3GPP TSG-RAN WG1 Meeting #113	R1-2305431

Incheon, Korea, May 22nd – 26th, 2023

Title:	[Draft] Reply LS to RAN2 on multicast reception in RRC_INACTIVE

Response to:	R1-2304325 (R2-2304330)

Release:	Rel-18

Work Item:	NR_MBS_enh-Core

Source:	RAN1,

To:	RAN2

Cc:

Contact Person:

Name:                    Teng Ma

E-mail Address:	mateng1@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 thanks RAN2’s LS in R2-2304330 (R1-2304325) and would like to provide RAN1’s responses to the following question asked by RAN2.

- Question 1: Is the following RAN2 assumption feasible? If feasible, whether both DCI format 4-1 and DCI format 4-2 are needed? - For MTCH, RAN2 assumes to reuse the same DCI formats of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used. - Question 2: Is the following RAN2 understanding correct? - RAN2 understanding is that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 for multicast MTCH in RRC_CONNECTED as well as for broadcast MTCH). - Question 3: Is it feasible to reuse the following Rel-17 CSS design for multicast MTCH and multicast MCCH? - 3.1) Reusing the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED). - 3.2) Separate CSS(es) for multicast MCCH and multicast MTCH in RRC_INACTIVE.

RAN1 would like to provide the following answer from RAN1’s perspective:

A1: RAN2’s assumption may not be feasible to use DCI format 4_0 scheduling MCCH and DCI format 4_1/4_2 scheduling MTCH for multicast in RRC_INACTIVE.

A2: Rel-17 PDSCH aggregation mechanism and principle can be reused for multicast MTCH in RRC_INACTIVE.

A3: It is feasible to reuse Rel-17 CSS design for multicast MTCH and multicast MCCH within the same CFR.

2. Actions:

To RAN2: RAN1 respectfully asks RAN2 to take the above response into account in the future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#114	21-25 August 2023   	Toulouse, FR

RAN1#114bis	9-13 October 2023   	Xiamen, CN