3GPP TSG RAN WG1 #113		                                                       R1-2305446

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	9.16.5

Source: 	OPPO

Title: 	Discussion on UE features for NR NTN enhancements

Document for:	Discussion and decision

- Introduction

This contribution includes an initial RAN1 UE feature list from our perspective for the Rel-18 WI of NR NTN enhancements.

- UE feature list for NR NTN enhancements

| X. NR_NTN_enh-Core | X-1 | MSG4 PUCCH repetition | - read repetition factor in SIB1 - read threshold in SIB1 - Send repetition request in MSG3 if RSRP<threshold - Send capability report in MSG3 if threshold is not configured or threshold value = X | [per UE/per band] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- |
| X. NR_NTN_enh-Core | X-1a | MSG4 PUCCH repetition | - Indicate repetition request in MSG3 - Indicate capable of MSG3 PUCCH repetition - Reinterpret XXX field in DCI format 1-0 with TC-RNTI, where the field is to indicate repetition factor - Perform MSG4 PUCCH repetition according to indicated repetition factor | [per UE/per band] | Optional with capability signalling |
| X. NR_NTN_enh-Core | X-2 | PUSCH DMRS bundling | - Determine actual TDW - Not perform TA adjustment within actual TDW | [per UE/per band] | Optional with capability signalling |
| X. NR_NTN_enh-Core | X-2a | PUSCH DMRS bundling | Indicate capable of performing TA adjustment within actual TDW while keeping phase continuity | [per UE/per band] | Optional with capability signalling |
| X. NR_NTN_enh-Core | X-3 | [multi-RTT for single serving satellite] | [indicate support multi-RTT for single serving satellite] | [per UE/per band] | Optional with capability signalling |