3GPP TSG RAN WG1 #113	 	                    R1-2305460

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	On sub use cases and other aspects of AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for:	Discussion and Decision

Introduction

In the recent RAN1 meetings, after several rounds of discussion on the representative sub-use cases of CSI feedback enhancement, CSI compression and time domain CSI prediction were selected as typical sub-use cases that need to be further studied. Other potential cases proposed in the early stage, e.g. CSI-RS configuration and overhead reduction, Resource allocation and scheduling, Joint CSI prediction and CSI compression, CSI accuracy enhancement based on traditional codebook and Temporal-spatial-frequency domain CSI compression using two-sided model, have been decided to lower the priority in R18.

In terms of potential standardization impact, agreements have been achieved on the training cooperation level, CSI reporting, CSI model output and data collection in RAN1#110[1], which are conducive to subsequent studies. In RAN1#110bis-e meeting[2], there were more discussions on performance monitoring and quantization issues. In RAN1#111 meeting[3], detailed agreements on training collaboration type 3 and quantization issues have been made. In RAN1#112 meeting[4], agreements/conclusions were achieved on CQI determination, CSI configuration/report, UE/NW side data collection and potential specification impact for intermediate KPIs based performance monitoring. In RAN1#112bis meeting[5], the pros/cons of different offline training collaboration types were discussed. Agreements on NW/UE-side model monitoring, ground-truth CSI and candidate quantization schemes were achieved.

In this contribution, we will continue to discuss the AI-based CSI enhancement, with the focus on corresponding specification impact.

CSI compression

2.1 Training collaboration

In RAN1#110 meeting, 3 different training collaboration types were agreed, i.e. (1) joint training of the two-sided model at a single side/entity, (2) joint training of the two-sided model at network side and UE side, and (3) separate training at network side and UE side respectively.

- For training collaboration type 1, UE side and NW side models will work well together as they are trained jointly. With aligned datasets, model structures and hyperparameter configurations, the training performance of type 1 is superior to the other two schemes, because the gradient quantization required by type 2 and the separate training required by type 3 may introduce unexpected performance loss during the training procedure. However, a drawback of type 1 training is it needs model exchange between different nodes. For example, after training a model on the UE side, the decoder needs to be transmitted to the NW for use, or vice versa. There will be concerns about model privatization among the participating nodes during above processes.

The potential standardization impact of training collaboration type 1 is mainly on the model transfer.

- For training collaboration type 2, the advantage is that UE and NW need not to reveal their local models to the other side. Only partial parameters or gradients (e.g. parameters or gradients corresponding to encoder output layers and/or decoder input layers) may need exchange between different nodes. However, this solution relies on the real-time interaction of FP/BP exchange between UE and NW. Besides, the overhead caused by FP and BP exchange would be extremely huge if the training process requires rounds of training to obtain a converged model. The potential standardization impact of training collaboration type2 is mainly on the demand for uplink and downlink transmission caused by periodic interactions for the FP/BP exchange. In addition, since the model is trained on UE and NW respectively, there are also potential standardization impacts on issues such as how to align the model interfaces as well as the quantification schemes

In RAN1#111 meeting, companies have agreed that “In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI”.

- For training collaboration type 3, it is also a solution to handle the concerns on model privatization. It is more flexible than the type 2 training as it does not require too much collaboration during the training process.

However, the disadvantage of type 3 scheme is it needs to transmit a large amount of training data. The performance of training depends on the quality and quantity of these training data, e.g. as discussed in [6], about 2GB training data is required and needs to be transmitted to ensure that the model performance obtained through training type 3 is equivalent to the model performance obtained through ideal training mode.

In addition, another concern related to training collaboration type 3 is that the upper bounder of the AI/ML model may be controlled by the node(NW or UE) providing training data to the other side. For example, the performance of UE-side local encoder will be limited by the training data set that obtained from NW side, e.g. in [6] it is obvious that the given training data set determines the upper bounder of the CSI model performance.

Besides, for NW first training, the CSI-input format adopted by UE may be constrained to the interface format of the dataset obtained from NW. For example, when the data input type given by NW is eigenvector while the model input type expected by UE is raw channel, the UE cannot effectively conduct training/inferencing directly from raw channel information.

In view of above concerns and uncertainties, for training collaboration type 3, the transmission of training data should meet the different needs of users in terms of model performance, transmission cost, data characteristics, and interface types.

Proposal 1: In training collaboration type 3,

- For NW first training, NW needs to be able to provide UE with training data sets that meet different requirements, e.g. on model performance, transmission cost, data characteristics and CSI input types

- For UE first training, UE needs to be able to provide NW with training data sets that meet different requirements, e.g. on model performance, transmission cost and data characteristics

In RAN1#112 meeting, a list of metrics to evaluate different training collaboration types has been concluded as below,

Conclusion (RAN1#112) In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects: - Whether model can be kept proprietary - Requirements on privacy-sensitive dataset sharing - Flexibility to support cell/site/scenario/configuration specific model - gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware - Model update flexibility after deployment - feasibility of allowing UE side and NW side to develop/update models separately - Model performance based on evaluation in 9.2.2.1 - Whether gNB can maintain/store a single/unified model - Whether UE device can maintain/store a single/unified model - Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use - Whether training data distribution can be matched to the device that will use the model for inference - Whether device capability can be considered for model development - Other aspects are not precluded - Note: training data collection and dataset/model delivery will be discussed separately

To further discuss the pros/cons of different training collaboration types, we give our views as below:

| Characteristics | Training collaboration type 1 | Training collaboration type 3 |
| --- | --- | --- |
| Whether model can be kept proprietary | No NW/UE needs to deliver a model or a sub-model to the other side | Yes Model or sub-model delivery is not needed |
| Requirements on privacy-sensitive dataset sharing | No | No Assume PMI is not privacy sensitive data. |
| Flexibility to support cell/site/scenario/configuration specific model | For NW side training: Yes, Cell/site/scenario/configuration specific model could be supported and delivered from NW to UE For UE side training: Yes. With assisted information signaling to identify cell/site/scenario /configuration for the data collection and/or model training Regarding “UE side training is less flexible than type1 NW side training” that discussed in RAN1 #112bis, clarification on this proposal by note would be helpful, e.g. as explained in the last meeting “consider NW side taking different decoders from UE can be quite difficult” | For NW side training: Semi-flexible, Cell/site/scenario/configuration specific model could be supported and delivered from NW to UE For UE side training: Semi-flexible. With assisted information signaling to identify cell/site/scenario/configuration for the data collection and/or model training Regarding “Semi-flexible” that discussed in RAN1 #112bis, clarification on this proposal by note would be helpful, e.g. as explained in the last meeting “dataset exchange for training type 3 can be an overhead” |
| gNB/device specific optimization | Support, under condition. For example, (1) NW/UE have to prepare models that match different devices/gNB. For example, when some devices are more compatible with transformer type models, while others require deployment of other model structures (CNN or fully connected networks), model providers must prepare multiple models to meet device specific optimization requirements. Alternatively, (2) NW/UE does not directly use the obtained decoder/encoder, but instead regenerates training data for matching devices based on the obtained model. | Support. For example, UE can achieve device specific optimization based on the obtained training data and its capabilities. |
| Model update flexibility after deployment | Support, Flexible NW can send new models to UE to update the UE side model, or vice versa For both type1 and type3 training, in the last meeting, more restrictive descriptions, e.g. “flexible or semi-flexible”, “with assisted info or not”, “conditional or not”, are proposed by companies. From our understanding, if we need to capture these restrictive descriptions in the table or in the TR eventually, clarifications/explanations for all the terms should be added in notes. Otherwise, the reasons and sources of these restrictive descriptions are hard to understand and aligned among companies. Regarding the “flexible or semi-flexible on model update flexibility after deployment”, in the last meeting, it proposed that offline effects bring the above differences. Examples of offline effect in note would be helpful. Regarding the “assisted information”, in the last meeting, it proposed that assisted information refers to the categorizing assisted information in data collection. | Support, Semi-flexible NW can send new training data to UE to update the UE side model, or vice versa |
| Feasibility of allowing UE side and NW side to develop/update models separately | Support with limitation For example, after deploying Model 1 on the UE side, a new UE model can be obtained by using Model 1 as the teacher model and using knowledge distillation method | Support For example, UE can retrain new models based on the obtained training data |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | For NW training: Yes For UE training: No | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | For NW training: No For UE training: Yes | Pending evaluation in 9.2.2.1 |
| Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Support with limitation As discussed above, after deploying Model 1 on the UE side, the update of the UE model can be achieved by using Model 1 as a teacher model and using knowledge distillation to obtain a new UE model | Support As discussed above, UE can retrain a new model by the obtained training data |
| Whether training data distribution can be matched to the device that will use the model for inference | Support, under condition. For NW side training: If UE directly uses the model passed to UE by NW, there is no guarantee that training data distribution can be matched to the device that will use the model for inference. Unless (1) NW prepares models that match to different devices, or (2) UE does not directly use the obtained encoder locally, but instead regenerates training data for matching devices based on the obtained encoder. | Support, under condition. For NW side training: If UE directly uses the data passed to UE by NW, there is no guarantee that the training data distribution can be matched to the UE that will use the model for inference. Unless (1) NW prepares different training data that match to different devices, or (2) UE does not directly use the obtained training data locally, but instead adjusts/ regenerates training data for matching devices based on the obtained training data sets. |
| Whether device capability can be considered for model development | Support, under condition. For NW training: - For device specific model: Yes - For device-agnostic model: No For UE training: Yes | Support NW/UE can construct and train local models based on received training data and their own abilities to complete Type 3 training. However, due to mismatched CSI encoders and decoders, it may result in extra performance loss. |

It can be seen that for training collaboration type1 and type3, the conclusions for most questions are the same, but the implementation methods are different. The core feature of training collaboration type1 still lies in the need for model transfer/delivery, as well as the secondary utilization and redevelopment based on the obtained model, without any additional performance loss caused by training collaboration types. The core features of type3 are the demand for training data transmission, as well as device level optimization and utilization based on the obtained data, but there is potential performance loss. From our perspective, these two training collaboration types are not exclusive and both can be considered in subsequent research

Observation 1: For training collaboration types 1 and type 3, the conclusions for most of the questions listed below are the same, but the implementation methods are different. These two training collaboration types are not exclusive, and both can be considered in subsequent research

For all the listed training collaboration types(NW/UE side training, NW/UE first training) and characteristics, in the last meeting, several restrictive descriptions, e.g. “flexible or semi-flexible”, “with assisted info or not”, “conditional or not”, are proposed by companies[7]. From our understanding, if restrictive descriptions are needed to differentiate the pros and cons for different training types and different characteristics in a table or in the TR eventually, clarifications/explanations for all restrictive terms as well as the scope (e.g. for the whole table, or for a given characteristic, or a given training type) should be added e.g. in notes. Otherwise, the reasons and sources of these restrictive descriptions are hard to understand and aligned among companies.

Observation 2: If restrictive descriptions are needed to differentiate the pros and cons for different training types and different characteristics in a table or in the TR eventually, clarifications/explanations for all restrictive terms as well as the scope (e.g. for the whole table, or for a given characteristic, or a given training type) should be added e.g. in notes.

2.2 Inference related spec impact

2.2.1.  CSI Report

Regarding CSI report, in the legacy CQI calculation, UE could derive the PMI for a CSI report and then calculate the CQI based on the PMI. The CQI in UE side is aligned with the on in NW since the same PMI is assumed to generate the precoder. For AI/ML-based CSI compression, if a UE does not have the decoder for CSI recovery, it is hard for the UE get to know the output CSI recovered by the CSI reconstruction part on NW side. Therefore, misalignment for the CQI calculation arises during the CSI feedback procedure.

In the last meeting, agreements on this issue have been achieved as below,

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

For option1, the basic assumption is that UE does not have a well-trained decoder for inference and cannot obtain the accurate output CSI to align with NW-side CSI. To handle this problem, different schemes are proposed, e.g. CQI can be calculated based on target CSI from realistic channel, or CQI can be calculated based on traditional codebook.

For option2, the CSI reconstruction part could be used/ should be available on the UE side and there would be no gap between the CQI calculated by UE and the one calculated by NW.

For the listed options/sub-options, it is necessary to first verify the feasibility of some solutions. For example, how to compensate for the gap between CSI calculated on the UE and NW sides. In addition, if some compensation operations can be performed, they can also be handed over to the implementation for resolution without the need for standardization. For some schemes, there are significant issues with additional RS and extra signaling overhead, such as using a two-stage method to calculate CQI. In addition, it is also necessary to consider the complexity of each CQI calculation method. Higher complexity will bring additional system burden. For example, when calculating RI, if a method based on traversing different ranks is used to determine the target RI, a highly complex CQI calculation scheme will bring huge computational overhead and latency. According to our understanding, if the impact of the different schemes mentioned above on system scheduling is not significant, there is no need to introduce additional complex mechanisms to avoid the impact on CQI determination, i.e., CQI could be calculated just based on the target CSI with realistic channel measurements.

Proposal 2: CQI should be calculated based on target CSI with realistic channel measurement

2.2.2.  Input of CSI generation part

Regarding the input of CSI generation part, companies have different views on whether the input of CSI compression model needs to be standardized. One proposal is the interface does not need to be defined in 3GPP. For example, if the training and model delivery are achieved by third parties and transparent to 3GPP, the interface does not need to be defined in 3GPP protocols. Different manufacturers can process the acquired channel information according to their own requirements. The other proposal is to clearly define the model input interface in 3GPP scope, e.g. when a gNB transmits an AI/ML model to a UE, a clear interface definition(e.g. data type, dimension size, input range, normalization/quantification schemes) is conductive to better use of the model. Otherwise, it will be so difficult for the UE to determine which information can be used as the input of the CSI compression model.

From our understanding, when UE obtains the CSI generation part from NW, a clear interface definition is required. The only difference is whether this definition is determined outside 3GPP or within the 3GPP protocol. For the first one, it can be considered as a kind of hypothesis that does not need too much 3GPP protocol supports. For the second one, it is necessary to study whether some corresponding standardization works will be required for the interface description and indication, e.g. content and configuration.

Proposal 3: Regarding the CSI input,

- when UE obtains the CSI generation part from NW in a 3GPP non-transparent way, the network needs to explicitly or implicitly indicate the input interface format of the CSI generation part, e.g. data type, dimension size, normalization/quantification schemes.

- when UE obtains the CSI generation part in a 3GPP transparent way, no need to indicate the input interface through 3GPP protocols

2.2.3.  Quantization

For the quantization, a basic principle is the quantization/dequantization method should be aligned between UE and NW.

From our understanding, at least two factors need to be addressed. One is whether the quantitative function is involved in the model training process. In the last meeting, three cases have been agreed for further discussion, including,

- Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training

- Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase

- Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase

Case 1 is a basic solution in which the quantization/dequantization module can be added after the training procedure.

For case 2-1, quantization/dequantization module is involved in the CSI model training process, but the quantization module itself does not need to be trained or updated, e.g. a fixed scalar quantization/dequantization scheme is utilized. The advantage is that the impact of quantization/dequantization has been considered when training a CSI encoder and a corresponding decoder, so that better CSI compression and recovery performance can be expected from the training results.

For case 2-2, quantization/dequantization module is involved in the CSI model training process, and the quantization module itself also needs training and updating during the CSI model training process, e.g. a trainable vector quantization/dequantization scheme is embedded into the CSI model, it can make the quantization/dequantization scheme better match the CSI data to be quantified and the CSI model to be used.

The benefits of different quantization/dequantization methods need to be checked through solid simulations. In addition, the training complexity, calculation complexity, signaling cost for indication and standardization impact of different quantization/dequantization methods also need to be further evaluated. If the quantization scheme is not a key contributor to CSI compression/recovery performance, quantization schemes that are relatively simple, easy to indicate and have less standardization impact should be selected. An example is shown in table1, the comparison on different quantization schemes is presented. It can be seen that vector quantization(VQ) methods utilized in the simulation do not show a significant performance gain compared to the scalar quantization(SQ) schemes. Actually, compared to some SQ design(e.g. 2bit/3bit mixed SQ), the quantization performance of VQ may not be the optimal one[6].

Table 1 Comparison on different quantization schemes

| Quantization | 2bit SQ | 3bit SQ | VQ method 1 | VQ method 2 | 2bit/3bit mixed SQ |
| --- | --- | --- | --- | --- | --- |
| SGCS | 0.846 | 0.852 | 0.869 | 0.853 | 0.878 |

Note: the total feedback payload is 174bit for each approach.

Proposal 4: The performance gain, training complexity, inference complexity, signaling cost for indication and standardization impact of different quantization/dequantization methods need to be evaluated.

- If the quantization/dequantization scheme is not a key contributor to CSI compression/recovery performance, the quantization/dequantization scheme(s) that is relatively simple, easy to indicate and have less standardization impact(e.g. SQ in case 2-1) should be selected first.

The impact of different training types on the use of quantization/dequantization schemes should be considered as well. For training collaboration type 1, the alignment of quantization/dequantization modules can be solved through implementation. For training collaboration type 3, especially when quantization/dequantization modules are involved in the CSI model training process, it may be necessary to specify the quantization/dequantization methods to ensure the encoder and encoder to be well trained and could work together.

Proposal 5: In CSI compression using two-sided model use case, further study potential specification impact on the quantization/dequantization method for the compressed CSI, including

- At least for training collaboration type3, quantization/dequantization methods should be specified and aligned to ensure the encoder and encoder to be well trained and could work together

  - For NW first training, network should indicate the quantization [or the dequantization] method for the compressed CSI to UE.

  - For UE first training, UE should indicate the dequantization [or the quantization] method for the compressed CSI to NW.

- Study potential signaling and procedure to indicate the quantization/dequantization method

2.3 Life cycle management

LCM is a key issue that has been discussed in the General aspects of AI/ML framework. Based on the agreement, the definition and necessity on data collection, model training, [model registration], model deployment, [model configuration], model inference operation, model selection/activation/deactivation/switching/ fallback operation, model monitoring, model update, model transfer, UE capability need to be further studied.

Regarding the performance monitoring metrics/methods, several options have been agreed in the last meeting for further study, e.g.

- Intermediate KPIs as monitoring metrics (e.g., SGCS)

- Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

- Legacy CSI based monitoring: schemes using additional legacy CSI reporting

- Other monitoring solutions, at least including the following option:

  - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

From our understanding, similar to the RLF/BFR procedures defined in previous releases, for the AI/ML model monitoring, eventual KPIs(e.g., hypothetical BLER) should be utilized for the performance monitoring. Other options can be used to equivalent convert into the eventual KPI by implementation. For example, UE can estimate the system performance by measuring/monitoring some intermediate results, e.g. UE can estimate the change of BLER/ throughput through the measurement of SGCS. This kind of evaluation may also require the network to provide some assisted information, e.g. labels for SGCS evaluation. Besides, UE can try to evaluate the impact of data drift between training dataset and observed dataset on eventual system performance as well. For example, when UE obtains the AI/ML model, it can also obtain some distribution information of the expected input data, or directly obtain the data drift evaluation model that matches the CSI model to be used.

Proposal 6: Regarding the performance monitoring metrics/methods for AI/ML model monitoring, eventual KPIs(e.g., hypothetical BLER) should be utilized for the performance monitoring, other options can be used to equivalent convert the eventual KPI.

For the performance monitoring, it could be done either on UE side or NW side. For UE-side monitoring, UE monitors the model performance and report results to NW. NW side makes the decision of model activation/deactivation/updating/switching. For NW-side monitoring, NW monitors the model performance and make decisions by itself. In the last meeting, agreements on this issue have been made as below

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

In addition to discussing how to perform one-shot performance evaluation, the stability of the performance evaluating and decision-making mechanism should be further studied to avoid the interference of random effects on the evaluation results. For example, multiple attempts within an evaluation window both in PHY and high layers would be helpful to obtain a relatively stable evaluation result. And then effectively trigger subsequent actions, such as model updating and switching. Besides, multi-user involved mechanism should be addressed as well, e.g. it is necessary to consider whether the model should be updated or optimized if only parts of the UE report the AI/ML model is failure. If NW makes a model failure judgment until receiving model failure indications from a large amount of UE, it may cause a delayed model update. While if NW directly updates the AI/ML model after receiving a few users’ model failure indications, it may lead to ping-pong switching of AI/ML models and cause unnecessary control overheads.

Proposal 7: The stability of the performance evaluating and decision-making mechanism should be further studied to avoid the interference of random effects on the evaluation results.

- multiple attempts within an evaluation window both in PHY and high layers would be helpful to obtain a relatively stable evaluation result

- multi-user involved mechanism should be addressed

2.4 Capability

Different UE may have different AI/ML capabilities to support different AI/ML feature/FGs. In 9.2.1 general part, discussions on how to align NW side and UE side AI/ML capability(or align the supported AI/ML feature/FG) are still ongoing. For CSI compression, as a typical two-sided model, the necessity on AI/ML capability alignment between NW and UE is more prominent. From our understanding, follow options to align NW/UE side AI/ML capability(or supported AI/ML feature/FG) can be considered for further study

Option1： UE initial, including

Option1-1: UE capability report on whether AI/ML based CSI feedback(or AI/ML based CSI generation) is supported

UE reports its AI/ML capability on AI/ML based CSI feedback (e.g. 1bit indication for whether AI/ML based CSI generation is supported). According to the obtained UE side AI/ML capability, NW could decide whether to activate AI/ML based CSI feedback, or whether to trigger performance monitoring for AI/ML based CSI feedback.

Option1-2: UE capability report on whether AI/ML based CSI feedback(or AI/ML based CSI generation) is supported, NW indication on whether AI/ML based CSI feedback(or AI/ML based CSI reconstruction) is supported.

UE reports its AI/ML capability on AI/ML based CSI feedback(e.g. 1bit indication for whether AI/ML based CSI generation is supported). After that, NW indicates UE whether AI/ML based CSI feedback feature is supported.

According to the UE side AI/ML capability(or supported AI/ML feature/FGs) and NW side AI/ML capability(or supported AI/ML feature/FGs), NW could decide whether to activate AI/ML based CSI feedback, or whether to trigger performance monitoring for AI/ML based CSI feedback. UE could decide whether to trigger performance monitoring for AI/ML based CSI feedback, if needed.

Option2：NW initial, including

Option2-1: NW indication on whether AI/ML based CSI feedback(or AI/ML based CSI reconstruction) is supported

NW indicates whether AI/ML based CSI feedback is supported(e.g. 1bit indication for whether AI/ML based CSI reconstruction is supported). According to the obtained NW side AI/ML feature/FGs, UE could decide whether to trigger performance monitoring for AI/ML based CSI feedback, if needed.

Option2-2: NW indication on whether AI/ML based CSI feedback(or AI/ML based CSI reconstruction) is supported, UE indication on whether AI/ML based CSI feedback(or AI/ML based CSI generation) is supported

NW indicates whether AI/ML based CSI feedback is supported(e.g. 1bit indication for whether AI/ML based CSI reconstruction is supported ).  After that, UE indicates whether AI/ML based CSI feedback is supported(e.g. 1bit indication for whether AI/ML based CSI generation is supported).

According to the UE side AI/ML capability(or supported AI/ML feature/FGs) and NW side AI/ML capability(or supported AI/ML feature/FGs), NW could decide whether to activate AI/ML based CSI feedback, or whether to trigger performance monitoring for AI/ML based CSI feedback. UE could decide whether to trigger performance monitoring for AI/ML based CSI feedback, if needed.

Proposal 8: To align NW/UE side AI/ML capability(or supported AI/ML feature/FG), follow options could be considered,

Option1： UE initial, including

Option1-1: UE capability report on whether AI/ML based CSI feedback(or CSI generation) is supported

Option1-2: UE capability report on whether AI/ML based CSI feedback(or CSI generation) is supported, NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported.

Option2：NW initial, including

Option2-1: NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported

Option2-2: NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported, UE indication on whether AI/ML based CSI feedback(or CSI generation) is supported

Conclusion

In this contribution, we provide some discussions on possible specification impacts on AI/ML for the CSI feedback enhancement. Based on the discussions, we have following proposals:

Observation 1: For training collaboration types 1 and type 3, the conclusions for most of the questions listed below are the same, but the implementation methods are different. These two training collaboration types are not exclusive, and both can be considered in subsequent research

Observation 2: If restrictive descriptions are needed to differentiate the pros and cons for different training types and different characteristics in a table or in the TR eventually, clarifications/explanations for all restrictive terms as well as the scope (e.g. for the whole table, or for a given characteristic, or a given training type) should be added e.g. in notes.

Proposal 1: In training collaboration type 3,

- For NW first training, NW needs to be able to provide UE with training data sets that meet different requirements, e.g. on model performance, transmission cost, data characteristics and CSI input types

- For UE first training, UE needs to be able to provide NW with training data sets that meet different requirements, e.g. on model performance, transmission cost and data characteristics

Proposal 2: CQI should be calculated based on target CSI with realistic channel measurement

Proposal 3: Regarding the CSI input,

- when UE obtains the CSI generation part from NW in a 3GPP non-transparent way, the network needs to explicitly or implicitly indicate the input interface format of the CSI generation part, e.g. data type, dimension size, normalization/quantification schemes.

- when UE obtains the CSI generation part in a 3GPP transparent way, no need to indicate the input interface through 3GPP protocols.

Proposal 4: The performance gain, training complexity, inference complexity, signaling cost for indication and standardization impact of different quantization/dequantization methods need to be evaluated.

- If the quantization/dequantization scheme is not a key contributor to CSI compression/recovery performance, the quantization/dequantization scheme(s) that is relatively simple, easy to indicate and have less standardization impact(e.g. SQ in case 2-1) should be selected first.

Proposal 5: In CSI compression using two-sided model use case, further study potential specification impact on the quantization/dequantization method for the compressed CSI, including

- At least for training collaboration type3, quantization/dequantization methods should be specified and aligned to ensure the encoder and encoder to be well trained and could work together

  - For NW first training, network should indicate the quantization [or the dequantization] method for the compressed CSI to UE.

  - For UE first training, UE should indicate the dequantization [or the quantization] method for the compressed CSI to NW.

- Study potential signaling and procedure to indicate the quantization/dequantization method

Proposal 6: Regarding the performance monitoring metrics/methods for AI/ML model monitoring, eventual KPIs(e.g., hypothetical BLER) should be utilized for the performance monitoring, other options can be used to equivalent convert the eventual KPI.

Proposal 7: The stability of the performance evaluating and decision-making mechanism should be further studied to avoid the interference of random effects on the evaluation results.

- multiple attempts within an evaluation window both in PHY and high layers would be helpful to obtain a relatively stable evaluation result

- multi-user involved mechanism should be addressed

Proposal 8: To align NW/UE side AI/ML capability(or supported AI/ML feature/FG), follow options could be considered for further study

Option1： UE initial, including

Option1-1: UE capability report on whether AI/ML based CSI feedback(or CSI generation) is supported

Option1-2: UE capability report on whether AI/ML based CSI feedback(or CSI generation) is supported, NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported.

Option2：NW initial, including

Option2-1: NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported

Option2-2: NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported, UE indication on whether AI/ML based CSI feedback(or CSI generation) is supported

References

- Chair’s notes for RAN1#110

- Chair’s notes for RAN1#110bis-e

- Chair’s notes for RAN1#111

- Chair’s notes for RAN1#112

- Chair’s notes for RAN1#112bis

- R1- 2305459 Evaluation methodology and results on AI/ML for beam management

- R1- 2303982 Summary #4 on other aspects of AI/ML for CSI enhancement