3GPP TSG RAN WG1 #113		R1-2305464

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	On sub use cases and other aspects of AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.2

Document for:	Discussion and Decision

- Introduction

As a promising technology, AI/ML based solutions have been widely used and has shown impressive performance in various areas, e.g., computer vision, speech recognition, natural language processing (NLP), language translation and so on.

Motivated by the great success of AI/ML in the afore-mentioned areas, 3GPP also started study on AL/ML for wireless communications in SA2 and RAN3. On the basis of the existing study in 3GPP, RAN#94e meeting decided to start a new study item (SI) on AI/ML for NR air interface in Rel-18 [1]. According to the SID, this study will focus on three typical use cases of physical layer in the first step:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In the recent RAN1 meetings, there were intensive discussions on the typical use cases and the potential spec impacts. Some achieved agreements are as below [2][3][4]:

Agreement For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI. Agreement Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305 Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects - Request/report of training data - Ground truth label - Measurement corresponding to model input - Associated information of ground truth label and/or measurement corresponding to model input - Assistance signaling and procedure to facilitate generating training data - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc. - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed - Note2: Study may consider different entity to generate training data as well as different types of training data when applicable - Note3: study considers both of the following cases when applicable - when the training entity is the same entity to generate training data - when the training entity is not the same entity to generate training data Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network Agreement For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning are selected as representative sub-use cases. Agreement Regarding training data generation for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods - Network entity generates ground truth label based on positioning methods - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study Agreement Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects - Associated information of training data - Quality indicator at least for ground truth label (if needed) - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc. - Assistance signaling and procedure to facilitate generating/collecting training data - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signaling other than above 2 for data collection - E.g., requested quality of training data Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded Agreement Regarding monitoring for AI/ML based positioning, at least the following entities are identified to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model) Working Assumption Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified. - Ground truth label - At least for model training - Report from the label data generation entity - Measurement (corresponding to model input) - At least for model training - Report from the measurement data generation entity - Quality indicator - For and/or associated with ground truth label and/or measurement at least for model training - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity - RS configuration(s) - At least for deriving measurement - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement - Time stamp - At least for and/or associated with training data for model training - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities - Report from data generation entity together with training data and/or as LMF assistance signaling - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement - FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection - Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed - Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective Agreement Regarding monitoring for AI/ML based positioning, at least the following aspects are identified for further study on benefit(s), feasibility, necessity and potential specification impact for each case (Case 1 to 3b) - Assistance signaling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring - Assistance signaling from UE/PRU for network-side model monitoring - Model monitoring based on provided ground truth label (or its approximation) - Monitoring metric: statistics of the difference between model output and provided ground truth label - Provisioning of ground truth label and associated label quality - Model monitoring using at least statistics of measurement(s) without ground truth label - Monitoring metric: e.g., statistics of measurement(s) compared to the statistics associated with the training data - Note1: the measurement(s) may or may not be the same as model input - Note2: other monitoring methods (e.g., based on statistics of model output without ground truth label, based UE motion sensor and/or jointly based on multiple monitoring metrics) are not precluded Agreement Regarding LCM of AI/ML based positioning accuracy enhancement, at least for Case 1 and Case 2a (model is at UE-side), further study the following aspects on information related to the conditions - What are the conditions for functionality-based LCM - which aspects should be specified as conditions of a Feature/FG available for functionality - What are the conditions for model-ID-based LCM - Which aspects should be considered as additional conditions, and how to include them into model description information during model identification

In this contribution, we will continue to discuss the use case of AI-based positioning accuracy enhancement, with the focus on sub use cases, the collaboration levels and the corresponding spec impacts on NR.

Discussion

Typical sub use cases/schemes

During the evolution of R16 NR, 3GPP developed multiple RAT-based positioning methods to provide high accuracy positioning for NR cellular system:

- DL-TDOA positioning

- UL-TDOA positioning

- DL-AoD positioning

- UL-AoA positioning

- Multi-RTT positioning

In RAN1#111 meeting, the following two types of AI/ML based positioning schemes were selected as the representative sub-use cases:

- Cat.1: Direct AI/ML positioning

- Cat.2: AI/ML assisted positioning

In order to facilitate the subsequence discussions, we take the following schemes as examples for direct AI/ML positioning:

- Direct: DL RSTD

  - UE measures and reports the DL RSTD results by following the existing mechanism.

    - Optionally, RSRP measurements can also be measured and reported. In this case, we refer to it as “Direct: DL RSTD + RSRP”.

  - LMF uses an AI model (e.g., neural network) to estimate the location of a given UE based on the reported Dl RSTD measurement results.

- Direct: Normalized CIR

  - UE estimates the channel impulse response (CIR) and reports its normalized versions to LMF.

    - Optionally, RSRP measurements can also be measured and reported. In this case, we refer to it as “Direct: Normalized CIR + RSRP”.

  - LMF uses an AI model to estimate the location of a given UE.

We can have similar schemes for AI/ML assisted positioning as below

- Assisted: an existing type of measurement

- Assisted: a new type of measurement

Based on the discussion on previous meetings, one aspect of AI/ML assisted positioning need to be clarified. During the discussion, different companies have different understanding on the AI/ML assisted positioning regarding the information of how many TRPs are used as the input of the AI/ML model inference. Speaking of this, there are two different alternatives:

- Alt.1: The information of single TRP is used as the input for AI/ML model inference. The same model is used for each TRP.

- Alt.2: The information of all measurement TRPs are used as the input for AI/ML model inference.

According to the evaluation results in our companion tdoc, the performance of Alt.1 if TOA is the output of AI/ML model is much worse than Alt.2, and Alt.1 cannot meet the requirement of positioning accuracy. Thus, Alt.2 should be focused on for AI/ML assisted positioning.

Proposal 1: For AI/ML assisted positioning, the following alternative should be prioritized if the TOA-like output is used for AI/ML model

- The measurement results corresponding to all TRPs are used as the input for AI/ML model inference (i.e., Multi-TRP construction).

In our understanding, AI/ML assisted positioning is an intermediate step for UE-based positioning method, and it is likely transparent to the NW. Thus, in the following discussion, we will only focus on UE-assisted or NG-RAN node assisted positioning method when we are talking about AI/ML assisted positioning.

Data collection

Data collection for AI model training

To train an AI/ML model, the positioning-related measurement results and associate ground-truth labels are used. For direct AI/ML positioning, the ground-truth labels are the location of the UEs that do the measurement. For AI/ML assisted positioning, the ground-truth labels are the ideal information of UE measurement/reporting (e.g., LOS/NLOS identification, RSTD, etc.).

In RAN1#112 meetings, it was agreed that PRU or some entity with known PRU location can generate ground truth labels for UE-based positioning with UE-side model (Case 1), UE-assisted positioning with UE-side model (Case 2a), UE-assisted/LMF-based positioning with LMF-side model (Case 2b), NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b). There is an FFS part on whether UE/NW entity can generate ground-truth label based on non-NR and/or NR RAT-dependent positioning methods.

Generally speaking, no matter what the contents of the labels are, the ground-truth labels are difficult to be obtained for UE. In theory, a normal UE can get some information of its own location via NR positioning or some other technologies, e.g., GPS, UWB and so on. However, there some potential challenges for normal UE to ground-truth labels, e.g.,

  - Other technologies may not be available for some scenarios

  - The accuracy of location information is uncertainty and cannot be guaranteed

In our companion tdoc submitted Agenda item 9.2.4.1, we show the evaluation results for an AI model trained with the tables obtained from NR RAT-dependent positioning methods. From these evaluation results, we can have the following observation:

Observation 1: For UE based and UE-assisted positioning method, if the model for direct AI/ML positioning is trained based on the labels that obtained by NR DL-TDOA scheme,

- there is significant performance degradation compared to the ideal ground-truth labels

- there is no obvious performance gain compared to legacy NR positioning methods

In summary, NW and normal UE seem unable to collect the data with ground-truth labels for AI/ML model training. Moreover, if the labels are obtained based on existing NR RAT-dependent positioning methods, there will be large performance loss for the trained AI/ML model. However, it is possible for UE and TRP to collect the measurement results without ground-truth labels.

Proposal 2: For AI/ML based positioning, regarding the data collection for AI model training, NOT support UE to report the ground-truth labels of its location(s)

If a UE-side model is also trained at UE side (e.g., a server deployed by UE vendor to train AI model), UE will collect training data under proper PRS configurations. Different UE may have different capability for training data collections. Thus, it would be beneficial to send UE’s request of preferred or supported configurations to NW so that LMF can chose a better and more efficient configurations.  Whether this request information is reported via UE capability signaling or other signaling can be discussed further.

Proposal 3: For the training data collection at UE side (e.g. UE-based positioning with UE-side model (Case 1), UE-assisted positioning with UE-side model (Case 2a)), study the potential spec impact on that UE sends request of preferred or supported configurations to LMF

- FFS: Whether this request is sent by UE capability signaling or other signaling.

Regarding UE-assisted/LMF-based positioning with LMF-side model (Case 2b), there may be various types of training data for AI model training:

- PRU: PRU can collect the measurement results corresponding to model input. The corresponding label(s) can be reported along with these measurement results, or LMF maybe know the label(s) in advance.

- UE: UE may collect and report the measurements results without labels. This kind of data is used for semi-supervised learning.

For these two cases, LMF needs to signal the PRS configurations for measurement.

Proposal 4: For the training data collection at NW side for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), study the potential spec impact (including necessity and benefit) of the following options

- PRU: PRU reports measurement results (corresponding to model input) optionally with labels via LPP or other new signaling defined by RAN2.

  - Other network entity (e.g., LMF) maybe know the label(s) in advance

- UE: UE reports measurement results (corresponding to model input) without labels via LPP or other new signaling defined by RAN2

  - This kind of data is used for semi-supervised learning.

Regarding NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b), there may be various types of training data for AI model training:

- TRP can collect the measurement results (corresponding to model input) based on PRU’s transmission. The corresponding label(s) can be derived from the know location of correspond PRUs.

- TRP can collect the measurement results (corresponding to model input) based on UE’s transmission. There may be no corresponding label(s).

For the above data collection, gNB needs to signal the configuration of SRS or SRS for positioning.

Proposal 5: For the training data collection at NW side NG-RAN node assisted positioning with gNB-side model (Case 3a) or NG-RAN node assisted positioning with LMF-side model (Case 3b), study the potential spec impact (including necessity and benefit) of the following options

- TRP can collect the measurement results (corresponding to model input) based on PRU’s transmission and report them via LPP or other new signaling defined by RAN2.

  - Other network entity (e.g., LMF) generates the associated labels based on the know location of the corresponding PRU

- TRP can collect the measurement results (corresponding to model input) based on UE’s transmission and report them via LPP or other new signaling defined by RAN2.

  - This kind of data is used for semi-supervised learning.

In RAN1#112bis-e, regarding the generation of labels, Proposal 1-1-1a was suggested for discussion. As there is no consensus on whether this proposal is workable or not, it was not agreed.

(Proposal 1-1-1a) Regarding ground truth label generation for AI/ML based positioning, the following options of entity to generate ground truth label are identified (in addition to entities from previous agreement)

- UE generates ground truth label when label quality satisfy the requirement

  - based on non-NR and/or NR RAT-dependent positioning methods

  - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a)

  - FFS potential specification impact, e.g., the required label quality, necessary assistance signaling to UE

    - Note: user data privacy needs to be preserved

- Network entity generates ground truth label when label quality satisfy the requirement

  - based on positioning methods

  - At least for UE-assisted/LMF-based positioning with LMF-side model (Case 2b),  NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b)

  - FFS potential specification impact, e.g., the required label quality

In our companion tdoc, we evaluated the impact of the non-ideal labels on AI-based positioning via two different ways to obtain/model the non-ideal labels for various cases/settings. The main observations are summarized as below:

- The non-ideal labels are obtained via legacy NR positioning

  - If the training data are all obtained by legacy NR positioning, AI-based positioning has no performance gain compared to legacy NR positioning.

  - If there is some training data with ideal (e.g., 5000 training data), the AI-based positioning will suffer performance degradation when additional training data with non-ideal labels are used together for AI model training.

- The non-ideal label is modelled by truncated Gaussian distribution

  - Accuracy of AI-based positioning is similar to or worse than the labels. That means the AI-based positioning will not provide any performance gain compared to the scheme by which the labels are obtained.

Based on the above observations, we can see that Proposal 1-1-1a is not workable for the positioning accuracy improvement as it does not provide any performance gain. Accordingly, the first step should be to study its feasibility and benefits. Meanwhile, there are some other positioning methods which may provide more training data for AI/ML model training.  Thus, we have the following proposal:

Proposal 6: Regarding ground truth label generation for AI/ML based positioning, study the feasibility, necessity and potential benefits for the following option

- UE/NW generates ground truth label when label quality satisfies the requirement

  - based on non-NR and/or NR RAT-dependent, and/or RAT-independent (e.g., WIFI, blue tooth, UWB, sensor, etc.) positioning methods

  - FFS: label quality

  - FFS: requirement

Data collection for AI model inference

When direct AI/ML positioning is used for UE-assisted positioning method, the input of AI model inference will be based on UE reporting of the measurement results. If the existing type of measurement results are reused, the current spec/signaling/procedure can be reused to facilitate the AI model inference. If new type of measurement result is justified and introduced, some new or enhanced signaling/mechanism for the reporting of new measurement type will be needed.

When direct AI/ML positioning is used for UE-base positioning method or AI/ML assisted positioning is used, the input will be based on the measurement results at UE side. If the model is trained by UE/chipset vendor, the inputs are up to UE implementation and are likely transparent from the perspective of air interface. On the other side, if the model is trained by NW vendor, the size/content of inputs will have to be pre-defined/pre-configured. Similar discussions are also applicable to the NG-RAN node assisted positioning method. Thus, we have the following proposal.

Proposal 7: For the data collection used for AI model inference

- When direct AI/ML positioning is used for UE-base positioning method (Case 1) or AI/ML assisted positioning is used for UE-assisted positioning method (Case 2a), the UE will collect measurement for the input of AI model

  - If the model is trained at the same side, the inputs/data collection are up to UE implementation and transparent from the perspective of air interface

  - If the model is trained at NW side and AI model inference is performed at UE side (when model transfer is supported), the size/contents of inputs will need to be pre-defined or pre-configured.

- When direct AI/ML positioning is used for UE-assisted positioning method (Case 2b), the target UE will report the measurement results to LMF via LPP signaling

  - FFS: type of measurement (e.g., existing measurement type, new measurement type), RS configuration for measurement

- When AI/ML assisted positioning is used for NG-RAN node assisted positioning method (Case 3a), the TRP will collect measurement for the input of AI model by implementation and transparent from the perspective of specification

- When direct AI/ML positioning is used for NG-RAN node assisted positioning method (Case 3b), the TRP will report the measurement results to LMF via NRPPa signaling

AI model training and deployment

The main phases of AI/ML based positioning consist of model training and inference. The collaboration levels and potential spec impacts will be different for model training phase and inference phase, respectively. Thus, we prefer to discuss model training and inference separately.

In generally, there are two types of AI model training: online training and offline training. Online training can update the AI model upon the reception of timely samples, which may be more flexible for the dynamic operation environment. On the other hand, online training will suffer more issues:

- How to evaluate/justify the performance/benefit of online training

  - The well-established methodology for online training of AI model is still missing in 3GPP

- More spec impact on NR system

  - E.g., new procedure(s)/signal(s) to support online training

- Potential more overhead due to data collection/information sharing

- Feasibility for UE/gNB to deploy an updated AI/ML model in (nearly) real-time manner

- …

In RAN2#121bis-e meeting, online training was agreed to be deprioritized from RAN2 perspective for the whole study item. The corresponding agreement is copied as below

R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

Considering the limited timeline of Rel-18 AI study and the afore-mentioned RAN2 agreement, we prefer to focus on offline training in R18 and continue to study online training in later release(s).

Proposal 8: For both direct AI/ML positioning and AI/ML assisted positioning, offline training of AI model(s) is prioritized in Rel-18

- Enhancements dedicated to online training can be discussed in the future release(s)

If the AI model is trained by one side and the inference is performed at the other side, it will have to support model transfer between these two sides, which is expected to lead to more challenges for specification design, product implementation and practical deployment, e.g.,

- More standardization efforts will be needed for the mechanism supporting AI/ML model transfer between different nodes (e.g., AI model is transferred from network to UE, or vice versa).

- Interoperability and the corresponding test will require a significant amount of effort.

- The optimization/deployment of transferred AI/ML model may require advanced UE capability.

Thus, we suggest to focus on the schemes with the model training and inference are done in the same side and without model transfer in Rel-18.

Proposal 9: For both direct AI/ML positioning and AI/ML assisted positioning, the AI model training and inference are assumed to be done in the same side in Rel-18, i.e., no model transfer

- AI model training and inference at UE side, or

- AI model training and inference at NW side

- Study model transfer in future release(s).

AI model inference

For UE-based positioning method, if AI model is used to estimate the location, then estimated location information can be reported by existing LPP signaling. One potential enhancement is to report the associated confidence information of estimated location additionally.

Regarding the direct AI/ML scheme for UE-assisted positioning method, there are some potential enhancements from the following aspects:

- Whether can some new type of measurement/reporting improve the performance of AI model? If so, what’s the new measurement/reporting type and the associated LPP signaling from UE to LMF?

- The current measurement/reporting can be used as the input of AI model in a transparent way. Whether there is some enhancement needed to improve the AI model performance based on existing measurement/reporting, e.g., more bits to report the existing measurement results. The parts will involve the LPP signaling from UE to LMF.

- If any of the above enhancement is introduced, then some new LPP signaling from LMF to UE will be needed for the configuration/triggering of measurement/reporting.

The above-mentioned aspects are also applicable to other cases/combinations in similar way. Table 2 provides a summary on the spec impacts of AI model inference.

Table 2: Spec impacts of AI model inference

|  | Positioning method | AI model inference at which side | Spec impacts |
| --- | --- | --- | --- |
| Direct AI/ML positioning | UE-based (Case 1) | UE | 1. whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF) 2. whether some additional assistant information is needed or not |
|  | UE-Assisted (Case 2b) | LMF | 1. whether/what new type of measurement /reporting (LPP signaling from UE to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (LPP signaling from UE to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration including RS (LPP signaling from LMF to UE) |
|  | NG-RAN node assisted (Case 3b) | LMF | 1. whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (NRPPa signaling from gNB to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration (NRPPa signaling from LMF to gNB) |
| AI/ML assisted positioning | UE-Assisted (Case 2a) | UE | 1. whether/what new type of measurement /reporting (LPP signaling from UE to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (LPP signaling from UE to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration (LPP signaling from LMF to UE) |
|  | NG-RAN node assisted (Case 3a) | TRP | 1. whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF) 2. whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) (NRPPa signaling from gNB to LMF) 3. whether/what enhancement for measurement/reporting triggering/configuration (NRPPa signaling from LMF to gNB) |

Based on the submitted evaluations results, the existing reporting usually can offer good performance and with limited performance gap compared to the new type of measurement results that needs a larger amount of reporting overhead. Thus, the AI/ML operation based on the existing reporting should be supported.

Proposal 10: For AI/ML based positioning,

- if UE-based positioning method is used (Case 1), study the following aspect on spec impact

  - support to report the confidence of the AI estimated location on top of location information (LPP signaling from UE to LMF)

- if UE assisted positioning method is used (Case 2a, 2b), study the following aspects on spec impact

  - Support of the existing reporting via LPP signaling from UE to LMF

    - whether/what enhancement, e.g., finer granularity for the measurement result quantization)

  - whether/what new type of measurement /reporting (LPP signaling from UE to LMF)

    - the tradeoff of performance gain, generalization performance and reporting overhead should be carefully considered.

- if NG-RAN node assisted positioning method is used  (Case 3a, 3b), study the following aspects on spec impact

  - Support of the existing reporting via NRPPa signaling from gNB to LMF

    - whether/what enhancement, e.g., finer granularity for the measurement result quantization)

  - whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF)

    - the tradeoff of performance gain, generalization performance and reporting overhead should be carefully considered.

If the AI model is deployed at LMF/gNB side for inference, the existing mechanism can be used for the measurement/reporting of UE/TRP. Some potential enhancement(s) on top of NR existing mechanism can also be considered.

If the AI model is deployed at UE side for inference, regarding the conditions/criteria of AI/ML model used for positioning, at least the TRP deployments and RS configuration for positioning should kept consistency for AI model inference and AI model training. Otherwise, there may be some mismatch between the trained AI model and the applied scenarios.

For UE-sided model, some type of “indication” is needed to determine the right matching of the AI model and the scenarios/configurations

For NW-sided model, the model indication may not be needed as it can be up to NW implementation

Proposal 11: For UE-side model is used for AI/ML based positioning (e.g., Case 1, Case 2a), support implicit or explicit information from LMF to UE to indicate/identify the scenarios/configuration so that UE can choose a suitable AI model matching the target case(s)

- For NW-side model, such type of signaling may not be needed.

Model monitoring and management (selection, activation, deactivation, switching, and fallback operation)

For the use cases of AI-based CSI compression or beam prediction, there are some existing metric(s), in an explicit or implicit way, to reflect the performance of AI model inference. For example, some potential metrics can be used for AI-based CSI compression:

- Some metrics of communication performance can be used to reflect the AI model performance directly. If an AI model achieves good performance, then the system will have good throughput and high transmission reliability (e.g., lower BLER). Thus, the throughput or BLER can be used a metric to monitor the AI model performance.

- The estimated channel by UE can be also used to compare the output of AI model. Thus, the accuracy of re-constructed channel/precoder can be monitored.

For the use case of AI/ML based positioning, the output of AI model usually is not used for communication directly. Thus, the metrics reflecting communication performance cannot reflect the positioning accuracy performance of AI model inference. Meanwhile, as we discussed in previous section, a normal UE cannot get the ground-truth information for location information or some intermediate results (e.g., TOA, TDOA, NLOS/LOS identification, etc.). Thus, the UE cannot get some type of “target output” for the AI/ML model performance monitoring. As a result, the PRU seems the only choice to collect information for the AI model monitoring. Meanwhile, there are some open issues regarding the usefulness/efficiency for PRU use for AI model monitoring, for example

- The PRU may not be deployed in each cell or each scenario. In such a cell or scenario, no PRU can used for AI model monitoring.

- The PRU and the target UE are in different locations. Thus, can the performance of AI model for a UE in one location reflect the performance of the same AI model for another UE in a different location? There maybe need further study and justification.

Meanwhile, there were some proposals to monitor AI/ML model based on the distributions of AI inputs and/or AI outputs. However, there is no common understanding how to use the input/output distribution for the AI model monitoring. Thus, more justifications via evaluation are needed for these methods.

Proposal 12: Regarding AI model monitoring for AI/ML based positioning, Not support UE to report “target output” or “label” for the comparison with the output of the AI/ML model

Proposal 13: For AI/ML based positioning, if PRU is utilized to collect data for AI model performance monitoring, at least study the following aspects

- evaluate/justify whether the performance of the same AI model for PRU and a given UE in different locations are the same or similar

- availability of PRU for typical deployment

Collaboration level and potential impact on specification

In this section, we will discuss the above-mentioned mechanisms from the perspective of collaboration levels between NW and UE and the corresponding specification impacts.

RAN1#109e meeting made the following agreement for the general framework of collaboration level [2]:

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels Level x: No collaboration Level y: Signaling-based collaboration without model transfer Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

More clarification on the boundary of different collaboration levels were made in RAN1#110bis-e [4]:

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

AI/ML assisted positioning

For AI/ML assisted positioning, the AI/ML model will produce some intermediate results, based on which the location will be calculated. For the perspective of air interface, there seems no much difference between direct AI/ML positioning and AI/ML assisted positioning for UE-based positioning method. Thus, the following discussions will focus on UE-assisted positioning method.

For UE-assisted positioning method, the inference of AI/ML model is performed only at UE side. This model can be regarded as UE-side model. UE will use this AI/ML model to generate UE measurement results for some existing type(s) and the generated measurement results can be reported by existing NR signaling. If this model is trained by UE/chipset vendor, there seems no need to specify the input as it is up to UE implementation.

Observation 2: For UE-assisted positioning method, if the model for AI/ML assisted positioning is trained by UE/chipset vendor and generates UE measurement results for some existing type(s), the reporting can reuse existing NR signaling and there is no strong motivation to specify the input of AI/ML model

According to Proposal 10, there still be some implicit or explicit information needed for UE-side model to ensure the current scenario/configuration is suitable for AI operations.

If the output of AI model for AI/ML positioning assisted is some new type of UE measurement (e.g., the scheme “Assisted: a new type of measurement”), some specification enhancement(s) may be needed, e.g.,

- Signaling for the configuration of new type of measurement and reporting

- New reporting format

- New type of measurement at UE side and corresponding requirement(s)

Observation 3: For UE-assisted positioning method, if the outputs of the model for AI/ML assisted positioning are some new type(s) of UE measurement, specification enhancement will be needed.

- e.g., new reporting format, new type of measurement and corresponding requirement

In summary, no matter whether some new type of UE measurement/reporting is introduced or not, there will be some signaling to collaborate UE and NW. Thus, we have following proposal:

Proposal 14: For UE-assisted positioning method with AI/ML assisted positioning (Case 2a), collaboration level y is prioritized.

Following the similar discussions, we have the proposals for NG-RAN node assisted positioning as below:

Proposal 15: For NG-RAN node assisted positioning method with AI/ML assisted positioning (Case 3a), collaboration level y is prioritized.

Direct AI/ML positioning

In general, direct AI/ML positioning can be used for both UE-based positioning method and UE assisted positioning method.

For UE-based positioning method, the inference of AI/ML model is performed only at UE side. In this case, this model can be regarded as UE-side model. UE will use this AI/ML model to generate the estimated location, which can be reported by existing NR signaling. If this model is trained by UE/chipset vendor, there seems no need to specify the input as it is up to implementation.

Observation 4: For UE-based positioning method, if the model for direct AI/ML positioning is trained by UE/chipset vendor, the reporting can reuse existing NR signaling and there is no strong motivation to specify the input

Similar to UE-assisted positioning method with AI/ML assisted positioning (Case 2a), there still be some implicit or explicit information needed for UE-side model to ensure the current scenario/configuration is suitable for AI operations. Thus, we have the following proposal:

Proposal 16: For UE-based positioning method with direct AI/ML positioning (Case 1), collaboration level y is prioritized for direct AI/ML positioning in Rel-18

For UE-assisted positioning method, the inference of AI/ML model is performed only at network side. In this case, this model can be regarded as network-side model. Network will use this AI/ML model to generate the estimated location based on the measurement results reported by UE.  If this approach is based on the existing UE reporting (e.g., the scheme “Direct: DL RSTD + RSRP”), the corresponding AI operations at network side can be transparent to UE. Thus, no collaboration between UE and network is needed in this case. Consequently, these will no spec impact from the perspective of RAN side.

Observation 5: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on existing UE measurement and reporting, the AI operations at network side can be transparent to UE.

Thus, we have following proposal:

Proposal 17: For UE-assisted positioning method with direct AI/ML positioning (Case 2b), collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing UE measurement and reporting (e.g., the scheme “Direct: DL RSTD +RSRP”)

- Note: Only from the perspective of AI model inference

If the input of direct AI/ML positioning is some new type(s) of UE measurement (e.g., the scheme “Direct: Normalized CIR + RSRP”), some specification enhancement(s) is likely to be needed, e.g.,

- Signaling for the configuration of new type of measurement and reporting

- New reporting format

- New type of measurement at UE side and corresponding requirement(s)

Although the new type of measurement result and reporting are introduced with the aim to facilitate the AI/ML operation at LMF, whether the AI/ML operation or non-AI operation is applied at LMF is transparent to UE. In other word, LMF can still use traditional algorithm(s) to calculate the location information of the given UE based on the new measurement results if it wants. Thus, the corresponding spec enhancement(s) is only regarding the new type of measurement and reporting, without any explicit/direct information on AI/ML operations.

Observation 6: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on new type(s) of UE measurement/reporting, specification enhancement will be needed.

- e.g., new reporting format, new type of measurement and corresponding requirement

Thus, we have following proposal:

Proposal 18: For UE-assisted positioning method with direct AI/ML positioning (Case 2b), collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of UE measurement/reporting (e.g., the scheme “Direct: Normalized CIR + RSRP”)

- Note: Only from the perspective of AI model inference

Following the similar discussions, we have the proposals for NG-RAN node assisted positioning as below:

Proposal 19: For NG-RAN node assisted positioning with direct AI/ML positioning (Case 3b), collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing TRP measurement and reporting

- Note: Only from the perspective of AI model inference

Proposal 20: For NG-RAN node assisted positioning with direct AI/ML positioning (Case 3b), collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of TRP measurement/reporting

- Note: Only from the perspective of AI model inference

In the above discussion, we haven’t included life cycle management. We summarize the above discussions in Table 3 as below:

Table 3: Potential spec impact(s) and collaboration level(s) from the perspective of AI model inference

| AI schemes/ sub use cases | Positioning method | Model | Potential impact on NR specifications | Collaboration level between NW and UE |
| --- | --- | --- | --- | --- |
| direct AI/ML positioning | UE-based (Case 1) | UE-side model | - implicit or explicit information to indicate the current scenario/ configuration is suitable for AI operations - whether additional information (e.g., confidence of the AI estimated location) is needed or not on top of location information (LPP signaling from UE to LMF) | Level y |
| AI/ML assisted positioning based on existing type(s) of UE measurement/reporting (e.g., “Assisted: an existing type of measurement”) | UE-assisted (Case 2a) | UE-side model (Stage 1) | implicit or explicit information to indicate the current scenario/ configuration is suitable for AI operations | Level y |
| AI/ML assisted positioning based on new type(s) of UE measurement/reporting (e.g., “Assisted: a new type of measurement”) | UE-assisted (Case 2a) | UE-side model (Stage 1) | - implicit or explicit information to indicate the current scenario/ configuration is suitable for AI operations - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at UE side and corresponding requirement(s) | Level y |
| direct AI/ML positioning based on existing measurement/reporting (e.g., “Direct: DL RSTD + RSRP”) | UE-assisted (Case 2b) | LMF-side model | whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) | Level x FFS: level y |
| direct AI/ML positioning based on new measurement/reporting (e.g., “Direct: Normalized CIR + RSRP”) | UE-assisted (Case 2b) | LMF-side model | - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at UE side and corresponding requirement(s) | Level y |
| AI/ML assisted positioning based on existing type(s) of TRP measurement/reporting | NG-RAN node assisted (Case 3a) | gNB-side model | implicit or explicit information to indicate the current scenario/ configuration is suitable for AI operations | Level y |
| AI/ML assisted positioning based on new type(s) of TRP measurement/reporting | NG-RAN node assisted (Case 3a) | gNB-side model | - implicit or explicit information to indicate the current scenario/ configuration is suitable for AI operations - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at TRP side and corresponding requirement(s) | Level y |
| direct AI/ML positioning based on existing TRP measurement/reporting | NG-RAN node assisted (Case 3b) | LMF-side model | whether/what enhancement for existing reporting (e.g., finer granularity for the measurement result quantization) | Level x FFS: level y |
| direct AI/ML positioning based on new TRP measurement/reporting | NG-RAN node assisted (Case 3b) | LMF-side model | - New signaling for the configuration of new type of measurement and reporting - New reporting format - New type of measurement at TRP side and corresponding requirement(s) | Level y |
| Note: Any potential spec impact of other components (e.g., training) of life cycle management is not included here. |  |  |  |  |

- Functionality-based and Model-ID-based LCM

One key issue for functionality-based and model-ID-based LCM is how to define the applicable condition(s). From the functional perspective, the main purpose for UE to report the applicable condition(s) is to ensure the common understanding between NW and UE on its AI-related capability. According to the agreements of Agenda item 9.2.1 (General part of AI/ML for air interface), there is another terminology of “additional condition(s)”. In our understanding, “condition(s)” and “additional condition(s)” are two different concepts and should be discussed separately.

It was agreed that the UE capability framework is the starting point for functionality identification. Thus, similar to the traditional UE capability, the “condition(s)” should refer to what the configuration(s) UE can support for functionality-based LCM. Otherwise, NW doesn’t know what configurations can be applicable to the AI operations of this UE. The similar principle should also be applied to the model-ID-based LCM.

Proposal 21: Regarding UE-side model (e.g., Case 1 and Case 2a) of AI/ML based positioning accuracy enhancement,

- for functionality-based LCM, the applicable conditional(s) at least includes the supported configuration(s) associated with given functionality(ies)

- for model-ID-based LCM, the applicable conditional(s) at least includes the supported configuration(s) associated with given model(s)

As for the “additional conditions”, there were some examples discussed in previous meetings, e.g., scenarios, sites, and datasets. The similar concept (e.g., scenarios) was also discussed in RAN2. The corresponding RAN2 agreements are copied as below

R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

Thus, if 3GPP agrees the information about scenario/site/dataset are feasible and beneficial for the functionality-based and/or model-ID-based LCM, it can be regard as a part of meta information. In order to avoid the duplicated discussion in RAN1 and RAN2, we have the following proposal:

Proposal 22: Regarding the “additional conditions” for functionality-based and model-ID-based LCM of AI/ML based positioning accuracy enhancement, suspend RAN1 discussion on the detailed “additional conditions” (e.g., scenarios, sites, and datasets) to wait for more RAN2 progress on meta information

- Conclusions

In this contribution, we discussed the sub use cases for AI based positioning accuracy improvement. Starting with some general principles for the use case selection, we discussed some typical sub use cases for AI-based positioning accuracy enhancement, followed by the discussion on potential impact on NR specification and collaboration between NW and UE. Based on the above discussion, we have the following observations, proposals and the summary of spec impact and collaboration level for typical sub use cases.

Observation 1: For UE based and UE-assisted positioning method, if the model for direct AI/ML positioning is trained based on the labels that obtained by NR DL-TDOA scheme,

- there is significant performance degradation compared to the ideal ground-truth labels

- there is no obvious performance gain compared to legacy NR positioning methods

Observation 2: For UE-assisted positioning method, if the model for AI/ML assisted positioning is trained by UE/chipset vendor and generates UE measurement results for some existing type(s), the reporting can reuse existing NR signaling and there is no strong motivation to specify the input of AI/ML model

Observation 3: For UE-assisted positioning method, if the outputs of the model for AI/ML assisted positioning are some new type(s) of UE measurement, specification enhancement will be needed.

- e.g., new reporting format, new type of measurement and corresponding requirement

Observation 4: For UE-based positioning method, if the model for direct AI/ML positioning is trained by UE/chipset vendor, the reporting can reuse existing NR signaling and there is no strong motivation to specify the input

Observation 5: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on existing UE measurement and reporting, the AI operations at network side can be transparent to UE.

Observation 6: For UE-assisted positioning method, if the model for direct AI/ML positioning is trained by network vendor and the input is based on new type(s) of UE measurement/reporting, specification enhancement will be needed.

- e.g., new reporting format, new type of measurement and corresponding requirement

Proposal 1: For AI/ML assisted positioning, the following alternative should be prioritized if the TOA-like output is used for AI/ML model

- The measurement results corresponding to all TRPs are used as the input for AI/ML model inference (i.e., Multi-TRP construction).

Proposal 2: For AI/ML based positioning, regarding the data collection for AI model training, NOT support UE to report the ground-truth labels of its location(s)

Proposal 3: For the training data collection at UE side (e.g. UE-based positioning with UE-side model (Case 1), UE-assisted positioning with UE-side model (Case 2a)), study the potential spec impact on that UE sends request of preferred or supported configurations to LMF

- FFS: Whether this request is sent by UE capability signaling or other signaling.

Proposal 4: For the training data collection at NW side for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), study the potential spec impact (including necessity and benefit) of the following options

- PRU: PRU reports measurement results (corresponding to model input) optionally with labels via LPP or other new signaling defined by RAN2.

  - Other network entity (e.g., LMF) maybe know the label(s) in advance

- UE: UE reports measurement results (corresponding to model input) without labels via LPP or other new signaling defined by RAN2

  - This kind of data is used for semi-supervised learning.

Proposal 5: For the training data collection at NW side NG-RAN node assisted positioning with gNB-side model (Case 3a) or NG-RAN node assisted positioning with LMF-side model (Case 3b), study the potential spec impact (including necessity and benefit) of the following options

- TRP can collect the measurement results (corresponding to model input) based on PRU’s transmission and report them via LPP or other new signaling defined by RAN2.

  - Other network entity (e.g., LMF) generates the associated labels based on the know location of the corresponding PRU

- TRP can collect the measurement results (corresponding to model input) based on UE’s transmission and report them via LPP or other new signaling defined by RAN2.

  - This kind of data is used for semi-supervised learning.

Proposal 6: For the training data collection, regarding ground truth label generation for AI/ML based positioning, study the feasibility, necessity and potential benefits for the following option

- UE/NW generates ground truth label when label quality satisfies the requirement

  - based on non-NR and/or NR RAT-dependent, and/or Non-RAT-dependent (e.g.,WIFI, blue tooth, UWB, sensor, etc.) positioning methods

  - FFS: label quality

  - FFS: requirement

Proposal 7: For the data collection used for AI model inference

- When direct AI/ML positioning is used for UE-base positioning method (Case 1) or AI/ML assisted positioning is used for UE-assisted positioning method (Case 2a), the UE will collect measurement for the input of AI model

  - If the model is trained at the same side, the inputs/data collection are up to UE implementation and transparent from the perspective of air interface

  - If the model is trained at NW side and AI model inference is performed at UE side (when model transfer is supported), the size/contents of inputs will need to be pre-defined or pre-configured.

- When direct AI/ML positioning is used for UE-assisted positioning method (Case 2b), the target UE will report the measurement results to LMF via LPP signaling

  - FFS: type of measurement (e.g., existing measurement type, new measurement type), RS configuration for measurement

- When AI/ML assisted positioning is used for NG-RAN node assisted positioning method (Case 3a), the TRP will collect measurement for the input of AI model by implementation and transparent from the perspective of specification

- When direct AI/ML positioning is used for NG-RAN node assisted positioning method (Case 3b), the TRP will report the measurement results to LMF via NRPPa signaling

Proposal 8: For both direct AI/ML positioning and AI/ML assisted positioning, offline training of AI model(s) is prioritized in Rel-18

- Enhancements dedicated to online training can be discussed in the future release(s)

Proposal 9: For both direct AI/ML positioning and AI/ML assisted positioning, the AI model training and inference are assumed to be done in the same side in Rel-18, i.e., no model transfer

- AI model training and inference at UE side, or

- AI model training and inference at NW side

- Study model transfer in future release(s).

Proposal 10: For AI/ML based positioning,

- if UE-based positioning method is used (Case 1), study the following aspect on spec impact

  - support to report the confidence of the AI estimated location on top of location information (LPP signaling from UE to LMF)

- if UE assisted positioning method is used (Case 2a, 2b), study the following aspects on spec impact

  - Support of the existing reporting via LPP signaling from UE to LMF

    - whether/what enhancement, e.g., finer granularity for the measurement result quantization)

  - whether/what new type of measurement /reporting (LPP signaling from UE to LMF)

    - the tradeoff of performance gain, generalization performance and reporting overhead should be carefully considered.

- if NG-RAN node assisted positioning method is used  (Case 3a, 3b), study the following aspects on spec impact

  - Support of the existing reporting via NRPPa signaling from gNB to LMF

    - whether/what enhancement, e.g., finer granularity for the measurement result quantization)

  - whether/what new type of measurement /reporting (NRPPa signaling from gNB to LMF)

    - the tradeoff of performance gain, generalization performance and reporting overhead should be carefully considered.

Proposal 11: For UE-side model is used for AI/ML based positioning (e.g., Case 1, Case 2a), support implicit or explicit information from LMF to UE to indicate/identify the scenarios/configuration so that UE can choose a suitable AI model matching the target case(s)

- For NW-side model, such type of signaling may not be needed.

Proposal 12: Regarding AI model monitoring for AI/ML based positioning, Not support UE to report “target output” or “label” for the comparison with the output of the AI/ML model

Proposal 13: For AI/ML based positioning, if PRU is utilized to collect data for AI model performance monitoring, at least study the following aspects

- evaluate/justify whether the performance of the same AI model for PRU and a given UE in different locations are the same or similar

- availability of PRU for typical deployment

Proposal 14: For UE-assisted positioning method with AI/ML assisted positioning (Case 2a), collaboration level y is prioritized.

Proposal 15: For NG-RAN node assisted positioning method with AI/ML assisted positioning (Case 3a), collaboration level y is prioritized.

Proposal 16: For UE-based positioning method with direct AI/ML positioning (Case 1), collaboration level y is prioritized for direct AI/ML positioning in Rel-18

Proposal 17: For UE-assisted positioning method with direct AI/ML positioning (Case 2b), collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing UE measurement and reporting (e.g., the scheme “Direct: DL RSTD +RSRP”)

- Note: Only from the perspective of AI model inference

Proposal 18: For UE-assisted positioning method with direct AI/ML positioning (Case 2b), collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of UE measurement/reporting (e.g., the scheme “Direct: Normalized CIR + RSRP”)

- Note: Only from the perspective of AI model inference

Proposal 19: For NG-RAN node assisted positioning with direct AI/ML positioning (Case 3b), collaboration level x is prioritized for direct AI/ML positioning if the AI model is based on existing TRP measurement and reporting

- Note: Only from the perspective of AI model inference

Proposal 20: For NG-RAN node assisted positioning with direct AI/ML positioning (Case 3b), collaboration level y is prioritized for direct AI/ML positioning if the AI model is based on new type(s) of TRP measurement/reporting

- Note: Only from the perspective of AI model inference

Proposal 21: Regarding UE-side model (e.g., Case 1 and Case 2a) of AI/ML based positioning accuracy enhancement,

- for functionality-based LCM, the applicable conditional(s) at least includes the supported configuration(s) associated with given functionality(ies)

- for model-ID-based LCM, the applicable conditional(s) at least includes the supported configuration(s) associated with given model(s)

Proposal 22: Regarding the “additional conditions” for functionality-based and model-ID-based LCM of AI/ML based positioning accuracy enhancement, suspend RAN1 discussion on the detailed “additional conditions” (e.g., scenarios, sites, and datasets) to wait for more RAN2 progress on meta information

- Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Chair’s notes of RAN1#111 meeting

- Chair’s notes of RAN1#112 meeting

- Chair’s notes of RAN1#112bis-e meeting