3GPP TSG RAN WG1 #113		        R1-2305465

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

# Introduction

The NR duplex evaluation assumptions, including deployment scenarios, channel model, interference model, traffic model and performance metrics, are almost finalized in the past meetings. In this contribution, we provide some initial evaluation results, with the adopted simulation assumptions given in Appendix.

# Evaluation results

Some key assumptions used in evaluation include:

- The channel bandwidth is 100MHz (273RBs at SCS=30kHz) and SBFD pattern in frequency domain is {DUD} with < ND, NU, NG > = <104, 55, 5>.

- Four slot patterns {DDDSU, XXXXU, XXXXX, DXXXU} are evaluated for low traffic load and medium traffic load, respectively.

- The FTP3 traffic arrival model parameter is determined with Type-2 RU in legacy TDD (DDDSU) case. The traffic packets generation for legacy TDD case and SBFD cases follow the same random seed for generations of traffic arrival times, as well as the same fixed packet size under the given load condition.

- UE-UE co-channel inter-subband CLI, gNB self-interference, co-site inter-sector co-channel inter-subband CLI are modeled, following the CLI modeling details described in Appendix.

More information on general SLS assumptions can be found in Table A-1 in Appendix.

## UPT Analysis

Figure 1 Average UPT per UE with 0.5Mbytes DL & 0.125Mbytes UL

Figure 1 shows that, for UE UPT with large packet size (0.5Mbytes DL & 0.125Mbytes UL),

- The setup of SBFD does not change the tendency that the UE DL UPT increases as traffic load decreases.

- For a given load case,

  - XXXXU results in the highest UL UPT among four slot patterns. This is because the UL resource pool size for XXXXU is highest among four slot patterns. A traffic packet with 0.125Mbytes may need more resources than UL resource in one “X” slot.

  - For XXXXX case, the UL UPT is larger than or similar to DDDSU but mostly smaller than XXXXU and DXXXU. This is because XXXXX has similar UL resource pool size to DDDSU but more scheduling occasions, and it has smaller UL resource pool size than XXXXU and DXXXU with similar scheduling occasions.

- For Indoor office and dense urban macro cases, the DL UPT for XXXXX is similar with DDDSU, because the DL resource pool size and scheduling occasions are similar for these two slot patterns. XXXXU and DXXXU have smaller DL resource pool size and similar scheduling occasions compared with DDDSU, so that the DL UPT for these two SBFD patterns are lower than that of DDDSU.

Table 1 show the UPT gain for SBFD patterns. For large packet case, the setup of SBFD improves UL UPT significantly, but at a cost of DL UPT reduction for almost all cases.

Table 1 Gain for average DL/UL UPT with 0.5Mbytes DL & 0.125Mbytes UL

|  |  | Indoor office |  | Dense Urban Macro |  | Urban Macro |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Gain for Avg. DL UPT | Gain for Avg. UL UPT | Gain for Avg. DL UPT | Gain for Avg. UL UPT | Gain for Avg. DL UPT | Gain for Avg. UL UPT |
| High load | XXXXU | -29.27% | 101.85% | -26.08% | 85.64% | -69.50% | 89.06% |
|  | XXXXX | -4.66% | 19.06% | -0.40% | 41.46% | -50.44% | 84.75% |
|  | DXXXU | -25.81% | 70.10% | -14.38% | 69.00% | -61.10% | 68.12% |
| Medium load | XXXXU | -23.88% | 92.49% | -27.16% | 114.71% | -43.51% | 157.98% |
|  | XXXXX | 2.08% | 17.66% | -3.38% | 51.18% | -28.14% | 101.90% |
|  | DXXXU | -25.55% | 68.89% | -21.02% | 88.46% | -50.26% | 120.42% |
| Low load | XXXXU | -15.14% | 77.61% | -17.60% | 103.52% | -28.06% | 156.70% |
|  | XXXXX | 5.42% | -3.50% | 3.01% | 44.92% | -10.87% | 104.97% |
|  | DXXXU | -12.49% | 57.38% | -14.90% | 77.01% | -25.73% | 120.32% |

Figure 2 Average UPT per UE with 4Kbytes DL & 1Kbytes UL

Figure 2 shows that, for UE UPT with small packet size,

- Under low-to-medium load, XXXXX offers the top DL UPT and UL UPT among four slot patterns, except for the medium load in Urban Macro scenario. This is because the UL subband in each of “X” slot is large enough for small packet size and moderate-to-low traffic load, and the same is for DL subbands in each of “X” slot. When it comes to Urban Macro, the cell-edge UE may need more “X” resources for uplink transmission, which may impact its downlink UPT due to half-duplex limitation. It is also observed that the UL UPT’s of XXXXU, XXXXX and DXXXU are far better than that of DDDSU.

Observation 1: The setup of SBFD does not change the tendency that the UE DL UPT decreases as traffic load increases.

Observation 2: The setup of UL subband over DL symbols improves the UL UPT per UE.

Observation 3: The setup of UL subband over DL symbols impact the DL UPT almost all evaluated cases when packet size is large (0.5Mbytes DL & 0.125Mbytes UL).

Although our evaluations cover low-to-medium traffic loads, in these evaluations we already observed the numerous occurrences that, at the time a UE is picked by scheduler for scheduling in a slot, the simulation already holds both DL to-be-transmitted traffic and UL to-be-transmitted traffic for the UE. Then the scheduler has to prioritize between DL and UL given the UE has to be in half-duplex mode. We expect this kind of DL-UL collision to occur more often in the high traffic load case. This brings a question whether the performance results corresponding to the following two UE distribution assumptions are comparable:

Agreement For any deployment cases where clustering is not used and where M UEs are distributed per direction, - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M UEs, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M UEs.

The key difference between the two options is that the first option completely removes the half-duplex restriction from the evaluation, which somehow translate the 2M half-duplex UEs into M full-duplex UEs under a 2-to-1 UE-pairing without taking into account the UE locations. This duplex relaxation behind the first option does not match any real field operation in the long run. We think the evaluation results corresponding to these two UE distribution options should not be mixed together to derive study conclusions.

Proposal 1: RAN1 examines the comparability of evaluation results corresponding to the two agreed UE distribution options (i.e., M DL-only UEs and M UL-only UEs vs. M dual-direction UEs) and, if necessary, summarizes the evaluation results corresponding to these two options separately.

## Link budget evaluation

Table 2 LLS evaluation for coverage performance

| TDD/SBFD | Required SNR | MCL | MIL | MPL | Key assumptions |
| --- | --- | --- | --- | --- | --- |
| TDD（DDDSU） | 0.5 | 129.75 | 138.52 | 107.79 | Single slot PUSCH transmission 30RB, 2815bit, MCS4 |
| SBFD（XXXXX） | -3.8 | 133.55 | 142.32 | 111.59 | Single slot PUSCH transmission 30RB, 1096bit,MCS0 |
| SBFD（XXXXU） | -3 | 132.75 | 141.52 | 110.79 | Single slot PUSCH transmission 30RB, 1096bit,MCS0 |
| SBFD（XXXXX） | -6.2 | 135.95 | 144.72 | 113.99 | Repetition type A (N=4) 30RB, 2815bit, MCS4 |

During the link level evaluation, using ∆=10*log10NSBFD-10*log10NTDD, ∆=6dB to model the noise for SBFD symbols and UL only symbols. For XXXXU case, the modeled noise level in “U” slot and “X” slot are different.

For single slot PUSCH transmission, XXXXU and XXXXX have better coverage performance than DDDSU, because there are more transmission occasions for XXXXU and XXXXX, so that the required MCS(TBS) can be set smaller than that of DDDSU for the given target data rate (1Mbps).

When using PUSCH repetition type A, XXXXX achieves better coverage performance than single slot PUSCH transmission in DDDSU, with same required MCS (TBS).

Observation 4: The setup of UL subband over DL symbols could improve coverage performance at least 3 dB for a given target data rate (1Mbps).

# Conclusion

In this contribution, we show our views on evaluation on NR duplex evolution with following proposals:

Observation 1: The setup of SBFD does not change the tendency that the UE DL UPT decreases as traffic load increases.

Observation 2: The setup of UL subband over DL symbols improves the UL UPT per UE.

Observation 3: The setup of UL subband over DL symbols impact the DL UPT almost all evaluated cases when packet size is large (0.5Mbytes DL & 0.125Mbytes UL).

Observation 4: The setup of UL subband over DL symbols could improve coverage performance at least 3 dB for a given target data rate (1Mbps).

Proposal 1: RAN1 examines the comparability of evaluation results corresponding to the two agreed UE distribution options (i.e., M DL-only UEs and M UL-only UEs vs. M dual-direction UEs) and, if necessary, summarizes the evaluation results corresponding to these two options separately.

# Reference

- RP-213591 New SI: Study on evolution of NR duplex operation, CMCC

- R1-2208856 Discussion on evaluation on NR duplex evolution, OPPO

- R1-2300286 Discussion on evaluation on NR duplex evolution, OPPO

- R1-2302546 Discussion on evaluation on NR duplex evolution, OPPO

# Appendix

## Simulation parameters

Table A-1 simulation parameters for SBFD

| Parameters | Value |  |  |
| --- | --- | --- | --- |
|  | Indoor office | Urban Macro | Dense Urban Macro |
| Scenario | InH(2*6 site) | 7 cells, 3 sectors/cell |  |
| Inter-BS distance | 20m | 500m | 200m |
| Carrier frequency | 4GHz |  |  |
| System bandwidth | 100MHz |  |  |
| SCS | 30KHz |  |  |
| TDD pattern | DDDSU / XXXXU / XXXXX |  |  |
| SBFD pattern | {DUD}, < ND, NU, NG > = <104, 55, 5> |  |  |
| BS Antenna Configuration | (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) (dH, dV) = (0.5λ, 0.5λ) | (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;2,8) (dH, dV) = (0.5λ, 0.5λ) |  |
| UE Antenna Configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |  |
| Transmit Power | DL: 24dBm UL: 23dbm | DL: 53dBm UL: 23dbm |  |
| Antenna Height | 3 m for BS and 1.5 m for UE | 25 m for BS and 1.5 m for UE | 25 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |  |
| UE speed | 3km/h |  |  |
| Scheduling Algorithm | SU-MIMO + PF |  |  |
| Power control for PUSCH | P0=-60dbm, alpha=0.6 | P0=-80dbm, alpha=0.8 |  |
| Traffic model | FTP3, with 0.5Mbytes for DL, 0.125Mbyte for UL |  |  |

## UE-UE co-channel inter-subband CLI

IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB

where

- IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value).

- PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value)

- CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE

- NULRB is the total number of UL RBs in the UL subband

- 10*log10(ICSUE)=33dBc

- 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein,

  - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored

  - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1

  - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'.

  - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1.

    - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS

    - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS

  - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs)

  - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs.

## gNB self-interference

ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB

    - Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB.

    - Nused-DL-RB is the number of DL RBs allocated for DL transmission.

    - αSI=PtxmaxISImax*NDLRBNULRB

      - with sensitivity degradation of 1dB, ISImax can be computed based on 10*log10ISImax+NN=1dB, where N is the noise floor over the UL subband given by N(dB)=-174 + 10*log10 (20e6) + 5 = -96 dBm, assuming 20MHz UL subband and 5dB noise figure.

## co-site inter-sector co-channel inter-subband CLI

Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB

Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB

- Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB.

- Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale).

- NDLRB is the total number of DL RBs in the DL subbands.

- Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x.

- αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI.

  - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

    - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment.

  - 10*log10ACLRBS=45dB

  - 10*log10ACSBS=46dB

  - spatial isolationdB=75dB

## Link level evaluation assumption

| System configuration | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| Carrier frequency (GHz) | 4.00 | 4.00 | 4.00 | 4.00 |
| BS antenna heights (m) | 25.00 | 25.00 | 25.00 | 25.00 |
| UT antenna heights (m) | 1.50 | 1.50 | 1.50 | 1.50 |
| Pathloss model (select from LoS or NLoS) | NLOS | NLOS | NLOS | NLOS |
| UE speed (km/h) | 3.00 | 3.00 | 3.00 | 3.00 |
| Channel for evaluation | PUSCH | PUSCH | PUSCH | PUSCH |
| UL-DL configuration for TDD | DDDSU | XXXXU | XXXXX | XXXXX |
| Subcarrier Spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Channel model for link level simulation | TDL-C | TDL-C | TDL-C | TDL-C |
| Frequency hopping | w/o | w/o | w/o | w/o |
| number of PRBs, TBS and MCS | 30RB, 2815bit, MCS4 | 30RB, 1096bit,MCS0 | 30RB, 1096bit,MCS0 | 30RB, 2815bit,MCS4 |
| BWP size | 100MHz | 100MHz | 100MHz | 100MHz |
| DMRS configuration | 1 DMRS | 1 DMRS | 1 DMRS | 1 DMRS |
| Waveform | DFT-S-OFDM | DFT-S-OFDM | DFT-S-OFDM | DFT-S-OFDM |
| Repetition | w/o | w/o | w/o | w/ type A 4 repetition |
| HARQ configuration | w/o | w/o | w/o | w/o |
| Target error rate (BLER, miss detection, false alarm, etc.) | 10% iBLER | 10% iBLER | 10% iBLER | 10% iBLER |
| Target data rate (Mbps) | 1 | 1 | 1 | 1 |