3GPP TSG RAN WG1 #113	                                                            	 R1-2305467

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.3

Document for:	Discussion and Decision

# Introduction

In RAN1 #112 bis meeting, potential enhancements on dynamic/flexible TDD were discussed with the following agreements:

Agreement

For the gNB-gNB co-channel CLI measurement, both RSRP and RSSI can be used as measurement metric for evaluation purposes only.

Agreement

Study the effect on DL performance and the UL performance of DL Tx power adjustment to evaluate the feasibility of such scheme to overcome the gNB-to-gNB co-channel CLI.

Agreement

Study the effect on DL/UL performance and specification impact of applying separate open-loop/closed-loop power control parameters with cochannel CLI and without cochannel CLI for the uplink power control of a UE

Agreement

For gNB-gNB co-channel CLI measurement and channel measurement, study the impact on system performance because of CLI measurement inaccuracy at victim gNB due to misalignment between UL timing at victim gNB and DL reception timing at victim gNB of CLI measurement resource transmitted from one or more aggressor gNB.

- Including potential impact on UL performance

Agreement

For enhancement of gNB-to-gNB co-channel CLI measurement and/or channel measurement, following options are studied for UL resource muting.

- Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource)

- Option 2: Non-transparent UL resource muting method (e.g., define UL resource muting pattern with one or more RE/RB muting patterns)

Agreement

For UE-to-UE co-channel CLI measurement, study the impact on system performance because of CLI measurement inaccuracy at victim UE due to misalignment between DL reception timing at victim UE of DL channel/signal transmitted from serving gNB and DL reception timing at victim UE of CLI measurement resource transmitted from aggressor UE(s).

In this contribution, we share our views on the inter-gNB and inter-UE CLI issues.

# Discussion

# Inter-gNB CLI measurement

Besides NZP-CSI-RS and SSB, a more general resource like Rel-16 CLI-RSSI resource is also suitable for inter-gNB CLI measurement. Firstly, CLI-RSSI resource is a periodic resource, so frequent exchange of configuration information over Xn interface can be avoided. Secondly, the configuration of CLI-RSSI resource is very flexible, so all kinds of CLI measurement needs can be satisfied by reasonable configuration.

Proposal 1: A CLI-RSSI-alike resource, i.e. defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset, can be used for gNB-to-gNB CLI measurement.

Both short-term and long term inter-gNB CLI measurement can be considered. Short-term inter-gNB CLI measurement, e.g. L1-based RSRP/RSSI, is more desirable to reflect the dynamic interference. Given L1/L2 UE-to-UE CLI measurement has been agreed in RAN1, we think L1 gNB-to-gNB CLI measurement should be supported, and L1 gNB-to-gNB CLI reporting with timestamp should be exchanged over Xn interface.

Proposal 2: RAN1 targets to support L1 gNB-to-gNB CLI measurement.

- L1-based RSRP/RSSI can be considered;

- L1 gNB-to-gNB CLI measurement reporting with timestamp is exchanged over Xn interface.

# Inter-UE CLI measurement

In R16, SRS resources and CLI-RSSI resources are configured to UE by RRC signaling for L3-based SRS-RSRP and CLI-RSSI measurement. In our opinion, L1-based SRS-RSRP and L1-based CLI-RSSI are very straightforward for inter-UE CLI measurement to reflect short-term interference. Both R16 SRS resources and CLI-RSSI resources should be reused. The L1 UE-to-UE CLI measurement reporting can be reported as a separated CSI report with the extension of R17 CSI reference resource definition to include the SRS resource and CLI-RSSI resource for UE-to-UE CLI measurement, so R15/16 CSI processing delay should be satisfied.

Proposal 3: RAN1 targets to support L1-based SRS-RSRP and L1-based CLI-RSSI measurement for UE-to-UE CLI measurement.

- R16 configuration of SRS and CLI-RSSI resources should be reused.

Proposal 4: L1 UE-to-UE CLI measurement reporting can be a separated CSI report.

- R17 CSI reference resource definition should be extended to include the SRS resource and CLI-RSSI resource for UE-to-UE CLI measurement;

- R15/16 CSI processing delay should be satisfied.

# Coordinated scheduling

The exchange of intended TDD DL-UL configuration over Xn and F1 interfaces has been supported since R16, but the slot configuration list is limited to a 10-ms period. More flexible configuration exchange over Xn and F1 interfaces should be studied for R18 dynamic/flexible TDD, such as SBFD time/frequency configuration and TDD DL-UL configuration with a periodicity longer than 10-ms.

Proposal 5: To support coordinated scheduling between gNBs, more flexible configuration exchange over Xn/F1 interfaces should be studied, e.g. SBFD time/frequency configuration and TDD DL-UL configuration with periodicity longer than 10-ms.

# UL power control

For one UE, the power for UL transmissions with CLI and without CLI can be very different. As mentioned above, boosting power can be used for UL transmissions with CLI, which motivates separated closed loop power control or separated configuration of open loop power control for UL transmissions with CLI and without CLI. Comparing to separated closed loop power control, separated configurations of open loop power control for UL transmissions with CLI and without CLI is preferred because it does not require interaction between UL transmissions with CLI and without CLI. In R15, open loop power control parameter is configured separately between configured grant and dynamic grant. In R16 URLLC, two values of P0 can be dynamically indicated to UE for PUSCH transmissions. Existing power control mechanism is flexible enough to allow separated handling of transmission power for UL transmissions with CLI and without CLI.

Proposal 6: Existing power control mechanism with separate open loop power control parameters can be reused for UL transmissions with CLI and without CLI.

# Conclusions

In this contribution, we discuss inter-gNB and inter-UE CLI issues for dynamic/flexible TDD with the following proposals:

Proposal 1: A CLI-RSSI-alike resource, i.e. defined by a starting RB/symbol and a number of RBs/symbols together with a time-domain pattern given by periodicity/offset, can be used for gNB-to-gNB CLI measurement.

Proposal 2: RAN1 targets to support L1 gNB-to-gNB CLI measurement.

- L1-based RSRP/RSSI can be considered;

- L1 gNB-to-gNB CLI measurement reporting with timestamp is exchanged over Xn interface.

Proposal 3: RAN1 targets to support L1-based SRS-RSRP and L1-based CLI-RSSI measurement for UE-to-UE CLI measurement.

- R16 configuration of SRS and CLI-RSSI resources should be reused.

Proposal 4: L1 UE-to-UE CLI measurement reporting can be a separated CSI report.

- R17 CSI reference resource definition should be extended to include the SRS resource and CLI-RSSI resource for UE-to-UE CLI measurement;

- R15/16 CSI processing delay should be satisfied.

Proposal 5: To support coordinated scheduling between gNBs, more flexible configuration exchange over Xn/F1 interfaces should be studied, e.g. SBFD time/frequency configuration and TDD DL-UL configuration with periodicity longer than 10-ms.

Proposal 6: Existing power control mechanism with separate open loop power control parameters can be reused for UL transmissions with CLI and without CLI.

# Reference

- R1-2301978, “Summary #3 of potential enhancement on dynamic/flexible TDD”, Moderator (LG Electronics)