3GPP TSG RAN WG1 #113		                                                       R1-2305470

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	9.16.7

Source: 	OPPO, Huawei, HiSilicon, LG Electronics

Title: 	Initial UE features list for Rel-18 NR sidelink evolution WI

Document for:	Discussion and decision

- Introduction

This contribution includes an initial RAN1 UE features list for the Rel-18 NR sidelink evolution WI.

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-1 | SL channel access for dynamic channel access mode | UE supports - SL Type 1 channel access and contention window size adjustment - SL Type 2A channel access - SL Type 2B channel access - SL Type 2C channel access - 20MHz LBT bandwidth - CP extension up to 1 symbol in 15kHz SCS - CP extension up to 2 symbols in 30kHz and 60kHz SCS | At least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | No | UE does not support channel access for NR sidelink operation in unlicensed spectrum. | Per band | n/a | n/a |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-2 | [SL multi-channel access for dynamic channel access mode] | - UE supports multi-channel access procedures for PSSCH transmission in multiple RB sets - UE supports Type A and Type B multi-channel access procedures for concurrent PSFCH transmissions in multiple RB sets | 47-1, 15-11, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in unlicensed spectrum. | Per band | n/a | n/a |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-3 | Receiving UE to UE COT sharing information | - UE supports monitoring SCI to read COT sharing information | 47-1, 15-1, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support UE-to-UE COT sharing for NR sidelink operation in unlicensed spectrum. | Per band | n/a | n/a |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-4 | Transmitting UE to UE COT sharing information | - [UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing] - UE supports indicating in SCI COT sharing information | 47-1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support UE-to-UE COT sharing for NR sidelink operation in unlicensed spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-5 | MCSt | UE supports - MCSt in SL Mode 1 resource allocation - MCSt in SL Mode 2 resource allocation | At least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support multi-consecutive slots transmission | Per UE | n/a | n/a |  |  | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based PSCCH/PSSCH transmission | - UE supports transmitting interlace RB-based PSCCH/PSSCH | 47-1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH transmission | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation.] | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m2 | Interlace RB-based PSCCH/PSSCH reception | 1) UE supports receiving interlace RB-based PSCCH/PSSCH | 47-1, [15-1 except Component 5] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH reception | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation.] | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | - UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 47-1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | - UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | - UE supports N PSFCH occasion(s) per PSCCH/PSSCH | 15-11 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {TBD} | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | - UE supports transmitting legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain | 15-4, 47-1 | No | No | UE does not support transmission of legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] Candidate values for N are {TBD} | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m7 | Receiving SSB repetitions within one RB set | - UE supports receiving S-SSB which is transmitted in a manner that repeat legacy S-PSS/S-SSS/PSBCH N times in frequency domain | 15-4 | No | No | UE does not support reception of S-SSB which is transmitted in a manner that repeat legacy S-PSS/S-SSS/PSBCH N times in frequency domain | Per band | N/A | N/A |  |  | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | - UE supports transmitting S-SSB on additional S-SSB occasion(s) | 15-4, 47-1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) | Per band | N/A | N/A |  |  | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-m9 | Receiving of S-SSB on additional S-SSB occasion(s) | - UE supports receiving S-SSB transmitted on additional S-SSB occasion(s) | 15-4 | No | No | UE does not support receiving S-SSB transmitted on additional S-SSB occasion(s) | Per band | N/A | N/A |  |  | [Optional with capability signalling] |
| 47. NR_SL_enh2 | 47-s1 | Avoidance of NR PSCCH/PSSCH overlapping with EUTRA SL resources in dynamic resource pool sharing | - UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] that excludes NR SL candidate resources overlapping with EUTRA SL reserved resources by other EUTRA SL UE - UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] that excludes NR SL candidate resources overlapping with EUTRA SL resources selected to be used for EUTRA SL module’s own transmission - UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] that excludes NR SL candidate resources overlapping with EUTRA SL resources associated with non-monitored subframe in EUTRA SL module - UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs in 30kHz SCS that selects at least the first of NR SL slots overlapping with an EUTRA SL subframe and can select the subsequent overlapping NR SL slot | 15-3, 15-6 UE supports EUTRA SL in the same band | [Yes] | No |  | [Per band] | N/A | N/A |  | [This is the basic FG for LTE/NR sidelink co-channel coexistence.] | [Optional with capability signalling] [For UE supports NR sidelink co-channel coexist with EUTRA sidelink, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-s2 | Avoidance of NR PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing | - UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] that excludes NR SL candidate resources where the corresponding NR PSFCH transmission occasions overlap with EUTRA SL reserved resources by other EUTRA SL UE in time domain | 47-s1, 15-11 | [No] | No |  | [Per band] | N/A | N/A |  | [[This is the basic FG for LTE/NR sidelink co-channel coexistence.] | [Optional with capability signalling] [For UE supports NR sidelink co-channel coexist with EUTRA sidelink, UE must indicate this FG is supported.] |