3GPP TSG RAN WG1 #113	R1-2305487

Incheon, Korea, May 22nd – May 26th, 2023

# Title:	Draft reply LS on unified TCI-state and fast SCell activation

# Response to:	LS R1-2304329 (R2-2304549)

# Release:	Rel-18

# Work Item:	NR_feMIMO-Core

Source:	3GPP TSG-RAN WG1 (Samsung)

To:	3GPP TSG-RAN WG2

Cc:	3GPP TSG-RAN WG4

Contact Person:

Name: Emad Farag

E-mail Address: e.farag@samsung.com

# Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for their LS in R2-2304549 on unified TCI-state and fast SCell activation and provide replies to the question asked.

RAN2 asked the following questions:

Question:   Is the reference signal used for fast SCell activation allowed to be configured with the qcl-Info that is indicated by TCI-state from the dl-OrJointTCI-stateList, by the current RAN1 specification?

RAN1’s reply to the Question: The QCL info can be a Rel-17 TCI state as well as a Rel-15 TCI state. The information element of SCellActivationRS-Config-r17 is given by

SCellActivationRS-Config-r17 ::= SEQUENCE {
      scellActivationRS-Id-r17 	SCellActivationRS-ConfigId-r17,
      resourceSet-r17 			NZP-CSI-RS-ResourceSetId,
      gapBetweenBursts-r17 		INTEGER (2..31) OPTIONAL, -- Need R
      qcl-Info-r17 			TCI-StateId,

}

The TCI state ID can be a Rel-15/16 TCI state Id or a Rel-17 TCI state Id. There is no need to restrict to Rel-15/16 TCI state only.

2. Actions:

To RAN4

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG RAN1 Meeting #114		21st – 25th AUG 2023				Toulouse, FR

TSG RAN1 Meeting #114	-bis	09th – 13th OCT 2023				Xiamen, CN