3GPP TSG RAN WG1 #113	    			            R1-2305521

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.5.2

Source: 	Samsung

Title: 	On NR DL and UL Carrier Phase Positioning

Document for:	Discussion and Decision

# Introduction

Based on the findings of the Rel-18 study item to expand and improve NR positioning [1], as documented in TR 38.859 [2], a new work item [3] has been approved in RAN1#98e to specify carrier phase positioning measurements and associated signaling. The scope of the work item is:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this document, we present Samsung’s views on the carrier-phase measurements.

# Discussion

Carrier phase (CP) positioning relies on measuring a carrier phase at the RF frequency of a signal transmitted from one device (e.g., device A which can be a base station or a UE) and received by another device (e.g., device B which can be a UE or a base station). When device A is a base station and device B is a UE, this is DL carrier phase measurement. When device A is a UE and device B is a base station, this is UL carrier phase measurement. The carrier phase measured at device B is a function of the propagation time, and consequently the propagation distance, from transmitter of device A to the receiver of device B.

The propagation time Tp from the transmitter of device A to the receiver of device B can be expressed as a sum of an integer number of cycles at the carrier frequency, N Tc, where Tc is the carrier frequency period, and a fraction part of a cycle Tf, where Tf<Tc. Therefore,

Tp= N Tc+Tf  	                                                                                 (1)

The measured carrier phase, (phase difference between device A and device B) is given by φp=2πfcTp=2πfcTf. The accuracy of the carrier phase measurement can be in the range of 0.01 to 0.05 cycles [2]. For a carrier frequency of 3 GHz, the wavelength is 10 cm, this corresponds to 1 mm to 5 mm accuracy, which is well within cm-level accuracy. To use the carrier phase measurement (φp) for positioning using equation (1), there are a few aspects to consider:

- The signal transmitted from device A to device B is a single carrier frequency with period Tc (frequency fc). However, positioning signals (DL PRS or positioning SRS in UL) are transmitted on more than one sub-carrier that modulates an RF carrier. Leveraging multi-carrier transmission of the DL or UL positioning reference signals is further discussed in section 2.2.

- There are typical synchronization mismatches between the devices involved in the carrier-phase measurement. There are two types for synchronization mismatches. The first is due to clock bias and drift. The second is due to random initial phase at device A and device B. Without mitigating the clock bias and the random initial phase between device A and device B, the phase measurement can’t be used for positioning. This is further discussed in section 2.3.

- The carrier phase method measures fractional propagation delay Tf (φp=2πfcTf). The integer number of cycles N Tc, cannot be measured with a carrier phase single carrier frequency. This gives rise to the well-known, integer ambiguity problem. This is further discussed in section 2.4.

- Equation (1) measures the propagation delay between device A and device B. The propagation delay depends on the path travelled from device A to device B. Having a line-of-sight path gives one propagation delay, but having a non-line-of-sight path gives a different propagation delay. Therefore, to use the propagation delay to find a distance between device A and device B we need to determine the line of sight condition. This is further discussed section.

## Downlink and Uplink Carrier-phase Measurement

In RAN1#112b-e, the reference point of the UE/TRP carrier phase measurement was discussed. There are two options to down-select from based on the following agreement:

Agreement

Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113).

- Option 1:

  - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2.

  - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2.

  - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting.

- Option 2:

  - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2.

  - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF

  - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame

Given that the carrier phase can be reported with other positioning measurements such as DL RSTD from the UE and UL RTOA from the TRP, it seems natural to reuse reference point of DL RSTD at the UE for downlink RSCP, and the reference point of UL RTOA at the TRP for uplink RSCP, i.e., to down-select to option 1 of the aforementioned agreement. For DL RSCP, the reference point can be defined as (same as the reference point of DL RSTD):

- In FR1: The antenna connector of the UE.

- In FR2: The antenna of the UE.

For UL RSCP, the reference point can be defined as (same as the reference point of UL RTOA):

- Type 1-C base station: The Rx antenna connector

- Type 1-O or 2-O base station: The Rx antenna (i.e., the center location of the radiating region of the Rx antenna)

- Type 1-H base station: The Rx Transceiver Array Boundary connector.

Proposal 1: For carrier phase positioning measurement, the DL RSCP reference point is defined the same as the reference point of DL RSTD.

Proposal 2: For carrier phase positioning measurement, the UL RSCP reference point is defined the same as the reference point of UL RTOA.

Another point of discussion in RAN1#112b-e is the impact of phase delays on Tx/Rx chains. Several option have been discussed for potential down-selection in the following agreement:

Agreement

To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113):

- Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals

  - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

  - FFS: the details of \the UE/TRP Tx/Rx PEGs

- Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals.

  - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs

  - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID.

- Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals.

  - The transmission/reception associated with the same ARP ID is assumed from the same ARP.

  - FFS: the maximum number of ARP IDs.

- Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value.

- Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

The carrier phase measurement should be associated with the ID of the reference point used in measuring the carrier phase. The reference point is linked to the hardware used for measuring the carrier phase, which can be a receive antenna or a receive RF chain. Indeed, if the same reference signal is received using different antennas or different receive antenna chains, the carrier phase measurement from each antenna or each antenna chain would be different. Hence, the measurement report can include the ID of the Rx antenna or the ID of the Rx RF chain used to receive the reference signal used for the carrier phase measurement.

Proposal 3: The carrier phase measurement is associated with an ID corresponding to reference point. This can be the Rx antenna ID or the Rx RF chain ID.

On the transmit side, the transmit antenna or the transmit antenna chain used to transmit a positioning reference signal (e.g., DL PRS from a TRP and an SRS for positioning from a UE) can impact the carrier phase measurement of the device receiving the positioning reference signal. Indeed, of a reference signal is transmitted using different antennas or different transmit antenna chains, the carrier phase measurement would be different at the receiving device. Hence, a UE transmitting SRS for positioning can indicate the transmit antenna ID or the transmit chain ID used to transmit the SRS. A TRP transmitting DL PRS can indicate the transmit antenna ID or the transmit chain ID used to transmit the DL PRS.

Proposal 4: A UE transmitting SRS for positioning indicates the Tx antenna ID or the Tx RF chain ID used to transmit SRS.

Proposal 5: A gNB/TRP transmitting DL PRS indicates the Tx antenna ID or the Tx RF chain ID used to transmit the DL PRS.

In RAN1#112b-e options for associating a time stamp with a measurement were discussed. The following two options are under consideration.

Agreement

Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113):

- Option 1:

  - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot.

  - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp.

- Option 2:

  - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements.

The time stamp can be enhanced to include the OFDM symbol index of the DL PRS or SRS for positioning within a slot used for the positioning measurement. In addition to the OFDM symbol index, a reference time associated with the symbol for measuring the phase should be defined. The carrier phase measurement changes depending on the selected reference time. The carrier phase depends on the time it is measured for. At different time instances, the carrier phase is different. Consider the diagram shown in Figure 2, the PRS (DL PRS or positioning SRS) is transmitted in symbol n. In the first device transmitting the PRS the carrier phase is zero after the cyclic prefix (TS 38.211 clause 5.4). The signal arrives at the second device with a zero phase after the CP at the time of arrival. If time between the reference time in the second device and time of the symbol n after the CP is Tn, as shown in Figure 2, the phase at the reference time of the received signal is -2πf Tn mod 2π. This can be the reported phase. The reference time can be chosen such that Tn is independent of the symbol location. For example, the reference time can be the time of symbol n according to the time reference of the second device (e.g., the start of the corresponding Tx symbol), Tn in this case would depend on the propagation delay from the first device to the second device and any synchronization mismatch between the two devices. Another option is to have a reference time that is common for multiple symbols (not PRS symbol dependent) for example the reference time can be at the start of the slot or the subframe or the frame that includes the PRS according to the time reference of the second device (e.g., the Tx time at the start of the slot or sub-frame or frame respectively).  Tn would depend on the location of symbol n in addition to the propagation delay from the first device to the second device and any synchronization mismatch between the two devices.

Figure 2: Carrier Phase reference time.

Proposal 6: For carrier phase positioning measurement, the reported carrier phase is the carrier phase at the start of the slot or symbol containing the reference signal on which the measurement is performed.

If the reported carrier phase is the average phase calculated from multiple PRS symbols, the carrier phase can be averaged form the individual carrier phase measurements from different symbols, however if the carrier phase depends on the location of the PRS, before combining the effect the symbol location on the carrier phase measurement should be removed. For example, if the reference time is at the start of slot, the phase due to the time of the nominal start of symbol n and the CP duration of symbol n should be removed to get a carrier phase independent of symbol location, and that can be combined across multiple symbols.

Proposal 7: For carrier phase positioning measurement, when combining carrier phase cross multiple symbols, the carrier phase measurement should be independent of the symbol location.

For DL RSCPD, the UE provides the difference between the carrier phase from one TRP and the carrier phase from another TRP. The carrier phases can be measured at different times. The UE should maintain continuity between the two measurement instances.

Proposal 8: For carrier phase positioning measurement, for RSCPD, a UE should maintain phase continuity between carrier-phase measurement of TRP1 and TRP2.

## Integer Ambiguity of Carrier-phase Measurement

As described in [5], one method to mitigate the integer ambiguity is to use determine the number of integer cycles in equation (1) by the use of legacy position methods with a coarse positioning estimate that is within the carrier’s period.

Observation 1: If the coarse positioning estimate of the legacy-based positioning methods is accurate enough, the carrier phase measurement can be used to further improve the positioning accuracy.

In RAN1#112 the following agreement was made regarding options to consider for integer ambiguity:

Agreement

To support NR carrier phase positioning, further consider the following options:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier

  - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

- Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF

- Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

The Rel-16 DL PRS and SL positioning SRS include multiple sub-carriers. The multiple sub-carriers can be leveraged to enhance the performance of the carrier phase measurement.

First, consider using two sub-carriers for the carrier phase measurement. The first sub-carrier, at frequency fc1, has a measured carrier phase with integer ambiguity N1, and propagation delay Tp, given by:

φ1=2πfc1Tp-2πN1

For second sub-carrier, at frequency fc2, has a measured carrier phase with integer ambiguity N2, and propagation delay Tp, given by:

φ2=2πfc2Tp-2πN2

Subtracting φ2 from φ1 we get

φ1-φ2=2πfc1-fc2TP-2πN1-N2

This creates a virtual carrier frequency with frequency fcv=fc1-fc2, and virtual phase φv=φ1-φ2.  If fc1 and fc2 are close in value, fcv≪fc1 and fcv≪fc2. Hence, the tolerance (error) of the legacy-based positioning methods can allow the traditional positioning method to estimate the number of cycles for the virtual carrier. This corresponds to option 1 in the aforementioned agreement.

Observation 2: The phase difference between two sub-carriers of DL/UL PRS provides a carrier-phase of a virtual carrier with a lower virtual frequency where the integer number of cycles can be more accurately determined.

Second, the phase measurement across the multiple sub-carriers can be exploited to eliminate the integer ambiguity. The carrier phase measurement can be given by:

φ=2πfcTp+∆T+∆φi mod 2π

φ=2πfcTp+∆T+∆φi-2πN

Where, ∆T is a clock mismatch between the devices involved in the carrier phase measurement, and ∆φi is an initial phase offset between the devices involved in the carrier phase measurement. Reporting measurements based on the carrier phase differentials corresponds to the second option in the aforementioned agreement. Furthermore, by taking the derivative with respect to the carrier frequency eliminates the integer ambiguity.

The clock mismatch remains, but this can be eliminated by one of the methods described in section 2.3.

ddfcφ=2πTp+∆T

The last equation eliminates the integer ambiguity factor. To measure ddfcΔφue, the phase for each subcarrier can be calculated and then the slope of the best line that fits the data can be estimated. Alternatively, the average phase difference between each two consecutive sub-carriers can be measured and reported. The phase is unwrapped to avoid any 2π discontinuities. Figure 1 illustrates an example of getting the carrier phase slope by measuring the sub-carrier phase for each sub-carrier of the DL positioning reference signal or the UL positioning sounding reference signal.

Figure 3: Carrier phase slope.

Observation 3: The carrier phase measurement of DL/UL PRS sub-carriers (e.g., by calculating the slope of the carrier phase measurement relative to frequency) can eliminate the integer ambiguity.

Proposal 9: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase of multiple frequencies of DL PRS or UL positioning SRS (option 1).

Proposal 10: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase difference of two sub-carriers of DL PRS or UL positioning SRS (option 2).

Regarding option 3, “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF”, if the UE is already providing a time of arrival measurement or time difference measurement, with a certain level of accuracy, this is already providing an implicit integer ambiguity. For example, the NR-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement).

NR-TimingQuality-r16 ::= SEQUENCE {

timingQualityValue-r16			INTEGER (0..31),

timingQualityResolution-r16		ENUMERATED {mdot1, m1, m10, m30, ...},

...

}

The timingQualityResolution provides the resolution used in the timingQualityValue field. This can correspond to a resolution as low as 0.1 meters.

Observation 4: The positioning measurements provided by the UE include a timing quality, which can be used by the LMF to estimate the integer ambiguity.

Proposal 11: Don’t support option 3 (UE/TRP to optionally report an estimated integer ambiguity).

Regarding option 4, “Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data”. Similar to the discussion about option 3, if the LMF provides a time measurement with a certain level of accuracy/uncertainty, this is already providing an implicit integer ambiguity. For example, the UE assistance information includes: nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, for RSTD, this can be used by the UE to estimate a range for the integer ambiguity.

Observation 5: The UE assistance information to UE includes nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, which can be used by the UE to determine the expected integer ambiguity range.

Proposal 12: Don’t support option 4 (LMF to provide the expected integer ambiguity range).

## Impact of synchronization mismatch on Carrier-phase Measurement

There are two types of synchronization mismatch that can occur between devices involved in the carrier-phase measurement (i.e., the device transmitting the PRS used for the carrier-phase measurement (Device-A) and the device performing the carrier phase measurement (Device-B)).

- Clock bias. There is a clock bias between the timeline of Device-A and Device-B. This clock bias can be represented by ∆T.

- Initial phase difference. The initial phase difference between Device-A and Device be is a random variable, ∆φ over the range 0,2π.

The measured phase difference with these synchronization mismatches becomes φp=2πfcTp+∆T+∆φ. Without mitigating the synchronization mismatches, the measured carrier phase can’t be used for positioning.

Observation 6: Without mitigating the synchronization mismatches between the devices involved in the carrier-phase measurement, the carrier-phase measurement can’t be used for positioning.

As described in [5], one method to mitigate the synchronization mismatches is to use the round-trip carrier phase, by measuring the carrier phase of a signal transmitted from Device-A (e.g., gNB) at Device-B (e.g., UE), and measuring the carrier phase of a signal transmitted from Device-B (e.g., UE) at Device-A (e.g., gNB) and adding the two measured carrier phase together. By doing so, the clock biases and phase mismatches cancel out.

Observation 7: Round-trip carrier phase can be used to mitigate the effect of initial phase offset difference, if phase coherency can be maintained between the RF transmit chain and RF receive chain of the UE and the gNB used for DL and UL carrier-phase measurement.

As described in [5], another method to mitigate the synchronization mismatches is to use a positioning reference unit (PRU), which is a device with a known location can that can be used to estimate the clock biases and phase mismatches between TRPs. The PRU should be determined as device with a known accurate location and close to the target device whose position is being determined. It would be desirable to achieve the PRU’s functionality, which is eliminating clock biases, without having to introduce a new node type. In legacy positioning, the use of PRU is transparent. However, since it is critical to have PRU for properly using the carrier-phase measurement for positioning, it’s essentially beneficial for RAN1 to study how to provide PRU or PRU-like functionality under existing system, e.g., how to determine a PRU and apply the PRU during the positioning procedure.

Agreement

To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements:

- Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s).

  - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time

- Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s).

  - Note: this may be a different indicated time window

Agreement

To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements:

- Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s).

- FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

In last meeting, the following agreement is reached.

In both agreements, the PRU could measure the PRS or transmit the SRS in an indicated time window. One issue is whether the UL SRS or the PRS for the PRU should also be indicated or not. For the purpose of having PRU, which is providing the calibration function, it is not reasonable to ask the PRU to measure all the PRS for the target UE or transmit all the SRS for the target UE. On the other hand, one PRU could help multiple UEs at the same time window, or overlapped time window, so the DL PRS or UL SRS for positioning configured for PRU could be just a masked version of the that from the target UE. For example, given existing PRS or SRS configuration, the PRU is given a resource mask indication on which PRS(s) or SRS(s) are to measure or transmit. By doing this, the usage of PRU could be much more efficient and controllable to network side.

Observation 8: A PRU can be considered for elimination of gNB and UE initial phase offset difference. Wherein, a PRU device can be determined as a device with a known accurate location.

Proposal 13: For simultaneous transmission/reception of UL SRS/DL PRS for positioning by a target UE and a PRU, support the PRU transmits/receives the indicated UL SRS/DL PRS by masking the resource from corresponding target UE.

Another aspect to consider, when receiving the UL SRS for positioning at a TRP from the target UE and the PRU is to have the same Rx RF chain used to receive the UL SRS for positioning from the target UE and the PRU to ensure the same synchronization and phase errors for both the target UE and the PRU. For example, the network can configure a UE to use the same Rx RF chain to receive the UL SRS for positioning from the target UE and the PRU using the same Rx RF chain.

Proposal 14: A UE receives the UL SRS for positioning from the target UE and the PRU using the same Rx RF chain.

As described earlier another method to eliminate synchronization errors when using the carrier phase measurement is use the round-trip carrier phase, where the UL RSCP measured at a TRP and the DL RSCP measure at a UE are added together to eliminate or cancel synchronization errors between the TRP and the UE. To ensure that the synchronization can be cancelled phase coherency is assumed between Rx RF chain and the Tx RF chain of the gNB/TRP and phase coherency is assumed between Rx RF chain and the Tx RF chain of the UE.

Proposal 15: For carrier phase positioning measurement, RAN1 considers providing measurements and necessary configuration for round-trip carrier phase measurement.

## Carrier-Phase Measurement Report Content

In RAN1#112b-e, it was agreed to report the DL RSCPD with the RSTD, the DL RSCP with the UE Rx-Tx time difference, the UL RSCP with the RTOA and the gNB Rx-Tx time difference. However, it is still open whether to support standalone measurement reports with carrier phase measurements. We see it beneficial to support standalone carrier phase reporting. For example, when the carrier phase report includes the carrier phase of multiple sub-carriers or the carrier phase difference between sub-carriers, the impact of integer ambiguity is reduced and a standalone carrier phase report can be used to determine the position of the UE. This is also useful for applications that require accurate user tracking at different time instances, with small change in user position between one-time instance and the next, for these applications, it is sufficient to provide a standalone carrier phase measurement report.

Proposal 16: Support standalone carrier phase measurement report for:

- DL reference signal carrier phase (DL RSCP).

- DL reference signal carrier phase difference (DL RSCPD).

- UL reference signal carrier phase (UL RSCP).

The measurement report for the carrier phase measurement can include the following parameters:

- The carrier phase measurement in radians (or degrees)

  - This can be UL carrier phase measurement of positioning SRS from a UE or DL carrier phase measurement of PRS from a TRP or DL carrier phase difference measurement between two TRPs (a reference TRP and a target TRP)

  - Carrier phase measurement can be a single measurement (e.g., at the carrier frequency of the carrier). Alternatively, the carrier phase measurement can be at two or more sub-carriers.

  - Measurement can be provided derived from the carrier phase. For example, this can the average of the carrier phase difference between two consecutive sub-carries.

- Reference signal ID of the reference signal used for the carrier phase measurement. If the carrier phase measurement is the difference between the carrier phase of two TRPs, a reference signal ID can be provided for each TRP

- A time stamp(s) can provided for the time (e.g., slot) at which the carrier phase measurement(s) was performed.

- Quality indicator for the carrier phase measurement, e.g., based on the quality of the first (LOS) component.

- A phase continuity indicator, this can indicate if the device performing phase measurements has maintained phase continuity since the last reported measurement. This can be useful for the LMF when combining multiple phase measurements.

Proposal 17: For carrier phase positioning measurement, consider including the following fields in the measurement report:

- Carrier phase measurement

- Reference signal ID

- Time stamp

- Quality indicator

- Phase continuity indicator since last measurement

# Conclusions

The following observations and proposals have been made regarding the carrier phase measurement

Proposal 1: For carrier phase positioning measurement, the DL RSCP reference point is defined the same as the reference point of DL RSTD.

Proposal 2: For carrier phase positioning measurement, the UL RSCP reference point is defined the same as the reference point of UL RTOA.

Proposal 3: The carrier phase measurement is associated with an ID corresponding to reference point. This can be the Rx antenna ID or the Rx RF chain ID.

Proposal 4: A UE transmitting SRS for positioning indicates the Tx antenna ID or the Tx RF chain ID used to transmit SRS.

Proposal 5: A gNB/TRP transmitting DL PRS indicates the Tx antenna ID or the Tx RF chain ID used to transmit the DL PRS.

Proposal 6: For carrier phase positioning measurement, the reported carrier phase is the carrier phase at the start of the slot or symbol containing the reference signal on which the measurement is performed.

Proposal 7: For carrier phase positioning measurement, when combining carrier phase cross multiple symbols, the carrier phase measurement should be independent of the symbol location.

Proposal 8: For carrier phase positioning measurement, for RSCPD, a UE should maintain phase continuity between carrier-phase measurement of TRP1 and TRP2.

Observation 1: If the coarse positioning estimate of the legacy-based positioning methods is accurate enough, the carrier phase measurement can be used to further improve the positioning accuracy.

Observation 2: The phase difference between two sub-carriers of DL/UL PRS provides a carrier-phase of a virtual carrier with a lower virtual frequency where the integer number of cycles can be more accurately determined.

Observation 3: The carrier phase measurement of DL/UL PRS sub-carriers (e.g., by calculating the slope of the carrier phase measurement relative to frequency) can eliminate the integer ambiguity.

Proposal 9: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase of multiple frequencies of DL PRS or UL positioning SRS (option 1).

Proposal 10: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase difference of two sub-carriers of DL PRS or UL positioning SRS (option 2).

Observation 4: The positioning measurements provided by the UE include a timing quality, which can be used by the LMF to estimate the integer ambiguity.

Proposal 11: Don’t support option 3 (UE/TRP to optionally report an estimated integer ambiguity).

Observation 5: The UE assistance information to UE includes nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, which can be used by the UE to determine the expected integer ambiguity range.

Proposal 12: Don’t support option 4 (LMF to provide the expected integer ambiguity range).

Observation 6: Without mitigating the synchronization mismatches between the devices involved in the carrier-phase measurement, the carrier-phase measurement can’t be used for positioning.

Observation 7: Round-trip carrier phase can be used to mitigate the effect of initial phase offset difference, if phase coherency can be maintained between the RF transmit chain and RF receive chain of the UE and the gNB used for DL and UL carrier-phase measurement.

Observation 8: A PRU can be considered for elimination of gNB and UE initial phase offset difference. Wherein, a PRU device can be determined as a device with a known accurate location.

Proposal 13: For simultaneous transmission/reception of UL SRS/DL PRS for positioning by a target UE and a PRU, support the PRU transmits/receives the indicated UL SRS/DL PRS by masking the resource from corresponding target UE.

Proposal 14: A UE receives the UL SRS for positioning from the target UE and the PRU using the same Rx RF chain.

Proposal 15: For carrier phase positioning measurement, RAN1 considers providing measurements and necessary configuration for round-trip carrier phase measurement.

Proposal 16: Support standalone carrier phase measurement report for:

- DL reference signal carrier phase (DL RSCP).

- DL reference signal carrier phase difference (DL RSCPD).

- UL reference signal carrier phase (UL RSCP).

Proposal 17: For carrier phase positioning measurement, consider including the following fields in the measurement report:

- Carrier phase measurement

- Reference signal ID

- Time stamp

- Quality indicator

- Phase continuity indicator since last measurement

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, 3GPP RAN#94e, e-Meeting, December, 2021.

- TR 38.859, “Study on expanded and improved NR positioning”, V18.0.0, January 2023.

- RP-223549, “New WID on Expanded and Improved NR Positioning”, 3GPP RAN#98e, e-Meeting, December, 2022.

- Pratap Misra, and Per Enge, Global Positioning System: Signals, Measurements, and performance, Ganga-Jamuna Press, 2012.

- R1-2212859, “Discussion on NR Carrier Phase Positioning Measurements”, Samsung, 3GPP, RAN1#111, Toulouse, France, November 2022.