3GPP TSG RAN WG1 #113			R1-2305538

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:		9.12.1

Source:				Samsung

Title:					PRACH coverage enhancements

Document for:	Discussion

# Introduction

In the R18 WI for Further NR coverage enhancements [1], one important direction is to enhance the random access channel for a coverage-limited UE.

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

This contribution discusses the design consideration for multiple PRACH transmission with same beam as well as different beams.

# Multiple PRACH with different beams

Interests for multiple PRACH transmission has been raised almost from first days in RAN1 design on NR initial access. The unique, beam-based directional transmission in NR triggered the need for multiple PRACH transmissions to explore either spatial diversity gain or the repetition gain for FR2. The fundamental reason is that a UE has no knowledge of which direction is the best for it to transmit a PRACH.

In Rel-15 RAN1 NR Ad-Hoc#2 [2], RAN1 had agreed to support multiple msg1 transmission with supporting both same and different UE Tx beams. The argument at that time was that, for a UE with beam correspondence, the UE knows the preferred UL Tx beam based on the DL Rx beam, then it can enjoy the repetition gain for quick access. For the UE without beam correspondence, the UE does not know the preferred UL Tx beam thus it needs to try with different beam directions. So multiple PRACH transmissions with different Tx beams could allow such UE to quickly do UL beam sweeping and get identified by the gNB.

Observation 1: Both UEs with or without beam correspondence could benefit from multiple PRACH transmissions with same or different beams.

During the continuous design of NR, with joint discussion with RAN1 and RAN4, the beam correspondence became a UE mandatory feature for a NR UE with different condition. It is still realistically impossible for all UEs to know the preferred UL Tx beam during initial access. Based on the description for the UE's beam correspondence capability IE, beamCorrespondenceWithoutUL-BeamSweeping, as captured below, a UE not presented with such IE needs to satisfy the corresponding requirement with UL beam sweeping. This suggests that a UE without such capability needs multiple chances for different beam transmissions.

From TS 38.101-2

6.6.4.1	General

The beam correspondence requirement for power class 3 UEs consists of three components: UE minimum peak EIRP (as defined in Clause 6.2.1.3), UE spherical coverage (as defined in Clause 6.2.1.3), and beam correspondence tolerance (as defined in Clause 6.6.4.2).  The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions, depending on the UE's beam correspondence capability IE beamCorrespondenceWithoutUL-BeamSweeping, as defined in TS 38.306 [14]:

-	If beamCorrespondenceWithoutUL-BeamSweeping is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 with its autonomously chosen UL beams and without uplink beam sweeping.  Such a UE is considered to have met the beam correspondence tolerance requirement.

-	If beamCorrespondenceWithoutUL-BeamSweeping is not present, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.3-1 and spherical coverage requirement according to Table 6.2.1.3-3 with uplink beam sweeping.  Such a UE shall meet the beam correspondence tolerance requirement defined in Clause 6.6.4.2 and shall support uplink beam management, as defined in TS 38.306 [14].

In addition, since Rel-16/17 specified multi-TRP operation, PRACH transmission with different beams could increase the possibility of being detected by any of the TRPs which may be located in different directions.

Thus, for a UE that initiates the contention-based 4-step RACH and without the beam correspondence capability, it is necessary to support different beams for multiple PRACH transmission.

Observation 2: if beamCorrespondenceWithoutUL-BeamSweeping is not present, a UE needs to conduct uplink beam sweeping.

Proposal 1: RAN1 supports to specify multiple PRACH transmissions with different beams for 4-step RACH procedure.

# Design for multiple PRACH transmissions

In case of the design for multiple PRACH transmissions, there will be several major aspects to be considered. One aspect is related to PRACH configuration, which includes the RO/preamble resource determination as well as the beam determination. Another aspect is the impact on UE procedure when multiple PRACH transmissions are enabled, such as power control, power ramping, RAR monitoring, and so on. In addition, the MPE constraint can also influence the design.

## RACH resource aspects

### RO/RO group determination

Agreement from RAN1#112b-E

- Multiple PRACH transmissions within one RACH attempt are only performed within one RO group.

  - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions.

    - Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value.

    - Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions.

    - Note 3: Valid RO(s) refers to what is defined in existing specification.

Agreement from RAN1#112

For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions.

Note 1: All ROs in one RO group is associated with the same SSB(s).

Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Note 3: whether/how to define “RO group” in specification will be discussed separately

Note 4: Valid RO(s) refers to what is defined in existing specification

FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH.

FFS: the time span of RO group.

FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions.

FFS: other details

Agreement from RAN1#112

Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

Agreement

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

- FFS: details

Based on last meeting agreement on the RO group, which is the same concept as the RO bundle in our previous discussion paper. A clearer design on how the UE uses the RO group is needed to facilitate multiple PRACH transmissions. In addition, there are also agreements on supporting the configuration of multiple numbers of multiple PRACH transmissions.

In this section, several aspects related to RO group determination will be discussed, including the general procedure to determine the RO group pattern, and handling of RO collision between single PRACH and multiple PRACH, the constraints on the time span of RO group, RO group for different N values and the possibility of FH within RO group.

#### RO group determination process

As commonly understood, the RO group defined in last meeting is considered after SSB-RO association. This means after UE reading the PRACH configuration, finishing the RO validation, and performing the SSB-RO association, the UE starts to determine the RO group pattern. So, at this stage, the UE already has the RO pattern for associated SSBs. Thus, it’s beneficial to determine the RO group pattern based on the RO pattern for each associated SSBs within the SSB-RO association pattern period, for the following reasons:

- The number of ROs is same for every associated SSB

- The RO pattern associated with every SSB will be repeated in every SSB-RO association pattern period.

With above two important features, so the UE only needs to derive the RO group pattern within one SSB-RO association pattern period, these RO group pattern for each associated SSB could be also repeated with any other SSB-RO association pattern period. This outcome is aligned with the design principle of SSB-RO association pattern period.

Observation 3: it’s beneficial to determine the RO group pattern based on the RO pattern for each associated SSB within the SSB-RO association pattern period.

The possible procedure could be illustrated by following example:

After the SSB-RO association, the two SSBs (SSB0 and SSB1) have corresponding associated ROs in SSB-RO association pattern as shown in following figure 1. The RO pattern is the outcome of configuration->validation-> association.

Fig.1 – illustration of RO group pattern determination process

So, when the UE is configured with N=2, it starts to determine the possible RO group with 2 adjacent ROs in time domain from the beginning of the SSB-RO association pattern period till the end of it, similar thing happened if UE is being configured with N=4 or N=8. Note that there could be some leftover ROs which cannot form a complete RO group, as shown by the dished box in the figure. These ROs cannot be used for multiple PRACH transmissions and the gNB should ensure that there is at least one RO group available to use if the corresponding N value is configured.

As discussed in last meeting, the left proposal from FL is shown in below:

Proposal

A set of RO group(s) for a configured number of multiple PRACH transmissions is [determined/configured] within a time period X, starting from frame 0. The [determined/configured] set of RO groups repeats every period X.

- FFS: the determination of time period X.

- FFS: whether the same time period X is applied to all the configured number(s) of multiple PRACH transmissions.

To align our discussion in above, the proposal is a good way forward, and for the choice of [determined/configured], the “determined” is more accurate since there should be some UE operation to final confirm the available PRACH resources. Regarding the time period X, we have to emphasize that two important features on determining such time period X:

- The set of RO groups determined in the time period X should repeat. The outcome of asking UE to continuously determine the RO group pattern is NOT acceptable, and also against of the vital reason to introducing such time period X. Besides, this is also the same logic why eventually we introduced the SSB-RO association pattern period in Rel15;

- The time period X should accommodate enough valid ROs in time domain for completing at least one RO group for a configured N value. To achieve this, there could be two ways:

  - gNB aligns the configuration of N and number of RO in time domain in a SSB-RO association pattern period, thus ensure at least one RO group is available to use if the corresponding N value is configured;

  - extend to K>1 SSB-RO association pattern period.

For these two ways, the first one is preferred since the configuration is all on gNB implementation and extension to K SSB-RO association pattern period could complicated the design.

Proposal 2: support determination of RO group pattern for a SSB within the SSB-RO association pattern period;

Proposal 3: it’s up to gNB implementation to ensure that there is at least one RO group available to use if the corresponding N value is configured.

Regarding whether the same time period X is applied to all the configured number(s) of multiple PRACH transmissions, we think it’s important to align the starting position of each configured N value and confine these RO groups in the same time period X to ease the resource management. Otherwise, the RO group pattern could be quite complicated to maintain especially when multiple N value configured. Thus, overall, we have following proposal.

Proposal 4: A set of RO group(s) for a configured number of multiple PRACH transmissions is determined within a time period X, starting from frame 0. The determined set of RO groups repeats every period X.

- the determination of time period X is one SSB-RO association pattern period.

- the same time period X is applied to all the configured number(s) of multiple PRACH transmissions.

Note: it’s up to gNB implementation to ensure that there is at least one RO group available to use if the corresponding N value is configured.

#### Collision of RO

As we agreed the RO for multiple PRACH and singe PRACH could be differentiated by separate PRACH configuration. Given the limited choice of the PRACH configuration index, there is a chance that the configured RO for multiple PRACH is overlapped with the configured RO for single PRACH. However, it’s not a new issue since the 2-step RACH with separate RO have the same overlap as well. And the suitable solution is also the same, to leave to gNB configuration to solve the collision, e.g., when the overlapped RO is unavoidable, gNB could configure different preamble sets for the single PRACH or multiple PRACH. This solution is similar to the shared RO case but under the separate RO configuration.

Proposal 5: the handling of RO collision between single PRACH and multiple PRACH is up to gNB configuration.

Proposal

If one or more PRACH transmission(s) of the multiple PRACH transmission in one RACH attempt are dropped based on the rules causing to drop PRACH transmission in existing spec., the dropped PRACH transmission is not postponed.

  - FFS: whether to introduce new rules causing to drop PRACH transmission.

  - FFS: whether there is standard impact if of the dropped RACH transmission affect the remaining RACH transmission within the same RO group.

In addition, there could be another collision handling for multiple PRACH transmission, as FL proposal in following:

The existing rules defined in current spec could cause the PRACH Tx dropping in corresponding RO location. Besides, there could be some impact to the PRACH transmission before/after such collided PRACH Tx. For example, if PRACH transmission in RO 2 gets cancelled by existing rules, by DL Rx (in another CC), so UE may cancel such PRACH transmission and because there could be switching time needed before or after such RO. Note that the restrictions on the valid RO cannot be used for DL channel reception (section 11.1 in TS38.213) is for single CC based on the common understanding when the decision made in Rel15. In addition, although be configuration, the RO1 might be in flexible symbol and not configured as DL, but since the RO 2 has DL reception, the symbols in RO1 are likely to be DL as well. Thus, the PRACH transmissions in both RO1 and RO3 could be dropped.

Fig. 2 illustration of impact of other ROs.

Proposal 6: if PRACH Tx in ith RO within a RO group is cancelled for multiple PRACH transmission based on existing rules, the PRACH Tx in (i-1)th RO and (i+1)th RO (if exist) are also cancelled.

#### Time span of RO group

Due to the plenty outcome of RO pattern after configuration, validation and association, the determined RO group might not be a very nicely formation, e.g., there could be some gap between different ROs within one RO group. From configuration perspective, it should not be realistic or reasonable to have a RO group time span covers quite large time duration. This might be ensured by gNB configuration as well, although it’s also an open point to discuss whether to put some limits on the time span of the RO group, e.g., the time span of one RO group cannot exceed a certain time duration threshold. This additional rule might ask the UE to drop some RO groups whose time span does not satisfy the requirement, which causes resource to be wasted. Thus, maybe combination of both directions could be also considered as the compromise to handle such issue.

Fig.3 illustration of possible time span restriction

Proposal 7: RAN1 considers the following alternatives to ensure the time span of the RO group to be kept for a reasonable time duration:

Alt. 1: leave to gNB configuration;

Alt.2: the time span of one RO group is required not to exceed a certain time duration threshold

#### RO group for different N values

As agreed in last meeting, multiple different N values can be configured by gNB. But the details on how to differentiate the different N values are still FFS. And the configuration details are also not clear. Let’s say, there are m N values for multiple PRACH, so that the value of “m” and the corresponding “N” values need to be configured to UE.

Then, when a UE reads the configuration, it knows how many N values and what N values are configured in the system. In order to differentiate them, in addition to normal methods like different preamble and/or different RO which come from different PRACH configuration, we can also use the FDMed RO to differentiate. As agreed in RAN1, the RO group is a time domain concept. So, if gNB configures multiple FDMed ROs (in current spec, it can be up to 8 FDMed ROs), one or multiple FDMed ROs could be associated with a certain N value. Thus, if the UE selects corresponding RO group with the associated FDMed RO, the gNB will also determine the UE’s selection based on the different FDMed RO.

Fig. 4 example of using FDMed RO to differentiate multiple N values

Proposal 8: RAN1 considers the following alternatives to differentiate multiple N values:

Alt. 1: separate preamble with shared RO;

Alt.2: separate RO;

Alt. 3: FDMed RO (in same PRACH configuration)

#### FH within RO group

Another issue that was discussed in the previous meeting was whether to allow the FDM RO or CDM/SDM in one RO for multiple PRACH transmission.

Agreement

- For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions.

- FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

- FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

For the first FFS point, the intention is to explore the frequency domain diversity for the multiple PRACH transmissions. It might be beneficial for the case with the larger frequency selective channel condition. However, such benefits may not be essential for multiple PRACH transmissions with same Tx beam, in which the key point is the combination gain through time domain.  For the second FFS point, however, the applicable scenario will be much more limited. It first requires a UE equipped with multiple panel/multiple Tx chains with separate power supply. In addition, for multiple transmission with same tx beam, it cannot add the spatial domain diversity unless it uses different preambles targeting to different SSB with is reception spatial domain diversity.

Proposal 9: Different frequency location of the ROs for one multiple PRACH transmission may or may not be considered.

### Shared or separate PRACH resource

In last meeting, the PRACH resource used for multiple RACH transmission is as working assumption for whether it should be shared or separate PRACH resource with legacy PRACH.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

As we can see, the Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17)  are designed to be very flexible and forward compatible so that reusing it for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning is very natural choice.

Proposal 10: for separate RO configuration, support reusing Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17).

### Beam determination

Even though whether different Tx beams are supported or not is still under study, the choice for beam determination could follow the current NR RACH procedure, that is, the Tx beam for preamble transmission is up to UE implementation. For the case of same UE Tx beam, the UE Tx beam for the first PRACH transmission of the multiple PRACH transmissions in a RACH attempt should apply to all PRACH transmissions in the RACH attempt. The UE can change the UE Tx beam for another RACH attempt which is up to UE implementation.

Proposal 11: The UE Tx beam for multiple PRACH transmissions is up to UE implementation.

Proposal 12: For multiple PRACH transmissions with same Tx beam, the Tx beam determined for the first PRACH transmission of the multiple PRACH in a RACH attempt should apply to all PRACH transmissions in the RACH attempt.

## RACH procedure aspects

In the previous discussion for multiple PRACH (a.k.a., multiple msg1) transmissions, one important principle is to keep one RACH procedure for a UE at any given time. This constrains the complexity of multiple RACH design in both PHY layer and higher layer. Similar to legacy behaviour, there is always one RACH procedure ongoing for a UE.

Proposal 13: The multiple PRACH transmission is kept as one RACH procedure.

### switching to higher number of multiple PRACH transmission

In last meeting, the following agreement was reached.

Agreement

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

FFS: details

Following previous design logic, the UE will select one N value based on the DL-RSRP, but the question is, whether the UE is allowed to switch to a higher N value during the multiple PRACH transmissions. For example, if a UE initially selects N=2, whether it can switch to N=4, or N=8 if configured.

There are pros and cons for such potential flexibility. The beneficial part is that it could be more flexible for UE to access the system quicker and possibly use the actual largest power (considering the N time combination) in the system. This could be good for the coverage limited UE which wrongly determines the N value initially. However, it could also complicate the behaviour to have switching operation, such the suitable switching condition should be studied and determined. The reason that a UE fails in a RACH attempt could be quite diverse and increasing the N value is a solution to just one possible reason. If this is wrongly used, the unnecessary interference could also be caused.

Proposal 14: FFS whether allow to switch to higher number of multiple PRACH transmission if applicable.

### PDCCH order

For a UE with RRC connection, the UE can be triggered for PRACH transmission not only by higher layers, but also by a PDCCH order from the gNB. Therefore, more discussion is needed for the case of multiple PRACH transmissions that are triggered by PDCCH order. For example, if the trigger for multiple PRACH transmissions is still based on UE determination of SSB RSRP, which is not known to the gNB, may not be suitable for the case of PDCCH order, and some more study is needed. For example, the gNB signalling can indicate to the UE whether to perform single PRACH transmission or multiple PRACH transmissions. In addition, the DCI format for PDCCH order may need to be further studied for the case of multiple PRACH transmissions.

Proposal 15: Further study multiple PRACH transmissions triggered by PDCCH order.

### Power control

During one RACH attempt with multiple PRACH transmission, the intended transmission power for each of the PRACH should be the same. The intended transmission power means the calculated Tx power based on the power control procedure specified in TS 38.213. It should be noted that the actual transmission power will consider the power prioritization rules in the event of overlap with other UL signals.

Proposal 16: The calculated transmission power for each of the PRACH in one attempt should be the same.

In order to keep the same calculated Tx power for all PRACH transmissions in one attempt, the same measurement of the same reference signal to calculate the pathloss should be applied, otherwise, there is great possibility that the path loss value could vary from different reference signals or from different measurements from same reference signal.

Proposal 17: The same measurement of the same reference signal to calculate the pathloss should be applied for each of the Multiple PRACH transmissions in one attempt.

In case of multiple N values configured, one issue is whether use same set of P0 alpha or to use different set of P0 alpha for different N value. In legacy NB-IOT or eMTC discussion, such configuration is allowed. But as we understand it is because the PRACH transmission time is quite large (e.g., 64, 128), so the reception level is quite different from single PRACH. In our current discussion in multiple PRACH, the agreed value is only 2, 4, 8 which is based on the single PRACH transmission. For that reason, we think it’s ok to use the same set of P0 and alpha configured single PRACH.

Proposal 18: The multiple PRACH transmission use the same set of P0 and alpha configuration from single PRACH transmission.

### Power ramping

In last meeting, it agrees

Agreement

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

In legacy R15, the power ramping counter is introduced in addition to the legacy preamble transmission counter. The power ramping counter will increase during the RACH re-attempt if the selected UL Tx beam and the selected SSB doesn’t change, otherwise, the power ramping counter should be kept unchanged. A similar principle can apply to multiple PRACH transmissions with same Tx beam. For multiple PRACH transmissions with different Tx beam, a different manner to consider the change of UL Tx beam(s) should be studied.

Proposal 19: FFS power ramping behaviour if the case with different Tx beams is supported.

### RA-RNTI

In last meeting, the following option is agreed:

Agreement from RAN1#112

For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

Agreement from RAN1#112b-E

The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions.

Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213.

Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

For the issue in the RA-RNTI calculation, for separate RO case, there is not much optimization necessity, but for the shared RO case, there could be one. The RA-RNTI calculation in legacy is not intended to separate them in RO, which means, as long as the UE, for whatever feature, selects the same RO, they will have same calculated RA-RNTI, and they will detect the same PDDCH no matter gNB detects whichever preamble in that RO.

In previous release, this might not be an issue since the feature number is not large (only 2 step RACH and 4 step RACH), but given now the Rel17 introduces up to 16 new separate PRACH configurations to different features/combinations and now the introduction of multiple PRACH transmission with configuration of multiple N values in Rel-18, such “collision” could happen far more easily. This could have lot of more negative impacts. Not only consuming UE’s time and resource to detect and decode the PDDCH which is never intended for it, but in the given UE handling for current procedure, it could suspend the RAR monitoring in the window after the PDCCH-PDSCH decoding if RAPID is not matched. Thus, we think it is important to separate UE from selecting same RO but for different N value by RA-RNTI.

Observation 4: it’s beneficial to separate UE from selecting same RO but for different N value by RA-RNTI.

One solution could assign the possible symbol index and the RO location within the RO group. More specifically, in current design of RA-RNTI, it is:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id,

Without impacting the equation and the legacy behaviour, where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14) for single PRACH transmission. The s_id for multiple PRACH transmission case could be like: the s_id is the index o f the second/last OFDM symbol of the xth (x=N/2) PRACH occasion within a RO group (0 ≤ s_id < 14) for multiple PRACH transmission with number N (N=2,4,8). As shown in following figure, by this solution, it has following benefits:

- the single PRACH can separate with multiple PRACH (with any number N) by the OFMD symbol used in RA-RNTI calculation;

- the multiple PRACH transmission with different N values can be separated with different RO selection for s_id, i.e., the x value;

Fig.5 – illustration of s_id for RA-RNTI calcuation

Proposal 20: for RA-RNTI calculation,  the s_id is the index of the second/last OFDM symbol of the xth (x=N/2) PRACH occasion within a RO group (0 ≤ s_id < 14) for multiple PRACH transmission with number N (N=2,4,8).

### Preamble/PRACH transmission max

In normal RACH procedure, when UE goes to a new PRACH attempt, it will increase the preamble transmission counter by 1, and the overall procedure is constraint by the max allowed preamble transmission number. This works for single PRACH transmission since UE would only transmit one preamble at one attempt.

However, for multiple PRACH transmission, there could be two directions if we still want to have such similar constraint. One is still using preamble transmission counter and limited by the max allowed preamble transmission number, only for each re-attempt, we should add the used N value rather than just one. The other direction, is that we can have a PRACH attempt counter, only counts the PRACH attempt times and limits it by gNB configured max allowed attempt number.

Proposal 21: to constraint the total number of multiple PRACH transmission, considering following to alternatives:

At.1: reuse preamble transmission counter by adding on the multiple transmission number for each attempt;

Alt.2: introduce the PRACH attempt counter and adding one for each attempt.

### Impact of maximum permissible exposure (MPE)

While DL coverage can be well determined and handled based on RSRP measurements, UL coverage may be impacted by additional aspects, such as UE Tx power limitations due to maximum permissible exposure (MPE) issues. Basically, the UE needs to apply additional power back-off, referred to as P-MPR, to meet regulatory requirements. Therefore, UL coverage for a UE may be worse than the UE’s DL coverage since not only the RSRP but also the P-MPR / MPE impact the UL coverage.

Handling the MPE issue was considered since NR Rel-15, where RAN4 specified corresponding P-MPR values [TS 38.101-2]. Additional handling for MPE for Connected-mode UEs was considered by RAN2 in NR Rel-16, where a new power headroom report (PHR) for MPE was adopted, which was later enhanced by RAN1 under Rel-17 feMIMO WI to support beam-specific PHR for MPE reporting [TS 38.321].

However, during initial access, there is no RRC connection established for the UE with the gNB, so the above enhancements will not be applicable. Therefore, a UE facing MPE issue (i.e., large P-MPR power back-off) will experience more significant UL coverage issue, and requires further coverage enhancement:

- MPE considerations, in addition to RSRP considerations, can be used for determination of the number of Multiple PRACH transmissions, and power settings for Multiple PRACH transmissions. For example, a UE with large P-MPR power back-off (i.e., worse MPE issue) may be allowed to perform larger number of Multiple PRACH transmissions. In addition, the Multiple PRACH transmission trigger may also be impacted by this MPE limitation. For example, a UE with moderate RSRP that would otherwise not qualify for multiple PRACH transmission, may be allowed to trigger multiple PRACH transmissions when facing large P-MPR power back-off (i.e., worse MPE issue).

- MPE, in addition to RSRP, can be also considered for selection of SSB(s) associated with the multiple PRACH transmissions. For example, when a UE identifies only a first SSB with RSRP above the threshold, TS 38.321 specifies that the UE selects the first SSB for PRACH association. However, if UE Tx beams associated with the first SSB have MPE issues (i.e., large P-MPR power back-off), then PRACH transmission associated with the first SSB would be UL coverage limited. In such case, it may be preferable for UL coverage that the UE uses a second SSB, possibly with RSRP below the threshold, that has no MPE issues.

Proposal 22: Further study multiple PRACH transmission enhancements when UE experiences MPE issues, e.g., impact of MPE on: number of multiple PRACH transmission, power settings, the trigger for multiple PRACH transmission, and SSB selection for PRACH association.

# Conclusion

This contribution discusses the design consideration for multiple PRACH transmission. Observations and proposals are summarized as follows:

Observation 1: Both UEs with or without beam correspondence could benefit from multiple PRACH transmissions with same or different beams.

Observation 2: if beamCorrespondenceWithoutUL-BeamSweeping is not present, a UE needs to conduct uplink beam sweeping.

Proposal 1: RAN1 supports to specify multiple PRACH transmissions with different beams for 4-step RACH procedure.

Observation 3: it’s beneficial to determine the RO group pattern based on the RO pattern for each associated SSB within the SSB-RO association pattern period.

Proposal 2: support determination of RO group pattern for a SSB within the SSB-RO association pattern period;

Proposal 3: it’s up to gNB implementation to ensure that there is at least one RO group available to use if the corresponding N value is configured.

Proposal 4: A set of RO group(s) for a configured number of multiple PRACH transmissions is determined within a time period X, starting from frame 0. The determined set of RO groups repeats every period X.

- the determination of time period X is one SSB-RO association pattern period.

- the same time period X is applied to all the configured number(s) of multiple PRACH transmissions.

Note: it’s up to gNB implementation to ensure that there is at least one RO group available to use if the corresponding N value is configured.

Proposal 5: the handling of RO collision between single PRACH and multiple PRACH is up to gNB configuration.

Proposal 6: if PRACH Tx in ith RO within a RO group is cancelled for multiple PRACH transmission based on existing rules, the PRACH Tx in (i-1)th RO and (i+1)th RO (if exist) are also cancelled.

Proposal 7: RAN1 considers the following alternatives to ensure the time span of the RO group to be kept for a reasonable time duration:

Alt. 1: leave to gNB configuration;

Alt.2: the time span of one RO group is required not to exceed a certain time duration threshold

Proposal 8: RAN1 considers the following alternatives to differentiate multiple N values:

Alt. 1: separate preamble with shared RO;

Alt.2: separate RO;

Alt. 3: FDMed RO (in same PRACH configuration)

Proposal 9: Different frequency location of the ROs for one multiple PRACH transmission may or may not be considered.

Proposal 10: for separate RO configuration, support reusing Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17).

Proposal 11: The UE Tx beam for multiple PRACH transmissions is up to UE implementation.

Proposal 12: For multiple PRACH transmissions with same Tx beam, the Tx beam determined for the first PRACH transmission of the multiple PRACH in a RACH attempt should apply to all PRACH transmissions in the RACH attempt.

Proposal 13: The multiple PRACH transmission is kept as one RACH procedure.

Proposal 14: FFS whether allow to switch to higher number of multiple PRACH transmission if applicable.

Proposal 15: Further study multiple PRACH transmissions triggered by PDCCH order.

Proposal 16: The calculated transmission power for each of the PRACH in one attempt should be the same.

Proposal 17: The same measurement of the same reference signal to calculate the pathloss should be applied for each of the Multiple PRACH transmissions in one attempt.

Proposal 18: The multiple PRACH transmission use the same set of P0 and alpha configuration from single PRACH transmission.

Proposal 19: FFS power ramping behaviour if the case with different Tx beams is supported.

Observation 4: it’s beneficial to separate UE from selecting same RO but for different N value by RA-RNTI.

Proposal 20: for RA-RNTI calculation,  the s_id is the index of the second/last OFDM symbol of the xth (x=N/2) PRACH occasion within a RO group (0 ≤ s_id < 14) for multiple PRACH transmission with number N (N=2,4,8).

Proposal 21: to constraint the total number of multiple PRACH transmission, considering following to alternatives:

At.1: reuse preamble transmission counter by adding on the multiple transmission number for each attempt;

Alt.2: introduce the PRACH attempt counter and adding one for each attempt.

Proposal 22: Further study multiple PRACH transmission enhancements when UE experiences MPE issues, e.g., impact of MPE on: number of multiple PRACH transmission, power settings, the trigger for multiple PRACH transmission, and SSB selection for PRACH association.

# References

- RP-221858, Revised WID on Further NR coverage enhancements, RAN#96, June, 2022.

- Chairman Note, RAN WG1 NR Ad-Hoc#2, Qingdao, China, June, 2017