- NR_MIMO_evo_DL_UL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI framework extension for MTRP operation | - Support TCI state activation and update for two indicated joint/DL/UL TCI states - Support TCI selelction from two indicated joint/DL/UL TCI states for applying to channels/signals - Supprot UL Tx power detetminaiton for PUSCH/PUCCH/SRS Tx occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the applied indicated joint/UL TCI state |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-1-1a | TCI selection for aperiodic CSI-RS in S-DCI based MTRP | - Support “per resource” or “per resource set” configuration to inform that the UE shall apply the first or the second indicated joint/DL TCI state to an aperiodic CSI-RS resource or an aperiodic CSI-RS resource set |  |  |  |  |  |  |  |  | Component candidate values: {per resource, per resource set, both} | Optional with capability signalling |
|  | 40-1-2 | CJT Tx scheme for PDSCH | - Support PDSCH-CJT with two indicated joint TCI states |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-1-3 | TRP-specific BFR with unified TCI framework | - Support TRP-specific BFR with unified TCI framework for M-DCI based MTRP operation |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | 1. Support for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement 2. Support of Rx timing difference between two DL reference timings being no larger than CP length 3. Support for the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index to be corresponding to one TAG | 16-2a |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | 1. Support for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement 2. Support of Rx timing difference between two DL reference timings being no larger than CP length 3. Support for the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index to be corresponding to one TAG | 23-4 |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 40-2-3 | Rx timing difference larger than CP length | 1. Support of Rx timing difference between the two DL reference timings to be larger than CP length | 16-2a or 23-4 |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | [40-2-4] | [TCI states of one CORESET Pool associated to both TAGs] | [1. Support for the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index to be corresponding to both TAGS] | [16-2a or 23-4] |  |  |  |  |  |  |  |  | [16-2a or 23-4] |
|  | 40-3-1-1 | Basic features of CSI-mTRPCJT | Comp1: Mode2 Comp2: Mode1 with O3=1 Comp3: - R16: paraCombs for {Ln} with L=2,4 for NTRP=1 - R17: paraCombs for {M,beta} with M=1 Comp4: R=1 |  |  |  |  |  |  |  |  |  | Mandatory |
|  | 40-3-1-2 | Optional features of CSI-mTRPCJT | Comp1: CSI-RS resource selection (TRP selection) Comp2: 2N1N2NTRP>32 Comp3: - 6th combination of ParaCombs for (pv,beta), i.e., ({pv},beta)=({1/2,1/2,1/2,1/2},1/2) - R16: paraCombs for {Ln} with L=6 for NTRP=1 - R17: paraCombs for {M,beta} with M=2 Comp4: Mode 1 with O3=4 Comp5: R=2 |  |  |  |  |  |  |  |  |  | Optional |
|  | 40-3-2-1 | Basic features of CSI-Doppler | Comp1: X=1, one slot (l) Comp2: - R16: paraCombs with L=2,4 - R17: paraCombs with M=1 Comp3: R=1 |  |  |  |  |  |  |  |  |  | Mandatory |
|  | 40-3-2-2 | Optional features of CSI-Doppler | Comp1: l = (n – nCSI,ref ) Comp2: value(s) of N4 (TBD) Comp3: {X=1, two slots, l and l+WCSI-1}, X=2 Comp4: K=12 Comp5: - R16: paraCombs with L=6 - R17: paraCombs with M=2 Comp6: R=2 |  |  |  |  |  |  |  |  | For a UE that supports UE-side prediction, the support of l = (n – nCSI,ref ) is UE optional. | Optional |
|  | 40-3-3-1 | Basic features of CSI-TDCP | Comp1: Y=1 and delay <= Dbasis, amplitude only Comp2: D=4 symbols, 1,2,3,4,5 slots |  |  |  |  |  |  |  |  |  | Mandatory |
|  | 40-3-3-2 | Optional features of CSI-TDCP | Comp1: Y=1 and delay > Dbasis; Y>=1, amplitude and phase Comp2: D=6,10 slots |  |  |  |  |  |  |  |  |  | Optional |
|  | 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH | 1. Supported DMRS type: eType1 2. Supported value of maxLength: 1 3. Support of DL PTRS RE mapping for Rel.18 enhanced DMRS ports, if PTRS is suppored |  | Yes |  | Rel.18 enhanced DMRS ports for PDSCH is not supported | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-1a | Additional support of eType2 DMRS for PDSCH | 1. Additional support of eType2 DMRS for PDSCH | 40-4-1 | Yes |  | eType2 DMRS for PDSCH is not supported | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-1b | Additional support of maxLength = 2 for enhanced DMRS for PDSCH | 1. Additional support of maxLength = 2 for enhanced DMRS for PDSCH | 40-4-1 | Yes |  | Rel.18 DMRS ports with maxLength = 2 for PDSCH is not supported. | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-1-1 | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports |  | 40-4-1 | Yes |  | UE can receive PDSCH with eType1 DMRS ports only if the scheduling restriction is satisfied | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-1-2 | Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP | 1. Supported DMRS type(s) 2. Supported value of maxLength(s) | 40-4-1 | Yes |  | Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP is not supported | Per band | No | No |  | Component 1 candidate values: {eType1 only, both eType1 and eType2} Component 2 candidate values: {1 only, both 1 and 2} | Optional with capability signalling |
|  | 40-4-2 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 1. Supported DMRS type: eType1 2. Supported value of maxLength: 1 3. Support of UL PTRS RE mapping for Rel.18 enhanced DMRS ports, if PTRS is suppored |  | Yes |  | Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-2a | Additional support of eType2 DMRS for PUSCH | 1. Additional support of eType2 DMRS for PUSCH | 40-4-2 | Yes |  | eType2 DMRS for PUSCH is not supported | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-2b | Additional support of maxLength = 2 for enhanced DMRS for PUSCH | 1. Additional support of maxLength = 2 for enhanced DMRS for PUSCH | 40-4-2 | Yes |  | Rel.18 DMRS ports with maxLength = 2 for PUSCH is not supported. | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-4 | UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 1. Support of UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-2 | Yes |  | UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | Per band | No | No |  |  | Optional with capability signalling |
|  | 40-4-10 | Rel.15 DMRS ports for PUSCH with rank 5-8 | 1. Supported DMRS type(s) 2. Supported value of maxLength(s) | 40-7-1-1 | Yes |  | Rel.15 DMRS ports for PUSCH with rank 5-8 is not supported | Per band | No | No |  | Component 1 candidate values: {Type1, Type2, both} Component 2 candidate values: {1, 2, both} | Optional with capability signalling Note: for UE supporting FG 40-7-1-1 shall report at least one of FG40-4-10 or FG40-4-11 |
|  | 40-4-11 | Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 1. Supported DMRS type(s) 2. Supported value of maxLength(s) | 40-4-2, 40-7-1-1 | Yes |  | Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | Per band | No | No |  | Component 1 candidate values: {eType1 only, both eType1 and eType2} Component 2 candidate values: {1 only, both 1 and 2} | Optional with capability signalling Note: for UE supporting FG 40-7-1-1 shall report at least one of FG40-4-10 or FG40-4-11 |
|  | 40-4-12 | UL PTRS for PUSCH with rank 5-8 | 1. Supported PTRS port(s) 2. Support of UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-7-1-1 | Yes |  | UL PTRS for rank 5-8 PUSCH with rank 5-8 is not supported | Per band | No | No |  | Component 1 candidate values: {1, 2, both} | Optional with capability signalling |
|  | 40-5a | SRS comb offset hopping/cyclic shift hopping | 1. Support SRS comb offset hopping 2. Support SRS cyclic shift hopping | 2-52, 2-53 | Yes |  | gNB cannot configure SRS comb offset hopping/cyclic shift hopping | Per UE | No | No |  |  | Optional with capability signalling |
|  | 40-5a-0 | Comb offset hopping time-domain behavior when reprtiton factor R>1 | 1. Hopping depends on the OFDM symbol index l' of each symbol 2. Hopping depends on the OFDM symbol index l' of the first symbol across the R repetitions | 40-5a | Yes |  | gNB cannot configure SRS comb offset hopping when reprtiton factor R>1 | Per UE | No | No |  |  | Optional with capability signalling |
|  | 40-5a-1 | Comb offset hopping combined with one of group / sequence hopping on a SRS resource | 1. Comb offset hopping combined with group hopping on a SRS resource 2. Comb offset hopping combined with sequence hopping on a SRS resource | 40-5a | Yes |  | gNB cannot configure SRS comb offset hopping combined with one of group / sequence hopping on a SRS resource | Per UE | No | No |  |  | Optional with capability signalling |
|  | 40-5b | SRS with 8 Tx ports | 1. Support transmission of SRS for 8T8R based antenna switching and SRS for CB/NCB with up to 8 layers | 2-52, 2-53 | Yes |  | gNB cannot configure 8 Tx ports SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, or a SRS resource set with more than 4 and up to 8 1-port SRS resources and with usage ‘nonCodebook’ | Per Band | No | No |  |  | Optional with capability signalling |
|  | 40-5b-0 | SRS with 8 Tx ports and comb 8 | 1. Support transmission of SRS with comb 8 for 8T8R based antenna switching and SRS for CB/NCB with up to 8 layers | 40-5b, 23-8-8 | Yes |  | gNB cannot configure comb 8 for SRS with 8 Tx ports | Per Band | No | No |  |  | Optional with capability signalling |
|  | 40-5b-1 | SRS with TDMed 8 Tx ports | 1. Support transmission of SRS for 8T8R based antenna switching and SRS for CB with up to 8 layers, and the 8 ports are split and mapped to more than 1 OFDM symbol with TDM | 40-5b | Yes |  | gNB cannot configure TDM for SRS with 8 Tx ports | Per Band | No | No |  |  | Optional with capability signalling |
|  | 40-6-1-1 | Basic features of STx2P SDM scheme for PUSCH | 1.Support STx2P SDM scheme for PUSCH transmission 2.Maximal number of MIMO layers per UE panel in SDM scheme, the candidate values are {1, 2} |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-6-1-2 | Optional feature for STx2P SDM scheme | 1.Support the new entry {0, 2, 3} in DMRS table for the layer combination {1+2} of SDM scheme |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-6-2-1 | Basic features of STx2P SFN scheme for PUSCH | 1.Support STx2P SFN scheme for PUSCH transmisson 2.Maximal number of layers of PUSCH in SFN scheme and the candidate values are {1, 2} |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-6-3-1 | Basic feature of STx2P PUSCH+PUSCH in multi-DCI based mTRP system | 1. Support two indepent PUSCHs with fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain in a same active BWP 2. Maximal number of layers in each PUSCH of STx2P PUSCH+PUSCH overlapping in time domain and the candidate values are {1,2} |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-6-4-1 | Basic feature of STx2P SFN scheme for PUCCH | 1.Support STx2P SFN scheme for PUCCH transmission |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-6-5-1 | Support grouped-based beam reporting for STx2P | 1.Support group-based beam reporting for STx2P. 2. The maximum number of reported CRI/SSBRI pairs 3. The maximum number of CSI-RS and SSB configured for group-based beam reporting for STx2P. |  |  |  |  |  |  |  |  |  |  |
|  | 40-7-1-1 | Basic features of UL-8Tx | 1. Support UL-8Tx based PUSCH transmission |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 40-7-1-2 | MaxMIMO layers for UL-8Tx | 1. Maximal number of MIMO layers for UL-8Tx based PUSCH transmission | 40-7-1-1 | Yes |  |  |  |  |  |  | Component candidate values: {4, 8} | Optional with capability signalling |
|  | 40-7-2-1 | Number of antenna groups (Ng) | 1. Supported number of antenna groups for UL-8Tx based PUSCH transmission | 40-7-1-1 | Yes |  |  |  |  |  |  | Component candidate values: {1,2,4} | Optional with capability signalling |
|  | 40-7-3-1 | Full-coherent codebook parameters | 1. Supported combination of a pair of parameters (N1, N2) | 40-7-1-1 | Yes |  |  |  |  |  |  | Component candidate values: {(4,1),(2,2)} | Optional with capability signalling |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-z |  |  |  |  |  |  |  |  |  |  |  |  |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. Netw_Energy_NR | 42-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 42. Netw_Energy_NR | 42-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 42. Netw_Energy_NR | 42-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_netcon_repeater

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 43. NR_netcon_repeater | 43-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 43. NR_netcon_repeater | 43-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. NR_NTN_enh | 44-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 44. NR_NTN_enh | 44-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 44. NR_NTN_enh | 44-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. NR_Mob_enh2 | 45-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 45. NR_Mob_enh2 | 45-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 45. NR_Mob_enh2 | 45-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_UAV

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_UAV | 46-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 46. NR_UAV | 46-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 46. NR_UAV | 46-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 47. NR_SL_enh2 | 47-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 47. NR_SL_enh2 | 47-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_redcap_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_redcap_enh | 48-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 48. NR_redcap_enh | 48-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 48. NR_redcap_enh | 48-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 49. NR_MC_enh | 49-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 49. NR_MC_enh | 49-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_XR_Enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 50. NR_XR_Enh | 50-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 50. NR_XR_Enh | 50-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_FR1_lessthan_5MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 51. NR_FR1_lessthan_5MHz_BW | 51-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 51. NR_FR1_lessthan_5MHz_BW | 51-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_DSS_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_DSS_enh | 52-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 52. NR_DSS_enh | 52-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 52. NR_DSS_enh | 52-z |  |  |  |  |  |  |  |  |  |  |  |  |

- BWP_wor

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. BWP_wor | 53-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 53. BWP_wor | 53-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 53. BWP_wor | 53-z |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 54. NR_cov_enh2 | 54-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 54. NR_cov_enh2 | 54-z |  |  |  |  |  |  |  |  |  |  |  |  |

- TEI18

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 55. TEI18 | 55-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 55. TEI18 | 55-z |  |  |  |  |  |  |  |  |  |  |  |  |