3GPP TSG RAN WG1 #113	                                              R1-2305566

Incheon, Korea, May 22nd – May 26th, 2023

Source:	ZTE

Title:	Discussion on LS on RACH-less Handover

Agenda Item:	5

Document for: Discussion

Introduction

              - In last meeting, RAN2 has sent the LS to RAN1 [1] with following content:

RAN2 has also identified some issues which are more relevant to RAN1 and would like to check RAN1 views on the following aspects for NTN RACH-less HO. 1. Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission? 2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)? 3. Regarding the power control for initial UL transmission, whether it follows the rules specified for PUSCH scheduled by Random Access grant or by configured grant or others?

              - With required action in RAN1 to provide responses to above questions. In this contribution, detailed views are shared.

Discussion on RACH-less Handover

Question 1: Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission?

              - In the TN system, normally, each SSB may be associated to the different beams (e.g., digital or analog) and the received signaling quality can be different due to complicated propagation channel condition, e.g., blockage of signal for certain direction. However, in NTN case, when one or multiple SSBs are associated with one beam, the UE is not expected to detect multiple SSBs with different signal quality during HO. For example, in case that single SSB is mapped to one beam, since the beam coverage is much larger than TN and different frequency reuse pattern is considered among beams, the detectable SSB at certain location will represent the proper beam for following communication. If multiple SSBs are mapped to one beam, since same satellite set-up and propagation is shared, the channel quality will be always same. Then, it’s clear that UE does not need to perform SSB selection in RACH-less HO.

              - Regarding whether the resource will be mapped to SSBs, the necessity seems also limited.

              - Proposal 1: For NTN RACH-less HO, both SSB-selection and association of resource with SSB(s) are not needed.

              - Question 2: To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

              - As commented in the response for Question 1, UE is not expected to detect multiple beams in NTN due to large beam coverage. Therefore, there is no need of beam selection to monitor target cell PDCCH for dynamic grant for initial UL transmission.

              - Proposal 2: Beam selection is not needed to monitor target cell PDCCH for dynamic grant for initial UL transmission.

              - Question 3: Regarding the power control for initial UL transmission, whether it follows the rules specified for PUSCH scheduled by Random Access grant or by configured grant or others?

              - In legacy HO with RACH, the power control for PUSCH scheduled by Random Access grant will be followed for initial UL transmission. In RACH-less HO, the main difference is that the RACH procedure is not needed to achieve UL synchronization due to UE pre-compensation. While for power control, similar rule as in legacy HO should be followed. Hence, in our view, the power control rules for PUSCH scheduled by Random Access grant should be followed for initial UL transmission in RACH-less handover.

              - Proposal 3: Regarding power control for initial UL transmission in RACH-less HO, rules specified for PUSCH scheduled by Random Access grant should be followed.

Conclusions

In this contribution, the RAN2 questions are analyzed with following proposals and observations:

              - Proposal 1: For NTN RACH-less HO, both SSB-selection and association of resource with SSB(s) are not needed.

              - Proposal 2: Beam selection is not needed to monitor target cell PDCCH for dynamic grant for initial UL transmission.

              - Proposal 3: Regarding power control for initial UL transmission in RACH-less HO, rules specified for PUSCH scheduled by Random Access grant should be followed.

- Reference

- R1-2304322 LS on RACH-less Handover, RAN1#113