3GPP TSG RAN WG1 #113			R1-2305591

Incheon, Korea, May 22nd – May 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of the evaluation on use cases as following.

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, the AI/ML-based CSI feedback enhancement on overhead reduction and accuracy improvement are discussed.

- Evaluation on AI/ML for CSI feedback enhancements

  - Sub use-case description

Fig. 1 illustrates the framework of spatial-frequency domain CSI compression using two-sided AI models. As shown in Fig. 1, UE is equipped with an AI/ML encoder to compress CSI into encoded bits, while the corresponding AI/ML decoder is deployed on gNB to reconstruct CSI from encoded bits. In CSI compression with two-sided models, UE calculates downlink CSI, such as channel matrix or precoding matrix, and feeds the CSI into the encoder for compression. After the AI/ML encoder extracts essential features and outputs the encoded bits, UE reports the encoded bits to gNB where CSI can be reconstructed from encoded bits with the AI/ML decoder.

Figure 1. The framework of auto-encoders of CSI feedback.

With this AI/ML-based CSI compression, accuracy improvements under a certain overhead of CSI reports and/or overhead reduction for CSI report achieving a certain performance can be expected. In the subsequent sections, we discuss the evaluation methodology and simulation results of this sub-use-case

  - Evaluation methodology

2.2.1	KPI for intermediate performance

At the RAN1#112 meeting, the following working assumption was confirmed as follows [2],

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

where the SGCS was defined at the RAN1#110bis-e meeting as follows [3],

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2. - Method 1: Average over all layers - Method 2: Weighted average over all layers SGCS=E1Ni=1NjKλijk=1Kλik¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij. - Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Since Method 3 could show the performance for each layer independently for the rank > 1 case, we use Method 3 to show the intermediate KPI SGCS for the multiple layers.

2.2.2	Simulation assumptions

The detailed simulation assumptions for packet throughput performance evaluations are given in Table 1.

Table 1. Simulation parameters

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD/TDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | 4GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 44dBm for 20MHz |
| BS antenna height | 25m |
| UE antenna height & gain | According to TR36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size = 8448 bits |
| Numerology | 30KHz, 14 OFDM symbol slot |
| Simulation bandwidth/granularity | 20MHz (48RB)/12 subbands (4 RBs per subband) |
| Frame structure | Slot format 0 for all slots |
| MIMO scheme | SU/MU with rank adaptation Maximum 8 MU layers |
| CSI feedback | CSI feedback periodicity: 5 ms, Scheduling delay: 4 ms |
| Overhead | 2-symbol |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| RU | 20%/50%/70% |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC |
| Feedback assumption | Realistic |
| Channel estimation | Ideal channel estimation for dataset construction Ideal channel estimation for CSI acquisition Realistic channel estimation for demodulation |
| KPI | 5% UPT, Average UPT |
| Baseline | Rel-16 Type II codebook |
| (De-)quantization method | 2-bit uniform before/after decoder/encoder |
| Input for AI/ML model | Precoding matrix(eigenvector) |
| Output for AI/ML model | Precoding matrix(eigenvector) |

  - Evaluation cases & results

2.3.1	Extendibility performance of Type 3 and Type 4 training procedures

At the RAN1#111 meeting, the following agreements were made regarding the evaluation of Type 3 training [4]:

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different - FFS: different quantization methods between NW side and UE side - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - FFS: whether/how to report overhead of dataset NW-first training; UE-first training.

At the RAN1#112 meeting, the following training steps were discussed as a hybrid case of Type 2 and Type 3 training [5], which can be named as Type 4 training.

Step A: first side is trained, and after the training for the first side is finished Step B: the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, while the first side is frozen

In this contribution, we assume that separate training is sequential training because only sequential training has been observed as a feasible separate training procedure with simulation results as far as we know. In sequential training, the model on one side is trained after the model training on the other side.

Since it is not preferable to perform joint training with all device types every time a new device comes into the market, training a new model compatible with the existing model is necessary. Thus, the extensibility performance is important in the practical scenario. In this contribution, we evaluate the performance of extensibility for both Type 3 and Type 4 training schemes.

Type 3 training

Type 3 NW-first training process can be performed as following for the extendibility scenario (refer to Figure 2):

1) A pair of encoder and decoder is firstly trained on NW-side using collected dataset from UE1;

2) Collected dataset from UE2 are fed into the encoder on NW-side to obtain the encoded output, then the input and the output are combined together and are sent back to UE2;

3) UE2 uses this delivered dataset to train the local encoder at UE2.

Figure 2. Type 3 NW-first training process

Type 4 training

This training procedures allows the FPs and BPs exchanging between two sides for the training, meanwhile the models at two sides can be trained sequentially. In this contribution, we evaluated the extendibility of an implementation of Type 4 training as shown in Figure 3. The detailed procedures of our implementation of Type 4 training is,

1) A pair of encoder and decoder is firstly trained on NW-side using collected dataset from UE1;

2) An encoder on UE2 is jointly trained with the frozen decoder on NW-side using collected data from UE2.

Figure 3. Type 4 NW-first training process

Extendibility with Environment Change

Table 2 and Table 3 compares the performance of Type 3 and Type 4 training when the payload is 88 bits and 608 bits respectively. Four options are compared in our evaluation:

- Benchmark 1: A pair of encoder and decoder is jointly trained on UMa 4GHz dataset.

- Benchmark 2: A pair of encoder and decoder is jointly trained on UMi 4GHz dataset.

- Type 3: Following the Type 3 training paradigm, NW-side firstly uses UMa 4GHz dataset to train an encoder and decoder pair, then UE2 uses UMi 4GHz dataset to separately train an encoder.

- Type 4: Following the Type 4 training paradigm, NW-side firstly uses UMa 4GHz dataset to train an encoder and decoder pair, then UE2 uses UMi 4GHz dataset to jointly train an encoder with the decoder on NW-side.

Table 2. Performance (SGCS) of Type 3 and Type 4 when payload is 88 bits.

| Training Method | Evalset |  |
| --- | --- | --- |
|  | UMa 4GHz | UMi 4GHz |
| Benchmark 1 | 0.52 | 0.51 |
| Benchmark 2 | 0.50 | 0.53 |
| Type 3 | - | 0.49 |
| Type 4 | - | 0.51 |

Table 3. Performance (SGCS) of Type 3 and Type 4 when payload is 608bits.

| Training Method | Evalset |  |
| --- | --- | --- |
|  | UMa 4GHz | UMi 4GHz |
| Benchmark 1 | 0.77 | 0.78 |
| Benchmark 2 | 0.77 | 0.81 |
| Type 3 | - | 0.74 |
| Type 4 | - | 0.78 |

Observation 1: Compared with Benchmark 2, when using Type 3 training strategy for UE2 training, a significant decrease can be observed.

Observation 2: Compared with Benchmark 2, when using Type 4 training strategy for UE2 training, a slight decrease can be observed.

Observation 3: When trainset on UE2 is different from dataset for NW-side training, the performance of Type 4 training is better than that of Type 3 training.

Extendibility with Different Device Types

During the RAN1#112 meeting, the extendibility of models with different UE types was raised. Due to the discrepancy in the data distribution between different UEs, directly applying a model trained on one UE to another UE for inference may result in performance degradation. Type 3 and Type 4 training paradigms provide methods to compensate for this discrepancy.

To study the extendibility of models with different device types, we preliminary evaluate the performance of CSI compression and regeneration models under different datasets that reflect to different UE implementations or UE environments. These differences include UE located in different environments, different UE antenna layouts, and UE antenna imbalance, i.e., there is a gap on the antenna gains of two UE antennas due to the different antenna positions on the UE or the shadowing by other UE components (e.g., PCB).

We use the following 4 datasets in the evaluations,

Table 4. Datasets used to model different UE types

|  | Scenario | UE antenna layout | UE antenna power imbalance |
| --- | --- | --- | --- |
| Dataset 1 | UMa 4GHz | (1,1,2,1,1,1,1) | 0 dB |
| Dataset 2 | UMi 4GHz | (1,1,2,1,1,1,1) | 0 dB |
| Dataset 3 | UMa 4GHz | (1,1,2,1,1,1,1) | 2 dB |
| Dataset 4 | UMa 4GHz | (1,2,1,1,1,1,1) | 0 dB |

Following Table 5 shows the simulation cases,

Table 5. Evaluation cases for different UE types

|  | Dataset for NW-UE1 Training | Dataset for UE2 Test | UE2 Training Method |
| --- | --- | --- | --- |
| Case 1 | Dataset 1/2 | Dataset 1 | No Type 3/4 training (Direct use UE1 model) |
| Case 2 | Dataset 1/2 | Dataset 2 | No Type 3/4 training (Direct use UE1 model) |
| Case 3 | Dataset 1/2 | Dataset 3 | No Type 3/4 training (Direct use UE1 model) |
| Case 4 | Dataset 1/2 | Dataset 4 | No Type 3/4 training (Direct use UE1 model) |
| Case 5 | Dataset 1/2 | Dataset 2 | Type 3 w/ data generated from Dataset 2 |
| Case 6 | Dataset 1/2 | Dataset 2 | Type 4 w/ FP/BP generated from Dataset 2 |

Table 6 shows the simulation results when UE1 and NW is initially trained with different datasets.

Table 6. Performance of six UEs with different evaluation datasets

|  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| SGCS (Initial Training on NW-UE1 w/ Dataset 1) | 0.52 | 0.51 | 0.53 | 0.58 | 0.49 | 0.51 |
| SGCS (Initial Training on NW-UE1 w/ Dataset 2) | 0.50 | 0.53 | 0.52 | 0.56 | 0.49 | 0.51 |

Observation 4: There is no significant performance loss on SGCS when directly applying a well-trained model based on UMa/UMi dataset to the cases that UE is located in different environment and has different antenna layouts or antenna gain imbalance.

Observation 5: For the cases that UE is in different environment, Type 3 or Type 4 training for adaption to the environment has similar performance with the cases that directly apply the model.

Based on these observations we can conclude that,

Observation 6: The CSI compression model is robust against non-ideal factors including different UE antenna layout, imbalance of UE antenna gains, or different channel environments.

2.3.2	Eventual performance with different CQI determination methods

At the RAN1#112bie-e meeting, some CQI determination methods were raised and discussed, and the following proposal was made [6]:

Agreement For the evaluation of CSI compression, companies are allowed to report (by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g., - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW - Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder - Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation - Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment - Option 1c: CQI is calculated based on traditional codebook - Other options if adopted, to be described by companies

In this contribution, we compare the performance of Option 2a-1 and Option 1a. In addition, there are also several alternatives for the RI determination with different CSI assumptions, i.e., RI is determined based on CSI reconstruction output or target CSI. We consider the following combinations of CQI and RI determination methods and evaluated their performances,

Table 7. Simulation Cases on Different CQI/RI Determination Methods

| Case | CQI Determination | RI Determination |
| --- | --- | --- |
| 1 | Option 2a-1 | Based on the same CSI as Option 2a-1 |
| 2 | Option 2a-1 | Based on the same CSI as Option 1a |
| 3 | Option 1a | Based on the same CSI as Option 1a |

Figure 4 shows the procedures of these RI and CQI calculation methods.

Figure 4.  CQI & RI determination method

Note that for Case 2, although the reconstructed CSI is available at UE side, the RI is not necessary to be calculated based on it. It should be verified whether an aggressive RI based on target CSI is helpful to the system-level performance.

The system-level performance of these cases is shown in Table 8. We take Case 1 where both RI and CQI is decided with output CSI as the baseline. Both the absolute UPT and the relative gain to the baseline case are shown in the table for Case 2 and 3. In the table, we also provide the ratio of different Rank values and the ratio of SU-/MU-MIMO after the user scheduling.

Table 8. System-level Performance with Different CQI/RI Determination Methods

Based on the evaluation results, we have the following observations,

Observation 7: Due to the mechanisms such as closed-loop MCS selection based on HARQ-ACK, the CQI/RI determination methods do not significantly impact the average packet throughput (about 5% performance loss or less) when the RU is at a low to medium level, or the CSI payload size is at a medium to high level. Only for high RU and low CSI payload case, the CQI/RI determination methods have significant impacts (>13%) on the network performance.

Observation 8: Given the RI determination method with Option 1a CSI, the CQI determination does not significantly impact the average packet throughput.

2.3.3	Evaluations on performance monitoring

At the RAN1#112bis-e meeting, the following agreements were made regarding the evaluation of performance monitoring [6]:

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, the model monitoring methodology is considered as: - Step1: Generate test dataset including K test samples - FFS how to obtain the K test samples - Step2: For each of K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff= f(KPIActual, KPIGenie), where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI. - Step3: Calculate the statistical result of the KPIDiff over K test samples which represents the monitoring accuracy performance. - Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network. - Note: the complexity, overhead and latency of the monitoring scheme are reported by companies. FFS how to evaluate latency. Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, for Step2 of the model monitoring methodology, the per sample KPIDiff is considered for - Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements. - KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32. - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample. - Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE: - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI. - Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100% - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual) - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI. - FFS how to train the proxy model and the resulting monitoring performance, to be reported by companies. - FFS whether/how to evaluate the generalization performance of the proxy model. - Case 3: others are not precluded Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of - Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie; - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap. - Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_2,KPIGenie>KPIth_3, where KPIth_2 can be same or different from KPIth_3. - Monitoring accuracy is the percentage of the samples for which KPIDiff=0. - FFS other metrics: Misdetection, False alarm, etc. - FFS the values of KPIth_1, KPIth_2, KPIth_3. - FFS whether/how to evaluate the monitoring metrics for Rank>1

In this contribution, we study the performance of NW side monitoring based on intermediate KPI (SGCS). The diagram of the procedures to calculate KPI difference is shown in Figure 5. The KPI genie is calculated based on the ground-truth CSI in FLOAT32 format. For KPI actual is calculated based on ground-truth CSI feedback with scalar quantization schemes in format from FLOAT16 to FLOAT4.

Figure 5.  Diagram of NW side performance monitoring

On KPI_Diff calculation, our evaluations adopt the Option 1, where the gap between KPI_Actual and KPI_Genie is used.

As to the Option 2, we listed all possible cases following the agreements from RAN1 #112bis-e in the Figure 6.

Figure 6.  KPI_Diff value based on agreements from RAN1 #112bis-e when KPI_th,2 is not equal to KPI_th,3

From Figure 6, we found that the KPI_Diff defined in Option 2 fails to output correct values to reflect the performance of monitoring.  We believe there is a typo in the definition where the KPI_Diff should be defined as KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_3,KPIGenie>KPIth_2. With such correction, the value of KPI_Diff is shown in following Figure 7, which are more reasonable.

Figure 7.  KPI_Diff value after correcting the typo in agreements.

Proposal 1: Correct the typo in the agreement related to KPI_Diff calculation of RAN1 #112bis-e as following,

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of - Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie; - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap. - Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_3,KPIGenie>KPIth_2, where KPIth_2 can be same or different from KPIth_3. - Monitoring accuracy is the percentage of the samples for which KPIDiff=0. - FFS other metrics: Misdetection, False alarm, etc. - FFS the values of KPIth_1, KPIth_2, KPIth_3. - FFS whether/how to evaluate the monitoring metrics for Rank>1

After the correction, we have following observations on Option 2 of the KPI_Diff calculations.

Observation 9: When either KPI_Actual or KPI_Genie falls into the rage [KPI_th,3, KPI_th,2], the KPI_Diff is always 0 no matter how large the gap between KPI_Actual and KPI_Genie is.

Based on this observation, we do not think Option 2 is good performance metric to evaluate the monitoring and have following proposal,

Proposal 2: Use Option 1 as mandatory metric for the evaluation of intermediate KPI based monitoring and FFS other metrics.

Following we show the evaluation results adapting KPI_Diff calculation Option 1. In our evaluations, we use SGCS as the intermediate KPI and the number of test samples (K) is set to 1 and 10, respectively.

Figure 8. CDF of SGCS Diff in absolute value when K=1

Figure 9. CDF of SGCS Diff in percentage value when K=1

Figure 8 and Figure 9 show the CDF of SGCS Diff in absolute value and percentage value respectively when K=1, which means the SGCS Diff is calculated based on only one sample.

Observations 10: When KPI_Diff (SGCS Diff) is calculated based on only one test sample (K = 1),

- It is not necessary to use FLOAT32 format for ground-truth CSI feedback to obtain accurate performance monitoring results.

- When scalar quantization is considered for ground-truth CSI feedback, the bit width for each scalar can be reduced to 4 bits where 95% of the KPI_Actual has a gap less than 5% to the KPI_Genie.

- Since the CSI regeneration model is trained with the datasets in FLOAT32 format, the KPI_Actual is usually smaller than KPI_Genie, which introduces a systematic bias as shown in the CDF figures. After compensating this systematic bias, the performance of monitoring can be further improved.

Figure 10. CDF of SGCS Diff in absolute value when K=10

Figure 10 shows the CDF of SGCS Diff in absolute value and percentage value respectively when K=10, which means the SGCS Diff is calculated based on an averaged value from 10 samples.

Observation 11: When KPI_Diff (SGCS Diff) is calculated based on 10 test sample (K = 10),

- The performance of monitoring can be acceptable with the ground-truth CSI feedback in 4 bit scalar quantization. Especially, after compensating the bias, the variance of KPI_Diff is ignorable.

  - Observations on submitted evaluation results

In this section, we briefly summarize the observations from our submitted results to the result collection sheets.

For the results submitted to Table 1, we have the following observations

Observation 12:

- For Rank 2, AI/ML method outperforms the Rel-16 eType II in term of SGCS. With the low and high feedback overhead, AI-based CSI feedback can obtain around 11% SGCS gains for Layer 1 and 10% SGCS gains for Layer 2; with the medium feedback overhead, AI-based CSI feedback can obtain up to 1.6% SGCS gains for Layer 1 and 1.9% SGCS gains for Layer 2.

- For Rank 1, AI/ML method also outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI feedback can obtain 13.5% SGCS gains; with the medium feedback overhead, AI-based CSI feedback can obtain 7.9% SGCS gains; with the high feedback overhead, AI-based CSI feedback can obtain 9.3% SGCS gains.

For the results submitted to Table 2, we have the following observations

Observation 13:

- Generalization can be achieved by training model with dataset with mixed data from various deployment scenario (UMa and UMi). Same method can also be used for scenarios with different UE distributions or different carrier frequencies.

For the results submitted to Table 3, we have the following observations

Observation 14:

- Scalability on bandwidth and Tx ports

  - Training with mixed bandwidth configurations achieve good generalization performance across all possible bandwidth configurations. This similar trend can also be observed in Tx ports generalization.

- Scalability on CSI payloads

  - No significant performance degradation can be observed by using unified scalable model compared to payload-specific models.

- Scalability on Rank

  - Compared with Option3-1, Option3-2 achieves better performance in Layer 1 and slightly degrades in Layer 2.

- Conclusion

In this contribution, we discussed evaluation on AI/ML for CSI feedback enhancement. Based on the discussion we made the following observations and proposal.

Observation 1: Compared with Benchmark 2, when using Type 3 training strategy for UE2 training, a significant decrease can be observed.

Observation 2: Compared with Benchmark 2, when using Type 4 training strategy for UE2 training, a slight decrease can be observed.

Observation 3: When trainset on UE2 is different from dataset for NW-side training, the performance of Type 4 training is better than that of Type 3 training.

Observation 4: There is no significant performance loss on SGCS when directly applying a well-trained model based on UMa/UMi dataset to the cases that UE is located in different environment and has different antenna layouts or antenna gain imbalance.

Observation 5: For the cases that UE is in different environment, Type 3 or Type 4 training for adaption to the environment has similar performance with the cases that directly apply the model.

Observation 6: The CSI compression model is robust against non-ideal factors including different UE antenna layout, imbalance of UE antenna gains, or different channel environments.

Observation 7: Due to the mechanisms such as closed-loop MCS selection based on HARQ-ACK, the CQI/RI determination methods do not significantly impact the average packet throughput (about 5% performance loss or less) when the RU is at a low to medium level, or the CSI payload size is at a medium to high level. Only for high RU and low CSI payload case, the CQI/RI determination methods have significant impacts (>13%) on the network performance.

Observation 8: Given the RI determination method with Option 1a CSI, the CQI determination does not significantly impact the average packet throughput.

Observation 9: When either KPI_Actual or KPI_Genie falls into the rage [KPI_th,3, KPI_th,2], the KPI_Diff is always 0 no matter how large the gap between KPI_Actual and KPI_Genie is.

Observation 10: When KPI_Diff (SGCS Diff) is calculated based on only one test sample (K = 1),

- It is not necessary to use FLOAT32 format for ground-truth CSI feedback to obtain accurate performance monitoring results.

- When scalar quantization is considered for ground-truth CSI feedback, the bit width for each scalar can be reduced to 4 bits where 95% of the KPI_Actual has a gap less than 5% to the KPI_Genie.

- Since the CSI regeneration model is trained with the datasets in FLOAT32 format, the KPI_Actual is usually smaller than KPI_Genie, which introduces a systematic bias as shown in the CDF figures. After compensating this systematic bias, the performance of monitoring can be further improved.

Observation 11: When KPI_Diff (SGCS Diff) is calculated based on 10 test sample (K = 10),

- The performance of monitoring can be acceptable with the ground-truth CSI feedback in 4 bit scalar quantization. Especially, after compensating the bias, the variance of KPI_Diff is ignorable.

Observation 12:

- For Rank 2, AI/ML method outperforms the Rel-16 eType II in term of SGCS. With the low and high feedback overhead, AI-based CSI feedback can obtain around 11% SGCS gains for Layer 1 and 10% SGCS gains for Layer 2; with the medium feedback overhead, AI-based CSI feedback can obtain up to 1.6% SGCS gains for Layer 1 and 1.9% SGCS gains for Layer 2.

- For Rank 1, AI/ML method also outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI feedback can obtain 13.5% SGCS gains; with the medium feedback overhead, AI-based CSI feedback can obtain 7.9% SGCS gains; with the high feedback overhead, AI-based CSI feedback can obtain 9.3% SGCS gains.

Observation 13:

- Generalization can be achieved by training model with dataset with mixed data from various deployment scenario (UMa and UMi). Same method can also be used for scenarios with different UE distributions or different carrier frequencies.

Observation 14:

- Scalability on bandwidth and Tx ports

  - Training with mixed bandwidth configurations achieve good generalization performance across all possible bandwidth configurations. This similar trend can also be observed in Tx ports generalization.

- Scalability on CSI payloads

  - No significant performance degradation can be observed by using unified scalable model compared to payload-specific models.

- Scalability on Rank

  - Compared with Option3-1, Option3-2 achieves better performance in Layer 1 and slightly degrades in Layer 2.

Proposal 1: Correct the typo in the agreement related to KPI_Diff calculation of RAN1 #112bis-e as following,

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of - Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie; - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap. - Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_3,KPIGenie>KPIth_2, where KPIth_2 can be same or different from KPIth_3. - Monitoring accuracy is the percentage of the samples for which KPIDiff=0. - FFS other metrics: Misdetection, False alarm, etc. - FFS the values of KPIth_1, KPIth_2, KPIth_3. - FFS whether/how to evaluate the monitoring metrics for Rank>1

Proposal 2: Use Option 1 as mandatory metric for the evaluation of intermediate KPI based monitoring and FFS other metrics.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Draft Report of 3GPP TSG RAN WG1 #112, Mar. 2023.

- Report of 3GPP TSG RAN1#110bis-e, Oct. 2022.

- Report of 3GPP TSG RAN1 #111, Nov. 2022.

- Moderator (Huawei), R1-2301936, Summary#1 for CSI evaluation of [112-R18-AI/ML] , Feb. 2023.

- Draft Report of 3GPP TSG RAN WG1 #112bis-e, Apr. 2023.