3GPP TSG RAN WG1 #113			R1-2305596

Incheon, Korea, May 22nd – May 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for: 	Discussion

- Introduction

At the RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss on the evaluation of NR duplex evolution.

- System level simulation

  - Calibration results

Based on simulation and calibration assumptions for the SLS that were agreed in the RAN1 meetings, the calibration has been performed for Urban Macro (FR1) and Dense Urban Macro (FR 2-1). Coupling loss with both large and small scale fading for gNB-UE, gNB-gNB, and UE-UE are shown in Fig.2-1, and detail assumption for the calibration is summarized in Appendix A.

- FR1 : Urban Macro

- FR2-1 : Dense Urban Macro

Figure 2-1. SLS calibration results

  - Initial evaluation results (FR1 Urban Macro)

In this section, our initial evaluation results for FR1 Urban macro of Deployment Case 1 (Non-coexistence case with single SBFD subband configuration) are introduced, and UL performance is evaluated as an initial step. The key assumptions listed in Table 1 are selected for the evaluation and detail evaluation assumption is summarized in Appendix A.

Table 1. Key assumptions for the evaluation

|  | Parameters |
| --- | --- |
| Co-site inter-sector CLI modelling (Spatial isolation + digital isolation) | 100dB |
| SBFD slot configuration | Alt-2: {DDDSU} vs. {XXXXU} |
| BS transmit power | 53dBm |
| SBFD antenna configuration | Twice area&same TxRUs |
| Packet Size | UL: 0.5Mbyte |

Figure 2-2 show the evaluation results for FR1 Urban macro and they are summarized in Table 2. Based on the evaluation results we observed that in low traffic load with Type-2 RU less than 10%, the gain of SBFD compared with static TDD is about 43%, 34% and 102% in terms of mean, 5% and 50% UE UL packet throughput. In medium traffic load with Type-2 RU about 20%, the gain of SBFD is about 19%, 6% and 31% in terms of mean, 5% and 50% UE UL packet throughput. In high traffic load with Type-2 RU about 50%, the gain of SBFD is about 7%, 7% and 3% in terms of mean, 5% and 50% UE UL packet throughput. The lower traffic load, the higher gain of SBFD is achieved since the self-interference, gNB-gNB co-channel inter-subband CLI and co-site inter-sector co-channel inter-subband CLI in lower traffic scenario are relative smaller than those in high traffic scenario.

Observation 1: In FR1 Urban macro, SBFD with {XXXXU} achieves 43%, 19% and 7% mean UE packet throughput gain compared to static TDD with {DDDSU} in low, medium and high traffic load, respectively.

Figure 2-2. SLS evaluation results for FR1 Urban macro

Table 2. Summary of evaluation results

| Simple description for the sub-case (e.g., 100dB inter-sector isolation, SBFD Alt2, Twice area&same TxRUs, UL: 0.5Mbyte, …) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain /Increase | TDD | SBFD | Gain /Increase | TDD | SBFD | Gain /Increase |
| UL Average-UPT (Mbps) | Mean | 41.12 | 58.98 | 43.44% | 38.92 | 46.51 | 19.48% | 34.96 | 37.30 | 6.70% |
|  | 5% | 1.70 | 2.29 | 34.24% | 1.68 | 1.78 | 6.26% | 1.57 | 1.69 | 7.40% |
|  | 50% | 20.76 | 41.94 | 102.00% | 21.85 | 28.53 | 30.61% | 21.85 | 22.43 | 2.67% |
| UL Packet-Latency CDF (ms) | Mean | 277 | 208 | -24.9% | 294.5 | 251.5 | -14.6% | 300 | 272.5 | -9.2% |
|  | 5% | 18.5 | 12.5 | -32.4% | 18.5 | 15.5 | -16.2% | 21 | 18 | -14.3% |
|  | 50% | 98.5 | 50 | -49.2% | 98.5 | 73.5 | -25.4% | 96 | 93.5 | -2.6% |
| UL RU (%) | Type-1 | 1.16% | 2.00% |  | 5.27% | 10.19% |  | 9.11% | 18.31% |  |
|  | Type-2 | 5.78% | 5.53% |  | 26.33% | 28.21% |  | 45.57% | 50.71% |  |

- Link level simulation for coverage metric

  - Evaluation assumptions for link level simulation

At the RAN1#112bis-e meeting, the assumptions for the evaluation using link level simulation (LLS) were discussed and following agreement was made [2].

Agreement For LLS coverage evaluation, RAN1 should consider self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference in TDD system and SBFD system. Option-1 - The modelling method is as below: - For TDD UL slot, additive white Gaussian noise with variance of NTDD is generated, where NTDD=IUE-gNB+N0 - IUE-gNB is UE-gNB interference and N0 is noise (in linear scale). - For SBFD slot, additive white Gaussian noise with variance of NSBFD is generated, where NSBFD=ISI+Ico-site+IgNB-gNB+IUE-gNB+N0 - ISI, Ico-site, IgNB-gNB, IUE-gNB are self-interference, co-site inter-sector interference, inter-site gNB-gNB co-channel inter-subband CLI and UE-gNB interference (in linear scale), respectively - Companies to report the details of deriving NTDD and NSBFD. Some examples are as below: - Example-1: IUE-gNB and IgNB-gNB are derived based on a certain assumption of the topology of gNBs and UEs (ISI is derived based on 1dB desense and Ico-site is derived based on αco-site as agreed in last meeting). In this example, the interference is pre-receiver interference. - Note: link budget analysis can be applied in this example - Example-2: ∆=10*log10NSBFD-10*log10NTDD is derived based on statistic in SLS, and then ∆ is used in LLS to increase the Gaussian noise power in SBFD symbol compared to TDD UL symbol. In this example, the interference is post-receiver interference. - Example-3: NTDD and NSBFD can be derived based on statistic in SLS. In this example, the interference is post-receiver interference. - Companies to report the RU assumption for the interference. - Note: For simplicity, the interference is independently updated/generated in each slot. - Note: Companies are encouraged to report whether and how channel estimation and interference estimation will be impacted by IUE-gNB and IgNB-gNB. - Based on the modelling method, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 1: For legacy TDD system, assume the SNR in UL only slot is SNRTDD=SNTDD, perform LLS to get the required SNR (SNRTDD) with which UE can achieve a certain bit rate in UL - Step 2: For SBFD system with frame structure XXXXU, assume the SNR in UL only slot is SNRSBFD-UL=SNTDD and the SNR in SBFD slot is SNRSBFD-X=SNSBFD. Perform LLS to get the required SNR (SNRSBFD-UL) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.) - Step 3: Use Link budget template to obtain MPL, MCL and MIL for legacy TDD and SBFD. - For legacy TDD, the required SNR (SNRTDD) obtained in Step 1 is used to calculate MPL, MCL, MIL. - For SBFD, the required SNR (SNRSBFD-UL) obtained in Step 2 is used to calculate MPL, MCL, MIL. Option-2 - The UE-gNB interference and inter-site gNB-gNB co-channel inter-subband CLI in LLS coverage evaluation are explicitly modelled based on a given topology of aggressor UEs and gNBs. The UE-gNB and gNB-gNB fast fading channels are explicitly modelled in LLS. The signal model is as follows - Y=HkSk +n≠kHnSn +HgSg+HCLI,mWmSm+Iself+N - Y is the received signal vector at the victim gNB - Hk is the channel matrix from target UE to gNB, Sk is the transmitted signal of the target user - Hn, Sn, n≠k, are the channel matrix and transmitted signal of the UE in the same cell as the target user - Hg and Sg are the channel matrix and transmitted signal of the UEs in the adjacent cell - HCLI,m, Wm and Sm are the channel matrix, the precoding matrix, and leakage CLI signal from aggressor gNB m to the victim gNB. - The power of the signal and interference is included in the channel marix respectively - Iself and N are the self-interference vector of the co-site sectors and the thermal noise signal vector on the receiving antennas - Companies to report the topology of gNBs and UEs to derive the detailed signals and interferences above. One example is as below - Based on the above modelling, the following high-level evaluation method can be used as an example for coverage performance evaluation: - Step 1: For legacy TDD system, perform LLS to get the required SNR (SNRTDD=SN0) with which UE can achieve a certain bit rate in UL - Step 2: For SBFD system with frame structure XXXXU, perform LLS to get the required SNR (SNRSBFD-UL=SN0) with which UE can achieve a certain bit rate in UL for a given SBFD coverage enhancement scheme (e.g., SBFD with PUSCH repetition type A, etc.) - Step 3: Use Link budget template to obtain MPL, MCL and MIL for legacy TDD and SBFD. - For legacy TDD, the required SNR (SNRTDD) obtained in Step 1 is used to calculate MPL, MCL, MIL. - For SBFD, the required SNR (SNRSBFD-UL) obtained in Step 2 is used to calculate MPL, MCL, MIL.

Regarding the schemes for link level evaluation of PUSCH coverage performance for SBFD, we calculate the power of interference from a link budget analysis with using Example-1 in Option-1. Interference is derived based on the topology illustrated in the Figure 3-1, with considering 4 aggressor gNBs, and 1 aggressor UE.

Figure 3-1. The illustration of the topology in LLS

    - Interference modeling in LLS for FR1

The details of respective interference modeling based on the topology shown in Figure 3-1 are as follows:

- Self-interference is modelled as additive white gaussian noise with fixed INR = - 6 dB

- Co-site inter-sector interference is modelled as additive white gaussian noise with fixed INR = 14.7 dB and the value is calculated as Table 3-1.

Table 3-1. Co-site inter-sector interference on link budget analysis for FR1(Low Load)

| A | BS transmit power over 80MHz per RB [dBm] = BS Tx power (49) + LOG10(80MHz/100MHz) -10*LOG10(NRB), NRB = 208 | 25.7 |
| --- | --- | --- |
| B | frequency isolation [dB] | 42.5 |
| C | spatial isolation [dB] | 75.0 |
| D | digital cancelation [dB] | 0.0 |
| E | resource utilization (RU) [%] | 10.0 |
| F | inter-sector interference level per RB per sector[dB] = A-B-C-D+10*LOG10(E) | -101.7 |
| G | inter-sector interference level per RB (2 sectors) [dB] | -98.7 |
| H | noise power per RB [dBm] = -174+10*LOG10(30kHz*12) + noise figure (5) | -113.4 |
|  | INR of inter-sector CLI [dB] = G-H | 14.7 |

- Inter-site gNB-gNB co-channel inter-subband CLI is modelled as additive white gaussian noise with fixed INR = -3.4 and the value is calculated as Table 3-2.

Table 3-2. Inter-site gNB-gNB co-channel subband CLI on link budget analysis for FR1(Low Load)

| A | BS transmit power per RB [dBm] = BS Tx power (49) + LOG10(80MHz/100MHz) -10*LOG10(NRB), NRB = 208 | 25.7 |
| --- | --- | --- |
| B | inter-gNB coupling loss [dB] using CDF of 95% for gNB-gNB coupling loss from SLS | -96.1 |
| C | frequency isolation [dB] | 42.5 |
| D | resource utilization (RU) [%] | 10.0 |
| E | inter-gNB interference level per RB per aggressor [dB] = A+B-C+10*LOG10(D) | -122.9 |
| F | inter-gNB interference level per RB (4 aggressors) [dB] | -116.8 |
| G | noise power per RB [dBm] = -174+10*LOG10(30kHz*12) + noise figure (5) | -113.4 |
|  | INR of inter-gNB CLI [dB] = F-G | -3.4 |

- UE-gNB interference is modelled as additive white gaussian noise with fixed INR = -19.7 dB and the value is calculated as Table 3-3.

Table 3-3. UE-gNB interference on link budget analysis for FR1(Low Load)

| A | UE transmit power per RB [dBm] = UE Tx power (23) -10*LOG10(NRB), NRB = 55 | 5.6 |
| --- | --- | --- |
| B | UE-gNB coupling loss [dB] using CDF of 25% for UE-gNB coupling loss from SLS | -128.8 |
| C | resource utilization (RU) [%] | 10.0 |
| D | UE-gNB interference level per RB [dB] = A+B+10*LOG10(C) | -133.2 |
| E | noise power per RB [dBm] = -174+10*LOG10(30kHz*12) + noise figure (5) | -113.4 |
|  | INR of UE-gNB interference [dB] = D-E | -19.7 |

    - Interference modeling in LLS for FR2-1

The respective interference modeling details based on the topology are as follows:

- Self-interference is modelled as additive white gaussian noise with fixed INR = - 6 dB

- Co-site inter-sector interference is modelled as additive white gaussian noise with fixed INR = -5.0 dB and the value is calculated as Table 3-4.

Table 3-4. Co-site inter-sector interference on link budget analysis for FR2-1(Low Load)

| A | BS transmit power over 160MHz per RB [dBm] = BS Tx power (30) + LOG10(160MHz/200MHz) -10*LOG10(NRB), NRB = 104 | 9.7 |
| --- | --- | --- |
| B | frequency isolation [dB] | 22.2 |
| C | spatial isolation [dB] | 88.0 |
| D | digital cancelation [dB] | 0.0 |
| E | resource utilization (RU) [%] | 10.0 |
| F | inter-sector interference level per RB per sector [dB] = A-B-C-D+10*LOG10(E) | -110.4 |
| G | inter-sector interference level per RB (2 sectors) [dB] | -107.4 |
| H | noise power per RB [dBm] = -174+10*LOG10(120kHz*12) + noise figure (10) | -102.4 |
|  | INR of inter-sector CLI [dB] = G-H | -5.0 |

- Inter-site gNB-gNB co-channel inter-subband CLI is modelled as additive white gaussian noise with fixed INR = -3.4 dB and the value is calculated as Table 3-5.

Table 3-5. Inter-site gNB-gNB co-channel subband CLI on link budget analysis for FR2-1(Low Load)

| A | BS transmit power over 160MHz per RB [dBm] = BS Tx power (30) + LOG10(160MHz/200MHz) -10*LOG10(NRB), NRB = 104 | 9.7 |
| --- | --- | --- |
| B | inter-gNB coupling loss [dB] using CDF of 95% for gNB-gNB coupling loss from SLS | -128.6 |
| C | frequency isolation [dB] | 22.2 |
| D | resource utilization (RU) [%] | 10.0 |
| E | inter-gNB interference level per RB per aggressor [dB] = A+B-C+10*LOG10(D) | -151.1 |
| F | inter-gNB interference level per RB (4 aggressors) [dB] | -145.1 |
| G | noise power for RB [dBm] = -174+10*LOG10(120kHz*12) + noise figure (10) | -102.4 |
|  | INR of inter-gNB CLI [dB] = F-G | -42.7 |

- UE-gNB interference is modelled as additive white gaussian noise with fixed INR = -19.7 dB and the value is calculated as Table 3-6.

Table 3-6. UE-gNB interference on link budget analysis for FR2-1(Low Load)

| A | UE transmit power per RB [dBm] = UE Tx power (23) -10*LOG10(NRB), NRB = 26 | 8.9 |
| --- | --- | --- |
| B | UE-gNB coupling loss [dB] using CDF of 5% for UE-gNB coupling loss from SLS | -104.8 |
| C | resource utilization (RU) [%] | 10.0 |
| D | UE-gNB interference level per RB [dB] = A+B+10*LOG10(C) | -106.0 |
| E | noise power per RB [dBm] = -174+10*LOG10(120kHz*12) + noise figure (10) | -102.4 |
|  | INR of UE-gNB interference [dB] = D-E | -3.6 |

  - Evaluation results

    - Evaluation results for FR1

The evaluation results for FR1 are shown in Figure 3-2, and the link budget which is derived using required SNR is summarized in Table 3-7.

(a)  Low Load

(b)  Medium Load

Figure 3-2. Evaluation results of LLS for FR1

Table 3-7. Link budget analysis for FR1

| PUSCH-FR1-Urban Macro |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company name | TDD/SBFD | Required SNR | MCL | MIL | MPL | Key assumptions |
| DOCOMO | TDD | -5.25 | 108 | 130.95 | 138.73 | Low Load (RU 10%) |
|  | SBFD | -5.12 | 107.87 | 130.82 | 138.6 |  |
|  | Gain | -0.13 | -0.13 | -0.13 | -0.13 |  |
| DOCOMO | TDD | -5.13 | 107.88 | 130.83 | 138.61 | Medium Load (RU 30%) |
|  | SBFD | -4.99 | 107.74 | 130.69 | 138.47 |  |
|  | Gain | -0.14 | -0.14 | -0.14 | -0.14 |  |

The evaluation results in FR1 did not show any gain by increased UL transmission opportunities with SBFD operation. In this evaluation, the first PUSCH is transmitted in the non-SBFD slot (e.g., UL slot or non-SBFD flexible slot) and the succeeding PUSCH repetitions are transmitted in UL subband in the SBFD slots. Therefore, the results mean that the impact of performance degradation due to interference cancels the gain of PUSCH repetitions using SBFD slots, so that PUSCH repetitions using SBFD slots can’t contribute the improvement of coverage performance.

Observation 2: In the case of FR1, SINR improvement is not found for SBFD operation with PUSCH repetition, since interference due to SBFD is too strong.

    - Evaluation results for FR2-1

The evaluation results for FR2-1 are shown in Figure 3-3, and the link budget which is derived using required SNR is summarized in Table 3-8.

- Low Load

- Medium Load

Figure 3-3. Evaluation results of LLS for FR2-1

Table 3-8. Link budget analysis for FR2-1

| PUSCH-FR2-1-Dense Urban Macro |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company name | TDD/SBFD | Required SNR | MCL | MIL | MPL | Key assumptions |
| DOCOMO | TDD | 3.83 | 122.33 | 102.44 | 136.53 | Low Load (RU 10%) |
|  | SBFD | -3.53 | 129.69 | 109.8 | 143.89 |  |
|  | Gain | 7.36 | 7.36 | 7.36 | 7.36 |  |
| DOCOMO | TDD | 6.07 | 120.09 | 100.2 | 134.29 | Medium Load (RU 30%) |
|  | SBFD | -1 | 127.16 | 107.27 | 141.36 |  |
|  | Gain | 7.07 | 7.07 | 7.07 | 7.07 |  |

In the case of FR2-1, the gNB transmission power is smaller than that in FR1. Accordingly, the impact of inter-sector interference and inter-site CLI are smaller compared with FR1, and hence the effect of interference is reduced. Thus, in the case of FR2-1, although the SBFD slot suffers from interference to some extent, the increased transmission opportunities and HARQ combination with retransmissions resulted in a gain of about 7 dB.

Observation 3: In the case of FR2-1, SINR improvement of about 7 dB is observed for SBFD with PUSCH repetition thanks to smaller inter-sector interference and inter-site CLI in SBFD slot compared with that in FR1.

- Conclusion

In this contribution, we discussed on the evaluation of NR duplex operation. Based on the discussion we made the following observations.

Observation 1: In FR1 Urban macro, SBFD with {XXXXU} achieves 43%, 19% and 7% mean UE packet throughput gain compared to static TDD with {DDDSU} in low, medium and high traffic load, respectively.

Observation 2: In the case of FR1, SINR improvement is not found for SBFD operation with PUSCH repetition, since interference due to SBFD is too strong.

Observation 3: In the case of FR2-1, SINR improvement of about 7 dB is observed for SBFD with PUSCH repetition thanks to smaller inter-sector interference and inter-site CLI in SBFD slot compared with that in FR1.

References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- 3GPP, RAN1#112bis-e, RAN1 Chairman’s Notes, Apr. 2023.

Appendix A: Calibration and evaluation assumptions for SLS

Table A-1:  Calibration assumptions for SLS.

|  | Urban Macro (FR1) | Dense Urban Macro (FR2-1) |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 30 GHz |
| System bandwidth | 100 MHz | 100 MHz |
| Numerology | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS = 30 kHz | SCS = 120 kHz |
| BS transmit power: Legacy TDD | 53 dBm for 100MHz | 40 dBm for 100 MHz |
| UE Tx power | 23 dBm | 23 dBm |
| Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |
| Inter-BS (2D) distance | 500 m | 200 m |
| Minimum BS-UE (2D) distance | 35 m | 35 m |
| Minimum UE-UE (2D) distance | 1 m | 1 m |
| UE distribution | UE Clustering | UE Clustering |
| BS antenna height | 25 m | 25 m |
| UE antenna height | 1.5 m | 1.5 m |
| gNB-UE Channel model (both large and small scale fading) | Macro-to-UE: UMa in TR 38.901; AAS and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |  |
| gNB-gNB Channel model (both large and small scale fading) | Macro-to-Macro: UMa in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. |  |
| UE-UE Channel model (both large and small scale fading) | UE-to-UE: UMi-Street canyon in TR 38.901 |  |
| BS antenna array configuration for legacy TDD | (M,N,P,M_g,N_g;M_p,N_p ) = (8,8,2,1,1; 2,8) | (M,N,P,M_g,N_g;M_p,N_p ) = (4,16,2,2,2; 1,1) |
|  | (d_H,d_V )= (0.5, 0.8)λ, +45°/-45° polarization | (d_H,d_V )= (0.5, 0.5)λ, +45°/-45° polarization |
| BS antenna array configuration for SBFD | l Option 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD | l Option 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD |
| UE antenna configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1), (dH,dV) = (0.5, 0.5)λ, 0°,90° polarization |
|  | 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |  |

Table A-2:  Evaluation assumptions for SLS.

|  |  | Parameters |  |
| --- | --- | --- | --- |
|  |  | Urban Macro (FR1) | Dense Urban Macro (FR2-1) |
| Interference Modelling | gNB self-interference - αSI | 150.09dB | 135.09dB |
| SBFD subband and slot configuration | SBFD slot configuration | {XXXXU} |  |
|  | SBFD Subband configuration | 273 Subband: < ND, NU, NG > = <104, 55, 5> | 66 subband: < ND, NU, NG > = <25, 14, 1> |
|  | UL/DL resource percentage per TDD period | 0.593 | 0.61 |
| BS transmit power & antenna configuration | BS transmit power for SBFD | 53 dBm | 40 dBm |
|  | BS antenna configuration for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization |
|  | BS antenna configuration for SBFD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization | (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization |
|  | BS antenna radiation pattern | 8 dBi | 8 dBi |
|  | UE antenna configuration | 0 dBi | 5 dBi |
| Traffic Model | DL/UL traffic assignment for the same UE | Option 1: Each UE is either assigned UL traffic or DL traffic. |  |
|  | DL/UL FTP packet size | UL FTP packet size 0.5Mbytes |  |
|  | DL/UL traffic load for legacy TDD | UL traffic load: 10%/30%/50% Type-2 RU DL traffic load: 10%/30%/50% Type-2 RU |  |
| Channel model | gNB-gNB | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) |  |
|  | UE-UE | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). |  |
| Others | Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -86 dBm, alpha = 0.9 |
|  | UE receiver | MMSE-IRC only suppresses the legacy interference |  |
|  | Channel estimation | Realistic |  |
|  | Transmission scheme | OFDM |  |

Appendix B: Link budget analysis for LLS

Table B-1:  Link budget analysis for FR1

| enarios for evaluation | Urban Macro |  |  |  |
| --- | --- | --- | --- | --- |
| Scenarios | Legacy TDD Low Load | SBFD Low Load | Legacy TDD Medium Load | SBFD Medium Load |
| Carrier frequency (GHz) | 4 GHz | 4 GHz | 4 GHz | 4 GHz |
| BS antenna heights (m) | 25.00 | 25.00 | 25.00 | 25.00 |
| UT antenna heights (m) | 1.50 | 1.50 | 1.50 | 1.50 |
| Cell area reliability (%) | - | - | - | - |
| Lognormal shadow fading std deviation (dB) | - | - | - | - |
| Pathloss model | - | - | - | - |
| UE speed (km/h) | 3 km/h | 3 km/h | 3 km/h | 3 km/h |
| Channel for evaluation | PUSCH | PUSCH | PUSCH | PUSCH |
| UL-DL configuration for TDD | DDDSU S=[12D:2G:0U] | XXXXU | DDDSU S=[12D:2G:0U] | XXXXU |
| Subcarrier Spacing | 30 kHz | 30 kHz | 30 kHz | 30 kHz |
| Channel model for link level simulation | TDL-C | TDL-C | TDL-C | TDL-C |
| Frequency hopping | w/o | w/o | w/o | w/o |
| number of PRBs, TBS and MCS | PRB = 30, MCS = 4 | PRB = 30, MCS = 4 | PRB = 30, MCS = 4 | PRB = 30, MCS = 4 |
| BWP size | 100 MHz | 100 MHz | 100 MHz | 100 MHz |
| DMRS configuration | Length : 1 Additional : pos1 | Length : 1 Additional : pos1 | Length : 1 Additional : pos1 | Length : 1 Additional : pos1 |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |
| Repetition | 5.00 | 5.00 | 5.00 | 5.00 |
| HARQ configuration | w/o | w/o | w/o | w/o |
| Latency requirements for voice | - | - | - | - |
| PUCCH format type | - | - | - | - |
| Tx Diversity | - | - | - | - |
| Target error rate (BLER, miss detection, false alarm, etc.) | 10% iBLER | 10% iBLER | 10% iBLER | 10% iBLER |
| PRACH format | - | - | - | - |
| Number of SSB | - | - | - | - |
| Correlation for TxRU at BS | - | - | - | - |
| Transmitter |  |  |  |  |
| (1) Number of transmit antenna elements. | 2.00 | 2.00 | 2.00 | 2.00 |
| (2) (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink | - | - | - | - |
| (2a) Number of transmit chains modelled in LLS | 2.00 | 2.00 | 2.00 | 2.00 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23.00 | 23.00 | 23.00 | 23.00 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 10800000.00 | 10800000.00 | 10800000.00 | 10800000.00 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) | 12.67 | 12.67 | 12.67 | 12.67 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) | 10800000.00 | 10800000.00 | 10800000.00 | 10800000.00 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 | 23.00 | 23.00 | 23.00 |
| (4) total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) - (4b) (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (4a) antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ( (1) / (2) ) (dB) for downlink, and = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (4b) antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (4c) gain of antenna element (dBi) | 0.00 | 0.00 | 0.00 | 0.00 |
| (5) total antennna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (5b) antena gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1.00 | 1.00 | 1.00 | 1.00 |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm | 22.00 | 22.00 | 22.00 | 22.00 |
| Receiver |  |  |  |  |
| (10) Number of receive antenna elements | 192.00 | 192.00 | 192.00 | 192.00 |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | 32.00 | 32.00 | 32.00 | 32.00 |
| (10b) Number of receive chains modelled in LLS | 4.00 | 4.00 | 4.00 | 4.00 |
| (11) total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) | 11.78 | 11.78 | 11.78 | 11.78 |
| (11a) antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink = (11c) + 10 log ( (10)/(10b) ) (dB) for downlink | 11.78 | 11.78 | 11.78 | 11.78 |
| (11b) antena gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (11c) gain of antenna element (dBi) | 4.00 | 4.00 | 4.00 | 4.00 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 9.03 | 9.03 | 9.03 | 9.03 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 9.03 | 9.03 | 9.03 | 9.03 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0.00 | 0.00 | 0.00 | 0.00 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3.00 | 3.00 | 3.00 | 3.00 |
| (13) Receiver noise figure (dB) | 5.00 | 5.00 | 5.00 | 5.00 |
| (14) Thermal noise density (dBm/Hz) | -174.00 | -174.00 | -174.00 | -174.00 |
| (15) Receiver interference density (dBm/Hz) | -172.00 | -172.00 | -172.00 | -172.00 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -166.00 | -166.00 | -166.00 | -166.00 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -95.67 | -95.67 | -95.67 | -95.67 |
| (19) Required SNR (dB) | -5.25 | -5.12 | -5.13 | -4.99 |
| (20) Receiver implementation margin (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | 0.00 | 0.00 | 0.00 | 0.00 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -98.92 | -98.79 | -98.80 | -98.66 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 130.95 | 130.82 | 130.83 | 130.69 |
| (23) Hardware link budget, a.k.a MIL = (9) + (11) + (11bis) - (12) - (22)(dB) | 138.73 | 138.60 | 138.61 | 138.47 |
| Calculation of available pathloss |  |  |  |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation ) (dB) | 4.48 | 4.48 | 4.48 | 4.48 |
| (26) BS selection/macro-diversity gain (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (27) Penetration margin (dB) | 26.25 | 26.25 | 26.25 | 26.25 |
| (28) Other gains (dB) (if any please specify) | 0.00 | 0.00 | 0.00 | 0.00 |
| (29) MPL : Available path loss = (23) – (25) + (26) – (27) + (28) (dB) | 108.00 | 107.87 | 107.88 | 107.74 |

Table B-2:  Link budget analysis for FR2-1

| Senarios for evaluation | Dense Urban Macro |  |  |  |
| --- | --- | --- | --- | --- |
| Scenarios | Legacy TDD Low Load | SBFD Low Load | Legacy TDD Medium Load | SBFD Medium Load |
| Carrier frequency (GHz) | 28 GHz | 28 GHz | 28 GHz | 28 GHz |
| BS antenna heights (m) | 25.00 | 25.00 | 25.00 | 25.00 |
| UT antenna heights (m) | 1.50 | 1.50 | 1.50 | 1.50 |
| Cell area reliability (%) | - | - | - | - |
| Lognormal shadow fading std deviation (dB) | - | - | - | - |
| Pathloss model | - | - | - | - |
| UE speed (km/h) | 30 km/h | 30 km/h | 30 km/h | 30 km/h |
| Channel for evaluation | PUSCH | PUSCH | PUSCH | PUSCH |
| UL-DL configuration for TDD | DDDSU S=[12D:2G:0U] | XXXXU | DDDSU S=[12D:2G:0U] | XXXXU |
| Subcarrier Spacing | 120 kHz | 120 kHz | 120 kHz | 120 kHz |
| Channel model for link level simulation | TDL-A | TDL-A | TDL-A | TDL-A |
| Frequency hopping | w/o | w/o | w/o | w/o |
| number of PRBs, TBS and MCS | PRB = 26, MCS = 7 | PRB = 26, MCS = 7 | PRB = 26, MCS = 7 | PRB = 26, MCS = 7 |
| BWP size | 200 MHz | 200 MHz | 200 MHz | 200 MHz |
| DMRS configuration | Length : 1 Additional : pos2 | Length : 1 Additional : pos2 | Length : 1 Additional : pos2 | Length : 1 Additional : pos2 |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |
| Repetition | 5.00 | 5.00 | 5.00 | 5.00 |
| HARQ configuration | w/o | w/o | w/o | w/o |
| Latency requirements for voice | - | - | - | - |
| PUCCH format type | - | - | - | - |
| Tx Diversity | - | - | - | - |
| Target error rate (BLER, miss detection, false alarm, etc.) | 10% iBLER | 10% iBLER | 10% iBLER | 10% iBLER |
| PRACH format | - | - | - | - |
| Number of SSB | - | - | - | - |
| Correlation for TxRU at BS | - | - | - | - |
| Transmitter |  |  |  |  |
| (1) Number of transmit antenna elements. | 8.00 | 8.00 | 8.00 | 8.00 |
| (2) (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink | - | - | - | - |
| (2a) Number of transmit chains modelled in LLS | 2.00 | 2.00 | 2.00 | 2.00 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23.00 | 23.00 | 23.00 | 23.00 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 37440000.00 | 37440000.00 | 37440000.00 | 37440000.00 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) | 7.27 | 7.27 | 7.27 | 7.27 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) | 37440000.00 | 37440000.00 | 37440000.00 | 37440000.00 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 | 23.00 | 23.00 | 23.00 |
| (4) total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) - (4b) (dB) | 11.02 | 11.02 | 11.02 | 11.02 |
| (4a) antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ( (1) / (2) ) (dB) for downlink, and = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink | 11.02 | 11.02 | 11.02 | 11.02 |
| (4b) antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (4c) gain of antenna element (dBi) | 5.00 | 5.00 | 5.00 | 5.00 |
| (5) total antennna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (5b) antena gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0.00 | 0.00 | 0.00 | 0.00 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1.00 | 1.00 | 1.00 | 1.00 |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm | 33.02 | 33.02 | 33.02 | 33.02 |
| Receiver |  |  |  |  |
| (10) Number of receive antenna elements | 256.00 | 256.00 | 256.00 | 256.00 |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | 2.00 | 2.00 | 2.00 | 2.00 |
| (10b) Number of receive chains modelled in LLS | 2.00 | 2.00 | 2.00 | 2.00 |
| (11) total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) | 27.07 | 27.07 | 27.07 | 27.07 |
| (11a) antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink = (11c) + 10 log ( (10)/(10b) ) (dB) for downlink | 27.07 | 27.07 | 27.07 | 27.07 |
| (11b) antena gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (11c) gain of antenna element (dBi) | 6.00 | 6.00 | 6.00 | 6.00 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0.00 | 0.00 | 0.00 | 0.00 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0.00 | 0.00 | 0.00 | 0.00 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0.00 | 0.00 | 0.00 | 0.00 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3.00 | 3.00 | 3.00 | 3.00 |
| (13) Receiver noise figure (dB) | 10.00 | 10.00 | 10.00 | 10.00 |
| (14) Thermal noise density (dBm/Hz) | -174.00 | -174.00 | -174.00 | -174.00 |
| (15) Receiver interference density (dBm/Hz) | -172.00 | -172.00 | -172.00 | -172.00 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -161.00 | -161.00 | -161.00 | -161.00 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -85.27 | -85.27 | -85.27 | -85.27 |
| (19) Required SNR (dB) | 3.83 | -3.53 | 6.07 | -1.00 |
| (20) Receiver implementation margin (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | 0.00 | 0.00 | 0.00 | 0.00 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -79.44 | -86.80 | -77.20 | -84.27 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 102.44 | 109.80 | 100.20 | 107.27 |
| (23) Hardware link budget, a.k.a MIL = (9) + (11) + (11bis) - (12) - (22)(dB) | 136.53 | 143.89 | 134.29 | 141.36 |
| Calculation of available pathloss |  |  |  |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation ) (dB) | 5.20 | 5.20 | 5.20 | 5.20 |
| (26) BS selection/macro-diversity gain (dB) | 0.00 | 0.00 | 0.00 | 0.00 |
| (27) Penetration margin (dB) | 9.00 | 9.00 | 9.00 | 9.00 |
| (28) Other gains (dB) (if any please specify) | 0.00 | 0.00 | 0.00 | 0.00 |
| (29) MPL : Available path loss = (23) – (25) + (26) – (27) + (28) (dB) | 122.33 | 129.69 | 120.09 | 127.16 |