3GPP TSG RAN WG1 #113          	          	      		                       R1- 2305632

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item: 	9.4.2

Source:   	LG Electronics

Title: 	Discussion on co-channel coexistence for LTE sidelink and NR sidelink

Document for:   	Discussion and decision

- Introduction

According to WID of Rel-18 NR SL evolution [1], the objective for co-channel coexistence for LTE SL and NR SL is as follows:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

The following agreements on the co-channel coexistence for LTE SL and NR SL were made in RAN1#112bis-e meeting.

Agreement: In NR SL resource (re)selection procedure, for dynamic resource pool sharing, the list of initial SL RSRP thresholds is separately (pre)configured (i.e., Alt 3 in the agreement of RAN1#112 meeting) for the PHY layer of NR SL module to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE. - FFS: whether a different initial SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH. Agreement: In NR SL resource (re)selection procedure, for determining the LTE SL periodic reserved resources by other LTE SL UE, the formula of Q in Section 8.1.4 in TS 38.214 is used. - FFS: whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas Note: The reservation period and the location of the LTE SL periodic reserved resources are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213. Agreement: In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe (in which UE has not monitored due to its transmission) in LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - The same formula of Q for determining the LTE SL periodic reserved resources by other SL UE is used - Note: The reservation period and the location of the LTE SL resources repeated Q times in the above procedure are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213 - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether the set SA is initialized to the set of all the candidate single-slot resources excluding NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframes when the amount of candidate single-slot resources is not sufficient as specified in Step 5a in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in NR SL slots that overlap with LTE SL resource pool where NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframes of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: The NR SL module excludes NR SL candidate resources overlapping with NR SL resources (including NR SL resource reservation period(s) (pre)configured in a NR SL resource pool) associated with non-monitored slot as specified in Section 8.1.4 of TS 38.214 (i.e., NR SL slot in which UE has not monitored due to its own transmission), which is the existing Rel-16/17 NR SL behavior - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module Agreement: In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in a NR SL slot where NR SL periodic resources are in the NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR SL module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - Alt 1: The above procedure is applied at least when the priority of LTE SL transmission is higher than the priority of NR SL transmission - It is up to UE implementation whether or not to apply the above procedure when the priority of LTE SL transmission is not higher than the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module Agreement: For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot Agreement: When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing - FFS: whether/how to differently handle the case when different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected. Agreement: RAN1 does not pursue further enhancements except for the following to handle NR SL candidate resources overlapping with LTE PSCCH for non-adjacent LTE SL resource pool in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. - NR SL module uses LTE SL RSRP measurement results (as specified in TS 36.213) shared from LTE SL module to determine excluding NR SL candidate resources overlapping with LTE PSCCH and/or LTE PSSCH resources reserved by other LTE SL UE in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. Agreement: In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain, - Down-selection one of followings: - Option 1: - When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - The list of the above initial SL RSRP thresholds is separately (pre)configured - FFS: whether this initial SL RSRP threshold list can be (pre)configured per subset of PSFCH resources - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR SL module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE and the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether to apply the above procedure based on the priority of the LTE SL reserved resources and/or the priority of the NR SL transmission - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE in time domain according to Step 5 in Section 8.1.4 of TS 38.214. - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module Agreement: The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module. - The starting LTE SL subframe is no later than the time (n-T_start) - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214 - Down-selection one of followings for T_start: - Option 1-1: T_start is T_0 as defined in clause 8.1.4 of TS 38.214 - Option 1-2: T_start is 1100ms - Option 1-3: T_start is up to UE implementation - The ending LTE SL subframe is no earlier than the time (n-T_valid2) - Down-selection one of followings for T_valid2: - Option 2-1: T_valid2 is T - Option 2-2: T_valid2 <= T+4ms - Option 2-3: T_valid2 is up to UE implementation

In this contribution, we discuss several remaining essential issues in supporting the co-channel coexistence for LTE SL and NR SL.

- Discussion

One of remaining issues is whether/how to differently handle the case where different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected when compared to the case where the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe. At this moment, we do not see any motivation to define different solutions for these two scenarios in terms of avoiding the AGC impact on the LTE SL reception. In other words, even in the former case, it can be defined that it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with the LTE SL subframe.

Proposal 1: When different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with the LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing.

We think that the RAN1 needs to discuss what criteria/conditions the PHY layer of NR SL module uses to excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain. For example, the method in which the PHY layer of NR SL module performs this kind of exclusion operation in Step 5 in Section 8.1.4 of TS 38.214 can result in a problem in which the number of remaining NR SL candidate resources is insufficient frequently. Considering this aspect, the SL RSRP value associated with the LTE SL reserved resources can be defined to be considered when the PHY layer of NR SL module determines whether to perform the exclusion operation of the above-mentioned NR SL candidate resources. Also, in terms of giving the network controllability for the impact of PSFCH transmission on the LTE SL reception, the initial list of these SL RSRP thresholds can be (pre)configured separately from those used when the PHY layer of NR SL module determines whether to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE.

Proposal 2: In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain,

- When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission and the list of the initial SL RSRP thresholds is separately (pre)configured.

Further discussion is also necessary on whether/how to define the starting/ending LTE SL subframe of information to be used by NR SL module from the information shared by LTE SL module. Since the NR SL module is expected to set these timings in terms of maximizing its performance, we think that it is unclear whether the additional/explicit specification work is required. Note that this expected behaviour of NR SL module will also ensure the performance of LTE SL UE coexisting in the same resource pool.

Proposal 3: The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-T_start), where n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214 and T_start is up to UE implementation.

- The ending LTE SL subframe is no earlier than the time (n-T_valid2), where T_valid2 is up to UE implementation.

- Conclusion

In this contribution, we discussed the remaining essential aspects need to be resolved to support the coexistence between LTE SL and NR SL in the same frequency channel. The following proposals are given.

Proposal 1: When different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with the LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing.

Proposal 2: In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain,

- When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission and the list of the initial SL RSRP thresholds is separately (pre)configured.

Proposal 3: The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-T_start), where n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214 and T_start is up to UE implementation.

- The ending LTE SL subframe is no earlier than the time (n-T_valid2), where T_valid2 is up to UE implementation.

- Reference

[1] RP-230077, “WID revision: NR sidelink evolution,” OPPO.