3GPP RAN1#113                	R1-2305645

# Athens, 22th – 26th May, 2023

Agenda item:		9.10.2

Source:	MediaTek Inc.

Title:	UL timing management to reduce handover latency

Document for:		Discussion

# 1	Introduction

Agreement

- For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

  - If reception of RAR is not configured/indicated (without RAR)

  - If reception of RAR is configured/indicated (with RAR), FFS

  - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- For PDCCH-order based PRACH for candidate cell study the following issues:

  - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed

  - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission

- For PDCCH ordered-RACH, if reception of RAR is not configured

  - Whether power ramping is performed or not is determined from PDCCH order

  - If power ramping is performed,

  - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism)

  - power ramping-up value is configured

  - else, the power should be determined by open-loop power control

- If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell.

  - The maximum number of TA values memorized by UE is a UE capability

  - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

In RAN1 #112 Bis meeting, most of key agreements regarding Rel-18 mobility enhancement TA management had been achieved [1]. In this contribution, we discuss remaining issues that have not yet been concluded.

# 2	PDCCH order RACH procedure

According to the previous RAN1 agreement, UE need to acquire TA of the candidate cell(s) in Rel-18 LTM. As shown in Figure 1, UE might switch to the beams of neighboring cell and need to transmit PRACH toward candidate beams of multiple cells before the cell switch command.

Figure 1: Activate asynchronous TCI states of different cells before the cell switch command

RAN1 had agreed PDCCH ordered-RACH for candidate cells, whether reception of RAR is needed will be configured by the Network. Furthermore, UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM. However, RAN1 have not reached consensus on the default scheme. Considering that requiring the UE to maintain TA for neighboring cells before camping-on to them has no practical use, we believe that the RAR-less approach is simpler and more intuitive. So we suggest that

Proposal 1: Supporting of RAR-less PDCCH ordered RACH procedure is the baseline UE approach for LTM

If a PARCH transmission to an LTM candidate cell overlaps (in time and frequency) with serving cell UL transmission, performance degradation on the serving cell is expected due to power sharing with the neighboring cell. Because of poor reception signal quality, the base station may not correctly receive data packets from the UE, including ACK/NACK feedback, resulting in an increased packet loss rate and unreliable communication. Therefore, we believe that the uplink transmission of the serving cell should be prioritized. When the uplink transmission of the serving cell and the PRACH transmission of the neighboring cell conflict in time, the UE shall keep the uplink transmission of the serving cell and give up the PRACH transmission to the neighboring cell.

Proposal 2: The serving cell UL transmission and neighboring cell PRACH transmission should not overlap in time. If conflicts occur, the UE should maintain the serving cell UL transmission and cancel the PRACH transmission to neighboring cell(s)

RAN1 already agreed that for PDCCH-ordered RACH, if reception of RAR is not configured, whether power ramping is performed or not is determined from PDCCH order. Some companies have concerns on no indication in PDCCH order to inform UE whether it is initial transmission or retransmission. However, in legacy systems, even if the UE doesn't know whether the PDCCH order is an initial transmission or a retransmission, it does not cause the PRACH procedure to fail. The UE may consider the first successfully decoded PDCCH order as an initial transmission. If the network indicates a power ramp, the initial transmission may ignore the indication and perform the power ramp the next time it transmits to the same target.

Proposal 3: UE considers the first successfully decoded PDCCH order as an initial transmission. If the network indicates a power ramp, the initial transmission may ignore the indication and perform the power ramp the next time it transmits PRACH to the same target

We will next discuss the scenario where RAR is configured to be received. In this case, RAN1 has already agreed that at least the TA of the candidate cell should be included in the RAR. Considering that the UE might only be able to receive the candidate cell's RAR from the serving cell, we believe the information of cell identity should also be included. This way, the UE can distinguish which cell the current RAR is sent from.

Proposal 4: For PDCCH-ordered RACH for candidate cell(s), if reception of RAR is configured (with RAR), the content of RAR should include

- TA of candidate cell,

- explicit indication or implicit indication of candidate cell identify (can be cell ID or TCI state ID/index of activated TCI state)

In this case, some companies believe that associating TA/TAG with candidate cell(s) is necessary. However, currently only the PDCCH order RACH procedure is agreed upon, and the UE is not allowed to choose a particular target TCI state or SSB to maintain and represent the TA of a specific TAG. There is no need for the UE to know whether the target TCI states or SSBs belong to the same cell or not, as that information can only be maintained on the network side.

Observation 1: Association of existing TAG ID for serving cell and candidate cell is not needed in the configuration for candidate cell(s).

For the number of TA(s)/TAG(s) per candidate cell, since in case of CA or mTRP operations up to two TA/TAGs can be supported, the same rules can be applied for LTM. In such cases, for a candidate cell, the UE may be provided configurations to acquire multiple TAs (via multiple or single PDCCH order), but only one of them will be chosen and provided to the UE in the cell switch command.

Proposal 5: Up to two TAs can be supported per candidate cell for Rel-18 LTM

# 3	UE-based TA measurement

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

In the RAN1 #112 meeting, companies discussed the feasibility of UE-based TA measurement.

The advantage of this method is that the UE no longer needs to send PRACH to neighboring cells in order to obtain the corresponding TA values. However, we believe that for this measurement method to be accurate, it must be based on certain conditions. As shown in Figure 2, to ensure accurate measurement results, the UE must not only know the propagation delay tt but also the time difference (clock error) between the serving cell and neighboring cell on Network side y. If either of these two pieces of information is missing, it will lead to significant errors in the UE's estimation result. Here, we provide an example to illustrate the reason.

The timing difference between cells estimated on UE side is

Δt = ttarget – tsource
        = y + tt - ts.

And the known information for the UE is the TA of serving cell ts. Assuming no other information provided, the estimated TA of neighboring cell is

Estimated neighboring cell’s TA

=  – [Δt + ts]
         =  – y - tt

Estimation error  
= Estimated neighboring cell’s TA – 
   True neighboring cell’s TA
=  [ T – y – tt] – [ T + y – tt]
= -2y

Figure 2: UE based TA measurement procedure

We can see that the timing difference between cells on Network side y would be doubled in the estimated TA value of neighboring cell. In other words, if there exists a certain clock error of 3μs between cells on network side and UE is unaware of that error value, the UE’s estimated result will differ from the actual TA value of neighboring cell by 6 μs. We don’t think that this error is acceptable. So we suggest that UE-based TA measurement can only be agreed upon if the following two conditions are met; otherwise, we risk misleading RAN4 in setting unreasonable requirements.

Proposal 6: UE-based TA measurement can only be agreed upon if

- the network can guarantee that the clock error between 2 cells is less than X nanoseconds, where value of x is FFS, and

- the network can inform UE of the timing different (including the clock error) between cells y on network side, allowing UE to derive neighboring cell’s TA by itself (no need for PRACH transmission toward neighboring cell)

# 4	Summary

In this contribution, we have following observations

Observation 1: Association of existing TAG ID for serving cell and candidate cell is not needed in the configuration for candidate cell(s).

and we propose

Proposal 1: Supporting of RAR-less PDCCH ordered RACH procedure is the baseline UE approach for LTM

Proposal 2: The serving cell UL transmission and neighboring cell PRACH transmission should not overlap in time. If conflicts occur, the UE should maintain the serving cell UL transmission and cancel the PRACH transmission to neighboring cell(s)

Proposal 3: UE considers the first successfully decoded PDCCH order as an initial transmission. If the network indicates a power ramp, the initial transmission may ignore the indication and perform the power ramp the next time it transmits PRACH to the same target

Proposal 4: For PDCCH-ordered RACH for candidate cell(s), if reception of RAR is configured (with RAR), the content of RAR should include

- TA of candidate cell,

- explicit indication or implicit indication of candidate cell identify (can be cell ID or TCI state ID/index of activated TCI state)

Proposal 5: Up to two TAs can be supported per candidate cell for Rel-18 LTM

Proposal 6: UE-based TA measurement can only be agreed upon if

- the network can guarantee that the clock error between 2 cells is less than X nanoseconds, where value of x is FFS, and

- the network can inform UE of the timing different (including the clock error) between cells y, allowing UE to derive neighboring cell’s TA by itself (no need for PRACH transmission toward neighboring cell)

# Reference

- 3GPP TSG-RAN WG1 Meeting #112 Bis Chairman Notes, Apr., 2023.