3GPP TSG-RAN WG1 Meeting #113	R1-2305646

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	5

Source:	MediaTek Inc.

Title:	Evaluation of SRS antenna switching with insertion loss

Document for:	Discussion

# 1	Introduction

At RAN1#112bis-e, discussion took place on the LS in [1] and is summarised in [2]. The present document provides some further evaluation on the impact of Tx insertion loss on the performance of SRS based DL precoding for the UE with 8Rx antennas. We also provide analysis for 4 Rx antennas although this was not highlighted as an issue by RAN4 but raised by a few companies in last RAN1 meeting.

# 2	Discussion

## 2.1	Expected UE behaviour and feasibility for UE to perform IL compensation

As stated in [3], TS38.213 expects equal transmit power to be used on each SRS antenna port within an SRS resource set. However, as also stated, TS38.101-1 also specifies ∆TRxSRS for configured maximum output power requirements to allow for some deviation from the maximum configured output power for the UE when transmitting SRS.

There was a claim documented in [2] that the UE will never perform any insertion loss compensation when setting the SRS power even when there is remaining unused Tx power available at the UE. However, the text in TS38.213 seems to conflict with that statement. It seems to be feasible for a UE to compensate for additional insertion loss on additional antenna ports relative to primary ports if it has remaining Tx power available to overcome any losses. However, when the UE is operating at close to maximum output power, overcoming additional insertion loss would be more challenging so imbalance can be expected.

Observation 1: It seems feasible for a UE implementation to compensate for additional insertion loss on additional antenna Tx ports relative to primary Tx ports where it has sufficient remaining power available to perform such compensation.

Observation 2: When the UE does not have sufficient remaining Tx power available, it can be expected that there may be some SRS power imbalance.

## 2.2	Performance analysis

Analysis on results presented previously

Results from [4] seem to show that at medium to mid-high CNR (where the UE is unlikely to be Tx power limited) while signalling SRS power offsets to gNB does provide some gain when the UE does not perform IL compensation, the DL performance is still degraded compared to the case where there was zero imbalance.

Given that achieving “zero SRS power imbalance” appears to facilitate much better DL performance than a UE “with SRs power imbalance” reporting SRS power offsets to gNB. However, in such a scenario we believe it is likely that the UE has Tx power available such that it could compensate IL imbalance itself.

Observation 3: Results from [4] show that at mid-high CNR, relying on gNB SRS power imbalance compensation via signalled power offsets is inferior in terms of improving DL performance compared to the “zero imbalance” scenario. In this scenario we assume that the UE would likely have Tx power available to compensate potential SRS power imbalance by itself.

Results from [4] also seem to show that the benefit of signalling any SRS offset for UEs operating at low CNR is negligible (approx. 0.5%).

Observation 4: Results from [4] show that, at low CNR, the benefit of gNB SRS power imbalance compensation via signalled power offsets is negligible in terms of benefit to DL performance.

New analysis

We have performed its own evaluation of system-level performance based on different schemes. Simulation assumptions can be found in the Annex of this document, and the results are shown in the figures below, considering different IL compensation approaches for 8Rx and 4Rx UEs (no IL, UE-based, UE+gNB based on dynamic reports, and gNB-only based on static reports).

Figure 1: Average cell UPT vs reference (%)

Figure 2: 5%-ile (cell edge) throughput vs reference (%)

From Figures 1 and 2 the following can be observed:

- No solution seems able to fully recover ideal “non-IL SRS-based CSI” performance for all UEs.

- For the 5%-ile users where IL impact for SRS-based CSI is worst, Type 1 CSI outperforms all compensation options.

- Even with ideal dynamic SRS IL offset reporting combined with UE IL self-compensation, additional system gains of gNB performing complementary IL compensation are negligible and can have a negative impact. Such dynamic reporting would also lead to more overheads in UL.

- Replacing UE IL self-compensation with gNB performing IL compensation based on static reports has no gain and can cause a loss.

  - Note: While static reporting is not modelled for 5%-ile case, it can be assumed that it would be no better than the dynamic reporting results shown.

- We attribute the performance loss of IL compensation by the gNB to the fact that channel estimation error is amplified by the gNB when compensating the signal, which can degrade precoding performance.

Observation 5: Results show that, in the 4Rx and 8Rx scenarios modelled, there is no DL performance benefit in the UE reporting SRS offset values and gNB performing compensation of IL compared to the UE performing IL compensation alone.

Observation 6: Results show that, for UEs operating at low SINR in both the 4Rx and 8Rx scenarios modelled, Type 1 CSI-RS based CSI reporting enables superior DL performance when compared to SRS-based CSI reporting when the Tx port contains IL.

Proposal: Reply to RAN4 that RAN1 does not see the benefit in specifying reporting of SRS IL offsets due to IL imbalance, because it does not provide benefit in relevant practical scenarios for SRS-based CSI usage.

# 3	Conclusion

The following observations and proposals are made:

Observation 1: It seems feasible for a UE implementation to compensate for additional insertion loss on additional antenna Tx ports relative to primary Tx ports where it has sufficient remaining power available to perform such compensation.

Observation 2: When the UE does not have sufficient remaining Tx power available, it can be expected that there may be some SRS power imbalance.

Observation 3: Results from [4] show that at mid-high CNR, relying on gNB SRS power imbalance compensation via signalled power offsets is inferior in terms of improving DL performance compared to the “zero imbalance” scenario. In this scenario we assume that the UE would likely have Tx power available to compensate potential SRS power imbalance by itself.

Observation 4: Results from [4] show that, at low CNR, the benefit of gNB SRS power imbalance compensation via signalled power offsets is negligible in terms of benefit to DL performance.

Observation 5: Results show that, in the 4Rx and 8Rx scenarios modelled, there is no DL performance benefit in the UE reporting SRS offset values and gNB performing compensation of IL compared to the UE performing IL compensation alone.

Observation 6: Results show that, for UEs operating at low SINR in both the 4Rx and 8Rx scenarios modelled, Type 1 CSI-RS based CSI reporting enables superior DL performance when compared to SRS-based CSI reporting when the Tx port contains IL.

Proposal: Reply to RAN4 that RAN1 does not see the benefit in specifying reporting of SRS IL offsets due to IL imbalance, because it does not provide benefit in relevant practical scenarios for SRS-based CSI usage.

# 4	References

R1-2302267, LS on the UE SRS IL imbalance issue, RAN4

R1-2304284, Summary of email discussion [112bis-e-LS-04], Moderator (Huawei)

R1-2303270, Discussion on RAN4 LS on the UE SRS IL imbalance issue, Nokia

R4-2300696, 8RX UE RF requirements, Qualcomm

3GPP TS 38.101-1 specification

# Annex: Simulation assumptions

| Parameter | Value |
| --- | --- |
| Evaluation configuration | Configuration A from IMT-2020 evaluations |
| Channel model | UMa_A |
| ISD | 500 m |
| Frame structure | DSUDD |
| Carrier Frequency | 4 GHz |
| System bandwidth | 20MHz |
| Subcarrier spacing | 30KHz |
| Number of TXRU per TRxP | 32 TXRUs |
| Number of antenna elements per TRxP | (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1, 2,8) for 32TXRU |
| Transmit power per TRxP | 43 dBm |
| TRxP number per site | 3 |
| Mechanic tilt | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt | 105deg in LCS |
| UE power class | PC2 26dBm |
| Number of antenna elements per UE | 4Rx or 8Rx 0°,90° polarization antenna (dH, dV) = (0.5λ, 0.5λ) (M,N,P,Mg,Ng,Mp,Np) = (1,2,2,1,1, 1,2) for 4RX (M,N,P,Mg,Ng,Mp,Np) = (1,4,2,1,1, 1,4) for 8RX |
| UT attachment | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling | PF scheduler |
| ACK/NACK delay | Next available UL slot |
| MIMO mode | SU-MIMO with rank 4 or 8 adaptation and OLLA per user |
| BS/MS receiver type | MMSE-IRC |
| Channel estimation | realistic |