3GPP TSG RAN WG1 Meeting #113	R1-2305685

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Sharp

Title:	Measurements and reporting for SL positioning

Agenda Item:	9.5.1.2

Document for:	Discussion and Decision

Introduction

A new work item on expanded and improved NR Positioning [1] was approved in RAN#98-e meeting, with one of the objectives to “specify solutions for support of sidelink positioning (including ranging) in NR systems”, with one objective to “specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA”, and another objective to “specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios”.

In this document, we share our views on a few aspects of measurements and reporting for SL positioning.

Discussion

Triggering of SL positioning measurement report

The following was agreed in RAN1#112bis-e,

Agreement Support higher layer signaling for sidelink positioning measurement report and report triggering. Up to RAN2 to discuss detailed signaling design. FFS on SCI based report triggering.

Regarding the FFS on SCI based report triggering, we think at least for aperiodic SL-PRS resources, triggering of the corresponding measurement reports, if by higher layer signaling, would be very complex due to the uncertainty of reception time of the triggering indication. Therefore, at least for such cases we think SCI based triggering should be supported.

Proposal 1: At least for aperiodic SL-PRS resources, support SCI based measurement report triggering.

Time domain behaviour of SL positioning measurement report

With the use of higher layer signalling to convey SL positioning measurement reports, all time domain behaviours which are possible for legacy SL data transmissions can be (and should be) supported for SL positioning measurement reports.

Proposal 2: Periodic, semi-persistent and aperiodic measurement reports are supported for SL positioning.

Priority of SL positioning measurement report

With the use of higher layer signalling to convey SL positioning measurement reports, the priority associated with the PSCCH/PSSCH transmission conveying a SL positioning measurement report should be based on the priority of the measured SL-PRS, in order to match the desired priority of the positioning service for which the SL-PRS and the corresponding measurement reports are transmitted.

Proposal 3: Priority of the SL positioning measurement report is based on priority of the measured SL-PRS.

Identification information in SL positioning measurement report

The following was agreed in RAN1#112,

Agreement Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL-PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded

And the following was agreed in RAN1#112bis-e,

Agreement - A SL PRS resource refers to a time-frequency resource within a slot of a dedicated SL PRS resource pool that is used for SL PRS transmission. - FFS: for a shared resource pool - Characteristics associated with a SL PRS resource include at least: - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation, - Note: Additional parameters can be included as/when identified. - FFS: other time domain aspects, if any - A SL PRS resource is identified by a SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. NOTE 1: The above does not imply need for signalling/(pre-)configuration of all these parameters

In Uu positioning, a DL-PRS resource ID is UE specifically configured by the network and is only meaningful between the UE and the network, i.e. from network perspective a same value of DL-PRS resource ID can be reused for another DL-PRS resource configured for another UE. However, according to the above RAN1 agreement, the association between a SL-PRS resource ID and the corresponding SL-PRS resource is defined per slot of a dedicated resource pool, and the association is common to all UEs. By definition, a “SL-PRS resource ID” can only uniquely identify a SL-PRS resource in a given slot of a given resource pool. Although it may seem straightforward to indicate a “resource pool index” along with a SL-PRS resource ID in a measurement report, we don’t think such an indication is desirable, because in a SL-based system the set of transmit resource pools may not be exactly the same for all UEs communicating with each other (e.g. in the case of partial coverage). Instead, a SL-PRS resource should be identified in a way independent of the resource pool, e.g. by a lowest PRB index (which can be used for both shared resource pools and dedicated resource pools, even if SL-PRS resourced ID is not defined for shared resource pools), a physical slot index, and a SL-PRS resource ID.

And, depending on how a “SL-PRS sequence ID” is derived as discussed in agenda item 9.5.1.1, it may be also useful to identify a SL-PRS resource in a measurement report with a SL-PRS sequence ID, e.g. if it is agreed to be provided by higher layers (although we are not in favor of this option, as stated in our companion paper [2]).

Proposal 4: The following is used to identify a SL-PRS resource in a measurement report,

- A lowest PRB index.

- A physical slot index (i.e. combination of SFN/DFN and slot index within a frame).

- A SL-PRS resource ID (in case of a dedicated resource pools).

Timestamp information in SL positioning measurement report

The following FL proposal was discussed in RAN1#112,

#### [ROUND1][Medium] FL proposal 4.2.2-v1: Time stamp for SL positioning measurement report includes at least the followings. - SFN/DFN - Slot number - FFS associated reference of the time stamp

In our understanding the above is how a physical slot index is indicated, and should be agreed.

Proposal 5: Time stamp for SL positioning measurement report includes at least the following,

- SFN/DFN.

- Slot number.

Conclusion

In this contribution, we discuss a few aspects relating to measurements and reporting for SL positioning, and make the following proposals.

Proposal 1: At least for aperiodic SL-PRS resources, support SCI based measurement report triggering.

Proposal 2: Periodic, semi-persistent and aperiodic measurement reports are supported for SL positioning.

Proposal 3: Priority of the SL positioning measurement report is based on priority of the measured SL-PRS.

Proposal 4: The following is used to identify a SL-PRS resource in a measurement report,

- A lowest PRB index.

- A physical slot index (i.e. combination of SFN/DFN and slot index within a frame).

- A SL-PRS resource ID (in case of a dedicated resource pools).

Proposal 5: Time stamp for SL positioning measurement report includes at least the following,

- SFN/DFN.

- Slot number.

References

RP-223549, “New WI: Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, RAN#98e.

R1-2305684, “SL positioning reference signal”, Sharp, RAN1#113.