3GPP TSG RAN WG1 #113		R1-2305689

Incheon, South Korea, May 22nd – May 26thth, 2023

Agenda Item:	9.2.4.1

Source:	Indian Institute of Tech (M), IIT Kanpur

Title:	Evaluation of AI/ML for Positioning Accuracy Enhancement

Document for:	Discussion

# Introduction

This document includes our discussions on the Evaluation of AI/ML for Positioning Accuracy Enhancement for Release 18. Specifically, we focus on comparisons between the approaches described in the agreements reached in the RAN1 #112 bis-e meeting [1]. In this section, we reproduce the relevant agreements from the prior meeting.

RAN1 #112 bis-e [1]

Agreement For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs include the following approaches. Proponent of each approach provide analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity. - Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0. - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed. - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically. - Note: for Approach 1, one model is provided to cover the entire evaluation area. - Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed. - For Approach 2: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18 N’TRP) TRPs. - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models. Agreement In the evaluation of AI/ML based positioning, if N’TRP<18, the set of N’TRP TRPs that provide measurements to model input of an AI/ML model are reported using the TRP indices shown below.

# Discussion

2.1 Background on AI/ML Approaches for Positioning

Two different types of AI/ML approaches were agreed upon in previous meetings. We summarize them below.

The first is the direct AI/ML approach in which the AI/ML model takes as input, a measurement such as CIR and predicts the position as the output. The second is the AI/ML assisted approach in which an AI/ML model takes as input, a measurement such as CIR and estimates intermediate parameters as its output such as LoS/NLoS classification or ToA. These intermediate parameters are used to estimate the position either by a separate AI/ML approach or a Non-AI/ML approach.

Figure 1 Two different types of AI/ML approaches for Positioning Estimation

2.2 Scenario Description and Datasets

The focus is on enhancement of UE positioning accuracy in an Indoor Factory Scenario (InF). The layout is shown in the agreements in the previous section. There are 18 base stations (TRPs) arranged in a 3x6 grid. The UEs can be located anywhere in the rectangular area. In our evaluation we simulate a single drop in which 80000 UEs are placed uniformly within the rectangular area. For the purposes of this document, we focus on the InF-DH scenario

which is characterized by high clutter density and high TRP height. Specifically, we assume 60% clutter density, 6m clutter height and 2m clutter size [2]. We utilize publicly available datasets from [3].

The dataset contains 80000 instances of both CIR and RSRP obtained from simulated DL PRS transmissions. The size of the CIR for each UE, is 18 x 256 x 2 where the 18 corresponds to number of TRPs, 256 corresponds to the CIR length (taps) and the 2 corresponds to the real and imaginary parts of the CIR coefficients. In each instance of data, 18 real valued RSRPs are provided. In each case, the label is the (x, y) co-ordinates of the UE position.

We divide the dataset into 3 sets. 78% for training, 20% for validation and 2% for testing.

2.3 Evaluation Results

We use the combination of CIR and RSRP as the input to the AI/ML model. Treating the CIR+RSRP as an image with 2 channels, we utilize a ResNet based AI/ML model architecture to estimate the position.

Observation 1: Combination of CIR and RSRP across different dimensions provides different positioning error.

We have considered 3 ways of combining the CIR and RSRP.

Case 1: Append the RSRP to the CIR in the TRP dimension – resultant dimension is (80000x36x256x2)

Case 2: Interleave the CIR and RSRP values in the TRP dimension – resultant dimension is (80000x36x256x2)

Case 3: Append the RSRP to the CIR in the CIR length dimension  – resultant dimension is (80000x18x257x2)

The CDF of the UE horizontal position error (m) for each of the 3 cases is shown below. We also show the 10, 50 and 90 percentile values. We have found that case 3 is the better approach (Table 1 and Fig 2).

Table 1: Horizontal Position Error (m) and Model Complexity for 3 cases of combining CIR and RSRP

| Approach | 10 % | 50 % | 90% | Model Complexity |
| --- | --- | --- | --- | --- |
| Case 1 | 1.15 | 2.89 | 5.39 | 2.66M parameters |
| Case 2 | 1.09 | 2.92 | 5.45 | 2.66M parameters |
| Case 3 | 1.07 | 2.63 | 5.04 | 1.48M parameters |

Figure 2: Horizontal Position Error (m) for 3 cases of combining CIR and RSRP

Observation 2: If (NTRP - N’TRP) is small, there is only a small difference between approaches 1A and 2.

Table 2: Horizontal Position Error (m) and Model Complexity with first column of TRPs not providing input

| Approach | 10 % | 50 % | 90% | Model Complexity |
| --- | --- | --- | --- | --- |
| 1A | 1.08 | 2.94 | 5.82 | 1.48M parameters |
| 2 | 1.09 | 2.80 | 5.70 | 1.28M parameters |

Figure 3: Horizontal Position Error (m) with first column of TRPs not providing input

In this simulation (Table 2 and Fig 3), we assume that the first column of TRPs (0, 1 and 2) is not providing input to the AI/ML model. We have also run simulations with TRP 0 not providing input and TRPs 0 and 1 not providing inputs. We have found that when a smaller number of TRPs are excluded from the model input, there is only a small different between Approaches 1A and 2.

Observation 3: If (NTRP - N’TRP) is large, then Approach 2 is better than Approach 1A.

Table 3: Horizontal Position Error (m) and Model Complexity with Top row of TRPs not providing input

| Approach | 10 % | 50 % | 90% | Model Complexity |
| --- | --- | --- | --- | --- |
| 1A | 1.50 | 3.94 | 7.13 | 1.48M parameters |
| 2 | 1.34 | 3.40 | 6.34 | 1.08M parameters |

Table 4: Horizontal Position Error (m) and Model Complexity with Middle row of TRPs not providing input

| Approach | 10 % | 50 % | 90% | Model Complexity |
| --- | --- | --- | --- | --- |
| 1A | 1.55 | 4.16 | 8.02 | 1.48M parameters |
| 2 | 1.25 | 3.16 | 6.45 | 1.08M parameters |

Figure 4: Horizontal Position Error (m) with

Top Row of TRPs not providing input

Figure 5: Horizontal Position Error (m) with

Middle Row of TRPs not providing input

This simulation shows two scenarios (Tables 3 and 4, Figs 4 and 5). In the first, the top row of TRPs (0, 3, 6, 9, 12, 15) is not contributing to the model input. We note that this is statistically equivalent to the scenario in which the bottom row is excluded. In the second scenario, the middle row of TRPs (1, 4, 7, 10, 13, 16) is not contributing to the model input.

In both cases, the number of contributing TRPs is constant at 12. In this case, the presence of zeros (corresponding to the TRPs not contributing) in Approach 1A carries no additional information and is detrimental to the learning of the model. Consequently, Approach 2 with a reduced input size, is better.

Observation 4: If (NTRP - N’TRP) is random, then Approach 1B is better than Approach 2.

Table 5: Horizontal Position Error (m) and Model Complexity with N’TRP = 12, random TRPs providing inputs

| Approach | 10 % | 50 % | 90% | Model Complexity |
| --- | --- | --- | --- | --- |
| 1B | 1.27 | 3.52 | 7.02 | 1.48M parameters |
| 2 | 3.60 | 9.29 | 18.28 | 1.08M parameters |

Figure 6: Horizontal Position Error (m) N’TRP = 12, random TRPs providing inputs

In this simulation (Table 5, Fig 6), we assume N’TRP = 12 but the TRPs providing input to the model are randomly chosen for each UE in the drop. In this case we observe that Approach 1B is better than Approach 2. Our understanding is that when the TRPs are randomly chosen, the number of zeros in the model input carries some information that the model is using to learn. When the TRPs are constant, the zeros have no information.

# Conclusion

In this contribution, we have expanded upon our previous contributions [4] and previous agreements to provide a discussion on the Evaluation of AI/ML for Positioning Accuracy Enhancement. The discussion has resulted in the following observations.

Observation 1: Combination of CIR and RSRP across different dimensions provides different positioning error.

Observation 2: If (NTRP - N’TRP) is small, there is only a small difference between approaches 1A and 2.

Observation 3: If (NTRP - N’TRP) is large, then Approach 2 is better than Approach 1A.

Observation 4: If (NTRP - N’TRP) is random, then Approach 1B is better than Approach 2.

# References

[1] Chair’s Notes, RAN1 #112 bis-e, April 2023

[2] 3GPP TR 38.901 V16.1.0 (2019-12), “Study on channel model for frequencies from 0.5 to 100 GHz.”

[3] Wireless Intelligence AI/ML Datasets https://wireless-intelligence.com/

[4] R1-2301667 Contributions on AIML based Positioning Accuracy Enhancement, IIT Madras, CEWiT, IIT Kanpur, RAN1 #112 Feb-March 2023