3GPP TSG RAN WG1 #113	R1-2305718

Incheon, Korea , 22nd – 26th May, 2023

Agenda item:		9.16.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for MIMO evolution WI

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1]. In RAN1#113 an extended set of topics will be included, where rapporteur’s input on UE features for each WI will be presented for discussion as well. Hence, in this contribution we provide our initial views on UE features for MIMO evolution WI.

# Discussion

Based on the agreements reached so far under the MIMO evolution WI, a skeleton of UE FGs structure is proposed in Table 1 . The Release 18 MIMO is discussing technical enhancements in several technical directions, and it is very likely that the main corresponding Release 18 UE features would be independent of each other. These technical categories are:

- Unified TCI extension for multi-TRP enhancements

- Two TAs for multi-DCI

- CSI enhancements

- DM-RS Reference signals enhancements

- SRS Reference Signals enhancements

- UL precoding for Multi-Panel transmission

- SRI/TPMI enhancements for enabling 8Tx UL transmission

Proposal: consider at least 7 independent discussion areas related to the UE features for Rel18 MIMO. Dependencies between these technical areas can be considered at the point when technical details are agreed.

Table 1: Set of new Rel-18 UE capabilities for MIMO evolution

| FG | FG name | Components | Note |
| --- | --- | --- | --- |
| 40-1-1 | Support unified TCI framework extension for M-DCI based MTRP | - Support up to 4 TCI states that can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - Support of indication of multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list by the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with joint DL/UL TCI mode, support one joint TCI state being mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, support mapping of a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - Support association of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value | Agreement (RAN1#110 Toulouse) On unified TCI framework extension, at least for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation Agreement (RAN1#110bis-e) On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement (RAN1#110bis-e) On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value |
| 40-1-2 | Support unified TCI framework extension for S-DCI based MTRP | - Support up to 4 TCI states that can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - Support indication of multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list by the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) - Support RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET - Support RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Support reception of an indicator field in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 | Agreement (RAN1#109e) On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list |
| 40-1-2-x | Support PDSCH-CJT with unified TCI framework | - Support 1 or 2 indicated joint TCI states for PDSCH-CJT | Agreement (RAN1#110bis-e) Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability |
| 40-1-2-x | Support of CSI-RS resource mapping to joint/DL TCI state in S-DCI based MTRP |  | Agreement (RAN1#112bis-e) On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability |
| 40-2-1 | Support of Two TAs for multi-DCI multi-TRP, for intra-cell and inter-cell cases | - Support of two TAGs belonging to a serving cell. - Support of two DL reference timings each associated to a TAG. - Association of TAG to UL/joint TCI state. - For intercell case, one additional PRACH configuration is supported for each configured additional PCI - Support of up to two n-TimingAdvanceOffset - Support RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS. | Agreement (RAN1#110 Toulouse) For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell. Agreement (RAN1#110 Toulouse) For multi-DCI multi-TRP operation with two TAs, up to two n-TimingAdvanceOffset value per serving cell is supported Agreement (RAN1#110bis-e) For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected Agreement (RAN1#112 Athens) For associating TAGs to target UL channels/signals for multi-DCI based multi- TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel- 15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS Agreement (RAN1#112 Athens) Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI |
|  | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA, at least for the inter-cell case. |  | Agreement (RAN1#111 Toulouse) For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. NOTE: This feature provides good benefits and flexibility, especially for the intercell case (to allow a TRP in the serving cell to trigger PRACH towards a TRP in another cell). Better to support it as a mandatory/basic feature for UEs supporting the corresponding scenario. Pre-requisite: 40-2-1 |
|  | Support of Rx timing difference between the two DL reference timings that can be assumed to be larger than CP length. |  | Agreement (RAN1#110bis-e) • For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG: • baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length • as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length |
| 40-3-1 | Support of Rel-16 based Type-II-CJT |  | The work scope of Type-II codebook refinement for CJT mTRP includes refinement of the following codebooks: - Rel-16 eType-II regular codebook |
| 40-3-1a |  |  |  |
| 40-3-2 | Support of Rel-17 based Type-II-CJT |  | The work scope of Type-II codebook refinement for CJT mTRP includes refinement of the following codebooks: - Rel-17 FeType-II port selection (PS) codebook |
| 40-3-3 | Support of Rel-16 based Type-II-Doppler |  | The Rel-18 Type-II codebook refinement for high/medium velocities comprises refinement of the following codebooks: - Refinement of the Rel-16 eType-II regular codebook, with N4>=1 |
| 40-3-4 | Support of Rel-17 based Type-II-Doppler for N4=1 |  | The Rel-18 Type-II codebook refinement for high/medium velocities comprises refinement of the following codebooks: - Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the common design with the Refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations, with N4=1 - Time-/Doppler-domain reciprocity is not assumed |
| 40-3-5 | Support of TRS-based aperiodic TDCP reporting |  | For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays For the Rel-18 TRS-based TDCP reporting, there is no consensus in supporting periodic, semi-persistent, and event-triggered/UE-initiated TDCP reporting. |
| 40- 4-1 | Enhanced DM-RS type support for PUSCH | 1.support of FD-OCC4 DMRS for PUSCH Tansmission of Cyclic shift based FD-OCC 4 sequence. | Agreement (RAN1#112 Athens) For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group Agreement (RAN1#111 Toulouse) – 38.211 For FD-OCC length 4 for PDSCH/PUSCH, select the following: - Opt.1-1 (Walsh matrix) for PDSCH - Opt.1-2 (Cyclic shift) for PUSCH |
| 40-4-2 | Enhanced DM-RS type support for PDSCH | 1.support of FD-OCC4 DMRS for PUSCH Reception of Walsh based FD-OCC4 seqeunce. 2. support of skipping scheduling restriction | Agreement (RAN1#112 Athens) For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group Agreement (RAN1#110bis-e) For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. |
| 40-4-3 | DMRS for 8TX UL support | 1.Support of 8TX UL for Rel-18 DMRS 2. Support of PTRS for 8TX UL | Agreement (RAN1#111 Toulouse) For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). |
| 40-5-1 | 8 port SRS for codebook | 1.support of 8 port SRS for codebook based UL 2.support of TDM operation of 8 port SRS 3.the number of TDM subset s {n1(default), n2, [n4],[n8]} | Agreement (RAN1#110bis-e) For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - n can be 8 - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement (RAN1#112 Athens) For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. |
| 40-5-2 | 8 port SRS for antenna switching | 1.support of 8 port SRS for antenna switching 2.support of TDM operation of 8 port SRS 3.the number of TDM subset s {n1(default), n2, [n4],[n8]} | Agreement (RAN1#110 Toulouse) For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol. Agreement (RAN1#112 Athens) For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. |
| 40-5-3 | SRS for 8TX operation for non-codebook PUSCH | 1. support of upto 8 single port SRS resources in a SRS resource sert for non-codebook PUSCH 2. maximum number of SRS resources in a SRS resource set | Agreement (RAN1#110 Toulouse) For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols. - Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling. |
| 40-5-4 | SRS cyclic shift hopping and comb offset hopping | 1.support of SRS cyclic shift hopping and comb offset hopping | Agreement (RAN1#112 Athens) For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability |
| 40-5-4a | Time domain behavior for comb offset hopping with repeition | 1. Support of comb offset hopping at every symbol or at every SRS repetition. | Agreement (RAN1#112bis-e) For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details. - UE can indicate whether it supports one or both the options. Details to be discussed in UE feature. |
| 40-6-1 | Support of single- DCI based STxMP PUSCH transmission | Support the following schemes for s-DCI based PUSCH: - SDM - SFN | Agreement (RAN1#110bis-e) Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM - SFN - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission Agreement (RAN1#112 Athens) - Dynamic switching between sTRP and mTRP with SRS resource set indicator - Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with one CW Agreement (RAN1#112 Athens) - On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN |
| 40-6-1a | Support indication of DMRS ports in different CDM groups for s-DCI based PUSCH | - Layer combination {1+2}: new entry {0, 2, 3} in DMRS table | - This is optional UE capability for UE that supports sDCI based STxMP SDM |
| 40-6-1b | Support of switching between s-DCI based SDM/SFN and Rel-17 mTRP PUSCH TDM scheme | -RRC based switching between SDM/SFN and Rel-17 mTRP PUSCH TDM | Agreement (RAN1#110bis-e) Support RRC-based switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme Conclusion (RAN1#112 Athens) There is no consensus to support dynamic switching between STxMP SDM/SFN scheme and Rel-17 mTRP PUSCH TDM scheme |
| 40-6-1-2 | Support of single DCI based STxMP PUCCH transmission | Support SFN scheme for s-DCI based PUCCH: | Agreement (RAN1#111 Toulouse) Support the SFN scheme for single-DCI based STxMP PUCCH transmission |
| 40-6-2 | Support of multi- DCI based STxMP PUSCH transmission | - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - Maximum number of layers of each PUSCH overlapping in time can be 1 or 2 s.t. UE capability - PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain - DG-PUSCH+DG-PUSCH - CG-PUSCH + DG-PUSCH - CG PUSCH + CG PUSCH | Agreement (RAN1#110bis-e) Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. Agreement (RAN1#110bis-e) Multi-DCI based STxMP PUSCH+PUSCH transmission supports the following PUSCH combinations: - DG-PUSCH+DG-PUSCH - CG-PUSCH+DG-PUSCH - CG-PUSCH+CG-PUSCH Agreement (RAN1#110bis-e) The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain Agreement (RAN1#112bis-e) - For STxMP PUSCH+PUSCH transmission in multi-DCI based system: - The maximal number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain can be 1 or 2 subject to UE capability |
| 40-7-1 | Support for SRI/TPMI enhancements for enabling 8Tx UL transmission | Number of Antenna groups Ng 1, 2, 4, 8 | Support the following cases for codebook design for 8TX precoders - Full coherent precoders with Ng=1 - FFS: Full coherent precoders with Ng=2, Ng=4 - Partial coherent precoders with Ng=2 and Ng=4 - This does not imply any relation with the number of TPMI indications for 8TX precoder - Non-coherent precoders |
| 40-7-2 | Support for SRI/TPMI enhancements for enabling 8Tx UL transmission | Extension Max layer to 8, over existing layers (up to 4): {4, 8} | Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e |
| 40-7-2a | Support for SRI/TPMI enhancements for enabling 8Tx UL transmission | Support of two antenna configurations (N, N1)=(4,1) or (2,2) | For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2)` A pair of (N1, N2) can be configured with subject to UE capability. |
| 40-7-2b | Support for SRI/TPMI enhancements for enabling 8Tx UL transmission | 8Tx full Tx mode Mode {0, 1, 2} | Agreement Framework for full power PUSCH transmission by an 8TX UE - To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used. - FFS if any change is required in the specifications. - Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used per rank. - Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration |

Proposal: Consider table 1 as starting point for discussions on MIMO evolution WI.

# Conclusion

In this contribution we presented our initial views on UE features for MIMO evolution, and we have made the following observations and proposals:

Proposal: Consider table 1 as starting point for discussions on MIMO evolution WI.

# References

[1] R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023