3GPP TSG RAN WG1 #113	R1- 2305719

Incheon (KR), 22nd – 26th May, 2023

Agenda item:		9.16.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for expanded and improved NR positioning

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1]. In RAN1#113 an extended set of topics will be included, where rapporteur’s input on UE features for each WI will be presented for discussion as well. Hence, in this contribution we provide our initial views on UE features for expanded and improved NR positioning WI [2].

# Discussion

RAN1 related work on the WI can be divided to several sub-categories based on the WI objectives and sub-objectives:

- Sidelink positioning

  - SL PRS RSRP measurement and reporting

  - SL PRS RSRPP measurement and reporting

  - SL PRS based RTOA measurement and reporting

  - SL PRS based RSTD measurement and reporting

  - SL PRS based AOA/ZOA measurement and reporting

  - SL PRS based Rx-Tx time difference measurement and reporting

  - [SL PRS triggering, too early to say if this is a capability or a basic component]

  - [SL PRS resource allocation, too early to say if this is a capability or a basic component]

- NR DL and UL carrier phase positioning

  - UE-based DL carrier phase positioning support

  - UE-assisted DL carrier phase positioning support

  - Support for LMF request for DL PRS measurements

  - PRU support

- LPHAP (Low Power High Accuracy Positioning)

  - Support for SRS validity area

- Bandwidth aggregation for positioning measurements

  - Support for PRS based measurements with bandwidth-aggregated PRS for DL PRS RSTD measurement

  - Support for PRS based measurements with bandwidth-aggregated PRS for UE Rx-Tx time difference measurement

  - Support for SRS bandwidth aggregation in RRC CONNECTED and INACTIVE

- Positioning for RedCap UEs

  - Support for PRS with Rx frequency hopping with measurement gaps

  - Support for SRS with Tx frequency hopping with switching gaps

Similarly to previous releases, this is expected to result in families of rather independent FGs, without strong relationship among them, with pre-requisites defined case-by-case. One basic structure for upcoming discussions is as presented in Table 1 below, which follows a similar structure as that for Rel-16 positioning WI.

Table 1: Initial structure for UE features for Rel-18 positioning enhancements

| Index | Feature group | Components | Notes |
| --- | --- | --- | --- |
| 41-1-1 | Common SL PRS Processing Capability | - Maximum SL PRS bandwidth - FFS others | Basic FG for positioning techniques based on SL PRS measurements |
| 41-1-1a | SL PRS transmission capability | FFS | Basic FG for positioning techniques based on SL PRS measurements FFS if different FGs needed for mode 1 and mode 2 |
| 41-1-2 | SL PRS measurement report for RTOA | FFS | Pre-requisite: FG 41-1-1 |
| 41-1-3 | SL PRS measurement report for RSTD | FFS | Pre-requisite: FG 41-1-1 |
| 41-1-4 | SL PRS measurement report for AoD/ZoA | FFS | Pre-requisite: FG 41-1-1 |
| 41-1-5 | SL PRS measurement report for Rx-Tx time difference | FFS | Pre-requisite: FG 41-1-1 |
| 41-1-6 | SL PRS RSRP measurement and reporting | FFS | Pre-requisite: FG 41-1-1 |
| 41-1-7 | SL PRS RSRPP measurement and reporting | FFS | Pre-requisite: FG 41-1-1 |
| 41-2-1 | UE-based DL carrier phase positioning support with RSTD | FFS | Pre-requisite: FG 13-6 |
| 41-2-1 | UE-based DL carrier phase positioning support with UE Rx-Tx time difference | FFS | Pre-requisite: FG 13-11 |
| 41-2-3 | UE-assisted DL carrier phase positioning support | FFS |  |
| 41-2-6 | Support for LMF request for DL PRS measurements | - Support of RSCPD request with RSTD - Support of RSCP request with UE Rx-Tx time difference measurement |  |
| 41-2-7 | PRU UE | - Support of reporting UE location. - Support of different measurements: {RSCP, RSCPD, RSTD, UE Rx-Tx} - FFS: Support of SRS for positioning transmission? |  |
| 41-3-1 | Support for SRS validity area for LPHAP | - Support of spatial relation information - Support of path-loss reference RS - Support of validity area-specific TA timer |  |
| 41-4-1 | Support for PRS based measuremtents with bandwidth-aggregated PRS | - Support of simultaneous reception of multiple PRS resources across multiple PFLs - Support of the maximum number of PFLs to be aggregated - [ Support of additional (N,T) ] |  |
| 41-4-2 | Support for SRS bandwidth aggregation in RRC CONNECTED state | - Support the maximum number of CCs to guarantee the phase coherency for the simultaneous transmission of SRS resources. |  |
| 41-4-2 | Support for SRS bandwidth aggregation in RRC INACTIVE state | - Support the maximum number of CCs to guarantee the phase coherency for the simultaneous transmission of SRS resources. | Pre-requisite: FG 27-15 |
| 41-5-1 | Support for PRS with Rx frequency hopping with measurement gaps | - the different types of measurements that are supported via Rx hopping. - Switching times FR1: {70, 140} us, FR2: {35, 70, 140} us [NOTE this is from RAN4 LS] 3. Frequency overlap between hops. Candidate values {1 PRB, 4 PRBs} | Pre-requisite: FG 13-1 (Common DL PRS Processing Capability), one_of {28-1 (Rel-17 RedCap), 48-x ( Rel-18 RedCap PR1), 48-y (Rel-18 RedCap PR1 + BW3)} |
| 41-5-2 | Support for SRS with Tx frequency hopping with switching gaps | - Switching times FR1: {70, 140} us, FR2: {35, 70, 140} us [NOTE this is from RAN4 LS] - Frequency overlap between hops. Candidate values {0, 1 PRB, 4 PRBs} - Support of SRS configuration separate from UL BWP | Pre-requisite: FG 13-8 (SRS Resources for Positioning), one_of {28-1 (Rel-17 RedCap), 48-x ( Rel-18 RedCap PR1), 48-y (Rel-18 RedCap PR1 + BW3)} |

Proposal: Consider table 1 above as baseline for UE FG definitions for Rel-18 positioning WI.

# Conclusion

In this contribution we have provided our initial views on UE features for expanded and improved NR positioning WI, and we have made the following proposal:

Proposal: Consider table 1 above as baseline for UE FG definitions for Rel-18 positioning WI.

# References

- R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation.