3GPP TSG RAN WG1 #113	R1- 2305740

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Sharp

Title:	Views on 8 TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

Background

In RAN plenary #94e, the work item on MIMO Evolution for Downlink and Uplink was approved [1]. According to the WID, the following work item needs to be studied, and if justified, specified.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we share our views on 8TX UL transmission.

Discussions

  - Codebook design for partial/non-coherent

For codebook design of partial/non-coherent UE, the following working assumption and agreement were achieved.

Agreement (#112) For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, Partial-coherent precoders are used Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, Alt1 is supported where - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Further study codebook size reduction

From the agreements in #112 and #112b-e meeting, the codebook design corresponding to partial coherent can be described as follows.

Observation 1: From the previous discussion, the codebook design corresponding to partial coherent is described as follows.

- Ng = 2

  - W= A104×rank(A2)04×rank(A1)A28×rank

    - A1 and A2 are precoding matrices taken from Rel-15 4TX UL codebook

    - A1 and A2 correspond to only FC precoder.

- Ng = 4

  - W= A102×rank(A2)02×rank(A3)02×rank(A4)02×rank(A1)A202×rank(A3)02×rank(A4)02×rank(A1)02×rank(A2)A302×rank(A4)02×rank(A1)02×rank(A2)02×rank(A3)A48×rank

    - A1, A2, A3 and A4 are precoding matrices taken from Rel-15 2TX UL codebook

    - A1, A2, A3 and A4 correspond to only FC precoder.

For layer splitting, the following agreements were achieved.

Agreement (#112) For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated. Working Assumption (#112b-e) For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3).

We need to consider the TPMI overhead to determine the supported layer splitting of the codebook design for partial coherent. Through the previous meeting, there is common understanding that need to reduce the TPMI overhead. The overhead become an issue especially for the partial-coherent codebook. In the case as the partial-coherent precoding structure consist of two precoding matrices taken from Rel-15 4TX UL codebook (A1, A2 described as observation 1), it needs as two times as the conventional TPMI overhead. As the codebook Subset is same as existing configuration (E.g. codebookSubset=fullyAndPartialAndNonCoherent), the number of PMI candidates become 62 for each 4TX UL codebook. Considered when the rank(A1) or the rank(A2) is 0, the total number of TPMI is 63*63-1 = 3968 for 8TX partial-coherent codebook. This TPMI indication needs 12 bits.

Table 1: Number of PMI candidate of 4TX UL codebook for each coherence type

|  | FC | PC | NC | FCPCNC |
| --- | --- | --- | --- | --- |
| Rank 1 | 16 | 8 | 4 | 28 |
| Rank 2 | 8 | 8 | 6 | 22 |
| Rank 3 | 4 | 2 | 1 | 7 |
| Rank 4 | 2 | 2 | 1 | 5 |
| sum | 30 | 20 | 12 | 62 |

Table 2: Number of PMI candidate of 2TX UL codebook for each coherence type

|  | FC | NC(PC) | FCPCNC |
| --- | --- | --- | --- |
| Rank 1 | 4 | 2 | 6 |
| Rank 2 | 2 | 1 | 3 |
| sum | 6 | 3 | 9 |

On the other hand, if codebookSubset is separated by one coherent capability, the total number of TPMI is 31*31-1 = 960, which needs 10 bits. In other words, 2 bits of overhead can be reduced. Therefore, the codebookSubset should be separated so that the codebookSubset contains only one coherent capability.

Proposal 1: The codebookSubset should be separated by one coherent capability.

Also, when 8TX UL codebook consists of four 2TX UL codebooks for Ng=4, the total number of TPMI is 10*10*10*10-1 = 9999, which needs 14 bits. Even if only TPMIs corresponding to fully-coherent were included, the total number of TPMI would be as large as 7*7*7*7-1 = 2400 which needs 12bits.

For L1 = rank(A1) and L2 = rank(A2), the PMI candidates for each rank combination is shown in the tables below. Since the number of candidates becomes large when all rank combinations are considered, it is expected to reduce the number of PMI candidates by supporting only some combinations. That is the intention for layer splitting.

Table 3: Number of PMI candidates of 4TX UL codebook for each rank combination corresponding to FC precoder

| maxRank | L1 | L2 |  | N(A1) | N(A2) | N(A1)*N(A2) | sum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  | 16 | 1 | 16 | 32 |
|  |  | 1 |  | 1 | 16 | 16 |  |
| 2 | 2 |  |  | 8 | 1 | 8 | 272 |
|  | 1 | 1 |  | 16 | 16 | 256 |  |
|  |  | 2 |  | 1 | 8 | 8 |  |
| 3 | 3 |  |  | 4 | 1 | 4 | 264 |
|  | 2 | 1 |  | 8 | 16 | 128 |  |
|  | 1 | 2 |  | 16 | 8 | 128 |  |
|  |  | 3 |  | 1 | 4 | 4 |  |
| 4 | 4 |  |  | 2 | 1 | 2 | 196 |
|  | 3 | 1 |  | 4 | 16 | 64 |  |
|  | 2 | 2 |  | 8 | 8 | 64 |  |
|  | 1 | 3 |  | 16 | 4 | 64 |  |
|  |  | 4 |  | 1 | 2 | 2 |  |
| 5 | 4 | 1 |  | 2 | 16 | 32 | 128 |
|  | 3 | 2 |  | 4 | 8 | 32 |  |
|  | 2 | 3 |  | 8 | 4 | 32 |  |
|  | 1 | 4 |  | 16 | 2 | 32 |  |
| 6 | 4 | 2 |  | 2 | 8 | 16 | 48 |
|  | 3 | 3 |  | 4 | 4 | 16 |  |
|  | 2 | 4 |  | 8 | 2 | 16 |  |
| 7 | 4 | 3 |  | 2 | 4 | 8 | 16 |
|  | 3 | 4 |  | 4 | 2 | 8 |  |
| 8 | 4 | 4 |  | 2 | 2 | 4 | 4 |
|  |  |  |  |  |  | total | 960 |

Table 4: Number of PMI candidates of 2TX UL codebook for each rank combination corresponding to FC precoder

| maxRank | L1 | L2 | L3 | L4 |  | N(A1) | N(A2) | N(A3) | N(A4) | N(A1)* N(A2)* N(A3)* N(A4) | sum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  |  | 4 | 1 | 1 | 1 | 4 | 16 |
|  |  | 1 |  |  |  | 1 | 4 | 1 | 1 | 4 |  |
|  |  |  | 1 |  |  | 1 | 1 | 4 | 1 | 4 |  |
|  |  |  |  | 1 |  | 1 | 1 | 1 | 4 | 4 |  |
| 2 | 2 |  |  |  |  | 2 | 1 | 1 | 1 | 2 | 104 |
|  |  | 2 |  |  |  | 1 | 2 | 1 | 1 | 2 |  |
|  |  |  | 2 |  |  | 1 | 1 | 2 | 1 | 2 |  |
|  |  |  |  | 2 |  | 1 | 1 | 1 | 2 | 2 |  |
|  | 1 | 1 |  |  |  | 4 | 4 | 1 | 1 | 16 |  |
|  | 1 |  | 1 |  |  | 4 | 1 | 4 | 1 | 16 |  |
|  | 1 |  |  | 1 |  | 4 | 1 | 1 | 4 | 16 |  |
|  |  | 1 | 1 |  |  | 1 | 4 | 4 | 1 | 16 |  |
|  |  | 1 |  | 1 |  | 1 | 4 | 1 | 4 | 16 |  |
|  |  |  | 1 | 1 |  | 1 | 1 | 4 | 4 | 16 |  |
| 3 | 2 | 1 |  |  |  | 2 | 4 | 1 | 1 | 8 | 352 |
|  | 2 |  | 1 |  |  | 2 | 1 | 4 | 1 | 8 |  |
|  | 2 |  |  | 1 |  | 2 | 1 | 1 | 4 | 8 |  |
|  | 1 | 2 |  |  |  | 4 | 2 | 1 | 1 | 8 |  |
|  |  | 2 | 1 |  |  | 1 | 2 | 4 | 1 | 8 |  |
|  |  | 2 |  | 1 |  | 1 | 2 | 1 | 4 | 8 |  |
|  | 1 |  | 2 |  |  | 4 | 1 | 2 | 1 | 8 |  |
|  |  | 1 | 2 |  |  | 1 | 4 | 2 | 1 | 8 |  |
|  |  |  | 2 | 1 |  | 1 | 1 | 2 | 4 | 8 |  |
|  | 1 |  |  | 2 |  | 4 | 1 | 1 | 2 | 8 |  |
|  |  | 1 |  | 2 |  | 1 | 4 | 1 | 2 | 8 |  |
|  |  |  | 1 | 2 |  | 1 | 1 | 4 | 2 | 8 |  |
|  | 1 | 1 | 1 |  |  | 4 | 4 | 4 | 1 | 64 |  |
|  | 1 | 1 |  | 1 |  | 4 | 4 | 1 | 4 | 64 |  |
|  | 1 |  | 1 | 1 |  | 4 | 1 | 4 | 4 | 64 |  |
|  |  | 1 | 1 | 1 |  | 1 | 4 | 4 | 4 | 64 |  |
| 4 | 2 | 2 |  |  |  | 2 | 2 | 1 | 1 | 4 | 664 |
|  | 2 |  | 2 |  |  | 2 | 1 | 2 | 1 | 4 |  |
|  | 2 |  |  | 2 |  | 2 | 1 | 1 | 2 | 4 |  |
|  |  | 2 | 2 |  |  | 1 | 2 | 2 | 1 | 4 |  |
|  |  | 2 |  | 2 |  | 1 | 2 | 1 | 2 | 4 |  |
|  |  |  | 2 | 2 |  | 1 | 1 | 2 | 2 | 4 |  |
|  | 2 | 1 | 1 |  |  | 2 | 4 | 4 | 1 | 32 |  |
|  | 2 | 1 |  | 1 |  | 2 | 4 | 1 | 4 | 32 |  |
|  | 2 |  | 1 | 1 |  | 2 | 1 | 4 | 4 | 32 |  |
|  | 1 | 2 | 1 |  |  | 4 | 2 | 4 | 1 | 32 |  |
|  | 1 | 2 |  | 1 |  | 4 | 2 | 1 | 4 | 32 |  |
|  |  | 2 | 1 | 1 |  | 1 | 2 | 4 | 4 | 32 |  |
|  | 1 | 1 | 2 |  |  | 4 | 4 | 2 | 1 | 32 |  |
|  | 1 |  | 2 | 1 |  | 4 | 1 | 2 | 4 | 32 |  |
|  |  | 1 | 2 | 1 |  | 1 | 4 | 2 | 4 | 32 |  |
|  | 1 | 1 |  | 2 |  | 4 | 4 | 1 | 2 | 32 |  |
|  | 1 |  | 1 | 2 |  | 4 | 1 | 4 | 2 | 32 |  |
|  |  | 1 | 1 | 2 |  | 1 | 4 | 4 | 2 | 32 |  |
|  | 1 | 1 | 1 | 1 |  | 4 | 4 | 4 | 4 | 256 |  |
| 5 | 2 | 2 | 1 |  |  | 2 | 2 | 4 | 1 | 16 | 704 |
|  | 2 | 2 |  | 1 |  | 2 | 2 | 1 | 4 | 16 |  |
|  | 2 | 1 | 2 |  |  | 2 | 4 | 2 | 1 | 16 |  |
|  | 2 |  | 2 | 1 |  | 2 | 1 | 2 | 4 | 16 |  |
|  | 2 | 1 |  | 2 |  | 2 | 4 | 1 | 2 | 16 |  |
|  | 2 |  | 1 | 2 |  | 2 | 1 | 4 | 2 | 16 |  |
|  | 1 | 2 | 2 |  |  | 4 | 2 | 2 | 1 | 16 |  |
|  |  | 2 | 2 | 1 |  | 1 | 2 | 2 | 4 | 16 |  |
|  | 1 | 2 |  | 2 |  | 4 | 2 | 1 | 2 | 16 |  |
|  |  | 2 | 1 | 2 |  | 1 | 2 | 4 | 2 | 16 |  |
|  | 1 |  | 2 | 2 |  | 4 | 1 | 2 | 2 | 16 |  |
|  |  | 1 | 2 | 2 |  | 1 | 4 | 2 | 2 | 16 |  |
|  | 2 | 1 | 1 | 1 |  | 2 | 4 | 4 | 4 | 128 |  |
|  | 1 | 2 | 1 | 1 |  | 4 | 2 | 4 | 4 | 128 |  |
|  | 1 | 1 | 2 | 1 |  | 4 | 4 | 2 | 4 | 128 |  |
|  | 1 | 1 | 1 | 2 |  | 4 | 4 | 4 | 2 | 128 |  |
| 6 | 2 | 2 | 2 |  |  | 2 | 2 | 2 | 1 | 8 | 416 |
|  | 2 | 2 |  | 2 |  | 2 | 2 | 1 | 2 | 8 |  |
|  | 2 |  | 2 | 2 |  | 2 | 1 | 2 | 2 | 8 |  |
|  |  | 2 | 2 | 2 |  | 1 | 2 | 2 | 2 | 8 |  |
|  | 2 | 2 | 1 | 1 |  | 2 | 2 | 4 | 4 | 64 |  |
|  | 2 | 1 | 2 | 1 |  | 2 | 4 | 2 | 4 | 64 |  |
|  | 2 | 1 | 1 | 2 |  | 2 | 4 | 4 | 2 | 64 |  |
|  | 1 | 2 | 2 | 1 |  | 4 | 2 | 2 | 4 | 64 |  |
|  | 1 | 2 | 1 | 2 |  | 4 | 2 | 4 | 2 | 64 |  |
|  | 1 | 1 | 2 | 2 |  | 4 | 4 | 2 | 2 | 64 |  |
| 7 | 2 | 2 | 2 | 1 |  | 2 | 2 | 2 | 4 | 32 | 128 |
|  | 2 | 2 | 1 | 2 |  | 2 | 2 | 4 | 2 | 32 |  |
|  | 2 | 1 | 2 | 2 |  | 2 | 4 | 2 | 2 | 32 |  |
|  | 1 | 2 | 2 | 2 |  | 4 | 2 | 2 | 2 | 32 |  |
| 8 | 2 | 2 | 2 | 2 |  | 2 | 2 | 2 | 2 | 16 | 16 |
|  |  |  |  |  |  |  |  |  |  | total | 2400 |

From Table 3, the number of PMI candidates when all combinations are supported is 960, and the number of PMI candidates when Ng=2 corresponding to the working assumption in the previous meeting is 736. We don’t support the working assumption because the number of required bits remains the same even if layer combinations are restricted.

Proposal 2: We don’t support the working assumption for the layer splitting.

Proposal 3: Support the all layer combination in case of Ng = 2.

For the Ng=4 case, it is necessary to eliminate at least 2400-2048=352 PMI candidates to reduce the number by 1 bit. without performance degradation. Basically, the easiest way is to support all combinations, but it is possible to discuss ways to reduce the number of bits without degrading performance.

Observation 2: For the Ng=4 case, it is necessary to eliminate at least 352 PMI candidates to reduce the number by 1 bit.

For the codebook cofiguration, the following agreement was achieved.

Agreement (112b-e) For codebook -based 8TX PUSCH transmission, down-select from, - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE, with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE, with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE, Ng =8, can only be configured with precoders considered for Ng = 8 - Alt2 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE, with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE, with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8 - FFS which Ng value(s), to be considered - A non-coherent UE, with Ng =8, can only be configured with precoders considered for Ng = 8 - FFS whether/how the configuration can be done via RRC or MAC-CE. - Alt3 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1 - A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2 - A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4 - A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8 - Other alternatives are not precluded Note: For an 8TX UE, Ng =8 can represent a non-coherent UE.

From the above discussion, it is assumed that the codebook size corresponding to a partial coherent UE will be larger (may require 10~12bit). The conventional nested codebookSubset is undesirable because it requires a huge number of bits. In other words, as mentioned in Proposal 1, separated codebookSubset should be used, and we support Alt3.

Proposal 4: Support the Alt3 for the codebook configuration.

- Alt3

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1

  - A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2

  - A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4

  - A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8

  - Full power operation

For full power operation, the following agreements were achieved.

Agreement Framework for full power PUSCH transmission by an 8TX UE - To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used. - FFS if any change is required in the specifications. - Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used per rank. - Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration Agreement (#112) For CB-based 8TX PUSCH transmission, where Mode 2 uplink full power transmission (if supported) is not used, re-use legacy Rel-15 mechanism, that is - when only one SRS resource in a resource set is configured, the SRI field in DCI is absent, - when two SRS resources are configured in a resource set, 1 bit of SRI field in DCI is used to indicate the selected SRS resource in the set. Agreement (#112) Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder, - Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3 - For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112. - This can be used for other UE Power Classes as well. 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA. Regular UE; P1=P2= …=P8=14 dBm (Full power supported by Mode1). Full-power capable UE; Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm. Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm. (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm. (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm. (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm.

Since fullpowermode 1 can be specified by inserting a full-coherent precoder into the free bit index of a partial-coherent precoder corresponding to Ng=2/4, the working assumption regarding mode 1 is supported.

Proposal 5: Support the full power transmission Mode 1.

As for fullpowermode 2, it seems difficult to discuss precoder groups before the codebook design specification is completed. Therefore, we support leaving the working assumption as it is.

Proposal 6: Discussion on full power transmission Mode 2 should be postponed until the codebook design specification is completed.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: The codebookSubset should be separated by one coherent capability.

Proposal 2: We don’t support the working assumption for the layer splitting.

Proposal 3: Support the all layer combination in case of Ng = 2.

Proposal 4: Support the Alt3 for the codebook configuration.

- Alt3

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1

  - A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2

  - A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4

  - A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8

Proposal 5: Support the full power transmission Mode 1.

Proposal 6: Discussion on full power transmission Mode 2 should be postponed until the codebook design specification is completed.

Observation 1: From the previous discussion, the codebook design corresponding to partial coherent is described as follows.

- Ng = 2

  - W= A104×rank(A2)04×rank(A1)A28×rank

    - A1 and A2 are precoding matrices taken from Rel-15 4TX UL codebook

    - A1 and A2 correspond to only FC precoder.

- Ng = 4

  - W= A102×rank(A2)02×rank(A3)02×rank(A4)02×rank(A1)A202×rank(A3)02×rank(A4)02×rank(A1)02×rank(A2)A302×rank(A4)02×rank(A1)02×rank(A2)02×rank(A3)A48×rank

    - A1, A2, A3 and A4 are precoding matrices taken from Rel-15 2TX UL codebook

    - A1, A2, A3 and A4 correspond to only FC precoder.

Observation 2: For the Ng=4 case, it is necessary to eliminate at least 352 PMI candidates to reduce the number by 1 bit.

References

RAN Meeting#94, RP-213598, New WID: MIMO Evolution for Downlink and Uplink

RAN1 Meeting#112bis-e, Chair's notes RAN1#112bis-e.