3GPP TSG RAN WG1 Meeting #113		R1-2305774

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:       9.1.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	UL enhancements for enabling 8Tx UL transmission

Document for:     Discussion and Decision

# 1	Introduction

In RAN#94-e two new objectives were agreed related for UL MIMO enhancements in Rel-18 NR [1], to address SRI/TPMI enhancements for enabling 8Tx UL transmission:

5. Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

Good progress on uplink 8Tx have been made in recent RAN1 meetings with agreements on fully/partially coherent codebook design, dual CW support, TRI and TPMI, and full Tx power feature. We are providing further discussions on remaining issues: 8Tx UL codebook design for both full coherent and partial coherent codebook design, full Tx power support, and multi-layer support for 8Tx transmission.

# 2	UL 8Tx Codebook Design

Based on the WID, “8Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices” is under the Rel-18 study, and if justified, for specifying. Since there is no 8Tx UL codebook in NR, we shall consider introducing 8Tx codebook for NR uplink.

## 2.1	Full coherent NR 8Tx codebook

There is an agreement in RAN1 #111 meeting on full coherent 8Tx precoding, following discussions of several RAN1 meetings. DL Type I codebook is agreed as the starting point to design the full coherent 8Tx codebook for uplink:

Agreement For a fully coherent uplink precoding by an 8TX UE, - Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a. - No LS to RAN4 will be needed

Given that the DL Type I codebook is agreed, let’s review Type I codebook with emphasis on its design parameters.

### 2.1.1	DL Type I codebook

DL type-I codebook is an example CB of the general 2-stage codebook. This is quite different from Rel-15 uplink 2Tx and 4Tx codebooks, which are single stage codebook.

Given the precoding is defined with

y=Wx,

where precoded signal y is a P×1 vector with P antenna ports, x is a rank-v data symbol vector with size of v×1, and the matrix W of size P×v is the precoder related with P antenna ports and rank v. For Rel-15 NR, uplink codebooks are single-stage codebooks which define codewords W. While for the two-stage codebook design in Type-I codebook, the precoder W has the form

W=W1W2

where W1 contains the wideband component of the precoder and W2 usually contains the sub-band specific components of the precoder. For wideband only support, we may pick single W2 for all sub-bands because there is no agreement to support frequency selective precoding in uplink.

Type-I codebook design has some general assumptions:

- The codebook design is based on 2-D DFT beam grid with oversampling.

- Each 2-D DFT beam in the type-I codebook has two components, related with azimuth and elevation direction of the 2-D antenna array.

- Cross-polarization antennas are assumed.

With the 2-D DFT beam grid, Type-I single beam codebook with rank 1 has its choice for W1 and W2 as

W1(1)i11,i12=vi11,i1200vi11,i12

W2(1)i2=1P1φ1(1)i2

where φ1(1)∈1,j,-1,-j, and vi11,i12 is defined as a DFT matrix. Note that a single beam is selected at wideband, and co-phasing across polarizations is performed per subband.

One 2-D DFT beam has an azimuth component and an elevation component, as

tl=1, ej2πlO1N1, ⋯, ej2πlN1-1O1N1T

um=1, ej2πmO2N2, ⋯, ej2πmN2-1O2N2T

Each 2-D beam is defined as

vl,m=tl⊗um

The antenna port configuration per polarization is given with (N1, N2), and oversampling factors O1 and O2 for two components, respectively. For DL Type-I codebook, these configurations are used:

Table 1    DL Type-I codebook configuration: 4Tx and 8Tx

| # of CSI-RS Ports | (N1,N2) | (O1,O2) |
| --- | --- | --- |
| 4 | (2, 1) | (4, 1) |
| 8 | (2, 2) | (4, 4) |
|  | (4, 1) | (4, 1) |

### 2.1.2	Full Coherent Uplink 8Tx Codebook Design

There are two (N1,N2) antenna configurations for Type-I 8Tx codebook, as (2, 2) and (4, 1). The configuration of (2, 2) provides better elevation capability. These two configurations, (2, 2) and (4, 1) are considered as Case 1 of UE antenna layouts, based on an agreement in RAN1 #109-e. RAN1 #112 have these agreements on the oversampling factors (O1, O2), and (N1, N2)

Agreement Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2) A pair of (N1, N2) can be configured with subject to UE capability.

The precoders of 8Tx Type-I codebook depends on codebook indices i1,1, i1,2, i2 for layers {1, 5, 6, 7, 8}, and codebook indices i1,1, i1,2, i1,3, i2 for layers {2, 3, 4}. Besides, there are two sets of Type-I codebook: single beam L=1 (codebookMode=1) and multiple beam L=4 codebook (codebookMode=2).

With OVS O1=1 and O2=1, we list all possible numbers of precoders for codebookMode=1, covering rank 1 to rank 8 in Table 2.

Table 2    List of numbers of precoders for L=1 type-I CB, with O1=1, O2=1, codebookMode=1

| Rank | i1,1 | i1,2 | i1,3 | i2 | # of precoders with N1,N2=(4,1) | # of precoders with (N1, N2)=(2,2) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0, 1, ⋯, N1-1 | 0, 1, ⋯, N2-1 | - | 0, 1, 2, 3 | 16 | 16 |
| 2 | 0, 1, ⋯, N1-1 | 0, 1, ⋯, N2-1 | 0, 1, 2, 3 | 0, 1 | 32 | 32 |
| 3 | 0, 1, ⋯, N1-1 | 0, 1, ⋯, N2-1 | 0, 1, 2 | 0, 1 | 24 | 24 |
| 4 | 0, 1, ⋯, N1-1 | 0, 1, ⋯, N2-1 | 0, 1, 2 | 0, 1 | 24 | 24 |
| 5 | 0, 1, ⋯, N1-1 | 0, 1, ⋯, N2-1 | - | 0, 1 | 8 | 8 |
| 6 | 0, 1, ⋯, N1-1 | 0, 1, ⋯, N2-1 | - | 0, 1 | 8 | 8 |
| 7 | 0, 1 | 0, 1, ⋯, N2-1 | - | 0, 1 | 4 | 8 |
| 8 | 0, 1 | 0, 1, ⋯, N2-1 | - | 0, 1 | 4 | 8 |

The two antenna configurations (N1,N2)=(4, 1) and (N1, N2)=(2,2) are included in the table.

For the Type-I CB with codebookMode=2, rank 1 and rank 2 precoders are different from codebookMode=1 cases. There are more precoders of codebookMode=2 than that of codebookMode=1. The numbers are listed in Table 3:

Table 3    List of numbers of Rank-1 and Rank-2 precoders, O1=1, O2=1, codebookMode=2

| (N1, N2) | Rank | i1,1 | i1,2 | i1,3 | i2 | # of precoders |
| --- | --- | --- | --- | --- | --- | --- |
| (4, 1) | 1 | 0, 1 | 0 | - | 0, 1, …, 15 | 32 |
|  | 2 | 0, 1 | 0 | 0, 1, 2, 3 | 0, 1, …, 7 | 64 |
| (2, 2) | 1 | 0, 1 | 0, 1 | - | 0, 1, …, 15 | 64 |
|  | 2 | 0, 1 | 0, 1 | 0, 1, 2, 3 | 0, 1, …, 7 | 128 |

Also the two antenna configurations are included in the table.

The total number of precoders with type-I CB can be summarized in Table 4:

Table 4    Summary of number of precoders for two modes of Type-I CB

| (N1, N2) | # of precoders (codebookMode = 1) | # of precoders (codebookMode = 2) |
| --- | --- | --- |
| (4, 1) | 120 | 168 |
| (2, 2) | 128 | 272 |

As expected in type-I CB design, there are more precoders with the multi-beam design than that of single beam CB design for rank-1 and rank-2 Tx. Using the multi-beam design would need higher overhead than that of single-beam type-I codebook design. The extra cost would be an increasing of 1 bit or 2 bits of TPMI overhead. Based on DL type-I codebook design principle, mode 1 selects one beam which can vary the cophasing in each subband.  Mode 2 selects a group of four beams which one of those four is chosen in each subband in addition to varying the cophasing across polarizations.  This additional freedom is designed to provide improved performance. Given that uplink 8Tx CB is based on wideband, it is not clear how much benefit of using Mode 2 CB when compared with Mode 1.

Proposal 1: Considering feedback overhead for 8Tx uplink, support Type-I codebookMode=1 CB as fully coherent codebook.

### 2.1.3	OVS and System Simulation Results

RAN1 #112b-e meeting reached this conclusion on OVS ratio for fully-coherent CB design:

Conclusion For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), there is no consensus to support any optional over-sampling ratio.

There are significant gains of using (O1, O2)=(4, 1) for (N1,N2)=(4, 1), and (O1, O2)=(4, 4) for (N1, N2) = (2, 2), as indicated in the system-level simulations shown below, compared to (1,1) OVS. The cost of introduction of (O1, O2)=(4,1) and (O1, O2)=(4,4) would be 2-extra bits and 4-extra bits, compared to (1, 1) OVS. With our system-level simulation results, we have this proposal:

Proposal 2: For (N1, N2) = (4,1), consider supporting at least (O1, O2) = (4,1) based on UE capability.  For (N1, N2) = (2,2), consider supporting at least (O1, O2) = (4,4) based on UE capability.

Following the Type I based table, it can be observed that (O1, O2) has direct impact on the total number of precoders. Given that the 8Tx precoding is based on wideband, there is no sub-band components in the TPMI for Type-I codebook design. TPMI signaling overhead will be determined by an indication of wideband precoder. By inspecting Table 2 for UL precoding overhead, it can be observed that 1-extra bit is needed to support (O1, O2) = (2, 1) from (O1, O2) = (1, 1), 2-extra bits are needed to support (O1, O2) = (4, 1) from (O1, O2) = (1, 1), and 4-extra bits are needed to support (O1,O2) = (4, 4) from (O1, O2) = (1, 1):

Observation 1: To support (O1, O2)=(2, 1), 1-extra bit is needed in TPMI, compared to (O1, O2)=(1, 1).

Observation 2: To support (O1, O2)=(4, 1), 2-extra bits are needed in TPMI, compared to (O1, O2)=(1, 1).

Observation 3: To support (O1, O2)=(4, 4), 4-extra bits are needed in TPMI, compared to (O1, O2)=(1, 1).

Table 5 – UL Precoding overhead (codebookModel=1, Type-I CB)

| Configuration parameters | Number of precoders using NR Rel-15 single panel DL Type I |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (N1, N2, O1, O2) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total | Max TPMI signalling overhead (bits) |
| (2, 2, 1, 1) | 16 | 32 | 24 | 24 | 8 | 8 | 8 | 8 | 128 | 7 |
| (2, 2, 2, 1) | 32 | 64 | 48 | 48 | 16 | 16 | 16 | 16 | 256 | 8 |
| (2, 2, 2, 2) | 64 | 128 | 96 | 96 | 32 | 32 | 32 | 32 | 512 | 9 |
| (2, 2, 4, 1) | 64 | 128 | 96 | 96 | 32 | 32 | 32 | 32 | 512 | 9 |
| (2, 2, 4, 2) | 128 | 256 | 192 | 192 | 64 | 64 | 64 | 64 | 1024 | 10 |
| (2, 2, 4, 4) | 256 | 512 | 384 | 384 | 128 | 128 | 128 | 128 | 2048 | 11 |
| (4, 1, 1, 1) | 16 | 32 | 24 | 24 | 8 | 8 | 4 | 4 | 120 | 7 |
| (4, 1, 2, 1) | 32 | 64 | 48 | 48 | 16 | 16 | 8 | 8 | 240 | 8 |
| (4, 1, 2, 2) | 64 | 128 | 96 | 96 | 16 | 16 | 8 | 8 | 432 | 9 |
| (4, 1, 4, 1) | 64 | 128 | 96 | 96 | 32 | 32 | 16 | 16 | 480 | 9 |
| (4, 1, 4, 2) | 128 | 256 | 192 | 192 | 32 | 32 | 16 | 16 | 864 | 10 |
| (4, 1, 4, 4) | 256 | 512 | 384 | 384 | 32 | 32 | 16 | 16 | 1632 | 11 |

From another side, if a fewer precoders are used, potential performance degradation is expected. The question is whether the performance degradation is justified with a potential few bits saving in TPMI feedback. We shall conduct system-level simulations to evaluate the system-level impact of using various OVS.

#### 2.1.3.1 Fixed Rank Scheduling:

System level simulations were generated for both the Outdoor FWA and Indoor FWA scenarios with full buffer traffic to evaluate the performance of the different choices for (O1, O2) for when the rank is fixed at a specific value for all scheduled UEs. For the Outdoor FWA scenario, Figure 1 shows the relative sector spectral efficiencies for the UE configuration (N1, N2) = (4,1) and three combinations of O1, O2: (O1, O2)=(1,1), (2, 1) and (4,1).  In Figure 1, the left two plots are for ranks 1 through 4, whereas the right two plots are for ranks 5 through 8.  The top two plots show the Mean UE spectral efficiency normalized to the largest value across both plots. The bottom two plots show the edge UE spectral efficiency also normalized to the largest value across both plots.  Figure 2 shows the corresponding plots for the Indoor FWA scenario.  For both Outdoor FWA and Indoor FWA,  the (O1, O2) = (4,1) configuration provides the highest performance for all ranks as expected.  However, the different combinations of (O1, O2) perform differently depending on the fixed rank value.  For ranks 1 through 4, there is relatively little difference between the three combinations of (O1, O2).  However, for ranks 5 through 8, there are severe losses in going from (4,1) to (2,1) and also from (2,1) to (1,1).

Figures 3 and 4 show the relative sector spectral efficiencies for the UE configuration (N1, N2) = (2,2) and six combinations of O1, O2: (O1, O2) = (1,1), (2, 1), (2,2), (4,1), (4,2) and (4,4).  Figure 3 is for the Outdoor FWA scenario, and Figure 4 is for the Indoor FWA scenario.  Figures 3 and 4 are arranged similarly to Figures 1 and 2: the left plots show ranks 1 through 4, the right plots show ranks 5 through 8, top two plots show the mean UE spectral efficiency normalized to the largest value across both plots, and the bottom two plots show the edge UE spectral efficiency also normalized to the largest value across both plots.  Similar trends are seen as with the (N1, N2) = (4,1) case:  For all ranks, the (O1, O2) = (4,4) configuration provides the highest performance as expected, and the magnitude of the degradation with the other configurations depends on the value of the fixed rank.  For ranks 1 through 4, there is relatively little difference between the six combinations of (O1, O2).  For ranks 5 through 8, there are severe losses in going from (4,4) to any of the other configurations.

Observation 4: for both Outdoor FWA and Indoor FWA and for both (N1, N2) = (2,2) and (N1, N2) = (4,1) with fixed rank selection and full buffer traffic:  for ranks fixed at 1, 2, 3 or 4, there is little performance difference between the different choices of (O1, O2).  For ranks fixed at values higher than 4, the performance difference between the highest overhead configuration and the other configurations can be moderate to severe.

Figure 1: Spectral Efficiency for (N1,N2)=(4,1) for ranks 1 through 4 (left) and for ranks 5 through 8 (right).  Relative Sector SE (top) and Relative Edge UE SE (bottom).  Full Buffer traffic, Outdoor FWA scenario

Figure 2: Spectral Efficiency for (N1,N2)=(4,1) for ranks 1 through 4 (left) and for ranks 5 through 8 (right).  Relative Sector SE (top) and Relative Edge UE SE (bottom).  Full Buffer traffic, Indoor FWA scenario

Figure 3: Spectral Efficiency for (N1,N2)=(2,2) for ranks 1 through 4 (left) and for ranks 5 through 8 (right).  Relative Sector SE (top) and Relative Edge UE SE (bottom).  Full Buffer traffic, Outdoor FWA scenario

Figure 4: Spectral Efficiency for (N1,N2)=(2,2) for ranks 1 through 4 (left) and for ranks 5 through 8 (right).  Relative Sector SE (top) and Relative Edge UE SE (bottom).  Full Buffer traffic, Indoor FWA scenario

#### 2.1.3.2 Adaptive Rank Selection

System level simulations results were generated for both the Outdoor FWA and Indoor FWA scenarios with both FTP1 and full buffer traffic to evaluate the performance of the different choices for (O1, O2) for when the rank is adaptively selected for all scheduled UEs.  Figure 5 shows the performance in the Outdoor FWA scenario for (N1, N2) = (4,1) for bursty traffic at target loads of 20% (top left), 50% (top right), 70% (bottom left), and also for full buffer traffic (bottom right).  Figure 6 shows the corresponding plots for the Indoor FWA scenario for (N1, N2) = (4,1).  Figure 7 shows the corresponding plots for the Outdoor FWA scenario for (N1, N2) = (2,2).  Figure 8 shows the corresponding plots for the Indoor FWA scenario for (N1, N2) = (2,2).  The arrival rates for these target resource utilization values were the same arrival rates used in the simulation results presented in [6], which are also contained in the Appendix.

As shown in Figures 5 and 6, for (N1, N2) = (4,1), we see moderate losses from using (O1, O2) = (2,1) or (1,1) relative to (O1, O2) = (4,1).  Those losses are more severe in the bursty traffic cases compared to the full buffer traffic case, which is likely due how the bursty traffic cases tend to involve higher rank transmissions where the losses from the lower overhead configurations are more severe.  Similar trends are shown in Figures 7 and 8 for (N1, N2) = (2,2), where we see moderate losses from using choices other than (O1, O2) = (4,4).

Observation 5: for adaptive rank selection, the losses from not using the highest overhead configuration can be moderate to severe, where the losses are higher in situations where higher ranks tend to get selected more often (e.g., FTP1).

Figure 5: Relative UE spectral efficiency for (N1, N2) = (4,1): Mean and cell edge: FTP1 with target resource utilization of 20% (top left), 50% (top right), 70% (bottom left), and full buffer (bottom right). Outdoor FWA scenario

Figure 6: Relative UE spectral efficiency for (N1, N2) = (4,1): Mean and cell edge: FTP1 with target resource utilization of 20% (top left), 50% (top right), 70% (bottom left), and full buffer (bottom right). Indoor FWA scenario

Figure 7: Relative UE spectral efficiency for (N1, N2) = (2,2): Mean and cell edge: FTP1 with target resource utilization of 20% (top left), 50% (top right), 70% (bottom left), and full buffer (bottom right). Outdoor FWA scenario

Figure 8: Relative UE spectral efficiency for (N1, N2) = (2,2): Mean and cell edge: FTP1 with target resource utilization of 20% (top left), 50% (top right), 70% (bottom left), and full buffer (bottom right). Indoor FWA scenario

#### 2.1.3.3 Discussion

As shown in the results in the previous subsections, not supporting the highest overhead configuration can severely impact UL system performance.  Given that support for ranks higher than 4 will be specified, it is important to avoid sacrificing the performance when a rank higher than 4 is selected, otherwise the overall benefits of supporting ranks higher than 4 will be reduced, significantly so in some situations.

Observation 6: As shown in the system level simulation results, not supporting the highest overhead configuration for 8Tx can severely impact UL system performance.  Given that support for ranks higher than 4 will be specified, it is important to avoid sacrificing the performance when a rank higher than 4 is selected, otherwise the overall benefits of supporting ranks higher than 4 will be reduced, significantly so in some situations.

Based on these comprehensive system level simulation results and observations, we propose that

Proposal 2: For (N1, N2) = (4,1), consider supporting at least (O1, O2) = (4,1) based on UE capability.  For (N1, N2) = (2,2), consider supporting at least (O1, O2) = (4,4) based on UE capability.

Based on Table 5, supporting (O1, O2)=(4,1) will need extra 2-bit of indication overhead, compared with (O1,O2)=(1,1). Supporting (O1,O2)=(4,4) needs extra 4-bit overhead, compared with (O1,O2)=(1,1).

## 2.2	Partial coherent NR 8Tx codebook

Following RAN1’s agreement on the partial coherent 8Tx codebook design,

Agreement (#110b) For 8TX UE codebook-based uplink transmission, - For partially/non-coherent precoding, support NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook

and RAN1’s agreement on antenna groups Ng,

Agreement Support the following cases for codebook design for 8TX precoders - Full coherent precoders with Ng=1 - FFS: Full coherent precoders with Ng=2, Ng=4 - Partial coherent precoders with Ng=2 and Ng=4 - This does not imply any relation with the number of TPMI indications for 8TX precoder - Non-coherent precoders Agreement For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used

We shall use the same principle of Rel-15 UL 2Tx/4Tx CB design for 8Tx. Let’s consider Ng=2 and Ng=4 separately in the following sub-sections.

### 2.2.1	Antenna group Ng=2

RAN1 has this agreement for Ng=2:

Agreement (RAN1 #112) For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). Note: Above is not relevant to how precoders are indicated. Working Assumption (RAN1 #112b-e) For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3).

When the antenna group Ng=2, each antenna group shall have 4 coherent Tx ports. We may consider Rel-15 4Tx UL codebook. The Rel-15 4Tx codebook is based on ULA such as this placement

This placement with cross-polarzation is not supported in the Rel-15 4Tx codebook.

Consider that the (M, N, P) = (2, 2, 2) and (1, 4, 2) are considered for 8Tx design, we shall consider x-pol support, based on DL Type-I codebooks.

Alternatively, we may reuse certain design principle from Rel-15 uplink 2Tx codebook. For example, with full rank support up to 8, the 8Tx precoders can be

W=W100W2

where W1 and W2 are 4Tx coherent precoders. This design provides rank support: 2, 3, 4, 5, 6, 7, 8.

If half rank (up to rank-4), the 8Tx precoders can be

W10, 0W2

where W1 and W2 are 4Tx coherent precoders, with these rank support: 1, 2, 3, 4

If W1 and W2 are independently indicated, there will be two TPMIs, each of which indicates a Rel-15 4Tx precoder. Based on Rel-15 spec, each TPMI field as “Precoding information and number of layers” has 6-bit payload for “codebookSubset = fullyAndPartialAndNonCoherent”. Separate TPMI means the double the TPMI payload to 12-bit, with maximum flexibility to support various ranks.

Certain constraints to TPMI can have some overhead saving. For example, is there a need to support all 4Tx fully/partial/non-coherent codebooks, given that these 4Tx port in an antenna group are coherent. We can consider to support only fully coherent 4Tx in each TPMI. This approach can save 1-bit per TPMI, with a total 8Tx TPMI payload as 10-bit.

Further reduction of the overhead is possible by using a single 4Tx TPMI, instead of 2 TPMI. This approach will keep the overall 8Tx TPMI payload to 6-bit, which is the same of 4Tx. The drawback is the lack of flexibility in precoder selections, which might impact system-level performance.

Table 6    TPMI design choices for Ng=2, with full max rank per TPMI

|  | Design | TPMI overhead (bits) | Rank | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| 1 | Rel-15 4Tx baseline | 6 | 1 to 4 |  |  |
| 2 | Two independent TPMI, full/partial/non-coherent support per TPMI | 12 | 2 to 8 | Flexible TPMI to support all 4Tx precoder combinations | Large overhead |
| 3 | Two independent TPMI, full coherent support per TPMI | 10 | 2 to 8 | Flexible TPMI combinations with reduced overhead | Reduced number of precoders |
| 4 | Single TPMI for two antenna groups | 6 | 2, 4, 6, 8 | Low overhead | Limited 8Tx precoders |

Table 6 lists these TMPI design possibilities with their corresponding pros and cons. Given that these designs will have different number of precoders, this will impact system-level performance, which can be evaluated with system-level simulations.

RAN1 agreement listed all possible rank combinations for Ng=2 with layer split. However, this type of design does not have a clear connection with the feedback payload size.

With the existing agreements on layer splitting between the two antenna groups, the list of numbers of precoders for all ranks is shown in Table 7. All possible combinations of two possible 4Tx precoders (fully coherent) are considered in the table.

Table 7    List of numbers of precoders for all ranks: Ng=2

| Rank | Ng=2 Combination | # of precoders for each groups | Total # of precoders per group combination | Total # of precoders per rank |
| --- | --- | --- | --- | --- |
| 1 | (1, 0) | (16, 0) | 16 | 32 |
|  | (0, 1) | (0, 16) | 16 |  |
| 2 | (2, 0) | (8, 0) | 8 | 272 |
|  | (0, 2) | (0, 8) | 8 |  |
|  | (1, 1) | (16, 16) | 256 |  |
| 3 | (3, 0) | (4, 0) | 4 | 264 |
|  | (0, 3) | (0, 4) | 4 |  |
|  | (1, 2) | (16, 8) | 128 |  |
|  | (2, 1) | (8, 16) | 128 |  |
| 4 | (4, 0) | (2, 0) | 2 | 68 |
|  | (0, 4) | (0, 2) | 2 |  |
|  | (2, 2) | (8, 8) | 64 |  |
| 5 | (2, 3) | (8, 4) | 32 | 64 |
|  | (3, 2) | (4, 8) | 32 |  |
| 6 | (3, 3) | (4, 4) | 16 | 16 |
| 7 | (3, 4) | (4, 2) | 8 | 16 |
|  | (4, 3) | (2, 4) | 8 |  |
| 8 | (4, 4) | (2, 2) | 4 | 4 |
| Sum |  |  |  | 736 |

With this list, there are a total of 736 8Tx precoders for Ng=2. We have two observations:

- The majority of the precoders is determined by the combination of lower rank precoders. For example, for Rank-2 with (1, 1) antenna group combination, the total number of precoders is 256 because each 4Tx group can support 16 precoders, as defined in Rel-15. If all combinations of these 16 precoders are supported, there will be 256 precoders.

- Removing certain layer splitting, such as (3, 2), won’t impact the total number of precoders much. The layer splitting (3, 2) has 32 precoders; also the layer splitting of (4, 3) for rank-7 Tx has only 8 precoders.

Proposal 3: Consider limiting certain 4Tx precoder combinations to reduce overhead for Ng=2 partially coherent CB design.

### 2.2.2	Antenna group Ng=4

Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - The following rank and layer splitting cases are supported, Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups. 1; (1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1). 2; (2,0,0,0), (0,2,0,0), (0,0,2,0), (0,0,0,2). 2; Transmission by 2 of the 4 antenna groups: (1,1,0,0), (1,0,1,0), (1,0,0,1) (0,1,1,0), (0,1,0,1), (0,0,1,1). 4; (1,1,1,1). 4; Transmission by 2 of the 4 antenna groups: (2,2,0,0), (2,0,2,0), (2,0,0,2) (0,2,2,0), (0,2,0,2), (0,0,2,2). 8; (2, 2, 2, 2). Note: Above is not relevant to how precoders are indicated. Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - In addition to the previously agreed cases, down-select from the rank and layer splitting cases listed below Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2).

When Ng=4, the number of coherent groups is 4, each of which has two coherent antenna ports (x-pol antenna ports in a typical case). If we also reuse design principle of Rel-15 uplink codebook to reuse Rel-15 UL 2Tx codebook precoders W1,W2,W3,W4, the 8Tx precoder can have this form

W10000W20000W30000W4

with rank support of 4, 5, 6, 7, 8.

Alternatively, we may have these 8Tx precoders

| W100W20000 | W10000W300 | W1000000W4 | 00W200W300 |
| --- | --- | --- | --- |
| 00W20000W4 | 0000W300W4 |  |  |

to support rank of 2, 3, 4.

The choices of W1,W2,W3,W4 are based on 2Tx codebooks. From TS 38.211, we have

Table 6.3.1.5-1: Precoding matrix  for single-layer transmission using two antenna ports.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 5 |  |  |  |  |  |  | - | - |

Table 6.3.1.5-4: Precoding matrix  for two-layer transmission using two antenna ports with transform precoding disabled.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 2 |  |  |  |  |

From TS 38.212, with maxRank=2, the “Precoding information and number of layers” field in DCI for the 2Tx TMPI needs 4-bits to support these 9 TPMI indices.

Following the similar approach for Ng=4, we can have 4 independent TPMIs, each of which may be a Rel-15 2Tx TPMI with 4-bits. The total payload for 8Tx TPMIs will be 16 bits. Certain limitation on the 2Tx TPMI is possible. For example, we may remove the rank-2 precoder , which is a rank-2 non-coherent precoder. The 2Tx TPMI payload can be reduced to 3-bit for 8 TPMI indices. The total number of 8Tx TPMI payload would be 12 bits. Further limitation of the 2Tx TPMI will further lower the 8Tx TPMI overhead.

An alternative is to use 2 TPMIs, instead of 4 to have a tradeoff between overhead and performance. Out of Ng=4 groups, two groups shall share one TPMI and the other two groups will share another TPMI. The total overhead bits will be reduced to 8, instead of 16 bits when 4 TPMIs are used. We can also combine this with some limitations to the 2Tx TPMIs.

If single TPMI is shared for all 4 antenna groups, this design will provide lowest overhead with limited 8Tx precoders. All these possible designs are listed in Table 4.

Table 8    TPMI design choices for Ng=4, with full max rank=2 per TPMI

|  | Design | TPMI overhead (bits) | Rank | Pros | Cons |
| --- | --- | --- | --- | --- | --- |
| 1 | Rel-15 2Tx baseline | 4 | 1, 2 |  |  |
| 2 | 4 independent TPMI, full/partial/non-coherent support per TPMI | 16 | 4 to 8 | Flexible TPMI to support all four 2Tx precoder combinations | Large overhead |
| 3 | 4 independent TPMI, with limitation at 2Tx TPMI | 12 | 4 to 8 | Flexible TPMI combinations with reduced overhead | Reduced number of precoders |
| 4 | 2 independent TPMIs | 8 | 4, 6, 8 | Reduced overhead | Reduced number of precoders |
| 5 | 2 independent TPMI, with limitation at 2Tx TPMI | 6 | 4, 6, 8 | Further reduced overhead | Reduced number of precoders |
| 6 | Single TPMI for 4 antenna groups | 4 | 4, 8 | Low overhead | Limited 8Tx precoders |

There is clear tradeoff between TPMI overhead and the number of precoders, which may have impact on uplink performance.

Following the existing agreements, a list of numbers of precoders for Ng=4 is shown in Table 9.

Table 9    List of numbers of precoders for Ng=4

With this list, there are a total of 2056 8Tx precoders for Ng=4, because there are many possible combinations for 4 antenna groups.

Here are some observations, similar to that of Ng=2:

- The majority of the precoders is determined by the combination of lower rank precoders. For example, for Rank-4 with (1, 1, 1, 1) antenna group combination, the total number of precoders is 256 because each 2Tx group can support 4 rank-1 precoders, as defined in Rel-15. If all combinations of these rank-1 precoders are supported, there will be 256 precoders.

- Removing certain layer splitting won’t impact the total number of precoders much. Instead, we shall focus on reducing certain combinations.

Proposal 4: Consider limiting certain 2Tx precoder combinations to reduce overhead for Ng=4 partially coherent CB design.

### 2.2.3	Other Antenna groups

Other than Ng=2 and Ng=4, there may be other possibilities in UE implementation for coherent antenna grouping. For example, it is possible to have Ng=3, where 8Tx ports are grouped as (2, 2, 4), where one coherent group has 2 coherent antenna ports, another group has 2 coherent ports, and the 3rd group has 4 coherent ports. Among the 3 groups, theses antennas are not coherent. This implementation cannot fit the cases of Ng=2 and Ng=4. It is an open question on how to support this type of coherence implementations. Besides, there are more possible implementations that won’t fit into the current design of Ng=2 or Ng=4.

Proposal 5: Study other possible implementations other than Ng=2 and Ng=4

## 2.3	Codebook configuration

RAN1 #112b-e have this agreement on CB configuration for fully-coherent, partially-coherent, and non-coherent UE.

Agreement For codebook -based 8TX PUSCH transmission, down-select from, - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8 - Alt2 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8 - FFS which Ng value(s), to be considered - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8 - FFS whether/how the configuration can be done via RRC or MAC-CE. - Alt3 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1 - A partially-coherent UE , with Ng =2, can only use precoders considered for Ng =2 - A partially-coherent UE , with Ng =4, can only use precoders considered for Ng =4 - A non-coherent UE , with Ng =8, can only use precoders considered for Ng = 8 - Other alternatives are not precluded Note: For an 8TX UE, Ng =8 can represent a non-coherent UE.

Alt 3 is the simple choice by restricting UE within its capability. There is another issue related to UE feature, which is under discussion:

For an 8TX UE, down-select from

- Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one of supported Ng values.

- Option 2 – An 8TX UE may report only a single Ng value as part of its UE capability. No RRC configuration is needed.

If Option 1 is supported, we shall support Alt 3, where the UE can be configured to support one Ng, subject to its capability.

Proposal 6: Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one of supported Ng values. Based on UE indicated capability of supporting Ng, UE can be configured to operate with its supported Ng values with RRC.

## 2.4	Non-codebook based Tx

For non-CB-based UL transmission, there were two agreements in the last two meetings:

Agreement (RAN1 #112) For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - All SRS port combinations are supported - For SRI indication, down-select from, - Option 1: Use an NSRS bit length bitmap - Option 2: Use a legacy-based solution - Consideration of Lmax for SRI indication For NSRS≤4, Rel-15 SRI indication is reused Agreement (RAN1 #112b-e) For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - Support Option 2 where a legacy-based solution is used by extending the existing SRI indication tables to include NSRS=8 and lmax=8

Option 2 shall be based the legacy SRI design, which supports up to 4Tx. Given that all SRS port combinations shall be supported, there would be an extensive table to list all possible port combinations for 8Tx.

Proposal 7: Further study is needed to mitigate the spec impact on SRI table to support 8TX NCB Tx.

## 2.5	System simulation evaluation

RAN1 #109-e meeting has this agreement on EVM for system-level simulation for 8Tx codebook:

Agreement Adopt the following Table as the reference EVM for SLS evaluation.- - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS , 30 KHz. Scenario; Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h. Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h. Industrial (38.901): Indoor Office (Inh ), 3Km/h. Channel model; 38.901. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); Outdoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. gNB antenna radiation pattern parameters; Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW Industrial: IMT.2412 Table 10,5 dBi , 90° HPBW. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; - Up to 64 QAM - Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. Traffic model; - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional). Suggested benchmarking; R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption. Precoder granularity; Wideband. Power control; Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101). Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT.

For the study phase of this objective, there are several key questions to be answered:

- What gains can be achieved from supporting 8TX on the UL while retaining the current limitation of a maximum of 4 transmission layers?

- What gains can be achieved if Rel-18 supports a maximum of 8 transmission layers on the UL in addition to supporting 8TX?

To answer these questions, an extensive set of system simulation results were generated according to the parameters listed in the above table as part of the agreed evaluation methodology.  These simulation results were generated in both the outdoor and indoor FWA scenarios with configuration 1B for 8TX with both omni-directional UE antennas as well as with the directional antennas agreed to in the EVM.  Coherent UE configurations were assumed with the wideband TPMI selected according to the downlink Type I CSI (L=1) for both 8TX and 4TX.  Given the potential performance impact of the maximum supported modulation on the UL, system results are also provided for two maximum modulation orders: 64 QAM and 256QAM.

As shown in the appendix, the gains from supporting 8TX and the additional gains from supporting up to 8 transmission layers are highly variable and depend on the scenario, the traffic characteristics, and the maximum modulation order.  However, the gains from supporting 8TX and up to 8 transmission layers are quite substantial across both the Outdoor and Indoor FWA scenarios, both types of UE antenna patterns (omni and directional), both maximum modulation orders (64QAM and 256QAM) and across the traffic types specified in the EVM for this objective (bursty and full buffer).

Observation 7: The gains from supporting 8Tx and up to 8 Tx layers are quite substantial across both the Outdoor and Indoor FWA scenarios, both types of UE antenna patterns, both maximum modulation orders, and across traffic types.

Given this observation from system level simulations, there is a significant gain from supporting 8Tx over 4Tx, and also a significant gain for supporting a maximum of 8 transmission layers over a maximum of 4 transmission layers, for uplink transmission scenarios specified in the EVM.

## 2.6	Channel Modelling for Performance Specification

To specify UL 8TX features with multiple codewords and practically relevant differing per layer/per CW performance, a spatially aware channel model must be utilized.

We would like to draw to attention the discussion occurring in RAN4, specifically those detailed in [7] from RAN4 #106 and RAN4 #106-bis [8]. Where RAN4 have been discussing the requirement to utilize either a Cluster Delay Line model, or a correlation model to achieve this effect. Prior to these contributions RAN4, was defining requirements for (in this case 8Rx, 2 Codewords) in a tap delay line channel, with no spatial component. As is demonstrated in the example below this creates limited variation per layer for the post equalization SNR; this observation has initially led RAN4 to look to specify performance with a common MCS across both CW, however this activity is ongoing, and efforts are being made to specify a more realistic channel and different MCS across both CW.

Furthermore, the choice of precoding vector per layer does not matter in TDL low (i.e., no) correlation environments. Every precoder will exhibit the same performance, hence the common observations that any precoder index and random precoding perform the same in TDL.

Observation 8: TDL channel models without any spatial component are not sufficient to emulate MIMO per layer and per codeword performance.

In a real deployment, it is observed that some MIMO layers will perform worse than others, with the maximum achievable per CW/TB performance being dominated by the performance of the worse layers within a CW.
Note: Depending on coding rates, the MCS choice is likely dominated by the worst or second to worst layer, as the coding rate decides how much payload bits the CW can stand to lose from the layers being undecodable.

This illustrated in the figure below, which shows example per layer (average) performance or rather quality, where the MCS (vertical bar) is chosen to be different across both CW in either a real deployment, or a simulation whereby variable performance per layer can be emulated.

In order to find an acceptable way to quickly model differencing average per layer SNR, RAN4 has been conducting studies on using a fixed MIMO precoder (PMI) along with TDL channels and spatial correlation.

It was detailed in [9] that TDL model can be enhanced with a simple linear spatial transformation on top of the independently created channel coefficients to create spatial correlation effects, and it is well known that CDL models exhibit the per layer and spatially dependent performance naturally.

Proposal 8: Study channel models that capture the effect of differing average per layer BLER performance for link level evaluation of 8TX.

For example, CDL or TDL with correlation and selected non-random precoders.

# 3	8Tx UE full-power modes

RAN1 #111 has these two agreement for 8Tx full power modes:

Agreement Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder, - Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3 - For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112. - This can be used for other UE Power Classes as well. 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA. Regular UE; P1=P2= …=P8=14 dBm (Full power supported by Mode1). Full-power capable UE; Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm. Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm. (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm. (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm. (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm. Agreement For an 8TX partial/non-coherent precoder, for study on full power codebook-based PUSCH transmissions, use Rel-16 full power modes as the starting point for the design. Note: This does not mandate support of all Rel-16 modes.

RAN1 #112b-e meeting reached this agreement:

Agreement Framework for full power PUSCH transmission by an 8TX UE - To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used. - FFS if any change is required in the specifications. - Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used per rank. - Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration

Rel-16 mode-1 can be extended to 8Tx where a subset of the 8Tx codebook shall support full power. For non-coherent case with Ng=8, one example precoder for rank-1 would be

W=12211111111

to support full Tx power transmission for 8Tx ports.

For other than rank-1, similar approach can be applied to limit TPMI in the 8Tx codebook. For other Ng cases, the TPMI limitation can be applied for the 8Tx partial coherent codebooks.

The Rel-16 full power mode-2 can also be applied with virtualized SRS ports. One simple approach is to virtualize 8Tx ports to a single virtual SRS port.

Proposal 9: Confirm the WA and support Rel-16 full power mode 1 and mode 2 for 8Tx support.

For non-coherent Tx with Ng=8, these are possible PA architectures to reach full 23dBm for PC-3 UE:

- Homogeneous PA

  - 14dBm max power per PA, where all 8 PAs shall be used for full power Tx

  - 17dBm max power per PA, where 4 PAs are needed for full power Tx

  - 20dBm max power per PA, where 2 PAs are needed for full power Tx

- Heterogeneous PA

  - Single PA with 23dBm max power, while other lower-power PAs

  - Two PAs with 20dBm max power each, with other lower-power PAs

For partial coherent TX, RAN1 has these agreed antenna layouts for Ng=2 and Ng=4, based on an agreement in RAN1 #109-e meeting:

Given these two general types of antenna layout with antenna groups, one assumption is that PAs in one antenna group shall have the same max Tx power capability. This assumption is based on that all Tx ports in one antenna group are coherent. Each antenna group may or may not deliver full Tx power collectively. With this assumption, further PA architectures can be studied to enable full Tx power support.

Proposal 10: Use these two antenna layouts with Ng=2 and Ng=4 to support model-1 and model-2 for full Tx power feature.

# 4	Multiple Layers and Codewords

Based on WID, 8Tx operation is targeted for CPE/FWA/Industrial applications. As demonstrated in system-level simulation results shown in Appendix and discussions in Section 2.2, with maximum rank of 8 for 8Tx UE, there is clear indication that full rank operation will provide UE throughput gains. Besides, for CPE/FWA/Industrial applications, high peak Tx rate transmission in uplink could be necessary for certain use cases.

RAN1 #112b-e meeting reached these agreements on the dual CW design

Agreement To support dual CW PUSCH operation by an 8TX UE, if CBG-based transmission is configured, the DL principle for CBGTI DCI field is reused where, - The first half of CBGTI field bits is used to indicate the transmission state of CBGs of the first transport block, while the second half of CBGTI field bits is used to indicate the transmission state of CBGs of the second transport block. - The bit field may be configured to have a length of N bits that can support operation of N/2 CBGs , where N=[2, 4, 6 or 8]. Agreement For 8TX UE supporting dual CW PUSCH (Maximum number of layers configured for the UE is larger than 4) - Alt1 – DL principle is reused for disabling transmission of a transport block, where - The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable (when transmission rank<=4) transmission of its corresponding TB - The enabled transport block is mapped to the first CW. - Note: When the transmission of a transport block is disabled, the number of layers is ≤ 4. - Note: the first CW refers to the enabled CW. Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the scheduled CWs - Alt2: The CW with the higher MCS index (if MCS indices are the same, UCI is multiplex on the first CW) - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

This proposal on 8Tx CG PUSCH is under discussion:

Support CG for dual CW PUSCH transmission by an 8TX UE,

- For Type-1 CG: A second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant

- For Type-2 CG: A second MCS field is added in DCI format activating a Type-2 CG-PUSCH

There is a question on whether configured grant shall be supported for 8Tx uplink transmission. For CPE/FWA/Industrial applications, certain use cases such XR, AGV etc may need low-latency and high throughput uplink transmission. Therefore, we shall support CG for dual CW PUSCH transmission.

Proposal 11: Support CG for dual CW PUSCH transmission for 8Tx.

# 5	List of RRC Parameters

There are potential RRC parameters for 8Tx uplink Tx, based on current agreements.

RAN2 parameters:

- Max number of MIMO layers

- (N1, N2) values for fully coherent 8Tx CB based on UE capability

- 8Tx full power Tx modes based on UE capability

- Number of antenna groups (Ng): 1, 2, 4

# 6	Conclusions

In this contribution, we are addressing the Objective 5 of the Rel-WID [1] on the uplink enhancements to enable 8Tx UL transmission. Here is the list of our observations:

Observation 1: To support (O1, O2)=(2, 1), 1-extra bit is needed in TPMI, compared to (O1, O2)=(1, 1).

Observation 2: To support (O1, O2)=(4, 1), 2-extra bits are needed in TPMI, compared to (O1, O2)=(1, 1).

Observation 3: To support (O1, O2)=(4, 4), 4-extra bits are needed in TPMI, compared to (O1, O2)=(1, 1).

Observation 4: for both Outdoor FWA and Indoor FWA and for both (N1, N2) = (2,2) and (N1, N2) = (4,1) with fixed rank selection and full buffer traffic:  for ranks fixed at 1, 2, 3 or 4, there is little performance difference between the different choices of (O1, O2).  For ranks fixed at values higher than 4, the performance difference between the highest overhead configuration and the other configurations can be moderate to severe.

Observation 5: for adaptive rank selection, the losses from not using the highest overhead configuration can be moderate to severe, where the losses are higher in situations where higher ranks tend to get selected more often (e.g., FTP1).

Observation 6: As shown in the system level simulation results, not supporting the highest overhead configuration for 8Tx can severely impact UL system performance.  Given that support for ranks higher than 4 will be specified, it is important to avoid sacrificing the performance when a rank higher than 4 is selected, otherwise the overall benefits of supporting ranks higher than 4 will be reduced, significantly so in some situations.

Observation 7: The gains from supporting 8Tx and up to 8 Tx layers are quite substantial across both the Outdoor and Indoor FWA scenarios, both types of UE antenna patterns, both maximum modulation orders, and across traffic types.

Observation 8: TDL channel models without any spatial component are not sufficient to emulate MIMO per layer and per codeword performance.

Our proposals are listed below:

Proposal 1: Considering feedback overhead for 8Tx uplink, support Type-I codebookMode=1 CB as fully coherent codebook.

Proposal 2: For (N1, N2) = (4,1), consider supporting at least (O1, O2) = (4,1) based on UE capability.  For (N1, N2) = (2,2), consider supporting at least (O1, O2) = (4,4) based on UE capability.

Proposal 3: Consider limiting certain 4Tx precoder combinations to reduce overhead for Ng=2 partially coherent CB design.

Proposal 4: Consider limiting certain 2Tx precoder combinations to reduce overhead for Ng=4 partially coherent CB design.

Proposal 5: Study other possible implementations other than Ng=2 and Ng=4

Proposal 6: Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one of supported Ng values. Based on UE indicated capability of supporting Ng, UE can be configured to operate with its supported Ng values with RRC.

Proposal 7: Further study is needed to mitigate the spec impact on SRI table to support 8TX NCB Tx.

Proposal 8: Study channel models that capture the effect of differing average per layer BLER performance for link level evaluation of 8TX.

Proposal 9: Confirm the WA and support Rel-16 full power mode 1 and mode 2 for 8Tx support.

Proposal 10: Use these two antenna layouts with Ng=2 and Ng=4 to support model-1 and model-2 for full Tx power feature.

Proposal 11: Support CG for dual CW PUSCH transmission for 8Tx.

# References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink,” RAN#94e.

- RAN1 109-e meeting Chair notes, RAN1 109-e meeting, May 2022

- RAN1 110 meeting Chair Notes, RAN1 #110 meeting, Toulouse, August 2022

- RAN1 110b-e meeting Chair Notes, RAN1 #110b-e meeting, October 2022

- RAN1 111 meeting Chair Notes, RAN1 #111 meeting, November 2022

- R1-2207550, Nokia, Nokia Shanghai Bell, “UL enhancements for enabling 8Tx UL transmission”, RAN1#110

- R4-2300612, Discussion on 8Rx UE demodulation and CSI requirements, Nokia, Nokia Shanghai Bell, RAN4 #106

- R4-2304097, PDSCH Requirements for 8Rx UE performance, Nokia, Nokia Shanghai Bell, RAN4 #106-bis-e

- Stephenne, A., & Champagne, B. (2000). Effective multi-path vector channel simulator for antenna array systems. IEEE Transactions on vehicular technology, 49(6), 2370-2381.

# Appendix – Detailed Simulation Results

## Gain of 8TX with max 4 layers over 4TX with max 4 layers (UE config 1B)

These results show the gains that can be achieved in Rel-18 simply by enabling 8TX while keeping the current maximum of 4 transmission layers.  Small gains with full buffer, but much larger gains with bursty traffic.

Figure A1:  Outdoor FWA scenario with UE config 1B:  gain of 8TX with max 4 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.  Four configurations: Max 64QAM vs Max 256QAM and omni UE antennas vs directional UE antennas.

Figure A2:  Indoor FWA scenario with UE config 1B:  gain of 8TX with max 4 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

## Gain of 8TX with max 8 layers over 8TX with max 4 layers (UE config 1B)

These results show the additional gains that can be obtained by enabling the 8TX UL to support up to 8 layers rather than 4 layers.  Gains are variable, but quite substantial.

Figure A3:  Outdoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 8TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

Figure A4:  Indoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 8TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

## Gain of 8TX with max 8 layers over 4TX with max 4 layers (UE config 1B)

These results show the total possible gain that can be achieved with Rel-18 compared to Rel-17 if Rel-18 enables both 8TX and up to 8 transmission layers.    Most of these gains are rather large no matter what the combination of maximum modulation or whether the UE uses omni or directional antennas, etc.

Figure A5:  Outdoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

Figure A6:  Indoor FWA scenario with UE config 1B:  gain of 8TX with max 8 layers over 4TX with max 4 layers: Left plot is the gain in the Mean UE SE, and the right plot is the gain in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

## Impact of wideband phase errors (UE config 1B)

These results show the loss in performance with wideband phase errors with φ=180o.

Figure A7:  Indoor FWA scenario with UE config 1B with omni UE antennas (max MCS level is 256QAM):  loss from wideband phase errors with φ=180 with the Rel-15 NR downlink Type I CSI used on the uplink: Left plot is the loss in the Mean UE SE, and the right plot is the loss in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

Figure A8:  Outdoor FWA scenario with UE config 1B with omni UE antennas (max MCS level is 256QAM):  loss from wideband phase errors with φ=180 with the Rel-15 NR downlink Type I CSI used on the uplink: Left plot is the loss in the Mean UE SE, and the right plot is the loss in the Edge UE SE.  Four traffic scenarios: FTP model 1 with Target RU = 0.2, 0.5, 0.7 and Full Buffer.

# RAN1 Previous Agreements

## RAN1 #109-e meeting Agreements

Agreement

Study fully-coherent, partially-coherent and non-coherent UEs for uplink transmission with 8TX UEs .

Agreement

Study full power transmission for 8TX UEs .

- Details are FFS upon completion of codebook design

Agreement

Adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 8Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers.

R1-2205221	FL Summary on SRI/TPMI Enhancements; Second Round	Moderator (InterDigital Inc.)

Agreement

Adopt the following Table as the reference EVM for SLS evaluation.-

- Companies may provide additional evaluation results per their case of interest.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS , 30 KHz |
| Scenario | Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h |
|  | Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h |
|  | Industrial (38.901): Indoor Office (Inh ), 3Km/h |
| Channel model | 38.901 |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | Outdoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| gNB antenna radiation pattern parameters | Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW Industrial: IMT.2412 Table 10,5 dBi , 90° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE TX antenna configuration | To be defined according to outcome of Proposal 2.1 |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101) |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

R1-2205497	FL Summary on SRI/TPMI Enhancements; Third Round	Moderator (InterDigital Inc.)

Agreement

For 8TX UE, consider the following UE antenna layouts for codebook design,

- For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration

- For fully/partial-coherent UEs, consider linear array (1D/2D)

  - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration

  - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types

    - An example of an antenna group is a panel

  - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details.

Additional information for definition of antenna layout

- Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively

  - Further down-selection can be done in the next meeting, if needed

  - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases.

  - Other antenna layouts for other use cases are not precluded.

  - To start companies may report their results according to their preferred layout.

| Case | Ng | (M, N, P) per group | Antenna Layout | Antenna Pattern/Antenna Element Gain |
| --- | --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2), (1, 4, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 2 | 2 | (1, 2, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 3 | 4 | (1, 1, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial |

o   Other UE antenna assumption for the purpose of evaluation

|  | Outdoor FWA | Indoor FWA | Industrial |
| --- | --- | --- | --- |
| UE antenna height | 6, 3 m (To start) | According to 36.873 | According to 38.901 |

Agreement

For 8TX UE codebook-based uplink transmission, down-select one of

- Alt1-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs

- Alt1-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs

- Alt2-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt2-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt3:

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

## RAN1 #110 meeting Agreements

Agreement

8TX PUSCH is supported in Rel-18

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed

- (O1, O2) = (1,1), (2,1), (2,2)

- Note: Other values may be used and reported by companies

- Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered

Agreement

RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e.

- Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

Agreement

For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

Agreement

Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE,

- FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded.

- Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms

## RAN1 #110b-e meeting Agreements

Agreement

Support the following cases for codebook design for 8TX precoders

- Full coherent precoders with Ng=1

  - FFS: Full coherent precoders with Ng=2, Ng=4

- Partial coherent precoders with Ng=2 and Ng=4

  - This does not imply any relation with the number of TPMI indications for 8TX precoder

- Non-coherent precoders

Agreement

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7}

- Note: Other alternatives which are not foreseen are not precluded

Agreement

For SRI and/or transmitter precoder matrix indication for codebook-based uplink transmission by an 8TX UE, study

- Whether/how to indicate one or multiple TPMI/SRI, according to the number of antenna groups, coherence capability, codebooksubset configuration, etc.

- Whether/how to extend Rel-17 framework, e.g., TPMI/SRI indication in MTRP PUSCH

- Whether/how to separate/joint indication of rank and precoding information.

- Whether/how to indicate n (<=Ng) selected antenna group(s) separately from TPMI/TRI indication

Agreement

In Rel-18, on support of full power operation by a partial/non-coherent 8TX UE configured with codebook-based transmission,

- Identify and agree on at least one potential PA architecture by RAN1 meeting #111

Agreement

For 8TX UE codebook-based uplink transmission,

- For partially/non-coherent precoding, support NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook

Agreement

For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

Agreement

For SRS configuration supporting codebook -based UL transmission for an 8TX UE ,

- Support configuration of 1 SRS resource set containing up to X 8-port SRS resource(s), where X = 2

  - FFS : Other values for X, if needed

- FFS : Configuration of at least one SRS resource set, configured with more than one SRS resources where each SRS resource may have the same or different number of SRS ports, e.g., for support full power operation, if supported

- FFS : Configuration of at least one SRS resource set, configured with 8/M of M-port SRS resources, for example,

  - Configuration of an SRS resource set, configured with at least 4 of 2-port SRS resources

  - Configuration of an SRS resource set, configured with at least 2 of 4-port SRS resources

Working Assumption

For uplink transmission with rank>4, support dual CW transmission.

Agreement

If dual CW is supported for uplink transmission with Rank>4 by an 8TX UE, reuse DL Rel-15 codeword to layer mapping for both codebook-based and non-codebook-based transmission.

## RAN1 #111 meeting Agreements

Agreement

For a fully coherent uplink precoding by an 8TX UE,

- Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook

  - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a.

- No LS to RAN4 will be needed

Agreement

For PUSCH transmission with rank>4 by an 8TX UE, to support dual CW transmission,

- specify MCS, NDI, RV indication for the second CW

- specify PUSCH Scrambling for the second CW

- specify UCI multiplexing on PUSCH for dual CW transmission

- study whether/how Enabling/Disabling the second CW

FFS: Optimization of DCI to indicate the above

Note: Strive to reuse Rel-15 NR DL schemes where possible.

Agreement

For PUSCH transmission with rank>4 by an 8TX UE, to support UCI multiplexing on PUSCH, down-select at least one of the following options in RAN1#112,

- Option1: UCI is always multiplexed on one of the CWs

- Option2: UCI is multiplexed on both CWs

- Option3: Based on UCI (e.g., type, payload size, etc.) UCI is multiplexed on one or both CWs

- Option4: UCI is multiplexed only when single CW is enabled

- Option5: UCI is repeated across the two CWs

- Other options are not precluded

Agreement

For CB-based 8TX PUSCH transmission, for rank indication, down-select among the following

- Separate indication of TRI and TPMI

- Joint indication of TRI and TPMI

Agreement

Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder,

- Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3

- For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered

  - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112.

  - This can be used for other UE Power Classes as well.

| 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA |  |  |
| --- | --- | --- |
|  | Regular UE | P1=P2= …=P8=14 dBm (Full power supported by Mode1) |
|  | Full-power capable UE | Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm |
|  |  | Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm |
|  |  | (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm |
|  |  | (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm |
|  |  | (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm |

Agreement

For an 8TX partial/non-coherent precoder, for study on full power codebook-based PUSCH transmissions, use Rel-16 full power modes as the starting point for the design.

Note: This does not mandate support of all Rel-16 modes.

## RAN1 #112 meeting Agreements

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported,

- (N1, N2) = (4, 1)

- (N1, N2) = (2, 2)

A pair of (N1, N2) can be configured with subject to UE capability.

Agreement

Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook

- Precoding matrices generated according to (O1, O2) = (1, 1) is supported

- Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1

  - Subject to UE capability

- FFS: Different O1, O2 values for different ranks

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support

- Alt.2: A second MCS field (5 bits) is indicated for the second codeword

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated.

- FFS: Details on how to signal

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q∈{0,1},

cinit=nRNTI∙215+q∙214+nID

where nRNTI, and nID are defined similar to the legacy single CW PUSCH transmission.

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1),

- Study whether/how to support (O1, O2) = (2,1), (2,2)

  - whether for all rank, or rank 1-2, or rank 3-8

  - applicability of different (O1, O2) values per agreed (N1, N2)

  - companies are encouraged to submit simulation results

Agreement

To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from,

- Alt1: First CW

- Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

Agreement

For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission.

10000000,01000000,00100000,00010000,00001000,00000100,00000010,00000001

with the scaling factor of 122.

Agreement

For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set,

- All SRS port combinations are supported

- For SRI indication, down-select from,

  - Option 1: Use an NSRS bit length bitmap

  - Option 2: Use a legacy-based solution

- Consideration of Lmax for SRI indication

For NSRS≤4, Rel-15 SRI indication is reused

Agreement

For CB-based 8TX PUSCH transmission, where Mode 2 uplink full power transmission (if supported) is not used, re-use legacy Rel-15 mechanism, that is

- when only one SRS resource in a resource set is configured, the SRI field in DCI is absent,

- when two SRS resources are configured in a resource set, 1 bit of SRI field in DCI is used to indicate the selected SRS resource in the set.

Agreement

For partially coherent uplink precoding by an 8TX UE codebook,

- When Ng=2

  - Precoding design is based on Rel-15 UL 4TX codebook,

    - Full-coherent precoders are used

      - FFS whether partial-coherent precoders are needed

- When Ng=4, down-select from,

  - Alt1:

    - Precoding design is based on Rel-15 UL 2TX codebook,

      - Full-coherent precoders are used

  - Alt2:

    - Precoding design is based on Rel-15 UL 4TX codebook,

      - Partial-coherent precoders are used

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=2,

- Following rank and layer splitting cases are supported

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 1 | (1,0), (0,1) |  |
| 8 |  | (4,4) |

- Select from the following cases based on the performance and overall DCI overhead

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (2,1), (1,2) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2), (3,1), (1,3) |
| 5 |  | (4,1), (1,4), (2,3), (3,2) |
| 6 |  | (4,2), (2,4), (3,3) |
| 7 |  | (4,3), (3,4) |

Note: Above is not relevant to how precoders are indicated.

## RAN1 #112b-e meeting Agreements

Conclusion

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), there is no consensus to support any optional over-sampling ratio.

Working Assumption

For partially coherent uplink precoding by an 8TX UE, Ng=2,

- At least the following combinations of layer splitting are supported

  - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

Agreement

To configure PUSCH transmission by an 8TX UE,

- Alt2: Max number of MIMO layers is RRC configured by extending the range of the legacy parameter maxRank and maxMIMO-Layers to 8

Agreement

To support dual CW PUSCH operation by an 8TX UE, if CBG-based transmission is configured, the DL principle for CBGTI DCI field is reused where,

- The first half of CBGTI field bits is used to indicate the transmission state of CBGs of the first transport block, while the second half of CBGTI field bits is used to indicate the transmission state of CBGs of the second transport block.

- The bit field may be configured to have a length of N bits that can support operation of N/2 CBGs , where N=[2, 4, 6 or 8].

Agreement

Framework for full power PUSCH transmission by an 8TX UE

- To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used.

  - FFS if any change is required in the specifications.

- Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used.

  - FFS if more than one of the 8TX full coherent precoders is used per rank.

- Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used.

  - FFS definition of precoder groups (G0, G1, …)

  - FFS enhancements for SRS configuration

Agreement

For 8TX UE supporting dual CW PUSCH (Maximum number of layers configured for the UE is larger than 4)

- Alt1 – DL principle is reused for disabling transmission of a transport block, where

- The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable (when transmission rank<=4) transmission of its corresponding TB

- The enabled transport block is mapped to the first CW.

- Note: When the transmission of a transport block is disabled, the number of layers is ≤ 4.

- Note: the first CW refers to the enabled CW.

R1-2302308	FL Summary SRI/TPMI Enhancements; Second Round	Moderator (InterDigital, Inc.)

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, Alt1 is supported where

- Precoding design is based on Rel-15 UL 2TX codebook,

  - Full-coherent precoders are used

- Further study codebook size reduction

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=4,

- The following rank and layer splitting cases are supported,

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups |
| --- | --- | --- |
| 1 | (1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1) |  |
| 2 | (2,0,0,0), (0,2,0,0), (0,0,2,0), (0,0,0,2) |  |
| 2 |  | Transmission by 2 of the 4 antenna groups: (1,1,0,0), (1,0,1,0), (1,0,0,1) (0,1,1,0), (0,1,0,1), (0,0,1,1) |
| 4 |  | (1,1,1,1) |
| 4 |  | Transmission by 2 of the 4 antenna groups: (2,2,0,0), (2,0,2,0), (2,0,0,2) (0,2,2,0), (0,2,0,2), (0,0,2,2) |
| 8 |  | (2, 2, 2, 2) |

Note: Above is not relevant to how precoders are indicated.

Agreement

For non-coherent uplink precoding with rank≤8 by an 8TX UE, down-select from

- Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported

- Alt2. – Only a subset of Alt1. is supported, striving for a substantial reduction in the number of precoders

R1-2302309	FL Summary SRI/TPMI Enhancements; Third Round	Moderator (InterDigital, Inc.)

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=4,

- In addition to the previously agreed cases, down-select from the rank and layer splitting cases listed below

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups (All possible permutations) |
| --- | --- | --- |
| 3 |  | Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |
| 4 |  | Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1) |
| 5 |  | Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1) |
| 6 |  | Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2 |
| 7 |  | Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2) |

Agreement

For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set,

- Support Option 2 where a legacy-based solution is used by extending the existing SRI indication tables to include NSRS=8 and lmax=8

Agreement

To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the scheduled CWs

- Alt2: The CW with the higher MCS index (if MCS indices are the same, UCI is multiplex on the first CW)

  - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

Agreement

For partially coherent 8TX precoding with Ng =2, the precoder is based on up to two full-coherent 4TX precoders. Down-select one of the following options for precoder indication,

- Option 3 – Up to two 4TX TPMIs are indicated,

  - When two TMPIs are indicated, the first is applied on one of antenna group, and the second is applied on the other antenna group,

  - FFS : details of TPMI indication when one antenna group is used

- Option 4 – A single 8TX TPMI is indicated

- Other options are not precluded

Agreement

For codebook -based 8TX PUSCH transmission, down-select from,

- Alt1

  - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which combinations of Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8

    - FFS which combinations of Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more  Ng cases, i.e., Ng= 4, 8

    - FFS which combinations of Ng value(s), if any, to be considered

  - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8

- Alt2

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8

    - FFS which Ng value(s), to be considered

  - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8

  - FFS whether/how the configuration can be done via RRC or MAC-CE.

- Alt3

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1

  - A partially-coherent UE , with Ng =2, can only use precoders considered for Ng =2

  - A partially-coherent UE , with Ng =4, can only use precoders considered for Ng =4

  - A non-coherent UE , with Ng =8, can only use precoders considered for Ng = 8

- Other alternatives are not precluded

Note: For an 8TX UE, Ng =8 can represent a non-coherent UE.