3GPP TSG-RAN WG1 Meeting #113	R1- 2305755

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN4 LS on RS supported for group-based reporting

Document for:	Discussion

# 1	Introduction

In [1], RAN4 asked the following questions to RAN1 regarding RS supported for group-based reporting:

Question 1: RAN4 understands that two resource sets consisting of SSB + SSB or CSI-RS + CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report. However, RAN4 is not clear whether the two resource sets configured for Rel-17 group-based L1-RSRP measurement and report can contain a mix of SSB and CSI-RS. Can RAN1 clarify whether a mix of SSB and CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report?

Question 2: When CSI-RS resources are configured as resource set for Rel-17 group-based L1-RSRP measurement and report, can the CSI-RS resource with repetition ON be configured for Rel-17 group-based L1-RSRP report?

Question 3: Does RAN1 specification define UE reporting behaviour for the case when NW configures UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report (e.g., the measured L1-RSRP of at least one of the beams of any beam pair is lower than a certain value, e.g., -140dBm)?

In this contribution, we provide our views on the above three questions.

# 2	Discussion

Question 1:  Regarding this issue, in our understanding the two resource sets can consist of either SSB +  SSB or CSI-RS + CSI-RS.   RAN1 has not previously discussed supporting a mix of SSBs in one set and CSI-RS in the other set.  Hence, we propose the following reply to Question 1:

Answer to Question 1:  a mix of SSB and CSI-RS cannot be configured for Rel-17 group-based L1-RSRP measurement and reporting.

Question 2:  Regarding this issue, the intention of Rel-17 group-based L1-RSRP measurement and report is to identify two SSBs or CSI-RSs in a group that can be simultaneously received.  Hence, in our understanding, CSI-RS resource with repetition ON cannot be configured for Rel-17 group-based L1-RSRP report:

Answer to Question 2:  CSI-RS resource with repetition ON cannot be configured for Rel-17 group-based L1-RSRP report.

Question 3:  Regarding this issue, in our understanding, the RAN1 specifications does not define UE reporting behavior for the case when NW configured UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report:

Answer to Question 3:  the RAN1 specifications does not define UE reporting behavior for the case when NW configured UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report.

# References

R1-2304311, LS on RS supported for group-based reporting, RAN4, RAN1#113, May 2023.