3GPP TSG WG RAN1 #113	R1-2305756

Incheon, May 22nd – 26th, 2023

# Title:	[DRAFT] Reply on LS 2TA for multi-DCI multi-TRP

# Response to:	R1-2304326 LS on 2TA for multi-DCI multi-TRP

# Release:	Rel-18

# Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	Ericsson [RAN1]

To:	RAN2

Cc:

Contact Person:

Name:	[Claes Tidestav]

E-mail Address:	[claes.tidestav@ericsson.com]

1. Overall Description:

RAN1 thanks RAN2 for the LS on 2TA for multi-DCI multi-TRP and would like to provide the following response.

Question 1 on TAG grouping

RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs.

Q1a:   For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs?

Answer:  RAN1 made the following agreement:

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

The statement “A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG” is a restriction that RAN1 aims to capture in the UE feature descriptions.

Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group?

Answer:  RAN1 has not identified a need to increase the number of TAGs per cell group.

Question 2 on operation

Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation?

Answer: RAN1 has not discussed what happens when the time-alignment timer corresponding to one TRP expires. Note that RAN1 has agreed on an association between a TAG ID and target UL channels/signals. Hence, RAN1 has agreed on an association between a time-alignment timer and target UL channels/signals.

2. Actions:

To RAN2 group:

ACTION:

RAN1 respectfully asks RAN2 to take the above answers into consideration.

3. Date of Next RAN1 Meetings:

TSG WG RAN1 #114		21st– 25th August 2023	Toulouse, France

TSG WG RAN1 #114-bis		9th – 13th October 2023	Xiamen, China