3GPP TSG RAN WG1 #113			R1-2305816

Incheon, Korea, May 22nd – May 26th, 2023

Source:	WILUS Inc.

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda item:	9.3.3

Document for:	Discussion

Introduction

In the previous RAN1#112b-e meeting, the following agreements were made on potential enhancements on dynamic/flexible TDD [1].

Agreement at RAN1#112b-e: For the gNB-gNB co-channel CLI measurement, both RSRP and RSSI can be used as measurement metric for evaluation purposes only. Agreement at RAN1#112b-e: Study the effect on DL performance and the UL performance of DL Tx power adjustment to evaluate the feasibility of such scheme to overcome the gNB-to-gNB co-channel CLI. Agreement at RAN1#112b-e: Study the effect on DL/UL performance and specification impact of applying separate open-loop/closed-loop power control parameters with cochannel CLI and without cochannel CLI for the uplink power control of a UE. Agreement at RAN1#112b-e: For gNB-gNB co-channel CLI measurement and channel measurement, study the impact on system performance because of CLI measurement inaccuracy at victim gNB due to misalignment between UL timing at victim gNB and DL reception timing at victim gNB of CLI measurement resource transmitted from one or more aggressor gNB. - Including potential impact on UL performance - Option 1: Only one set of parameters for open loop power control can be reused. - Option 2: Separate sets of parameters for open loop power control can be applied with/without CLI. Agreement at RAN1#112b-e: For enhancement of gNB-to-gNB co-channel CLI measurement and/or channel measurement, following options are studied for UL resource muting. - Option 1: Transparent UL resource muting method (e.g., avoid the scheduling on measurement resource) - Option 2: Non-transparent UL resource muting method (e.g., define UL resource muting pattern with one or more RE/RB muting patterns) Agreement at RAN1#112b-e: For UE-to-UE co-channel CLI measurement, study the impact on system performance because of CLI measurement inaccuracy at victim UE due to misalignment between DL reception timing at victim UE of DL channel/signal transmitted from serving gNB and DL reception timing at victim UE of CLI measurement resource transmitted from aggressor UE(s).

In this contribution, we provide our views on potential enhancements on dynamic/flexible TDD.

Discussion on potential enhancements on dynamic/flexible TDD

The objective of Rel-18 Duplex SI is to study inter-gNB (gNB-to-gNB) inter-cell CLI and inter-UE (UE-to-UE) inter-cell CLI handling and identify solutions to manage them, considering both dynamic TDD operation and SBFD (subband non-overlapping full duplex) operation. Target scenarios to handle such kinds of inter-cell CLI are depicted in Figure 1.

Figure 1. gNB-to-gNB CLI and UE-to-UE CLI.

In Figure 1-(a), DL transmission from aggressor (gNB#1) causes gNB-to-gNB inter-cell CLI to UL reception at victim (gNB#2). In Figure 1-(b), UL transmission from aggressor (UE#2) causes UE-to-UE inter-cell CLI to DL reception at victim (UE#1). In this contribution, we provide our views on gNB-to-gNB inter-cell co-channel CLI and UE-to-UE inter-cell co-channel CLI.

gNB-to-gNB inter-cell co-channel interference

gNB-to-gNB co-channel CLI measurement for gNB-to-gNB co-channel CLI handling

In the previous RAN1#112b-e meeting, it was agreed to study both UE transparent UL muting and UE non-transparent UL muting for enhancement of gNB-to-gNB co-channel CLI measurement and/or channel measurement. UE transparent UL muting denotes that UL muting resources are not known to UEs in a victim cell, while UE non-transparent UL muting denotes that UL muting is known at least for Rel-18 UEs in a victim cell. UE transparent UL muting can be implemented by a gNB scheduling. Thus, the gNB can manage to configure or indicate to UEs not to transmit any UL signal/channel on the resources for gNB-to-gNB CLI handling. On the other hand, an explicit configuration or indication of UL muting resources is necessary at least for Rel-18 UEs in a victim cell for UE non-transparent UL muting.

Between two schemes for UL muting, UE non-transparent UL muting is beneficial in terms of efficient resource utilization. Regarding that the UL muting resource can be RE-level (i.e., REs for NZP CSI-RS from aggressor UE), the UL signal/channel can be transmitted on REs not used for gNB-to-gNB CLI measurement. For example, PUSCH can be rate-matched around REs of UL muting resource. On the other hand, if at least one or more REs are overlapped between UL signal/channel and UL muting resource, the gNB should avoid scheduling in the overlapped RE(s).

Under the UE non-transparent UL muting, UE behaviors on UL muting resources can be further studied. We provide our views on UE behaviors on UL muting resource with respect to UL signal/channel.

- PUSCH can be rate-matched around REs of UL muting resource in both RB-level and RE-level. For example, a UE can transmit PUSCH by rate-matching around RBs of SSB or REs of NZP CSI-RS transmitted from aggressor gNB. Since rate-matching procedure for PUSCH is not supported in the current specification, it can be studied based on the Rel-15 PDSCH rate-matching as we proposed in [2]. Also, different UE behavior for the PUSCH multiplexed with UCI can be studied, e.g., whether or not REs for HARQ-ACK are overlapped with UL muting resource.

- UE behaviors on PUCCH can be categorized according to UCI type and PUCCH format. For example, PUCCH format 0 is scheduled to transmit in 1 or 2 symbols and 1 RB with low PAPR sequence. Then, if at least one RE is not used for PUCCH transmission, a gNB cannot detect the PUCCH format 0 in scheduled resources. Also, since PUCCH format 0 is scheduled to transmit HARQ-ACK and/or SR that should be prioritized to transmit, the UE can transmit PUCCH format 0 even in the resources overlapped with UL muting resources. For another example, PUCCH format 2 is scheduled to transmit in maximum 16 RBs with polar coded bits. Then, even some RBs or REs are overlapped with UL muting resource, the UE can transmit PUCCH format 2 by rate-matching around RBs or REs of UL muting resource.

- SRS can be transmitted or cancelled in symbol-level, i.e., partial cancellation is supported. Therefore, symbol-level transmission or cancellation of SRS can also be used for UE behaviors on UL muting resource. For example, if at least one RE in a symbol of SRS is overlapped with UL muting resource, SRS cannot be transmitted in the symbol. Otherwise, if no RE in a symbol of SRS is overlapped with UL muting resource, SRS can be transmitted in the symbol.

- PRACH transmission can be prioritized over RBs or REs in UL muting resource. Since PRACH is essential channel for uplink synchronization, TA alignment, SR, BFR, and on-demand SI request, it seems better to transmit PRACH even in RBs or REs for UL muting.

In addition, different UE behaviors according to PHY priority (i.e., eMBB or URLLC) can be studied. For example, a UE does not transmit UL signal/channel with low priority on RBs or REs for UL muting, while transmits UL signal/channel with high priority.

- Proposal 1: RAN1 to study UE non-transparent UL muting for gNB-to-gNB CLI handling by considering UE behaviors on UL muting resource with respect to the UL signal/channel and PHY priority.

UE-to-UE inter-cell co-channel interference

UE-to-UE co-channel CLI measurement and reporting for UE-to-UE co-channel CLI handling

In Rel-16, UE-to-UE CLI measurement and reporting procedures at victim UE was specified with two types of CLI measurement and reporting metrics, i.e., CLI-RSSI and SRS-RSRP. For CLI-RSSI measurement, the victim UE measures linear average of the total received power observed in the configured OFDM symbols of the configured measurement time resource(s), in the configured measurement bandwidth from all sources. For SRS-RSRP measurement, the victim UE measures linear average of the power contributions of the resource elements carrying SRS resource(s) which is/are transmitted from one or multiple aggressor UE(s). Then, Layer 3 filtering can be applied to the measurement result for both CLI-RSSI and SRS-RSRP measurements. For measurement result reporting for both CLI-RSSI and SRS-RSRP measurements, event-triggered and periodic reporting are supported.

In addition to the Rel-16 UE-to-UE CLI measurement and reporting, UE-to-UE CLI measurement and reporting at aggressor UE can be studied in Rel-18. Thus, UE-to-UE CLI measurement and reporting at both the victim UE and the aggressor UE can be studied. More efficient CLI handling is expected via bi-directional CLI measurement and reporting. For example, an aggressor UE can be indicated with reduced UL power control parameter set if reported CLI from the aggressor UE is high.

For this, Rel-16 CLI measurement and reporting framework can be a baseline as an agreement in the RAN1#110b-e meeting, i.e., consider reusing existing channel(s)/signal(s)/measurement resource(s) as baseline for UE-to-UE CLI measurement. For example, aggressor UE can be configured to measure SRS-RSRP in the configured resource that SRS is transmitted from victim UE. Otherwise, aggressor UE can be configured to measure CLI-RSSI in the configured resource in certain symbols and RBs.

- Proposal 2: RAN1 to study UE-to-UE CLI measurement and reporting at aggressor UE side for UE-to-UE CLI handling.

In the previous RAN1#112 meeting, it was agreed to use existing CSI reporting framework as the baseline for studying L1/L2 UE-to-UE CLI measurement and reporting. We provide our views on L1/L2 UE-to-UE CLI measurement and reporting details.

In the existing CSI reporting framework, a UE can be configured or indicated to receive CSI-RS for downlink channel state measurement and reporting. CSI reporting framework have three types: periodic, semi-persistent, and aperiodic. A UE can be configured with one of three types and report quantity per CSI reporting configuration with ID. For example, the UE can be configured with {Reporting configuration ID = 0, periodic, cri-RSRP} by CSI-ReportConfig. Then, the UE receives CSI-RS in corresponding CSI-RS resource and measures RSRP of the CSI-RS (i.e., CSI-RSRP). After that, the UE reports measured CSI-RSRP and corresponding CSI-RS resource ID to gNB via PUCCH transmission according to configured slot offset/periodicity.

L1/L2 UE-to-UE CLI measurement and reporting can be included as IEs (information elements) of CSI-ReportConfig. For example, a UE can be configured with {Reporting configuration ID = 1, aperiodic, sri-RSRP} by CSI-ReportConfig, where sri-RSRP denotes victim UE’s SRS resource indicator and corresponding SRS-RSRP. Then, the UE receives SRS in the corresponding SRS resource and measures RSRP of the SRS (i.e., SRS-RSRP) if the UE is indicated to trigger aperiodic CSI reporting by DCI. After that, the UE reports measured SRS-RSRP and corresponding SRS resource ID to gNB via PUSCH transmission scheduled by DCI. In addition to SRS-RSRP, CLI-RSSI, which was also agreed as baseline metric, and corresponding time/frequency resources can be included as an IE of CSI-ReportConfig.

Although CLI measurement reporting is not related with downlink channel state measurement and reporting, there is no ambiguity between gNB and UE among CSI or CLI reporting since it can be configured with separate reporting configuration ID. Also, it’s beneficial in specification point of view by reusing existing configuration, thereby additional signaling overhead is not needed.

- Proposal 3: IEs (information elements) of L1/L2 UE-to-UE CLI measurement and reporting can be included in CSI reporting configuration (i.e., CSI-ReportConfig) with new report quantities to measure and report UE-to-UE CLI.

Power control-based solution

In the RAN1#111 meeting, it was agreed to study whether/how to enhance UL power control mechanism, while considering the existing UL power control mechanism as a baseline. For UE-to-UE CLI handling, UL power reduction at aggressor UE can be considered to mitigate the UE-to-UE CLI. We propose to study UL power control based on the aggressor UE side CLI measurement and reporting as shown in Figure 2.

Figure 2. Power control-based solution for UE-to-UE CLI handling.

① CLI-RS measurement

As discussed above, in addition to Rel-16 UE-to-UE CLI measurement and reporting, UE-to-UE CLI measurement and reporting at aggressor UE can be studied for better UE-to-UE CLI handling. Therefore, an aggressor UE (UE#2) can be configured to measure received signal/channel power of CLI-RS from a victim UE (UE#1). Rel-16 metrics for UE-to-UE CLI measurement can be reused, i.e., CLI-RSSI and SRS-RSRP.

② CLI reporting

After CLI-RS measurement, the aggressor UE (UE#2) can report measurement result via L1, L2, or L3 UE-to-UE CLI reporting to gNB (gNB#2). It’s worth noting that the L1/L2 UE-to-UE CLI reporting is beneficial to reflect the UE-to-UE CLI instantly than L3 UE-to-UE CLI reporting. As discussed above, the aggressor UE (UE#2) can be configured to measure and report UE-to-UE CLI in one of CSI reporting configurations.

③ UL power control

Based on UE-to-UE CLI reporting from aggressor UE (UE#2), gNB (gNB#2) can indicate aggressor UE (UE#2) to adjust UL transmission power. For example, an aggressor UE can be indicated with reduced open-loop UL power control parameter set (P0 or alpha) or closed-loop UL power control parameter set (TPC command) if reported UE-to-UE CLI from the aggressor UE is high.

For the above power control-based solution for UE-to-UE CLI handling, existing UL power control parameter set can be reused unless the motivation of parameter modification is clarified (e.g., increased step size of TPC command).

- Proposal 4: RAN 1 to study UL power control-based solution for UE-to-UE CLI handling based on L1/L2 UE-to-UE CLI measurement and reporting at aggressor UE side.

  - Existing UL power control parameter set can be reused.

Conclusion

In this contribution, we discussed potential enhancements on dynamic/flexible TDD and summarize our views as the following:

- Proposal 1: RAN1 to study UE non-transparent UL muting for gNB-to-gNB CLI handling by considering UE behaviors on UL muting resource with respect to the UL signal/channel and PHY priority.

- Proposal 2: RAN1 to study UE-to-UE CLI measurement and reporting at aggressor UE side for UE-to-UE CLI handling.

- Proposal 3: IEs (information elements) of L1/L2 UE-to-UE CLI measurement and reporting can be included in CSI reporting configuration (i.e., CSI-ReportConfig) with new report quantities to measure and report UE-to-UE CLI.

- Proposal 4: RAN 1 to study UL power control-based solution for UE-to-UE CLI handling based on L1/L2 UE-to-UE CLI measurement and reporting at aggressor UE side.

  - Existing UL power control parameter set can be reused.

References

- Draft Report of 3GPP TSG RAN WG1 #112b-e v0.1.0

- R1-2305815, “Discussion on subband non-overlapping full duplex,” WILUS Inc.