3GPP TSG RAN WG1 #113			R1-2305822

Incheon, Korea, May22nd – 26th, 2023

# Title:	[DRAFT] Reply LS on multicast reception in RRC_INACTIVE

# Response to:	R1-2304325

Release:	Rel-18

Work Item:	NR_MBS_enh-Core

Source:	Ericsson [RAN1]

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Florent Munier

#### E-mail Address:	florent.munier@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for the question in the LS in R1-2304325. The questions are copied below for convenience:

LS from R1-2304325 1. Overall Description: For Rel-18 MBS enhancement, RAN2 has discussed multicast reception in RRC_INACTIVE and made several agreements (chair notes can be found in R2-2304207). RAN2 would like to inform RAN1 of the following which may be relevant to RAN1: RAN2#121bis-e agreements: - CFR for multicast reception in RRC_INACTIVE - From the location&bandwidth and SCS configuration perspective, follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE. - Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise). - Case B and case D are not supported for multicast CFR in RRC_INACTIVE. - Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed. - The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs. - HARQ Operation (including beam and DCI format) - HARQ feedback related information in the DCI is not needed or can be ignored for multicast transmission to RRC_INACTIVE UE. - The HARQ operation for multicast reception in RRC_INACTIVE is same as the operation without HARQ feedback in RRC_CONNECTED state. - The multicast transmission in RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e. beam information is not needed in DCI). - For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used. We will ask RAN1 to confirm whether it is feasible and whether both 4-1 and 4-2 are needed.. Additionally, RAN2 has made some assumptions on aspects relevant to RAN1 and would like to check RAN1 views on the following for multicast reception in RRC_INACTIVE: - Question 1: Is the following RAN2 assumption feasible? If feasible, whether both DCI format 4-1 and DCI format 4-2 are needed? - For MTCH, RAN2 assumes to reuse the same DCI formats of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used. - Question 2: Is the following RAN2 understanding correct? - RAN2 understanding is that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 for multicast MTCH in RRC_CONNECTED as well as for broadcast MTCH). - Question 3: Is it feasible to reuse the following Rel-17 CSS design for multicast MTCH and multicast MCCH? - 3.1) Reusing the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED). - 3.2) Separate CSS(es) for multicast MCCH and multicast MTCH in RRC_INACTIVE. 2. Actions: To RAN1 group ACTION: RAN2 respectfully asks RAN1 to take the above agreements into consideration and provide responses to the above questions.

Based on the discussion in RAN1#113, RAN1 reached the following conclusions:

Regarding Question 1:

Regarding the assumption, the DCI formats for multicast can be used in RRC inactive, both of the multicast formats are suitable for use in RRC_INACTIVE. The PRI Field regarding HARQ feedback in DCI 4_1 or 4_2, should be ignored by the UE since no harq feedback wil be provided. For MCCH, DCI 4_0 can be used also in RRC_INACTIVE.

Regarding Question 2:

PDSCH aggregation is configured as part of the part of the G-RNTI configuration in MBS-RNTI-SpecificConfig, and therefore can also be supported in RRC_INACTIVE.

Regarding Question 3:

Reusing the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE is feasible from RAN1 perspective.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above answers into account.

3. Date of Next RAN1 Meetings:

R1#114 		2023‑08‑21 - 2023‑08‑25, Toulouse, France

R1#114-bis		2023‑10‑09 - 2023‑10‑13, Xiamen, China