3GPP TSG-RAN WG1 Meeting #113                                                                                                      R1-2305943

Incheon, Korea, 22-26 May, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Document for:	Discussion and Decision

Introduction

In previous meeting, the following LS [1] was triggered by RAN2.

# 1 Overall description RAN2 has discussed possible clarifications on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs based on specification text in RAN2 and relevant sections in RAN1 and RAN4. Current RAN2 specifications do not explicitly specify what happens for UEs in half duplex mode if a paging occasion conflicts with a CG-SDT occasion. It is RAN2’s understanding that although information pertaining to this can be found in e.g., 38.213, clause 17.2 or in 38.133, clause 5.1B.2.6, the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. Similar to connected mode behaviour, since the UE is only required to monitor the paging in any paging occasion at least once per modification period, there should be other paging occasions available (within the modification period) to monitor the paging for SI change even if some of them overlap with the CG-SDT occasion(s). Hence, RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications. # 2 Actions To RAN WG1 and RAN WG4 ACTION: RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications for CG-SDT with HD-FDD Redcap.

In this contribution, the UE behavior for paging occasions monitoring conflicting with a CG-SDT occasion is discussed.

Discussion

As mentioned in the LS, it is RAN2’s understanding that the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

And the behavior is already captured in RAN2 38.331 spec as following.

##### 5.2.2.2.2 SI change indication and PWS notification … UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

For HD-FDD RedCap UEs, the concern from RAN2 is that UE behavior for paging occasions monitoring conflicting with a CG-SDT occasion is not specified.

From RAN1 perspective, paging for SI change indication is treated as common PDCCH reception and CG-SDT occasions is treated as higher layers configured uplink transmission.

For HD-FDD RedCap UEs, the UE behavior for common PDCCH reception and higher layer configured uplink transmission is already captured in RAN1 spec as following.

## 17.2 Half-Duplex UE in paired spectrum … A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols.

Proposal 1:  Send reply LS to RAN2 with the following information.

- For HD-FDD RedCap UEs, the UE behavior for paging occasions monitoring conflicting with a CG-SDT occasion has already captured in RAN1 spec 38.213 as following. No additional action is needed in RAN1.

  - “A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols.”

Conclusions

Based on the analysis, we have the following proposal:

Proposal 1:  Send reply LS to RAN2 with the following information.

- For HD-FDD RedCap UEs, the UE behavior for paging occasions monitoring conflicting with a CG-SDT occasion has already captured in RAN1 spec 38.213 as following. No additional action is needed in RAN1.

  - “A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols.”

- References

- R2-2304562, “LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs” RAN2, RAN2#121bis-e.