3GPP TSG-RAN WG1 Meeting #113	R1-2305945

Incheon, Korea, 22-26 May, 2023

Agenda Item:	9.16.16

Source:	Huawei, HiSilicon

Title:	UE feature for non-contiguous multi-PUSCH scheduling with single DCI 0_1 in FR1

Document for:	Discussion and Decision

# Introduction

Agreement

Introduce UE feature(s) for multi-PUSCH scheduling with single DCI 0_1 for non-contiguous slots in FR1 for all defined SCSs

- Note: there is no RAN1 impact

In RAN1#112bis-e, a TEI to extend non-contiguous multi-PUSCH scheduling with single DCI 0_1 in FR1 for all SCSs was endorsed according to the following agreement [1]. In this paper, we discuss how the UE feature group can be added.

# Discussion

In Rel-16 NR-U, according to TR38.822 [2], the UE feature group for contiguous multi-PUSCH scheduling with single DCI 0_1 was introduced for all SCSs for a shared spectrum operation, in order to avoid unnecessary LBT attempts before individual PUSCHs are due to receive UL grants in DL.

| Index | Feature group | Components | Prerequisite feature groups | Type | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 |  | Per band | Optional with capability signalling |

When this feature group was captured in TS38.306 [3], the restriction to apply this on only the unlicensed band was removed. It was also extended to FR2-1 for all SCS.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| multiPUSCH-UL-grant-r16 Indicates whether the UE supports scheduling up to 8 PUSCH with a single DCI 0_1. This capability is also applicable to a frequency band that does not require shared spectrum access. | Band | No | N/A | N/A |

In Rel-17 FR2-2, the feature for non-contiguous multi-PUSCH scheduling with single DCI 0_1 was designed based on its contiguous version for 480kHz and 960kHz SCS in order to reduce the PDCCH BD within the unit time. They were included as a component of the FG for basic support of 480kHz and 960kHz SCS for UL.

| Index | Feature group | Components | Prerequisite feature groups | Type | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Per band | Optional with capability signalling |
| 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1a, 24-5 | Per band | Optional with capability signalling |

It was further extended to 120kHz SCS for FR2-2 and FR2-1 during the UE features discussions.

| 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Per band | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- |
| 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Per band | Optional with capability signalling |

The FG24-1g is captured in TS38.306 [3] as follows:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PUSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 with non-contiguous allocation. | Band | No | N/A | N/A |

In order to support non-contiguous multi-PUSCH scheduling with single DCI 0-1 in FR1 for all SCSs, FG 24-1g can be taken as reference. Considering the non-contiguous version is enhanced based on the contiguous version, FG10-17 can be the requisite for the new FG. Accordingly, we propose that the following FG can be introduced:

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FR1/FR2 differentiation | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Enhanced Multi-PUSCH UL grant in FR1. | 1.Multi-PUSCH scheduling by single DCI with non-contiguous allocation for all SCS in FR1 | 10-17 | Yes | Non-contiguous multi-PUSCH scheduling is not supported in FR1 | per band | FR1 only | Optional with capability signalling |

# Conclusions

According to the above discussions, we have the following proposals:

Proposal: The following FG can be introduced to support non-contiguous multi-PUSCH scheduling with single DCI 0_1 in FR1 for all SCSs.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FR1/FR2 differentiation | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Enhanced Multi-PUSCH UL grant in FR1. | 1.Multi-PUSCH scheduling by single DCI with non-contiguous allocation for all SCS in FR1 | 10-17 | Yes | Non-contiguous multi-PUSCH scheduling is not supported in FR1 | per band | FR1 only | Optional with capability signalling |

# References

- Chairman’s Notes, 3GPP RAN1#112bis-e, April 2022.

- TS 38.822 v17.0.0, “User Equipment (UE) feature list (Release 17)”, March 2023.

- TS 38.306 v17.4.0, “User Equipment (UE) radio access capabilities (Release 17)”, March 2023.