3GPP TSG-RAN WG1 Meeting #113		R1- 230xxxx

Incheon, May 22-26, 2023

Agenda item:	9.2.1

Source: 	Moderator (Qualcomm)

Title: 	Summary 1 of General Aspects of AI/ML Framework

Document for:	Discussion/Decision

# Introduction

In this contribution, we summarize issues regarding AI/ML general aspects agenda in RAN1 #113.

## Contact information

Please provide/update your contact information.

| Company | Name | Email |
| --- | --- | --- |
| Moderator (Qualcomm) | Taesang Yoo | taesangy@qti.qualcomm.com |
| vivo | Peng SUN | sunpeng@vivo.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Google | Yushu Zhang | yushuzhang@google.com |
| OPPO | Jia SHEN | sj@oppo.com |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| Futurewei | Chunhui (Allan) Zhu | chunhui.zhu@futurewei.com |
| Ericsson | Henrik Ryden | henrik.a.ryden@ericsson.com |
| NTT DOCOMO | Haruhi Echigo | haruhi.echigo.fw@nttdocomo.com |
| NEC | Peng Guan | guan_peng@nec.cn |
| LG Electronics | Jiwon Kang | jw.kang@lge.com |
| Lenovo | Jianfeng Wang Vahid Pourahmadi | wangjf20@lenovo.com vpourahmadi@lenovo.com |
| Mediatek | Yuanyuan Zhang | Yuany.zhang@mediatek.com |
| ZTE | Guozeng, ZHENG | zheng.guozeng@zte.com.cn |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| KDDI | Taishi Watanabe | ta-watanabe@kddi.com |
| Intel | Bishwarup Mondal | bishwarup.mondal@intel.com |

| AT&T | Salam Akoum Isfar Tariq | Salam.akoum@att.com Isfar.tariq@att.com |
| --- | --- | --- |
| Fujitsu | Wang Xin | wangxin@fujitsu.com |
| Spreadtrum | Hualei Wang | Hualei.wang@unisoc.com |
| ETRI | Hanjun Park Anseok Lee | hjunpark@etri.re.kr alee@etri.re.kr |
| CMCC | Yuhua Cao Yi Zheng | caoyuhua@chinamobile.com zhengyi@chinamobile.com |
| Panasonic | Hidetoshi Suzuki | suzuki.hidetoshi@jp.panasonic.com |
| Nokia | Keeth Jayasinghe | Keeth.jayasinghe@nokia.com |
| Samsung | Ameha | amehat.abebe@samsung.com |
| Intel Corporation | Bishwarup Mondal | bishwarup.mondal@intel.com |
| SEU | Jiajia Guo | jiajiaguo@seu.edu.cn |
| Sony | Hiroki Matsuda | hiroki.matsuda@sony.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Apple | Huaning Niu | huaning_niu@apple.com |
| Rakuten Mobile | Koichiro Kitagawa | koichiro.kitagawa@rakuten.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Fraunhofer | Georgios Kontes Ebrahim Amiri | georgios.kontes@iis.fraunhofer.de ebrahim.amiri@iis.fraunhofer.de |
| Indian Institute of Tech (M)/IIT Madras | Jeeva Keshav S Sai Prasanth Anil Kumar Yerrapragada Rohit Singh Sai Prasad P | jeevak@5gtbiitm.in saiprasanth@5gtbiitm.in anilkumar@5gtbiitm.in rohit@5gtbiitm.in venkatasiva@5gtbiitm.in |
| TCL | Jack Ahmed Mikaeil Qu Miao | xiaoyong.tnag@tcl.com ahmed.mikaeil@tcl.com miaoqu.tcl.com |
| China Telecom | Bei Yang | yangbei1@chinatelecom.cn |
| New H3C | Lei Zhou | zhou.leih@h3c.com |
| Continental Automotive | Hojin Kim | hojin.kim@continental.com |

# Summary of contributions in RAN1#113

## Definitions of terms, symbols and abbreviations

### Proposals for changes

##### Company proposals

Fraunhofer:

Proposal 1: Change the Reinforcement Learning (RL) Definition as follows:

Reinforcement Learning (RL): A process of training an AI/ML model (policy) to interact with an environment and take actions (model’s output) based on the environment’s current state (model’s input), with the goal of maximizing the expected cumulative reward (feedback signal). For the AI/ML model (policy) training, direct interaction with the environment, available logged data from the environment, or a combination of both can be used.

ZTE:

| Model activation | enable an AI/ML model for a specific function AI/ML-enabled feature |
| --- | --- |
| Model deactivation | disable an AI/ML model for a specific function AI/ML-enabled feature |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function AI/ML-enabled feature |

### New terminologies

##### Company proposals

Fraunhofer:

Proposal 5: The following concepts/terms shall be introduced:

- Fault: a specific problem caused by the performance degradation of the AI/ML model. For example, for a beam management model, the RSRP/SINR values of the chosen beams are declining.

- Fault indication: signs that could imply the existence of a fault. For example, a mismatch between the statistics of input data in the AI/ML model during Inference and the training data for the specific AI/ML model, could indicate a problem on the model’s performance.

- Fault type or root cause of a fault: the underlying reason a fault is observed. For example, we have a blockage or reflections in the radio environment and the AI/ML model’s performance degrades, as it is not trained for this.

##### Issue 7-1: terminologies

###### [FL1] Proposal 7-1a:

- Revise the following terminologies for model activation, model deactivation, and model switching as follows

| Model activation | Enable an AI/ML model for a specific function AI/ML-enabled feature |
| --- | --- |
| Model deactivation | Disable an AI/ML model for a specific function AI/ML-enabled feature |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function AI/ML-enabled feature |

|  | Yes | No |
| --- | --- | --- |
| Agree? | Vivo, Panasonic, Futurewei |  |
| Company | Comments |  |
| vivo | Support |  |
| Indian Institute of Tech (M)/IIT Madras | We agree with the revised terminologies. |  |
| OPPO | Agree. |  |
| ZTE | Support. |  |
| NTT DOCOMO | Generally support the proposal. But prefer the small modification of model switching description to make it more clear. Deactivating a currently active AI/ML model and activating a different AI/ML model forwithin a specific function AI/ML-enabled feature |  |
| Google | We do not see a strong motivation for the change. But if majority is ok, we can also accept the proposal. |  |
| Futurewei | Support |  |
| Fujitsu | Regarding model switching, there are two potential cases: - The activated model and deactivated model belongs to the same functionality - The activated model and deactivated model belongs to different functionalities. For case 2, it covers functionality switching, is it the intention to do this modificiton? |  |
| Lenovo | Agree |  |
| NEC | Generally OK. |  |
| LG | OK |  |
| KDDI | Support. |  |
| ETRI | Support. |  |
| CAICT | Support. |  |
| Ericsson | Support. This is more inline with other agreements. |  |
| Samsung | Ok |  |

## General AI/ML Framework

### Description of the stages of Machine Learning

##### Previous agreements

RAN2 #121-bis-e Agreement The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage.

##### Company proposals

FUTUREWEI:

Proposal 1: RAN1 to take the RAN2 decision on functional framework as the base and handle the rest of the discussion going forward, instead of leaving it to RAN2.

Proposal 2: Adopt the functional framework in Figure 2 as the starting point for RAN1 AI/ML study item. Note the dash lines indicate the operation/data/signaling may or may not be needed depending on where the model is stored in network entities.

Figure 2. Proposed AI/ML functional framework

Huawei, HiSilicon:

Proposal 1: Figure 1 can be considered as the diagram for high-level general AI/ML framework.

Figure 1 Diagram of the general framework for air-interface AI/ML

ZTE:

The functional framework about the relationship between different functions should be discussed in RAN1. The entity mapping for the functions in the functional framework can be further discussed by RAN2.

Adopt the following functional framework as a starting point for further discussion:

Fig.1 Functional framework

Vivo:

Figure 6-2: Updated general framework of AI for air interface.

Proposal 17: Based on the Framework for RAN intelligence, RAN1 to introduce an updated general framework that can reflect the key components of AI for air interface.

Spreadtrum:

Figure 2. Functional Framework for AI/ML enabled air interface

Proposal 1: The general framework of Figure 2 can be considered as the AI functional framework for the air interface.

CATT:

Figure 1 Functional framework of AI/ML in NR air interface

Proposal 2: Adopt Figure 1 in R1-2304721 as the diagram for AI/ML framework in NR air interface.

Proposal 3: Discuss whether to introduce functional framework diagram for functionality-based LCM.

- If introduced, it is preferred to reuse the one for model-ID-based LCM as much as possible.

Intel:

Figure 1: Functional framework

Proposal-1 (framework): Support a modification of the RAN3 functional framework (37.817) by using components according to the agreed terminology in RAN1 (including model storage introduced in RAN2)

Ericsson:

- Adopt the model functional framework in Figure 1 for anchoring discussions related to model LCM.

Figure 1. A functional framework for discussing model LCM aspects.

Fujitsu:

Proposal-1: To cover the monitoring methods that are not performance-based, it is suggested that the name of model management/performance monitoring is changed to model monitoring/management in the framework diagram.

Proposal-2: Adopt the diagram in Figure-1 as a starting point of general framework diagram. Further updates can be added according to RAN1/2 future discussions.

- Note: the block and arrows with dish line need to be further studied.

Figure-1 General framework diagram.

Xiaomi:

Figure 2 Proposed updated diagram of LCM

LG:

Proposal #1: Support AI/ML functional framework for air interface.

- Modification from the functional framework in TR37.817 can be considered (e.g. removing actor function, adding model management function)

- Coordination with RAN2 is necessary for this topic

CAICT:

Fig.1 Sample of general AI/ML framework

Proposal 1: The general AI/ML functional architecture could be agreed in RAN2 to avoid duplicate discussion.

MediaTek:

Model Inference

Model Training

Model Management

Model transfer/ delivery

Model Performance Feedback

Outputs

Training data

Inference data

Data collection

Model Storage

Model Deployment/Model delivery

Model control

Monitoring data

Figure 1 Example of Functional Framework for AI/ML over air interface

Proposal 1: The AI/ML functional framework for air interface includes the functions of data collection, model training, model storage, model inference and model management. RAN1 takes Figure 1 as starting point for functional framework discussion.

Nvidia:

Proposal 1: The defining stages of AI/ML related algorithms, including the model generation, e.g., model training (including input/output, pre-/post-process, online/offline, etc.), model validation, model testing, the model inference operation, e.g., input/output, pre-/post-process, and the associated complexity, need to be analysed case by case.

Apple:

Fig 2: Proposed functional framework with performance monitoring

Proposal 12: Use TR 37.817 functional framework as the starting point for RAN1 functional framework discussion.

Proposal 13: Considering additional performance monitoring block in the functional framework.

Lenovo:

Figure 1 A high-level functional framework for the study on AI/ML for NR air interface.

Proposal 1: Adopt Figure 1 as the general functional framework, including Data Collection, Model Training, Model Management, Model Inference and optional Model Storage, where at least the Model Management and Model Inference are in RAN.

Qualcomm:

Figure 1: A general AI/ML framework

Proposal 1: Adopt the general AI/ML framework diagram shown in Figure 1.

Proposal 2: Model storage can be necessary for the general AI/ML framework to reflect all cases in “Model transfer/delivery agreement” in RAN1 #112.

ATT:

Proposal 1: Study a common framework for model and functionality identification to indicate a common understanding between network and UE across all sub-use cases.

Proposal 2: include the following blocks as a starting point for high-level AI/ML framework diagram:

Data collection, model training, model management/performance monitoring, model inference. Model storage.

Proposal 3: The following figure is used as the starting point for general AI/ML framework.

Model deployment/delivery/transfer

Note:

- A block may be implemented in one or multiple entities, some of which may be 3GPP or non-3GPP entities.

- The interaction between block may or may not have impact on 3GPP signaling.

NEC:

Figure 1 AI/ML Framework Description

Proposal 1: Adopt Figure 1 as starting point for discussion for AI/ML functional framework and discuss the potential enhancements to support different use cases/variations of AI/ML models.

TCL Communication:

Proposal 1: The MEC can be taken as the Starpoint for AI/MI model LCM, and RAN should focus on AI/ML model inference-relate processes designing, including configuration, active/deactivate/switching mechanism, monitoring and CSI enhancement etc.

Indian Institute of Tech (M), IIT Kanpur:

Figure 1: Block diagram representation of different stages of AI/ML General Framework

##### Issue 7-2: framework diagram

###### [FL1] FL remark 7-2a:

The FL thank companies for proposals regarding the framework diagrams. Generally, companies think that the diagram from 6-1c of the last meeting, copied below, is a good starting point, and companies have various proposals for modifications.

However, given that RAN2 is also discussing this, the FL would like to dedicate the meeting time on other more important topics.

There was also a proposal on whether the group needs to consider to add a similar diagram for a functionality-based LCM.

Please share your views on this.

| Company | Comments |
| --- | --- |
| vivo | Support the diagram. |
| ZTE | It’s not an important issue in our thinking. It’s fine to only capture general aspects. A separate diagram for functionality-based LCM is not necessary. Our suggestion is to have separate blocks for model management and performance monitoring. The model storage can be kept, which doesn’t imply the actual entity mapping. Therefore, it’s fine to merge model management and model storage. |
| Panasonic | OK with FL suggestion. |
| Huawei/HiSilicon | As not all the blocks in the diagrame are mandatory (e.g., select, switch, model delivery/transfer/update), this diagram seems also applicable to functionality based LCM (where model select/switch is not needed). |
| NTT DOCOMO | Share the same view as FL. |
| Google | We suggest we split the data collection into 3 blocks (training, monitoring and inference), as the data collection for the 3 functionalities could be different. We also suggest we use ‘dot-line’ for ‘model training control’, as this could be an optional step. |
| Futurewei | - Given that RAN1 has spent quick some time on this diagram, we think RAN1 is very close to reaching agreements. As we mentioned, RAN2 discussed it once during online meeting time and they tried to reach agreement. We think RAN1 has better understanding about the air-interface use cases and requirements so RAN1 should lead and complete the discussion/work on the general framework. However, RAN1 does need to coordinate well with RAN2. - We support using the diagram as the starting point and incorporate acceptable comments from last meeting, for example, adding model deployment for the arrow from Model Storage to Model Inference. |
| Fujitsu | OK with the suggesiton |
| Lenovo | Generally OK, and we suggest to add a dot line from ‘Model management’ to ‘Model storage’. We think another diagram for functionality-based LCM could not be necessary, since the operations on the AI/ML models have the same intentions. |
| NEC | OK with the proposed diagrams. Also OK with FL suggestions. |
| LG | Our understanding of this diagram is to explain how AI/ML algorithm operates for air-interface and not directly related to spec enhancements. Thus, not sure the need for different diagram for different LCM approaches. |
| ETRI | Support. |
| CAICT | Support. |
| Ericsson | Share the view that we can await RAN2 progress. |
| Samsung | To be more generic the following can be modified: “model/functionality management/performance monitoring”. This block doesn’t cover on-device training. Is that FL’s intention? |

### Collaboration levels

##### Previous agreements

Agreement (from RAN1 #109-e) Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary Working Assumption (from RAN1 #110-bis-e) - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z. - Clarifying note: Level y includes cases without model delivery. Agreement (from RAN1 #110-bis-e) - Clarify Level x/y boundary as: Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

##### Company proposals

CATT:

Table 1 Relationship between collaboration levels, functional/model identification and LCM procedures

| Identification | Model transfer | Collaboration level | LCM procedure within 3GPP |
| --- | --- | --- | --- |
| No identification | N/A | Level x | No. LCM is out of 3GPP network. |
| Functionality identification | Not support | Level y | Functionality-based LCM, e.g. AI/ML functionality activation/deactivation, fallback to non-AI/ML based approach, performance monitoring. |
| Model identification | Not support | Level y | Model-ID-based LCM (without model transfer), e.g. AI/ML model activation/deactivation, model selection/switching/monitoring, etc. |
|  | Support | Level z | Model-ID-based LCM, all LCM procedures in level y + model transfer. |

Proposal 4: From UE’s perspective,

- If UE/UE-side supports functionality identification, collaboration level y can be achieved. Functionality-based LCM can be supported, e.g. AI/ML functionality activation/deactivation, fallback to non-AI/ML based approach, performance monitoring.

- If UE/UE-side supports model identification but not model transfer, collaboration level y can be achieved. Most model-ID-based LCM procedures within 3GPP network can be supported, e.g. AI/ML model activation/deactivation, model selection, model switching, model monitoring, fallback, etc.

- If UE/UE-side supports model identification and model transfer, collaboration level z can be achieved. All model-ID-based LCM procedures can be supported.

Sony:

Proposal 1: RAN1 should prioritise the study of collaboration level y&z between transmitter and receiver to identify issues and solutions.

LG:

Proposal #9: For collaboration level z, RAN1 should focus on RAN1-specific issues only or deprioritize it for further study.

CMCC:

Proposal 1: Confirm the working assumption on Level y-z boundary.

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

Nvidia:

Proposal 2: RAN1 to further clarify the meaning of “dedicated AI/ML-specific enhancement.”

- For example, if RAN1 introduced the feature that “UE to report the measurement results of more than 4 beams in one reporting instance” for AI/ML based beam management, would the feature be qualified as “dedicated AI/ML-specific enhancement”?

Rakuten:

Proposal 1
Confirm the following working assumption:

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

- Clarifying note: Level y includes cases without model delivery.

Proposal 2

Further clarification of the AI/ML collaboration Level y includes:

- Level y-1: NW based AI/ML application

- Level y-2: Dual-sided AI/ML application

- Level y-3: UE based AI/ML application

The above clarification can be independently defined as framework, instead of clarification of the collaboration levels.

Proposal 3
For collaboration level z, controllable model parameters should be aligned with collaboration level y, at least.

NEC:

Proposal 21: Support to define network-UE collaboration levels based on one-sided AI/ML model or two-sided AI/ML model.

### ML model Life Cycle Management

##### Previous agreements

RAN1 #110 Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

#### Data collection

##### Previous agreements

Conclusion from RAN#1 110-bis-e Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN2 #121 agreement Proposal 1: - RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity.​ Proposal 2: - Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc.​ Proposal 3: - RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted.​ Proposal 4: - Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics.​ Proposal 5: - When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy.​ Proposal 6: - Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed.​ Proposal 7: - Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required. Proposal 8: - For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary P1-P8 are loosely endorsed with the understanding that we can also go beyond, e.g., analyse other methods. RAN2 #121 agreement R2 may consider including the existing EVEX framework for this SI, FFS exactly what this means, can discuss next meeting RAN2 #121-bis-e Agreement • Extend the previously endorsed table with 3 columns: Inference, Monitoring and Training, and explain in free text the applicability of the data collection method to the LCM purpose and the use case(s). • P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information. • P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement. FFS how we do the formatting/presentation of the results. • P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results. • P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training) - Model sidedness (UE side, NW side, two sided) FFS - Use case mapping FFS • P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS.

##### Company proposals

Ericsson

Proposal 19: Regarding NW-sided training data collection, send an LS to RAN2 taking the following requirements into account for each use case, with example values below for BM/CSI use case as a starting point,

| Use case | Size of one data sample (ballpark numbers) | Type of data | Latency requirement (time to collect 1,…N samples) | Periodicity req. (how often gNB should collect samples) | Direction |
| --- | --- | --- | --- | --- | --- |
| BM-spatial pred. | ~1k bits (supporting >100 of beams in set A+B) | L1-RSRP | Non-real time | Days/Weeks | UE-gNB |
| CSI compression | ~13k | High-resolution CSI | Non-real time | Days/Weeks | UE-gNB |
| …. |  |  |  |  |  |

Proposal 20: Regarding NW-sided monitoring data collection, send an LS to RAN2 taking the following requirements into account for each use case, with example values below for BM/CSI use case as a starting point,

| Use case | Size of one data sample (ballpark numbers) | Type of data | Latency requirement (time to collect 1,…N samples) | Periodicity req. (how often gNB should collect samples) | Direction |
| --- | --- | --- | --- | --- | --- |
| BM-spatial pred. | ~1k bits (supporting >100 of beams in set A+B) | L1-RSRP | FFS | FFS | UE-gNB |
| CSI compression | ~13k | High-resolution CSI | FFS | FFS | UE-gNB |
| …. |  |  |  |  |  |

FutureWei:

Proposal 3: Consider at least the following aspects when studying data collection and, if applicable, the corresponding potential specification impact related to data collection:

- Measurement configuration and reporting

- Contents, type and format of data including:

  - Data related to model input

  - Data related to ground truth of model output

  - Quality of the data

    - FFS: how to indicate quality of the data

  - Other information

- Signaling of assistance information for categorizing the data

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

- Method of indicating the capabilities (e.g., storage capacity) of one side to the other side

- Mechanisms of reducing the size of data needs to be transmitted over the air interface

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by appropriate working groups.

Continental Automotive

Proposal 3: Data subsetting is used to split the complete dataset collection for selective data transfer based on data quality measurement.

Proposal 4: Mapping relationship between data collection and assistance information is used to further reduce signaling overhead related to data collection of each LCM phases.

Vivo

Proposal 18: Study the following assistance information for data collection:

- Assistance information for categorizing the data/dataset;

- Additional conditions, such as cell ID, timestamp and SNR.

Proposal 19: Study the data compression for multiple samples in collected data reporting.

ZTE

Proposal 4: 	Send LS to RAN2 about the identified requirements in RAN1 on data collection for various use cases and LCM purposes, at least consider the following aspects:

- Report type and content;

- Data size per reporting;

- Report latency and periodicity;

- The amount of data samples.

- Note: the following table can be adopted for collecting the requirements:

| Use case | Report type | Report content | Data size per reporting | Report latency | Report periodicity | The amount of samples (if applicable) | LCM purposes (e.g., model training, model monitoring or model inference) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: 	 For the purpose of identifying a dataset between network side and UE side, further define the following terminology:

- Dataset identification: A process/method of identifying a dataset for the common understanding between the NW and the UE

- Note: The process/method of dataset identification may or may not be applicable.

- Note: Information regarding the dataset may be shared during dataset identification.

Huawei, HiSilicon

Proposal 5: Study the potential spec impact of data collection from realistic networks for supporting the LCM of AI/ML model, including at least:

- Enhanced/dedicated RS design.

- Enhanced UE measurement/report.

- Type/format of the data sample(s).

- Signaling for indicating/requesting data collection.

Proposal 6: For data collection, study the procedure/signaling to generate/carry data sample(s), including both L1 and L3 measurement/reporting.

Proposal 7: Study the following aspects to improve the quality of dataset during data collection:

- Improving the quality of data samples, e.g., improving the accuracy of the measured labels.

- Indicating the quality requirement of data samples to be reported.

Proposal 8: The necessity of introducing new assistance information for data collection/categorization needs to be clarified/justified, considering:

- UE can sense the scenario autonomously without being notified by gNB or with legacy signaling.

- The categorization or granularity of the scenarios identified by Network vendor may not match the categorization principle of the UE side.

- Generalized model can be trained over scenarios/configurations.

Proposal 9: The assistance information for data collection/categorization, if studied, should be in forms of virtualized ID to avoid the disclosure of proprietary.

- Such assistance information can be sent from Network to UE or from UE to Network.

Proposal 10: Send the following LS to RAN2 to provide RAN1 observations on data collection.

1. Overall Description: RAN1 has identified NW side data collection is one of the key procedures of AI/ML model LCM and may be performed for different purposes e.g., model training and model monitoring. Data collection may have different requirements depending on the purpose and sub use case. Note: the size of dataset collected by the Network side depends on the purpose (e.g., training or monitoring), requirement (e.g., required performance of the model, whether the training is from scratch or fine-tuning, etc.) and UE capability (e.g., number of data samples supported by UE), and the NW side dataset can be constructed by collecting data from multiple UEs, so RAN2 could assume the number of data samples within one message of data collection for per UE is flexible/configurable and subject to the current capacity of RRC signaling. 2. Actions To TSG RAN WG2 ACTION: RAN1 respectfully asks RAN2 to take the following RAN1 observations into account. Table 1 Data collection requirements for model training Size of one data sample; Type of data; Latency requirement; Frequency requirement; Direction. CSI compression; 1000~6000 bits; Eigenvector; Non real time; Several days, weeks or months; UE->gNB. Beam prediction in spatial domain; 250~1000 bits; RSRP (set A/set B); Non real time; Several days, weeks or months; UE->gNB. Beam prediction in temporal domain; 250~1000 bits; RSRP (set A/set B); Non real time; Several days, weeks or months; UE->gNB. Direct AI/ML positioning; ~8000 bits; Multipath Profile; Non real time; Several days, weeks or months; UE/PRU->LMF (Case 2b) gNB->LMF (Case 3b). Table 2 Data collection requirements for model monitoring Size of one data sample; Type of data; Latency requirement; Frequency requirement; Direction. CSI compression; 1000~6000 bits; Eigenvector/CSI feedback; ~100ms level; On demand, e.g., several minutes or hours; UE->gNB. Beam prediction in spatial domain; 250~1000 bits; RSRP (set A/set B); <500ms; On demand, e.g., several minutes or hours; UE->gNB. Beam prediction in temporal domain; 250~1000 bits; RSRP (set A/set B); <1s; On demand, e.g., several minutes or hours; UE->gNB. Direct AI/ML positioning; ~8000 bits; Multipath Profile; ~1s level; On demand, e.g., several hours or days; UE/PRU->LMF (Case 2b) gNB->LMF (Case 3b).

CATT

Proposal 24: Regarding data collection, RAN1 sends an LS to RAN2, in which collecting the following information for the purpose of model training and performance monitoring in each sub use case respectively.

-	Data type (e.g. raw channel or eigenvector for CSI compression),

-	Data sample size,

-	Dataset size (magnitude of amount),

-	Requirement on latency and/or periodicity.

Xiaomi

Proposal 1: Data collection from network to UE and potential network operation to facilitate the data collection on UE side should be supported

Proposal 2:  Study the data collection requirement to facilitate RAN2’s discussion

AT & T

Proposal 10: For data collection for the general AI/ML framework, consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection:

- Measurement configuration and reporting

- Contents, type and format of data including:

  - Data related to model input

  - Data related to ground truth output

  - Quality of the data

  - Other information

- Signaling of assistance information for categorizing the data

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by appropriate working groups.

NEC

Proposal 4: Study whether and how the legacy CSI framework, BM framework and positioning framework can provide sufficient data for model training (including fine-tuning) and model inference.

Proposal 5: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

CAICT

Proposal 2: As for the general aspects for data collection, Proposal 6-4g discussed in last meeting could be agreed.

Proposal 3: L1-signaling could be used for high frequency and small data samples transmission while RRC signaling could be used for low frequency and large data samples transmission.

CMCC

Proposal 3: For data collection, study the potential spec impact of dataset delivery based on 3GPP signaling.

Proposal 4: To enable the development of a set of specific models, study the way to associate the dataset with a specific scenario/configuration/site.

Proposal 5: To further improve the system performance, study the mechanism to reduce overhead of data collection in LCM.

Proposal 6: Consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection:

- Measurement configuration and reporting

- Contents, type, and format of data including:

  - Data related to model input

  - Data related to ground truth

  - Quality of the data

  - Other information

- Signaling of assistance information for categorizing the data / dataset

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by appropriate working groups.

Nvidia

Proposal 9: For AI/ML LCM, study potential specification impact related to data collection for different purposes, including model training, model inference, model monitoring, model selection, model update, etc.

Proposal 10: For AI/ML model training in each NR air interface enhancement, study potential specification impact related to training data type, training data source determination, and assistance signalling and procedure for training data collection.

Intel

Proposal-2 (data collection – training): Support assistance information from NW to UE and UE to NW to facilitate data collection for training at the UE and at the NW respectively

Lenovo

Proposal 15: Associate the dataset for the AI/ML model with scenario/configuration/site-specific setting, which benefit the performance conformance test and the application condition indication in functionality identification.

Qualcomm

Proposal 29:  Mechanism for training data collection needs architectural considerations and should be handled by other working groups.

Proposal 30: For data collection for model training, UE-side may take auxiliary inputs such as SNR, Doppler, sensor measurements, etc. that do not need to be standardized.

Samsung

Proposal#9: Study the necessity, requirement and specification of data collection for two cases of purposes

- Case1: : Real-time purposes, e.g., model monitoring, inference, selection, switching, etc.

- Case2:  Non-real-time purposes, e.g., model training, update

Proposal#10: Study the following two directions of data collection where applicable, assess their pros and cons and specification impact:

- Network-side data collection and assistance information from UE

- UE-side data collection and assistance information from network

Proposal#11: Study per each use case the feasibility/frequency/overhead of dataset collection before designing the dataset sharing framework

NTT Docomo

Proposal 9: RAN1 should discuss the dataset collection requirement for training and monitoring in each sub use case. In our view, at least RAN1 should discuss what data should be collected, required latency of the dataset collection, and how often and large data should be collected.

Proposal 10: Assistance signalling including configuration/deployment ID should be assumed for dataset collection, model training, and model inference of scenario/configuration-specific AI models.

Proposal 11: Study DL RS configuration request mechanism for UE side data collection.

TCL Communication

Proposal 4: AI-specific CSI measurement framework should be introduced, to support various CSI measurement scenarios, which would be used for different AI functions, or different AI procedure that associated with the logical model ID.

Proposal 5: Data collection payload can be conveyed through UCI, RRC or data plane signaling, and UCI format would be enhanced to support real-time AI/ML-specific data transmission.

Proposal 6: Data plane would be introduced in the network system, which dedicated for data collection, including AI-based data collection, also other data collections would be involved in future, e.g., NWADF, MDT, BOSS etc.

MediaTek

Proposal 2: Data collection comprises multiple functional entities serving for different purposes of functions. The functional entity of data collection is co-located with the function for which the dataset is used.

Proposal 3: RAN1 study the requirements of data collection for model offline training, model monitoring and model inference with following aspects: need of specified data content/format, data size, data collection latency and data collection frequency.

Proposal 4: Consider UE data transfer from UEs to the OTT server with the following ways. FFS on others.

•	Option 1: Data transfer from UEs to OTT server with RAN awareness

•	Option 2: Data transfer from UEs to OTT server without RAN awareness

Proposal 5: Study the following mechanisms for data collection: utilize existing or extension of existing L1/L3 measurement and report procedure or utilize procedure particular for data collection request and control.

Proposal 6: Study the proprietary way, 3GPP specified way and combination of them for dataset exchange for two-sided model Type 3 training.

Proposal 7: A Dataset ID is assigned to the data with assistance information for the purpose of categorizing the data.

Proposal 8: Assistance information may consist of (sub)use cases, purposes (training/inference/monitoring), scenarios, and requirements (latency), timestamps, if applicable.

##### Issue 7-3: Data collection requirement

As RAN2 is expecting RAN1 input on data collection requirements, the following proposed conclusion is intended to capture requirements associated with the different purposes of data collection to serve as guidance for RAN2 and other working groups.

###### [FL1] Proposal 7-3a:

The different purposes of data collection are associated with different latency and data size requirements as described in the following table:

| Purpose of data collection | Latency requirement | Data size requirement | Example of data collected |
| --- | --- | --- | --- |
| Offline initial training / redevelopment | Relaxed (e.g., hours or days) | Large | many measurements from many devices to server |
| Model validation | Relaxed (e.g., minutes or hours) | Large | many measurements / metrics from many devices using the model |
| Slow monitoring | Relaxed (e.g., minutes or hours) | Large | many measurements / metrics from one device |
| Fast monitoring | Near-real-time (e.g., 100s of ms to a few seconds) | Limited | a few measurements / metrics from one device |
| Inference | Time-critical (e.g., a few ms) | Limited | a single measurement from one device |

|  | Yes | No |
| --- | --- | --- |
| Agree? | Panasonic |  |
| Company | Comments |  |
| vivo | In general fine with the observation. One comment regarding model validation: we did not touch this component either in framework or for each use case thus prefer not to include this for now. |  |
| Indian Institute of Tech (M)/IIT Madras | Agree in general. Request further clarifications on why Training and Validation are considered separately. |  |
| OPPO | General OK with the first three types. But what is the difference between fast monitoring and inference? Why we need to differentiate the two? What is the use case for fast monitoring (e.g., 100s of ms to a few seconds)? We think it needs more justification. |  |
| ZTE | We’re fine with the direction to send LS to RAN2. Please find more comments: - For offline training at network side, the measurement should be reported from UE. Our suggestion is to split last column into two columns, i.e., ‘data type’ and ‘measurement report direction’ - Not clear how to understand ‘model validation’. If it’s ‘model testing’, it should belong to model training. If it’s about model monitoring, we don’t need separate row for it. - Some use cases already made progress on data collection. If we want to send LS to RAN2 in this agenda, we prefer to have more concrete information in RAN1. For example, we can give exact values for the data size per sample, data type for different use cases and LCM purposes. |  |
| Huawei/HiSilicon | Agree that RAN1 needs to send RAN2 some useful info. on data collection in forms of LS. Some suggested changes: For “Offline initial training / redevelopment”, we may change it as “Traininig” for simplicity. - Under “Example of data collected” column, the content should be changed from “many measurements from many devices to server” as “from UE to gNB/LMF”. From UE to UE side server is implementation. For “Model validation”, it should be part of training, and no need for a dedicated entry. For “Data size requirement”, we should distinguish the size (no. of bits) of per data sample from the size (no. of samples) of the dataset. For the data collection, the overall size of the dataset does not impact the RAN2 signaling (L3/MDT), since gNB can collect the data from massive UEs, each of which only contributes one or a few data samples; or, even for a single UE, it does not need to feedback all its data samples in one batch, but split them into multiple RRC signalings. The essential point that impacts the RAN2 spec is whether the the size (no. of bits) of per data sample, or the overall size (no. of bits) of data samples in one report needs new requirements. From what we have evaluated in RAN1, the current RRC signaling (9K Bytes) can basically carry one/multiple data sample(s) without difficulty. Therefore, the content of this entry should be changed as “the current RRC signaling (9K Bytes) can carry one/multiple data sample(s) without difficulty”. In addition, we think “frequency requirement” is needed as an additional column, to address whether it is NW triggered, or periodic. Purpose of data collection; Latency requirement; Frequency requirement; Data size requirement; Example of data collected. Offline initial training / redevelopment Training; Relaxed (e.g., hours or days); NW-triggered or periodic report; Large current RRC signaling (9K Bytes) is capable to carry one/multiple data sample(s); many measurements from many devices to server gNB/LMF. Model validation; Relaxed (e.g., minutes or hours); Large; many measurements / metrics from many devices using the model. Slow monitoring; Relaxed (e.g., minutes or hours); NW-triggered or periodic report; Large current RRC signaling (9K Bytes) is capable to carry one/multiple data sample(s); many measurements / metrics from one device to gNB/LMF. Fast monitoring; Near-real-time (e.g., 100s of ms to a few seconds); NW-triggered; Limited; One/a few measurements / metrics from one device to gNB/LMF. Inference; Time-critical (e.g., a few ms); NW-triggered or periodic report; Limited; a single measurement from one device to gNB/LMF. |
| NTT DOCOMO | Fine with the principle. However, the example of near-real-time is not clear description. Should it be like a few ms to a few second? Also, the boundary of slow monitoring and fast monitoring is unclear. We suggest the folling update for the common understanding between companies. ・Slow monitoring => monitoring for the model update/fine-tuning ・Fast monitoring => monitoring for the fallback operation decision and model/funcitonlaity switching |  |
| Google | We also think it is better to clarify what “slot monitoring” and “fast monitoring” mean. For monitoring, we also think ‘one measurement’ should be included. |  |
| Futurewei | Agree in general. |  |
| Fujitsu | It is supportivie to provide such kind of information to RAN2. We also think the part of Model validation needs to be further clarified. |  |
| Lenovo | We are fine with the intention to provide the requirements on data collection to RAN2, which are useful to select, design and/or enhance the signaling to satisfy the requirements. However, the values are supposed to be use case specific, and as a general principle discussed here, we can have some range values for both latency and data size. |  |
| NEC | Slow and fast monitoring is not well discussed. We would like to understand the typical useage of “slow monitoring”. Since we are discussing physical layer techniques, all monitoring should be “fast monitoring”. |  |
| LG | Similar question as NTT for the boundary between slow and fast monitoring. Further clarification is necessary. |  |
| KDDI | Agree in general. However the meaning of “model validation” should be calrified. |  |
| ETRI | We understand this as a discussion of data collection. Therefore, we think it is needed to categorize the processes that are distinct from the data collection point of view. From this point of view, the motivation to separate data collection processes for training and validation does not seem clear as mentioned by other companies. Similary, it is also not clear why monitoring is divided into slow monitoring and fast monitoring. Are we going to consider two different data collection processes for monitoring?. |  |
| CAICT | Agree in general. |  |
| Ericsson | Support in general, prefer the table from Huawei. We would also like to agree on a potential LS to RAN2, using the table from Huawei proposal 10 as a starting point. |  |
| Rakuten Mobile | We are fine in general. Definition of fast and slow monitoring is not clear, and clarification is needed. E.g., By mentioning latency requirement for the monitoring explicitly in the first column, it would be clarified. |  |

##### Issue 7-4: Data collection study aspects (continued from 6-4)

(FL note: The following proposal is the same as Proposal 6-4g from the e-mail discussion in RAN1#112bis-e.)

###### [FL1] Proposal 7-4a:

Consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection:

- Measurement configuration and reporting

- Contents, type and format of data including:

  - Data related to model input

  - Data related to ground truth

  - Quality of the data

  - Other information

- Signaling of assistance information for categorizing the data

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by appropriate working groups.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | We are supportive of the study. |  |
| OPPO | OK to study them. But considering the remaining meeting time for Rel-18. We suggest to prioritize some bullets in this list, e.g. - Measurement configuration and reporting - Contents, type and format of data including: And leave signaling for second priority. |  |
| ZTE | OK with the general guidance. |  |
| Panasonic | On "signaling of assistance information for categorizing the data", such assistance information usage is not limited to categorizaing the data. Therefore, to add "at least" like " Signaling of assistance information at least for categorizing the data" is proposed. |  |
| Huawei, HiSilicon | Support. |  |
| NTT DOCOMO | Support |  |
| Google | For ‘data related to ground-truth’, we suggest we add ‘(if feasible)’. It is hard to get some kind of ‘ground-truth’ data. |  |
| Futurewei | Partially agree. As we have not talked about the quality of data, we suggest adding the following FFS below the sub-bullet “Quality of the data” - FFS: how to indicate quality of the data |  |
| Fujitsu | OK |  |
| Lenovo | Agree |  |
| NEC | Support |  |
| LG | Seems better fit to each use-case level study but ok to agree on this in this agenda for more structured discussion in each agenda. |  |
| KDDI | Support. |  |
| ETRI | Support. |  |
| CAICT | Support. |  |
| Ericsson | Support |  |
| Samsung | Ok. |  |

#### Model development and training

##### Previous agreements

RAN2 Agreement from 121-bis-e R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).

##### Company proposals

Ericsson

Proposal 13: Deprioritize studies and discussion on over-the-air training between NW and UE.

Spreadtrum

Proposal 2: For model training for one-sided model, the following model training types can be further discussed:

- Type 0: Training at a single side/entity without model transfer

- Type 1: Training at a single side/entity, and model transfer to another side/entity

Proposal 3: For one-sided AI/ML model, model training and model inference at the same side should be considered as a starting point, i.e.,

- Network-side training for Network-side model

- UE-side training for UE-side model

Proposal 4: Offline AI/ML model training is the first priority.

Huawei, HiSilicon

Proposal 11: The discussion of online/offline training should be decoupled with whether the data collection/dataset delivery is performed via air-interface or non-air-interface.

Proposal 12: For the study of one-sided AI/ML model, model training without model transfer/delivery should be considered as a starting point, i.e.,

- On-Network training for Network-side model.

- On-UE training for UE-side model.

Google

Proposal 1: For 1-side mode, Rel-18 should focus on the scenario that the model inference and training are in the same side.

Sony

Proposal 2: RAN1 should study what signalling information would be needed for training and how to transfer an AI/ML model.

Intel

Proposal-5 (NW side involvement): Model training, particularly after an initially developed model is acquired, should be supported at a NW-side server that allows customization of a model to the local environment. Subsequent model transfer/delivery to the UE can happen through 3GPP or non-3GPP mechanisms

Oppo

Proposal 12: In the early stage of Rel-18 study, prioritize study of the AI/ML inference over the study of AI/ML training.

- Study offline training with high priority and as the default training type.

##### Issue 7-5: Discussion on online training and over-the-air training at the UE-side

(FL note: The following proposed conclusion is intended to capture analysis on the use case, benefits, challenges and specification impact of online training on the UE-side and over-the-air training)

###### [FL1] Proposed conclusion 7-5a:

The benefits and challenges of online and over-the-air training are described in the table below:

|  | Online training of a UE-side model by the UE | Over-the-air training of a UE-side model |
| --- | --- | --- |
| Use case | U1 | U1 |
| Benefits | B1 | B1 |
| Challenges / Requirements | C1, C2, C3, C4 | C1, C2, C3, C4,C5, C6 |
| Potential Spec Impact | S1, S2 | S1, S2, S3 |

Use cases:

- U1: Fast-time-scale adaptation of a deployed UE-side model

Benefits:

- B1: Can adapt to the data distribution of new scenarios quickly

Challenges / Requirements:

- C1: Requires advanced device capability of training the model

- C2: Potentially suboptimal performance and power consumption due to inability to quantize the model optimally.

- C3: Each UE’s model may evolve differently, leading to proliferation of many different models (LCM challenges) and lack of reuse across UEs.

- C4: No performance guarantee since updated model is not tested or validated.

- C5: Large air-interface overhead for training

- C6: Large specification impact

Potential Spec Impact:

- S1: Model identification, performance validation and monitoring, and other LCM impacts

- S2: Real-time training data collection – input and/or label

- S3: Real-time training related exchange over-the-air (e.g., activation / gradient)

| Company | Comments |
| --- | --- |
| vivo | Seems RAN2 has already down-prioritized online training. Other companies can also check whether this is correct understanding. |
| OPPO | We suggest to deprioritize online training, considering the limited time for Rel-18 study. The proposal is good reference for future study. |
| ZTE | It’s not an important issue. We don’t expect to specify the training procedure in the near future. We can simply down-prioritize the two training types in Rel-18. |
| Panasonic | If down-prioritizing means at least to capture this level, we support it to cover overall study. |
| Huawei, HiSilicon | We do not think this discussion is needed. There is no discussion at either evaluation agendas or spec impact agendas for use cases on the online training to dynamically update the model at UE device-it is beyond UE capability. |
| Google | We also do not know why this needs to be discussed. |
| Futurewei | RAN2 has reached the following conclusion about online/real-time train. “R2 will deprioritize aspects of on-line/real-time training for the whole SI (unless R1 identifies that it is needed for one of the studied use cases).” Note the wording “…for the whole SI…”. Therefore we don’t think it is necessary to discuss this. Also, we are not clear about the difference between “real-time training” and “over-the-air training”; does “real-time training” mean “real-time, but non-over-the-air training”? |
| Lenovo | We think they can be discussed in future, e.g., Rel-19. |
| LG | We are confused on the intention of the proposal. As captured above, RAN2 already agreed to deprioritize this study but RAN1 want to keep studying? |
| Ericsson | Share same view as Vivo. Propose to downprio. |
| Samsung | Fine in general. But this may only apply for initial model training (development) and may not apply for model update/finetuning. If so, that can be indicated. |

#### Two-sided model development and training

##### Previous agreements

## Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at Network side and UE side, respectively. - Type 3: Separate training at Network side and UE side, where the UE-side CSI generation part and the Network-side CSI reconstruction part are trained by UE side and Network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with Network side training [, or parallel training] at UE and Network - Other collaboration types are not excluded.

##### Company proposals

FutureWei:

Proposal 8: For the three types of two-sided model training, study and compare their performance, signaling overhead and potential standard impacts. There is no need for down selections.

Proposal 9: For Type 1 two-sided training, when the joint training is done at the network side, make the perform-at-network the baseline solution.

Huawei, HiSilicon

Proposal 13: For training Type 1 (joint training of the two-sided model at a single side/entity), prioritize the study of joint model training at Network side and transfer/deliver the model to the UE side.

Qualcomm

Proposal 36: Adopt the following two-sided model development/training framework:

Case 1: Initial (non-backward-compatible) development/training of “nominal encoder + nominal decoder”

  - The use of the nominal encoder at the UE-side is not mandated

    - If needed, UE-side may implement a different proprietary encoder based on this decoder using Case 2.

    - As the encoders are only nominal, input used in the training process is only a nominal input. The actual input to the CSI encoders may be different and of proprietary choice.

  - The use of the nominal decoder at the NW-side is not mandated

    - If needed, NW-side may implement a different proprietary decoder based on this encoder using Case 3.

Case 2: Encoder development/training to be interoperable with existing decoders (e.g., encoders for new UEs or updating encoders for existing UEs):

  - UE-side vendor trains new encoders based on the existing decoders.

  - Infra vendor should make the existing decoders available (via either a run-time image or an API for training) for the encoder training.

Case 3: Decoder development/training to be interoperable with existing encoders (e.g., decoders for new cell sites or updating decoders for existing cell sites):

  - Network-side vendor trains new decoders based on the existing encoders.

  - FFS: Need for encoder availability for decoder training

Samsung

Proposal #10: Deprioritize two-sided model training collaboration that requires extensive sharing over the air interface of training, validation and testing dataset.

Proposal #7: Consider the following and study their impacts for the two-side model development approaches,

- Requirements on privacy-sensitive dataset sharing

- Scalability, i.e., whether the number of models one vendor should develop increases with the collaborating vendors

- Whether the model development approaches adhere to 3GPP’s open and fair framework

#### Functionality/model identification and methods of LCM

##### Previous agreements

RAN1 #110-bis-e Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models - FFS: Detailed discussion of model ID with associated information and/or model functionality. - FFS: usage of model ID with associated information and/or model functionality based LCM procedure - FFS: whether support of model ID FFS: the detailed applicable AI/ML operations RAN2 #119bis-e agreement R2 assumes that a model is identified by a model ID. Its usage is FFS. RAN2 #119bis-e agreement R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. RAN1 #111 Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Funtionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs RAN1 #111 Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately. RAN2 #120 agreement R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. RAN2 #120 agreement R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN1 #112 Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model RAN1 #112 Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. RAN1 #112 Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. RAN2 #120 agreement R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. RAN2 #120 agreement R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN2 #121 agreement RAN2 assumes that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified RAN1 #112-bis-e Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. RAN1 #112-bis-e Conclusion From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. - When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. RAN1 #112-bis-e Agreement - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models. RAN2 #121-bis-e Agreement FFS if For UE capability for AIML methods we use the UE capability mechanisms as defined for RRC reported and LPP reported capabilities. RAN2 #121-bis-e Agreement R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information. RAN2 #121-bis-e Agreement Model ID can be used to identify model or models for the following LCM purposes: model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step). (e.g. for so called “model ID based LCM”) If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose. RAN2 #121-bis-e Agreement How to achieve globality of the Model ID is FFS. Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point: Direction1: Pre-defined/hard-coded global unique model ID Direction3: Assigned global unique model ID via specific ID management node. Note: Other global unique model ID definition is not precluded. Model ID structure, if any, is FFS

##### Company proposals

FUTUREWEI:

Proposal 4: A model ID is a unique index that differentiates one model from other models within a network. The model IDs may or may not be globally unique.

Proposal 5: Revise the statement of Type A of FL proposal 6-11f in [2] as below.

- Type A: NW-initiated model identification to NW (if applicable) and UE-initiated model identification to UE (if applicable) without over-the-air signaling.

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification.

  - FFS: Spec impact to other WGs including SA

  - Note: no matter which side initiates the model identification, models are identified at the network.

Proposal 6: Each model ID should be associated with a list of meta information that describes the functionalities, associated features, and other characteristics etc. of the model.

Proposal 7: RAN1 to focus on model identification in the SI phase and defer the study of functionality identification details till Rel-19 work item phase.

Huawei, HiSilicon:

Proposal 14: Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) are the conditions in forms of RRC IE(s) reported by UE capability. The following steps can be considered:

- Step 1: UE capability report of the supported functionalities in forms of a list of relevant RRC parameters.

- Step 2: Network activates one preferred functionality from the set of UE reported functionalities.

Proposal 15: Categorize the functionality identification modes into the following two modes

- Mode 1: Functionality identification-basic, where NO globally unique ID is needed.

- Mode 2: Functionality identification-enhanced, where globally unique scenario ID and/or globally unique dataset ID for representing additional conditions is needed.

Proposal 16: For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows:

- Type A: Model is identified between Network and UE without over-the-air signaling

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling for the LCM procedure after model identification.

- Type B: Model is identified via over-the-air signaling

  - Type B1: UE-initiated model identification of a model trained at the UE side

    - Model identification initiated by the UE, and Network assists the remaining steps of the model identification

  - Type B2: Network-initiated model identification of a model trained at the Network side

    - Model identification initiated by the Network, and UE responds (if applicable) for the remaining steps of the model identification

    - Note: Type B2 is used in conjunction with model transfer from Network to UE.

Proposal 17: Consider model identification as a parallel mode with functionality identification (i.e., model identification mechanism is not supported on top of functionality identification mechanism).

Proposal 18: For two-sided AI/ML model, UE can report the set of multiple UE part model IDs currently supported by the UE as well as UE’s preferences. From the set, gNB can select a UE part model that can pair with the Network part model at the gNB and achieves good performance.

ZTE:

- Proposal 6: Model identification process via over-the-air signaling should always be accompanied by the model transfer, model description information, and model ID assignment.

- Proposal 7: For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact.

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling.

  - The model is assigned with a global ‘unique’ model ID during the model identification, which may be used in over-the-air signaling after the model identification process.

- Type B: Model is identified from UE to NW via over-the-air signaling.

  - A model is transferred from a UE to network. The corresponding model description information (e.g., applicable conditions of the model) about the model is also provided along with the model transfer.

  - The model is assigned with a global ‘unique’ model ID by NW side.

- Type C: Model is identified from NW to UE via over-the-air signaling.

  - A model is transferred from network to a UE. The corresponding model description information (e.g., applicable conditions of the model) about the model is provided along with the model transfer.

  - A global ‘unique’ model ID to the model is also provided to UE from network side.

- FFS: UE capabilities to support the model identification process

- Note: This does not imply that model identification is necessary.

- Note: Model identification process may have specification impacts on other WGs including SA

- Proposal 8: The signaling framework for UE functionality/model alignment should consider various options to be compatible with future extensions to the rapid development of AI/ML technologies.

- Proposal 9: Further study the UE capability report for an AI/ML-enabled feature, including the following aspects:

- Whether applicable conditions of a functionality is determined during the UE capability report or after the UE capability report;

- Whether the model can be deployed before or after the UE capability report;

- Whether functionality/model can be configured by NW(i.e., via model transfer) after the UE capability report;

- How to report the UE capability for the model that is already identified before.

- Proposal 10: Further study the solutions to report UE capability for an AI/ML-enabled feature, at least including the following options:

- Option 1: UE indicates its supported candidate values for each condition (or component) of an AI/ML-enabled feature as the legacy UE capability report.

- Option 2: UE indicates its supported functionality of an AI/ML-enabled feature in a UE capability report, where the applicable conditions of a functionality are indicated by a combination of the conditions.

- Option 3: UE indicates its supported model IDs (e.g., global ‘unique’ model IDs) of an AI/ML-enabled feature in a UE capability report.

- Proposal 11: Support of NW to assign a local identifier to the functionality/model reported by UE or configured by NW (i.e., via model transfer). From network perspective, the local identifier is mapped to a function unit operated at UE that is not transparent to network no matter the function unit is associated with a functionality or a model.

- Proposal 12: Totally different signaling frameworks for functionality-based LCM and model-ID-based LCM are not preferred. Support of a unified signaling framework based on the local identifier assigned by NW for various LCM procedures, e.g., measurement report, performance monitoring, activation/deactivation, selection, or switching.

- Proposal 18: UE can indicate its applicable functionalities/models by indicating the local identifier associated with the functionalities/models.

Vivo:

Proposal 1:	It is not needed to address additional conditions in functionality based LCM.  Functionality is used to handle the case where the AI/ML capabilities are statically implemented in UEs, while for the case with dynamic additional conditions, model identification is more appropriate.

Proposal 2:	Signaling to support functionality-based LCM operations can be discussed in each sub use cases and should be studied in WI.

Proposal 3:	Functionality-based LCM and model-ID-based LCM should be designed to follow one general framework and use same aspects as many as possible.

Proposal 4:	The scenarios where updates on applicable functionality(s) may be needed should be clarified among companies to study the necessity.

Proposal 5:	Study updates on applicable model(s) for the case where UE power consumption or computation resources are dynamically varying.

Proposal 6:	Functionality-based LCM contains two steps as legacy UE features, UE capability report and RRC/MAC-CE/DCI enabling/disabling procedures.

Proposal 7:	Consider to define the procedures as in Figure 2-1 for model/applicability -based LCM, which contains the following four steps:

- Step1: UE AI/ML-enabled feature report to network;

- Step2: Alignment of additional conditions between network and UE;

- Step3: LCM control or assistance by network using model ID or applicability ID, including model selection or switching;

- Step4: Additional conditions updating during usage.

Figure 2-1. The generally procedure of the functionality/model/applicability -based LCM.

Proposal 8:	Consider the following model identification types.

- Type A (Step0 in Figure 2-1): Model is identified offline to network (if applicable) and/or UE (if applicable).

- Type B (Step1, Step2b and Step4 in Figure 2-1): Model is identified via signaling from UE to network.

- Type C (Step2a, Step2b and Step4 in Figure 2-1): Model is identified via signaling from network to UE.

Oppo:

Proposal 1: Support both functionality-based LCM and model-ID-based LCM.

Proposal 2: Local ID is supported for indication of model selection/activation/deactivation/switching/fallback within an AI/ML functionality.

- FFS: Local ID-based model indication across multiple functionalities.

Proposal 3: The AI/ML functionality identification and configuration procedure can be as below:

- (1) UE reports the static UE capability on supported AI/ML functionalities;

- (2) NW configures the identified functionality list according to the UE capability and NW capability (or NW’s interest);

- (3) UE updates the dynamic UE capability on applicable AI/ML functionalities (optional);

- (4) NW configures the sub-set functionality list according to the updated UE capability and NW capability (or NW’s interest).

- FFS: Whether Step (2) is needed if Step (3) is performed.

Proposal 4: Support Local ID-based model identification.

- Focus on design of Local ID-based model identification assuming non-3GPP-based model transfer.

- FFS: 3GPP-based model transfer/training.

  - First focus on following aspects:

    - Required KPI (e.g. packet size, data rate, latency, reliability), so to select the design (e.g. in which layer/channel).

    - Model transfer format (if needed).

  - Study AI/ML model transfer for training with lower priority.

Proposal 5: At least for LCM with non-3GPP-based model transfer,

- Local ID can be a simple number, which is similar to the resource/configuration ID in the legacy NR specification and does not include explicit information about the model, e.g., use case, scenario, configuration.

- FFS: Whether Global ID is needed and whether the Global ID needs to be defined in 3GPP specification.

Proposal 6: For LCM with 3GPP-based model transfer,

- Local ID can be a simple number, which is similar to the resource/configuration ID in the legacy NR specification and does not include explicit information about the model, e.g., use case, scenario, configuration.

- Global ID may contain the information about the model (explicitly or implicitly).

  - FFS the information, e.g., use case, scenario, configuration.

- FFS: The mapping between the two types of IDs.

Spreadtrum:

Proposal 5: For AI/ML model/functionality identification, model ID (for model identification), model function, model applicable condition, model input, and model output can be considered to be included into capability information.

Proposal 6: For functionality identification, other information such as applicable condition can be denoted in the form of components of one FG or FG.

Proposal 7: Legacy signaling, e.g., RRC signaling for CSI reporting configuration, can be utilized for functionality-based LCM operation.

Proposal 8: For model identification,

- Regarding UE-sided model, model description along with (local) model ID can be reported by UE, and then global or local model ID can be allocated by NW;

- Regarding UE part of two-sided model, model ID is reported by UE with the assumption that offline coordination and/or model transfer are considered.

Proposal 9: Capability reporting for Model identification can be considered to correspond to one subset of capability reporting for functionality identification.

Proposal 10: Applicable functionalities are reported from UE as a subset of identified functionalities.

Proposal 11: Dynamic UE capability update on applicable functionality(es) should be considered.

Nokia:

Proposal 2: At least for UE-side models and UE-part of two-sided models, define the sub-use case specific conditions for functionalities and identify the common conditions for functionalities across different sub-use cases.

Proposal 3: At least for UE-side models and UE-part of two-sided models, the conditions for functionalities shall contain the following (see Table 4),

- Conditions on inference (use case specific)

- Conditions on Performance monitoring (use case specific)

- Conditions on functionality configurations (generic)

- Conditions on functionality validation procedure (use case specific)

- Conditions on supporting fallback (use case specific)

- If applicable, context information (use case specific)

Proposal 4: For functionality identification and functionality-based LCM, the UE capability reporting is to be used only for reporting static conditions.

Proposal 5: If model identification is supported via 3GPP signaling, RAN1 to study an additional reporting method (not the UE capability reporting) for reporting additional conditions.

Proposal 6: At least for UE-side models and UE-part of two-sided models, RAN1 to align with the understanding that the NW configures functionalities to the UE with each functionality referring to a configuration message (e.g., RRC or LPP) that contains NW-selected conditions (according to the UE capability).

Proposal 7: At least for UE-side models and UE-part of two-sided models, after functionality identification (i.e., after UE capability reporting and functionality configuration(s) are available at the UE), study reporting methods of applicable functionalities.

Proposal 10: RAN1 to discuss dynamic methods, triggered by the NW or the UE, for verifying compatibility of the Functionalities at the UE-side and gNB-side for the use cases with two-sided ML models.

Proposal 11: At least for UE-side models and UE-part of two-sided models, for the case of no model identification, to ensure performance is not impacted due to UE’s autonomous model-LCM changes (e.g., model update, switch) for a used/activated functionality, RAN1 to support NW to track the performance variations due to the UE’s model-based LCM changes and partially control UE’s model-based LCM changes.

- Study the exact performance monitoring framework and control mechanism to enable such a controllability

Proposal 12: At least for UE-side models and UE-part of two-sided models, prior to supporting 3GPP signaling details for (logical) ML model identification, RAN1 shall define and identify the additional conditions where this would be needed for the studied sub-use-cases.

CATT:

Proposal 9: Further study the necessity, feasibility and detailed procedure of the following three types of model identification for UE-side or UE-part of two-sided models:

- Type A: Model is identified to NW and UE without OTA signaling.

- Type B: Model is identified from UE to NW via OTA signaling.

- Type C: Model is identified from NW to UE via OTA signaling, in conjunction with model transfer.

Proposal 10: From RAN1 perspective, the following information shall be considered for metadata of an AI/ML model:

- Applicable conditions

  - Associated AI/ML-enabled Feature/FG.

  - Association with specific configurations/conditions, including, e.g. configuration for nominal input and/or nominal output (e.g. payload size for CSI compression), configuration for RS measurement, configuration for quantization, etc.

  - Additional conditions, e.g. scenarios, sites, datasets, paring information (for two-sided model, if applicable), concurrent use with other AI/ML models and/or non-AI/ML features.

- Model performance indicators

  - Performance of inference accuracy or system performance, which may be hypothetic or predicted one.

Proposal 11: From RAN1 perspective, a local model ID can be allocated to an AI/ML model temporally for model-ID-based LCM, rather than directly apply the global ID assumed by RAN2.

Proposal 15: Functionality identification reflects the applicable conditions for supported AI/ML functionality at UE side indicated by UE capability, which does not include subsequent procedures like functionality configuration reflecting NW’s interest.

Proposal 16: Functionality corresponds to a specific configuration or a set of configurations of the AI/ML enabled Feature/FG.

Proposal 18: For functionality-based LCM, network does not need to know about the model-level LCM at UE-side. Instead, functionality performance should be monitored by network, or monitored by UE but reported to network.

Proposal 20: Both functionality and model shall refer to AI/ML-enabled Feature/FG. No need to identify a model via referring to functionality.

Proposal 22: Only the applicable model(s)/functionality(es) can be updated by UE report. Do not extend this mechanism to other features of UE capability.

Proposal 23: For reporting the applicable functionalities, at least further consider the mechanism of reporting applicable functionalities among identified functionalities.

Intel:

Proposal-3 (physical model): The concept of a physical model is relevant for model development, training and transfer (specification impact outside RAN1), in other contexts (that are primarily impacting RAN1 specifications) a logical model concept is sufficient.

Proposal-4 (model-ID). From RAN1 perspective there is no need for uniqueness of a model-ID (in model-ID based LCM) since the meta-information for the model clarifies the distinction between different UEs. The namespace for a model-ID for L1 signaling purposes depends on specific use-cases and can wait for later discussions.

Proposal-7 (Model identification types): Model identification to NW/NW-side is a process through which NW acquires meta information associated with a physical or a logical model. For model identification to UE/UE-side clarify the relevant use-cases (whether it is z4/z5).

Proposal-8 (offline email- functionality LCM): Clarify that functionality identification is a process by which a UE identifies (advertises) its capabilities to the NW based on the current UE capability framework. A configuration and UE reporting of applicability may determine the granularity associated with a functionality (which could be dynamic). However, the configuration and reporting process should not affect a functionality identification process.

Proposal-9 (model ID LCM): A model-ID based LCM requires a model identification process – in contrast, a functionality based LCM is not associated with an identification procedure, it is not applicable for model transfer/delivery (collaboration level-z) and it is not applicable to two-sided models

Proposal-10 (relationship between model ID and functionality): we suggest to develop model-ID based LCM and functionality based LCM procedures separately – it is not critical to address how they work with each other at this time. whether a model may be or must be identified referring to functionality(s) can also be postponed until further development of the LCMs.

InterDigital:

Proposal 1: Mechanism for the UE to report updates on applicable functionality(ies) can be studied after the details of additional conditions are agreed.

Proposal 2: At least for some LCM procedures (e.g., model (de)activation, model switching) locally unique (at least within the UE) Model ID may be sufficient and globally unique model ID may not be necessary.

Proposal 3: The exact scope of model ID used for model transfer/update can be studied in RAN2.

Sony:

Proposal 5: RAN1 should consider supporting the individual functionality-based LCM and the common functionality-based LCM for indication of activation/deactivation/switching/fallback.

Proposal 6: RAN1 should study which LCM can support the common functionality-based LCM.

Ericsson:

Proposal 2 Conclude that a possible frequent update of models or applicable conditions (if needed) does not imply frequent updates in UE capabilities within the existing capability framework.

Proposal 3 RAN1 should focus on outlining the of conditions part of “static” UE capability signalling, and which conditions that could be handled via a “dynamic” approach. Exact signalling details are up to RAN2

Proposal 4 Study on a use-case basis the content of conditions that are part/not part of UE capability

Proposal 5 Conclude that functionality-based LCM actions such as activation/deactivation/switching is based on NW (re)-configuration.

Proposal 6 Model identification (“physical model” ID or “logical model” ID) for a model trained and monitored at the UE-side is not needed.

Proposal 7 Functionality identification is sufficient for a model trained at UE-side but monitored (if needed) at the NW-side. Model identification (“physical model” ID or “logical model” ID) is not needed.

Proposal 8 Methods for supporting pairing of compatible UE part and NW part of a two-sided model should be studied (e.g., for CSI-compression, selecting an encoder of a connected UE to pair with a decoder used by the serving gNB of a network vendor).

Proposal 9 Model identification (“physical model” ID or “logical model” ID) is not needed for two-sided models. Study methods to support pairing of the UE-part and NW-part of a two-sided model.

Proposal 24 Support of ML model-based functionalities, but not related model details, may be reported using the UE capability framework.

Fujitsu:

Proposal-3: Study the procedures and potential STD impacts of applicable functionality referring to the assumptions in Alt 1 proposed by FL.

- Configurable functionality is synonymous to identified functionality.

- Configured functionalities are determined by NW as a subset of identified functionalities.

- Applicable functionalities are reported from UE as a subset of configured functionalities.

- NW activates one functionality out of applicable functionalities.

Proposal-4: Regarding functionality-based LCM operations, at least the following approaches are studied:

- NW-side performance monitoring at functionality level and following-up actions.

- UE initiated the request on functionality operations to NW based on its model-level-based monitoring.

Proposal-5: Regarding model-ID-LCM,

- Global model ID is assumed to be used in model identification/model registration procedure.

- Local model ID is introduced to facilitate model activation/deactivation/selection/switching.

- FFS: the relationship between the global model ID and the local model ID.

Xiaomi:

Proposal 3: one feature refers to one sub-use case.

Proposal 4: One model could support one or more than one functionalities.

Proposal 5: Which aspects should be specified as conditions of a Feature/FG  for functionality will be discussed in each sub-use-case agenda during normative work.

Proposal 6: Consider the following steps / components for the functionality-based LCM

- Step 1: Identify the applicable functionality

- Step 2: Functionality selection and Functionality configuration

- Step 3: Functionality activation

- Step 4: Functionality(model) switch/fallback/deactivation；

- Performance monitoring and functionality management for all the steps

Proposal 7: Additional conditions could help the functionality management and the model management

- Additional condition of the functionality can be reported to network during functionality identification

- Mechanism to acquire additional condition from network should be supported to facilitate the model operation on UE side

Proposal 8: Categorize the model identification for the case without model delivery/transfer and the case with model delivery/transfer separately

- Consider the following model identification types for the case without model delivery/transfer

Proposal 9: Confirm the necessity of defining local model index. The detailed design can be left to WI phase

Proposal 10: Confirm the necessity for UE to report updates on applicable functionality(es) / applicable model(s) among [configured/identified] functionality(es) /model(s)

Proposal 11: Further study the following directions for the report updates on applicable functionality(es) and applicable model(s)

- Proactive report

- Passive response

Panasonic:

Proposal 1:  When the AI/ML model is trained is different NW or UE side with the selection of AI/ML model, the AI/ML model is expressed by the training data and expected output. This AI/ML model is logical AI/ML model. More concrete case is the AI/ML mode is trained in UE side and the selection of AI/ML model is NW side.

Proposal 2:

- In functionality based LCM, UE determines what physical AI/ML model is used for what conditions/site. UE may use other than AI/ML model for the specific condition and/or the selection of the condition.

- In logical model-ID-based LCM, NW determines what logical AI/ML model is used for what conditions/site. UE further determines what physical model is used.

- In physical model-ID-based LCM, NW determines what physical AI/ML model is used for what condition/site. The AI/ML model may be only the parameters management.

Proposal 3:  For functional LCM, validation may be similar to non-AI/ML operation as all adaptation to the real deployment is rather UE side responsibility. For model-ID-based LCM, the validation using specific UE is required as a kind of test trial. This test trial can be skipped when the same AI/ML model is used for the same manufacture's same model in the same software version. This test trial is required even if the model is trained by the NW as the compilation, quantization and alignment to the UE internal processing related to AI/ML operating environment.

Proposal 5: Model update is restricted to the process updating the model parameters and/or model structure of a model of the same usage/purpose/applicable condition. It is same meaning with version update. When different usage/purpose/applicable condition is applied, it is new model.

Proposal 6: Model identification is "a process/method of identifying an AI/ML model for the common understanding of the usage/purpose/applicable condition between the NW and the UE".

LG:

Proposal #2: Clarify that the key difference between model-based LCM and functionality-based LCM is on whether LCM signaling shall be triggered whenever status of model(s) is changed, e.g. via activation/deactivation/switching or not.

Proposal #3: Before going into details of LCM signaling, identify the case(s) when model-level LCM is needed and when it is not needed, i.e. functionality-based LCM is sufficient.

Proposal #4: Functionality-based LCM should be adopted as a baseline approach, which is applicable for most cases. On top of that, model-based LCM can be considered for some special cases, e.g. two-sided model and/or model transfer scenario, with more focused work scope.

Proposal #6: For the granularity of functionality, start from FG defined for UE capability, and further consider whether a functionality can cover multiple performance reference of the same feature or not.

Proposal #7: Consider dynamic reconfiguration of UCI reporting for the case of functionality switch/update for the same sub-use-case.

Samsung:

Proposal #3: Study functionality-based LCM for UE-side model where

Alt 1: UE reports the supported AI/ML functionalities

Alt2: UE reports the supported AI/ML functionalities by mapping them to logical models. i.e., by indicating the group of functionalities supported by a single model.

Note: Logical model implies a reported AI/ML model. UE may transparently have multiple implementations of a logical model.

Proposal #4: For UE side and UE part of two-sided models, study mechanisms to manage

- Timeline and delay requirements for AI/ML operations, e.g., AI/ML model/functionality activation, switching,

- Processing capability for concurrently activated AI/ML models/functionalities

Proposal#5: For UE-sided AI/ML models functionality-based LCM is adopted.

  - Network provides LCM assistance on the basis of specified AI/ML functionalities.

  - Capability reporting relies on the specified list of functionalities.

FFS: whether UE reports the supported AI/ML functionalities or the supported AIML functionalities by mapping them to logical models. i.e., by indicating the group of functionalities supported by a single model.

Proposal#6: For two-sided AI/ML models, differ the conclusion on whether to adopt model-ID or functionality based LCM after evaluating the feasibility of

  - Case1: Two-sided models that work in a vendor-specific manner.

  - Case2: Two-sided models that work in a vendor-agnostic manner.

CAICT:

Proposal 4: A model could be identified referring to the functionality(s) with the same application conditions.

Proposal 5: For the same application conditions for the same AI/ML-enabled Feature, it is not necessary to operate functionality-based LCM and model-ID based LCM simultaneously.

Proposal 6: Functionality ID could be assigned to identified functionality(s) for functionality-based LCM.

ETRI:

Proposal 1: For the LCM of AI/ML in NR air interface, study whether/how to report application conditions of AI/ML that depend on scenario/site/dataset through a separate process other than UE capability reporting.

Proposal 2: For the LCM of AI/ML in NR air interface, study the following two-step identification process:

- Step 1: AI/ML functionality identification step

  - The gNB sends a UE capability enquiry to the UE.

  - The UE reports UE capability information to the gNB.

  - UE capability information includes supportable configurations for each AI/ML functionality.

- Step 2: AI/ML model identification step

  - The gNB sends configurations related to AI/ML functionalities.

  - Configurations related to AI/ML functionalities include functionality ID

  - The UE reports model ID(s) that can be supported for the configurations/functionality ID.

Proposal 3: For the LCM of AI/ML in NR air interface, study AI/ML operation scenario and/or operation area identification during the AI/ML model identification.

Proposal 4: For the LCM of AI/ML in NR air interface, study the following two options for AI/ML model information report procedure during/after the AI/ML model identification.

- Option 1: AI/ML model information report in response to the AI/ML related configuration

- Option 2: AI/ML model information report in response to the AI/ML model enquiry

Proposal 5: For the LCM of AI/ML in NR air interface, study a unified procedure to support both functionality-based LCM and model-ID based LCM.

CMCC:

Proposal 7: For the model description information during model identification, the following aspects could be considered:

- Model functionality

- Model applicability scenarios, configurations

- Information on model input

- Information on model output

- Information on assistance information

Proposal 8: For functionality identification, the functionality ID can be assigned by the network to facilitate functionality-based LCM procedure.

Proposal 9: For the description information during functionality identification, the following aspects could be considered:

- Functionality

- Applicability scenarios, configurations of models for the functionality

- Information on model input type(s)

- Information on model output type(s)

- Information on assistance information

Proposal 10: Functionality identification is based on the UE capability UE reported and the associated applicable conditions for each functionality.

Proposal 11: Configured functionalities is a subset of identified functionalities, as identified functionalities refers to what NW could potentially configure to UE, and configured functionalities refers to what NW actually configures to UE as a subset of identified functionalities.

Proposal 12: UE could report the updates on the identified functionality(es) in a more dynamic manner than UE capability report.

MediaTek:

Proposal 16: The global model ID is a permanent ID, which is assigned and managed by the network.

Proposal 17: The global model ID for each AI/ML model is used for the following purposes:

- Model test certification

- UE capability reporting to indicate which AI/ML model is available at the UE

- Model paring of the AI/ML models between the UE and network for two-sided model

Proposal 18: Model Identification/Registration should be done before model transfer/delivery.

Proposal 19: A model index is assigned to each model for model activation/deactivation/switch/selection by the network through model configuration.

Proposal 20: Study what associated information needs to be provided through model identification for the case that UE sided model is generated and training at the UE side and leave the model identification procedure to RAN2 discussion.

Proposal 21: For each AI/ML model, at least following associated information needs to be known. FFS on other information.

- Applicable AI/ML-enabled Feature/FG

- Information on model input

- Information on model output

- Information on pairing between UE-sided part and network-sided part of two-sided model

Proposal 22: For model/functionality identification for UE-sided model when it is generated at the UE sided (over OTT server)， UE report updates on applicable functionality(es)/model(s) has following options:

- Option 1: Model/functionality identification through coordination between the server and RAN/CN during model delivery

- Option 2a: Model/functionality identification with UE report updates on applicable functionality(es)/model(s) over RRC messages to RAN

- Option 2b: Model/functionality identification with UE report updates on applicable functionality(es)/model(s) over NAS message to CN

Nvidia:

Proposal 8: Coordinate with SA5 on AI/ML model life cycle management.

Proposal 15: For AI/ML based enhancements for NR air interface, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

Apple:

Proposal 1: Use needForGap UE capability report framework at a starting point to support scenario-specific, site-specific, configuration-specific and/or dataset-specific AI model UE capability report.  If UE indicate the configuration/scenario/site is NOT supported in RRCReconfigurationComplete, NW should not configure the AI functionality or AI model.

Proposal 2: Use UAI framework as a starting point to feedback additional UE preference due to temporary model un-availability such as desirable UE power saving, model concurrency etc.

Proposal 3: Define functionality-based LCM and model ID based LCM independently.

Proposal 4: Functionality granularity is sub-use case dependent and can be defined in later stage of WI similar to legacy UE feature discussion.

Proposal 5: Functionality based LCM procedure can be used for one sided model without model transfer.

Proposal 6: Model ID based LCM procedure can be used for two-sided model, and one-sided model with model transfer.

Proposal 7: AI model identification can be done between vendors/operators during offline training collaboration for two-sided model, or part of feature alignment for one sided model for known model structure.

Proposal 8: RAN1 discussion focus on the use cases and requirement for model ID.

Proposal 9: 3GPP define model ID and model description. For two-sided model, the model ID is to facilitate the corresponding UE part or NW part model paring in LCM procedure.

Proposal 10: For two-sided model, the model ID can be defined via:

- NW part model with NW vendor identification, and other applicable conditions.

- UE part model with UE vendor identification, and other applicable conditions.

- Both NW part model and UE part model, i.e., explicating link information is listed, and other applicable conditions.

Proposal 11: Model description include scenarios/configurations for model inferencing, model input/output information, model file type/size/compression status etc.

Lenovo:

Proposal 2: The information and signaling exchanging between NW and UE for Model-ID-based LCM can be tagged with a Model ID to facilitate LCM.

Proposal 3: The model-related information, i.e., model description, to be shared during model identification needs to be discussed per sub use case.

Proposal 4: Study the feasibility and values of the following for all sub use cases as the information to be shared during model identification:

- AI/ML-enabled Feature(s), e.g., configurations and applicable conditions

- Properties of nominal model input/output, e.g., quantization format

- Assistance information, e.g., type of labeled data for monitoring

- Model complexity, e.g., FLOP

Proposal 6: The information, i.e., configurations and application conditions, shared in functionality identification needs to facilitate functionality-based LCM.

Proposal 7: The configuration of AI/ML-enabled Feature/FG’ for the functionality is a set of system setting parameters indicated by RRC IEs, and the detailed configurations need to be studied per use case in the normative stage.

Proposal 8: Study the approaches to indicate the applicable conditions for the AI/ML functionalities per sub use case to facilitate functionality-based LCM.

Proposal 9: Functionality-based LCM can provide feasibility for UE to select AI/ML model, based on the assistance information after identification procedure.

Proposal 10: To update an identified functionality, an identifier, e.g., Functionality ID, can be introduced to facilitate the signaling exchanging between NW and UE.

Proposal 11: A set of dedicated signals, e.g., RRC signaling, can be defined to update the identified functionality.

Proposal 12: Study the feasibility of functionality-based only, model-ID-based only and joint functionality-based and model-ID-based LCM alternatives for each sub use case.

Proposal 13: A hierarchical Model ID, one level is for functionality indication and the other level is for the multiple models within the functionality, can be applied for both Model ID based LCM and Functionality based LCM.

Qualcomm:

Proposal 3: Similar to the UE capability signaling for existing features, UE may indicate the static envelope configuration for AI/ML-enabled features representing the possible supported configurations at the UE.

Proposal 4: Functionality can be seen as a unit for activation/deactivation/switching, that may correspond to a specific configuration or a set of configurations.

Proposal 5: Functionality identification reflects conditions indicated by UE capability.

Proposal 6: UEs report updates on applicable functionalities as necessary or whenever NW inquires.

Proposal 7: The source gNB may request UE to provide the functionalities applicable by UE for the target and candidate cells in case of handover. Alternatively, applicable functionalities may be requested by the target cell.

Proposal 8:

| Meta information | Supplemental information being provided about a model during model identification process. |
| --- | --- |

Proposal 9: UEs can utilize the identified model IDs to indicate the applicable models at one time to the network.

Proposal 10: Adopt the three Types of model identification categories – Type A, B1, and B2.

Proposal 11: Consider the Table 1 to associate “initial model identification” and “updated model identification” with model identification types.

Table 1: Model identification types with initial and update model identification

|  | Initial model identification | Updated model identification |
| --- | --- | --- |
| Type A | Applicable | Applicable |
| Type B1 | Applicable | Applicable |
| Type B2 | Not applicable | Applicable |

Proposal 12: Deprioritize B1 for initial model identification.

Proposal 13: Deprioritize further discussion of Type B1 for updated model identification.

Proposal 14: During the initial model identification procedure, the network should map the provided meta information with the assigned ID for subsequent operations.

Proposal 15: Model description information may not contain the related functionality if model identification is done offline since functionalities can be created with over-the-air signaling.

Proposal 16: Consider Table 2 for configurations and additional conditions of the relevant sub-use cases.

Table 2: Configurations and additional conditions for sub-use cases

|  | Conditions of a Feature/FG available for functionality | Additional conditions (e.g., scenarios, sites, and datasets) |
| --- | --- | --- |
| CSI compression | Configuration related to CSI feedback analogous to legacy framework (e.g., CSI-RS measurement config, CSI reporting config – subband, antenna port layout, rank restriction, payload configuration) | Pairing ID to identify compatible UE-part; Other ID for network settings / scenario / site / dataset, if not captured in pairing ID. |
| Spatial domain beam prediction & Temporal domain beam prediction | Configurations for Set A, Set B (including configuration of associated RS and associated signaling/report) [Assistance information from NW to UE, including codebook ID, association/mapping/relationship between beams within Set A and beams within Set B (e.g., relative beam pointing angles of beams within Set A and beams within Set B), cell ID] | [Assistance information from NW to UE, including codebook ID, association/mapping/relationship between beams within Set A and beams within Set B (e.g., relative beam pointing angles of beams within Set A and beams within Set B), cell ID] |
| Direct AI/ML positioning | Assistance data/information considered in legacy NR-RAT UE-based positioning (e.g. DL PRS config., etc.) (from LMF to UE) | Validity area for direct AI/ML positioning (expressed as zone info [lat./long./alt.] or cell-ID[phy./global]) (from UE to LMF and/or from LMF to UE) |
| AI/ML-assisted positioning | Assistance data/information considered in legacy NR-RAT UE-assisted positioning (e.g. DL PRS config., reporting config. etc.) and new measurement reporting type(s) and configurations (from LMF to UE) | Validity area for AI/ML assisted positioning (expressed as zone info [lat./long./alt.] or cell-ID[phy./global]) (from UE to LMF and/or from LMF to UE) |

Proposal 17: Functionality-based LCM is applicable to UE-side models in Collaboration Level y.

Proposal 18: Functionality activation, deactivation and switching are NW implementation based on identified functionalities assocociated with UE capability and do not need to be studied in this agenda item 9.2.1.

Proposal 19: NW can provide dataset ID for additional conditions that are not a part of configurations.

Proposal 20: The model ID and associated model description information can be utilized for various LCM steps for model management and control purposes.

Proposal 21: Models are identified by model IDs, and associated meta information known at the network is used for selection of the right model at the inference time.

Proposal 22: Model-ID-based LCM is useful for one-sided models when additional conditions such as scenarios, datasets, custom configurations are addressed outside specification, via vendor collaboration.

Proposal 23: Model-ID-based LCM is applicable for both one-sided and UE-part of two-sided models.

Proposal 24: Model-ID-based LCM is applicable to Collaboration Level z.

KDDI:

Proposal 1：

Study how to link functionality and Feature/FGs in a functionality-based LCM.

Proposal 2:

Functionality activation/deactivation/switching/monitoring in the case of Alt1 and Alt2 is done as follows

Alt 1

- Functionality Activation

  - Activate one of the Applicable functionalities.

- Functionality Deactivation

  - Deactivate an Activated functionality.

- Functionality Switching

  - Switching to one of the Applicable functionalities.

- Functionality monitoring

  - The functionalities to be monitored are a subset of the Applicable functionalities.

  - Note: A subset may be a full set.

Alt 2

- Functionality Activation

  - Activate one of the Configured functionalities.

- Functionality Deactivation

  - Deactivate an Activated functionality.

- Functionality Switching

  - Switching to one of the Configured functionalities.

- Functionality monitoring

  - The functionalities to be monitored are a subset of the Configured functionalities.

  - Note: A subset may be a full set.

Rakuten:

Proposal 4
Model ID should be changed upon the model update so that UE could know the model is updated.

Proposal 5
Performance monitoring should be done with awareness of model ID.

Proposal 6

Both gNB and UE can indicate model ID depending on the framework deployed.

Proposal 7

Model ID should be able to be indicated separately with functionality ID, in addition to the indication within functionality ID.

Proposal 8

Functionality should be related with the model ID and the collaboration level.

Figure 1 Relationship between different model definition

Proposal 9

If functionality identification is performed within 3GPP framework, it should be performed before the model identification, if two different approaches coexist in in AI/ML operation for the same use case.

From perspective of the model identification, the problem is how to identify model structure. There are multiple possibilities to solve the problem:

- Solution 1: UE driven - identify model structure identically in functionality identification,

- Solution 2: NW driven - identify model structure identically in functionality identification,

- Solution 3: identify model structure by differentiating model ID with consideration of model structure,

- Solution 4: UE indicate deployed model structure to the network, separately,

- Solution 5: Model performance requirements are specified in spec. for each use case. UE selects appropriate model structure that satisfies specified requirements, without awareness from the network.

In solution 1, based on the information about available AI/ML use cases, UE decides model structure to apply. On the other hand, in solution 2, the network decides the model structure to apply based on the report from UE about AI/ML related capability.

There are other ways without involving functionality identification. In solution 3, model structure is identified by indicating model ID, by including model structure ID information to the model ID (hierarchical ID). In solution 4, model structure is identified by any schemes outside the 3GPP specification. In solution 5, only the model performance is specified with applicable model structure. By referring to the specification, applied model structure is identified depending on the use cases.

Proposal 10

Discuss further about preferable solutions to identify model structure with/without functionality identification.

AT&T:

Proposal 4:  Characterize AI/ML model identification as a function of the associated information needed for the common understanding between the UE and the network, used for AI/ML LCM procedure

Proposal 5: Define three levels of AI/ML model identification categories: model functionality, AI/ML untrained model structure, and AI/ML trained model.

Proposal 7:

For UE-side models or UE part of two-sided models, functionality identification is based on the conditions indicated by the UE capability report.

FFS: Whether/how to report the NW capability [and NW interests] to UE after functionality identification for UE-side models or UE part of two-sided models

Proposal 8: A functionality refers to a specific configuration of the Feature/FG or a set of configurations of Feature/FG that can be activated/deactivated/switched using functionality-based LCM.

Proposal 9: The term functionality(/ies) used at various part of functionality-based LCM can be referred as

- Identified functionalities: Set of functionalities indicated by UE capability report during functionality identification.

  - This term corresponds to all functionalities for a given Feature/FG that can be supported by the UE.

- Configured functionalities: Set of functionalities that can be configured by the NW.

  - This term corresponds to the set of functionalities that are the intersection of UE capability and NW capability (NW interests). It is a subset of identified functionalities.

- Applicable functionalities: Set of functionalities that are currently applicable among the configured/identified functionalities.

  - This term corresponds to the set of functionalities that can be activated for the given Feature/FG for the current time. It is a subset of configured functionalities.

- Activated functionality: The functionality that is currently activated from the set of applicable functionalities.

Proposal 11: Study the use of a local model ID that can be used for model control purpose (model activation/deactivation/switching/selection) after model identification.

- FFS: Any relationship between local model ID and global model ID.

- FFS: If local model ID is identified for common understanding between NW and UE during model identification or after model identification.

- FFS: If local model ID can be used to indicate compatibility between UE part and NW part of two-sided models.

Proposal 12: Functionality based LCM procedure is used for both UE-sided and two-sided models.

Proposal 13: Functionality-based LCM meta info, including AI/ML-enabled features/FGs indicated by capability signaling, their applicable conditions, and model structure (if provided) can be optionally associated with one or multiple model IDs in order to support both functionality identification and model identification as well as LCM functions including model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, model-specific performance monitoring, and testing and calibration performed by a network operator.

Proposal 14: For UE-side models and UE-part of two-sided models, functionality-based LCM is used.

Proposal 15: For UE-sided models, model-ID-based LCM procedure can be optional.

Proposal 16: For two-sided models, both functionality and model-ID-based LCM are used.

Proposal 17: For CSI prediction using UE sided model study the following configurations and their granularity that will be signaled through the functionality, and the corresponding specification impact in functionality-based LCM

- UE speed

- Frequency PRB’s

- Prediction window

- Observation window

- Scenario (Uma etc.)

- Performance requirement/monitoring

- Other additional configurations

Proposal 18: Study the necessity for model-ID based LCM to support any additional functionalities that will not be supported through functionality-based LCM.

NEC:

Proposal 2: Study model-ID based lifecycle management with a higher priority.

Proposal 3: For a two-sided model, study methods to align NW part and UE part of one AI/ML model, e.g., including NW part ID and UE part ID as parts of the model ID.

Proposal 5: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

NTT Docomo:

Proposal 1: Conditions indicated by UE capability should include all static information that NW should be aware of for the NW operation. At least, the following information should be reported as the condition in UE capability.

・Nominal input information such as required measurements and assistance information for model input

・Nominal output information such as derived information from the functionality

・Applicable configuration/deployment associated with functionalities

Proposal 2: AI/ML enabled feature can be defined by input type and output type.

Proposal 3: Regarding how to address the additional conditions, focus on UE reporting of the applicable functionality update according to additional conditions rather than reporting additional conditions themselves.

Proposal 4: Performance requirement under the indicated condition (e.g., configuration/deployment) should be verified by the test.

Proposal 5: Further study the following functionality-based LCM procedure.

・Step1: UE capability report of conditions on functionalities (= identified functionality)

・Step2: NW configures some or all identified functionalities (= configured functionality)

・Step3: UE indicates the applicable functionalities among configured functionalities

・Step4: NW determines which functionality is activated among applicable functionalities

Proposal 6: Model level LCM within a functionality should not include the fallback operation.

Proposal 7: Discuss the pros and cons of following potential relationship between model ID-based LCM and functionality-based LCM.

・Relationship 1: Hierarchical LCM framework.

- Functionality-based LCM is basic, and model ID-based LCM is optional.

- Model = functionality + additional condition

・Relationship 2: Unified LCM framework.

- Functionality-based LCM is the same as model ID-based LCM.

- Model = functionality

・Relationship 3: Independent two LCM frameworks

- Functionality-based LCM and model ID-based LCM are independent

- Model ≠ functionality

TCL Communication:

Proposal 2: The logical model ID, which consists of scenario, feature, function, and local ID, will be used for air interface signalling. Other auxiliary information will be obtained through a model-ID based lookup inside UE or gNB entity.

Proposal 3: The network’s unique ID (including UE IEMI, and gNB’s unique ID(Cell ID)) together with AI/ML model’s global unique ID should be used in AI-MAO for management.

NYCU, NTPU:

Proposal 1: From RAN1 perspective, the model ID indicates the corresponding logical model.

Proposal 2: The logical model should contain information of model functionality, applicable conditions and model capability. Information of pairing is also should be contained in the two-sided model case

Proposal 3: Study necessity and feasibility of local model ID (temporary index) for model control purposes.

Proposal 4: Support sharing the view of local model ID to RAN2.

Continental Automotive:

Proposal 1: The index is allocated between NW and UE so that it can be used to represent different combinations of the assigned models (e.g., model IDs) for LCM signaling purposes.

Proposal 2: The index is also used to indicate the paired models.

Proposal 8: Relationship of quasi-based similarity between model/data properties need to be further studied for different LCM phases (model training/inferencing/monitoring, etc.).

Indian Institute of Tech (M), IIT Kanpur:

Proposal 1: Model management can be initiated by either the NW or the UE for a particular model. Upon request from the NW or UE, Model Management will select the model(s) for monitoring and collect the appropriate data.

Proposal 3: In the event that the NW doesn’t initiate the model identification process, the UE can request the NW for model identification.

Proposal 4: In addition to the NW identifying the models (Observation 4), we believe that for the UE-part of two-sided models, it is also possible for the UE to directly report the Model IDs along with the conditions (under which a functionality could use an AI/ML based approach) in the capability report.

Proposal 5: There should be a distinction between AI/ML functionalities and Non-AI/ML functionalities.

##### Issue 7-8: Clarification of functionality terminology

After the agreement 6-7f and 6-12f in RAN1 #112-bis-e regarding “overall framework of functionality and model operation”, email discussions were held among companies to further clarify the term functionality.

During the email discussion, one confusion arose due to the interpretation of “configuration(s)” in the agreement:

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

A number of companies interpreted “configuration(s)” as functionality configuration(s), but a prevailing understanding seems that “configuration(s)” refer to configuration parameter, i.e., the RRC IE. The latter was also the FL’s intention/interpretation leading to the agreement. So, it will be good to clarify this and make an agreement to avoid future confusion.

Regarding what is meant by “functionality”, the FL asked two alternatives:

- Alt 1: a specific configuration of the Feature/FG (i.e., a particular choice from NW for potential RRC configuration to UE)

- Alt 2: a set of configurations of the Feature/FG (i.e., the set of configurations that the NW could choose from for potential RRC configuration to UE)

Based on the companies’ inputs, the FL has summarized:

- Functionality may refer to a specific configuration of the Feature/FG or a set of configurations of the Feature/FG. We can perhaps say that a Functionality is a unit for activation/deactivation/switching, that may correspond to a specific configuration or a set of configurations.

Also, during email discussion, companies discussed various terminologies related to functionality, which was summarized by FL as follows:

Alt 1

- Configurable functionality is synonymous to identified functionality.

- Configured functionalities are determined by NW as a subset of identified functionalities.

- Applicable functionalities are reported from UE as a subset of configured functionalities.

- NW activates one functionality out of applicable functionalities.

Alt 2

- Applicable functionalities are reported from UE as a subset of identified functionalities.

- Configurable functionality is synonymous to applicable functionality.

- Configured functionalities are determined by NW as a subset of applicable functionalities.

- NW activates one functionality out of configured functionalities.

The difference between these two alternatives is whether the applicable functionalties are reported only for configured functionalities. It is seen that the majority of companies think Alt1 is more appropriate. While it may not be essential to discuss/decide which Alternative is better, it will be very helpful to at least align terminologies used for discussion.

###### [FL1] Proposal 7-8a:

Further clarification on functionality terminology:

- Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configurations are the conditions in forms of RRC IE(s) reported by UE capability.

- A functionality refers to a specific configuration of the Feature/FG or a set of configurations of Feature/FG and may serve as a unit of activation/deactivation/switching in functionality-based LCM.

The term functionality(/ies) used at various part of functionality-based LCM can be referred to

- Identified functionalities: functionalities corresponding to conditions indicated by UE capability

- Configured functionalities: functionalities that are configured by the NW among identified functionalities

- Applicable functionalities: functionalities that are currently applicable at the UE among identified functionalities

- Activated functionality: the functionality that is currently activated from the set of applicable functionalities

- FFS: Whether applicable functinoaltiy is a subset of configured functionalities, or vice versa.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | We prefer to not touch applicable functionality for now since they are still under study in another agreement and needs more clarification on the intention. Other parts are fine. |  |
| OPPO | Generally fine for us. But we think the FFS point is essential for this proposal. We may try to clarify the FFS point in this meeting. If we can not make consensus on the FFS point. We are OK to support the proposal also. |  |
| ZTE | The proposal is related to signaling structure about the functionalities. We don’t think it’s really necessary to discuss it right now. The clarification may somehow create more ambiguities. Our suggestion is to address it in WI phase. For example, it’s not clear to us about the identified functionalities and configured functionalities. - Whether applicable conditions of a functionality are determined during the UE capability report or after the UE capability report. For Option 1, the applicable conditions of a functionality are determined by UE capability. For Option 2, the the applicable conditions of a functionality are determined after UE capability. - Whether a functionality can be configured by NW (i.e., via model transfer) after the UE capability report. For example, in the following two figures, Type 2 functionality is available after model transfer from NW. Therefore, we don’t know the configured functionality can include model transfer or not. - Option 1: UE indicates its supported candidate values for each condition (or component) of an AI/ML-enabled feature as the legacy UE capability report. - Option 2: UE indicates its supported functionality of an AI/ML-enabled feature in a UE capability report, where the applicable conditions of a functionality are indicated by a combination of the conditions. We can live with the first part for now with ‘RRC’ removed as positioning uses LPP signaling. Further clarification on functionality terminology: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configurations are the conditions in forms of RRC IE(s) reported by UE capability. |  |
| Panasonic | We share simialr view with vivo. The need of applicable functionality in functional framework is necessary or not is uncertain. It should be managed by model identification based. |  |
| Huawei, HiSilicon | In our understanding, the procedure includes two steps: Step 1: UE report of the supported functionalities. (Identified functionalities?) Step 2: NW activates one preferred functionality from the set of UE reported functionalities in Step 1. (Activated functionality?) What are the need of the other “functionalities” (“Applicable functionalities” and “Configured functionalities”)? If “Applicable functionalities” means the dynamic update of the availability of functionalities, then it can be part of Step 1; for “Configured functionalities”, we do not see a storng need why NW has to make two steps to activate a UE side/part model – a direct activation is enough. |  |
| NTT DOCOMO | Support the proposal. |  |
| Google | We share similar view with HW. |  |
| Futurewei | We need some clarifications. It is not clear to us the difference between “a specific configuration of the Feature/FG” and “a set of configurations of Feature/FG”. Does “a set of configurations” mean multiple “specific configuration”? And when “a set of configurations of Feature/FG” serve as a unit of activation/deactivation/switching in functionality-based LCM, will all the (set of) configurations be considered a unit? Our overall feeling about functionality identification and functionality-based LCM is that, after all the discussions, there are still many aspects related to functionality ID that are not clear and, based on our observations, would take the group huge effort to sort it out (if possible). Some of our concerns are - Although the group decided to use legacy 3GPP framework of Features as a starting point for the study of functionality identification, there is not a well-defined “3GPP framework of Features”. - There is not clear understanding of the relationship between model Functionality and the Feature/FG. For example, how to link a Functionality to multiple UE Features/FGs? Note the agreement only says that “functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG”. Then, can we link one Functionality ID to multiple Features/FGs? - It would be hard to define Functionality without the definitions of each Feature if Functionalities will be based on Features (which we think they would). Based on this thinking, we think RAN1 should focus on model identification in the SI phase and defer the study of functionality identification details till Rel-19 work item phase. |  |
| Fujitsu | We share the similar view as that of vivo and Panasonic |  |
| Lenovo | We think new terms need to be carefully introduced. We need to align the agreements and procedures in advance. We share the similar view with HW that the procedure with two steps is enough, and also other possible LCM operations, e.g., updating selection and switching, can be used for the purposed discussed here. |  |
| NEC | From our perspective, there are some benefits of having concept of configured functionalities where one of the configured functionalities is finally used for activation. A UE can be configured with multiple functionalities and one of the functionalities may be activated by the network/UE depending on the applicable scenario. For e.g., network may configure multiple functionalities for different radio condition of UE (poor signal, good signal) and can switch easily between the configured functionality quickly without incurring significant signalling overhead. Hence, we agree to support identified, configured and activated functionalities. |  |
| LG | Fine in general but not sure whether we need to define exact ‘unit’ for functionality unless we are going to index functionalities from zero to X. To our understanding, it is a configuration so may not need to define such unit as current specification. |  |
| KDDI | Agree in general. But we have similar view with vivo. Applicable functionality should be clarified in another proposal. |  |
| ETRI | We share similar views with vivo and Panasonic. Introducing applicable functionality without clear scope of applicable conditions that functionality can support can add confusion. For functionality identification, it was agreed to further study how to address additional conditions (e.g., scenarios, sites, and datasets). We can discuss this FFS point first. |  |
| CAICT | Similar view with HW. |  |
| Ericsson | Similar view as Huawei, the 4 circles are bit confusing and overly complicated. Regardinig the dynamic update of the availability of functionalities, it is important that this dynamic mechnis, are outside of existing UE capability framework, since it is seldomly peformed. Hence there could be a motivation for a another step according to below. Step 1a: UE report of the supported functionalities. (Identified functionalities?) (static) Step 1b: UE may report applicable functionallites (this procedure is dynamic, based on applicable functionalities not part of step 1a) Step 2: NW activates one preferred functionality from the set of UE reported functionalities in Step 1 . (Activated functionality?) |  |
| Samsung | We agree with direction. Similar to Rel-15/16 TCI framework, activation after configuration may have benefits. The network can activate a subset of configured functionalities to make the UE ready for inference. |  |

##### Issue 7-9: Remaining issues on the functionality/model identification and LCM

Following the agreements related to functionality/model identification and related LCMs, it would be useful to discuss further details and clarify some aspects. One of them is provided in the below figure associated with the feature, functionality and model. There are some other aspects as well.

###### [FL1] Proposal 7-9a:

For functionality/model identification and LCM,

- In functionality-based LCM, Functionality ID may be introduced to facilitate the signaling exchanging between NW and UE.

- A model ID used for model identification and LCM purposes may be local or global.

- One model identified by a model ID may support one or more than one Functionalities.

- In model-ID-based LCM, UE indicates supported AI/ML models for a given AI/ML-enabled Feature/FG in a UE capability report or in a subsequent procedure.

  - FFS: Whether and how UE indicates supported AI/ML models in relation to identified/configured functionalities.

- Functionality-based LCM is applicable for the case with functionality identification.

- Both functionality-based LCM and model-based LCM are applicable for the case with model identification.

- A possible frequent update of models or applicable conditions (if needed) does not imply frequent updates in UE capabilities within the existing capability framework

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | We have the following comments: - Functionality ID is a signaling design that is up to WI and other WG to determine. It is inappropriate to say there is such ID for now. - The case that functionality-based LCM applied to model identification can be supported with the assumption that functionality is limited to existing signaling framework. Otherwise if functionality becomes dynamic, there will be overlapping between functionality-based LCM and model ID based LCM. ###### [FL1] Proposal 7-9a: For functionality/model identification and LCM, - In functionality-based LCM, Functionality ID may be introduced to facilitate the signaling exchanging between NW and UE. - A model ID used for model identification and LCM purposes may be local or global. - One model identified by a model ID may support one or more than one Functionalities. - In model-ID-based LCM, UE indicates supported AI/ML models for a given AI/ML-enabled Feature/FG in a UE capability report or in a subsequent procedure. - FFS: Whether and how UE indicates supported AI/ML models in relation to identified/configured functionalities. - Functionality-based LCM is applicable for the case with functionality identification. - Both functionality-based LCM and model-based LCM are applicable for the case with model identification with the assumption that functionality identification uses existing capability framework. - A possible frequent update of models or applicable conditions (if needed) may need signaling different fromdoes not imply frequent updates in UE capabilities within the existing capability framework |  |
| OPPO | We do not quite understand in Bullet 5, why functionality-based LCM is applicable for the case with model identification. If models have been identified, model-based LCM will provide better than life cycle management than functionality-based LCM. In which case we need functionality-based LCM with model identification? |  |
| ZTE | - We should have a unified solution for functionality-based LCM and model-ID based LCM. To our understanding, either the functionality or an identified model should be allocated with a local identifier. However, the local identifier doesn’t need to differentiate whether it’s associated with a model or a functionality. From network perspective, the local identifier is just a function unit operated at UE that is not transparent to network, which can facilitate the follow-up indication on various operations. - We don’t think a hierarchical structure for functionality and model is needed, the model or functionality is just a function unit operated at UE that is not transparent to network. Therefore, there is no too much difference for model-ID based LCM functionality-based LCM. We acknowledge that an ID may be introduced to facilitate the signaling exchanging between NW and UE. The ID can be either associated with a model or a functionality. - In addition, our understanding is that a functionality can include both semi-static parts and dynamic parts. It’s too early to say that the functionality identification should follow existing UE capability framework. How to enable the dynamic parts can be discussed separately. It’s fine to address it in WI phase. With about comments, we prefer the following version: For functionality/model identification and LCM, - A local ID may be introduced to facilitate the signaling exchanging between NW and UE. The local ID may either be associated with a functionality or a model. - A model ID used for model identification may be global. UE indicates supported AI/ML model(s) via the model ID(s) for a given AI/ML-enabled Feature/FG in a UE capability report or in a subsequent procedure. |  |
| Panasonic | Although functionality ID is described as "may be introduced", we are not sure the need of functionality ID or more traditional way of the signalling of the functionality as the configuration. Therefore, we propose to have both possibilities. - In functionality-based LCM, Functionality ID may be introduced to facilitate the signaling exchanging between NW and UE. Or functionality may be expressed as the configuration(s) as the signalling. In case different functions of the same (physical) model, just to have different model ID for each function may be sufficient. It may be related to validation and testing discussion. - One model identified by a model ID may support one or more than one Functionalities. Or a model ID may be allocated for each functionality even the actual model is same. |  |
| Huawei, HiSilicon | 1) For local model ID, as we commented in the last meeting, RAN1 does not identify the need – it could be RAN2 scope. - A model ID used for model identification and LCM purposes may be local or global. 2) We do not see the need to build model identification LCM on top of functionality identification LCM. In model identification LCM, after model is identified between NW and UE, UE directly reports the supported model ID(s)-then the NW can activate a model from the reported model IDs. The needed RRC parameters, conditions, etc. are all implicitly indicated by the model ID (these meta information associated with the model ID have been addressed to NW side during model identification procedure). Both functionality-based LCM and model-based LCM are applicable for the case with model identification One model identified by a model ID may support one or more than one Functionalities |  |
| NTT DOCOMO | As the model identified by model ID may be logical, the boundary between model and functionality is not distinct. We think the difference between model and functionality should be studied first, before agreeing the model and functionality are hierarchical structure. |  |
| Google | We do not see the need to introduce ‘functionality ID’. |  |
| Futurewei | We suggest leaving functionality based topics to the R19 WI phase and focusing on model ID based approach in this SI. Agree in general for the model ID related statements. |  |
| Fujitsu | For functionality-based LCM, we also think it is too eary to conclude functionality ID is the only way for functionality control. For the 3rd bullet, we think a model can support more than one functionality in terms of implementation,. But referring to a logical model, it would be better links to only one functionality. |  |
| Lenovo | We suggest not to mix ID discussion and functionality/model-ID-based LCM. For the ID issues, we share the similar view with ZTE that ‘A local ID may be introduced to facilitate the signaling exchanging between NW and UE. The local ID may either be associated with a functionality or a model.’ The relationship between functionality-based and model-ID-based LCM can be discussed in another issue, Issue 7-17. |  |
| NEC | We don’t understand the subbullet “Both functionality-based LCM and model-based LCM are applicable for the case with model identification.” Does it imply that functionality-based LCM have model indentification component? |  |
| LG | - 1st bullet: as commented in 7-8, we are not sure the need for functionality ID. - 2nd bullet: this does not seem to be helpful for progress. We need to be more specific when it is local and when it is global. - 3rd bullet: unclear how this is applicable and why this is needed unless we are going to apply both LCM approaches simultaneously. - 4th bullet: this is only for UE-side/part model to our understanding. For UE-side model, i.e. one-side model, we don’t think model-ID based LCM is necessary so we suggest to discuss when model-ID-based LCM is necessary firstly before discussing related signaling aspect. - 6th bullet: unclear why both functionality-based LCM and model-based LCM need to be applied when model is identified? - 7th bullet: intension is fine but whether dynamic functionality/model update can be viewed as dynamic capability update or not may not be understood differently per company. |  |
| ETRI | We think that functionality/model identification and LCM have their own strengths, and it is worth considering taking advantage of the strengths of both approaches. For the functionality identification and LCM, we think that resolving dynamic application conditions may not enough. For example, if models are not identified, the UE may not receive feedback from the NW on the system performance monitoring result of each model. Moreover, in the case of a two-sided model, the exchange of model information may be essential for parining. For the model identification and LCM, we think that reporting all the model IDs to the NW may not efficient. For example, considering the scenario, site, and dataset conditions, the number of models can be very large. If model IDs are reported before the feature is clarified, model IDs will be reported unnecessarily even for the feature that the network will not use, causing excessive signaling overhead. In addition, model IDs are reporting without feature information, NW should perform an indexing process to find the feature/configuration information corresponding to each model ID whenever model IDs are reported, and this process can be inefficient. |  |
| CAICT | General fine. Some clarifications on “Both functionality-based LCM and model-based LCM are applicable for the case with model identification.” Is needed. For functionality-based LCM only, model identification might not be necessary. |  |
| Ericsson | Not supportive to add “Functionallity ID”. This would add another layer on confusion for functionalities, we don’t see the need of an ID. - In functionality-based LCM, Functionality ID may be introduced to facilitate the signaling exchanging between NW and UE. It would be beneficial to clarify the relation between model and functionalities prior to discussing this proposal in our view. For example, first agree on: One model identified by a model ID may support one or more than one Functionalities. |  |
| Samsung | We are ok with the direction. When functionality refers to a single configuration, the configuration ID may serve as functionality identification. When functionality refers to a set of (multiple) configurations, the configurations can be linked by higher layer parameter. |  |

##### Issue 7-11: dynamic functionality/model applicability

Applicable functionalities at UE may change over time. Reasons may include site-, scenario- and/or dataset-specific models underlying a functionality. Additionally, UE’s memory usage, battery status, or any other hardware limitations and temporary unavailability of a model (e.g., time to download a model upon transparent model switching) affect applicable functionalities.

Likewise, applicable models at UE may also change over time due to UE’s memory usage, battery status, or any other hardware limitations in addition to temporary unavailability of a model (e.g., time to download a model upon transparent model switching).

Therefore, in RAN1 #112-bis-e, it was agreed

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

These changes in applicable functionalities and models can be attributed to additional conditions. In the RAN1 #112-bis-e agreement, additional conditions are FFS.

FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

Additional conditions can be defined as the information provided by NW to UE such as scenario/dataset ID, pairing information for two-sided model operation, site/cell ID. Furthermore, UE’s internal conditions such as memory, battery, other hardware limitations, temporarily unavailability of a model due to the need of model download can also be considered as additional conditions.

###### [FL1] Proposal 7-11a:

Confirm the necessity for UE to report updates on applicable functionality(es) / applicable model(s) among [configured/identified] functionality(es) /model(s).

Categorize additional conditions as follows:

Category 1: Additional conditions provided by the NW, e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID.

Category 2: UE’s internal conditions unknown at the NW such as memory, battery, other hardware limitations, temporary unavailability of a model due to the need of model download

Take the following mechanisms for handling the above two Categories of additional conditions as the baseline and further study them in general agenda and in each sub-use-case:

- In Functionality based LCM

  - Category 1 additional conditions

    - Alt 1: UE may indicate Category 1 conditions via UE capability or other mechanisms.

    - Alt 2: UE may dynamically report updates on applicable functionalities due to changes in Category 1 conditions

  - Category 2 additional conditions

    - UE may dynamically report updates on applicable functionalities due to changes in Category 2 conditions.

- In Model ID based LCM

  - Category 1 additional conditions

    - Category 1 conditions for a model is provided via meta info during model identification. UE indicates supported models during UE capability.

  - Category 2 additional conditions

    - UE may dynamically report updates on applicable models due to changes in Category 2 conditions.

For dynamically reporting updates, further study the following mechanisms

Reports upon Network’s request

Proactive reports from UE

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | We are fine for the mode ID based LCM to support Category 1 and Category 2. We don’t support the funcationality to be associated with scenario/dataset ID. Thus the applicable functionality part should be deleted. Confirm the necessity for UE to report updates on applicable functionality(es) / applicable model(s) among [configured/identified] functionality(es) /model(s). Categorize additional conditions as follows: Category 1: Additional conditions provided by the NW, e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID. Category 2: UE’s internal conditions unknown at the NW such as memory, battery, other hardware limitations, temporary unavailability of a model due to the need of model download Take the following mechanisms for handling the above two Categories of additional conditions as the baseline and further study them in general agenda and in each sub-use-case: - In Functionality based LCM - Category 1 additional conditions - Alt 1: UE may indicate Category 1 conditions via UE capability or other mechanisms. - Alt 2: UE may dynamically report updates on applicable functionalities due to changes in Category 1 conditions - Category 2 additional conditions - UE may dynamically report updates on applicable functionalities due to changes in Category 2 conditions. - In Model ID based LCM - Category 1 additional conditions - Category 1 conditions for a model is provided via meta info during model identification. UE indicates supported models during UE capability. - Category 2 additional conditions - UE may dynamically report updates on applicable models due to changes in Category 2 conditions. For dynamically reporting updates, further study the following mechanisms Reports upon Network’s request Proactive reports from UE |  |
| OPPO | Generally OK for the bullets. We support both the functionality-based part and model-based part in the proposal. But some examples in the bullet are still pending clarification: What is relation between scenario and cell/site? If we have the scenario-based condition, do we still need the cell/site-based condition, and verse visa? We still have concerns about the pairing information for two-sided model. Should it be separate from model ID? Or it can be expressed by model ID? We suggest to avoid listing too many examples, especially the unclear ones. |  |
| ZTE | We prefer not to mix two issues together. We prefer to only focus on category 1 in this proposal. - Additional conditions, as discussed in last meeting, which are the applicable conditions of a model/functionality. It’s to define generalization capability of a model. It should be the attributes of a model/functionality. It’s not provided or configured by network. It should be reported by UE during the model/functionality identification. - For category 2, the operation is on a whole model/functionality for it’s applicability status. It’s nothing about the applicable conditions. Additional conditions e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID may be provided during the model/functionality identification process - UE may indicate the additional conditions via UE capability or other mechanisms. - The additional conditions for a model is provided via model description information during model identification. |  |
| Panasonic | We have similar view with vivo on the need of additional condition for functionality based LCM is not sure. |  |
| NTT DOCOMO | In Alt1 for funcitonlaity-based LCM, UE is expected to indicate Category 1 conditions. However, it contradicts the definition of Category 1, since Category 1 is conditions provided by NW. We think Category 1 and Category 2 can be differentiated according to whether the additional condition is transparent to NW. Category 1 is non-transparent condition between NW and UE because NW and UE exchange that information, while Category 2 is the transparent condition to NW. Hence, we suggest the following update Category 1: Additional conditions that is not transparent to NW, which could be provided by NW or reported by UE, e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID. Category 2: UE’s internal conditions unknown at the NW such as memory, battery, other hardware limitations, temporary unavailability of a model due to the need of model download |  |
| Google | We also think this can be for model base LCM |  |
| Futurewei | Again, suggest deferring the discussions on functionality based approaches. We only comment on model ID based approach here. For model ID based approach, we are not clear on the following. Category 1: Additional conditions provided by the NW, e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID. The definition says the additional conditions are provided by the NW. Then under model ID based LCM, the proposal says “Category 1 conditions for a model is provided via meta info during model identification. UE indicates supported models during UE capability.” The two statements seem controditary to each other; are these additional conditions provided by the NW, or the UE? |  |
| LG | For functionality-based LCM, Cat1-alt1 seems not belong to functionality-based LCM. |  |
| ETRI | For Category 1, we have similar views with other companies. It is unclear how to identify applicable conditions of scenario/site/dataset by functioanlity. After this is clarified, we can discuss further. For Category 2, it should be clarified whether the UE simply reports UE preference or if the UE can initiate model deactivation of the AI/ML functionality/model. For example, in the former case, it can be supported by the existing UE assistance information report, but in the latter case, a new process needs to be introduced. |  |
| CAICT | General OK with the proposal. |  |
| Ericsson | - Supportive regarding the categories and functionality-based LCM. - For Model ID based LCM, we prefer to avoid introducing the new term “meta info”, it should be first agreed upon before using in sub-bullets. - Category 1 conditions for a model is provided via meta info during model identification. UE indicates supported models during UE capability. |  |
| Samsung | We can start with the below categorization, anf first study the detailed applicable conditions in a sub-use case level first. The mechanism can be studied later. Categorize additional conditions as follows: Category 1: Additional conditions provided by the NW, e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID. Category 2: UE’s internal conditions unknown at the NW such as memory, battery, other hardware limitations, temporary unavailability of a model due to the need of model download |  |

##### Issue 7-14: Applicability of LCM methods (continued from 6-14)

In the previous agreement, applicability of LCM methods are marked as a separate discussion as follows.

• Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Functionality based LCM and model ID based LCM can be both useful for different cases. Identifying these cases can be helpful in many ways, e.g., for discussion on detailed LCM signaling aspects. To identify these cases more clearly, the major differences can be considered in terms of the main steps of LCM. To illustrate, for UE-sided models functionality based LCM can be utilized if there is no model transfer from network to UE, i.e., collaboration level y in an attempt to simplify the model management complexity in network and avoid more frequent signaling with larger payload. If there is model transfer from network to UE, i.e., collaboration level z and/or if the model is two-sided, model ID based LCM can provide some benefits in terms of model management and control. However, employing these LCMs for different scenarios does not necessarily mean that they need to be designed from completely indepent frameworks. On the contrary, functionality-based LCM and model-ID-based LCM may follow one general framework since they are derived from similar configurations/conditions and additional conditions. This may be also useful from efficient signaling design perspective.

###### [FL1] Proposal 7-14a:

For UE-side models and UE-part of two-sided models, it is observed that

- Scenarios where functionality based LCM can be useful

  - For UE-side models when there is no model transfer from NW, i.e., Collaboration Level y

- Scenarios where model ID based LCM can be useful

  - For two-sided models

  - When there is model transfer from NW, i.e., Collaboration Level z

    - For addressing custom

  - additional conditions (e.g., scenarios, datasets, configurations) outside specification, via vendor collaboration.

From the general framework point of view, support both functionality-based LCM and model-ID-based LCM. Each sub-use-case can further study whether to allow only one of the two LCM flavors for the sub-use-case.

Study shall strive for a unified framework for functionality based LCM and model-ID based LCM as much as possible.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
|  | For UE-side models and UE-part of two-sided models, it is observed that - Scenarios where functionality based LCM can be useful - For UE-side models when there is no model transfer from NW and there is no need to associate additional conditions with AI/ML enabled feature in a specified way, i.e., Collaboration Level y - Scenarios where model ID based LCM can be useful - For two-sided models - When there is model transfer from NW, i.e., Collaboration Level z - For addressing custom - additional conditions (e.g., scenarios, datasets, configurations) that need to be associated with AI/ML enabled feature outside specification, via vendor collaboration. From the general framework point of view, support both functionality-based LCM and model-ID-based LCM. Each sub-use-case can further study whether to allow only one of the two LCM flavors for the sub-use-case. Study shall strive for a unified framework for functionality based LCM and model-ID based LCM as much as possible. |  |
| OPPO | Generally fine. |  |
| ZTE | - Please see our comments on Proposal 7-9a. - We agree that a unified framework for functionality based LCM and model-ID based LCM is necessary. Either a functionality or an identified model should be allocated with a local identifier. However, the local identifier doesn’t need to differentiate whether it’s associated with a model or a functionality. From network perspective, the local identifier is just a function unit operated at UE that is not transparent to network, which can facilitate the follow-up indication on various operations. - In addition, it’s too early to preclude that functionality-based LCM cannot support model transfer. We think it’s not future-proof if model transfer is only applied to identified models. The outcome of this SI is not only for Rel-19, but also for long term development of AI/ML in 5G and beyond. |  |
| Panasonic | We support the modification by vivo (assuming empty name box is from vivo), i.e. " there is no need to associate additional conditions with AI/ML enabled feature in a specified way" is fo functoinality based LCM". |  |
| NTT DOCOMO | We think functionality-based LCM is also applicable to the two-sided model, as long as the paring info is exchanged between UE and NW. Also, we do not find any benefit to confine functionality-based LCM to one sided model. RAN1 should avoid introducing many frameworks for unnecessary reasons. |  |
| Google | OK |  |
| Futurewei | Most part of this proposal are observations. The real proposals seem to be the following. - Support both functionality-based LCM and model-ID-based LCM - Study shall strive for a unified framework for functionality based LCM and model-ID based LCM as much as possible. The first point seems obvious as we are doing it. We support the second point about unified framework. |  |
| NEC | Generally OK. |  |
| LG | Suggest following modification: For UE-side models and UE-part of two-sided models, it is observed that - Scenarios where functionality based LCM can be useful - For UE-side models when there is no model transfer from NW, i.e., Collaboration Level y - Scenarios where model ID based LCM can be useful - For two-sided models - When there is model transfer from NW, i.e., Collaboration Level z - For addressing custom - additional conditions (e.g., scenarios, datasets, configurations) outside specification, via vendor collaboration. From the general framework point of view, support both functionality-based LCM and model-ID-based LCM. Each sub-use-case can further study whether to allow only one of the two LCM flavors for the sub-use-case. Study shall strive for a unified framework for functionality based LCM and model-ID based LCM as much as possible. |  |
| ETRI | We are generally fine. |  |
| CAICT | Generally fine. For UE-side models, model ID based LCM might also be needed. |  |
| Ericsson | Propose to remove the subbullet below, it is unclear what is meant by “custom” and what should be part of outside specification. - For addressing custom additional conditions (e.g., scenarios, datasets, configurations) outside specification, via vendor collaboration. |  |
| Samsung | Functionality-based LCM applied to the following cases too - Two-sided models when a UE does not have to keep multiple UE-part models for a single functionality (to pair with models from multiple network vendors. - To collaboration level-Z when the network does not transfer multiple models for a single configuration (functionality). |  |

##### Issue 7-15: Model identification types (continued from 6-11)

Model identification procedure involves assigning model ID and sharing mode description information. This can be performed offline or online. For the online case, model identifition can be initiated either by UE or NW. In the previous meeting Type A, B1 and B2 are formed to include these cases.

Additionally, for model identification types it can be helpful to consider the two scenarios for model identification as “initial model identification” and “updated model identification”.  Initial model identification refers to a model that has not been identified before and it is not explicitly linked with any previous model ID. On the other hand, updated model identification refers to a new model with a new model ID, but linked with a previous model ID. For example, updated model identification may be used as a result of model parameter update. In this case, a new model being identified is related to the previously identified model via the same model structure, and therefore, an “updated model identification” may be used to explicitly provide the linkage to the previous identified model, and model description information for updated model identification can be  mostly inherited from the previously identified model.

The first part of the proposal is identifical to 6-11f.

###### [FL1] Proposal 7-15a:

For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any).

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification.

  - FFS: Spec impact to other WGs including SA

- Type B: Model is identified via over-the-air signaling

  - Type B1: UE-initiated model identification of a model known model at UE

    - Model identification initiated by the UE, and NW assists the remaining steps of the model identification

    - FFS: details of steps

  - Type B2: NW-initiated model identification of a model known model at NW

    - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps of the model identification

    - Note: Type B2 may be used in conjunction with model transfer from NW to UE.

    - FFS: details of steps

- Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

Study the following two scenarios for model identification: “initial model identification” and “updated model identification”.

- Initial model identification refers to a model that has not been identified before and it is not explicitly linked with any previous model ID

- Updated model identification refers to a new model with a new model ID, but linked with a previous model ID.

Study the necessity, feasibility, applicability, and detailed procedure of the above three Types for initial and updated model identification, taking the following Table as a starting point for discussion.

|  | Initial model identification | Updated model identification |
| --- | --- | --- |
| Type A | Applicable | Applicable |
| Type B1 | Applicable | Applicable |
| Type B2 | Not applicable | Applicable |

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | Support. And at least for Type A, we should consider send LS to SA. Do not know how RAN1 studies whether there is SA/RAN2 impact. |  |
| OPPO | It is suggested to split the proposal to two separate proposals. We generally support the first part. But there may be some typo in it. - Type B: Model is identified via over-the-air signaling - Type B1: UE-initiated model identification of a model known model at UE - Model identification initiated by the UE, and NW assists the remaining steps of the model identification - FFS: details of steps - Type B2: NW-initiated model identification of a model known model at NW - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps of the model identification - Note: Type B2 may be used in conjunction with model transfer from NW to UE. - FFS: details of steps The second part including the table for updated model identification can be discussed after the first part is agreed. |  |
| ZTE | - Our understanding is that the model identification is a separate model ‘naming’ process. Additional UE capabilities may need to be defined. We don’t need to mix the issue with model-ID LCM and functionality-based LCM. - For all the model identification types, it should explicitly indicated the necessity of model ID assignment. It maybe ‘globally’ unique as agreed in RAN2. - For Type B1, if model transfer doesn’t happen, we’re not sure why we need to assign an ID to model description information only. It’s not clear to us how NW can know the performance of corresponding model. - Regarding the “initial model identification” and “updated model identification”, we prefer not to discuss it for now. Our understanding is that Type B2 can be applied to both model transfer case z4 and z5. |  |
| Panasonic | "UE-initiated" or "NW-initiated" is ambiguuous on the following case of the behaviour. Case X: When the network reconfigures some parameters inclduing handover, UE responds model identification procedure. Case Y: When the network reconfigures some parameters including handover, NW also informs model identification. Both of the parameter changes are intiated by NW. On the other hand, model identification itself is triggered by UE in case X and model identification is informed simultaneosly informed by NW in case Y. Case X is categorized as B1 and case Y is categorized as B2. Or such exact definition is FFS? For the definition of updated model identification, just "linked" is ambiguious. The text before the proposal can be added like following. - Updated model identification refers to a new model with a new model ID, but linked with a previous model ID. It is related to the previously identified model via the same model structure. |  |
| Huawei, HiSilicon | A question on the table: How will NW and UE build the initial model identification under Type B2 (model transfer)? Does it mean the model ID (linked to a structure) is identified with an offline (A) or from UE to NW (B1) behavior, after which the model transfer can be performed? |  |
| Google | OK |  |
| Futurewei | In this proposal, Type A allows UE to assign model IDs to models when models are identified to UE. However, this is not aligned with the agreement in Meeting 112, which says only NW can assign model IDs. - For UE-side models and UE-part of two-sided models… for AI/ML model identification, models are identified by model ID at the Network. UE indicates supported AI/ML models. - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. Therefore, before the group agrees that UE is allowed to identify models, Model is identified to UE should be removed from Type A. In addition, we wonder whether it is necessary to define the term “updated model identification” because “Updated model identification refers to a new model with a new model ID”; this doesn’t need to be called an update since both the model and the model ID are new. The link between this model and the previous model can be indicated by other means. |  |
| Lenovo | We agree with the definition on the identification types, i.e., Type A/B. The intention to introduce ‘initial/update’ model identification and the table needs to be further discussed. |  |
| ETRI | We have similar view with Panasonic. We do not consider the definition above to be essential and the “initiated” is a quite ambiguous to us. For example, Type B1 imply that NW may not initiate the model identification of the UE-sided model. However, when considering that some applicable conditions (e.g., Category 1 in Proposal 7-11a) may be provided by NW, it seems be more reasonable for NW to initiate the model identification process by providing Category 1 conditions. It can be discussed after the each step of model identification process is clarified. |  |
| CAICT | We are not sure when a model is updated with some parameter changing, whether a new model ID is required or not. If the answer is yes, the proposal is fine. |  |
| Rakuten Mobile | Support the proposal. Initiation of the update depends on where KPI monitoring is done. In that sense, this classification is useful to be aware about the entity which conduct the monitroing. |  |

##### Issue 7-17: Awareness of model operations in functionality-based LCM

In RAN1 #112, it was agreed to study whether and how much awareness/interaction NW should have about model-level LCM in functionality-based LCM.

RAN1 #112 Agreement For UE-side models and UE-part of two-sided models: - In functionality-based LCM - … - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM

###### [FL1] Discussion 7-17a:

Please share your views on whether Network may need some awareness of UE’s model level operations in functionality-based LCM, including its purposes and use cases, scenarios, necessity, and potential mechanisms. Please note that some aspects are being discussed in 7-11.

| Company | Comments |
| --- | --- |
| vivo | We don’t see the need of functionality operation where model level awareness is needed. If needed, we should go to model identification procedure. Otherwise there would be redundant design for the same purpose. |
| OPPO | We have no contribution at this moment for this type of use cases. |
| ZTE | Our understanding is that the model operations within a functionality are transparent to network. The functionality is just a virtulization of model(s). If UE would like to disclose more information about the models, UE can simply report more functionalities. |
| Panasonic | It depends on the boundary between functionality-based LCM amd model based LCM discussed in Proposal 7-11a. If functionality-based LCM does not have additional conditions exchange between UE and NW, all model handling is up to within UE similar to rather non-AI based functoinality handling. It has more similarity with level x. Then model awareness from NW is not required. If functionality-based LCM have additional conditions exchange between UE and NW, we think such additional condition is linked to logical model. Then model awareness from NW is required on what model is used. |
| Huawei, HiSilicon | The principle of functionality based LCM is to conceal the model operations. For the given examples, NW does not need to know. But as UE reports different functionalities (with functionality IDs), gNB will know that when functionality changes, there may potentially correspond to model switching (then gNB will trigger a monitoring process)? Is this deemed as awareness? |
| NTT DOCOMO | First UE’s model level operation should be clarified. If it means the physical model management within a functionality, we think it can be transparent to NW. However, if UE’s model level operation includes the fallback operation or functionality switching, NW should be aware of it as it affects the derived information and the configuration. |
| Google | We do think the awareness of UE’s model level operations in functionality-based LCM is needed. |
| Futurewei | This is another example that functionality identification/functionality-based LCM is a complicated topic and we should defer it. |
| Fujitsu | We think there is a need for NW to be aware some model level operations, such as model switching. For example, the consequence of a model switching from one to the other would be a better performance at functionality level. Although the model switching at UE-side can be an implementation-based choice, it would be helpful to inform or indicate the NW side such kind of model level operations. With it, functionality monitoring at NW-side can adjust its monitoring record and decision. For example, NW may decide to deactivate an AI/ML functionality for its poor performance. But it will keep watching for a while if being indicated a model switching just happened at UE-side. |
| Lenovo | In general, we think the operations based on functionality could not be excluded for the model-ID-based LCM (i.e., model level operations), because if multiple models have common configurations, they can be also indicated by functionality for operation with less signalling overhead. |
| LG | Model-level operation shall be transparent to NW but it may induce some related functionality-level LCM signaling. |
| ETRI | We think that functionality identification can be jointly considered with the model identification and model-ID based LCM. For example, model identification and LCM can be done on top of functionality identification. In our understanding, this case is in a scope of model-ID based LCM. For the functionality-based LCM, we have same understanding that there is no model level operation. |
| Ericsson | We don’t see any need for model level awareness. If so, it will add another source of confusion for functionality based LCM. |
| Rakuten Mobile | Share same opinion with Lenovo. |

#### Model configuration

##### Company proposals

Nvidia:

Proposal 11: For AI/ML based enhancements for NR air interface, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

#### Model deployment

No proposal

#### Model delivery and transfer

##### Previous agreements

Working Assumption from RAN1 #111 Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared RAN2 #120 agreement For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions.

##### FL recommendation 3-53d: FL recommendation: Consider the following aspects for discussion of model delivery format options and their feasibility, pros, and cons. The list is provided as a guideline for companies to bring discussion in the next meeting. - Interoperability: does a model transferred from NW/UE side to UE/NW side work in a plug-and-pay manner? - Device capability for compiling and running the model - Hardware efficiency (device-specific optimization) - Proprietary information disclosure across vendors - Testability aspects: including how to involve testing equipment - Offline co-engineering efforts - Feasibility for deployment involving multiple vendors - Flexibility for model update - Model performance - If AI/ML model is transferred from one node to other, which entity guarantees performance, e.g., inference latency? - Impact on other common KPIs (e.g., over-the-air overhead, inference complexity, training complexity, LCM related complexity and storage overhead, [latency]) - Specification effort Model delivery format options under consideration include, at least - Vendor-specific format (e.g., device-specific run-time binary image) - 3GPP-standardized/adopted model representation format (MRF) (e.g.,.h5, .pt, .mlmodel, ONNX, or custom 3GPP-developed MRF) - Any other aspects Note: The discussion is about model delivery format. Note: For the 3GPP-standardized/adopted MRF, if adopted, the choice of format is FFS. Note: Some aspects may belong to RAN2 discussion, in which case RAN1 can give appropriate guidance based on RAN1 understanding. FFS: Applicability to collaboration level y and z

##### Proposed conclusions 4-19b from RAN1 #111 (not agreed): Conclusion on device-specific vs. agnostic model: Concerns with a model developed without device specific considerations: - Suboptimal performance due to unoptimized model design for the device (e.g., device specific input, device specific pre-processing, device specific power/complexity/performance trade-off considerations) - Hardware efficiency (lack of device-specific optimization for hardware implementation) - Interoperability issue: Models may not run in a plug-and-play manner at the device. - Model performance (e.g., inference latency) at the target device may not be guaranteed. - Lack of vendor differentiation, potentially hindering innovation Benefits of a vendor-/device-agnostic model: - Less offline engineering effort across vendors for two-sided model training Conclusion on model delivery using an open-format model compared to proprietary-format models Concerns with model delivery in an open format: - It may require device capability for compiling and running the model. - FFS: device capability for parameter-only update - Proprietary model information is disclosed across vendors. - Specification impact Benefits of model delivery in an open format: - Shorter model update timescale compared to proprietary-format models that need offline model re-training, compiling, and testing Conclusion on model parameter update after deployment Concerns - Unlike in offline training where the trained model can go through extensive functionality and performance testing, model parameter update after deployment may lead to unoptimized and/or unexpected device behavior/performance.

FL comment from RAN1 #111 There are varying opinions on the need of model delivery/transfer. FL encourages - Proponents to bring discussions on why model delivery/transfer may be useful and their use cases - Opponents to bring discussions on why model delivery/transfer is not needed - In which scenarios model delivery/transfer may or may not be needed

Agreement from RAN1 #112 To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary RAN2 #121 agreement Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. Solution 4: Server (e.g., OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g., transparent to 3GPP)

##### Company proposals

Huawei, HiSilicon:

Proposal 2: For the study of model transfer/delivery cases:

- Deprioritize the Case z1, z2 and z3.

- Further study the Case z4 and z5.

- No need to discuss the potential spec impact on Case y.

Proposal 3: Deprioritize the model transfer/delivery from UE to Network.

Proposal 4: For the study of model transfer/delivery from Network to UE, small model size (e.g., to ensure no strong impact to legacy RRC signaling) should be assumed as a starting point for the potential spec impact analysis.

ZTE:

Proposal 13: RAN1 concludes the pros and cons of supporting “proprietary model” and “open-format model”. Then, RAN1 should send LS to other working groups (e.g., RAN2 and SA2) to check the feasibility of supporting open-format models.

Proposal 14: Leave the discussion on model storage and delivery options to RAN2. RAN1 can proceed current work assuming model delivery may be from a 3GPP entity (e.g., gNB, LMF, and CN) or from a non-3GPP entity (e.g., OAM, OTT).

Vivo:

Proposal 15: RAN1 concludes typical model size, frequency of model transfer/update and latency requirement and send LS to RAN2 to facilitate the discussion of solutions for the model transfer.

Proposal 16: Model transfer capability may consider the alignment between UE and network on supported structures, quantization and processing.

Proposal 23: Send LS to SA2 and SA4 to study the potential specification impact of at least model transfer/deliver, model training, data collection and model identification.

CATT:

Proposal 13: Leave the discussion of model delivery/transfer to RAN2. RAN1 may provide some input by sending LS if relative agreements are reached.

Proposal 14: If RAN1 sends an LS to RAN2 on the size of UE-side models, capture CATT’s input in Table 3 in this contribution.

Intel:

Proposal-5 (NW side involvement): Model training, particularly after an initially developed model is acquired, should be supported at a NW-side server that allows customization of a model to the local environment. Subsequent model transfer/delivery to the UE can happen through 3GPP or non-3GPP mechanisms

Proposal-6 (NW side involvement): For model transfer in a proprietary format support a NW to update the parameters of a model at the UE – (including indirect means involving a UE-side server if needed)

Sony:

Proposal 2: RAN1 should study what signalling information would be needed for training and how to transfer an AI/ML model.

Ericsson:

Proposal 10	Model transfer is deprioritized for use cases using one-sided model.

Proposal 11	For CSI with two-sided model, alternatives that do not require model transfer are prioritized in this SI.

Proposal 12 	For model delivery/transfer, consider only model delivery (case y). Model transfer (case z1-z5) are not supported.

Xiaomi:

Proposal 12: At least model delivery/transfer from network to UE should be considered.

Google:

Proposal 6: Since AI/ML models are not expected to be specified, the model transfer and update procedure could be deprioritized.

Samsung:

Proposal #1: Concerning with the feasibility and practicality of AI/ML model transfer, RAN1 should study aspects such as

-   Interoperability: does a model transferred from one node to another node work in a plug-and-pay manner, i.e., without extensive receiving-node-specific optimization, compiling and testing?

-    Proprietary issues: If AI/ML models are considered proprietary assets, model transfer in an open format discloses them.

-   Model transfer format (MTF): does RAN1 need to adopt a common MTF so that a model exchanged between two nodes from different vendors compiles and runs?

-   Performance guarantee: If AI/ML model is transferred from one node to the other, which entity guarantees performance, e.g., inference latency?

Proposal#2: In regards to model delivery/transfer deprioritize cases z1, z2, z3 ,z4, z5 in this study item

- Cases z2, z3, z4 and z5, do not allow the UE to receive and run AI/ML models in a ‘plug-and-play’ manner, i.e., without additional steps for UE-specific compilation and optimization.

- Specification support for case-z1 is not justified as the same UE vendor would train the model. Hence, proprietary solutions, e.g., case-y, can be used.

CAICT:

Proposal 7: Proposal 6-19f discussed in last meeting could be approved.

Proposal 8: For model delivery/transfer cases, z4/z5 could be considered with higher priority than z1/z2/z3.

CMCC:

Proposal 2: Study the following options and potential spec impact of model delivery.

- Opt1. UE specific format based model transfer

- Opt2. Standard format based model transfer

- Opt3. OTT/OAM based model delivery

Proposal 15: Regarding the boundary between z4 and z5,

- In model delivery/transfer Case z4, the “known model structure” means an exact model structure that as has been previously identified between NW and UE and for which whose model ID the UE has and explicitly indicated its support in its UE capability reporting.

- In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not covered appearing in z4, including any model structure that is only partially known.

MediaTek:

Proposal 23: For model transfer, RAN1 focuses on what kind of information needs to be delivered for model transfer. RAN1 leaves model transfer channel (CP or UP) and model format to RAN2 discussion. RAN1 should provide inputs for the requirements of model transfer/delivery in terms of model size, latency and model update frequency to RAN2 to evaluate the applicability of different model transfer/delivery solutions.

Apple:

Proposal 14: For level z, 3GPP consider endorse a few existing AI model formats. 3GPP does not specify its own model format for model delivery.

Lenovo:

Proposal 14: Agree with FL’s proposal on the boundary of z4 and z5 as
“In model delivery/transfer Case z4, the ‘known model structure’ means a model structure that has been previously identified between NW and UE and for which the UE has explicitly indicated its support. 
In model delivery/transfer Case z5, the ‘unknown model structure’ means any other model structure not covered in z4.”

Qualcomm:

Proposal 31: Model training needs to consider feasibility of disclosing proprietary model information to the other side.

Proposal 32: Model structure design needs to consider device-specific design optimization and capability.

Proposal 33: Deprioritize Case z5.

Proposal 34: Model training and conversion to executable involves device-specific optimization.

Proposal 35: Prioritize proprietary format (Cases z1 and z2) over open-format model (Cases z3 and z4) for model transfer.

AT&T:

Proposal 6: Consider “proprietary AI/ML model structure” and “open-source AI/ML model structure” as two separate AI/ML model structure categories for RAN1 discussion,

| Proprietary AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and/or input/outputs that are proprietary from 3GPP perspective. |
| --- | --- |
| Open-source AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and input/outputs that are mutually recognizable across vendors and can allow interoperability, from 3GPP perspective. |

NEC:

Proposal 17: Study AI/ML model transfer with 3GPP network assistance at least for the case of two-sided AI/ML model or when online training is required for an AI/ML model.

Proposal 18: Study AI/ML model transfer using open AI/ML format. FFS details of open format, support of vendor specific algorithms.

NTT Docomo:

Proposal 12: Deprioritize the case z5 model transfer in Rel-18 AI/ML SI discussion.

Proposal 13: Discuss the feasibility of case z4 model transfer before further studying it.

##### Issue 7-21: Model delivery and transfer discussion

I believe that the group has made decent progress on essential aspects of AI/ML framework and for each sub-use-case, centering around inference operations assuming offline training (w/ multi-vendor collaboration/alignment), offline model conversion and testing, and Collaboration Level y. As the basic framework for essential aspects are getting into shape, we’d like to capture analysis, for model transfer/delivery, on the need/benefit, feasibility, and specification impact assessment.

According to the agreement from RAN1 #112, the cases y, and z1-z5 are defined as follows

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

The boundary between z4/z5 is further clarified in 7-23.

For ease of understanding, the Cases y and z1-z5 are illustrated in the below diagram, which was recreated from vivo’s contribution with some monidification.

To have a better understanding, it can be good to list all the pros and cons of each option in addition to the relevant use cases. It is also worth considering the potential spec impact of each option.

- Why is it needed? What are the use cases and benefits?

- What are challenges and feasibility issues to support it? What problems would it cause that needs to be addressed to make it work?

- Potential specification impact?

The purpose of this is so that we’d like to capture analysis summary, and recommendations, if possible, into the TR. This will at least serve as a scoping for future specification.

###### [FL1] Proposed conclusion 7-21a:

The following summarizes the use cases, benefits, challenges/requirements, and potential specification impact of model delivery/transfer Cases.

|  | Use cases | Benefits | Challenges / requirements | Potential specification impact |
| --- | --- | --- | --- | --- |
| Y |  | B1, B2, B3 |  |  |
| Z1 |  | B2 | C1, C2, C8 |  |
| Z2 |  | B2 | C1, C2, C3 |  |
| Z3 |  | B1, B2, B3 | C1, C3, C4, C6, C8 | S1 |
| Z4 |  | B1, B2, B3 | C3, C4, C5, C6, C7, C9 | S1 |
| Z5 |  | B1, B2, B3 | C3, C4, C5, C6, C7, C9, C10, C11 | S1, S2 |

Benefits:

- B1: Shorter model update timescale

- B2: Joint model development across vendors

- B3: Flexibility for model update

Challenges and requirements:

- C1: Larger latency

- C2: Offline co-engineering efforts

- C3: Preservation of proprietary design

- C4: UE capability for compiling, updating and running the model

- C5: Lack of performance guarantee due to lack of testing fully developed model

- C6: Specification effort for model delivery format for open format

- C7: Testability aspects

- C8: Lack of per cell or area optimization if dataset ID is not available

- C9: Full model optimization

- C10: Device specific optimization

- C11: Device capability of running an unknown model structure

Potential specification impact:

- S1: Specification of model format for open-format model transfer

- S2: Flexible UE capability mechanism beyond model ID-based approach

| Company | Comments |
| --- | --- |
| vivo | We mainly have the following comments: - For level y, - there is no benefit of B1 and B3 since it would require complicated co-engineering or exchange of information before deploying the model. - There are following challenges for level y - Complicated co-engineering efforts would be needed for two sided cases and for data not available at single side. - Data collection efforts would be required for UE side which may cause devices storage and need user consent. - Additional risk at device side exists that propriatery information would be exposed. - Large amount of models need to be stored at UE side for site specific operations. Additional user consent is needed if the model delivered consumes user data link resources. - C8 is also the challenge for level y. - For level z4, - With widely used models, there is no challenge of C3; - With aligned model structure, there is no challenge for C4 since the trained model should be aligned with supported UE capability. - C5 and C7 do not exist as long as the RAN4 tests are well defind. These aspects are RAN4 related and are pending on RAN4 discussion. - C6 does not create any issue. SA2 has already specified token based mechanism for interoperability. - We don’t understand C9. What is full model optimization? |
| Panasonic | We may have more comment but current identifiied topics are following. - The meaning of "flexibility for model update" is not yet clear to us. It may be possible in z1 and z2. On the other hand, flexibility for model udpate may not be available when all model handling is within UE in level Y. Even in level Z, if multiple models are transferred to UE and some activations are within UE, model handling is not visble to NW. - On B2 in Z1, as training is neutral or UE side and stroage is NW side, we are not sure " Joint model development across vendors" of B1 is feasible. - On C6 in Z2, as NW side training and UE side compilling, we expect some limitation on the model similar to z4 although some more flexibility is available compared with z4. Therefore, we don't see so big difference between C6 aspect between Z2 and Z4. - C7: testability aspect would be commonly applied when B1: shorter model update time scale is applied as shorter model update time scale may not have sufficient time for the test. - C9: full model optimization and D10 Device specific optimization are applied in all case NW trains the model. |
| Huawei, HiSilicon | 1) Why “B2 Joint model development across vendors” is a benefit? We think it should be drawback as it incurs offline interoperation? 2) For C6, regardless of the model transfer format (proprietary format, open format), there will be effort on the format between vendors – either it is spec effort, or effort offline interoperation with vendor customization. 3) Case y has longer model update timescale (B1) – after it is updated, it needs to be delivered to the non-3GPP server, then deployed to the UE device via FOTA. Same for z3, since it needs to be delivered to the non-3GPP server for compile. 4) For z4, C4/5/7/9 may not exist: UE has performed test/compiling on the model structure. The parameters are directly updated to the binary image w/o re-compiling; the network performance will be ensured by monitoring. |
| NTT DOCOMO | If model is trained at NW side and delivered from NW to UE in case y, the model delivery requires the offline co-engineering effors between UE and NW. So, C2 should be added in case y as well. |
| Google | It seems this needs more discussion. Probably we can try to agree on the categories in B/C/S and come back with the table at next meeting. |
| LG | As some companies proposed in their tdocs, some prioritization of scenario(s) is prefered before going into studying each scenario. |
| CAICT | For Z4, we propose C5 could be removed from challenge/requirements. |
| Samsung | Ok with the direction. As Google suggested may be clearer categorization of aspects may help for convergence. |

##### Issue 7-22: Need of model transfer

###### [FL1] Proposal 7-22a:

Alt 1: Support specification of model transfer for both UE-sided models and UE-part of two-sided models

Alt 2: Support specification of model transfer for UE-part of two-sided models. Model transfer is not needed for UE-sided models.

Alt 3: Do not support specification of model transfer

Note: Further discussion and downscoping can be done in each sub-use-case.

|  | Support | Not support |
| --- | --- | --- |
| Alt 1 |  |  |
| Alt 2 |  |  |
| Alt 3 |  |  |
| Company | Comments |  |
| vivo | We would like to clarify what does “support” mean in the proposal? This is study item and we are trying to study aspects for various options on table. If this recommendation for WI work, we think it is more appropriate to derive the conlusion based on study of options. And based on above analysis in 7-22, we see the benefits of Alt1 being recommended for a future-proof framework. Also from RAN perspective, we don’t see much difference between z1~z5 for specification effort. |  |
| OPPO | We suggest to deprioritize model transfer in Rel-18. We are open to further study it in future release. |  |
| Panasonic | What topics are going to be Release 19 is the outcome of the study item. We are not sure the meaning of "support". In addition, it can be "support for further study" or "supprot for work" can be also different. |  |
| NTT DOCOMO | We could not understand the motivation of this proposal. SI does not need to determine which feature is supported. If the intention of this proposal is not to further discuss the potential specification impacts of model transfer in R18, it makes sense. |  |
| Google | Probably we can say “model transfer is deprioritized in R18”. |  |
| LG | We guess that the proposal is for deprioritizing certain scenario for further study (whether to support or not would be RANP scoping discussion for Rel-19 WI(if supported) or WI-level discussion). Given the very limited time for this SI, we support deprioritizing further study of model transfer. |  |
| CAICT | Alt.1 is preferred. |  |
| Ericsson | We think the model transfer discussion should be downpriortized in this study item, instead focusing on clarifying the functionality and model identification/LCM topic. We should rather agree to downprioritize it than identify the need of it. |  |
| Samsung | We are ok with proposal. What does the Note mean? We think the question here is about feasibility, why should it be use-case specific? |  |

##### Issue 7-23: Model delivery/transfer Case z4/z5 clarification

This is the same as 6-19f that was put for an email approval in RAN1 #112-bis-e. It was very stable without any objection but could not be endorsed as this version was available for review for less than 20 hours before the deadline.

###### [FL1] Proposal 7-23a (same as 6-19f):

In model delivery/transfer Case z4, the “known model structure” means an exact model structure that as has been previously identified between NW and UE and for which whose model ID the UE has and explicitly indicated its support support in its UE capability reporting.

In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not covered appearing in z4, including any model structure that is only partially known.

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | Fine with the clarification. And we would like to clarify that for the case of partially known model, transferring the partillay know part belongs to z4. But the whole model if including the unknown part would belong to z5. There are possible cases that some adaptation layers may be transferred for network to UE but the unknown UE side processing and feature extraction can be kept unknown. |  |
| ZTE | Support. |  |
| Panasonic | Although we are ok with the current separation, instead of discussing only model structure aspect, to discuss what is more feasible and what is more unfeasible in general is more meaningful discussion. As difficulty is not only model structure knowledge. |  |
| Google | OK |  |
| Futurewei | OK with these definitions. |  |
| Lenovo | Agree |  |
| CAICT | Support. |  |

#### Model inference operation

##### Previous agreements

##### Company proposals

Huawei, HiSilicon

Proposal 19: Study the following aspects for pre/post-processing:

- Pre/post-processing methods, e.g. scalability to different input/output dimensions, channel conversion, quantization methods, etc.

- Potential spec impact on how to align the pre/post-processing methods between Network and UE.

Google

Proposal 2: Study parallel model inference based on the same or different AI/ML models.

Nvidia

Proposal 13: For AI/ML based enhancements for NR air interface, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing.

Proposal 14: For AI/ML based enhancements for NR air interface, study potential specification impact related to report/feedback of model inference output and post-processing.

NTT Docomo

Proposal 16: Study the processing unit framework for model inference, when multiple models are compiled at UE.

TCL Communication

Proposal 11: Some constraints shall be added on the post-processing, in order to avoid obtaining an oversimplified low-performance model from post-processing.

#### Functionality/model selection, activation, deactivation, switching, and fallback operation

##### Previous agreements

RAN1 #110bis-e Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms RAN2 #121-bis-e Agreement For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model). For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

##### Company proposals

Continental Automotive

Proposal 9: The pre-determined list of models are used to switch models based on the configured triggering information about activating alternative models for model switching.

Vivo

Proposal 10: To fight against the AI/ML generalization problem, generic model would typically have larger computation complexity and storage overhead, while zone/site specific models would need simple model structure and small model size.

ZTE

Proposal 1: 	Revise the terminologies agreed for model activation, model deactivation, and model switching as follows,

| Model activation | enable an AI/ML model for a specific function AI/ML-enabled feature |
| --- | --- |
| Model deactivation | disable an AI/ML model for a specific function AI/ML-enabled feature |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function AI/ML-enabled feature |

Proposal 15: Model/functionality selection, activation, deactivation, switching and fallback operation should be discussed separately from performance monitoring.

Proposal 17: Network should make final decision on selection, activation, deactivation, switching, and fallback operations of the function unit (either a functionality or a model operated at UE) that is not transparent to network.

Spreadtrum

Proposal 12: For network sided model, the following mechanisms can be further studied for model selection, activation, deactivation, switching, and fallback for network sided models:

- Decision by the network

  - Network-initiated

Huawei, HiSilicon

Proposal 24: For model selection, activation, deactivation, switching, and fallback at least for UE-side models and two-sided models, take the following modes as baseline:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network, and the network indicates UE to execute the decision accordingly

Proposal 25: Study the case where Network pre-monitors the performance of an inactive UE part model or UE-side model before model activation/switching for guaranteeing the network performance.

Nokia

Proposal 9: RAN1 to prioritize Network-controlled Functionality selection, (de)activation, switching, and fallback, and focus only on the following variants:

- Decision by the network – for all Functionality level actions

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE – for deactivation of an activated Functionality

  - Event-triggered as configured by the network, UE’s decision is reported to network

Proposal 10: RAN1 to discuss dynamic methods, triggered by the NW or the UE, for verifying compatibility of the Functionalities at the UE-side and gNB-side for the use cases with two-sided ML models.

CATT

Proposal 1: Add the following definition of functionality switching in the terminology list:

| Functionality switching | Deactivating a currently active AI/ML functionality and activating a different AI/ML functionality for a specific AI/ML-enabled feature. |
| --- | --- |

Proposal 5: To achieve good performance across different scenarios, model generalization, model switching and model update are supported from standardization point of view. Which option will be adopted/implemented is up to vendor’s choice.

Proposal 12: For the purpose of model activation/selection/switching, study necessity, feasibility and potential specification impact for methods to evaluate the applicability of inactive AI/ML models/functionalities, including the following examples:

- Evaluation by comparing the model’s applicability condition to the current conditions

- Evaluation based on input data distribution

- Evaluation through model monitoring by using the inactive model(s) for monitoring purpose and measuring the inference accuracy/system performance

  - FFS: how to mitigate resulting system performance impact if any.

Fujitsu

Proposal-6: For the purpose of model activation/selection/switching, study necessity, feasibility and potential specification impact for methods to evaluate the applicability of inactive AI/ML models, at least including the following examples:

- Evaluation by comparing the model’s applicability condition to the current conditions.

- Evaluation based on input data distribution.

- Evaluation based on output of inactive model.

- Historic knowledge on model performance.

Google

Proposal 3: For 1-side mode, the model selection/switching could be transparent.

Proposal 4: For 2-side mode, the model selection/switching can be configured by the NW or reported by the UE

Proposal 5: Consider to use lower layer signaling, e.g., MAC CE, for model activation/deactivation/fallback operation.

Xiaomi

Proposal 15: Study processing time for the AI model activation/deactivation/switch/fallback

AT & T

Proposal 19: For the purpose of model activation/selection/switching, study necessity, feasibility and potential specification impact for methods to evaluate the applicability of inactive AI/ML models/functionalities, including the following examples:

- Evaluation by comparing the model’s applicability condition to the current conditions

- Evaluation based on input data distribution

- Evaluation through model monitoring by using the inactive model(s) for monitoring purpose and measuring the inference accuracy/system performance

  - FFS: how to mitigate resulting system performance impact if any

NEC

Proposal 6: Study adaptive model selection, activation, deactivation, switching, and fallback without explicit LCM signaling.

Proposal 7: Study autonomous model activation procedure for AI/ML models with assistance of network broadcast signaling.

Sony

Proposal 4: RAN1 should consider supporting the network initiated AI/ML model switching and event-trigger based AI/ML model switching for AI/ML model switching.

CMCC

Proposal 13: For the mechanism of model selection, activation, deactivation, switching, and fallback, if the decision is made by UE, UE’s decision should be reported to the network.

Nvidia

Proposal 11: For AI/ML based enhancements for NR air interface, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

Proposal 12: For AI/ML based enhancements for NR air interface, study potential specification impact related to assistance signalling and procedure for model performance monitoring, model update/tuning, and model selection/switching.

Fraunhofer

Proposal 2: A cost is associated with AI/ML functionality/model activation/deactivation/selection/switching. This cost can encapsulate, for example, the required overhead for measurement, signaling and coordination between UE and NW, as well as the complexity of the functionality/model to be activated.

Proposal 3: To predict the expected performance of inactive AI/ML model(s) without explicitly using them for monitoring purpose and measuring the inference accuracy/system performance, train an estimator from data that can predict the expected benefit of activating the functionality/model, considering the expected performance/QoS the functionality/model will bring, as well as the expected cost due to selection/activation/deactivation/switching of the candidate functionality/model.

LG

Proposal #5: For UE-sided models, model selection/activation/deactivation/switching should be decided by the UE and no need to inform NW on the number of AI/ML models and the currently activated AI/ML model among them. Instead, UE may report updated UE capability/functionality and/or reliability/confidence of the reported values for NW to decide whether or not to use it.

Proposal #7: Consider dynamic reconfiguration of UCI reporting for the case of functionality switch/update for the same sub-use-case.

Proposal #8: For two-sided models, model selection/activation/deactivation/switching can be decided by the NW.

Qualcomm

Proposal 25: Model selection / switching should be based on well-defined conditions that are available during inference. Model monitoring is not a desirable mean for model selection / switching due to potentially high complexity, overhead, and/or latency.

Oppo

Proposal 7: Besides generalized AI/ML models, scenario-dependent AI/ML models should be supported.

- Model switching should be supported because its specification impact is limited if the Local ID is supported.

- FFS: Specification impacts of model update.

Proposal 8: In Rel-18, for model selection, activation, deactivation, switching, and fallback,

- The NW-sided model is decided by NW.

  - The decision may be informed to UE, if needed.

- The UE-sided model is decided by UE.

  - The decision may be informed to NW, if needed.

- For two-sided models, focus on the mechanisms based on network decision.

Samsung

Proposal #14: For approaches to achieve good performance across different scenarios/configurations/sites, e.g., model generalization, model switching, model update, etc., study

- Approaches to discover and/or report scenarios/configurations/sites

- Model input/output pre/post-processing and the additional side-information required to achieve model generalization

TCL Communication

Proposal 9: To minimize the overhead of switching between AI/ML models, it is important to ensure that the models can be switched which sharing the same reference signal configuration.

Proposal 10: The generalization of an ML model is needed to be discussed, according to model deployment, model switching, and alignment of applicable settings.

MediaTek

Proposal 9: Use ‘model control’ to stand for model selection, activation, deactivation, switching, and fallback to facilitate the discussion.

Proposal 10: Clarify model selection as the operation to select the first AI/ML model at the very beginning when AI/ML is enabled.

Proposal 11: Clarify the difference between model switching and model activation/deactivation and discuss the need to keep both.

ETRI

Proposal 6: For the LCM of AI/ML in NR air interface, study AI/ML functionality ID and model ID based activation/deactivation of AI/ML functionality/model.

Proposal 7: For the LCM of AI/ML in NR air interface, study AI/ML functionality/model activation and/or deactivation reflecting other UE status (e.g., DRX).

##### Issue 7-25: model control decision (continuation from 6-23a)

(FL note: The following proposal intends to limit the model control decision to the network side – either directly decided by the network, or UE-request based, or event-triggered as configured by the network. UE autonomous approach is only applicable if the models are transparent to the network.)

###### [FL1] Proposal 7-25a:

For models that are not transparent to the network, UE-autonomous mechanisms should not be considered for selection, activation, deactivation, switching, and fallback and the final decision should be made by the network:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

|  | Yes | No |
| --- | --- | --- |
| Agree? | DCM |  |
| Company | Comments |  |
| vivo | This seems to be use case specific discussion. |  |
| ZTE | It should be applied to both model and functionality. As we commented before, either a model or a functionality is just a function unit operated at UE that is not transparent to network. If it’s not transparent to network, the finally decision should be made by network. |  |
| Panasonic | We agree “models not transpared to the NW” is obviously “UE autonomous mechanism” should be prevented. |  |
| Huawei, HiSilicon | Agree. |  |
| NTT DOCOMO | Support the proposal. |  |
| Google | OK in general. We suggest some revision as follows: - Decision by the UE - Event-triggered as configured by the network or predefined by spec, UE’s decision is reported to network |  |
| Futurewei | Support in general. We may need to clarify the words “decision” and “execution”. In current discussion, we seem to imply that the entire making the decision will execute the corresponding operation. But we have also seen companies using the term “execute”; in this case, the entity making the decision is not the same one that executes the operation. |  |
| Fujitsu | OK |  |
| Lenovo | Agree |  |
| ETRI | Support. |  |
| CAICT | OK. |  |
| Ericsson | Agree with Vivo that it should be use case specific. To identify the scenarios where we think the models are not transparent to the NW. |  |
| Samsung | Some clarification as Futurewei suggested may help. |  |

#### Functionality/model monitoring

##### Previous agreements

RAN1 #110bis-e Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN1 #110bis-e Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system erformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE RAN1 #110bis-e Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

##### Company proposals

Ericsson

Proposal 14: Add the following as model monitoring KPIs: false alarm rate (FAR) and missed detection rate (MDR).

Proposal 15: One-sided models at the NW-side are monitored by the NW. Study data collection mechanisms to support inference-accuracy based model monitoring for NW-sided models if needed for a given use case.

Proposal 16: Study triggering conditions/events/signalling for UE-sided model fallback per use case.

Proposal 17: One-sided models at the UE-side are monitored by the UE.

Proposal 18: The performance monitoring of two-sided models is performed at the NW side. Study mechanisms to support inference-accuracy based model monitoring for two-sided models at the NW side, including UE reporting ground-truth together with UE-part model output to NW, periodic and event-triggered model monitoring procedures.

Continental Automotive

Proposal 5: To identify candidate inactive models need to be further studied in terms of improving model switching performance and minimizing any potential impact (e.g., signaling overhead).

Proposal 6: The relationship between candidate inactive models and the pre-configured parameters (e.g., data drift) can be further studied as a guide of inactive model selection.

ZTE

Proposal 16: For UE-side/part model, depending on which side to calculate the monitoring metrics and whether the monitoring metrics are reported, further study following options:

- UE-side performance monitoring: monitoring metrics are calculated by UE, and the monitoring metrics are not reported to network side.

- Network-side performance monitoring: monitoring metrics are calculated by network (with/without the potential to inform UE about the monitoring metrics).

- Hybrid performance monitoring: monitoring metrics are calculated by UE, and then the monitoring metrics are reported to network side.

Huawei, HiSilicon

Proposal 20: Study the potential procedures included by model monitoring, including data collection, measurement and report, AI/ML and non-AI/ML co-existence.

Proposal 21: For the container of Network side monitoring based on intermediate KPI, at least consider L1 signaling for the UE report to enable fast identification of AI/ML model failure.

Proposal 22: Study the following three modes of model monitoring:

- Mode 1: Network collects inputs for monitoring, calculates monitoring KPI, and makes monitoring decision.

  - This case is applicable to Network-side model and two-sided model.

- Mode 2: UE collects inputs for monitoring, calculates monitoring KPIs which are then fed back to Network, and Network makes monitoring decision.

  - This case is applicable to Network-side model, UE-side model, and two-sided model.

- Mode 3: UE collects inputs for monitoring, calculates monitoring KPI, makes monitoring decision, and reports the decision to Network; Network will indicate UE to execute the decision accordingly.

  - This case is applicable to UE-side model.

  - Network may configure a threshold criterion (e.g., threshold throughput/RSRP, or threshold intermediate KPIs) to facilitate UE to make decision.

Proposal 23: Study the benchmark solutions for model monitoring, including at least:

- Non-AI/ML solution, to make the decision of deactivation/fallback based on the performance comparison with the undergoing AI/ML solution.

- AI/ML solution subject to an inactive model, to make the decision of model switching based on the performance comparison with the undergoing AI/ML solution.

Nokia

Proposal 8: At least for UE-side models and UE-part of two-sided models, functionality or functionalities associated with the ML-enabled feature are always monitored based on an NW-controlled manner.

CATT

Proposal 17: Further study how to monitor the performance of AI/ML functionality. Model monitoring method can be a starting point.

Google

Proposal 8: For model monitoring interval, study the following options:

- Option 1: The model monitoring is performed based on the similar periodicity as RLM/BFD, e.g., every N ms.

- Option 2: The model monitoring is performed with a larger periodicity, e.g., every N second

- Option 3: The model monitoring is performed after each prediction.

Proposal 9: After a detection of the performance failure for a prediction based on a AI/ML model, study the following options:

- Option 1: The model is assumed to be “invalid” for further communication

- Option 2: The detected prediction instance could be “invalid”, but the model may still be used for further communication

Xiaomi

Proposal 13: study the performance monitoring from the following two aspects

- Monitor the performance of activated AI model to assess whether to deactivate this model or update this model

- Monitor the possible performance of AI model not activated to assess whether to activate the AI model.

Proposal 14: Study the mechanism to enable fast performance report

NEC

Proposal 8: Consider to support model monitoring of multiple AI/ML models for the same functionality.

Proposal 9: Support configuring an AI/ML model functionality for monitoring without activation. Further study impact on UE reporting procedure for monitoring and LCM signaling design.

Proposal 10: Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Proposal 11: For model monitoring based on inference accuracy, study methods of ‘ground truth’ data collection.

Proposal 12: For UE-based monitoring, network should be able to configure one or more criteria to UE per AI/ML model/functionality to allow UE to determine model failure.

Proposal 13: Study L1/L2 based mechanism for UE reporting of model failure for UE sided model monitoring.

Proposal 14: For UE sided monitoring, study UE procedure for AI/ML model/functionality handling when UE reports an AI/ML model failure.

Sony

Proposal 3: RAN1 should consider supporting both of periodic and aperiodic feedback some assistance information from UE-side for AI/ML model monitoring.

CMCC

Proposal 14: For NW-sided AI/ML model, study the following mechanism for model monitoring

- Atl1. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Proposal 22: When the performance monitoring metric/method is the input or output data-based monitoring method, such as data drift between training dataset and observed dataset or out-of-distribution detection, the overhead of storing these data and the complexity to compute the input or output data-based KPIs need to be considered.

Fraunhofer

Proposal 6: The AI/ML monitoring has extended AI/ML model fault management capabilities. It encapsulates a Fault Detection and a Fault Diagnosis function.

Proposal 7: Consider the requirements and applicability for simultaneous inference and monitoring at the UE.

Proposal 8: The AI/ML monitoring at the UE, can provide information to the NW at least on AI/ML model functionality, detected fault indicators and associated recommended actions, even for models identified only as logical models in the NW.

Proposal 9: In two-sided model for CSI feedback use case, the gNB monitors the performance of the AI model and detect any possible fault based on the transmitted CSI report from the UE.

Lenovo

Proposal 5: Study the requirements on model monitoring for different AI/ML model LCM purposes, e.g., model selection, switching and fallback, per sub use case

Qualcomm

Proposal 26: To avoid performance issues due to training and target platform differences, it is highly desirable for the trained model to be converted (quantized, compiled) and tested offline prior to being delivered to UE.

Proposal 27: Real-time performance monitoring that incurs high overhead, high complexity, or high latency should be deprioritized.

Oppo

Proposal 9: Target to design a unified AI/ML inference monitoring mechanism supporting AI/ML model switching, 3GPP-based model transfer and model re-training.

- Consider communication performance-based metrics (e.g. MSE, BLER, throughput) as starting point. Complexity and overhead are not considered as metrics for model performance monitoring.

- Study performance prediction mechanism for an unused model.

- Model re-training is considered with low priority.

- Study evaluation methodology for performance monitoring approaches.

Proposal 10: Study on AI/ML training performance monitoring is low priority.

Proposal 11: For AI/ML model performance monitoring,

- The performance monitor of a UE-sided model is the UE.

- The performance monitor of a NW-sided model is the NW.

Samsung

Proposal #8: Study direct model monitoring, e.g., monitoring based on inference latency, and indirect monitoring, e.g., monitoring based on system performance, input/output data distribution, application condition, per use case.

- Prioritize methods that do not require specifying monitoring metrics unless justified.

NTT Docomo

Proposal 14: In performance monitoring, (near) real time-scale performance metrics should be provided to NW for reliable model activation/deactivation/switching/fallback in case of NW-controlled models

Proposal 15: Study the performance monitoring mechanism for inactive models and fallback operation in addition to active models.

TCL Communication

Proposal 7: Monitoring event types include serving become better and serving become worse would be defined on the network, also the associated signaling would be future researched.

Proposal 8: The CSI framework should be enhanced to support monitoring event-based mechanism.

IITM, IITK

Proposal 2: Model monitoring can be conducted with new data (which we refer to as monitoring data). Unlike inference data, monitoring data is one in which the ground truth is known. It is our view that additional test-ing with monitoring data can augment the analysis of inference results and KPIs.

MediaTek

Proposal 12: For network-decided and network-initiated mechanism, model monitoring is performed at the network side.

Proposal 13: For network-sided and UE-initiated mechanism, consider the cases that model monitoring is performed at the UE side or at both UE and network side.

Proposal 14: For UE-decided mechanisms, model monitoring is performed at the UE side.

Proposal 15: If model control is UE-autonomous without reporting the decision to the network, model monitoring and model control at the UE side is implementation specific and will not be specified.

##### Issue 7-26: model validation and monitoring of inactive models (related to 6-25 and 6-28)

(FL note: During RAN1#112bis-e, there were several comments on the similarity between the proposal on inactive models and proposal on validating a model before first active use. The following proposal unifies the two aspects and discusses the problem of evaluating the applicability and performance an inactive model/functionality for potential activation.)

###### [FL1] Proposal 7-26a:

For the purpose of activation/selection/switching of models/functionalities, study necessity, feasibility and potential specification impact for methods to validate the applicability and expected performance of an inactive model/functionality, including the following examples:

- Validation based on the additional conditions associated with the model/functionality

- Validation based on input data distribution

- Validation through monitoring by using the inactive model/functionality for monitoring purpose and measuring the inference accuracy/system performance

  - FFS: how to mitigate resulting system performance impact if any

- Validation based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs)

- FFS: Requirements for the validation to be reliable (e.g., sufficient data coverage during evaluation)

|  | Yes | No |
| --- | --- | --- |
| Agree? |  |  |
| Company | Comments |  |
| vivo | Support the study for model part but not for functionality part. The intention should also be communicated with RAN4 and ask for their views on this. For the purpose of activation/selection/switching of models/functionalities, study necessity, feasibility and potential specification impact for methods to validate the applicability and expected performance of an inactive model/functionality, including the following examples: - Validation based on the additional conditions associated with the model/functionality - Validation based on input data distribution - Validation through monitoring by using the inactive model/functionality for monitoring purpose and measuring the inference accuracy/system performance - FFS: how to mitigate resulting system performance impact if any - Validation based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs) - FFS: Requirements for the validation to be reliable (e.g., sufficient data coverage during evaluation) |  |
| OPPO | Ok to study, but not the high priority for Rel-18. |  |
| ZTE | OK with the direction. It’s not clear about the specification impacts of the following bullet: - Validation based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs) |  |
| Panasonic | Support the view from vivo. |  |
| NTT DOCOMO | Generally fine with the proposal. We suggest adding the note “the combination of above validation methods is also possible”. |  |
| Google | We suggest adding the following options for validation. If the AI/ML can outperform the non-AI/ML, the performance could be good enough. - Validation through monitoring by using the non-AI/ML functionality for monitoring purpose and measuring the inference accuracy/system performance |  |
| Futurewei | Agree in general for model ID based approach. |  |
| Fujitsu | We think the purpose and the procedures of model validation and monitoring on inactive model are quite different, we suggest to discuss two issus separately: - For model validation, it is on validating a model before first active use, thus the possible latency could be large and needs to consider data coverage issue. - While for monitoring inactive model, it relates to model selection/switching/activation, short latency and limited data collection would be desired. |  |
| NEC | To our understanding, validation is a step of model training and should be discussed together with model training, not activation/switching/selection. |  |
| ETRI | Support. |  |
| CAICT | Support. |  |
| Ericsson | Not support. We don’t see the need of also supporting “inactive” models at this stage. We should first progress on how to monitor activated functionalities/models . |  |

#### Model update

##### Company proposals

NEC

Proposal 15: Study the methods to update AI/ML model with minimum interruptions of AI/ML model inference.

Proposal 16: In addition to Functionality (or Feature) and logical model ID, study whether additional information needs to be provided to UE on which model is to be updated.

CAICT

Proposal 9: The process of model updating only applies for deactivated model. If an activated model needs to be updated, it should be deactivated first.

#### UE capability

##### Company proposals

FUTUREWEI:

Proposal 10: When studying UE AI/ML related capabilities, separate physical capabilities from functional capabilities.

Proposal 11: For UE physical capabilities, consider categorizing them that reflects their ability in handling various AI/ML complexities, including pre- and post-processing.

Vivo:

Proposal 20: Study ways for UE to report its capability for data collection regarding expected pre-processing, data storage, feature extraction and report for data collection.

Proposal 21: Study ways for UE to report its capability for latencies with respect to the model inference.

Proposal 22: Study UE capability on supported quantization levels.

CATT:

Proposal 21: For support of AI/ML-based approach at UE, the following capabilities can be considered as optional:

- Capability of online training,

- Capability of implementing transferred AI/ML model (at least for Case z5).

Google:

Proposal 7: For AI/ML based operation, the following UE types should be considered:

- Type 1 UE (low performance UE): AI/ML based operation is based on general processing unit (GPU)

- Type 2 UE (high performance UE): AI/ML based operation can be based on neural processing unit (NPU)

ETRI:

Proposal 8: For the LCM of AI/ML model in NR air interface, study UE capability for AI/ML model inference performance report:

- Capability A: Not capable to provide AI/ML model inference performance

- Capability B: Capable to provide AI/ML model inference performance with GT

- Capability C: Capable to provide AI/ML model inference performance estimate without GT

CMCC:

Proposal 16: For AI-related UE capability, how to define and report the capability of training, power, computation, storage should be studied.

NEC:

Proposal 19: Introduce AI/ML processing units (APUs) to reflect UE capability of AI/ML operations.

Proposal 20: Support UE capability reporting on the supported number of AI/ML models for parallel model monitoring and on the supported methods for model monitoring.

#### Interoperability and testability aspects

##### Previous agreements

Agreement (RAN1 #110bis-e) Study various approaches for achieving good performance across different scenarios/configurations/sites, including - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.] - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example. FL recommendation 3-73d from RAN1 #110bis-e Companies are encouraged to bring discussion on interoperability and testability aspects, including, but not limited to, the following: - Discussion on testing model generalization performance - Discussion on two-sided AI/ML model interoperability and testing - Discussion on involvement of multiple parties including UE, NW, and TE vendors how to support full NW-UE interoperability - Discussion on how to handle multiple models (e.g., model switching, model selection) - Discussion on how to handle model update (e.g., offline and online model update) - Whether and how to test LCM This discussion can also serve as an input for later RAN4 study.

##### Company proposals

Ericsson

Proposal 23: RAN1 should consider requirement setting and testing feasibility of proposed AI PHY solutions but any requirement- and testing-related decisions should be taken by RAN4.

FutureWei

Proposal 12: Study common assumptions, topics, and guidelines for the discussion of interoperability.

Note: this may be use case dependent.

Panasonic

Proposal 3:  For functional LCM, validation may be similar to non-AI/ML operation as all adaptation to the real deployment is rather UE side responsibility. For model-ID-based LCM, the validation using specific UE is required as a kind of test trial. This test trial can be skipped when the same AI/ML model is used for the same manufacture's same model in the same software version. This test trial is required even if the model is trained by the NW as the compilation, quantization and alignment to the UE internal processing related to AI/ML operating environment.

Vivo

Proposal 9:	Send LS to RAN4 on recommendations on reference model structure for each use case.

Huawei, HiSilicon

Proposal 26: The performance test at RAN4 can consider the following two options

- Option 1: Static scenario, where a fixed scenario is considered to construct the testing dataset.

- Option 2: Variable scenario, where more than one scenario is considered to construct the testing dataset to separately test the same AI/ML model.

Proposal 27: RAN1 may need to provide information on the typical scenarios for performance test in RAN4, e.g., by referring the set of scenarios for the generalization verification.

Proposal 28: RAN1 may need to provide training dataset related information to RAN4 for determining the performance requirements. The following two options can be considered:

- Option 1: Construct a common training dataset for each scenario.

- Option 2: The methodology of generating the training dataset, e.g., the EVM of simulation for each scenario, and the method to sample the data in simulation.

Proposal 29: To facilitate RAN4 to perform the test on training, RAN1 can provide information on the procedures of training types that have been identified.

CATT

Proposal 25: From RAN4 perspective, reference model and reference dataset can be considered.

- Reference model serves for requirement design, similar to reference receiver. It does not aim at forcing vendors to implement such reference model in their products.

- Reference dataset serves for test case design, to guarantee the reproducibility of the test.

Fujitsu

Proposal-7: Study the necessity and feasibility of a procedure to validate a UE-side/UE part model after assigned an identification but before used for the first time, including at least:

- NW-side assessment method.

- Reliable validation with the consideration of sufficient data coverage.

- Identification of untested/unvalidated models and tested/validated models.

Fraunhofer

Proposal 4: To ensure sufficient input data coverage during verification of a UE-side or two-sided ML model after identification but prior to its first active use, the UEs capable of performing this validation are given specific patterns of input data (and/or side information) as queries to look for. Once such patterns are detected, data collection for the model validation process is triggered.

Qualcomm

Proposal 28: Study feasibility of RAN4-like tests over-the-air on deployed UEs.

NTT Docomo

Proposal 8: The combination of performance monitoring and RAN4 test under the indicated conditions should be assumed to guarantee the performance.

## Use cases

<empty>

## Evaluations

### Common evaluation methodology and KPIs

#### Dataset and model disclosure

##### Company proposals

CMCC

Proposal 17: A common data set for each use case could be encouraged to be constructed for evaluation and cross-checking of performance.

Nvidia

Proposal 3: Companies are encouraged to contribute real data to the 3GPP Rel-18 AI/ML study for NR air interface to help start to build up sets of real data in 3GPP.

Samsung

Proposal #12: For evaluation purpose, companies should report their results with, at least, a higher level description of their AI/ML model. higher level description includes

-	Types of neural network for AI/ML model, e.g., CNN, LSTM, transformer, etc.

-	Number of layers

#### Model generalization

##### Company proposals

CMCC

Proposal 18: The average performance under multiple configurations / scenarios should be evaluated to evaluate the generalization capability of AI/ML model.

Proposal 19: The performance loss of intermediate or eventual performance KPIs using configurations / scenarios-common models over configurations / scenarios-specific models can also be adopted as the metric for evaluating the generalization performance.

Nvidia

Proposal 5: From a common framework’s perspective, introduce “in-distribution generalization” and “out-of-distribution generalization” in the terminology list and leave the details of generalization types to the discussion of each use case.

Proposal 6: In-distribution generalization: training and test data have the same distribution.

Proposal 7: Out-of-distribution generalization: training and test data do not have the same distribution.

Samsung

Proposal #13: The following cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

#### Common KPIs

##### Previous agreements

Agreement from RAN1 #110 The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIs Note: Use-case specific KPIs may be additionally considered for the given use-case. Agreement from RAN1 #110-bis-e The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency) Conclusion from RAN#1 110-bis-e This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP. Agreement from RAN1 #112 For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

##### Company proposals

Ericsson

Proposal 21: For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values based on HLO representations before optimization (i.e., not accelerator-optimized computational complexity values). Otherwise, the reported computation complexity value cannot be included for a fair cross-company comparison.

Proposal 22: To further 3GPP discussion and preparation of observations/conclusions for the technical report, three model size classes are defined per use case, as follows:

- Small models (e.g., < 1 M model parameters for positioning use cases)

- Medium-size models (e.g., 1 – 8 M model parameters for positioning use cases)

- Large models (e.g., > 8 M model parameters for positioning use cases)

Huawei, HiSilicon

Proposal 31:  Adopt power consumption in common KPI for evaluating the performance benefit of AI/ML. Companies are encouraged to report power consumption for the AI/ML model as part of the evaluation.

CATT

Proposal 6: At least for offline training, companies can voluntarily share their training strategies, but no need to set up training complexity KPIs for comparison.

Proposal 7: It is more proper to discuss inference latency under UE/network capability, rather than common KPI for AI/ML model.

Proposal 8: Power consumption is not considered as a common KPI for AI/ML-based approach.

- It is up to companies’ interest to report their results of power consumption.

CMCC

Proposal 20: The model size can be adopted as one representative KPI to evaluate the overhead of model delivery/transfer.

Proposal 21: The inference latency can be adopted as one common KPI when evaluating the performance of AI/ML model.

Nvidia

Proposal 4: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for NR air interface.

Lenovo

Proposal 16: Consider latency as one of the KPIs/Metrics for the common aspects of an evaluation methodology:

- Latency

  - Latency for data collection for model training and update.

  - Latency for LCM procedures, e.g., model monitoring, update, training data transfer, model activation, deactivation, switching, and fallback operation

Proposal 17: Evaluations of an AI/ML scheme should include analysis of the latency/delays introduced by the AI/ML procedures (e.g., model training, update) and comparisons with the latency requirement of the system and latency for baseline Rel-17 schemes.

NTT Docomo

Proposal 17: Companies can voluntarily provide their models estimating power consumption model based on FLOPs with their expected implementations.

TCL Communication

Proposal 11: To reduce the signaling overhead between the UE and the gNB, a rule is needed to roughly classify the model complexity.

## Potential Specification Impact Assessment

<empty>

### General observations

<empty>

### PHY layer aspects

<empty>

### Protocol aspects

<empty>

### Interoperability and testability aspects

<empty>

## SI structure

##### Company proposals

vivo:

- Observation: RAN2 has agreed to deprioritize aspects of on-line/real-time training for the whole SI.

### RAN1 sub-agendas

### Coordination with RAN2 and SA

##### Company proposals

vivo:

Support to reuse the mechanism defined in SA2 (interoperability token) for aligning model description format for model transfer.

RAN1 concludes typical model size, frequency of model transfer/update and latency requirement and send LS to RAN2 to facilitate the discussion of solutions for the model transfer.

Send LS to SA2 and SA4 to study the potential specification impact of at least model transfer/deliver, model training, data collection and model identification.

##### Issue 7-33: SA2/SA4/SA5 impact

###### [FL1] Conclusions 7-33a:

The following potential impacts to SA2, SA4, and SA5 have been identified during the Rel-18 AI/ML air interface study.

- AI/ML model format for model transfer in open-format models

- Model storage and training at Network of UE-side/part models

- Two-sided model training involving dataset exchange and/or model training API between the Network and the UE-side server.

- Offline model identification (Model identification Type A)

- Certain data collection mechanisms (e.g. EVEX)

| Company | Comments |
| --- | --- |
| vivo | Support and we should send LS to SA for them to study further and provide feedback to RAN. |
| ZTE | Fine with the direction. Two comments from our side: - All the model identification types may have SA impacts. - The signaling framework for data collection is under discussion in RAN2. We don’t need to include the last bullet. |
| Huawei, HiSilicon | Given the limited time of SI, we can focus on the discussions within RAN. |
| NTT DOCOMO | We do not find the benefit to standardize the model storage at NW. Could proponents explain the benefit for that? |
| Google | Agree with HW |

### Coordination with RAN4

##### Company proposals

vivo:

Send LS to RAN4 on recommendations on reference model structure for each use case.

## Others

<empty>

# GTW session (Monday)

###### [FL1] Proposal 7-4a:

Consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection:

- Measurement configuration and reporting

- Contents, type and format of data including:

  - Data related to model input

  - Data related to ground truth

  - Quality of the data

  - Other information

- Signaling of assistance information for categorizing the data

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by appropriate working groups.

###### [GTW1] Proposal 7-15b:

For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any).

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification.

  - FFS: Spec impact to other WGs including SA

- Type B: Model is identified via over-the-air signaling

  - Type B1: UE-initiated model identification of a model known model at UE

    - Model identification initiated by the UE, and NW assists the remaining steps of the model identification

    - FFS: details of steps

  - Type B2: NW-initiated model identification of a model known model at NW

    - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps of the model identification

    - Note: Type B2 may be used in conjunction with model transfer from NW to UE.

    - FFS: details of steps

- Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

###### [GTW1] Proposal 7-11b:

Categorize additional conditions as follows:

Category 1: Additional conditions provided by the NW, e.g., scenario/dataset ID, pairing information for two-sided model operation, site/cell ID.

Category 2: UE’s internal conditions unknown at the NW such as memory, battery, other hardware limitations, temporary unavailability of a model due to the need of model download

###### [GTW1] Proposal 7-25b:

For UE sided models and two-sided models, for models that are not transparent to the network, UE-autonomous mechanisms should not be considered for selection, activation, deactivation, switching, and fallback and the final decision should be made by the network:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network or predefined by spec, UE’s decision is reported to network

# GTW session (placeholder)

# GTW session (placeholder)

# FL notes for the next meeting

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- 3GPP TR 37.817, Technical Specification Group RAN; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancement for Data Collection for NR and EN-DC (Release 17)

- R1-2205522, “Summary of General Aspects of AI/ML Framework”, RAN WG1 #109-e, Moderator (Qualcomm)

- Draft Report of 3GPP TSG RAN WG1 #109-e

- R1-2208178, Summary#1 of General Aspects of AI/ML Framework, RAN WG1 #110, Moderator (Qualcomm)

- Draft Report of 3GPP TSG RAN WG1 #110

- R1-2210708, Summary#5 of General Aspects of AI/ML Framework, RAN WG1 #110bis-e, Moderator (Qualcomm)

- R1-2210801, Report of RAN1#110bis-e meeting

- R1-2213003, Revised final summary of General Aspects of AI/ML framework, RAN WG1 #111, Moderator (Qualcomm)

- R1-xxxxxx, Report of RAN1#111

- R1- 2301868, Final Summary of General Aspects of AIML Framework, RAN WG1 #112, Moderator (Qualcomm)

- R1- 2304054, Final summary of General Aspects of AI/ML Framework, RAN WG1 #112-bis-e, Moderator (Qualcomm)

- R1-2304301, Final Report of 3GPP TSG RAN WG1 #112bis-e v1.0.0

# Working list of terminologies

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Working Assumption

Table 3: Working list of terminologies

| Terminology | Description |
| --- | --- |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model download | Model transfer from the network to UE |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model parameter update | Process of updating the model parameters of a model |
| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model upload | Model transfer from UE to the network |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Unsupervised learning | A process of training a model without labelled data. |
| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

# Agreement from RAN#1 109-e

Agreement

- Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

- Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

# Agreement from RAN#1 110

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIs

Note: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

# Agreement from RAN#1 110-bis-e

Summary#1 of General Aspects of AI/ML Framework	Moderator (Qualcomm Incorporated)	(rev of R1-2210375)

From Oct 11th GTW session

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

R1-2210472	Summary#2 of General Aspects of AI/ML Framework	Moderator (Qualcomm Incorporated)

From Oct 13th GTW session

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

- FFS: Detailed discussion of model ID with associated information and/or model functionality.

- FFS: usage of model ID with associated information and/or model functionality based LCM procedure

- FFS: whether support of model ID

- FFS: the detailed applicable AI/ML operations

Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

R1-2210661	Summary#3 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

From Oct 18th GTW session

Conclusion

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

- Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

- Monitoring based on inference accuracy, including metrics related to intermediate KPIs

- Monitoring based on system performance, including metrics related to system peformance KPIs

- Other monitoring solutions, at least following 2 options.

  - Monitoring based on data distribution

    - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

    - Output-based: e.g., drift detection of output data

  - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

From Oct 19th GTW session

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

- Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

- Overhead (e.g., signaling overhead associated with model monitoring)

- Complexity (e.g., computation and memory cost for model monitoring)

- Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

- FFS: Power consumption

- Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

- Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

- Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

  - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

- Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Conclusion

This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

# Agreement from RAN#1 111

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Funtionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

# Agreement from RAN#1 112

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

      - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

      - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement

For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

# Agreement from RAN#1 112-bis-e

Agreement

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

  - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

  - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

  - FFS: relationship between functionality-based LCM and model-ID-based LCM

- Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Conclusion

From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

- When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model.

Agreement

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities.

- Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

Working Assumption

The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

Working Assumption

| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation |
| --- | --- |

# Agreement from RAN#1 113