3GPP TSG-RAN WG1 Meeting #113	  R1-2306063

Incheon, Korea, May 22 – 26, 2023

Agenda Item:	9.2.2.1

Source:	Moderator (Huawei)

Title:	Summary#6 for CSI evaluation of [113-R18-AIML]

Document for:	Discussion and Decision

# Introduction

In RAN#94-e plenary meeting, a new SID on AI/ML for air-interface was approved for Rel-18 [1], where the use cases include CSI feedback enhancement, and evaluations would be performed for this use case:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This document summarizes the key issues discussed under agenda item 9.2.2.1, and aims to discuss a set of issues for the evaluation of the AI/ML-based CSI feedback enhancements in this meeting.

## 1st round email discussions

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table. Please update your contact information on top of the last meeting if needed.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Samsung | Ameha | amehat.abebe@samsung.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| ZTE | Lun Li Guozeng Zheng | li.lun1@zte.com.cn zheng.guozeng@zte.com.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Intel | Victor Sergeev | victor.sergeev@intel.com |
| Ericsson | Mattias Frenne | Mattias.frenne@ericsson.com |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| AT&T | Isfar Tariq Salam Akoum | isfar.tariq@att.com salam.akoum@att.com |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Mavenir | Ali Fatih Demir Yuanlong Yang | ali.demir@mavenir.com yuanlong.yang@mavenir.com |
| NTT DOCOMO | Haruhi Echigo Liu Liu Xin Wang | haruhi.echigo.fw@nttdocomo.com liul@docomolabs-beijing.com.cn wangx@docomolabs-beijing.com.cn |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| IIT Kanpur | Abhishek Kumar Singh | Abhishekks@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| vivo | Jianming Wu | jianming.wu@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| Mavenir | Fan Yang | fan.yang@mavenir.com |
| Fujitsu | Xin Wang Qun Zhang | wangxin@fujitsu.com zhangqun@fujitsu.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Apple | Huaning Niu | huaning_niu@apple.com |
| MediaTek | Gyubum Kyung Pedram Kheirkhah Sangdeh | gyubum.kyung@mediatek.com Pedram.kheirkhah@mediatek.com |
| InterDigital | MoonIl Lee | MoonIl.Lee@InterDigital.com |
| Nokia | Tosato, Filippo | filippo.tosato@NOKIA.COM |
| Xiaomi | Min Liu | liumin10@xiaomi.com |
| China Telecom | Bei Yang | yangbei1@chinatelecom.cn |
| Fraunhofer | Ebrahim Amiri | ebrahim.amiri@iis.fraunhofer.de |

# Remaining EVM issues

## 1st/2nd round email discussions

### 2.1-1: Monitoring of intermediate KPI for CSI compression

#### Issue#2-1 (Medium priority) How to obtain the K test samples for Step 1

| BJTU | Proposal #1: For the method of obtaining test samples for model monitoring, the field data is preferred |
| --- | --- |
| Lenovo | - Proposal 6: The “K” test samples should include samples: - Drawn from the same scenario/configuration that the model is designed (trained), and - Also samples drawn from scenarios other than the ones used for training of the model. - Proposal 7: For the CSI feedback compression use case, samples of the test dataset do not need to have time-dependency. |
| Qualcomm | - Proposal 1: For Step 1 of the model monitoring methodology, for “FFS how to obtain the K test samples”, use the following methodology – - the K test samples refer to K different monitoring occasions over which the overall monitoring accuracy of a scheme is calculated - each monitoring occasion may further use M CSI samples from the same UE for estimating one instance of KPIactual for that occasion. |
| Samsung | - For evaluation of the performance of the intermediate KPI based monitoring mechanism for CSI compression, the number of samples, eigenvectors, considered for monitoring accuracy evaluation to be reported |

Moderator note: In the last meeting, there is one FFS issue for Step 1 on how to obtain the K test samples.

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, the model monitoring methodology is considered as: - Step1: Generate test dataset including K test samples - FFS how to obtain the K test samples

From the inputs of companies for this meeting, it should avoid sampling all the K test samples from a short time window of a single UE where all the samples are quite time correlated.

On the time/frequency granularity of the K test samples, and whether/how to perform time/frequency domain filtering to the test samples, it may be implementation (as different companies may have different flavors on whether/how to perform filtering)? So, it may be reported by companies.

Moderator note[Rd2]: No change on the question. Please provide more inputs!

Question 2.1.1: For the intermediate KPI monitoring of CSI compression, on how to obtain the K test samples, they should include samples from different UEs or from one UE with time-independency.

- The layer, granularity, whether/how to perform filtering to the samples are reported by companies.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Lenovo | We are not clear with the meening of the filtering so not sure about the billet Also based on Question 2.1.2, we also suggest to add a term like Question 2.1.1: For the intermediate KPI monitoring of CSI compression, on how to obtain the K test samples, they should include samples from different UEs/scenrios or from one UE with time-independency. - The layer, granularity, whether/how to perform filtering to selection of the samples are reported by companies. |
| Qualcomm | In our view, the K test samples refer to K different monitoring occasions over which the overall monitoring accuracy of a scheme is calculated. This should include samples from across different UEs in the test dataset. Each monitoring occasion may further use M CSI samples from the same UE for estimating one instance of KPIactual for that occasion. This assumption should also be clarified, and the corresponding impact on latency must be captured. |
| vivo | It depends on whether a monitoring window is used or not. If the monitoring window is used, the K test samples should be time-related and generated from one UE with time-dependency. If the monitoring window is not used, namely, the suitable switching/fallback decision is made based on the K test samples, which can be generated from different UEs with time-independency. |
| NTT DOCOMO | We think whether time-independency should be considered or not depends on the simulatoin results for performance monitoring. Given that there is no conclusion from the simulation results, the proposal should be more generic without the time-independency description. |
| ZTE | We agree with the K samples should not be quite time-related. However, we are not clear the intention of the bullet, which needs further clarification. |
| OPPO | In our view, the K test samples should be representative of the realistic test dataset. Whether the K test samples are from one UE or multiple UEs, one time occasion or multiple time occasions depend on how the monitored model is deployed and how the test dataset is composed. |

#### Issue#2-2 (Medium priority) How to evaluate the generalization performance of the proxy model for Step 2

| Huawei, HiSilicon | Proposal 2: For intermediate KPI based monitoring mechanism for CSI compression, to evaluate the generalization performance of the proxy model (Case 2), generalization Case 1 and generalization Case 2 can be reused: - Generalization Case 1: The proxy model is trained based on training dataset from one Scenario#A, and then the proxy model is tested on a dataset from the same Scenario#A. - Generalization Case 2: The proxy model is trained based on training dataset from one Scenario#A, and then the proxy model is tested on the dataset from a different Scenario#B. Note: due to imbalanced generalization performance of the actual NW part model and the proxy model, the monitoring accuracy of NW side monitoring may still be robust but it may degrade under UE side monitoring based on proxy model. |
| --- | --- |

Moderator note: In the last meeting, there is one FFS issue for Case 2 (UE side monitoring) on whether/how to evaluate the generalization performance of the proxy model.

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, for Step2 of the model monitoring methodology, the per sample KPIDiff is considered for … - Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE: - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI. - Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100% - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual) - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI. - FFS how to train the proxy model and the resulting monitoring performance, to be reported by companies. - FFS whether/how to evaluate the generalization performance of the proxy model. - Case 3: others are not precluded

The intention of this FFS is to evaluate whether the proxy model can achieve the same or different generalization capability as the actual NW part model which is generally assumed to be with larger size. E.g., if the actual NW part model is capable to achieve generalized performance over different scenarios, whether the proxy model can achieve the same generalized performance (to keep the fixed gap with the actual NW part model); or, it suffers degradation when scenario changes, and thus the gap with the actual NW part model enlarges/fluctuates?

Moderator note[Rd2]: No change on the question. Please provide more inputs!

Question 2.1.2: For the intermediate KPI monitoring of CSI compression, for Case 2: UE side monitoring of intermediate KPI with a proxy model, do you agree to evaluate the generalization performance of the proxy model under various scenarios to observe values of KPIDiff separately?

- E.g., generalization Case 1 and generalization Case 2 are reused to derive KPIDiff_GenrCase1 and KPIDiff_GenrCase2, separately.

| Support/Can accept | NTT DOCOMO |
| --- | --- |
| Object/Concern | vivo |

| Company | View |
| --- | --- |
| Lenovo | If we understand the FL point correctly, To us it is not a generalization performance and it is part of the performance of the monitoring schme, i.e., test samples should be selected from similar and different dataset that is used for training. |
| Fujitsu | Support. The generalization capability of the proxy model is crucial for the feasibility of the UE-side monitoring. The size of the proxy model, which is not expected to be large, should be a KPI in the evaluation of the performance of proxy model. |
| vivo | The proxy model is not used for the generalization, but for checking the performance of its paired actual model instead. One simple way to realize the monitoring relying on the proxy model is: first, prepare N paired models each associated with an actual model and a proxy model, and then, check the best SGCS performance among the N proxy models, whereby make the suitable switching/fallback decision. |
| Moderator | @vivo as the model size is different from the actual model, how can we guarantee it has the stable gap with the actual pair of models? |
| OPPO | We also think this is not issue about the generalization performance about proxy model. We just want to improve the monitoring performance of proxy model, instead of guarantee its generalization performance under various scenarios. |

#### Issue#2-3 (Medium priority) values of thresholds for Step 3 - KPIth_1 for Option 1

| Huawei, HiSilicon | Quantitative monitoring accuracy values are captured in Table 15. Three values for threshold of SGCS gap (i.e., KPIth_1) are considered: 0.01, 0.02, and 0.05; considering the resulting SGCS of inference ranges from 0.7 to 0.9 for layer 1 (take 0.8 for average) and ranges from 0.6 to 0.8 for layer 2 (take 0.7 for average), the threshold of +/- 0.01, +/- 0.02, +/- 0.05 contributes a bias ratio of 2.5%, 5%, and 12.5% for layer 1, and 2.9%, 5.7%, and 14.3% for layer 2. Proposal 3: For UE side monitoring of intermediate KPI based on a proxy CSI reconstruction part (Case 2-1), KPIActual is calculated as KPIActual=KPIproxy+KPIcomp, where KPIproxy is the direct SGCS between the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI, and KPIcomp is the fixed compensation value determined at the training phase. Note: KPIActual is calculated as compensated SGCS of the proxy model, i.e., KPIActual=KPIproxy+KPIcomp, so that the same SGCS threshold KPIth_2=KPIth3 is applied for KPIActual and KPIGenie. |
| --- | --- |
| Intel | Proposal 2: - KPIth_2 and KPIth_3 are defined as a X%-tile of KPIActual and KPIGenie distributions respectively - The distributions are based on DL/UL channel datasets generated by using the same set of UEs - The value of X% can be up to each company or fixed (e.g., X% = 75%) |
| Fujitsu | Observation 22 For NW-side monitoring with the ground-truth CSI quantized by Rel-16 type II-like method, the monitoring error (threshold) can be less than 0.07/0.04/0.03/0.025 under 90% monitoring accuracy by using the Rel-16 type II-like codebook of PC#1/ PC#2/ PC#3/ PC#4, respectively; The AI/ML model with a larger feedback payload for CSI compression needs a higher resolution codebook to achieve the good monitoring performance |
| Vivo | Observation: The accuracy of intermediate KPI based monitoring at UE side with proxy model (evaluated by the percentage of the samples for which KPIDiff<KPIth_1) is ~5% for KPIDiff<0.05, ~1% for KPIDiff<0.10, and ~0.1% for KPIDiff<0.20 if generalization case 1 is considered (i.e., training at scenario#A UMi and testing at scenario#A UMi). Proposal 8: For the case that binary state where KPIActual and KPIGenie is considered to reflect the monitoring accuracy (i.e., computing KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_2,KPIGenie>KPIth_3), clarify how to determine KPIth_2 and KPIth_3 for all K samples in the test datasets. |
| Samsung | Proposal #1: To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, - If monitoring accuracy is computed as the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap, KPIth_1 is computed from legacy benchmark baseline with the closest feedback overhead. |

Moderator note: In the last meeting, we have the following agreement and FFS.

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of - Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie; - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap. … - FFS the values of KPIth_1, KPIth_2, KPIth_3.

For Option 1, several threshold values have been evaluated by companies, e.g., 0.01, 0.025, 0.05, etc., or a percentage of CDF. It is Moderator’s feeling that the values of KPIth_1, at least the Max value is aligned over companies. As one principle, the monitoring accuracy should at least guarantee the relationship between AI/ML and R16 benchmark should not be mixed up (as suggested by Samsung)?

Therefore, we may refer to Issue##3-1, where the SGCS ranges for benchmark (for Layer 1, rank 1) are 0.67~0.72 (~0.7 in average) for CSI payload X, 0.74~0.82 (~0.78 in average) for CSI payload Y, and 0.83~0.87 (~0.85 in average) for CSI payload Z. The gains of AI/ML are 2.6%~ 8.8% (5.7% in average) for CSI payload X, 0%~ 8.1% (4% in average) for CSI payload Y, and 0.9%~ 7% (4% in average) for CSI payload Z. The average SGCS gap between AI/ML and benchmark is then roughly 0.04 for X, 0.03 for Y, and 0.034 for Z. Considering some margin, the max threshold can be 0.06? Companies can select other values lower than 0.06 to observe the sensitivity of solutions.

Moderator note[Rd2]: No change on the question. Please provide more inputs!

Question 2.1.3: For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_1 in Option 1, what is the Max threshold value in your view?

- Note: It is up to companies to simulate smaller values than the Max threshold.

| Company | View |
| --- | --- |
| Qualcomm | We have used 0.05 in our results and propose the same value. |
| vivo | The suitable Max threshould of KPIth_1 depends on what types of scenarios are in use. For instance, in the scenario of LoS, the Max of threshould of KPIth_1 could be small, say 0.1, while in the scenario of NLoS, the Max of threshould of KPIth_1 could be large, say 0.2. Therefore, if we expect to set a Max threshold for evaluation purpose, it is better to couple the Max threshold with the simulation scenario. |

#### Issue#2-3a (Medium priority) values of thresholds for Step 3 – relationship of KPIth_2, KPIth_3 for Option 2

| Huawei, HiSilicon | Proposal 3: For UE side monitoring of intermediate KPI based on a proxy CSI reconstruction part (Case 2-1), KPIActual is calculated as KPIActual=KPIproxy+KPIcomp, where KPIproxy is the direct SGCS between the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI, and KPIcomp is the fixed compensation value determined at the training phase. Note: KPIActual is calculated as compensated SGCS of the proxy model, i.e., KPIActual=KPIproxy+KPIcomp, so that the same SGCS threshold KPIth_2=KPIth3 is applied for KPIActual and KPIGenie. |
| --- | --- |
| Intel | Proposal 2: - KPIth_2 and KPIth_3 are defined as a X%-tile of KPIActual and KPIGenie distributions respectively - The distributions are based on DL/UL channel datasets generated by using the same set of UEs - The value of X% can be up to each company or fixed (e.g., X% = 75%) |
| NTT DOCOMO | Proposal 1: Correct the typo in the agreement related to KPI_Diff calculation of RAN1 #112bis-e as following, Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of - Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie; - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap. - Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_3,KPIGenie>KPIth_2, where KPIth_2 can be same or different from KPIth_3. - Monitoring accuracy is the percentage of the samples for which KPIDiff=0. - FFS other metrics: Misdetection, False alarm, etc. - FFS the values of KPIth_1, KPIth_2, KPIth_3. - FFS whether/how to evaluate the monitoring metrics for Rank>1. After the correction, we have following observations on Option 2 of the KPI_Diff calculations. Observation 9: When either KPI_Actual or KPI_Genie falls into the rage [KPI_th,3, KPI_th,2], the KPI_Diff is always 0 no matter how large the gap between KPI_Actual and KPI_Genie is. Proposal 2: Use Option 1 as mandatory metric for the evaluation of intermediate KPI based monitoring and FFS other metrics. |
| Vivo | As the training objective is to minimize the variance of KPI gap, KPIActual is computed by shifting the intermediate KPI between inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI, where the shifting bias is obtained at training stage. |
| Lenovo | As one example, we have used Case 2-1 to evaluate the performance of the following two different schemes. In both schemes we have assumed KPIth2=KPIth3=0.9 for illustration. |
| ZTE | In this simulation, the same threshold value is applied to KPIth 2 and KPIth 3 |
| Samsung | Proposal #1: To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, - If monitoring accuracy is computed based on binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., set KPIth_2=KPIth_3 and computed from legacy benchmark baseline with the closest feedback overhead. |

Moderator note: In the last meeting, we have the following agreement and FFS.

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of … - Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_2,KPIGenie>KPIth_3, where KPIth_2 can be same or different from KPIth_3. - Monitoring accuracy is the percentage of the samples for which KPIDiff=0. … - FFS the values of KPIth_1, KPIth_2, KPIth_3.

The principle for how to determine KPIth_2 and KPIth_3 has been discussed in Tdocs for this meeting. To make it simple, as many companies (Huawei, HiSilicon, vivo, Lenovo, ZTE) assumed in the evaluation assumption or suggested in the proposal, we may assume equal value to KPIth_2 and KPIth_3. For the case where the proxy model has lower performance than the actual decoder at the NW side, we may consider a fixed compensation on top of the proxy model, i.e., KPIActual=KPIproxy+KPIcomp as suggested by Huawei, HiSilicon.

Moderator note[Rd2]: Changed from “Note” to an “example” based on Lenovo comments. Please provide more inputs!

Upd Proposal 2.1.1: For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_2 and KPIth_3 in Option 2, consider KPIth_2=KPIth_3.

- E.g., Note: KPIActual is calculated as KPIActual=KPIproxy+KPIcomp, where  KPIproxy is the intermediate KPI of/calculated by the direct output of the proxy model, and KPIcomp is the fixed compensation value. It is up to companies to determine KPIcomp, e.g., it can be determined at the training phase of the proxy model.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Lenovo | We are okay with the main proposal but prefer to remove the bullet point. |
| NTT DOCOMO | Support the proposal. |
| vivo | It seems that in different cases, the definition of KPIActual is different. We need to define KPIcomp, first, which impact the value of KPIActual. |
| ZTE | We agree with the main text, while for e.g., we think the fixed compensate value is strongly related the training procedure among companies, the KPIcomp can be positive value, negative value, or maybe zero. |
| OPPO | We are general okay with this proposal. We think that how the KPIcomp is determined and obtained should also be discussed. |

#### Issue#2-3b (Medium priority) values of thresholds for Step 3 – value of KPIth_2, KPIth_3 for Option 2

Regarding the value of KPIth_2/KPIth_3, Huawei, HiSilicon adopts 0.85 and 0.9, ZTE adopts 0.7 and 0.8, and Lenovo adopts 0.9.

It is Moderator’s understanding that the selection of depends on the absolute/mean value of the SGCS of AI/ML. E.g., as described in Issue#2-3, if the AI/ML based SGCS is ~0.74 in average for CSI payload X, ~0.81 in average for CSI payload Y, and ~0.88 in average for CSI payload Z, the threshold could be between 0.7~0.9 and related with CSI payload size.

Therefore, either we align one or two values for KPIth_2/KPIth_3 for each CSI payload size (e.g., two values based on averaged SGCS +/-0.05), or we do not align KPIth_2/KPIth_3, but leave it to companies to report.

Moderator note[Rd2]: No change on the question. Please provide more inputs!

Question 2.1.4: For the intermediate KPI monitoring of CSI compression, for the values of threshold of KPIth_2/KPIth_3 in Option 2, which of the following options do you prefer?

- Option 1: Align the KPIth_2/KPIth_3 over companies for each CSI payload size X/Y/Z, e.g., 0.7/0.8 for CSI payload X, 0.75/0.85 for CSI payload Y, and 0.8/0.9 for CSI payload Z.

- Option 2: No need to align. Up to companies to report KPIth_2/KPIth_3.

| Option 1 | NTT DOCOMO |
| --- | --- |
| Option 2 |  |

| Company | View |
| --- | --- |
| Lenovo | Assuming that we agree on that KPIth=KPIth_2=KPIth_3 then we can user ROC curev and then report AUC of the curve for the comparison. |
| NTT DOCOMO | Prefer Option1. If it is not aligned, we are not sure how to derive the RAN1’s observation based on the collected values over the companies. |
| vivo | It can be discussed after Issue#2-3 and Issue#2-3a. |
| ZTE | We are OK with Option 1, since it is hard to derive an observation if the collected results are not aligned. However, regarding the AI/ML models are different among companies, the threshold for X,Y,Z needs further discussed. In addition, we agree with Lenovo that KPIth=KPIth_2=KPIth_3 can be assumed. |
| OPPO | Agree with vivo’s comments. |

#### Issue#2-4 (Medium priority) other metrics: Misdetection, False alarm for Step 3

| Intel | Proposal 2: - For Option 2 KPIDiff calculation, misdetection and false alarm metrics can be optionally disclosed by companies on top of the KPIDiff |
| --- | --- |
| Lenovo | Proposal 8: For a model monitoring scheme, the consequence/cost associated with miss-detection and false-alarm event are not the same. Therefore, we suggest companies to report evaluate/report the Miss-detection-rate and False-alarm-rate separately when evaluating different monitoring schemes. |
| ZTE | - Proposal 9: FAR (false alarm rate) and MAR (miss alarm rate) can be used in addition to monitoring accuracy to analyze monitoring performance. The definition of FAR and MAR are as following: FAR: samples for (KPIActual<KPIth 2,KPIGenie>KPIth 3)samples for (KPIGenie>KPIth 3)×100% MAR: samples for (KPIActual>KPIth 2,KPIGenie<KPIth 3)samples for (KPIGenie<KPIth 3)×100% |

Moderator note: In the last meeting, the metric of monitoring accuracy has been agreed. For whether/how to additionally consider the misdetection and false alarm, there is one FFS issue. In this meeting, ZTE and Lenovo propose to capture misdetection and false alarm as additional metric, and Intel propose to optionally consider these two metrics. On the other hand, in the last meeting a couple of companies think the monitoring accuracy is enough, and no need to introduce other metrics. To collect views of other companies for this meeting, the following question is raised.

Moderator note[Rd2]: No change on the question. Please provide more inputs!

Question 2.1.5: For the intermediate KPI monitoring of CSI compression, besides the metric of monitoring accuracy, do you think it is needed to futher introduce other metrics of misdetection and false alarm?

- For Option 1, misdetection is the percentage of the samples for which KPIDiff>KPIth_1, false alarm is the percentage of the samples for which KPIDiff<- KPIth_1, where KPIth_1 is a positive value.

- For Option 2, considering K test samples in the test dataset,

  - misdetection is the percentage of

Number of samples for (KPIActual>KPIth_2,KPIGenie<KPIth_3)K

  - false alarm is the percentage of

Number of samples for (KPIActual<KPIth_2,KPIGenie>KPIth_3)K

| Support/Can accept |  |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Lenovo | We are in favor of Option-2 with possible simiplification that we assume KPIth_2=KPIth_3 |
| Qualcomm | Whether a sample should be considered a false alarm or misdetection should be discussed based on the implication of the decision. For example, a false alarm could be defined as a case where the true performance is good but the model monitoring decision is a failure. However, that would require more discussion and could lead to more complicated evaluations. We prefer to use the intermediate KPI accuracy as the accuracy metric. |
| vivo | The misdetection and false alarm should be introduced in normative work. |
| ZTE | We support the statistical method of Option 2, and agree with Lenovo that KPIth_2=KPIth_3 can be assumed. |

### 2.1-2: Categorization of training types (On hold)

### 2.1-3: Template related issues

#### Issue#2-5 (Medium priority) RI calculation method

| Apple | Proposal 1: When e-type II codebook is used, two options for RI determination: - Option 1: Sequential approach. RI is determined based on ideal eigen-vector. PMI is searched based on RI. - Option 2: Joint approach. RI is determined based on the best PMI for each RI hypothesis. Proposal 2: When AI based CSI compression is used, two options for RI determination: - Option 1: RI is determined based on ideal eigen-vector. RI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation. - Option 2: RI is calculated based on the output of CSI reconstruction part from the realistic channel estimation. RI is determined based on the best PMI for each RI hypothesis. Proposal 3: In result reporting table, add a row to describe the RI determination method for AI and e-type II separately. |
| --- | --- |

Moderator note: In the last meeting, we have agreed to add an entry in the template to report the CQI calculation method for AI/ML. One key reason is that different options may have different spec impacts. For the RI determination, as Apple pointed out, it is similar to the CQI report, that different mechanisms may have different spec impacts. Therefore, we may change the entry to be “CQI/RI” to be complete.

On the other hand, for the CQI/RI determination methods of benchmark, different companies may have quite different implementation methods, thus, how much useful information it addresses is not clear.

Moderator note[Rd2]: Seems fine to companies. Changed to a proposal and Medium priority.

QuestionProposal 2.1.6: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, update the entry of CQI determination method(s) to include also the RI determination:

| Common description | Input type |
| --- | --- |
|  | Output type |
|  | Quantization /dequantization method |
|  | Rank/layer adaptation settings for rank>1 |
|  | CQI/RI determination method(s) for AI/ML (Option 1a/1b/1c/2a/2b, etc.) |

| Support/Can accept | Apple, vivo, NTT DOCOMO, Fujitsut, OPPO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Apple | For e-type II, whether UE choose to perform joint RI/PMI/CQI or perform RI selection first is up to implementation. But the e-type II design enables the implemeantion choice. For AI based solution, due to the fact UE does not have decoder, the joint RI/PMI/CQI is difficult. Therefore, it should be studied whether similar flexibility like e-type 2 can be enabled by AI solution. In general, we observe SLS performance when joint RI/PMI/CQI selection is done is much higher than perform RI selection first, particularly at lower parameter configuration. This observation is true for both e-type 2 and AI based solution. If e-type 2 choose joint RI/PMI/CQI implemention, while AI based one choose eigen-vector based solution, we see large performance loss. |
| NTT DOCOMO | Support the proposal. We observe that the RI determination affects the eventual KPI more than CQI. It is better to take the RI determination method into consideration, when comparing the evaluation results over the companies. |

#### Issue#2-6 (Low priority) Additional CSI feedback overhead D

| ZTE | Proposal 2: For rank 2/3/4, it is necessary to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= 1.9, and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits. |
| --- | --- |

Moderator note: In the last meeting, for the WA of the CSI feedback overhead, there is one FFS issue on the additional CSI feedback overhead D.

Working Assumption For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback overhead for the metric of eventual KPI (e.g., mean/5% UPT) is re-determined as: - CSI feedback overhead A: <=β* 80 bits. - CSI feedback overhead B: β* (100bits – 140 bits). - CSI feedback overhead C: >=β* 230 bits. - Note: β=1 for max rank = 1, andβ=1.5 for max rank = 2/3/4. - FFS for rank 2/3/4, whether to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= [1.9], and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits. - Note: companies additionally report the exact CSI feedback overhead they considered

From the submitted results, it seems only one company submitted results on the additional CSI feedback overhead D. To Moderator’s understanding, it seems no strong need to add this entry on top of the template/observations which are already complicated.

Moderator note[Rd2]: No change on the question. Please provide more inputs!

Question 2.1.7: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, for rank 2/3/4, do you think it is necessary to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= [1.9], and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits?

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | Although we observe different performance gains from CSI feedback overhead C and D, if only ZTE provides the simulation results for CSI overhead D, we can compromise to the original version of CSI feedback overhead A/B/C as long as our results can be incorporated into FL’s summary. |

### 2.1-4: Others

Question 2.1.8: For the evaluation methodology related aspects on CSI compression or CSI prediction, what other issues do you think is necessary to be discussed?

| Company | View |
| --- | --- |
| Samsung | It is useful if some observations are drawn by comparing the complexity of AI/ML models and the algorithms to obtain legacy CSI. Companies have already submitted the complexity in terms of FLOPs for their AI/ML models. The same can be done for legacy CSI calculation. Moreover, it can be left up to companies how to obtain the complecity of legacy CSI but companies can employ the basic FLOPs counting methods for matrix/vectors operations such as multiplications, additions, SVD, etc. |

## 3rd round email discussions

#### Issue#2-2 (Medium priority) How to evaluate the generalization performance of the proxy model for Step 2

Moderator note[Rd3]: As clarified in the first round, the intention of evaluating whether the proxy model can achieve the same or different generalization capability as the actual NW part model which is generally assumed to be with larger size.

E.g., if the actual NW part model is capable to achieve generalized performance over different scenarios, whether the proxy model can achieve the same generalized performance (to keep the fixed gap with the actual NW part model); or, it suffers degradation when scenario changes, and thus the gap with the actual NW part model enlarges/fluctuates?

Upd Question 2.2.1: For the intermediate KPI monitoring of CSI compression, for Case 2: UE side monitoring of intermediate KPI with a proxy model, evaluate the generalization performance of the proxy model under various scenarios to observe values of KPIDiff separately?

- E.g., generalization Case 1 and generalization Case 2 are reused to derive KPIDiff_GenrCase1 and KPIDiff_GenrCase2, separately.

- Note: the target is to evaluate whether the performance of the proxy model and the actual pair of models are equally impacted when the scenario changes.

#### Issue#2-3 (Medium priority) values of thresholds for Step 3 - KPIth_1 for Option 1

Moderator note[Rd3]: Limited inputs in the first/second round. Made as a proposal in the third round.

Proposal 2.2.1: For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_1 in Option 1, the Max threshold value is set as one of the following options:

- Option 1: 0.05

- Option 2: 0.1

- Option 3: other values

- Note: It is up to companies to simulate smaller values of KPIth_1 than the Max threshold.

| Option 1 | NTT DOCOMO, Qualcomm |
| --- | --- |
| Option 2 |  |
| Option 3 | vivo |

| Company | View |
| --- | --- |
| vivo | The suitable Max threshould of KPIth_1 depends on the types of scenarios and the size of proxy model in use. For instance, in the scenario of LoS, the Max of threshould of KPIth_1 could be small, say 0.1, while in the scenario of NLoS, the Max of threshould of KPIth_1 could be large, say 0.2. Therefore, if we expect to set a Max threshold for evaluation purpose, it is better to couple the Max threshold with the simulation scenario. In addtion, the larger the size of proxy model, the smaller the gap in between, and vice versa. |

#### Issue#2-3a (Medium priority) values of thresholds for Step 3 – relationship of KPIth_2, KPIth_3 for Option 2

Moderator note[Rd3]: No change on top of the updated version in the second round. More inputs in the third round.

Proposal 2.2.2: For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_2 and KPIth_3 in Option 2, consider KPIth_2=KPIth_3.

- E.g., KPIActual is calculated as KPIActual=KPIproxy+KPIcomp, where  KPIproxy is the intermediate KPI of/calculated by the direct output of the proxy model, and KPIcomp is the fixed compensation value. It is up to companies to determine KPIcomp, e.g., it can be determined at the training phase of the proxy model.

| Support/Can accept | NTT DOCOMO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Lenovo | We are okay with the main proposal but we do are in line with the “E.g.” part as we are not clear why we have KPIActual=KPIproxy+KPIcomp. |
| NTT DOCOMO | Support the proposal. |
| vivo | It seems that in different cases, the definition of KPIActual is different. We need to define KPIcomp, first, which impact the value of KPIActual. For sake of simplicity, however, we can set KPIth_2=KPIth_3. |
| ZTE | We agree with the main text, while for e.g., we think the fixed compensate value is strongly related the training procedure among companies, the KPIcomp can be positive value, negative value, or maybe zero. |
| OPPO | We are general okay with this proposal. We think that how the KPIcomp is determined and obtained should also be discussed. |

#### Issue#2-3b (On hold) values of thresholds for Step 3 – value of KPIth_2, KPIth_3 for Option 2

Moderator note[Rd3]: To be discussed after Issue#2-3a is resolved.

## 4th round email discussions

#### Issue#2-2 (Medium priority) How to evaluate the generalization performance of the proxy model for Step 2

Moderator note[Rd4]: Forgot to add table for company views in the 3rd round. Please start your input in the fourth round!

As clarified in the first round, the intention of evaluating whether the proxy model can achieve the same or different generalization capability as the actual NW part model which is generally assumed to be with larger size.

E.g., if the actual NW part model is capable to achieve generalized performance over different scenarios, whether the proxy model can achieve the same generalized performance (to keep the fixed gap with the actual NW part model); or, it suffers degradation when scenario changes, and thus the gap with the actual NW part model enlarges/fluctuates?

Question 2.3.1: For the intermediate KPI monitoring of CSI compression, for Case 2: UE side monitoring of intermediate KPI with a proxy model, evaluate the generalization performance of the proxy model under various scenarios to observe values of KPIDiff separately?

- E.g., generalization Case 1 and generalization Case 2 are reused to derive KPIDiff_GenrCase1 and KPIDiff_GenrCase2, separately.

- Note: the target is to evaluate whether the performance of the proxy model and the actual pair of models are equally impacted when the scenario changes.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | We agree to evaluate the performance of model monitoring when scenario changes. But it is not suitable to use ‘generalization performance’ here. We suggest to delete the word ‘generalization’ in the main bullet. It should also be clearly defined that for Case 2 in the e.g., both the UE-side CSI generation part model + proxy model and the UE-side CSI generation part model +NW side part model are trained on scenario#A and tested on scenario#B. |

#### Issue#2-3b (Medium priority) values of thresholds for Step 3 – value of KPIth_2, KPIth_3 for Option 2

Moderator note[Rd4]: As we have agreed KPIth_2=KPIth_3 on Issue#2-3a, next we need to handle the values of KPIth_2.

It is Moderator’s understanding that the selection of depends on the absolute/mean value of the SGCS of AI/ML. E.g., as described in Issue#2-3, if the AI/ML based SGCS is ~0.74 in average for CSI payload X, ~0.81 in average for CSI payload Y, and ~0.88 in average for CSI payload Z, the threshold could be between 0.7~0.9 and related with CSI payload size.

Therefore, either we align one or two values for KPIth_2/KPIth_3 for each CSI payload size (e.g., two values based on averaged SGCS +/-0.05), or we do not align KPIth_2/KPIth_3, but leave it to companies to report.

As the ranges of SGCS from companies may be quite diverse, maybe it is not practical to force the companies to align to a single SGCS threshold; thus, Option 2 is sligntly preferred by Moderator.

Proposal 2.3.1: For the intermediate KPI monitoring of CSI compression, for the values of threshold of KPIth_2 (equal to KPIth_3) in Option 2, down select from the following options.

- Option 1: Align the KPIth_2 over companies for each CSI payload size X/Y/Z, e.g., 0.7 or 0.8 for CSI payload X, 0.75 or 0.85 for CSI payload Y, and 0.8 or 0.9 for CSI payload Z.

- Option 2: No need to align. Up to companies to report KPIth_2/KPIth_3.

| Option 1 | NTT DOCOMO |
| --- | --- |
| Option 2 |  |

| Company | View |
| --- | --- |
| Lenovo | Assuming that we agree on that KPIth=KPIth_2=KPIth_3 then we can user ROC curev and then report AUC of the curve for the comparison. |
| NTT DOCOMO | Prefer Option1. If it is not aligned, we are not sure how to derive the RAN1’s observation based on the collected values over the companies. |
| vivo | It can be discussed after Issue#2-3 and Issue#2-3a. |
| ZTE | We are OK with Option 1, since it is hard to derive an observation if the collected results are not aligned. However, regarding the AI/ML models are different among companies, the threshold for X,Y,Z needs further discussed. In addition, we agree with Lenovo that KPIth=KPIth_2=KPIth_3 can be assumed. |
| OPPO | Agree with vivo’s comments. |
| ZTE | We prefer Option 2. |

#### Issue#2-3c (High priority) Down selection of Option1 and Option 2 for Step 3

Moderator note[Rd4]: In the last meeting, we have agreed two options.

Agreement To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of - Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie; … - Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_2,KPIGenie>KPIth_3, where KPIth_2 can be same or different from KPIth_3. …

For this meeting, some companies prefer to down select to Option 1 (vivo, DOCOMO) since it is simple, and the threshold is easily aligned over companies since the threshold represents a relative gap; on the contrary, the appropriate threshold value of Option 2 could be related with the channel status, CSI payload size, etc., which is an absolute value, and can hardly be aligned over companies. Therefore, let’s see if we can make Option 1 as mandatory, and Option 2 as optional.

Proposal 2.3.2: For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIDiff achieved in the RAN1#112bis-e meeting, consider Option 1 (Gap between KPIActual and KPIGenie) as mandatory, and Option 2 (Binary state of KPIActual and KPIGenie relationship) as optional.

Moderator note[Rd4]-Upd: Updated based on today’s offline.

Upd Proposal 2.3.2: For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIDiff achieved in the RAN1#112bis-e meeting, as baseline for calibration purpose, consider Option 1 (Gap between KPIActual and KPIGenie).

- Option 2 (Binary state of KPIActual and KPIGenie relationship) as optional and up to companies to report.

  - Results subject to Option 2, may be captured as a note in observation

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Intel | We prefer to use word baseline instead of mandatory. |
| Futurewei | We agree with Intel that using “baseline” wording is better. |
| ZTE | We can compromise to this proposal, however, we suggest the valid results of Option 2 should be captured into TR. |
| OPPO | Agree with Intel’s comments. |

#### Issue#2-5 (High priority) RI calculation method

Moderator note[Rd4]: Seems fine to companies. No changes on top of the second round discussion.

Proposal 2.3.3: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, update the entry of CQI determination method(s) to include also the RI determination:

| Common description | Input type |
| --- | --- |
|  | Output type |
|  | Quantization /dequantization method |
|  | Rank/layer adaptation settings for rank>1 |
|  | CQI/RI determination method(s) for AI/ML (Option 1a/1b/1c/2a/2b, etc.) |

| Support/Can accept | Apple, vivo, NTT DOCOMO, Fujitsut, OPPO, Futurewei |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Apple | For e-type II, whether UE choose to perform joint RI/PMI/CQI or perform RI selection first is up to implementation. But the e-type II design enables the implemeantion choice. For AI based solution, due to the fact UE does not have decoder, the joint RI/PMI/CQI is difficult. Therefore, it should be studied whether similar flexibility like e-type 2 can be enabled by AI solution. In general, we observe SLS performance when joint RI/PMI/CQI selection is done is much higher than perform RI selection first, particularly at lower parameter configuration. This observation is true for both e-type 2 and AI based solution. If e-type 2 choose joint RI/PMI/CQI implemention, while AI based one choose eigen-vector based solution, we see large performance loss. |
| NTT DOCOMO | Support the proposal. We observe that the RI determination affects the eventual KPI more than CQI. It is better to take the RI determination method into consideration, when comparing the evaluation results over the companies. |

## 5th round email discussions

#### Issue#2-2 (Medium priority) How to evaluate the generalization performance of the proxy model for Step 2

Moderator note[Rd5]: Changes made based on OPPO comments.

As clarified in the first round, the intention of evaluating whether the proxy model can achieve the same or different generalization capability as the actual NW part model which is generally assumed to be with larger size.

E.g., if the actual NW part model is capable to achieve generalized performance over different scenarios, whether the proxy model can achieve the same generalized performance (to keep the fixed gap with the actual NW part model); or, it suffers degradation when scenario changes, and thus the gap with the actual NW part model enlarges/fluctuates?

Upd Proposal 2.4.1: For the intermediate KPI monitoring of CSI compression, for Case 2: UE side monitoring of intermediate KPI with a proxy model, evaluate the generalization performance of the proxy model under various scenarios to observe values of KPIDiff separately.

- E.g., generalization Case 1 and generalization Case 2 are reused to derive different KPIDiff valuesKPIDiff_GenrCase1 and KPIDiff_GenrCase2, separately; for generalization Case 2, both the actual pair of models and the proxy model are trained under scenario#A and tested on scenario#B.

- Note: the target is to evaluate whether the performance of the proxy model and the actual pair of models are equally impacted when the scenario changes.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | We agree to evaluate the performance of model monitoring when scenario changes. But it is not suitable to use ‘generalization performance’ here. We suggest to delete the word ‘generalization’ in the main bullet. It should also be clearly defined that for Case 2 in the e.g., both the UE-side CSI generation part model + proxy model and the UE-side CSI generation part model +NW side part model are trained on scenario#A and tested on scenario#B. |

#### Issue#2-5 (High priority) RI calculation method

Moderator note[Rd5]: Seems fine to companies. No changes on top of the previous round.

Proposal 2.4.2: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, update the entry of CQI determination method(s) to include also the RI determination:

| Common description | Input type |
| --- | --- |
|  | Output type |
|  | Quantization /dequantization method |
|  | Rank/layer adaptation settings for rank>1 |
|  | CQI/RI determination method(s) for AI/ML (Option 1a/1b/1c/2a/2b, etc.) |

| Support/Can accept | Apple, vivo, NTT DOCOMO, Fujitsut, OPPO, Futurewei |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Apple | For e-type II, whether UE choose to perform joint RI/PMI/CQI or perform RI selection first is up to implementation. But the e-type II design enables the implemeantion choice. For AI based solution, due to the fact UE does not have decoder, the joint RI/PMI/CQI is difficult. Therefore, it should be studied whether similar flexibility like e-type 2 can be enabled by AI solution. In general, we observe SLS performance when joint RI/PMI/CQI selection is done is much higher than perform RI selection first, particularly at lower parameter configuration. This observation is true for both e-type 2 and AI based solution. If e-type 2 choose joint RI/PMI/CQI implemention, while AI based one choose eigen-vector based solution, we see large performance loss. |
| NTT DOCOMO | Support the proposal. We observe that the RI determination affects the eventual KPI more than CQI. It is better to take the RI determination method into consideration, when comparing the evaluation results over the companies. |

# Observations/conclusions for CSI compression

## 1st/2nd round email discussions

### 3.1-1: Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability

#### Issue#3-1 (High priority) SGCS gain over benchmark

Moderator note: Summarized SGCS resulst are provided in the table below. “L” means “Layer”, “R” means “Rank”. In addition, as only 4 companies submitted results for rank 4, and observe broad range on SGCS (due to loss function?), the rank 4 results are not captured.

Principle of filtering: For some results which largely bias from the majority (either benchmark is too low/too high, or the gain bias from the majority), they are marked in grey and not captured into the observation.

Moderator note[Rd2]:

- Add one note based on FW comment (add the LUT based VQ solution, with a “note” added to clarify the results bias from the majority). Please see if it is acceptable.

- Add Apple to L1, R1/L1, R2, and L2, R2.

- As comments from ETRI, the SGCS gain for L1, R1 Y is updated as 0.9%~8.1%

|  |  | SGCS L1, R1 | SGCS L1, R2 | SGCS L1, R4 | SGCS L2, R2 | SGCS L2, R4 | SGCS L3, R4 | SGCS L4, R4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW#1 (FTP) TF | X | 8.8% | 8.8% |  | 14% |  |  |  |
|  | Y | 4.5% | 4.5% |  | 7% |  |  |  |
|  | Z | 6.5% | 6.5% |  | 9% |  |  |  |
| Nokia #1 (FTP/FB) TF | X | 5.6% | 4.2% |  | 14.3% |  |  |  |
|  | Y | 1.2% | 1.3% |  | 3.6% |  |  |  |
|  | Z | 2.2% | -0.2% |  | 5.9% |  |  |  |
| Futurewei#1 (FTP) | X | 10.2% | 4.04% |  | 1.13% |  |  |  |
|  | Y | 4.4% | 3.13% |  | 1.45% |  |  |  |
|  | Z | 4.6% | 1.4% |  | -0.39% |  |  |  |
| Futurewei#2 (FTP) | X | 11.6% |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| Lenovo#1 (FTP) TF | X |  |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |
|  | Z | 0.9% |  |  |  |  |  |  |
| ZTE#1 (FTP/FB) TF | X | 6.43% | 4.67% | 3.96% | 5.92% | 6.77% | 32.12% | 91.85% |
|  | Y | 6.44% | 4.04% | 3.11% | 5.90% | 3.72% | 15.02% | 37.16% |
|  | Z | 6.97% | 6.44% | 5.64% | 9.95% | 4.79% | 14.92% | 31.94% |
| Vivo#1 (FTP) TF | X | 4.97% |  |  |  |  |  |  |
|  | Y | 3.54% |  |  |  |  |  |  |
|  | Z | 6.44% |  |  |  |  |  |  |
| Vivo#2 (FB) TF | X | 6.09% | 6.08% |  | 17.8% |  |  |  |
|  | Y | 3.61% | 3.59% |  | 9.23% |  |  |  |
|  | Z | 6.28% | 6.32% |  | 12.19% |  |  |  |
| OPPO#1 (FTP/FB) TF | X | 8.45% | 8.45% |  | 30.2% |  |  |  |
|  | Y | 3.8% | 3.8% |  | 23.08% |  |  |  |
|  | Z | 3.45% | 3.45% |  | 12.99% |  |  |  |
| ETRI#1 (FTP) TF | X | 4.04% |  |  |  |  |  |  |
|  | Y | 1.9% |  |  |  |  |  |  |
|  | Z | 3.6% |  |  |  |  |  |  |
| Spreadtrum#1(FTP) TF | X | 18.7% |  |  |  |  |  |  |
|  | Y | 21.27% |  |  |  |  |  |  |
|  | Z | 19.47% |  |  |  |  |  |  |
| Fujitsu#1 (FTP/FB) TF | X | 2.6% | 7.7% |  | 11.9% |  |  |  |
|  | Y | 0.9% | 2.9% |  | 6.2% |  |  |  |
|  | Z | 2.1% | 3.6% |  | 7.8% |  |  |  |
| NTT DOCOMO#1(FTP) TF | X | 13.5% | 11.5% |  | 12.2% |  |  |  |
|  | Y | 7.9% | 1.6% |  | 1.9% |  |  |  |
|  | Z | 9.3% | 9.3% |  | 10.6% |  |  |  |
| CMCC (FTP) TF | X | 4.74% |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| E///#1 (FTP) ResNet-CNN | X |  | 3.9% |  | 10.9% |  |  |  |
|  | Y |  | 0.7% |  | 1.5% |  |  |  |
|  | Z |  | 2.2% |  | 4.4% |  |  |  |
| Xiaomi#1 (FB) TF | X |  | 9.69% |  | 9.68% |  |  |  |
|  | Y | 7.31% | 14.19% |  | 16.46% |  |  |  |
|  | Z |  | 18.12% |  | 22.96% |  |  |  |
| QC#1 (FTP/FB) TF | X |  | 11% | 17% | 26% | 25% | 101% | 177% |
|  | Y |  | 4% | 20% | 8% | 18% | 18% | 16% |
|  | Z |  |  | 11% |  | 6% | -1% | -5% |
| CATT (FTP) TF | X |  |  | 7.84% |  | 54.3% | 66.3% | 27.3% |
|  | Y |  |  | 4.32% |  | 42.2% |  |  |
|  | Z |  |  |  |  |  |  |  |
| Apple#1 (FTP) TF | X | 7% | 7% | 11% | 12% | 13% | 92% | 166% |
|  | Y | 4% | 4% | 6% | 4% | -2% | 7% | 10% |
|  | Z | 6% | 6% | 5% | 2% | -5% | -7% | -7% |
| China Telecom#1 (FB) TF | X | 7.7% 4.2% |  |  |  |  |  |  |
|  | Y | 8.1% |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| MTK#1 (FB) TF | X | 3.4% |  |  |  |  |  |  |
|  | Y | 1.7% |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| Intel#1 (FTP/FB) TF | X |  | 8% |  | 10% |  |  |  |
|  | Y |  | 12% |  | 10% |  |  |  |
|  | Z |  | 8% |  | 12% |  |  |  |
| BJTU CsiNet+ | X |  |  |  |  |  |  |  |
|  | Y | 8% |  |  |  |  |  |  |
|  | Z | 7% |  |  |  |  |  |  |
| Benchmark SGCS | X | 0.67~0.72 | 0.66~0.72 | 0.59~0.74 | 0.48~0.56 | 0.34~0.56 | 0.15~0.44 | 0.11~0.34 |
|  | Y | 0.74~0.82 | 0.76~0.82 | 0.72~0.79 | 0.61~0.7 | 0.42~0.64 | 0.5~0.51 | 0.38~0.40 |
|  | Z | 0.83~0.87 | 0.83~0.86 | 0.80~0.88 | 0.73~0.79 | 0.70~0.76 | 0.59~0.61 | 0.48~0.51 |
| Summary | X | 2.6%~ 8.8% | 3.9%~ 11% | 3.64%~ 17% | 5.92%~ 30.2% | 6.77%~ 54.3% | 32.12%~101% | 72%~ 177% |
|  | Y | 0.9%~ 8.1% | 0.7%~ 4.5% | 3.11%~ 20% | 1.5%~ 23.08% | 3.72%~ 42.2% | 8%~ 18% | 16%~ 37.16% |
|  | Z | 0.9%~ 7% | -0.2% 6.5% | 5%~ 11% | 4.4%~ 12.99% | 2%~ 6% | -1%~ 17% | -5%~ 31.94% |

Upd Proposed Observation 3.1.1: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of SGCS,

- For Max rank 1, Layer 1,

  - 11 sources [Huawei, Nokia, ZTE, vivo, OPPO, ETRI, Fujitsu, CMCC, China Telecom, MediaTek, Apple] observe the performance gain of 2.6%~ 8.8% at CSI payload X (small payload);

  - 14 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, ETRI, Fujitsu, NTT DOCOMO, Xiaomi, China Telecom, MediaTek, BJTU, Apple] observe the performance gain of 0.9%~ 8.1% at CSI payload Y (medium payload);

  - 11 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, ETRI, Fujitsu, BJTU, Apple] observe the performance gain of 0.9%~ 7% at CSI payload Z (large payload);

  - Note: 1 source [Futurewei] observes the performance gain of 11.6% at CSI payload X (small payload) which biases from the majority range.

- For Max rank 2, Layer 1,

  - 12 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 3.9%~ 11% at CSI payload X (small payload);

  - 11 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Qualcomm, Apple] observe the performance gain of 0.7%~ 4.5% at CSI payload Y (medium payload);

  - 9 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Apple] observe the performance gain of -0.2%~ 6.5% at CSI payload Z (large payload);

- For Max rank 2, Layer 2, more gains are observed in general compared with Layer 1 of Max rank 2:

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 5.92%~ 30.2% at CSI payload X (small payload);

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 1.5%~ 23.08% at CSI payload Y (medium payload);

  - 10 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Intel, Apple] observe the performance gain of 4.4%~ 12.99% at CSI payload Z (large payload);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1 of Max rank 1 or Layer 1/2 of Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Lenovo, ZTE, Qualcomm, OPPO, vivo, NTT DOCOMO, LG Electronics |
| --- | --- |
| Object/Concern | Futurewei |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML. Please provide your view on whether it is acceptable. |
| Futurewei | For the filtering rules as indicated by the FL “Principle of filtering: For some results which largely bias from the majority (either benchmark is too low/too high, or the gain bias from the majority), they are marked in grey and not captured into the observation. “. We think there should be a clear definition of what is too high/low and agreed among companies. As we explained in the email, our benchmark SGCS for max rank =1 is high because we used ideal channel estimation, but the relatively gain should be reasonable. We compared the relative gains for R1, L1 among companies, the average is 7.52% and the STD is 0.041825, our relative gain for R1, L1 is 10.2% which is within mean +/1 STD, thus we think our result for R1, L1 should be included in the observation summary. In addition, our result for using VQ (LUT-based) is not captured in the table. We understand our LUT-based approach is different than approaches most companies adopted but it’s still based on vector quantization with the assumption that either the CSI reconstruction part or a derived CSI codebook is available at UE side, thus, there should be at least a note to indicate our approach and results. |
| Moderator | @Futurewei: - As the majority companies have benchmark of the following ranges, FW has 0.83 for X, 0.92 for Y, and 0.92 for Z. These are 8%~18% bias from the majority. Note that only based on a more or less calibrated results, we may observe useful information. Note: If the reason is the ideal CE, then it is not consistent with the agreed EVM. BTW, for CSI payload Y/Z, FW results are captured, right? - For LUT, For the LUT solution, it looks the UE does no need an encoder at the inference phase? If so, it conflicts with the agreement for CSI compression Agreement For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.. |
| Apple | Updated Apple’s SGCS for rank 1 and 2, when max rank is 1 or 2. We did not simulated throughput for max rank-2. For max rank 4, the table is not complete. When max rank=4, selected rank is 2 or 3 or 4, the SGCS is e-type 2 for each layer is all different. However, for AI, if layer common and rank common model is used, i.e., the SGCS of each layer are all the same. Precentage of gain/loss over e-type 2 are all different when RI=2, 3 or 4. In the table, only RI=4 is captured. We think it is important to separate rank specific and rank common model for SGCS metrics. |
| Futurewei | We appreciate FL’s feedback. Regarding the agreement: “At least for inference, the CSI generation part is located at the UE side”, our LUT-based approach does not have any conflict with this agreement as the “CSI generation part” is still located at UE side during inference phase, just the CSI generation is utilizing the CSI codebook vs. encoder + quantization codebook. It should be left to UE/companies to decide how to perform the CSI generation at UE side. |
| ETRI | We appreciate FL for the efforts. For Max rank 1, Layer 1 and Payload size Y, the minimum observed performance gain seems 0.9% (not 0%). |
| LG Electronics | We are generally fine with this observation. One clarification question is whether we need to note the rank/layer AI/ML model setting or not. |

#### Issue#3-2 (High priority) Down selection of model input/output

Moderator note: As the intput/output type would have impact to the potential spec impact discussion, e.g., the type, format, etc. Therefore, it is Moderator’s understanding that proper downselection of input/output type is helpful to relieve our spec efforts. As a vast majority adopt eigenvectors (for subbands) as model input/output, it is then proposed to prioritize eigenvector.

Moderator note[Rd2]: Please note that the “eigenvectors” here means eigenvectors for subbands which do not include eType II-like report.

Proposal Proposed Conclusion 3.1.2: For the evaluation of CSI compression, for the type of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part), prioritize eigenvector(s).

- Note: For the evaluations of CSI compression with 1-on-1 joint training, 21 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, Spreadtrum, Fujitsu, NTT DOCOMO, Xiaomi, Qualcomm, Intel, InterDigital, CATT, Apple, China Telecom, MediaTek, BJTU, ETRI, CMCC] take eigenvector(s) without angular-delay domain convertion as the model input/output; 1 source [Ericsson] takes eigenvectors with angular-delay domain representation as the model input/output.

| Support/Can accept | NTT DOCOMO, ETRI, vivo, LG Electronics, OPPO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-3 (High priority) Mean UPT, FTP traffic

Moderator note: Summarized Mean UPT resulst for FTP are provided in the table below. “R” means “Rank”.

Principle of filtering: For some results which largely bias from the majority (either benchmark is too low/too high, or the gain bias from the majority), they are marked in grey and not captured into the observation. But for a group of results, if partial results fall into the majority range while other do not, then the partial results in the majority range are captured to the observation.

Moderator note[Rd2]:

- QC results for rank 2 added

- ZTE results updated.

- Futurewei results updated.

|  | RU | CSI overhead |  | R1 |  | R2 |  | R4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW#1 | <=39% | A |  | 2% |  | 6% |  |  |
|  |  | B |  | 1% |  | 2% |  |  |
|  |  | C |  | 1% |  | 3% |  |  |
|  | 40%-69% | A |  | 3% |  | 10% |  |  |
|  |  | B |  | 2% |  | 5% |  |  |
|  |  | C |  | 4% |  | 6% |  |  |
|  | >=70% | A |  | 9% |  | 15% |  |  |
|  |  | B |  | 5% |  | 8% |  |  |
|  |  | C |  | 6% |  | 11% |  |  |
| Nokia#1 | <=39% | A |  | 1% |  | 2% |  |  |
|  |  | B |  | 0.5% |  | 0.5% |  |  |
|  |  | C |  | 0.6% |  | -0.3% |  |  |
|  | 40%-69% | A |  | 2.5% |  | 4.3% |  |  |
|  |  | B |  | 1.6% |  | 1.2% |  |  |
|  |  | C |  | 0.1% |  | -0.5% |  |  |
|  | >=70% | A |  | 4.5% |  | 13% |  |  |
|  |  | B |  | 3.2% |  | 6.8% |  |  |
|  |  | C |  | 1.0% |  | -0.2% |  |  |
| ZTE#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  | 4.09% |  | 12.22% |
|  |  | B |  |  |  | 4.08% |  | 7.04% |
|  |  | C |  |  |  | 4.55% |  | 8.19% |
|  | >=70% | A |  | 0.38% |  | 5.30% |  | 14.89% |
|  |  | B |  | 0.62% |  | 4.92% |  | 6.64% |
|  |  | C |  | 0.23% |  | 6.94% |  | 8.40%6.88% |
| Vivo#1 | <=39% | A |  | 0.29% |  |  |  |  |
|  |  | B |  | 0.20% |  |  |  |  |
|  |  | C |  | 0.33% |  |  |  |  |
|  | 40%-69% | A |  | 1.09% |  |  |  |  |
|  |  | B |  | 0.80% |  |  |  |  |
|  |  | C |  | 1.30% |  |  |  |  |
|  | >=70% | A |  | 2.27% |  |  |  |  |
|  |  | B |  | 1.66% |  |  |  |  |
|  |  | C |  | 2.28% |  |  |  |  |
| OPPO#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 3% |  | 11% |  |  |
|  |  | B |  | 1% |  | 3% |  |  |
|  |  | C |  | 1% |  | 1% |  |  |
| E///#1 | <=39% | A |  |  |  | 1% |  |  |
|  |  | B |  |  |  | 1% |  |  |
|  |  | C |  |  |  | 0% |  |  |
|  | 40%-69% | A |  |  |  | 3% |  |  |
|  |  | B |  |  |  | 4% |  |  |
|  |  | C |  |  |  | 1% |  |  |
|  | >=70% | A |  |  |  | 5% |  |  |
|  |  | B |  |  |  | 6% |  |  |
|  |  | C |  |  |  | 2% |  |  |
| QC#1 | <=39% | A |  |  |  | 1% |  | 6% |
|  |  | B |  |  |  | 1% |  | 6% |
|  |  | C |  |  |  | 2% |  |  |
|  | 40%-69% | A |  |  |  | 4% |  | 11% |
|  |  | B |  |  |  | 4% |  | 11% |
|  |  | C |  |  |  | 1% |  | 3% |
|  | >=70% | A |  |  |  | 12% |  | 17% |
|  |  | B |  |  |  | 8% |  | 17% |
|  |  | C |  |  |  | 2% |  | 6% |
| CATT#1/#2 | <=39% | A |  |  |  |  |  | 7.4% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
| Apple#1 | <=39% | A |  |  |  |  |  | 2.5% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -4% |
|  | 40%-69% | A |  |  |  |  |  | 3% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1.8% |
|  | >=70% | A |  |  |  |  |  | 3% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1% |
| Futurewei#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 21.5% |  | 9.65% |  |  |
|  |  | B |  | 8.1% |  | 8.4% |  |  |
|  |  | C |  | 10.1% |  | 12.71% |  |  |
| Spreadtrum#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  | 12.77% |  |  |  |  |
|  |  | B |  | 15.18% |  |  |  |  |
|  |  | C |  | 21.21% |  |  |  |  |
|  | >=70% | A |  | 7.47% |  |  |  |  |
|  |  | B |  | 11.23% |  |  |  |  |
|  |  | C |  | 11.44% |  |  |  |  |
| Intel#1 | <=39% | A |  |  |  | 3% |  |  |
|  |  | B |  |  |  | 6% |  |  |
|  |  | C |  |  |  | 6% |  |  |
|  | 40%-69% | A |  |  |  | 4% |  |  |
|  |  | B |  |  |  | 9% |  |  |
|  |  | C |  |  |  | 9% |  |  |
|  | >=70% | A |  |  |  | 6% |  |  |
|  |  | B |  |  |  | 9% |  |  |
|  |  | C |  |  |  | 10% |  |  |
| NTT DOCOMO#1 | <=39% | A |  |  |  | 19.9% |  |  |
|  |  | B |  |  |  | 11.5% |  |  |
|  |  | C |  |  |  | 7% |  |  |
|  | 40%-69% | A |  |  |  | 19.7% |  |  |
|  |  | B |  |  |  | 22% |  |  |
|  |  | C |  |  |  | 16.3% |  |  |
|  | >=70% | A |  |  |  | 23% |  |  |
|  |  | B |  |  |  | 24.6% |  |  |
|  |  | C |  |  |  | 26.8% |  |  |
| InterDigital#1 | <=39% | A |  |  |  | 30% |  |  |
|  |  | B |  |  |  | 25% |  |  |
|  |  | C |  |  |  | 21% |  |  |
|  | 40%-69% | A |  |  |  | 23% |  |  |
|  |  | B |  |  |  | 21% |  |  |
|  |  | C |  |  |  | 8% |  |  |
|  | >=70% | A |  |  |  | 10% |  |  |
|  |  | B |  |  |  | 9% |  |  |
|  |  | C |  |  |  | 5% |  |  |
| Fujitsu | <=39% | A |  |  |  | 8.80% |  |  |
|  |  | B |  |  |  | 9.50% |  |  |
|  |  | C |  |  |  | 5.5% |  |  |
|  | 40%-69% | A |  |  |  | 15% |  |  |
|  |  | B |  |  |  | 20.80% |  |  |
|  |  | C |  |  |  | 10.00% |  |  |
|  | >=70% | A |  |  |  | 15.80% |  |  |
|  |  | B |  |  |  | 20.20% |  |  |
|  |  | C |  |  |  | 12% |  |  |
| Summary | <=39% | A | 0.29%~2% |  | 1%~6% |  | 2.5%~7.4% |  |
|  |  | B | 0.2%~1% |  | 0.5%~6% |  | 6% |  |
|  |  | C | 0.33%~1% |  | -0.3%~6% |  | -4% |  |
|  | 40%-69% | A | 1.09%~3% |  | 3%~10% |  | 3%~12.22% |  |
|  |  | B | 0.80%~2% |  | 1.2%~9% |  | 7.04~11% |  |
|  |  | C | 0.1%~4% |  | -0.5%~9% |  | -1.8~8.19% |  |
|  | >=70% | A | 0.38%~9% |  | 5%~15% |  | 3%~17% |  |
|  |  | B | 0.62%~5% |  | 3%~9% |  | 6.64~17% |  |
|  |  | C | 0.23%~6% |  | -0.2%~12% |  | -1%~6.88% |  |

Proposed Observation 3.1.3: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under FTP traffic, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.29%~2% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~1% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.33%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.09%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4% at CSI overhead C (large overhead);

  - For RU>=70%

    - 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.38%~9% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.62%~5% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.23%~6% at CSI overhead C (large overhead);

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%

    - 4 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm] observe the performance gain of 1%~6% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.5%~6% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, Ericsson, Intel, Fujitsu, Qualcomm] observe the performance gain of -0.3%~6% at CSI overhead C (large overhead);

  - For RU 40%-69%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 3%~10% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 1.2%~9% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of -0.5%~9% at CSI overhead C (large overhead);

  - For RU>=70%

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 5%~15% at CSI overhead A (small overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 3%~9% at CSI overhead B (medium overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Fujitsu, Qualcomm] observe the performance gain of -0.2%~12% at CSI overhead C (large overhead);

- For Max rank 4:

  - For RU<=39%

    - 3 sources [CATT, Apple, Qualcomm] observe the performance gain of 2.5%~7.4% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 6% at CSI overhead B (medium overhead);

    - 1 source [Apple] observes the performance gain of -4% at CSI overhead C (large overhead);

  - For RU 40%-69%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~12.22% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 7.04%~11% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~8.19% at CSI overhead C (large overhead);

  - For RU>=70%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.64%~17% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~8.40% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Qualcomm (comment), OPPO, vivo, LG Electronics |
| --- | --- |
| Object/Concern | Futurewei |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML. Please provide your view on whether it is acceptable. |
| Futurewei | Similar as our comments for Issue#3-1, we think there should be a clear definition of what is too high/low and agreed among companies. For max rank =2, our results are missing from the table, thus I filled in the numbers (gains), they should be included in the observation summary. In addition, our result for using VQ (LUT-based) is not captured in the table. We understand our LUT-based approach is different than approaches most companies adopted but it’s still based on vector quantization with the assumption that either the CSI reconstruction part or a derived CSI codebook is available at UE side, thus, there should be at least a note to indicate our approach and results. |
| ZTE | Thanks for FL’s summary. A minor comment for the results of R4, RU>=70%, CSI feedback overhead C, in the evaluation Table 1, the maximum performance gain for CSI overhead C in is 8.40% under 430 bits, not 6.88% for CSI overhead D. Also, the conclusion can be revised as - For RU>=70% - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~6.88% 8.40% at CSI overhead C (large overhead); |
| Qualcomm | Our results for max-rank 2 seem to have not been captured in the table. |
| LG Electronics | Same question on Issue# 3-1. |

#### Issue#3-4 (High priority) 5% UPT, FTP traffic

Moderator note: Summarized 5% UPT resulst for FTP are provided in the table below. “R” means “Rank”.

Principle of filtering: For some results which largely bias from the majority (either benchmark is too low/too high, or the gain bias from the majority), they are marked in grey and not captured into the observation. But for a group of results, if partial results fall into the majority range while other do not, then the partial results in the majority range are captured to the observation.

Moderator note[Rd2]:

- FW results for rank 2 recovered (Original observation correctly captured the company name)

- Fujitsu results for rank 2 recovered (Original observation correctly captured the company name)

|  | RU | CSI overhead |  | R1 |  | R2 |  | R4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW#1 | <=39% | A |  | 3% |  | 5% |  |  |
|  |  | B |  | 1% |  | 3% |  |  |
|  |  | C |  | 3% |  | 5% |  |  |
|  | 40%-69% | A |  | 7% |  | 13% |  |  |
|  |  | B |  | 2% |  | 4% |  |  |
|  |  | C |  | 3% |  | 8% |  |  |
|  | >=70% | A |  | 16% |  | 24% |  |  |
|  |  | B |  | 7% |  | 9% |  |  |
|  |  | C |  | 8% |  | 12% |  |  |
| Nokia#1 | <=39% | A |  | 1.9% |  | 1.1% |  |  |
|  |  | B |  | 1.2% |  | -2% |  |  |
|  |  | C |  | 1.7% |  | -0.5% |  |  |
|  | 40%-69% | A |  | 2.8% |  | 7.2% |  |  |
|  |  | B |  | 2.7% |  | 0.3% |  |  |
|  |  | C |  | 0.1% |  | -4% |  |  |
|  | >=70% | A |  | 9.5% |  | 22.5% |  |  |
|  |  | B |  | 3.6% |  | 11.4% |  |  |
|  |  | C |  | 1.7% |  | -1.3% |  |  |
| ZTE#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  | 9.72% |  | 9.25% |
|  |  | B |  |  |  | 20% |  | 6.17% |
|  |  | C |  |  |  | 25.62% |  | 9.47% |
|  | >=70% | A |  | 20.43% |  | 10.26% |  | 23.27% |
|  |  | B |  | 10.13% |  | 15.02% |  | 10.2% |
|  |  | C |  | 0.85% |  | 13.67% |  | 6.83% |
| Vivo#1 | <=39% | A |  | 1.72% |  |  |  |  |
|  |  | B |  | 0.80% |  |  |  |  |
|  |  | C |  | 1.68% |  |  |  |  |
|  | 40%-69% | A |  | 4.13% |  |  |  |  |
|  |  | B |  | 1.22% |  |  |  |  |
|  |  | C |  | 3.25% |  |  |  |  |
|  | >=70% | A |  | 5.45% |  |  |  |  |
|  |  | B |  | 1.68% |  |  |  |  |
|  |  | C |  | 4.28% |  |  |  |  |
| OPPO#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 4% |  |  |  |  |
|  |  | B |  | 1% |  |  |  |  |
|  |  | C |  | 1% |  |  |  |  |
| E///#1 | <=39% | A |  |  |  | 2% |  |  |
|  |  | B |  |  |  | 2% |  |  |
|  |  | C |  |  |  | 0% |  |  |
|  | 40%-69% | A |  |  |  | 7% |  |  |
|  |  | B |  |  |  | 5% |  |  |
|  |  | C |  |  |  | 3% |  |  |
|  | >=70% | A |  |  |  | 12% |  |  |
|  |  | B |  |  |  | 12% |  |  |
|  |  | C |  |  |  | 2% |  |  |
| QC#1 | <=39% | A |  |  |  | 5% |  | 10% |
|  |  | B |  |  |  | 3% |  | 5% |
|  |  | C |  |  |  | 4% |  | 1% |
|  | 40%-69% | A |  |  |  | 7% |  | 17% |
|  |  | B |  |  |  | 8% |  | 23% |
|  |  | C |  |  |  | -1% |  | 5% |
|  | >=70% | A |  |  |  | 37% |  | 31% |
|  |  | B |  |  |  | 10% |  | 30% |
|  |  | C |  |  |  | 40% |  | 15% |
| Apple#1 | <=39% | A |  |  |  |  |  | 8% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1.60% |
|  | 40%-69% | A |  |  |  |  |  | 5% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1.70% |
|  | >=70% | A |  |  |  |  |  | 5.80% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | 2% |
| Futurewei#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 50% |  | 0% |  |  |
|  |  | B |  | 24% |  | 7.69% |  |  |
|  |  | C |  | 23% |  | 9.87% |  |  |
| Spreadtrum#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  | 21.04% |  |  |  |  |
|  |  | B |  | 18.61% |  |  |  |  |
|  |  | C |  | 15.87% |  |  |  |  |
|  | >=70% | A |  | 34.83% |  |  |  |  |
|  |  | B |  | 27.23% |  |  |  |  |
|  |  | C |  | 20.2% |  |  |  |  |
| Intel#1 | <=39% | A |  |  |  | 8% |  |  |
|  |  | B |  |  |  | 8% |  |  |
|  |  | C |  |  |  | 9% |  |  |
|  | 40%-69% | A |  |  |  | 8% |  |  |
|  |  | B |  |  |  | 15% |  |  |
|  |  | C |  |  |  | 20% |  |  |
|  | >=70% | A |  |  |  | 3% |  |  |
|  |  | B |  |  |  | 11% |  |  |
|  |  | C |  |  |  | 10% |  |  |
| NTT DOCOMO#1 | <=39% | A |  |  |  | 23.3% |  |  |
|  |  | B |  |  |  | 17.7% |  |  |
|  |  | C |  |  |  | 19.9% |  |  |
|  | 40%-69% | A |  |  |  | 13.4% |  |  |
|  |  | B |  |  |  | 29.7% |  |  |
|  |  | C |  |  |  | 22.5% |  |  |
|  | >=70% | A |  |  |  | 21.6% |  |  |
|  |  | B |  |  |  | 23.1% |  |  |
|  |  | C |  |  |  | 35.4% |  |  |
| InterDigital#1 | <=39% | A |  |  |  | 9% |  |  |
|  |  | B |  |  |  | 7% |  |  |
|  |  | C |  |  |  | 2% |  |  |
|  | 40%-69% | A |  |  |  | 5% |  |  |
|  |  | B |  |  |  | -2% |  |  |
|  |  | C |  |  |  | -8% |  |  |
|  | >=70% | A |  |  |  | -5% |  |  |
|  |  | B |  |  |  | -10% |  |  |
|  |  | C |  |  |  | -10% |  |  |
| Fujitsu | <=39% | A |  |  |  | 24% |  |  |
|  |  | B |  |  |  | 12.6% |  |  |
|  |  | C |  |  |  | 3.80% |  |  |
|  | 40%-69% | A |  |  |  | 25.80% |  |  |
|  |  | B |  |  |  | 17.80% |  |  |
|  |  | C |  |  |  | 7.90% |  |  |
|  | >=70% | A |  |  |  | 12.40% |  |  |
|  |  | B |  |  |  | 18.60% |  |  |
|  |  | C |  |  |  | 18.10% |  |  |
| Summary | <=39% | A | 1.72%~3% |  | 1.1%~5% |  | 8%~10% |  |
|  |  | B | 0.80%~1.2% |  | -2%~3% |  | 5% |  |
|  |  | C | 1.68%~3% |  | -0.5%~5% |  | -1.6%~1% |  |
|  | 40%-69% | A | 2.8%~7% |  | 7%~13% |  | 5%~17% |  |
|  |  | B | 1.22%~2.7% |  | 0.3%~8% |  | 6.17%~23% |  |
|  |  | C | 0.1%~3.25% |  | -4%~8% |  | -1.7%~9.47 |  |
|  | >=70% | A | 4%~20.43% |  | 10.26%~24% |  | 5.8%~31% |  |
|  |  | B | 1%~10.13% |  | 9%~15.02% |  | 10.2%~30% |  |
|  |  | C | 0.85%~8% |  | -1.3%~13.67% |  | 2%~15% |  |

Proposed Observation 3.1.4: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.72%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~1.2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.68%~3% at CSI overhead C (large overhead);

  - For RU 40%-69%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 2.8%~7% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.22%~2.7% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~3.25% at CSI overhead C (large overhead);

  - For RU>=70%

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 4%~20.43% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 1%~10.13% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~8% at CSI overhead C (large overhead);

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 1.1%~5% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of -2%~3% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -0.5%~5% at CSI overhead C (large overhead);

  - For RU 40%-69%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 7%~13% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.3%~8% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu] observe the performance gain of -4%~8% at CSI overhead C (large overhead);

  - For RU>=70%

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO] observe the performance gain of 10.26%~24% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 9%~15.02% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Futurewei, Intel] observe the performance gain of -1.3%~13.67% at CSI overhead C (large overhead);

- For Max rank 4:

  - For RU<=39%

    - 3 sources [Apple, Qualcomm] observe the performance gain of 8%~10% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 5% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.17%~23% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~9.47% at CSI overhead C (large overhead);

  - For RU>=70%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5.8%~31% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 10.2%~30% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~15% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Qualcomm, OPPO, vivo |
| --- | --- |
| Object/Concern | Futurewei, Fujitsu |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML. Please provide your view on whether it is acceptable. |
| Futurewei | Similar as our comments for Issue#3-1, we think there should be a clear definition of what is too high/low and agreed among companies. Our result for max rank = 2 (7.69%) is well within mean +/- 1 STDV calculated from companies results and should be included in the observation summary. In addition, our result for using VQ (LUT-based) is not captured in the table. We understand our LUT-based approach is different than approaches most companies adopted but it’s still based on vector quantization with the assumption that either the CSI reconstruction part or a derived CSI codebook is available at UE side, thus, there should be at least a note to indicate our approach and results. |
| Fujitsu | The result highlighted (<=39%, C) is within the range in the summary, but was not captured. |

#### Issue#3-5 (Medium priority) Mean UPT, full buffer

Moderator note: Summarized Mean UPT resulst for full buffer are provided in the table below. “R” means “Rank”.

Principle of filtering: For some results which largely bias from the majority (either benchmark is too low/too high, or the gain bias from the majority), they are marked in grey and not captured into the observation. But for a group of results, if partial results fall into the majority range while other do not, then the partial results in the majority range are captured to the observation.

Moderator note[Rd2]: No changes. Please continue your inputs at second round.

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| HW#1 | A | 11% | 15% |  |
|  | B | 7% | 9% |  |
|  | C | 11% | 14% |  |
| Nokia #1 | A | 6.5% | 8.5% |  |
|  | B | 5.8% | 5% |  |
|  | C | 1.1% | 0.2% |  |
| vivo#1 | A | 7.92% | 11.15% |  |
|  | B | 6.02% | 7.09% |  |
|  | C | 9.53% | 10.28% |  |
| OPPO#1 | A | 6% |  |  |
|  | B | 3% |  |  |
|  | C | 3% |  |  |
| Fujitsu#1 | A | 9% | 9.2% |  |
|  | B | 5.6% | 7% |  |
|  | C | 2% | 13.7% |  |
| Xiaomi#1 | A |  | 24.47% |  |
|  | B |  | 28.24% |  |
|  | C |  | 27.36% |  |
| QC#1 | A |  | 8% |  |
|  | B |  | 10% |  |
|  | C |  | 3% |  |
| Intel#1 | A |  | 7% |  |
|  | B |  | 9% |  |
|  | C |  | 7% |  |
| InterDigital#1 | A |  | 4% |  |
|  | B |  | 4% |  |
|  | C |  | 0.8% |  |
| ZTE#1 | A |  |  | 8.51% |
|  | B |  |  | 7.44% |
|  | C |  |  | 9.95% |
| Summary | A | 6%~11% | 4%~15% | 8.51% |
|  | B | 3%~7% | 4%~10% | 7.44% |
|  | C | 1.1%~11% | 0.2%~14% | 9.95% |

Proposed Observation 3.1.5: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1,

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 6%~11% at CSI overhead A (small overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11% at CSI overhead C (large overhead);

- For Max rank 2,

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~15% at CSI overhead A (small overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~10% at CSI overhead B (medium overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~14% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Qualcomm, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML. Please provide your view on whether it is acceptable. |

#### Issue#3-6 (Medium priority) 5% UPT, full buffer

Moderator note: Summarized 5% UPT resulst for full buffer are provided in the table below. “R” means “Rank”.

Principle of filtering: For some results which largely bias from the majority (either benchmark is too low/too high, or the gain bias from the majority), they are marked in grey and not captured into the observation. But for a group of results, if partial results fall into the majority range while other do not, then the partial results in the majority range are captured to the observation.

Moderator note[Rd2]: No changes. Please continue your inputs at second round.

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| Nokia #1 | A | 2.5% | 4.1% |  |
|  | B | 2.3% | 0.3% |  |
|  | C | 0% | 2.11% |  |
| vivo#1 | A | 8.81% | 13.8% |  |
|  | B | 8.66% | 3.05% |  |
|  | C | 6.62% | 6.03% |  |
| Fujitsu#1 | A | 20.9% | 14.90% |  |
|  | B | 17.4% | 3.50% |  |
|  | C | 6.1% | -7% |  |
| Xiaomi#1 | A |  | 30.17% |  |
|  | B |  | 21% |  |
|  | C |  | 8.76% |  |
| QC#1 | A |  | 9% |  |
|  | B |  | 3% |  |
|  | C |  | 0 |  |
| Intel#1 | A |  | 5% |  |
|  | B |  | 4% |  |
|  | C |  | 3% |  |
| ZTE#1 | A |  |  | 5.36% |
|  | B |  |  | 4.72% |
|  | C |  |  | 3.59% |
| Summary | A | 2.5%~20.9% | 3%~14.9% | 5.36% |
|  | B | 2.3%~17.4% | 0.3%~7% | 4.72% |
|  | C | 0%~6.62% | -7%~6.03% | 3.59% |

Proposed Observation 3.1.6: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer,

- For Max rank 1,

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.5%~20.9% at CSI overhead A (small overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.3%~17.4% at CSI overhead B (medium overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~6.62% at CSI overhead C (large overhead);

- For Max rank 2,

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 3%~14.9% at CSI overhead A (small overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 0.3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~6.03% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Qualcomm, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-7 (Low priority) CSI overhead reduction

Moderator note: Summarized Mean UPT resulst for full buffer are provided in the table below. “R” means “Rank”.

Moderator note[Rd2]: QC results updated; observation updated accordingly.

|  | RU | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- | --- |
| HW#1 (FTP) | <=39% | A |  |  |  |
|  |  | B | 38% |  |  |
|  |  | C | 47% |  |  |
|  | 40%-69% | A |  |  |  |
|  |  | B | 39% |  |  |
|  |  | C | 55% |  |  |
|  | >=70% | A |  |  |  |
|  |  | B | 50% |  |  |
|  |  | C | 47% |  |  |
| ZTE#1 (FTP) | <=39% | A |  |  |  |
|  |  | B |  |  |  |
|  |  | C |  |  |  |
|  | 40%-69% | A |  | 48.60% | 56.01% |
|  |  | B |  | 39.80% | 36.10% |
|  |  | C |  | 38.66% | 38.14% |
|  | >=70% | A | 10.24% | 44.44% | 60.29% |
|  |  | B | 15.62% | 38.12% | 36.41% |
|  |  | C | 14.37% | 36.47% | 37.93% |
| QC#1 (FTP) Option 2-1 | <=39% | A |  | 52% | 67% |
|  |  | B |  | 46% | 70% |
|  |  | C |  | 29% | 8% |
|  | 40%-69% | A |  | 51% | 57% |
|  |  | B |  | 49% | 61% |
|  |  | C |  | 10% | 12% |
|  | >=70% | A |  | 54% | 56% |
|  |  | B |  | 52% | 62% |
|  |  | C |  | 12% | 21% |
| QC#2 (FTP) Option 3-1 | <=39% | A |  |  | 50% |
|  |  | B |  |  | 61% |
|  |  | C |  |  | 13% |
|  | 40%-69% | A |  |  | 65% |
|  |  | B |  |  | 67% |
|  |  | C |  |  | 41% |
|  | >=70% | A |  |  | 79% |
|  |  | B |  |  | 78% |
|  |  | C |  |  | 58% |
| Futurewei#1 (FTP) | <=39% | A |  |  |  |
|  |  | B |  |  |  |
|  |  | C |  |  |  |
|  | 40%-69% | A |  |  |  |
|  |  | B |  |  |  |
|  |  | C |  |  |  |
|  | >=70% | A |  | 20.83% |  |
|  |  | B |  | 22.22% |  |
|  |  | C |  | 58.33% |  |
| HW#2 (FB) |  | A |  |  |  |
|  |  | B | 37% | 52% |  |
|  |  | C | 50% | 54% |  |
| QC#2 (FB) |  | A |  | 56% |  |
|  |  | B |  | 52% |  |
|  |  | C |  | 22% |  |
| ZTE#2 (FB) |  | A |  |  | 70.53% |
|  |  | B |  |  | 47.74% |
|  |  | C |  |  | 42.59% |
| Sumamry- FTP | <=39% | A |  | 52% | 50%~67% |
|  |  | B | 38% | 46% | 61%~70% |
|  |  | C | 47% | 29% | 8%~13% |
|  | 40%-69% | A |  | 20.83%~54% | 56.01%~65% |
|  |  | B | 39% | 22.22%~52% | 36.10%~67% |
|  |  | C | 55% | 10%~58.33% | 12%~41% |
|  | >=70% | A | 10.24% | 44.44% | 56%~79% |
|  |  | B | 15.62%~50% | 38.12% | 36.41%~78% |
|  |  | C | 14.37%~47% | 36.47% | 21%~58% |
| Summary-FTP |  | A | 10.24% | 20.83%~54% | 50%~79% |
|  |  | B | 15.62%~50% | 22.22%~52% | 36.10%~78% |
|  |  | C | 14.37%~55% | 10%~58.33% | 8%~58% |
| Summary-FB |  | A |  | 56% | 70.53% |
|  |  | B | 37% | 52% | 47.74% |
|  |  | C | 50% | 22%~54% | 42.59% |

Upd Proposed Observation 3.1.7: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of CSI feedback reduction,

- For Max rank = 1,

  - For CSI overhead A (small overhead), 1 source [ZTE] observes the CSI feedback reduction of 10.24% for FTP traffic;

  - For CSI overhead B (medium overhead), 2 sources [Huawei, ZTE] observe the CSI feedback reduction of 15.62%~50%, and 1 source [Huawei] observes the CSI feedback reduction of 37% for full buffer;

  - For CSI overhead C (large overhead), 2 sources [Huawei, ZTE] observe the CSI feedback reduction of 14.37%~55%, and 1 source [Huawei] observes the CSI feedback reduction of 50% for full buffer;

- For Max rank = 2,

  - For CSI overhead A (small overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 20.83%~54% for FTP traffic, and 1 source [Qualcomm] observes the CSI feedback reduction of 56% for full buffer;

  - For CSI overhead B (medium overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 22.22%~52%, and 2 sources [Huawei, Qualcomm] observe the CSI feedback reduction of 52% for full buffer;

  - For CSI overhead C (large overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 10%~58.33%, and 2 sources [Huawei, Qualcomm] observe the CSI feedback reduction of 22%~54% for full buffer;

- For Max rank = 4,

  - For CSI overhead A (small overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 50%~79% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 70.53% for full buffer;

  - For CSI overhead B (medium overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 36.10%~78%, and 1 source [ZTE] observes the CSI feedback reduction of 47.74% for full buffer;

  - For CSI overhead C (large overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 8%~58%, and 1 source [ZTE] observes the CSI feedback reduction of 42.59% for full buffer;

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Qualcomm (comment) |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Qualcomm | Some of the numbers in QC#1 for rank 2 case seem to be missing. |

#### Issue#3-8 (High priority) Quantization method-training awareness cases

Moderator note: Summarized resulst for quantization methods, including quantization non-aware/aware (Case 1/2-1/2-2) for SQ and VQ, are provided in the table below.

Principle of filtering: For some results which largely bias from the majority, they are marked in grey and not captured into the observation. Only the results with different training awareness cases are captured; if all results are subject to the same training awareness case, they are not captured. In addition, only Layer 1 results are captured.

Moderator note[Rd2]: A note is added to address QC concern.

|  |  |  | Case 1 | Case 2-1 | Case 2-2 |
| --- | --- | --- | --- | --- | --- |
| QC#1~QC#7 | SQ | X |  |  |  |
|  |  | Y | 0.648 (-11.56%) | config#1: 0.796 (8.64%) 22.8% gain over Case 1 config#2: 0.738 (0.72%) 13.9% gain over Case 1 | 0.788 (7.55%) 21.6% gain over Case 1 |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  | 0.798 (8.91%) 23.1% gain over Case 1 | VQ-dim2: 0.823 (12.32%) 27% gain over Case 1 3.1% gain over Case 2-1 VQ-dim4: 0.828 (13.01%) 27.8% gain over Case 1 3.8% gain over Case 2-1 |
|  |  | Z |  |  |  |
| HW#1/#5 | SQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X | 0.69 (-2%) |  | 0.767 (8.8%) 11.1% gain over Case 1 |
|  |  | Y | 0.738 (-10%) |  | 0.861 (4.5%) 16.7% gain over Case 1 |
|  |  | Z | 0.829 (-4%) |  | 0.918 (6.5%) 10.7% gain over Case 1 |
| vivo#1~#5 | SQ | X |  |  |  |
|  |  | Y | 0.735 (-5.9%) | 0.862 (6.7%) 17.3% gain over Case 1 |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  | 0.855 (6.0%) 16.3% gain over Case 1 | VQ-config#1: 0.871 (7.6%) 18.5% gain over Case 1 1.9% gain over Case 2-1 VQ-config#2: 0.866 (7.1%) 17.8% gain over Case 1 1.3% gain over Case 2-1 |
|  |  | Z |  |  |  |
| ZTE#1/#5 | SQ | X |  | L1: 0.7283(6.01%) L2: 0.5935(7.17%) |  |
|  |  | Y |  | L1: 0.8118(5.37%) L2: 0.6994(7.14%) |  |
|  |  | Z |  | L1: 0.9040(5.47%) L2: 0.8380(8.42%) |  |
|  | VQ | X |  |  | L1: 0.7191(4.67%) L2: 0.5866(5.92%) |
|  |  | Y |  |  | L1: 0.8015(4.04%) L2: 0.6913(5.90%) |
|  |  | Z |  |  | L1: 0.9123(6.44%) L2: 0.8498(9.95%) |
| Xiaomi#1/#2 | SQ | X |  |  |  |
|  |  | Y |  | 0.7533(7.31%) |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  |  | 0.7591(8.13%) |
|  |  | Z |  |  |  |
| E///#1/#2 | SQ | X | 0.410 (-43.2%) | 0.751(3.9%) 83.2% gain over Case 1 |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
| Nokia | SQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  | 1.2% (52 bits) |
|  |  | Y |  |  | 2.9% (104 bits) |
|  |  | Z |  |  | 1.5% (256 bits) |
| Fujitsu | SQ | X | 0.729(5.85%) | 0.7684(11.57%) 5.4% gain over Case 1 |  |
|  |  | Y | 0.8135(4.2%) | 0.8431(7.99%) 3.6% gain over Case 1 |  |
|  |  | Z | 0.9258(4.84%) | 0.9343(5.8%) 0.9% gain over Case 1 |  |
|  | VQ | X |  |  | 0.784(13.83%) 7.5% gain over Case 1 2% gain over Case 2-1 |
|  |  | Y |  |  | 0.8713(11.6%) 7.1% gain over Case 1 3.3% gain over Case 2-1 |
|  |  | Z |  |  | 0.9411(6.57%) 1.7% gain over Case 1 0.7% gain over Case 2-1 |
| OPPO | SQ | X | Xxx | Xxx y% gain over Case 1 | Xxx z% gain over Case 1; w% gain over Case 2-1 |
|  |  | Y | Xxx | Xxx y% gain over Case 1 | Xxx z% gain over Case 1; w% gain over Case 2-1 |
|  |  | Z | Xxx | Xxx y% gain over Case 1 | Xxx z% gain over Case 1; w% gain over Case 2-1 |
|  | VQ | X | Xxx | Xxx y% gain over Case 1 | Xxx z% gain over Case 1; w% gain over Case 2-1 |
|  |  | Y | Xxx | Xxx y% gain over Case 1 | Xxx z% gain over Case 1; w% gain over Case 2-1 |
|  |  | Z | Xxx | Xxx y% gain over Case 1 | Xxx z% gain over Case 1; w% gain over Case 2-1 |
| Apple | SQ | X | 0.513 (-26%) | 0.722(4.49%) 26% gain over Case 1 | N/A |
|  |  | Y | Xxx | N/A | N/A |
|  |  | Z | Xxx | N/A | N/A |
|  | VQ | X | 0.709 (2.6%) (VQ codebook is searched separately using traditional methods, based on latent space distribution, after encoder/decoder is trained without quantization) | 0.73(5.64%) 3% gain over Case 1 | 0.735 (6.37%) 3.66% gain over Case 1; 0.7% gain over Case 2-1 |
|  |  | Y | Xxx | N/A | N/A |
|  |  | Z | Xxx | N/A | N/A |
| Summary; gain over benchmark, gain over Case 1 | SQ | X | -43.2%; 5.85% | 3.9%~11.57%, 83.2%; 5.4% |  |
|  |  | Y | -5.9%~-11.56%; 4.2% | 5.37%~8.64%, 17.3%~22.8%; 3.6% | 7.55%, 21.6% |
|  |  | Z | 4.84% | 5.47%~5.8%, 0.9% |  |
|  | VQ | X | -2% | 6.01% | 4.67%~8.8% 7.5% |
|  |  | Y | -10% | 6.0%~8.91%, 16.3%~23.1% | 4.04%~13.01%, 7.1%; 16.7%~27.8% |
|  |  | Z | -4% | N/A | 6.44%~6.5%, 1.7%; 10.7% |
| Summary; gain over benchmark, gain over Case 1 | SQ | X/Y/Z | -5.9%~-43.2% (QC, vivo, E///) 4.2%~5.85% (Fujitsu) | 3.9%~8.64% (QC, vivo, ZTE, Xiaomi, E///), 17.3%~83.2% (QC, vivo, E///) 0.9%~5.4% (Fujitsu) | 7.55%(QC), 21.6%(QC) |
|  | VQ | X/Y/Z | -2%~-10% (HW) | 6.0%~8.91% (QC, vivo), 16.3%~23.1% (QC, vivo) | 4.67%~13.01% (QC, vivo, HW, ZTE, Xiaomi), 10.7%~27.8% (QC, vivo, HW), 1.7%~7.5% (Fujitsu), 0.7%~3.8% gain over Case 2-1(QC, vivo, Fujitsu) |

Upd Proposed Observation 3.1.8: For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, training non-aware quantization (Case 1) is in general inferior to the training aware quantization (Case 2-1/2-2), and may lead to lower performance than the benchmark.

- For scalar quantization, compared with benchmark,

  - -5.9%~-43.2% degradations are observed for training non-aware quantization (Case 1) from 3 sources [Qualcomm, vivo, Ericsson].

  - 3.9%~8.64% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 5 sources [Qualcomm, vivo, ZTE, Xiaomi, Ericsson], which are 17.3%~83.2% gains over training non-aware quantization (Case 1) from 3 sources [Qualcomm, vivo, Ericsson] and 0.9%~5.4% gains over training non-aware quantization (Case 1) from 1 source [Fujitsu].

    - Note: 0.72% gains are observed for Case 2-1 from 1 source [Qualcomm] due to inappropriate SQ parameter chosen, which achieves 13.9% gains over Case 1.

  - 7.55% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 1 source [Qualcomm], which are 21.6% gains over training non-aware quantization (Case 1) from 1 source [Qualcomm].

- For vector quantization, compared with benchmark,

  - -2%~-10% degradations are observed for training non-aware quantization (Case 1) from 1 source [Huawei].

  - 6.0%~8.91% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 2 sources [Qualcomm, vivo], which are 16.3%~23.1% gains over training non-aware quantization (Case 1) from 2 sources [Qualcomm, vivo].

  - 4.67%~13.01% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 5 sources [Qualcomm, vivo, Huawei, ZTE, Xiaomi], which are 10.7%~27.8% gains over training non-aware quantization (Case 1) from 3 sources [Qualcomm, vivo, Huawei] and 1.7%~7.5% gains over training non-aware quantization (Case 1) from 1 source [Fujitsu].

  - It is observed by 3 sources [Qualcomm, vivo, Fujitsu] that Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | ZTE, Qualcomm (comment), vivo, LG Electronics |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| Apple | Apple’s results are missing. Table V: Summary of quantization evaluation Method; SGCS. e-type II configuration 1 baseline; 0.691. Float point, no quantization (30 float value); 0.773. Uniform scaler quantization non-aware (60 bits); 0.513. Fixed VQ codebook quantization non-aware (60bits); 0.709. Uniform scaler quantization aware (60bits); 0.722. Case 2-1: Fixed VQ codebook quantization aware (60bits); 0.730. Case 2-2: Joint VQ and AI model; 0.735. |
| ZTE | We are fine to only summarize the results for 1st layer, and 2nd layer results can be further referred to. |
| Qualcomm | Regarding our SQ Case 2-1 config #2 result which has been marked as lower than majority, the main reason is that the quantization parameters are not chosen based on the statistics of the data. In our view, this result should be captured with appropriate description. For Case 2-1, if the parameters of the fixed scheme are carefully chosen based on the statistical distribution of the latent vector, the performance is expected to be better. Companies should clarify which approach was followed. |
| OPPO | OPPO’s results are provided in our Tdoc: Table 1 Comparison on different quantization schemes Quantization; 2bit SQ; 3bit SQ; VQ method 1; VQ method 2; 2bit/3bit mixed SQ. SGCS; 0.846; 0.852; 0.869; 0.853; 0.878. Here the UMa#595k and UMa#5k are used for training and testing, respectively with 174bit CSI feedback payload. Two kinds of VQ methods with different hyper-parameters are given as VQ method 1 and VQ method 2, respectively. The 2bit/3bit mixed SQ is a kind of non-uniform quantization SQ, where the former 90bits is 3bit quantized from 30 float numbers and the later 84bits is 2bit quantized from 42 float numbers. Firstly, the 2bit/3bit mixed SQ achieves highest SGCS performance with about 1% gain than VQ method 1 and 3% gain than 3bit SQ. The SGCS for both uniform SQ with 2bit and 3bit are inferior than VQ and 2bit/3bit mixed SQ. Actually, VQ with well-chosen hyper-parameters is potential of achieving higher SGCS performance than 2bit/3bit mixed SQ. However, the optimization of hyper-parameters for VQ training with different CSI feedback payloads is very time-consuming, and the alignment between UE and NW side in Type 3 training is also difficult. As comparison, the alignment between UE and NW side for non-uniform SQ (e.g. 2bit/3bit mixed SQ) is easy to realize. Therefore, we prefer to use non-uniform SQ (e.g. 2bit/3bit mixed SQ) during quantization-aware training. |
| Moderator | @Apple @OPPO Your results are supposed to be submitted to the templates before the meeting (not during the meeting)!!! Please fill the entries in the table above by yourselves before Tue 23:59AM (Incheon), otherwise your results may not be captured to the observation in 113. |

#### Issue#3-9 (High priority) Quantization method-quantization format

Moderator note: Summarized resulst for quantization methods, including SQ and VQ, are provided in the table below.

Principle of filtering: For some results which largely bias from the majority, they are marked in grey and not captured into the observation. In addition, only Layer 1 results are captured.

Moderator note[Rd2]: A note is added to address QC concern.

|  |  |  | Case 2-1 | Case 2-2 |
| --- | --- | --- | --- | --- |
| QC#1~QC#7 | SQ | X |  |  |
|  |  | Y | config#1: 0.796 (8.64%) config#2: 0.738 (0.72%) 13.9% gain over Case 1 | 0.788 (7.55%) |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y | 0.798 (8.91%) 0.3% gain over Case 2-1-SQ | VQ-dim2: 0.823 (12.32%) 3.4% gain over Case 2-1-SQ 4.4% gain over Case 2-2-SQ VQ-dim4: 0.828 (13.01%) 4% gain over Case 2-1-SQ 5.1% gain over Case 2-2-SQ |
|  |  | Z |  |  |
| vivo#1~#5 | SQ | X |  |  |
|  |  | Y | 0.862 (6.7%) |  |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y | 0.855 (6.0%) -0.8% gain over Case 2-1-SQ | VQ-config#1: 0.871 (7.6%) 1% gain over Case 2-1-SQ VQ-config#2: 0.866 (7.1%) 0.5% gain over Case 2-1-SQ |
|  |  | Z |  |  |
| ZTE#1/#5 | SQ | X | L1: 0.7283(6.01%) L2: 0.5935(7.17%) |  |
|  |  | Y | L1: 0.8118(5.37%) L2: 0.6994(7.14%) |  |
|  |  | Z | L1: 0.9040(5.47%) L2: 0.8380(8.42%) |  |
|  | VQ | X |  | L1: 0.7191(4.67%) -1.3% gain over Case 2-1-SQ L2: 0.5866(5.92%) |
|  |  | Y |  | L1: 0.8015(4.04%) -1.3% gain over Case 2-1-SQ L2: 0.6913(5.90%) |
|  |  | Z |  | L1: 0.9123(6.44%) 1% gain over Case 2-1-SQ L2: 0.8498(9.95%) |
| Xiaomi#1/#2 | SQ | X |  |  |
|  |  | Y | 0.7533(7.31%) |  |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y |  | 0.7591(8.13%) 0.8% gain over Case 2-1-SQ |
|  |  | Z |  |  |
| Nokia | SQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  |  |
|  | VQ | X |  | 1.2% (52 bits) |
|  |  | Y |  | 2.9% (104 bits) |
|  |  | Z |  | 1.5% (256 bits) |
| Lenovo#1/#2 | SQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  | 0.844 |
|  | VQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  | 0.882 0.8% gain over Case 2-2-SQ |
| Apple | SQ | X | 0.722 (2.6%) | xxx |
|  |  | Y | Xxx | xxx |
|  |  | Z | Xxx | xxx |
|  | VQ | X | 0.73 (5.64%) 1.1% gain over Case 2-1-SQ | 0.735 (6.37%) 1.8% gain over Case 2-1-SQ 0.7% gain over Case 2-2-SQ |
|  |  | Y | xxx y% gain over Case 2-1-SQ | xxx y% gain over Case 2-1-SQ z% gain over Case 2-2-SQ |
|  |  | Z | xxx y% gain over Case 2-1-SQ | xxx y% gain over Case 2-1-SQ z% gain over Case 2-2-SQ |
| Summary | X/Y/Z |  | Case 2-1-VQ has -0.8% loss over Case 2-1-SQ [vivo], 0.3% gain over Case 2-1-SQ [QC] | Case 2-2-VQ has 0.5% ~5.1% gain over Case 2-1-SQ [QC, vivo, ZTE, Xiaomi] Case 2-2-VQ has 0.8% ~5.1% gain over Case 2-2-SQ [Lenovo] Case 2-2-VQ has -1.3% loss over Case 2-1-SQ [ZTE] |

Upd Proposed Observation 3.1.9:  For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, in general vector quantization (VQ) has comparable performance with scalar quantization (SQ):

- For SQ and VQ under the same training case, it is observed by 1 source [Qualcomm] that VQ under Case 2-1 has -0.8% degradation over SQ under Case 2-1, observed by 1 source [vivo] that VQ under Case 2-1 has 0.3% gain over SQ under Case 2-1, and observed by 1 source [Lenovo] that that VQ under Case 2-2 has 0.8% gain over SQ under Case 2-2.

  - Note: VQ under Case 2-1 has 8% gains over SQ under Case 2-1 as observed from 1 source [Qualcomm] due to inappropriate SQ parameter chosen.

- For SQ and VQ across training cases, it is observed by 4 sources [Qualcomm, vivo, ZTE, Xiaomi] that VQ under Case 2-2 has 0.5% ~5.1% gain over SQ under Case 2-1, and observed by 1 source [ZTE] that VQ under Case 2-2 has -1.3% degradation over SQ under Case 2-1.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | ZTE, vivo |
| --- | --- |
| Object/Concern | Qualcomm (comment) |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| Lenovo | As we also have some simulation results on using both VQ and SQ, we would like to capture that observation as well. For combined SQ and VQ, it is observed by 1 sources [Lenovo] that Case2-2 with VQ+SQ has ~0.2% gain with respect to case2-2 with only VQ |
| Apple | Apple’s results are missing. Table V: Summary of quantization evaluation Method; SGCS. e-type II configuration 1 baseline; 0.691. Float point, no quantization (30 float value); 0.773. Uniform scaler quantization non-aware (60 bits); 0.513. Fixed VQ codebook quantization non-aware (60bits); 0.709. Uniform scaler quantization aware (60bits); 0.722. Case 2-1: Fixed VQ codebook quantization aware (60bits); 0.730. Case 2-2: Joint VQ and AI model; 0.735. |
| Qualcomm | For Case 2-1, there are two possibilities – the fixed quantization parameters are chosen matched to the statistics of the latent samples, or they are chosen independently of the data. For SQ, we presented results for both the cases. For VQ, we presented results only for the case where the parameters are chosen independent of the data statistics. The VQ result should therefore be compared with the corresponding SQ result, which would result in a larger gain than 0.3%. For Case 2-2, we have shown gains with VQ over SQ if the quantization is jointly trained with the models. Based on this, we do not support the conclusion that VQ has comparable performance as SQ at this point. |
| Moderator | @Apple Your results are supposed to be submitted to the templates before the meeting (not during the meeting)!!! Please fill the entries in the table above by yourselves before Tue 23:59AM (Incheon), otherwise your results may not be captured to the observation in 113. @Lenovo: SQ+VQ is not in the examples of any agreements. How is it achieved? Maybe it is subject to VQ? |

#### Issue#3-10 (High priority) High resolution ground-truth CSI for training

Moderator note: Summarized resulst for high resolution ground-truth CSI format for training, are provided in the table below.

Principle of filtering: For some results which biases from the intuition (e.g., positive gain over Float32), they are marked in grey and not captured into the observation.

Moderator note[Rd2]: A note is added as per the comments from QC.

|  |  | QC#8/#9 | HW#1~#4 | vivo#4~#7 | ZTE#1/3~5 | Apple |
| --- | --- | --- | --- | --- | --- | --- |
| Float32 | X | 0.735 (13.6%) | 0.767 (8.8%) |  | L1: 0.7191(4.67%) L2: 0.5866(5.92%) | 0.726(5.19%) |
|  | Y |  | 0.861 (4.5%) | 0.871 (7.6%) |  |  |
|  | Z |  | 0.918 (6.5%) |  | L1: 0.9123(6.44%) L2: 0.8498(9.95%) |  |
| Scalar quantz: 8 bits (75% OH reduction) | X |  | 0.762 (8.1%) -0.5% loss |  |  | 0.725(4.9%) |
|  | Y |  | 0.852 (3.4%) -0.9% loss |  |  |  |
|  | Z |  | 0.911 (5.7%) -0.7% loss |  |  |  |
| Float16 (50% OH reduction) | X |  |  |  |  | 0.727(5.29%) |
|  | Y |  |  | 0.866 (7.1%) -0.6% loss |  |  |
|  | Z |  |  |  |  |  |
| R16 Type II-like CB: Legacy resolution (PC5) (99.1% OH reduction) | X |  |  |  |  |  |
|  | Y |  |  | 0.819 (2.4%) -6% loss |  |  |
|  | Z |  |  |  |  |  |
| R16 Type II-like CB: Legacy resolution (PC8) (98.8% OH reduction) | X | 0.726 (12.2%) -1.2% loss | 0.76 (7.8%) -0.7% loss |  | L1: 0.6913(0.63%) -3.9% loss L2: 0.5579(0.74%) -4.9% loss |  |
|  | Y |  | 0.832 (1%) -2.9% loss |  |  |  |
|  | Z |  | 0.887 (2.9%) -3.1% loss |  | L1: 0.8691(1.40%) -4.7% loss L2: 0.7933(2.64%) -6.6% loss |  |
| ParaSet#1: R16 Type II-like CB: High resolution (>legacy PC8) L=10, p=0.9, beta=0.31; Ref-amp: 6 bits, Diff-amp: 4 bits, phase: 6 bits (96% OH reduction) | X |  | 0.768 (8.9%) +0% 1% gain on PC8 |  |  |  |
|  | Y |  | 0.854 (3.6%) -0.7% loss; 2.6% gain on PC8 |  |  |  |
|  | Z |  | 0.917 (6.4%) -0.1% loss; 3.4% gain over PC8 |  |  |  |
| ParaSet#2: R16 Type II-like CB: High resolution (>legacy PC8) L=12, M=6, beta=1.0 (91.2% OH reduction) | X |  |  |  |  |  |
|  | Y |  |  | 0.854 (5.9%) -2% loss; 4.3% gain over PC5 |  |  |
|  | Z |  |  |  |  |  |
| ParaSet#3: R16 Type II-like CB: High resolution (>legacy PC8) L=8, p_v=0.75, β=0.31 (97.5% OH reduction) | X |  |  |  | L1: 0.7015(2.11%) -2.4% loss; 1.5% gain over PC8 L2: 0.5834(5.34%) -0.5% loss; 4.6% gain over PC8 |  |
|  | Y |  |  |  |  |  |
|  | Z |  |  |  | L1: 0.9063(5.74%) -0.7% loss; 4.3% gain over PC8 L2: 0.8440(9.20%) -0.7% loss; 6.4% gain over PC8 |  |
| ParaSet#4: R16 Type II-like CB: High resolution (>legacy PC8) L=10, p_v=0.9, β=0.31 (96.2% OH reduction) | X |  |  |  | L1: 0.7147(4.03%) -0.6% loss; 3.4% gain over PC8 L2: 0.5865(5.90%) -0% loss; 5.1% gain over PC8 |  |
|  | Y |  |  |  |  |  |
|  | Z |  |  |  | L1: 0.9163(6.91%) +0.4% gain L2: 0.8586(10.19%) 1% gain |  |

Proposed Observation 3.1.10:  For the evaluation of high resolution quantization of the ground-truth CSI for CSI compression, till the RAN1#113 meeting, compared to the upper-bound of Float32, quantized high resolution ground-truth CSI can achieve significant overhead reduction with minor performance loss if the parameters are appropriately selected.

- For high resolution scalar quantization,

  - Float16 achieves 50% overhead reduction and -0.6% performance loss from 1 source [vivo]

  - 8 bits scalar quantization achieves 75% overhead reduction and -0.5%~-0.9% performance loss from 1 source [Huawei]

- For high resolution R16 eType II-like quantization, R16 eType II CB with new parameters can outperform that with legacy parameters (e.g., PC1~PC8) with slight increase of overhead:

  - R16 eType II-like quantization with legacy PC5 achieves 99.1% overhead reduction, and -6% loss at CSI payload Y (medium payload) from 1 source [vivo].

  - R16 eType II-like quantization with new parameter assumptions [set#2] which achieves 91.2% overhead reduction, has -2% loss to Float32 and 4.3% gain over legacy PC5 at CSI payload Y (medium payload) from 1 source [vivo]

  - R16 eType II-like quantization with legacy PC8 achieves 98.8% overhead reduction, and

    - -4.9%~-0.7% loss at CSI payload X (small payload) from 3 sources [Qualcomm, Huawei, ZTE],

    - -2.9% loss at CSI payload Y (medium payload) from 1 source [Huawei],

    - -6.6%~-3.1% loss at CSI payload Z (large payload) from 2 sources [Huawei, ZTE].

    - Note: it is pointed out by 1 source [Qualcomm] that using R16 eType II-like quantization with legacy PC may achieve close performance to Float32 by special data handling.

  - R16 eType II-like quantization with new parameter assumptions [set#1/3/4] which achieve 96%~97.5% overhead reduction, have

    - -2.4%~0% loss to Float32 and 1%~1.5% gain over legacy PC8, at CSI payload X (small payload) from 2 sources [Huawei, ZTE]

    - -0.7% loss to Float32 and 2.6% gain over legacy PC8, at CSI payload Y (medium payload) from 1 source [Huawei]

    - -0.7%~0% loss to Float32 and 3.4%~6.4% gain over legacy PC8, at CSI payload Z (large payload) from 2 sources [Huawei, ZTE]

- Note: the new parameters include at least one from the follows:

  - L= 8, 10, 12;

  - pv = 0.75, 0.9;

  - beta = 1;

  - reference amplitude = 6 bits; differential amplitude = 4bits; phase = 6 bits;

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1/2.

| Support/Can accept | ZTE, vivo |
| --- | --- |
| Object/Concern | Qualcomm (comment) |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| Apple | Our results are missing. Table VI: SGCS versus quantization of target CSI with 60bit UCI per layer 32 bits; 16bits; 8bits; 4 bits. U1; 0.726563; 0.727325; 0.724561; 0.697744. U2; 0.550424; 0.554369; 0.548524; 0.496096. U3; 0.424542; 0.428177; 0.423602; 0.402386. U4; 0.32442; 0.331857; 0.330396; 0.31255. |
| Qualcomm | We do not support the second observation that eType II CB can outperform legacy PC. It gives an impression that there is a gain in user performance metrics from using eType II CB with new parameters. This statement is premature to conclude considering that the results are from very few sources. Moreover, even there the gains observed are small except in the Z payload range, where the SGCS is quite high already. Our results show that even the gap of PC8 to floating point resolution is not very large, and hence the gap to eType II CB with new parameters will also be small. |

#### Issue#3-11 (Low priority) Rank>1 solutions

Moderator note: Summarized resulst for rank>1 options, including Option 2-1 and Option 3-1, are provided in the table below. Note that only two companies submitted results to compare different rank>1 options to the template. If you have additional results, please put your results to the company views column.

Moderator note[Rd2]: No change on the issue. Please provide more inputs!

|  |  | Option 2-1 |  | Option 3-1 |  |
| --- | --- | --- | --- | --- | --- |
|  | CSI payload | R2 | R4 | R2 | R4 |
| Intel#1/#2 | X | L1: 0.7103 (12%) L2: 0.5290 (20%) |  | L1: 0.6990 (10%) L2: 0.5413 (22%) |  |
|  | Y | L1: 0.8431 (16%) L2: 0.7143 (22%) |  | L1: 0.8378 (15%) L2: 0.7206 (23%) |  |
|  | Z | L1: 0.9166 (10%) L2: 0.8415 (16%) |  | L1: 0.9152 (10%) L2: 0.8377 (15%) |  |
| QC#8 | X |  | L1: 15% L2: 25% L3: 102% L4: 179% |  | L1: 17% L2: 25% L3: 101% L4: 177% |
|  | Y |  | L1: 19% L2: 19% L3: 17% L4: 17% |  | L1: 20% L2: 18% L3: 18% L4: 16% |
|  | Z |  | L1: 10% L2: 5% L3: -2% L4: -4% |  | L1: 11% L2: 6% L3: -1% L4: -5% |

Proposed Observation 3.1.11: For the evaluation of CSI compression, for the rank>1 solutions, Option 3-1 has comparable performance with Option 2-1 from 2 sources [Qualcomm, Intel], where 0~+/-2% gap is observed for layer 1/2 for Max rank 2 or layer 1/2/3/4 for Max rank 4 in terms of gain% over benchmark of Rel-16 eType II codebook.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS.

  - Benchmark is Rel-16 Type II codebook.

| Support/Can accept | Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-11a (High priority) Rank>1 solutions-down selection

Moderator note: As the rank>1 options would have impact to the potential spec impact discussion, e.g., the type, format, etc. Therefore, it is Moderator’s understanding that proper downselection of rank>1 options is helpful to relieve our spec efforts. As a vast majority adopt Option 2-1/3-1/3-2, it is then proposed to prioritize Option 2-1/3-1/3-2.

Moderator note[Rd2]: Changed to high priority.

Proposed Conclusion 3.1.12: For the evaluation of CSI compression, for the rank>1 solutions, prioritize Option 2-1 (layer specific and rank common), Option 3-1 (layer common and rank common), and Option 3-2 (layer common and rank specific).

- Note: For the evaluations of CSI compression with 1-on-1 joint training,

  - Option 1-2 is not adopted;

  - Option 2-2 is not adopted;

  - Over the 11 sources [Huawei, Ericsson, Nokia, ZTE, OPPO, Intel, Fujitsu, NTT DOCOMO, InterDigital, Qualcomm, Futurewei] for FTP/fullbuffer, Max Rank 2,

    - Option 1-1 is adopted by 1 source [ZTE]

    - Option 2-1 is adopted by 3 sources [Ericsson, Intel, Qualcomm]

    - Option 3-1 is adopted by 8 sources [Huawei, Nokia, ZTE, OPPO, Fujitsu, Futurewei, Xiaomi, vivo]

    - Option 3-2 is adopted by 2 sources [NTT DOCOMO, InterDigital]

  - Over the 4 sources [Qualcomm, ZTE, CATT, Apple] for FTP/fullbuffer, Max Rank 4,

    - Option 2-1 is adopted by 1 source [Qualcomm]

    - Option 3-1 is adopted by 2 sources [Qualcomm, ZTE]

    - Option 3-2 is adopted by 1 source [Apple]

| Support/Can accept | vivo, LG Electronics, NTT DOCOMO(w/ comments), OPPO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | We are supportive of Option 3-1, but we are open to Option 2-1 and Option 3-2. |
| LG Electronics | We prefer option 3-1. |
| NTT DOCOMO | We also provided the results on Option 3-1 in Table 3. We are open to both Option 3-1 and 3-2. |
| OPPO | Supportive of option 3-1, also open to option 2-1 and option 3-2. |

### 3.1-2: Table 2. Evaluation results for CSI compression with model generalization

#### Issue#3-12 (High priority) Generalization-Deployment scenarios

Moderator note: Summarized results for generalization over deployment scenarios.

Principle of filtering: For some results which largely bias from the majority (e.g., too low SGCS under generalization Case 3), they are marked in grey and not captured into the observation. But for a group of results, if partial results fall into the majority range while other do not, then the partial results in the majority range are captured to the observation.

Moderator note[Rd2]:

- Table updated by correcting some errors.

- Content of the observation updated. Filtering is canceled, and some significant degradation results under Case 3 are also captured with a note.

| Training, Generalization Case 1 |  |  | UMa |  |  | UMi |  |  | InH |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | UMi | InH |  | UMa | InH |  | UMa | UMi |
| Training, Generalization Case 3 |  |  | Uma+ UMi | Uma+ InH | Uma+ UMi+ InH | Uma+ UMi | Umi+ InH | Uma+ UMi+ InH | Uma+ InH | Umi+ InH | Uma+ UMi+ InH |
| Testing |  |  | UMa |  |  | UMi |  |  | InH |  |  |
|  | Case |  |  |  |  |  |  |  |  |  |  |
| HW#1/#2 | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | 0.916 |  |  |  |  |  | 0.968 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  | -7% |  |  |  |  | -0.6% |  |
|  | Case 3 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  | -1% |  |  |  |  | -0.4% |  |  |
| Futurewei#1#2 | Case 1 | X | 0.92 |  |  | 0.936 |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | X |  | -14.2% |  |  | -18.5% |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 3 | X | 1.8% |  |  | -0.6% |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| Lenovo#1#2 | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | 77.2 |  |  |  |  |  | 96.4 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  | -21.76% |  |  |  |  | -3.6% |  |
|  | Case 3 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| Intel#1/#2 | Case 1 | X | L1:0.7103 L2:0.5290 |  |  | L1:0.7208 L2:0.5610 |  |  |  |  |  |
|  |  | Y | L1:0.8431 L2:0.7143 |  |  | L1:0.8603 L2:7273 |  |  |  |  |  |
|  |  | Z | L1:0.9166 L2:0.8415 |  |  | L1:0.9320 L2:0.8600 |  |  |  |  |  |
|  | Case 2 | X |  | L1:-3.8% L2:-4.6% |  |  | L1:-2.5% L2:-8.1% |  |  |  |  |
|  |  | Y |  | L1:-1.6% L2:-2.4% |  |  | L1:-2.5% L2:-3.6% |  |  |  |  |
|  |  | Z |  | L1:-0.4% L2:-3.1% |  |  | L1:-1.2% L2:-1.4% |  |  |  |  |
|  | Case 3 | X | L1:2.2% L2:-4.5% |  |  | L1:2.1% L2:-5.5% |  |  |  |  |  |
|  |  | Y | L1:0.1% L2:-6.7% |  |  | L1:-0.9% L2:-6.4% |  |  |  |  |  |
|  |  | Z | L1:0.1% L2:-2.1% |  |  | L1:-0.2% L2:-0.2% |  |  |  |  |  |
| Vivo#1#2#3#4 | Case 1 | X | 0.8002 |  |  | 0.8111, 0.8035 |  |  | 0.8574 |  |  |
|  |  | Y | 0.8265 |  |  | 0.8596, 0.8564 |  |  | 0.9025 |  |  |
|  |  | Z | 0.8954 |  |  | 0.9125, 0.9093 |  |  | 0.9411 |  |  |
|  | Case 2 | X |  | 0.04% |  |  | -0.1% | -20% |  |  | -29.9% |
|  |  | Y |  | -0.5% |  |  | -0.75% | -20% |  |  | -29.6% |
|  |  | Z |  | -0.5% |  |  | -0.26% | -18% |  |  | -27.8% |
|  | Case 3 | X | 0.17% |  |  | 0.1% | -0.8% |  |  | -0.3% |  |
|  |  | Y | 0.35% |  |  | 0.02% | -3.3% |  |  | 0.5% |  |
|  |  | Z | 0.3% |  |  | 0.06% | -4% |  |  | 0.7% |  |
| OPPO#1#2 | Case 1 | X | 0.786 |  |  | 0.784 |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | X |  | -1.8% |  |  | -1.4% |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 3 | X | -0.6% |  |  | -1.1% |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| CATT | Case 1 | X | 0.677 |  |  | 0.646 |  |  | 0.841 |  |  |
|  |  | Y | 0.756 |  |  | 0.746 |  |  | 0.872 |  |  |
|  |  | Z | 0.816 |  |  | 0.84 |  |  | 0.904 |  |  |
|  | Case 2 | X |  | 1.18% | -13.74% |  | -4.8% |  |  | 4.64% |  |
|  |  | Y |  | 1.46% | -19.44% |  | -4.02% |  |  | 3.21% |  |
|  |  | Z |  | -2.21% | -20.96% |  | -4.76% |  |  | -0.11% |  |
|  | Case 3 | X | -1.48% | -1.92% |  | -4.49% |  |  | 4.76% |  |  |
|  |  | Y | -1.85% | -2.78% |  | -4.96% |  |  | 3.78% |  |  |
|  |  | Z | -0.74% | -1.35% |  | -4.64% |  |  | 2.10% |  |  |
| Xiaomi#1#2 | Case 1 | X | L1: 0.7168 L2: 0.564 |  |  | L1:0.7094 L2:0.5529 |  |  |  |  |  |
|  |  | Y | L1:0.7767 L2:0.6476 |  |  | L1:0.7864 L2:0.6522 |  |  |  |  |  |
|  |  | Z | L1:0.8893 L2:0.8108 |  |  | L1:0.8834 L2:0.7983 |  |  |  |  |  |
|  | Case 2 | X |  | L1:-0.2% L2: -1.8% |  |  | L1:-3.4% L2:-4.4% |  |  |  |  |
|  |  | Y |  | L1: -0.2% L2: -2.8% |  |  | L1: -3.2% L2: -3.2% |  |  |  |  |
|  |  | Z |  | L1: -3% L2: -6.3% |  |  | L1: -0.5% L2: 0.4% |  |  |  |  |
|  | Case 3 | X | L1: -1% L2: -0.5% |  |  | L1: 2.3% L2: 3.8% |  |  |  |  |  |
|  |  | Y | L1: -0.3% L2: -1.7% |  |  | L1: -1.3% L2: -3.3% |  |  |  |  |  |
|  |  | Z | L1: -2.6% L2: -4.8% |  |  | L1: -2.8% L2: -6% |  |  |  |  |  |
| ZTE#1#2#3#4#5#6#7#8 | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | L1:0.9218 L2:0.8661 |  |  | L1:0.9314 L2:0.8812 |  |  | L1:0.9504 L2:0.9309 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  | L1:0.09% L2:-0.29% | L1:-5.55% L2:-7.96% |  | L1:-1.9% L2:-2.86% | L1:-8.63% L2:-12.79% |  | L1:-1.74% L2:-2.38% | L1:-1.44% L2:-2.41% |
|  | Case 3 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | L1:0.09% L2:0.08% |  | L1:-0.08% L2:-0.1% | L1:-0.92% L2:-1.32% |  | L1:-1.04% L2:-1.37% |  |  | L1:1.1% L2:1.19% |
| NTT DOCOMO#1#2#3#4#5#6 | Case 1 | X | L1:0.59 L2:0.46 |  |  | L1:0.59 L2:0.47 |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | X |  | L1: -3.39% L2: -6.5% |  |  | L1: -3.39% L2:-4.26% |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 3 | X | L1:-1.7%/-1.7%/-1.7% L2:-2.17%/0%/0% |  |  | L1:-1.7%/0%/-1.7% L2:-2.1%/0%/-2.1% |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| InterDigital#1#2 | Case 1 | X | L1:0.623 L2:0.444 |  |  | L1:0.652 L2:0.469 |  |  |  |  |  |
|  |  | Y | L1:0.71 L2:0.536 |  |  | L1:0.741 L2:0.57 |  |  |  |  |  |
|  |  | Z | L1:0.803 L2:0.671 |  |  | L1:0.829 L2:0.694 |  |  |  |  |  |
|  | Case 2 | X |  | L1: -0.8% L2:-2.25% |  |  | L1:-3.68% L2:-4.26% |  |  |  |  |
|  |  | Y |  | L1:-1.69% L2:-3.36% |  |  | L1:-2.83% L2:-4.56% |  |  |  |  |
|  |  | Z |  | L1:-3.6% L2:-7.75% |  |  | L1:-1.81% L2:-1.87% |  |  |  |  |
|  | Case 3 | X | L1:2.1% L2:0% |  |  | L1:1.07% L2:0.64% |  |  |  |  |  |
|  |  | Y | L1:1.83% L2:2.05% |  |  | L1:1.62% L2:1.75% |  |  |  |  |  |
|  |  | Z | L1:1.868% L2:2.23% |  |  | L1:1.93% L2:3.9% |  |  |  |  |  |
| MTK#1#2 | Case 1 | X | 0.696 |  |  | 0.704 |  |  |  |  |  |
|  |  | Y | 0.771 |  |  | 0.767 |  |  |  |  |  |
|  |  | Z | 0.831 |  |  | 0.826 |  |  |  |  |  |
|  | Case 2 | X |  | -1.4% |  |  | -0.9% |  |  |  |  |
|  |  | Y |  | -2.8% |  |  | 0.2% |  |  |  |  |
|  |  | Z |  | -2.1% |  |  | 0.1% |  |  |  |  |
|  | Case 3 | X | -1.6% |  |  | -0.8% |  |  |  |  |  |
|  |  | Y | -1.0% |  |  | 0.1% |  |  |  |  |  |
|  |  | Z | -0.5% |  |  | 0.2% |  |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.6%) |  | Xiaomi, Interdigital, MTK, vivo, Intel, ZTE |  |  | Vivo, Oppo, MTK, Intel, Xiaomi |  |  | HW, CATT | ZTE |
|  |  | Moderate/Significant loss (-1.6%~-4%) |  | Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT,MTK |  |  | ZTE, Interdigital, CATT, Xiaomi, Intel, NTT DOCOMO, |  |  | Lenovo, ZTE | Vivo, ZTE |
|  |  | Significant loss (<-4%) |  | FW, Intel, Xiaomi, Interdigital, NTT DOCOMO | HW, Lenovo, CATT, ZTE |  | FW, CATT, NTT DOCOMO, Interdigital, Xiaomi, Intel | ZTE, vivo |  |  | Vivo, |
|  |  | Positive gain |  | Vivo, CATT, ZTE |  |  | MTK, Xiaomi |  |  | CATT, |  |
|  | Case 3 | Minor loss (>-1.6%) | CATT, Xiaomi, NTT DOCOMO, MTK, Interdigital,OPPO | HW, CATT | ZTE | Intel, Futurewei, vivo, xiaomi, OPPO, MTK, ZTE, NTT DOCOMO | Vivo | ZTE | HW | vivo |  |
|  |  | Moderate loss (-1.6% ~ -4%) | NTT DOCOMO, CATT, Xiaomi, Intel | CATT |  | Xiaomi, NTT DOCOMO | vivo |  | CATT |  |  |
|  |  | Significant loss (<-4%) | Intel, Xiaomi, |  |  | Intel, CATT, Xiaomi |  |  |  |  |  |
|  |  | Positive gain | Futurewei, Intel, vivo, Interdigital, ZTE |  | ZTE | Vivo, Interdigital, MTK, NTT DOCOMO, Intel, xiaomi |  |  | CATT | Vivo | ZTE |

Upd Proposed Observation 3.1.13: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - If deployment scenario#A is UMi & deployment scenario#B is UMa, deployment scenario#A is UMa & deployment scenario#B is UMi, or deployment scenario#A is UMa & deployment scenario#B is InH:

    - 9 sources [Xiaomi, InterDigital, MTK, vivo, Intel, ZTE, OPPO, Huawei, CATT] observe that generalized performance can be achieved:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Xiaomi, InterDigital, MTK, vivo, Intel, ZTE, CATT] observe less than -1.6% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 5 sources [vivo, OPPO, MTK, Intel, Xiaomi] observe less than -1.4% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [Huawei, CATT] observe less than -0.6% degradation or positive gain

    - 10 sources [Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT, ZTE, Lenovo, MTK, Futurewei] observe that moderate/significant degradations are suffered under generalization Case 2:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 8 sources [Futurewei, MTK Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT] observe larger than -1.69% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 7 sources [Futurewei, NTT DOCOMO, ZTE, InterDigital, CATT, Xiaomi, Intel] observe larger than -1.81% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [ZTE, Lenovo] observe larger than -1.74% degradation.

  - If deployment scenario#A is InH & deployment scenario#B is Uma/UMi, significant performance degradations are observed under generalization Case 2:

    - For deployment scenario#A is InH & deployment scenario#B is UMa, 4 sources [Huawei, CATT, Lenovo, ZTE] observe -5.55%~-21.76% degradation.

    - For deployment scenario#A is InH & deployment scenario#B is UMi, 2 sources [vivo, ZTE] observe -8.63%~-20% degradation.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MTK, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Minor loss (0%~-1.48%) are observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MTK, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Moderate loss (-1.6%~-4%) are observed by 5 sources [Xiaomi, CATT, vivo, ZTE, NTT DOCOMO, Intel].

  - Positive gains are observed by 8 sources [ZTE, Interdigital, MTK, vivo, Intel, Xiaomi, Futurewei, CATT NTT DOCOMO].

  - Note: Significant degradations of up to -6.7% are still observed by 2 sources [Intel, Xiaomi] for deployment scenario#B subject to UMa, and by 2 sources [Intel, CATT] for deployment scenario#B subject to UMi.

- Note: For generalization Case 2, if deployment scenario#A is UMi & deployment scenario#B is InH, 2 sources [vivo, ZTE] observe different trends, where significant performance degradations of -27.8%~-29.9% are observed by [vivo], while moderate performance degradations of -1.44%~-2.41% are observed by [ZTE].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Results refer to Table 5.1-2 of R1-2306058.

| Support/Can accept | Lenovo, ZTE, OPPO, vivo, NTT DOCOMO, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| Futurewei | For this observation, we are generally ok. We only studied range X using vector quantization and the higher performance loss for generalization Case 2 may be due to the very low number of CSI bits and we will try using higher number of bits (i.e., via scalar quantization) and submit our results in the next meeting. However, similar comments as indicated previously, we think there should be a clear definition of what is too high/low and agreed among companies. |

#### Issue#3-13 (High priority) Generalization-UE distributions

Moderator note: Summarized results for generalization over UE distributions.

Principle of filtering: For some results which largely bias from the majority (e.g., too low SGCS under generalization Case 3), they are marked in grey and not captured into the observation. But for a group of results, if partial results fall into the majority range while other do not, then the partial results in the majority range are captured to the observation.

Moderator note[Rd2]: Content of the table/observation updated by canceling the Filtering; and some significant degradation results under Case 3 are also captured with a note.

| Training, Generalization Case 1 |  |  | Indoor |  |  | Outdoor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | Outdoor |  |  | Indoor |  |
| Training, Generalization Case 3 |  |  | I:O=5:5 | I:O=8:2 |  | I:O=5:5 | I:O=8:2 |  |
| Testing |  |  | Indoor |  |  | Outdoor |  |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| HW#1/#2 | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z | 0.916 |  |  | 0.948 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  | -3% |  |  | 0.1% |  |
|  | Case 3 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z | -0.2% |  |  | 0.1% |  |  |
| QC#1/#2 | Case 1 | X |  |  |  |  |  |  |
|  |  | Y | 0.7624 |  |  | 0.8915 |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  |  |  |
|  |  | Y |  | -11.5% |  |  | +0.96% |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X |  |  |  |  |  |  |
|  |  | Y |  | + 0.9% |  |  | +1.8% |  |
|  |  | Z |  |  |  |  |  |  |
| Nokia#1/#2 | Case 1 | X | 0.685 |  |  | 0.837 |  |  |
|  |  | Y | 0.741 |  |  | 0.854 |  |  |
|  |  | Z | 0.881 |  |  | 0.926 |  |  |
|  | Case 2 | X |  | -2.9% |  |  | -0.7% |  |
|  |  | Y |  | -5.5% |  |  | 1.2% |  |
|  |  | Z |  | -3.4% |  |  | 0.5% |  |
|  | Case 3 | X |  | 2.5% |  |  | -0.2% |  |
|  |  | Y |  | -3.9% |  |  | 0.1% |  |
|  |  | Z |  | -1.0% |  |  | 0.4% |  |
| NTT DOCOMO#1#2#3#4#5#6 | Case 1 | X | 0.6 0.47 |  |  | 0.65 0.56 |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  | 0% 0% |  |  | 1.54% 3.57% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X | 0% 0% | 1.67% 2.1% |  | 0% 1.75% | 1.54% 3.57% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.6%) |  | NTT DOCOMO |  |  | Nokia |  |
|  |  | Moderate/significant loss (<-1.6%) |  | Nokia, QC, HW |  |  |  |  |
|  |  | Positive gain |  |  |  |  | QC, Nokia, HW, NTT DOCOMO |  |
|  | Case 3 | Minor loss (>-1.6%) | NTT DOCOMO, HW | Nokia |  |  | Nokia |  |
|  |  | Moderate loss (-1.6%~-4% |  | Nokia |  |  |  |  |
|  |  | Positive gain |  | QC, Nokia, NTT DOCOMO |  | NTT DOCOMO, HW | QC, Nokia, NTT DOCOMO |  |

Upd Proposed Observation 3.1.14: For the generalization verification of AI/ML based CSI compression over various UE distributions, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE distribution#B and applied for inference with a same UE distribution#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE distribution#A and UE distribution#B but not for others

  - If UE distribution#A is Outdoor & UE distribution#B is Indoor, 3 sources [Nokia, Qualcomm, Huawei] observe that moderate/significant degradations of larger than -2.9% degradation are suffered,

    - Note: 1 source [NTT DOCOMO] observes 0% degradation

  - If UE distribution#A is Indoor & UE distribution#B is Outdoor, 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei] observe less than -0.7% degradation or positive gain

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-1% loss or positive gain) for UE distribution#B subject to any of Outdoor and Indoor, if the training dataset is constructed with data samples subject to multiple UE distributions including UE distribution#B, as observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Minor loss (0%~-1%) are observed by 3 sources [NTT DOCOMO, Nokia, Huawei].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Note: Moderate degradations of up to -3.9% are still observed by 1 source [Nokia] for deployment scenario#B subject to Indoor.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

#### Issue#3-14 (Low priority) Generalization-Carrier frequencies

Moderator note: Summarized results for generalization over carrier frequencies.

Moderator note[Rd2]: No change on the issue. Please provide more inputs!

| Training, Generalization Case 1 |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | 3.5/4GHz |  |  | 2GHz |  |
| Training, Generalization Case 3 |  |  | 2GHz+3.5/4GHz | 5.5GHz+3.5/4GHz |  | 2GHz+3.5/4GHz | 5.5G+3.5/4GHz |  |
| Testing |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| vivo#1/#2 | Case 1 | X |  |  |  | 0.788 |  |  |
|  |  | Y |  |  |  | 0.847 |  |  |
|  |  | Z |  |  |  | 0.9013 |  |  |
|  | Case 2 | X |  |  |  |  | 0.782/-0.76% |  |
|  |  | Y |  |  |  |  | 0.838/-1% |  |
|  |  | Z |  |  |  |  | 0.8992/-0.23% |  |
|  | Case 3 | X |  |  |  | 0.788/0% | 0.789/0.127% |  |
|  |  | Y |  |  |  | 0.845/0.236% | 0.848/0.12% |  |
|  |  | Z |  |  |  | 0.9015/0.02% | 0.9002/-0.12% |  |
| NTT DOCOMO #1#2#3#4#5#6 | Case 1 | X | 0.58 |  |  | 0.58 |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  | -1.72% |  |  | 1.72% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X | 0%/0%/1.72% |  |  | 1.72%/1.72%/1.72% |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
| Nokia#1#2 | Case 1 | X | 0.715 |  |  | 0.691 |  |  |
|  |  | Y | 0.763 |  |  | 0.796 |  |  |
|  |  | Z | 0.886 |  |  | 0.880 |  |  |
|  | Case 2 | X |  | -0.8% |  |  | 0.1% |  |
|  |  | Y |  | 4.8% |  |  | -6.6% |  |
|  |  | Z |  | 0.1% |  |  | -0.9% |  |
|  | Case 3 | X | -0.1% |  |  | 0.3% |  |  |
|  |  | Y | 1.0% |  |  | -4.9% |  |  |
|  |  | Z | 0.1% |  |  | -0.3% |  |  |
| MTK#1#2 | Case 1 | X | 0.696 |  |  | 0.702 |  |  |
|  |  | Y | 0.771 |  |  | 0.773 |  |  |
|  |  | Z | 0.831 |  |  | 0.835 |  |  |
|  | Case 2 | X |  | 0.3% |  |  | -0.1% |  |
|  |  | Y |  | -0.5% |  |  | 0.5% |  |
|  |  | Z |  | -0.2% |  |  | 0.4% |  |
|  | Case 3 | X | 0.0% |  |  | -0.3% |  |  |
|  |  | Y | -0.8% |  |  | -0.3% |  |  |
|  |  | Z | -0.2% |  |  | 0.1% |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  | NTT DOCOMO, Nokia, MTK |  |  | Vivo, Nokia, MTK |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  |  | Nokia |  |
|  |  | Positive gain |  | MTK, Nokia |  |  | NTT DOCOMO, Nokia, MTK |  |
|  | Case 3 | Minor loss (>-1.6%) | Nokia, MTK | Nokia |  | Nokia, MTK | Vivo |  |
|  |  | Significant loss (<-4%) |  |  |  | Nokia |  |  |
|  |  | Positive gain | NTT DOCOMO, Nokia | Nokia |  | Vivo, NTT DOCOMO, Nokia, MTK | vivo |  |

Upd Proposed Observation 3.1.15: For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B,

- For generalization Case 2, generalized performance may be achieved in general

  - If carrier frequency#A is 3.5/4GHz & carrier frequency#B is 2GHz, 3 sources [NTT DOCOMO, Nokia, MTK] observe generalized performance of less than -1.72% degradation.

  - If carrier frequency#A is 2GHz & carrier frequency#B is 3.5/4GHz, 4 sources [NTT DOCOMO, Nokia, vivo, MTK] observe generalized performance of less than -1% degradation or positive gain,

    - Note: 1 source [Nokia] observes significant degradations of -6.6%.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-0.8% loss or positive gain) for carrier frequency#B subject to any of 2GHz and 3.5/4GHz, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B, as observed by 4 sources [NTT DOCOMO, Nokia, vivo, MTK].

  - Minor loss (0%~-0.8%) are observed by 3 sources [Nokia, vivo, MTK].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, vivo, MTK].

  - Note: Significant degradations of up to -4.9% are still observed by 1 source [Nokia] for deployment scenario#B subject to 3.5/4GHz

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Antenna layouts are assumed as the same over the different frequency carriers.

| Support/Can accept | NTT DOCOMO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

#### Issue#3-15 (Low priority) Generalization-TxRU mappings

Moderator note: Summarized results for generalization over TxRU mappings

Moderator note[Rd2]: No change on the issue. Please provide more inputs!

| Training, Generalization Case 1 |  |  | [8,8,2] |  | [2,8,2] |  | [4,4,2] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | [2,8,2] | [4,4,2] | [8,8,2] | [8,8,2] | [2,8,2] |
| Training, Generalization Case 3 |  |  | [8,8,2]+ [2,8,2] | [8,2,2]+[4,4,2] | [8,8,2]+ [2,8,2] | [2,8,2]+[4,4,2] | [8,8,2]+ [4,4,2] | [2,8,2]+ [4,4,2] |
| Testing |  |  | [8,8,2] |  | [2,8,2] |  | [4,4,2] |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| HW#1/#2 | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z | 0.916 |  | 0.922 |  |  |  |
|  | Case 2 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  | -3% |  | -0.9% |  |  |
|  | Case 3 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z | -0.3% |  |  | -0.2% |  |  |
| Apple#1 | Case 1 | X | 0.7163 |  |  |  |  |  |
|  |  | Y | 0.7767 |  |  |  |  |  |
|  |  | Z | 0.8506 |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  | 0.5213/-27.2% |  |
|  |  | Y |  |  |  |  | 0.5664/-27% |  |
|  |  | Z |  |  |  |  | 0.6382/-24.97% |  |
|  | Case 3 | X |  | 0.7146/-0.2% |  |  |  |  |
|  |  | Y |  | 0.7632/-1.7% |  |  |  |  |
|  |  | Z |  | 0.8566/0.7% |  |  |  |  |
| Nokia#1/#2 | Case 1 | X |  |  | 0.691 |  | 0.750 |  |
|  |  | Y |  |  | 0.796 |  | 0.848 |  |
|  |  | Z |  |  | 0.880 |  | 0.919 |  |
|  | Case 2 | X |  |  | -23.6% |  |  | -34.1% |
|  |  | Y |  |  | -16.3% |  |  | -36.1% |
|  |  | Z |  |  | -22.6% |  |  | -24.7% |
|  | Case 3 | X |  |  |  | -0.9% |  | -4.4% |
|  |  | Y |  |  |  | -1.0% |  | -2.0% |
|  |  | Z |  |  |  | -1.1% |  | -2.5% |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  |  |  | HW |  |  |
|  |  | Moderate/significant loss (<-1.8%) |  | HW | Nokia |  | Apple | Nokia |
|  |  | Positive gain |  |  |  |  |  |  |
|  | Case 3 | Minor loss (>-1.8%) | HW | Apple |  | HW, Nokia |  |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  |  |  | Nokia |
|  |  | Positive gain |  | Apple |  |  |  |  |

Proposed Observation 3.1.16: For the generalization verification of AI/ML based CSI compression over various TxRU mappings, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain TxRU mapping#B and applied for inference with a same TxRU mapping#B,

- For generalization Case 2, significant performance degradations are observed in general:

  - If TxRU mapping#A is [4,4,2] & TxRU mapping#B is [2,8,2], 1 source [Nokia] observe significant performance degradations of -16.3%~-23.6%.

  - If TxRU mapping#A is [8,8,2] & TxRU mapping#B is [4,4,2], 1 source [Apple] observes significant performance degradations of -25%~-27.2%.

  - If TxRU mapping#A is [2,8,2] & TxRU mapping#B is [4,4,2], 1 source [Nokia] observes significant performance degradations of -24.7%~-36.1%.

  - if TxRU mapping#A is [2,8,2] & TxRU mapping#B is [8,8,2], 1 source [Huawei] observes that moderate degradations of -3% are suffered.

  - Note: if TxRU mapping#A is [8,8,2] & TxRU mapping#B is [2,8,2], 1 source [Huawei] observes that generalized performance of -0.9% degradation can be achieved

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4.4% loss or positive gain) for TxRU mapping#B subject to any of [8,8,2] and [2,8,2] and [4,4,2], if the training dataset is constructed with data samples subject to multiple TxRU mappings including TxRU mapping#B, as observed by 3 sources [Huawei, Nokia, Apple].

  - Minor loss (0%~1.7%) are observed by 3 sources [Huawei, Apple, Nokia].

  - Moderate loss (2%~4.4%) are observed by 1 source [Nokia].

  - Positive gains are observed by 1 source [Apple].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

### 3.1-3: Table 3. Evaluation results for CSI compression with model scalability

#### Issue#3-16 (Low priority) Scalability-Bandwidths

Moderator note: Summarized results for scalability over various bandwidths.

Moderator note[Rd2]: No change on the issue. Please provide more inputs!

| Training, Generalization Case 1 |  |  | 10MHz/52RB |  | 20MHz | 5MHz | 48RB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  | 20MHz | 48RB | 10MHz | 10MHz |  |
| Training, Generalization Case 3 |  |  | 20MHz+10MHz | 52RB+48RB | 20MHz+10MHz |  | 52RB+48RB |
| Testing |  |  | 10MHz/52RB |  | 20MHz | 5MHz | 48RB |
|  | Case | CSI payload |  |  |  |  |  |
| Nokia#1/#2 | Case 1 | X | 0.618, 0.66 |  |  |  | 0.619, 0.677 |
|  |  | Y | 0.727 |  |  |  | 0.728 |
|  |  | Z | 0.808 |  |  |  | 0.806 |
|  | Case 2 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 3 | X |  | 2.2%, -0.1% |  |  | 2.7% -2.5% |
|  |  | Y |  | 1.2% |  |  | 0.9% |
|  |  | Z |  | -1.7% |  |  | -1.6% |
| ZTE#1/#2 | Case 1 | X | L1:0.7191 L2:0.5866 |  | L1:0.6926 L2:0.5495 |  |  |
|  |  | Y | L1:0.8033 L2:0.6913 |  | L1:0.7835 L2:0.6525 |  |  |
|  |  | Z | L1:0.8978 L2:0.8275 |  | L1:0.877 L2:0.7882 |  |  |
|  | Case 2 | X | L1:-0.47% L2:-1.28% |  | L1:0% L2:-0.27% |  |  |
|  |  | Y | L1:-0.22% L2:-0.04% |  | L1:-0.46% L2:-0.49% |  |  |
|  |  | Z | L1:-0.85% L2:-1.1% |  | L1:-0.64% L2:-0.44% |  |  |
|  | Case 3 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
| NTT DOCOMO#1/#2 | Case 1 | X | L1:0.73 L2:0.63 |  | L1:0.68 L2:0.57 |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 3 | X | L1:-1.37% L2:-1.59% |  | L1:-1.47% L2:-1.75% |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
| InterDigital#1 | Case 1 | X |  |  |  | L1:0.623 L2:0.426 |  |
|  |  | Y |  |  |  | L1:0.683 L2:0.525 |  |
|  |  | Z |  |  |  | L1:0.745 L2:0.56 |  |
|  | Case 2 | X |  |  |  | L1:-4.3% L2:-3.3% |  |
|  |  | Y |  |  |  | L1:-2.5% L2:-11.4 |  |
|  |  | Z |  |  |  | L1:-8.46% L2:-12.7 |  |
|  | Case 3 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
| QC#1 | Case 1 | X |  |  | 0.75 |  |  |
|  |  | Y |  |  | 0.825 |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 3 | X |  |  | 4.4% |  |  |
|  |  | Y |  |  | -5.4% |  |  |
|  |  | Z |  |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.6%) | ZTE |  | ZTE |  |  |
|  |  | Moderate/significant loss (<-1.6%) |  |  |  | Interdigital |  |
|  |  | Positive gain |  |  |  |  |  |
|  | Case 3 | Minor loss (>-1.8%) | NTT DOCOMO | Nokia | NTT DOCOMO |  | Nokia |
|  |  | Moderate/significant loss (<-1.8%) |  | Nokia | QC |  | Nokia |
|  |  | Positive gain |  | Nokia | QC |  | Nokia |

Proposed Observation 3.1.17: For the scalability verification of AI/ML based CSI compression over various bandwidths, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain bandwidth#B and applied for inference with a same bandwidth#B,

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-2.5% loss or positive gain) for bandwidth#B, if the training dataset is constructed with data samples subject to multiple bandwidths including bandwidth#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, as observed by 3 sources [NTT DOCOMO, Nokia, QC].

  - Minor loss (0%~-1.75%) are observed by 2 sources [NTT DOCOMO, Nokia].

  - Moderate loss (-2.5%) are observed by 1 source [Nokia].

  - Positive gains are observed by 2 sources [Nokia, QC].

  - Note: Significant loss (-5.4%) is observed by 1 source [QC]

  - Note: Pre/post-processing of truncation/padding is adopted by 3 sources [NTT DOCOMO, Nokia, QC].

- Note: For generalization Case 2, 1 source [ZTE] observes generalized performance of 0%~-1.28% loss using proportional bandwidth and SB size.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

| Support/Can accept | NTT DOCOMO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

#### Issue#3-17 (High priority) Scalability-Tx port number

Moderator note: Summarized results for scalability over various Tx port numbers.

Moderator note[Rd2]: Content of the table/observation updated by canceling the Filtering; and some significant degradation results under Case 3 are also captured with a note.

| Training, Generalization Case 1 |  |  | 32ports |  | 16ports | 32ports |
| --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | 16ports |  | 32ports |
| Training, Generalization Case 3 |  |  | 32ports+16ports |  | 32ports+16ports |  |
| Testing |  |  | 32ports |  | 16ports |  |
|  | Case | CSI payload |  |  |  |  |
| HW#1/#2 | Case 1 | X | 0.767 |  | 0.831 |  |
|  |  | Y | 0.861 |  | 0.894 |  |
|  |  | Z | 0.918 |  | 0.944 |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | -0.1% |  | -0.5% |  |
|  |  | Y | -1% |  | -0.2% |  |
|  |  | Z | -0.5% |  | -0.2% |  |
| OPPO#1#2 | Case 1 | X | 0.855 |  | 0.886 |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  | -21.8% |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | -0.2% |  | 1.35% |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
| Fujitsu | Case 1 | X |  |  | 0.8401 |  |
|  |  | Y |  |  | 0.8924 |  |
|  |  | Z |  |  | 0.9568 |  |
|  | Case 2 | X |  |  |  | -11.6% |
|  |  | Y |  |  |  | -12.26% |
|  |  | Z |  |  |  | -8.8% |
|  | Case 3 | X |  |  | -1% |  |
|  |  | Y |  |  | 0.01% |  |
|  |  | Z |  |  | 0.18% |  |
| Nokia#1#2 | Case 1 | X |  |  | 0.768 |  |
|  |  | Y | 0.754 |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X |  |  | -4% |  |
|  |  | Y | -3.6% |  |  |  |
|  |  | Z |  |  |  |  |
| ZTE#1#2 | Case 1 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z | L1:0.9218 L2:0.8771 |  | L1:0.947 L2:0.9047 |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  | L1:-3.37% L2:-5.77% |
|  | Case 3 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z | L1:-0.89% L2:-1.47% |  | L1:0.88% L2:1.34% |  |
| NTT DOCOMO#1#2 | Case 1 | X | L1:0.58 L2:0.46 |  | L1:0.68 L2:0.57 |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | L1:0% L2:-2.17% |  | L1:-1.47% L2:-1.75% |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
| CATT#1#2 | Case 1 | X | L1:0.7231 L2:0.5675 L3:0.2662 L4:0.1447 |  | L1:0.7802 L2:0.5993 L3:0.2609 L4:0.1429 |  |
|  |  | Y | L1:0.7562 L2:0.5889 L3:0.2633 L4:0.1428 |  | L1:0.807 L2:0.6196 L3:0.2666 L4:0.1487 |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | L1:-1.62% L2:-3.42% L3:-5.03% L4:-0.35% |  | L1:-2.08% L2:-6.08% L3:-2.49% L4:-1.11% |  |
|  |  | Y | L1:-0.03% L2:-0.15% L3:0% L4:-0.7% |  | L1:- 1.84% L2:-1.18% L3:-2.4% L4:-1.75% |  |
|  |  | Z |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  |  |  |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  | Oppo, Fujistu, ZTE |
|  |  | Positive gain |  |  |  |  |
|  | Case 3 | Minor loss (>-1.8%) | HW, OPPO, NTT DOCOMO, CATT, ZTE |  | HW, Fujistu, NTT DOCOMO |  |
|  |  | Moderate loss (-1.8%~-4%) | CATT, NTT DOCOMO, Nokia |  | CATT, Nokia |  |
|  |  | Significant loss (<-4%) | CATT |  | CATT |  |
|  |  | Positive gain |  |  | OPPO, ZTE, Fujistu |  |

Proposed Observation 3.1.18: For the scalability verification of AI/ML based CSI compression over various Tx port numbers, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain Tx port number#B and applied for inference with a same Tx port number#B,

- For generalization Case 2, significant performance degradations are observed in general, if Tx port number#A is 32 & Tx port number#B is 16, as larger than -3.37% degradations are observed by 3 sources [OPPO, Fujitsu, ZTE]

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gains) for Tx port number#B subject to any of 16 and 32, if the training dataset is constructed with data samples subject to multiple Tx port numbers including bandwidth#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, as observed by 7 sources [HW, OPPO, NTT DOCOMO, CATT, ZTE, Fujistu, Nokia].

  - Minor loss (0%~-1.75%) are observed by 6 sources [HW, OPPO, Fujistu, CATT, ZTE, NTT DOCOMO].

  - Moderate loss (-1.84%~-4%) are observed by 3 sources [Nokia, CATT, NTT DOCOMO].

  - Positive gains are observed by 3 sources [OPPO, ZTE, Fujistu].

  - Note: Significant degradations of up to -6.08% are still observed by 1 source [CATT] for deployment scenario#B subject to 32 ports, and for deployment scenario#B subject to 16 ports

  - Note: Pre/post-processing of truncation/padding is adopted by 6 sources [HW, OPPO, NTT DOCOMO, ZTE, Fujistu, Nokia], and adaptation layer in the AL/ML model is adopted by 1 source [CATT].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

| Support/Can accept | NTT DOCOMO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

### 3.1-4: Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability

#### Issue#3-18 (On hold) Case 2 (1 NW part to M>1 UE parts)

Moderator note:

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-19 (On hold) Case 3 (N>1 NW parts to 1 UE part)

Moderator note:

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### 3.1-5: Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability

#### Issue#3-20 (Medium priority) NW first training-1 NW to 1/M>1 UE

Moderator note: Summarized results for NW first separate training with Case 1 (1 NW to 1 UE or 1 NW to M>1 UEs).

Principle of filtering: Only the separate training subject to the examples of the following agreement is captured. Hence the observation is for “NW first separate training with dataset sharing manner”. Other manners such as parallel training, iterative training, end-to-end training, “freeze-and-train”, etc., are not captured. In addition, for the Model pair for Type 3 does not equal to model pair of benchmark (joint training), the results are not captured since such comparison is unfair if the model structures are not aligned.

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2 - For NW-first training - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - For UE-first training - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

Moderator note[Rd2]: No changes for the second round. More inputs are welcome!

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip |  |  |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.5% |  | HW, Apple, CMCC, |  |  |

Proposed Observation 3.1.19: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone [or similar architecture] is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 4 sources [Ericsson, Nokia, ZTE，Fujitsu] observe -0%~-0.5% degradation, 4 sources [Nokia, QC, ZTE, vivo] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [HW, Apple, CMCC] observe -0%~-0.5% degradation.

- Note: For the NW first separate training case where different backbones [or substantially different architectures] are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [QC, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| Moderator | @Lenovo That is why the note is added: it only limits the results are from the one way dataset sharing (from the simulation assumption of majority companies). For other special handlings, we may have further observations. |

#### Issue#3-20a (Medium priority) NW first training-1 NW to 1/M>1 UE- Impact of size of shared training dataset

Moderator note: Summarized results for NW first separate training with Case 1 (1 NW to 1 UE or 1 NW to M>1 UEs) with various sizes of datasets for sharing.

Moderator note[Rd2]: No changes for the second round. More inputs are welcome!

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) dataset size | Same backbone, same/diff dataset size | Diff backbone; Cap_UE model < Cap_NW model, same/diff dataset size |
| --- | --- | --- | --- | --- |
| HW#1/2 | Descrip | TF#A-TF#1, 300k | TF#A-TF#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.1%/-0.3% Gap: -0.2% |  |
|  | Z |  | 0%/-0.1% Gap: -0.1% |  |
|  | Descrip | CNN#A-CNN#1, 300k | CNN#A-CNN#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.4%/-0.5% Gap: -0.1% |  |
|  | Z |  | -0.3%/-0.5% Gap: -0.2% |  |
| CMCC#1~7 | Descrip | TF#A-TF#1, 154k | TF#A-TF#1, 154k/100k/50k/ 10k/5k |  |
|  | X |  |  |  |
|  | Y |  | -0.22%/-0.22%/-0.55%/-1.86%/-2.95% Gap: 0/-0.3%/-1.64%/-2.73% |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-CNN#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -4.04%/-4.59%/-5.90%/-8.30%/-9.61% Gap: -0.55/-1.86%/-4.26%/-5.55% |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-MLP#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -0.76%/-0.98%/-1.53%/-6.33%/-9.17% Gap: -0.22%/-0.77%/-5.57%/-8.41% |
|  | Z |  |  |  |
| vivo#1/2 | Descrip | TF#A-TF#1, 300k; TF#A-TF#2, 300k; | TF#A-TF#1, 300k/50k; TF#A-TF#2, 300k/50k; |  |
|  | X |  | -0.6%/-2.6%; -0.9%/-2.6% |  |
|  | Y |  |  |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-CNN#1, 300k |  | TF#A-CNN#1, 300k/50k |
|  | X |  |  | -5.2%/ -9.4% Gap: -3.8% |
|  | Y |  |  |  |
|  | Z |  |  |  |
| Summary: gap of additional loss | -0%~-0.55% |  | HW, CMCC, | CMCC, vivo |
|  | -0.55%~-8.41% |  |  |  |

Proposed Observation 3.1.20: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [HW, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |

#### Issue#3-21 (On hold) NW first training- N>1 NW to 1/M>1 UE

Moderator note:

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-22 (Medium priority) UE first training-1/N>1 NW to 1 UE

Moderator note: Summarized results for NW first separate training with Case 1 (1 NW to 1 UE or N>1 NWs to 1 UE).

Principle of filtering: Only the separate training subject to the examples of the following agreement is captured. Hence the observation is for “UE first separate training with dataset sharing manner”. Other manners such as parallel training, iterative training, end-to-end training, “freeze-and-train”, etc., are not captured.

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2 - For NW-first training - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - For UE-first training - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

Moderator note[Rd2]: No changes for the second round. More inputs are welcome!

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model > Cap_NW model | Diff backbone or largely diff structure; Cap_UE model < Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A/B/C-TF#1 | TF#A/B/C-TF#1 |  |  |
|  | X |  | 0%/-0.4%/-0.2% |  |  |
|  | Y |  | 0%/-0.6%/-0.2% |  |  |
|  | Z |  | 0%/-0.6%/-0.1% |  |  |
|  | Descrip | CNN#A/B-CNN#1 | CNN#A/B-CNN#1 |  | TF#A- CNN#1 |
|  | X |  | -0.6%/-0.6% |  | -0.3% |
|  | Y |  | -0.5%/-0.6% |  | 0% |
|  | Z |  | -0.5%/-0.7% |  | 0% |
| Nokia#3/4 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | +1.7%/-0.2% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A/B/C /D-TF#1 | TF#A/B/C /D-TF#1 |  |  |
|  | X |  | -1.8%/ -2.3%/ -2.6%/ -2.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: -1.13% L2: -1.55% |  |  |
|  | Y |  | L1: -1.07% L2: -1.52% |  |  |
|  | Z |  | L1: -1.05% L2: -1.75% |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  | L1: -0.99% L2: -1.06% |  |
|  | Y |  |  | L1: -0.98% L2: -1.88% |  |
|  | Z |  |  | L1: -0.74% L2: -0.92% |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  | L1: -0.85% L2: -0.73% |
|  | Y |  |  |  | L1: -0.84% L2: -1.74% |
|  | Z |  |  |  | L1: -0.67% L2: -0.78% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.03% |  |  |
|  | Y |  | -0.31% |  |  |
|  | Z |  | -0.14% |  |  |
|  | Descrip | TF#A-TF#1 |  | CNN #A-TF#1 |  |
|  | X |  |  | -5.82% |  |
|  | Y |  |  | -5.51% |  |
|  | Z |  |  | -4.18 % |  |
| CATT#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.14%/ 0.21% |  |  |
|  | Y |  | -0.23%/-0.27% |  |  |
|  | Z |  | -0.14%/-0.35% |  |  |
|  | Descrip | CNN#A-TF #1 |  | CNN#A-TF #1 |  |
|  | X |  |  | 1.07% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -0.90% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +5.5%; +1.3%/ +2.7%; +17.1%/ +9.2% |  |  |
|  | Descrip |  |  |  | TF#A/B-CNN#1; TF#A/B-CNN#2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A/B- TF#1; |  | CNN#A/B- TF#1; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | 0.0%/ -1.4% | +5.8%/ +7.4% |
| vivo#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.4%/ -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A- TF#1 |  | CNN#A- TF#1 |  |
|  | X |  |  | -1.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | Nokia, Fujitsu, CATT, vivo |  |  |
|  | -0.5%~-1% |  |  | CATT, ZTE | ZTE |
|  | -1%~-1.88% |  | ZTE | ZTE, vivo | ZTE |
| Summary (Before Q) | -0%~-0.5% |  | HW |  |  |
|  | -0.5%~-1% |  | HW |  |  |
|  | -1%~-2.9% |  | Apple, |  |  |

Proposed Observation 3.1.21: For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1/N>1 NW to 1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone [or similar architecture] is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 4 sources [Nokia, Fujitsu, CATT, vivo] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [HW] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones [or substantially different architectures] are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We have also reported simulation results for the cases - When backbones are almost similar (Lenovo#1 Normal Type3) For the case where the shared input of the UE side CSI reconstruction part is after quantization, 4 sources [Nokia, Fujitsu, CATT, vivo, Lenovo] observe -0%~-0.5%-0.8% degradation - When backbones are very different (Lenovo#2 Normal Type3) If the backbone of the NW part model is less capable than the UE part model, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation and 1 sources [Lenovo] observe -1.2%~-4.2% degradation We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| Qualcomm | We ask the moderator to please clarify the comment that the format of our result is incorrect. |
| Moderator | @Lenovo Your results are submitted to Case 2 (dataset mixing case, e.g. M>1 UEs to 1 NW). The observation is for Case 1 (1 UE to 1 NW) @QC As there is only one NW part model in the assumptions, why are there results for two NWs? Why for layer 1 there are two results (two UEs?) and for other layers there is only one result? |

#### Issue#3-23 (On hold) UE first training-1/N>1 NW to 1 UE- UE first training-1/N>1 NW to M>1 UE

Moderator note:

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-24 (Medium priority) Benchmark of joint training for comparison

Moderator note: As in Issue#3-20/3-22, for NW first training and UE first training, Huawei, Fujitsu, and CMCC do not compare the Type 3 training with the benchmark with the same pair of models, the results will degrade or outperform the benchmark. Such comparison may be unfair since the reason of the degradation/gain may probably due to the capability of the model rather than the dataset sharing behavior. Therefore, the following proposal is given to align the pair of models to compare Type 3 with benchmark of joint training.

Moderator note[Rd2]: No change on the issue. Please provide more inputs!

Proposal 3.1.22: For the evaluation of training Type 3 under CSI compression, for the benchmark case (1-on-1 joint training) for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared.

- E.g., if the Type 3 is Transformer#1 for NW part model and CNN#1 for UE part model, then the benchmark case for performance comparison is also Transformer#1 for NW part model and CNN#1 for UE part model with joint training.

| Support/Can accept | Fujitsu, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### 3.1-6: Others

#### Issue#3-25 (Low priority) Others

Question 3.1.1: For the evaluation of the AI/ML based CSI compression sub use cases, what other aspects related with evaluations or observations do you think is necessary to be discussed?

| Company | View |
| --- | --- |

## 3rd round email discussions

#### Issue#3-2 (High priority) Down selection of model input/output

Moderator note[Rd3]: The intention of the proposal is due to the agreement on the study of the input/output types in 9.2.2.2; whether Option 2 is to be further studied depends on the evaluations in 9.2.2.1. As far as is known by Moderator, there is no company submitted results based on raw channel matrix, so it is proposed to prioritize Option 1 precoding matrix.

RAN1#112 9.2.2.2 Agreement In CSI compression using two-sided model use case, further study potential specification impact of the following output-CSI-UE and input-CSI-NW at least for Option 1: - Option 1: Precoding matrix - 1a: The precoding matrix in spatial-frequency domain - 1b: The precoding matrix represented using angular-delay domain projection - Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel) - 2a: raw channel is in spatial-frequency domain - 2b: raw channel is in angular-delay domain - Note: Whether Option 2 is also studied depends on the performance evaluations in 9.2.2.1. - Note: RI and CQI will be discussed separately

Proposal 3.2.1: For the evaluation of CSI compression, for the type of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part), prioritize precoding matrix for further studies eigenvector(s).

- Note: For the evaluations of CSI compression with 1-on-1 joint training, 21 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, Spreadtrum, Fujitsu, NTT DOCOMO, Xiaomi, Qualcomm, Intel, InterDigital, CATT, Apple, China Telecom, MediaTek, BJTU, ETRI, CMCC] take eigenvector(s) without angular-delay domain convertion as the model input/output; 1 source [Ericsson] takes eigenvectors with angular-delay domain representation as the model input/output. No company submitted explicit channel matrix as input.

| Support/Can accept | NTT DOCOMO, ETRI, vivo, LG Electronics, OPPO, Lenovo, ZTE, Futurewei, Xiaomi, Ericsson, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Futurewei | We are ok to prioritize using precoding matrix considering there is only 1 meeting left for Rel-18. |

#### Issue#3-5 (Medium priority) Mean UPT, full buffer

Moderator note[Rd3]: A similar “note” is added to mark the company with biased results.

Proposed Observation 3.2.2: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11%

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 6%~11% at CSI overhead A (small overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11% at CSI overhead C (large overhead);

- For Max rank 2, 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~15%

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~15% at CSI overhead A (small overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~10% at CSI overhead B (medium overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~14% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 24.47%~28.24%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Gain of CSI comporession over R16 Type II benchmark – Mean UPT, full buffer

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| HW#1 | A | 11% | 15% |  |
|  | B | 7% | 9% |  |
|  | C | 11% | 14% |  |
| Nokia #1 | A | 6.5% | 8.5% |  |
|  | B | 5.8% | 5% |  |
|  | C | 1.1% | 0.2% |  |
| vivo#1 | A | 7.92% | 11.15% |  |
|  | B | 6.02% | 7.09% |  |
|  | C | 9.53% | 10.28% |  |
| OPPO#1 | A | 6% |  |  |
|  | B | 3% |  |  |
|  | C | 3% |  |  |
| Fujitsu#1 | A | 9% | 9.2% |  |
|  | B | 5.6% | 7% |  |
|  | C | 2% | 13.7% |  |
| Xiaomi#1 | A |  | 24.47% |  |
|  | B |  | 28.24% |  |
|  | C |  | 27.36% |  |
| QC#1 | A |  | 8% |  |
|  | B |  | 10% |  |
|  | C |  | 3% |  |
| Intel#1 | A |  | 7% |  |
|  | B |  | 9% |  |
|  | C |  | 7% |  |
| InterDigital#1 | A |  | 4% |  |
|  | B |  | 4% |  |
|  | C |  | 0.8% |  |
| ZTE#1 | A |  |  | 8.51% |
|  | B |  |  | 7.44% |
|  | C |  |  | 9.95% |
| Summary | A | 6%~11% | 4%~15% | 8.51% |
|  | B | 3%~7% | 4%~10% | 7.44% |
|  | C | 1.1%~11% | 0.2%~14% | 9.95% |

| Support/Can accept | Qualcomm, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | For rank 4, our simulation results seem not to be incorporated in the summary since only ZTE provides the results. Could FL incorporate our evaluation results in summary as a reference for other companies’ further study? We understand the FL’s concern and we can wait for one more meeting for more inputs of this case, but we really hope our simulation results can be captured into TR to provide evidence for further study. |

#### Issue#3-6 (Medium priority) 5% UPT, full buffer

Moderator note[Rd3]: A similar “note” is added to mark the company with biased results. In addition, some values wrongly captured in the last round are corrected.

Proposed Observation 3.2.3: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer,

- For Max rank 1, 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~20.9%

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.5%~20.9% at CSI overhead A (small overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.3%~17.4% at CSI overhead B (medium overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~6.62% at CSI overhead C (large overhead);

- For Max rank 2, 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~14.9%

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 4.1%~14.9% at CSI overhead A (small overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 0.3%~4% at CSI overhead B (medium overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~6.03% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 8.76%~30.17%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Gain of CSI comporession over R16 Type II benchmark – 5% UPT, full buffer

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| Nokia #1 | A | 2.5% | 4.1% |  |
|  | B | 2.3% | 0.3% |  |
|  | C | 0% | 2.11% |  |
| vivo#1 | A | 8.81% | 13.8% |  |
|  | B | 8.66% | 3.05% |  |
|  | C | 6.62% | 6.03% |  |
| Fujitsu#1 | A | 20.9% | 14.90% |  |
|  | B | 17.4% | 3.50% |  |
|  | C | 6.1% | -7% |  |
| Xiaomi#1 | A |  | 30.17% |  |
|  | B |  | 21% |  |
|  | C |  | 8.76% |  |
| QC#1 | A |  | 9% |  |
|  | B |  | 3% |  |
|  | C |  | 0 |  |
| Intel#1 | A |  | 5% |  |
|  | B |  | 4% |  |
|  | C |  | 3% |  |
| ZTE#1 | A |  |  | 5.36% |
|  | B |  |  | 4.72% |
|  | C |  |  | 3.59% |
| Summary | A | 2.5%~20.9% | 4.1%~14.9% | 5.36% |
|  | B | 2.3%~17.4% | 0.3%~4% | 4.72% |
|  | C | 0%~6.62% | -7%~6.03% | 3.59% |

| Support/Can accept | Qualcomm, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | The same comment as Issue#3-5. |

#### Issue#3-7 (Low priority) CSI overhead reduction

Moderator note[Rd3]: Some editorial updates.

Proposed Observation 3.2.4: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of CSI feedback reduction,

- For Max rank = 1,

  - For CSI overhead A (small overhead), 1 source [ZTE] observes the CSI feedback reduction of 10.24% for FTP traffic;

  - For CSI overhead B (medium overhead), 2 sources [Huawei, ZTE] observe the CSI feedback reduction of 15.62%~50% for FTP traffic, and 1 source [Huawei] observes the CSI feedback reduction of 37% for full buffer;

  - For CSI overhead C (large overhead), 2 sources [Huawei, ZTE] observe the CSI feedback reduction of 14.37%~55% for FTP traffic, and 1 source [Huawei] observes the CSI feedback reduction of 50% for full buffer;

- For Max rank = 2,

  - For CSI overhead A (small overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 20.83%~54% for FTP traffic, and 1 source [Qualcomm] observes the CSI feedback reduction of 56% for full buffer;

  - For CSI overhead B (medium overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 22.22%~52% for FTP traffic, and 2 sources [Huawei, Qualcomm] observe the CSI feedback reduction of 52% for full buffer;

  - For CSI overhead C (large overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 10%~58.33% for FTP traffic, and 2 sources [Huawei, Qualcomm] observe the CSI feedback reduction of 22%~54% for full buffer;

- For Max rank = 4,

  - For CSI overhead A (small overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 50%~79% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 70.53% for full buffer;

  - For CSI overhead B (medium overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 36.10%~78% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 47.74% for full buffer;

  - For CSI overhead C (large overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 8%~58% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 42.59% for full buffer;

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is CSI overhead reduction5% UPT for Max rank 1/2/4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Gain of CSI comporession over R16 Type II benchmark – CSI overhead reduction

|  | RU | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- | --- |
| HW#1 (FTP) | <=39% | A |  |  |  |
|  |  | B | 38% |  |  |
|  |  | C | 47% |  |  |
|  | 40%-69% | A |  |  |  |
|  |  | B | 39% |  |  |
|  |  | C | 55% |  |  |
|  | >=70% | A |  |  |  |
|  |  | B | 50% |  |  |
|  |  | C | 47% |  |  |
| ZTE#1 (FTP) | <=39% | A |  |  |  |
|  |  | B |  |  |  |
|  |  | C |  |  |  |
|  | 40%-69% | A |  | 48.60% | 56.01% |
|  |  | B |  | 39.80% | 36.10% |
|  |  | C |  | 38.66% | 38.14% |
|  | >=70% | A | 10.24% | 44.44% | 60.29% |
|  |  | B | 15.62% | 38.12% | 36.41% |
|  |  | C | 14.37% | 36.47% | 37.93% |
| QC#1 (FTP) Option 2-1 | <=39% | A |  | 52% | 67% |
|  |  | B |  | 46% | 70% |
|  |  | C |  | 29% | 8% |
|  | 40%-69% | A |  | 51% | 57% |
|  |  | B |  | 49% | 61% |
|  |  | C |  | 10% | 12% |
|  | >=70% | A |  | 54% | 56% |
|  |  | B |  | 52% | 62% |
|  |  | C |  | 12% | 21% |
| QC#2 (FTP) Option 3-1 | <=39% | A |  |  | 50% |
|  |  | B |  |  | 61% |
|  |  | C |  |  | 13% |
|  | 40%-69% | A |  |  | 65% |
|  |  | B |  |  | 67% |
|  |  | C |  |  | 41% |
|  | >=70% | A |  |  | 79% |
|  |  | B |  |  | 78% |
|  |  | C |  |  | 58% |
| Futurewei#1 (FTP) | <=39% | A |  |  |  |
|  |  | B |  |  |  |
|  |  | C |  |  |  |
|  | 40%-69% | A |  |  |  |
|  |  | B |  |  |  |
|  |  | C |  |  |  |
|  | >=70% | A |  | 20.83% |  |
|  |  | B |  | 22.22% |  |
|  |  | C |  | 58.33% |  |
| HW#2 (FB) |  | A |  |  |  |
|  |  | B | 37% | 52% |  |
|  |  | C | 50% | 54% |  |
| QC#2 (FB) |  | A |  | 56% |  |
|  |  | B |  | 52% |  |
|  |  | C |  | 22% |  |
| ZTE#2 (FB) |  | A |  |  | 70.53% |
|  |  | B |  |  | 47.74% |
|  |  | C |  |  | 42.59% |
| Sumamry- FTP | <=39% | A |  | 52% | 50%~67% |
|  |  | B | 38% | 46% | 61%~70% |
|  |  | C | 47% | 29% | 8%~13% |
|  | 40%-69% | A |  | 20.83%~54% | 56.01%~65% |
|  |  | B | 39% | 22.22%~52% | 36.10%~67% |
|  |  | C | 55% | 10%~58.33% | 12%~41% |
|  | >=70% | A | 10.24% | 44.44% | 56%~79% |
|  |  | B | 15.62%~50% | 38.12% | 36.41%~78% |
|  |  | C | 14.37%~47% | 36.47% | 21%~58% |
| Summary-FTP |  | A | 10.24% | 20.83%~54% | 50%~79% |
|  |  | B | 15.62%~50% | 22.22%~52% | 36.10%~78% |
|  |  | C | 14.37%~55% | 10%~58.33% | 8%~58% |
| Summary-FB |  | A |  | 56% | 70.53% |
|  |  | B | 37% | 52% | 47.74% |
|  |  | C | 50% | 22%~54% | 42.59% |

| Support/Can accept | Qualcomm, Futurwei (with comments) |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Futurewei | Our result for VQ is missing from the table, in which the overhead reduction for “B” (compared with155 bits in Rel-16 Type II codebook) is 83% using LUT-based approach. |

#### Issue#3-8 (High priority) Quantization method-training awareness cases

Moderator note[Rd3]: Apple results added. The reference of company names reformulated.

| Source#1: Qualcomm | Source#2: vivo | Source#3: Ericsson | Source#4: ZTE | Source#5: Xiaomi |
| --- | --- | --- | --- | --- |
| Source#6: Fujitsu | Source#7: Huawei | Source#8: Apple |  |  |

Proposed Observation 3.2.5: For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, training non-aware quantization (Case 1) is in general inferior to the training aware quantization (Case 2-1/2-2), and may lead to lower performance than the benchmark.

- For scalar quantization, compared with benchmark,

  - -5.9%~-43.2% degradations are observed for training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8].

  - 3.9%~8.64% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 5 sources [Source#1, Source#2, Source#3, Source#4, Source#5], which are 17.3%~83.2% gains over training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8] and 0.9%~5.4% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

    - Note: 0.72% gains are observed for Case 2-1 from 1 source [Source#1] due to inappropriate SQ parameter chosen, which achieves 13.9% gains over Case 1.

  - 7.55% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 1 source [Source#1], which are 21.6% gains over training non-aware quantization (Case 1) from 1 source [Source#1].

- For vector quantization, compared with benchmark,

  - -2%~-10% degradations are observed for training non-aware quantization (Case 1) from 1 source [Source#7].

  - 6.0%~8.91% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 2 sources [Source#1, Source#2], which are 16.3%~23.1% gains over training non-aware quantization (Case 1) from 2 sources [Source#1, Source#2].

  - 4.67%~13.01% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 6 sources [Source#1, Source#2, Source#4, Source#5, Source#7, Source#8], which are 10.7%~27.8% gains over training non-aware quantization (Case 1) from 3 sources [Source#1, Source#2, Source#7] and 1.7%~7.5% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

  - In general, Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains, as observed by 4 sources [Source#1, Source#2, Source#6, Source#8].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: Qualcomm (R1-2305328); Source#2: vivo (R1-2304471); Source#3: Ericsson (R1-2304521); Source#4: ZTE (R1-2304534); Source#5: Xiaomi (R1-2304893); Source#6: Fujitsu (R1-2304764); Source#7: Huawei, HiSilicon (R1-2304653); Source#8: Apple (R1-2305234).

  - Note: Results refer to Table x of R1-230xxxx

Table X. Quantization method of CSI comporession – training awareness

|  |  |  | Case 1 | Case 2-1 | Case 2-2 |
| --- | --- | --- | --- | --- | --- |
| QC#1~QC#7 | SQ | X |  |  |  |
|  |  | Y | 0.648 (-11.56%) | config#1: 0.796 (8.64%) 22.8% gain over Case 1 config#2: 0.738 (0.72%) 13.9% gain over Case 1 | 0.788 (7.55%) 21.6% gain over Case 1 |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  | 0.798 (8.91%) 23.1% gain over Case 1 | VQ-dim2: 0.823 (12.32%) 27% gain over Case 1 3.1% gain over Case 2-1 VQ-dim4: 0.828 (13.01%) 27.8% gain over Case 1 3.8% gain over Case 2-1 |
|  |  | Z |  |  |  |
| HW#1/#5 | SQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X | 0.69 (-2%) |  | 0.767 (8.8%) 11.1% gain over Case 1 |
|  |  | Y | 0.738 (-10%) |  | 0.861 (4.5%) 16.7% gain over Case 1 |
|  |  | Z | 0.829 (-4%) |  | 0.918 (6.5%) 10.7% gain over Case 1 |
| vivo#1~#5 | SQ | X |  |  |  |
|  |  | Y | 0.735 (-5.9%) | 0.862 (6.7%) 17.3% gain over Case 1 |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  | 0.855 (6.0%) 16.3% gain over Case 1 | VQ-config#1: 0.871 (7.6%) 18.5% gain over Case 1 1.9% gain over Case 2-1 VQ-config#2: 0.866 (7.1%) 17.8% gain over Case 1 1.3% gain over Case 2-1 |
|  |  | Z |  |  |  |
| ZTE#1/#5 | SQ | X |  | L1: 0.7283(6.01%) L2: 0.5935(7.17%) |  |
|  |  | Y |  | L1: 0.8118(5.37%) L2: 0.6994(7.14%) |  |
|  |  | Z |  | L1: 0.9040(5.47%) L2: 0.8380(8.42%) |  |
|  | VQ | X |  |  | L1: 0.7191(4.67%) L2: 0.5866(5.92%) |
|  |  | Y |  |  | L1: 0.8015(4.04%) L2: 0.6913(5.90%) |
|  |  | Z |  |  | L1: 0.9123(6.44%) L2: 0.8498(9.95%) |
| Xiaomi#1/#2 | SQ | X |  |  |  |
|  |  | Y |  | 0.7533(7.31%) |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  |  | 0.7591(8.13%) |
|  |  | Z |  |  |  |
| E///#1/#2 | SQ | X | 0.410 (-43.2%) | 0.751(3.9%) 83.2% gain over Case 1 |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
| Nokia | SQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  | 1.2% (52 bits) |
|  |  | Y |  |  | 2.9% (104 bits) |
|  |  | Z |  |  | 1.5% (256 bits) |
| Fujitsu | SQ | X | 0.729(5.85%) | 0.7684(11.57%) 5.4% gain over Case 1 |  |
|  |  | Y | 0.8135(4.2%) | 0.8431(7.99%) 3.6% gain over Case 1 |  |
|  |  | Z | 0.9258(4.84%) | 0.9343(5.8%) 0.9% gain over Case 1 |  |
|  | VQ | X |  |  | 0.784(13.83%) 7.5% gain over Case 1 2% gain over Case 2-1 |
|  |  | Y |  |  | 0.8713(11.6%) 7.1% gain over Case 1 3.3% gain over Case 2-1 |
|  |  | Z |  |  | 0.9411(6.57%) 1.7% gain over Case 1 0.7% gain over Case 2-1 |
| Apple | SQ | X | 0.513 (-26%) | 0.722(4.49%) 26% gain over Case 1 | N/A |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X | 0.709 (2.6%) (VQ codebook is searched separately using traditional methods, based on latent space distribution, after encoder/decoder is trained without quantization) | 0.73(5.64%) 3% gain over Case 1 | 0.735 (6.37%) 3.66% gain over Case 1; 0.7% gain over Case 2-1 |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
| Summary; gain over benchmark, gain over Case 1 | SQ | X | -43.2%; 5.85% | 3.9%~11.57%, Signf: 83.2%; Modrt: 5.4% |  |
|  |  | Y | -5.9%~-11.56%; 4.2% | 5.37%~8.64%, Signf: 17.3%~22.8%; Modrt: 3.6% | 7.55%, 21.6% |
|  |  | Z | 4.84% | 5.47%~5.8%, 0.9% |  |
|  | VQ | X | -2% | 6.01% | 4.67%~8.8% 7.5% |
|  |  | Y | -10% | 6.0%~8.91%, 16.3%~23.1% | 4.04%~13.01%, Modrt: 7.1%; Signf:16.7%~27.8% |
|  |  | Z | -4% | N/A | 6.44%~6.5%, Modrt: 1.7%; Signf:10.7% |
| Summary; gain over benchmark, gain over Case 1 | SQ | X/Y/Z | -5.9%~-43.2% (QC, vivo, E///, Apple) 4.2%~5.85% (Fujitsu) | 3.9%~8.64% (QC, vivo, ZTE, Xiaomi, E///), Signf:17.3%~83.2% (QC, vivo, E///, Apple) Modrt:0.9%~5.4% (Fujitsu, Apple) | 7.55%(QC), 21.6%(QC) |
|  | VQ | X/Y/Z | -2%~-10% (HW) 2.6% (Apple) | 6.0%~8.91% (QC, vivo), 16.3%~23.1% (QC, vivo) | 4.67%~13.01% (QC, vivo, HW, ZTE, Xiaomi, Apple), Signf:10.7%~27.8% (QC, vivo, HW), Modrt:1.7%~7.5% (Fujitsu, Apple), 0.7%~3.8% gain over Case 2-1(QC, vivo, Fujitsu, Apple) |

| Support/Can accept | Qualcomm (comment), vivo, LG Electronics, ZTE(comment), Ericsson |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | @Apple @Fujitsu Can you clarify the reason why the gain of Case 2-1 over Case 1 is lower than other companies? |
| ZTE | Thanks FL’s summary. For the last sub-bullet in the 2nd bullet, we also observe that Case 2-2 has about 1% gain over Case 2-1 for CSI payload size Z, so could you add ZTE’s observation in the summary? Thanks. - For vector quantization, compared with benchmark, - In general, Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains, as observed by 4 5 sources [Source#1, Source#2, Source#4, Source#6, Source#8]. |
| Xiaomi | Thanks FL’s summary. For the last sub-bullet in the 2nd bullet, we also observe that Case 2-2 has about 0.82% gain over Case 2-1 for CSI payload size Y, so could you add Xiaomi’s observation in the summary? Thanks. - For vector quantization, compared with benchmark, In general, Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains, as observed by 6 sources [Source#1, Source#2, Source#4, Source#5, Source#6, Source#8]. |
| Qualcomm | In the new note related to our results, we request to replace “due to inappropriate SQ parameter chosen” with “because the SQ parameter is not optimized based on the distribution of the latent vector values”. |

#### Issue#3-9 (High priority) Quantization method-quantization format

Moderator note[Rd3]: Apple results updated. The reference of company names reformulated.

| Source#1: vivo | Source#2: Qualcomm | Source#3: Apple | Source#4: Lenovo | Source#5: ZTE |
| --- | --- | --- | --- | --- |
| Source#6: Xiaomi |  |  |  |  |

Proposed Observation 3.2.6:  For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, in general vector quantization (VQ) has comparable performance with scalar quantization (SQ):

- For SQ and VQ under the same training case, it is observed by 1 source [Source#1] that VQ under Case 2-1 has -0.8% degradation over SQ under Case 2-1, observed by 1 source [Source#2, Source#3] that VQ under Case 2-1 has 0.3%~1.1% gain over SQ under Case 2-1, and observed by 3 sources [Source#2, Source#3, Source#4] that that VQ under Case 2-2 has 0.7%~5.1% gain over SQ under Case 2-2.

  - Note: VQ under Case 2-1 has 8% gains over SQ under Case 2-1 as observed from 1 source [Source#2] due to inappropriate SQ parameter chosen.

- For SQ and VQ across training cases, it is observed by 5 sources [Source#1, Source#2, Source#3, Source#5, Source#6] that VQ under Case 2-2 has 0.5% ~4% gain over SQ under Case 2-1, and observed by 1 source [Source#5] that VQ under Case 2-2 has -1.3% degradation over SQ under Case 2-1.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: vivo (R1-2304471); Source#2: Qualcomm (R1-2305328); Source#3: Apple (R1-2305234); Source#4: Lenovo (R1-2305202); Source#5: ZTE (R1-2304534); Source#6: Xiaomi (R1-2304893);.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Quantization method of CSI comporession – quantization format

|  |  |  | Case 2-1 | Case 2-2 |
| --- | --- | --- | --- | --- |
| QC#1~QC#7 | SQ | X |  |  |
|  |  | Y | config#1: 0.796 (8.64%) config#2: 0.738 (0.72%) 13.9% gain over Case 1 | 0.788 (7.55%) |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y | 0.798 (8.91%) 0.3% gain over Case 2-1-SQ | VQ-dim2: 0.823 (12.32%) 3.4% gain over Case 2-1-SQ 4.4% gain over Case 2-2-SQ VQ-dim4: 0.828 (13.01%) 4% gain over Case 2-1-SQ 5.1% gain over Case 2-2-SQ |
|  |  | Z |  |  |
| vivo#1~#5 | SQ | X |  |  |
|  |  | Y | 0.862 (6.7%) |  |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y | 0.855 (6.0%) -0.8% gain over Case 2-1-SQ | VQ-config#1: 0.871 (7.6%) 1% gain over Case 2-1-SQ VQ-config#2: 0.866 (7.1%) 0.5% gain over Case 2-1-SQ |
|  |  | Z |  |  |
| ZTE#1/#5 | SQ | X | L1: 0.7283(6.01%) L2: 0.5935(7.17%) |  |
|  |  | Y | L1: 0.8118(5.37%) L2: 0.6994(7.14%) |  |
|  |  | Z | L1: 0.9040(5.47%) L2: 0.8380(8.42%) |  |
|  | VQ | X |  | L1: 0.7191(4.67%) -1.3% gain over Case 2-1-SQ L2: 0.5866(5.92%) |
|  |  | Y |  | L1: 0.8015(4.04%) -1.3% gain over Case 2-1-SQ L2: 0.6913(5.90%) |
|  |  | Z |  | L1: 0.9123(6.44%) 1% gain over Case 2-1-SQ L2: 0.8498(9.95%) |
| Xiaomi#1/#2 | SQ | X |  |  |
|  |  | Y | 0.7533(7.31%) |  |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y |  | 0.7591(8.13%) 0.8% gain over Case 2-1-SQ |
|  |  | Z |  |  |
| Nokia | SQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  |  |
|  | VQ | X |  | 1.2% (52 bits) |
|  |  | Y |  | 2.9% (104 bits) |
|  |  | Z |  | 1.5% (256 bits) |
| Lenovo#1/#2 | SQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  | 0.844 |
|  | VQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  | 0.882 0.8% gain over Case 2-2-SQ |
| Apple | SQ | X | 0.722 (2.6%) |  |
|  |  | Y |  |  |
|  |  | Z |  |  |
|  | VQ | X | 0.73 (5.64%) 1.1% gain over Case 2-1-SQ | 0.735 (6.37%) 1.8% gain over Case 2-1-SQ 0.7% gain over Case 2-2-SQ |
|  |  | Y |  |  |
|  |  | Z |  |  |
| Summary | X/Y/Z |  | Case 2-1-VQ has -0.8% loss over Case 2-1-SQ [vivo], 0.3%~1.1% gain over Case 2-1-SQ [QC, Apple] | Case 2-2-VQ has 0.5% ~4% gain over Case 2-1-SQ [QC, vivo, ZTE, Xiaomi Apple] Case 2-2-VQ has 0.7% ~5.1% gain over Case 2-2-SQ [Lenovo, QC, Apple] Case 2-2-VQ has -1.3% loss over Case 2-1-SQ [ZTE] |

| Support/Can accept | ZTE, vivo |
| --- | --- |
| Object/Concern | Qualcomm (comment) |

| Company | View |
| --- | --- |
| Lenovo | As we have reported the results for the case of SQ+VQ we suggest to add the following to the observation as well: - It is observed by 1 sources [Source#4] that under Case 2-2 VQ+SQ outperforms only VQ and only SQ by ~0.2% and ~4% gain, respectivly. |
| Qualcomm | In the new note related to our results, we request to replace “due to inappropriate SQ parameter chosen” with “because the SQ parameter is not optimized based on the distribution of the latent vector values”. As for the overall conclusion, the summary seems to show that more companies show a gain compared to a loss in all the cases. This does not seem to justify concluding “comparable performance”. Could we say “VQ has a small gain over SQ”? |

#### Issue#3-10 (High priority) High resolution ground-truth CSI for training

Moderator note[Rd3]: Apple results updated. The reference of company names reformulated.

| Source#1: vivo | Source#2: Qualcomm | Source#3: Huawei | Source#4: ZTE | Source#5: Apple |
| --- | --- | --- | --- | --- |

Proposed Observation 3.2.7:  For the evaluation of high resolution quantization of the ground-truth CSI for CSI compression, till the RAN1#113 meeting, compared to the upper-bound of Float32, quantized high resolution ground-truth CSI can achieve significant overhead reduction with minor performance loss if the parameters are appropriately selected.

- For high resolution scalar quantization,

  - Float16 achieves 50% overhead reduction and -0.6% or less performance loss from 2 sources [Source#1, Source#5]

  - 8 bits scalar quantization achieves 75% overhead reduction and -0.1%~-0.9% performance loss from 2 sources [Source#3, Source#5]

- For high resolution R16 eType II-like quantization, R16 eType II CB with new parameters can outperform that with legacy parameters (e.g., PC1~PC8) with slight increase of overhead:

  - R16 eType II-like quantization with legacy PC5 achieves 99.1% overhead reduction, and -6% loss at CSI payload Y (medium payload) from 1 source [Source#1].

  - R16 eType II-like quantization with new parameter assumptions [set#2] which achieves 91.2% overhead reduction, has -2% loss to Float32 and 4.3% gain over legacy PC5 at CSI payload Y (medium payload) from 1 source [Source#1]

  - R16 eType II-like quantization with legacy PC8 achieves 98.8% overhead reduction, and

    - -4.9%~-0.7% loss at CSI payload X (small payload) from 3 sources [Source#2, Source#3, Source#4],

    - -2.9% loss at CSI payload Y (medium payload) from 1 source [Source#3],

    - -6.6%~-3.1% loss at CSI payload Z (large payload) from 2 sources [Source#3, Source#4].

    - Note: it is pointed out by 1 source [Source#2] that using R16 eType II-like quantization with legacy PC may achieve close performance to Float32 by special data handling.

  - R16 eType II-like quantization with new parameter assumptions [set#1/3/4] which achieve 96%~97.5% overhead reduction, have

    - -2.4%~0% loss to Float32 and 1%~1.5% gain over legacy PC8, at CSI payload X (small payload) from 2 sources [Source#3, Source#4]

    - -0.7% loss to Float32 and 2.6% gain over legacy PC8, at CSI payload Y (medium payload) from 1 source [Source#3]

    - -0.7%~0% loss to Float32 and 3.4%~6.4% gain over legacy PC8, at CSI payload Z (large payload) from 2 sources [Source#3, Source#4]

- Note: the new parameters include at least one from the follows:

  - L= 8, 10, 12;

  - pv = 0.75, 0.9;

  - beta = 1;

  - reference amplitude = 6 bits; differential amplitude = 4bits; phase = 6 bits;

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1/2.

  - Source#1: vivo (R1-2304471); Source#2: Qualcomm (R1-2305328); Source#3: Huawei, HiSilicon (R1-2304653); Source#4: ZTE (R1-2304534); Source#5: Apple (R1-2305234).

  - Note: Results refer to Table x of R1-230xxxx

Table X. Quantization method of CSI comporession – High resolution ground-truth CSI format

|  |  | QC#8/#9 | HW#1~#4 | vivo#4~#7 | ZTE#1/3~5 | Apple |
| --- | --- | --- | --- | --- | --- | --- |
| Float32 | X | 0.735 (13.6%) | 0.767 (8.8%) |  | L1: 0.7191(4.67%) L2: 0.5866(5.92%) | 0.726(5.19%) |
|  | Y |  | 0.861 (4.5%) | 0.871 (7.6%) |  |  |
|  | Z |  | 0.918 (6.5%) |  | L1: 0.9123(6.44%) L2: 0.8498(9.95%) |  |
| Scalar quantz: 8 bits (75% OH reduction) | X |  | 0.762 (8.1%) -0.5% loss |  |  | 0.725(4.9%) -0.1% loss |
|  | Y |  | 0.852 (3.4%) -0.9% loss |  |  |  |
|  | Z |  | 0.911 (5.7%) -0.7% loss |  |  |  |
| Float16 (50% OH reduction) | X |  |  |  |  | 0.727(5.29%) |
|  | Y |  |  | 0.866 (7.1%) -0.6% loss |  |  |
|  | Z |  |  |  |  |  |
| R16 Type II-like CB: Legacy resolution (PC5) (99.1% OH reduction) | X |  |  |  |  |  |
|  | Y |  |  | 0.819 (2.4%) -6% loss |  |  |
|  | Z |  |  |  |  |  |
| R16 Type II-like CB: Legacy resolution (PC8) (98.8% OH reduction) | X | 0.726 (12.2%) -1.2% loss | 0.76 (7.8%) -0.7% loss |  | L1: 0.6913(0.63%) -3.9% loss L2: 0.5579(0.74%) -4.9% loss |  |
|  | Y |  | 0.832 (1%) -2.9% loss |  |  |  |
|  | Z |  | 0.887 (2.9%) -3.1% loss |  | L1: 0.8691(1.40%) -4.7% loss L2: 0.7933(2.64%) -6.6% loss |  |
| ParaSet#1: R16 Type II-like CB: High resolution (>legacy PC8) L=10, p=0.9, beta=0.31; Ref-amp: 6 bits, Diff-amp: 4 bits, phase: 6 bits (96% OH reduction) | X |  | 0.768 (8.9%) +0% 1% gain on PC8 |  |  |  |
|  | Y |  | 0.854 (3.6%) -0.7% loss; 2.6% gain on PC8 |  |  |  |
|  | Z |  | 0.917 (6.4%) -0.1% loss; 3.4% gain over PC8 |  |  |  |
| ParaSet#2: R16 Type II-like CB: High resolution (>legacy PC8) L=12, M=6, beta=1.0 (91.2% OH reduction) | X |  |  |  |  |  |
|  | Y |  |  | 0.854 (5.9%) -2% loss; 4.3% gain over PC5 |  |  |
|  | Z |  |  |  |  |  |
| ParaSet#3: R16 Type II-like CB: High resolution (>legacy PC8) L=8, p_v=0.75, β=0.31 (97.5% OH reduction) | X |  |  |  | L1: 0.7015(2.11%) -2.4% loss; 1.5% gain over PC8 L2: 0.5834(5.34%) -0.5% loss; 4.6% gain over PC8 |  |
|  | Y |  |  |  |  |  |
|  | Z |  |  |  | L1: 0.9063(5.74%) -0.7% loss; 4.3% gain over PC8 L2: 0.8440(9.20%) -0.7% loss; 6.4% gain over PC8 |  |
| ParaSet#4: R16 Type II-like CB: High resolution (>legacy PC8) L=10, p_v=0.9, β=0.31 (96.2% OH reduction) | X |  |  |  | L1: 0.7147(4.03%) -0.6% loss; 3.4% gain over PC8 L2: 0.5865(5.90%) -0% loss; 5.1% gain over PC8 |  |
|  | Y |  |  |  |  |  |
|  | Z |  |  |  | L1: 0.9163(6.91%) +0.4% gain L2: 0.8586(10.19%) 1% gain |  |

| Support/Can accept | ZTE, vivo, Ericsson |
| --- | --- |
| Object/Concern | Qualcomm (comment) |

| Company | View |
| --- | --- |
| Qualcomm | We still do not support the second observation that eType II CB can outperform legacy PC. Comparison with PC5 should not be used to draw this conclusion. Even with legacy PC8, the performance is similar to the floating point performance, even at high payload settings, as shown in our contribution. This can be achieved using dithering to avoid the effect of quantization. For example, with a payload size of 512 bits, PC8 quantized training dataset can achieve SGCS of 0.94 as compared to floating point training dataset performance of 0.95. Hence the gap to eType II CB with new parameters will also be small. |

#### Issue#3-11a (High priority) Rank>1 solutions-down selection

Moderator note[Rd3]: No Change.

Proposed Conclusion 3.2.8: For the evaluation of CSI compression, for the rank>1 solutions, prioritize Option 2-1 (layer specific and rank common), Option 3-1 (layer common and rank common), and Option 3-2 (layer common and rank specific).

- Note: For the evaluations of CSI compression with 1-on-1 joint training,

  - Option 1-2 is not adopted;

  - Option 2-2 is not adopted;

  - Over the 11 sources [Huawei, Ericsson, Nokia, ZTE, OPPO, Intel, Fujitsu, NTT DOCOMO, InterDigital, Qualcomm, Futurewei] for FTP/fullbuffer, Max Rank 2,

    - Option 1-1 is adopted by 1 source [ZTE]

    - Option 2-1 is adopted by 3 sources [Ericsson, Intel, Qualcomm]

    - Option 3-1 is adopted by 8 sources [Huawei, Nokia, ZTE, OPPO, Fujitsu, Futurewei, Xiaomi, vivo]

    - Option 3-2 is adopted by 2 sources [NTT DOCOMO, InterDigital]

  - Over the 4 sources [Qualcomm, ZTE, CATT, Apple] for FTP/fullbuffer, Max Rank 4,

    - Option 2-1 is adopted by 1 source [Qualcomm]

    - Option 3-1 is adopted by 2 sources [Qualcomm, ZTE]

    - Option 3-2 is adopted by 1 source [Apple]

| Support/Can accept | vivo, LG Electronics, NTT DOCOMO(w/ comments), OPPO, ZTE, Futurewei, Xiaomi, Ericsson |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | We are supportive of Option 3-1, but we are open to Option 2-1 and Option 3-2. |
| LG Electronics | We prefer option 3-1. |
| NTT DOCOMO | We also provided the results on Option 3-1 in Table 3. We are open to both Option 3-1 and 3-2. |
| OPPO | Supportive of option 3-1, also open to option 2-1 and option 3-2. |
| Qualcomm | It would be good to clarify whether the prioritization is for evaluation, capturing results, or for specification impact discussion. If it is for specification impact, then it is not clear how we can decide not to discuss specification impact based on the fact that it was not evaluated by any company. |

#### Issue#3-14 (Medium priority) Generalization-Carrier frequencies

Moderator note[Rd3]: No change on the issue. Please provide more inputs!

Proposed Observation 3.2.9: For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B,

- For generalization Case 2, generalized performance may be achieved in general

  - If carrier frequency#A is 3.5/4GHz & carrier frequency#B is 2GHz, 3 sources [NTT DOCOMO, Nokia, MTK] observe generalized performance of less than -1.72% degradation.

  - If carrier frequency#A is 2GHz & carrier frequency#B is 3.5/4GHz, 4 sources [NTT DOCOMO, Nokia, vivo, MTK] observe generalized performance of less than -1% degradation or positive gain,

    - Note: 1 source [Nokia] observes significant degradations of -6.6%.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-0.8% loss or positive gain) for carrier frequency#B subject to any of 2GHz and 3.5/4GHz, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B, as observed by 4 sources [NTT DOCOMO, Nokia, vivo, MTK].

  - Minor loss (0%~-0.8%) are observed by 3 sources [Nokia, vivo, MTK].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, vivo, MTK].

  - Note: Significant degradations of up to -4.9% are still observed by 1 source [Nokia] for deployment scenario#B subject to 3.5/4GHz

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Antenna layouts are assumed as the same over the different frequency carriers.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Generalization of CSI compression– carrier frequencies

| Training, Generalization Case 1 |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | 3.5/4GHz |  |  | 2GHz |  |
| Training, Generalization Case 3 |  |  | 2GHz+3.5/4GHz | 5.5GHz+3.5/4GHz |  | 2GHz+3.5/4GHz | 5.5G+3.5/4GHz |  |
| Testing |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| vivo#1/#2 | Case 1 | X |  |  |  | 0.788 |  |  |
|  |  | Y |  |  |  | 0.847 |  |  |
|  |  | Z |  |  |  | 0.9013 |  |  |
|  | Case 2 | X |  |  |  |  | 0.782/-0.76% |  |
|  |  | Y |  |  |  |  | 0.838/-1% |  |
|  |  | Z |  |  |  |  | 0.8992/-0.23% |  |
|  | Case 3 | X |  |  |  | 0.788/0% | 0.789/0.127% |  |
|  |  | Y |  |  |  | 0.845/0.236% | 0.848/0.12% |  |
|  |  | Z |  |  |  | 0.9015/0.02% | 0.9002/-0.12% |  |
| NTT DOCOMO #1#2#3#4#5#6 | Case 1 | X | 0.58 |  |  | 0.58 |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  | -1.72% |  |  | 1.72% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X | 0%/0%/1.72% |  |  | 1.72%/1.72%/1.72% |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
| Nokia#1#2 | Case 1 | X | 0.715 |  |  | 0.691 |  |  |
|  |  | Y | 0.763 |  |  | 0.796 |  |  |
|  |  | Z | 0.886 |  |  | 0.880 |  |  |
|  | Case 2 | X |  | -0.8% |  |  | 0.1% |  |
|  |  | Y |  | 4.8% |  |  | -6.6% |  |
|  |  | Z |  | 0.1% |  |  | -0.9% |  |
|  | Case 3 | X | -0.1% |  |  | 0.3% |  |  |
|  |  | Y | 1.0% |  |  | -4.9% |  |  |
|  |  | Z | 0.1% |  |  | -0.3% |  |  |
| MTK#1#2 | Case 1 | X | 0.696 |  |  | 0.702 |  |  |
|  |  | Y | 0.771 |  |  | 0.773 |  |  |
|  |  | Z | 0.831 |  |  | 0.835 |  |  |
|  | Case 2 | X |  | 0.3% |  |  | -0.1% |  |
|  |  | Y |  | -0.5% |  |  | 0.5% |  |
|  |  | Z |  | -0.2% |  |  | 0.4% |  |
|  | Case 3 | X | 0.0% |  |  | -0.3% |  |  |
|  |  | Y | -0.8% |  |  | -0.3% |  |  |
|  |  | Z | -0.2% |  |  | 0.1% |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  | NTT DOCOMO, Nokia, MTK |  |  | Vivo, Nokia, MTK |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  |  | Nokia |  |
|  |  | Positive gain |  | MTK, Nokia |  |  | NTT DOCOMO, Nokia, MTK |  |
|  | Case 3 | Minor loss (>-1.6%) | Nokia, MTK |  |  | Nokia, MTK | Vivo |  |
|  |  | Significant loss (<-4%) |  |  |  | Nokia |  |  |
|  |  | Positive gain | NTT DOCOMO, Nokia |  |  | Vivo, NTT DOCOMO, Nokia, MTK | vivo |  |

| Support/Can accept | NTT DOCOMO, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-16 (Low priority) Scalability-Bandwidths

Moderator note[Rd3]: No change on the issue. Please provide more inputs!

Proposed Observation 3.2.10: For the scalability verification of AI/ML based CSI compression over various bandwidths, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain bandwidth#B and applied for inference with a same bandwidth#B,

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-2.5% loss or positive gain) for bandwidth#B, if the training dataset is constructed with data samples subject to multiple bandwidths including bandwidth#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, as observed by 3 sources [NTT DOCOMO, Nokia, QC].

  - Minor loss (0%~-1.75%) are observed by 2 sources [NTT DOCOMO, Nokia].

  - Moderate loss (-2.5%) are observed by 1 source [Nokia].

  - Positive gains are observed by 2 sources [Nokia, QC].

  - Note: Significant loss (-5.4%) is observed by 1 source [QC]

  - Note: Pre/post-processing of truncation/padding is adopted by 3 sources [NTT DOCOMO, Nokia, QC].

- Note: For generalization Case 2, 1 source [ZTE] observes generalized performance of 0%~-1.28% loss using proportional bandwidth and SB size.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Scalability of CSI compression – bandwidths

| Training, Generalization Case 1 |  |  | 10MHz/52RB |  | 20MHz | 5MHz | 48RB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  | 20MHz | 48RB | 10MHz | 10MHz |  |
| Training, Generalization Case 3 |  |  | 20MHz+10MHz | 52RB+48RB | 20MHz+10MHz |  | 52RB+48RB |
| Testing |  |  | 10MHz/52RB |  | 20MHz | 5MHz | 48RB |
|  | Case | CSI payload |  |  |  |  |  |
| Nokia#1/#2 | Case 1 | X | 0.618, 0.66 |  |  |  | 0.619, 0.677 |
|  |  | Y | 0.727 |  |  |  | 0.728 |
|  |  | Z | 0.808 |  |  |  | 0.806 |
|  | Case 2 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 3 | X |  | 2.2%, -0.1% |  |  | 2.7% -2.5% |
|  |  | Y |  | 1.2% |  |  | 0.9% |
|  |  | Z |  | -1.7% |  |  | -1.6% |
| ZTE#1/#2 | Case 1 | X | L1:0.7191 L2:0.5866 |  | L1:0.6926 L2:0.5495 |  |  |
|  |  | Y | L1:0.8033 L2:0.6913 |  | L1:0.7835 L2:0.6525 |  |  |
|  |  | Z | L1:0.8978 L2:0.8275 |  | L1:0.877 L2:0.7882 |  |  |
|  | Case 2 | X | L1:-0.47% L2:-1.28% |  | L1:0% L2:-0.27% |  |  |
|  |  | Y | L1:-0.22% L2:-0.04% |  | L1:-0.46% L2:-0.49% |  |  |
|  |  | Z | L1:-0.85% L2:-1.1% |  | L1:-0.64% L2:-0.44% |  |  |
|  | Case 3 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
| NTT DOCOMO#1/#2 | Case 1 | X | L1:0.73 L2:0.63 |  | L1:0.68 L2:0.57 |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 3 | X | L1:-1.37% L2:-1.59% |  | L1:-1.47% L2:-1.75% |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
| InterDigital#1 | Case 1 | X |  |  |  | L1:0.623 L2:0.426 |  |
|  |  | Y |  |  |  | L1:0.683 L2:0.525 |  |
|  |  | Z |  |  |  | L1:0.745 L2:0.56 |  |
|  | Case 2 | X |  |  |  | L1:-4.3% L2:-3.3% |  |
|  |  | Y |  |  |  | L1:-2.5% L2:-11.4 |  |
|  |  | Z |  |  |  | L1:-8.46% L2:-12.7 |  |
|  | Case 3 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
| QC#1 | Case 1 | X |  |  | 0.75 |  |  |
|  |  | Y |  |  | 0.825 |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 3 | X |  |  | 4.4% |  |  |
|  |  | Y |  |  | -5.4% |  |  |
|  |  | Z |  |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.6%) | ZTE |  | ZTE |  |  |
|  |  | Moderate/significant loss (<-1.6%) |  |  |  | Interdigital |  |
|  |  | Positive gain |  |  |  |  |  |
|  | Case 3 | Minor loss (>-1.8%) | NTT DOCOMO | Nokia | NTT DOCOMO |  | Nokia |
|  |  | Moderate/significant loss (<-1.8%) |  | Nokia | QC |  | Nokia |
|  |  | Positive gain |  | Nokia | QC |  | Nokia |

| Support/Can accept | NTT DOCOMO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-20 (Medium priority) NW first training-1 NW to 1/M>1 UE

Moderator note[Rd3]: Minor changes for the third round.

Proposed Observation 3.2.11: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone [or similar architecture] is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 4 sources [Ericsson, Nokia, ZTE，Fujitsu] observe -0%~-0.5% degradation, 4 sources [Nokia, QC, ZTE, vivo] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [HW, Apple, CMCC] observe -0%~-0.5% degradation.

- Note: For the NW first separate training case where different backbones [or substantially different architectures] are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [QC, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Note: Results refer to Table x of R1-230xxxx

Table X. NW first training for CSI compression -1 NW to 1/M>1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip |  |  |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.5% |  | HW, Apple, CMCC, |  |  |

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| Moderator | @Lenovo That is why the note is added: it only limits the results are from the one way dataset sharing (from the simulation assumption of majority companies). For other special handlings, we may have further observations. |

#### Issue#3-20a (Medium priority) NW first training-1 NW to 1/M>1 UE- Impact of size of shared training dataset

Moderator note[Rd3]: Minor changes for the third round.

Proposed Observation 3.2.12: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [HW, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table x of R1-230xxxx

Table X. NW first training for CSI compression -1 NW to 1/M>1 UE with various dataset settings

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) dataset size | Same backbone, same/diff dataset size | Diff backbone; Cap_UE model < Cap_NW model, same/diff dataset size |
| --- | --- | --- | --- | --- |
| HW#1/2 | Descrip | TF#A-TF#1, 300k | TF#A-TF#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.1%/-0.3% Gap: -0.2% |  |
|  | Z |  | 0%/-0.1% Gap: -0.1% |  |
|  | Descrip | CNN#A-CNN#1, 300k | CNN#A-CNN#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.4%/-0.5% Gap: -0.1% |  |
|  | Z |  | -0.3%/-0.5% Gap: -0.2% |  |
| CMCC#1~7 | Descrip | TF#A-TF#1, 154k | TF#A-TF#1, 154k/100k/50k/ 10k/5k |  |
|  | X |  |  |  |
|  | Y |  | -0.22%/-0.22%/-0.55%/-1.86%/-2.95% Gap: 0/-0.3%/-1.64%/-2.73% |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-CNN#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -4.04%/-4.59%/-5.90%/-8.30%/-9.61% Gap: -0.55/-1.86%/-4.26%/-5.55% |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-MLP#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -0.76%/-0.98%/-1.53%/-6.33%/-9.17% Gap: -0.22%/-0.77%/-5.57%/-8.41% |
|  | Z |  |  |  |
| vivo#1/2 | Descrip | TF#A-TF#1, 300k; TF#A-TF#2, 300k; | TF#A-TF#1, 300k/50k; TF#A-TF#2, 300k/50k; |  |
|  | X |  | -0.6%/-2.6%; -0.9%/-2.6% |  |
|  | Y |  |  |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-CNN#1, 300k |  | TF#A-CNN#1, 300k/50k |
|  | X |  |  | -5.2%/ -9.4% Gap: -3.8% |
|  | Y |  |  |  |
|  | Z |  |  |  |
| Summary: gap of additional loss | -0%~-0.55% |  | HW, CMCC, | CMCC, vivo |
|  | -0.55%~-8.41% |  |  |  |

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |

#### Issue#3-22 (Medium priority) UE first training-1/N>1 NW to 1 UE

Moderator note[Rd3]: Minor changes for the third round.

Proposed Observation 3.2.13: For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1/N>1 NW to 1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone [or similar architecture] is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 4 sources [Nokia, Fujitsu, CATT, vivo] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [HW] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones [or substantially different architectures] are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Note: Results refer to Table x of R1-230xxxx

Table X. UE first training for CSI compression-1/N>1 NW to 1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model > Cap_NW model | Diff backbone or largely diff structure; Cap_UE model < Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A/B/C-TF#1 | TF#A/B/C-TF#1 |  |  |
|  | X |  | 0%/-0.4%/-0.2% |  |  |
|  | Y |  | 0%/-0.6%/-0.2% |  |  |
|  | Z |  | 0%/-0.6%/-0.1% |  |  |
|  | Descrip | CNN#A/B-CNN#1 | CNN#A/B-CNN#1 |  | TF#A- CNN#1 |
|  | X |  | -0.6%/-0.6% |  | -0.3% |
|  | Y |  | -0.5%/-0.6% |  | 0% |
|  | Z |  | -0.5%/-0.7% |  | 0% |
| Nokia#3/4 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | +1.7%/-0.2% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A/B/C /D-TF#1 | TF#A/B/C /D-TF#1 |  |  |
|  | X |  | -1.8%/ -2.3%/ -2.6%/ -2.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: -1.13% L2: -1.55% |  |  |
|  | Y |  | L1: -1.07% L2: -1.52% |  |  |
|  | Z |  | L1: -1.05% L2: -1.75% |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  | L1: -0.99% L2: -1.06% |  |
|  | Y |  |  | L1: -0.98% L2: -1.88% |  |
|  | Z |  |  | L1: -0.74% L2: -0.92% |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  | L1: -0.85% L2: -0.73% |
|  | Y |  |  |  | L1: -0.84% L2: -1.74% |
|  | Z |  |  |  | L1: -0.67% L2: -0.78% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.03% |  |  |
|  | Y |  | -0.31% |  |  |
|  | Z |  | -0.14% |  |  |
|  | Descrip | TF#A-TF#1 |  | CNN #A-TF#1 |  |
|  | X |  |  | -5.82% |  |
|  | Y |  |  | -5.51% |  |
|  | Z |  |  | -4.18 % |  |
| CATT#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.14%/ 0.21% |  |  |
|  | Y |  | -0.23%/-0.27% |  |  |
|  | Z |  | -0.14%/-0.35% |  |  |
|  | Descrip | CNN#A-TF #1 |  | CNN#A-TF #1 |  |
|  | X |  |  | 1.07% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -0.90% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +5.5%; +1.3%/ +2.7%; +17.1%/ +9.2% |  |  |
|  | Descrip |  |  |  | TF#A/B-CNN#1; TF#A/B-CNN#2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A/B- TF#1; |  | CNN#A/B- TF#1; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | 0.0%/ -1.4% | +5.8%/ +7.4% |
| vivo#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.4%/ -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A- TF#1 |  | CNN#A- TF#1 |  |
|  | X |  |  | -1.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | Nokia, Fujitsu, CATT, vivo |  |  |
|  | -0.5%~-1% |  |  | CATT, ZTE | ZTE |
|  | -1%~-1.88% |  | ZTE | ZTE, vivo | ZTE |
| Summary (Before Q) | -0%~-0.5% |  | HW |  |  |
|  | -0.5%~-1% |  | HW |  |  |
|  | -1%~-2.9% |  | Apple, |  |  |

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Lenovo | We have also reported simulation results for the cases - When backbones are almost similar (Lenovo#1 Normal Type3) For the case where the shared input of the UE side CSI reconstruction part is after quantization, 4 sources [Nokia, Fujitsu, CATT, vivo, Lenovo] observe -0%~-0.5%-0.8% degradation - When backbones are very different (Lenovo#2 Normal Type3) If the backbone of the NW part model is less capable than the UE part model, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation and 1 sources [Lenovo] observe -1.2%~-4.2% degradation We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| Qualcomm | We ask the moderator to please clarify the comment that the format of our result is incorrect. |
| Moderator | @Lenovo Your results are submitted to Case 2 (dataset mixing case, e.g. M>1 UEs to 1 NW). The observation is for Case 1 (1 UE to 1 NW) @QC As there is only one NW part model in the assumptions, why are there results for two NWs? Why for layer 1 there are two results (two UEs?) and for other layers there is only one result? |
| Lenovo | If this proposal is related to Case-1, then we are okay with this. |
| Qualcomm | We thank the moderator for the explanation. Please find our responses to your questions below: As there is only one NW part model in the assumptions, why are there results for two NWs? - The result for layer 1 was indeed evaluated with two NWs. The detail was inadvertently missed. Please add “CNN” as the NW#2 part model backbone/structure, and "440k” as the NW#2 part training dataset description and size. Why for layer 1 there are two results (two UEs?) and for other layers there is only one result? - Yes, for layer 1, the two results are for two UEs. For the other layers, we only evaluated them for the single UE case. |

#### Issue#3-24 (High priority) Benchmark of joint training for comparison

Moderator note[Rd3]: No change on the issue. Please provide more inputs!

Proposal 3.2.14: For the evaluation of training Type 3 under CSI compression, for the benchmark case (1-on-1 joint training) for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared.

- E.g., if the Type 3 is Transformer#1 for NW part model and CNN#1 for UE part model, then the benchmark case for performance comparison is also Transformer#1 for NW part model and CNN#1 for UE part model with joint training.

| Support/Can accept | Fujitsu, vivo, ZTE, Xiaomi, NTT DOCOMO, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

## 4th round email discussions

#### Issue#3-2 (Closed) Down selection of model input/output

Moderator note[Rd4]: No change on top of the third round.

The intention of the proposal is due to the agreement on the study of the input/output types in 9.2.2.2; whether Option 2 is to be further studied depends on the evaluations in 9.2.2.1. As far as is known by Moderator, there is no company submitted results based on raw channel matrix, so it is proposed to prioritize Option 1 precoding matrix.

RAN1#112 9.2.2.2 Agreement In CSI compression using two-sided model use case, further study potential specification impact of the following output-CSI-UE and input-CSI-NW at least for Option 1: - Option 1: Precoding matrix - 1a: The precoding matrix in spatial-frequency domain - 1b: The precoding matrix represented using angular-delay domain projection - Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel) - 2a: raw channel is in spatial-frequency domain - 2b: raw channel is in angular-delay domain - Note: Whether Option 2 is also studied depends on the performance evaluations in 9.2.2.1. - Note: RI and CQI will be discussed separately

Proposal 3.2.1: For the evaluation of CSI compression, for the type of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part), prioritize precoding matrix for further studies eigenvector(s).

- Note: For the evaluations of CSI compression with 1-on-1 joint training, 21 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, Spreadtrum, Fujitsu, NTT DOCOMO, Xiaomi, Qualcomm, Intel, InterDigital, CATT, Apple, China Telecom, MediaTek, BJTU, ETRI, CMCC] take eigenvector(s) without angular-delay domain convertion as the model input/output; 1 source [Ericsson] takes eigenvectors with angular-delay domain representation as the model input/output. No company submitted explicit channel matrix as input.

| Support/Can accept | NTT DOCOMO, ETRI, vivo, LG Electronics, OPPO, Lenovo, ZTE, Futurewei, Xiaomi, Ericsson, Qualcomm |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Futurewei | We are ok to prioritize using precoding matrix considering there is only 1 meeting left for Rel-18. |

#### Issue#3-5 (High priority) Mean UPT, full buffer

Moderator note[Rd4]: ZTE results for rank 4 added as a note.

Proposed Observation 3.3.1: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11%

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 6%~11% at CSI overhead A (small overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11% at CSI overhead C (large overhead);

- For Max rank 2, 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~15%

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~15% at CSI overhead A (small overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~10% at CSI overhead B (medium overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~14% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 24.47%~28.24%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 7.44%~9.95% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table X of R1-230xxxx

Table X. Gain of CSI comporession over R16 Type II benchmark – Mean UPT, full buffer

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| HW#1 | A | 11% | 15% |  |
|  | B | 7% | 9% |  |
|  | C | 11% | 14% |  |
| Nokia #1 | A | 6.5% | 8.5% |  |
|  | B | 5.8% | 5% |  |
|  | C | 1.1% | 0.2% |  |
| vivo#1 | A | 7.92% | 11.15% |  |
|  | B | 6.02% | 7.09% |  |
|  | C | 9.53% | 10.28% |  |
| OPPO#1 | A | 6% |  |  |
|  | B | 3% |  |  |
|  | C | 3% |  |  |
| Fujitsu#1 | A | 9% | 9.2% |  |
|  | B | 5.6% | 7% |  |
|  | C | 2% | 13.7% |  |
| Xiaomi#1 | A |  | 24.47% |  |
|  | B |  | 28.24% |  |
|  | C |  | 27.36% |  |
| QC#1 | A |  | 8% |  |
|  | B |  | 10% |  |
|  | C |  | 3% |  |
| Intel#1 | A |  | 7% |  |
|  | B |  | 9% |  |
|  | C |  | 7% |  |
| InterDigital#1 | A |  | 4% |  |
|  | B |  | 4% |  |
|  | C |  | 0.8% |  |
| ZTE#1 | A |  |  | 8.51% |
|  | B |  |  | 7.44% |
|  | C |  |  | 9.95% |
| Summary | A | 6%~11% | 4%~15% | 8.51% |
|  | B | 3%~7% | 4%~10% | 7.44% |
|  | C | 1.1%~11% | 0.2%~14% | 9.95% |

| Support/Can accept | Qualcomm, vivo, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | For rank 4, our simulation results seem not to be incorporated in the summary since only ZTE provides the results. Could FL incorporate our evaluation results in summary as a reference for other companies’ further study? We understand the FL’s concern and we can wait for one more meeting for more inputs of this case, but we really hope our simulation results can be captured into TR to provide evidence for further study. |
| ZTE | Really appreciate FL incorporates our results, we are OK with this observation. |

#### Issue#3-6 (High priority) 5% UPT, full buffer

Moderator note[Rd4]: ZTE results for rank 4 added as a note.

Proposed Observation 3.3.2: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer,

- For Max rank 1, 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~20.9%

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.5%~20.9% at CSI overhead A (small overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.3%~17.4% at CSI overhead B (medium overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~6.62% at CSI overhead C (large overhead);

- For Max rank 2, 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~14.9%

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 4.1%~14.9% at CSI overhead A (small overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 0.3%~4% at CSI overhead B (medium overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~6.03% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 8.76%~30.17%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 3.59%~5.36% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1 or Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Gain of CSI comporession over R16 Type II benchmark – 5% UPT, full buffer

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| Nokia #1 | A | 2.5% | 4.1% |  |
|  | B | 2.3% | 0.3% |  |
|  | C | 0% | 2.11% |  |
| vivo#1 | A | 8.81% | 13.8% |  |
|  | B | 8.66% | 3.05% |  |
|  | C | 6.62% | 6.03% |  |
| Fujitsu#1 | A | 20.9% | 14.90% |  |
|  | B | 17.4% | 3.50% |  |
|  | C | 6.1% | -7% |  |
| Xiaomi#1 | A |  | 30.17% |  |
|  | B |  | 21% |  |
|  | C |  | 8.76% |  |
| QC#1 | A |  | 9% |  |
|  | B |  | 3% |  |
|  | C |  | 0 |  |
| Intel#1 | A |  | 5% |  |
|  | B |  | 4% |  |
|  | C |  | 3% |  |
| ZTE#1 | A |  |  | 5.36% |
|  | B |  |  | 4.72% |
|  | C |  |  | 3.59% |
| Summary | A | 2.5%~20.9% | 4.1%~14.9% | 5.36% |
|  | B | 2.3%~17.4% | 0.3%~4% | 4.72% |
|  | C | 0%~6.62% | -7%~6.03% | 3.59% |

| Support/Can accept | Qualcomm, vivo, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | The same comment as Issue#3-5. |
| ZTE | We check our results and it needs a minor modification for our results: - Note: For Max rank 4, 1 source [ZTE] observes gain of 3.59%~5.36% 4.72%~6.15% over CSI overhead A/B/C. |

#### Issue#3-10a (New) High resolution ground-truth CSI for training-Benchmark of lower bound

Moderator note[Rd4]: As per concerns from some companies, the observations of Issue#3-10 has limited inputs, especially the companies who compared the high resolution R16 eType II-like quantization with new parameter values with the R16 eType II-like quantization with legacy values of PC8. Therefore, the following proposal is given to set up a benchmark/lower bound, to see how much gap between PC8 and Float32, and how much gain can be achieved by new parameters compared to PC8.

Upd Proposal 3.3.3: For the evaluation of the R16 eType II-like codebook based high resolution quantization of the ground-truth CSI in the CSI compression for AI/ML training, regarding the evaluation of new values of eType II parameters, consider the legacy values of PC8 as the baseline/lower-bound of performance comparison.

- Note: it has been agreed that Float32 is adopted as the baseline/upper-bound of performance comparison.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-11a (High priority) Rank>1 solutions-down selection

Moderator note[Rd4]: Number of companies added.

Proposed Conclusion 3.3.4: For further study the evaluation of CSI compression, for the rank>1 solutions, prioritize Option 2-1 (layer specific and rank common), Option 3-1 (layer common and rank common), and Option 3-2 (layer common and rank specific).

- Note: For the evaluations of CSI compression with 1-on-1 joint training,

  - Option 1-2 is not adopted;

  - Option 2-2 is not adopted;

  - Over the 11 sources [Huawei, Ericsson, Nokia, ZTE, OPPO, Intel, Fujitsu, NTT DOCOMO, InterDigital, Qualcomm, Futurewei] for FTP/fullbuffer, Max Rank 2,

    - Option 1-1 is adopted by 1 source [ZTE]

    - Option 2-1 is adopted by 3 sources [Ericsson, Intel, Qualcomm]

    - Option 3-1 is adopted by 9 sources [Huawei, Nokia, ZTE, OPPO, Fujitsu, Futurewei, Xiaomi, vivo, NTT DOCOMO]

    - Option 3-2 is adopted by 2 sources [NTT DOCOMO, InterDigital]

  - Over the 4 sources [Qualcomm, ZTE, CATT, Apple] for FTP/fullbuffer, Max Rank 4,

    - Option 2-1 is adopted by 1 source [Qualcomm]

    - Option 3-1 is adopted by 2 sources [Qualcomm, ZTE]

    - Option 3-2 is adopted by 1 source [Apple]

| Support/Can accept | vivo, LG Electronics, NTT DOCOMO(w/ comments), OPPO, ZTE, Futurewei, Xiaomi, Ericsson |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | We are supportive of Option 3-1, but we are open to Option 2-1 and Option 3-2. |
| LG Electronics | We prefer option 3-1. |
| NTT DOCOMO | We also provided the results on Option 3-1 in Table 3. We are open to both Option 3-1 and 3-2. |
| OPPO | Supportive of option 3-1, also open to option 2-1 and option 3-2. |
| Qualcomm | It would be good to clarify whether the prioritization is for evaluation, capturing results, or for specification impact discussion. If it is for specification impact, then it is not clear how we can decide not to discuss specification impact based on the fact that it was not evaluated by any company. |
| Moderator | @Qualcomm It is also for spec impact. The rationale is that we do believe companies, after 1 year of the study, have tried more than one type of the rank>1 options. We believe the above 3 options are justified to be the ones with the tradeoff of best performance/lowest complexity - that is the reason why they survived. |

#### Issue#3-14 (High priority) Generalization-Carrier frequencies

Moderator note[Rd3]: No change on the issue. Please provide more inputs!

Proposed Observation 3.3.5: For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B,

- For generalization Case 2, generalized performance may be achieved in general

  - If carrier frequency#A is 3.5/4GHz & carrier frequency#B is 2GHz, 3 sources [NTT DOCOMO, Nokia, MediaTek] observe generalized performance of less than -1.72% degradation.

  - If carrier frequency#A is 2GHz & carrier frequency#B is 3.5/4GHz, 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek] observe generalized performance of less than -1% degradation or positive gain,

    - Note: 1 source [Nokia] observes significant degradations of -6.6%.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-0.8% loss or positive gain) for carrier frequency#B subject to any of 2GHz and 3.5/4GHz, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B, as observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Minor loss (0%~-0.8%) are observed by 3 sources [Nokia, vivo, MediaTek].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Note: Significant degradations of up to -4.9% are still observed by 1 source [Nokia] for carrier frequency#B subject to 3.5/4GHz

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Antenna layouts are assumed as the same over the different frequency carriers.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Generalization of CSI compression– carrier frequencies

| Training, Generalization Case 1 |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | 3.5/4GHz |  |  | 2GHz |  |
| Training, Generalization Case 3 |  |  | 2GHz+3.5/4GHz | 5.5GHz+3.5/4GHz |  | 2GHz+3.5/4GHz | 5.5G+3.5/4GHz |  |
| Testing |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| vivo#1/#2 | Case 1 | X |  |  |  | 0.788 |  |  |
|  |  | Y |  |  |  | 0.847 |  |  |
|  |  | Z |  |  |  | 0.9013 |  |  |
|  | Case 2 | X |  |  |  |  | 0.782/-0.76% |  |
|  |  | Y |  |  |  |  | 0.838/-1% |  |
|  |  | Z |  |  |  |  | 0.8992/-0.23% |  |
|  | Case 3 | X |  |  |  | 0.788/0% | 0.789/0.127% |  |
|  |  | Y |  |  |  | 0.845/0.236% | 0.848/0.12% |  |
|  |  | Z |  |  |  | 0.9015/0.02% | 0.9002/-0.12% |  |
| NTT DOCOMO #1#2#3#4#5#6 | Case 1 | X | 0.58 |  |  | 0.58 |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  | -1.72% |  |  | 1.72% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X | 0%/0%/1.72% |  |  | 1.72%/1.72%/1.72% |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
| Nokia#1#2 | Case 1 | X | 0.715 |  |  | 0.691 |  |  |
|  |  | Y | 0.763 |  |  | 0.796 |  |  |
|  |  | Z | 0.886 |  |  | 0.880 |  |  |
|  | Case 2 | X |  | -0.8% |  |  | 0.1% |  |
|  |  | Y |  | 4.8% |  |  | -6.6% |  |
|  |  | Z |  | 0.1% |  |  | -0.9% |  |
|  | Case 3 | X | -0.1% |  |  | 0.3% |  |  |
|  |  | Y | 1.0% |  |  | -4.9% |  |  |
|  |  | Z | 0.1% |  |  | -0.3% |  |  |
| MTK#1#2 | Case 1 | X | 0.696 |  |  | 0.702 |  |  |
|  |  | Y | 0.771 |  |  | 0.773 |  |  |
|  |  | Z | 0.831 |  |  | 0.835 |  |  |
|  | Case 2 | X |  | 0.3% |  |  | -0.1% |  |
|  |  | Y |  | -0.5% |  |  | 0.5% |  |
|  |  | Z |  | -0.2% |  |  | 0.4% |  |
|  | Case 3 | X | 0.0% |  |  | -0.3% |  |  |
|  |  | Y | -0.8% |  |  | -0.3% |  |  |
|  |  | Z | -0.2% |  |  | 0.1% |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  | NTT DOCOMO, Nokia, MTK |  |  | Vivo, Nokia, MTK |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  |  | Nokia |  |
|  |  | Positive gain |  | MTK, Nokia |  |  | NTT DOCOMO, Nokia, MTK |  |
|  | Case 3 | Minor loss (>-1.6%) | Nokia, MTK |  |  | Nokia, MTK | Vivo |  |
|  |  | Significant loss (<-4%) |  |  |  | Nokia |  |  |
|  |  | Positive gain | NTT DOCOMO, Nokia |  |  | Vivo, NTT DOCOMO, Nokia, MTK | vivo |  |

| Support/Can accept | NTT DOCOMO, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

#### Issue#3-20 (High priority) NW first training-1 NW to 1/M>1 UE

Moderator note[Rd4]: CATT results added.

Proposed Observation 3.3.6: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone [or similar architecture] is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 4 sources [Ericsson, Nokia, ZTE，Fujitsu] observe -0%~-0.5% degradation, 5 sources [Nokia, QC, ZTE, CATT, vivo] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Huawei, Apple, CMCC] observe -0%~-0.5% degradation.

- Note: For the NW first separate training case where different backbones [or substantially different architectures] are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [Qualcomm, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Note: Results refer to Table X of R1-230xxxx

Table X. NW first training for CSI compression -1 NW to 1/M>1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip |  |  |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo, CATT | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.5% |  | HW, Apple, CMCC, |  |  |

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| Moderator | @Lenovo That is why the note is added: it only limits the results are from the one way dataset sharing (from the simulation assumption of majority companies). For other special handlings, we may have further observations. |
| OPPO | Add OPPO’s results as follows: 6 sources [Nokia, QC, ZTE, CATT, vivo, OPPO] observe -0.5%~-1% degradation |

#### Issue#3-20a (Medium priority) NW first training-1 NW to 1/M>1 UE- Impact of size of shared training dataset

Moderator note[Rd4]: No changes for the fourth round.

Proposed Observation 3.3.7: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [Huawei, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table X of R1-230xxxx

Table X. NW first training for CSI compression -1 NW to 1/M>1 UE with various dataset settings

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) dataset size | Same backbone, same/diff dataset size | Diff backbone; Cap_UE model < Cap_NW model, same/diff dataset size |
| --- | --- | --- | --- | --- |
| HW#1/2 | Descrip | TF#A-TF#1, 300k | TF#A-TF#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.1%/-0.3% Gap: -0.2% |  |
|  | Z |  | 0%/-0.1% Gap: -0.1% |  |
|  | Descrip | CNN#A-CNN#1, 300k | CNN#A-CNN#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.4%/-0.5% Gap: -0.1% |  |
|  | Z |  | -0.3%/-0.5% Gap: -0.2% |  |
| CMCC#1~7 | Descrip | TF#A-TF#1, 154k | TF#A-TF#1, 154k/100k/50k/ 10k/5k |  |
|  | X |  |  |  |
|  | Y |  | -0.22%/-0.22%/-0.55%/-1.86%/-2.95% Gap: 0/-0.3%/-1.64%/-2.73% |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-CNN#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -4.04%/-4.59%/-5.90%/-8.30%/-9.61% Gap: -0.55/-1.86%/-4.26%/-5.55% |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-MLP#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -0.76%/-0.98%/-1.53%/-6.33%/-9.17% Gap: -0.22%/-0.77%/-5.57%/-8.41% |
|  | Z |  |  |  |
| vivo#1/2 | Descrip | TF#A-TF#1, 300k; TF#A-TF#2, 300k; | TF#A-TF#1, 300k/50k; TF#A-TF#2, 300k/50k; |  |
|  | X |  | -0.6%/-2.6%; -0.9%/-2.6% |  |
|  | Y |  |  |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-CNN#1, 300k |  | TF#A-CNN#1, 300k/50k |
|  | X |  |  | -5.2%/ -9.4% Gap: -3.8% |
|  | Y |  |  |  |
|  | Z |  |  |  |
| Summary: gap of additional loss | -0%~-0.55% |  | HW, CMCC, | CMCC, vivo |
|  | -0.55%~-8.41% |  |  |  |

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. Please also provide your views on the highlighted part. |
| Lenovo | We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| OPPO | Add OPPO’s results. - If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 3 sources [CMCC, vivo, OPPO]. |

#### Issue#3-22 (High priority) UE first training-1/N>1 NW to 1 UE

Moderator note[Rd4]: Qualcomm results added.

Proposed Observation 3.3.8: For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1/N>1 NW to 1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone [or similar architecture] is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [Huawei] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones [or substantially different architectures] are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Note: Results refer to Table x of R1-230xxxx

Table X. UE first training for CSI compression-1/N>1 NW to 1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model > Cap_NW model | Diff backbone or largely diff structure; Cap_UE model < Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A/B/C-TF#1 | TF#A/B/C-TF#1 |  |  |
|  | X |  | 0%/-0.4%/-0.2% |  |  |
|  | Y |  | 0%/-0.6%/-0.2% |  |  |
|  | Z |  | 0%/-0.6%/-0.1% |  |  |
|  | Descrip | CNN#A/B-CNN#1 | CNN#A/B-CNN#1 |  | TF#A- CNN#1 |
|  | X |  | -0.6%/-0.6% |  | -0.3% |
|  | Y |  | -0.5%/-0.6% |  | 0% |
|  | Z |  | -0.5%/-0.7% |  | 0% |
| Nokia#3/4 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | +1.7%/-0.2% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1; CNN#A-CNN#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | L1: +0.2%; L2: -0.04%; L3: +0.03%; L4:+0.2%; L1: -0.2% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  | +0.6% |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -0.3% |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A/B/C /D-TF#1 | TF#A/B/C /D-TF#1 |  |  |
|  | X |  | -1.8%/ -2.3%/ -2.6%/ -2.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: -1.13% L2: -1.55% |  |  |
|  | Y |  | L1: -1.07% L2: -1.52% |  |  |
|  | Z |  | L1: -1.05% L2: -1.75% |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  | L1: -0.99% L2: -1.06% |  |
|  | Y |  |  | L1: -0.98% L2: -1.88% |  |
|  | Z |  |  | L1: -0.74% L2: -0.92% |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  | L1: -0.85% L2: -0.73% |
|  | Y |  |  |  | L1: -0.84% L2: -1.74% |
|  | Z |  |  |  | L1: -0.67% L2: -0.78% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.03% |  |  |
|  | Y |  | -0.31% |  |  |
|  | Z |  | -0.14% |  |  |
|  | Descrip | TF#A-TF#1 |  | CNN #A-TF#1 |  |
|  | X |  |  | -5.82% |  |
|  | Y |  |  | -5.51% |  |
|  | Z |  |  | -4.18 % |  |
| CATT#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.14%/ 0.21% |  |  |
|  | Y |  | -0.23%/-0.27% |  |  |
|  | Z |  | -0.14%/-0.35% |  |  |
|  | Descrip | CNN#A-TF #1 |  | CNN#A-TF #1 |  |
|  | X |  |  | 1.07% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -0.90% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +5.5%; +1.3%/ +2.7%; +17.1%/ +9.2% |  |  |
|  | Descrip |  |  |  | TF#A/B-CNN#1; TF#A/B-CNN#2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A/B- TF#1; |  | CNN#A/B- TF#1; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | 0.0%/ -1.4% | +5.8%/ +7.4% |
| vivo#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.4%/ -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A- TF#1 |  | CNN#A- TF#1 |  |
|  | X |  |  | -1.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | Nokia, Fujitsu, CATT, vivo, QC | QC |  |
|  | -0.5%~-1% |  |  | CATT, ZTE | ZTE |
|  | -1%~-1.88% |  | ZTE | ZTE, vivo | ZTE |
| Summary (Before Q) | -0%~-0.5% |  | HW |  |  |
|  | -0.5%~-1% |  | HW |  |  |
|  | -1%~-2.9% |  | Apple, |  |  |

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Lenovo | We have also reported simulation results for the cases - When backbones are almost similar (Lenovo#1 Normal Type3) For the case where the shared input of the UE side CSI reconstruction part is after quantization, 4 sources [Nokia, Fujitsu, CATT, vivo, Lenovo] observe -0%~-0.5%-0.8% degradation - When backbones are very different (Lenovo#2 Normal Type3) If the backbone of the NW part model is less capable than the UE part model, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation and 1 sources [Lenovo] observe -1.2%~-4.2% degradation We are not in favor of the “Note” in blue part as it is limiting the possibilities of Type-3 to only the example that wa presented in RAN#111. So we prefer to remove that. The bullet point in blue is generally okay but if the structures are different we may need to say which one them should be used for the 1-on-1 joint training, for example the more capable structure should be used. |
| Qualcomm | We ask the moderator to please clarify the comment that the format of our result is incorrect. |
| Moderator | @Lenovo Your results are submitted to Case 2 (dataset mixing case, e.g. M>1 UEs to 1 NW). The observation is for Case 1 (1 UE to 1 NW) @QC As there is only one NW part model in the assumptions, why are there results for two NWs? Why for layer 1 there are two results (two UEs?) and for other layers there is only one result? |
| Lenovo | If this proposal is related to Case-1, then we are okay with this. |
| Qualcomm | We thank the moderator for the explanation. Please find our responses to your questions below: As there is only one NW part model in the assumptions, why are there results for two NWs? - The result for layer 1 was indeed evaluated with two NWs. The detail was inadvertently missed. Please add “CNN” as the NW#2 part model backbone/structure, and "440k” as the NW#2 part training dataset description and size. Why for layer 1 there are two results (two UEs?) and for other layers there is only one result? - Yes, for layer 1, the two results are for two UEs. For the other layers, we only evaluated them for the single UE case. |
| Moderator | @Qualcomm the positive values are filtered, assuming your type 3 model pair is the same as the 1-on-1 joint training model pair. |
| OPPO | Add OPPO’s results. For the case where the shared input of the UE side CSI reconstruction part is after quantization, 6 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm, OPPO] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation. If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 3 sources [ZTE, vivo, OPPO] observe -1%~-1.88% degradation. |

## 5th round email discussions

#### Issue#3-20 (High priority) NW first training-1 NW to 1/M>1 UE

Moderator note[Rd5]: Samsung results added. 1 NW to 1/M>1 UE changed to 1 NW to 1 UE (Case 1) for clarity.

Upd Proposed Observation 3.4.1: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 5 sources [Ericsson, Nokia, ZTE，Fujitsu, Samsung] observe -0%~-0.5% degradation, 6 sources [Nokia, Qualcomm, ZTE, CATT, vivo, Samsung] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Huawei, Apple, CMCC] observe -0%~-0.8% degradation.

- Note: For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [Qualcomm, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table X of R1-230xxxx

Table X. NW first training for CSI compression -1 NW to 1 UE (Case 1)

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip | TF#A-TF#1/TF#2; CNN#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +0.3%/-0.4%; -0.09% | -17.5% |  |
| SS#1 (After Q) | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | -4.5% +4.2% |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu, SS | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo, CATT, SS | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.5% |  | HW, Apple, CMCC, |  |  |

#### Issue#3-20a (Medium priority) NW first training-1 NW to 1/M>1 UE- Impact of size of shared training dataset

Moderator note[Rd5]: No change on top of the fourth round.

Proposed Observation 3.4.2: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [Huawei, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table X of R1-230xxxx

Table X. NW first training for CSI compression -1 NW to 1 UE 1 NW to 1 UE (Case 1) with various dataset settings

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) dataset size | Same backbone, same/diff dataset size | Diff backbone; Cap_UE model < Cap_NW model, same/diff dataset size |
| --- | --- | --- | --- | --- |
| HW#1/2 | Descrip | TF#A-TF#1, 300k | TF#A-TF#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.1%/-0.3% Gap: -0.2% |  |
|  | Z |  | 0%/-0.1% Gap: -0.1% |  |
|  | Descrip | CNN#A-CNN#1, 300k | CNN#A-CNN#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.4%/-0.5% Gap: -0.1% |  |
|  | Z |  | -0.3%/-0.5% Gap: -0.2% |  |
| CMCC#1~7 | Descrip | TF#A-TF#1, 154k | TF#A-TF#1, 154k/100k/50k/ 10k/5k |  |
|  | X |  |  |  |
|  | Y |  | -0.22%/-0.22%/-0.55%/-1.86%/-2.95% Gap: 0/-0.3%/-1.64%/-2.73% |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-CNN#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -4.04%/-4.59%/-5.90%/-8.30%/-9.61% Gap: -0.55%/-1.86%/-4.26%/-5.55% |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-MLP#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -0.76%/-0.98%/-1.53%/-6.33%/-9.17% Gap: -0.22%/-0.77%/-5.57%/-8.41% |
|  | Z |  |  |  |
| vivo#1/2 | Descrip | TF#A-TF#1, 300k; TF#A-TF#2, 300k; | TF#A-TF#1, 300k/50k; TF#A-TF#2, 300k/50k; |  |
|  | X |  | -0.6%/-2.6%; -0.9%/-2.6% |  |
|  | Y |  |  |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-CNN#1, 300k |  | TF#A-CNN#1, 300k/50k |
|  | X |  |  | -5.2%/ -9.4% Gap: -3.8% |
|  | Y |  |  |  |
|  | Z |  |  |  |
| Summary: gap of additional loss | -0%~-0.55% |  | HW, CMCC, |  |
|  | -0.55%~-8.41% |  |  | CMCC, vivo |

#### Issue#3-22 (High priority) UE first training-1/N>1 NW to 1 UE

Moderator note[Rd5]: 1/N>1 NW to 1 UE changed to 1 NW to 1 UE (Case 1) for clarity..

Proposed Observation 3.4.3: For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1/N>1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [Huawei] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table X of R1-230xxxx

Table X. UE first training for CSI compression-1 NW to 1 UE (Case 1)

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model > Cap_NW model | Diff backbone or largely diff structure; Cap_UE model < Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A/B/C-TF#1 | TF#A/B/C-TF#1 |  |  |
|  | X |  | 0%/-0.4%/-0.2% |  |  |
|  | Y |  | 0%/-0.6%/-0.2% |  |  |
|  | Z |  | 0%/-0.6%/-0.1% |  |  |
|  | Descrip | CNN#A/B-CNN#1 | CNN#A/B-CNN#1 |  | TF#A- CNN#1 |
|  | X |  | -0.6%/-0.6% |  | -0.3% |
|  | Y |  | -0.5%/-0.6% |  | 0% |
|  | Z |  | -0.5%/-0.7% |  | 0% |
| Nokia#3/4 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | +1.7%/-0.2% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1; CNN#A-CNN#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | L1: +0.2%; L2: -0.04%; L3: +0.03%; L4:+0.2%; L1: -0.2% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  | +0.6% |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -0.3% |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A/B/C /D-TF#1 | TF#A/B/C /D-TF#1 |  |  |
|  | X |  | -1.8%/ -2.3%/ -2.6%/ -2.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: -1.13% L2: -1.55% |  |  |
|  | Y |  | L1: -1.07% L2: -1.52% |  |  |
|  | Z |  | L1: -1.05% L2: -1.75% |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  | L1: -0.99% L2: -1.06% |  |
|  | Y |  |  | L1: -0.98% L2: -1.88% |  |
|  | Z |  |  | L1: -0.74% L2: -0.92% |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  | L1: -0.85% L2: -0.73% |
|  | Y |  |  |  | L1: -0.84% L2: -1.74% |
|  | Z |  |  |  | L1: -0.67% L2: -0.78% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.03% |  |  |
|  | Y |  | -0.31% |  |  |
|  | Z |  | -0.14% |  |  |
|  | Descrip | TF#A-TF#1 |  | CNN #A-TF#1 |  |
|  | X |  |  | -5.82% |  |
|  | Y |  |  | -5.51% |  |
|  | Z |  |  | -4.18 % |  |
| CATT#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.14%/ 0.21% |  |  |
|  | Y |  | -0.23%/-0.27% |  |  |
|  | Z |  | -0.14%/-0.35% |  |  |
|  | Descrip | CNN#A-TF #1 |  | CNN#A-TF #1 |  |
|  | X |  |  | 1.07% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -0.90% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +5.5%; +1.3%/ +2.7%; +17.1%/ +9.2% |  |  |
|  | Descrip |  |  |  | TF#A/B-CNN#1; TF#A/B-CNN#2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A/B- TF#1; |  | CNN#A/B- TF#1; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | 0.0%/ -1.4% | +5.8%/ +7.4% |
| vivo#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.4%/ -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A- TF#1 |  | CNN#A- TF#1 |  |
|  | X |  |  | -1.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | Nokia, Fujitsu, CATT, vivo, QC | QC |  |
|  | -0.5%~-1% |  |  | CATT, ZTE | ZTE |
|  | -1%~-1.88% |  | ZTE | ZTE, vivo | ZTE |
| Summary (Before Q) | -0%~-0.5% |  | HW |  |  |
|  | -0.5%~-1% |  | HW |  |  |
|  | -1%~-2.9% |  | Apple, |  |  |

# Observations/conclusions for CSI prediction

## 1st/2nd round email discussions

### 4.1-1: Table 6. Evaluation results for CSI prediction without model generalization/scalability

#### Issue#4-1 (High priority) Benchmark#1-SGCS gain-Impact of UE speed

Moderator note: In the last meeting, we have analyzed the impact of input type to the SGCS gain over benchmark#1. In this meeting, we will further analyze the impact of UE speed to the SGCS gain over benchmark#1.

Moderator note[Rd2]: A note is made based on vivo inputs. More inputs are welcome!

| OB window | 3/5ms~ 5/5ms |  |  | 6/5ms~10/5ms |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10km/h | 30km/h | 60km/h | 10km/h | 30km/h | 60km/h |
| HW#1 |  |  | 18% |  |  |  |
| ZTE#1/3 |  | 18.72%~ 22.05% |  |  | 25.51%~ 26.47% |  |
| SS#1/2 | 2.4% |  | 11.2% |  |  |  |
| Fujitsu#1/2/3 | L1:2.6% L2:4% | L1:28% L2:35% | L1:5% L2:3.4% |  |  |  |
| Apple#1 |  | L1: 20.2232% L2: 27.0090% |  |  |  |  |
| vivo#1 |  |  |  |  |  | 13.8% |
| InterDigital#1~ 3 | L1: 6% L2: 9% | L1: 35% L2: 46% | L1: -3% L2: -4% |  |  |  |
| MTK#1 |  | 40.7% |  |  |  |  |
| ETRI#1 |  | 10.43% |  |  |  |  |
| Xiaomi#1~6 | 1.03% | 20.25% | 66.02% | 1.03% | 20.57% | 66.55% |
| CMCC#6/8/9 | 21.93% | 41.75% | 19.98% |  |  |  |
| OPPO#1 |  | 6% |  |  |  |  |
| Spreadtrum#1 |  | 31.3% |  |  |  |  |
| Summary | 10km/h |  | 30km/h |  | 60km/h |  |
|  | 4 sources:1.03%~6%; [Fujitsu, SS, Xiaomi, InterDigital] 1 source: 21.93% [CMCC] |  | 2 sources: 6%~10.43% [OPPO, ETRI] 4 sources: 18.72%~28% [ZTE, Fujitsu, Apple, Xiaomi] 3 sources: 35%~ 41.75% [InterDigital, MTK, CMCC] |  | 2 sources: -3%~5% [Fujitsu, InterDigital] 4 sources: 11.2%~19.98% [HW, SS, vivo, CMCC] |  |

Upd Proposed Observation 4.1.1: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, SS, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 4 sources [ZTE, Fujitsu, Apple, Xiaomi] observe 18.72%~28% gain, and 3 sources [InterDigital, MTK, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [HW, SS, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - No spatial consistency is considered

| Support/Can accept | ZTE, OPPO, vivo (comments), ETRI |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| vivo | In the previous and current meetings, we have submitted the results relying on both non-spatial consestancy and spatial consestancy. We found that, the gain tendency is the same. It should be captured in the results. |
| Spreadtrum | Our simulation result in R1-2304550 was not captured. Sorry for missing collecting the result. In our simulation under 30km/h UE speed, AI based CSI prediction can achieve about 31.3% SGCS gain than The nearest historical CSI w/o prediction. |
| Moderator | @Spreadtrum: what is the speed? What is the observation window? Please fill the entry in the table above by yourself before Tue 23:59AM (Incheon), otherwise your results may not be captured to the observation in 113 |
| Spreadtrum | We have updated the results in the above table. |

#### Issue#4-2 (Medium priority) Benchmark#1-SGCS gain- Impact of observation window size

Moderator note: In the last meeting, we have analyzed the impact of input type to the SGCS gain over benchmark#1. In this meeting, we will further analyze the impact of observation window size to the SGCS gain over benchmark#1.

Moderator note[Rd2]: A note is made same as Issue#4-1. Please provide more inputs!

| 30km/h | OB window size |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4/5ms | 5/5ms | 8/5ms | 10/5ms | 15/5ms |
| ZTE#1/3 | 18.72%~ 22.05% |  | 26.28% | 26.47% |  |
| Xiaomi#1~6 |  | 20.25% | 20.57% |  |  |
| CMCC#6/8/9 |  | 41.75% |  |  | 52.07% |

Proposed Observation 4.1.2: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from observation window length perspective, in general the gain of AI/ML based solution is increased with the increase of the length for the observation window:

- 1 source [ZTE] observes the gain over benchmark is increased from 18.72%~ 22.05% gain to 26.47% gain when the observation window is increased from 4/5ms to 10/5ms.

- 1 source [Xiaomi] observes the gain over benchmark is increased from 20.25% gain to 20.57% gain when the observation window is increased from 5/5ms to 8/5ms.

- 1 source [CMCC] observes the gain over benchmark is increased from 41.75% gain to 52.07% gain when the observation window is increased from 5/5ms to 15/5ms.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - No spatial consistency is considered

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

#### Issue#4-3 (Medium priority) Benchmark#1-SGCS gain- Impact of prediction window length

Moderator note: In the last meeting, we have analyzed the impact of input type to the SGCS gain over benchmark#1. In this meeting, we will further analyze the impact of prediction window length to the SGCS gain over benchmark#1.

Moderator note[Rd2]: No change on the issue. Please provide more inputs!

| 30km/h | Prediction window length |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | +2.5ms | +5ms | +7.5ms | +10ms | +15ms |
| ZTE#1 |  | 22.05% |  | 10.04% | 7.48% |
| ZTE#2 |  | 26.47% |  | 23.93% | 13.36% |
| Apple#1 | 7.2442% | 20.2232% | 23.1582% | 10.4327% |  |
| MTK#2 |  | 40.7% |  | 50.3% | 35.5% |
| ETRI#1 |  | 10.43% |  | 8.75% | 7.62% |
| CMCC#5 |  | -41.45dB |  | -37.53dB |  |
| OPPO#1 |  | 6% |  | 5% | 5% |

Proposed Observation 4.1.3: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS/NMSE, from prediction window length perspective, in general the gain of AI/ML based solution is related with the prediction length in terms of the distance to the applicable time of the predicted CSI:

- 4 sources [ZTE, ETRI, CMCC, OPPO] observe the gain over benchmark is reduced with the increase of the prediction length since after 5ms;

- 2 sources [Apple, MTK] observe the gain over benchmark is increased when the prediction length is appropriately increased from 2.5ms to 5ms or from 5ms to 10ms, and decreased when the prediction length is further increased from 5ms to 10ms.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value/NMSE in dB value for layer 1.

  - No spatial consistency is considered.

  - No post processing is considered.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

#### Issue#4-4 (Low priority) Benchmark#2-SGCS gain

Moderator note: Summarized SGCS gain of AI/ML based CSI prediction over Benchmark#2 are provided in the table below.

Moderator note[Rd2]: A note is made same as Issue#4-1. Please provide more inputs!

| OB window | 3/5ms~ 5/5ms |  |  | 6/5ms~10/5ms |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10km/h | 30km/h | 60km/h | 10km/h | 30km/h | 60km/h |
| HW#1 |  |  | 3.6% |  |  |  |
| ZTE#1/3 |  | 0.85%~12.92% |  |  | -1.1%~-0.3% |  |
| vivo#1 |  |  |  |  |  | 6.2% |
| InterDigital#1~ 3 | L1: 0% L2: 0% | L1: -1% L2: -2% | L1: -14% L2: -16% |  |  |  |
| MTK#1 |  | 0.16% |  |  |  |  |
| Summary | 4 sources: 0.16%~12.92% [HW, vivo, ZTE, MTK] 2 sources: -16%~0% [ZTE, Interdigital] |  |  |  |  |  |

Proposed Observation 4.1.4: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#2 of a non-AI/ML based CSI prediction approach, more companies observe gains are achieved by AI/ML based CSI prediction than the companies who observe degradations are suffered by AI/ML:

- 4 sources [HW, vivo, MTK, ZTE] observe 0.16%~12.92% gain.

- 2 sources [Interdigital, ZTE] observe -15%~0% performance loss.

- Note: 1 source [ZTE] observes the gains reduce or becomes loss if observation window length increases (0.85%~12.92% gain under 15ms~20ms observation window, and -0.3%~-1.1% loss under 30ms~50ms observation window).

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1/2.

  - No post processing is considered.

  - No spatial consistency is considered

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |

#### Issue#4-5 (High priority) UPT gain-Mean UPT

Moderator note: Summarized Mean UPT resulst for FTP and full buffer are provided in the table below.

Moderator note[Rd2]: A note is made based on vivo inputs same as Issue#4-1. More inputs are welcome!

| BM#1 | 30km/h |  |  | 60km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1-FTP |  |  |  | 1.2% (62bits) 1.7% (111bits) 2.1% (279bits) | 1.8% (62bits) 3.9% (111bits) 3.5% (279bits) | 2.5% (62bits) 3.1% (111bits) 4.2% (279bits) |
| Apple#1- FTP | 7.6% | 8.5% | 8.5% |  |  |  |
| Vivo#1- FTP |  |  |  | 9.7% (64 bits) | 15.1%(64 bits) | 17.2%(64 bits) |
| Vivo#2-FB |  |  |  | 8.7% |  |  |
| Nokia#1-FB | 2%~3% (303 bits) |  |  |  |  |  |
| MTK | 22.6%~29.1% (279bit) |  |  |  |  |  |

| BM#2 | 30km/h |  |  | 60km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1- FTP |  |  |  | 0.7% (62bits) 0.9% (111bits) 0.8% (279bits) | 2.3% (62bits) 3.1% (111bits) 2.5% (279bits) | 2.4% (62bits) 2% (111bits) 2.5% (279bits) |
| Vivo#1- FTP |  |  |  | 3.4% (64 bits) | 5.1%(64 bits) | 7.0%(64 bits) |
| Vivo#2-FB |  |  |  | 8.1% |  |  |
| MTK |  |  |  | 1.22%~1.17% (279bit) |  |  |

Upd Proposed Observation 4.1.5: For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [HW] observes 1.2%~4.2% gain;

    - 1 source [Apple] observes 7.6%~8.5% gain;

    - 1 source [vivo] observes 9.7%~17.2% gain.

  - For full buffer traffic:

    - 1 source [Nokia] observes 2%~3% gain;

    - 1 source [vivo] observes 8.7% gain.

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 1 source [HW] observes 0.7%~3.1% gain;

    - 1 source [vivo] observes 3.4%~7.0% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 8.1% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| vivo | In the previous and current meetings, we have submitted the results relying on both non-spatial consestancy and spatial consestancy. We found that, the gain tendency is the same. It should be captured in the results. |

#### Issue#4-6 (High priority) UPT gain-5% UPT

Moderator note: Summarized 5% UPT resulst for FTP and full buffer are provided in the table below.

Moderator note[Rd2]: A note is made based on vivo inputs same as Issue#4-1. More inputs are welcome!

| BM#1 | 30km/h |  |  | 60km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1-FPT |  |  |  | 4.5% (62bits) 18.3% (111bits) 8% (279bits) | 11.3% (62bits) 9.3% (111bits) 8.6% (279bits) | 17.8% (62bits) 13.4% (111bits) 16.4% (279bits) |
| Apple#1-FPT | 12.9% | 20.1% | 15.8% |  |  |  |
| Vivo#1-FPT |  |  |  | 6.9% (64 bits) | 13.0% (64 bits) | 20.0% (64 bits) |
| Vivo#2-FB |  |  |  | 17.5% |  |  |
| Nokia#1-FB | 6%~15% (303 bits) |  |  |  |  |  |

| BM#2 | 30km/h |  |  | 60km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1-FPT |  |  |  | 2.5% (62bits) 5.2% (111bits) 4.3% (279bits) | 7.7% (62bits) 7% (111bits) 8.6% (279bits) | 14.8% (62bits) 6.7% (111bits) 4.2% (279bits) |
| Vivo#1-FPT |  |  |  | 0.5% (64 bits) | 3.1%(64 bits) | 16%(64 bits) |
| Vivo#2-FB |  |  |  | 11% |  |  |

Upd Proposed Observation 4.1.6: For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 2 sources [HW, vivo] observes 4.5%~9.3% gain;

    - 3 sources [HW, Apple, vivo] observes 11.3%~20.1% gain;

  - For full buffer traffic:

    - 2 sources [Nokia, vivo] observe 6%~17.5% gain;

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 2 sources [HW, vivo] observes 0.5%~16% gain;

  - For full buffer traffic:

    - 1 source [vivo] observes 11% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

| Support/Can accept | vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| vivo | In the previous and current meetings, we have submitted the results relying on both non-spatial consestancy and spatial consestancy. We found that, the gain tendency is the same. It should be captured in the results. |

### 4.1-2: Table 7. Evaluation results for CSI prediction with model generalization

#### Issue#4-7 (High priority) Generalization-UE speed

Moderator note: Summarized results for generalization over UE speeds.

Moderator note[Rd2]:

- A note is made based on vivo inputs same as Issue#4-1.

- Add Sptreadtrum results for Case 3.

| Training Genr-Case 1 (km/h) |  | 10 |  |  |  | 30 |  |  |  | 60 |  |  |  | 120 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training Genr-Case 2 (km/h) |  |  | 30 | 60 | 120 |  | 10 | 60 | 120 |  | 10 | 30 | 120 |  | 30 | 60 |
| Training Genr-Case 3 (km/h) |  | X+10 |  |  |  | X+30 |  |  |  | Y+60 |  |  |  | Z+120 |  |  |  |
| Test |  | 10km/h |  |  |  | 30km/h |  |  |  | 60km/h |  |  |  | 120km/h |  |  |  |
| HW | Case 1 |  |  |  |  | 0.979 |  |  |  | 0.861 |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  |  | -6% |  |  |  | -31% |  |  |  |  |
|  | Case 3 |  |  |  |  | -0.9% |  |  |  | -1.4% |  |  |  |  |  |  |  |
| Apple | Case 1 |  |  |  |  | L1: 0.9984 L2: 0.9977 L3: 0.9968 L4: 0.997 |  |  |  | L1: 0.8201 L2: 0.7578 L3: 0.6651 L4: 0.6441 |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 3 |  |  |  |  | L1: -0.46% L2: -0.73% L3: -1.34% L4: -1.24% |  |  |  | L1: -1.26% L2: -1.65% L3: -0.4% L4: -2% |  |  |  |  |  |  |  |
| ZTE | Case 1 |  |  |  |  | 0.9576 |  |  |  | 0.7435 |  |  |  | 0.7289 |  |  |
|  | Case 2 |  |  |  |  |  |  | -18.44% | -21.02% |  |  | -14.34% | -3% |  | -28.44% | -3.4% |
|  | Case 3 |  |  |  |  | -10.29% |  |  |  | -0.05% |  |  |  | -3.1% |  |  |  |
| ETRI | Case 1 |  |  |  |  | 0.8841 |  |  |  | 0.7987 |  |  |  | 0.7273 |  |  |
|  | Case 2 |  |  |  |  |  |  | -16.7% | -45.6% |  |  | -11.6% | -36.1% |  | -32.3% | -27.4% |
|  | Case 3 |  |  |  |  | -13.3% -22.2% |  |  |  | -5% |  |  |  | -2% |  |  |  |
| SS | Case 1 | 0.9596 |  |  |  |  |  |  |  | 0.5745 |  |  |  | 0.4727 |  |  |
|  | Case 2 |  |  | -53% | -61% |  |  |  |  |  | -18% |  | -26.4% |  |  | -13% |
|  | Case 3 | -8% |  |  |  |  |  |  |  | -6.7% |  |  |  | -2.6% |  |  |  |
| Fujitsu | Case 1 |  |  |  |  | 0.9368 |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  | -21% |  |  |  |  |  |  |  |  |  |
|  | Case 3 |  |  |  |  | -9.8% |  |  |  |  |  |  |  |  |  |  |  |
| vivo | Case 1 |  |  |  |  | 0.9896 |  |  |  | 0.8102 |  |  |  | 0.6156 |  |  |
|  | Case 2 |  |  |  |  |  |  | -6.8% | -10.7% |  |  | -30.8% | -7.81% |  | -18% | -7.55% |
|  | Case 3 |  |  |  |  | -4.07% |  |  |  | -3.76% |  |  |  | -4.45% |  |  |  |
| CMCC | Case 1 | -42.822dB |  |  |  | -28.426dB |  |  |  | -9.036dB |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 3 | +24.297dB |  |  |  | + 12.47dB |  |  |  | + 1.981dB |  |  |  |  |  |  |  |
| InterDi gital | Case 1 | L1:0.999 L2:0.999 |  |  |  | L1:0.938 L2:0.912 |  |  |  | L1:0.57 L2:0.497 |  |  |  |  |  |  |
|  | Case 2 |  | L1: -2% L2: -2% |  |  |  | L1: -17% L2:-22% | L1: -32% L2: -40% |  |  |  |  |  |  |  |  |
|  | Case 3 | L1: -0.6% L2: -0.8% |  |  |  | L1: -3% L2: -3% |  |  |  | L1: -14% L2: -15% |  |  |  |  |  |  |  |
| MTK | Case 1 | -32.83dB |  |  |  | -23.62dB |  |  |  | -0.87dB |  |  |  | -0.5dB |  |  |
|  | Case 2 |  | +2.16dB |  | +27.5dB |  |  |  | +21.85 |  |  | +20.17dB | 0dB |  | +17.48dB |  |
|  | Case 3 | +10.31dB |  |  |  | +8.21dB |  |  |  | +0.26dB |  |  |  | +0.29dB |  |  |  |
| Xiaomi | Case 1 | 0.984 |  |  |  | 0.972 |  |  |  | 0.943 |  |  |  |  |  |  |
|  | Case 2 |  | -0.4% | -2.7% |  |  | -12.3% | -3% |  |  | -52% | -43% |  |  |  |  |
|  | Case 3 | -0.6% |  |  |  | -0.8% |  |  |  | -1.2% |  |  |  |  |  |  |  |
| CATT | Case 1 | 0.833 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  | -1.51% |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spreadtrum | Case 1 | 0.9919 |  |  |  | 0.9182 |  |  |  | 0.8302 |  |  |  |  |  |  |
|  | Case 2 |  | -1.2% | -2.1% |  |  | -18.7% |  |  |  | -34.6% |  |  |  |  |  |
|  | Case 3 | -1% |  |  |  | -2.2% |  |  |  | -2% |  |  |  |  |  |  |
| Summ ary | Case 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  | -0.4%~ -2% [3 companies] +2.16dB | -2.7%[1 company], -53%[1] | -61%[1company] +27.5dB |  | -12.3%~ -22%[3 companies] | -3%[1 company]; -6~ -40%[5 companies] | -10.7%~-45.6[3 companies] +21.85dB |  | -18%~-52%[2 companies] | -11.6%~-43%[4 companies] +20.17dB | -3%[1 company], -7.81%~-36[3 companies] 0dB |  | -18%~-32.3%[3 companies] +17.48dB | -3.4%[1 company], 7.55~-27.4[3companies] |
|  | Case 3 | -0.6% ~ -1%[3 companies]; -8% [1 company] +10.31dB~+24.297dB |  |  |  | -0.08%~-1.34% [3 companies]; -2.2%~-4.07% [3 companies]; -5.9~-26.5% [3 companies]; +8.21dB~+ 12.47dB |  |  |  | -0.05~2% [4 companies] -2%~-3.76% [2 company] -5%~-15% [3 companies] + 0.26~1.981dB |  |  |  | -2~-4.45% [4 companies] +0.29dB |  |  |  |
| Case 2 | Minor(>-2%) |  | CATT, Xiaomi, Interdigital |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Modr(-2~-5% loss) |  |  | Xiaomi |  |  |  | xiaomi |  |  |  |  | ZTE |  |  | ZTE |
|  | Signf(<-5% loss) |  |  | SS | SS |  | Xiaomi, Interdigital, Fujitsu | ZTE, ETRI, Interdigital, vivo, HW | vivo, ETRI,ZTE |  | SS, Xiaomi | Xiaomi, Fujitsu, ETRI, ZTE | Vivo, SS, ETRI, |  | ZTE, ETRI, vivo | Vivo,SS, ETRI |
| Case 3 | Minor(>-2%) | Xiaomi, Interdigital, Spreadtrum |  |  |  | Xiaomi, Apple, HW |  |  |  | ZTE, Apple, Xiaomi, HW |  |  |  |  |  |  |  |
|  | Modr(-2~-5% loss) |  |  |  |  | Interdigital, vivo Spreadtrum |  |  |  | vivo Spreadtrum |  |  |  | Vivo, SS, ETRI, ZTE |  |  |  |
|  | Signf(<-5% loss) | SS |  |  |  | ETRI, ZTE, Fujitsu |  |  |  | Interdigital, ETRI, SS |  |  |  |  |  |  |  |

Upd Proposed Observation 4.1.7: For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, 3 sources [Xiaomi, CATT, Interdigital] observe a generalized performance of less than -2% degradation.

  - If UE speed#B is either 30 km/h or 60 km/h or 120 km/h, or if speed#B is 10km/h and speed#A is either 60km/h or 120km/h, 8 sources [Xiaomi, SS, Interdigital, Fujitsu, ZTE, ETRI, vivo, HW] observe that moderate/significant performance degradations are suffered:

    - For UE speed#B is 10 km/h & UE speed#A is either 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-2.7% loss), 1 source [SS] observes significant degradation (-53%~-61% loss).

    - For UE speed#B is 30 km/h & UE speed#A is either 10 km/h, 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-3% loss), 7 sources [Xiaomi, Interdigital, Fujitsu, vivo, ZTE, HW, ETRI] observe significant degradation (-6%~-45.6% loss).

    - For UE speed#B is 60 km/h & UE speed#A is either 10 km/h, 30 km/h or 120 km/h, 1 source [ZTE] observes moderate degradation (-3% loss), 6 sources [SS, Xiaomi, Fujitsu, ETRI, ZTE, vivo] observe significant degradation (-7.8%~-52% loss).

    - For UE speed#B is 120 km/h & UE speed#A is either 30 km/h or 60 km/h, 1 source [ZTE] observes moderate degradation (-3.4% loss), 4 sources [ZTE, ETRI, vivo, SS] observe significant degradation (-7.55%~-32.3% loss).

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 9 sources [Xiaomi, Interdigital, Apple, HW, ZTE, SS, ETRI, vivo, Spreadtrum].

  - For UE speed#B is 10 km/h, minor loss (-0.6%~-1%) are observed by 3 sources [CATT, Xiaomi, Spreadtrum].

  - For UE speed#B is 30 km/h, minor loss (-0.08%~-1.34%) are observed by 3 sources [Xiaomi, Apple, HW], moderate loss (-2.2%~-4.07%) are observed by 3 sources [Interdigital, vivo, Spreadtrum].

  - For UE speed#B is 60 km/h, minor loss (-0.05%~-2%) are observed by 4 sources [ZTE, Apple, Xiaomi, HW], moderate loss (-2%~-3.76%) are observed by 2 sources [vivo, Spreadtrum].

  - For UE speed#B is 120 km/h, moderate loss (-2%~-4.45%) are observed by 4 sources [vivo, SS, ETRI, ZTE].

  - Note: For generalization Case 3, 5 sources [ETRI, ZTE, SS, Interdigital, Fujitsu] observe significant performance degradations (-5%~-26.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - No spatial consistency is considered

| Support/Can accept | Vivo, ETRI |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| vivo | In the previous and current meetings, we have submitted the results relying on both non-spatial consestancy and spatial consestancy. We found that, the gain tendency is the same. It should be captured in the results. |
| Spreadtrum | Our simulation result in R1-2304550 was not captured. Sorry for missing collecting the result. - For generalization Case 2 - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, the result is -2.5% degradation. - If UE speed#B is 10 km/h & UE speed#A is either 60 km/h, the result is -45% degradation. - UE speed#B is 30 km/h & UE speed#A is either 10 km/h or 60 km/h, the result is 0% ~ -29% degradation. - For generalization Case 3 - For UE speed#B is 10 km/h, the loss is 1%. - For UE speed#B is 30 km/h, the loss is 2.2%. - For UE speed#B is 60 km/h, the loss is 2.0%. |
| Moderator | @Sptreadtrum Case 3 captured. Case 2 seems some results are inconsistent. Please fill the entry “Case 2” in the table above by yourselves before Tue 23:59AM (Incheon), otherwise your results may not be captured to the observation in 113. |
| Spreadtrum | We have updated the results in the above table. |

### 4.1-3: Others

#### Issue#4-8 (Low priority) Others

Question 4.1.1: For the evaluation of the AI/ML based CSI prediction sub use cases, what other aspects related with EVM or observation do you think is necessary to be discussed?

| Company | View |
| --- | --- |
| vivo | For AI-based CSI prediction, the performance results evaluated based on the preprocessing-based scaling mechanism (e.g., the stretching/shrinking of historical CSI and prediction CS) should be captured in the table. We may not need to have a table to capture it, but with a simple observation that “Using the preprocessing-based model scaling mechanism, the model trained at one speed can scales to other speeds without notable performance degradation”. |

## 3rd round email discussions

#### Issue#4-3 (Medium priority) Benchmark#1-SGCS gain- Impact of prediction window length

Moderator note[Rd3]: The reference of company names reformulated

| Source#1: ZTE | Source#2: ETRI | Source#3: CMCC | Source#4: OPPO | Source#5: Apple |
| --- | --- | --- | --- | --- |
| Source#6: MediaTek |  |  |  |  |

Proposed Observation 4.2.1: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS/NMSE, from prediction window length perspective, in general the gain of AI/ML based solution is related with the prediction length in terms of the distance to the applicable time of the predicted CSI:

- 4 sources [Source#1, Source#2, Source#3, Source#4] observe the gain over benchmark is reduced with the increase of the prediction length since after 5ms;

- 2 sources [Source#5, Source#6] observe the gain over benchmark is increased when the prediction length is appropriately increased from 2.5ms to 5ms or from 5ms to 10ms, and decreased when the prediction length is further increased from 5ms to 10ms.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value/NMSE in dB value for layer 1.

  - No spatial consistency is considered.

  - No post processing is considered.

  - Source#1: ZTE (R1-2304534); Source#2: ETRI (R1-2305789); Source#3: CMCC (R1-2305085); Source#4: OPPO (R1-2305459); Source#5: Apple (R1-2305234); Source#6: MediaTek (R1-2305655);

  - Note: Results refer to Table x of R1-230xxxx

Table X. Gain of CSI compression over Benchmark#1–Impact of prediction window length

| 30km/h | Prediction window length |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | +2.5ms | +5ms | +7.5ms | +10ms | +15ms |
| ZTE#1 |  | 22.05% |  | 10.04% | 7.48% |
| ZTE#2 |  | 26.47% |  | 23.93% | 13.36% |
| Apple#1 | 7.2442% | 20.2232% | 23.1582% | 10.4327% |  |
| MTK#2 |  | 40.7% |  | 50.3% | 35.5% |
| ETRI#1 |  | 10.43% |  | 8.75% | 7.62% |
| CMCC#5 |  | -41.45dB |  | -37.53dB |  |
| OPPO#1 |  | 6% |  | 5% | 5% |
| vivo #1 | 4.28% (+3ms) | 9.98% | 18.15% (+8ms) | 21.35% |  |

| Support/Can accept | ZTE, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Besides the values, some subjective information is also given as highlighted, with the intention of addressing the applicable cases of AI/ML solutions/options. Please provide your view on whether it is acceptable. |
| vivo | vivo added resuts in the Table X, highlighted by Yellow. Gain of CSI compression prediction over Benchmark#1–Impact of prediction window length, which can be further captured in the observation vivo #1 AL/ML model backbone; 2D-FCN. [Pre-processing]; /. [Post-processing]; /. FLOPs/M for model; 77. FLOPs/M for pre/post processing. Parameters/M; 0.29. Storage /Mbytes. Input type; raw channel. Output type; raw channel. UE speed; 30km/h. CSI feedback periodicity; 5ms. Observation window (number/distance); 10/5ms. Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance); /. Whether/how to adopt spatial consistency; spatial consistency procedure a. Codebook type for CSI report. |

## 4th/5th round email discussions

#### Issue#4-3 (Medium priority) Benchmark#1-SGCS gain- Impact of prediction window length

Moderator note[Rd5]: No change on top of the fourth round.

Proposed Observation 4.3.1: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS/NMSE, from prediction window length perspective, in general the gain of AI/ML based solution is related with the prediction length in terms of the distance to the applicable time of the predicted CSI:

- When the prediction length is increased from 10ms to 15ms, the gain over benchmark is reduced (gap from -1.13%~-14.8%), as observed by 3 sources [ZTE, ETRI, MediaTek].

- When the prediction length is increased from 2.5ms/3ms to 5ms, the gain over benchmark is increased (gap from +5.7%~+13%), as observed by 2 sources [Apple, vivo].

- When the prediction length is increased from 5ms to 10ms, 5 sources [ZTE, Apple, ETRI, CMCC, OPPO] observe the gain over benchmark is reduced (gap from -1%~-9.79% for SGCS, +3.92dB for NMSE) while 2 sources [MediaTek, OPPO] observe the gain over benchmark is increased (+9.6%~+11.55%).

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value/NMSE in dB value for layer 1.

  - No spatial consistency is considered.

  - No post processing is considered.

  - Note: Results refer to Table x of R1-230xxxx

Table X. Gain of CSI prediction over Benchmark#1–Impact of prediction window length

| 30km/h | Prediction window length |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | +2.5ms | +5ms | +7.5ms | +10ms | +15ms |
| ZTE#1 |  | 22.05% |  | 10.04% Gap with +10ms: -12.1% | 7.48% Gap with +10ms: -2.56% |
| ZTE#2 |  | 26.47% |  | 23.93% Gap with +10ms: -2.54% | 13.36% Gap with +10ms: -10.57% |
| Apple#1 | 7.2442% | 20.2232% Gap with +2.5/3ms: 13% | 23.1582% | 10.4327% Gap with +10ms: -9.79% |  |
| MTK#2 |  | 40.7% |  | 50.3% Gap with +10ms: +9.6% | 35.5% Gap with +10ms: -14.8% |
| ETRI#1 |  | 10.43% |  | 8.75% Gap with +10ms: -1.68% | 7.62% Gap with +10ms: -1.13% |
| CMCC#5 |  | -41.45dB |  | -37.53dB Gap with +10ms: +3.92dB |  |
| OPPO#1 |  | 6% |  | 5% Gap with +10ms: -1% | 5% |
| vivo #1 | 4.28% (+3ms) | 9.98% Gap with +2.5/3ms: 5.7% | 18.15% (+8ms) | 21.35% Gap with +10ms: +11.55% |  |

| Support/Can accept | ZTE, vivo |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | vivo added resuts in the Table X, highlighted by Yellow. Gain of CSI compression prediction over Benchmark#1–Impact of prediction window length, which can be further captured in the observation vivo #1 AL/ML model backbone; 2D-FCN. [Pre-processing]; /. [Post-processing]; /. FLOPs/M for model; 77. FLOPs/M for pre/post processing. Parameters/M; 0.29. Storage /Mbytes. Input type; raw channel. Output type; raw channel. UE speed; 30km/h. CSI feedback periodicity; 5ms. Observation window (number/distance); 10/5ms. Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance); /. Whether/how to adopt spatial consistency; spatial consistency procedure a. Codebook type for CSI report. |

# Potential proposals for GTW/offline

## 22 May (Mon.) GTW

#### Issue#4-1 (High priority) Benchmark#1-SGCS gain-Impact of UE speed

Proposed Observation 4.1.1: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, SS, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 4 sources [ZTE, Fujitsu, Apple, Xiaomi] observe 18.72%~28% gain, and 3 sources [InterDigital, MTK, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [HW, SS, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - Results refer to Table 5.1-1 of R1-2306058.

#### Issue#3-12 (High priority) Generalization-Deployment scenarios

Proposed Observation 3.1.13: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - If deployment scenario#A is UMi & deployment scenario#B is UMa, deployment scenario#A is UMa & deployment scenario#B is UMi, or deployment scenario#A is UMa & deployment scenario#B is InH:

    - 9 sources [Xiaomi, InterDigital, MTK, vivo, Intel, ZTE, OPPO, Huawei, CATT] observe that generalized performance can be achieved:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Xiaomi, InterDigital, MTK, vivo, Intel, ZTE, CATT] observe less than -1.6% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 5 sources [vivo, OPPO, MTK, Intel, Xiaomi] observe less than -1.4% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [Huawei, CATT] observe less than -0.6% degradation or positive gain

    - 8 sources [Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT, ZTE, Lenovo] observe that moderate/significant degradations are suffered under generalization Case 2:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Futurewei, Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT] observe larger than -1.6% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 7 sources [Futurewei, NTT DOCOMO, ZTE, InterDigital, CATT, Xiaomi, Intel] observe larger than -1.4% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [ZTE, Lenovo] observe larger than -0.6% degradation.

  - If deployment scenario#A is InH & deployment scenario#B is Uma/UMi, significant performance degradations are observed under generalization Case 2:

    - For deployment scenario#A is InH & deployment scenario#B is UMa, 4 sources [Huawei, CATT, Lenovo, ZTE] observe -5.55%~-21.76% degradation.

    - For deployment scenario#A is InH & deployment scenario#B is UMi, 2 sources [vivo, ZTE] observe -8.63%~-20% degradation.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MTK, Futurewei, vivo, Oppo, Intel, Huawei, ZTE].

  - Minor loss (0%~-1.48%) are observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MTK, Futurewei, vivo, Oppo, Intel, Huawei, ZTE].

  - Moderate loss (-1.6%~-4%) are observed by 5 sources [Xiaomi, CATT, vivo, ZTE, Intel].

  - Positive gains are observed by 8 sources [ZTE, Interdigital, MTK, vivo, Intel, Xiaomi, Futurewei, NTT DOCOMO].

  - Note: Significant degradations of up to -6.7% are still observed by 2 sources [Intel, Xiaomi] for deployment scenario#B subject to UMa, by 2 sources [Intel, CATT] for deployment scenario#B subject to UMi, and by 1 sources [CATT] for deployment scenario#B subject to InH.

- Note: For generalization Case 2, if deployment scenario#A is UMi & deployment scenario#B is InH, 2 sources [vivo, ZTE] observe different trends, where significant performance degradations of -27.8%~-29.9% are observed by [vivo], while moderate performance degradations of -1.44%~-2.41% are observed by [ZTE].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Results refer to Table 5.1-2 of R1-2306058.

## 22 May (Mon.) GTW

#### Issue#4-1 (High priority) Benchmark#1-SGCS gain-Impact of UE speed

Proposed Observation 4.1.1: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, SS, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 4 sources [ZTE, Fujitsu, Apple, Xiaomi] observe 18.72%~28% gain, and 3 sources [InterDigital, MTK, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [HW, SS, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - Results refer to Table 5.1-1 of R1-2306058.

#### Issue#3-12 (High priority) Generalization-Deployment scenarios

Proposed Observation 3.1.13: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - If deployment scenario#A is UMi & deployment scenario#B is UMa, deployment scenario#A is UMa & deployment scenario#B is UMi, or deployment scenario#A is UMa & deployment scenario#B is InH:

    - 9 sources [Xiaomi, InterDigital, MTK, vivo, Intel, ZTE, OPPO, Huawei, CATT] observe that generalized performance can be achieved:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Xiaomi, InterDigital, MTK, vivo, Intel, ZTE, CATT] observe less than -1.6% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 5 sources [vivo, OPPO, MTK, Intel, Xiaomi] observe less than -1.4% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [Huawei, CATT] observe less than -0.6% degradation or positive gain

    - 8 sources [Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT, ZTE, Lenovo] observe that moderate/significant degradations are suffered under generalization Case 2:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Futurewei, Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT] observe larger than -1.6% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 7 sources [Futurewei, NTT DOCOMO, ZTE, InterDigital, CATT, Xiaomi, Intel] observe larger than -1.4% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [ZTE, Lenovo] observe larger than -0.6% degradation.

  - If deployment scenario#A is InH & deployment scenario#B is Uma/UMi, significant performance degradations are observed under generalization Case 2:

    - For deployment scenario#A is InH & deployment scenario#B is UMa, 4 sources [Huawei, CATT, Lenovo, ZTE] observe -5.55%~-21.76% degradation.

    - For deployment scenario#A is InH & deployment scenario#B is UMi, 2 sources [vivo, ZTE] observe -8.63%~-20% degradation.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MTK, Futurewei, vivo, Oppo, Intel, Huawei, ZTE].

  - Minor loss (0%~-1.48%) are observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MTK, Futurewei, vivo, Oppo, Intel, Huawei, ZTE].

  - Moderate loss (-1.6%~-4%) are observed by 5 sources [Xiaomi, CATT, vivo, ZTE, Intel].

  - Positive gains are observed by 8 sources [ZTE, Interdigital, MTK, vivo, Intel, Xiaomi, Futurewei, NTT DOCOMO].

  - Note: Significant degradations of up to -6.7% are still observed by 2 sources [Intel, Xiaomi] for deployment scenario#B subject to UMa, by 2 sources [Intel, CATT] for deployment scenario#B subject to UMi, and by 1 sources [CATT] for deployment scenario#B subject to InH.

- Note: For generalization Case 2, if deployment scenario#A is UMi & deployment scenario#B is InH, 2 sources [vivo, ZTE] observe different trends, where significant performance degradations of -27.8%~-29.9% are observed by [vivo], while moderate performance degradations of -1.44%~-2.41% are observed by [ZTE].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Results refer to Table 5.1-2 of R1-2306058.

## 23 May (Tue.) Offline/GTW

#### Issue#4-1 (High priority) Benchmark#1-SGCS gain-Impact of UE speed

Upd Proposed Observation 4.1.1: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, Samsung, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 5 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] observe 18.72%~31.3% gain, and 3 sources [InterDigital, MediaTek, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [Huawei, Samsung, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - No spatial consistency is considered by 11 sources [Fujitsu, Samsung, Xiaomi, InterDigital, CMCC, OPPO, ETRI, ZTE, Apple, Huawei, Spreadtrum]. 1 source [vivo] provides both results with spatial consistency and results w/o spatial consistency.

  - Note: Results refer to Table 5.1-1 of R1-2306059

Table 5.1-1 SGCS gain of CSI prediction over Benchmark#1-Impact of UE speed

| OB window | 3/5ms~ 5/5ms |  |  | 6/5ms~10/5ms |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 10km/h | 30km/h | 60km/h | 10km/h | 30km/h | 60km/h |
| HW#1 |  |  | 18% |  |  |  |
| ZTE#1/3 |  | 18.72%~ 22.05% |  |  | 25.51%~ 26.47% |  |
| SS#1/2 | 2.4% |  | 11.2% |  |  |  |
| Fujitsu#1/2/3 | L1:2.6% L2:4% | L1:28% L2:35% | L1:5% L2:3.4% |  |  |  |
| Apple#1 |  | L1: 20.2232% L2: 27.0090% |  |  |  |  |
| vivo#1 |  |  |  |  |  | 13.8% |
| InterDigital#1~ 3 | L1: 6% L2: 9% | L1: 35% L2: 46% | L1: -3% L2: -4% |  |  |  |
| MTK#1 |  | 40.7% |  |  |  |  |
| ETRI#1 |  | 10.43% |  |  |  |  |
| Xiaomi#1~6 | 1.03% | 20.25% | 66.02% | 1.03% | 20.57% | 66.55% |
| CMCC#6/8/9 | 21.93% | 41.75% | 19.98% |  |  |  |
| OPPO#1 |  | 6% |  |  |  |  |
| Spreadtrum#1 |  | 31.3% |  |  |  |  |
| Summary | 10km/h |  | 30km/h |  | 60km/h |  |
|  | 4 sources:1.03%~6%; [Fujitsu, SS, Xiaomi, InterDigital] 1 source: 21.93% [CMCC] |  | 2 sources: 6%~10.43% [OPPO, ETRI] 5 sources: 18.72%~31.3% [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] 3 sources: 35%~ 41.75% [InterDigital, MTK, CMCC] |  | 2 sources: -3%~5% [Fujitsu, InterDigital] 4 sources: 11.2%~19.98% [HW, SS, vivo, CMCC] |  |

#### Issue#3-1 (High priority) SGCS gain over benchmark

Upd Proposed Observation 3.1.1: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of SGCS,

- For Max rank 1, Layer 1,

  - 11 sources [Huawei, Nokia, ZTE, vivo, OPPO, ETRI, Fujitsu, CMCC, China Telecom, MediaTek, Apple] observe the performance gain of 2.6%~ 8.8% at CSI payload X (small payload);

  - 14 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, ETRI, Fujitsu, NTT DOCOMO, Xiaomi, China Telecom, MediaTek, BJTU, Apple] observe the performance gain of 0.9%~ 8.1% at CSI payload Y (medium payload);

  - 11 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, ETRI, Fujitsu, BJTU, Apple] observe the performance gain of 0.9%~ 7% at CSI payload Z (large payload);

  - Note: 1 source [Futurewei] observes the performance gain of 11.6% at CSI payload X (small payload) which biases from the majority range.

- For Max rank 2, Layer 1,

  - 12 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 3.9%~ 11% at CSI payload X (small payload);

  - 11 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Qualcomm, Apple] observe the performance gain of 0.7%~ 4.5% at CSI payload Y (medium payload);

  - 9 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Apple] observe the performance gain of -0.2%~ 6.5% at CSI payload Z (large payload);

- For Max rank 2, Layer 2, more gains are observed in general compared with Layer 1 of Max rank 2:

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 5.92%~ 30.2% at CSI payload X (small payload);

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 1.5%~ 23.08% at CSI payload Y (medium payload);

  - 10 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Intel, Apple] observe the performance gain of 4.4%~ 12.99% at CSI payload Z (large payload);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1 of Max rank 1 or Layer 1/2 of Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-2 of R1-2306059

Table 5.1-2 Gain of CSI comporession over R16 Type II benchmark - SGCS

|  |  | SGCS L1, R1 | SGCS L1, R2 | SGCS L1, R4 | SGCS L2, R2 | SGCS L2, R4 | SGCS L3, R4 | SGCS L4, R4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW#1 (FTP) TF | X | 8.8% | 8.8% |  | 14% |  |  |  |
|  | Y | 4.5% | 4.5% |  | 7% |  |  |  |
|  | Z | 6.5% | 6.5% |  | 9% |  |  |  |
| Nokia #1 (FTP/FB) TF | X | 5.6% | 4.2% |  | 14.3% |  |  |  |
|  | Y | 1.2% | 1.3% |  | 3.6% |  |  |  |
|  | Z | 2.2% | -0.2% |  | 5.9% |  |  |  |
| Futurewei#1 (FTP) | X | 10.2% | 4.04% |  | 1.13% |  |  |  |
|  | Y | 4.4% | 3.13% |  | 1.45% |  |  |  |
|  | Z | 4.6% | 1.4% |  | -0.39% |  |  |  |
| Futurewei#2 (FTP) | X | 11.6% |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| Lenovo#1 (FTP) TF | X |  |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |
|  | Z | 0.9% |  |  |  |  |  |  |
| ZTE#1 (FTP/FB) TF | X | 6.43% | 4.67% | 3.96% | 5.92% | 6.77% | 32.12% | 91.85% |
|  | Y | 6.44% | 4.04% | 3.11% | 5.90% | 3.72% | 15.02% | 37.16% |
|  | Z | 6.97% | 6.44% | 5.64% | 9.95% | 4.79% | 14.92% | 31.94% |
| Vivo#1 (FTP) TF | X | 4.97% |  |  |  |  |  |  |
|  | Y | 3.54% |  |  |  |  |  |  |
|  | Z | 6.44% |  |  |  |  |  |  |
| Vivo#2 (FB) TF | X | 6.09% | 6.08% |  | 17.8% |  |  |  |
|  | Y | 3.61% | 3.59% |  | 9.23% |  |  |  |
|  | Z | 6.28% | 6.32% |  | 12.19% |  |  |  |
| OPPO#1 (FTP/FB) TF | X | 8.45% | 8.45% |  | 30.2% |  |  |  |
|  | Y | 3.8% | 3.8% |  | 23.08% |  |  |  |
|  | Z | 3.45% | 3.45% |  | 12.99% |  |  |  |
| ETRI#1 (FTP) TF | X | 4.04% |  |  |  |  |  |  |
|  | Y | 1.9% |  |  |  |  |  |  |
|  | Z | 3.6% |  |  |  |  |  |  |
| Spreadtrum#1(FTP) TF | X | 18.7% |  |  |  |  |  |  |
|  | Y | 21.27% |  |  |  |  |  |  |
|  | Z | 19.47% |  |  |  |  |  |  |
| Fujitsu#1 (FTP/FB) TF | X | 2.6% | 7.7% |  | 11.9% |  |  |  |
|  | Y | 0.9% | 2.9% |  | 6.2% |  |  |  |
|  | Z | 2.1% | 3.6% |  | 7.8% |  |  |  |
| NTT DOCOMO#1(FTP) TF | X | 13.5% | 11.5% |  | 12.2% |  |  |  |
|  | Y | 7.9% | 1.6% |  | 1.9% |  |  |  |
|  | Z | 9.3% | 9.3% |  | 10.6% |  |  |  |
| CMCC (FTP) TF | X | 4.74% |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| E///#1 (FTP) ResNet-CNN | X |  | 3.9% |  | 10.9% |  |  |  |
|  | Y |  | 0.7% |  | 1.5% |  |  |  |
|  | Z |  | 2.2% |  | 4.4% |  |  |  |
| Xiaomi#1 (FB) TF | X |  | 9.69% |  | 9.68% |  |  |  |
|  | Y | 7.31% | 14.19% |  | 16.46% |  |  |  |
|  | Z |  | 18.12% |  | 22.96% |  |  |  |
| QC#1 (FTP/FB) TF | X |  | 11% | 17% | 26% | 25% | 101% | 177% |
|  | Y |  | 4% | 20% | 8% | 18% | 18% | 16% |
|  | Z |  |  | 11% |  | 6% | -1% | -5% |
| CATT (FTP) TF | X |  |  | 7.84% |  | 54.3% | 66.3% | 27.3% |
|  | Y |  |  | 4.32% |  | 42.2% |  |  |
|  | Z |  |  |  |  |  |  |  |
| Apple#1 (FTP) TF | X | 7% | 7% | 11% | 12% | 13% | 92% | 166% |
|  | Y | 4% | 4% | 6% | 4% | -2% | 7% | 10% |
|  | Z | 6% | 6% | 5% | 2% | -5% | -7% | -7% |
| China Telecom#1 (FB) TF | X | 7.7% 4.2% |  |  |  |  |  |  |
|  | Y | 8.1% |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| MTK#1 (FB) TF | X | 3.4% |  |  |  |  |  |  |
|  | Y | 1.7% |  |  |  |  |  |  |
|  | Z |  |  |  |  |  |  |  |
| Intel#1 (FTP/FB) TF | X |  | 8% |  | 10% |  |  |  |
|  | Y |  | 12% |  | 10% |  |  |  |
|  | Z |  | 8% |  | 12% |  |  |  |
| BJTU CsiNet+ | X |  |  |  |  |  |  |  |
|  | Y | 8% |  |  |  |  |  |  |
|  | Z | 7% |  |  |  |  |  |  |
| Benchmark SGCS | X | 0.67~0.72 | 0.66~0.72 | 0.59~0.74 | 0.48~0.56 | 0.34~0.56 | 0.15~0.44 | 0.11~0.34 |
|  | Y | 0.74~0.82 | 0.76~0.82 | 0.72~0.79 | 0.61~0.7 | 0.42~0.64 | 0.5~0.51 | 0.38~0.40 |
|  | Z | 0.83~0.87 | 0.83~0.86 | 0.80~0.88 | 0.73~0.79 | 0.70~0.76 | 0.59~0.61 | 0.48~0.51 |
| Summary | X | 2.6%~ 8.8% | 3.9%~ 11% | 3.64%~ 17% | 5.92%~ 30.2% | 6.77%~ 54.3% | 32.12%~101% | 72%~ 177% |
|  | Y | 0.9%~ 8.1% | 0.7%~ 4.5% | 3.11%~ 20% | 1.5%~ 23.08% | 3.72%~ 42.2% | 8%~ 18% | 16%~ 37.16% |
|  | Z | 0.9%~ 7% | -0.2% 6.5% | 5%~ 11% | 4.4%~ 12.99% | 2%~ 6% | -1%~ 17% | -5%~ 31.94% |

#### Issue#3-3 (High priority) Mean UPT, FTP traffic

Proposed Observation 3.1.3: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under FTP traffic, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~2%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.29%~2% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~1% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.33%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.09%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4% at CSI overhead C (large overhead);

  - For RU>=70%, 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.23%~9%

    - 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.38%~9% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.62%~5% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.23%~6% at CSI overhead C (large overhead);

  - Note: 2 sources [Spreadtrum, Futurewei] observe gain of 12.77%~21.21% at RU 40%-69%, 11.23%~21.5% at RU>=70%, which bias from the majority ranges.

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 6 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm, Fujitsu] observe the performance gain of -0.3%~6%

    - 5 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm] observe the performance gain of 1%~6% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.5%~6% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Intel, Fujitsu, Qualcomm] observe the performance gain of -0.3%~6% at CSI overhead C (large overhead);

  - For RU 40%-69%, 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm, InterDigital] observe the performance gain of -0.5%~10%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 3%~10% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 1.2%~9% at CSI overhead B (medium overhead);

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm, InterDigital] observe the performance gain of -0.5%~9% at CSI overhead C (large overhead);

  - For RU>=70%, 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of -0.2%~15%

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 5%~15% at CSI overhead A (small overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 3%~9% at CSI overhead B (medium overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Fujitsu, Qualcomm] observe the performance gain of -0.2%~12% at CSI overhead C (large overhead);

  - Note: 4 sources [Futurewei, NTT DOCOMO, InterDigital, Fujitsu] observe gain of 7%~30% at RU<=39%, 10%~23% at RU 40%-69%, 12.71%~26.8% at RU>=70%, which bias from the majority ranges.

- For Max rank 4:

  - For RU<=39%, 3 sources [CATT, Apple, Qualcomm] observe the performance gain of -4%~7.4%

    - 3 sources [CATT, Apple, Qualcomm] observe the performance gain of 2.5%~7.4% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 6% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -4%~0% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~12.22%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~12.22% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 7.04%~11% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~8.19% at CSI overhead C (large overhead);

  - For RU>=70%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~17%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.64%~17% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~8.40% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-3 of R1-2306059

Table 5.1-3 Gain of CSI comporession over R16 Type II benchmark – Mean UPT, FTP

|  | RU | CSI overhead |  | R1 |  | R2 |  | R4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW#1 | <=39% | A |  | 2% |  | 6% |  |  |
|  |  | B |  | 1% |  | 2% |  |  |
|  |  | C |  | 1% |  | 3% |  |  |
|  | 40%-69% | A |  | 3% |  | 10% |  |  |
|  |  | B |  | 2% |  | 5% |  |  |
|  |  | C |  | 4% |  | 6% |  |  |
|  | >=70% | A |  | 9% |  | 15% |  |  |
|  |  | B |  | 5% |  | 8% |  |  |
|  |  | C |  | 6% |  | 11% |  |  |
| Nokia#1 | <=39% | A |  | 1% |  | 2% |  |  |
|  |  | B |  | 0.5% |  | 0.5% |  |  |
|  |  | C |  | 0.6% |  | -0.3% |  |  |
|  | 40%-69% | A |  | 2.5% |  | 4.3% |  |  |
|  |  | B |  | 1.6% |  | 1.2% |  |  |
|  |  | C |  | 0.1% |  | -0.5% |  |  |
|  | >=70% | A |  | 4.5% |  | 13% |  |  |
|  |  | B |  | 3.2% |  | 6.8% |  |  |
|  |  | C |  | 1.0% |  | -0.2% |  |  |
| ZTE#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  | 4.09% |  | 12.22% |
|  |  | B |  |  |  | 4.08% |  | 7.04% |
|  |  | C |  |  |  | 4.55% |  | 8.19% |
|  | >=70% | A |  | 0.38% |  | 5.30% |  | 14.89% |
|  |  | B |  | 0.62% |  | 4.92% |  | 6.64% |
|  |  | C |  | 0.23% |  | 6.94% |  | 8.40% |
| Vivo#1 | <=39% | A |  | 0.29% |  |  |  |  |
|  |  | B |  | 0.20% |  |  |  |  |
|  |  | C |  | 0.33% |  |  |  |  |
|  | 40%-69% | A |  | 1.09% |  |  |  |  |
|  |  | B |  | 0.80% |  |  |  |  |
|  |  | C |  | 1.30% |  |  |  |  |
|  | >=70% | A |  | 2.27% |  |  |  |  |
|  |  | B |  | 1.66% |  |  |  |  |
|  |  | C |  | 2.28% |  |  |  |  |
| OPPO#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 3% |  | 11% |  |  |
|  |  | B |  | 1% |  | 3% |  |  |
|  |  | C |  | 1% |  | 1% |  |  |
| E///#1 | <=39% | A |  |  |  | 1% |  |  |
|  |  | B |  |  |  | 1% |  |  |
|  |  | C |  |  |  | 0% |  |  |
|  | 40%-69% | A |  |  |  | 3% |  |  |
|  |  | B |  |  |  | 4% |  |  |
|  |  | C |  |  |  | 1% |  |  |
|  | >=70% | A |  |  |  | 5% |  |  |
|  |  | B |  |  |  | 6% |  |  |
|  |  | C |  |  |  | 2% |  |  |
| QC#1 | <=39% | A |  |  |  | 1% |  | 6% |
|  |  | B |  |  |  | 1% |  | 6% |
|  |  | C |  |  |  | 2% |  | 0% |
|  | 40%-69% | A |  |  |  | 4% |  | 11% |
|  |  | B |  |  |  | 4% |  | 11% |
|  |  | C |  |  |  | 1% |  | 3% |
|  | >=70% | A |  |  |  | 12% |  | 17% |
|  |  | B |  |  |  | 8% |  | 17% |
|  |  | C |  |  |  | 2% |  | 6% |
| CATT#1/#2 | <=39% | A |  |  |  |  |  | 7.4% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
| Apple#1 | <=39% | A |  |  |  |  |  | 2.5% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -4% |
|  | 40%-69% | A |  |  |  |  |  | 3% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1.8% |
|  | >=70% | A |  |  |  |  |  | 3% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1% |
| Futurewei#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 21.5% |  | 9.65% |  |  |
|  |  | B |  | 8.1% |  | 8.4% |  |  |
|  |  | C |  | 10.1% |  | 12.71% |  |  |
| Spreadtrum#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  | 12.77% |  |  |  |  |
|  |  | B |  | 15.18% |  |  |  |  |
|  |  | C |  | 21.21% |  |  |  |  |
|  | >=70% | A |  | 7.47% |  |  |  |  |
|  |  | B |  | 11.23% |  |  |  |  |
|  |  | C |  | 11.44% |  |  |  |  |
| Intel#1 | <=39% | A |  |  |  | 3% |  |  |
|  |  | B |  |  |  | 6% |  |  |
|  |  | C |  |  |  | 6% |  |  |
|  | 40%-69% | A |  |  |  | 4% |  |  |
|  |  | B |  |  |  | 9% |  |  |
|  |  | C |  |  |  | 9% |  |  |
|  | >=70% | A |  |  |  | 6% |  |  |
|  |  | B |  |  |  | 9% |  |  |
|  |  | C |  |  |  | 10% |  |  |
| NTT DOCOMO#1 | <=39% | A |  |  |  | 19.9% |  |  |
|  |  | B |  |  |  | 11.5% |  |  |
|  |  | C |  |  |  | 7% |  |  |
|  | 40%-69% | A |  |  |  | 19.7% |  |  |
|  |  | B |  |  |  | 22% |  |  |
|  |  | C |  |  |  | 16.3% |  |  |
|  | >=70% | A |  |  |  | 23% |  |  |
|  |  | B |  |  |  | 24.6% |  |  |
|  |  | C |  |  |  | 26.8% |  |  |
| InterDigital#1 | <=39% | A |  |  |  | 30% |  |  |
|  |  | B |  |  |  | 25% |  |  |
|  |  | C |  |  |  | 21% |  |  |
|  | 40%-69% | A |  |  |  | 23% |  |  |
|  |  | B |  |  |  | 21% |  |  |
|  |  | C |  |  |  | 8% |  |  |
|  | >=70% | A |  |  |  | 10% |  |  |
|  |  | B |  |  |  | 9% |  |  |
|  |  | C |  |  |  | 5% |  |  |
| Fujitsu | <=39% | A |  |  |  | 8.80% |  |  |
|  |  | B |  |  |  | 9.50% |  |  |
|  |  | C |  |  |  | 5.5% |  |  |
|  | 40%-69% | A |  |  |  | 15% |  |  |
|  |  | B |  |  |  | 20.80% |  |  |
|  |  | C |  |  |  | 10.00% |  |  |
|  | >=70% | A |  |  |  | 15.80% |  |  |
|  |  | B |  |  |  | 20.20% |  |  |
|  |  | C |  |  |  | 12% |  |  |
| Summary | <=39% | A | 0.29%~2% |  | 1%~6% |  | 2.5%~7.4% |  |
|  |  | B | 0.2%~1% |  | 0.5%~6% |  | 6% |  |
|  |  | C | 0.33%~1% |  | -0.3%~6% |  | -4%~0% |  |
|  | 40%-69% | A | 1.09%~3% |  | 3%~10% |  | 3%~12.22% |  |
|  |  | B | 0.80%~2% |  | 1.2%~9% |  | 7.04~11% |  |
|  |  | C | 0.1%~4% |  | -0.5%~9% |  | -1.8~8.19% |  |
|  | >=70% | A | 0.38%~9% |  | 5%~15% |  | 3%~17% |  |
|  |  | B | 0.62%~5% |  | 3%~9% |  | 6.64~17% |  |
|  |  | C | 0.23%~6% |  | -0.2%~12% |  | -1%~6.88% |  |

#### Issue#3-4 (High priority) 5% UPT, FTP traffic

Proposed Observation 3.1.4: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.8%~3%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.72%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~1.2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.68%~3% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~7%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 2.8%~7% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.22%~2.7% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~3.25% at CSI overhead C (large overhead);

  - For RU>=70%, 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~20.43%

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 4%~20.43% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 1%~10.13% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~8% at CSI overhead C (large overhead);

  - Note: 2 sources [Spreadtrum, Futurewei] observe gain of 15.87%~21.04% at RU 40%-69%, 20.2%~50% at RU>=70%, which bias from the majority ranges.

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -2%~5%

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 1.1%~5% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of -2%~3% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -0.5%~5% at CSI overhead C (large overhead);

  - For RU 40%-69%, 7 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel, Fujitsu] observe the performance gain of -4%~13%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 7%~13% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.3%~8% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu] observe the performance gain of -4%~8% at CSI overhead C (large overhead);

  - For RU>=70%, 9 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO, Qualcomm, Futurewei] observe the performance gain of -1.3%~24%

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO] observe the performance gain of 10.26%~24% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 9%~15.02% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Futurewei, Intel] observe the performance gain of -1.3%~13.67% at CSI overhead C (large overhead);

  - Note: 5 sources [Intel, NTT DOCOMO, InterDigital, Fujitsu, ZTE] observe gain of 7%~24% at RU<=39%, -8%~-2%, 13.4%~29.7% at RU 40%-69%, -5%~-10%, 18.1%~35.4% at RU>=70%, which bias from the majority ranges.

- For Max rank 4:

  - For RU<=39%, 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~10%

    - 2 sources [Apple, Qualcomm] observe the performance gain of 8%~10% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 5% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~23%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.17%~23% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~9.47% at CSI overhead C (large overhead);

  - For RU>=70%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~31%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5.8%~31% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 10.2%~30% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~15% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-4 of R1-2306059

Table 5.1-4 Gain of CSI comporession over R16 Type II benchmark – 5% UPT, FTP

|  | RU | CSI overhead |  | R1 |  | R2 |  | R4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW#1 | <=39% | A |  | 3% |  | 5% |  |  |
|  |  | B |  | 1% |  | 3% |  |  |
|  |  | C |  | 3% |  | 5% |  |  |
|  | 40%-69% | A |  | 7% |  | 13% |  |  |
|  |  | B |  | 2% |  | 4% |  |  |
|  |  | C |  | 3% |  | 8% |  |  |
|  | >=70% | A |  | 16% |  | 24% |  |  |
|  |  | B |  | 7% |  | 9% |  |  |
|  |  | C |  | 8% |  | 12% |  |  |
| Nokia#1 | <=39% | A |  | 1.9% |  | 1.1% |  |  |
|  |  | B |  | 1.2% |  | -2% |  |  |
|  |  | C |  | 1.7% |  | -0.5% |  |  |
|  | 40%-69% | A |  | 2.8% |  | 7.2% |  |  |
|  |  | B |  | 2.7% |  | 0.3% |  |  |
|  |  | C |  | 0.1% |  | -4% |  |  |
|  | >=70% | A |  | 9.5% |  | 22.5% |  |  |
|  |  | B |  | 3.6% |  | 11.4% |  |  |
|  |  | C |  | 1.7% |  | -1.3% |  |  |
| ZTE#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  | 9.72% |  | 9.25% |
|  |  | B |  |  |  | 20% |  | 6.17% |
|  |  | C |  |  |  | 25.62% |  | 9.47% |
|  | >=70% | A |  | 20.43% |  | 10.26% |  | 23.27% |
|  |  | B |  | 10.13% |  | 15.02% |  | 10.2% |
|  |  | C |  | 0.85% |  | 13.67% |  | 6.83% |
| Vivo#1 | <=39% | A |  | 1.72% |  |  |  |  |
|  |  | B |  | 0.80% |  |  |  |  |
|  |  | C |  | 1.68% |  |  |  |  |
|  | 40%-69% | A |  | 4.13% |  |  |  |  |
|  |  | B |  | 1.22% |  |  |  |  |
|  |  | C |  | 3.25% |  |  |  |  |
|  | >=70% | A |  | 5.45% |  |  |  |  |
|  |  | B |  | 1.68% |  |  |  |  |
|  |  | C |  | 4.28% |  |  |  |  |
| OPPO#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 4% |  |  |  |  |
|  |  | B |  | 1% |  |  |  |  |
|  |  | C |  | 1% |  |  |  |  |
| E///#1 | <=39% | A |  |  |  | 2% |  |  |
|  |  | B |  |  |  | 2% |  |  |
|  |  | C |  |  |  | 0% |  |  |
|  | 40%-69% | A |  |  |  | 7% |  |  |
|  |  | B |  |  |  | 5% |  |  |
|  |  | C |  |  |  | 3% |  |  |
|  | >=70% | A |  |  |  | 12% |  |  |
|  |  | B |  |  |  | 12% |  |  |
|  |  | C |  |  |  | 2% |  |  |
| QC#1 | <=39% | A |  |  |  | 5% |  | 10% |
|  |  | B |  |  |  | 3% |  | 5% |
|  |  | C |  |  |  | 4% |  | 1% |
|  | 40%-69% | A |  |  |  | 7% |  | 17% |
|  |  | B |  |  |  | 8% |  | 23% |
|  |  | C |  |  |  | -1% |  | 5% |
|  | >=70% | A |  |  |  | 18% |  | 31% |
|  |  | B |  |  |  | 11% |  | 30% |
|  |  | C |  |  |  | 10% |  | 15% |
| Apple#1 | <=39% | A |  |  |  |  |  | 8% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1.60% |
|  | 40%-69% | A |  |  |  |  |  | 5% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | -1.70% |
|  | >=70% | A |  |  |  |  |  | 5.80% |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  | 2% |
| Futurewei#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | >=70% | A |  | 50% |  | 0% |  |  |
|  |  | B |  | 24% |  | 7.69% |  |  |
|  |  | C |  | 23% |  | 9.87% |  |  |
| Spreadtrum#1 | <=39% | A |  |  |  |  |  |  |
|  |  | B |  |  |  |  |  |  |
|  |  | C |  |  |  |  |  |  |
|  | 40%-69% | A |  | 21.04% |  |  |  |  |
|  |  | B |  | 18.61% |  |  |  |  |
|  |  | C |  | 15.87% |  |  |  |  |
|  | >=70% | A |  | 34.83% |  |  |  |  |
|  |  | B |  | 27.23% |  |  |  |  |
|  |  | C |  | 20.2% |  |  |  |  |
| Intel#1 | <=39% | A |  |  |  | 8% |  |  |
|  |  | B |  |  |  | 8% |  |  |
|  |  | C |  |  |  | 9% |  |  |
|  | 40%-69% | A |  |  |  | 8% |  |  |
|  |  | B |  |  |  | 15% |  |  |
|  |  | C |  |  |  | 20% |  |  |
|  | >=70% | A |  |  |  | 3% |  |  |
|  |  | B |  |  |  | 11% |  |  |
|  |  | C |  |  |  | 10% |  |  |
| NTT DOCOMO#1 | <=39% | A |  |  |  | 23.3% |  |  |
|  |  | B |  |  |  | 17.7% |  |  |
|  |  | C |  |  |  | 19.9% |  |  |
|  | 40%-69% | A |  |  |  | 13.4% |  |  |
|  |  | B |  |  |  | 29.7% |  |  |
|  |  | C |  |  |  | 22.5% |  |  |
|  | >=70% | A |  |  |  | 21.6% |  |  |
|  |  | B |  |  |  | 23.1% |  |  |
|  |  | C |  |  |  | 35.4% |  |  |
| InterDigital#1 | <=39% | A |  |  |  | 9% |  |  |
|  |  | B |  |  |  | 7% |  |  |
|  |  | C |  |  |  | 2% |  |  |
|  | 40%-69% | A |  |  |  | 5% |  |  |
|  |  | B |  |  |  | -2% |  |  |
|  |  | C |  |  |  | -8% |  |  |
|  | >=70% | A |  |  |  | -5% |  |  |
|  |  | B |  |  |  | -10% |  |  |
|  |  | C |  |  |  | -10% |  |  |
| Fujitsu | <=39% | A |  |  |  | 24% |  |  |
|  |  | B |  |  |  | 12.6% |  |  |
|  |  | C |  |  |  | 3.80% |  |  |
|  | 40%-69% | A |  |  |  | 25.80% |  |  |
|  |  | B |  |  |  | 17.80% |  |  |
|  |  | C |  |  |  | 7.90% |  |  |
|  | >=70% | A |  |  |  | 12.40% |  |  |
|  |  | B |  |  |  | 18.60% |  |  |
|  |  | C |  |  |  | 18.10% |  |  |
| Summary | <=39% | A | 1.72%~3% |  | 1.1%~5% |  | 8%~10% |  |
|  |  | B | 0.80%~1.2% |  | -2%~3% |  | 5% |  |
|  |  | C | 1.68%~3% |  | -0.5%~5% |  | -1.6%~1% |  |
|  | 40%-69% | A | 2.8%~7% |  | 7%~13% |  | 5%~17% |  |
|  |  | B | 1.22%~2.7% |  | 0.3%~8% |  | 6.17%~23% |  |
|  |  | C | 0.1%~3.25% |  | -4%~8% |  | -1.7%~9.47 |  |
|  | >=70% | A | 4%~20.43% |  | 10.26%~24% |  | 5.8%~31% |  |
|  |  | B | 1%~10.13% |  | 9%~15.02% |  | 10.2%~30% |  |
|  |  | C | 0.85%~8% |  | -1.3%~13.67% |  | 2%~15% |  |

#### Issue#3-12 (High priority) Generalization-Deployment scenarios

Upd Proposed Observation 3.1.13: For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - If deployment scenario#A is UMi & deployment scenario#B is UMa, deployment scenario#A is UMa & deployment scenario#B is UMi, or deployment scenario#A is UMa & deployment scenario#B is InH:

    - 9 sources [Xiaomi, InterDigital, MediaTek, vivo, Intel, ZTE, OPPO, Huawei, CATT] observe that generalized performance can be achieved:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Xiaomi, InterDigital, MediaTek, vivo, Intel, ZTE, CATT] observe less than -1.6% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 5 sources [vivo, OPPO, MediaTek, Intel, Xiaomi] observe less than -1.4% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [Huawei, CATT] observe less than -0.6% degradation or positive gain

    - 10 sources [Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT, ZTE, Lenovo, MediaTek, Futurewei] observe that moderate/significant degradations are suffered under generalization Case 2:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 8 sources [Futurewei, MediaTek, Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT] observe -1.69%~-14.2% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 7 sources [Futurewei, NTT DOCOMO, ZTE, InterDigital, CATT, Xiaomi, Intel] observe -1.81%~-18.5% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [ZTE, Lenovo] observe -1.74%~-3.6% degradation.

  - If deployment scenario#A is InH & deployment scenario#B is Uma/UMi, significant performance degradations are observed under generalization Case 2:

    - For deployment scenario#A is InH & deployment scenario#B is UMa, 4 sources [Huawei, CATT, Lenovo, ZTE] observe -5.55%~-21.76% degradation.

    - For deployment scenario#A is InH & deployment scenario#B is UMi, 2 sources [vivo, ZTE] observe -8.63%~-20% degradation.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MediaTek, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Minor loss (0%~-1.48%) are observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MediaTek, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Moderate loss (-1.6%~-4%) are observed by 5 sources [Xiaomi, CATT, vivo, NTT DOCOMO, Intel].

  - Positive gains are observed by 8 sources [ZTE, Interdigital, MediaTek, vivo, Intel, Xiaomi, Futurewei, CATT].

  - Note: Significant degradations of up to -6.7% are still observed by 2 sources [Intel, Xiaomi] for deployment scenario#B subject to UMa, and by 2 sources [Intel, CATT] for deployment scenario#B subject to UMi.

- Note: For generalization Case 2, if deployment scenario#A is UMi & deployment scenario#B is InH, 2 sources [vivo, ZTE] observe different trends, where significant performance degradations of -27.8%~-29.9% are observed by [vivo], while moderate performance degradations of -1.44%~-2.41% are observed by [ZTE].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.1-5 of R1-2306059

Table 5.1-5 Generalization of CSI compression-Deployment scenarios

| Training, Generalization Case 1 |  |  | UMa |  |  | UMi |  |  | InH |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | UMi | InH |  | UMa | InH |  | UMa | UMi |
| Training, Generalization Case 3 |  |  | Uma+ UMi | Uma+ InH | Uma+ UMi+ InH | Uma+ UMi | Umi+ InH | Uma+ UMi+ InH | Uma+ InH | Umi+ InH | Uma+ UMi+ InH |
| Testing |  |  | UMa |  |  | UMi |  |  | InH |  |  |
|  | Case |  |  |  |  |  |  |  |  |  |  |
| HW#1/#2 | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | 0.916 |  |  |  |  |  | 0.968 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  | -7% |  |  |  |  | -0.6% |  |
|  | Case 3 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  | -1% |  |  |  |  | -0.4% |  |  |
| Futurewei#1#2 | Case 1 | X | 0.92 |  |  | 0.936 |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | X |  | -14.2% |  |  | -18.5% |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 3 | X | 1.8% |  |  | -0.6% |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| Lenovo#1#2 | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | 77.2 |  |  |  |  |  | 96.4 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  | -21.76% |  |  |  |  | -3.6% |  |
|  | Case 3 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| Intel#1/#2 | Case 1 | X | L1:0.7103 L2:0.5290 |  |  | L1:0.7208 L2:0.5610 |  |  |  |  |  |
|  |  | Y | L1:0.8431 L2:0.7143 |  |  | L1:0.8603 L2:7273 |  |  |  |  |  |
|  |  | Z | L1:0.9166 L2:0.8415 |  |  | L1:0.9320 L2:0.8600 |  |  |  |  |  |
|  | Case 2 | X |  | L1:-3.8% L2:-4.6% |  |  | L1:-2.5% L2:-8.1% |  |  |  |  |
|  |  | Y |  | L1:-1.6% L2:-2.4% |  |  | L1:-2.5% L2:-3.6% |  |  |  |  |
|  |  | Z |  | L1:-0.4% L2:-3.1% |  |  | L1:-1.2% L2:-1.4% |  |  |  |  |
|  | Case 3 | X | L1:2.2% L2:-4.5% |  |  | L1:2.1% L2:-5.5% |  |  |  |  |  |
|  |  | Y | L1:0.1% L2:-6.7% |  |  | L1:-0.9% L2:-6.4% |  |  |  |  |  |
|  |  | Z | L1:0.1% L2:-2.1% |  |  | L1:-0.2% L2:-0.2% |  |  |  |  |  |
| Vivo#1#2#3#4 | Case 1 | X | 0.8002 |  |  | 0.8111, 0.8035 |  |  | 0.8574 |  |  |
|  |  | Y | 0.8265 |  |  | 0.8596, 0.8564 |  |  | 0.9025 |  |  |
|  |  | Z | 0.8954 |  |  | 0.9125, 0.9093 |  |  | 0.9411 |  |  |
|  | Case 2 | X |  | 0.04% |  |  | -0.1% | -20% |  |  | -29.9% |
|  |  | Y |  | -0.5% |  |  | -0.75% | -20% |  |  | -29.6% |
|  |  | Z |  | -0.5% |  |  | -0.26% | -18% |  |  | -27.8% |
|  | Case 3 | X | 0.17% |  |  | 0.1% | -0.8% |  |  | -0.3% |  |
|  |  | Y | 0.35% |  |  | 0.02% | -3.3% |  |  | 0.5% |  |
|  |  | Z | 0.3% |  |  | 0.06% | -4% |  |  | 0.7% |  |
| OPPO#1#2 | Case 1 | X | 0.786 |  |  | 0.784 |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | X |  | -1.8% |  |  | -1.4% |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 3 | X | -0.6% |  |  | -1.1% |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| CATT | Case 1 | X | 0.677 |  |  | 0.646 |  |  | 0.841 |  |  |
|  |  | Y | 0.756 |  |  | 0.746 |  |  | 0.872 |  |  |
|  |  | Z | 0.816 |  |  | 0.84 |  |  | 0.904 |  |  |
|  | Case 2 | X |  | 1.18% | -13.74% |  | -4.8% |  |  | 4.64% |  |
|  |  | Y |  | 1.46% | -19.44% |  | -4.02% |  |  | 3.21% |  |
|  |  | Z |  | -2.21% | -20.96% |  | -4.76% |  |  | -0.11% |  |
|  | Case 3 | X | -1.48% | -1.92% |  | -4.49% |  |  | 4.76% |  |  |
|  |  | Y | -1.85% | -2.78% |  | -4.96% |  |  | 3.78% |  |  |
|  |  | Z | -0.74% | -1.35% |  | -4.64% |  |  | 2.10% |  |  |
| Xiaomi#1#2 | Case 1 | X | L1: 0.7168 L2: 0.564 |  |  | L1:0.7094 L2:0.5529 |  |  |  |  |  |
|  |  | Y | L1:0.7767 L2:0.6476 |  |  | L1:0.7864 L2:0.6522 |  |  |  |  |  |
|  |  | Z | L1:0.8893 L2:0.8108 |  |  | L1:0.8834 L2:0.7983 |  |  |  |  |  |
|  | Case 2 | X |  | L1:-0.2% L2: -1.8% |  |  | L1:-3.4% L2:-4.4% |  |  |  |  |
|  |  | Y |  | L1: -0.2% L2: -2.8% |  |  | L1: -3.2% L2: -3.2% |  |  |  |  |
|  |  | Z |  | L1: -3% L2: -6.3% |  |  | L1: -0.5% L2: 0.4% |  |  |  |  |
|  | Case 3 | X | L1: -1% L2: -0.5% |  |  | L1: 2.3% L2: 3.8% |  |  |  |  |  |
|  |  | Y | L1: -0.3% L2: -1.7% |  |  | L1: -1.3% L2: -3.3% |  |  |  |  |  |
|  |  | Z | L1: -2.6% L2: -4.8% |  |  | L1: -2.8% L2: -6% |  |  |  |  |  |
| ZTE#1#2#3#4#5#6#7#8 | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | L1:0.9218 L2:0.8661 |  |  | L1:0.9314 L2:0.8812 |  |  | L1:0.9504 L2:0.9309 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  | L1:0.09% L2:-0.29% | L1:-5.55% L2:-7.96% |  | L1:-1.9% L2:-2.86% | L1:-8.63% L2:-12.79% |  | L1:-1.74% L2:-2.38% | L1:-1.44% L2:-2.41% |
|  | Case 3 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z | L1:0.09% L2:0.08% |  | L1:-0.08% L2:-0.1% | L1:-0.92% L2:-1.32% |  | L1:-1.04% L2:-1.37% |  |  | L1:1.1% L2:1.19% |
| NTT DOCOMO#1#2#3#4#5#6 | Case 1 | X | L1:0.59 L2:0.46 |  |  | L1:0.59 L2:0.47 |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | X |  | L1: -3.39% L2: -6.5% |  |  | L1: -3.39% L2:-4.26% |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 3 | X | L1:-1.7%/-1.7%/-1.7% L2:-2.17%/0%/0% |  |  | L1:-1.7%/0%/-1.7% L2:-2.1%/0%/-2.1% |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
| InterDigital#1#2 | Case 1 | X | L1:0.623 L2:0.444 |  |  | L1:0.652 L2:0.469 |  |  |  |  |  |
|  |  | Y | L1:0.71 L2:0.536 |  |  | L1:0.741 L2:0.57 |  |  |  |  |  |
|  |  | Z | L1:0.803 L2:0.671 |  |  | L1:0.829 L2:0.694 |  |  |  |  |  |
|  | Case 2 | X |  | L1: -0.8% L2:-2.25% |  |  | L1:-3.68% L2:-4.26% |  |  |  |  |
|  |  | Y |  | L1:-1.69% L2:-3.36% |  |  | L1:-2.83% L2:-4.56% |  |  |  |  |
|  |  | Z |  | L1:-3.6% L2:-7.75% |  |  | L1:-1.81% L2:-1.87% |  |  |  |  |
|  | Case 3 | X | L1:2.1% L2:0% |  |  | L1:1.07% L2:0.64% |  |  |  |  |  |
|  |  | Y | L1:1.83% L2:2.05% |  |  | L1:1.62% L2:1.75% |  |  |  |  |  |
|  |  | Z | L1:1.868% L2:2.23% |  |  | L1:1.93% L2:3.9% |  |  |  |  |  |
| MTK#1#2 | Case 1 | X | 0.696 |  |  | 0.704 |  |  |  |  |  |
|  |  | Y | 0.771 |  |  | 0.767 |  |  |  |  |  |
|  |  | Z | 0.831 |  |  | 0.826 |  |  |  |  |  |
|  | Case 2 | X |  | -1.4% |  |  | -0.9% |  |  |  |  |
|  |  | Y |  | -2.8% |  |  | 0.2% |  |  |  |  |
|  |  | Z |  | -2.1% |  |  | 0.1% |  |  |  |  |
|  | Case 3 | X | -1.6% |  |  | -0.8% |  |  |  |  |  |
|  |  | Y | -1.0% |  |  | 0.1% |  |  |  |  |  |
|  |  | Z | -0.5% |  |  | 0.2% |  |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.6%) |  | Xiaomi, Interdigital, MTK, vivo, Intel, ZTE |  |  | Vivo, Oppo, MTK, Intel, Xiaomi |  |  | HW, CATT | ZTE |
|  |  | Moderate/Significant loss (-1.6%~-4%) |  | Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT,MTK |  |  | ZTE, Interdigital, CATT, Xiaomi, Intel, NTT DOCOMO, |  |  | Lenovo, ZTE | ZTE |
|  |  | Significant loss (<-4%) |  | FW, Intel, Xiaomi, Interdigital, NTT DOCOMO | HW, Lenovo, CATT, ZTE |  | FW, CATT, NTT DOCOMO, Interdigital, Xiaomi, Intel | ZTE, vivo |  |  | Vivo, |
|  |  | Positive gain |  | Vivo, CATT, ZTE |  |  | MTK, Xiaomi |  |  | CATT, |  |
|  | Case 3 | Minor loss (>-1.6%) | CATT, Xiaomi, NTT DOCOMO, MTK, Interdigital,OPPO | HW, CATT | ZTE | Intel, Futurewei, vivo, xiaomi, OPPO, MTK, ZTE, NTT DOCOMO | Vivo | ZTE | HW | vivo |  |
|  |  | Moderate loss (-1.6% ~ -4%) | NTT DOCOMO, CATT, Xiaomi, Intel | CATT |  | Xiaomi, NTT DOCOMO | vivo |  |  |  |  |
|  |  | Significant loss (<-4%) | Intel, Xiaomi, |  |  | Intel, CATT, Xiaomi |  |  |  |  |  |
|  |  | Positive gain | Futurewei, Intel, vivo, Interdigital, ZTE |  |  | Vivo, Interdigital, MTK, NTT DOCOMO, Intel, xiaomi |  |  | CATT | Vivo | ZTE |

#### Issue#3-13 (High priority) Generalization-UE distributions

Upd Proposed Observation 3.1.14: For the generalization verification of AI/ML based CSI compression over various UE distributions, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE distribution#B and applied for inference with a same UE distribution#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE distribution#A and UE distribution#B but not for others

  - If UE distribution#A is Outdoor & UE distribution#B is Indoor, 3 sources [Nokia, Qualcomm, Huawei] observe that moderate/significant degradations of -2.9%~-11.5% degradation are suffered,

    - Note: 1 source [NTT DOCOMO] observes 0% degradation

  - If UE distribution#A is Indoor & UE distribution#B is Outdoor, 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei] observe minor loss of less than -0.7% degradation or positive gain

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-1% loss or positive gain) for UE distribution#B subject to any of Outdoor and Indoor, if the training dataset is constructed with data samples subject to multiple UE distributions including UE distribution#B, as observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Minor loss (0%~-1%) are observed by 3 sources [NTT DOCOMO, Nokia, Huawei].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Note: Moderate degradations of up to -3.9% are still observed by 1 source [Nokia] for deployment scenario#B subject to Indoor.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.1-6 of R1-2306059

Table 5.1-6 Generalization of CSI compression-UE distributions

| Training, Generalization Case 1 |  |  | Indoor |  |  | Outdoor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | Outdoor |  |  | Indoor |  |
| Training, Generalization Case 3 |  |  | I:O=5:5 | I:O=8:2 |  | I:O=5:5 | I:O=8:2 |  |
| Testing |  |  | Indoor |  |  | Outdoor |  |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| HW#1/#2 | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z | 0.916 |  |  | 0.948 |  |  |
|  | Case 2 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  | -3% |  |  | 0.1% |  |
|  | Case 3 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z | -0.2% |  |  | 0.1% |  |  |
| QC#1/#2 | Case 1 | X |  |  |  |  |  |  |
|  |  | Y | 0.7624 |  |  | 0.8915 |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  |  |  |  |  |  |
|  |  | Y |  | -11.5% |  |  | +0.96% |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X |  |  |  |  |  |  |
|  |  | Y |  | + 0.9% |  |  | +1.8% |  |
|  |  | Z |  |  |  |  |  |  |
| Nokia#1/#2 | Case 1 | X | 0.685 |  |  | 0.837 |  |  |
|  |  | Y | 0.741 |  |  | 0.854 |  |  |
|  |  | Z | 0.881 |  |  | 0.926 |  |  |
|  | Case 2 | X |  | -2.9% |  |  | -0.7% |  |
|  |  | Y |  | -5.5% |  |  | 1.2% |  |
|  |  | Z |  | -3.4% |  |  | 0.5% |  |
|  | Case 3 | X |  | 2.5% |  |  | -0.2% |  |
|  |  | Y |  | -3.9% |  |  | 0.1% |  |
|  |  | Z |  | -1.0% |  |  | 0.4% |  |
| NTT DOCOMO#1#2#3#4#5#6 | Case 1 | X | 0.6 0.47 |  |  | 0.65 0.56 |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  | 0% 0% |  |  | 1.54% 3.57% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X | 0% 0% | 1.67% 2.1% |  | 0% 1.75% | 1.54% 3.57% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.6%) |  | NTT DOCOMO |  |  | Nokia |  |
|  |  | Moderate/significant loss (<-1.6%) |  | Nokia, QC, HW |  |  |  |  |
|  |  | Positive gain |  |  |  |  | QC, Nokia, HW, NTT DOCOMO |  |
|  | Case 3 | Minor loss (>-1.6%) | NTT DOCOMO, HW | Nokia |  |  | Nokia |  |
|  |  | Moderate loss (-1.6%~-4% |  | Nokia |  |  |  |  |
|  |  | Positive gain |  | QC, Nokia, NTT DOCOMO |  | NTT DOCOMO, HW | QC, Nokia, NTT DOCOMO |  |

#### Issue#3-17 (High priority) Scalability-Tx port number

Upd Proposed Observation 3.1.18: For the scalability verification of AI/ML based CSI compression over various Tx port numbers, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain Tx port number#B and applied for inference with a same Tx port number#B,

- For generalization Case 2, significant performance degradations are observed in general, if Tx port number#A is 32 & Tx port number#B is 16, as -3.37%~-21.8% degradations are observed by 4 sources [OPPO, Fujitsu, ZTE, vivo]

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gains) for Tx port number#B subject to any of 16 and 32, if the training dataset is constructed with data samples subject to multiple Tx port numbers including Tx port number#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, as observed by 7 sources [Huawei, OPPO, NTT DOCOMO, CATT, ZTE, Fujistu, Nokia].

  - Minor loss (0%~-1.75%) are observed by 6 sources [Huawei, OPPO, Fujistu, CATT, ZTE, NTT DOCOMO].

  - Moderate loss (-1.84%~-4%) are observed by 3 sources [Nokia, CATT, NTT DOCOMO].

  - Positive gains are observed by 3 sources [OPPO, ZTE, Fujistu].

  - Note: Significant degradations of up to -6.08% are still observed by 1 source [CATT] for deployment scenario#B subject to 32 ports, and for deployment scenario#B subject to 16 ports

  - Note: Pre/post-processing of truncation/padding is adopted by 6 sources [Huawei, OPPO, NTT DOCOMO, ZTE, Fujistu, Nokia], and adaptation layer in the AL/ML model is adopted by 1 source [CATT].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - Note: Results refer to Table 5.1-7 of R1-2306059

Table 5.1-7 Scalability of CSI compression-Tx port number

| Training, Generalization Case 1 |  |  | 32ports |  | 16ports | 32ports |
| --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | 16ports |  | 32ports |
| Training, Generalization Case 3 |  |  | 32ports+16ports |  | 32ports+16ports |  |
| Testing |  |  | 32ports |  | 16ports |  |
|  | Case | CSI payload |  |  |  |  |
| HW#1/#2 | Case 1 | X | 0.767 |  | 0.831 |  |
|  |  | Y | 0.861 |  | 0.894 |  |
|  |  | Z | 0.918 |  | 0.944 |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | -0.1% |  | -0.5% |  |
|  |  | Y | -1% |  | -0.2% |  |
|  |  | Z | -0.5% |  | -0.2% |  |
| OPPO#1#2 | Case 1 | X | 0.855 |  | 0.886 |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  | -21.8% |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | -0.2% |  | 1.35% |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
| Fujitsu | Case 1 | X |  |  | 0.8401 |  |
|  |  | Y |  |  | 0.8924 |  |
|  |  | Z |  |  | 0.9568 |  |
|  | Case 2 | X |  |  |  | -11.6% |
|  |  | Y |  |  |  | -12.26% |
|  |  | Z |  |  |  | -8.8% |
|  | Case 3 | X |  |  | -1% |  |
|  |  | Y |  |  | 0.01% |  |
|  |  | Z |  |  | 0.18% |  |
| Nokia#1#2 | Case 1 | X |  |  | 0.768 |  |
|  |  | Y | 0.754 |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X |  |  | -4% |  |
|  |  | Y | -3.6% |  |  |  |
|  |  | Z |  |  |  |  |
| ZTE#1#2 | Case 1 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z | L1:0.9218 L2:0.8771 |  | L1:0.947 L2:0.9047 |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  | L1:-3.37% L2:-5.77% |
|  | Case 3 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z | L1:-0.89% L2:-1.47% |  | L1:0.88% L2:1.34% |  |
| NTT DOCOMO#1#2 | Case 1 | X | L1:0.58 L2:0.46 |  | L1:0.68 L2:0.57 |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | L1:0% L2:-2.17% |  | L1:-1.47% L2:-1.75% |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
| Vivo#1 | Case 1 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  | -15.2% |
|  |  | Z |  |  |  |  |
|  | Case 3 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
| CATT#1#2 | Case 1 | X | L1:0.7231 L2:0.5675 L3:0.2662 L4:0.1447 |  | L1:0.7802 L2:0.5993 L3:0.2609 L4:0.1429 |  |
|  |  | Y | L1:0.7562 L2:0.5889 L3:0.2633 L4:0.1428 |  | L1:0.807 L2:0.6196 L3:0.2666 L4:0.1487 |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 3 | X | L1:-1.62% L2:-3.42% L3:-5.03% L4:-0.35% |  | L1:-2.08% L2:-6.08% L3:-2.49% L4:-1.11% |  |
|  |  | Y | L1:-0.03% L2:-0.15% L3:0% L4:-0.7% |  | L1:- 1.84% L2:-1.18% L3:-2.4% L4:-1.75% |  |
|  |  | Z |  |  |  |  |
| Summary | Case 1 | X |  |  |  |  |
|  |  | Y |  |  |  |  |
|  |  | Z |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  |  |  |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  | Oppo, Fujistu, ZTE |
|  |  | Positive gain |  |  |  |  |
|  | Case 3 | Minor loss (>-1.8%) | HW, OPPO, NTT DOCOMO, CATT, ZTE |  | HW, Fujistu, NTT DOCOMO |  |
|  |  | Moderate loss (-1.8%~-4%) | CATT, NTT DOCOMO, Nokia |  | CATT, Nokia |  |
|  |  | Significant loss (<-4%) | CATT |  | CATT |  |
|  |  | Positive gain |  |  | OPPO, ZTE, Fujistu |  |

#### Issue#4-5 (High priority) UPT gain-Mean UPT

Upd Proposed Observation 4.1.5: For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 1.2%~4.2% gain;

    - 1 source [Apple] observes 7.6%~8.5% gain;

    - 1 source [vivo] observes 9.7%~17.2% gain.

    - 1 source [MediaTek] observes 22.6%~ 48.6% gain.

  - For full buffer traffic:

    - 1 source [Nokia] observes 2%~3% gain;

    - 1 source [vivo] observes 8.7% gain.

    - 1 source [MediaTek] observes 1.01% gain.

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.7%~3.1% gain;

    - 1 source [vivo] observes 3.4%~7.0% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 8.1% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-8 of R1-2306059

Table 5.1-8 Gain of CSI prediction– Mean UPT

| BM#1 | 30km/h |  |  | 60km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1-FTP |  |  |  | 1.2% (62bits) 1.7% (111bits) 2.1% (279bits) | 1.8% (62bits) 3.9% (111bits) 3.5% (279bits) | 2.5% (62bits) 3.1% (111bits) 4.2% (279bits) |
| Apple#1- FTP | 7.6% | 8.5% | 8.5% |  |  |  |
| Vivo#1- FTP |  |  |  | 9.7% (64 bits) | 15.1%(64 bits) | 17.2%(64 bits) |
| MTK FTP | 22.6%~ 29.1% (279bit) |  | 34.6%~ 48.6% (279bits) |  |  |  |
| Vivo#2-FB |  |  |  | 8.7% |  |  |
| Nokia#1-FB | 2%~3% (303 bits) |  |  |  |  |  |
| BM#2 | 30km/h |  |  | 60km/h |  |  |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1- FTP |  |  |  | 0.7% (62bits) 0.9% (111bits) 0.8% (279bits) | 2.3% (62bits) 3.1% (111bits) 2.5% (279bits) | 2.4% (62bits) 2% (111bits) 2.5% (279bits) |
| Vivo#1- FTP |  |  |  | 3.4% (64 bits) | 5.1%(64 bits) | 7.0%(64 bits) |
| MTK FTP | 1.01% (279bits) |  | 1.01% (279bits) |  |  |  |
| Vivo#2-FB |  |  |  | 8.1% |  |  |

#### Issue#4-6 (High priority) UPT gain-5% UPT

Upd Proposed Observation 4.1.6: For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 4.5%~9.3% gain;

    - 3 sources [Huawei, Apple, vivo] observes 11.3%~20.1% gain;

  - For full buffer traffic:

    - 2 sources [Nokia, vivo] observe 6%~17.5% gain;

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 0.5%~16% gain;

  - For full buffer traffic:

    - 1 source [vivo] observes 11% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-9 of R1-2306059

Table 5.1-9 Gain of CSI prediction– 5% UPT

| BM#1 | 30km/h |  |  | 60km/h |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1-FPT |  |  |  | 4.5% (62bits) 18.3% (111bits) 8% (279bits) | 11.3% (62bits) 9.3% (111bits) 8.6% (279bits) | 17.8% (62bits) 13.4% (111bits) 16.4% (279bits) |
| Apple#1-FPT | 12.9% | 20.1% | 15.8% |  |  |  |
| Vivo#1-FPT |  |  |  | 6.9% (64 bits) | 13.0% (64 bits) | 20.0% (64 bits) |
| Vivo#2-FB |  |  |  | 17.5% |  |  |
| Nokia#1-FB | 6%~15% (303 bits) |  |  |  |  |  |
| BM#2 | 30km/h |  |  | 60km/h |  |  |
|  | RU<=39% | RU 40%-69% | RU >=70% | RU<=39% | RU 40%-69% | RU >=70% |
| HW#1-FPT |  |  |  | 2.5% (62bits) 5.2% (111bits) 4.3% (279bits) | 7.7% (62bits) 7% (111bits) 8.6% (279bits) | 14.8% (62bits) 6.7% (111bits) 4.2% (279bits) |
| Vivo#1-FPT |  |  |  | 0.5% (64 bits) | 3.1%(64 bits) | 16%(64 bits) |
| Vivo#2-FB |  |  |  | 11% |  |  |

#### Issue#4-7 (High priority) Generalization-UE speed

Upd Proposed Observation 4.1.7: For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, 4 sources [Xiaomi, CATT, Interdigital, Spreadtrum] observe a generalized performance of less than -2% degradation.

  - If UE speed#B is either 30 km/h or 60 km/h or 120 km/h, or if UE speed#B is 10km/h and UE speed#A is either 60km/h or 120km/h, 8 sources [Xiaomi, Samsung, Interdigital, Fujitsu, ZTE, ETRI, vivo, Huawei] observe that moderate/significant performance degradations are suffered:

    - For UE speed#B is 10 km/h & UE speed#A is either 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-2.7% loss), 1 source [Samsung] observes significant degradation (-53%~-61% loss).

    - For UE speed#B is 30 km/h & UE speed#A is either 10 km/h, 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-3% loss), 8 sources [Xiaomi, Interdigital, Fujitsu, vivo, ZTE, Huawei, ETRI, Spreadtrum] observe significant degradation (-6%~-45.6% loss).

    - For UE speed#B is 60 km/h & UE speed#A is either 10 km/h, 30 km/h or 120 km/h, 1 source [ZTE] observes moderate degradation (-3% loss), 7 sources [Samsung, Xiaomi, Fujitsu, ETRI, ZTE, vivo, Spreadtrum] observe significant degradation (-7.8%~-52% loss).

    - For UE speed#B is 120 km/h & UE speed#A is either 30 km/h or 60 km/h, 1 source [ZTE] observes moderate degradation (-3.4% loss), 4 sources [ZTE, ETRI, vivo, Samsung] observe significant degradation (-7.55%~-32.3% loss).

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 9 sources [Xiaomi, Interdigital, Apple, Huawei, ZTE, Samsung, ETRI, vivo, Spreadtrum].

  - For UE speed#B is 10 km/h, minor loss (-0.6%~-1%) are observed by 3 sources [CATT, Xiaomi, Spreadtrum].

  - For UE speed#B is 30 km/h, minor loss (-0.08%~-1.34%) are observed by 3 sources [Xiaomi, Apple, Huawei], moderate loss (-2.2%~-4.07%) are observed by 3 sources [Interdigital, vivo, Spreadtrum].

  - For UE speed#B is 60 km/h, minor loss (-0.05%~-2%) are observed by 4 sources [ZTE, Apple, Xiaomi, Huawei], moderate loss (-2%~-3.76%) are observed by 2 sources [vivo, Spreadtrum].

  - For UE speed#B is 120 km/h, moderate loss (-2%~-4.45%) are observed by 4 sources [vivo, Samsung, ETRI, ZTE].

  - Note: For generalization Case 3, 5 sources [ETRI, ZTE, Samsung, Interdigital, Fujitsu] observe significant performance degradations (-5%~-26.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-10 of R1-2306059

Table 5.1-10 Generalization of CSI prediction-UE speeds

| Training Genr-Case 1 (km/h) |  | 10 |  |  |  | 30 |  |  |  | 60 |  |  |  | 120 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training Genr-Case 2 (km/h) |  |  | 30 | 60 | 120 |  | 10 | 60 | 120 |  | 10 | 30 | 120 |  | 30 | 60 |
| Training Genr-Case 3 (km/h) |  | X+10 |  |  |  | X+30 |  |  |  | Y+60 |  |  |  | Z+120 |  |  |  |
| Test |  | 10km/h |  |  |  | 30km/h |  |  |  | 60km/h |  |  |  | 120km/h |  |  |  |
| HW | Case 1 |  |  |  |  | 0.979 |  |  |  | 0.861 |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  |  | -6% |  |  |  | -31% |  |  |  |  |
|  | Case 3 |  |  |  |  | -0.9% |  |  |  | -1.4% |  |  |  |  |  |  |  |
| Apple | Case 1 |  |  |  |  | L1: 0.9984 L2: 0.9977 L3: 0.9968 L4: 0.997 |  |  |  | L1: 0.8201 L2: 0.7578 L3: 0.6651 L4: 0.6441 |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 3 |  |  |  |  | L1: -0.46% L2: -0.73% L3: -1.34% L4: -1.24% |  |  |  | L1: -1.26% L2: -1.65% L3: -0.4% L4: -2% |  |  |  |  |  |  |  |
| ZTE | Case 1 |  |  |  |  | 0.9576 |  |  |  | 0.7435 |  |  |  | 0.7289 |  |  |
|  | Case 2 |  |  |  |  |  |  | -18.44% | -21.02% |  |  | -14.34% | -3% |  | -28.44% | -3.4% |
|  | Case 3 |  |  |  |  | -10.29% |  |  |  | -0.05% |  |  |  | -3.1% |  |  |  |
| ETRI | Case 1 |  |  |  |  | 0.8841 |  |  |  | 0.7987 |  |  |  | 0.7273 |  |  |
|  | Case 2 |  |  |  |  |  |  | -16.7% | -45.6% |  |  | -11.6% | -36.1% |  | -32.3% | -27.4% |
|  | Case 3 |  |  |  |  | -13.3% -22.2% |  |  |  | -5% |  |  |  | -2% |  |  |  |
| SS | Case 1 | 0.9596 |  |  |  |  |  |  |  | 0.5745 |  |  |  | 0.4727 |  |  |
|  | Case 2 |  |  | -53% | -61% |  |  |  |  |  | -18% |  | -26.4% |  |  | -13% |
|  | Case 3 | -8% |  |  |  |  |  |  |  | -6.7% |  |  |  | -2.6% |  |  |  |
| Fujitsu | Case 1 |  |  |  |  | 0.9368 |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  | -21% |  |  |  |  |  |  |  |  |  |
|  | Case 3 |  |  |  |  | -9.8% |  |  |  |  |  |  |  |  |  |  |  |
| vivo | Case 1 |  |  |  |  | 0.9896 |  |  |  | 0.8102 |  |  |  | 0.6156 |  |  |
|  | Case 2 |  |  |  |  |  |  | -6.8% | -10.7% |  |  | -30.8% | -7.81% |  | -18% | -7.55% |
|  | Case 3 |  |  |  |  | -4.07% |  |  |  | -3.76% |  |  |  | -4.45% |  |  |  |
| CMCC | Case 1 | -42.822dB |  |  |  | -28.426dB |  |  |  | -9.036dB |  |  |  |  |  |  |
|  | Case 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 3 | +24.297dB |  |  |  | + 12.47dB |  |  |  | + 1.981dB |  |  |  |  |  |  |  |
| InterDi gital | Case 1 | L1:0.999 L2:0.999 |  |  |  | L1:0.938 L2:0.912 |  |  |  | L1:0.57 L2:0.497 |  |  |  |  |  |  |
|  | Case 2 |  | L1: -2% L2: -2% |  |  |  | L1: -17% L2:-22% | L1: -32% L2: -40% |  |  |  |  |  |  |  |  |
|  | Case 3 | L1: -0.6% L2: -0.8% |  |  |  | L1: -3% L2: -3% |  |  |  | L1: -14% L2: -15% |  |  |  |  |  |  |  |
| MTK | Case 1 | -32.83dB |  |  |  | -23.62dB |  |  |  | -0.87dB |  |  |  | -0.5dB |  |  |
|  | Case 2 |  | +2.16dB |  | +27.5dB |  |  |  | +21.85 |  |  | +20.17dB | 0dB |  | +17.48dB |  |
|  | Case 3 | +10.31dB |  |  |  | +8.21dB |  |  |  | +0.26dB |  |  |  | +0.29dB |  |  |  |
| Xiaomi | Case 1 | 0.984 |  |  |  | 0.972 |  |  |  | 0.943 |  |  |  |  |  |  |
|  | Case 2 |  | -0.4% | -2.7% |  |  | -12.3% | -3% |  |  | -52% | -43% |  |  |  |  |
|  | Case 3 | -0.6% |  |  |  | -0.8% |  |  |  | -1.2% |  |  |  |  |  |  |  |
| CATT | Case 1 | 0.833 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  | -1.51% |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Spreadtrum | Case 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  | -1.2% | -2.1% |  |  | -18.7% |  |  |  | -34.6% |  |  |  |  |  |
|  | Case 3 | -1% |  |  |  | -2.2% |  |  |  | -2% |  |  |  |  |  |  |
| Summ ary | Case 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Case 2 |  | -0.4%~ -2% [3 companies] +2.16dB | -2.7%[1 company], -53%[1] | -61%[1company] +27.5dB |  | -12.3%~ -22%[3 companies] | -3%[1 company]; -6~ -40%[5 companies] | -10.7%~-45.6[3 companies] +21.85dB |  | -18%~-52%[2 companies] | -11.6%~-43%[4 companies] +20.17dB | -3%[1 company], -7.81%~-36[3 companies] 0dB |  | -18%~-32.3%[3 companies] +17.48dB | -3.4%[1 company], 7.55~-27.4[3companies] |
|  | Case 3 | -0.6% ~ -1%[3 companies]; -8% [1 company] +10.31dB~+24.297dB |  |  |  | -0.08%~-1.34% [3 companies]; -2.2%~-4.07% [3 companies]; -5.9~-26.5% [3 companies]; +8.21dB~+ 12.47dB |  |  |  | -0.05~2% [4 companies] -2%~-3.76% [2 company] -5%~-15% [3 companies] + 0.26~1.981dB |  |  |  | -2~-4.45% [4 companies] +0.29dB |  |  |  |
| Case 2 | Minor(>-2%) |  | CATT, Xiaomi, Interdigital, Spreadtrum |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Modr(-2~-5% loss) |  |  | Xiaomi |  |  |  | xiaomi |  |  |  |  | ZTE |  |  | ZTE |
|  | Signf(<-5% loss) |  |  | SS | SS |  | Xiaomi, Interdigital, Fujitsu Spreadtrum | ZTE, ETRI, Interdigital, vivo, HW | vivo, ETRI,ZTE |  | SS, Xiaomi, Spreadtrum | Xiaomi, Fujitsu, ETRI, ZTE | Vivo, SS, ETRI, |  | ZTE, ETRI, vivo | Vivo,SS, ETRI |
| Case 3 | Minor(>-2%) | Xiaomi, Interdigital, Spreadtrum |  |  |  | Xiaomi, Apple, HW |  |  |  | ZTE, Apple, Xiaomi, HW |  |  |  |  |  |  |  |
|  | Modr(-2~-5% loss) |  |  |  |  | Interdigital, vivo Spreadtrum |  |  |  | vivo Spreadtrum |  |  |  | Vivo, SS, ETRI, ZTE |  |  |  |
|  | Signf(<-5% loss) | SS |  |  |  | ETRI, ZTE, Fujitsu |  |  |  | Interdigital, ETRI, SS |  |  |  |  |  |  |  |

## 24 May (Wed.) Offline/GTW

#### Issue#3-8 (High priority) Quantization method-training awareness cases

Proposed Observation 3.2.5: For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, training non-aware quantization (Case 1) is in general inferior to the training aware quantization (Case 2-1/2-2), and may lead to lower performance than the benchmark.

- For scalar quantization, compared with benchmark,

  - -5.9%~-43.2% degradations are observed for training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8].

  - 3.9%~8.64% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 5 sources [Source#1, Source#2, Source#3, Source#4, Source#5], which are 17.3%~83.2% gains over training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8] and 0.9%~5.4% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

    - Note: 0.72% gains are observed for Case 2-1 from 1 source [Source#1] due to non-optimized SQ parameter chosen, which achieves 13.9% gains over Case 1.

  - 7.55% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 1 source [Source#1], which are 21.6% gains over training non-aware quantization (Case 1) from 1 source [Source#1].

- For vector quantization, compared with benchmark,

  - -2%~-10% degradations are observed for training non-aware quantization (Case 1) from 1 source [Source#7].

  - 6.0%~8.91% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 2 sources [Source#1, Source#2], which are 16.3%~23.1% gains over training non-aware quantization (Case 1) from 2 sources [Source#1, Source#2].

  - 4.67%~13.01% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 6 sources [Source#1, Source#2, Source#4, Source#5, Source#7, Source#8], which are 10.7%~27.8% gains over training non-aware quantization (Case 1) from 3 sources [Source#1, Source#2, Source#7] and 1.7%~7.5% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

  - In general, Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains, as observed by 6 sources [Source#1, Source#2, Source#4, Source#5, Source#6, Source#8].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: Qualcomm (R1-2305328); Source#2: vivo (R1-2304471); Source#3: Ericsson (R1-2304521); Source#4: ZTE (R1-2304534); Source#5: Xiaomi (R1-2304893); Source#6: Fujitsu (R1-2304764); Source#7: Huawei, HiSilicon (R1-2304653); Source#8: Apple (R1-2305234).

  - Note: Results refer to Table 5.4-1 of R1-2306060

Table 5.4-1. Quantization method of CSI comporession – training awareness

|  |  |  | Case 1 | Case 2-1 | Case 2-2 |
| --- | --- | --- | --- | --- | --- |
| QC#1~QC#7 | SQ | X |  |  |  |
|  |  | Y | 0.648 (-11.56%) | config#1: 0.796 (8.64%) 22.8% gain over Case 1 config#2: 0.738 (0.72%) 13.9% gain over Case 1 | 0.788 (7.55%) 21.6% gain over Case 1 |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  | 0.798 (8.91%) 23.1% gain over Case 1 | VQ-dim2: 0.823 (12.32%) 27% gain over Case 1 3.1% gain over Case 2-1 VQ-dim4: 0.828 (13.01%) 27.8% gain over Case 1 3.8% gain over Case 2-1 |
|  |  | Z |  |  |  |
| HW#1/#5 | SQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X | 0.69 (-2%) |  | 0.767 (8.8%) 11.1% gain over Case 1 |
|  |  | Y | 0.738 (-10%) |  | 0.861 (4.5%) 16.7% gain over Case 1 |
|  |  | Z | 0.829 (-4%) |  | 0.918 (6.5%) 10.7% gain over Case 1 |
| vivo#1~#5 | SQ | X |  |  |  |
|  |  | Y | 0.735 (-5.9%) | 0.862 (6.7%) 17.3% gain over Case 1 |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  | 0.855 (6.0%) 16.3% gain over Case 1 | VQ-config#1: 0.871 (7.6%) 18.5% gain over Case 1 1.9% gain over Case 2-1 VQ-config#2: 0.866 (7.1%) 17.8% gain over Case 1 1.3% gain over Case 2-1 |
|  |  | Z |  |  |  |
| ZTE#1/#5 | SQ | X |  | L1: 0.7283(6.01%) L2: 0.5935(7.17%) |  |
|  |  | Y |  | L1: 0.8118(5.37%) L2: 0.6994(7.14%) |  |
|  |  | Z |  | L1: 0.9040(5.47%) L2: 0.8380(8.42%) |  |
|  | VQ | X |  |  | L1: 0.7191(4.67%) L2: 0.5866(5.92%) |
|  |  | Y |  |  | L1: 0.8015(4.04%) L2: 0.6913(5.90%) 1% gain over Case 2-1-VQ |
|  |  | Z |  |  | L1: 0.9123(6.44%) L2: 0.8498(9.95%) |
| Xiaomi#1/#2 | SQ | X |  |  |  |
|  |  | Y |  | 0.7533(7.31%) |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  |  | 0.7591(8.13%) 0.82% gain over Case 2-1-VQ |
|  |  | Z |  |  |  |
| E///#1/#2 | SQ | X | 0.410 (-43.2%) | 0.751(3.9%) 83.2% gain over Case 1 |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
| Nokia | SQ | X |  |  |  |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X |  |  | 1.2% (52 bits) |
|  |  | Y |  |  | 2.9% (104 bits) |
|  |  | Z |  |  | 1.5% (256 bits) |
| Fujitsu | SQ | X | 0.729(5.85%) | 0.7684(11.57%) 5.4% gain over Case 1 |  |
|  |  | Y | 0.8135(4.2%) | 0.8431(7.99%) 3.6% gain over Case 1 |  |
|  |  | Z | 0.9258(4.84%) | 0.9343(5.8%) 0.9% gain over Case 1 |  |
|  | VQ | X |  |  | 0.784(13.83%) 7.5% gain over Case 1 2% gain over Case 2-1 |
|  |  | Y |  |  | 0.8713(11.6%) 7.1% gain over Case 1 3.3% gain over Case 2-1 |
|  |  | Z |  |  | 0.9411(6.57%) 1.7% gain over Case 1 0.7% gain over Case 2-1 |
| Apple | SQ | X | 0.513 (-26%) | 0.722(4.49%) 26% gain over Case 1 | N/A |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
|  | VQ | X | 0.709 (2.6%) (VQ codebook is searched separately using traditional methods, based on latent space distribution, after encoder/decoder is trained without quantization) | 0.73(5.64%) 3% gain over Case 1 | 0.735 (6.37%) 3.66% gain over Case 1; 0.7% gain over Case 2-1 |
|  |  | Y |  |  |  |
|  |  | Z |  |  |  |
| Summary; gain over benchmark, gain over Case 1 | SQ | X | -43.2%; 5.85% | 3.9%~11.57%, Signf: 83.2%; Modrt: 5.4% |  |
|  |  | Y | -5.9%~-11.56%; 4.2% | 5.37%~8.64%, Signf: 17.3%~22.8%; Modrt: 3.6% | 7.55%, 21.6% |
|  |  | Z | 4.84% | 5.47%~5.8%, 0.9% |  |
|  | VQ | X | -2% | 6.01% | 4.67%~8.8% 7.5% |
|  |  | Y | -10% | 6.0%~8.91%, 16.3%~23.1% | 4.04%~13.01%, Modrt: 7.1%; Signf:16.7%~27.8% |
|  |  | Z | -4% | N/A | 6.44%~6.5%, Modrt: 1.7%; Signf:10.7% |
| Summary; gain over benchmark, gain over Case 1 | SQ | X/Y/Z | -5.9%~-43.2% (QC, vivo, E///, Apple) 4.2%~5.85% (Fujitsu) | 3.9%~8.64% (QC, vivo, ZTE, Xiaomi, E///), Signf:17.3%~83.2% (QC, vivo, E///, Apple) Modrt:0.9%~5.4% (Fujitsu, Apple) | 7.55%(QC), 21.6%(QC) |
|  | VQ | X/Y/Z | -2%~-10% (HW) 2.6% (Apple) | 6.0%~8.91% (QC, vivo), 16.3%~23.1% (QC, vivo) | 4.67%~13.01% (QC, vivo, HW, ZTE, Xiaomi, Apple), Signf:10.7%~27.8% (QC, vivo, HW), Modrt:1.7%~7.5% (Fujitsu, Apple), 0.7%~3.8% gain over Case 2-1(QC, vivo, Fujitsu, Apple) |

#### Issue#3-9 (High priority) Quantization method-quantization format

Proposed Observation 3.2.6:  For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, in general vector quantization (VQ) has comparable performance with scalar quantization (SQ):

- For SQ and VQ under the same training case, it is

  - observed by 1 source [Source#1] that VQ under Case 2-1 has -0.8% degradation over SQ under Case 2-1,

  - observed by 2 sources [Source#2, Source#3] that VQ under Case 2-1 has 0.3%~1.1% gain over SQ under Case 2-1, and

  - observed by 3 sources [Source#2, Source#3, Source#4] that VQ under Case 2-2 has 0.7%~5.1% gain over SQ under Case 2-2.

  - Note: VQ under Case 2-1 has 8% gains over SQ under Case 2-1 as observed from 1 source [Source#2] due to non-optimized SQ parameter chosen.

- For SQ and VQ across training cases, it is

  - observed by 5 sources [Source#1, Source#2, Source#3, Source#5, Source#6] that VQ under Case 2-2 has 0.5%~4% gain over SQ under Case 2-1, and

  - observed by 1 source [Source#5] that VQ under Case 2-2 has -1.3% degradation over SQ under Case 2-1.

- Note: in general, more companies (Source#1, Source#2, Source#3, Source#4, Source#5, Source#6) observing gain of VQ over SQ than companies observing loss (Source#1, Source#5).

- Note: it is observed by 1 source [Source#5] that combined SQ and VQ under Case 2-2 has minor gain of 0.2% over VQ only under Case 2-2.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: vivo (R1-2304471); Source#2: Qualcomm (R1-2305328); Source#3: Apple (R1-2305234); Source#4: Lenovo (R1-2305202); Source#5: ZTE (R1-2304534); Source#6: Xiaomi (R1-2304893);.

  - Note: Results refer to Table 5.4-2 of R1-2306060

Table 5.4-2. Quantization method of CSI comporession – quantization format

|  |  |  | Case 2-1 | Case 2-2 |
| --- | --- | --- | --- | --- |
| QC#1~QC#7 | SQ | X |  |  |
|  |  | Y | config#1: 0.796 (8.64%) config#2: 0.738 (0.72%) 13.9% gain over Case 1 | 0.788 (7.55%) |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y | 0.798 (8.91%) 0.3% gain over Case 2-1-SQ | VQ-dim2: 0.823 (12.32%) 3.4% gain over Case 2-1-SQ 4.4% gain over Case 2-2-SQ VQ-dim4: 0.828 (13.01%) 4% gain over Case 2-1-SQ 5.1% gain over Case 2-2-SQ |
|  |  | Z |  |  |
| vivo#1~#5 | SQ | X |  |  |
|  |  | Y | 0.862 (6.7%) |  |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y | 0.855 (6.0%) -0.8% gain over Case 2-1-SQ | VQ-config#1: 0.871 (7.6%) 1% gain over Case 2-1-SQ VQ-config#2: 0.866 (7.1%) 0.5% gain over Case 2-1-SQ |
|  |  | Z |  |  |
| ZTE#1/#5 | SQ | X | L1: 0.7283(6.01%) L2: 0.5935(7.17%) |  |
|  |  | Y | L1: 0.8118(5.37%) L2: 0.6994(7.14%) |  |
|  |  | Z | L1: 0.9040(5.47%) L2: 0.8380(8.42%) |  |
|  | VQ | X |  | L1: 0.7191(4.67%) -1.3% gain over Case 2-1-SQ L2: 0.5866(5.92%) |
|  |  | Y |  | L1: 0.8015(4.04%) -1.3% gain over Case 2-1-SQ L2: 0.6913(5.90%) |
|  |  | Z |  | L1: 0.9123(6.44%) 1% gain over Case 2-1-SQ L2: 0.8498(9.95%) |
| Xiaomi#1/#2 | SQ | X |  |  |
|  |  | Y | 0.7533(7.31%) |  |
|  |  | Z |  |  |
|  | VQ | X |  |  |
|  |  | Y |  | 0.7591(8.13%) 0.8% gain over Case 2-1-SQ |
|  |  | Z |  |  |
| Nokia | SQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  |  |
|  | VQ | X |  | 1.2% (52 bits) |
|  |  | Y |  | 2.9% (104 bits) |
|  |  | Z |  | 1.5% (256 bits) |
| Lenovo#1/#2 | SQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  | 0.844 |
|  | VQ | X |  |  |
|  |  | Y |  |  |
|  |  | Z |  | 0.882 0.8% gain over Case 2-2-SQ |
| Apple | SQ | X | 0.722 (2.6%) |  |
|  |  | Y |  |  |
|  |  | Z |  |  |
|  | VQ | X | 0.73 (5.64%) 1.1% gain over Case 2-1-SQ | 0.735 (6.37%) 1.8% gain over Case 2-1-SQ 0.7% gain over Case 2-2-SQ |
|  |  | Y |  |  |
|  |  | Z |  |  |
| Summary | X/Y/Z |  | Case 2-1-VQ has -0.8% loss over Case 2-1-SQ [vivo], 0.3%~1.1% gain over Case 2-1-SQ [QC, Apple] | Case 2-2-VQ has 0.5% ~4% gain over Case 2-1-SQ [QC, vivo, ZTE, Xiaomi Apple] Case 2-2-VQ has 0.7% ~5.1% gain over Case 2-2-SQ [Lenovo, QC, Apple] Case 2-2-VQ has -1.3% loss over Case 2-1-SQ [ZTE] |

#### Issue#2-3 (Medium priority) values of thresholds for Step 3 - KPIth_1 for Option 1

Proposal 2.2.1: For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_1 in Option 1, the Max threshold value is set as one of the following options:

- Option 1: 0.05

- Option 2: 0.1

- Option 3: other values

- Note: It is up to companies to simulate smaller values of KPIth_1 than the Max threshold.

#### Issue#2-3a (Medium priority) values of thresholds for Step 3 – relationship of KPIth_2, KPIth_3 for Option 2

Proposal 2.2.2: For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_2 and KPIth_3 in Option 2, consider KPIth_2=KPIth_3.

- E.g., KPIActual is calculated as KPIActual=KPIproxy+KPIcomp, where  KPIproxy is the intermediate KPI of/calculated by the direct output of the proxy model, and KPIcomp is the fixed compensation value. It is up to companies to determine KPIcomp, e.g., it can be determined at the training phase of the proxy model.

#### Issue#3-24 (High priority) Benchmark of joint training for comparison

Proposal 3.2.14: For the evaluation of training Type 3 under CSI compression, for the benchmark case (1-on-1 joint training) for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared.

- E.g., if the Type 3 is Transformer#1 for NW part model and CNN#1 for UE part model, then the benchmark case for performance comparison is also Transformer#1 for NW part model and CNN#1 for UE part model with joint training.

#### Issue#3-11a (High priority) Rank>1 solutions-down selection

Proposed Conclusion 3.2.8: For further study the evaluation of CSI compression, for the rank>1 solutions, prioritize Option 2-1 (layer specific and rank common), Option 3-1 (layer common and rank common), and Option 3-2 (layer common and rank specific).

- Note: For the evaluations of CSI compression with 1-on-1 joint training,

  - Option 1-2 is not adopted;

  - Option 2-2 is not adopted;

  - Over the 11 sources [Huawei, Ericsson, Nokia, ZTE, OPPO, Intel, Fujitsu, NTT DOCOMO, InterDigital, Qualcomm, Futurewei] for FTP/fullbuffer, Max Rank 2,

    - Option 1-1 is adopted by 1 source [ZTE]

    - Option 2-1 is adopted by 3 sources [Ericsson, Intel, Qualcomm]

    - Option 3-1 is adopted by 8 sources [Huawei, Nokia, ZTE, OPPO, Fujitsu, Futurewei, Xiaomi, vivo]

    - Option 3-2 is adopted by 2 sources [NTT DOCOMO, InterDigital]

  - Over the 4 sources [Qualcomm, ZTE, CATT, Apple] for FTP/fullbuffer, Max Rank 4,

    - Option 2-1 is adopted by 1 source [Qualcomm]

    - Option 3-1 is adopted by 2 sources [Qualcomm, ZTE]

    - Option 3-2 is adopted by 1 source [Apple]

#### Issue#3-20 (Medium priority) NW first training-1 NW to 1/M>1 UE

Proposed Observation 3.2.11: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 4 sources [Source#1, Source#2, Source#3, Source#4] observe -0%~-0.5% degradation, 4 sources [Source#2, Source#3, Source#5, Source#6] observe -0.5%~-1% degradation, and 2 sources [Source#2, Source#3] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Source#7, Source#8, Source#9] observe -0%~-0.5% degradation.

- Note: For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [Source#3] observes -0%~-0.5% degradation, 2 sources [Source#3, Source#10] observe -0.5%~-1% degradation, and 2 sources [Source#5, Source#6] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [Source#3] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Source#1: Ericsson (R1-2304521); Source#2: Nokia (R1-2304681); Source#3: ZTE (R1-2304534); Source#4: Fujitsu (R1-2304764); Source#5: Qualcomm (R1-2305328); Source#6: vivo (R1-2304471); Source#7: Huawei, HiSilicon (R1-2304653); Source#8: Apple (R1-2305234); Source#9: CMCC (R1-2305085); Source#10: CATT (R1-2304722);

  - Note: Results refer to Table 5.4-3 of R1-2306060

Table 5.4-3. NW first training for CSI compression -1 NW to 1/M>1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip |  |  |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.5% |  | HW, Apple, CMCC, |  |  |

## 25 May (Thu.) Offline/GTW

#### Issue#2-3c (New) Down selection of Option1 and Option 2 for Step 3

Proposal 2.3.2: For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIDiff achieved in the RAN1#112bis-e meeting, consider Option 1 (Gap between KPIActual and KPIGenie) as mandatory, and Option 2 (Binary state of KPIActual and KPIGenie relationship) as optional.

Proposal 2.3.2: For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIDiff achieved in the RAN1#112bis-e meeting, as baseline for calibration purpose, consider Option 1 (Gap between KPIActual and KPIGenie).

- Option 2 (Binary state of KPIActual and KPIGenie relationship) as optional and up to companies to report.

  - Results subject to Option 2, may be captured as a note in observation

#### Issue#3-5 (High priority) Mean UPT, full buffer

Proposed Observation 3.3.1: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11%

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 6%~11% at CSI overhead A (small overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11% at CSI overhead C (large overhead);

- For Max rank 2, 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~15%

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~15% at CSI overhead A (small overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~10% at CSI overhead B (medium overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of -0.2%~14% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 24.47%~28.24%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 7.44%~9.95% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.5-1 of R1-2306061

Table 5.5-1. Gain of CSI comporession over R16 Type II benchmark – Mean UPT, full buffer

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| HW#1 | A | 11% | 15% |  |
|  | B | 7% | 9% |  |
|  | C | 11% | 14% |  |
| Nokia #1 | A | 6.5% | 8.5% |  |
|  | B | 5.8% | 5% |  |
|  | C | 1.1% | -0.2% |  |
| vivo#1 | A | 7.92% | 11.15% |  |
|  | B | 6.02% | 7.09% |  |
|  | C | 9.53% | 10.28% |  |
| OPPO#1 | A | 6% |  |  |
|  | B | 3% |  |  |
|  | C | 3% |  |  |
| Fujitsu#1 | A | 9% | 9.2% |  |
|  | B | 5.6% | 7% |  |
|  | C | 2% | 13.7% |  |
| Xiaomi#1 | A |  | 24.47% |  |
|  | B |  | 28.24% |  |
|  | C |  | 27.36% |  |
| QC#1 | A |  | 8% |  |
|  | B |  | 10% |  |
|  | C |  | 3% |  |
| Intel#1 | A |  | 7% |  |
|  | B |  | 9% |  |
|  | C |  | 7% |  |
| InterDigital#1 | A |  | 4% |  |
|  | B |  | 4% |  |
|  | C |  | 0.8% |  |
| ZTE#1 | A |  |  | 8.51% |
|  | B |  |  | 7.44% |
|  | C |  |  | 9.95% |
| Summary | A | 6%~11% | 4%~15% | 8.51% |
|  | B | 3%~7% | 4%~10% | 7.44% |
|  | C | 1.1%~11% | 0.2%~14% | 9.95% |

#### Issue#3-6 (High priority) 5% UPT, full buffer

Proposed Observation 3.3.2: For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer,

- For Max rank 1, 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~20.9%

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.5%~20.9% at CSI overhead A (small overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.3%~17.4% at CSI overhead B (medium overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~6.62% at CSI overhead C (large overhead);

- For Max rank 2, 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~14.9%

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 4.1%~14.9% at CSI overhead A (small overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 0.3%~4% at CSI overhead B (medium overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~6.03% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 8.76%~30.17%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 3.59%~5.36% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.5-2 of R1-2306061

Table 5.5-2. Gain of CSI comporession over R16 Type II benchmark – 5% UPT, full buffer

|  | CSI overhead | R1 | R2 | R4 |
| --- | --- | --- | --- | --- |
| Nokia #1 | A | 2.5% | 4.1% |  |
|  | B | 2.3% | 0.3% |  |
|  | C | 0% | -2.11% |  |
| vivo#1 | A | 8.81% | 13.8% |  |
|  | B | 8.66% | 3.05% |  |
|  | C | 6.62% | 6.03% |  |
| Fujitsu#1 | A | 20.9% | 14.90% |  |
|  | B | 17.4% | 3.50% |  |
|  | C | 6.1% | -7% |  |
| Xiaomi#1 | A |  | 30.17% |  |
|  | B |  | 21% |  |
|  | C |  | 8.76% |  |
| QC#1 | A |  | 9% |  |
|  | B |  | 3% |  |
|  | C |  | 0 |  |
| Intel#1 | A |  | 5% |  |
|  | B |  | 4% |  |
|  | C |  | 3% |  |
| ZTE#1 | A |  |  | 5.36% |
|  | B |  |  | 4.72% |
|  | C |  |  | 3.59% |
| Summary | A | 2.5%~20.9% | 4.1%~14.9% | 5.36% |
|  | B | 2.3%~17.4% | 0.3%~4% | 4.72% |
|  | C | 0%~6.62% | -7%~6.03% | 3.59% |

#### Issue#3-10a (New) High resolution ground-truth CSI for training-Benchmark of lower bound

Upd Proposal 3.3.3: For the evaluation of the R16 eType II-like codebook based high resolution quantization of the ground-truth CSI in the CSI compression for AI/ML training, regarding the evaluation of new values of eType II parameters, consider the legacy values of PC8 as the baseline/lower-bound of performance comparison.

- Note: it has been agreed that Float32 is adopted as the baseline/upper-bound of performance comparison.

#### Issue#3-11a (High priority) Rank>1 solutions-down selection

Moderator note[Rd4]: Number of companies added.

Proposed Conclusion 3.3.4: For further study the evaluation of CSI compression, for the rank>1 solutions, prioritize Option 2-1 (layer specific and rank common), Option 3-1 (layer common and rank common), and Option 3-2 (layer common and rank specific).

- Note: For the evaluations of CSI compression with 1-on-1 joint training,

  - Option 1-2 is not adopted;

  - Option 2-2 is not adopted;

  - Over the 11 sources [Huawei, Ericsson, Nokia, ZTE, OPPO, Intel, Fujitsu, NTT DOCOMO, InterDigital, Qualcomm, Futurewei] for FTP/fullbuffer, Max Rank 2,

    - Option 1-1 is adopted by 1 source [ZTE]

    - Option 2-1 is adopted by 3 sources [Ericsson, Intel, Qualcomm]

    - Option 3-1 is adopted by 9 sources [Huawei, Nokia, ZTE, OPPO, Fujitsu, Futurewei, Xiaomi, vivo, NTT DOCOMO]

    - Option 3-2 is adopted by 2 sources [NTT DOCOMO, InterDigital]

  - Over the 4 sources [Qualcomm, ZTE, CATT, Apple] for FTP/fullbuffer, Max Rank 4,

    - Option 2-1 is adopted by 1 source [Qualcomm]

    - Option 3-1 is adopted by 2 sources [Qualcomm, ZTE]

    - Option 3-2 is adopted by 1 source [Apple]

#### Issue#3-14 (High priority) Generalization-Carrier frequencies

Proposed Observation 3.3.5: For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B,

- For generalization Case 2, generalized performance may be achieved in general

  - If carrier frequency#A is 3.5/4GHz & carrier frequency#B is 2GHz, 3 sources [NTT DOCOMO, Nokia, MediaTek] observe generalized performance of less than -1.72% degradation.

  - If carrier frequency#A is 2GHz & carrier frequency#B is 3.5/4GHz, 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek] observe generalized performance of less than -1% degradation or positive gain.

    - Note: 1 source [Nokia] observes significant degradations of -6.6%.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-0.8% loss or positive gain) for carrier frequency#B subject to any of 2GHz and 3.5/4GHz, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B, as observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Minor loss (0%~-0.8%) are observed by 3 sources [Nokia, vivo, MediaTek].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Note: Significant degradations of up to -4.9% are still observed by 1 source [Nokia] for carrier frequency#B subject to 3.5/4GHz

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Antenna layouts are assumed as the same over the different frequency carriers.

  - Note: Results refer to Table 5.5-3 of R1-2306061

Table 5.5-3. Generalization of CSI compression– carrier frequencies

| Training, Generalization Case 1 |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Training, Generalization Case 2 |  |  |  | 3.5/4GHz |  |  | 2GHz |  |
| Training, Generalization Case 3 |  |  | 2GHz+3.5/4GHz | 5.5GHz+3.5/4GHz |  | 2GHz+3.5/4GHz | 5.5G+3.5/4GHz |  |
| Testing |  |  | 2GHz |  |  | 3.5/4GHz |  |  |
|  | Case | CSI payload |  |  |  |  |  |  |
| vivo#1/#2 | Case 1 | X |  |  |  | 0.788 |  |  |
|  |  | Y |  |  |  | 0.847 |  |  |
|  |  | Z |  |  |  | 0.9013 |  |  |
|  | Case 2 | X |  |  |  |  | 0.782/-0.76% |  |
|  |  | Y |  |  |  |  | 0.838/-1% |  |
|  |  | Z |  |  |  |  | 0.8992/-0.23% |  |
|  | Case 3 | X |  |  |  | 0.788/0% | 0.789/0.127% |  |
|  |  | Y |  |  |  | 0.845/-0.236% | 0.848/0.12% |  |
|  |  | Z |  |  |  | 0.9015/0.02% | 0.9002/-0.12% |  |
| NTT DOCOMO #1#2#3#4#5#6 | Case 1 | X | 0.58 |  |  | 0.58 |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | X |  | -1.72% |  |  | 1.72% |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 3 | X | 0%/0%/1.72% |  |  | 1.72%/1.72%/1.72% |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
| Nokia#1#2 | Case 1 | X | 0.715 |  |  | 0.691 |  |  |
|  |  | Y | 0.763 |  |  | 0.796 |  |  |
|  |  | Z | 0.886 |  |  | 0.880 |  |  |
|  | Case 2 | X |  | -0.8% |  |  | 0.1% |  |
|  |  | Y |  | 4.8% |  |  | -6.6% |  |
|  |  | Z |  | 0.1% |  |  | -0.9% |  |
|  | Case 3 | X | -0.1% |  |  | 0.3% |  |  |
|  |  | Y | 1.0% |  |  | -4.9% |  |  |
|  |  | Z | 0.1% |  |  | -0.3% |  |  |
| MTK#1#2 | Case 1 | X | 0.696 |  |  | 0.702 |  |  |
|  |  | Y | 0.771 |  |  | 0.773 |  |  |
|  |  | Z | 0.831 |  |  | 0.835 |  |  |
|  | Case 2 | X |  | 0.3% |  |  | -0.1% |  |
|  |  | Y |  | -0.5% |  |  | 0.5% |  |
|  |  | Z |  | -0.2% |  |  | 0.4% |  |
|  | Case 3 | X | 0.0% |  |  | -0.3% |  |  |
|  |  | Y | -0.8% |  |  | -0.3% |  |  |
|  |  | Z | -0.2% |  |  | 0.1% |  |  |
| Summary | Case 1 | X |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |
|  | Case 2 | Minor loss (>-1.8%) |  | NTT DOCOMO, Nokia, MTK |  |  | Vivo, Nokia, MTK |  |
|  |  | Moderate/significant loss (<-1.8%) |  |  |  |  | Nokia |  |
|  |  | Positive gain |  | MTK, Nokia |  |  | NTT DOCOMO, Nokia, MTK |  |
|  | Case 3 | Minor loss (>-1.6%) | Nokia, MTK |  |  | Nokia, Vivo MTK | Vivo |  |
|  |  | Significant loss (<-4%) |  |  |  | Nokia |  |  |
|  |  | Positive gain | NTT DOCOMO, Nokia |  |  | Vivo, NTT DOCOMO, Nokia, MTK | vivo |  |

#### Issue#3-20 (High priority) NW first training-1 NW to 1/M>1 UE

Upd Proposed Observation 3.3.6: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 4 sources [Ericsson, Nokia, ZTE，Fujitsu] observe -0%~-0.5% degradation, 5 sources [Nokia, Qualcomm, ZTE, CATT, vivo] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Huawei, Apple, CMCC] observe -0%~-0.8% degradation.

- Note: For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [Qualcomm, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.5-4 of R1-2306061

Table 5.5-4. NW first training for CSI compression -1 NW to 1/M>1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip |  |  |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo, CATT | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.8% |  | HW, Apple, CMCC, |  |  |

#### Issue#3-20a (Medium priority) NW first training-1 NW to 1/M>1 UE- Impact of size of shared training dataset

Proposed Observation 3.3.7: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1/M>1 UE, as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [Huawei, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table 5.5-5 of R1-2306061

Table 5.5-5. NW first training for CSI compression -1 NW to 1/M>1 UE with various dataset settings

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) dataset size | Same backbone, same/diff dataset size | Diff backbone; Cap_UE model < Cap_NW model, same/diff dataset size |
| --- | --- | --- | --- | --- |
| HW#1/2 | Descrip | TF#A-TF#1, 300k | TF#A-TF#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.1%/-0.3% Gap: -0.2% |  |
|  | Z |  | 0%/-0.1% Gap: -0.1% |  |
|  | Descrip | CNN#A-CNN#1, 300k | CNN#A-CNN#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.4%/-0.5% Gap: -0.1% |  |
|  | Z |  | -0.3%/-0.5% Gap: -0.2% |  |
| CMCC#1~7 | Descrip | TF#A-TF#1, 154k | TF#A-TF#1, 154k/100k/50k/ 10k/5k |  |
|  | X |  |  |  |
|  | Y |  | -0.22%/-0.22%/-0.55%/-1.86%/-2.95% Gap: 0/-0.3%/-1.64%/-2.73% |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-CNN#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -4.04%/-4.59%/-5.90%/-8.30%/-9.61% Gap: -0.55%/-1.86%/-4.26%/-5.55% |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-MLP#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -0.76%/-0.98%/-1.53%/-6.33%/-9.17% Gap: -0.22%/-0.77%/-5.57%/-8.41% |
|  | Z |  |  |  |
| vivo#1/2 | Descrip | TF#A-TF#1, 300k; TF#A-TF#2, 300k; | TF#A-TF#1, 300k/50k; TF#A-TF#2, 300k/50k; |  |
|  | X |  | -0.6%/-2.6%; -0.9%/-2.6% |  |
|  | Y |  |  |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-CNN#1, 300k |  | TF#A-CNN#1, 300k/50k |
|  | X |  |  | -5.2%/ -9.4% Gap: -3.8% |
|  | Y |  |  |  |
|  | Z |  |  |  |
| Summary: gap of additional loss | -0%~-0.55% |  | HW, CMCC, |  |
|  | -0.55%~-8.41% |  |  | CMCC, vivo |

#### Issue#3-22 (High priority) UE first training-1/N>1 NW to 1 UE

Proposed Observation 3.3.8: For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1/N>1 NW to 1 UE, as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [Huawei] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.5-6 of R1-2306061

Table 5.5-6. UE first training for CSI compression-1/N>1 NW to 1 UE

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model > Cap_NW model | Diff backbone or largely diff structure; Cap_UE model < Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A/B/C-TF#1 | TF#A/B/C-TF#1 |  |  |
|  | X |  | 0%/-0.4%/-0.2% |  |  |
|  | Y |  | 0%/-0.6%/-0.2% |  |  |
|  | Z |  | 0%/-0.6%/-0.1% |  |  |
|  | Descrip | CNN#A/B-CNN#1 | CNN#A/B-CNN#1 |  | TF#A- CNN#1 |
|  | X |  | -0.6%/-0.6% |  | -0.3% |
|  | Y |  | -0.5%/-0.6% |  | 0% |
|  | Z |  | -0.5%/-0.7% |  | 0% |
| Nokia#3/4 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | +1.7%/-0.2% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1; CNN#A-CNN#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | L1: +0.2%; L2: -0.04%; L3: +0.03%; L4:+0.2%; L1: -0.2% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  | +0.6% |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -0.3% |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A/B/C /D-TF#1 | TF#A/B/C /D-TF#1 |  |  |
|  | X |  | -1.8%/ -2.3%/ -2.6%/ -2.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: -1.13% L2: -1.55% |  |  |
|  | Y |  | L1: -1.07% L2: -1.52% |  |  |
|  | Z |  | L1: -1.05% L2: -1.75% |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  | L1: -0.99% L2: -1.06% |  |
|  | Y |  |  | L1: -0.98% L2: -1.88% |  |
|  | Z |  |  | L1: -0.74% L2: -0.92% |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  | L1: -0.85% L2: -0.73% |
|  | Y |  |  |  | L1: -0.84% L2: -1.74% |
|  | Z |  |  |  | L1: -0.67% L2: -0.78% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.03% |  |  |
|  | Y |  | -0.31% |  |  |
|  | Z |  | -0.14% |  |  |
|  | Descrip | TF#A-TF#1 |  | CNN #A-TF#1 |  |
|  | X |  |  | -5.82% |  |
|  | Y |  |  | -5.51% |  |
|  | Z |  |  | -4.18 % |  |
| CATT#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.14%/ 0.21% |  |  |
|  | Y |  | -0.23%/-0.27% |  |  |
|  | Z |  | -0.14%/-0.35% |  |  |
|  | Descrip | CNN#A-TF #1 |  | CNN#A-TF #1 |  |
|  | X |  |  | 1.07% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -0.90% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +5.5%; +1.3%/ +2.7%; +17.1%/ +9.2% |  |  |
|  | Descrip |  |  |  | TF#A/B-CNN#1; TF#A/B-CNN#2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A/B- TF#1; |  | CNN#A/B- TF#1; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | 0.0%/ -1.4% | +5.8%/ +7.4% |
| vivo#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.4%/ -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A- TF#1 |  | CNN#A- TF#1 |  |
|  | X |  |  | -1.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | Nokia, Fujitsu, CATT, vivo, QC | QC |  |
|  | -0.5%~-1% |  |  | CATT, ZTE | ZTE |
|  | -1%~-1.88% |  | ZTE | ZTE, vivo | ZTE |
| Summary (Before Q) | -0%~-0.5% |  | HW |  |  |
|  | -0.5%~-1% |  | HW |  |  |
|  | -1%~-2.9% |  | Apple, |  |  |

## 26 May (Fri.) Offline/GTW

#### Issue#2-2 (Medium priority) How to evaluate the generalization performance of the proxy model for Step 2

Upd Proposal 2.4.1: For the intermediate KPI monitoring of CSI compression, for Case 2: UE side monitoring of intermediate KPI with a proxy model, evaluate the performance of the proxy model under various scenarios to observe values of KPIDiff separately.

- E.g., generalization Case 1/3 and generalization Case 2 are reused to derive different KPIDiff values, separately; for generalization Case 2, both the actual pair of models and the proxy model are trained under scenario#A/#A+#B and tested on scenario#B.

- Note: the target is to evaluate whether the performance of the proxy model and the actual pair of models are equally impacted when the scenario changes.

#### Issue#2-5 (High priority) RI calculation method

Proposal 2.4.2: For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, update the entry of CQI determination method(s) to include also the RI determination:

| Common description | Input type |
| --- | --- |
|  | Output type |
|  | Quantization /dequantization method |
|  | Rank/layer adaptation settings for rank>1 |
|  | CQI/RI determination method(s) for AI/ML (Option 1a/1b/1c/2a/2b, etc.) |

#### Issue#3-20 (High priority) NW first training-1 NW to 1/M>1 UE

Upd Proposed Observation 3.4.1: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 5 sources [Ericsson, Nokia, ZTE，Fujitsu, Samsung] observe -0%~-0.5% degradation, 6 sources [Nokia, Qualcomm, ZTE, CATT, vivo, Samsung] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Huawei, Apple, CMCC] observe -0%~-0.8% degradation.

- Note: For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [Qualcomm, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.6-1 of R1-2306062

Table 5.6-1. NW first training for CSI compression -1 NW to 1 UE (Case 1)

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model < Cap_NW model | Diff backbone or largely diff structure; Cap_UE model > Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1/2/3 |  |  |
|  | X |  | -0.2%/-0.4%/-0.3% |  |  |
|  | Y |  | -0.1%/ -0.5%/ -0.2% |  |  |
|  | Z |  | 0%/ 0%/ -0.1% |  |  |
|  | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1/2 |  | CNN#A- TF#1 |
|  | X |  | -0.2%/ -0.4% |  | +0.1% |
|  | Y |  | -0.4%/ -0.4% |  | +0.2% |
|  | Z |  | -0.3%/ -0.5% |  | 0% |
| E#1 (After Q) | Descrip | CNN#A-CNN#1 | CNN#A-CNN#1 |  |  |
|  | X |  | L1: -0.3% L2: -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Nokia#2 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | 0%/-1.3% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.7% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -2.1 % |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A-TF#1/3/4 (similar size) | TF#A-TF#1/3/4 (similar size) |  |  |
|  | X |  | -0.1%/-0.4%/-0.3% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A (2.5M size)-TF#2 (550k size) | TF#A (2.5M size)-TF#2 (550k size) |  |  |
|  | X |  | -0.8% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: 0.00% L2: -0.05% |  |  |
|  | Y |  | L1: -1% L2: -1.22% |  |  |
|  | Z |  | L1: -0.02% L2: -0.07% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | L1: -0.03% L2: -0.09% |  |
|  | Y |  |  | L1: -0.13% L2: -0.94% |  |
|  | Z |  |  | L1: -0.06% L2: -0.15% |  |
|  | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  | L1: -0.08% L2: -0.26% |
|  | Y |  |  |  | L1: -0.19% L2: -0.64% |
|  | Z |  |  |  | L1: -0.12% L2: -0.20% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.33% |  |  |
|  | Y |  | -0.2% |  |  |
|  | Z |  | -0.12% |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.42% |  |
|  | Y |  |  | -7.46 |  |
|  | Z |  |  | -4.62% |  |
| CMCC#1/3/5 (Before Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | -0.22% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-TF#1 |  | TF#A-CNN#1/MLP#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -4.04%/-0.76% |  |
| CATT#1 (After Q) | Descrip | TF#A-TF#1/TF#2 | TF#A-TF#1/TF#2 |  |  |
|  | X |  | -0.52%/ -0.59% |  |  |
|  | Y |  | -0.63%/ -0.66% |  |  |
|  | Z |  | -0.71%/ -0.79% |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -0.92% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -1.00% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; | CNN#A-CNN#1/2; CNN#B-CNN#1/2; TF#A-TF#1/2; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +1.4%; +4.2%/ +4.1%; +1.23%/ -8.6% |  |  |
|  | Descrip | CNN#A-TF#1/2; CNN#B-TF#1/2 |  |  | CNN#A-TF#1/2; CNN#B-TF#1/2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | +4.2%/ +2.7%; +5.6%/ +2.7% |
|  | Descrip | TF#A-CNN#1/2; |  | TF#A-CNN#1/2; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | -20.0%/ -17.6% |  |
| vivo#1 (After Q) | Descrip | TF#A-TF#1/2 | TF#A-TF#1/2 |  |  |
|  | X |  | -0.6%/ -0.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  | -5.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| SS#1 (After Q) | Descrip | TF#A-TF#1/TF#2; CNN#A-CNN#1 |  | TF#A-CNN#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +0.3%/-0.4%; -0.09% | -17.5% |  |
| SS#1 (After Q) | Descrip | CNN#A-TF#1 |  |  | CNN#A-TF#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  | -4.5% +4.2% |
| Summary (After Q) | -0%~-0.5% |  | E///, Nokia, ZTE，Fujitsu, SS | ZTE |  |
|  | -0.5%~-1% |  | Nokia, QC, ZTE, vivo, CATT, SS | ZTE, CATT |  |
|  | -1%~-1.3% |  | Nokia, ZTE |  |  |
|  | -2.1%~-5.2% |  |  | QC, vivo |  |
| Summary (Before Q) | -0%~-0.5% |  | HW, Apple, CMCC, |  |  |

#### Issue#3-20a (Medium priority) NW first training-1 NW to 1/M>1 UE- Impact of size of shared training dataset

Proposed Observation 3.4.2: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [Huawei, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table 5.6-2 of R1-2306062

Table 5.6-2. NW first training for CSI compression -1 NW to 1 UE (Case 1) with various dataset settings

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) dataset size | Same backbone, same/diff dataset size | Diff backbone; Cap_UE model < Cap_NW model, same/diff dataset size |
| --- | --- | --- | --- | --- |
| HW#1/2 | Descrip | TF#A-TF#1, 300k | TF#A-TF#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.1%/-0.3% Gap: -0.2% |  |
|  | Z |  | 0%/-0.1% Gap: -0.1% |  |
|  | Descrip | CNN#A-CNN#1, 300k | CNN#A-CNN#1, 300k/100k |  |
|  | X |  | -0.2%/-0.5% Gap: -0.3% |  |
|  | Y |  | -0.4%/-0.5% Gap: -0.1% |  |
|  | Z |  | -0.3%/-0.5% Gap: -0.2% |  |
| CMCC#1~7 | Descrip | TF#A-TF#1, 154k | TF#A-TF#1, 154k/100k/50k/ 10k/5k |  |
|  | X |  |  |  |
|  | Y |  | -0.22%/-0.22%/-0.55%/-1.86%/-2.95% Gap: 0/-0.3%/-1.64%/-2.73% |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-CNN#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -4.04%/-4.59%/-5.90%/-8.30%/-9.61% Gap: -0.55%/-1.86%/-4.26%/-5.55% |
|  | Z |  |  |  |
|  | Descrip | TF#A-TF#1, 154k |  | TF#A-MLP#1, 154k/100k/50k/ 10k/5k |
|  | X |  |  |  |
|  | Y |  |  | -0.76%/-0.98%/-1.53%/-6.33%/-9.17% Gap: -0.22%/-0.77%/-5.57%/-8.41% |
|  | Z |  |  |  |
| vivo#1/2 | Descrip | TF#A-TF#1, 300k; TF#A-TF#2, 300k; | TF#A-TF#1, 300k/50k; TF#A-TF#2, 300k/50k; |  |
|  | X |  | -0.6%/-2.6%; -0.9%/-2.6% |  |
|  | Y |  |  |  |
|  | Z |  |  |  |
|  | Descrip | TF#A-CNN#1, 300k |  | TF#A-CNN#1, 300k/50k |
|  | X |  |  | -5.2%/ -9.4% Gap: -3.8% |
|  | Y |  |  |  |
|  | Z |  |  |  |
| Summary: gap of additional loss | -0%~-0.55% |  | HW, CMCC, |  |
|  | -0.55%~-8.41% |  |  | CMCC, vivo |

#### Issue#3-22 (High priority) UE first training-1/N>1 NW to 1 UE

Proposed Observation 3.4.3: For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [Huawei] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.6-3 of R1-2306062

Table 5.6-3. UE first training for CSI compression-1 NW to 1 UE (Case 1)

|  | CSI payload | Benchmark (#A~D: NW part; #1~4: UE part) | Same backbone | Diff backbone; Cap_UE model > Cap_NW model | Diff backbone or largely diff structure; Cap_UE model < Cap_NW model |
| --- | --- | --- | --- | --- | --- |
| HW#1/2 (Before Q) | Descrip | TF#A/B/C-TF#1 | TF#A/B/C-TF#1 |  |  |
|  | X |  | 0%/-0.4%/-0.2% |  |  |
|  | Y |  | 0%/-0.6%/-0.2% |  |  |
|  | Z |  | 0%/-0.6%/-0.1% |  |  |
|  | Descrip | CNN#A/B-CNN#1 | CNN#A/B-CNN#1 |  | TF#A- CNN#1 |
|  | X |  | -0.6%/-0.6% |  | -0.3% |
|  | Y |  | -0.5%/-0.6% |  | 0% |
|  | Z |  | -0.5%/-0.7% |  | 0% |
| Nokia#3/4 (After Q) | Descrip | TF#A-TF#1/ TF#B-TF#2 | TF#A-TF#1/ TF#B-TF#2 |  |  |
|  | X |  |  |  |  |
|  | Y |  | +1.7%/-0.2% |  |  |
|  | Z |  |  |  |  |
| QC#1 (After Q) | Descrip | TF#A-TF#1; CNN#A-CNN#1 | TF#A-TF#1 |  |  |
|  | X |  |  |  |  |
|  | Y |  | L1: +0.2%; L2: -0.04%; L3: +0.03%; L4:+0.2%; L1: -0.2% |  |  |
|  | Z |  |  |  |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  |  |
|  | Y |  |  |  | +0.6% |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  |  |  |
|  | Y |  |  | -0.3% |  |
|  | Z |  |  |  |  |
| Apple#1 (Before Q) | Descrip | TF#A/B/C /D-TF#1 | TF#A/B/C /D-TF#1 |  |  |
|  | X |  | -1.8%/ -2.3%/ -2.6%/ -2.9% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| ZTE#1/2/ 3 (after Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | L1: -1.13% L2: -1.55% |  |  |
|  | Y |  | L1: -1.07% L2: -1.52% |  |  |
|  | Z |  | L1: -1.05% L2: -1.75% |  |  |
|  | Descrip | CNN#A-TF#1 |  | CNN#A-TF#1 |  |
|  | X |  |  | L1: -0.99% L2: -1.06% |  |
|  | Y |  |  | L1: -0.98% L2: -1.88% |  |
|  | Z |  |  | L1: -0.74% L2: -0.92% |  |
|  | Descrip | TF#A-CNN#1 |  |  | TF#A-CNN#1 |
|  | X |  |  |  | L1: -0.85% L2: -0.73% |
|  | Y |  |  |  | L1: -0.84% L2: -1.74% |
|  | Z |  |  |  | L1: -0.67% L2: -0.78% |
| Fujitsu#1 (After Q) | Descrip | TF#A-TF#1 | TF#A-TF#1 |  |  |
|  | X |  | -0.03% |  |  |
|  | Y |  | -0.31% |  |  |
|  | Z |  | -0.14% |  |  |
|  | Descrip | TF#A-TF#1 |  | CNN #A-TF#1 |  |
|  | X |  |  | -5.82% |  |
|  | Y |  |  | -5.51% |  |
|  | Z |  |  | -4.18 % |  |
| CATT#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.14%/ 0.21% |  |  |
|  | Y |  | -0.23%/-0.27% |  |  |
|  | Z |  | -0.14%/-0.35% |  |  |
|  | Descrip | CNN#A-TF #1 |  | CNN#A-TF #1 |  |
|  | X |  |  | 1.07% |  |
|  | Y |  |  | -0.98% |  |
|  | Z |  |  | -0.90% |  |
| MTK#1~ 3 (After Q) | Descrip | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; | CNN#A/B-CNN#1; CNN#A/B-CNN#2; TF#A/B-TF#1; |  |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  | +1.4%/ +5.5%; +1.3%/ +2.7%; +17.1%/ +9.2% |  |  |
|  | Descrip |  |  |  | TF#A/B-CNN#1; TF#A/B-CNN#2 |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A/B- TF#1; |  | CNN#A/B- TF#1; |  |
|  | X |  |  |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  | 0.0%/ -1.4% | +5.8%/ +7.4% |
| vivo#1 (After Q) | Descrip | TF#A/B-TF#1 | TF#A/B-TF#1 |  |  |
|  | X |  | -0.4%/ -0.2% |  |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
|  | Descrip | CNN#A- TF#1 |  | CNN#A- TF#1 |  |
|  | X |  |  | -1.2% |  |
|  | Y |  |  |  |  |
|  | Z |  |  |  |  |
| Summary (After Q) | -0%~-0.5% |  | Nokia, Fujitsu, CATT, vivo, QC | QC |  |
|  | -0.5%~-1% |  |  | CATT, ZTE | ZTE |
|  | -1%~-1.88% |  | ZTE | ZTE, vivo | ZTE |
| Summary (Before Q) | -0%~-0.5% |  | HW |  |  |
|  | -0.5%~-1% |  | HW |  |  |
|  | -1%~-2.9% |  | Apple, |  |  |

#### Issue#3-26 (New) How to evaluate the price of complexity

Moderator note[Rd5]: In the fourth round, it is raised by Samsung that we may need observation for the complexity of the AI/ML model, e.g., as the comparison to the legacy eType II CB. From the submitted results, the reported values of FLOPs over companies range from <1 M to 800M. It is Moderator’s understanding that it may be helpful to address the comprehensive information of introducing AI/ML, not only based on performance but also on the price, to provide a trade-off AI/ML solution, including the model structure, rank>1 options, etc.

| Samsung | It is useful if some observations are drawn by comparing the complexity of AI/ML models and the algorithms to obtain legacy CSI. Companies have already submitted the complexity in terms of FLOPs for their AI/ML models. The same can be done for legacy CSI calculation. Moreover, it can be left up to companies how to obtain the complecity of legacy CSI but companies can employ the basic FLOPs counting methods for matrix/vectors operations such as multiplications, additions, SVD, etc. |
| --- | --- |

Question 5.6.1: For the evaluation of AI/ML based CSI compression and CSI prediction, whether/how to draw observations in terms of the complexity metric?

- Option 1: Compare the FLOPs and/or memory storage between AI/ML and legacy non-AI/ML (e.g., eType II, non-AI/ML based CSI prediction, etc.) where how to calculate FLOPs and/or memory storage for legacy is up to companies.

- Option 2: Compare the FLOPs and/or memory storage among different AI/ML solutions, e.g., rank>1 options for CSI compression, etc.

# Agreements for #113

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, Samsung, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 5 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] observe 18.72%~31.3% gain, and 3 sources [InterDigital, MediaTek, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [Huawei, Samsung, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - No spatial consistency is considered by 11 sources [Fujitsu, Samsung, Xiaomi, InterDigital, CMCC, OPPO, ETRI, ZTE, Apple, Huawei, Spreadtrum]. 1 source [vivo] provides both results with spatial consistency and results w/o spatial consistency.

  - Note: Results refer to Table 5.1-1 of R1-2306059

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of SGCS,

- For Max rank 1, Layer 1,

  - 11 sources [Huawei, Nokia, ZTE, vivo, OPPO, ETRI, Fujitsu, CMCC, China Telecom, MediaTek, Apple] observe the performance gain of 2.6%~ 8.8% at CSI payload X (small payload);

  - 14 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, ETRI, Fujitsu, NTT DOCOMO, Xiaomi, China Telecom, MediaTek, BJTU, Apple] observe the performance gain of 0.9%~ 8.1% at CSI payload Y (medium payload);

  - 11 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, ETRI, Fujitsu, BJTU, Apple] observe the performance gain of 0.9%~ 7% at CSI payload Z (large payload);

  - Note: 1 source [Futurewei] observes the performance gain of 11.6% at CSI payload X (small payload) which biases from the majority range.

- For Max rank 2, Layer 1,

  - 12 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 3.9%~ 11% at CSI payload X (small payload);

  - 11 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Qualcomm, Apple] observe the performance gain of 0.7%~ 4.5% at CSI payload Y (medium payload);

  - 9 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Apple] observe the performance gain of -0.2%~ 6.5% at CSI payload Z (large payload);

- For Max rank 2, Layer 2, more gains are observed in general compared with Layer 1 of Max rank 2:

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 5.92%~ 30.2% at CSI payload X (small payload);

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 1.5%~ 23.08% at CSI payload Y (medium payload);

  - 10 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Intel, Apple] observe the performance gain of 4.4%~ 12.99% at CSI payload Z (large payload);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1 of Max rank 1 or Layer 1/2 of Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-2 of R1-2306059

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under FTP traffic, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~2%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.29%~2% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~1% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.33%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.09%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4% at CSI overhead C (large overhead);

  - For RU>=70%, 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.23%~9%

    - 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.38%~9% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.62%~5% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.23%~6% at CSI overhead C (large overhead);

  - Note: 2 sources [Spreadtrum, Futurewei] observe gain of 12.77%~21.21% at RU 40%-69%, 11.23%~21.5% at RU>=70%, which bias from the majority ranges.

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 6 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm, Fujitsu] observe the performance gain of -0.3%~6%

    - 5 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm] observe the performance gain of 1%~6% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.5%~6% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Intel, Fujitsu, Qualcomm] observe the performance gain of -0.3%~6% at CSI overhead C (large overhead);

  - For RU 40%-69%, 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm, InterDigital] observe the performance gain of -0.5%~10%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 3%~10% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 1.2%~9% at CSI overhead B (medium overhead);

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm, InterDigital] observe the performance gain of -0.5%~9% at CSI overhead C (large overhead);

  - For RU>=70%, 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of -0.2%~15%

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 5%~15% at CSI overhead A (small overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 3%~9% at CSI overhead B (medium overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Fujitsu, Qualcomm] observe the performance gain of -0.2%~12% at CSI overhead C (large overhead);

  - Note: 4 sources [Futurewei, NTT DOCOMO, InterDigital, Fujitsu] observe gain of 7%~30% at RU<=39%, 10%~23% at RU 40%-69%, 12.71%~26.8% at RU>=70%, which bias from the majority ranges.

- For Max rank 4:

  - For RU<=39%, 3 sources [CATT, Apple, Qualcomm] observe the performance gain of -4%~7.4%

    - 3 sources [CATT, Apple, Qualcomm] observe the performance gain of 2.5%~7.4% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 6% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -4%~0% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~12.22%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~12.22% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 7.04%~11% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~8.19% at CSI overhead C (large overhead);

  - For RU>=70%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~17%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.64%~17% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~8.40% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-3 of R1-2306059

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under FTP, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.8%~3%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.72%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~1.2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.68%~3% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~7%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 2.8%~7% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.22%~2.7% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~3.25% at CSI overhead C (large overhead);

  - For RU>=70%, 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~20.43%

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 4%~20.43% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 1%~10.13% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~8% at CSI overhead C (large overhead);

  - Note: 2 sources [Spreadtrum, Futurewei] observe gain of 15.87%~21.04% at RU 40%-69%, 20.2%~50% at RU>=70%, which bias from the majority ranges.

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -2%~5%

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 1.1%~5% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of -2%~3% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -0.5%~5% at CSI overhead C (large overhead);

  - For RU 40%-69%, 7 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel, Fujitsu] observe the performance gain of -4%~13%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 7%~13% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.3%~8% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu] observe the performance gain of -4%~8% at CSI overhead C (large overhead);

  - For RU>=70%, 9 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO, Qualcomm, Futurewei] observe the performance gain of -1.3%~24%

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO] observe the performance gain of 10.26%~24% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 9%~15.02% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Futurewei, Intel] observe the performance gain of -1.3%~13.67% at CSI overhead C (large overhead);

  - Note: 5 sources [Intel, NTT DOCOMO, InterDigital, Fujitsu, ZTE] observe gain of 7%~24% at RU<=39%, -8%~-2%, 13.4%~29.7% at RU 40%-69%, -5%~-10%, 18.1%~35.4% at RU>=70%, which bias from the majority ranges.

- For Max rank 4:

  - For RU<=39%, 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~10%

    - 2 sources [Apple, Qualcomm] observe the performance gain of 8%~10% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 5% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~23%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.17%~23% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~9.47% at CSI overhead C (large overhead);

  - For RU>=70%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~31%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5.8%~31% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 10.2%~30% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~15% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-4 of R1-2306059

Observation

For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - If deployment scenario#A is UMi & deployment scenario#B is UMa, deployment scenario#A is UMa & deployment scenario#B is UMi, or deployment scenario#A is UMa & deployment scenario#B is InH:

    - 9 sources [Xiaomi, InterDigital, MediaTek, vivo, Intel, ZTE, OPPO, Huawei, CATT] observe that generalized performance can be achieved:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Xiaomi, InterDigital, MediaTek, vivo, Intel, ZTE, CATT] observe less than -1.6% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 5 sources [vivo, OPPO, MediaTek, Intel, Xiaomi] observe less than -1.4% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [Huawei, CATT] observe less than -0.6% degradation or positive gain

    - 10 sources [Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT, ZTE, Lenovo, MediaTek, Futurewei] observe that moderate/significant degradations are suffered under generalization Case 2:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 8 sources [Futurewei, MediaTek, Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT] observe -1.69%~-14.2% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 7 sources [Futurewei, NTT DOCOMO, ZTE, InterDigital, CATT, Xiaomi, Intel] observe -1.81%~-18.5% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [ZTE, Lenovo] observe -1.74%~-3.6% degradation.

  - If deployment scenario#A is InH & deployment scenario#B is Uma/UMi, significant performance degradations are observed under generalization Case 2:

    - For deployment scenario#A is InH & deployment scenario#B is UMa, 4 sources [Huawei, CATT, Lenovo, ZTE] observe -5.55%~-21.76% degradation.

    - For deployment scenario#A is InH & deployment scenario#B is UMi, 2 sources [vivo, ZTE] observe -8.63%~-20% degradation.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MediaTek, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Minor loss (0%~-1.48%) are observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MediaTek, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Moderate loss (-1.6%~-4%) are observed by 5 sources [Xiaomi, CATT, vivo, NTT DOCOMO, Intel].

  - Positive gains are observed by 8 sources [ZTE, Interdigital, MediaTek, vivo, Intel, Xiaomi, Futurewei, CATT].

  - Note: Significant degradations of up to -6.7% are still observed by 2 sources [Intel, Xiaomi] for deployment scenario#B subject to UMa, and by 2 sources [Intel, CATT] for deployment scenario#B subject to UMi.

- Note: For generalization Case 2, if deployment scenario#A is UMi & deployment scenario#B is InH, 2 sources [vivo, ZTE] observe different trends, where significant performance degradations of -27.8%~-29.9% are observed by [vivo], while moderate performance degradations of -1.44%~-2.41% are observed by [ZTE].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.1-5 of R1-2306059

Observation

For the generalization verification of AI/ML based CSI compression over various UE distributions, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE distribution#B and applied for inference with a same UE distribution#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE distribution#A and UE distribution#B but not for others

  - If UE distribution#A is Outdoor & UE distribution#B is Indoor, 3 sources [Nokia, Qualcomm, Huawei] observe that moderate/significant degradations of -2.9%~-11.5% degradation are suffered,

    - Note: 1 source [NTT DOCOMO] observes 0% degradation

  - If UE distribution#A is Indoor & UE distribution#B is Outdoor, 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei] observe minor loss of less than -0.7% degradation or positive gain

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-1% loss or positive gain) for UE distribution#B subject to any of Outdoor and Indoor, if the training dataset is constructed with data samples subject to multiple UE distributions including UE distribution#B, as observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Minor loss (0%~-1%) are observed by 3 sources [NTT DOCOMO, Nokia, Huawei].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Note: Moderate degradations of up to -3.9% are still observed by 1 source [Nokia] for deployment scenario#B subject to Indoor.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.1-6 of R1-2306059

Observation

For the scalability verification of AI/ML based CSI compression over various Tx port numbers, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain Tx port number#B and applied for inference with a same Tx port number#B,

- For generalization Case 2, significant performance degradations are observed in general, if Tx port number#A is 32 & Tx port number#B is 16, as -3.37%~-21.8% degradations are observed by 4 sources [OPPO, Fujitsu, ZTE, vivo]

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gains) for Tx port number#B subject to any of 16 and 32, if the training dataset is constructed with data samples subject to multiple Tx port numbers including Tx port number#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, as observed by 7 sources [Huawei, OPPO, NTT DOCOMO, CATT, ZTE, Fujistu, Nokia].

  - Minor loss (0%~-1.75%) are observed by 6 sources [Huawei, OPPO, Fujistu, CATT, ZTE, NTT DOCOMO].

  - Moderate loss (-1.84%~-4%) are observed by 3 sources [Nokia, CATT, NTT DOCOMO].

  - Positive gains are observed by 3 sources [OPPO, ZTE, Fujistu].

  - Note: Significant degradations of up to -6.08% are still observed by 1 source [CATT] for deployment scenario#B subject to 32 ports, and for deployment scenario#B subject to 16 ports

  - Note: Pre/post-processing of truncation/padding is adopted by 6 sources [Huawei, OPPO, NTT DOCOMO, ZTE, Fujistu, Nokia], and adaptation layer in the AL/ML model is adopted by 1 source [CATT].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - Note: Results refer to Table 5.1-7 of R1-2306059

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 1.2%~4.2% gain;

    - 1 source [Apple] observes 7.6%~8.5% gain;

    - 1 source [vivo] observes 9.7%~17.2% gain.

    - 1 source [MediaTek] observes 22.6%~ 48.6% gain.

  - For full buffer traffic:

    - 1 source [Nokia] observes 2%~3% gain;

    - 1 source [vivo] observes 8.7% gain.

    - 1 source [MediaTek] observes 1.01% gain.

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.7%~3.1% gain;

    - 1 source [vivo] observes 3.4%~7.0% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 8.1% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-8 of R1-2306059

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 4.5%~9.3% gain;

    - 3 sources [Huawei, Apple, vivo] observes 11.3%~20.1% gain;

  - For full buffer traffic:

    - 2 sources [Nokia, vivo] observe 6%~17.5% gain;

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 0.5%~16% gain;

  - For full buffer traffic:

    - 1 source [vivo] observes 11% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-9 of R1-2306059

Observation

For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, 4 sources [Xiaomi, CATT, Interdigital, Spreadtrum] observe a generalized performance of less than -2% degradation.

  - If UE speed#B is either 30 km/h or 60 km/h or 120 km/h, or if UE speed#B is 10km/h and UE speed#A is either 60km/h or 120km/h, 8 sources [Xiaomi, Samsung, Interdigital, Fujitsu, ZTE, ETRI, vivo, Huawei] observe that moderate/significant performance degradations are suffered:

    - For UE speed#B is 10 km/h & UE speed#A is either 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-2.7% loss), 1 source [Samsung] observes significant degradation (-53%~-61% loss).

    - For UE speed#B is 30 km/h & UE speed#A is either 10 km/h, 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-3% loss), 8 sources [Xiaomi, Interdigital, Fujitsu, vivo, ZTE, Huawei, ETRI, Spreadtrum] observe significant degradation (-6%~-45.6% loss).

    - For UE speed#B is 60 km/h & UE speed#A is either 10 km/h, 30 km/h or 120 km/h, 1 source [ZTE] observes moderate degradation (-3% loss), 7 sources [Samsung, Xiaomi, Fujitsu, ETRI, ZTE, vivo, Spreadtrum] observe significant degradation (-7.8%~-52% loss).

    - For UE speed#B is 120 km/h & UE speed#A is either 30 km/h or 60 km/h, 1 source [ZTE] observes moderate degradation (-3.4% loss), 4 sources [ZTE, ETRI, vivo, Samsung] observe significant degradation (-7.55%~-32.3% loss).

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 9 sources [Xiaomi, Interdigital, Apple, Huawei, ZTE, Samsung, ETRI, vivo, Spreadtrum].

  - For UE speed#B is 10 km/h, minor loss (-0.6%~-1%) are observed by 3 sources [CATT, Xiaomi, Spreadtrum].

  - For UE speed#B is 30 km/h, minor loss (-0.08%~-1.34%) are observed by 3 sources [Xiaomi, Apple, Huawei], moderate loss (-2.2%~-4.07%) are observed by 3 sources [Interdigital, vivo, Spreadtrum].

  - For UE speed#B is 60 km/h, minor loss (-0.05%~-2%) are observed by 4 sources [ZTE, Apple, Xiaomi, Huawei], moderate loss (-2%~-3.76%) are observed by 2 sources [vivo, Spreadtrum].

  - For UE speed#B is 120 km/h, moderate loss (-2%~-4.45%) are observed by 4 sources [vivo, Samsung, ETRI, ZTE].

  - Note: For generalization Case 3, 5 sources [ETRI, ZTE, Samsung, Interdigital, Fujitsu] observe significant performance degradations (-5%~-26.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-10 of R1-2306059

Observation

For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, training non-aware quantization (Case 1) is in general inferior to the training aware quantization (Case 2-1/2-2), and may lead to lower performance than the benchmark.

- For scalar quantization, compared with benchmark,

  - -5.9%~-43.2% degradations are observed for training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8].

  - 3.9%~8.64% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 5 sources [Source#1, Source#2, Source#3, Source#4, Source#5], which are 17.3%~83.2% gains over training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8] and 0.9%~5.4% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

    - Note: 0.72% gains are observed for Case 2-1 from 1 source [Source#1] due to SQ parameter chosen without matching latent distribution, which achieves 13.9% gains over Case 1.

  - 7.55% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 1 source [Source#1], which are 21.6% gains over training non-aware quantization (Case 1) from 1 source [Source#1].

- For vector quantization, compared with benchmark,

  - -2%~-10% degradations are observed for training non-aware quantization (Case 1) from 1 source [Source#7].

  - 6.0%~8.91% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 2 sources [Source#1, Source#2], which are 16.3%~23.1% gains over training non-aware quantization (Case 1) from 2 sources [Source#1, Source#2].

  - 4.67%~13.01% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 6 sources [Source#1, Source#2, Source#4, Source#5, Source#7, Source#8], which are 10.7%~27.8% gains over training non-aware quantization (Case 1) from 3 sources [Source#1, Source#2, Source#7] and 1.7%~7.5% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

  - In general, Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains, as observed by 6 sources [Source#1, Source#2, Source#4, Source#5, Source#6, Source#8].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: Qualcomm (R1-2305328); Source#2: vivo (R1-2304471); Source#3: Ericsson (R1-2304521); Source#4: ZTE (R1-2304534); Source#5: Xiaomi (R1-2304893); Source#6: Fujitsu (R1-2304764); Source#7: Huawei, HiSilicon (R1-2304653); Source#8: Apple (R1-2305234).

  - Note: Results refer to Table 5.4-1 of R1-2306060

Observation

For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, in general vector quantization (VQ) has comparable performance with scalar quantization (SQ):

- For SQ and VQ under the same training case, it is

  - observed by 1 source [Source#1] that VQ under Case 2-1 has -0.8% degradation over SQ under Case 2-1,

  - observed by 2 sources [Source#2, Source#3] that VQ under Case 2-1 has 0.3%~1.1% gain over SQ under Case 2-1, and

  - observed by 3 sources [Source#2, Source#3, Source#4] that VQ under Case 2-2 has 0.7%~5.1% gain over SQ under Case 2-2.

  - Note: VQ under Case 2-1 has 8% gains over SQ under Case 2-1 as observed from 1 source [Source#2] due to non-optimized SQ parameter chosen.

- For SQ and VQ across training cases, it is

  - observed by 5 sources [Source#1, Source#2, Source#3, Source#5, Source#6] that VQ under Case 2-2 has 0.5%~4% gain over SQ under Case 2-1, and

  - observed by 1 source [Source#5] that VQ under Case 2-2 has -1.3% degradation over SQ under Case 2-1.

- Note: in general, more companies (Source#1, Source#2, Source#3, Source#4, Source#5, Source#6) observing gain of VQ over SQ than companies observing loss (Source#1, Source#5).

- Note: it is observed by 1 source [Source#5] that combined SQ and VQ under Case 2-2 has minor gain of 0.2% over VQ only under Case 2-2.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: vivo (R1-2304471); Source#2: Qualcomm (R1-2305328); Source#3: Apple (R1-2305234); Source#4: Lenovo (R1-2305202); Source#5: ZTE (R1-2304534); Source#6: Xiaomi (R1-2304893);.

  - Note: Results refer to Table 5.4-2 of R1-2306060

Agreement

For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_1 KPIth_1 in Option 1, the candidate threshold values are set as 0.02, 0.05 and 0.1

Agreement

For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_2 and KPIth_3  in Option 2, consider KPIth_2   = KPIth_3.

Agreement

For the evaluation of training Type 3 under CSI compression, for the benchmark case (1-on-1 joint training) for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared.

E.g., if the Type 3 is Transformer#1 for NW part model and CNN#1 for UE part model, then the benchmark case for performance comparison is also Transformer#1 for NW part model and CNN#1 for UE part model with joint training.

Agreement

For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIdiffKPIDiff achieved in the RAN1#112bis-e meeting, as baseline for calibration purpose, consider Option 1 (Gap between KPIActualKPIActual and KPIGenieKPIGenie).

- Option 2 (Binary state of KPIActual  and KPIGenie relationship) as optional and up to companies to report.

  - Results subject to Option 2, may be captured as a note in observation

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11%

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 6%~11% at CSI overhead A (small overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11% at CSI overhead C (large overhead);

- For Max rank 2, 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~15%

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~15% at CSI overhead A (small overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~10% at CSI overhead B (medium overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of -0.2%~14% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 24.47%~28.24%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 7.44%~9.95% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.5-1 of R1-2306061

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer,

- For Max rank 1, 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~20.9%

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.5%~20.9% at CSI overhead A (small overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.3%~17.4% at CSI overhead B (medium overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~6.62% at CSI overhead C (large overhead);

- For Max rank 2, 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~14.9%

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 4.1%~14.9% at CSI overhead A (small overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 0.3%~4% at CSI overhead B (medium overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~6.03% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 8.76%~30.17%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 3.59%~6.15% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.5-2 of R1-2306061

Agreement

For the evaluation of the R16 eType II-like codebook based high resolution quantization of the ground-truth CSI in the CSI compression for AI/ML training, regarding the evaluation of new values of eType II parameters, consider the legacy values of PC6&PC8 as the baseline/lower-bound of performance comparison.

- Note: it has been agreed that Float32 is adopted as the baseline/upper-bound of performance comparison.

Observation

For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B,

- For generalization Case 2, generalized performance may be achieved in general

  - If carrier frequency#A is 3.5/4GHz & carrier frequency#B is 2GHz, 3 sources [NTT DOCOMO, Nokia, MediaTek] observe generalized performance of less than -1.72% degradation.

  - If carrier frequency#A is 2GHz & carrier frequency#B is 3.5/4GHz, 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek] observe generalized performance of less than -1% degradation or positive gain.

    - Note: 1 source [Nokia] observes significant degradations of -6.6%.

- For generalization Case 3, generalized performance of the AI/ML model may be achieved (0%~-0.8% loss or positive gain) for carrier frequency#B subject to any of 2GHz and 3.5/4GHz, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B, as observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Minor loss (0%~-0.8%) are observed by 3 sources [Nokia, vivo, MediaTek].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Note: Significant degradations of up to -4.9% are still observed by 1 source [Nokia] for carrier frequency#B subject to 3.5/4GHz

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Antenna layouts are assumed as the same over the different frequency carriers.

  - Note: Results refer to Table 5.5-3 of R1-2306061

Working Assumption

For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, update the entry of CQI determination method(s) to include also the RI determination:

| Common description | Input type |
| --- | --- |
|  | Output type |
|  | Quantization /dequantization method |
|  | Rank/layer adaptation settings for rank>1 |
|  | CQI/RI determination method(s) for AI/ML (Option 1a/1b/1c/2a/2b, etc.) |

Observation

For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 5 sources [Ericsson, Nokia, ZTE，Fujitsu, Samsung] observe -0%~-0.5% degradation, 6 sources [Nokia, Qualcomm, ZTE, CATT, vivo, Samsung] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Huawei, Apple, CMCC] observe -0%~-0.8% degradation.

- Note: For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [Qualcomm, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.6-1 of R1-2306062

Observation

For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [Huawei, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table 5.6-2 of R1-2306062

Observation

For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [Huawei] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.6-3 of R1-2306062

# FL Summary and potential issues for the next meeting

Following progress has been achieved in the 113 meeting:

- Remaining EVM issues

  - Issue#2-3: For the FFS issue on the value of threshold of KPIth_1KPIth_1 in Option 1, 3 baseline threshold values are considered: 0.02, 0.05, and 0.1.

  - Issue#2-3a: For the FFS issue on the value of threshold of KPIth_2 and KPIth_3 in Option 2, they are considered as equal.

  - Issue#2-3c: As the down-selection of two options for monitoring, Option 1 (Gap between KPIActualKPIActual and KPIGenie) is considered as baseline; Option 2 is optionally reported.

  - Issue#2-5: The “CQI determination method(s)” entry in the template is updated as “CQI/RI determination method(s)”, to reflect the RI selection method considered by companies also.

  - Issue#3-24: For training Type 3, the new case benchmark is considered to use the same pair of models as the pair of models for separate training for fair comparison.

  - Issue#3-10a: For justifying the new values of eType II parameters for R16 eType II-like codebook based high resolution quantization of the ground-truth CSI for AI/ML training, PC6 and PC8 are introduced. Companies are invited to simulate the training dataset using PC6, PC8, and new parameters, compare the gap between PC6/PC8 and the upperbound Float32, and compare the performance gain of using enhanced values of eType II parameters over PC6/PC8.

- Observations

  - Table 1 - AI/ML over benchmark: performance observed in terms of SGCS (Issue#3-1), Mean UPT/5% UPT for FTP traffic/full buffer (Issue#3-3/ Issue#3-4/ Issue#3-5/ Issue#3-6).

  - Table 1 - quantization/dequantization methods (Issue#3-8/Issue#3-9): Training aware outperforms training non-aware in general; Case 2-2 outperoforms Case 2-1 under VQ in general.

  - Table 2 – generalization (Issue#3-12/Issue#3-13/Issue#3-14): Generalization Case 2 may achieve generalized performance for some testing cases but not for others; Generalization Case 3 may in principle achieve generalized performance.

  - Table 3 – scalability (Issue#3-17): Generalization Case 2 may suffer significant degradation; Generalization Case 3 may in principle achieve generalized performance.

  - Table 5 – Case 1 (Issue#3-20/20a/22): minor loss for 1 NW to 1 UE separate training under the same backbone. Observed performances for different backbones are diverse.

  - Table 6 - AI/ML over benchmark: performance observed in terms of SGCS over benchmark#1 from the impact factor of UE speed (Issue#4-1), as well as in terms of Mean UPT/5% UPT for FTP traffic/full buffer over benchmark#1 and benchmark#2 (Issue#4-5/ Issue#4-6).

  - Table 7 – Generalization over UE speeds (Issue#4-7): Generalization Case 2 may achieve generalized performance for some (limited) testing cases but not for (most) others; Generalization Case 3 may in principle achieve generalized performance.

Following aspects may be at least discussed in the next meeting:

- Remaining EVM issues

  - Note: As the target is to finish the evaluation agenda at 114 meeting, no EVM issues will be discussed at the 114 meeting unless necessary. In that regar, for the issues which we do not have consensus (Issue#2-1, Issue#2-4, Issue#2-6), they will not be raised in the next meeting.

- Template for monitoring of CSI compression

  - As we did not discuss the template during the 113 meeting (due to limited time), the draft template will be provided by Moderator as a post meeting offline email. Companies can provide your comments and polish the template via email. It is the current feeling of Moderator that the template will be added as a new sheet of Table 1.

- Observations on CSI compression

  - Principle on how to handle the aspects where observations have been drawn in 112b-e/113 meetings: We will have a quick go through in the 114 meeting with updated results from companies. The structure of the observations would not be changed (if no revert to the trends that have been observed); only update the number of sources/company names. These updated observations will be made as a package in the FL summary and to be endorsed in together (if no comments); the updated observations made at 114 will be captured to TR, rather than the old observations achieved at 112b-e/113.

  - Table 1. 1-on-1 joint training

    - Basic AI/ML performance gain for CSI compression (including eventual KPI). The basic performance gain in terms of SGCS and eventual KPI (Mean UPT, 5% UPT) are already captured in old observations. The new observation to be treated includes CSI overhead reduction (Issue#3-7).

    - Comparison of AI/ML solutions (in terms of intermediate KPI)

      - Comparison of input/output types (Issue#3-2): Discussed at this meeting but with no consensus. Will come back at the next meeting. For companies who are still in favor of keeping raw channel matrix on the table, please provide simulation results to show the performance (compared with precoding matrix) and applicable cases of raw channel matrix.

      - Comparison of trainig awareness solutions and quantization/dequantization methods. More details on the quantization details to be evaluated and submitted, e.g., number of bits for SQ, uniform vs non-uniform of SQ, segmentation methods for VQ, etc.

      - Comparison of Rank>1 options (Issue#3-11/11a). It is Moderator’s understanding that Option 2-1, 3-1, and 3-2 are natural survivals after being comparing by companies with other rank>1 options. For companies who are still in favor of keeping Option 1-1/1-2/2-2 on the table, please provide simulation results to show their performances (compared with Option 2-1/3-1/3-2) and their applicable cases.

      - Comparison of high resolution ground-truth CSI format (Issue#3-10). As guided by the agreement, companies are welcome to bring the comparison between ground-truth CSI subject to PC6, PC8, and new values of eType II like parameters.

      - Comparison of CQI determination method for AI/ML. This issue is not a MUST, but would provide meaningful information for the down-selection of options at 9.2.2.2.

    - Monitoring

      - Monitoring accuracy based on the agreed threshold values for Option 1 are to be collected as baseline. Companies can report the simulated cases (Case 1/2-1/2-2), and the threshold values for Option 2 if optionally selected. Assumptions of ground-truth CSI parameters for Case 1 and proxy model for Case 2-1/2-2 are also reported and to be compared.

      - Geneneralization (Issue#2-2). Though we do not have conclusion for this meeting, companies are welcome to submit your results to separate columns of the template.

    - Complexity (Issue#3-26)

      - Companies are welcome to complete the complexity entries and provide how to compare the performance and the price of complexity for CSI compression/prediction.

  - Table 2. Generalization (Issue#3-15). Try to complete the observation at the next meeting, with similar structure with other generalization aspects.

  - Table 3. Scalability (Issue#3-16). Try to complete the observation at the next meeting, with similar structure with other scalability aspects.

  - Table 4. Multi-vendor joint training (Issue#3-18/19)

    - To be observed for 1 NW to M>1 UEs (Case 2) and N>1 NWs to 1 UE (Case 3). Please companies report whether your solution is simultaneous training or sequential training (as agreed in 9.2.2.2)!

  - Table 5. Separate training

    - Performance impact of different backbones (please companies take the same pair of models for 1-on-1 benchmark as agreed for comparison!).

    - Performance impact of N NW to M UEs for both NW/UE first training (Issue#3-21/23). It is noticed by Moderator that the current performances are diverse over companies (some have minor loss while other have huge loss), and for those who have huge loss, it may due to the fact that no special handling is performed (e.g., adaptation layer). So, it is encouraged that companies report whether special handling is performed during the dataset mixing.

- Observations on CSI prediciton

  - Table 6. CSI prediction w/o generalization

    - Basic AI/ML performance gain for CSI prediction (including eventual KPI).

      - Limited inputs for the comparison with benchmark#2 (Issue#4-4). More results are encouraged from companies to draw more concrete observations.

      - Limited inputs for the comparison with benchmark#1/2 in terms of UPT. More results are encouraged from companies to draw more concrete observations.

      - In addition, proponents are encouraged to compare the results (SGCS/UPT) w/ the modeling of spatial consistency and w/o modeling spatial consistency. We need to understand whether introducing spatial consistency will bring a significant difference on performance.

    - Comparison of AI/ML solutions (in terms of intermediate KPI)

      - Comparison of different observation window settings (Issue#4-2)

      - Comparison of different prediction window settings (Issue#4-3)

  - Table 7. CSI prediction w/ generalization: no remaining critical issues identified.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6-17, 2021.

- R1-2304371	Discussion and evaluation of AI/ML for CSI feedback enhancement FUTUREWEI

- R1-2304471	Evaluation on AI/ML for CSI feedback enhancement  vivo

- R1-2304521	Evaluations of AI-CSI	Ericsson

- R1-2304534	Evaluation on AI for CSI feedback enhancement	ZTE

- R1-2304550  Discussion on evaluation on AIML for CSI feedback enhancement  Spreadtrum Communications, BUPT

- R1-2304653	Evaluation on AI/ML for CSI feedback enhancement  Huawei, HiSilicon

- R1-2304681  Evaluation of ML for CSI feedback enhancement  Nokia, Nokia Shanghai Bell

- R1-2304722	Evaluation on AI/ML for CSI feedback enhancement  CATT

- R1-2304764  Evaluation on AI/ML for CSI feedback enhancement  Fujitsu

- R1-2304779	Evaluation on AI/ML for CSI feedback enhancement  InterDigital, Inc.

- R1-2304813  Evaluation for CSI feedback enhancements  Intel Corporation

- R1-2304842  On Evaluation of AI/ML based CSI  Google

- R1-2304854	Evaluation on AI/ML for CSI feedback enhancement  China Telecom

- R1-2304893	Discussion on evaluation on AI/ML for CSI feedback enhancement  Xiaomi

- R1-2304948	Discussion on AI/ML for CSI feedback enhancement	AT&T

- R1-2305015  Some discussions on evaluation on AI-ML for CSI feedback	 CAICT

- R1-2305085  Discussion on evaluation on AI/ML for CSI feedback enhancement  CMCC

- R1-2305160  Evaluation of AI and ML for CSI feedback enhancement  NVIDIA

- R1-2305202  Evaluation on AI/ML for CSI feedback  Lenovo

- R1-2305234  Evaluation for AI/ML based CSI feedback enhancement  Apple

- R1-2305296  Evaluation on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2305328  Evaluation on AI/ML for CSI feedback enhancement  Qualcomm Incorporated

- R1-2305459	Evaluation methodology and results on AI/ML for CSI feedback enhancement  OPPO

- R1-2305505	Evaluation on AI ML for CSI feedback enhancement	Samsung

- R1-2305591  Discussion on evaluation on AI/ML for CSI feedback enhancement  NTT DOCOMO, INC.

- R1-2305655  Evaluation on AI/ML for CSI feedback enhancement  MediaTek Inc.

- R1-2305688 	Evaluation on AI for CSI feedback enhancement	Mavenir

- R1-2305730  Evaluation and preliminary results on AI/ML-based CSI feedback enhancement  BJTU

- R1-2305789  Evaluation on AI/ML for CSI feedback enhancement  ETRI

- R1-2305894  Evaluation of AI/ML for CSI feedback Enhancement  CEWiT

- Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, Table 1. 1-on-1 joint training_v025_Fujitsu2_Intel2.xlsx

- Table 2. Evaluation results for CSI compression with model generalization, Table 2. CSI compression with model generalization_v019_Xiaomi2_MediaTek.xlsx

- Table 3. Evaluation results for CSI compression with model scalability, Table 3. CSI compression with model scalability_v013_ZTE2_Mavenir.xlsx

- Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, Table 4. Multi-vendor joint training_v005_Ericsson_Ericsson2.xlsx

Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, 
Table 5. Separate training_v009_SS_MediaTek.xlsx

- Table 6. Evaluation results for CSI prediction without model generalization/scalability, Table 6. CSI prediction without generalization_v015_CATT2_OPPO.xlsx

- Table 7. Evaluation results for CSI prediction with model generalization, Table 7. CSI prediction with generalization_v015_Xiaomi2_ZTE2.xlsx

# Appendix I: Agreement list

### Agreements of the 109-e meeting

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement (modified by May 23rd post)

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

Note: Above agreement is updated as follows

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

  - Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

### Agreements of the 110 meeting

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

  - Note: Companies to report the ratio for dataset mixing

  - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted.

Agreement

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point:

- Various deployment scenarios (e.g., UMa, UMi, InH)

- Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

- Companies to report the selected scenarios for generalization verification

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

  - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

  - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

### Agreements of the 110bis-e meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | At least, FTP model 1 with packet size 0.5 Mbytes is assumed Other options are not precluded. |
| --- | --- |

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy,

- Use the target CSI from ideal channel and use output CSI from the realistic channel estimation

  - The target CSI from ideal channel equally applies to AI/ML based CSI feedback enhancement, and the baseline codebook

Note: there is no restriction on model training

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook (if it outperforms Type II Codebook) can be optionally considered for comparing AI/ML schemes up to companies

- Note: Type II Codebook is baseline as agreed

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, and if the AI/ML model outputs multiple predicted instances, the intermediate KPI is calculated for each prediction instance

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations:

- Raw channel matrixes

- Eigenvector(s)

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the evaluation of CSI prediction:

- Companies are encouraged to report the assumptions on the observation window, including number/time distance of historic CSI/channel measurements as the input of the AI/ML model, and

- Companies to report the assumptions on the prediction window, including number/time distance of predicted CSI/channel as the output of the AI/ML model

Conclusion

If ideal DL channel estimation is considered (which is optional) for the evaluations of CSI feedback enhancement, there is no consensus on how to use the ideal channel estimation for dataset construction, or performance evaluation/inference.

- It is up to companies to report whether/how ideal channel is used in the dataset construction as well as performance evaluation/inference.

Conclusion

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example:

- For each FP/BP loop,

  - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side

  - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side

  - Step 3: UE side trains the CSI generation part based on the BP information from NW side

- Note: the dataset between UE side and NW side is aligned.

- Other Type 2 training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training):

- Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly

- Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part

- Step3: UE side trains the UE side CSI generation part based on the received set of information

- Other Type 3 NW-first training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training):

- Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly

- Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part

- Step3: NW side trains the NW side CSI reconstruction part based on the received set of information

- Other Type 3 UE-first training approaches are not precluded and reported by companies

Working assumption

In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors

- Note: Eventual KPI can still be used to compare the performance

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2.

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Agreement

In CSI compression using two-sided model use case, evaluate and study quantization of CSI feedback, including at least the following aspects:

- Quantization non-aware training

- Quantization-aware training

- Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc.

- How to use the quantization methods

Agreement

For evaluating the performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, e.g., including at least the following options

- High resolution scalar quantization, e.g., Float32, Float16, etc.

    - FFS select one of the scalar quantization resolutions as baseline

- High resolution codebook quantization, e.g., R16 Type II-like method with new parameters

    - FFS new parameters

- Other quantization methods are not precluded

Agreement

For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement which is optionally considered by companies, the following case is taken

- The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations:

- Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side

- Case 2: Not aligned AI/ML model structures between NW side and UE side

    - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers)

- FFS different sizes of datasets between NW side and UE side

- FFS aligned/different quantization/dequantization methods between NW side and UE side

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

Agreement

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors,

- Case 1 (baseline): Type 2 training between one NW part model to one UE part model

- Case 2: Type 2 training between one NW part model and M>1 separate UE part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at UE part models, e.g., whether the same or different dataset(s) are used among M UE part models

- Case 3: Type 2 training between one UE part model and N>1 separate NW part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at NW part models, e.g., whether the same or different dataset(s) are used among N NW part models

- FFS N NW part models to M UE part models

- FFS different quantization/dequantization methods between NW and UE

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

- FFS: companies to report the training order of UE-NW pair(s) in case of M UE part models and/or N NW part models

- FFS: whether/how to report overhead

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations

- Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit

    - Companies to report the raw channel is in frequency domain or delay domain

- Precoding matrix

    - Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation)

- Other input/output types are not precluded

- Companies to report the combination of input (for CSI generation part) and output (for CSI reconstruction part),

    - Note: the input and output may be of different types

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for SLS, spatial consistency procedure A with 50m decorrelation distance from 38.901 is used (if not used, company should state this in their simulation assumptions)

- UE velocity vector is assumed as fixed over time in Procedure A modeling

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for the calculation of intermediate KPI, the following is considered as the granularity of the frequency unit for averaging operation

- For 15kHz SCS: For 10MHz bandwidth: 4 RBs; for 20MHz bandwidth: 8 RBs

- For 30kHz SCS: For 10MHz bandwidth: 2 RBs; for 20MHz bandwidth: 4 RBs

- Note: Other frequency unit granularity is not precluded and reported by companies

### Agreements of the 111 meeting

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS the value or range of payload size X/Y/Z

- FFS the description and results for different training types/cases may need a separate table

- FFS: training related overhead

Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#1, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#2, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#2, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Gain for Mean UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for 5% UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors

- Case 1 (baseline): Type 3 training between one NW part model and one UE part model

  - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model

  - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model

  - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different

  - FFS: different quantization methods between NW side and UE side

- Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models

  - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models

  - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model

  - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models

- Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models

  - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models

  - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models

  - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models

- FFS: whether/how to report overhead of dataset

Working Assumption

For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example

  - It does not imply any restriction on future specification for CSI prediction

- FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1.

- Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn.

- Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1.

- Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn.

- Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization.

- Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison.

Agreement

For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies:

- Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training

  - Fixed/pre-configured quantization method/parameters is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

- Case 2: Quantization aware training, where quantization/dequantization is involved in the training process

  - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

  - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase

    - Companies to report how to update the quantization method/parameters during the training

- Note: the above cases apply for training Type 1/2/3

- Others are not precluded.

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2

- For NW-first training

  - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization.

- For UE-first training

  - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

Working Assumption

For the AI/ML based CSI prediction sub use case, the following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction for the case without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS whether/how to capture the muliptle predicted CSI instances and their mapping to slots

Table X. Evaluation results for CSI prediction without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | UE speed |  |  |
|  | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Benchmark 1 |  |  |  |
| Intermediate KPI #1 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 1 |  |  |  |
| Intermediate KPI #2 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 1 |  |  |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| Benchmark 2 |  |  |  |
| Intermediate KPI #1 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 2 |  |  |  |
| Intermediate KPI #2 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 2 |  |  |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| FFS others |  |  |  |

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated

- Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions.

- Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions

- Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions

- Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, if R16 Type II-like method is considered, companies to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc.

### Agreements of the 112 meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method.

Agreement

Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

Conclusion

For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory.

- It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR)

Agreement

For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison.

Agreement

- Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

Agreement

For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

  - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate template is introduced to capture the evaluation results for “separate training”

- FFS: additional KPIs for each template, e.g., overhead, latency, etc.

Agreement

For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison.

- FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

  - FFS: input/output type

- Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is

    - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

    - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

- Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is

    - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

    - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

- Other options not precluded.

Agreement

The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

- For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank.

- For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB

  - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank

  - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank

Working Assumption

For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as:

- X is <=80bits

- Y is 100bits-140bits

- Z is  >=230bits

Working Assumption

X, Y and Z are applicable for per layer

Working assumption

The following initial template is considered to replace the template achieved in the working assumption in the RAN1#111 meeting, for companies to report the evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS the description and results for generalization/scalability may need a separate table

- Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting

- FFS: training related overhead

- FFS: how to capture CSI overhead reduction to the template

  - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction.

Table X. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AI/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AI/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
|  | Rank/layer adaptation settings for rank>1 |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) |  |  |  |
|  | Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  |  |  |  |
| SGCS of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| SGCS of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| NMSE of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| NMSE of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| Other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for Mean UPT (for a specific CSI feedback overhead) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for 5% UPT | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| [CSI feedback reduction (%)] |  |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Note: “Benchmark” means the type of Legacy CB used for comparison.

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working assumption

A separate table to capture the evaluation results of generalization/scalability verification for AI/ML-based CSI compression is given in the following initial template

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
| SGCS, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| SGCS, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other settings for Case 1) |  |  |  |
| … |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Case 2) |  |  |  |
| … (other settings for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
| … (results for Case 3) |  |  |  |
| … (other settings for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Fine-tuning) |  |  |  |
| … (other settings for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI prediction with model generalization, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model description (e.g., backbone, structure) |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 1) |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for multi-vendor joint training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | [Training method] |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Train/k |  |  |
|  | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Case 1 (baseline): NW#1-UE#1 | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
| ... (other NW-UE combinations for Case 1) |  |  |  |
| Case 2 (1 NW part to M>1 UE parts) | NW part model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
| Case 3 (N>1 NW parts to 1 UE part) | UE part model backbone/structure |  |  |
|  | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| Case 1: NW#1-UE#1: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other NW-UE combinations for Case 1) |  |  |  |
| Case 2: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for sequentially separate training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of separate training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Shared output of CSI generation part/input of reconstruction part is before or after quantization |  |  |
| Dataset description | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| [Benchmark: NW#1-UE#1 joint training] | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
|  | Training dataset size |  |  |
| ... (other NW-UE combinations for benchmark) |  |  |  |
| Case 1-NW first training | NW part AI/ML model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 1-UE first training | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
|  | UE part model backbone/structure |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 2-UE first training | UE#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE part AI/ML model backbone/structure |  |  |
|  | NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge) |  |  |
| Case 3-NW first training | NW#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | UE part model backbone/structure |  |  |
|  | UE part training dataset description and size (e.g., description/size of dataset from N NWs and how to merge) |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| NW#1-UE#1 joint training: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other 1-on-1 NW-UE joint training combinations) |  |  |  |
| Case 1-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 1-UE first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 2-NW first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

### Agreements of the 112bis-e meeting

Agreement

For the rank >1 options under AI/ML-based CSI compression, for a given configured Max rank=K, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

Agreement

For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

Working assumption

For the forms of the intermediate KPI results for the following templates:

Table 2. Evaluation results for CSI compression with model generalization Table 3. Evaluation results for CSI compression with model scalability, Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, Table 7. Evaluation results for CSI prediction with model generalization

- The intermediate KPI results are in forms of absolute values and the gain over benchmark, e.g., in terms of “absolute value (gain over benchmark)”

- The intermediate KPI results are in forms of linear value for SGCS and dB value for NMSE

Working Assumption

For the per layer CSI payload size X/Y/Z in the templates of CSI compression, as a clarification, the X/Y/Z ranges in the working assumption achieved in RAN1#112 meeting is applicable to Max rank = 1/2. For Max rank (v) = 3/4, the per layer basis X/Y/Z ranges are re-determined as:

- X is <=160/vbits

- Y is 200/vbits-280/vbits

- Z is >=460/vbits

Working Assumption

For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback reduction is provided for 3 CSI feedback overhead ranges, where for each CSI feedback overhead range of the benchmark, it is calculated as the gap between the CSI feedback overhead of benchmark and the CSI feedback overhead of AI/ML corresponding to the same mean UPT.

- Note: the CSI feedback overhead reduction and gain for mean/5%tile UPT are determined at the same payload size for benchmark scheme

| CSI feedback reduction (%) (for a given CSI feedback overhead in the benchmark scheme) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |

Note: for result collection for the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#112bis-e meeting,

- 15 sources show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

  - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, deployment scenario#A is UMi, deployment scenario#B is UMa, or deployment scenario#A is InH, deployment scenario#B is UMa/UMi.

  - 6 sources observe that if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor.

    - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, or deployment scenario#A is UMi, deployment scenario#B is UMa.

- 6 sources show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#A and deployment scenario#B, and the trained AI/ML model applies inference on either deployment scenario#A or deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa and/or UMi.

- 3 sources show that, compared to the case where the AI/ML model is trained on scenario#A and applied for inference on deployment scenario#B, the generalization performance can be improved, if the AI/ML model, after trained on deployment scenario#A, is updated based on a fine-tuned dataset subject to deployment scenario#B, and performs inference on deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa or UMi.

Agreement

For the AI/ML based CSI prediction, add an entry for “Table 6. Evaluation results for CSI prediction without model generalization/scalability” to report the Codebook type for CSI report.

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance [between prediction instances/distance from the last observation instance to the 1st prediction instance]) |
|  | Whether/how to adopt spatial consistency |
|  | Codebook type for CSI report |

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, the model monitoring methodology is considered as:

- Step1: Generate test dataset including K test samples

  - FFS how to obtain the K test samples

- Step2: For each of K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff= f(KPIActual, KPIGenie), where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI.

- Step3: Calculate the statistical result of the KPIDiff over K test samples which represents the monitoring accuracy performance.

- Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network.

- Note: the complexity, overhead and latency of the monitoring scheme are reported by companies. FFS how to evaluate latency.

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, for Step2 of the model monitoring methodology, the per sample KPIDiff is considered for

- Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where

  - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements.

  - KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32.

  - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample.

- Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE:

  - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI.

    - Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100%

  - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual)

  - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI.

  - FFS how to train the proxy model and the resulting monitoring performance, to be reported by companies.

  - FFS whether/how to evaluate the generalization performance of the proxy model.

- Case 3: others are not precluded

Conclusion

For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, the complexity metric of FLOPs may be reported separately for the AI/ML model and the pre/post processing.

- How to calculate the FLOPs for pre/post processing is up to companies.

- While reporting the FLOPs of pre-processing and post-processing the following boundaries are considered.

  - Estimated raw channel matrix per each frequency unit as an input for pre-processing of the CSI generation part

  - Precoding vectors per each frequency unit as an output of post-processing of the CSI reconstruction part

Agreement

For the evaluation of CSI compression, companies are allowed to report (by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g.,

- Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

  - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW

  - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW

- Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder

- Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation

- Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment

- Option 1c: CQI is calculated based on traditional codebook

- Other options if adopted, to be described by companies

Agreement

For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

- E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2 or Case 3.

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of

- Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff= KPIActual-KPIGenie;

  - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth_1, where KPIth_1 is a threshold of the intermediate KPI gap.

- Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth_2,KPIGenie<KPIth_3 OR KPIActual<KPIth_2,KPIGenie>KPIth_3, where KPIth_2 can be same or different from KPIth_3.

  - Monitoring accuracy is the percentage of the samples for which KPIDiff=0.

- FFS other metrics: Misdetection, False alarm, etc.

- FFS the values of KPIth_1, KPIth_2, KPIth_3.

- FFS whether/how to evaluate the monitoring metrics for Rank>1

Working Assumption

For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback overhead for the metric of eventual KPI (e.g., mean/5% UPT) is re-determined as:

- CSI feedback overhead A: <=β* 80 bits.

- CSI feedback overhead B: β* (100bits – 140 bits).

- CSI feedback overhead C: >=β* 230 bits.

- Note: β=1 for max rank = 1, andβ=1.5 for max rank = 2/3/4.

- FFS for rank 2/3/4, whether to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= [1.9], and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits.

- Note: companies additionally report the exact CSI feedback overhead they considered

Observation

For the scalability verification of AI/ML based CSI compression over various CSI payload sizes, till the RAN1#112bis-e meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain CSI payload size#B and applied for inference with a same CSI payload size#B,

- Generalized performance of the AI/ML model can be achieved (0%~5.9% loss) under generalization Case 3 for the inference on either CSI payload size#A or CSI payload size#B, if the training dataset is constructed with data samples subject to multiple CSI payload sizes including CSI payload size#A and CSI payload size#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, shown by 7 sources (Note *) (6 sources (Note **) showing 0%~2.2% loss, 3 sources (Note ***) showing 2.35%~5.9% loss). The scalability solution is adopted as follows:

  - Pre/post-processing of truncation/padding, adopted by 3 sources (Note ****), showing 0.2%~5.9% loss.

  - Various quantization granularities, adopted by 1 source (Note *****), showing 1.8%~4.7% loss.

  - Adaptation layer in the AL/ML model, adopted by 3 sources (Note ******), showing 0%~4.05% loss.

- Note: the above results are based on the following assumptions

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - Input/output scalability dimension Case 3 is adopted: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

  - The performance metric is SGCS in linear value for layer 1/2.

- Note *: Huawei, HiSilicon (R1-2302358), Ericsson (R1-2302918), OPPO (R1-2302540), Fujitsu (R1-2302904), CMCC (R1-2303224), MediaTek (R1-2303336), NTT DOCOMO (R1-2303705).

- Note **: Huawei, HiSilicon (R1-2302358), Ericsson (R1-2302918), Fujitsu (R1-2302904), CMCC (R1-2303224), MediaTek (R1-2303336), NTT DOCOMO (R1-2303705).

- Note ***: Ericsson (R1-2302918), OPPO (R1-2302540), MediaTek (R1-2303336).

- Note ****: OPPO (R1-2302540), Fujitsu (R1-2302904), CMCC (R1-2303224).

- Note *****: Ericsson (R1-2302918).

- Note ******: Huawei, HiSilicon (R1-2302358), MediaTek (R1-2303336), NTT DOCOMO (R1-2303705).

Observation

For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting,

- 11 sources (Note *) show that the AI/ML-based CSI prediction outperforms the benchmark of the nearest historical CSI, wherein

  - 5 sources (Note **) show the gain of 14% ~ 26.47% using raw channel matrix as input.

  - 2 sources (Note ***) show the gain of 5.64% ~ 9.49% using precoding matrix as input, which is in general worse than using raw channel matrix as input

- Note 1: spatial consistency is adopted in 1 source (Note ****) and not adopted in 5 sources (Note *****).

- Note 2: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - UE speed is 30km/h.

  - The performance metric is SGCS in linear value for layer 1.

- Note *: Huawei, HiSilicon (R1-2302358), ZTE (R1-2302437), Spreadtrum Communications, BUPT, (R1-2302593), Nokia, Nokia Shanghai Bell (R1-2302628), CATT (R1-2302695), Fujitsu (R1-2302904), Samsung (R1-2303120), ETRI (R1-2303194), CMCC (R1-2303224), NVIDIA (R1-2303435), Apple (R1-2303475).

- Note **: ZTE (R1-2302437), Nokia, Nokia Shanghai Bell (R1-2302628), Spreadtrum Communications, BUPT (R1-2302593), NVIDIA (R1-2303435), Apple (R1-2303475).

- Note ***: ZTE (R1-2302437), Fujitsu (R1-2302904).

- Note ****: Nokia, Nokia Shanghai Bell (R1-2302628).

- Note *****: Huawei, HiSilicon (R1-2302358), ZTE (R1-2302437), ETRI (R1-2303194), CMCC (R1-2303224), Apple (R1-2303475).

Agreement

For the AI/ML based CSI compression, for the submission of simulation results to the RAN1#113 meeting, for Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, companies are encouraged to take the following assumptions as baseline for the calibration purpose:

- Benchmark: R16 eType II CB;

  - Others can be additionally submitted, e.g., Type I CB.

- Input/Output type: Eigenvectors of the current CSI

  - Other can be additionally submitted, e.g., eigenvectors with additional past CSI, eType II-like input, raw channel matrix, etc.

- Ground-truth CSI quantization method: Float32, i.e., without quantization

  - Other high resolution CSI quantization methods can be additionally submitted for comparison, e.g., R16 Type II-like method with new parameters, scalar quantization, etc.

- Rank/layer adaptation settings for rank>1: Option 3-1, i.e., layer common and rank common

  - Other rank>1 options can be additionally submitted for comparison, e.g., Option 1-1/1-2/2-1/2-2/3-2.

- Quantization method: quantization-aware training (Case 2-1 or Case 2-2)

  - Quantization non-aware training can be additionally submitted for comparison

  - SQ and/or VQ is up to companies; companies are encouraged to provide results of various cases for comparison.

- Performance metric for intermediate KPI: SGCS

  - NMSE can be additionally submitted.

Agreement

For the AI/ML based CSI prediction, for the submission of simulation results to the RAN1#113 meeting,

- for Table 6. Evaluation results for CSI prediction without model generalization/scalability, companies are encouraged to take the following assumptions as baseline for the calibration purpose:

  - UE speed: 10km/h, 30km/h, 60km/h;

    - Others can be additionally submitted, e.g., 120km/h.

  - Input/Output type: Raw channel matrix

    - Other can be additionally submitted, e.g., eigenvectors.

  - Observation window: 5/5ms, 10/5ms

    - Other observation window configurations can be additionally submitted for comparison, e.g., 3/5ms, 4/5ms, 8/2.5ms, 10/4ms, etc.

  - Prediction window: 1/5ms/5ms

    - Other prediction window configurations can be additionally submitted for comparison, e.g., 3/5ms/5ms, 5/5ms/5ms, 4/2.5ms/2.5ms, 5/4ms/4ms, etc.

  - Performance metric for intermediate KPI: SGCS

    - NMSE can be additionally submitted.

  - Spatial consistency configuration (optional): procedure A with 50m decorrelation distance and channel updating periodicity of 1 ms.

- for Table 7. Evaluation results for CSI prediction with model generalization, companies are encouraged to take the following assumption as baseline for the calibration purpose:

  - Performance metric for intermediate KPI: SGCS

    - NMSE can be additionally submitted.