3GPP TSG RAN WG1 Meeting #113	R1-2306065

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #2 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

25 contributions [2-26] have been submitted to Agenda Item 9.1.3.2 of RAN1#113 on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

## Comb offset hopping and cyclic shift hopping reinitialization periodicity

We had the following agreement:

Agreement

For SRS comb offset hopping / cyclic shift hopping, support reinitialization at the beginning of every N radio frame(s), where N ≥ 1.

- FFS: N is fixed or configurable.

Regarding the FFS points colored in red, the general positions are:

Option 1: N is fixed

  - Supporting: CATT, CMCC, Ericsson, FUTUREWEI, Google, Huawei, HiSilicon, Intel, Lenovo, NEC, NTT DOCOMO, Sharp, OPPO, vivo, ZTE (11 15 proponents)

  - Option 1A: N = 1: this is the same mechanism as used in legacy group/sequence hopping, and the random pattern may repeat itself every 10 ms.

    - Supporting: Ericsson, Intel, NTT DOCOMO, OPPO, Sharp (4 5 proponents)

  - Option 1B: N = 1024: this effectively leads to the time-domain hopping behavior dependent on the system frame number (SFN), and leads to the longest possible random pattern, i.e., repeat itself 10240 ms.

    - Supporting: Apple, CATT, CMCC, FUTUREWEI, Google, Huawei, HiSilicon, Lenovo, NEC, OPPO, vivo (7 11 proponents)

  - Option 1C: N = 32 or other small number

    - Supporting: ZTE

Option 2: N is configurable

  - Supporting: CMCC, Futurewei, InterDigital, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm, Samsung, Spreadtrum, vivo, xiaomi (9 12 proponents)

  - Option 2A: N is based on SRS periodicity P

    - Supporting: CMCC, InterDigital, Samsung, Spreadtrum, xiaomi (3 5 proponents)

  - Option 2B: N is chosen from the set {2n}, where n = 0,1,…,5 or 0,1,…,10

    - Supporting: Apple, Futurewei, InterDigital, Qualcomm, Samsung, Spreadtrum (2 6 proponents)

We will need to make a decision in this meeting. All these options can work and have their pros and cons. We have the following initial proposal.

Proposal 2.1: For SRS comb offset hopping / cyclic shift hopping reinitialization periodicity of N radio frame(s), down select one from the following options:

- Option 1A: N = 1

- Option 1B: N = 1024

- Option 2: N ≥ 1 is configurable

  - Option 2A: N ≥ 1 is configurable and based on SRS periodicity P, and FFS details

  - Option 2B: N ≥ 1 is configurable and chosen from the set {2n}, and FFS n = 0,1,…,5 or 0,1,…,10

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Support Option 2B. Also, for this option, we do not see the need for going beyond 32 frames. For Option 2A, we do not understand how it can work. Should network configure the same SRS periodicity for all UEs that need to hop consistently to maintain orthogonality? |
| Google | - Support Option 1B, which is a simple solution. |
| Samsung | We are fine with either one in Option 2. @QC, we would like to ask whether gNB shall configure/have same reinitialization periodicity for all UEs in a cell or not. |
| NEC | Support Option 1B. The value of N should be cell-specific or at least group common, where reusing 1024 (SFN) is a simple and efficient solution. |
| OPPO | We support a fixed value of N, and either Option 1A or 1B is fine. |
| ZTE | - We prefer a smaller fixed N (e.g., 32) - First, we didn’t see the necessity of a configurable N. Besides, N = 1 means following legacy, and no enhancement is introduced. Regarding the value of N, we agree with QC that there’s no need for going with such a large N, i.e., N = 1024. Probably a fixed N = 32 is more appropriate. |
| Huawei, HiSilicon | Support Option 1B. |
| Lenovo | Support the proposal and prefer Option 1B on account of simplicity. |
| New H3C | Support Option 1B |
| CMCC | We can be fine with either Option 1B or Option 2. We have agreed that N can be >=1 to achieve better randomization gain for SRS hopping, especially for SRS with long periodicity. So, the determination of reinitialization periodicity should consider the periodicity of SRS. Or the N can be configured with a relatively large value, like 1024, to accommodate the above case. |
| InterDigital | Support Option 2, either option is fine. |
| CATT | Support Option 1B. We’re also open to other fixed value as long as N >= 32. |
| vivo | Support either Option 1B or Option 2. |
| Xiaomi | We support Option2. For different SRS periodicities, the value N should be configured as different values to randomize SRS interference. For example, if SRS is configured with larger periodicity, the value N is increased as well so that better performance is obtained due to more randomization interference of SRS. Otherwise, the larger N is not necessary. |
| Nokia/NSB | We support option 2. The value N need to be configurable according to the SRS periodicity, number of resources, comb-type, number of TRPs etc. |
| Apple | Option 1B or Option 2B. |
| FL | @ZTE: We can add your preferred solution to the proposal if there is more support. |
| Ericsson | Support Option 1A to reinitialize comb-offset/cyclic-shift hopping every radio frame (which is the same as for legacy group/sequence hopping) |
| NTT DOCOMO | We are ok with option 2. |
| Spreadtrum | Support option 2, 2A and 2B are both fine. |

### Follow-up after Monday online

We had the following working assumption:

Working Assumption

For SRS comb offset hopping / cyclic shift hopping reinitialization periodicity of N radio frame(s):

- N = 128

We will decide if we confirm it or not.

| Company | View |
| --- | --- |
| Sharp | We are OK to confirm. |
| OPPO | - Fine with the value. |
| Lenovo | We are fine with either 128 or 1024. |
| ZTE | Support to confirm. |
| Spreadtrum | Support |
| FL2 | Please continue to discuss. |
| Huawei, HiSilicon | We conduct a simple simulation to prove the benefit of larger N at some cases. Parameter; Value. Scenario; CJT with N_TRP = 2. Carrier frequency and subcarrier spacing; 3.5 GHz with 30 kHz SCS. System bandwidth; 20MHz. Channel model; CDL-B with 300ns delay spread. UE speed; 3km/h. Antennas at UE; 4T4R. Antennas at gNB; 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ. SRS configurations; SRS periodicity = 1280ms. |

## Subset(s) for cyclic shift hopping and comb offset hopping

We had the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Regarding the FFS point colored in red, the general positions are:

Support configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

  - Supporting: CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Intel, Lenovo, New H3C, Nokia, Nokia Shanghai Bell, Samsung, xiaomi, ZTE (10 15 proponents)

  - Against: Apple, Sharp, LG, Qualcomm, Spreadtrum (5 opponents)

The benefits of configuring a subset of resources for hopping include potential collision avoidance between different SRS resources when hopping is enabled. For example, if a Rel-18 SRS resource configured with cyclic shifts hopping is CDMed with another SRS resource not supporting hopping, the Rel-18 SRS resource can be configured with a subset of cyclic shifts excluding the other SRS resource’s cyclic shifts, so that these two resources would never collide. The other SRS resource may be a legacy SRS or aperiodic SRS / SRS with usages CB/NCB/AS not supporting the new hopping (if concluded). Based on the views, we have the following proposal.

Proposal 2.2: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Do not support. This reduces the gain of hopping (less interference randomization). We think network should instead use other dimensions for legacy UEs. If comb offset hopping is used, legacy Ues can be multiplexed in CS domain. If CS hopping is used, legacy Ues can be multiplexed in comb offset domain. In addition, network can always use different symbols. Furthermore, this proposal makes the design / hopping formula very complicated especially for multi-port SRS. |
| Google | - Support. This is beneficial with regard to multiplexing with legacy UE. |
| Samsung | Support. This could guarantee the performance of legacy UE, and efficient resource allocation. |
| OPPO | We think this feature is beneficial only when finer cyclic shift granularity is introduced. |
| ZTE | - Support. Configuring a CS/ comb offset subset for CS/ com offset hopping can provide higher scheduling flexibility in supporting backward compatibility of legacy Ues. - We agree that legacy Ues can be multiplexed in CS/ comb offset/ time domains. However, this means legacy and Rel-18 SRS ports cannot overlap in CS/ comb offset/ time domains, which would limit the scheduling flexibility at gNB side. - To address the issue of degradation of interference randomization, we propose to combine CS hopping and comb offset hopping, then the hopping can be limited within a 2-D resource subset. Regarding Proposal 2.2, can we add an FFS: “configuring a 2-D (CS/ comb offset) resource subset for combined CS hopping and comb offset hopping (if supported)” ? |
| Huawei, HiSilicon | Support, given the realistic resource allocation challenge faced by the gNB when there exists legacy UE and controllable spec. impact. |
| Lenovo | Support on account of benefit for interference coordination and multiplexing SRS between legacy Ues and CJT UE. For coexistence, SRS of legacy Ues and SRS of Rel.18 Ues may hop in different subsets of comb offsets or CS and the siganlling for the indication of subset is needed. |
| New H3C | Support |
| CMCC | Support. It is beneficial to multiplexing with legacy Ues. |
| LGE | Do not support. This limits the number of configurable offsets, so it brings decreased degree of freedom for interference randomization. In addition, collision avoidance can be achieved not only through configuring a subset of comb offset/cyclic shift but also through FDM-based scheduling between legacy and Rel-18 UE. |
| CATT | Support the proposal. The SRS resource allocation algorithm can be simplified at gNB by configuring a subset for the hopping. |
| Vivo | We understand the motivation to introduce the subset, but there are several points we would like to be further clarified. - There are many dimensions to multiplex R18 SRS and legacy SRS, such as TDM/CDM/FDM, even partial sounding. What is the difficulty to multiplex R18 SRS and legacy SRS based on these legacy methods without the subset? - For inter-cell scenarios, when the target cell doesn’t know the SRS assignment in neighbor cells, it seems not feasible to configure an optimal subset to avoid interference from the legacy SRS in neighbor cells. - The CS and comb offset assignment of legacy SRS is not consecutive, which would increase the complexity of designing elements of the subset and the formula for CS/comb offset hopping. It is better to clarify the principle of how to design the subset firstly. - If the subset is introduced, the performance gain of CS/comb offset hopping would be degraded. How much gain can be provided is not clear at this stage. |
| Xiaomi | Support and agree the FL’s observation. |
| Nokia/NSB | Support. |
| Apple | This is not high priority. Since the hopping sequence ID can be configured by the NW already with full flexibility, we do not support this proposal |
| Ericsson | Support the FL proposal. In particular for CS hopping, restricting the hopping to a limited set of CSs is needed for co-scheduling legacy UEs on the same comb offset. For comb-offset hopping, we may want to clarify that the restriction is only for transmission comb K_TC > 2 (otherwise there is no comb-offset hopping for k_TC = 2). |
| Spreadtrum | Do not support. Similar views like QC and vivo. Multiplexing with legacy SRS is not big issue, which can be easily solved via TDM/FDM etc. By configuring subset of comb offsets/cyclic shifts will degrade the performance and make spec more complicated. |

### Follow-up after Monday online

We had the following updated proposal from Monday online:

Proposal 2.2:

Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

- The subset configuration applies to all the ports

There were some concerns:

Some companies were concerned about the subset configuration would reduce the randomization benefit, but as the chairman explained, this can be a network decision to make if the standards provide sufficient flexibility to the network.

Some companies were concerned about the further details have not been discussed and are unclear. We can discuss them but it seems to me that the details can be straightforward.

Some UE vendors were concerned about the impact on UE implementation, but this can be a UE-optional feature.

- We can consider the following based on Monday online version:

Proposal 2.2-1: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value on an OFDM symbol.

This is a UE-optional feature.

| Company | View |
| --- | --- |
| Sharp | We are OK with Proposal 2.2-1. |
| Lenovo | - Support. We think this is a good tradeoff proposal (as a UE optional feature) on account of the benefit of multiplexing SRS between legacy UEs and CJT UE and interference coordination. |
| Huawei, HiSilicon | Support. Regarding the first concern, we should try our best to harvest the randomization effect without affecting the legacy UEs’ channel estimation performance. Regarding the second concern, an exemplary simple implementation is given as below to address companies’ concern. Actually we’re open to any implementation achieving similar effect and other proponents are welcome to give candidates. Following is the suggested proposal accordingly: Proposal 2.2-1A: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. - Option 1 khopping(pi)=m=07c8nslotframe,μnf mod NNsymbslot+ns,fμNsymbslot+l0+l'+m∙2m mod L(pi) where L(pi) represents the length of the subset, which means the candidate offset values belong to {0, 1, …, L(pi)-1}. Regarding the third concern, we can compromise this as a UE optional feature, but we’d like to emphasize that UE supporting this feature, which greatly facilitates the SRS resource allocation of gNB, may have more chance to be enabled with CS/comb offset hopping. |
| Ericsson | Support CS/comb-offset hopping being restricted to a subset of CSs/comb offsets to support co-scheduling legacy UEs and/or other UEs configured with hopping on the same time/frequency resources. However, we would like to clarify that the subset does not need to be explicitly configured (e.g., a list of CSs and/or comb offsets). Our preference is that it is implicitly RRC configured by legacy parameters cyclicShift and combOffset, an example of how this can be done is provided below for CS hopping and transmission comb 2. The figure shows SRS ports P0—P3 over four-time instances. Here, the subset is {ncs, ncs+2, ncs+4, ncs+6}mod ncs,max, where ncs is the RRC configured CS. Note that SRS ports P0—P3 can belong to either a single 4-port SRS resource, two different 2-port SRS resources, or four different 1-port SRS resources. For example, for a 4-port SRS resource, the hopping pattern in the figure could be achieved by configuring said SRS resource with CS 1 and comb offset 0. Proposal 2.2-2: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value on an OFDM symbol. This is a UE-optional feature. The subset is configured by legacy parameters cyclicShift and combOffset for cyclic shift hopping and comb offset hopping, respectively. |
| ZTE | Support. From technical perspective, we do not think it is necessary to make it an UE-optional feature, but we can comprise for progress. We agree to discuss the detailed designs. |
| Samsung | Support the proposal. We have same understanding that all the port(s) shall have same hopping offset. We can live with a UE-optional feature. In order to make simple, we think max 2 subsets are enough. Then, a UE performs hopping within a certain subset. Based on this, our proposal on top of the proposal 2.2-1 is as follows: (updated) Proposal 2.2-1: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value on an OFDM symbol. This is a UE-optional feature. The number of subsets is fixed as 2, and a subset is defined as half of all possible hop locations. - For comb-offset hopping with comb-X (X=2, 4, or 8), 1st subset can be comb-offset 0 ~ X/2-1, and 2nd subset can be comb-offset X/2 ~ X-1. - E.g., for comb-8, comb-offset 0 ~ 3 can be one subset, comb-offset 4 ~7 can be one another subset. - If an initial comb-offset is 0, then comb-offset hopping is performed within 1st subset. - 1st subset; - 2nd subset. - 0; - 1; - 2; - 3; - 4; - 5; - 6; - 7. - For cyclic shift hopping with maxCS = Y (Y=8, 12, 6) for SRS resource with 1 port, 1st subset can be cyclic shift 0 ~ Y/2-1, and 2nd subset can be cyclic shift Y/2 ~ Y-1. - E.g., for maxCS = 12 with 1-port with initial comb-offset as 0, CS 0 ~ 5 can be one subset, CS 6 ~ 11 can be one another subset. - If an initial comb-offset is 0, then comb-offset hopping is performed within 1st subset. - 1st subset; - 2nd subset. - 0; - 1; - 2; - 3; - 4; - 5; - 6; - 7; - 8; - 9; - 10; - 11. - For cyclic shift hopping with maxCS = Y (Y=8,12,6) for SRS resource with >1 ports, 1st subset can be cyclic shift CS 0 ~ Y/4-1 & CS Y/2 ~ 3Y/4-1, and 2nd subset can be cyclic shift CS Y/4 ~ CS Y/2-1 & CS 3Y/4 ~ Y-1. - E.g., for maxCS = 12 with 2-port with initial comb-offset as 0 and 6, CS 0 ~ 2 & CS 6 ~ 8 can be one subset, and CS 3 ~ 5 & CS 9 ~ 11 can be one another subset. - If an initial comb-offset is 0 and 6 for 2 ports, then comb-offset hopping is performed within 1st subset. - 1st subset; - 2nd subset; - 1st subset; - 2nd subset. - 0; - 1; - 2; - 3; - 4; - 5; - 6; - 7; - 8; - 9; - 10; - 11. |
| FL2 | Based on the inputs, the detailed options are provided in the following updated proposal. Companies can add more options for down selection. Proposal 2.2-3: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured. The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol. [This is a UE-optional feature.] Down select from the following options: - Option 1: khopping(pi)=m=07c8nslotframe,μnf mod NNsymbslot+ns,fμNsymbslot+l0+l'+m∙2m mod L(pi) where L(pi) represents the length of the subset, which means the candidate offset values belong to {0, 1, …, L(pi)-1}. - Option 2: The subset is configured by legacy parameters cyclicShift and combOffset for cyclic shift hopping and comb offset hopping, respectively. - Option 3: The number of subsets is fixed as 2, and a subset is defined as half of all possible hop locations. - For comb-offset hopping with comb-X (X=2, 4, or 8), 1st subset can be comb-offset 0 ~ X/2-1, and 2nd subset can be comb-offset X/2 ~ X-1. - For cyclic shift hopping with maxCS = Y (Y=8, 12, 6) for SRS resource with 1 port, 1st subset can be cyclic shift 0 ~ Y/2-1, and 2nd subset can be cyclic shift Y/2 ~ Y-1. - For cyclic shift hopping with maxCS = Y (Y=8,12,6) for SRS resource with >1 ports, 1st subset can be cyclic shift CS 0 ~ Y/4-1 & CS Y/2 ~ 3Y/4-1, and 2nd subset can be cyclic shift CS Y/4 ~ CS Y/2-1 & CS 3Y/4 ~ Y-1. |
| Xiaomi | Support the updated proposal. The details of subset configuration can be FFS. |

## Applicability to SRS other than P/SP SRS for antennaSwitching

We had the following agreement:

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Regarding the FFS point colored in red, the general positions are:

Applicable to aperiodic SRS

  - Supporting: Apple, CATT, Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, Lenovo, LG, NEC, Nokia, Nokia Shanghai Bell, Spreadtrum, Qualcomm, xiaomi, vivo, ZTE (6 17 proponents)

  - Against: Google, New H3C, OPPO, Samsung (4 opponents)

Applicable to SRS with usage codebook, nonCodebook, or beamManagement

  - Supporting: Futurewei, Qualcomm

  - Against: CATT, ETRI, Samsung, xiaomi (4 proponents)

In my personal view, supporting hopping for these other types of SRS can improve randomization and enhance multiplexing with new SRS with hopping. How much necessity/benefit this can provide can be discussed. Based on the views, we have the following proposal.

Proposal 2.3: SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Support. There is no reason to preclude these hopping schemes for AP-SRS. Even legacy group/sequence hopping today is applicable to AP-SRS. In our understanding, w/o this, AP-SRS becomes less useful when network configures hopping for P/SP-SRS as ensuring orthogonality between SRS resources with different time-domain behavior (P/SP/AP) requires configuring many AP-SRS resource sets and trigger one of them based on other P/SP-SRS resources (which may be difficult given 2-bits SRS request field). Furthermore, there is no additional spec effort. Hence, why should we impose an artificial restriction that these schemes cannot be supported for AP-SRS? |
| Google | - Do not support. The slot offset for AP-SRS can be dynamically indicated by the gNB. There is no benefit for this proposal. |
| Samsung | Not support. Same view with Google. The reason why P/SP SRS is agreed was to randomize persistent interference. |
| NEC | Fine with the majority view. |
| OPPO | We also think the benefit is unclear for AP-SRS. |
| ZTE | - Support. CS/ comb offset hopping should be applied to P/SP/AP SRS to maintain orthogonality among SRS ports. |
| Huawei, HiSilicon | Agree with QC. Seems no harm to support. |
| Lenovo | Support the proposal 2.3 since it can achieve unified scheme for perioid/semi-persistent/aperiodic SRS and this can simplify gNB’s configuration for SRS. |
| New H3C | The benefit of AP-SRS isn’t clear to us although there is no additional spec effort. |
| CMCC | Support. Now that P/SP SRS is supported, it seems no additional spec impact. |
| LGE | Same view as QC. |
| InterDigital | Support FL’s proposal. |
| CATT | Support the proposal. |
| Vivo | Support the proposal. For inter-cell interference scenarios, it seems difficult to dynamically indicate AP-SRS without cross-SRS interference when the target cell doesn’t know the information of CS/comb offset/symbol of SRS in neighbor cells. In this case, comb offset hopping / cyclic shift hopping can help aperiodic SRS randomize the interference from SRS in neighbor cells, especially based on the hopping pattern per symbol. |
| Xiaomi | Support and share similar view with QC and Lenovo. |
| Nokia/NSB | Support. |
| Apple | We are fine with the proposal. |
| Ericsson | Support the proposal. |
| Spreadtrum | Support |

### Tuesday offline

Applicable to aperiodic SRS

  - Supporting: Apple, CATT, Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, Lenovo, LG, NEC, Nokia, Nokia Shanghai Bell, Spreadtrum, Qualcomm, xiaomi, vivo, ZTE (6 17 proponents)

  - Against: Google, New H3C, OPPO, Samsung (3 opponents)

Proposal 2.3: SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

| Company | View |
| --- | --- |
| Sharp | Support. We think no restriction is needed. |
| OPPO | - Though we think it is not essential, we can be fine with the hopping for AP SRS. |
| Lenovo | Support |
| Huawei, HiSilicon | Support. |
| Ericsson | Support. There is no reason to restrict. There clearly are benefits with hopping for AP SRS: E.g., inter-cell interference can be randomized by hopping over a multi-symbol SRS resource within a slot. |
| Spreadtrum | Support |
| FL2 | We did not get the time to discuss this in Tuesday offline. Please continue to discuss. |

## Time-domain hopping behavior for comb offset hopping with repetition factor R > 1

- We had the following agreement:

Agreement

For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l’ of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details.

- UE can indicate whether it supports one or both the options. Details to be discussed in UE feature.

The general positions regarding the configuration FFS are:

Option 1: When KTC<R, the time-domain hopping behavior depends on the OFDM symbol index l'  of each symbol. Otherwise, the time-domain hopping behavior depends on the OFDM symbol index  l' of the first symbol across the R repetitions.

  - Supporting: xiaomi

Option 2: Introduce a one-bit RRC parameter to configure the time-domain hopping behavior.

  - Supporting: Futurewei

Since there is no enough inputs, we can see more companies’ views on this issue, but it is preferable if this can be decided soon, especially if it has RAN2 RRC impact.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Our understanding is that Option 2 is already agreed. |
| Google | - OK with option 2. |
| Samsung | We can live with Option 2, but it could be up to RAN2 the detail design on RRC parameter (whether it is one-bit or not). |
| NEC | Fine with option 2. |
| OPPO | It is up to RAN2 design. No additional agreement is needed. |
| ZTE | - Support Option2. We share the same view with QC that Option 2 is already agreed. Besides, there is no technical justification of Option 1. |
| Huawei, HiSilicon | Fine with option 2. |
| Lenovo | Support option 2 as a straight forward RRC configuration for the time-domain hopping behavior depending on scheme 1 (i.e. the OFDM symbol index l’ of each symbol) or scheme 2 (i.e. the first symbol across the R repetitions). |
| New H3C | Support option 2 |
| CMCC | Support Option 2, but it can be up to RAN2 design. |
| LGE | Same understanding as QC. |
| InterDigital | Leave the design up to RAN2. |
| CATT | Ok with option 2, but this should be RAN2 work. |
| Vivo | We also think Option 2 is agreed in the last meeting. In our understanding, we don’t think time-domain hopping behavior should be determined based on the value of KTC and R. |
| Xiaomi | When KTC<R, partial SRS repetition gain is still obtained even though comb offset is hopped based on each OFDM symbol index. Therefore, comb offset hopping based on each OFDM symbol index can be supported when KTC<R. Otherwise, there is no SRS repetition gain for comb offset hopping when KTC>R. This is our motivation for Option1. For Option 2, if UE indicates that only one option, e.g., comb offset hopping depending on the first symbol across the R repetitions, is supported, it is not necessary to introduce one-bit for configuring the time domain behavior. |
| Nokia/NSB | One RRC parameter (not 1-bit) to indicate. This is UE capability to indicate what option it is used when R>1. For example comboffsetHoppingPerRepeition ENUMERATED{enabled} |
| Apple | Option 2 |
| FL | I think companies made a good point in leaving the detailed design to RAN2. So we can still discuss Xiaomi’s suggestion for Option 1 to see if it is agreeable, otherwise no further discussion in RAN1 is needed except for discussing in the RRC parameter list. |
| Ericsson | Support Option 2 and agree that detailed design should be up to RAN2. |
| NTT DOCOMO | Agree with Option 2. |

## Whether to allow combining cyclic shift hopping and comb offset hopping

We had the following agreement:

Agreement

For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

Regarding the FFS point colored in red, it has been discussed extensively in the last meeting, but no agreement was made. The general positions are:

Option 1: Only one of cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time

  - Supporting: CMCC, ETRI, Futurewei, LG, NTT DOCOMO, OPPO, Qualcomm, Samsung,  vivo (9 proponents)

Option 2: One or both of cyclic shift hopping and comb offset hopping can be configured for a SRS resource at a time (which may also be subject to UE capabilities/feature)

  - Supporting:  Apple, CATT, Ericsson, Google, Huawei, HiSilicon, Intel, Lenovo, NEC, New H3C, Nokia, Nokia Shanghai Bell, Sharp, Spreadtrum, xiaomi, ZTE (12 14 proponents)

  - In addition, one joint/combined cyclic shift + comb offset hopping may be supported.

    - Supporting: Ericsson, ZTE

    - Against: CATT

Based on the views, we have the following proposal.

Proposal 2.5: Whether SRS comb offset hopping and cyclic shift hopping can be combined on a SRS resource depends on UE feature/capability design.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | Do not support Proposal 2.5. Support Option 1. |
| Google | - OK with the proposal |
| Samsung | Not support proposal 2.5. |
| NEC | Support |
| OPPO | Don’t support the proposal. |
| ZTE | - Support Proposal 2.5, and support Option 2. Due to limited CS/comb offset resources (especially when , and ) , separate CS hopping and comb offset hopping may NOT provide sufficient interference randomization. If separate CS hopping and comb offset hopping are limited to a subset of CS/comb offset resources to support the backward compatibility of legacy UEs, this issue would become more serious. To address this issue, combined CS hopping and comb offset hopping is needed. |
| Huawei, HiSilicon | Support. |
| Lenovo | Support the proposal 2.5. We think potential gain and flexibility can be achieved by option 2. |
| New H3C | Support |
| CMCC | Not support proposal 2.5. |
| LGE | Do not support Proposal 2.5. It is difficult to see significant performance improvement. |
| CATT | Support the proposal. |
| Vivo | Don’t support. Our simulation result has shown that the additional gain of the combination of CS hopping and comb offset hopping is limited. |
| Xiaomi | We are fine with the proposal. |
| Nokia/NSB | We support option 2. The proposal is still unclear. |
| Apple | Fine with the proposal |
| Ericsson | Support the proposal. |
| Spreadtrum | Support |
| Sharp | Support |
| FL2 | Please continue to discuss. |

## Cyclic shift hopping with finer cyclic shift granularity

Regarding cyclic shift hopping with finer cyclic shift granularity, the general positions are:

Support cyclic shift hopping with finer cyclic shift granularity

  - Supporting: Futurewei, Lenovo, Huawei, HiSilicon, New H3C, vivo (4 6proponents)

  - Against: Google, LG, Nokia, Nokia Shanghai Bell, Qualcomm, Sharp (6 opponents)

The benefits of introducing finer cyclic shift granularity include further randomization for cyclic shift hopping, which is also demonstrated using simulations. There were concerns regarding the increase of the maximum number of cyclic shifts used for each comb, but it seems this enhancement does not increase the maximum number of cyclic shifts. Based on the views, we have the following proposal.

Proposal 2.6: For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

•	Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

•	Note: This does not increase the maximum number of cyclic shifts.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| QC | As we mentioned in the previous meeting, Proposal 2.6 is not consistent with the conclusion below. The proposal effectively increases the max # of possible cyclic shifts that can be used for SRS. Conclusion - No further discussion of increasing the maximum number of cyclic shifts for CJT SRS. - No further discussion of partial frequency sounding extensions for CJT SRS Even if we ignore the conclusion above, this does not seem to be an essential feature for interference randomization and is second-order optimization in our view. |
| Google | - Agree with QC. |
| ZTE | - We can discuss. |
| Huawei, HiSilicon | Support. I really don’t see the reason why some companies still stubbornly repeat the unreasonable comment that the proposal contradicts previous conclusion after both us and even FL have explained for multiple rounds and the restrictive notes have already been added. For companies’ information, let me explain again: Technically, I believe we have a common understanding that the reason why increasing the maximum number of cyclic shifts is precluded is that, it will shrink the zero correlation zone length and lead to SRS channel estimation performance degradation especially under large delay-spread scenario. However, the intention of finer CS hopping granularity is only applying finer-granularity CS hopping offset (which is shared by a group of orthogonal SRS ports, i.e., the delay-domain distance among SRS ports is unchanged and the channel estimation performance is totally unaffected) during CS hopping, which can be ensured by above notes. Literally, the nSRScs,max in the current spec. will remain unchanged. The only minor spec impact is a coefficient may need to be considered during the calculation of CS hopping offset (which also guarantee the “static” resource allocation by RRC is totally unaffected). Now that the CS hopping is already agreed, why not give this scheme more chance to harvest significant benefit under different scenarios (multiple companies have shown the performance benefit for several meetings) by a simple extension with minimal spec. impact, rather than revisit in the future release? Companies’ consideration is highly appreciated. |
| Lenovo | We are fine with the proposal on account of potential gain. |
| New H3C | Support |
| CMCC | We are OK to discuss this further enhancement. |
| LGE | Agree with QC. |
| Vivo | Support. As CS hopping has already been supported, finer time-delay-domain granularity is a solution that can provide significant gain with almost no additional spec effort. No additional signaling is needed, the only effort is to slightly modify the formula of CS mapping with factor K. |
| Xiaomi | We are fine to discuss this proposal. |
| Nokia/NSB | Agree with QC. |
| FL | It seems that a key technical discussion point for this proposal is, whether or not, the direction of this proposal is conflicting with the previous conclusion. We can see more companies’ inputs. We may spend a little bit online or offline time to discuss this and see if a consensus on this discussion point can be reached. If deemed help, the group may consider a conclusion such as “the finer CS hopping granularity does not conflict with the conclusion of no increase of max CS number” or the opposite to facilitate progress. |
| NTT DOCOMO | Ok to discuss further enhancement. |

### Tuesday offline

Support cyclic shift hopping with finer cyclic shift granularity

  - Supporting: Futurewei, Google (based on offline discussion), Lenovo, LGU+, Huawei, HiSilicon, New H3C, Spreadtrum, vivo (4 9 proponents)

  - Open/fine to discuss: CMCC, NTT DOCOMO, Xiaomi, ZTE

  - Against: Google, LG, Nokia, Nokia Shanghai Bell, Qualcomm, Sharp (6 5 opponents)

Proposal 2.6: For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

•	Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

•	Note: This does not increase the maximum number of cyclic shifts.

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Given our explanation above showing the proposal does not contradicts with previous conclusion, the minimal spec. impact and the performance benefit observed by multiple companies’ simulation, we believe it’s the good timing to support this. |
| Spreadtrum | - Support. Performance enhancement is expected by using finer granularity. Meanwhile, the spec efforts seems to be limited. |
| FL2 | Updated supporting list based on all inputs. Please continue to discuss. |
| China Unicom | Given the shown benefit, we support the proposal |
| Xiaomi | - Considering performance gain and limited specification impact, we are fine with the proposal. |

## Cyclic shift hopping combined with group / sequence hopping

Regarding cyclic shift hopping combined with group / sequence hopping, the general positions are:

Support cyclic shift hopping combined with group / sequence hopping, subject to UE capabilities/feature

  - Supporting: Apple, CATT, CMCC, Ericsson, Google, Lenovo, InterDigital, NEC, New H3C, Nokia, Nokia Shanghai Bell, Samsung, Sharp, Spreadtrum, OPPO, Qualcomm, ZTE (6 17 proponents)

  - Against: ETRI, Futurewei, LG, vivo, xiaomi (4 opponents)

We can try the following proposal.

Proposal 2.7: Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

Views can be provided for the above enhancement.

| Company | View |
| --- | --- |
| QC | Support. We think the benefit is even more compared to comb offset hopping combined with group/sequence hopping (which is already agreed based on UE capability). This is because the gain of cyclic shift hopping is quite sensitive to the pair of two SRS sequences (see our earlier evaluations in R1-2301399). |
| Google | - Support |
| Samsung | We can live with this proposal. |
| NEC | Support |
| OPPO | We are fine with the proposal. |
| ZTE | - Support. Combing CS hopping with group/ sequence hopping can enhance interference randomization. - Nevertheless, when CS hopping and group/ sequence hopping are enabled simultaneously, two SRS ports may collide due to the usage of a same CS and a same group/ sequence number. This issue can be avoided by jointly design the hopping patterns of CS hopping and group/ sequence hopping. Can we add an FFS “jointly design the hopping patterns of CS hopping and group/ sequence hopping” ? |
| Lenovo | - We are fine with the proposal since they are independent features and may have potential gain if both of them are supported. |
| New H3C | - Support |
| CMCC | - Support. |
| LGE | - Not support. We are not sure that the benefit is clear enough. Configuring either of cyclic shift hopping or group / sequence hopping seems sufficient since they are both code-domain interference randomization schemes. |
| InterDigital | - Support FL’s proposal. |
| CATT | - Fine with the proposal. |
| Vivo | Don’t support. - We have given the simulation result to show that the additional gain of the combination of CS hopping and sequence/group hopping is limited. Besides, the optimal pair of two SRS sequences can be pre-configured to maximize the gain of CS hopping. Based on this, the additional gain of the combination of CS hopping and sequence/group hopping would be further limited. |
| Xiaomi | - Not support. We have same view with LGE. |
| Nokia/NSB | - Support the proposal. Group/sequence hopping is necessary to average inter-group interference between cells. CS/comb-offset hopping cannot complement the imbalance of cross-correlation property. |
| Apple | - Fine with the proposal. |
| FL | - @ZTE: I think your point of a joint CS + group/sequence hopping design is an interesting point. However it is not quite related to the current proposal which is limited on UE capability / feature discussion. For now, I put it in Sec. 2.8 “Other proposals”. If there is more support, we can add a subsection to discuss it. |
| Ericsson | - We are fine with the proposal. |
| Sharp | - Support. |
| FL2 | Please continue to discuss. |

## Other proposals comb offset hopping / cyclic shift hopping:

The following proposals are also for comb offset hopping / cyclic shift hopping:

- Enh. 1: Combined with enhanced CS configurations, e.g., per-port CS assignment, non-equidistant CS assignment

  - Supported by: Samsung, Futurewei

- Enh. 2: jointly design the hopping patterns of CS hopping and group/ sequence hopping

  - Supported by: ZTE

Views can be provided for the above enhancements. Any other views, issues, and clarifications can also be provided.

| Company | View |
| --- | --- |
| QC | Don’t we also need to discuss hopping formula? Without hopping formula, it may not be clear to the editors how to implement the agreements in the spec. |
| ZTE | - We agree with QC that the hopping formula should be discussed once we reach an agreement on the reinitialization periodicity. |
| Huawei, HiSilicon | - Open to discuss. |
| CMCC | - Agree with QC. The hopping formula could be discussed in this meeting. |
| Xiaomi | - The hopping equation should be discussed firstly. |
| Nokia/NSB | - Agree with QC. |
| FL | - The hopping equations may depend on the reinitialization periodicity design. For now, a placeholder subsection is added below and we will provide more details after the reinitialization periodicity is decided. |

## Comb offset hopping / cyclic shift hopping equations

5 companies provided inputs to comb offset hopping / cyclic shift hopping equations. These are:

CATT 1: For an SRS resource with comb offset hopping configured, k0(pi) is determined by

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+kcomb, offsetl' mod KTC

where

kcomb, offsetl'=m=07c8*ns,fμNsymbslot+l0+l'+m*2mmod KTC.

CATT 2: For an SRS resource with cyclic shift hopping configured, the cyclic shift αi for port pi can be determined by

- αi=2πnSRScs,i+nSRScs,offsetmod nSRScs,maxnSRScs,max

where

- nSRScs,offset=m=07c8*ns,fμNsymbslot+l0+l'+m*2mmod nSRScs,max.

- Futurewei: For SRS comb offset hopping and/or cyclic shift hopping M hopping values, the hopping pattern formula is given by

- koffsets,f,l'=m=072mc8ns,fμNsymbslot+8l0+8l'+m mod M for comb offset hopping

- ncs offset,is,f,l'=m=072mc8ns,fμNsymbslot+8l0+8l'+m mod M for cyclic shift hopping

Qualcomm: For the hopping formula in cyclic shift hopping or comb offset hopping, use: f=(m=07c8∙t+m∙2m) mod Y, where t=mod(SFN,N)Nslotframe,μNsymbslot+ns,fμNsymbslot+l', and

- For cyclic shift hopping, Y=nSRScs,max, and l' is the OFDM symbol index of SRS within the slot.

- For comb offset hopping, Y=KTC, and l' is the OFDM symbol index of SRS within the slot or the OFDM symbol index of the first symbol across the R repetitions.

Xiaomi: For the hopping equations of comb offset hopping and cyclic shift hopping, the following two ways can be considered:

- Option1: fchns,fμ,l'=m=07c8Nsymbslotns,fμ+l0+l'+m∙2m mod K, where K=KTC or nSRScs,max

- Option2: fchns,fμ,l'=m=0 K-1c KNsymbslotns,fμ+l0+l'+m∙2mmod K or fchns,fμ,Y=m=0 K-1c KNsymbslotns,fμ+l0+Y+m∙2mmod K, where Y=t/T, t and T respectively denote the slot of SRS transmission and the periodicity of SRS.

ZTE: By referring to the legacy group hopping scheme, the hopping functions of separate CS hopping and comb offset hopping should be defined as

nSRSCS,i, hopns,fμ, l'=nSRSCS, i+m=0M-1cM∙ns,fμNsymbolslot+l0+l'+m∙2mmod nSRSCS, max

kTC(pi),hopns,fμ, l'=kTC(pi)+m=0M-1cM∙ns,fμNsymbolslot+l0+l'+m∙2mmod KTC

- M is a fixed value, CS hopping and comb offset hopping can use one common M or two individual M.

Most of the equations are very similar. I suggest the following initial proposal:

Proposal 2.9: For the hopping formula in cyclic shift hopping or comb offset hopping, use: f=(m=07c8∙t+m∙2m) mod Y, where t=mod(SFN,N)Nslotframe,μNsymbslot+ns,fμNsymbslot+l', where Y is the number of possible values for hopping, and

- For cyclic shift hopping, l' is the OFDM symbol index of SRS within the slot.

- For comb offset hopping, l' is the OFDM symbol index of SRS within the slot or the OFDM symbol index of the first symbol across the R repetitions.

| Company | View |
| --- | --- |

# SRS enhancements targeting 8 Tx operation

It is well known that increasing UE Tx antenna ports can significantly improve various performance metrics for UL/DL transmissions. 8 Tx transmissions can be feasible for at least CPE/FWA/vehicle/industrial devices and hence can be beneficial.

Some remaining issues on the number of SRS resources for 8 Tx SRS and the number of SRS resource sets for 8 Tx SRS will be discussed in agenda item 9.1.4.2 covering “SRI/TPMI enhancement for enabling 8 TX UL transmission; To support up to 4 or more layers per UE in UL targeting CPE/FWA/vehicle/industrial devices”.

## Non-TDMed 8 Tx SRS cyclic shift locations across the comb offsets for comb 4 and comb 8

We had the following agreement:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 or comb 8, decide one of the following options:

- Option 1: the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

  - For comb KTC=4, k=2. For comb KTC=8, k=4. For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max.

- Option 2: the cyclic shift positions are unaligned across the comb offsets on the same OFDM symbol for comb 4, and the cyclic shift positions are aligned on only 2 of the 4 comb offsets on the same OFDM symbol for comb 8.

  - For comb KTC=4, k=2. For comb KTC=8, k=4.  Example: For port pi, nSRScs,i=nSRScs+(pi-1000mod k)+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max. FFS equation details.

- FFS: potential impact on PAPR, if any.

This has been discussed for some time and companies are familiar with the pros and cons of the options, including the PAPR aspect. The general positions are:

Option 1: The cyclic shifts are completely aligned across the multiple comb offsets

  - Supporting: Apple, CATT, CMCC, Futurewei, Google, Huawei, HiSilicon, Lenovo, LG, New H3C, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo, xiaomi (14 17 proponents)

Option 2: The cyclic shift positions have minimum alignment across the multiple comb offsets

  - Supporting: Ericsson, Futurewei, NEC, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, ZTE (5 8 proponents)

Based on the views, I suggest the following proposal:

Proposal 3.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 on 2 comb offsets (KTC=4, kTC=2) or comb 8 on 4 comb offsets (KTC=8, kTC=4), the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

- For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kTCNapSRS/kTC mod nSRScs,max.

Views can be provided for the above proposal.

| Company | View |
| --- | --- |
| Google | Support |
| Samsung | - We are okay with the proposal. |
| NEC | Same CS mapping on different REs will lead high PAPR, so we prefer option 2, which can reduce PAPR. |
| OPPO | Support the proposal. |
| ZTE | Do NOT support. We prefer to use unaligned CSs across different comb offsets (Option 1). Especially, for transmission comb 4 with 12 available CSs, it could be a waste if only 4 of 12 CSs are used (Option 2). |
| Huawei, HiSilicon | Support. |
| Lenovo | Support. |
| New H3C | Support |
| LGE | Support |
| QC | We support the proposal. |
| CMCC | Support |
| CATT | Support. |
| vivo | Support |
| Xiaomi | Support |
| Nokia/NSB | Do not support. We see the gain from the unaligned CS. PAPR improved with option2, and also share view with ZTE. |
| Apple | Support |
| NTT DOCOMO | Ok |

This discussion is closed with the following agreement:

Proposal 3.1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 on 2 comb offsets (KTC=4, kTC=2) or comb 8 on 4 comb offsets (KTC=8, kTC=4), the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

- For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kTCNapSRS/kTC mod nSRScs,max.

## 8 Tx SRS with TDM

### Resource allocation for the s subsets of ports over m OFDM symbols

We had the following agreement:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), the SRS transmissions within each of the m/s groups of {1, 2, …, s} use the same set of subcarriers. If consecutive groups of {1, 2, …, s} are configured as repetition, then the SRS transmissions of the consecutive groups use the same set of subcarriers.

- Note: applicable to the SRS resource with or without FH/RPFS.

- FFS the scenario where comb offset hopping is configured for the SRS resource.

A decision to make, which will impact the specifications, is to decide how to interpret repetition when TDM is configured. There are two interpretations:

- Interpretation 1: R is a multiple of s, and the group of {1,2,…,s} is repeated R/s times in the slot. For this interpretation, the s OFDM symbols for TDM are viewed as a type of repetition since they are assigned with exactly the same frequency-domain resources, though with different ports.

  - Supporting: Apple, NTT DOCOMO (2 proponents)

- Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot. For this one, repetition is interpreted in a stricter sense, i.e., TDMed ports on different OFDM symbols are not viewed as repetition.

  - Supporting: CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Lenovo, LG, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo, xiaomi, ZTE (6 20 proponents)

Based on the best of our knowledge, either interpretation works and can have their respective advantages in terms of how much of the existing specifications/equations needs to be modified. Given companies’ views, I suggest we go with Interpretation 2.

Several companies discussed the resource allocation for the s subsets of ports over the m OFDM symbols in a slot. For example, m should be divisible by sR (based on Interpretation 2) or R (based on Interpretation 1). This seems already been agreed in above agreement “the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}).” If companies think there is a need to clarify, it can be further clarified after the interpretation is decided.

Regarding the FFS in the agreement, if comb offset hopping is configured for the SRS resource with TDM, when multiple groups of {1,2,…,s} are transmitted, across the groups the hopping pattern can follow the RRC configuration to be determined in Sec. 2.4 for comb offset hopping. Within each group of {1,2,…,s}, there can be hopping or non-hopping. With hopping, it is easier to allow multiplexing with other SRS, but without hopping, it may simplify UE/gNB behavior. The general positions are:

- Option 1: The time-domain behavior of cyclic shift hopping and/or comb offset hopping depends on the OFDM symbol index l’ of each symbol.

  - Supporting: CATT, Qualcomm, Huawei, HiSilicon, NTT DOCOMO, ZTE (3 5 proponents)

- Option 2: The time-domain behavior of cyclic shift hopping and/or comb offset hopping depends on the OFDM symbol index l’ of the first of the m OFDM symbol (or the first of the s OFDM symbol).

  - Supporting: CATT, FUTUREWEI, NTT DOCOMO, ZTE (3 4 proponents)

- Option 3: Do not support cyclic shift hopping or comb offset hopping for 8-port SRS.

  - Supporting: vivo (1 proponent)

We can see more companies’ views on this issue.

Proposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot and with repetition factor R, adopt the following interpretation:

Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot.

Views can be provided for the above proposal, and companies can feel free to suggest proposals for the other two issues.

| Company | View |
| --- | --- |
| Google | OK |
| Samsung | - We have same understanding as interpretation 2. |
| OPPO | We support the proposal. |
| ZTE | Support. Both interpretations 1 and 2 cause spec modification, while interpretation 2 is more straightforward. |
| Huawei, HiSilicon | Support the proposal. Regarding the FFS in the previous agreement, support option 1. |
| Lenovo | Support. All the 8 SRS ports should be sounded together. In case some symbols are dropped due to collision, the gNB can obtain the full channel information via one of the repetitions. |
| New H3C | Support |
| LGE | OK |
| QC | We support the proposal. By the way, on the comb offset and CS hopping. We are open to discuss all options. We don’t only support option 1. We updated the supporting companies list as following. In general, we think it is not mature to study this problem, given Proposal 3.2.1 is still pending. Because the comb offset hopping can support per symbol hopping or per R symbol hopping, in case of SRS repetition is configured, the formulation of option 1 seems missing the case when R is configured. Also, another option missing is determine CS hopping and/or comb offset hopping per s OFDM symbol, i.e., the TDMed SRS ports hopping together, similar to frequency hopping. - Option 1: The time-domain behavior of cyclic shift hopping and/or comb offset hopping depends on the OFDM symbol index l’ of each symbol. - Supporting: CATT, Qualcomm, ZTE (32 proponents) - Option 2: The time-domain behavior of cyclic shift hopping and/or comb offset hopping depends on the OFDM symbol index l’ of the first of the m OFDM symbol (or the first of the s OFDM symbol). - Supporting: FUTUREWEI, NTT DOCOMO, ZTE (3 proponents) - Option 3: Do not support cyclic shift hopping or comb offset hopping for 8-port SRS. - Supporting: vivo (1 proponent) |
| CMCC | Support |
| CATT | Support. Regarding the FFS in previous agreement, either option 1 or the same scheme as that agreed for a non-TDM based SRS resource configured with comb offset hopping ( i. e., if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l’ of each symbol or the first symbol across the R repetitions based on RRC configuration) is acceptable. |
| Vivo | Support |
| Xiaomi | support |
| Nokia/NSB | Support |
| Apple | We do not support the proposal, it is unneccesary. |
| FL | @Qualcomm: For Option 1, it is applicable to both cases with and without repetition. And the case you mentioned seems to be included in “(or the first of the s OFDM symbol)” part. @Apple: We could skip the proposal, but it’s better if the group can align the understanding. If your suggestion is to leave this for the editor, I think that should work but our other proposals need to be written in a way compatible to both interpretations. |
| Ericsson | Support the proposal. |
| NTT DOCOMO | Ok with the proposal. For the subsequent discussion on three options, we are fine with option1 and/or option 2. |
| Sharp | Support. |

#### Tuesday offline

- Interpretation 1: R is a multiple of s, and the group of {1,2,…,s} is repeated R/s times in the slot.

  - Supporting: Apple, NTT DOCOMO (2 proponents)

- Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot.

  - Supporting: CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Lenovo, LG, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo, xiaomi, ZTE (20 proponents)

Proposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot and with repetition factor R, adopt the following interpretation:

Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot.

If this cannot be agreed, we could consider the following alternative:

Proposal 3.2.1-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot and with repetition factor R, there are two interpretations and it is up to the editor(s) to decide which one to adopt:

Interpretation 1: R is a multiple of s, and the group of {1,2,…,s} is repeated R/s times in the slot, where R is a multiple of s.

Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot, and m is a multiple of sR.

| Company | View |
| --- | --- |
| Sharp | We support Proposal 3.2.1, and fine with Proposal 3.2.1-1. There is no technical difference between Interpretation 1 and Interpretation 2. |
| OPPO | - Common understanding in RAN1 is preferred. We support to agree with proposal 3.2.1 to follow majority understanding. |
| Lenovo | Support Proposal 3.2.1 |
| ZTE | Support Proposal 3.2.1. |
| Ericsson | After Tuesday’s offline discussion, it is our understanding that there is still no consensus between companies regarding how SRS ports are mapped to OFDM symbols for SRS TDM (based on the previous agreements we’ve made) and the above proposals above does not really address this issue. There are (at least) three different understandings, expressed by different companies. To help us reach a common understanding, these three cases are illustrated below for the case of an SRS resource configured with TDM factor s=2, repetition factor R=2, and with frequency hopping (two different sets of PRBs). Here, the first and second subset of SRS ports are illustrated in blue and red, respectively. Understanding 1 Understanding 2 Understanding 3 Note that all above figures are different examples of cyclic mapping. It would be great if companies could share their understanding. We prefer Understanding 1as it preserves the interpretation of the repetition factor as the number of consecutive OFDM symbols containing the same set of SRS ports. This provides maximum robustness towards channel aging. If Understanding 1 is not adopted, we will have a different (compared to legacy specification) interpretation of the repetition factor depending on whether TDM is configured or not, which will be confusing. Furthermore, with Understanding 1, we can straightforwardly extend existing frequency-hopping formulas to the case of TDM. To make it clear how SRS ports are mapped to OFDM symbols, we propose the following: Proposal 3.2.1-2: For SRS TDM: SRS ports 0, 1,…,NapSRSs-1 are transmitted in R consecutive SRS symbols starting from 0, Rs, 2Rs, 3Rs, ..., SRS ports NapSRS/s, NapSRS/s+1, ..., 2NapSRSs-1, are transmitted in R consecutive SRS symbols starting from R, R(s + 1), R(2s + 1), R(3s + 1), ..., etc., where s=2 is the TDM factor and SRS symbols are measured from first OFDM symbol configured for the SRS resource. The above proposal agrees with the first of the above figures (Understanding 1): SRS ports 0—3 are transmitted in 2 consecutive OFDM symbols starting from 0 + 6 = 6 and 4 + 6 = 10, SRS ports 4—7 are transmitted in 2 consecutive OFDM symbols starting from 2 + 6 = 8 and 6 + 6 = 12. |
| FL2 | Regarding Interpretation 1 and Interpretation 2: Proposal 3.2.1 can still be discussed, and we can see if companies can converge. Otherwise, Proposal 3.2.1-1 can be considered. Regarding Understandings 1, 2, and 3 from Ericsson: Based on my discussions with the companies, Understanding 3 is the one agreed per the following agreement: Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, the s subsets of ports are mapped cyclically as {{1, 2, …, s}, …, {1, 2, …, s}} on the m OFDM symbols. In any case, as this is a critical issue, companies please provide your view on the 3 understandings and Ericsson’s proposal. If Understanding 3 is confirmed by most companies, to avoid any potential confusion, I suggest the group to agree on a clarification conclusion: Proposed conclusion: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, the s subsets of ports are mapped cyclically as {{1, 2, …, s}, …, {1, 2, …, s}} on the m OFDM symbols, and each value in {1,2, …, s} corresponds to one subset of ports and one OFDM symbol. |

### TDM factor s

We had the following agreement:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

At least s = 2.

  - FFS: s = 4, s = 8.

m = 2,4,8, 10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Regarding the FFS points colored in red, the general positions are:

Support s = 4

  - Supporting: Apple, CATT, CMCC, Futurewei, Google, Huawei, HiSilicon, KDDI, Lenovo, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Sharp, xiaomi, ZTE (12 18 proponents)

  - Against: Ericsson, Intel, LG, NTT DOCOMO, Samsung, vivo (6 opponents)

Support s = 8

  - Supporting: KDDI, Sharp (2 proponents)

  - Against: Ericsson, Intel, LG, NTT DOCOMO, OPPO, Samsung, vivo, xiaomi, ZTE (9 opponents)

Determination of s

  - Implicit determination based on configured parameters: Ericsson, Nokia, Nokia Shanghai Bell, OPPO (4 opponents)

  - Explicit determination based on configured parameters: CATT, Nokia, Nokia Shanghai Bell, OPPO (4 opponents)

Given the large number of proponents for s=4, we can see if an agreement can be achieved to support it.

Proposal 3.2.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4.

Views can be provided for the above enhancements. Proponents for implicit determination based on configured parameters can further suggest proposals.

| Company | View |
| --- | --- |
| Google | OK |
| Samsung | - Not support more TDM factors (s values), only support s=2. Longer s cause more collision probability. |
| OPPO | Fine with s=4. |
| ZTE | Support. |
| Huawei, HiSilicon | Support. |
| Lenovo | Support. S=4 is useful for UE with Ng=4 antenna groups and can further improve the sounding performance with higher SRS transmission power. |
| New H3C | Support |
| LGE | Do not support. Same view as Samsung. |
| QC | We are fine with s=4. |
| CMCC | Support. |
| CATT | Support. |
| Vivo | s=2 is enough. When 8 ports of one SRS resource is mapped on 2 OFDM symbol, 4 ports can be mapped in each OFDM symbol, which is similar to two 4-port SRS resource mapped in two symbols to handle the large delay spread case. |
| Xiaomi | support |
| Nokia/NSB | Support |
| Apple | Fine with the proposal. |
| KDDI | We support the proposal. TDM factor s=4 has benefits in terms of partial coherent UEs with Ng=4 and higher SRS transmission power. |
| Ericsson | Do not support, we think s = 2 (4 ports per symbol, same as for legacy 4-port SRS resource) is enough. It is unclear why 8-port SRS resources would require more power per port (at the cost of an SRS resource occupying more OFDM symbols) compared to 4-port SRS resources. Furthermore, s = 2 is the only value of s that supports all values of m > 2, where m is the number of SRS symbols spanned by the SRS resource. |
| NTT DOCOMO | Ok |

- This discussion is closed with the following conclusion:

Conclusion

There is no consensus on the support of the following feature in RAN1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4.

### TDM power control related issues

- A few power control related issues for TDMed 8 ports were discussed by some companies:

The UE splits a linear value PSRS of SRS transmission power equally across the configured SRS ports within each OFDM symbol, at least for UE capable of transmitting at PCMAX per OFDM symbol with 8/s ports.

  - Supporting: Apple, CATT, CMCC, Futurewei, Google, Huawei, HiSilicon, Intel, Lenovo, LG, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo, xiaomi, ZTE (11 21 proponents)

  - Note that this may be captured in the specification in several different but essentially equivalent ways, e.g., using the above text without changing the equations, or using the existing text but changing the equations, etc. How it is captured should be left for the editor, and we note that how it may be best captured may also depend on the outcome of the discussion for the next bullet point.

For UE NOT capable of transmitting at PCMAX per OFDM symbol with 8/s ports, some detailed alternatives were provided:

  - Alt 1: Introduce a scaling factor for PCMAX

  - Alt 2: Introduce a scaling factor for the term calculated based on P0, alpha, PL, and TPC command (before comparing to PCMAX)

  - Alt 3: Introduce SRS-specific PCMAX for TDM, e.g., PCMAXSRS(i) or PCMAXSRS(i,l), which may require RAN4 inputs

  - Alt 4: Introduce a scaling factor for PSRS

  - Alt 5: Do not support TDM for such UEs

  - Alt 6: Reuse existing power control for such Ues

Maintain the same SRS transmission power over the m OFDM symbols

  - Supporting: OPPO, Qualcomm, Sharp, ZTE

Other power control related proposals supported by one or two companies:

  - Full power transmission capability reporting

  - Power scaling when SRS overlaps with another uplink signal

  - Allow different transmission power levels on different OFDM symbols in a slot

- We can first try to reach an agreement on the case where the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports. We can then discuss options for the other cases.

Proposal 3.2.3-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications.

Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide.

Proposal 3.2.3-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options:

Option 1: Introduce a power scaling factor without introducing PCMAXSRS, and FFS details.

Option 2: Introduce PCMAXSRS, and FFS details.

Option 3: Do not support TDM for such UE.

Option 4: Reuse existing power control mechanism for such UE.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | OK with both proposals. |
| Samsung | - We support Proposal 3.2.3-1, and in the Note, “it is up to editor” seems enough. - Regarding Proposal 3.2.3-2, if a UE is not capable of transmitting at P_CMAX per OFDM symbol with 8/s ports, our view is that the UE shall not support TDM, since power boosting is the key motivation and benefit from TDM scheme. |
| OPPO | Support both proposals. |
| ZTE | Current Proposal 3.2.3-2 is confusing. Does Option 1 mean introducing a power scaling factor for PCMAX? And does Option 4 mean splitting a linear value PSRS of SRS transmission power equally across all SRS ports? If so, we suggest the following modification of Proposal 3.2.3-2: Proposal 3.2.3-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options: Option 1: Introduce a power scaling factor for PCMAX, without introducing PCMAXSRS, and FFS details. Option 2: Introduce PCMAXSRS, and FFS details. Option 3: Do not support TDM for such UE. Option 4: Reuse existing power control, i.e., splitting a linear value PSRS of SRS transmission power equally across all SRS ports, for such UE. With the modified Proposal 3.2.3-2, we support option 1. Considering the high spec complexity, we suggest to preclude solution 2 at first. In comparison, solution 1 simply introduces a power scaling factor for current PCMAX, and can well utilize the PA capability. Therefore, solution 1 should be prioritized over others. |
| Huawei, HiSilicon | Support both proposals. |
| Lenovo | For Proposal 3.2.3-1: Support. For Proposal 3.2.3-2: The motivation of TDM based SRS scheme is to transmit the SRS with higher power by transmit less SRS ports in a symbol. If the UE does not support full power transmission with part of SRS ports in a symbol, TDM is not preferred to be configured. |
| New H3C | Support both proposals |
| LGE | We can live with the Proposal 3.2.3-1 since there could be power boosting for TDM case. Regarding Proposal 3.2.3-2, similar view as Samsung and support Option 3 |
| QC | Proposal 3.2.3-1: We support this proposal. Proposal 3.2.3-2: We are in general fine with option 1, 2, 3. But we are not sure option 4 can work. If UE is not capable of transmitting at PCMAX per OFDM symbol, but gNB follow existing power control mechanism to keep sending power control command to boost UE power exceeding UE’s capability, what should UE do? I am not a RAN4 expert. But I feel this UE might fail RAN4 max SRS Tx power test. |
| CMCC | Proposal 3.2.3-1: Support Proposal 3.2.3-2: We are fine with Option 1 and Option 2. |
| CATT | For Proposal 3.2.3-1: Support. For Proposal 3.2.3-2: For the Ues not capable of transmitting at P_CMAX per OFDM symbol with 8/s ports, but capable of transmitting at a power larger than P_CMAX/s, the benefit of power boosting can be achieved by option 1 or option 2. Therefore TDM based SRS transmission should be supported at least for such Ues. |
| Vivo | Support both proposals. |
| Xiaomi | Fine with the two proposals |
| Nokia/NSB | Support the proposal 3.2.3-1. For the proposal 3.2.3-2, we are open to more options. |
| Apple | Proposal 3.2.3-1 is okay. Proposal 3.2.3-2 needs further discussion. Why UE would do this which hurts the link budget. |
| FL | @ZTE: Currently, the formulation of Option 1 includes Alt 1 and Alt2, and which one to be further considered is part of the FFS, but in either case, Option 1 does not change the current definition of Pcmax and no RAN4 input is needed, which is why I grouped them together just for now. For Option 4, yes, that’s exactly what the existing mechanism is. @Qualcomm: For Option 4, the per-port transmission power is still PSRS/8, and hence the per-symbol power is PSRS/s, which should be achievable by the UE. |
| Ericsson | Support Proposal 3.2.3-1. For Proposal 3.2.3-2, we think it is not needed as UE not capable of transmitting P_CMAX per OFDM symbol, for s = 2, would not benefit from TDM and, hence, should not support it (Option 3). |
| NTT DOCOMO | Support 3.2.3-1. For 3.2.3-2, overall, we are not sure if it is good to have adjustment on PCMAX in RAN1 specification. PCMAX is calculated by UE based on quite some conditions as per 101 series. We agree PCMAX should be somewhat adjusted, but it is better to leave it to RAN4 from our perspective. We think any option can be handled by RAN4. |

#### Tuesday offline

Proposal 3.2.3-1 has been agreed, and we will discuss Proposal 3.2.3-2.

Proposal 3.2.3-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options:

Option 1: Introduce a power scaling factor without introducing PCMAXSRS, and FFS details.

Option 2: Introduce PCMAXSRS, and FFS details.

Option 3: Do not support TDM for such UE.

Option 4: Reuse existing power control mechanism for such UE.

Proposal 3.2.3-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options:

Option A: Do not support TDM for such UE. (Apple, Ericsson, Lenovo, LG, Samsung, Nokia)

Option B: Support TDM for such UE. (ZTE, Qualcomm)

  - Option B1: Reuse existing power control mechanism for such UE. (Apple, Lenovo, Nokia)

  - Option B2: Modify existing power control mechanism for such UE.

    - Option B2.1: Introduce a power scaling factor without introducing PCMAXSRS, and FFS details.

      - Option B2.1.1: Introduce a power scaling factor for PCMAX

      - Option B2.1.2: Introduce a power scaling factor for PSRS

      - Option B2.1.3: Introduce a scaling factor for the term calculated based on P0, alpha, PL, and TPC command (before comparing to PCMAX)

    - Option B2.2: Introduce PCMAXSRS, and FFS details.

| Company | View |
| --- | --- |
| Sharp | We support Option 2 and think details are up to RAN4. |
| OPPO | - We support Option 1. We prefer a unified power scaling mechanism for UE with different power capabilities. |
| Lenovo | We support Option 3 given that TDM cannot provide performance gain. |
| Ericsson | We don’t think this is needed after Proposal 3.2.3-1 has been agreed and since only s = 2 is supported. It is our view that UE not capable of supporting full power for the SRS ports in a subset would not support SRS TDM. |
| ZTE | Support option 1. Option 2 should be precluded first, considering we have to send a LS to RAN4 to ask how to define the PCMAX for TDMed SRS but only two meetings are left and the time for discuss is not sufficient |
| FL2 | Based on the offline discussion, we can focus on the following updated proposal: Proposal 3.2.3-3: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options: Option A: Do not support TDM for such UE. Option B: Support TDM for such UE. - Option B2: Modify existing power control mechanism for such UE. - Option B2.1: Introduce a power scaling factor without introducing PCMAXSRS, and FFS details. - Option B2.1.1: Introduce a power scaling factor for PCMAX - Option B2.1.2: Introduce a power scaling factor for PSRS - Option B2.1.3: Introduce a scaling factor for the term calculated based on P0, alpha, PL, and TPC command (before comparing to PCMAX) - Option B2.2: Introduce PCMAXSRS, and FFS details. |

### Collision handling for TDMed ports

We had the following agreement for further study:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, study at least the following solutions:

- Whether or not a UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage, coherency, and/or repetition configuration.

- Whether or not a UE changes the transmission order of the subsets of ports.

The general positions are:

Option 1: When the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, UE drops the SRS transmission on some or all of the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage and coherency.

  - Supporting: Huawei, HiSilicon, Intel, LG, NEC, NTT DOCOMO, OPPO, Qualcomm, Sharp, Spreadtrum (9 10 proponents)

  - Some detailed options include, e.g., all the s OFDM symbols are dropped for CB/AS, or the OFDM symbol(s) within a coherent group is dropped for CB, etc. These can be further discussed.

Option 2: Legacy per-OFDM symbol based dropping rules are kept for TDMed 8-port SRS.

  - Supporting: Apple, CATT, vivo, xiaomi, ZTE (5 proponents)

Option 3: UE changes the transmission order of the subsets of ports.

  - Supporting: Samsung

Based on the views, we can try the following proposal.

Proposal 3.2.4: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE drops the SRS transmission on some or all of the rest of OFDM symbols within the group of {1, 2, …, s} based on the usage and coherency, and FFS details.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | We suggest adding a bracket for “based on the usage and coherency” in the last line. We are not sure why coherency could be a factor to determine the rule. Besides, we sometimes the gNB may configure the overlapped SRS resources for SRS for codebook and antenna switching. It is better to have a unified solution for both codebook and antenna switching. |
| Samsung | - Our view is that both Option 1 and 3 could be combined and supported. - We understand that Option 1 can be applied to all cases generally. - Regarding Option 3, we focused on the case which time domain behaviour of SRS could be affected by overlapping severely. - Between P-SRS and SP-SRS, they can be overlapped at the common multiple of their periods. If so, some subsets of periodic SRS could not be transmitted. - For AP SRS, based on gNB scheduling and flexible slot offset adopted in Rel-17, overlapping could be avoided, hence the effect of dropping could be minimized. - Therefore, we consider the solution on P/SP SRS separately. - Hence, we think that Option 3 is also considered together with Option 1. |
| OPPO | We support google’s proposal to put a bracket for “based on the usage and coherency”, and also for “some or”. We also prefer a unified solution for different types of SRS. |
| ZTE | Do NOT support. We believe SRS transmissions on each OFDM symbol should be dropped independently. - First, this issue is related to the port splitting scheme. Based companies’ discussion, SRS ports belonging to a same coherent group should be mapped onto a same symbol. If this port splitting scheme is adopted, there is no coherency-related dropping issue, and UE should only drop the SRS transmissions on the symbols where collision happens. - Second, when repetition is configured, it is unlikely that all symbols carrying a same subset of SRS ports are dropped. Hence, SRS ports mapped onto symbols where no collision happens should be transmitted normally, then all the 8 SRS ports can be successfully sounded. The following figure gives an example. In the example, m = 4, s = 2, and R = 2, the second and third symbols collide with other signals, but the SRS transmissions on the first and fourth symbols can be transmitted normally, then all the 8 SRS ports are successfully sounded. |
| Huawei, HiSilicon | In current spec., when SRS is fully/partially overlapped with other transmissions with higher priority (e.g., PUCCH transmission), the overlapped SRS symbol(s) is dropped (Option 2). Following this principle, for a TDMed 8-port SRS resource, when SRS dropping occurs on part of the SRS symbols corresponding to a coherent group, the remaining SRS transmission on other SRS symbols corresponding to the same coherent group may become invalid due to the lost phase continuity, sending which will only bring extra SRS interference without any benefit. An intuitional way confronting above situation is dropping SRS transmission on all SRS symbols corresponding to the coherent group (named full dropping hereinafter). However, when the TDMed 8-port SRS resource is configured with repetition factor R>1, given that the SRS transmission on non-overlapped SRS symbols has the potential to be combined with continuous SRS transmission in other repetitions to obtain the channel information with guaranteed phase continuity, full dropping, which delays the channel information acquisition and wastes the allocated physical resource, is not preferred. An example is shown below, where a TDMed 8-port SRS resource is configured with TDM factor s=2 and repetition factor R=2, and all the 8 PUSCH ports respectively corresponding to 8 SRS ports belong to one coherent group (full-coherent). When SRS dropping occurs on the first and fourth symbol, the remaining SRS transmission on the second and third symbol can still be combined to conduct the complete channel estimation facilitating full-coherent UL transmission. Under this circumstance, it is better to retain the remaining SRS transmission rather than to conduct full dropping. Based on above discussion, we suggest to modify the agreement as below: Proposal 3.2.4: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the sm OFDM symbols within a group of {1, 2, …, s} is dropped, the UE may drops the SRS transmission on some or all of the rest of OFDM symbols within the group of {1, 2, …, s} based on the usage, repetition configuration and coherency, and FFS details. |
| Lenovo | We are fine with this proposal. |
| New H3C | Support |
| LGE | If the UE drops some of the rest of symbols within the group, fully 8-port can’t be transmitted. Then exact CSI cannot be obtained. So, we prefer to change it to clear “all” instead of "some or all". We are also prefer to put a bracket for “based on the usage and coherency”. |
| QC | To Google: coherency is determination factor here. In our view, With SRS usage of codebook based PUSCH, if codebook is coherent, even if one out of s OFDM is dropped, UE should drop all s OFDM symbols, because all ports in s OFDM symbol supposed to keep same behavior to maintain coherency. While for noncoherent codebook, legacy per OFDM dropping rule can be reused, because no coherence is required across the ports on s OFDM symbols. |
| CMCC | For SRS resource set with usage ‘antennaSwitching’, per OFDM dropping rule can be reused. We also agree with ZTE and Huawei that repetition configuration may be also added. |
| CATT | Not support. We support option 2. As noted by ZTE, with legacy per-OFDM symbol based dropping rules, although some SRS ports in some OFDM symbols are dropped, it is possible that all the 8 SRS ports are successfully sounded when repetition factor R > 1 is configured. We prefer to use a unified dropping rule for various usages, coherencies, etc. Option 3 is not preferred either, since it would increase the complexity of UEs and gNB. |
| vivo | Support to only drop the collision part. As some companies analyzed, when part of SRS ports is dropped in some symbols, it is still feasible to use the remaining SRS ports such as based on repetition. Moreover, even without repetition, the channel estimation results of the remaining ports can also help for gNB such as time filtering based on historical channel estimation results. |
| Xiaomi | Do not support. We think legacy dropping rule can be reused. |
| Nokia/NSB | We are fine with the proposal. Detail condition can be still discussed. |
| Apple | Legacy SRS dropping is per symbol based. Without any consensus, there is no need to change the UE behavior. All we need to discuss is whether there is consensus to change legacy SRS dropping, in which case we do not think there is any need. |
| NTT DOCOMO | Support |
| Spreadtrum | Fine with the proposals. |
| FL2 | Please continue to discuss. |

### Port splitting for TDM

- Regarding how the 8 ports are split into s subsets for TDM, some companies provided the following discussions.

Option 1: Within each of the s subsets, the port indexes are consecutive. For example, for s=2, the first subset has {1000,1001,1002,1003}, and the second subset has {1004, 1005, 1006, 1007}.

Option 2: The ports within a same coherent port group are mapped to one OFDM symbol if s ≤ Ng, otherwise the ports are mapped to two consecutive OFDM symbols.  The exact port indexing may be done in Agenda Item 9.1.4.2.

- We can see if the high-level principle of Option 2 is agreeable.

Proposal 3.2.5: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot the port,

For usage ‘codebook’ with Ng coherent port group(s) for the 8 ports, the 8/Ng ports in a coherent port group are mapped to s/Ng consecutive OFDM symbol(s).

  - FFS detailed port indexing per OFDM symbol based on the outcome from Agenda Item 9.1.4.2.

For usage ‘antennaSwitching’, adopt the same port indexing per OFDM symbol as ‘codebook’.

Views can be provided for the above enhancements.

| Company | View |
| --- | --- |
| Google | We think this needs more discussion. Ng can be 1,2,4,8. We are not sure how this can work for all possible Ng. |
| Samsung | - We think that it is better to postpone the discussion on this issue after finalizing issue 3.2.4 (collision handling) and port numbering in AI 9.1.4.2. |
| OPPO | We think the proposal is unable to cover Ng=1,2,4,8. |
| ZTE | Support Proposal 3.2.5 in general. However, do we need to configure s > Ng, e.g., s = 4 and Ng =2, for TDM SRS? |
| Huawei, HiSilicon | Seems reasonable. |
| Lenovo | For Ng>1, it is better to sound antenna ports within a same antenna group in an OFDM symbol to obtain better channel matrix corresponding to the antenna group. |
| NewH3C | Open to discuss |
| QC | We also don’t think this is a critical issue needs decision now. It is better to wait for the progress in AI 9.1.4.2. |
| CMCC | We may consider Ng=1,2,4,8. |
| CATT | Open to discuss. The proposal cannot cover which antenna groups are mapped to the same OFDM symbol(s) when Ng > s. |
| vivo | Can be discussed later after the outcome of AI 9.1.4.2. |
| Nokia/NSB | Open to discuss. |
| Apple | Low priority. The formulation is problematic, there is no need for ceiling operation. |
| FL | We can wait for the outcome of the discussion on s value(s) and also keep an eye on how 9.1.4.2 is going. I believe the current formulation cover the cases of Ng > s, since in those cases, s/Ng=1, which is what’s intended. Please let me know if I missed anything. |
| KDDI | Considering coherency between antenna groups, it is desirable to map SRS ports of the same coherent antenna group to consecutive symbols. Also, this proposal is related to the proposal 3.3.2. |
| NTT DOCOMO | Ok to discuss |
| FL2 | Please continue to discuss. |

## Other proposals for 8Tx SRS

Several companies further detailed enhancements for the 8Tx SRS, and some of the enhancements may be related to the outcomes of above discussions.

Enh. 1: Maintain phase/beam consistency over multiple OFDM symbols for TDM

  - Supported by: Sharp

Enh. 2: Whether to down select from existing resource mapping schemes for TDM

  - Discussed by: Ericsson

Enh. 3: Whether to downgrade configuration of SRS for antenna switching

  - Supported by: CMCC

Enh. 4: Same number of symbols and TDM pattern for SRS resources for a SRS resource set

  - Supported by: Apple

Views can be provided for the above enhancements, and new details / proposals can also be included.

| Company | View |
| --- | --- |
| Google | We think the power scaling for TDMed SRS needs to be discussed. Currently power scaling is performed per symbol. But for TDMed SRS, such power scaling approach could cause incorrect UL channel estimation for UL CSI measurement. |
| ZTE | Enh1: Support Enh2: We can further discuss. Enh3: We can further discuss. Enh4: Support. |

# Conclusions

## For Monday Online

Proposal 2.1: For SRS comb offset hopping / cyclic shift hopping reinitialization periodicity of N radio frame(s), down select one from the following options:

- Option 1A: N = 1

- Option 1B: N = 1024

- Option 2: N ≥ 1 is configurable

  - Option 2A: N ≥ 1 is configurable and based on SRS periodicity P, and FFS details

  - Option 2B: N ≥ 1 is configurable and chosen from the set {2n}, and FFS n = 0,1,…,5 or 0,1,…,10

Option 1: CATT, CMCC, Ericsson, FUTUREWEI, Google, Huawei, HiSilicon, Intel, Lenovo, NEC, NTT DOCOMO, Sharp, OPPO, vivo, ZTE (15 proponents)

  - Option 1A: Ericsson, Intel, NTT DOCOMO, OPPO, Sharp (5 proponents)

  - Option 1B: Apple, CATT, CMCC, FUTUREWEI, Google, Huawei, HiSilicon, Lenovo, NEC, OPPO, vivo (11 proponents)

  - Option 1C: N = 32 or other small number (ZTE)

Option 2: CMCC, Futurewei, InterDigital, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm, Samsung, Spreadtrum, vivo, xiaomi (12 proponents)

  - Option 2A: CMCC, InterDigital, Samsung, Spreadtrum, xiaomi (5 proponents)

  - Option 2B: Apple, Futurewei, InterDigital, Qualcomm, Samsung, Spreadtrum (6 proponents)

Proposal 3.2.3-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications.

Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide.

Proposal 3.2.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4.

Supporting: Apple, CATT, CMCC, Futurewei, Google, Huawei, HiSilicon, KDDI, Lenovo, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Sharp, xiaomi, ZTE (18 proponents)

Against: Ericsson, Intel, LG, NTT DOCOMO, Samsung, vivo (6 opponents)

Proposal 3.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 on 2 comb offsets (KTC=4, kTC=2) or comb 8 on 4 comb offsets (KTC=8, kTC=4), the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

- For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kTCNapSRS/kTC mod nSRScs,max.

Option 1: The cyclic shifts are completely aligned across the multiple comb offsets

  - Supporting: Apple, CATT, CMCC, Futurewei, Google, Huawei, HiSilicon, Lenovo, LG, New H3C, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo, xiaomi (17 proponents)

Option 2: The cyclic shift positions have minimum alignment across the multiple comb offsets

  - Supporting: Ericsson, Futurewei, NEC, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, ZTE (8 proponents)

Proposal 2.2: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

Supporting: CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Intel, Lenovo, New H3C, Nokia, Nokia Shanghai Bell, Samsung, xiaomi, ZTE (15 proponents)

Against: Apple, Sharp, LG, Qualcomm, Spreadtrum (5 opponents)

Proposal 2.3: SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

Supporting: Apple, CATT, Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, Lenovo, LG, NEC, Nokia, Nokia Shanghai Bell, Qualcomm, Spreadtrum, xiaomi, vivo, ZTE (17 proponents)

Against: Google, New H3C, OPPO, Samsung (4 opponents)

Proposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot and with repetition factor R, adopt the following interpretation:

Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot.

- Interpretation 1: R is a multiple of s, and the group of {1,2,…,s} is repeated R/s times in the slot.

  - Supporting: Apple, NTT DOCOMO (2 proponents)

- Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot.

  - Supporting: CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Lenovo, LG, New H3C, Nokia, Nokia Shanghai Bell, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo, xiaomi, ZTE (20 proponents)

## Agreements from Monday Online

Working Assumption

For SRS comb offset hopping / cyclic shift hopping reinitialization periodicity of N radio frame(s):

- N = 128

Proposal 3.2.3-1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications.

Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide.

Conclusion

There is no consensus on the support of the following feature in RAN1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4.

Proposal 3.1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 on 2 comb offsets (KTC=4, kTC=2) or comb 8 on 4 comb offsets (KTC=8, kTC=4), the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

- For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kTCNapSRS/kTC mod nSRScs,max.

## For Tuesday Offline

Proposal 3.2.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot and with repetition factor R, adopt the following interpretation:

Interpretation 2: The group of {1,2,…,s} is repeated R times in the slot.

Proposal 2.6: For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

•	Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

•	Note: This does not increase the maximum number of cyclic shifts.

Proposal 3.2.3-2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options:

Option 1: Introduce a power scaling factor without introducing PCMAXSRS, and FFS details.

Option 2: Introduce PCMAXSRS, and FFS details.

Option 3: Do not support TDM for such UE.

Option 4: Reuse existing power control mechanism for such UE.

Proposal 2.3: SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

## For Wednesday Online

Proposal 2.2-3: Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol.

[This is a UE-optional feature.]

Down select from the following options:

  - Option 1: khopping(pi)=m=07c8nslotframe,μnf mod NNsymbslot+ns,fμNsymbslot+l0+l'+m∙2m mod L(pi) where L(pi) represents the length of the subset, which means the candidate offset values belong to {0, 1, …, L(pi)-1}.

  - Option 2: The subset is configured by legacy parameters cyclicShift and combOffset for cyclic shift hopping and comb offset hopping, respectively.

  - Option 3: The number of subsets is fixed as 2, and a subset is defined as half of all possible hop locations.

    - For comb-offset hopping with comb-X (X=2, 4, or 8), 1st subset can be comb-offset 0 ~ X/2-1, and 2nd subset can be comb-offset X/2 ~ X-1.

    - For cyclic shift hopping with maxCS = Y (Y=8, 12, 6) for SRS resource with 1 port, 1st subset can be cyclic shift 0 ~ Y/2-1, and 2nd subset can be cyclic shift Y/2 ~ Y-1.

    - For cyclic shift hopping with maxCS = Y (Y=8,12,6) for SRS resource with >1 ports, 1st subset can be cyclic shift CS 0 ~ Y/4-1 & CS Y/2 ~ 3Y/4-1, and 2nd subset can be cyclic shift CS Y/4 ~ CS Y/2-1 & CS 3Y/4 ~ Y-1.

Potential RRC impact

CATT, CMCC, Ericsson, Futurewei, Google, Huawei, HiSilicon, Intel, Lenovo, New H3C, Nokia, Nokia Shanghai Bell, Samsung, Sharp, xiaomi, ZTE (16)

Concern: Apple, LG, Qualcomm, Spreadtrum (4)

Proposal 2.6: For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

•	Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

•	Note: This does not increase the maximum number of cyclic shifts.

Potential RRC impact

Support: China Unicom, Futurewei, Google, Lenovo, LGU+, Huawei, HiSilicon, New H3C, Spreadtrum, Xiaomi, vivo (11)

Open/fine to discuss: CMCC, NTT DOCOMO, ZTE (3)

Concern: LG, Nokia, Nokia Shanghai Bell, Qualcomm, Sharp (5)

Proposal 3.2.3-3: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, if the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications, down select one from the following options:

Option A: Do not support TDM for such UE.

Option B: Support TDM for such UE.

  - Option B2: Modify existing power control mechanism for such UE.

    - Option B2.1: Introduce a power scaling factor without introducing PCMAXSRS, and FFS details.

      - Option B2.1.1: Introduce a power scaling factor for PCMAX

      - Option B2.1.2: Introduce a power scaling factor for PSRS

      - Option B2.1.3: Introduce a scaling factor for the term calculated based on P0, alpha, PL, and TPC command (before comparing to PCMAX)

    - Option B2.2: Introduce PCMAXSRS, and FFS details.

Potential RAN4 impact

Proposal 2.3: SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

Apple, CATT, Ericsson, Futurewei, Huawei, HiSilicon, InterDigital, Lenovo, LG, NEC, Nokia, Nokia Shanghai Bell, Sharp, Spreadtrum, OPPO, Qualcomm, xiaomi, vivo, ZTE (18 proponents)

Concern: Google, New H3C, Samsung (3)

Proposal 3.2.4: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE drops the SRS transmission on some or all of the rest of OFDM symbols within the group of {1, 2, …, s} based on the usage and coherency, and FFS details.

Huawei, HiSilicon, Intel, LG, NEC, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, Spreadtrum (11)

Concern: Apple, CATT, vivo, xiaomi, ZTE (5)

Proposal 2.7: Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

Apple, CATT, CMCC, Ericsson, Google, Lenovo, InterDigital, NEC, New H3C, Nokia, Nokia Shanghai Bell, Samsung, Sharp, Spreadtrum, OPPO, Qualcomm, ZTE (17 proponents)

Concern: ETRI, Futurewei, LG, vivo, xiaomi

# References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#94-e.

R1-2304351, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2304378, Discussions on SRS enhancement in Rel-18, New H3C Technologies Co., Ltd..

R1-2304396, SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2304420, On SRS enhancements targeting TDD CJT and 8 TX operation, Ericsson.

R1-2304425, Further Details on SRS for CJT and 8TX UEs, InterDigital, Inc..

R1-2304467, Further discussion on SRS enhancements, vivo.

R1-2304546, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, Spreadtrum Communications.

R1-2304640, Discussion on SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon.

R1-2304709, On SRS enhancement for CJT and 8Tx operation, CATT.

R1-2304819, SRS Enhancements for Rel-18 NR, Intel Corporation.

R1-2304838, On SRS Enhancement, Google.

R1-2304877, Discussion on SRS enhancements, xiaomi.

R1-2304954, Discussion of SRS enhancement, Lenovo.

R1-2304971, Discussions on SRS enhancement targeting TDD CJT and 8 TX operation, KDDI Corporation.

R1-2304997, Discussion on SRS enhancement, NEC.

R1-2305081, Discussion on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2305230, Views on Rel-18 MIMO SRS enhancement, Apple.

R1-2305292, SRS enhancement targeting TDD CJT and 8 TX operation, LG Electronics.

R1-2305322, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2305405, SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2305501, Views on SRS enhancements, Samsung.

R1-2305587, Discussion on SRS enhancement, NTT DOCOMO, INC..

R1-2305735, SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2305752, SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

R1-2305787, Discussion on SRS enhancement targeting TDD CJT, ETRI.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |
| --- | --- | --- |
| Parameter |  | Value |
| Duplex, Waveform |  | TDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30kHz |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |
| Frame structure |  | DSUDD, or companies to state the used frame structure |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| DL Channel estimation |  | Realistic |
| Evaluation Metric |  | DL throughput |
| Baseline for performance evaluation |  | R17 SRS design |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |

## Appendix 2: Agreements from RAN1#110

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

## Appendix 3: Agreements from RAN1#110bis-e

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Conclusion

The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

Conclusion

- No further discussion of increasing the maximum number of cyclic shifts for CJT SRS.

- No further discussion of partial frequency sounding extensions for CJT SRS.

Agreement

For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Agreement

For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- n can be 8

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

## Appendix 4: Agreements from RAN1#111

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on:

Option 1: The hopping pattern is based on the pseudo-random sequence c(i), initialized with a network-configured ID.

FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

FFS: The relation between the legacy group / sequence hopping and the new hopping

Agreement

For SRS interference randomization, support one from the following options  (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

Conclusion

No consensus on enhanced signaling for flexible SRS transmission in Rel-18

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

## Appendix 5: Agreements from RAN1#112

Agreement

For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

At least s = 2.

  - FFS: s = 4, s = 8.

m = 2,4,8, 10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- Option 2: For comb 4, do not support 4 comb offsets.

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

FFS: At least support reinitialization at the beginning of each radio frame.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Conclusion

No consensus to support the following for TDD CJT SRS enhancement in Rel-18:

Further enhancements to frequency hopping

Sequence hopping/randomization, per-hop sequence from a long SRS sequence

Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

Precoded SRS for DL CSI acquisition

Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

Configuration of  (sequence index within a group) per SRS resource

Multiplying mask sequence to the legacy SRS sequence

Conclusion

No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18.

## Appendix 6: Agreements from RAN1#112bis-e

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID determined by cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

- The range of the new ID is from 0 to 1023

Agreement

For a SRS resource configured with comb offset hopping and/or cyclic shift hopping,

- If the repetition factor R = 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

- If the repetition factor R > 1,

  - For cyclic shift hopping, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

  - For comb offset hopping, within a slot, the time-domain hopping behavior depends on one of the following alternatives:

    - Alt1: The OFDM symbol index l' of the first symbol across the R repetitions.

    - Alt2: The OFDM symbol index l' of each symbol.

    - Alt3: The OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on configuration, and FFS configuration details.

Agreement

For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details.

- UE can indicate whether it supports one or both the options. Details to be discussed in UE feature.

Agreement

For SRS comb offset hopping / cyclic shift hopping, support reinitialization at the beginning of every N radio frame(s), where N ≥ 1.

- FFS: N is fixed or configurable.

Agreement

Whether SRS comb offset hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

- FFS: Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

FFS: UE feature/capability design details.

Conclusion

No consensus on enhanced per-TRP power control and/or power control of one SRS towards to multiple TRPs in Rel-18.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows:

- If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset

- If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with comb KTC=2 and with maximum nSRScs,max=8 cyclic shifts per comb offset, the number of comb offset(s) and the cyclic shift locations are determined based on the one RRC configured cyclic shift location nSRScs as follows:

- If nSRScs<nSRScs,max/2, then 1 comb offset is used, otherwise 2 comb offsets are used.

- The 8 cyclic shift locations for the 8 ports are {nSRScs, (nSRScs+1) mod nSRScs,max, …,(nSRScs+7) mod nSRScs,max}, reusing the existing equation nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max in 38.211 6.4.1.4.2.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 or comb 8, decide one of the following options:

- Option 1: the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

  - For comb KTC=4, k=2. For comb KTC=8, k=4. For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max.

- Option 2: the cyclic shift positions are unaligned across the comb offsets on the same OFDM symbol for comb 4, and the cyclic shift positions are aligned on only 2 of the 4 comb offsets on the same OFDM symbol for comb 8.

  - For comb KTC=4, k=2. For comb KTC=8, k=4.  Example: For port pi, nSRScs,i=nSRScs+(pi-1000mod k)+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max. FFS equation details.

- FFS: potential impact on PAPR, if any.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, the s subsets of ports are mapped cyclically as {{1, 2, …, s}, …, {1, 2, …, s}} on the m OFDM symbols.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), the SRS transmissions within each of the m/s groups of {1, 2, …, s} use the same set of subcarriers. If consecutive groups of {1, 2, …, s} are configured as repetition, then the SRS transmissions of the consecutive groups use the same set of subcarriers.

- Note: applicable to the SRS resource with or without FH/RPFS.

- FFS the scenario where comb offset hopping is configured for the SRS resource.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, study at least the following solutions:

- Whether or not a UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage, coherency, and/or repetition configuration.

- Whether or not a UE changes the transmission order of the subsets of ports.