3GPP TSG-RAN WG1 #113	R1-2306114

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	9.5.5

Source:	Ericsson

Title:	Feature Lead summary #1 for Positioning for RedCap UEs

Document for:	Discussion, Decision

# Introduction

This document summarises the proposals received as part of Agenda Item 9.5.5 for RAN1#113 for the Rel-18 work item on expanded and improved NR positioning [1]. The objectives relevant for this agenda item are as follow:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

# General issues

## Switching Time and LS from RAN4

### Background

Multiple contribution highlight the need for allocating time for retuning prior to and after the UE performs frequency hopping[6][15][21][22], and confirming the RAN4 LS for retuning time between hops [18][19].  This was also a question from RAN4 in their LS[25]. Therefore, we should capture an agreement on the issue and use it to respond to RAN4.

In [2], additional values for retuning time are proposed:

-	210us, 500us for FR1

-	210us for FR2

From the FL perspective, we have now received a list of candidate values. RAN4 is already discussing further values during their may meeting. Therefore, to avoid the turnaround time incurring by sending an LS and waiting for a reply, it is preferable to discuss these new values directly in the RAN4 WG without having RAN1 sending an LS on the issue.

Company views are summarized in the table below:

| Company | Proposal |
| --- | --- |
| [2] | Proposal 7 - For RedCap positioning frequency hopping switching time, the following values should be additional supported. - 210us, 500us for FR1 - 210us for FR2 - Send an LS to RAN4 to confirm above values |
| [6] | Proposal 8 - Additional switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP is relevant for positioning SRS with frequency hopping and can be discussed in RAN4. |
| [15] | Proposal 2: Send a reply LS to RAN4 with the following response: - Yes, the additional switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP is relevant. - In this case, numerology, bandwidth, Tx/Rx antennas between the initial/active BWP and the SRS hop(s) can be different Proposal 7: For an SRS resource used for Positioning frequency hopping: - It is associated with a CC and includes a configuration of a numerology and bandwidth. - Introduce a transmission/switching/retune gap before the first hop of such an SRS resource and after the last hop, so that the UE can tune from the active BWP and back to the active BWP. - Up to RAN4 the details |
| [18] | Proposal 4: a time gap configuration should be considered between hops, FFS the candidate value and applicable condition. Proposal 5: FFS how to compensate the impacts on phase offset and/or amplitude change, from the time varying channel property. Proposal 6: From RAN1 perspective, the potential other UL signal transmission/DL reception might require additional time for UE to handle, especially with different SCS or TA; |
| [19] | Proposal 1 - The switching time between hops should be {70us, 140us} for FR1 and {35us, 70us, 140us} for FR2 from RAN1 perspective. - Final decision of the gap values may be up to RAN4. |
| [21] | The RAN1 discussion of UE capability indication for the supported minimal RF retuning time can start after receiving further confirmation from RAN4. RF retuning time should also be considered prior and after transmission of the PRS with Rx hopping. |
| [22] | Proposal 2-1: The additional switch time between BWP and the first hop, and between last hop and the return to BWP may be needed. It could reuse the agreed value and it is same as that between hops, and there is no need to be further discussed in RAN4 Proposal 2-2: The SRS transmission duration may consider the switch time between BWP and the first hop, and between last hop and the return to BWP |

### Further considerations on the switching time [medium]

#### Round 1

The following proposal was advanced in [2]. From the FL perspective, it is preferred to leave it to RAN4. Therefore, we start with the following question:

Question 2.1-1:	should additional values for the switching time for RedCap positioning frequency hopping switching time, the following be discussed within RAN1?

Companies are encouraged to comment on the question in the table below:

Question 2.1-1:

| Company | Comment |
| --- | --- |
| Qualcomm | We think RAN1 can say that it should be RAN4 to make the agreement, however, from RAN1 perspective, it appears that it is needed. We make the following proposal: - From RAN1 perspective, the additional switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP seems relevant. However, it should be RAN4 to make the decision. - RAN1 would like to inform RAN4 that, in this case, numerology, bandwidth, Tx/Rx antennas between the initial/active BWP and the SRS hop(s) may be different. |
| Huawei, HiSilicon | The additional values should be discussed in RAN4. |
| Futurewei | It is not clear to us what is the rationale behind additional values for frequency hopping switching time on top of the switching time values provided by RAN4 in the replied LS? |
| Ericsson | We also think that any additional value should be discussed by RAN4 directly. RAN4 does not need to get engaged in this discussion. |

### Response to RAN4 LS [separate discussion]

The response to RAN4 LS will be discussed in a separate document, as decided by the following:

Rel-18 Positioning R1-2304316 LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs RAN4, Ericsson RAN4 response to RAN1 LS in R1-2302127. In addition, RAN4 is requesting RAN1 input on whether additional switch time may be needed for SRS transmission between the initial/active BWP. Discussion on response LS to be handled in agenda item 9.5. To be moderated by Florent (Ericsson).

## Subcarrier Spacing [low]

In [8], it is proposed to consider the use of small SCS for UEs with low speed:

| Company | Proposal |
| --- | --- |
| [8] | Proposal 15: Consider using small SCS to increase the positioning performance for RedCap UEs with low speed at least. |

From the FL perspective, this is an implementation issue and should not be impacting the specification. Therefore, it is proposed not to pursue the proposal further.

Companies are encouraged to comment on the proposal in the table below:

| Company | Comment |
| --- | --- |
| Qualcomm | Implementation issue. |
| NEC | Thanks for the summary. The small SCS we mentioned is SCS smaller than the normal one defined in specification. For example, 7.5kHz for FR1. Based on those much smaller SCS, the energy per PRS RE increases due to longer time duration, then the positioning performance improves further. And if no other companies are interested on this topic, we can accept to close the discussion in this stage. |
| Ericsson | Do not support this proposal. We see this as a network implementation issue. |

## RACH [low]

In [18], it is proposed to study the impact of RACH on the PRS Rx hopping:

| Company | Proposal |
| --- | --- |
| [18] | Proposal 2: RAN1 to study the impact of RACH related signal to the PRS Rx reception FH. |

Since only 1 company has raised the issue above, let’s first collect some comments on the proposals to see the level of support. Not that the issue was also brought up during RAN1#112b-e but did not receive any comment.

Comments can be entered in the table below:

| Company | comment |
| --- | --- |
| Qualcomm | We consider it low priority and not clear what is needed to be done. |
| Ericsson | We prefer to deprioritize and focus on the main issues. |

## Positioning methods [low]

In [11][20] it is proposed to consider time and angle-based methods for positioning of redcap UEs. In [7], it is proposed to support carrier phase based positioning for a single hop.

| Company | Proposal |
| --- | --- |
| [11] | Proposal 4: Support both DL/UL timing and angle based positioning methods for hopping based positioning methods |
| [20] | Proposal 1: Prioritize timing-based positioning techniques such as TDoA, RTT for Redcap positioning. |
| [7] | Proposal 9: Only single hop CP measurement is reported. - FFS the first hop |

This topic was also brought up in the RAN1#112b-e and RAN1#112 meeting. From the FL perspective, the WID does not specify a particular method to be supported. From the FL perspective, the support of CPP for redcap UE based on a single hop is not different from the support for CPP from non-RedCap UEs. the UE will report which band / DL PRS bandwidth it supports for CPP and perform the measurement accordingly. For the other methods mentioned in [11] and [20], the discussion so far does not restrict any existing method to be used.  Therefore, it is proposed not to pursue the discussion on supported method further.

Companies are encouraged to comment on the proposal in the table below:

| Company | Comment |
| --- | --- |
| Qualcomm | We don’t see the need to discuss this further |
| Ericsson | Same view as Qualcomm. |

# Measurement reporting

## Background

During RAN1#112b-e, we reached the following agreements:

Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One [or more] measurements where each measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

Within the contributions, the proposals discuss further the following issues (note: all proposal apply to both TRP and UE measurements, unless stated otherwise)

- Measurement per hops:

  - Report only a single hop is proposed in [1][6][10][16] due to overhead concern

  - Per-hop measurement is reported separately from the measurement based on multiple hops[2][7][9][11][14][18][21][22]

    - In [7], the proposal is to report either the wideband measurement or the single hop measurement. In [9][11][22], the single hop measurement is a fallback measurement reported when the wideband measurement fails.

    - In [14][15], the per hop measurement can be requested explicitely

  - Support of per-hop indication [2][5][7][9][11][12][13][14][18][21]

    - Note:[2] suggest that the hop index is up to UE choice

  - For DL measuements, the measurement bandwidth per hop is reported as a UE capability [13][21]

- Wideband measurement:

  - Support of indicating which hop were used in the wideband measurement [7]

    - In [15] indication of which hop were used in not supported, but instead the use of the quality metric is suggest to convey that a more accurate measurement is being reported.

    - In [22] it is proposed to include the measurement bandwidth obtained from stitching multiple hops in the measurement report

  - Restriction to use only contiguous hops in the wideband measurement [12]

- Connection to Measurement Gap: if a gap is too short, the UE reports on the hops within the gap[8]

Company views are shown in the table below:

| Company | Proposal |
| --- | --- |
| [1] | Proposal 1: For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One [or more] measurements where each measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are report, and whether the above measurements can be reported together |
| [2] | Proposal 2 - For frequency hopping report, support the following - For ‘a single measurement based on receiving multiple hops’ and ‘one [or more] measurements where each measurement is associated with one received hop’, they cannot be reported together. - For ‘one [or more] measurements where each measurement is associated with one received hop’, support indication of hop index along with each measurement. - Note: different hop indices are not associated with actual hop locations, but are only used to distinguish each other - Note: the associated hop indices are up to UE implementation other than network configuration |
| [5] | Proposal 1: An indicator is needed to indicate how many received hops or which received hops were used in the measurement report. |
| [6] | Proposal 3 - More than one measurement where each measurement is associated with one received hop is not supported for DL PRS or UL SRS for positioning with frequency hopping. |
| [7] | Proposal 2: For DL Rx hopping, support UE to report either - A single measurement based on receiving multiple hops of the DL PRS for positioning - The frequency range or hop indications of the multiple hops should be reported as well - Or one measurement which is associated with one received hop - The frequency range or hop indication of the hop should be reported as well Proposal 7: For UL Tx hopping, support TRP to report either. - A single measurement based on received multiple hops of SRS. - The corresponding hop indications should be reported as well - One single measurement which is associated with one received hop. - The hop indication should be reported as well |
| [8] | Proposal 7: Whether all the frequency hops are included in a single measurement gap or multiple measurement gaps, is determined by the measurement gap configuration and the PRS processing capability of RedCap UE. |
| [9] | Proposal 7: Support both options on measurement report when using DL Rx hopping or UL Tx hopping (i.e., Option 1 and Option 2). One single measurement for multiple hops is used when UE has capability to perform bandwidth stitching, while measurement per hop is used as a fallback operation when UE has no/partial bandwidth stitching capability. |
| [10] | Proposal 1: For DL Rx hopping or UL Tx hopping, - Support to report one [or more] measurement that is associated with a single received hop. - No need to indicate how many / which hops are used to acquire the measurement |
| [11] | Proposal 5: Support per-hop measurement by modifying the agreement made in RAN1#121b-e as follows, “One [or more] measurements where each measurement is associated with one received hop” Proposal 6: If not all hops in a pattern are received, the UE or gNB reports one or more measurements where each measurement is associated with one received hop of DL PRS or UL SRS for positioning Proposal 7: If all hops in a pattern are received, the UE or gNB reports coherently combined measurements (e.g., single measurement based on receiving multiple hops of DL PRS or UL SRS for positioning) Proposal 8 : The UE includes the identify of received hop(s) in the measurement report |
| [12] | Proposal 11: Support the UE to indicate which received hop(s) are associated with a given measurement. Proposal 12: Restrict multi-hop measurements to the case where the hops are contiguous in the frequency domain. |
| [13] | Proposal 5: For RedCap positioning, the number of hops, measurement bandwidth per hop, and overlapping bandwidth are UE’s capabilities. Proposal 6: The hop index and the number of hops used to perform should be reported along with the measurement. |
| [14] | Proposal 7: A single measurement can be fed back in the case that some of the hops are not combined in the measurement. To assist in the measurement, an indication of which received hops were used in the measurement report can be sent. Proposal 8: For the measurement per hop, this can (a) be configured to be reported always with the single measurement, (b) be reported in a fall-back mode (e.g. if there is a failure in the single measurement), or (c) be reported by itself based on an explicit measurement request. To assist in the measurement, an indication of which received hops were measured and sent can be reported. |
| [15] | Proposal 5: For DL Rx hopping or UL Tx hopping: - Do not support an additional indication of how many received hops / which received hops where used in the measurement report. - Note: A UE may, up to UE implementation, to use the quality metric and the ReportingGranularityfactor to inform the LMF that a more accurate measurement is being reported. - Up to RAN4 to define any conditions on when the measurements are reported, what/if requirements are needed to be specified if there collisions or hops are being dropped. Proposal 6: Support an LMF to include an explicit request in the Location Request Signaling for a device to perform and report measurements according to DL PRS Rx frequency hopping. |
| [16] | Proposal 7-1: For UL SRS-pos Tx hopping, reporting the multiple measurements per hop from gNB is not supported. Proposal 7-2: For DL PRS Rx hopping, reporting the multiple measurements per hop from UE is not supported. |
| [18] | Proposal 3: support FH based measurement reporting and associated FH part index within one complete PRS. |
| [21] | In the measurement reporting for DL PRS frequency hopping, one measurement result is associated with the accumulated bandwidth information if the accumulated bandwidth is not aligned with the full bandwidth of configured PRS transmission, and one accumulated bandwidth has up to one measurement result. It is up to UE capability to report narrow-bandwidth-based measurement result. In the measurement reporting for UL SRS frequency hopping, one measurement result is associated with the accumulated bandwidth information (i.e. the number of hops in frequency domain) if the accumulated bandwidth is not aligned with the full bandwidth of configured SRS transmission, and one accumulated bandwidth has up to one measurement result. |
| [22] | Proposal 3-1: The measurement reporting based on either one received hop or multiple hops could be considered. To report together is not preferred Proposal 3-2: If UE doesn't support measurements by the stitched and larger bandwidth, it is up to UE implementation to determine a location of the measurement bandwidth with good RSRP to measure and report. It also means, the reporting could be based on a selected received hop Proposal 3-3: A hop with good RSRP would be changed with time due to mobility. Then to report the measurement with which hop and the corresponding location is less meaningful and it is not preferred Proposal 3-4: For the measurement reporting by using multiple consecutive hops, since the overlapping RB number is up to implementation, UE could report based on the measurement bandwidth after stitching, instead of reporting the number of consecutive hops |

## Report of a single hop for per-hop measurements [high]

### Round 1

For per-hop measurement, the majority of proposals in contributions want to focus on a single-hop being reported.

Proposal 3.2-1: For DL and UL measurements associated with one received hop, a single measurement, associated with one hop, is reported.

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.2-1:

| Company | Comment |
| --- | --- |
| vivo | OK |
| Qualcomm | We can accept the proposal |
| Huawei, HiSilicon | Support. A single measurement (instead of multiple measurements) reporting for one hop is sufficient. |
| LGE | Support the proposal. |
| Futurewei | Support |
| Nokia/NSB | Okay in principle. Question for clarification: is the intention that a UE will be able to support both one measurement based on multiple hops and one additional measurement on only one hop? (e.g., RSTD based on multiple hops plus RSTD based on a single hop) |
| NEC | Support. |
| Samsung | Fine with intention. One further question, whether which hop is the reported one should be also informed? |
| CATT | Support the single hop for per-hop measurements. |
| ZTE | Ok. |
| mtk | Similar question as Nokia. And we don't think UE should report both, the results by one hop and multiple hops |
| Spreadtrum | Support |
| SONY | Support |
| NTT DOCOMO | Support |
| Ericsson | Support. We think in that case the hop information (i.e. which part of the bandwidth is reported) is needed. |

### Status before first online (wednesday)

Based on the received comments, it seems we have a stable proposal. let’s try bringing it to the floor online:

Proposal 3.2-1: For DL and UL measurements associated with one received hop, a single measurement, associated with one hop, is reported.

## Report of single hop measurement separately from the wideband measurement [high]

### Round 1

Several contribution proposed to use per-hop measurements as fall back to the main measurement, but there are also proposal to have the per-hop measurement independent from the main measurement. For both cases, there are proposals to let the LMF deceide on whether a fall-back measurement is of interest, or to request a per-hop measurement.

Proposal 3.3-1: DL and UL measurements associated with one received hop are reported separately from the measurement based on multiple hops.

- Measurements associated with one received hop can be reported as a fall back when a measurement based on multiple hops fails, or independently of a measurement based on multiple hops

- The LMF indicates whether the UE or TRP should report the single-hop measurement as fall back to the multiple hop measurement in the measurement request. If no indication is present, the UE or TRP reports measurement failure.

- FFS: The LMF can separately request a measurement based on one received hop

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.3-1:

| Company | Comment |
| --- | --- |
| vivo | We prefer to remove the 2nd sub-bullet considering how UE selects which type of measurement report can be UE implementation, so there is no need for LMF to indicate. |
| Qualcomm | We don’t really think that either 1st or 2nd subbulet is really necessary. We prefer a simpler solution as following: - The LMF requests the UE to report either a single-hop measurement or a multiple hop measurement or both. |
| Huawei, HiSilicon | Not support. We think these two measurements (the measurements associated with one received hop, the measurement based on multiple hops) can be reported together, which is helpful for positioning calculation, e.g. LMF can decide to use which one or maybe both to obtain the positioning results. |
| LGE | UE/gNB can decide whether to report main single measurement based on receiving multiple hops or per-hop single measurement associated with one received hop according to hopping reception failure. We can’t find any clear reason for reporting both measurements together. So, we are only fine with main bullet, but we don’t think both of 1st and 2nd sub-bullet is necessary. |
| Futurewei | It is not clear what the benefits of reporting single hop measurements other than possibly as fallback purposes. It is not necessary to report single hop and multiple hop measurements separately because such separate reporting consumes additional time-frequency resources and incurs delays. As such, we propose the following wording update: Proposal 3.3-1: DL and UL measurements associated with one received hop are reported separately from the measurement based on multiple hops. - Measurements associated with one received hop can be reported as a fall back when a measurement based on multiple hops fails |
| Nokia/NSB | We don’t think the sub-bullets are needed. |
| Samsung | “fallback” definition will complicate the situation. Maybe main bullet is enough. |
| CATT | We prefer two kinds of measurement reporting should be supported: one is measurements associated with one received hop and another is measurement based on multiple hops. It seems that the sub-bullets are not necessary. |
| ZTE | We do not think UE need to report both measurements based on one received hop and measurements based on multiple hops. |
| mtk | 1, keep main sentence is enough |
| SONY | We are OK with the main bullet. However, in our understanding, the intention is not the allow the UE reporting both per-hop measurement and multihop measurement. |
| NTT DOCOMO | OK with the main sentence. We don’t think both measurements based on one received hop and measurements based on multiple hops are needed. |
| Ericsson | We agree with Futurewei that a only fallback mechanism is needed for single-hop measurements. for independent measurements, we can use legacy rel17 positioning. |
| IIT Kanpur, CEWiT | We are fine with the main sentence. We don’t think these sub-bullets are required. |

### Status before first online (wednesday)

From the received comments, the situation is as follow:

- the measurement for a single hop is a fallback measurements: LGE, Futurewei, Ericsson

- the measurement for a single hop is reported separately: Huawei, Nokia, Samsung, CATT, ZTE (maybe fallback?), Mediatek, Sony (maybe fallback),

  - for these companies, my interpretation is that they favor a fallback measurement:NTT Docomo, sony, ZTE

at least the first bullet is supported, i.e, to have a separate measurement report for the single hop measurement.  Maybe a way forward regarding fallback is to have the condition for the reporting of the single hop measurements as FFS.

Proposal 3.3-1: DL and UL measurements associated with one received hop are reported separately from the measurement based on multiple hops.

- FFS: Conditions for reporting the single hop measurement as fallback to the multi-hop measurement

## Hop bandwidth capability for per-hop measurement [medium]closed

### Round 1

One contribution proposes to report the hop bandwidth as a UE capability.

Proposal 3.4-1: For DL measurements, measurement bandwidth for per-hop reporting is a UE capability

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.4-1:

| Company | Comment |
| --- | --- |
| Qualcomm | We tend to prefer to treat these in the corresponding UE feature agendas. However, we can point ou there, that it is not just the per-hop BW that needs to be a capability but also (at least) the amount of overlap: - One or more Frequency domain overlap(s) between hops which are supported and reported by UE - RF Rx retune times between consecutive hops which is supported and reported by the UE |
| Huawei, HiSilicon | Up to RAN4. |
| Futurewei | Based on our understanding, the measurement bandwidth per hop is similar to the RedCap UE legacy bandwidth, e.g., 20 MHz. |
| CATT | We prefer to discuss this issue in UE feature AI 9.16.2. |
| ZTE | For each hop, the supported maximum PRS bandwidth can refer to the existing RedCap UE capability. |
| mtk | Per hop BW, in our view is not necessary to report |
| Ericsson | Can wait for the general UE capability discussion. In our view, the capability is whether it can do fallback measurements reports if the wideband measurement fails. |
| IIT Kanpur, CEWiT | We support the proposal. |
| FL | it seems we could leave this discussion to UE features. Let’s close the issue. |

## Hop indication in per-hop measurements [high]

### Round 1

Several contribution point to the need to indicate which hop is reported for per-hop measurement. However, how the hop is indicated is not clear yet. For example, in the DL the hopping pattern is up to the UE implementation. In the UL the hopping pattern will be defined, but it remains to be decided.

Proposal 3.5-1: For DL and UL measurements associated with one received hop, the measurement report includes an indication of the hop that was used to perform the measurement

- FFS: how the hop is indicated

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.5-1:

| Company | Comment |
| --- | --- |
| vivo | For DL measurement, there may not be hopping pattern configured and it may be up to UE implementation to perform Rx hopping. So we prefer to indicate the measurement type (ie., multiple hop measurement, or one hop measurement) other than detailed hop indication |
| Qualcomm | We don’t think it is necessary to report which hop is used. It is an unnecessary optimization from our side. We can accept that the UE reports that the measurement is “single-hop” or a “multi-hop” measurement (a topic that is treated in an earlier proposal). |
| Huawei, HiSilicon | The hop indication is not necessary, we may consider using frequency information developed in CPP. |
| LGE | Same view with Qualcomm |
| Futurewei | This proposal is related to Proposal 3.3-1. As there are no clear benefits of hop indication in reporting single hop measurements, such a hop indication may not be needed. If it needs to be reported, it should be low priority. |
| Nokia/NSB | We support indication of the hop used. If UE reports a single hop measurement how is it useful at the LMF without the hop ID? |
| NEC | Support. The hop information for measurement determining can be used to consider the reliability of the estimations. |
| Samsung | We think it’s needed for such hop index information to better understanding the reported results. If this information is missing, it will be problematic on how to understand such reported results. For HW’s proposal on using F information, we think it’s not necessary since CPP may have different range of F domain for phase measurement, but for redcap, we agree to use 20Mhz per hop. |
| CATT | Support. In our opinion, an indicator associated with measurement is needed to indicate how many received hops or which received hops are used in the measurement report. For example, in a measurement report, LMF receives two RTOA measurements reported by a TRP from UL SRS-Pos from a UE. The first RTOA measurement corresponds to 3 hops and the second RTOA measurement corresponds to 5 hops, where the RTOA measurement has higher measurement accuracy with more hops. The LMF can use the indicator to choose the second RTOA measurement with 5 hops to do UL-TDOA positioning, which help to achieve higher positioning accuracy. |
| ZTE | Support. From our side, UE can inform LMF which received hop is used in the measurement report. Then, LMF can know if the PRS assistance data and PRS request is suitable, if not, an update may be needed. |
| mtk | It is not needed actually. |
| SONY | Support. This information can be utilized by the LMF. |
| Ericsson | Support for the case of single-hop as fallback to multiple hop. If the single-hop measurement is a fallback measurement, then the LMF does not know what is the bandwidth measured, and hop information is needed. If the single hop measurement is independent from the multi-hop measurement, it can have its own assistance data and no further indication is needed. |
| IIT Kanpur, CEWiT | We are fine with the proposal. |

### Status before first online (wednesday)

From the received comments, there is as many companies for and against hop indication. I would like to check at the offline if a compromise can be reached. Maybe one way is to make the indication configurable in the measurement report configuration?

Proposal 3.5-1: For DL and UL measurements associated with one received hop, the measurement report can be configured includes an indication of the hop that was used to perform the measurement

- FFS: how the hop is indicated

## Hop indication in multiple hop measurements [high]

### Round 1

Several companies showed interest for indicating which of the hops were used in forming the mesurement based on multiple hops. As alternative to indicating the hops used, it was also proposed to report the measurement bandwidth.

Proposal 3.6-1: For DL and UL measurements associated with multiple received hop, the measurement report includes

- Option 1: an indication of the hops that was used to perform the measurement

  - FFS: how the hop is indicated

- Option 2: the measurement bandwidth

  - FFS: details of the measurement bandwidth

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.6-1:

| Company | Comment |
| --- | --- |
| vivo | Not supported, for the measurement of multi-hops, we don’t see any difference with R16/17 specification. |
| Qualcomm | Same view with vivo. We don’t think it is really necessary for a UE to provide information of which hops were measured or the bandwidth. In short, we believe it is enough to have the following 3 steps: Step 1: UE sends a UE capability that it supports this PRS Rx frequency hopping feature Step 2: LMF sends a request for the UE to do multiple hop measurement. Step 3: UE reports the multiple hop measurement with minimum reporting requirements, side conditions requirements, measurement period requirements, etc according to RAN4. In other words, a UE will be required to report a multiple hop measurement in all cases, up to UE capability & LMF’s request and RAN4 minimum requirements. If needed we can add in the LMF request that a UE is requested to measure X BWs or X hops. The case that some hops are dropped appear to us to be a corner case and we prefer not to have the UE to report IDs of the hops that were measured. - Up to RAN4 to define any conditions on when the measurements are reported, what/if requirements are needed to be specified if there collisions or hops are being dropped. |
| Huawei, HiSilicon | Neither. We don’t think it is helpful to know the hop indication or the measurement BW. |
| LGE | Same view with vivo |
| Nokia/NSB | Same comment as in 3.5.1 |
| NEC | We prefer option 1. Similar view as proposal 3.6-1, the actual hop information used for measurement determining can be used to determine the reliability of reported measurements. |
| Samsung | Option1 is preferred. First, we think the hop index reporting is needed. RAN4 may eventually just have requirement on single hop measurement for redcap case, it’s also beneficial to know how many and which hops are being reported in the. Second, the option2 seems not enough because only the size is reported. For example, if all 40mhz, but which 40mhz also matters. |
| CATT | We prefer Option 1. |
| ZTE | We support Option 1. |
| mtk | Don't support. These detailed information is useful to LMF? |
| SONY | We prefer option 1 since UE may not be able to perform measurement based on all the hops. In the case when UE is failed to measure all of the hops, it may be beneficial to indicate the subset of hops in which the UE performs measurements. |
| Ericsson | No strong preference in supporting hop indication, but we agree that option 1 is more meaningfull than option 2. |

### Round 2

From the received comments, there is as many companies for and against hop indication, and the option for indication is in majority option 1.  Let’s try and discuss it if there is time:

Proposal 3.6-2: For DL and UL measurements associated with multiple received hop, the measurement report includes

- Option 1: an indication of the hops that was used to perform the measurement

  - FFS: how the hop is indicated

- Option 2: the measurement bandwidth

  - FFS: details of the measurement bandwidth

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.6-2:

| Company | Comment |
| --- | --- |

## Measurements with partially overlapping measurement gap [medium]closed.

### Round 1

[8] proposed to consider the case where the measurement gap is too short to overlap with the whole DL PRS duration so that only a subset of the Rx hopping can be performed. From the FL perspective, this is an unlikely case, since the UE requests the measurement gaps. However, companies are encouraged to comment on the proposal:

Proposal 3.7-1: For DL and UL measurements associated with multiple received hop, when a measurement gap overlap partially with the measured DL PRS, the UE reports the DL measurement based on the hops received within the measurement gaps.

Companies are encouraged to comment on the proposal in the table below:

Proposal 3.7-1:

| Company |  | Comment |  |  |
| --- | --- | --- | --- | --- |
| vivo |  | Considering the DL measurement is up to implementation, we prefer up to RAN4 to discuss the issue if it is needed |  |  |
| Huawei, HiSilicon |  | From RAN1 perspective, we could say this functionality can be allowed, and it is up to RAN4 to determine whether/how to set the requirements. |  |  |
| Futurewei |  | The problem of measurement gaps partially overlapping with measured PRS can be overcome by selecting the right measurement gap size for the UE to complete DL PRS measurements over a wide frequency hopping bandwidth. Several measurement gap sizes can be defined, which depend on parameters, e.g., the number of hops, hop switching time and the number of symbols occupied by DL PRS for each hop. |  |  |
| CATT |  | It seems that this is the RAN4 issue. |  |  |
| mtk |  | 1, For this “corner” case, maybe this issue could leave to RAN4 to determine whether test case will consider such condition |
| Ericsson |  | Do not support. We don’t think this is supported for non-redcap UEs. |  |  |
| FL |  | We can deprioritize the issue. |  |  |

# DL-PRS Frequency Hopping

## PPW support [high]

### Background

During RAN1#112b-e, support of PPW for DL PRS Rx hopping was discussed, but an agreement was not reached, so that the FFS on PPW was left unresolved.

Based on the received proposals support of PPW is as follow:

- Not supported: 10 Companies ([2][3][4][6][9][10][14][16][19][21])

- Supported: 5 Companies ([5][11][12][15][18])

  - This includes different level of support, eg. Different types of PPW.

| Company | Proposal |
| --- | --- |
| [2] | Proposal 1 - For PRS Rx frequency hopping, PPW-based method is not supported. |
| [3] | Proposal 2: Only measurement gap based measurement for DL PRS frequency hopping should be considered. |
| [4] | Proposal 1: Do not further consider PPW-based/ MG-less scheme for PRS Rx frequency hopping for the target UE. |
| [5] | Proposal 6: For RedCap UE positioning with DL Rx frequency hopping, support the use of PPW, e.g., one/multiple PPWs based BWP frequency retuning method. Proposal 7: For RedCap UEs positioning in HD-FDD, collision handling rules for DL PRS and other UL signals/channels within PPW should be defined by the one of the following two methods. - Method 1: Reuse collision handling rule for DL PRS and other DL signals/channels in PPW in Rel-17, with new definition of PPW types (Type 3/4). - Method 2: Reuse the collision handling rule for other DL signals/channels and UL signals/channels for RedCap UE in HD-FDD. Proposal 8: Support a RedCap UE to use an on-demand method to provide the recommended PPW-related configuration information to the network (serving gNB/LMF). |
| [6] | Proposal 2 - For DL PRS with Rx frequency hopping for RedCap UEs, only MG-based measurement is supported. Proposal 7 - For HD-FDD RedCap UE, collision handling between DL PRS and UL channels/signals within a configured PPW needs to be addressed at least for the case where DL PRS is configured without Rx frequency hopping. |
| [9] | Proposal 8: Down-prioritize the frequency hopping operation within a PRS Processing Window (PPW) for RedCap UE. |
| [10] | Proposal 2: For RedCap UEs, the PPW-based DL PRS measurement with Rx frequency hopping should be treated as low priority. |
| [11] | Proposal 3: Support PRS processing window to receive PRS via Rx hopping |
| [12] | Proposal 3: RAN1 should support DL PRS frequency hopping outside MG for RedCap UE at least for the case of Type-1A PPW. Proposal 4: RAN1 should specify solutions to effectively support DL PRS frequency hopping within PPW configurations. |
| [14] | Proposal 2: Additional design details DL PRS Rx Hopping are as follows: • Time domain repetition may have to account for the hops across the bandwidth as a single repetition is over multiple hops. • The muting pattern may either mute a single hop or may mute a hop set. • The measurement gap may have to accommodate the duration of the hop set. This may be accommodated by a single large MG or multiple instances of a MG. • Gapless measurements based on the PRS Processing Window (PPW) should be lower priority. |
| [15] | Proposal 4: Support Rx frequency hopping for MG-less PRS processing only for PPW Type-1A using the following principle as a starting point: - A UE may perform Rx frequency hopping within a PPW instance under the condition that the required retune time before and after each hop is treated as part of the PRS duration in the specified PRS prioritization/collision rules. - If the retune time before or after a low-priority PRS hop collides with a high-priority channel, then the UE does not perform the Rx frequency hopping. - Note: No additional specification impact beyond UE capabilities is expected from RAN1 perspective. |
| [16] | Proposal 1: Deprioritize DL-PRS frequency hopping outside MG. |
| [18] | Proposal 1: PPW only with PRS as higher priority could to be used for PRS Rx FH. |
| [19] | Proposal 2 - RAN1 may not need to support PPW with FH. |
| [21] | PPW is not supported with DL PRS Rx hopping. |

### Round 1

RAN1#113 is the third meeting where PPW support is discussed.  There is a majority of contribution that does not want to support PPW, or at least down-prioritize it. Regarding down-prioritization, RAN1 only has two meetings left, so down-prioritization now means it will not be supported in rel-18. Thus we should make a decision on PPW support during this meeting.

Based on the majority of contribution, it is proposed not to support PPW:

Proposal 4.1-1: PPW is not supported with DL PRS Rx hopping in Rel-18.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.1-1:

| Company | Comment |
| --- | --- |
| vivo | Support |
| Qualcomm | Even though we don’t see the need to deprioritize Type-1A, we can accept it for progress and due to the time limitations in this release. |
| Huawei, HiSilicon | Support. |
| LGE | Support |
| Nokia/NSB | Don’t support. In our understanding the biggest argument against PPW support is that it is disruptive to data communication. However, MG is more disruptive as it does not give gNB ability to configure the UE for data around the DL PRS reception. The other argument is specification impact but for Type-1A as pointed out by many companies the impact is extremely small. |
| NEC | Support. |
| Samsung | Just technical question, what is the issue to support PPW when PRS is high priority? |
| CATT | We prefer to support PPW with Type 1A which only has very small specification impact. |
| ZTE | Support. |
| mtk | support |
| Spreadtrum | Support |
| SONY | Support |
| NTT DOCOMO | Support |
| Ericsson | OK. |

### Status before first online (wednesday)

We are basically at the same point as we were in the previous meeting. Some companies see merit in supporting PPW, and some see it is as not worth supporting.

We have now two meetings left to complete the WI and I don’t see the situation changing, therefore my proposal is to conclude that the proposal is not pursued further.

Proposal 4.1-1 (for conclusion): PPW is not supported with DL PRS Rx hopping in Rel-18.

## Measurement gap configuration [high]

### Background

The issue of a single or multiple measurement gap was discussed in previous meetings. In this meeting, the majority of companies propose to support a single measurement gap[3] [5] [9] [15], and two contributions [10][15] propose to leave it to RAN4.  In [15] it is proposed to leave to RAN4 to decide the duration of the MG and of the number of hops (Rx retunings) covered by the MG.

| Company | Proposal |
| --- | --- |
| [3] | Proposal 3: UE should only apply a single instance of a measurement gap to complete one PRS frequency hopping reception. |
| [5] | Proposal 5: For DL PRS Rx hopping using measurement gap(s), a single measurement gap should be configured to overlap with at least 1 DL PRS hopping sequence. - Send an LS to RAN4 to check if there is any issue to support the above enhancement. |
| [9] | Proposal 9: For RedCap UE positioning, only support the use of a single instance of MG for DL PRS with Rx frequency hopping. |
| [10] | Proposal 3: It is up to RAN4 to define RRM requirements for UE performing DL PRS Rx frequency hopping using a measurement gap. • A LS to RAN4 can be sent. |
| [15] | Proposal 3: The UE is expected to perform up to N Rx Retunings during a single MG instance in order to measure multiple frequency parts of a single PRS resource Up to RAN4 to decide the maximum number of Rx Retuninings within a single MG instance |
| [21] | - Proposal: Gap duration for PRS Rx hopping can be handled by RAN4. |

### Round 1

Based on the received proposal we can discuss the use of a single measurement gap. RAN4 should be informed of the agreement.

Proposal 4.2-1: For DL PRS Rx hopping, support the use of a single instance of a measurement gap over all the hops for DL PRS with Rx frequency hopping.

- Send an LS to RAN4 including the agreement.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.2-1

| Company | comment |
| --- | --- |
| vivo | OK |
| Qualcomm | We support this feature. RAN4 is really waiting for RAN1 progress, see below the RAN4 agreements. RAN1 need to provide more guidance to let RAN4 do their work. Issue 2-3-1: When to start PRS measurements for RedCap with FH? Agreements: - RAN4 to start work on PRS requirements for RedCap with frequency hopping after RAN1 has made agreements on frequency hopping for RedCap. Issue 2-3-8: UE capability related to FH: Agreements: - Wait for RAN1 agreements and input on the UE capability related to FH |
| Huawei, HiSilicon | RAN1 could conclude it is up to RAN4. |
| LGE | Fine with proposal |
| Futurewei | Ok. In addition, we support measurement gaps with different lengths to accommodate different hop switching time values and the number of hops. |
| Nokia/NSB | We don’t support this proposal if PPW is not supported as it is technically the same for the UE to do both. |
| NEC | Support. |
| Samsung | Just question, “a single instance of a measurement gap” means one measurement gap, or the time span within the MG which can covers a complete FH? So that in MG there could be multiple instance. |
| CATT | Support. |
| ZTE | OK. |
| mtk | ok |
| Spreadtrum | Support |
| SONY | Support |
| NTT DOCOMO | Support |
| Ericsson | OK |
| IIT Kanpur, CEWiT | Support |

### Status before first online (wednesday)

There is quite a large majority supporting the proposal. Let’s try to get the proposal online for agreement

Proposal 4.2-1: For DL PRS Rx hopping, support the use of a single instance of a measurement gap over all the hops for DL PRS with Rx frequency hopping.

- Send an LS to RAN4 including the agreement.

## Further details on Configuration of Rx hopping

### Background

In [8][3], it is propose to further specify how repetitions are used to implement Rx hopping.

In other contributions it is propose to introduce parameters for the time between hops[7] ,  the overlap between hops [9][13]. In [14] it is propose to consider extending the existing parameters for repetitions in the time domain.

| Company | Proposal |
| --- | --- |
| [3] | Proposal 1: UE perform frequency hopping reception between different PRS transmission repetitions within one period of one PRS resource can be considered. |
| [7] | Proposal 1: With regards to Rx frequency hopping for positioning for RedCap UE: - UE reports a capability on the maximum number of supporting frequency hops to network, the candidates at least include {2, 3, 4, 5} - UE reports a capability on the overlapping PRB(s) between adjacent hops or a capability on the maximum equivalent bandwidth after combing all hops Proposal 10: The parameters are recommended for higher layer. - maximum number of supported frequency hops [1,2, ..., 5] - overlapping PRB(s) between adjacent hops or a capability on the maximum equivalent bandwidth after combing all hops [0,1,2, 4] - UE capability on support of on-demand PRS with Rx hopping [Yes/No] |
| [8] | Proposal 1: Repetition scheme for PRS can be considered to realize DL PRS frequency hopping in Rx. Proposal 2: Support an additional indication to indicate which repetitions will be received by UE as a frequency hopping pattern. |
| [9] | Proposal 1: Support frequency hopping with partial overlap with two adjacent frequency bands to compensate the performance loss due to the phase offset. Proposal 2: Introduce two higher layer parameters for configuration of hops, Tgap (the time gap between two hops) and F_ovl (the overlap resources in frequency domain), to facilitate bandwidth stitching in the frequency hopping operation. Proposal 3: These parameters (T_gap and F_ovl) should be configured for each measurement occasion or semi-consistent for multiple occasions. |
| [13] | Proposal 2: For UE-assisted Redcap positioning, bandwidth overlap between the frequency hops should be supported. |
| [14] | Proposal 1: For DL PRS Rx Hopping, the UE hops within a DL PRS resource. The specification impact includes the following: - Frequency domain: no change - Time domain: increase number of repetitions to enable mapping over BW. This may need an update to parameters like the L-PRS-ResourceRepetitionFactor, DL-PRS-ResourceTimeGap and the DL-PRS-Periodicity. Proposal 3: On the overlap between hops and number of hops, the hopping configuration needs to identify at least the following: - the number of hops, - the bandwidth of each hop, - the amount of overlap between hops. These are dependent on the UE capability. |
| [20] | Proposal 3: Specify the timing gap resulting from the PRS reception of different frequency hops. |
| [15] | Proposal 1: For DL-PRS Rx frequency hopping, support performing Rx hopping with overlapping tones and with non-overlapping tones. - Support a UE to report a per-band capability for the amount of overlap required for DL PRS frequency hopping - Note: RAN1 assumes that no additional UE requirements shall be specified for the case of Rx hopping with non-overlapping tones; e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops. - Send an LS to RAN4 with the above agreement |

From the received proposals, it seems at least either the amount of overlap between hops and the hopping bandwidth, or alternatively, total accumulated bandwidth across hops, should be reported as a UE capability in order for the network to form the assistance data (for example, the overall bandwidth of the DL PRS depends on the total bandwidth across hops).

Regarding the time gap between hops, it could  be reported to secure that repetitions are not too close from each other so allow the UE to perform retuning.

Regarding identifying the repetition used for Rx hopping, it should be noted that all repetitions from the network perspective are identical. The UE can use the repetition to either perform Rx beam sweeping or accumulate more power, based on its implementation. Thus the benefit of identifying the repetition involved in the Rx hopping is unclear.

### Capability reporting for the bandwidth across all hops [high]closed.

#### Round 1

Proposal 4.3.2-1: To report the UE capability for the DL PRS bandwidth across all hops,  the UE reports as a capability the hop bandwidth and the overlap between hops.

Companies are encouraged to comment on the proposal in the table below:

| Company | comment |
| --- | --- |
| vivo | From the RAN1 perspective, supporting the DL PRS measurement with overlap and without overlap between hops is enough, other parts can be up to RAN4 to define. |
| Qualcomm | We tend to prefer to treat the UE capabilities in the corresponding agenda, unless we have finished with all important remaining issues already in this agenda. However, we don’t think that this is the case yet. We prefer to first agree with the principle (similar to what vivo says), and then yes, we obviously support the UE to report capabilities for this feature. Example is shown below: For DL-PRS Rx frequency hopping, support performing Rx hopping with overlapping tones and with non-overlapping tones. - Support a UE to report a per-band capability for the amount of overlap required for DL PRS frequency hopping - Note: RAN1 assumes that no additional UE requirements shall be specified for the case of Rx hopping with non-overlapping tones; e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops. - Send an LS to RAN4 with the above agreement With regards to the UE capabilities we think all four components below are needed: - Maximum PRS BW per hop which is supported and reported by UE - One or more Frequency domain overlap(s) between hops which are supported and reported by UE - RF Rx retune times between consecutive hops which is supported and reported by the UE - Maximum DL PRS bandwidth across all hops |
| Huawei, HiSilicon | This capability should be introduced by RAN4. |
| Nokia/NSB | UE feature discussion, don’t support discussion here. |
| NEC | We suggest to report maximum overlapped bandwidth as UE’s capability. Because the actual overlapped bandwidth can be different for different scenarios with different positioning requirements, and different with different channel conditions. In addition, to make a more accuracy configuration on overlapped bandwidth, phase error should be reported as one kind of capability too. |
| CATT | We prefer to discuss this UE capability in RAN4. |
| ZTE | Ok to discuss in UE capability agenda. |
| mtk | Overlapping between hops is UE implementation, and we don't think UE should disclose this |
| Spreadtrum | Support |
| SONY | OK to discuss this topic in UE capability / feature discussion |
| NTT DOCOMO | Ok to discuss in UE capability agenda. |
| Ericsson | Ok to move to UE features. We also agree with Qualcomm’s proposal to capture that the measurement with rx hopping may or may not include overlap. |
| FL | We can close this issue and move it to UE features. |

### Capability reporting for the time gap between hops [high]closed

#### Round 1

Proposal 4.3.3-1: for DL PRS Rx hopping, the time gap between Rx retunings is reported by the UE as a capability.

Companies are encouraged to comment on the proposal in the table below:

| Company | comment |
| --- | --- |
| Qualcomm | We support it, but overall we think all 4 components are needed: - Maximum PRS BW per hop which is supported and reported by UE - One or more Frequency domain overlap(s) between hops which are supported and reported by UE - RF Rx retune times between consecutive hops which is supported and reported by the UE - Maximum DL PRS bandwidth across all hops |
| Huawei, HiSilicon | This capability should be covered by RAN4. |
| Nokia/NSB | UE feature discussion, don’t support discussion here. |
| CATT | We prefer to discuss this UE capability in RAN4. |
| ZTE | Up to RAN4 |
| Spreadtrum | Support |
| SONY | Support and we can discuss further in UE capability / feature discussion |
| NTT DOCOMO | Up to RAN4 |
| Ericsson | Same view as above |
| FL | We can close this issue and move it to UE features. |

### Identification of repetitions used for Rx hopping [low]

#### Round 1

Proposal 4.3.4-1: for DL PRS Rx hopping, the network indicates which of the repetitions are used for Rx hopping

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.3.4-1

| Company | comment |
| --- | --- |
| Qualcomm | We want to discuss this a bit more and we are not convinced yet it is needed. Why not the LMF indicating how many hops the UE should do? And let the UE decide where/how to do the hops? At the end of the day, the UE will have to meet the RAN4 requirements. E.g., - Step 1: UE says it is capable of processing 100 MHz PRS with 20 MHz hop in each one and Y PRBs overlap. RAN1 can say that the UE is expected to do the hops within a signle MG instance. - Step 2: RAn4 will say that, for such case, the UE needs to do X hops (by formulating a clear measurement period formula). - Step 3: LMF/gNB will know how long should the MG be, and how many repetitions should the PRS have (following the RAN4 minimum requrirements) by looking at UE capabilities and how these translate to minimum measurement period requirements in RAN4. Based on the above, we don’t see the need for an LMF to request the UE to perform Rx hopping on specification repetitions. It should be enough to ask the UE on the “total BW it should be measured”, or “number of hops to perform”. Furthermore, we think that the feature of LMF requesting a UE to perform measurements on a specific instance / PRS resources (discussed in the CPP agenda), will also be applicable for any other positioning feature, including this one. Then, an LMF will be able to ask the UE to perform measurements in a given PRS instance. |
| Huawei, HiSilicon | Not support. |
| NEC | Based on Rx frequency hopping of PRS for RedCap UE, repetitions of PRS are shared between eMBB UEs and RedCap UEs. When the number of repetitions is larger than the number of hops needed, then UE can select one group of combination with highest accuracy via implementation. However, the complexity to find the best one increases with the increased repetitions. If the best combination can be indicated by gNB based on some other information, then the complexity and accuracy of measurements are ensured simultaneously. |
| Ericsson | Do not support. We agree with qualcomm’s analysis regarding the way the measurement request will be formulated. Additionally, To answer to NEC, we do not think there is a “best repetition instance” from the network perspective. A priori, the network has configured the DL PRS with repetitions to support all the UEs receiving the same PRS. All repetitions are supposed to be usable coherently to accumulate power, so the network assumes the channel is static over all repetitions. Hence there is no “best repetition” from the LMF/gNB side. |

## Number of hops [high]closed.

### Background

In [7] [9] [20], it is proposed that the UE reports the maximum number of hops used in Rx hopping. Such capability could be used by the network to configure the number of DL PRS repetitions.

| Company | Proposal |
| --- | --- |
| [7] | Proposal 1: With regards to Rx frequency hopping for positioning for RedCap UE: - UE reports a capability on the maximum number of supporting frequency hops to network, the candidates at least include {2, 3, 4, 5} - UE reports a capability on the overlapping PRB(s) between adjacent hops or a capability on the maximum equivalent bandwidth after combing all hops |
| [9] | Proposal 5: Support the UE capability parameter to reflect the supported frequency hopping operation for NR RedCap UE. (i.e, by considering the RedCap UE constraints / limitations). |
| [20] | Proposal 4: Specify the configuration and related capabilities associated to the number of Rx hops to combine and number of DL-PRS samples across different hops to be stored for coherent Rx combining to achieve wideband DL-PRS measurement for RedCap devices. |

### Round 1

Proposal 4.4-1: for DL PRS Rx hopping, the UE reports a capability on the maximum number of hops it supports.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.4-1

| Company | comment |
| --- | --- |
| vivo | We wonder about the motivation and benefit of reporting the capability to LMF. |
| Qualcomm | We think it is important to add this UE feature, or the “maximum BW it can process”. We have some preference on the “maximum BW”: With regards to the UE capabilities we think all four components below are needed: - Maximum PRS BW per hop which is supported and reported by UE - One or more Frequency domain overlap(s) between hops which are supported and reported by UE - RF Rx retune times between consecutive hops which is supported and reported by the UE - Maximum DL PRS bandwidth across all hops |
| Huawei, HiSilicon | RAN1 could conclude it being up to RAN4. |
| Futurewei | It may not be necessary to report the maximum number of hops as capability because it depends on the maximum bandwidth per hop and the amount of overlapping frequency between two hops. |
| Nokia/NSB | UE feature discussion, don’t support discussion here. |
| NEC | Support. |
| CATT | We prefer RAN4 discuss this issue. |
| ZTE | Support |
| mtk | Up to RAN4 |
| Spreadtrum | Support |
| SONY | Support. The higher number of supported hops then the higher number of UE complexity / processing. |
| NTT DOCOMO | Up to RAN4 |
| Ericsson | We think this capability is relevant fort he network to configure the repetitions in a PRS resource. However we can wait fort he UE features discussion. |
| IIT Kanpur, CEWiT | Support |
| FL | Let’s move this discussion to UE features |

## Further assistance data to support Rx hopping [medium]

### Background

In [7] it is propose to include the hopping parameter in the assistance data. In [8] it is proposed to make the bandwidth overlap configurable for Rx Hopping.

| Company | Proposal |
| --- | --- |
| [7] | Proposal 3: For PRS reception with hopping, one or more of the following parameters in assistance data or location request should be introduced to facilitate UE’s PRS measurement. - Number of hops - Number of overlapped RB between hops - Total bandwidth of all hops |
| [8] | Proposal 3: For frequency hopping of PRS or SRS, support configurable overlapped bandwidth between two adjacent hops to address the influence caused by phase offset between hops. |

### Round 1

We can start by checking the support to introduce Rx hopping parameters in assistance data for DL PRS Rx hopping

Proposal 4.5-1: for DL PRS Rx hopping, the LMF sends the number of hops, overlap between hops and total bandwidth of all hops as part of the assistance data to the UE

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.5-1

| Company | comment |
| --- | --- |
| vivo | Not support. For ‘total bandwidth of all hops’, it is the same as the current PRS bandwidth. For ‘the number of hops’ and ‘overlap’, we don’t think they are needed, as it is up to UE implementation for Rx hopping. |
| Qualcomm | We are generally supportive to have the LMF to ask for a “total BW to be measured” or “number of hops”. However, we don’t think it should be part of the assistance data, rather the location request. |
| Huawei, HiSilicon | Not support. We don’t think it is necessary. |
| CATT | We failed to see the benefits of such assistance data from LMF to UE. |
| ZTE | Support |
| Ericsson | Do not support. The LMF will send the repetition factor in assistance data. The rest is up to the UE implementation. |

## Intra-slot hopping [medium]

### Background

Since rel16, the repetition framework of the DL PRS only support inter-slot hopping. In [10][21], intra slot hopping is proposed. [10] proposes to support it by configuring DL PRS patterns with repetition exceeding comb size, while [21] proposes to introduce intra-slot hopping parameters:

| Company | Proposal |
| --- | --- |
| [10] | Proposal 4: Both intra-slot and inter-slot DL PRS Rx frequency hopping can be supported. - No RAN1 specification enhancement is required. |
| [21] | Proposal 2 Support intra-slot (symbol) level repetitions, with a new intra-slot repetition factor and intra-slot resource time gap using symbol resolution. Potential values for resource time gap for intra-slot hopping includes 2, 4, 8 symbols. Potential values for intra-slot repetition include 1, 2, 3, 4. |

### Round 1

Since the proposal in [10] does not have specification impact, we can start by checking the support to introduce intra-slot hopping based on [21]:

Proposal 4.6-1: for DL PRS Rx hopping, intra-slot hopping is supported by a new intra-slot repetition factor and intra slot resource time gap

- FFS details on values for repetition factor and time gap.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.6-1

| Company | comment |
| --- | --- |
| Huawei, HiSilicon | Up to RAN4. |
| Ericsson | Support. RAN4 should be informed of the agreement. |

## DL PRS Processing [medium]

### Background

The DL processing capability is expressed since rel16 as the number of DL PRS resources N the UE is able to process in a time window T at a given maximum bandwidth B. in [9][14][20][22], it is proposed to discuss the capability of DL PRS processing in the context of RedCap UEs and DL Rx hopping

| Company | Proposal |
| --- | --- |
| [9] | Proposal 6: Support the RedCap UE’s processing time for Rx frequency hopping as part of the UE capability. |
| [14] | Proposal 6: A reply from the RAN4 LS is needed to update the existing sets of values for the UE DL PRS processing capability. |
| [20] | Proposal 5: Support Redcap PRS processing capabilities corresponding to different (N,T) values with reduced bandwidths e.g., 20MHz for FR1 and 100MHz for FR2 including a reduced Rx antenna/RF chain of a single antenna. |
| [22] | Proposal 4-1: The processing capability may depend on DL-PRS pattern, the RF switch time of UE and the processing strategy. As such, the processing capability could be determined and reported after the DL-PRS configuration is sent to the UE Proposal 4-2: The measurement capability could be defined as the measurement bandwidth per resource per TRP, and the total number of resource number times the TRP number within a time window |

### Round 1

The proposals do not mention a specific new set of values for DL Processing capability. Thus we start by checking whether new capability should be defined at all, and we will discuss further details later:

Proposal 4.7-1: for DL PRS Rx hopping, a new processing capability for processing of DL PRS with Rx hopping is defined.

- FFS: details of the new capability regarding number of resources, maximum bandwidth and time window.

Companies are encouraged to comment on the proposal in the table below:

Proposal 4.7-1

| Company | Comment |
| --- | --- |
| Qualcomm | We can be supportive of the feature. |
| Huawei, HiSilicon | Why cannot the existing capability be used for this? |
| Ericsson | OK to discuss. |

## PRS Tx hopping

In [6][9], it is proposed to re-consider the use of Tx hopping for the DL PRS:

| Company | Proposal |
| --- | --- |
| [6] | Proposal 1 - For DL PRS for RedCap UEs, frequency hopping pattern for the reception of DL PRS across different subbands is defined. - gNB may choose between the option of transmitting a single common DL PRS that may be received by RedCap and non-RedCap UEs and the option of transmitting DL PRS for RedCap UEs separate from that for non-RedCap UEs. |
| [9] | Proposal 10: Support frequency hopping is configurable across multiple DL PRS resources or resource-sets. |

Since the issue was resolved during RAN1#112 with the following agreement, it is proposed not to consider the issue further.

Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

Companies may leave their comment in the table below:

| Company | comment |
| --- | --- |
| Qualcomm | We cannot revisit this now. It is too late for this release. |
| Ericsson | We should not reopen this issue. |

## On-demand PRS [low]

### Background

In [7], it is proposed to include a UE capability for support of on-demand PRS with Rx hopping.

| Company | Proposal |
| --- | --- |
| [7] | Proposal 10: The parameters are recommended for higher layer. - maximum number of supported frequency hops [1,2, ..., 5] - overlapping PRB(s) between adjacent hops or a capability on the maximum equivalent bandwidth after combing all hops [0,1,2, 4] - UE capability on support of on-demand PRS with Rx hopping [Yes/No] |

### Round 1

We can start by discussing whether a new capability for on-demand PRS specifically for DL PRS with Rx hopping is needed.

Proposal 4.9-1: a new UE capability on support of on-demand PRS with Rx hopping is introduced.

- FFS: details of the capability

| Company | comment |
| --- | --- |
| Qualcomm | We support it |

## PRS Muting enhancements [low]

### Background

In [8], enhancements for the DL PRS muting framework to account for the use of Rx hopping are proposed to be studied:

| Company | Proposal |
| --- | --- |
| [8] | Proposal 8: For NR RedCap UEs, study the muting mechanism for frequency hopping sub-bands. |

### Round 1

The issue was brought up during RAN1#112b-e, but no feedback except from the proponent was received. Muting was not discussed during the SI phase, and no recommendation to extend the muting framework was captured. Therefore, we propose to restart the discussion again for this meeting with checking the support for the proposal above.

Comments can be entered in the table below:

| Company | comment |
| --- | --- |

# UL SRS Tx Hopping

## SRS Tx Hopping Pattern [high]

### Background

In RAN1#112b-e, we agreed to support the SRS tx hopping within one resource. In this meeting, many companies submitted proposals as to how to configure frequency hopping for the SRS.

We can see 3 different solution types presented in the contributions:

Solution 1:

Some companies focus on a so-called “staircase pattern”, which has hops overlapping consecutively as in figure X.  this is proposed by [1][3][6][7]

- In[3] the stair case pattern is controlled by a repetition factor wich controls the number of hops-.

- In [6], the stair case pattern is controlled by

  - In time: the starting symbol of the first hop, number of symbols per hop and switching period between hops.

  - In frequency: the starting frequency allocation for the first hop, overlap and hop bandwidth for each hop.

Solution 2:

Other companies instead propose to instead reuse the SRS mimo frequency hopping equation, with some adaptations[2][15], using the existing parameters for frequency hopping, and introduce a parameter to control the amount of overlap.

- In [2], an update to the SRS bandwidth configuration table, is proposed, with additional rows taylors for Tx hopping.

- Additionally, intra-slot FH is proposed to be supported with a new symbol-level time gap between hops in [2][15].

- The number of hop is proposed to be computed based on configured total number of symbols across all hops, repetition factor and the retuning time capability of the UE in [15], noting that there is already a configured number of symbols and repetition factor in RRC.

Solution 3:

Finally, contributions in [4][5][14][16][21] propose to have a configurable hopping pattern using a combination

- Starting symbol for the first hop and Time domain offset between hops

  - In [16], one option is to use slot based hopping and each hop starts in the same symbol position but in a different slot

- Starting frequency allocation and Frequency domain offset between time-consecutive hops

- Number of hops

In [1], it is clarified that the hopping pattern should fit within the channel bandwidth, excluding the channel edges’ guard bands.

In [12], it is proposed to ensure that the overlap covers both the upper and the lower part of the hop in frequency domain. While a gap between allocation of different UEs in different part of the spectrum of the same symbol is introduced, in order to make sure that as many UE can be multiplexed without interfering with each other. From the FL perspective this could be seen as an optimization of the parameters to configure the staircase pattern, that could be achieved by implementation using solution 3.

| Company | Proposal |
| --- | --- |
| [1] | Proposal 3: For UL SRS Tx hopping, support the following: - the staircase hopping pattern (as a starting point); and - FFS the staggered hopping pattern Proposal 4: For UL SRS frequency hopping with partial overlapping, support - excluding the guard bands on either side of the channel edge from the overlapping bandwidth determination - the overlapping bandwidth is a configurable parameter |
| [2] | Proposal 3 - For SRS for positioning frequency hopping within one SRS resource, support to reuse the framework of MIMO SRS frequency hopping as a starting point. Proposal 4 - For frequency domain design of SRS for positioning frequency hopping, support the following. - Reuse following parameters and mechanism for MIMO SRS frequency hopping - freqDomainShift: nshift - freqHopping:CSRS, BSRS and bhop - freqDomainPosition: nRRC - Frequency hopping enable mechanism: ‘bhop<BSRS’, e.g., bhop=0, BSRS=1 - Introduce ‘virtual UL BWP’ for hopping, similar to ‘SRS only BWP’ in RRC_INACTIVE state - Slightly update rows of Table ‘SRS bandwidth configuration’ in TS38.211, considering number of hops, hop bandwidth, frequency hopping total bandwidth, overlapping bandwidth, e.g., Table 6.4.1.4.3-1: SRS bandwidth configuration. …; …; …; …; …; …; …; …; …. 63; 272; 1; 16; 17; 8; 2; 4; 2. …; …; …; …; …; …; …; …; …. 64; A1; 1; B1; C1; /; /; /; /. 65; A2; 1; B2; C2; /; /; /; /. …; …; …; …; …; …; …; …; …. - Slightly update frequency hopping offset function in TS38.211 to subtract the effect of overlapping bandwidth, e.g., noffsetFH=b=0BSRS(mSRS,b-overlap_BW)NscRBnb Proposal 5 - For intra-slot time domain design of SRS for positioning frequency hopping, support the following: - Reuse following parameters and mechanism for MIMO SRS frequency hopping - The quantity of OFDM symbol number within a slot for an SRS resource: l'∈0,1,…,NsymbSRS-1 - Table of the koffsetl' (the table of relative RE offset) - Repetition factor R - Introduce symbol-level ‘time gap’ within an SRS resource - Update the existing concept of NsymbSRS for ‘concecutive OFDM symbols’, considering symbol-level ‘time gap’ - Update symbol position within a slot for an SRS resource: l'+l0, considering symbol-level ‘time gap’ - For inter-slot time domain design of SRS for positioning frequency hopping, support the following: - Introduce inter-slot repetition within an SRS resource - Update the function of the quantity nSRS considering inter-slot repetition within an SRS resource |
| [3] | Proposal 4: For SRS frequency hopping within one SRS resource, introducing the SRS repetition mechanism, UE achieve SRS frequency hopping between different SRS repetitions with one SRS resource. Proposal 6: Diagonal frequency hopping pattern should be supported and the overlapping hops are adjacent in the time domain. Proposal 9: For positioning for RedCap UEs, the number of hops for DL PRS Rx frequency hopping is a UE capability. |
| [4] | Proposal 5: For the enhancement of pos-SRS configuration to achieve overlapped or non-overlapped Tx frequency hopping within one SRS resource, support introducing the following parameters: - Time domain offsets between adjacent hops - Frequency domain offsets between (time-domain) adjacent hops - Number of hops for the pos-SRS resource - Note: The overlapping hops should be adjacent in the time domain for a single SRS hopping pattern. |
| [5] | Proposal 2: Both the diagonal hopping pattern of SRS-Pos and the hopping pattern of SRS-MIMO can be supported. Proposal 3: UL Tx frequency hopping pattern information should include the following SRS-Pos resource configuration information in time domain and frequency domain. - In time domain: - the start time of the frequency hopping pattern - the end time of the frequency hopping pattern - time gap between two consecutive hops (in unit of slot or OFDM symbol) - In frequency domain: - partial overlapping size(granularity: PRB or RE) - starting PRB index per frequency hopping - the number of frequency hopping N,N subject to UE capability |
| [6] | Proposal 5 - For SRS for positioning with frequency hopping for RedCap UEs, - At least a staircase-like frequency hopping pattern is supported. - In the time domain, the starting symbol of the first hop, number of symbols for SRS transmission and switching period between adjacent hops can be configured. - In the frequency domain, the starting PRB of the first hop, number of overlapping PRBs, size of subband for each hop can be configured. |
| [7] | Proposal 5: To improve positioning performance of SRS with hopping, overlapped PRBs are supported for two adjacent hops. - The hopping order in time is { nhop, nhop+1, …, nhop+N-1}mod N, where nhop is the start hop index and N is the number of hops Proposal 6: For RedCap UE with positioning SRS, the number of overlapping PRBs in adjacent hops is based on the bandwidth of each hop - Larger bandwidth of each hop corresponds to more overlapped PRBs, the candidates of PRBs can be {1, 2, 4} |
| [12] | Proposal 9: Support configuration of UL FH patterns which take advantage of both the top and bottom PRBs in the overall 100 MHz as shown in Figure 3. |
| [13] | Proposal 1: The effects of the time and phase offset and the UE speed should be considered while designing the hopping patterns for the RedCap UE Observation 2: The effects of the phase offset between the frequency hops should be mitigated to get the gains of the frequency hopping operation. Proposal 2: For UE-assisted Redcap positioning, bandwidth overlap between the frequency hops should be supported. Proposal 3: The exact pattern of frequency hops should not be restricted. |
| [14] | Proposal 4: For the UL SRS Tx hopping pattern, overlapping hops do not have to be adjacent in the time domain as long as they occur within a duration of the channel not changing. |
| [15] | Proposal 8: Downselect between the following alternatives for the SRS frequency hopping pattern: - Alt. 1: Use the current hopping formula of SRS as a starting point and update it such that there can be frequency domain overlap over 2 hops that are adjacent in frequency. - Example of the update in the frequency-domain starting position k0(pi) : k0(pi)=k0(pi)+b=0BSRS(KTCMsc,bSRSnb -Moverlap⋅nb⋅b'=b+1BSRSNb') - Alt.2 2: Use a staircase-like hopping formula with a parameter that controls the amount of overlap of frequency domain adjacent hops. Proposal 10: Support intra-slot SRS hopping and inter-slot hopping in X consecutive available slots for periodic, semi-persistent, aperiodic SRS for positioning. - Support at least Ninterslot={2} slots - FFS: Additional Ninterslot values - Introduce a new RRC parameter on the number of inter-slot repetitions - FFS: Definition of “available” slot for SRS frequency hopping Proposal 11: The nominal number of hops in a slot (Nh) is computed as follows: Nh= Ns+NRFNRF+R, where - NRF : UE capability for the number of symbols gap needed between hops - Ns : Configured total number of symbols of an SRS resource (nrofSymbols) in a slot. - Note: This configuration already exists in RRC. - R: Number of OFDM symbols within each hop, configured through the parameter repetitionFactor . - Note: This configuration already exists in RRC. |
| [16] | - The number of symbols of SRS-pos resource is used to determine the number of symbols of each hop Proposal 3: For the intra-slot + inter-slot SRS-pos Tx hopping mechanism: - Select one of the methods for resource mapping scheme for preventing the hop from crossing over the slot boundary - Alt 1: By limiting the start position according to the symbol length of each hop and switching gap between consecutive hops. - Alt 2: By repeating the time-domain hopping pattern of the first slot in other slots Proposal 4: SRS for positioning frequency hopping configured with frequency hopping configuration ID which is pre-defined via RRC should be supported. |
| [21] | Proposal 6 Introduce a new frequency domain offset to support multiplexing of SRS frequency hopping and its value range can be from 0 to [14]. Proposal 7 Introduce a new repetition number to indicate the number of SRS frequency hopping where each hop has one symbol in one SRS frequency hopping, or to indicate the number of symbols in one SRS hop, if partially overlapped SRS frequency hopping is configured. |
| [22] | Proposal 5-1: The SRS hopping pattern could be up to NW configuration except that, the duration between end of a hop and the beginning of next hop should be longer than RF switch time reported by a UE |

### Round 1

Before going into a detailed solution, we should start by identifying which of the 3 solution we should implement,

- A Staircase pattern

- Re-use the mimo FH equation with an update to account for overlap and retuning time

- A comfigurable pattern, not limited to the staircase pattern.

The details on how granular the configuration is (e.g. the starting symbol for each hop, inter-slot or intra slot hopping, etc) can be discussed afterward. The ambition is that we should continue discussing the details during this meeting, i.e. not downselect later.

Proposal 5.1-1: For the SRS Tx hopping pattern configuration support (select one)

- A Staircase pattern, where each hop follows the preceeding hop with an overlap in frequency

- Re-use the mimo FH equation with an update to account for overlap and retuning time

- A comfigurable pattern, not limited to the staircase pattern, where each hop overlap with another hop in the pattern, but the overlapping hops do not need to be adjacent in time.

Companies are encouraged to comment on the proposal  in the table below:

Proposal 5.1-1:

| Company | Comment |
| --- | --- |
| vivo | Support reusing MIMO SRS frequency hopping framework as a starting point. No matter which solution is selected, it all ends up in the specification of mapping between the SRS symbol and SRS subband, and MIMO SRS frequency hopping gives a basic design, we just need to update it a little bit which can minimize the spec impact. The design of frequency hopping includes the determination of subband of hop, counting number of hop and T/F hop mapping pattern. Based on the framework, even if staircase pattern is supported, it is easy to update the equation ‘F(nsrs)’ to adapt. |
| Qualcomm | We prefer to re-use the MIMO FH equation as a starting point instead of starting from scratch from a new pattern. It is significantly simpler from implementation standpoint to reuse the F(nsrs) that already exists by juts doing a few changes as/if needed. |
| Huawei, HiSilicon | We support to use the stair-case pattern; at least it should be the baseline considered in the evaluation, e.g. CFO, clock drift etc. |
| LGE | We prefer 1st bullet. By reusing some of legacy configuration parameters, e.g. start position in time/frequency domain, we can simply configure the staircase frequency pattern with the configuration parameters indicating the number of hops and overlap size which should be configured for SRS frequency hopping. From the perspective of spec impact, we are also fine with 2nd bullet. We would like to notify that our intention in our proposal about “the intra-slot + inter-slot SRS-pos Tx hopping mechanism” was to configure ‘staircase’ pattern which can be included in 1st bullet. For the clarification, we suggest to add the note: Note: The overlap between adjacent hops in frequency hopping may or may not be configured. |
| Futurewei | We understand there are pros and cons associated with the staircase pattern and reusing SRS MIMO. Regarding the staircase pattern, it is not feasible to obtain a complete staircase pattern with each hop follows the preceding hop with an overlap in frequency. This depends on the starting frequency location of the first hop. Regarding reusing the SRS MIMO, it contains a lot of features which are not needed by SRS Tx frequency hopping, which requires simplifications. |
| Nokia/NSB | Support in principle but we wonder if the first sub-bullet is needed given the last sub-bullet. |
| NEC | We prefer option 3, it’s more flexible, and option 1 and option 2 can be seen as specific cases of option 3. |
| Samsung | The first one and third one is ok for us. |
| CATT | We prefer to reuse the hopping pattern of SRS-MIMO as a starting point. Maybe some updates accounting for overlap and retuning time are needed. |
| ZTE | We prefer a staircase pattern. |
| Spreadtrum | We prefer a staircase pattern. |
| NTT DOCOMO | We prefer a staircase pattern. |
| Ericsson | We think the flexibility of the second and third options are the same, and we could choose between them. We have a preference for the third bullet option due to the possibility to control overlap between SRS transmitted by different UEs. |
| IIT Kanpur, CEWiT | We prefer option 3. |

### First offline (Tuesday)

Status:

- Option 1: Staircase pattern: ZTE, Spreadtrum, NTT Docomo, LGE, Huawei, Samsung, apple, intel, idg

- Option 2: Mimo SRS equation: CATT, Qualcomm, vivo, CMCC

- Option 3: Configurable hopping: Samsung, NEC, Nokia, apple, IIT, CEWiT, Ericsson

It seems this would benefit from more discussion during the offline:

Proposal 5.1-1: For the SRS Tx hopping pattern configuration support (select one)

- A Staircase pattern, where each hop follows the preceeding hop with an overlap in frequency

- Re-use the mimo FH equation with an update to account for overlap and retuning time

  - FFS: any required updates

- A configurable pattern, not limited to the staircase pattern, where each hop may overlap with another hop in the pattern, but the overlapping hops do not need to be adjacent in time.

### Status before first online (wednesday)

During the offline, we heard further opinions and looking at the count, it seems that option 1 has a majority.  Proponents of option 1 see it as too limiting, while on the other hand proponents of option 1 think that there are too many possible patterns in option 3.

Option 2, to re-use the MIMO SRS equation with some changes, had the least amount of support. Since it is clear that option 1 can be realized in option 3, I would like to check if a compromise can be reached to endorse option 3, but discuss further exactly which patterns should be specified. That is to say, it should be common understanding that a fixed list of pattern will be in the specification in some way.

Proposal 5.1-2: For the SRS Tx hopping pattern configuration support

- A configurable pattern, not limited to the staircase pattern, where each hop may overlap with another hop in the pattern, but the overlapping hops do not need to be adjacent in time.

  - FFS: restrictions / rules to for valid hopping patterns

## Collision rules [high]

### Background

Collision was discussed last meeting and two option were agreed for further discussion:

Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, study whether to support one or both of the following options, according to UE capabilities: - Option 1: UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels - FFS: details on the collision rules

In this meeting, the contributions show that companies have interest for both of the options:

- Option 1: UL time window [5][6][8][12][14][16][18][19][22]

  - Not supported by [11]

- Option 2: Additional collision rules: [2][3][4][7][8][10][11][15][16][22]

Regarding the UL time window, [5][8] proposes to specify a periodic time window, similar to DL measurement gap. [8] discuss whether the

Regarding the additional dropping rules, the following is proposed:

- [2][8] propose to discuss whether to drop all the SRS hops, only the affected hop, or the affected symbols

- [4] proposes to add a condition for dropping the SRS to come back to the active BWP. If the turnaround time to and from the active BWP is larger than the time between hops, the UE does not return to the active BWP

- [7] proposes to treat the SRS with low priority when outside of the active BWP, similar to SRS in RRC inactive in release 17.

- [10],[15] [21] propose to re-use formulation from SRS carrier switching to drop the SRS with Tx hopping if the UE receives notification of UL dynamic scheduling with enough advance. [15] proposes to drop only the colliding hop.

| Company | Proposal |
| --- | --- |
| [2] | Proposal 9 - For SRS for positioning frequency hopping collides with other DL/UL reception/transmission, Option 2 is supported with the following aspects. - The dropping rules should include the following based on different scenarios and UE capabilities - Alt 1: UE drops all the SRS hops - Alt 2: UE drops affected hops - Alt 3: UE drops affected symbols - The other DL/UL reception/transmission should include other UL signals/channels, DL signals/channels in TDD, DL signals/channels for half-duplex UE (HD-UE) in FDD |
| [3] | Proposal 7: When one or more SRS frequency hops collide with other UL and DL signals/channels in once SRS transmission, additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels need to be defined. |
| [4] | Proposal 6: Consider introducing the following collision rule between SRS and other UL&DL signals/channels. • To ensure the transmission of two adjacent SRS hops not collide with other signals/channels in the active UL BWP, if the sum of the retuning time to the active UL BWP after the first hop, and the retuning time from the active UL BWP prior to the second hop is larger than the configured time domain offsets, UE is not required to switch to the active UL BWP; otherwise UE is required to switch to the active UL BWP. |
| [5] | Proposal 4: Support the following parameters for UL time window indication from gNB to UE (Option 1), • UL time window ID(Indicates the pre-configured ID for UL time window configuration) • starting slot/symbol • Periodicity • Duration(indicates the length of DL-PRS processing window) |
| [6] | Proposal 6 • For collision handling of positioning SRS with frequency hopping, at least Option 1 (UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning) is supported. |
| [7] | Proposal 8: For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, - If the SRS hop is within the initial BWP, the existing dropping rule for RRC_CONNECTED state is reused. - If the SRS hop is outside the initial BWP, Rel-17 defined rule for SRS outside initial BWP in RRC_INACTIVE state is reused, i.e. SRS has low priority. |
| [8] | Proposal 10: Definition of DL measurement gap can be taken as a start point for UL time window definition, which includes period, offset and length at least. Proposal 11: Support same period configuration for DL measurement gap and UL time window for positioning, and the period can be the indicated one or two times of the indicated one when both DL measurement gap and UL time window are configured or activated. Proposal 12: Two level offsets can be considered to decrease the overhead of UL time window, where one offset is a legacy one in terms of subframe, and another one is in unit of slot with length in unit of slots simultaneously. Proposal 13: Several dropping alternatives can be considered when dropping positioning SRS with frequency hopping is needed, which are: Alt-1: Drop the collided symbols within the collided hops, the rest symbols within the collided hop and the rest hops are still transmitted. Alt-2: Drop the collided hop, the rest hops are still transmitted. Alt-3: Drop the collided hop and the rest hops after the collided hop. |
| [10] | Proposal 8: For RedCap UEs positioning transmitting the UL SRS with frequency hopping, support to define additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels: - If SRS resources for Tx frequency hopping collides with a Type 1 configured grant of UL/DL channels/signals, UE drops SRS resources for Tx frequency hopping; - If a DCI scheduling a dynamic grant or Type 2 configured grant of UL/DL channels/signals arrives before T ahead of the first symbol of SRS resources for Tx frequency hopping, and collides with SRS resources for Tx frequency hopping, UE drops SRS resources for Tx frequency hopping; otherwise, UE drops the schedule UL/DL channels/signals; - T is a preparation time for transmission of SRS resources for Tx frequency hopping and switching time. |
| [11] | Proposal 9: Do not support Option 1 “UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning” Proposal 10: Support Option 2 “Additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels” Proposal 11: Adopt collision rules between higher-priority UL channel/signal transmission and gap between SRS for positioning hops |
| [12] | Proposal 8: Support Option 1: an UL time window where the UE is not expected to receive/transmit other signals/channels and is only expecting to transmit FH SRS for positioning. |
| [14] | Proposal 9: RAN1 should support the UL time window concept for UL SS with frequency hopping to account for possible collisions between other UL and DL signals/channels and the UL SRS with frequency hopping. Proposal 10: An UL Hopping SRS instance collides with DL/UL signal/channel if any portion of the other DL/UL signal/channel overlaps with the time interval starting X symbols before the transmission and ending Y symbols after the transmission. Note that X and Y may be different for UL and DL signals and channels. Proposal 11: It is also necessary to discuss if the window spans a single hop or spans the entire hop sequence. This could be chosen based on a UE capability. The UL time window can be configured based on higher level signaling. Observation: If the UL time window spans the entire hop sequence and no additional collision rules are defined then, at least for SRS measurement, there is no need to discuss the per-hop or single measurement issue. Proposal 12: The use of any additional collision rules should be based on a UE capability. |
| [15] | Proposal 13: For the purpose of SRS frequency hopping, support additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels where the rules defined in the SRS carrier switching framework are used as starting point (Section 6.2.1.3 of 38.214). - An SRS hop is dropped whenever one or more symbols of the hop, including any interruption due to uplink RF retuning time collides with a higher priority channel |
| [16] | Proposal 6: For collision handling rule, support both following options: - Opt. 1) UL time domain window where UE is expected to transmit only SRS for positioning - Opt. 2) Hop level dropping with priority rule between SRS for positioning and other UL transmission |
| [18] | Proposal 7: the option 1 with a configured UL time window to prioritize SRS transmission is supported. |
| [19] | Proposal 3 - RAN1 should support UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning. |
| [21] | For RedCap UEs positioning transmitting the UL SRS with frequency hopping, a UL time window is supported. - The uplink time window starts Nstart symbols ahead of the first SRS symbol in the first hop and ends Nend symbols after the last SRS symbol in the last hop. - When the UE is scheduled with a dynamic PUSCH within the UL time window so that the UE receives a DCI scheduling PUSCH N2 symbols ahead of the UL time window or is scheduled to transmit a PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI, or aperiodic CSI within the window, the UE drops the SRS for positioning with Tx hopping transmission. - Otherwise, the UE drops the transmission of other signals/channels and transmits only the SRS for positioning. |
| [22] | Proposal 5-2: Support both options for collision issue during SRS hopping. Proposal 5-3: The collision rule may consider the dropping of SRS hopping in a per slot basis, if the time window is across slots |

### Round 1

Since there is clearly interest for both options, let’s see first if supporting both of the two studied option is agreeable, or if downselection is necessary:

Proposal 5.2-1:  For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping,  support both of the following options, according to UE capabilities:

- Option 1: UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning.

  - FFS details of an UL time window

  - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning

- Option 2: additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels

  - FFS: details on the collision rules

Comments can be entered in the table below:

Proposal 5.2-1

| Company | comment |
| --- | --- |
| vivo | We have concerns about the UL time window that UE only expects to perform Tx frequency hopping with high priority. Firstly, in R16/17, no matter SRS within BWP or outside BWP, SRS is always treated as low priority and dropped, and we don’t find any difference for frequency hopping. Then, during different hops, there may be other transmission/receptions interspersed, e.g., in TDD spectrum, so the time window only for SRS may not be applied. In addition, even in RRC_INACTIVE, the collision rules applies to SRS and corresponding switching time without the introduction of UL time window, so we don’t find the necessity for frequency hopping. |
| Qualcomm | We don’t see the need to support both options. We prefer to have collision rules as it is the case in legacy. |
| Huawei, HiSilicon | We think a down-selection should be made. Option 2 should be supported since Option 1 may have a huge impact on data transmission/reception. |
| LGE | We support both options as each options can be used in different cases. As can be seen from our evaluation results, dropping multiple slots can cause critical performance degradation. So, in the case that positioning accuracy has to be guaranteed, UL SRS frequency hopping without dropping should be supported to achieve a desired positioning performance. Otherwise, UE can apply the collision rules to UL SRS-pos resource outside UL time window. |
| Nokia/NSB | We still feel that Option 1 is the simplest option but we could accept both options as a compromise. |
| NEC | If both options are agreed, then how to combine the two options should be discussed further. |
| Samsung | We prefer option 1 only, because basically it already solve the priority debating. The argument by vivo is exactly what we are afraid to and mentioned in last meeting. In legacy, if you drop one SRS seems fine, but now a complete SRS will take 5 time occasion to transmit, and one drop will bring huge impact of the whole reception of all potential other SRS hops. That’s why, SRS hops should be somehow prioritized than before. |
| CATT | We prefer Option 1, and can live with supporting both options. |
| ZTE | We have similar understanding as Qualcomm. We shall reuse the legacy collision rule. |
| mtk | Support both |
| Spreadtrum | We prefer Option 2. |
| NTT DOCOMO | We prefer Option 1, which is simpler option than Option 2. |
| Ericsson | We prefer option 2. the collision rules for SRS with FH could be similar to those for SRS carrier switching. |
| IIT Kanpur, CEWiT | We prefer option 2. |

### Round 2

Status:

- Ok with both options: MTK, CATT (or opt1), NEC, Nokia (or opt1),LGE

- No to both options

  - Prefer option 1: Docomo, CATT (or both), Samsung, Nokia (or both),

  - Prefer option 2: Ericsson, Spreadtrum, ZTE, Huawei,Qualcomm, vivo

It seems that the compromise is to support both options, since at least some of the companies supporting option 1 are open to support both.

The proposal is repeated below, to see if this can be agreeable:

Proposal 5.2-1 (no changes):  For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options, according to UE capabilities:

- Option 1: UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning.

  - FFS details of an UL time window

  - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning

- Option 2: additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels

  - FFS: details on the collision rules

Comments can be entered in the table below:

Proposal 5.2-1

| Company | comment |
| --- | --- |

## Configuration outside the active BWP [medium]

### Background

In [4][10][15], it is propose to configure the SRS with Tx hopping outside of the active BWP, using either a virtual BWP, or a SRS configuration separate from the active UL BWP and associated with a CC.

| Company | Proposal |
| --- | --- |
| [4] | Proposal 3: Support SRS transmission outside the active UL BWP to support SRS Tx hopping based positioning of RedCap UEs in RRC_CONNECTED state. |
| [10] | Proposal 5: For RedCap UEs, consider the following enhancement on BWP configuration to support SRS for positioning frequency hopping: - Define a virtual UL BWP which is outside of RedCap UE active BWP limitation and covers all SRS for positioning hops; |
| [15] | Proposal 7: For an SRS resource used for Positioning frequency hopping: - It is associated with a CC and includes a configuration of a numerology and bandwidth. - Introduce a transmission/switching/retune gap before the first hop of such an SRS resource and after the last hop, so that the UE can tune from the active BWP and back to the active BWP. - Up to RAN4 the details |

### Round 1

There is consensus in the proposals that the configuration for SRS tx hopping should be outside of the active UL BWP. We can check whether further details can be included in the agreement:

Proposal 5.3-1: The SRS for positioning resources with Tx Hopping are configured in a configuration separate from the UL active BWP configuration.

- The configuration consists of

  - Option1: a virtual BWP containing the SRS configuration.

  - Option 2: a SRS resource configuration with an associated CC, numerology, and bandwidth.

Comments can be entered in the table below:

Proposal 5.3-1

| Company | comment |
| --- | --- |
| vivo | Option 1. A virtual BWP configuration in RRC_INACTIVE can be reused. |
| Qualcomm | We are worried that Option 1 might contradict the previous agreement: Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources Can we just say that the SRS resource configuration has its own numerology & bandwidth similar to what we said for RRC Inactive SRS in Rle-17? What we are worried with the “virtual BWP configuration” is that it might appear as if it is an actual BWP which is not the case. Eventually, the RRC editor might decide to create a virtual BWP, however, from agreement standpoint, it will be clear that we are not following the BWP framework (which entails many more things that we expliclty said that we are not going to copy over). The related agreement from Rel-17 was: Agreement: - For RRC_INACTIVE UEs, SRS for positioning bandwidth, SCS and CP type are configured by RRC and can be different from that of initial UL BWP configured by the system information What if we say the following: - For RRC_CONNECTED Redcap UEs, for the SRS for positioning resources with Tx Hopping, SCS, CP type, bandwidth are configured by RRC and can be different from that of the UL BWP configured to the UE. |
| Huawei, HiSilicon | Support Option 1 in principle. OK with the wording provided by QC, which in fact is the meaning of “virtual” to our understanding. |
| CATT | OK with the wording from Qualcomm |
| ZTE | Actually, we don’t think there is any need to introduce an IE regarding virtual BWP (e.g. virtual-BWP-config). We prefer Option2. |

### Round 2

We can use the Qualcomm proposal, which was supported by 2 further companies, to continue the discussion:

Proposal 5.3-2: For RRC_CONNECTED Redcap UEs, for the SRS for positioning resources with Tx Hopping, SCS, CP type, bandwidth are configured by RRC and can be different from that of the UL BWP configured to the UE.

Comments can be entered in the table below:

Proposal 5.3-2

| Company | comment |
| --- | --- |

## Configuration of the Bandwidth overlap between hops [high]

### Background

In [4], it is proposed to extend the support of DL and UL measurements with FH to the RRC_INACTIVE mode:

| Company | Proposal |
| --- | --- |
| [1] | values) to cover 100 MHz (FR1) and 400 MHz (FR2) bandwidth. Proposal 2: For Tx frequency hopping configured within one positioning SRS resource, support the following configurable parameters as a baseline: - Hop switching time which can be any of this set of values: {70 µs, 140 µs} for FR1 and {35 µs, 70 µs, 140 µs} for FR2 - Hopping bandwidth which can be any of this set of values: for instance, {5 MHz, 10 MHz, 15 MHz, 20 MHz} for FR1 and {50 MHz, 100 MHz} for FR2 - Short Zadoff-Chu sequence with length up to the hopping bandwidth or long Zadoff-Chu sequence with length up to the total (or wide) frequency-hopping bandwidth |
| [2] | Proposal 8 - For the sizes of overlapping bandwidth for different hops, the balance between phase error compensation performance and bandwidth span of frequency hopping should be considered. - A size smaller than 8 PRBs can be considered |
| [3] | Proposal 8: The number of overlapping frequency resources in adjacent hops required for the UE to perform phase offset compensation the overlap between hop for DL PRS Rx frequency hopping is a UE capability. |
| [8] | Proposal 3: For frequency hopping of PRS or SRS, support configurable overlapped bandwidth between two adjacent hops to address the influence caused by phase offset between hops. Proposal 6: For frequency hopping of PRS or SRS, support size of overlapped bandwidth between two adjacent hops is decided by channel quality. |
| [10] | Proposal 6: To support RS frequency hopping, the partial overlapping in the frequency domain should be considered to mitigate the phase discontinuity between different hops. Proposal 7: To support RS frequency hopping, at least consider the following additional parameters on top of that supported for Rel-15 SRS resources: - Number of overlapped PRB or PRB offset between two adjacent hops - The switching time gap between two adjacent hops |
| [11] | Proposal 1: The amount of overlapped bandwidth is associated with gap between two consecutive frequency hops for UL frequency hopping and Rx frequency hopping |
| [12] | Proposal 7: For the SRS frequency hopping, the SRS configuration independent with the existing UL BWP supports at least with the following configuration parameters - Starting RB index and the number of RBs of the SRS resource, - Starting slot, the number of slots and/or the number of symbols - Periodicity and offset - SRS sequence ID - Number of frequency hops, - Gap time between frequency hops, - Number of RBs overlapped between frequency hops |
| [16] | Proposal 2-1: Adopt following new parameters are included in SRS-pos frequency hopping configuration - Number of hops - Overlap size which can be set to zero Proposal 2-2: Following parameters in legacy SRS-pos resource configuration can be reused to SRS-pos frequency hopping configuration - Bandwidth of SRS-pos resource is used to determine the bandwidth of each hop - Starting position of SRS-pos resource in time/frequency domain is used to determine the Starting position in time/frequency domain for the first hop - The number of symbols of SRS-pos resource is used to determine the number of symbols of each hop |
| 13 | Proposal 4: For RedCap positioning, the overlap of bandwidth between the adjacent hops is X PRB. The possible values of X are 1,2,4,8 PRB. |
| [15] | Proposal 9: Support in the specification multiple overlap options between 2 frequency-adjacent hops with a minimum value of 1 PRB and a minimum granularity of 1 PRB. |
| [14] | Proposal 3: On the overlap between hops and number of hops, the hopping configuration needs to identify at least the following: - the number of hops, - the bandwidth of each hop, - the amount of overlap between hops. These are dependent on the UE capability. Proposal 5 : To support SRS for positioning frequency hopping, the hopping configuration needs to identify at least the following: - the number of hops, - the bandwidth of each hop, - the amount of overlap between hops. - The time between hops - For the overlap configuration of SRS Tx hopping include the starting PRB for each hop These are dependent on the UE capability. |
| [21] | Proposal 5 SRS for positioning Tx bandwidth hopping is supported for RedCap UEs, by extending the SRS configuration with at least i. The total BW to be covered over all hops ii. The gap (in symbols) between two adjacent hops iii. The overlap between two adjacent hops in frequency domain |

### Round 1

For the configuration of the SRS for positioning with Tx hopping, many parameters depends on how the Tx hopping pattern is built, either using an equation similar to the one for SRS Mimo FH, or a different configuration. For all solution, it seems at least necessary to introduce a parameter for the overlap between hops and based on the received proposal, the minimum overlap can be zero, i.e. no overlap:

Proposal 5.4-1: SRS Tx Frequency hopping is configured with an overlap between hops.

- Support in the specification multiple overlap options between 2 frequency-adjacent hops with a minimum value of 0 PRB and a minimum granularity of 1 PRB.

- FFS: The maximum value of the overlap

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.4-1:

| Company | Comment |
| --- | --- |
| vivo | Generally, support. In our view, a note may be need: Note: the overlap between hops can be configured explicitly or implicitly. For example, when we configure hop number, hop bandwidth, and total number of hops, the overlap bandwidth can be obtained by calculation. |
| Qualcomm | Support |
| Huawei, HiSilicon | 1 RB granularity may be contradictory to the existing SRS BW indication granularity. |
| LGE | Support |
| Futurewei | Support the following overlapping values between 2 adjacent frequency hops: 0 PRB, 1 PRB and FFS other values. The overlapping PRB(s) should exclude the guard bands on either side of the RedCap UE channel edge. Based on the above, we propose to update wording as follows: Proposal 5.4-1: SRS Tx Frequency hopping is configured with an overlap between hops. - Support in the specification multiple overlap options between 2 frequency-adjacent hops with 0 PRB, 1 PRB and a minimum granularity of 1 PRB. - FFS: other values of overlap Note: The overlapping PRB(s) should exclude the guard bands on either side of the RedCap UE channel edge. |
| Nokia/NSB | Okay |
| CATT | OK with the proposal |
| ZTE | Generally ok. |
| mtk | ok |
| Spreadtrum | Support |
| NTT DOCOMO | OK |
| Ericsson | OK |
| IIT Kanpur, CEWiT | Support |

### Round 2

Most companies are ok with the proposal. there are three comments to resolve:

- The overlap may be implicitely configured instead of explicitely with a parameter (vivo)

- Granularity of the overlap with 1 PRB is not supported for SRS for positioning, which has a granularity of 1 PRB (Huawei)

- Support 0 or 1 PRB overlap, and other values are FFS (Futurewei)

Note that from the RRC configuration, the starting position of the SRS has a resolution of 1 PRB. The SRS bandwidth resolution is 4 PRB.

We can continue the discussion using Futurewei’s proposal. a note reflective the comment from vivo is added:

Proposal 5.4-2: SRS Tx Frequency hopping is configured with an overlap between hops.

- Support in the specification multiple overlap options between 2 frequency-adjacent hops with 0 PRB, 1 PRB and a minimum granularity of 1 PRB.

- FFS: other values of overlap

Note: The overlap between hop may be explicitely (i.e. via a parameter), or implicitely configured

Note: The overlapping PRB(s) should exclude the guard bands on either side of the RedCap UE channel edge.

Companies are encouraged to comment on the proposal  in the table below:

Proposal 5.4-2:

| Company | Comment |
| --- | --- |

## Support of SRS Tx hopping in RRC_INACTIVE state [medium]

### Background

In [4], it is proposed to extend the support of DL and UL measurements with FH to the RRC_INACTIVE mode:

| Company | Proposal |
| --- | --- |
| [4] | Proposal 4: Define a virtual BWP beyond maximum RedCap UE bandwidth to achieve SRS Tx frequency hopping for RedCap UEs in RRC_INACTIVE or RRC_CONNECTED state. |

### Round 1

Proposal 5.5-1: SRS Tx Frequency hopping is supported for both RRC_CONNECTED and RRC_INACTIVE state.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.5-1:

| Company | Comment |
| --- | --- |
| vivo | Typo: remove a for before both RRC_CONNECTED FL: fixed, thanks! |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |
| Ericsson | OK |

### Status before first online (wednesday)

The proposal seems acceptable so let’s try to get it agreed Wednesday if time allows:

(stable)

Proposal 5.5-1: SRS Tx Frequency hopping is supported for both RRC_CONNECTED and RRC_INACTIVE state.

## Sequence design [medium]

### Background

The transmitted sequence in a given SRS resource with FH is discussed in [1][12]. In [1] it is propose to have the sequence span a single hop, while in [12] it is instead proposed to a sequence spanning all hops (where each hop thus transmit part of the sequence) and let the UE request the network to configure a sequence satisfying a certain PAPR level

| Company | Proposal |
| --- | --- |
| [1] | Proposal 2: For Tx frequency hopping configured within one positioning SRS resource, support the following configurable parameters as a baseline: - Hop switching time which can be any of this set of values: {70 µs, 140 µs} for FR1 and {35 µs, 70 µs, 140 µs} for FR2 - Hopping bandwidth which can be any of this set of values: for instance, {5 MHz, 10 MHz, 15 MHz, 20 MHz} for FR1 and {50 MHz, 100 MHz} for FR2 - Short Zadoff-Chu sequence with length up to the hopping bandwidth or long Zadoff-Chu sequence with length up to the total (or wide) frequency-hopping bandwidth |
| [12] | Proposal 5: When UE is FH within an SRS resource it should transmit part of the SRS resource/sequence (i.e., 1 SRS frequency hop) during one hop. Proposal 6: RAN1 supports UE to request an SRS sequence that satisfies a certain level of PAPR performance. |

### Round 1

From the existing specification for the SRS for MIMO, frequency hopping is done across an SRS resource, such that each hop is a  ZC sequence of size Msc,bSRS . Thus, it seems that if the same mapping used for SRS for MIMO is used for frequency hopping, a ZC sequence with the size of the hop will be used.

Further details may have to be discussed once the hopping patterns and how to generate them have been agreed. Regarding the proposal in [12] to have the UE request a certain level of PAPR, it is added as a possible FFS.

Proposal 5.6-1: for UL SRS Tx hopping, a single ZC sequence is mapped to each of the hop

- FFS remaining details

- FFS: whether the UE can request an SRS sequence that satisfies a certain level of PAPR performance.

Companies are encouraged to comment on the proposal in the table below:

Proposal 5.6-1

| Company | comment |
| --- | --- |
| Qualcomm | Do not support. This is a major change that we don’t think we have studied enough for this purpose. |
| Ericsson | Do not support, we think it’s an optimization with large spec impact. |

## MIMO SRS support for Tx Hopping [medium]

In [4] it is proposed to support the SRS for MIMO with frequency hopping to implement Tx hopping in UL.

| Company | Proposal |
| --- | --- |
| [4] | Proposal 7: Subject to UE capability, within a virtual BWP with bandwidth beyond maximum RedCap UE bandwidth to achieve SRS Tx frequency hopping, MIMO SRS can also be configured. |

Since there is only one company advancing this proposal, we should first collect views on the support of this proposal:

Companies are encouraged to comment on the proposal in the table below:

| Company | Comment |
| --- | --- |
| Ericsson | Support. In our understanding there is no spec impact beside allowing to instantiate the SRS-config from mimo into the Virtual BWP used for positioning with FH. |

## Pre-compensation of the Tx phase

In [12], it is proposed to support letting the UE perform pre-compensation prior to the transmission of the UL SRS.

| Company | Proposal |
| --- | --- |
| [12] | Proposal 2: RAN1 should support phase alignment for Multi-RTT and determine if phase alignment is needed for both UL at the gNB and DL at the UE. |

From the FL perspective, how pre-compensation is done is up to UE implementation. However, the UE could signal no need for overlap in the Tx hopping if it can maintain the phase by itself without the TRP requiring to perform phase compensation. Thus we propose instead to discuss the case of no overlap in the UL SRS configuration discussion.

Companies are encouraged to comment on the proposal in the table below:

| Company | Comment |
| --- | --- |

## BWP Switching [low]

### Background

In [17], it is proposed to use  BWP switching using an resource in each BWP:

| Company | Proposal |
| --- | --- |
| [17] | Proposal 1: For RedCap UEs positioning, support the SRS frequency hopping across multiple BWPs within one SRS resource. |

In the previous meeting, the following agreement was reached:

Agreement For RedCap UEs, SRS for positioning Tx frequency hopping is configured within one SRS for positioning resource.

Based on the previous agreement, the proposal seem to be contradicting the currently agreed design. Therefore, it will not be pursued. Companies are encouraged to comment below if further discussion is needed on the issue:

| Company | Comment |
| --- | --- |

## Support of Tx hopping with Aperiodic and Semi Persistent SRS [medium]

### Background

In [6][12][16] it is proposed to support aperiodic and semi persistent scheduling of the UL SRS with Tx hopping. Note that since the SRS configuration is outside of the active BWP transmission, the aperiodic SRS resource will have to be associated with a DCI sent in the active BWP.

| Company | Proposal |
| --- | --- |
| [6] | Proposal 4 - Semi-persistent and aperiodic SRS transmission are supported for SRS for positioning with Tx frequency hopping for RedCap UEs. |
| [12] | Proposal 10: RAN1 should discuss the way to reduce the time gap and unnecessary signalling overhead of RedCap FH for positioning (e.g., support a single DCI triggering all the switching). |
| [16] | Proposal 5: Support frequency hopping of SRS-pos is activated/deactivated by MAC-CE. |

### Round 1

Proposal 5.10-1: Semi-persistent and aperiodic SRS transmission are supported for SRS for positioning with Tx frequency hopping for RedCap UEs.

Proposal 5.10-1:

| Company | Comment |
| --- | --- |

## Power control of the UL SRS with Tx hopping [medium]

### Background

In [21] power control for the SRS while hopping is discussed, and it is proposed to use the same pathloss estimate across the hopping procedure.

| Company | Proposal |
| --- | --- |
| [16] | - Proposal 12 UE uses same pathloss estimation for SRS transmission during one SRS frequency hopping duration if uplink power control is configured. |

### Round 1

As only a single company has discussed the issue, we can start by collecting views.

Proposal 5.11-1: UE uses same pathloss estimation for SRS transmission during one SRS frequency hopping duration if uplink power control is configured.

Comments can be entered in the table below:

Proposal 5.11-1:

| Company | Comment |
| --- | --- |
| vivo | Okay for the motivation |

# Online discussions

## Wedesday online

Stable proposals:

(stable)

Proposal 3.2-1: For DL and UL measurements associated with one received hop, a single measurement, associated with one hop, is reported.

(stable)

Proposal 4.2-1: For DL PRS Rx hopping, support the use of a single instance of a measurement gap over all the hops for DL PRS with Rx frequency hopping.

- Send an LS to RAN4 including the agreement.

(stable)

Proposal 5.5-1: SRS Tx Frequency hopping is supported for both RRC_CONNECTED and RRC_INACTIVE state.

Additional proposal that could be discussed

Measurements:

Proposal 3.3-1: DL and UL measurements associated with one received hop are reported separately from the measurements based on multiple hops.

- FFS: Conditions for reporting the single hop measurement, e.g. as fallback to the multi-hop measurement

Proposal 3.5-1: For DL and UL measurements associated with one received hop, the measurement report can be configured includes an indication of the hop that was used to perform the measurement

- FFS: how the hop is indicated

DL PRS rx hopping:

The proposal is still at a deadlock and I don’t see it progressing, therefore I would like to close the issue:

Proposal (conclusion) 4.1-1: PPW is not supported with DL PRS Rx hopping in Rel-18.

UL SRS Rx hopping:

This proposal is not stable, but it would be beneficial to continue the discussion online to see it we are at least going in the right direction.

Proposal 5.1-2: For the SRS Tx hopping pattern configuration support

- A configurable pattern, not limited to the staircase pattern, where each hop may overlap with another hop in the pattern, but the overlapping hops do not need to be adjacent in time.

  - FFS: restrictions / rules to for valid hopping patterns

# Conclusion

TBD

# References

R1-2304368	On positioning for RedCap UEs in Rel-18	FUTUREWEI

R1-2304490	Discussion on positioning for RedCap UEs	vivo

R1-2304568	Discussion on positioning for RedCap UEs	Spreadtrum Communications

R1-2304627, Remaining issues on positioning for RedCap UEs Huawei, HiSilicon

R1-2304741, On positioning for RedCap UEs, CATT

R1-2304834, On positioning for RedCap UEs, Intel Corporation

R1-2304933, Discussion on Positioning for RedCap UEs, ZTE

R1-2304987, Discussion on positioning support for RedCap UEs, NEC

R1-2305045, On Positioning for RedCap UEs, Sony

R1-2305104, Discussion on RedCap UE positioning, CMCC

R1-2305133, Positioning for RedCap UEs, InterDigital, Inc.

R1-2305180, Views on Positioning for RedCap UEs, Nokia, Nokia Shanghai Bell

R1-2305183, Discussion on NR positioning for RedCap , IIT Kanpur, CEWiT

R1-2305253, On Positioning for RedCap UEs, Apple

R1-2305961, Positioning for Reduced Capabilities UEs, Qualcomm Incorporated (Revision of R1-2305347)

R1-2305389, Discussion on positioning support for RedCap UEs, LG Electronics

R1-2305416, Discussion on positioning for RedCap UEs, OPPO

R1-2305524, On Positioning for RedCap UEs, Samsung

R1-2305606, Discussion on positioning for RedCap UEs, NTT DOCOMO, INC.

R1-2305744, RedCap Positioning, Lenovo

R1-2305835, Positioning for RedCap UEs, Ericsson

R1-2305842, Positioning for RedCap UEs, MediaTek (Chengdu) Inc.

RP-223549, “New WID on expanded and improved NR positioning”, RAN#98-e, December 12-16, 2022.

RP-213588, “SID on study on expanded and improved NR positioning”, RAN#94e, December 2021.

R1-2304316	LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs	RAN4, Ericsson