3GPP TSG RAN WG1 #113	R1-2306129 (revised from R1-2305983)

Incheon, Korea, May 22nd – May 26th, 2023

Source:	vivo

Title:	Evaluation methodologies and results for LP-WUS/WUR

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #112bis e-meeting, the agreements for R18 low-power Wake-up Signal and Receiver for NR was approved [1]. In this contribution, the target use cases, design targets are firstly discussed. Then we give our views on the power evaluation methodologies for both RRC idle/inactive and connected modes, followed by the initial simulation results. Finally, LLS and coverage evaluation methodology for LP-WUS is discussed as well.

- Design targets

- Relative power of LP-WUR “on” state

The overall target for the power consumption of LP-WUR is to achieve longer battery life of device. For legacy UEs in RRC idle/inactive mode, the average UE power consumption is typically several or tens of mW. To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR “on” state needs to be reduced by 100 to 1000 times, corresponding to tens or hundreds of μW. Based on the evaluation results provided in section 3.2 and 4.2, to achieve substantial power saving gain (e.g., 80%), less than 1unit relative power of LP-WUR “on” state is desirable.

For a general usage, LP-WUS/WUR design should address all traffic characteristics for IoT/Wearable/eMBB. Specifically, for IoT devices with relatively sparse traffic arrival, battery life is critical. Thus, keeping the “on” state power consumption as low as possible is important as the results shown in the section 3.2.

Observation 1: In the case that the relative power of LP-WUR “on” state is 0.03~0.5unit, substantial power saving gain (i.e. 75% or higher) can be achieved.

Observation 2: LP-WUS/WUR design should address all traffic characteristics for IoT/Wearable/eMBB, thus keeping the “ON” state power as low as possible is important for increasing the battery life of the device.

Proposal 1: The target relative power of LP-WUR “on” state should be less than 1 unit.

- Latency

The latency by adopting LP-WUS/WUR is defined as the time interval between the DL traffic arrival and the time that the MR is ready to receive, which consists of two parts as follows:

- Wake-up delay:

- It depends on LP-WUS monitoring cycle

- MR transition time:

- The ramp-up transition time of main radio

- The sync/resync time of main radio (only for RRC idle/inactive mode)

Observation 3: The latency brought by using LP-WUS/WUR consists of two parts: wake-up delay and MR transition time.

For RRC idle/inactive mode, since there are variety of use cases with different latency requirements, it will be difficult to set certain latency targets. For IoT devices, latency in the order of seconds is generally required. Specially, for latency-sensitive IoT devices, target latency may need to be hundreds of milliseconds if necessary. In addition, latency within several seconds is expected for wearable devices.

For RRC connected mode, considering the stringent latency requirement of XR traffic, the additional latency by adopting LP-WUS/WUR should be no more than several milliseconds. For example, to be able to wake-up for data reception quickly, MR is expected to enter micro or light sleep state during LP-WUR continuous monitoring of wake-up signal, so as to control latency within 3ms (i.e., half of light sleep transition time). Apart from XR use case, the target latency of other eMBB use cases in RRC connected mode can be tens of milliseconds s so that discontinuous LP-WUR monitoring and/or main radio deep sleep could be allowed.

Proposal 2: Consider the following as the latency target for LP-WUS/WUR:

- Within 1 or 2 seconds for latency-sensitive IoT devices and within several or tens of seconds for other IoT devices;

- Within several seconds for wearable devices;

- Within several milliseconds for XR devices and within tens of milliseconds for other eMBB devices;

- Data rate

For idle UEs, the maximum data rate for paging can be calculated as R = 48 bits (UE ID length) * 32 (UEs per PO) * 4(POs per radio frame)/10ms = 614kbps. While for LP-WUS, highest data rate may not necessary, e.g., full UE ID delivery may be not required. Hence, around 100kbps data rate design target can be considered.

Proposal 3: Around 100kbps data rate can be considered as design target for LP-WUS.

- Evaluation methodologies and results for RRC idle/inactive mode

  - Assumptions for RRC idle/inactive mode

  - Sync/re-sync time and energy assumption for MR

It has been agreed that the number of SSBs for sync/re-sync of MR is up to 10. For evaluation, two different numbers of SSB corresponding to different SINR cases can be considered, e.g., 5 SSBs for low SINR case and 3 SSBs for medium or high SINR cases. Note that these SSBs can be used for both synchronization and RRM measurement. Besides, as shown in Figure 1, the detailed processing timeline of sync/re-sync is assumed as: UE performs continuous detection of SSB to determine the slot and symbol boundary (i.e. coarse timing sync)at the beginning of sync/re-sync process; After acquiring the coarse timing information, the rest SSBs can be measured discontinuously i.e., UE can enter sleep state between the time interval of two SSB bursts. According to these analyses, we provide the sync/re-sync time and energy as the following Table 1.

Figure 1. Sync/re-sync timeline including RRM measurement procedure by MR

Table 1. The assumptions of Sync/re-sync time and energy

|  | Sync/re-sync time [ms] | Sync/re-sync energy [ms*units] |
| --- | --- | --- |
| Number of SSBs for sync/re-sync =3[1] | 60 | 2180 |
| Number of SSBs for sync/re-sync =5[1] | 100 | 3340 |
| Note 1: 10ms average continuous detection for SSB at the beginning of sync/re-sync time is assumed. Note 2: LP-SS reception are not assumed to assist MR sync/re-sync after wake- up from ultra-deep sleep. |  |  |

Proposal 4: For sync/re-sync time and energy, assumptions shown in Table 1 can be considered.

  - Transition time between LP-WUR ‘ON’ and ‘OFF’ states for LP-WUR

For the transition from LP-WUR ‘OFF’ to ‘ON’ state, it mainly includes the starting and settling process of MCU or DSM. The MCUs boot up for a fixed number of cycles, or a fixed time period, to enable an external oscillator to become stabilized. After that, a DSM will initialize the runtime environment of the device etc. Accordingly, when assuming the stabilization of the digital circuit needs 1024 cycles and the clock is on the order of Msps, the transition time is approximately tens of milliseconds e.g., 10ms. Additionally, since DDR memory and control processor will take most of the transition time for LP-WUR from ON to OFF, if the two components in OFDM based LP-WUR can be drastically simplified, the transition latency from OFDM based LP-WUR ON to OFF may also be in the order of milliseconds. Besides, the transition time could reduce with the decrease of difference between the relative power of WUR ON and WUR OFF states.

Proposal 5: Transition time between LP-WUR ‘ON’ and ‘OFF’ states can be assumed as:

- 10ms, when the relative power of LP-WUR ON is more than 1 unit;

- 1ms or 5ms, when the relative power of LP-WUR ON is no more than 1 unit.

  - Power model of LP-WUR

In previous meetings, LP-WUR for detection of OFDMA-based WUS has been agreed for study. However, the current agreed power model for LP-WUR (i.e. 0.01/0.05/0.1/0.5/1/2/4) were assumed for LP-WUR corresponding to OOK or FSK based WUS. Thus, the power model for OFDMA-based LP-WUS and WUR should be discussed. The detailed analyses of the power consumption of each component for OFDMA receiver are provided in our companion contribution [2]. The relative powers of LP-WUS ‘ON’ and ‘OFF’ states are 10/20/30 units and 0.05/0.1/0.5 units, respectively.

Proposal 6: The relative power consumptions of LP-WUR for OFDM-based signals/channels detection are given as

- LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring:

  - 10/20 units (reduced complexity OFDM receiver),

  - 30~35 units (normal OFDM receiver)

- LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring:

- WUR-OFF value 0=0.001unit (Note: XO is not in ‘active mode’, memory is not kept, only RTC is running)

- WUR-OFF value 1=0.05/0.1unit (Note: XO is in ‘active mode’, assuming XO option 1/2)

- WUR-OFF value 2=0.5unit (Note: XO is in ‘active mode’, assuming XO option 3/4)

Hence, the power model for LP-WUR can be updated as below.

Proposal 7: For LP-WUR power evaluation,

- Relative Power (unit) for LP-WUR OFF state: 0.001/0.05/0.1/0.5

- 0.001unit for receiver types based on envelope detection for MC-ASK and MC-FSK

- 0.05/0.1/0.5unit for receiver type based on OFDM

- Relative Power (unit) for LP-WUR ON state: 0.01/0.05/0.1/0.5/1/2/4/10/20/30

- 10/20/30 units for receiver type based on OFDM

- 0.01/0.05/0.1/0.5/1/2/4 units for receiver types for based on envelope detection for MC-ASK and MC-FSK

  - RRM measurement assumptions for RRC idle/inactive mode

In RRC idle/inactive mode, to meet RRM measurement requirements defined by RAN4, a legacy UE is supposed to perform RRM measurement by MR every I-DRX cycle. For UEs configured with LP-WUS for power saving, to reduce the power consumption due to RRM measurement activities, MR should be allowed to perform relaxed RRM, and/or to offload RRM measurement to LP-WUR. Following three RRM measurement options can be considered.

- Option 1 (legacy behavior): RRM measurement is only performed by MR per I-DRX cycle.

- Option 2: MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 10 or larger). Additionally, LP-WUR may also perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

- Option 3: MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

Additionally, the feasibility and detailed design of RRM measurement performed by LP-WUR need to be further studied. Note that we assume that MR performs RRM measurement during the process of sync-/re-sync as shown in Figure 1. And the relative power of RRM measurement by LP-WUR is the same as that of LP-WUS detection.

Proposal 8: For RRM measurement assumptions in RRC idle/inactive mode, the following options can be considered.

- Option 1: RRM measurement is only performed by MR as in legacy releases.

- Option 2: MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 10 or larger). Additionally, LP-WUR may also perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

- Option 3: MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

  - Power evaluation results for RRC idle/inactive mode

Based on the agreed evaluation methodologies for RRC idle/inactive mode and the aforementioned additional assumptions, we provide the power evaluation results for the following configurations.

- Baselines: legacy PO monitoring with i-DRX with PEI, eDRX with PEI

- MR sleep state during LP-WUS monitoring: deep sleep or ultra-deep sleep

- The relative power of LP-WUR “ON” state: [0.01/0.05/0.1/0.5/1/2/4/10/20] units

- MR Ramp-up time: [400ms] and Ramp-up and down transition energy: [15000]

- Sync/re-sync time X: X= [60, 100ms] and sync/re-sync energy: [2180, 3340]

- RRM:

- Baseline: Option 1, RRM measurement is only performed by MR as in legacy releases.

- Option 2: MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 10 or larger). Additionally, LP-WUR may also perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

- Option 3: MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%]

Note: Alt-2 of FAR target is adopted, i.e. FAR is determined across a reference time duration (e.g., 1.28s) of one or multiple WUS attempts/trials.

- Per UE paging rate: RE=RE, REF= [1%, 0.1%, 0.01% or 0.001%], Y=YREF = 1.28s

- Per group paging rate: RG=1 – (1 – RE)N , N is the number of UEs in the group: [10, 20]

- LP-WUS monitoring:

  - Option 1: continuously monitoring;

  - Option 2: duty-cycled monitoring

- [200ms, 600ms, 1000ms] as the cycle periodicity, and within each cycle the ON ratio is [0.1%, 1%, 10%].

- LP-WUR ON-OFF transition time: 10ms.

Based on the above evaluation assumption and configurations, the evaluation results and observations are summarized as below.

- Power consumption and latency comparison among legacy I-DRX, legacy e-DRX and LP-WUS

Figure 2 shows the comparison among I-DRX, e-DRX and LP-WUR/WUS under the configurations in Table 2.

Table 2. Configurations for evaluation with different baselines.

| Parameters | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Values | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous | 0.001%, 1% | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 2.  Power consumption and latency comparison among I-DRX, e-DRX and LP-WUR/WUS

According to the results, the following observations are given:

Observation 4: Compared to I-DRX with PEI, LP-WUR/WUS with continuous monitoring configuration can achieve 46%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, and limited latency increase.

Observation 5: Compared to eDRX with PEI, LP-WUR/WUS with continuous monitoring configuration can largely reduce the paging latency (23x), with comparable UE power consumption.

Observation 6: LP-WUR/WUS provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with legacy I-DRX and eDRX.

Observation 7: Battery life is inversely proportional to relative power of LP-WUR “ON” especially when the traffic is extremely sparse e.g., per UE paging rate is 0.001%.

- Impact due to different LP-WUR “ON” state power: [0.01/0.05/0.1/0.5/1/2/4/10/20] units

Figure 3 shows the simulation results with different relative power of LP-WUR “ON” state under configurations in Table 3.

Table 3. Configurations for the evaluation of different LP-WUR “ON” state power.

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 400ms; 15000 | 100ms; 3340 | 0.1% | Continuous; Duty cycle | 0.001% | Ultra-deep sleep, Deep sleep | Option 1 for baseline Option 3 for LP-WUS |
| Notes: Considering latency sensitive use cases, the configuration for duty cycle of LP-WUS is: [200ms] as the duty cycle periodicity, and [2ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |  |

(Notes: When the WUR ON power=10/20, it is assumed that WUR OFF power = 0.01;

The transition time between LP-WUR “ON” and “OFF” is assumed as 10ms)

Figure 3.  The performance of LP-WUS/WUR with different relative powers of LP-WUR “ON” state.

According to the results, the following observations are given:

Observation 8:  Continuous LP-WUS monitoring is not a feasible configuration when the relative power of LP-WUR “ON” state is higher than 1unit.

Observation 9: Duty cycled LP-WUS monitoring can achieve more than 90% power saving benefit compared to legacy I-DRX with PEI, except for the case when the relative power of LP-WUR “ON” is 10 or 20units. The latency increase due to duty cycle LP-WUS monitoring is marginal.

Observation 10: With LP-WUR “ON” state power lower than 10 units, duty cycled LP-WUS monitoring can achieve lower power consumption than legacy e-DRX with PEI.

Observation 11: Up to 45% power saving gain will be lost if UE main radio goes into deep sleep rather than ultra-deep sleep during LP-WUS monitoring.

- Impact due to different LP-WUR “OFF” state power: [0.001/0.1] units

Figure 4 shows the simulation results with different LP-WUR “OFF” state power under configurations in Table 4.

Table 4. Configurations for the evaluation of different LP-WUR “OFF” state power

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | Assumed MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 400ms; 15000 | 100ms; 3340 | 0.1% | duty cycle | 0.001% | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |
| Notes: Considering latency sensitive use cases, the configuration for duty cycle of LP-WUS is: [200ms] as the duty cycle periodicity, and [2ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |  |

Figure 4.  The performance of LP-WUS/WUR scheme with different relative powers of LP-WUR.

According to the results, the following observations are given:

Observation 12: The power consumption will increase when the LP-WUR “OFF” state power increases from 0.001units to 0.1units even with 1% duty cycle ratio.

- Impact due to different LP-WUS monitoring duty cycle periodicity and ratio

Figure 5 shows the simulation results for different LP-WUS monitoring duty cycle periodicity and ratio under configurations in Table 5.

- [200ms, 600ms, 1000ms] as the duty cycle periodicity, and for each cycle the duty cycle ratio is [0.1%, 1%, 10%].

Table 5. Configurations for evaluation with different LP-WUS monitoring configurations.

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUR ON state power | Per UE paging rate | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 400ms; 15000 | 100ms; 3340 | 0.1% | 0.1 units, 1 units | 0.001% | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 5. The power and latency performances of LP-WUS/WUR scheme with different DRX cycle lengths and ratio.

According to the results, the following observations are given:

Observation 13: With fixed duty cycle ratio and LP-WUR “ON” state power, when LP-WUR “ON” state power is no more than 1unit, different LP-WUR duty cycle periodicity have no or less impact on UE power consumption.

Observation 14错误!未指定顺序。: Latency will be reduced with the increase of duty cycle ratio of LP-WUS monitoring, while UE power consumption will increase accordingly.

Observation 15: When the duty cycle ratio is 10%, the achievable UE power saving benefit is significantly degraded if the LP-WUR “ON” state power is 10 or 20 units.

- Impacts due to different RRM schemes

Figure 6  shows the simulation results with different RRM schemes under configurations in Table 6 , where the compared three RRM schemes are the same as mentioned before, i.e.

- Option 1 (MR): RRM measurement is only performed by MR as in legacy releases.

- Option 2 (MR+LR): MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 10 or larger). Additionally, LP-WUR may also perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

- Option 3 (LR): MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

Table 6. Configurations for evaluation with different RRM assumptions.

| Parameters | Baseline | LP-WUR ON state power | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | MR sleep state | Per UE paging rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 0.1/1 unit | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous; duty cycle (1% ratio) | Ultra-deep sleep | 0.001% |
| Note: Considering latency sensitive use cases, the configuration for duty cycle of LP-WUS is: [200ms] as the duty cycle periodicity, and [2ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |  |

Figure 6.  The power and latency performances of LP-WUS/WUR scheme with different RRM schemes.

According to the results, the following observations are given:

Observation 16: No power saving benefit by LP-WUR/WUS, if MR RRM measurement is not relaxed in RRC IDLE/INACTIVE modes. MR RRM measurement relaxation is an enabler to achieve power saving benefit by LP-WUR/WUS, where the MR RRM measurement relaxation includes to increase the MR RRM measurement period by X times, or to offload RRM from MR to LR.

Observation 17: No meaningful power saving gain can be obtained by duty cycled LP-WUS monitoring (with 1% duty cycle ratio) scheme, even with 20x MR measurement relaxation, when the LP-WUR “ON” power is 10 or 20 units.

Observation 18: Higher UE power saving benefit can be achieved if RRM measurement can be completely offloaded from MR to LR.

Proposal 9: It is recommended to support at least one of the following RRM measurement enhancements to achieve substantial UE power saving benefit by LP-WUR/WUS

- Relax the MR RRM measurement period by X times, where the X should be sufficiently large number, e.g. 20 or larger

- Offload the RRM measurements from MR to LR

- Impact due to different false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%]

Figure 7shows the simulation results with different FAR under configurations in Table 7

Table 7. Configurations for evaluation with different FAR.

| Parameters | Baseline: | Ramp-up time and transition energy | Per UE paging rate | Sync/re-sync time and energy | LP-WUS monitoring configuration | Relative power of LP-WUR ON | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 400ms; 15000 | 0.001% | 100ms; 3340 | continuous | 0.1 or 1units | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 7.  The power and latency performances of LP-WUS/WUR with different FAR.

(Note: FAR here refers to the FAR of the LP-WUS, which does not include FAR due to group paging.)

According to the above results, the following observations are given:

Observation 19: The power saving benefit of LP-WUR/WUS decreases when the FAR of LP-WUS is increased.

- Impact due to different per UE paging rate [1%, 0.1%, 0.01%, 0.001%] and group size N= [1, 10, 20] UEs per paging group

Figure 8shows the simulation results with different paging rate under configurations in Table 8.

Table 8. Configurations for evaluation with different paging rate.

| Parameters | Baseline | LP-WUR ON power | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 0.1 unit | 400ms; 15000 | 100ms; 3340 | 0.1% | continuous | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |
| Note: RE = 0.001% or 1%, N = [1 10 20] when calculate per group paging rate. |  |  |  |  |  |  |  |  |

Figure 8. The power and latency performances of LP-WUS/WUR scheme with different paging rates.

According to the results, the following observations are given:

Observation 20:  The power saving benefit of LP-WUR/WUS decreases when the paging rate is increased.

- UE type: eMBB or RedCap

Figure 9 shows the simulation results showing power saving benefit for different UE type under configurations in Table 9. It should be noted that 1Rx is assumed for RedCap UEs.

Table 9. Configurations for evaluation with different UE type.

| Parameters | Baseline: | Ramp-up time and transition energy | Per UE paging rate | Sync/re-sync time and energy | LP-WUS monitoring configuration | FAR | MR sleep state | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX with PEI | 400ms; 15000 | 0.001% | 100ms; 3340 | Continuous, Duty cycle(1%) | 0.1% | Ultra-deep sleep | Option 1 for baseline Option 3 for LP-WUS |

Figure 9. The power saving benefit of LP-WUR/WUS for different UE types (eMBB and RedCap)

Observation 21: RedCap UEs are more sensitive to the LP-WUR ON state power, i.e. the power saving benefit of LP-WUR/WUS decreases faster with the increase of LP-WUR ON state power than eMBB UEs.

- Evaluation methodologies and results for RRC connected mode

  - Usage of LP-WUR/WUS in RRC connected mode

XR is one of the target use cases for applying LP-WUS. Per the discussion in R17/18 XR, one of the identified important characteristics of XR traffic is the wide range and unpredictable jitter. To combat with the jitter while guarantee the latency and capacity, UE has to perform continuous PDCCH monitoring in the entire jitter range in order to receive the scheduling of XR traffic timely and avoid packet loss caused by exceeding the PDB (e.g., 10ms for XR video traffic). However, the existing R15/16/17 power saving schemes are not effective enough for such traffic. In this regard, the motivation to study LP-WUS/WUR in RRC connected mode is to reduce the unnecessary PDCCH monitoring due to jitter while guarantee the XR performance (i.e. latency, capacity)

The power model of LP-WUR agreed in the last meeting can also be adopted for power evaluation in RRC connected mode. To guarantee the latency requirement of XR traffic, main radio should enter micro or light sleep state rather than the ultra-deep sleep state during LP-WUS monitoring, corresponding to additional latency of 0 or 3ms. And the sum of LP-WUR “ON” state power and MR micro/light sleep state power will be close to 45 or 20 units, which is substantially lower than the MR PDCCH monitoring power, i.e.100 units.

As shown in Figure 10(b), enhanced C-DRX, i.e. aligned C-DRX cycle to XR traffic periodicity is adopted, and DRX onduration is configured to cover the whole jitter range. To further reduce the unnecessary PDCCH monitoring during the jitter range, a UE can firstly monitor LP-WUS instead of PDCCH at the beginning of DRX onduration. And after LP-WUS is detected, the UE starts to monitor PDCCH. As such, by replacing the unnecessary PDCCH monitoring with more power efficient LP-WUS detection, more power saving benefit can be achieved, compared to the existing power saving schemes as depicted in Figure 10 (a).

Observation 22: In RRC connected mode, LP-WUS/WUR can be used to reduce the unnecessary PDCCH monitoring due to jitter while guarantee the XR performance (i.e. latency, capacity).

Figure 10.  Potential working procedure for LP-WUS/WUR based jitter handling

In addition to XR traffic, LP-WUR/WUS can also be beneficial for FTP traffic for eMBB devices. In this case, UE can perform LP-WUS monitoring anytime outside and inside DRX onduration, so that gNB is able to wake-up UE whenever there is DL traffic arrival as shown in Figure 11. It can be observed from the evaluation results shown in later section that, with LP-WUR/WUS, same UPT performance can be achieved as always-on monitoring, while the UE power consumption can be similar or even lower than legacy CDRX, which is very much beneficial for both UE side (power consumption) and NW side (scheduling flexibility, UPT). Since FTP traffic is generally more delay tolerant than XR, deep/light/micro sleep states are possible for MR during LP-WUS monitoring by LR. However, ultra-deep sleep state of MR should not be applicable due to latency issues and frequent measurement activities (CSI, RLM, RRM, etc) in MR.

Figure 11.  Potential working procedure for LP-WUS/WUR for FTP3 use case

Observation 23: In RRC connected mode, LP-WUS/WUR can be used to achieve substantial UE power saving benefit without degradation of scheduling flexibility, latency and UPT of FTP traffic, compared with always-on monitoring.

Proposal 10: In RRC connected mode, ultra-deep sleep state of main radio should not be applied. And UE main radio can enter micro, light or deep sleep state during LP-WUS monitoring by LR.

  - Power evaluation results for LP-WUR/WUS in RRC connected mode

In the following evaluation, performance metrics including power consumption, latency, capacity, UPT are considered. Besides, both XR traffic and eMBB FTP traffic are evaluated, the detail evaluation methodologies and assumptions can be found in TR 38.838 and 38.840, respectively.

- Evaluation for XR traffic model (refer to TR38.838 and the agreed jitter model in R18 XR SI)

Based on the R17 and R18 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. As discussed in section 4.1, UE main radio can enter micro or light sleep during LP-WUS monitoring of LR. And the detailed configurations can be founded in Appendix B. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep during LP-WUS monitoring of LR) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for both low load and high loads are given, corresponding to 5 and 10 UEs per cell, respectively.

- InH, FR1, DL only XR traffic, 100MHz bandwidth, DDDSU TDD format

Figure 9 and 10 below shows the evaluation results for DL only XR traffic, i.e. 30Mbps and 60FPS. And LP-WUR ON power is assumed 0.1units with always-On monitoring.

Figure 12.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

Figure 13.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 12 and Figure 13, compared to always-On scheme (no DRX), R17 PDCCH monitoring adaptation scheme can obtain 10~19% power saving gain, while LP-WUS monitoring schemes can achieve about 25% or 30% power saving gain, meaning up to 15% additional power saving gain.

For both low load and high load cases, comparing to R17 PDCCH monitoring adaptation +enhanced DRX scheme, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {7%~15%} additional UE power saving gain with no capacity loss.

In addition, for low load case, comparing to R17 PDCCH monitoring adaptation +enhanced DRX scheme, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can bring {14%~25%} additional UE power saving gain with 10% (UE satisfaction metric as 99% packet successful rate) or no (UE satisfaction metric as 95% packet successful rate) capacity loss. But, for high load case, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can achieve similar additional UE power saving gain with as that in low load, but the capacity loss is significant.

In this sense, the LP-WUS monitoring combined with main receiver micro sleep can be applied to all the cases, while the combination with main receiver light sleep are mainly useful for low load scenarios. Moreover, it can be observed that the larger jitter range, the more additional power saving gain can be obtained by LP-WUS/WUR.

Observation 24: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {7%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases for DL only XR traffic.

Observation 25: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {14%~25%} additional UE power saving gain, with acceptable capacity impact at least in low load case for DL only XR traffic.

- InH, FR1, DL video+UL pose XR traffics, 100MHz bandwidth, DDDSU TDD format

Figure 14 below shows the evaluation results for mixed DL and UL traffics for XR. For DL, 30Mbps and 60FPS is assumed. While for UL, the pose traffic with arrival interval of 16.67ms and 10ms PDB is assumed. And LP-WUR ON power is assumed 0.1units with alwayson monitoring. Other details can be found in Appendix B.

Figure 14 Evaluation results for mixed DL (AR) and UL (pose) traffics in XR

According to the results shown in Figure 14, compared to always-On scheme (no DRX), R17 PDCCH monitoring skipping+ehanced DRX + SSSG switching scheme can obtain 10~14% power saving gain, while LP-WUS monitoring schemes can achieve about 19% ~29% power saving gain.

For both low load and high load cases, comparing to R17 PDCCH monitoring adaptation +enhanced DRX, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {8%~12%} additional UE power saving gain with no capacity loss.

In addition, for low load case, comparing to R17 PDCCH monitoring adaptation +enhanced DRX, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can bring {18%~19%} additional UE power saving gain with a little (UE satisfaction metric as 95% packet successful rate) capacity loss.

In this sense, the LP-WUS monitoring combined with main receiver micro sleep can be applied to all the cases, while the combination with main receiver light sleep are mainly useful for low load scenarios.

Observation 26: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {8%~12%} additional UE power saving gain with no capacity loss in both low load and high load cases for DL+UL XR traffics.

Observation 27: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {18%~19%} additional UE power saving gain, with acceptable capacity impact at least in low load case for DL+UL XR traffics.

- Evaluation for eMBB FTP traffic model (refer to TR 38.840)

Based on FTP model 3 traffic model (200ms mean arrival interval) and its corresponding CDRX configuration provided in TR 38.840, we further evaluate UE perceived throughput (UPT) and power consumption with the following schemes. Note that one UE per cell is assumed.

- Always-On (baseline): Without adopting any power saving mechanism.

- C-DRX:

| DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- |
| 160 | 8 | 100 |
| 160 | 8 | 40 |
| 40 | 4 | 25 |
| 40 | 4 | 10 |

- C-DRX +DCI format 2_6: the time gap between DCI format 2_6 and DRX onduration is 3ms, and the DCI format 2_6 monitoring duration is 2ms.

- R17 PDCCH monitoring adaptation + C-DRX +DCI format 2_6: PDCCH skipping duration is 100ms, SSSG with PDCCH monitoring every slot.

- LP-WUS monitoring+ PDCCH skipping: During PDCCH skipping duration, the UE performs continuous LP-WUS monitoring. Once detecting LP-WUS, UE will start PDCCH monitoring after a wake-up delay e.g., 0ms, 3ms, or 10ms (which corresponds to the ramp-up delay of main receiver from micro/light/deep sleep states).  The assumptions on the relative power of LP-WUR “ON” state is 0.01/1/10/20/30 units.

Figure 15. UPT and power consumption of evaluation schemes

Table 10. PSG and UPT loss compared with always-On scheme

| Schemes | LP-WUR “ON” power | PSG (compared to “AlwaysON”) | UPT loss (compared to “AlwaysON”) | UPT gain (compared to “CDRX”) |
| --- | --- | --- | --- | --- |
| CDRX (160ms-8ms-100ms) | - | 52.19% | 60.93% | - |
| CDRX (160ms-8ms-40ms) |  | 66.17% | 75.33% | -36.84% |
| CDRX (40ms-4ms-25ms) |  | 59.14% | 54.39% | 14.34% |
| CDRX (40ms-4ms-10ms)C-DRX |  | 63.36% | 58.12% | 7.18% |
| C-DRX(160ms-8ms-100ms) + DCI 2_6 | - | 53.78% | 63.74% | -7.17% |
| R17 PDCCH monitoring adaptation + C-DRX(160ms-8ms-100ms) | - | 77.30% | 84.86% | -61.24% |
| R17 PDCCH monitoring adaptation + C-DRX(160ms-8ms-100ms) + DCI 2_6 | - | 77.35% | 86.73% | -66.03% |
| LP-WUR (MR micro sleep) | 0.01 | 44.00% | 0.00% | 155.98% |
|  | 1 | 42.95% |  |  |
|  | 10 | 33.42% |  |  |
|  | 20 | 22.83% |  |  |
|  | 30 | 12.24% |  |  |
| LP-WUR (MR light sleep) | 0.01 | 69.00% | 25.98% | 89.47% |
|  | 1 | 67.95% |  |  |
|  | 10 | 58.41% |  |  |
|  | 20 | 47.83% |  |  |
|  | 30 | 37.24% |  |  |
| LP-WUR (MR deep sleep) | 0.01 | 86.73% | 57.20% | 9.57% |
|  | 1 | 85.68% |  |  |
|  | 10 | 76.14% |  |  |
|  | 20 | 65.56% |  |  |
|  | 30 | 54.98% |  |  |

As shown in Figure 15 and Table 10, the following observations are given

- For UPT:

- Compared to existing power saving schemes, LP-WUR/WUS can achieve large UPT gain, especially for cases with MR micro or light sleep during LP-WUS monitoring by LR.

- With MR micro sleep during LP-WUS monitoring by LR, same UPT performance as always-on scheme can be achieved.

- For UE power consumption:

- The legacy UE power schemes in Rel-15/16/17 reduced the UE power consumption, however, the latency and UPT performance are largely degraded compared with always-on scheme.

- Comparing with Rel-15 C-DRX1 scheme, LP-WUR/WUS with main radio enters light/deep sleep during LP-WUS monitoring can bring {35.2% / 72.3%} additional UE power saving gain.

- Comparing with C-DRX1 + DCI format 2_6 scheme, LP-WUR/WUS with main radio enters light/deep sleep during LP-WUS monitoring can bring {32.9% / 71.3%} additional UE power saving gain.

- Compared with R17 PDCCH monitoring adaptation + C-DRX1 + DCI 2_6 scheme, LP-WUR/WUS with main radio enters deep sleep during LP-WUS monitoring can bring 41% additional UE power saving gain.

Observation 28: With MR micro sleep during LP-WUS monitoring by LR, same UPT performance as always-on scheme can be achieved, while the UE power consumption can be similar or lower than Rel-15 C-DRX.

Observation 29: When the WUR “ON” state power is low (e.g. no more than 1unit), LP-WUR/WUS combined with Rel-17 PDCCH skipping can achieve the best trade-off between UPT and UE power saving, among all the evaluated UE power saving schemes.

Observation 30: When the WUR “ON” state power is high (e.g. 20 or 30 units), LP-WUR/WUS combined with Rel-17 PDCCH skipping has no power saving gain over existing power saving schemes.

- System performance

  - System resource overhead of LP-WUS

In order to study the system resource overhead by LP-WUS, we perform the resource overhead calculation with the following assumptions:

- The bandwidth of LP-WUS denoted as BWLP-WUS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a serving cell denoted as BWCell is 20MHz or 100MHz.

- The duration of LP-WUS represented by DLP-WUS.

- For RRC idle/inactive mode, the number of beams for LP-WUS transmission is assumed to be 8, which means the LP-WUS is transmitted repeatedly 8 times.

Hence, the resource overhead LP-WUS to the overall system resource can be calculated as below. Table 11 and Table 12 provide some reference values based on the above assumptions.

- RRC IDLE/INACTIVE mode:

- For RRC idle/inactive mode, paging configuration:

  - Number of pages for a UE per cell per second (NPage_per_cell_second) is calculated in Table 5

  - Number of UEs carried in a LP-WUS is NUE_per_WUS

  - Number of beams is Nbeam

RLP_WUS=BWLP_WUS*DLPWUS*NPagepercellsecond*NUEperWUS*Nbeam BWCell

Assuming the resource of one LP-WUS is:

- Alt 1: 5MHz * 4 OFDM symbol (30kHz) (Note: 1bit, 5dB better than the coverage of Msg.3 PUSCH, refer to LP-WUS 1bit-4symbols as shown in Figure 16)

- Alt 2: 5MHz * 14 OFDM symbol (30kHz) (Note: 1bit, comparable to the coverage of PDCCH AL8 and 9dB better than MSG.3 PUSCH, refer to LP-WUS 1bit-14symbols as shown in Figure 16)

- Alt 3: 5MHz * 7 OFDM symbol (30kHz) (Note: 12 bits, comparable to the coverage of Msg.3 PUSCH, refer to LP-WUS 12bit-7symbols as shown in Figure 16)

Figure 16. MIL of LP-WUS for Urban 2.6GHz for non-RedCap UEs

Table 11.  Resource overhead ratio to the overall system resource for RRC idle/inactive mode.

| Scenarios | Paging density |  | Alt | LP-WUS overhead (ratio of the resource compared to the total resources) |  |
| --- | --- | --- | --- | --- | --- |
|  | Scenarios description | Number of pages per cell per second |  | Per UE LP-WUS | Per group LP-WUS (NUE_per_WUS=4) |
| For IoT cases | Assuming 1,000,000 user/km2 connection density [ITU M.2412], with packet arrival every 1 message/2 hours/device [ITU M.2412], assuming 500m ISD with 3 sectors | 10 | Alt 1 | 0.06% (100MHz sysBW) | 0.01% (100MHz sysBW) |
|  |  |  |  | 0.29% (20MHz) | 0.07% (20MHz) |
|  |  |  | Alt 2 | 0.2% (100MHz sysBW) | 0.05% (100MHz sysBW) |
|  |  |  |  | 1.01% (20MHz) | 0.25% (20MHz) |
|  |  |  | Alt 3 | 0.1% (100MHz sysBW) | 0.025% (100MHz sysBW) |
|  |  |  |  | 0.5% (20MHz) | 0.125% (20MHz) |
| For eMBB cases | Assuming very high population density, e.g., sports event with 5,000 users/cell population density, and per UE paging arrival rate = 1% when paging i-DRX cycle is Tpagingcycle = 1.28s | 39 | Alt 1 | 0.22% (100MHz sysBW) | 0.06% (100MHz sysBW) |
|  |  |  |  | 1.12% (20MHz) | 0.28% (20MHz) |
|  |  |  | Alt 2 | 0.79% (100MHz sysBW) | 0.2% (100MHz sysBW) |
|  |  |  |  | 3.93% (20MHz) | 0.98% (20MHz) |
|  |  |  | Alt 3 | 0.4% (100MHz sysBW) | 0.1% (100MHz sysBW) |
|  |  |  |  | 1.97% (20MHz) | 0.49% (20MHz) |
| Note: - The above calculation assumes per cell paging. If the tracking area is larger and the base station needs to page UEs in a tracking area covering multiple cells, the LP-WUS overhead may be increased. - The coverage performance of LP-WUS based on Alt1 and 2 are far better than that of Msg.3 PUSCH. If we consider the comparable coverage performance with Msg.3 PUSCH, then the resource overhead of LP-WUS alt 1 and alt 2 will be much lower than the ones shown in Table 11. |  |  |  |  |  |

- RRC Connected mode:

For RRC connected mode, number of UE configured with LP-WUS per cell NUE_connected can be assumed as 10 (capacity upper bound for XR service in several scenarios). Different UEs can use orthogonal LP-WUS resources. The traffic interval TTrafficInterval can be assumed as 16.67ms (60FPS) for XR and 200ms for eMBB FTP traffic.

RLP_WUS=BWLP_WUS*DLP_WUS*NUE_connected BWCell*TTrafficInterval

As per the agreed assumption of aggregation level (AL) distributions captured in RedCap TR 38.875 below, we note that the distributions of AL1+AL2 accounts for the majority i.e., 70%~90% in typical case. So, LP-WUS matching to PDCCH with AL2 is assumed in our overhead evaluation.

TR 38.875 *** Unchanged text is omitted *** The aggregation level (AL) distributions in Table 6.2-5 are used by different sources for PDCCH blocking rate evaluations. Table 6.2-5: PDCCH AL distributions of AL [1,2,4,8,16], FR1 and FR2 PDCCH AL distributions of AL [1,2,4,8,16]. - Configuration 1 (A1): [0.5, 0.4, 0.05, 0.03, 0.02], assuming majority of the UEs are in is good coverage - Configuration 2 (A2): [0.1, 0.2, 0.4, 0.2, 0.1]: Majority of the UEs are in medium coverage - Configuration 3 (A3): [0.05, 0.05, 0.2, 0.3, 0.4]: Majority of the UEs are in poor coverage - Configuration 4 (A4): [0.3 0.5 0.1 0.06 0.04] - Configuration 5 (A5): [0.4 0.45 0.08 0.04 0.03] - Configuration 6 (A6): [0.2 0.55 0.14 0.06 0.05] - Configuration 7 (A7): [0.4 0.3 0.2 0.05 0.05]. Note: the results for A2/A3 may not represent a typical case, e.g., because of the assumptions of unfavourable channel conditions.. *** Unchanged text is omitted ***

As analysis above, to match the unicast PDCCH AL2-4Rx link budget, we consider the following different examples of resources used for LP-WUS with different information bits:

LP-WUS carry 1bit information:

- Alt 1: 5MHz * 14 OFDM symbol (30kHz) (Note: Refer to unicast LP-WUS 1bit-14symbols in Figure 17)

LP-WUS carry 12bit information:

- Alt 2: 5MHz *56 OFDM symbol (30kHz) (Note: Refer to unicast LP-WUS 12bit-56symbols in Figure 17)

Figure 17. MIL of unicast LP-WUS for Urban 2.6GHz for non-RedCap UEs

Table 12.  Resource overhead ratio to the overall system resource for RRC connected mode.

| Resource overhead ratio, RLP-WUS | RRC Connected mode |  |
| --- | --- | --- |
|  | XR traffic (10 users per cell, Maximum capacity) | eMBB FTP traffic (10 users per cell) |
| 100MHz, 30KHz SCS, Alt1 | 1.5% | 0.125% |
| 100MHz, 30KHz SCS, Alt2 | 6.0% | 0.5% |

Observation 31: For RRC IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1% for 100MHz system BW or less than 4% for 20MHz system BW even in highest UE connection density for both IoT and eMBB cases.

Observation 32: For RRC CONNECTED mode, the resource overhead of LP-WUS is

- Less than 6% in the cell with maximum XR capacity, i.e. 10 users.

- Less than 0.5% in the cell with 10 users of FTP traffic

| Resource overhead ratio, RLP-WUS | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic (10 users per cell, Maximum capacity) | eMBB FTP traffic (10 users per cell) |
| 20MHz, 30KHz SCS | 0.07% ~ 3.93% | - | - |
| 100MHz, 30KHz SCS | 0.01% ~ 0.79 % | 1.5%~6.0% | 0.125%~0.5% |

  - System resource overhead of LP-SS in RRC idle/inactive mode

LP-SS is beneficial for IDLE/INACTIVE UEs for offloading RRM measurements to LR, and/or synchronization for LP-WUS monitoring. However, there has been some concern regarding system resource overhead and network power consumption due to LP-SS.

We perform the resource overhead calculation of LP-SS with the following assumptions:

- The bandwidth of LP-SS denoted as BWLP-SS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a serving cell denoted as BWCell is 20MHz or 100MHz

- The duration of LP-SS represented by DLP-SS is one slot.

- According to the agreed evaluation assumption, the LP-SS periodicity represented by TLP-SS are assumed as 320ms (the most frequent LP-SS transmission), 1280ms (the same as the periodicity of I-DRX paging cycle) and 10240ms (the largest LP-SS periodicity).

- The number of beams for LP-SS transmission is assumed to be 8, which means the LP-WUS is transmitted repeatedly 8 times.

Hence, the resource overhead ratios of LP-SS to the overall system resource can be calculated as below in Table 13.

RLP_SS=BWLP_SS*DLP_SS*Nbeam BWCell*TLP_SS

Table 13.  Resource overhead ratio of LP-SS to the overall system resource in RRC idle/inactive mode

| Resource overhead ratio, RLP-SS | LP-SS periodicity |  |  |
| --- | --- | --- | --- |
|  | 320ms | 1280ms | 10240ms |
| 20MHz, 30KHz SCS | 0.315% | 0.079% | 0.01% |
| 100MHz, 30KHz SCS | 0.063% | 0.0158% | 0.002% |

In addition, LP-SS will not be assumed for RRC connected mode UEs as per the following agreement endorsed in 9.11.3 of the last meeting. Hence, there is no additional resource overhead caused by LP-SS for RRC connected mode.

Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR.

Observation 33: LP-SS in RRC IDLE/INACTIVE mode consumes no more than 0.32% of the system resource, even considering the most frequency LP-SS transmission i.e., 320ms LP-SS periodicity.

Observation 34: LP-SS is not needed for RRC connected mode UEs enabled with LP-WUR/WUS.

  - Network energy consumption

Network energy consumption for transmitting LP-WUS/LP-SS depends on the transmission occasion/periodicity configuration, gNB implementation and cell load etc. In general, there are two types of transmission: periodic transmission of LP-SS, and aperiodic transmission of LP-WUS. Note that the assumptions of different loads follow the agreed definition in R18 NES SI.

As LP-SS is transmitted in periodic manner, the network energy consumption has been a concern by some companies. In the following, the additional network energy consumption due to LP-SS transmission is evaluated, where the baseline assumption and LP-SS configuration is provided in Table 14 and additional evaluation assumptions can be found in Appendix D. The evaluation results under different network loads based on NES power model CAT 1 & CAT2 are provided in Table 15.

In general, LP-SS can be either FDM or TDM multiplexed with existing periodic signal/channels, e.g. SSB/SIB1. For FDM multiplexing of LP-SS and SSB/SIB 1, it is observed that the additional network energy consumption ratio is marginal, i.e., 0.002%~0.075% for a transmission periodicity 10240ms to 320ms across different network loads based on CAT 1 network power model and 0.002%~0.059% for a transmission periodicity 10240ms to 320ms across different network loads based on CAT 2 network power model.

For TDM multiplexing of LP-SS and SSB/SIB 1, the additional network energy consumption ratio is still low, i.e., 0.039%~1.724% for a transmission periodicity 10240ms to 320ms across different network loads based on CAT 1 network power model and 0.031%~1.038% for a transmission periodicity 10240ms to 320ms across different network loads based on CAT 2 network power model.

Table 14. Assumption on baseline and LP-SS configuration

| Scheme | Assumption |
| --- | --- |
| Baseline: SSB and SIB1 transmitted in FDM manner; RACH monitoring | - Periodicity of SSB/SIB1 transmission, RACH monitoring: 20ms - SSB: 4 slots with 2 SSBs in each slot, where 1 SSB occupies 4 OFDM symbols and 20 PRBs - SIB 1: occupies 4 slots and 48 PRBs - RACH: occupies 1 slot |
| LP-SS | - Periodicity of LP-SS: P=320, 640, 1280, 2560, 5120, 10240ms - 1 LP-SS occupies 4 slots (enabling beam-sweeping) and 11PRBs |

Table 15. The additional network energy power consumption for periodic LP-SS transmittion under different  network loads based on NES power model CAT 1 &CAT2

- Zero load case

| Cell load | Multiplexing between LP-SS and SSB/SIB1 | LP-SS transmission periodicity P (unit: ms) | Additional network power consumption (CAT-1 model) | Additional network power consumption (CAT- 2 model) |
| --- | --- | --- | --- | --- |
| Zero load | FDM with SSB/SIB 1 | P=320 | 0.075% | 0.059% |
|  |  | P=640 | 0.037% | 0.030% |
|  |  | P=1280 | 0.019% | 0.015% |
|  |  | P=2560 | 0.009% | 0.007% |
|  |  | P=5120 | 0.005% | 0.004% |
|  |  | P=10240 | 0.005% | 0.004% |
|  | TDM with SSB/SIB 1 (adjacent slots) | P=320 | 1.724% | 1.038% |
|  |  | P=640 | 0.862% | 0.519% |
|  |  | P=1280 | 0.431% | 0.259% |
|  |  | P=2560 | 0.216% | 0.130% |
|  |  | P=5120 | 0.108% | 0.065% |
|  |  | P=10240 | 0.108% | 0.065% |

- Low load case

| Cell load | Multiplexing between LP-SS and SSB/SIB1 | LP-SS transmission periodicity P (unit: ms) | Additional network power consumption (CAT-1 model) | Additional network power consumption (CAT-2 model) |
| --- | --- | --- | --- | --- |
| Low load | FDM with SSB/SIB 1 | P=320 | 0.057% | 0.049% |
|  |  | P=640 | 0.028% | 0.025% |
|  |  | P=1280 | 0.014% | 0.012% |
|  |  | P=2560 | 0.007% | 0.006% |
|  |  | P=5120 | 0.004% | 0.003% |
|  |  | P=10240 | 0.004% | 0.003% |
|  | TDM with SSB/SIB 1 (adjacent slots) | P=320 | 1.271% | 0.863% |
|  |  | P=640 | 0.598% | 0.435% |
|  |  | P=1280 | 0.325% | 0.221% |
|  |  | P=2560 | 0.163% | 0.112% |
|  |  | P=5120 | 0.083% | 0.057% |
|  |  | P=10240 | 0.083% | 0.057% |

- Light load case

| Cell load | Multiplexing between LP-SS and SSB/SIB1 | LP-SS transmission periodicity P (unit: ms) | Additional network power consumption (CAT-1 model) | Additional network power consumption (CAT-2 model) |
| --- | --- | --- | --- | --- |
| Light load | FDM with SSB/SIB 1 | P=320 | 0.033% | 0.032% |
|  |  | P=640 | 0.017% | 0.017% |
|  |  | P=1280 | 0.008% | 0.008% |
|  |  | P=2560 | 0.005% | 0.004% |
|  |  | P=5120 | 0.003% | 0.002% |
|  |  | P=10240 | 0.003% | 0.002% |
|  | TDM with SSB/SIB 1 (adjacent slots) | P=320 | 0.830% | 0.580% |
|  |  | P=640 | 0.402% | 0.298% |
|  |  | P=1280 | 0.202% | 0.141% |
|  |  | P=2560 | 0.102% | 0.073% |
|  |  | P=5120 | 0.060% | 0.041% |
|  |  | P=10240 | 0.060% | 0.041% |

- Medium load case

| Cell load | Multiplexing between LP-SS and SSB/SIB1 | LP-SS transmission periodicity P (unit: ms) | Additional network power consumption (CAT-1 model) | Additional network power consumption (CAT-2 model) |
| --- | --- | --- | --- | --- |
| Medium load | FDM with SSB/SIB 1 | P=320 | 0.019% | 0.021% |
|  |  | P=640 | 0.010% | 0.011% |
|  |  | P=1280 | 0.006% | 0.006% |
|  |  | P=2560 | 0.003% | 0.003% |
|  |  | P=5120 | 0.002% | 0.002% |
|  |  | P=10240 | 0.002% | 0.002% |
|  | TDM with SSB/SIB 1 (adjacent slots) | P=320 | 0.657% | 0.367% |
|  |  | P=640 | 0.312% | 0.188% |
|  |  | P=1280 | 0.156% | 0.099% |
|  |  | P=2560 | 0.079% | 0.056% |
|  |  | P=5120 | 0.039% | 0.031% |
|  |  | P=10240 | 0.039% | 0.031% |

Observation 35: The additional network energy consumption for periodic LP-SS transmission is marginal, especially when LP-SS is multiplexed with SSB/SIB 1 in FDM manner.

| Multiplexing between LP-SS and SSB/SIB1 | The additional network energy consumption for periodic LP-SS transmission with a periodicity of 10240ms to 320ms |  |
| --- | --- | --- |
|  | NES power model CAT 1 (across different network loads) | NES power model CAT 2 (across different network loads) |
| FDM with SSB/SIB 1 | 0.002%~0.075% | 0.002%~0.059% |
| TDM with SSB/SIB 1 | 0.039%~1.724% | 0.031%~1.038% |

- Link performance and coverage evaluation results

  - Assumptions in coverage Evaluation

In this section, we provide the evaluation assumptions and coverage evaluation results for LP-WUS, according to agreed evaluation methodologies.

- Frequency error/frequency drift

Working Assumption The following for usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done.. TBD: value(s); For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. [Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power] FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1 Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.

In our evaluation, time offset cumulated in the LP-WUR OFF period can be calculated based on the	 RTC with 20 ppm. frequency error. For time drift, assuming the LP-SS periodicity of 1.28 sec, the time drift ΔT in each of 1.28 sec can be calculated according to the relationship between the maximum frequency error (Fe) and time drift (ΔT) over a time (T), i.e. ΔT(sec)= Fe (ppm)* T(sec) =20 ppm*1.28s=25.6us.

For each LP-WUS monitoring occasion, UE start searching for preamble ΔT time before the actual starting time of the LP-WUS transmission in evaluation. For frequency offset, it is calculated assuming the oscillator opt-1, and we assume the oscillator always work in linear region. For carrier frequency offset(ΔF), max frequency error is assumed, and the frequency displacement in Hz is ΔF(Hz) = ±Fe (ppm) * Fcarrier (Hz)= ±200 ppm*2.6GHz =520kHz.

Figure 18.  Starting detection in advance due to time drifting

- Target FAR and number of attempts across a reference time duration T

In previous meeting, the definition of FAR in evaluation were discussed, and following working assumption were made.

Working Assumption For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE have N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design.

According to above conclusion, when the FAR target and reference time duration T is determined, the greater number of attempts leading to higher FAR, which will conversely limit the MDR performance.

For RRC idle/inactive UEs, always on monitoring of LP-WUS, assume I-DRX=1.28s and 1 slot for each LP-WUS transmission, UE detect LP-WUS with 2560 attempts for the 2560 slots/occasions across the I-DRX cycle. For sequence only LP-WUS, the sequence length should be long enough to achieve both MDR and FAR target. Since the sequence is detected by sliding correlation multiple times in the time duration which covers the time drift, periodical sync to calibrate the timing is needed to reduce the number of correlations within one attempt, to make it easier to achieve the FAR target. For LP-WUS structure with both preamble and payload (with CRC), the FAR should be overall FAR considering both sequence and payload part, and UE start to decode payload part and check CRC only when the correlation of preamble part is higher than certain threshold. With this, the actual number of CRC check per I-DRX cycle is greatly reduced. For example, the FAR of detection preamble part is 1%, UE only need to decode the payload part ~26 times per 1.28 seconds, even with continuous LP-WUS monitoring. To ensure the overall FAR rate lower than 1%, the CRC bits should be long enough, e.g., 12bits.

For RRC connected, main radio (MR) activities are frequent due to CSI/RLM/RRM measurements, etc. Therefore, the accurate timing can be assumed for LP-WUR with the assist from MR. UE only need one attempt for detection of the LP-WUS in each monitoring occasion. In this case, number of attempts is equivalent to number of monitoring occasions. For a typical 60FPS XR traffic with 16.67ms C-DRX cycle, UE need to detect 32 LP-WUS occasions, assuming 1 slot LP-WUS duration and 30kHz SCS, per C-DRX cycle for always on monitoring, and even less times for duty cycled monitoring. In this case, both sequence-only and sequence + payload (with CRC) formats for LP-WUS structure are feasible.

In our evaluation, we assume (FAR target, T, N) = {1%, 1.28sec, 1}. We assume that the periodicity of monitor occasions is 1.28sec, which is I-DRX cycle, and the LP-WUS is transmitted once per 1.28 sec. For detection of each LP-WUS transmission, UE performs multiple correlations for LP-WUS sequence, and these correlations are considered as UE implementation in ONE trial/attempt.

- Noise Figure

In previous meetings, it was agreed that noise figure can be selected from [9, 12, 15, 18, 21, 24] db. One of the above values should be reported together with MIL.

According to [3] [4], 8.7dB and 11dB noise figure can be achieved for LP-WUR, 12dB noise figure is assumed for MIL calculation in this section.

- Assumption for antenna/beamforming gain for LP-WUS and legacy NR channels

The DL coverage is highly dependent on the gNB transmission EIRP. While the EIRP is usually determined based on transmission power and beamforming gain. In Rel-15/Rel-16, for a UE specific downlink transmission, gNB may select beam with better quality to serve a UE through beam management procedure or taking advantage of channel reciprocity, hence large BF gain can be assumed for transmissions or receptions other than DL broadcast channels.

For DL broadcast channels, e.g. SSBs and PDCCH in type 0-2 CSSs, typically these channels are served by fixed and wider beams, the beamforming gain would be lower compared with UE specific channels. To reflect the coverage of the broadcast channels more accurately, the BF gain difference between broadcast channel and unicast channel should be reflected in link budget template. The beamforming gain correction should be considered for both paging PDCCH and LP-WUS received in idle state.

Similarly, for UL channels, difference between broadcast BF gain and unicast BF gain in gNB reception should be reflected. For Msg3 PUSCH, gNB receives the Msg3 using the same beam as SSB which UE selected to initiate the RACH procedure. Hence, the receiving beamforming gain can be same as the SSB beamforming gain, i.e., broadcast beamforming gain. The same correction factor as the correction factor for gNB Tx BF gain for broadcast channel can be assumed for UL channels transmitted in idle state, e.g., Msg3. While for PUSCH eMBB, unicast beamforming gain can be assumed due to beam management procedure.

According to [5], the correction factor for gNB Tx BF gain for broadcast channel and gNB Rx BF gain should be considered in link budget template.

- In FR1, the correction factor is about 8dB for carrier frequency 2.6GHz/4GHz; 0dB for 700MHz.

In the link budget template,

Proposal 11: The correction factor for gNB Tx/Rx BF gain for broadcast DL channel and Msg3, should be considered in link budget template to reflect the difference between broadcast beamforming and unicast beamforming

- The same correction factor should be assumed for broadcast DL channel and Msg3;

- In FR1, the correction factor is about 8dB for 2.6GHz/4GHz; 0dB for 700MHz.

In the link budget template provided in R1-2304151, the correction factors for modelling antenna gain loss including BF gain loss can be provided in the following rows.

(5b) antenna gain correction factor at antenna gain component 2 of transmitter (dB)
Note: zero for uplink

(11bis-b) antenna gain correction factor at antenna gain component 2 of receiver (dB)Note:  zero for downlink

  - Preliminary coverage evaluation results for LP-WUS

In the section, we provide our preliminary coverage evaluation results for LP-WUS. Detailed simulation assumptions are provided in Appendix C and several configurations are listed as following:

- Config-1: 4.32MHz/8.64MHz BW, preamble length-16 chips, payload-12bits, CRC-8bits, 1ms-duration

- Config-2: 4.32MHz/8.64MHzBW, sequence length-28 chips, 1-slot length (30kHz), 0.5ms-duration

- Config-3: 4.32MHz/8.64MHz BW, sequence length-8 chips, 4 OFDM symbols length (30kHz)

The modulation of the above three configurations is OOK-4 and the chip rate is 56kcps(cps, chip per second), which means there are 2 OOK ON/OFF chips within one OFDM symbol. We evaluated both 4.32MHz/8.32MHz LP-WUS BW, and more simulation parameters can be found in Appendix C.

In the following figures, we provide the coverage comparison between LP-WUS and legacy NR signals for both  non-RedCap UE and Redcap UE. As shown in following figures, all LP-WUS configs can achieve higher MIL than NR PUSCH and PUSCH Msg3. Among the three configurations above, LP-WUS Config-2 with 8.64MHz BW can achieve the best performance.

For non-RedCap UEs, the MIL of LP-WUS configs cannot achieve comparable MIL as PDCCH AL16-2Rx. The biggest performance gap is 7.5dB between LP-WUS config1 with 4.32MHz BW and PDCCH AL16-2Rx in urban scenarios for non-RedCap UE. And the MIL of LP-WUS Config-2 can achieve comparable MIL as PDCCH AL8.

For RedCap UEs with 1Rx, the MIL of LP-WUS config-2 with 8.64MHz BW can be comparable with PDCCH AL16, and the MIL of LP-WUS Config-2 with 8.64MHz BW is higher than the MIL of PDCCH AL16 in rural scenario. The maximum performance MIL gap between LP-WUS and PDCCH is within 5dB for RedCap UEs.

Note: The Results for Msg3 PUSCH is updated compared with R1-2304500.

Figure 19. MIL of LP-WUS for Urban 2.6GHz for non-RedCap UEs

Figure 20. MIL of LP-WUS for Rural 700MHz for non-RedCap UEs

Figure 21. MIL of LP-WUS for Urban 2.6GHz for Redcap UE

Figure 22. MIL comparison between PUSCH/PDCCH and LP-WUS for Rural 700MHz for Redcap UE

Observation 36: All LP-WUS configurations can achieve better MIL than PUSCH eMBB and PUSCH for Msg3, for both non-RedCap and RedCap UEs.

Observation 37: For non-RedCap UEs, the sequence-based LP-WUS with 0.5ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-2) can achieve comparable MIL with NR PDCCH AL8.

Observation 38: For non-RedCap UEs, the “sequence + payload” based LP-WUS with 1ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-1) can achieve up to 5.5dB worse MIL than NR PDCCH AL16, respectively.

Observation 39: For RedCap UEs, the sequence-based LP-WUS with 0.5ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-2) can achieve comparable MIL with NR PDCCH AL16.

Observation 40: For RedCap UEs, the “sequence + payload” based LP-WUS with 1ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-1) can achieve up to 3dB worse MIL than NR PDCCH AL16.

To further improve the detection performance of LP-WUS, power boosting, channel coding and more time/frequency resources allocation can be considered.

Proposal 11: To reduce the MIL gap between LP-WUS and NR PDCCH, coverage enhancement methods for LP-WUS such as power boosting, channel coding and more time/frequency domain resources can be studied.

- Conclusion

In this contribution, design targets and evaluation methodologies for R18 LP-WUS/WUR study are discussed with the following observations and proposals:

Observation 1: In the case that the relative power of LP-WUR “on” state is 0.03~0.5 unit, substantial power saving gain (i.e. 75% or higher)can be achieved.

Observation 2: LP-WUS/WUR design should address all traffic characteristics for IoT/Wearable/eMBB, thus keeping the “ON” state power as low as possible is important for increasing the battery life of the device.

Observation 3: The latency brought by using LP-WUS/WUR consists of two parts: wake-up delay and MR transition time.

Observation 4: Compared to I-DRX with PEI, LP-WUR/WUS with continuous monitoring configuration can achieve 46%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, and limited latency increase.

Observation 5: Compared to eDRX with PEI, LP-WUR/WUS with continuous monitoring configuration can largely reduce the paging latency (23x), with comparable UE power consumption.

Observation 6: LP-WUR/WUS provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with legacy I-DRX and eDRX.

Observation 7: Battery life is inversely proportional to relative power of LP-WUR “ON” especially when the traffic is extremely sparse e.g., per UE paging rate is 0.001%.

Observation 8:  Continuous LP-WUS monitoring is not a feasible configuration when the relative power of LP-WUR “ON” state is higher than 1unit.

Observation 9: Duty cycled LP-WUS monitoring can achieve more than 90% power saving benefit compared to legacy I-DRX with PEI, except for the case when the relative power of LP-WUR “ON” is 10 or 20units. The latency increase due to duty cycle LP-WUS monitoring is marginal.

Observation 10: With LP-WUR “ON” state power lower than 10 units, duty cycled LP-WUS monitoring can achieve lower power consumption than legacy e-DRX with PEI.

Observation 11: Up to 45% power saving gain will be lost if UE main radio goes into deep sleep rather than ultra-deep sleep during LP-WUS monitoring.

Observation 12: The power consumption will increase when the LP-WUR “OFF” state power increases from 0.001units to 0.1units even with 1% duty cycle ratio.

Observation 13: With fixed duty cycle ratio and LP-WUR “ON” state power, when LP-WUR “ON” state power is no more than 1unit, different LP-WUR duty cycle periodicity have no or less impact on UE power consumption.

Observation 14: Latency will be reduced with the increase of duty cycle ratio of LP-WUS monitoring, while UE power consumption will increase accordingly.

Observation 15: When the duty cycle ratio is 10%, the achievable UE power saving benefit is significantly degraded if the LP-WUR “ON” state power is 10 or 20 units.

Observation 16: No power saving benefit by LP-WUR/WUS, if MR RRM measurement is not relaxed in RRC IDLE/INACTIVE modes. MR RRM measurement relaxation is an enabler to achieve power saving benefit by LP-WUR/WUS, where the MR RRM measurement relaxation includes to increase the MR RRM measurement period by X times, or to offload RRM from MR to LR.

Observation 17: No meaningful power saving gain can be obtained by duty cycled LP-WUS monitoring (with 1% duty cycle ratio) scheme, even with 20x MR measurement relaxation, when the LP-WUR “ON” power is 10 or 20 units.

Observation 18: Higher UE power saving benefit can be achieved if RRM measurement can be completely offloaded from MR to LR.

Observation 19: The power saving benefit of LP-WUR/WUS decreases when the FAR of LP-WUS is increased.

Observation 20:  The power saving benefit of LP-WUR/WUS decreases when the paging rate is increased.

Observation 21: RedCap UEs are more sensitive to the LP-WUR ON state power, i.e. the power saving benefit of LP-WUR/WUS decreases faster with the increase of LP-WUR ON state power than eMBB UEs.

Observation 22: In RRC connected mode, LP-WUS/WUR can be used to reduce the unnecessary PDCCH monitoring due to jitter while guarantee the XR performance (i.e. latency, capacity).

Observation 23: In RRC connected mode, LP-WUS/WUR can be used to achieve substantial UE power saving benefit without degradation of scheduling flexibility, latency and UPT of FTP traffic, compared with always-on monitoring.

Observation 24: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {7%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases for DL only XR traffic.

Observation 25: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {14%~25%} additional UE power saving gain, with acceptable capacity impact at least in low load case for DL only XR traffic.

Observation 26: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {8%~12%} additional UE power saving gain with no capacity loss in both low load and high load cases for DL+UL XR traffics.

Observation 27: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {18%~19%} additional UE power saving gain, with acceptable capacity impact at least in low load case for DL+UL XR traffics.

Observation 28: With MR micro sleep during LP-WUS monitoring by LR, same UPT performance as always-on scheme can be achieved, while the UE power consumption can be similar or lower than Rel-15 C-DRX.

Observation 29: When the WUR “ON” state power is low (e.g. no more than 1unit), LP-WUR/WUS combined with Rel-17 PDCCH skipping can achieve the best trade-off between UPT and UE power saving, among all the evaluated UE power saving schemes.

Observation 30: When the WUR “ON” state power is high (e.g. 20 or 30 units), LP-WUR/WUS combined with Rel-17 PDCCH skipping has no power saving gain over existing power saving schemes.

Observation 31: For RRC IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1% for 100MHz system BW or less than 4% for 20MHz system BW even in highest UE connection density for both IoT and eMBB cases.

Observation 32: For RRC CONNECTED mode, the resource overhead of LP-WUS is less than 6% even in high load case such as 10 XR UEs per cell.

| Resource overhead ratio, RLP-WUS | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic (10 users per cell, Maximum capacity) | eMBB FTP traffic (10 users per cell) |
| 20MHz, 30KHz SCS | 0.07% ~ 3.93% | - | - |
| 100MHz, 30KHz SCS | 0.01% ~ 0.79 % | 1.5%~6.0% | 0.125%~0.5% |

Observation 33: LP-SS in RRC IDLE/INACTIVE mode consumes no more than 0.32% of the system resource, even considering the most frequency LP-SS transmission i.e., 320ms LP-SS periodicity.

Observation 34: LP-SS is not needed for RRC connected mode UEs enabled with LP-WUR/WUS.

Observation 35: The additional network energy consumption for periodic LP-SS transmission is marginal, especially when LP-SS is multiplexed with SSB/SIB 1 in FDM manner.

| Multiplexing between LP-SS and SSB/SIB1 | The additional network energy consumption for periodic LP-SS transmission with a periodicity of 10240ms to 320ms |  |
| --- | --- | --- |
|  | NES power model CAT 1 (across different network loads) | NES power model CAT 2 (across different network loads) |
| FDM with SSB/SIB 1 | 0.002%~0.075% | 0.002%~0.059% |
| TDM with SSB/SIB 1 | 0.039%~1.724% | 0.031%~1.038% |

Observation 36: All LP-WUS configurations can achieve better MIL than PUSCH, for both non-RedCap and RedCap UEs.

Observation 37: For non-RedCap UEs, the sequence-based LP-WUS with 0.5ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-2) can achieve comparable MIL with NR PDCCH AL8 and MSG3 PUSCH.

Observation 38: For non-RedCap UEs, the “sequence+payload” based LP-WUS with 1ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-1) can achieve up to 5.5dB worse and 5dB worse MIL than NR PDCCH AL16 and MSG3 PUSCH, respectively.

Observation 39: For RedCap UEs, the sequence-based LP-WUS with 0.5ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-2) can achieve comparable MIL with NR PDCCH AL16.

Observation 40: For RedCap UEs, the “sequence+payload” based LP-WUS with 1ms transmission duration and 8.64MHz BW (i.e. LP-WUS config-1) can achieve up to 3dB worse MIL than NR PDCCH AL16 and comparable MIL than MSG3 PUSCH, respectively.

Proposal 1: The target relative power of LP-WUR “on” state should be less than 1 unit.

Proposal 2: Consider the following as the latency target for LP-WUS/WUR:

- Within 1 or 2 seconds for latency-sensitive IoT devices and within several or tens of seconds for other IoT devices;

- Within several seconds for wearable devices;

- Within several milliseconds for XR devices and within tens of milliseconds for other eMBB devices;

Proposal 3: Around 100kbps data rate can be considered as design target for LP-WUS.

Proposal 4: For sync/re-sync time and energy, assumptions shown in Table 1 can be considered.

Proposal 5: Transition time between LP-WUR ‘ON’ and ‘OFF’ states can be assumed as:

- 10ms, when the relative power of LP-WUR ON is more than 1 unit;

- 1ms or 5ms, when the relative power of LP-WUR ON is no more than 1 unit.

Proposal 6: The relative power consumptions of LP-WUR for OFDM-based signals/channels detection are given as

- LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring:

  - 10/20 units (reduced complexity OFDM receiver),

  - 30~35 units (normal OFDM receiver)

- LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring:

- WUR-OFF value 0=0.001unit (Note: XO is not in ‘active mode’, memory is not kept, only RTC is running)

- WUR-OFF value 1=0.05/0.1unit (Note: XO is in ‘active mode’, assuming XO option 1/2)

- WUR-OFF value 2=0.5unit (Note: XO is in ‘active mode’, assuming XO option 3/4)

Proposal 7: For LP-WUR power evaluation,

- Relative Power (unit) for LP-WUR OFF state: 0.001/0.05/0.1/0.5

- 0.001unit for receiver types based on envelope detection for MC-ASK and MC-FSK

- 0.05/0.1/0.5unit for receiver type based on OFDM

- Relative Power (unit) for LP-WUR ON state: 0.01/0.05/0.1/0.5/1/2/4/10/20/30

- 10/20/30 units for receiver type based on OFDM

- 0.01/0.05/0.1/0.5/1/2/4 units for receiver types for based on envelope detection for MC-ASK and MC-FSK

Proposal 8: For RRM measurement assumptions in RRC idle/inactive mode, the following options can be considered.

- Option 1: RRM measurement is only performed by MR as in legacy releases.

- Option 2: MR performs relaxed RRM measurement every X I-DRX cycles (e.g. X can be 10 or larger). Additionally, LP-WUR may also perform assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

- Option 3: MR does not perform RRM measurements and RRM measurement is completely offloaded to LP-WUR.

Proposal 9: It is recommended to support at least one of the following RRM measurement enhancements to achieve substantial UE power saving benefit by LP-WUR/WUS

- Relax the MR RRM measurement period by X times, where the X should be sufficiently large number, e.g. 20 or larger

- Offload the RRM measurements from MR to LR

Proposal 10: In RRC connected mode, ultra-deep sleep state of main radio should not be applied. And UE main radio can enter micro, light or deep sleep state during LP-WUS monitoring by LR.

Proposal 11: To reduce the MIL gap between LP-WUS and NR PDCCH or MSG3 PUSCH, coverage enhancement methods for LP-WUS such as power boosting, channel coding and more time/frequency domain resources can be studied.

- The same correction factor should be assumed for broadcast DL channel and Msg3;

- In FR1, the correction factor is about 8dB for 2.6GHz/4GHz; 0dB for 700MHz.

# References

- Chairman’s Notes of RAN1 #112bis-e (April 17th – April 26th, 2023).

- R1-2304501, “Remaining issues on low power wake-up receiver architecture”, vivo.

- Jaeho Im, Jacob Breiholz, Shuo Li, Benton Calhoun, David D. Wenzloff , “A Fully Integrated 0.2V 802.11ba Wake-Up Receiver with -91.5dBm_Sensitivity”, 2020, IEEE Radio Frequency Integrated Circuits Symposium.

- Renzhi Liu, Asma Beevi K. T., Richard Dorranc, Deepak Dasalukunte, etc. “An 802.11ba 495W -92.6dBm-Sensitivity Blocker-Tolerant Wake-up Radio Receiver Fully Integrated with Wi-Fi Transceiver”, 2019, IEEE Radio Frequency Integrated Circuits Symposium.

# Appendix A – Processing timeline for the schemes in RRC idle/inactive mode

The assumed processing timelines of I-DRX paging, eDRX and LP-WUS/WUR schemes.

# Appendix B – Evaluation assumption for RRC connected mode

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring can be assumed to 0ms and 3ms corresponding to MR enters micro or light sleep states during LP-WUS monitoring.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table 16. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table 17. The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |

Table 18. UL traffic models of XR

| Traffic model | Mean packet size (Bytes) | STD of packet sizes (Bytes) | Min packet size (Bytes) | Max packet size (Bytes) | Packet arrival interval (ms) | PDB (ms) | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pose/control stream | 100 | 0 | 100 | 100 | 16.67 | 10 | No |

# Appendix C – Evaluation Results on Coverage for LP-WUS

In the evaluation results shown in Appendix D, some common parameters are assumed, provided as follows

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Carrier Frequency | 2.6GHz |  |  |
| Case name | LP-WUS Config1 | LP-WUS Config2 | LP-WUS Config3 |
| Channel structure | sync: 16 chips data: 12bits(24 chips) CRC: 8 bits (16 chips) | Sequence only: 28 chips | Sequence only: 8 chips |
| Chip rate | 56kcps | 56kcps | 56kcps |
| WUS duration | 2 slots | 1 slot | 4 symbols |
| Performance metric | FAR <0.1% MDR 1% | FAR < 1% MDR 1% | FAR < 1% MDR 1% |
| Waveform | OOK -4 |  |  |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) |  |  |
| Time error | RTC 20ppm |  |  |
| Frequency error | {200 ppm,0.1ppm/s} , in saturated region |  |  |
| Beacon periodicity | 1.28 sec Note: beacon periodicity is used to calculate the time drift for WUS monitoring |  |  |
| SCS | 30kHz |  |  |
| gNB Channel BW | 20MHz (50 RB) |  |  |
| WUS BW | 4.32MHz /8.64MHz |  |  |
| Guard band | 1RB on each side of LP-WUS bandwidth |  |  |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |  |  |
| ASCI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |  |  |
| Sampling Rate | 3.84 MHz |  |  |
| ADC bitwidth | 4 bit ADC |  |  |
| Channel Model | TDL-C 300 |  |  |

Detailed MIL calculation for LP-WUS for above configurations, are provided in the following attachment which is calculated according to the template.

# Appendix D – Evaluation assumptions on network energy consumption

- Assumption on network load

| Zero load | Low load | Light load | Medium load |
| --- | --- | --- | --- |
| NaN | DL FTP3 traffic with 0.5Mbytes packet size and 200ms mean packet interval |  |  |
| 10 UEs per cell | 2 UEs per cell | 6 UEs per cell | 10 UEs per cell |

- Assumption on BS power model

- Reference configuration

|  | Set 1 FR1 |
| --- | --- |
| Duplex | TDD |
| System BW | 100 MHz |
| SCS | 30 kHz |
| Number of TRP | 1 |
| Total number of DL TX RUs | 64 |
| Total DL power level | 55dBm |
| Total number of UL Rx RUs | 64 |

- BS power consumption model

| Power state |  | Relative Power P for Category 1 | Relative Power P for Category 2 |
| --- | --- | --- | --- |
|  |  | Set 1 FR1 | Set 1 FR1 |
| Deep sleep | P1 | 1 | 1 |
| Light sleep | P2 | 25 | 2.1 |
| Micro sleep | P3 | 55 | 5.5 |
| Active DL | P4 | 280 | 32 |
| Active UL | P5 | 110 | 6.5 |

- Assumption on simulation parameters

|  |  | Parameters |
| --- | --- | --- |
|  |  | Set 1 |
| Basic parameters | Channel model | 3D-Uma as in TR 38.901 (low-loss O2I penetration model) |
|  | Percentage of high loss and low loss building type | 100% low loss |
|  | Device deployment | 80% indoor, 20% outdoor |
|  | Inter-site distance | 500m |
|  | Network Topology | 7*3 Sector |
|  | Carrier Frequency | 4.0GHz |
|  | Multiple access | OFDMA |
|  | Duplexing | TDD |
|  | Numerology | 30kHz, 14 OFDM symbol slot |
|  | Guard band ratio on simulation bandwidth | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) |
|  | Simulation bandwidth | 100 MHz |
|  | Frame structure | DDDSU (S slot is assumed as 10D:2G:2U) |
|  | UT attachment | Based on RSRP |
|  | Wrapping around method | Geographical distance based wrapping |
| BS parameters | BS antenna height | 25 m |
|  | BS noise figure | 5 dB |
|  | BS Tx power | 55 dBm |
|  | BS antenna element gain | 8 dBi |
|  | Antenna configuration at TRxP | For 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8). based on 38.802 |
| UE parameters | UE power class | 23dBm |
|  | UE noise figure | 9 dB |
|  | UE antenna element gain | 0 dBi |
|  | UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
|  | Antenna configuration at UE | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Transmission parameters | Modulation | Up to 256 QAM |
|  | Transmission scheme | SU-MIMO |
|  | SU dimension | For 4Rx: Up to 4 layers |
|  | DL CSI measurement | Precoded CSI-RS based |
|  | DL codebook | non-PMI transmission |
|  | Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
|  | Scheduling | PF |
|  | Receiver | MMSE-IRC |
|  | Channel estimation | Non-ideal |
|  | HARQ scheme | Ideal |
|  | Max HARQ retransmission | 3 |
|  | Target BLER | 10% of first transmission |
|  | Power control parameters | Open loop, P0=-80dBm, alpha=0.8 |
| Common RS | SSB period | 20ms |
|  | SS blocks per SSB burst | Up to 8 |
|  | SSB time resource | 4 symbols for each SSB |
|  | SSB frequency resource | 20 RBs |