3GPP TSG RAN WG1 #113			R1-2306158

Incheon, Korea, May 22nd- May 26th, 2023

Agenda item:	9.10.2

Source: 	Moderator (CATT)

Title: 	Moderator summary on timing advance management for LTM: Round 3

Document for:	Discussion and Decision

- Introduction

In RAN #94e, the Rel-18 WID of Further NR mobility enhancements are approved [1]. In the approved WID, Timing Advance management is a part of RAN1 objectives,

To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

This summary includes the following:

- Summary of companies’ views on each of open issues raised by interested companies

- Observations and recommended proposals based on the summary of companies’ views

- Issue 1: TA acquisition of RACH-based solutions

Open issues on RACH-based solutions for TA acquisition of candidate target cell(s) and company views are summarized as follows.

Table 1. Summary of views on issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.1 | When reception of RAR is configured, the random access response window for inter-DU multi-TRP operation - Alt1: Postpone the starting point of the random access response window - Alt2: Extend the length of the random access response window - Alt 3: length and offset of the starting point of RAR window can be configured by RRC | Alt1: vivo Alt2: Spreadtrum, Ericsson Alt1 or Alt2: ZTE, CATT(up to RAN2/RAN3), Lenovo, LGE Alt 3: QC |
| 1.2 | When reception of RAR is configured, the container of RAR - Alt1: MAC PDU(like RAR MAC PDU in 4 step RACH) - Alt2: MAC CE(like absolute TAC in MsgB in 2step RACH) - Alt3: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI | Alt1: Spreadtrum, CATT, Nokia, Samsung(1st preference) Alt2: Apple Alt3: Huawei, vivo, Ericsson, NTTDoCoMo, Samsung(2nd preference), OPPO |
| 1.3 | When reception of RAR is configured, whether candidate cell ID is contained in RAR - Alt 1: Don’t need - Alt 2: identification of candidate cell is contained in RAR | Alt1: ZTE, vivo, Spreadtrum Alt2: Huawei, Samsung, CATT, CMCC, Nokia, OPPO |
| 1.4 | When reception of RAR is configured, QCL assumption for PDCCH scheduling RAR PDSCH and RAR PDSCH | - RAR PDSCH is QCLed with PDCCH scheduling RAR PDSCH: Nokia, Samsung, LGE - PDCCH scheduling RAR PDSCH is QCLed with corresponding PDCCH order: Spreadtrum, CATT, DoCoMo, ITRI |
| 1.5 | Power ramping - Alt1: UE performs power ramping based on a field explicitly indicating initial transmission or retransmission of PRACH in PDCCH order - Alt2: UE performs power ramping based on implicit indication in PDCCH order, e.g. same SSB index, preamble index, RO index etc | Alt1: Huawei, CATT, Spreadtrum, Ericsson, CAICT, CMCC, Nokia, LGE, OPPO, MTK, Transsion Holdings, FGI, Interdigital, ZTE, MTK, Samsung Alt2: ZTE, MTK |
| 1.6 | PL-RS for PRACH Tx power determination | For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination QC, Nokia |
| 1.7 | Overlapping of PARCH transmission to a LTM candidate cell and serving cell UL transmission - Alt1: Overlapping of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell can be avoided by NW implementation - Alt2: PDCCH ordered PRACH transmission for TA acquisition should not have a higher priority than the normal UL transmitted in the serving cell - Alt3: the PRACH transmission is prioritized and the overlapped CG-PUSCH/RRC-configured UL transmission on serving cell is dropped | Alt1: Huawei, CATT, vivo, Ericsson, NTTDoCoMo, MTK, ITRI, Spreadtrum Alt2: CMCC Alt3: Apple, MTK |
| 1.8 | Whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell) | Alt1: Huawei, Spreadtrum, CMCC, OPPO, ITRI, Interdigital Alt2: Ericsson, Nokia, Apple Alt3: Nokia, DoCoMo |

Companies are encouraged to show views/comments/suggestions on the following proposals.

P1-1: RAR window(closed)

Round 1

Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window for inter-DU multi-TRP operation

- Alt1: Postpone the starting point of the random access response window

- Alt2: Extend the length of the random access response window

- Alt 3: Length and offset of the starting point of RAR window can be configured by RRC

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | For Proposal 1-1, we prefer Alt3, which is most flexible for gNB to control |
| Futurewei | All the alternatives are feasible. Prefer Alt2: the simplest one. |
| NTT DOCOMO | Support in principle. However, we are not sure why multi-TRP operation is included in this proposal. In our understanding, multi-TRP operation has not been supported yet for R18 LTM. Thus, this discussion should be separate. We suggest deleting ‘multi-TRP’ in the main bullet: Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window for inter-DU scenario multi-TRP operation …. |
| Nokia | Agree with NTT, we also suggest removing the “multi-TRP” in the main proposal? We support Alt 3 (configuration of offset and length will be given in the RRC) |
| Samsung | Agree to NTT, and we support Alt 3. |
| Lenovo | Agree to NTT to remove multi-TRP, and support Alt3. |
| vivo | Agree with DOCOMO and Nokia that “multi-TRP” should be removed from the main bullet. For the proposed alternatives, we slightly prefer Alt1. |
| Xiaomi | Fine. And we prefer alt.2. |
| ZTE | Prefer Alt 1. From our point of view, we have no identify clear requirement to extend the length together with postpone the starting point. |
| Mod | @DOCOMO @Nokia: Thanks for the correction. It’s a typo. Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window for inter-DU scenario - Alt1: Postpone the starting point of the random access response window - vivo, ZTE - Alt2: Extend the length of the random access response window - Supported by: Futurewei, Xiaomi - Alt 3: Length and offset of the starting point of RAR window can be configured by RRC - Supported by: QC, Nokia, Samsung, Lenovo, ITRI |
| LGE | Fine with the proposal and support Alt 3. |
| Ericsson | OK, with comments: - Note that we don’t agree “for inter-DU”, this is up to NW configuration. - Just to clarify: this is separately configured from the normal RAR window? |
| ITRI | Agree with NTT, and we prefer Alt 3 |
| Spreadtrum | It is important to have unified RAR window for inter and intra-DU case. For P1-1, does it mean to have two RAR window, one for inter-DU and another for inra-DU? |
| IDCC | We are fine and prefer Alt 3. |
| Mod | The reason why this proposal is confined only for inter-DU case is just because the following agreement achieved in the last meeting, where the FFS is under inter-DU case. However, a unified solution for both inters and intra-DU cases seems to be more reasonable. Agreement When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues Regarding whether a separate configuration of RAR window is needed for candidate cell(s), further discussion is necessary. So, P1-1 is revised as follows. Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window for inter-DU scenario - Alt1: Postpone the starting point of the random access response window - Supported by (2): vivo, ZTE - Alt2: Extend the length of the random access response window - Supported by (2): Futurewei, Xiaomi - Alt 3: Length and offset of the starting point of RAR window can be configured by RRC - Supported by (7): QC, Nokia, Samsung, Lenovo, ITRI, LGE, IDCC [Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.] |
| CMCC | Agree with NTT, it should be focused on “inter-DU” following the discussion. Alt 3 is preferred. From our understanding, this should be a different RAR from the current one. Configurable offset and length can provide additional |
| Huawei, HiSilicon | The proposal can be discussed after P1-2. If MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, we do not think RAR window is necessary, as it is usually used for UE to identify the MOs of the search space.. |
| FGI | We are fine with the revised Proposal 1-1. And we prefer Alt 3. |
| Mod | Based on HW’s comment, one FFS is included in the following revision for further discussion. Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window for inter-DU scenario - Alt1: Postpone the starting point of the random access response window - Supported by (2): vivo, ZTE - Alt2: Extend the length of the random access response window - Supported by (2): Futurewei, Xiaomi - Alt 3: Length and offset of the starting point of RAR window can be configured by RRC - Supported by (8): QC, Nokia, Samsung, Lenovo, ITRI, LGE, IDCC, FGI [Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.] [FFS: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, whether RAR window is still necessary] |

Round 2

| Company | Input |
| --- | --- |
| Mod | Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window - Alt1: Postpone the starting point of the random access response window - Supported by (2): vivo, ZTE - Alt2: Extend the length of the random access response window - Supported by (2): Futurewei, Xiaomi - Alt 3: Length and offset of the starting point of RAR window can be configured by RRC - Supported by (8): QC, Nokia, Samsung, Lenovo, ITRI, LGE, IDCC, FGI [Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.] [FFS: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, whether RAR window is still necessary] |
| Lenovo | We prefer Alt3. |
| Huawei, HiSilicon | We think if P1-2 is deferred after RAN2 decision, P1-1 should also be deferred as they are related. We also think the FFS point in the bracket should be listed Alt 4 if FL hope to collect all alternatives for further discussion |
| NTT DOCOMO | We support the latest proposal and prefer Alt3. |
| Transsion | Support the latest proposal and prefer Alt1. |
| Futurewei | Either RAR or MAC CE is PDCCH scheduled data. We only need a time threshold (the upper limit of so called “response window”), after the time exceeds the threshold, if the RAR or MAC CE is still not arrived, then the RAR or MAC CE is considered lost. Therefore, an increased time threshold – enlarged “window” should serve the purpose. It is not clear for me why we care about the starting point of the “window.” It is not a window for signal detection. If the proponents can provide a reason for setting the starting point of the window, we don’t mind to adopt Alt 3. So far, we think Alt2 is good enough. Either at the source cell or at the UE, we need a time threshold to determine whether the TA response from the target cell is failed. |
| Xiaomi | Considering the additional time for the cooperation between candidate DU and serving DU, the straightforward way is to extend the length of RAR. While, we are still OK with Alt.3 if it is most companies’ preference. |
| Mod | @HW: thanks for the comment. Regarding whether RAR response window is needed in the case MAC CE is used, as mentioned at least by Futurewei, it’s still possible to have such window. However, it’s fine to include the FFS as one of the alternatives for further discussion. @Futurewei: thanks for sharing your understanding. In my opinion, the motivation of introducing such offset is based on the consideration that the TA value needs to be transmitted from candidate cell to serving cell. Therefore, the starting point of the window can be delayed to make sure that RAR can be received within the window. P1-1 is revised as follows for further discussion. Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window - Alt1: Postpone the starting point of the random access response window - Supported by (3): vivo, ZTE, Transsion - Alt2: Extend the length of the random access response window - Supported by (2): Futurewei, Xiaomi - Alt 3: Length and offset of the starting point of RAR window can be configured by RRC - Supported by (10): QC, Nokia, Samsung, Lenovo, ITRI, LGE, IDCC, FGI, DOCOMO, Xiaomi - Alt 4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed - Supported by (1): HW [Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.] |
| Samsung | We are O.K. with latest FL proposal. |

Note:

From RAN1 perspective, when reception of RAR is configured, there may have 4 alternatives to determine the randomaccess response window

- Alt1: Postpone the starting point of the random access response window

- Alt2: Extend the length of the random access response window

- Alt3: Length and offset of the starting point of RAR window can be configured by RRC

- Alt4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed

[Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.]

P1-2: container of RAR(closed)

Round 1

Proposal 1-2: When reception of RAR is configured, discuss and down-select among the following alternatives for RAR reception:

- Alt1: MAC PDU(like RAR MAC PDU in 4 step RACH)

- Alt3: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | For Proposal 1-2, support Alt3, which has no RA-RNTI, CSS, and Msg3 issues |
| Futurewei | We are fine with either alternative. |
| NTT DOCOMO | We are OK with FL proposal. |
| Nokia | RAN1 does not need to down select the container of the RAR. We should leave it to RAN2. We can just focus on the content of the RAR (next proposal). |
| Samsung | We prefer Alt 3. |
| Lenovo | We support Alt1 which can reuse the legacy procedure as much as possible. |
| vivo | Share similar views with Nokia. The design of MAC signaling which at least carries the TA value of the candidate cell is RAN2’s work, and RAN2 is also discussing this issue in this meeting. It would be better to wait for the response from RAN2. |
| Xiaomi | Support alt.3 |
| ZTE | Support Alt 1. In legacy, PDCCH ordered CFRA procedure completes after RAR reception, and there are no further considerations on Msg3 issues. The same case happens for LTM, it is confusing why we need to focus on Msg3 issues. Besides, we understand this issue should be discussed and determined by RAN2, not RAN1. |
| LGE | Fine with the proposal. |
| Ericsson | We prefer Alt3, since the transmission over L1 is more efficient, but the main responsibility lies with RAN2. |
| ITRI | Prefer Alt 3. |
| Spreadtrum | Alt 1 can work, do not know why new design is needed. |
| IDCC | We prefer Alt 3. |
| Mod | Views/positions of companies are summarized as follows. Proposal 1-2: When reception of RAR is configured, discuss and down-select among the following alternatives for RAR reception: - Alt1: MAC PDU(like RAR MAC PDU in 4 step RACH) - Supported by(3): Lenovo, ZTE, Spreadtrum - Alt3: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI - Supported by(8): QC, Samsung, Xiaomi, Ericsson, ITRI, IDCC, CMCC, HW - Up to RAN2 design - Supported by(2): Nokia, vivo |
| CMCC | Alt 3 is preferred, since currently only the TA and related candidate cell information are needed. Then there would be some redundant part if we reuse the legacy MAC PDU like RAR MAC PDU. |
| Huawei, HiSilicon | Alt 3 is preferred. In Alt 3, it should be clarified whether the regular USS or a separate SS like that in BFR is used. We preferred the former. |

Round 2

| Company | Input |
| --- | --- |
| Mod | As pointed out by at least Nokia and vivo, RAN2 is also discussing the issue of RAR container. If it’s also agreeable to other companies, I’d suggest we leave this issue to RAN2. |
| LGE | We are OK to leave it to RAN2. However, it should be noticed that P1-1/P1-3/P1-4 may depend on the consequence of the Proposal 1-2. |
| Lenovo | OK to leave it to RAN2. |
| Huawei, HiSilicon | Fine to see RAN2 conclusion at first |
| NTT DOCOMO | We are OK with FL proposal. |
| Transsion | We support Alt1 and also fine to go with majority view. |
| Futurewei | We support FL proposal. To us, either way works and either way has pro and cons. Consider RAN1 only care about what to be delivered and where to deliver in this case, probably we can let RAN2 to decide. We prefer UP to RAN2 design – it really is MAC design. |
| Xiaomi | OK to leave it to RAN2. |
| Mod | As shown below, P1-2 is updated for further discussion. Proposal 1-2 (proposed conclusion): When reception of RAR is configured, the following alternatives for RAR reception have been discussed in RAN1. It’s up to RAN2 design on how RAR reception is supported. - Alt1: MAC PDU(like RAR MAC PDU in 4 step RACH) - Alt3: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI |

P1-3: content of RAR(closed)

Round 1

Proposal 1-3: On whether candidate cell ID shall be contained in RAR when reception of RAR is configured, discuss and down-select among the following alternatives:

- Alt 1: Don’t need

- Alt 2: Identification of candidate cell is contained in RAR

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Prefer Alt1. Based on legacy rule, there is only 1 ongoing RACH procedure at each time. So it seems no need candidate cell ID. |
| Futurewei | Slightly prefer Alt2 for flexibility on timing of starting simultaneous early RACH.. |
| NTT DOCOMO | Support in principle. However, we may need to discuss whether one or multiple RACH(s) can be ongoing before this discussion. If only one RACH is always ongoing, indication of candidate cell may not be needed because UE can identify candidate cell for which TA information included in RAR is. |
| Nokia | Are we going to discuss additional proposal on other parameters (e.g., UE-ID)? If not, we suggest to simply update this proposal to consider what exactly is needed in the RAR from RAN1 perspective which will help RAN2 and may eliminate the need to back and forth LSs to clarify details on different parameters. We suggest the following rewording: Proposal 1-3: In addition to TA, on whether candidate cell ID shall be contained in RAR when reception of RAR is configured/indicated, discuss and down-select among the following alternatives: - Alt 1: Don’t need - Alt 2: Identification of candidate cell is contained in RAR We support Alt1, and then leave the further details on the RAR reception mechanism to RAN2. |
| Samsung | Support Alt 1 |
| Lenovo | Similar view with Docomo that it depends on whether multiple RACHs can be ongoing. |
| Vivo | Support Alt1. |
| Xiaomi | Support alt.1 |
| ZTE | Support Alt 1. |
| Mod | @Nokia: thanks for the question and suggestion. Actually, based on views of companies in their contributions, UE ID is mentioned nobody. So, I don’t think additional discussion on parameter like UE-ID is needed. Based on comments from Nokia, QC, DOCOMO and Lenovo, P1.3 is updated as follows. Proposal 1-3: In addition to TA, on whether candidate cell ID shall be contained in RAR when reception of RAR is configured, discuss and down-select among the following alternatives: - Alt 1: Don’t need - Supported by(13): QC, [DOCOMO], Nokia, Samsung, vivo, ITRI, Xiaomi, ZTE, LGE, ITRI, Spreadtrum, IDCC, FGI - Alt 2: Identification of candidate cell is contained in RAR - Supported by(2): Futurewei, Ericsson Note: there is only 1 ongoing RACH procedure at each time. |
| LGE | Support Alt 1. |
| Ericsson | Support Alt2. This would make the procedure more robust. |
| ITRI | Support Alt1. |
| Spreadtrum | Alt 1 |
| IDCC | Alt 1. |
| Mod | As shown above, views/positions are updated. |
| CMCC | If it is limited under that only 1 ongoing RACH procedure at each time, Alt 1 is preferred. But we still have questions on whether this TA acquisition should belong to the RACH procedure. |
| Huawei, HiSilicon | It depends on whether RAR is defined. If not defined, cell ID is necessary as parallel RAR reception may happen. |
| FGI | Support Alt 1. Since we already agreed that the reserved bit(s) in DCI format 1_0 can be used for indication of cell identity, further indicating the cell identity in the RAR is redundant. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Based on majority view shown in round-1 discussion, P1-3 is revised as follows. Proposal 1-3: In addition to TA, candidate cell ID is not needed in RAR when reception of RAR is configured. |
| LGE | One clarification question: Candidate cell ID indication is needed if Alt3(MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI) in P1-2 is supported, in order to indicate target of TA indication in the TAC MAC CE? If so, it would depend on outcome of P1-2. |
| Lenovo | OK with the latest Proposal. |
| Huawei, HiSilicon | We do not support this proposal. It is related to the RAR window discussion. |
| NTT DOCOMO | We support the FL proposal. |
| Transsion | We support the FL proposal. |
| Futurewei | We still think Alt 2 is better. Alt 1imposes restriction on when to start next PDCCH order. Consider there could be preamble retransmissions, one particular PDCCH order could last long time to finish. If before it is finished, we don’t allow serving cell to issue another order, it will introduce too much delay to get TAs from different candidates. Therefore, we suggest companies reconsider this issue. We understand Alt1 is simple, but consider there could be multiple candidates need early RACH and the delay impact, Alt2 worth the effort. |
| Xiaomi | Fine |
| Mod | @LGE: no matter TA is carried in MAC PDU or MAC CE, as stated in the note in P1-3 of round-1 discussion, if there is only 1 ongoing RACH procedure at each time, candidate cell ID may not needed. However, as mentioned by Futurewei, if more than one RACH procedures in overlapped period of time are allowed, such ID is still needed. @Futurewei:I can understand the intention to further reduce latency of RAR receptions of more than one candidate cells in Alt 2. We can further check the positions of companies in this round of discussion. Companies are encouraged to show your opinions on the following proposal. Proposal 1-3: In addition to TA, on whether candidate cell ID shall be contained in RAR when reception of RAR is configured, discuss and down-select among the following alternatives: - Alt 1: there is only one ongoing RACH procedure at each time, and the identification of candidate cell is not needed - Supported by: - Alt 2: more than one RACH procedures are allowed at each time, and the identification of candidate cell is contained in RAR - Supported by(1): Futurewei |
| Samsung | I wonder RAN1 can decide to support multiple of simultaneous PRACH process. We propose back to previous version. According to my RAN2 colleague, another RACH cannot be triggered till the previous process ends. |

Note:

In addition to TA, when reception of RAR is configured, RAN1 discussed the following alternatives:

- Alt 1: when there is only one ongoing RACH procedure at each time, the identification of candidate cell is not needed

- Alt 2: when more than one RACH procedures are allowed at each time, the identification of candidate cell is contained in RAR

P1-4: QCL(postponed)

Round 1

Proposal 1-4: On the QCL assumption for PDCCH scheduling RAR PDSCH and RAR PDSCH.

- PDCCH scheduling RAR PDSCH is QCLed with corresponding PDCCH order

- RAR PDSCH is QCLed with PDCCH scheduling RAR PDSCH

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Suggest to use same rule/wording for legacy PDCCH order. Proposal 1-4: On the QCL assumption for PDCCH scheduling RAR PDSCH and RAR PDSCH. - If PDCCH order is sent from SpCell, PDCCH scheduling RAR PDSCH is QCLed with has same QCL properties as those of corresponding PDCCH order - If PDCCH order is sent from SCell, PDCCH scheduling RAR PDSCH has same QCL properties as those of the CORESET associated with the Type1-PDCCH CSS set - RAR PDSCH is QCLed with PDCCH scheduling RAR PDSCH If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for a secondary cell, the UE may assume the DM-RS antenna port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set for receiving the PDCCH that includes the DCI format 1_0. |
| Futurewei | We are fine with the principle of Proposal 1-4 |
| NTT DOCOMO | Support in principle. |
| Nokia | The first bullet will depend on how the RAR is sent, i.e., using the type-1 CSS or USS. We can discuss this once the RAN2 makes agreement on the RAR container. |
| Samsung | We see association between this issue and 1-2. We suggest to postpone the discussion till down-selection is done for proposal 1-2. And we also prefer to rely on legacy rule for QCL. |
| Lenovo | We are fine with the principle. |
| vivo | Suggest postponing the discussion on Proposal 1-4 until the related discussion on Proposal 1-2 is complete in RAN2, since the consequence of Proposal 1-2 will affect the validity of Proposal 1-4. If carried by MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI, the QCL assumption of PDCCH and scheduled PDSCH should follow the indicated unified TCI state, and Proposal 1-4 is not necessary to discuss. |
| Xiaomi | Fine with proposal 1-4. |
| ZTE | Since PDCCH order and RAR are all from serving cell, we think the legacy QCL assumption can be reused. |
| LGE | As QC mentioned, the legacy QCL rule can be reused for PDCCH scheduling RAR PDSCH(e.g., either QCLed with PDCCH order or QCLed with the CORESET associated with the Type-1 CSS). Regarding RAR PDSCH, the legacy rule is that RAR PDSCH is QCLed with the SSB indicated by PDCCH order, which is not applicable for Rel-18 LTM since RACH transmission is toward candidate cell and RAR is transmitted by serving cell. So, we support the Proposal 1-4. |
| Ericsson | We are fine with P1-4. Note that if the RAR is not transmitted using Type1-CSS, it would seem strange to rely on that statement |
| ITRI | Support this proposal. |
| Spreadtrum | Support |
| IDCC | Support |
| CMCC | Support the proposal. |
| Huawei, HiSilicon | Share similar views as previous comments to defer the discussion. It relates to the SS scheduling the RAR. |

Round 2

| Company | Input |
| --- | --- |
| Mod | As mentioned by Nokia, vivo and HW, this issue can be revisited after related aspects are settled. |
| NTT DOCOMO | We are OK with FL proposal. |
| Transsion | Support the FL proposal. |
| Futurewei | Fine with the FL proposal. |
| Xiaomi | OK to postpone the discussion. |

P1-5: closed

Round 1

Proposal 1-5: On the determination of the PRACH transmission power when reception of RAR is not configured, a field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Should we say a “1-bit” field if that is the common understanding? |
| Futurewei | Support FL Proposal 1-5. |
| NTT DOCOMO | Support. |
| Nokia | Support the explicit indication. However, even after this proposal, we need to clarify other details on - Number of bits used for the indication - Power ramping formula to be used and whether it is counter based or just one step power ramping. If counter based, then the details on counter functionality, i.e., which layer maintains it and when the counter is increased and reset. |
| Samsung | Support |
| Lenovo | Ok with this proposal |
| vivo | Support |
| Xiaomi | OK |
| ZTE | Support |
| Mod | Revised based on comments above. Proposal 1-5: On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported. - [UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved. - The UE applies the power ramping for any subsequent re-transmissions for the PRACH preamble with the same target SSB - If UE receives a PDCCH order with a retransmission indication for a candidate cell, without receiving a prior PDCCH order with an initial transmission, the UE uses the initial value for the power ramping counter (i.e., counter = 1)] |
| Ericsson | Support the original version. We already agreed to have an explicit indicator. The relation to an SSB is not relevant: the UE follows the command. |
| ITRI | Support |
| Spreadtrum | We can agree with the main bullet first, and go with the subbullets next. At least the following can be studied: - For first sub-sub-bullet, - it would be clearer to say counter based. - Does it suggest same SSB + re-tx indication would lead to count +1, all the other fields can be same or different? Including different PRACH preamble and PRACH Mask index. For second sub-sub bullet, is it a kind of error case? |
| IDCC | We are fine with the first part of the proposal. But we need to discuss how the 1-bit is applied. One option is that the power ramping counter is controlled by the 1-bit, i.e., the value of the counter is incremented or reset with the 1-bit. Another option is that an additional power is added to the calculated power where for the calculated power power ramping counter is fixed at 1. We prefer the first option as it is more straightforward. Another issue is to how and when to reset the counter even for the same SSB. We think we can use a timer (e.g., a validity timer); when the timer expires, the counter is reset. This can be useful when a DCI with bit =0 is missed. |
| Mod | Thanks for the comments. To be safe, let’s try to take a smaller step first. Proposal 1-5: On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported. @Spreadtrum: regarding the second sub-sub bullet in the second version of P1-5, it’s possible that the PDCCH indicating initial PRACH transmission is not received correctly. So, the case mentioned there still needs to be considered. |
| CMCC | We are fine with the updated version. But we are confused with the last bullet part. It seems trying to solve a error case, which should not current stage’s focus. |
| Huawei, HiSilicon | Support the latest P1-5 by FL. |
| FGI | Support the latest revised proposal. It is enough to use the one-bit field included in the DCI to indicate whether the scheduled PRACH is the initial PRACH transmission or the PRACH retransmission. |

Agreement

On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported.

P1-5-1: power ramping

Round 2

Proposal 1-5-1: On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH

- UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved

- FFS: whether/how to reset the counter

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Lenovo | OK with the proposal. |
| Nokia | Support |
| Huawei, HiSilicon | Support in general. Not sure what is the counter in FFS used for? |
| NTT DOCOMO | Support. |
| Transsion | Support |
| Futurewei | Support |
| Xiaomi | OK |
| Mod | @HW: as mentioned by IDCC, a timer (e.g., a validity timer) can be considered if the reset of counter is supported. When the timer expires, the counter is reset. This can be useful when a DCI with bit =0 is missed. Another example is that the PDCCH indicating initial PRACH transmission is not received correctly, while the subsequent PDCCH indicates a retransmission of PRACH, the counter can also be reset. |

Round 3

| Company | Input |
| --- | --- |
| Lenovo | Fine with this proposal |
| IDCC | Fine with the propossal |
| ZTE | Thanks FL for clarification on FFS. Based on these examples on FFS listed by FL and IDCC, we understand that it mainly discusses missing detection and false detection issue on DCI. For these cases, we can separately discuss it later and first focus on the scenario where DCI can be correctly received. Since it is FFS, we are fine with current proposal. |
| Futurewei | Support the proposal |
| QC | For the counter reset, our view is that the previously increased power boost should reset to zero if different candidate cell and/or SSB are indicated in the PDCCH order, or if the PDCCH order is for initial transmission The issue is unnecessary power boost due to DCI miss detection. For example, suppose UE has received 3 retrans indicators for SSB 1 of cell 1 before, and UE currently receives the 1st retrans indicator for SSB 2 of cell 2 but with corresponding initial trans indicator missed for SSB 2 of cell 2. In this case, the additional power boost for the 1st retrans for SSB 2 of cell 2 will be 4*X dB, instead of the desired X dB. The unnecessary power increase can be avoided by resetting the power increase counter if the candidate cell or indicated SSB changes. |
| NTT DOCOMO | Support in principle. |
| Mod | Offline agreement Proposal 1-5-1: On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter |
| Mod | Based on what we have agreed in offline session on Wednesday, my suggestion is to take use of the rest of this week and further discuss the procedure of power ramping/resetting in more details. Based on inputs of companies, the following procedure is summarized: For a candidate cell, on the determination of the PRACH transmission power when reception of RAR is not configured, except for the following cases, UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission: - Case 1: the PDCCH order is for initial transmission. In this case, power ramping counter is reset to 1. - Case 2: the max allowed power is achieved. In this case, FFS the exact Tx power. - Case 3: PDCCH indicating a retransmission of PRACH is received without reception of previous PDCCH indicating initial transmission. In this case, power ramping counter is reset to 1. Please provide your comments on the above procedure. And then, if it’s stable enough, I can draft a proposal for further discussion/decision. |
| Xiaomi | Add another case that UE won’t increase the power. For a candidate cell, on the determination of the PRACH transmission power when reception of RAR is not configured, except for the following cases, UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission: - Case 1: the PDCCH order is for initial transmission. In this case, power ramping counter is reset to 1. - Case 2: the max allowed power is achieved. In this case, FFS the exact Tx power. - Case 3: PDCCH indicating a retransmission of PRACH is received without reception of previous PDCCH indicating initial transmission. In this case, power ramping counter is reset to 1. - Case 4: The ssb index indicated in PDCCH order is changed, counter is reset to 1. |

P1-6: closed

Round 1

Proposal 1-6: For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Support. Good to add a Note to ensure it has been periodically measured for path loss calculation. Ideally, we prefer gNB to explicitly indicate which SSB will be used for path loss computation for the best path loss accuracy Proposal 1-6: For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination. - Note: UE expects the above SSB is currently measured via L3 or L1 periodic measurement |
| Futurewei | Fine with FL Proposal 1-6 |
| NTT DOCOMO | Support. |
| Nokia | Support |
| Samsung | Support |
| Lenovo | Support |
| vivo | Support |
| Xiaomi | Support |
| ZTE | Support |
| LGE | OK in principle. |
| Ericsson | Support |
| ITRI | Support |
| Spreadtrum | Support |
| IDCC | Support |
| CMCC | Support. |
| Huawei, HiSilicon | Support |
| FGI | Support |

Agreement

For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination.

P1-7: overlapping (discuss together with 1-8)

Round 1

Proposal 1-7: Overlapping of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell can be avoided by NW implementation.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Support, since gNB can select the PRACH Tx time not affecting any traffic |
| Futurewei | Support FL Proposal 1-7 |
| NTT DOCOMO | Support. |
| Nokia | Not sure if we should define the rules based on dynamic vs configured scheduled transmission in serving cell. We agree that NW can control overlap in some certain scenarios, but it cannot always guarantee that overlap would not happen, as it will be more of an issue when a candidate cell is not one of the current serving cells, in those case, the current serving cell may not have good knowledge on how long the PRACH transmission to a non-serving candidate cell will take. We should simply define what UE should do when the PRACH transmission to a non-serving candidate cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with a following UL transmission to one of the serving cells: - PRACH transmission - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI - SRS transmission - Any other PUCCH/PUSCH transmission We think that at least PRACH transmission to a serving cell should be prioritized as this may be triggered due to BFR and it should be important to recover it as soon as possible. |
| Samsung | Support |
| Lenovo | Support |
| vivo | Support. |
| Xiaomi | Agree with Nokia. Whether this can be guaranteed by NW all time is not sure. It is better to define the UE behaviors when the collision happens. |
| ZTE | Seems that this issue cannot be avoided by NW implementation considering that Downlink timing of different candidate cells can be different, accordingly DL reference timing for PRACH transmission is not aligned. Hence it might be impossible to up to gNB to avoid physical uplink overlapping all the time. |
| LGE | Support. |
| Ericsson | Agree with Nokia. We don’t see that it is always possible to avoid this by scheduling, especially in an inter-DU scenario. |
| ITRI | Support |
| Spreadtrum | In legacy, if UE does not simultaneously transmit PRACH and PUSCH/PUCCH/SRS in intra-band CA, including overlapping, or the gap between the end of PRACH (PUSCH/PUCCH/SRS) and the start of PUSCH/PUCCH/SRS (PRACH) is smaller than a threshold, it up to UE implementation. Thus for the second issue of PARCH transmission to a LTM candidate cell overlapped (in time and frequency) with serving cell UL transmission, PRACH and other UL transmissions on the serving cell cannot simultaneously transmitted, it also can up to UE implementation, as same as legacy. |
| IDCC | Support |
| CMCC | Support |
| Huawei, HiSilicon | Support |
| FGI | Support |

Round 2

In round-2 discussion, P1-7 can be discussed together with P1-8.

Round 3

Based on discussion in 2nd round, separate discussion of P1-7 and P1-8 is preferred.

Proposal P1-7: For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a non-serving candidate cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells

- PRACH transmission

- PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI

- SRS transmission

- Any other PUCCH/PUSCH transmission

FFS: the UE behavior in this case

- Alt 1:Dropping rule is needed

  - e.g., transmission to a serving cell should be prioritized

  - Supported by(2): Nokia, Xiaomi

- Alt 2: up to UE implementation

  - Supported by(2): Spreadtrum, Lenovo

| Company | Input |
| --- | --- |
| Nokia | In scenarios when UE cannot do parallel multiple transmissions (e.g., specially in case of overlap in both time and frequency), UE needs to decide which transmission to be selected. NW cannot always guarantee that such overlap scenario does not happen, one example, periodic CSI reporting on a serving cell can collide with a PRACH transmission to a candidate cell. |
| Lenovo | We support Alt2 which follows the legacy spec. |
| Xiaomi | OK with proposal p1-7. And for the UE behavior, we support alt.1. |

P1-8: priority

Round 1

Proposal 1-8: On whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed, discuss and down-select among the following alternatives:

- Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction

- Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell

- Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell)

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| NTT DOCOMO | We are OK with FL proposal. |
| Nokia | We are fine as long as rule is defined. Alt2/3 are slightly preferred. |
| Samsung | OK with FL proposal |
| Lenovo | We support to reuse the legacy spec that is the PRACH to a candidate cell have a same priority of PRACH transmission on a serving cell other than the PCell. Therefore, we propose the following revision. Revised Proposal 1-8: On whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed, discuss and down-select among the following alternatives: - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell) - Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priorioty as a PRACH transmission on a serving cell other than PCell. |
| vivo | Do not support. Since the parallel transmission of PDCCH order triggered PRACH transmission to candidate cell and UL transmission on the serving cell can be avoided by NW scheduling. |
| Xiaomi | Fine with proposal 1-8. For now, we do not have a particular preference among these alternatives. |
| ZTE | We would like to align the understanding with other companies on this case, what we want to discuss or resolve the case is multiple PRACH transmission are transmitted to different candidate cells respectively and further discuss how to control transmission power of each PRACH transmission for UE to satisfy requirement of total power. Is it correct? If yes, we tend to support Alt3. |
| Ericsson | Why do we need this proposal? Are we excluding something? |
| ITRI | OK with FL proposal, and we prefer Alt.1 |
| Spreadtrum | We support the proposal. And open for down-selection, first preference is Alt 2. |
| IDCC | If overlap is avoided, do we need this proposal? |
| CMCC | Fine with the proposal. Alt 1 is slightly preferred. |
| Huawei, HiSillicon | We prefer Alt 1 or the Alt 4 by Lenovo. To my understanding, they are same. The PRACH to candidate cell is best effort procedure and should not affect UL transmission in serving cells. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Please provide your answer/understanding to the following questions. Question1: can the overlapping of PRACH to candidate cell and UL transmission in serving cell be avoided by NW implementation? Question 2: if the answer to Q1 is no, what’s your preference on the following alternatives. On whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed: - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell) - Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priority as a PRACH transmission on a serving cell other than PCell. |
| Nokia | We would like to point out the difference between the scenarios for P1-7 and P1-8. They are not the same and hence cannot be merged! P1-7 is when overlap happens and UE capability does not support parallel transmissions. Therefore, dropping rules need to be defined. However, P1-8 is needed when the UE supports simultaneous/parallel transmissions. In the legacy [section 7.5, 38.213] similar rules have been defined when the UE supports simultaneous/parallel transmission for example in single cell operation with multiple carriers or across CCs in an inter-band or intra-band non-contiguous CA scenarios. Therefore, power reduction rules are needed. Therefore, we need two different proposals. |
| Lenovo | Q1: Yes. Q2: We support Alt4 if overlapping exists. |
| Huawei, HiSilicon | To my understanding P1-7 and P1-8 is different. in P1-7, the resource is overlapped in time domain but not in frequency domain. We think it is possible in CA cases. Otherwise, UE will give all UL resource in other CCs than the CC to transmitted PRACH to candidate cell. in P1-8, the resource is overlapped in time and frequency domain. This scenario should be avoided by scheduling. |
| NTT DOCOMO | Question1: No. Question 2: We slightly prefer Alt1. We’re also okay with Alt4. |
| Transsion | Q1: Yes. Q2: We slightly prefer Alt4. |
| Fsuturewei | Q1: Yes in principle. But no requirement could be imposed on network. Q2: Fine with Alt4. |
| Xiaomi | Q1: no Q2: Slightly prefer alt.1. |
| Mod | @Nokia @HW: based on feedback from companies in round-1 discussion, some companies raised that if overlapping can be avoided, whether prioritization rule is still needed should be discussed. So, I merged the discussion of P1-7 and P1-8 together. Based on comments from Nokia, HW and Spreadtrum, the proposal is updated as follows. Proposal P1-7/8: For PDCCH-order based PRACH for candidate cell - If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a non-serving candidate cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells - PRACH transmission - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI - SRS transmission - Any other PUCCH/PUSCH transmission FFS: the UE behavior in this case - Alt 1:Dropping rule is needed - e.g., transmission to a serving cell should be prioritized - Supported by(1): Nokia - Alt 2: up to UE implementation - Supported by(1): Spreadtrum - When overlap of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell happens and the UE supports simultaneous/parallel transmissions - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction - Supported by(5): ITRI, CMCC, HW, DCM(1st preference), Xiaomi - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell - Supported by(2): Nokia, Spreadtrum - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell) - Supported by(2): Nokia, ZTE - Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priority as a PRACH transmission on a serving cell other than PCell. - Supported by(5): Lenovo, HW(Alt 1=Alt 4), DCM(2nd preference), Transsion, Fsuturewei |
| Samsung | We also think P1-7/P1-8 are separated issue. We are generally O.K. with FL’s latest proposal |

Round 3

| Company | Input |
| --- | --- |
| Lenovo | For UE behavior when overlapping, we support Alt2 which follows the legacy scheme. For power reduction priority, we support Alt4 which follows the legacy scheme. |
| Mod | Please further check and provide feedback for P1-8. Proposal P1-8: For PDCCH-order based PRACH for candidate cell, When overlap of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell happens and the UE supports simultaneous/parallel transmissions - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction - Supported by(6): ITRI, CMCC, HW, DCM(1st preference), Xiaomi, IDCC - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell - Supported by(2): Nokia, Spreadtrum - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell) - Supported by(2): Nokia, ZTE - Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priority as a PRACH transmission on a serving cell other than PCell. - Supported by(5): Lenovo, HW(Alt 1=Alt 4), DCM(2nd preference), Transsion, Futurewei |
| IDCC | We prefer Alt 1. We added the company name to the list. |
| ZTE | After further thinking, we realized that PDCCH order based PRACH is triggered by serving cell, we don’t understand why serving cell trigger a PRACH transmission for candidate cell that conflicts with its own UL transmission. If such a conflict does not exist at all, it doesn’t make sense to continue discussion the solution to it. So we think that we can first clarify the possibility of this situation. As for whether serving cell can know the timing of candidate cell, this issue has been discussed in RAN4 and have a common understanding that timing difference between serving cell and candidate cell can be derived by L3 measurement. Based on the above information, we think that this case can be avoided by implementation. |
| Futurewei | It is really based on the scenarios – whether fast cell switch is more important than serving cell interruption. If the network can make the decision, it is the best. If let the UE blindly make the decision, we slightly prefer Alt4. |
| QC | The proposal may not be needed. NW can avoid overlapping between PRACH and serving cell UL Tx. The interruption rule to be specified by RAN4 is sufficient, i.e. all interrupted serving cells will have UL Tx dropped in the PRACH Tx window, and gNB should minimize the interruption |
| Nokia | NW cannot always avoid overlapping. Currently, parallel PRACH transmission and another UL transmission to servings cells are possible with UE capability, specially in the case of inter-band CA and non-contiguous intra-band CA, and the power reduction rules are defined in section 7.5, TS 38.213. The same scenario may also be valid for PRACH transmission to a candidate cell and another UL transmission to a serving cell. We need to define the rules for this scenario as well, specially when a candidate cell is not a current serving cell. We don’t agree that NW implementation can solve it (please check section 7.5, 38.213) - UE needs to know how the power distribution should be done. |
| NTT DOCOMO | We support FL proposal and prefer Alt1 or Alt4. |
| Xiaomi | During LTM, from our understanding, the transmission between UE and serving cell is more important than the PDCCH order RACH for candidate cell TA measurement. Therefore, we still prefer Alt.1. |

- Issue 2 – Initial TA acquisition for RACH-less solutions

Open issues on TA acquisition of the candidate target cell(s) for RACH-less solutions and company views are summarized below.

Table 2. Summary of views on Issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | TA acquisition for RACH-less solutions - Alt2: RACH-less mechanism as in LTE - Alt3: UE based TA measurement Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported | Alt2 is supported by: ZTE, Samsung, Spreadtrum, CATT, Huawei Alt3is supported by: Futurewei, vivo, Nokia, OPPO, DoCoMo, Samsung |

Companies are encouraged to show views/comments/suggestions on the following proposals.

P2-1: closed

Round 1

Proposal 2-1 (proposed conclusion): From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command.

- Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

- Note 2: The feasibility can be further concluded by RAN2

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Support |
| Futurewei | We think this is to support LTE RACH-less scenarios. We are fine with the principle of Proposal 2-1. But we don’t think LTE like RACH-less should be handled with additional separate mechanism. With PDCCH ordered RACH without-RAR supported, we already agreed that cell switch command carries target cell TA, thus there will be a TA field in the MAC CE. The TA field need not to be tied with PDCCH ordered RACH. We just need to have an indication in the MAC CE to indicate whether this TA field is carrying the target TA or the source TA adjustment (delta source TA). If the indication is target-TA, then if it is set to ‘0’, it is the target TA=0 case. If the indication is source TA, the field carries the delta source TA which is used for UE-based TA measurement. If the indication is source TA and the TA field is set to ‘0’, it naturally covers the LTE target TA= source TA case. Therefore, our point is that as long as the UE based TA measurement is supported, LTE RACH-less scenarios can be covered naturally. Note 1: Letting the cell switch command to carry the most recent delta source TA update allows the UE to use the most accurate source TA for deriving the target TA. Note 2: after the UE connected to a serving cell, network source/serving cell does not maintain the absolute TA value, only the delta TA is measured and if necessary updated to the UE. Therefore, the second LTE case needs to be indicated by delta source TA=0 not the absolute source TA itself. The absolute TA is updated and maintained at the UE. |
| NTT DOCOMO | Support. |
| Nokia | Support |
| Samsung | Support in principle |
| Lenovo | Support in principle |
| vivo | Support |
| Xiaomi | Is this about how to support RACH-less mechanism as LTE? It should be specified in the main bullet. Proposal 2-1 (proposed conclusion): From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism as LTE can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command. Generally, we are fine with proposal 2-1. |
| ZTE | Support |
| Ericsson | Support. Note that this is not to support RACH-less like in LTE. Fundamentally, we assume that it is clear what TA=0 is clear – the only new thing is “same value as source”, which would have to be signaled using a special value. |
| IDCC | Support |
| CMCC | Fine in principle |
| Huawei, HiSilicon | Support |
| FGI | Support |

Agreement

From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or keeping the same value as source cell in cell switch command.

- Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

- Note 2: The feasibility and signalling can be further concluded by RAN2

P2-2: closed

Round 1

Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption:

Working Assumption

From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| QC | Support. The feasibility can be decided by RAN4/2 later. Btw, the mechanism similar to Rx timing difference measurement has been supported in positioning. ### 5.1.29 DL reference signal time difference (DL RSTD) Definition; DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE. |
| Futurewei | We support FL Proposal 2-2. We think with significant progress already made for PDCCH ordered RACH, we would need to start to work on the details of UE based TA measurement. We also support to extend current positioning RSTD definition suggested by QC, and consider to modify the sentence to: “Multiple DL PRS, SSB, CSI-RS resources can be used to determine the start of one subframe from a TP.” If CSI-RS is agreed for LTM we include it, otherwise we don’t include. |
| NTT DOCOMO | Support. |
| Nokia | Support |
| Samsung | Support |
| Lenovo | We are OK with it if majority support it. |
| vivo | According to the contributions of opponents, their concern on UE-based TA measurement is the estimation accuracy of downlink receiving timing based on SSB, especially for asynchronous scenarios. If the estimation error of downlink receiving timing difference is large, the TA value of candidate cell calculated by UE-based TA measurement cannot satisfy the uplink timing requirement, and uplink transmission to the candidate cell cannot be performed based on the uplink timing. In our view, the estimation error is a valid issue for the asynchronous scenario. To facilitate the discussion process, we suggest focusing on UE-based TA measurement in synchronous scenarios firstly in Rel-18, where the downlink receiving timing difference between candidate cell and the serving cell is within CP and the estimation error is small. For the scenario where the downlink receiving timing difference exceeds CP, the potential enhancement on UE-based TA measurement can be discussed if there is time left. |
| Xiaomi | Support |
| ZTE | We noticed that PRS rather than SSB is used as DL RS in positioning. It is still unclear whether accuracy of SSB based DL timing measurement can satisfy requirements. |
| Ericsson | Support |
| ZTE2 | Further comment: we understand that even if UE-based TA measurement is ultimately assessed as feasible by RAN4 in certain scenario, such as FR1, RAN1 still need to further clarify something. For example, whether to allow to configure “UE-based TA measurement” and “RACH-based TA acquisition” simultaneously. And whether UE needs to report an indication to let NW know that it has acquired TA through “UE-based TA measurement” mechanism and further clarify whether TA needs to be indicated by RAR or cell switch command. |
| IDCC | Support |
| Futurewei | @ZTE: As you suggested, we do need to start to discuss the details of UE based TA measurement so that we can provide RAN1 agreed framework to RAN4 for them to evaluate the feasibility and set up the minimum performance requirements. Regarding your above questions we would like to share our views with you: - We can further discussion the need of configuring different types of LTM RACH schemes simultaneously, the need of them for different mobility scenarios and the details of the configuration. But it is clear whenever the source/serving cell to trigger a TA acquisition, only one type of scheme is triggered and indicated in the MAC CE command. - We can discuss whether UE need to report/indicate UE already measured TA. It depends on whether we need to support early triggered UE based TA measurement. In our view, since we already agreed the principle that the early DL synchronization is performed at the measurement phase, the timings of all the candidate cells are acquired and tracked after DL synchronization is achieved. The UE needs to track the timing of all the candidate cells to maintain DL synchronization with each candidate (this is not only for UE based TA management but also for early RACH). Upon receiving the cell switch command, the UE is able to use the timings of the serving and the target candidate cells currently under tracking to calculated the target cell TA without delay. Therefore, UE TA measurement can be triggered by cell switch command. - The target cell TA is measured by the UE. In this case, there is no need to use RAR or cell switch command to deliver the candidate/target cell TA to the UE. |
| Mod | Thanks for further comments/clarifications from QC, Futurewei, vivo and ZTE. To address the concern from ZTE, can we consider the following revision of P2-2. Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption (with the following update): Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: - Whether to allow simultaneous configuration of “UE-based TA measurement” and “RACH-based TA acquisition”. - Whether UE needs to let NW know that it has acquired TA through UE-based TA measurement mechanism - Whether the TA value still needs to be indicated in RAR or cell switch command for UE-based solutions. - FFS: other impacts on RAN1 spec |
| CMCC | Support |
| Huawei, HiSilicon | we would like to add another FFS on the channels/signals the TA acquired by UE can be applied. The accuracy of UE based TA acquisition may not be sufficient for PUSCH transmission. |
| Mod | Further update based on HW’s comment. Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption (with the following update): Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec[, for example: - Whether to allow simultaneous configuration of “UE-based TA measurement” and “RACH-based TA acquisition” - Whether UE needs to let NW know that it has acquired TA through UE-based TA measurement mechanism - Whether the TA value still needs to be indicated in RAR or cell switch command for UE-based solutions - The channels/signals the TA acquired by UE can be applied] |

Agreement

Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism

Working Assumption

From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

- Other issues

Round 1

Table 3. Summary for other issues

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.1 | For TA acquisition of candidate cell(s) when reception of RAR is configured, whether the number of candidate cells can exceed the maximum number of TAs that can be maintained by UE capability | No: Huawei, CATT, Spreadtrum, Qualcomm, Lennovo, Futurewei Yes: ZTE |
| 3.2 | Support independent TAT configuration for candidate cell(s), and when UE receives initial TA value, corresponding TAT would be started. | vivo, OPPO |
| 3.3 | Association between TA and candidate cell - Alt2: Associate TA/TAG and candidate cell explicitly - Alt3: Do not support associating the TA with a candidate cell | Alt2: Spreadtrum, CMCC, LGE, DoCoMo, Transsion Holdings, FGI, CATT Alt3: Google |
| 3.4 | Clarification on UE behavior for the case that the reception of RAR is not configured - Whether to allow or need to initialing RAR window - How to identify the completing of random access procedure - E.g., PRACH transmission is transmitted to candidate cell. | ZTE |

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Futurewei | We do not support 3.1. We are fine with 3.2. |
| Lenovo | We do not support 3.1. |

Round 2

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Lenovo | We do not support 3.1 |
| Futurewei | We do not support 3.1. |

P3-1: number of candidate cell (closed)

Round 3

Proposal 3-1: For TA acquisition of candidate cell(s) when reception of RAR is configured, the number of candidate cells cannot exceed the maximum number of TAs that can be maintained by UE capability.

| Company | Input |
| --- | --- |
| Mod | Please share your views on the above proposal. |
| Lenovo | Support. |
| Mod | Based on discussion offline, majority companies thought this proposal is not needed. |
| ZTE | After offline discussion, we noticed that some companies want to support the case that the number of configured candidate cell can be larger than the UE capability to acquire to maintain the maximum number of TA. We can understand the motivation of this case. But what I want to say is that this proposal is to express that no matter how many cells are configured for UE to acquire TA, UE will always only maintain or obtain the number of TAs that match its own UE capabilities. To solve this issue mentioned above, we can discuss it case by case, such as: - If the number of configured candidate cell for TA acquisition > UE reporting capability, one of the following method can be used to determine the number of candidate cells that matches UE capability - Option 1: a MAC CE is used to select the number of candidate cells for TA acquisition that matches UE capability - Option 2: drop rule, such as drop some candidate cells that will not be moved or switched to temporarily. - If the number of configured candidate cell for TA acquisition <= UE reporting capability, no additional MAC CE is introduced, that is, UE can obtain TA only for these configured candidate cells |
| Futurewei | Number of candidate cells is already have separate limitation, should not be tied with number of TAs. |

P3-2: TAT

Round 3

Proposal 3-2: TBD

| Company | Input |
| --- | --- |
| Mod | Please provide views on the following issue: For TA acquisition of candidate cell(s) when reception of RAR is configured, whether/how to support independent TAT configuration for candidate cell(s). |
| ZTE | Is the TAT mentioned above same as the existing TAT design? Whether the existing TAT can be directly reused. If no, what additional design is needed. |
| Futurewei | Since PRACH is ordered from the serving cell, for PDCCH ordered PRACH, the network/serving cell maintains the TAT like timer seems the simplest. |
| NTT DOCOMO | We would like to clarify what independent TAT configuration for candidate cell(s). In legacy spec., TAT is associated with TAG. Is TAT configuration for candidate cell(s) associated with one candidate cell or multiple candidate cells which have same TA? In our view, TAT configuration can be associated with multiple candidate cells which have same TA because it would be beneficial. |
| Lenovo | Similar view with Docomo that a clarification is needed for the independent TAT configuration for candidate cell(s). |
| Mod | @ZTE: regarding whether the existing TAT mechanism can be reused, we don’t think we can answer this without discussion. However, before making such decision, a more important issue is whether TAT is needed for PDCCH-ordered RACH of candidate cell. In legacy system, when TAT is expired, UL transmission is not permitted until UL synchronization is obtained again. However, other than PRACH, there is no UL transmission to candidate cell before cell switch. What’s more, as also mentioned by Futurewei, the timer or some other equivalent measures can be kept at NW side, and PDCCH order triggering PRACH can be sent once it’s needed. In such case, do we still need to specify UE behavior as what we have in legacy system? |

- Issues to be discussed in online/offline sessions

5.1 Round 1

Proposal 2-2: On UE based TA measurement, confirm the following Working Assumption (with the following update):

Working Assumption

From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec[, for example:

  - Whether to allow simultaneous configuration of “UE-based TA measurement” and “RACH-based TA acquisition”

  - Whether UE needs to let NW know that it has acquired TA through UE-based TA measurement mechanism

  - Whether the TA value still needs to be indicated in RAR or cell switch command for UE-based solutions

  - The channels/signals the TA acquired by UE can be applied]

Proposal 2-1 (proposed conclusion): From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or the same value as source cell in cell switch command.

- Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order.

- Note 2: The feasibility can be further concluded by RAN2

Proposal 1-6: For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination.

Proposal 1-5: On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported.

Proposal 1-2: When reception of RAR is configured, discuss and down-select among the following alternatives for RAR reception:

- Alt1: MAC PDU(like RAR MAC PDU in 4 step RACH)

  - Supported by(3): Lenovo, ZTE, Spreadtrum

- Alt3: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

  - Supported by(8): QC, Samsung, Xiaomi, Ericsson, ITRI, IDCC, CMCC, HW

- Up to RAN2 design

  - Supported by(2): Nokia, vivo

Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window

- Alt1: Postpone the starting point of the random access response window

  - Supported by (2): vivo, ZTE

- Alt2: Extend the length of the random access response window

  - Supported by (2): Futurewei, Xiaomi

- Alt 3: Length and offset of the starting point of RAR window can be configured by RRC

  - Supported by (8): QC, Nokia, Samsung, Lenovo, ITRI, LGE, IDCC, FGI

[Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.]

[FFS: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, whether RAR window is still necessary]

Proposal 1-3: In addition to TA, on whether candidate cell ID shall be contained in RAR when reception of RAR is configured, discuss and down-select among the following alternatives:

- Alt 1: Don’t need

  - Supported by(13): QC, [DOCOMO], Nokia, Samsung, vivo, ITRI, Xiaomi, ZTE, LGE, ITRI, Spreadtrum, IDCC, FGI

- Alt 2: Identification of candidate cell is contained in RAR

  - Supported by(2): Futurewei, Ericsson

Note: there is only 1 ongoing RACH procedure at each time.

5.2 Round 2

Proposal 1-2 (proposed conclusion): When reception of RAR is configured, the following alternatives for RAR reception have been discussed in RAN1. It’s up to RAN2 design on how RAR reception is supported.

- Alt1: MAC PDU(like RAR MAC PDU in 4 step RACH)

- Alt3: MAC CE scheduled by PDCCH with CRC scrambled by C-RNTI

Proposal 1-1: When reception of RAR is configured, discuss and down-select among the following alternatives of determining the random access response window

- Alt1: Postpone the starting point of the random access response window

  - Supported by (3): vivo, ZTE, Transsion

- Alt2: Extend the length of the random access response window

  - Supported by (2): Futurewei, Xiaomi

- Alt 3: Length and offset of the starting point of RAR window can be configured by RRC

  - Supported by (10): QC, Nokia, Samsung, Lenovo, ITRI, LGE, IDCC, FGI, DOCOMO, Xiaomi

- Alt 4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed

  - Supported by (1): HW

[Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.]

Proposal 1-3: In addition to TA, on whether candidate cell ID shall be contained in RAR when reception of RAR is configured, discuss and down-select among the following alternatives:

- Alt 1: there is only one ongoing RACH procedure at each time, and the identification of candidate cell is not needed

  - Supported by (13): QC, [DOCOMO], Nokia, Samsung, vivo, ITRI, Xiaomi, ZTE, LGE, ITRI, Spreadtrum, IDCC, FGI

- Alt 2: more than one RACH procedures are allowed at each time, and the identification of candidate cell is contained in RAR

  - Supported by(2): Futurewei, Ericsson

Proposal 1-5-1: On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH

- UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved

- FFS: whether/how to reset the counter

Proposal 1-7/8: For PDCCH-order based PRACH for candidate cell

- If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a non-serving candidate cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells

  - PRACH transmission

  - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI

  - SRS transmission

  - Any other PUCCH/PUSCH transmission

FFS: the UE behavior in this case

  - Alt 1:Dropping rule is needed

    - e.g., transmission to a serving cell should be prioritized

    - Supported by(1): Nokia

  - Alt 2: up to UE implementation

    - Supported by(1): Spreadtrum

- When overlap of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell happens and the UE supports simultaneous/parallel transmissions

  - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction

    - Supported by(5): ITRI, CMCC, HW, DCM(1st preference), Xiaomi

  - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell

    - Supported by(2): Nokia, Spreadtrum

  - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell)

    - Supported by(2): Nokia, ZTE

  - Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priority as a PRACH transmission on a serving cell other than PCell.

    - Supported by(5): Lenovo, HW(Alt 1=Alt 4), DCM(2nd preference), Transsion, Fsuturewei

5.3 Round 2-1(offline)

Proposal 1-5-1: On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS

- UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved

- whether/how to reset the counter

Proposal 3-1: For TA acquisition of candidate cell(s) when reception of RAR is configured, the number of configured candidate cells for TA acquisition cannot exceed the maximum number of TAs that can be maintained by UE capability.

Proposal P1-7/8: For PDCCH-order based PRACH for candidate cell

- If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a non-serving candidate cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells

  - PRACH transmission

  - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI

  - SRS transmission

  - Any other PUCCH/PUSCH transmission

FFS: the UE behavior in this case

  - Alt 1:Dropping rule is needed

    - e.g., transmission to a serving cell should be prioritized

    - Supported by(1): Nokia

  - Alt 2: up to UE implementation

    - Supported by(1): Spreadtrum

- When overlap of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell happens and the UE supports simultaneous/parallel transmissions

  - Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction

    - Supported by(5): ITRI, CMCC, HW, DCM(1st preference), Xiaomi

  - Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell

    - Supported by(2): Nokia, Spreadtrum

  - Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell)

    - Supported by(2): Nokia, ZTE

  - Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priority as a PRACH transmission on a serving cell other than PCell.

    - Supported by(5): Lenovo, HW(Alt 1=Alt 4), DCM(2nd preference), Transsion, Fsuturewei

Proposal 3-2: TBD

For TA acquisition of candidate cell(s) when reception of RAR is configured, whether/how to support independent TAT configuration for candidate cell(s).

5.4 Round 3

Proposal P1-7: For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a non-serving candidate cell (including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells

- PRACH transmission

- PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI

- SRS transmission

- Any other PUCCH/PUSCH transmission

FFS: the UE behavior in this case

- Alt 1:Dropping rule is needed

  - e.g., transmission to a serving cell should be prioritized

  - Supported by(2): Nokia, Xiaomi

- Alt 2: up to UE implementation

  - Supported by(2): Spreadtrum, Lenovo

Proposal P1-8: For PDCCH-order based PRACH for candidate cell, When overlap of PRACH to candidate cell and dynamic scheduled UL transmission in serving cell happens and the UE supports simultaneous/parallel transmissions

- Alt1: A PRACH transmission to a LTM candidate cell has the lowest priority for power reduction

  - Supported by(6): ITRI, CMCC, HW, DCM(1st preference), Xiaomi, IDCC

- Alt2: Power reduction for a PRACH transmission to a LTM candidate cell has a same prioritization as PRACH transmission on the PCell

  - Supported by(2): Nokia, Spreadtrum

- Alt3: A PRACH transmission to an LTM candidate cell has same power allocation priority as PUCCH/PUSCH transmission with HARQ-ACK (i.e., lower than the PRACH transmission to current PCell)

  - Supported by(2): Nokia, ZTE

- Alt4: A PRACH transmission to a LTM candidate cell has a same power allocation priority as a PRACH transmission on a serving cell other than PCell.

  - Supported by(5): Lenovo, HW(Alt 1=Alt 4), DCM(2nd preference), Transsion, Futurewei

- References

- R1-2304353	Discussion on UE based RACH-less TA determination	FUTUREWEI

- R1-2304400	Enhancements on TA management to reduce latency	ZTE

- R1-2304499	Discussion on TA management for L1/L2 mobility	vivo

- R1-2304577	Discussion on timing advance management to reduce latency	Spreadtrum Communications

- R1-2304660	Timing advance management to reduce latency	Huawei, HiSilicon

- R1-2304713	Discussion on time advance management to reduce latency	CATT

- R1-2304786	Timing advance management for L1/L2 Mobility	Ericsson

- R1-2304881	Discussion on Timing advance management	xiaomi

- R1-2304958	Timing advancement management for L1L2 mobility	Lenovo

- R1-2305011	Discussion on timing advance management to reduce latency	Google

- R1-2305023	Discussion on timing advance management to reduce latency	CAICT

- R1-2305113	Discussion on timing advance management to reduce latency	CMCC

- R1-2305157	Timing Advance Management for L1/L2-triggered Mobility	Nokia, Nokia Shanghai Bell

- R1-2305264	Timing advance management for L1/L2 triggered mobility	Apple

- R1-2305303	Enhancements on TA management for mobility	LG Electronics

- R1-2305357	TA management to reduce latency for L1/L2 based mobility	Qualcomm Incorporated

- R1-2305419	Discussions on Timing Advance Management	OPPO

- R1-2305962	Candidate cell TA acquisition for NR L1/L2 mobility enhancement	Samsung

- R1-2305614	Timing advance enhancement for inter-cell mobility	NTT DOCOMO, INC.

- R1-2305645	UL Timing management to reduce handover latency	MediaTek Inc.

- R1-2305713	Discussion on TA management for L1/L2 mobility	Transsion Holdings

- R1-2305775	Discussion on TA management for mobility enhancement	ITRI

- R1-2305784	Discussion on TA management for LTM	FGI

- R1-2305857	Discussion on timing advance management	InterDigital, Inc.

- Previous agreements

RAN1 #110bis-e

Agreement

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

On mechanism to acquire TA of the candidate cells, the following solutions can be further studied:

•         RACH-based solutions

e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd

•         RACH-less solutions

e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

- the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

- the association is provided as a part of candidate cell(s) configuration

- the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

RAN1 #111

Agreement

On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.

- The PDCCH order is only triggered by source cell

- FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.

- Note: any other RACH-based solutions are for discussion separately

Agreement (Made in RAN1#110b-e)

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2)

Agreement

For PDCCH ordered RACH in LTM, at least the following enhancements are supported

- Introduce indication of candidate cell and/or RO of candidate cell in DCI

- configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order

- FFS: whether/how to transmit RAR

Agreement

On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study

- Alt 1: RAR is needed

- Alt 2: RAR is not needed

    - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command

- Alt 3: whether RAR is needed can be configured

Agreement

- TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW.

same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

RAN1 #112

Agreement

For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell.

Note: the detailed signalling is left to RAN2

Agreement

The PDCCH order from the source cell contains the indication of candidate cell.

- The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

Agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

Agreement

- For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported

Agreement

on whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives.

- Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1)

  - Alt 2: UE autonomous Re-transmission of PRACH is allowed,

    - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

Agreement

If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell.

- The maximum number of TA values memorized by UE is a UE capability

- FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR

Agreement

Whether RAR needs to be received is configured by RRC.

Agreement

study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured

- Whether gap between the DCI and PRACH longer than timeline defined in spec is needed

- Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

- FFS: other impacts on RAN1 spec

RAN1 #112bis

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax.

Agreement

When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case.

Agreement

When reception of RAR is configured, support RAR is received from serving cell in inter-DU case.

- FFS: RA response window related issues

Agreement

For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured,

- the UE stores(remembers/maintains/handles) a TA for at least one candidate cell

- storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability

  - detailed number of candidate cell is up to UE capability

Agreement

For PDCCH-order based PRACH for candidate cell study the following issues:

  - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed

  - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured

- Whether power ramping is performed or not is determined from PDCCH order

    - If power ramping is performed,

      - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism)

      - power ramping-up value is configured

    - else, the power should be determined by open-loop power control

Agreement

Send LS to RAN4 with the following info

- RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM.

- RAN1 believes that this will require that the time gap is increased at least for the following scenario

    - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell

- RAN1 relies on RAN4 to verify the need for the above additional latency and, if so, the corresponding value is needed

- RAN1 relies on RAN4 to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH

- RAN1 relies on RAN4 to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed

Potential RAN1 spec update will be based on RAN4’s feedback.

RAN1 #112bis-e

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax.

Agreement

When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case.

Agreement

When reception of RAR is configured, support RAR is received from serving cell in inter-DU case.

- FFS: RA response window related issues

Agreement

For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured,

- the UE stores(remembers/maintains/handles) a TA for at least one candidate cell

- storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability

  - detailed number of candidate cell is up to UE capability

Agreement

For PDCCH-order based PRACH for candidate cell study the following issues:

- whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed

- whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission

Agreement

For PDCCH ordered-RACH, if reception of RAR is not configured

- Whether power ramping is performed or not is determined from PDCCH order

    - If power ramping is performed,

      - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism)

      - power ramping-up value is configured

    - else, the power should be determined by open-loop power control

Agreement

Send LS to RAN4 with the following info

- RAN1 discussed the time gap between a PDCCH order and the corresponding PRACH transmission for LTM.

- RAN1 believes that this will require that the time gap is increased at least for the following scenario

    - For PDCCH-order based PRACH on a candidate cell that is not a current serving cell with PUCCH/PUSCH or inter-frequency with the current serving cell

- RAN1 relies on RAN4 to verify the need for the above additional latency and, if so, the corresponding value is needed

- RAN1 relies on RAN4 to investigate any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH

- RAN1 relies on RAN4 to verify the need for any update is required to ΔBWPSwitching, ΔDelay if so, the corresponding values and whether UE capability is needed

- Potential RAN1 spec update will be based on RAN4’s feedback.