3GPP TSG RAN WG1 #113		                                   					       												                          R1-2306165

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.16.14

Agenda Item:	9.16.14

Document for: 	Endorsement

### UE features for BWP without restriction

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell 3. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] Note: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig [Note: If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions | 1. UE is allowed to performs RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell 3. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. | [1-7, 2-24, 2-31,] 53-3] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP and meet interruptions requirements | [Per band] | No | No | n/a | This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of [the corresponding carrier(s) to be measured.] The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig [If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations.] [UE indicates at most one of FG 53-1 and 53- 2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell 3. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per band | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2304516	Discussion on UE features for BWP without restriction	vivo

R1-2304606	Discussion on UE feature for BWP without restriction	ZTE

R1-2304609	On UE features for BWP without restrictions	Dell Technologies

R1-2304670	UE features for BWP without restriction	Huawei, HiSilicon

R1-2304787	UE features for BWP without restriction	Ericsson

R1-2304945	Summary of UE features for BWP without restriction	Moderator (AT&T)

R1-2305056	Open issues in UE features for BWP without restrictions	Vodafone

R1-2305283	On  UE features for Rel-18 BWP without restriction	Apple

R1-2305377	UE features for BWP without restriction	Qualcomm Incorporated

R1-2305649	UE features for BWP without restriction	MediaTek Inc.

R1-2305985	UE features for BWP without restriction	MediaTek Inc.

Revision of R1-2305649

R1-2305728	On UE features for BWP without restriction	Nokia, Nokia Shanghai Bell