3GPP TSG RAN WG1 #113	R1-2306171

Incheon, Korea, May 22nd - May 26th, 2023

Agenda item:	9.5.1.3c

Sourcce: 	Moderator (Qualcomm)

Title: 	Moderator Summary #3 on resource allocation for SL PRS

Document for:	Discussion and Decision

# Introduction

At RAN#98, a new work item “Expanded and improved NR positioning” (NR_pos_enh2) was approved [1]. The relevant WID objective is copied here for convenience:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre)-configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we present the summary of the documents submitted in this subagenda

# Discussion Information

The proposals are ranked according to HIGH, MEDIUM, LOW. The intention is to start in the offline/online sessions from the HIGH proposals.

# SL Positioning Resource Allocation

## 3.1 General Aspects on Resource Pools

### 3.1.1 Slots for resource pool for positioning

The following agreements have been reached:

- Agreement - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool. - Agreement For a dedicated resource pool for positioning: - The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214). - FFS: additional slots that can be used for SL PRS is not precluded - Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration Agreement For a dedicated resource pool for positioning: - No additional slots are needed to be supported

The following related proposals were found in the submitted papers:

| OPPO | Proposal 1: For a dedicated SL PRS resource pool, slots with less than 7 OFDM symbols for SL operation and slots configured with PSFCH resources can be included. |
| --- | --- |

[LOW] Feature Lead Proposal 3.1.1-v0

For a dedicated SL PRS resource pool, slots with less than 7 OFDM symbols for SL operation can be included.

Companies views

| ZTE | We are not sure why the number 7 is brought up. We believe slots no larger than 9 OFDM symbols for a SL-PRS resource For a dedicated resource pool, except 2 or 3 symbols for PSCCH, one AGC symbol in the first symbol of a slot, one AGC proceeding and one gap symbol after a SL PRS resource, one SL PRS resource can at most occupy 8 or 9 symbols. |
| --- | --- |
| Futurewei | We do not think that is necessary. |
| CEWiT | Exact motivation is unclear. We believe no need of explicit limitation. Rather SL-PRS configuration (M & N ) value should be adopted according to the multiplexing of other channel. |
| InterDigital | We think it is not necessary to define a slot based on number of symbols, because it was agreed that multiple SL-PRS resources can be TDM:ed within a slot and thus the granularity of time domain resource allocano is per SL-PRS resource. We also support to have the start of SL-PRS resource in a slot (pre)configured in a resource pool and by doing that, we think sub-slot-based structure is already in place albeit the start and duration of sub-slot structure is based on SL-PRS resource and is (pre)configured in a resource pool. |

FL comment: There is no support or interest for this proposal. We can depriorize it for now.

### 3.1.2 Configuration parameters for Resource Pool for Positioning

Agreement For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier. - A UE can be (pre-)configured with one or more dedicated SL resource pools. - A UE can be (pre-)configured with one or more shared SL resource pools.

The following related proposals were found in the submitted papers:

| Huawei, HiSilicon | Proposal 4: For the dedicated resource pool, the configuration of the SL-PRS dedicated resource pool should at least include following information - Time domain resource, including at least available slots and symbols - Frequency domain resource, including at least starting PRB, bandwidth - PSCCH configurations, including at least bandwidth, number of symbols - SL-PRS configurations, including at least comb size, offset, bandwidth (same as the resource pool), start symbol and number of symbols Proposal 5: For the dedicated resource pool, SL-PRS configuration within the dedicated RP could uniquely determine all the parameters of each SL-PRS resource except the sequence ID. |
| --- | --- |
| CATT, GOHIGH | Proposal 1: For a dedicated resource pool for positioning, the following information should be (pre-) configured: - Time-domain bitmap for available slots - Start PRB position and the number of contiguous PRBs - Sub-channel size and sub-channel count - SL-PRS configuration - Resource allocation Scheme 2 related configuration - Power control configuration. - Reporting configuration. |
| Xiaomi | Proposal 3: The comb size, starting symbol(s) in a slot and number of symbols for a SL PRS transmission is determined based on the (pre)configuration in a SL PRS dedicated resource pool. Proposal 7. The start symbol or the number of symbols for SL PRS transmitted together with PSCCH/PSSCH is (pre)configured for the shared resource pool. |
| ZTE | Proposal 2: A dedicated SL-PRS resource pool should at least include the following configurations: - SL-PRS resource pool level configuration: time resource of resource pool, frequency resource of resource pool - SL-PRS resource configuration: SL-PRS resource ID, the periodicity of SL-PRS resource, number of symbols and start symbol of SL-PRS in a slot, comb size and RE offset, priority of SL-PRS resource - PSCCH configuration: candidates of time resource and frequency resource of PSCCH - Synchronization configuration: indicates the allowed synchronization reference(s) - Resource allocation scheme 2 related configuration: mapping between SL-PRS transmission parameter, SL-PRS CBR range and SL-PRS priority, SL-PRS resource reservation period, sensing window, selection window, SL-PRS RSRP threshold - Power control configuration: maximum SL-PRS transmission power, alpha value and P0 value for either sidelink pathloss based power control or downlink pathloss based power control |
| NEC | Proposal 9: With regard to the SL PRS time resource allocation within the resource pool, at least the candidate staring symbols of SL PRS in a slot should be (pre)configured for Scheme 2 allocation. |
| Interdigital | Proposal 3: Study (pre)configured SL-PRS starting position(s) with a slot in a dedicated resource pool. |
| Qualcomm | Proposal 21: Higher layers message (SLPP) contain positioning method, SL-PRS periodicity, comb size, comb offset, scrambling seed parameter, cast type, QoS, and identification information. In addition, they can include SL-PRS time and frequency resources. |
| LGE | Proposal 38: SL positioning resource pool configuration includes at least the following parameters. - Frequency domain resource granularity - PSCCH configuration - PSSCH configuration - Allowed SL positioning method (e.g. SL RTT, SL TDOA, SL AoA) - SL PRS resource configuration - Allowed multiplexing scheme (e.g. comb-based, TDM) - Enable/disable of each multiplexing scheme - Enable/disable of the feedback-based retransmission - Associated resource pool information (e.g. for SLPP, measurement report transmission) |
| Ericsson | - The network assigns SL PRS resource pools for scheme 1 and scheme 2, similarly to sl-TxPoolScheduling (mode 1 resource pools) and sl-TxPoolSelectedNormal (mode 2 resource pools) for legacy sidelink. Note that: - Each resoucrce pool can only be associated with either scheme 1 or scheme 2. - An SL PRS resource set can be assigned to both a scheme 1 and a scheme 2 resource pool simultaneously, which may result in an overlap between the two pools. |
| CEWiT | Proposal 5: Dedicated resource pool (pre)-configuration should include the time domain bit map of slots, reserved slots bit map, SL-start symbol per slot, start PRB position, the number of contiguous PRBs, sub-channel size and sub-channel count, SL-PRS configuration, SL-PSCCH configuration, SL-PSSCH configuration related to SL-PRS, power control configuration, reporting configuration, IUC configurations. |

### 3.1.3 Other general proposals

The following related proposals were found in the submitted papers:

| Nokia | Proposal 3: Discuss if a SL PRS transmission can occupy a wide bandwidth spanning across multiple SL resource pools. |
| --- | --- |
| Futurewei | Proposal 7: Support SL-PRS transmission in the available PSFCH occasions. |
| Continental | Proposal 1. 1: When dedicated and/or shared RPs are (pre-)configured for a UE, the network indicates to the UE which RP to select. Proposal 1.2: The UE selects the RP based on parameters related to the SL positioning operation (e.g., priority). Proposal 4.1: [Definition of dedicated RP for SL positioning] An RP in which only signals or data strictly necessary for SL positioning (e.g., SL PRS, SL positioning measurement data) may be exchanged. If there are data channels defined within such an RP, they may only carry reference signals and/or control and/or measurement information necessary for an ongoing, completed, or future positioning operation. Proposal 4.2: [Definition of shared RP for SL positioning] An SL communication RP as defined in 3GPP TS 38.211 [2], which may also be used for exchanging reference signals and/or control and/or measurement information necessary for an ongoing, completed, or future positioning operation, while using only the existing control and data channels. |
| NEC | Proposal 4: At least when a dedicated resource pool is used only for SL-PRS and the associated SCI without other channel/signal, the association between dedicated resource pool(s) and shared resource pool(s) should be (pre)configured. |
| Fraunhofer | Proposal 2: For the OFDM symbols including the PSCCH are allocated in a single subchannel, whereas for the SL-PRS several subchannels can be allocated. Proposal 3: For the shared resource pool the bandwidth of the SL-PRS shall be independent from the bandwidth selected for the PSCCH and PSSCH. - Option 1: The SL-PRS bandwidth is identical to the resource pool bandwidth and the PSCCH/PSSCH is selected according to the TB (transport block) size - Option 2: The SL-PRS bandwidth configurable independent from the bandwidth selected for PSCCH/PSSCH Proposal 4: In a shared resource pool: - For the PSCCH and PSSCH part FDMing is supported only (different UEs use different sub-channels) - For the SL-PRS itself TDMing (position in the slots number of symbols used for SL-PRS) and FDMing (COMB structure and/or PRB based subband or subchannel structure) shall be supported |
| Apple | Proposal 2: Associate a dedicated RP with a shared RP to allow for transmission of PSSCH/PSFCH information and support IUC. |
| LGE | Proposal 2: If SL PRS is transmitted in a dedicated resource pool, there should be an associated resource pool for transmission of the related PSSCH. - FFS on how to relate a dedicated resource pool and the associated resource pool |
| Sharp | Proposal 1: A legacy resource pool can be configured (by SL-ResourcePool-r16) to be either a shared resource pool, or not a shared resource pool. - By default, a legacy resource pool configured by SL-ResourcePool-r16 is not a shared resource pool. |
| Ericsson | - PRS Resource allocation for a PRS in a dedicated pool using a reservation from a shared pool is not supported. |

## 3.2 Dedicated Resource Pool for Positioning

### 3.2.1 General

| LGE | Proposal 5: TDM of multiple SL PRS resources from a single UE in a slot is supported in a dedicated resource pool. |
| --- | --- |

### 3.2.2 Slot structure

The following agreement was reached with regards to this topic:

Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above - Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH). Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

The following proposals were found in the contributions:

| Nokia | Proposal 7: Consider supporting SL PRS slots with/without PSCCHs in a dedicated resource pool: a SL PRS slot with PSCCHs can be configured occurring once in every K slots; the parameter K is (pre-)configurable and can be configured as 1. Proposal 8: Introduce a mini-slot time domain structure for SL PRS resources in dedicated resource pool. |
| --- | --- |
| Futurewei | Proposal 2: In dedicated resource pools support slot formats that carry only SL-PRS (no PSCCH). |
| vivo | - In the dedicated resource pool, PSCCH and the associated SL-PRS should be transmitted in the same slot (i.e., SL-PRS cannot be transmitted in a slot without associated PSCCH). |
| Spreadtrum | Proposal 1: SL PRS and its associated PSCCH are not in the same slot should be supported in dedicated resource pool. |
| Huawei, HiSilicon | Proposal 1: For the dedicated resource pool, SL-PRS should always be transmitted with the associated PSCCH within the same slot. |
| CATT, GOHIGH | Proposal 13: In a dedicated resource pool, each slot should consist of PSCCH resources and SL-PRS resources, and the SL-PRS can be transmitted in a slot without associated PSCCH. |
| Intel | - For dedicated resource pool, SL PRS only transmission without associated PSCCH is not supported. |
| Xiaomi | Proposal 2: SL-PRS transmission without associated PSCCH is not supported. |
| Sony | Proposal 5: SL-PRS can be transmitted in a slot without associated PSCCH (i.e, the PSCCH is in the previous slot). |
| CMCC | Proposal 1: Only SL-PRS and the corresponding PSCCH can be transmitted in a dedicated resource pool. - SL-PRS cannot be transmitted in a slot without associated PSCCH; - Other channels, i.e., PSSCH and PSFCH, is not necessary to be included in the dedicated resource pool. |
| Apple | Proposal 1: In a dedicated RP, SL-PRS should not be transmitted in a slot without an associated PSCCH. In addition, a mapping relationship between PSCCH and SL-PRS in a slot should be specified. |
| Qualcomm | Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool. |
| OPPO | Proposal 2: In a dedicated resource pool PSCCH carrying the SCI for reserving/indicating one or more SL PRS resource(s) uses at most one OFDM symbol. Proposal 3: In a dedicated resource pool the OFDM symbol used for PSCCH carrying the SCI for reserving/indicating one or more SL PRS resource(s) is adjacent to the OFDM symbol(s) for associated SL PRS, and occupies same bandwidth as the associated SL PRS. |
| OPPO | Proposal 6: In a dedicated/shared Scheme 1 resource pool, SL PRS transmission without an associated PSCCH within a slot from a single UE perspective should be supported, SL-MAC-CE or other higher-layer signaling is used for SL PRS resource reservation/indication. |
| Samsung | Proposal 1: For a dedicated resource pool for SL positioning, - SL-PRS can be transmitted in a slot without associated PSCCH. - PSSCH is not transmitted. Proposal 11: For indication/reservation of SL-PRS resource(s), consider the following options for SL control information - Option 1: Same-slot SL control information signalling - Option 2: Cross-slot SL control information signalling |
| LGE | Proposal 16: For a dedicated resource pool, SL PRS and the associated PSCCH are always transmitted together in a slot. |
| Ericsson | - SCI for SL PRS in dedicated resource pool is used to assist sensing and includes at least time domain resource allocation and resource reservation periods. FFS: additional fields, including priority, frequency domain resource allocation and SL PRS resource indication Proposal 25 SL PRS is always transmitted with PSCCH in the dedicated pool at least in scheme 2. |
| Mediatek | Proposal 3-1: SL-PRS transmission without the associated PSCCH in a slot is not supported |
| CEWiT | Proposal 1: The dedicated SL resource pool may include SL-PRS, corresponding PSCCH and PSSCH in a slot. PSSCH will be carrying the positioning protocol (SLPP) and SL-PRS related messages. Proposal 2: In the dedicated SL resource pool, dedicated SCI format in PSCCH should be defined for SL-PRS configuration, indication and/or triggering. |

In a dedicated resource pool, SL-PRS can be transmitted in a slot without associated PSCCH

- Support: Nokia, NSB, Futurewei, Spreadtrum, CATT, GOHIGH, Sony, OPPO (scheme 1), Samsung (7)

- Against: vivo, Huawei, HiSilicon, Intel, Xiaomi, CMCC, Apple, Qualcomm, LGE, Ericsson, Mediatek,  (10)

[HIGH] Feature Lead Proposal 3.2.2-v0

For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH.

Companies views

| vivo |  | Support. It would reduce the impact of half-duplexing and increase the transmission performance of SL-PRS. If not supported, once previous slots with PSCCH are missed, the scheduled PRS can not be received. |  |
| --- | --- | --- | --- |
| CMCC |  | Support. If SL-PRS in some slots can be transmitted without an associated PSCCH, it indicates that either a PSCCH in a dedicated RP can reserve more than one SL-PRS resouces, or the associated PSCCH is within a shared RP. We don’t support either, these solution may lead to large specification on, for example, sensing and resource selection, resource overbooking, or linkage between different pools. |  |
| ZTE |  | Support. |  |
| Continental |  | We support the proposal. |  |
| OPPO |  | For SL PRS transmitted by anchor UEs in TDOA method, if resources are allocated by configured grant type 1 or configured grant type 2, there is no dynamic information to be transmitted with PSCCH at all, anchor UE can use higher layer siganling to indicate the SL PRS to target UE(s). Transmitting PSCCH with SL PRS wastes several symbols for SL PRS without any benefit. The proposal is acceptable for us if it is only for Scheme 2 and dyanmic grant in Scheme 1, configured grant type 1/2 need more discussion. |  |
| LGE |  | Support. We understand FL proposal so that SL-PRS is always transmitted together with the associated PSCCH in a same slot. |  |
| Futurewei |  | Support. |  |
| Samsung |  | In case of the dedicated resource pool, SL PRS only transmission can reduce the signaling overhead due to SCI. We suggest to consider this feature. |  |
| CEWiT |  | Support. |  |
| CATT |  | In our view, in a dedicated resource pool, each slot should consist of PSCCH resources and SL-PRS resources, but the SL-PRS can be transmitted in a slot without associated PSCCH. When SL-PRS can be transmitted in a slot without associated PSCCH is supported, the UEs can operate the TX/RX turnaround of SL-PRS in the same slot as shown in Figure 4. UE3 can receive the SL-PRS scheduled by UE2 in slot 0, and also can transmit its own SL-PRS in slot N. However, it doesn’t mean that existing a slot can only carry SL-PRS, each slot should consist of PSCCH resources and SL-PRS resources in a dedicated resource pool. |  |
| Panasonic |  | Support |  |
| Spreadtrum |  | We also suggest to consider SL-PRS can be transmitted in a slot without associated PSCCH. |  |
| InterDigital |  | We don’t support this proposal. We think SL-PRS transmission without associated PSCCH can save signaling overhead and reduce UE SCI decoding processing, e.g., in Scheme 1 scenario where SL-PRS resource configuration parameters can be indicated in resource pool (pre)configuration. |  |
| Huawei, HiSilicon |  | Support If the slot without associated PSCCH, then to support Scheme-2 based resource allocation, the SCI will be transmitted by more complex manner. In addition, the Rx UE cannot determine whether the SL-PRS is coming from the original Tx UE or from another unintended Tx UE. |  |
| Sharp |  | We support the proposal. |  |
| SONY |  | Do not support. There are some scenarios where it would still be benefial to allow SL-PRS transmission in a slot without associated PSCCH (e.g., SL-PRS with configured grant). |  |
| Nokia, NSB |  | We can support this to make progress. |  |

[HIGH] Feature Lead Proposal 3.2.2-v0

For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH.

- Support: vivo, CMCC, ZTE, Continental, LGE, Futurewei, OPPO (only for Scheme 2 and dynamic scheme 1), CEWiT, Panasonic, Huawei, HiSilicon, Sharp, Nokia, Apple, Qualcomm, Toyota, Intel, Lenovo, Xiaomi

- Not support: Samsung, CATT, Spreadtrum, Interdigital, Sony

OFFLINE CONSENUS

For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH.

##### PSSCH in the dedicated RP?

The following proposals were found in the contributions:

| Nokia | Proposal 5: PSSCH is not included in dedicated resource pool for SL positioning. Proposal 6: Any signaling associated with a SL PRS transmission in a dedicated resource pool that does not fit into associated control channels (e.g., PSCCH or PSFCH) can be transmitted in an associated resource pool (which could be a shared resource pool or SL-communications-only resource pool). |
| --- | --- |
| vivo | So, we prefer to transmit SL-PRS without PSSCH transmission in the dedicated resource pool. PSCCH which carries SCI associated with SL-PRS transmission(s) is included in the dedicated resource pool (i.e., Option 2 is supported). |
| Huawei, HiSilicon | Proposal 2: Do not include PSSCH within the dedicated RP for SL-PRS transmission. |
| Xiaomi | Proposal 1: Only PSCCH and SL PRS need to be supported in a dedicated SL PRS resource pool. |
| ZTE | Proposal 1: For a dedicated resource pool for SL positioning, in addition to SL-PRS, PSCCH which carries SCI associated with SL-PRS transmission(s) is included in the resource pool. - PSSCH is not included in a dedicated resource pool |
| NEC | Proposal 5: With regard to which channels can be included in a dedicated resource pool for SL positioning, PSSCH associated with SL-PRS transmission(s) is not included. |
| CMCC | Proposal 1: Only SL-PRS and the corresponding PSCCH can be transmitted in a dedicated resource pool. - SL-PRS cannot be transmitted in a slot without associated PSCCH; - Other channels, i.e., PSSCH and PSFCH, is not necessary to be included in the dedicated resource pool. |
| Qualcomm | Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool. |
| Samsung | Proposal 1: For a dedicated resource pool for SL positioning, - SL-PRS can be transmitted in a slot without associated PSCCH. - PSSCH is not transmitted. |
| LGE | Proposal 1: For a dedicated resource pool, no PSSCH is transmitted. |
| CEWiT | Proposal 3: In the dedicated SL resource pool, if slot carries PSSCH then, - Alt 1: Dedicated 1st stage SCI format for SL-PRS and SCI 1-A should be multiplexed. - Alt 2: Dedicated SCI format should consider the transmission of PSSCH. |

PSSCH is included in dedicated resource pool for SL positioning.

- Support: CEWiT (?)

- Against:  Nokia, NSB, vivo, Huawei, HiSilicon, CATT, GOHIGH, Xiaomi, ZTE, NEC, CMCC, Qualcomm, Samsung, LGE

[HIGH] Feature Lead Proposal 3.2.2.1-v0

PSSCH is not included in dedicated resource pool for SL positioning.

Companies views

| vivo |  | Support Based on the following agreement, we think we have reached consensus no PSSCH in the dedicated resource pool, otherwise, why only single SCI is used? Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH |  |
| --- | --- | --- | --- |
| CMCC |  | Support |  |
| ZTE |  | Support. In our understanding, we had an agreement made in the last RAN1 meeting that only a single stage SCI is used for triggering/reserving SL-PRS resource(s), 2nd stage SCI is not supported and therefore there is no need to transmit PSSCH in dedicated resource pool. |  |
| OPPO |  | Support. |  |
| LGE |  | Support |  |
| Futurewei |  | Support. |  |
| Samsung |  | Support |  |
| CEWiT |  | Do not support. Our understanding from the last meeting agreement is that for SL-PRS transmission, only single stage SCI will be used and PSSCH discussion was postponed. Moreover PSSCH transmission over dedicated channel needs only SL-SCH related to the positioning message to be transmitted. It is possible that there need not to be second stage SCI appended to it. This could be one solution so last agreement can not be reason to exclude it. PSSCH As justification to the PSSCH over dedicated resource pool is that dedicated pool should be self sufficient from positioning point of view. It will make sure the optimized SL operation for SL positioning usecases. Further, it will avoid the fragmentation of the dedicated resources as mostly the allocation in time domain will be slot specific and it may not be necessary to use all symbols for SL-PRS transmission ([M,N] values are configurable) in a slot. |  |
| CATT |  | Support |  |
| Panasonic |  | Support |  |
| Spreadtrum |  | Support |  |
| InterDigital |  | We support that PSSCH is not included in a dedicated resource pool. |  |
| Huawei, HiSilicon |  | Support |  |
| Sharp |  | We support the proposal. |  |
| Nokia, NSB |  | OK |  |

[HIGH] Feature Lead Proposal 3.2.2.1-v0

PSSCH is not included in dedicated resource pool for SL positioning.

- Support: vivo, CMCC, ZTE, OPPO, LGE, Futurewei, Smasung, CATT, Panasonic, Spreadtrum, Interdigital, Huawei, HiSilicon, Sharp, Nokia , Xiaomi, Qualcomm, Apple, Intel, Toyota

- Not support: CEWiT

OFFLINE CONSENSUS

PSSCH is not included in dedicated resource pool for SL positioning.

- Support: vivo, CMCC, ZTE, OPPO, LGE, Futurewei, Smasung, CATT, Panasonic, Spreadtrum, Interdigital, Huawei, HiSilicon, Sharp, Nokia , Xiaomi, Qualcomm, Apple, Intel, Toyota

##### PSFCH in the dedicated RP?

The following proposals were found in the contributions:

| Nokia | Proposal 4: Support inclusion of PSFCH in dedicated resource pool for SL positioning. |
| --- | --- |
| Futurewei | We note that this scheme is suitable for both dedicated and shared resource pool transmissions for this reason PSFCH should be supported in dedicated resource pools. |
| Huawei, HiSilicon | Proposal 3: Do not include PSFCH within the dedicated RP for SL-PRS transmission. |
| CATT, GOHIGH | Proposal 14: Whether PSFCH could also be included will be decided according to the IUC design. |
| Xiaomi | Proposal 1: Only PSCCH and SL PRS need to be supported in a dedicated SL PRS resource pool. |
| Sony | Proposal 6: Support a dedicated resource pool to include PSFCH-like channel to be used for positioning purpose. |
| CMCC | Proposal 1: Only SL-PRS and the corresponding PSCCH can be transmitted in a dedicated resource pool. - SL-PRS cannot be transmitted in a slot without associated PSCCH; - Other channels, i.e., PSSCH and PSFCH, is not necessary to be included in the dedicated resource pool. |
| Interdigital | Proposal 24: Include PSFCH in a dedicated resource pool in addition to SL-PRS and PSCCH. |
| Qualcomm | Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool. |
| OPPO | Proposal 1: For a dedicated SL PRS resource pool, slots with less than 7 OFDM symbols for SL operation and slots configured with PSFCH resources can be included. |
| LGE | Proposal 3: For a dedicated resource pool, PSFCH transmission can be (pre)configured for the feedback-based SL PRS retransmission. |

PSFCH can be included in dedicated resource pool for SL positioning.

- Support: Nokia, NSB, Futurewei, Sony, Interdigital, OPPO, LGE

- Against:  Huawei, HiSilicon, , Xiaomi, CMCC, Qualcomm,

- Wait for IUC decision: CATT, GOHIGH

[LOW] Feature Lead Proposal 3.2.2.1-v0

Decide first whether to support IUC for dedicated resource pool, or whether to support feedback mechanisms.

Companies views

| vivo |  | Support. We need to decide whether to support feedback mechanism and/or scheme 2 based IUC in the SL positioning first. |  |
| --- | --- | --- | --- |
| ZTE |  | Wait for the decision on IUC. |  |
| Futurewei |  | Not sure what not support would mean in an answer to the proposal. We are OK to wait for the decision on IUC and feedback in order to decide if PSFCH is supported. |  |
| InterDigital |  | We agree with the proposal. In addition, we think it is important to support both SL PRS feedback and IUC Scheme 2. Note in a shared resource pool, IUC Scheme 2 can be applied to a transmission including both PSSCH (SL-SCH) and SL-PRS and thus SL-PRS in a shared resource pool can already benefit from IUC Scheme 2 using PSFCH and we thinks SL-PRS in a dedicated resource pool should at least has similar reliability enhancement. |  |
| Huawei, HiSilicon |  | Only IUC scheme-2 is associated with PSFCH, so IUC scheme2 is intention of this proposal to our understanding |  |

[MEDIUM] Feature Lead Proposal 3.2.2.1-v2

PSFCH is not included in dedicated resource pool for SL positioning.

Companies views

| CATT | Support. Since we had agreed that IUC signalling is not supported for the dedicated resource pool, and ACK/NACK feedback for SL-PRS is not supported in Release 18, PSFCH is not needed in dedicated resource pool for SL positioning. |
| --- | --- |
| ZTE | Support, same reason as CATT. |
| Samsng | Support |
| Sharp | We support the proposal. |
| OPPO | OK. As no IUC scheme 2 and no feedback based re-transmission, PSFCH is not needed in dedicated resource pool. |
| Lenovo | Support, similar understanding as CATT |
| LGE | Support as we have agreed no feedback on SL PRS and IUC in a dedicated resource pool. |
| Nokia, NSB | OK. We share CATT’s view. |

OFFLINE CONSENSUS

PSFCH is not included in dedicated resource pool for SL positioning.

##### PSCCH structure in the dedicated RP

Agreement For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH. Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

FL comment: It came to my attention that we don’t have a proposal on the location of PSCCH and related details for the dedicated resource pool. I would like to see if there is any company that thinks that there should be changes compared to legacy PSCCH structure, so we can discuss the following proposal:

[MEDIUM] Feature Lead Proposal 3.2.2.3-v2

In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects:

- the first PSCCH symbol is mapped to the 2nd symbol available for SL tranmsissions in a slot

- the number of PSCCH symbol(s) is (pre-)configured to 2 or 3 symbols using the sl-TimeResourcePSCCH

- the number of PRBs is (pre-)configured using the sl-FreqResourcePSCCH

- mapping of PSCCH and PSCCH DM-RS in the slot

Companies views

| CATT | Support |
| --- | --- |
| ZTE | Generally ok with clarification questions. Firstly, are we also assuming the first symbol of a slot is used as AGC and the last symbol of a slot is used as gap symbol? Furthermore, is the intention of this proposal to reuse the same IE (i.e. sl-TimeResourcePSCCH, sl-FreqResourcePSCCH)? we prefer to use another parameter name for SL positioning. |
| Samsung | OK. BTW, the 1st symbol would be AGC symbol. Is it separate discussion? |
| Qualcomm | We agree with the proposal since it reuses existing structure and does not increase control monitoring complexity. |
| Sharp | Fine with the direction of the proposal but no need to mention the higher layer parameter names (they would be up to RAN2), e.g. for the last but one bullet it is sufficient to say “(pre-)configuration of the number of PRBs”. |
| OPPO | Because SL PRS bandwidth can be same as resource pool, 1 symbol is sufficient to gurantee the coding rate of PSCCH and thus should be supported. Starting position of each PSCCH resource should be separately (pre-)configured if sub-channel is not (pre-)configured in the dedicted resource pool. Bandwidth of PSCCH can be derived based on the bandwith of resource pool W and the number of SL PRS resources multiplexed in a slot N, e.g., floor (M/N). |
| Lenovo | Agree to re-use existing structure as a starting point |
| LGE | We agree on the first 3 bullets. Regarding the 4th bullet, we think some improvement is needed to avoid overlap between PSCCHs when multiple SL PRS resources are multiplexed in a slot. |
| Nokia, NSB | OK |

[MEDIUM] Feature Lead Proposal 3.2.2.3-v2

In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects:

- the first PSCCH symbol is mapped to the 2nd symbol available for SL tranmsissions in a slot

  - Note: 1st symbol available for SL tranmsissions in a slot is for PSCCH AGC similar to legacy

- PSCCH DM-RS in the slot is being reused from legacy

- the number of PSCCH symbol(s) is (pre-)configured to

  - Alt. 1: 2 or 3 symbols (same as legacy)

    - LGE, CMCC, Huawei, HiSilicon, Qualcomm, Samsung, vivo, Sony, Lenovo, Fraunhofer, Intel, Apple

  - Alt. 3: 1, or 2 or 3 symbols

    - Ericsson, CEWiT, OPPO, CATT, ZTE

- the number of PRBs is (pre-)configured using the legacy values

  - FFS: reconsider if 1-symbol PSCCH is supported.

### 3.2.3 SCI for SL-PRS

Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

| Spreadtrum | Proposal 4: The frequency domain range of SCI blind detection needs to be limited in dedicated resource pool. |
| --- | --- |

### 3.2.4 Mapping Relationship b/w PSCCH and SL-PRS resources

| Nokia | Proposal 9: Consider mapping PSCCH locations to both mini-slot allocations and first PRB of the SL PRS in dedicated resource pools. |
| --- | --- |
| Futurewei | Proposal 4: Support an implicit one-to-one mapping between the location of PSCCH and the comb parameters of the SL-PRS multiplexed in the same slot. Proposal 7: Support implicit one-to-one mapping between the frequency location of PSCCH and the symbol location of the SL-PRS TDM multiplexed in the same slot. |
| Spreadtrum | Proposal 2: If SL PRS and its associated PSCCH are not in the same slot is supported, a mapping relationship between SL PRS resources and PSCCH resources need to be defined. Proposal 6: The SCI indicating to the receiving UE the SL-PRS allocation in the same resource pool as SL PRS resource at least should be supported. Proposal 8: The association between PSCCH frequency domain monitoring ranges and SL PRS in a slot should be supported for SL PRS resource selection. |
| Huawei, HiSilicon | Proposal 6: Support to introduce one–to-one mapping scheme between SL-PRS resource and PSCCH resource. Proposal 7: Support to divide the bandwidth of the dedicated RP into several sub-bands or sub-channels for PSCCH resources according to the number of the SL-PRS resources to be transmission within the same slot. Proposal 8: Support to study the following two association schemes between SL-PRS and PSCCH resources - Option1: The association is determined based on the order of the SL-PRS resource ID and sub-band or sub-channel index of PSCCH resource. - Option2: The association can be indicated during the (pre)configuration of SL-PRS resources. |
| CATT, GOHIGH | Proposal 12: For the mapping rule between PSCCH and SL-PRS in dedicated resource pool, the method of determination for PSFCH based on associated PSSCH can be as a starting point. |
| Intel | - For Scheme 2 SL PRS resource allocation in a dedicated resource pool, mapping between PSCCH subchannel and SL PRS resource is supported. - FFS: details, including whether explicit SL PRS resource indication in the single stage SCI is additionally supported. c |
| Xiaomi | Proposal 6: A mapping relationship is defined between resource of PSCCH and the associated SL PRS resource in a dedicated resource pool. |
| ZTE | Proposal 3: Support mapping between a PSCCH/SCI configuration and a SL-PRS resource for a dedicated resource pool, where: - Frequency resource occupied by SCI is limited to a range of bandwidth of dedicated resource pool and multiple SCIs can be multiplexed in FDM manner. |
| NEC | Proposal 6: At least for TDM-ed multiplexing of SL PRS transmissions from different UEs in dedicated resource pools, the mapping between SL PRSs and associated PSCCHs should be defined. |
| CMCC | Proposal 2: Mapping relationship b/w PSCCH and SL-PRS resources may need to be re-designed for NR SL positioning. - PSCCH-PSSCH mapping relationship can be a reference; - Comb-based multiplexing of SL PRS should be considered when designing the mapping relationship; - TDM-based multiplexing of SL PRS should also be considered when designing the mapping relationship. |
| LGE | Proposal 9: The location of PSCCH and PSSCH for SCI transmission is determined based on TDM index and/or comb RE offset of the associated SL PRS resource within a slot in a dedicated resource pool. Proposal 10: The candidate positions of SL PRS resource in a slot are determined based on the number of symbols of SL PRS resource. |
| Sharp | Proposal 6: In a dedicated resource pool for positioning, each SL-PRS resource is associated with a PSCCH which is TDMed with the SL-PRS in the same slot. Proposal 8: In a dedicated resource pool for positioning, for aperiodic reservations, SCI indicates a few SL-PRS resources with a same SL-PRS resource ID. - The interval between two adjacent SL-PRS resources is configurable (e.g. to be in slots, or in SL-PRS occasions). |

For a dedicated resource pool for SL positioning, support a mapping relationship between a PSCCH resource and the resource of the associated SL-PRS in a slot

- FFS: Details

Companies views

| vivo |  | Generally support. In our opinion, the following options can be taken into consideration to determine the resource of SL-PRS: Option 1. The mapping relationship between a PSCCH and the resource of the associated SL-PRS is configured by the higher layer parameters. Option 2. The resource of the SL-PRS can be indicated in the PSCCH. |  |
| --- | --- | --- | --- |
| CMCC |  | Support. With the FL proposal 3.2.2, it would be clearer to say “support a mapping relationship between a PSCCH resource and the resource of the associated SL-PRS in a the same slot” |  |
| ZTE |  | Generally ok. Our understanding is aligned with Option 1 as commented by vivo. The mapping between SCI and the associated SL-PRS can be configured for each dedicated resource pool. Also since we do not have beam related function in Rel-18 SL positioning, we suggest to first agree on one-to-one mapping a PSCCH resource and the resource of the associated SL-PRS in a slot. For a dedicated resource pool for SL positioning, support a one-to-one mapping relationship between a PSCCH resource and the resource of the associated SL-PRS in a slot |  |
| Futurewei |  | Support |  |
| CEWiT |  | Support |  |
| CATT |  | Support |  |
| Spreadtrum |  | Support |  |
| InterDigital |  | Yes. We support this mapping relationship. |  |
| Huawei, HiSilicon |  | Support We think one-to-one mapping scheme between SL-PRS and PSCCH resource should be introduced. |  |
| Sharp |  | We support the proposal. We are also fine with ZTE’s modification. Regarding CMCC’s comment, RAN1 already agreed in the last meeting that “PSCCH and associated SL-PRS are TDMed in the same slot.” |  |

FL comment: I am merging this proposal with 3.2.5 since it was commented that they are related. Lets continue the discussion there.

### 3.2.5 Fields in the SCI

| vivo | - For the single stage SCI used for the reservation/indication of SL-PRS, the following information should be carried in the SCI. - SL PRS timing/frequency resource, destination ID, source ID, period indication, priority, SL PRS resource ID. |
| --- | --- |
| spreadtrum | Proposal 7: Regarding on the content included in SCI for SL PRS transmission in dedicated resource pool, the following fileds should be included. - Source ID - Destination ID - Resource reservation period - SL-PRS Priority - SL PRS resource ID - 24-bit CRC Field - SL-PRS request |
| Huawei, HiSilicon | Proposal 9: In the dedicated resource pool, the SCI should at least include the following fields: - ID information, Destination ID and Source ID; - Priority; - Time resource assignment; - SL-PRS resource assignment; - Resource reservation period; |
| CATT, GOHIGH | Proposal 15: In the dedicated resource pool, SCI for SL-PRS should at least include the following fields: - 24-bit Source ID - 24-bit Destination ID - Resource reservation period - SL-PRS Priority - Cast type - Time-domain resource assignment (slot-level) - Frequency-domain resource assignment (slot-level) - SL-PRS configuration and/or time-frequency assignment information in a slot: - Opt. 1: when the associated parameters are (pre-)configured and lack of flexibility - Opt. 2: when the associated parameters are quite flexible - 24-bit CRC Field - SL-PRS request - Positioning Session ID |
| Xiaomi | Proposal 5: The following information elements are considered as candidate included in the single stage SCI for SL PRS in dedicated resource pool - Resource indication for the associated SL PRS transmission in the same slot - Resource reservation indication for future SL PRS transmission(s) - Priority information - Source and destination ID information - SL PRS measurement assistant information |
| ZTE | Proposal 5: In the dedicated resource pool for positioning, with regards to the SCI for SL-PRS, information carried in SCI for SL-PRS should at least include: - SL-PRS priority - The priority value of SL-PRS can be configured as decimal to distinguish the service priority between positioning and communication - Source ID (24 bits) - Destination ID (24 bits) - Cast type - Indicating either unicast, groupcast or broadcast SL-PRS transmission - Resource reservation period - SL-PRS resource allocation information - SCI only indicates time resource assignment |
| Apple | Proposal 3: In the dedicated RP, the SCI should contain the following fields: - Source ID - Destination ID - Resource reservation period - SL-PRS Priority - Cast type – needed to support unicast, groupcast and broadcast SL-PRS. - SL-PRS configuration and/or time-frequency assignment information: - Opt. 1: SCI includes a SL-PRS resource ID that indicates one of the SL-PRS resource(s) that have been provided to the UE with higher layer signaling |
| Qualcomm | Proposal 22: In a dedicated resource pool, the new SCI includes SL-PRS time and frequency allocation, periodicity, positioning session identifiers, and an index to an SL-PRS configuration provided by SLPP/LPP. |
| OPPO | Proposal 4: In the dedicated resource pool, SCI for SL-PRS does not include “Cast type” field and “SL-PRS request” field, whether “Session ID” is included or not is up to higher layer. |
| Sharp | Proposal 7: In a dedicated resource pool, SCI for SL-PRS should at least include the following fields, - Layer 2 source ID. - Layer 2 destination ID. - Resource reservation period. - SL-PRS priority. - Cast type. |
| CEWiT | Proposal 4: For Dedicated SL resource configuration for SL positioning, reporting of the measurements can be triggered by Tx UE in the first stage SCI. - Resource reservation mechanism of first stage SCI (e.g., SCI 1-A) can be used to decide the measurement reporting resources. - PRS report trigger field should be provided as additional signal using reserved bit. |

Companies views

| vivo |  | Due to low layer trigger is supported as WA in the previous meeting, we support SL PRS request at least for RTT positioning method. For the cast type, we prefer to discuss how to use the information in lower layer first, and put it as FFS. In addition, considering the support of SL PRS resource ID in section 9.5.1.1, SL PRS resource ID should be added for resource-based reservation. |  |
| --- | --- | --- | --- |
| CMCC |  | We are generally fine with most of the bullets. Regarding the second last bullet, we would like to first clarify what exactly is SL-PRS configuration? Does it include, for example, the comb size, RE offset, SL-PRS pattern, etc.? If so, we don’t think these configuration parameters can be carried in SCI, it can be per resource pool level (pre-)configured to simply the resource selection. |  |
| ZTE |  | Comment 1: For the “cast type”, we believe it is necessary. If unicast is applied, Tx UE’s destination ID needs to be matched with Rx UE’s source ID. However, if groupcast/broadcast is applied, Tx UE’s destination ID needs to be matched with Rx UE’s destination ID. Different cast type hasdifferent filtering pairs. Comment 2: For “SL-PRS configuration and/or time-frequency assignment information”, this bullet is related to FL proposal 3.2.4. If each PSCCH location is associated with one SL-PRS resource, there is no need to indicating SL-PRS resource ID at least for the same slot. Only time resource assignment is needed. The remaining issue is whether we can have different SL-PRS resources (e.g. different SL-PRS resource ID) for multiple SCI reserved time resources. Comment 3: Regarding the FFS part, Positioning session ID is a higher layer parameter and we doubt the necessity of including it in SCI. SL-PRS request is also not ready to be agreed, we should wait for the progress of FL Proposal 5.1. |  |
| Futurewei |  | Support in general. Not sure if the cast type is necessary if we already have the desitination ID. |  |
| CEWiT |  | We support the cast type to be included as similar opinion as ZTE. Regarding FFS we support of SL PRS request in general and it will be helpful to coordinate the transmission among the UEs. As the discussion on PSSCH is going on, FFS can be added on inclusion of PSSCH related parameter if supported. |  |
| CATT |  | OK with the proposal. We prefer to include the “cast type” in the SCI. |  |
| Spreadtrum |  | “SL PRS request” should be included in SCI. |  |
| InterDigital |  | We support the proposal. In addition, we proposed to add two more parameters for FFS - L-PRS feedback request (if SL-PRS feedback is supported): this indication can be used to request a feedback for high priority SL-PRS transmissions. - number of resource reservation periods: this indication is to indicate the how many periods the SL-PRS transmissions may be performed. SL-PRS transmissions with periodic reservation is agreed to use a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication. However, the number of reserved periods is conveyed in (pre)configured resource re-selection counter value. For SL-PRS transmission, we think the number of the periods can depend on SL-PRS session duration, latency requirement, etc. and it would be helpful to indicate this explicitly in SCI. In the dedicated resource pool, SCI for SL-PRS should at least indicate the following values: - Source ID - Destination ID - Resource reservation period - SL-PRS Priority - [Cast type] - SL-PRS configuration and/or time-frequency assignment information: - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, SL-PRS feedback request (if feedback is supported), number of resource reservation periods |  |
| Huawei, HiSilicon |  | Ok in general. |  |
| Sharp |  | We don’t think the last but one bullet is clear. We are fine with other bullets. |  |

FL comment: A related agreement with regards to the SL PRS resource bandwidth and SL PRS resource definition:

Agreement For a dedicated resource pool, at least the case where SL PRS bandwidth is same as resource pool bandwidth is supported. Agreement - A SL PRS resource refers to a time-frequency resource within a slot of a dedicated SL PRS resource pool that is used for SL PRS transmission. - FFS: for a shared resource pool - Characteristics associated with a SL PRS resource include at least: - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation, - Note: Additional parameters can be included as/when identified. - FFS: other time domain aspects, if any - A SL PRS resource is identified by a SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool.

FL comment: Please note i am merging some bullets from the Subsections 3.2.4 and Subsections 3.2.5 to try to make as complete the proposal on how the allocation will work for dedicated resource pool.

[MEDIUM] Feature Lead Proposal 3.2.5-v1

In the dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a

  - SL-PRS-resource-based allocation

- A single transmitting UE can reserve

  - Alt. 1.1: One SL PRS resource in a slot

  - Alt. 1.2.: One or more SL PRS resources in a slot that can be TDMed

- SCI for SL-PRS should at least indicate the following values:

  - Source ID

  - Destination ID

  - Resource reservation period

  - SL-PRS Priority

  - With regards to cast type

    - Alt. 2.1: It should be indicated

    - Alt. 2.2: It should not be indicated

  - With regards to the SL-PRS configuration and/or SL-PRS time assignment information

    - Alt. 3.1: support a one-to-one mapping relationship between a PSCCH resource and a SL-PRS resource of the associated SL-PRS in the slot.

      - Note: In this case, there is no need of an explicit signaling of which SL PRS resource.

    - Alt. 3.2: SL PRS resource ID

  - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, number of resource reservation periods

Companies views

| CATT | We prefer Alt. 1.2, Alt. 2.1, and Alt.3.1. For Alt. 1.1 and 1.2, this issue is being discussed in AI 9.5.1.1 as following proposal, we prefer to discuss this issue in one AI to avoid possible conflict with each other. ### [Medium] FL1 Proposal 2.4-2 (in AI 9.5.1.1) - TDM-ed SL PRS resources within a slot from a single UE in a dedicated/shared resource pool is: - Alt A1: Supported in both dedicated and shared resource pools - Alt A2: Supported in only dedicated resource pools - Alt B: Not supported. |
| --- | --- |
| ZTE | Here is our preference Alt. 1.1: A single transmitting UE can reserve one SL PRS resource in a slot. This is indeed also discussed in AI 9.5.1.1. Alt. 2.1: cast type should be indicated in SCI, the reason is same as we present in the first round discussion. Alt. 3.1 with comments: time resource assignment field is also needed. We believe one SCI can at most reserve 3 SL-PRS resources. |
| Samsung | Agree with CATT on duplicated discussion for Alt. 1.1 and 1.2. Support Alt. 1.2, Alt. 2.1, and Alt.3.1. |
| Qualcomm | Alt 1.1 Alt 2.1. The issue of cast type has been discussed in R16 and R17 and the conclusion is that destination ID is insufficient to determine the cast since there is no ID segmentation by cast type in the higher layers. Alt 3.2 Explicit indication of the SL-PRS resource simplifies design and easily addresses TDM and comb-based multiplexing. A mapping requires defining equations to for when only TDM is enabled, only comb-based multiplexing is enabled, both are enabled, neither is enabled. Our understanding is that RAN1 still needs to discuss how indicate these values. |
| Sharp | Regarding Alt. 1.1 and Alt. 1.2, we wonder if the intention of “in a slot” is actually “per slot”, i.e. in an SCI the UE can reserve SL-PRS resources in one or more slots, with a restriction on the number of SL-PRS resources per slot. Our first preference is Alt. 1.2 for future proofness, but can be also fine with Alt. 1.1. Regarding other bullets we support Alt. 2.1 and Alt. 3.1. And we think a field similar to TRIV in legacy sidelink is also needed in SCI. In fact we think the TRIV definition can be fully reused. |
| Continental | We support Alt 1.2 and Alt 2.1. For the bullet on SL-PRS configuration and/or SL-PRS time assignment information, we are not clear how Alt 3.1 or Alt 3.2 (assuming only one SL PRS resource ID) would work if multiple SL PRS resources are TDMed in a slot. |
| OPPO | Fine in general, we support Alt1.1, Alt 2.2, and Alt 3.1. For Alt. 3.2 maybe we can make it clearer: Alt. 3.2: explicit signaling of SL PRS resource ID |
| Lenovo | Alt 1.1 (simplified design), Alt 2.1, Alt (3.2) |
| LGE | For the 2nd bullet, we support Alt 1.2 so that UE has no constraint in using any TDM durations within a slot. This would help to reduce the latency and to improve the resource utilization. For the 3rd bullet, - For the 5th sub-bullet, the cast type needs to be included same as in SL communication. For the 6th sub-bullet, support Alt 3.1 because SL PRS resource ID is not directly related to the time/frequency location of SL PRS resource. |
| NEC | Support Alt 1.2, Alt 2.1 and Alt 3.1, in addition, we also think Positioning Session ID is needed for sessions in parallel which may associated with different SL PRS. |
| Nokia, NSB | Alt. 1.2 is our preference. But, we can discuss this once we have an outcome on discussion related to TDM-ed SL PRS resources within a slot from a single UE in AI 9.5.1.1. Alt. 2.1.- as discussed in previous releases, (destination id, cast type) together are required |

OFFLINE CONSENSUS

In the dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a

  - SL-PRS-resource-based allocation

- SCI for SL-PRS should at least indicate the following values:

  - Source ID

  - Destination ID

  - Resource reservation period

  - SL-PRS Priority

  - Cast type

  - With regards to the SL-PRS configuration and/or SL-PRS time assignment information

    - Alt. 3.1: support a one-to-one mapping relationship between a PSCCH resource and a SL-PRS resource of the associated SL-PRS in the same slot.

      - Note: In this case, there is no need of an explicit signaling of which SL PRS resource for the same slot

      - Note: Same number of PSCCH resource(s) and SL-PRS resource(s)

    - Alt. 3.2: explicit signaling of SL PRS resource in the same slot

    - Alt. 3.3: support a mapping relationship between a PSCCH resource and one or more SL-PRS resource(s) of the associated SL-PRS in the same slot and explicit signaling of SL PRS resource

    - FFS: Whether and how to indicate SCI resource(s) or SL-PRS resource (s) for a future slot

  - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, number of resource reservation periods

### 3.2.7 Resource allocation granularity

The following agreement was reached with regards to this topic:

Agreement Study further the granularity of time-domain resource allocation for SL-PRS transmission.

The following proposals were found in the contributions:

| Spreadtrum | Proposal 5: SL-PRS-resource-based allocation should be supported in dedicated resource pool. |
| --- | --- |
| Huawei, HiSilicon | Proposal 10: For a dedicated resource pool, with regards to the SL-PRS time-domain resource allocation, only one slot based SL-PRS resource can be reserved by one UE within each slot. Proposal 11: For a dedicated resource pool, with regards to the SL-PRS time-domain resource allocation, support SL-PRS-resource-based allocation, where the occupied OFDM symbols of each SL-PRS resource are (pre-)configured and cannot be dynamically changed by SCI. Proposal 12: For a dedicated resource pool, SL-PRS-resource-based allocation indicates that the bandwidth of each SL-PRS resource should be (pre-)configured and cannot be dynamically changed via SCI indication. |
| CATT, GOHIGH | Proposal 5: SL-PRS resource should be the resource allocation unit in time-domain for SL-PRS transmission. |
| Xiaomi | Proposal 4: PRS subchannel is defined as the frequency domain resource allocation granularity |
| ZTE | Proposal 6: With regards to the SL-PRS time-domain resource allocation within a dedicated resource pool, support a SL-PRS-resource-based allocation. - An allocated SL-PRS resource occupies at least one symbol within a slot. |
| NEC | Proposal 8: With regard to the SL PRS time resource allocation within the resource pool, SL-PRS-resource-based allocation can be supported for Scheme 1 allocation. |
| Sony | Proposal 7: support slot- or sub-slot-based SL-PRS resource allocation |
| Interdigital | Proposal 2: Support time domain resource allocation based on SL-PRS resource in a dedicated resource pool. |
| Qualcomm | Proposal 6: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size, comb offset, and TDM partition. Proposal 7: Sub-slot TDM SL-PRS resource partitioning, when supported by the UE and enabled for the dedicated resource pool, is semi-static. |
| OPPO | Proposal 5: For SL PRS time domain resource allocation in a dedicated resource pool SL PRS resource-based allocation is used. |
| LGE | Proposal 17: SL PRS resource is the granularity of the SL PRS resource allocation |

[MEDIUM] Feature Lead Proposal 3.2.7-v0

For a a dedicated resource pool,

- with regards to the SL-PRS time-domain resource allocation within the resource pool support a

  - SL-PRS-resource-based allocation

- The OFDM symbols of each SL-PRS resource in a slot

  - Alt. 1: cannot be dynamically changed by SCI

  - Alt. 2: can be dynamically changed by SCI

- A single transmitting UE can reserve

  - Alt. 1: One SL PRS resource in a slot

  - Alt. 2: One or more SL PRS resources in a slot that can be TDMed

Companies views

| vivo |  | Alt 1+Alt 1 For us, we prefer the resource used for SL-PRS in a slot is (pre-)configured in the resource pool, and UEs used the resource pool have a common understanding on the SL-PRS resource in a slot. Then, the indication in the SCI can be used for resource reservation and sensing. For last bullet, it is unclear to us why UE needs to reserve multiple resources in aresource pool. We think one reserved SL PRS resource in a slot is enough, and if a single transmitting UE rreserves multiple resources in a slot, the UE may need to support additional capability. |  |
| --- | --- | --- | --- |
| CMCC |  | We are fine with the 1st bullet. For the 2nd bullet, we support Alt. 1, we think that the starting symbols and number of symbols of a SL PRS resource in a slot should be semi-persistantly (pre-)configured per RP level. For the 3rd bullet, we would like to first clarify if more than one SL-PRS resources are reserved in a slot, does it intended for the same UE or can be for different UEs? Does it include the case that the multiple SL-PRS resources are the repetitions of one SL-PRS resource (which also is related to the discussion in AI 9.5.1.1)? |  |
| ZTE |  | - Agree with SL-PRS-resource-based allocation - Alt.1, we think the OFDM symbols of each SL-PRS resource in a slot is configured in a resource pool. - Alt.1, there will be more discussion point raised if Alt.2 is agreed. For example, if more than one resources are intended for different UEs, do we need more source/destination ID indicated in one SCI? Or does one UE transmit more than one SCI in a slot (cause we think there are one-to-one mapping between one SCI and one SL-PRS resource)? |  |
| Futurewei |  | Support first bullet. Alt 1 for second bullet, given that a fixed mapping is supported between PSCCH and SL-PRS Alt 1 for third bullet Is here an overlap with other AI? |  |
| CATT |  | Alt 1+Alt 1 |  |
| Spreadtrum |  | The OFDM symbols of each SL-PRS resource in a slot cannot be dynamically changed by SCI. A single transmitting UE can reserve one SL PRS resource in a slot. |  |
| InterDigital |  | We support the proposal. For 2nd bullet, we think either Alt. 1 or Alt. 2 can apply depending on the SL PRS resource (pre)configuration in a resource pool. So we should first discuss how SL-PRS resource can be (pre)configured in a resource pool. For example, if SL-PRS resource starting symbol location and duration is (pre)configured in each slot in a resource pool, then we think Alt. 1 is preferred. Otherwise, Alt. 2. can be considered. For 3rd bullet, we support Alt. 2 so a UE can reserve multiple TDM:ed SL-PRS resources in one slot, e.g. for SL-PRS repetition. |  |
| Huawei, HiSilicon |  | Support. For OFDM symbols in a slot, we support: Alt1. For single Tx UE reservation, we support: Alt1. |  |
| Sharp |  | We are fine with the first bullet. On the second bullet, we support Alt. 1. On the third bullet, we prefer Alt. 2 but can be also fine with Alt. 1 if that is the majority view. |  |

FL comment: By looking at these responses, i think we can merge this into the 3.2.4, and have one proposal that is trying to address how the SCI will be used for SL PRS resource indication.

### 3.2.8 Additional Proposals

The following proposals were found in the contributions:

## Shared Resource Pool

The following agreement was reached with regards to this topic:

Agreement With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 Ues should be ensured.

### SCI for SL-PRS

Agreement With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication - FFS: Details including a new or existing 2nd stage SCI

| vivo | - For the SCI in the shared resource pool, - SCI format 1-A can be reused. - SL-PRS flag indication can be indicated by the SCI. |
| --- | --- |
| Spreadtrum | Proposal 13: A new Second-order SCI can be introduced for R18 UE for SL-PRS transmission in shared resource pool. |
| Huawei, HiSilicon | Proposal 17: For a shared resource pool, indication of a PSSCH only transmission or a PSSCH+SL-PRS transmission by some reserved bits in the SCI-1 or by the indication of SL-PRS based second stage SCI format 2 can be considered. Proposal 18: For a shared resource pool, a new second stage SCI format associated with SL-PRS to indicate the reservation of SL-PRS with a granularity of SL-PRS resource can be considered. Proposal 19: For the Alt.C.2, the following signaling should be indicated in the second stage SCI: - The legacy contents of SCI format 2-A/2-B/2-C. - SL-PRS transmission parameters. |
| CATT, GOHIGH | Proposal 16: For the backward compatibility of shared resource pool, the PSCCH design for SL-PRS should keep the same as that in Rel-16/17. Proposal 17: A new 2nd-stage SCI should be introduced for scheduling SL-PRS in a shared resource pool. |
| Intel | - For Scheme 2 SL PRS resource allocation in a shared resource pool, existing SCI 1-A is reused and codepoint “11” in the field of 2nd-stage SCI format is used to indicate a new 2nd stage SCI format. |
| ZTE | Proposal 8: In a shared resource pool, in addition to SL PRS transmission, the UE transmits SCI1-A & a new 2nd stage SCI format used for SL-PRS indication - SCI1-A - Priority indicator of SL-PRS is indicated in SCI format 1-A, priority value of SL-PRS can be integer or decimal - A new 2nd stage SCI - Use one bit in the 2nd stage SCI to indicate whether only SL-PRS is indicated or both SL-PRS and SL-data are transmitted - If both SL-PRS and SL-data are indicated using the same 2nd stage SCI, it is expected that cast type, destination ID, source ID are shared for both SL-PRS transmission and SL-data transmission. |
| Panasonic | Proposal 5: To use N_reserved bits +”11” of 2nd-stage SCI format for new 2nd stage SCI format. Proposal 6: The new 2nd-stage SCI format has information for [24 bits] Source ID, [24 bits] Destination ID, Resource reservation period, SL-PRS Priority, Cast type, SL-PRS configuration and/or time-frequency assignment information, and additional information (e.g. SL-PRS request, Positioning Session ID, UE-types) Proposal 7: New or re-used parameters in the 2nd-stage SCI could be designed as a reception filtering so that SL UEs are not required to monitor all SL-PRS |
| Sony | Proposal 13: RAN1 to define new SCI format for SL-PRS resource allocation purpose.c |
| Interdigital | Proposal 16: Support a new SCI-2 format including at least SL PRS resource information e.g., comb pattern and RE offset in a shared resource pool. Proposal 17: Study SCI format 1-A indications for resource reservation of a SL-PRS transmission with SCI-2 only in a shared resource pool.c |
| Apple | Proposal 6: With regards to the SCI signaling in a shared RP, in addition to SL PRS transmission, a new 2nd stage SCI should be supported. This should include information to enable signaling of the SL-SCH and the SL-PRS. The destination ID for the SL-PRS should include at least the receiver for the SL-SCH channel. |
| Qualcomm | Proposal 23: In a shared resource pool, legacy SCI-1A format is used to indicate transmission time-frequency resources and periodicity. Proposal 24: In a shared resource pool, a new SCI-2 format is introduced and includes at least identifiers associating the SL-PRS with the positioning session (e.g. SLPP UE ID) and SL-PRS configuration index. |
| OPPO | Proposal 11: With regards to the SCI signaling in a shared resource pool, “2nd-stage SCI format” field in SCI 1-A is set to “10”, and the size of the 2nd stage SCI for SL-PRS indication is same as current SCI 2-C. |
| Samsung | Proposal 2: In a shared resource pool, introduce a new 2nd stage SCI for SL-PRS indication. Proposal 13: In a shared resource pool for SL PRS and SL communication, TX UE indicates whether SL PRS is transmitted or not for co-existence with SL communication. - The presence of SL-PRS is indicated in the 1st SCI by using the reserved bits. |
| LGE | Proposal 8: SCI indicates the information of all the SL PRS resources reserved by a UE, which are TDMed within a slot in a shared resource pool. Proposal 27: The 1st-stage SCI associated with SL PRS carries at least the following information. - Resource reservation information of PSSCH carrying the 2nd-stage SCI - Indication of a new 2nd-stage SCI format for SL PRS resource information Proposal 28: For a shared resource pool, a new 2nd SCI format is introduced for indicating the SL PRS resource reservation, which includes at least the followings. - SL PRS resource ID - SL PRS resource starting symbol index within a slot - RE offset of the first symbol of SL PRS resource - Full L2 source ID - Full L2 destination ID Proposal 33: For a shared resource pool, a new 2nd-stage SCI format is introduced. The new SCI format 2-X field above are supported. |
| Sharp | Proposal 2: With regards to the SCI signaling in a shared resource pool, a new 2nd stage SCI format is introduced. |
| lenovo | Proposal 12: SL Positioning SCI contains at least time-frequency and reservation indication information for SL-PRS. FFS details and other positioning related information such as comb pattern size, SL-PRS repetition information, etc. |
| Ericsson | - Resource allocation for the SL PRS in a shared resource pool uses SCI for the purpose of sensing. SCI for SL PRS allocation contains the same resource allocation information as legacy V2X: - Frequency resource assignment in subchannels - Time resource assignment - Resource reservation period - FFS: use of additional fields for positioning |
| CEWiT | Proposal 7: For legacy SL resource configuration for SL positioning, SL-PRS will be indicated using first stage SCI and new second stage SCI. Proposal 8: For legacy SL resource configuration for SL positioning, first stage SCI will be reinterpreted to convey SL-PRS used for SL positioning if no legacy channel is multiplied in the subchannel. Otherwise, reserved bit can be used to indicate the presence of SL-PRS. Proposal 9: For legacy SL resource configuration for SL positioning, a new second stage SCI should be designed to covey SLPP session ID, the positioning reporting configuration and corresponding PSSCH channel reservations for reporting. |

With regards to the SCI signaling in a shared resource pool, support a new format for 2nd stage SCI

- Support: Spreadtrum, Huawei, HiSilicon, CATT, GOHIGH, Intel, ZTE, Panasonic, Sony, Interdigital, Apple, Qualcomm, Samsung, LGE ,Sharp, CEWIT

- Against:

[HIGH] Feature Lead Proposal 3.3.1-v0

With regards to the SCI signaling in a shared resource pool,

- support a new format for 2nd stage SCI.

- If both SL-PRS and PSSCH are indicated using the same 2nd stage SCI, the cast type, destination ID, source ID are shared.

Companies views

| Vivo |  | We are not against the proposal, but we prefer to remove the first sub-bullet, and discuss the second bullet first. This is because all contents listed in the proposal 3.2.5-v0 are presented in the existing SCI, the motivation to support new format for SL PRS transmission is unclear to us. So, we prefer to discuss which parameter needs to be added compared to legacy SCI first, and then we revist whether new 2nd SCI format is needed. |  |
| --- | --- | --- | --- |
| CMCC |  | OK. |  |
| ZTE |  | Support. If both SL-PRS and SL-data are indicated using the 2nd stage SCI, it is expected that the cast type, destination ID, source ID are shared for both Tx UE’s SL-PRS transmission and SL-data transmission. One Tx UE is not expected to transmit SL data to UE1 but transmit SL-PRS to UE2 in order to save overhead/bits for 2nd stage SCI. |  |
| Continental |  | We are OK with the proposal. |  |
| OPPO |  | Regarding to the first sub-bullet, we propose to add “FFS how to indicate the new 2nd stage SCI format”, as there is only one remaining state (“11”) for “2nd-stage SCI format” field in SCI 1-A, for forward compatibility, “11” should not be used unless it has to be. For the 2nd sub-bullet, maybe the PSSCH should be SL-SCH, if the case, we propose to remove this sub-bullet, we have not agreed to support multiplexing SL PRS and SL-SCH in one slot yet. |  |
| LGE |  | We support the 1st sub-bullet. The 2nd sub-bullet depends on Proposal 3.3.3-v0. Let’s first make decision on Proposal 3.3.3-v0 and then decision on the 2nd sub-bullet. We support either PSSCH+SL-PRS or PSSCH+SL-SCH, not both SL-PRS and SL-SCH in a same slot. |  |
| Futurewei |  | OK |  |
| Samsung |  | OK in general. |  |
| CEWiT |  | Support |  |
| CATT |  | Support the first bullet. For the second bullet, we prefer to put it as FFS. |  |
| Panasonic |  | Support |  |
| Spreadtrum |  | Suppport |  |
| InterDigital |  | We support the proposal. |  |
| Huawei, HiSilicon |  | ok |  |
| Sharp |  | We propose to first discuss the condition in the 2nd bullet, i.e. whether “both SL-PRS and PSSCH are indicated using the same 2nd stage SCI”. In our view the condition should always hold for a shared resource pool. |  |
| SONY |  | We support the proposal |  |
| Nokia, NSB |  | OK; in the 2nd subbullet, “PSSCH” should be replaced by SL-SCH, since the 2nd stage SCI is carried over PSSCH and does not indicate that PSSCH, it indicates a TB. |  |

OFFLINE CONSENSUS

With regards to the SCI signaling in a shared resource pool,

- support a new format for 2nd stage SCI.

  - FFS how to indicate the new 2nd stage SCI format

- FFS: If a 2nd stage SCI indicates both SL-PRS and SL-SCH, the cast type, destination ID, source ID are shared.

### SL-PRS multiplexing within the S-RP

Agreement In a shared resource pool: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. A.1: Only TDMing is supported - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. B.1: Only TDMing is supported - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting): - Alt. C.1: 2nd SCI only - Alt. C.2: 2nd SCI and SL-SCH - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH” - FFS: Handling of PT-RS and SL-PRS

| Nokia | Proposal 10: In shared resource pools, PSSCH within the same slot as that of SL PRS can be used for transmitting 2nd SCI only or 2nd stage and SL-SCH, i.e., support Alt.C.3 Proposal 11: In shared resource pools, SL PRS can be TDM'ed and/or FDM’ed with PSSCH, within a slot, i.e., support Alt. A.3. Proposal 14: In shared resource pools, PSCCH and SL-PRS can be only TDM’ed within a slot, i.e., support Alt. B.1. Proposal 12: In shared resource pools, support FDM’ing of SL PRS and PSFCH (e.g., by using remaining resources from legacy SL transmissions) within a slot. |
| --- | --- |
| Futurewei | Proposal 4: With regards to PSSCH and SL-PRS multiplexing, - Alt. A.1: Only TDMing is supported. Proposal 5: With regards to PSCCH and SL-PRS multiplexing - Alt. B.1: Only TDMing is supported. Proposal 6: The PSSCH is used for - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH”. |
| Toyota | Proposal 7: Support FDMing of PSSCH and SL-PRS (Alt. A.2). |
| vivo | - The resource of SL-PRS should be restricted in the resource of the PSSCH in the slot scheduled by the associated SCI. - For a shared resource pool, - The PSSCH and SL-PRS should be TDMed in the slot for a UE. - The starting symbol and/or the number of SL-PRS should be configured by higher layer signaling and/or indicated by SCI. - If FDMing of PSSCH and SL-PRS is supported in a slot, the PSSCH should rate-match around SL-PRS RE to improve the resource utilization of the resource pool. - For a shared resource pool, the PSCCH and SL-PRS should be TDMed in a slot for a UE. - The PSSCH in a slot can carry 2nd SCI only, or carry 2nd SCI and SL-SCH. |
| Spreadtrum | Proposal 9: With regards to PSSCH and SL-PRS multiplexing in shared pool, only TDMing should be supported. Proposal 10: With regards to PSCCH and SL-PRS multiplexing in shared pool, only TDMing should be supported. Proposal 11: When SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot, the PSSCH in one slot is used for “2nd SCI only” transmission or “2nd SCI and SL-SCH” transmission. |
| Huawei, HiSilicon | - Proposal 13: Support TDMing only between SL-PRS and PSSCH in the same slot (Alt. A.1) for the shared resource pool. - Proposal 14: With regards to PSSCH and SL-PRS multiplexing Alt. A.1, the TBS for PSSCH should be adjusted accordingly. - The TBS determination for PSSCH due to existence of PSFCH as legacy can be considered as baseline. Proposal 15: For multiplexing PSSCH and SL-PRS within a slot, at least support Alt. C.2, i.e., the PSSCH is used for 2nd SCI and SL-SCH. - FFS Alt. C.3, taking into account the MAC subheader of carrying the Source Layer-2 ID. - Proposal 16: Support TDMing only between SL-PRS and PSCCH in the same slot (Alt. B.1) for the shared resource pool. |
| CATT, GOHIGH | Proposal 3: For shared resource pool, SL-PRS should be TDMed with PSSCH in the slot-level or symbol-level. Proposal 4: For shared resource pool, SL-PRS should be configured with the same bandwidth with PSSCH, when SL-PRS and PSSCH occupy different symbols in the same slot. Proposal 20: Considering the backward compatibility of SL-PRS in shared resource pool: - SL-PRS can’t be mapped on the PSSCH symbols carrying 2nd-SCI REs and CSI REs. - SL-PRS should be TDMed with PSSCH in symbol-level or slot-level. - SL-PRS should be TDMed with PSCCH. - The PSSCH can be used for “2nd SCI only” or “2nd SCI and SL-SCH”. - SL-PRS can’t be mapped on the resources for PT-RS by puncturing the SL-PRS. |
| Intel | - For shared resource pool, - For PSSCH and SL-PRS multiplexing, support Alt. A1 (only TDM’ed). - For PSCCH and SL-PRS multiplexing, support Alt. B1 (only TDM’ed). - PSSCH is used to carry 2nd SCI and SL-SCH (Alt. C2). - A UE does not map SL-PRS and PT-RS in the same OFDM symbol. |
| ZTE | Proposal 7: In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot. - With regards to PSSCH and SL-PRS multiplexing, - Only TDM is supported - With regards to PSCCH and SL-PRS multiplexing - Only TDM is supported - The PSSCH is used for “2nd SCI only” or “2nd SCI and SL-SCH” |
| NEC | Proposal 1: With regard to PSSCH and SL-PRS multiplexing in shared resource pools, only TDM-ed multiplexing is supported. Proposal 2: With regard to PSCCH and SL-PRS multiplexing in shared resource pools, only TDM-ed multiplexing is supported. Proposal 3: With regard to the usage of PSSCH in shared resource pools, “2nd SCI only” or “2nd SCI and SL-SCH” is supported. |
| Sony | Proposal 1: In a shared resource pool, only TDM-ing is supported for both PSSCH and SL-PRS multiplexing (known as Alt A.1) and PSCCH and SL-PRS multiplexing (known as Alt B.1). |
| CMCC | Proposal 3: For a shared resource pool, with regards to PSSCH and SL-PRS multiplexing, only TDMing is supported, i.e., Alt. A.1 should be supported. Proposal 4: For a shared resource pool, with regards to PSCCH and SL-PRS multiplexing, only TDMing is supported, i.e., Alt. B.1 should be supported. Proposal 5: For SL-PRS transmission in a shared resource pool, the associated PSSCH is only used to convey 2nd SCI, i.e., Alt. C.1 should be supported. |
| Interdigital | Proposal 5: With regards to PSSCH and SL-PRS multiplexing in a shared resource pool, support both TDMing and FDMing, i.e. Alt. A.3. Proposal 6: With regards to PSCCH and SL-PRS multiplexing in a shared resource pool, only TDMing is supported, i.e. Alt. B.1. Proposal 7: PSSCH in a share resource pool is used for “2nd SCI only” or “2nd SCI and SL-SCH”, i.e. Alt. C.3. |
| Apple | Proposal 7: In a shared resource pool SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot • With regards to PSSCH and SL-PRS multiplexing, o Alt. A.1: Only TDMing is supported. • With regards to PSCCH and SL-PRS multiplexing, o Alt. B.1: Only TDMing is supported. • The PSSCH is used for o Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH” • The PT-RS may be treated in the same manner as the DMRS i.e. the SL-PRS is not transmitted. |
| Qualcomm | Proposal 2: Regarding PSSCH and SL-PRS multiplexing in the shared resource pool, TDMing and FDMing of PSSCH and SL-PRS is supported with PSSCH rate-matched around SL PRS REs. Proposal 3: Regarding PSCCH and SL-PRS multiplexing in the shared resource pool, only TDMing is supported. Proposal 4: In the shared resource pool, PSSCH in a slot with SL-PRS is used for 2nd SCI and SL-SCH. |
| OPPO | Proposal 7: In shared resource pool PSSCH associated with SL PRS in a slot is used to convey 2nd SCI only, i.e., to support Alt. C.1. Proposal 8: In shared resource pool, PSSCH and associated SL-PRS are only TDMed in a slot, i.e., Alt. A.1 is supported. Proposal 9: In shared resource pool, PSCCH and associated SL-PRS are only TDMed in a slot, i.e., Alt. B.1 is supported. Proposal 10: In shared resource pool, in OFDM symbols with PSCCH or PSSCH DMRS, UE should transmit PSSCH to occupy all REs within allocated PRBs. |
| Samsung | Proposal 3: In a shared resource pool, - With regards to PSSCH and SL-PRS multiplexing, only TDMing is supported. - With regards to PSCCH and SL-PRS multiplexing, only TDMing is supported. - The PSSCH is used for “2nd SCI only” or “2nd SCI and SL-SCH”. |
| LGE | Proposal 11: For a shared resource pool, only TDM of SL PRS and the associated PSCCH is allowed in a slot. Proposal 12: For a shared resource pool, only TDM of SL PRS and the associated PSSCH is allowed in a slot. Proposal 13: For a shared resource pool, SL PRS and the associated PSSCH have the same bandwidth. Proposal 14: For a shared resource pool, in a slot where SL PRS is transmitted, the associated PSSCH carries the 2nd-stage SCI only. Proposal 15: For a shared resource pool, in a slot where SL PRS is transmitted, PSFCH can be transmitted by (pre)configuration. |
| Sharp | Proposal 3: With regards to PSCCH and SL-PRS multiplexing in a shared resource pool, support Alt. B.1 (i.e. “only TDMing is supported”). Proposal 4: With regards to PSSCH and SL-PRS multiplexing in a shared resource pool, do not further consider Alt. A.3 (i.e. “both Alt. A.1 and A.2 are supported in the specification”). Proposal 5: With regards to support of 2nd stage SCI and/or SL-SCH in PSSCH multiplexed with SL-PRS in a shared resource pool, support Alt. C.2 (i.e. “2nd SCI and SL-SCH”). |
| lenovo | Proposal 11: In the case of a shared resource pool for SL Positioning, support: - Alt. A.1 – TDM between SL-PRS and PSSCH - Alt. B.2 - TDMing or PRB/sub-channel-based FDMing is supported between PSCCH and SL-PRS - Alt C.2- The PSSCH is used to carry 2nd SCI and SL-SCH |
| Ericsson | - Regarding the multiplexing of the PSSCH and SL-PRS, support both TDMing and FDMing. FFS: on TDMing detail FFS: on FDMing detail - Regarding the multiplexing of the PSCCH and SL-PRS, support both TDMing and PRB/sub-channel-based FDMing. |
| Mediatek | Proposal 2-1: For multiplexing between PSSCH and SL-PRS, both TDM and FDM could be supported. The comb-1 SL-PRS may belong to the TDM condition with PSSCH Proposal 2-2: To support FDM between data REs and SL-PRS REs, the rate matching around the SL-PRS REs is considered due to the reasons, - Better resource utilization - SL-PRS to be transmitted with larger BW as compared with the FDM by PRB or subchannel level Proposal 2-3: For multiplexing between PSCCH and SL-PRS, TDM is considered. The reason not to support FDM is, the SL-PRS BW may be limited since the accuracy is all about the measurable RS bandwidth Proposal 2-4: For the PSSCH in same slot as SL-PRS, consider PSSCH to be used for “2nd SCI and SL-SCH”. Without data transmission, SL-PRS could actually be transmitted in dedicated resource pool. |
| CEWiT | Proposal 10: For legacy SL resource configuration for SL positioning, Subchannel will carry the SL-PRS, PSSCH with SL positioning protocol message, PSCCH with first stage SCI and second stage SCI piggybacked on PSSCH. The PSSCH and PSCCH will be multiplexed with SL-PRS in TDM manner only. Proposal 11: For legacy SL resource configuration for SL positioning, the PSSCH is used to transmit 2nd SCI only” OR “2nd SCI and SL-SCH”. PSSCH should belong to the same Tx UE transmitting the SL-PRS. Proposal 12: For legacy SL resource configuration for SL positioning, reporting of the measurement can be triggered by Tx UE in the first stage SCI. Resource reservation mechanism of first stage SCI (SCI 1-A) can be used to decide the measurement reporting resources using resource reservation field from stage 1 SCI and providing PRS report trigger field as additional signal using reserved bit. |

Summary:

  - With regards to PSSCH and SL-PRS multiplexing

    - Alt. A.1: Only TDMing is supported

      - Futurewei, vivo, Spreadtrum, Huawei, HiSilicon, CATT, GOHIGH, Intel, ZTE, NEC, Sony, CMCC, Apple, OPPO, Samsung, LGE, Lenovo, CEWiT

    - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported

      - Toyota

    - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification

      - Nokia, NSB, Interdigital, Qualcomm, Ericsson, Mediatek

  - With regards to PSCCH and SL-PRS multiplexing

    - Alt. B.1: Only TDMing is supported

      - Nokia, NSB, Futurewei, vivo, Spreadtrum, Huawei, HiSilicon, CATT, GOHIGH,Intel, ZTE, NEC, Sony, CMCC, Interdigital, Apple, Qualcomm, OPPO, Samsung, LGE, Sharp, Lenovo, Mediatek, CEWiT

    - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported

      - Ericsson

  - The PSSCH is used for

    - Alt. C.1: 2nd SCI only

      - , OPPO, LGE

    - Alt. C.2: 2nd SCI and SL-SCH

      - Huawei, HiSilicon, Intel, Qualcomm, Sharp, Lenovo, Mediatek

    - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH”

      - Nokia, NSB, Futurewei, vivo, Spreadtrum, CATT, GOHIGH, ZTE, NEC, Interdigital, Apple, Samsung, CEWiT, CMCC

Based on the above, we can start from the following proposal:

[HIGH] Feature Lead Proposal 3.3.3-v0

In shared resource pools,

  - With regards to PSSCH and SL-PRS multiplexing, support Alt. A.1 from previous agreement (i.e., Only TDMing is supported)

  - With regards to PSCCH and SL-PRS multiplexing, support Alt. B.1. from previous agreement (i.e., Only TDMing is supported)

  - The PSSCH is used for (continue discussion ONLY between C.2 and C.3 and dowselect during this meeting):

    - Alt. C.2: 2nd SCI and SL-SCH

    - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH”

Companies views

| vivo |  | Okay |  |
| --- | --- | --- | --- |
| CMCC |  | We are supportive of this proposal. Regarding the 3rd bullet, we support Alt. C.3. We believe that the case where only SL-PRS is transmitted as a special PSSCH should be included. Location service and data service are different, they may have different QoS type (e.g., priority, latency), different reservation period and resource selection window, etc. We cannot guarantee that the SL-PRS will always be transmiited together with a data. |  |
| ZTE |  | Support. For the last bullet, we support Alt.C.3. If a new 2nd stage SCI (e.g. SCI 2-D) is used for SL-PRS indication, it is possible to indicate both SL-PRS and SL-data in this new 2nd stage SCI. Therefore, we support to use one bit to indicate whether only SL-PRS is indicated or both SL-PRS and SL-data are indicated in this SCI. |  |
| OPPO |  | We are fine with the first 2 sub-bullets. We disagree with the last sub-bullet. “2nd SCI and SL-SCH” needs new TBS determination scheme, new slot structure, and many impacts to MAC layer, but transmitting SL-SCH and SL-PRS in a slot is not so necessary, because if a UE tranmsitted PSSCH with a bandwidth similar as SL PRS, the PSSCH DMRS can be used as SL PRS. |  |
| Futurewei |  | OK with first two bullets. Third bullet we prefer Alt C.3 |  |
| Samsung |  | OK. We support Alt C.3. |  |
| CEWiT |  | Support |  |
| CATT |  | Support. For the last bullet, we support Alt.C.3. |  |
| Panasonic |  | OK, prefer alt c3 |  |
| Spreadtrum |  | Support. We support Alt C.3. |  |
| InterDigital |  | We support the proposal and we prefer Alt. C. 3. |  |
| Huawei, HiSilicon |  | Ok for the proposal. Regarding Alt. C.3: for 2nd SCI only, we wonder how to address the remaining ID bits carrier by MAC subheader. |  |
| Sharp |  | We are fine with the proposal and we support Alt. C.2 for simplicity. |  |
| SONY |  | Support |  |
| Nokia, NSB |  | With regard to first bullet, we prefer to have both TDMing and FDMing options (Alt A.3) |  |

### 3.3.5 Additional Proposals

| Ericsson | - In a shared resource pool, priority handling of SL positioning and other V2X services, and potential modifications to resource allocation procedures, need to be studied. |
| --- | --- |
| OPPO | Proposal 17: In shared resource pool, the priority of SL PRS provided by higher layer and the priority of SL data should be comparable. |
| LGE | We support the 1st and the 2nd sub-bullets. We need clarification on the 3rd bullet. As commented in Proposal 3.3.1-v0, we support either PSSCH+SL-PRS or PSSCH+SL-SCH, not both SL-PRS and SL-SCH in a same slot. We suggest the following clarification. With the following clarification, we support Alt C.3 - Alt. C.3: “2nd SCI only” when SL-PRS is transmissted, or “2nd SCI and SL-SCH” otherwise. |

## Scheme 1

The following agreement was made with regards to the topic of SL Positioning Resource Allocation Modes:

Agreement With regards to the SL-PRS resource allocation, study the following two schemes: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. Gnb, LMF, Gnb & LMF) allocates resources for SL-PRS - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS - Applicable regardless of the network coverage - FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting Ues (e.g. IUC-like solutions). - Note: Other Schemes are not precluded to be studied - FFS how to handle resource allocation of SL-Positioning measurement report Agreement Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. Gnb, LMF, Gnb & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS Agreement Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from Gnb Up to further discussion which one or more of these shall be applicable Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further). Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details

| Nokia | Proposal 17: Scheme 1 to support resource allocation methods that enable multiple UEs transmitting SL PRS to be allocated SL resources at once. For this, a UE, e.g., target UE may send a resource allocation request, on behalf of other UEs involved in positioning. Proposal 18: For Scheme 1 resource allocation, discuss the option where LMF provides the SL PRS characteristics (e.g., bandwidth) required for resource allocation, directly to gNB. Proposal 19: For Scheme 1 resource allocation, LMF may act as an intermediary entity to inform resource allocation decisions of gNBs with each other, so as to avoid any SL resource conflicts across gNBs that are using the same or overlapping resource pools for SL positioning. Proposal 20: For Scheme 1 resource allocation, UEs may indicate network (e.g., gNB or LMF) a conflict between SL resources used by UEs served by different gNBs, e.g., in cell-edge scenarios. |
| --- | --- |
| Futurewei | Proposal 8: For Scheme 1 (Mode 1) SL PRS resource allocation use DCI Format 3_0 signalling as a starting point. |
| vivo | - New DCI format including some parameters (e.g., SL PRS resource, starting symbol for SL-PRS, etc) indication should be defined for scheme 1 SL-PRS resource allocation. |
| Spreadtrum | Proposal 14: For dedicated resource pool, the new DCI format and configured grant should be defined for SL-PRS in scheme 1. Proposal 15: In shared resource pool, DCI format 3_0 can be reused for SL PRS transmission in scheme 1 SL-PRS resource allocation. |
| Huawei, HiSilicon | Proposal 21: For scheme 1, support to reuse PUCCH for requesting resource for SL-PRS transmissions. Proposal 22: For scheme 1, support to send SL-PRS characteristics to gNB by MAC CE, the specific SL-PRS characteristics includes at least: - Number of SL-PRS transmissions - Bandwidth of SL-PRS resource Proposal 23: Support to reuse DCI 3_0 for shared resource pool for SL-PRS transmission. Proposal 24: Support to indicate following fields in DCI for dedicated resource pool - Time resource assignment; - SL-PRS resource assignment; |
| Continental | Proposal 3.1: Introduce new DCI format for Scheme 1 resource allocation to Rel-18 and beyond UEs by a network gNB for SL positioning in a shared RP. Proposal 3.2: The new DCI format would contain one of the following alternatives in any combination with already existing fields in DCI format 3_0. - Option 1: Explicit indication of time and frequency resource assignment for SL positioning, and configuration indexes to indicate the type of resource allocation. - Option 2: Indication of resource allocation via a resource index which maps to a combination of resource allocation parameters. Proposal 5: To enable Scheme 1 SL positioning resource allocation for UEs belonging to different cells, consider the following options: - Option 1: Indirect resource allocation with one UE acting as initiator/mediator for all UEs - Option 2: Direct resource allocation where UEs first coordinate among each other for resources required and individually request their own serving cells for resources - Option 3: Indirect or direct resource allocation depending on factors such as the participation of out-of-coverage UEs in the SL positioning operation. |
| CATT, GOHIGH | Proposal 7: A new DCI format 3_X (e.g., DCI format 3-2) should be introduced in order to support the scheduling of SL-PRS with the resource allocation Scheme 1 at least for a dedicated resource pool. Proposal 8: The new DCI format 3_X should at least include the following fields for SL-PRS transmission: - Resource pool index, which indicates the index of dedicated resource pool and/or shared resource pool, - SL-PRS resource ID, which indicates the configuration of SL-PRS resource, - Time gap, which indicates the slot offset between the slot of the DCI and the first SL-PRS transmission scheduled by DCI, - Lowest index of the subchannel allocation to the initial transmission of SL-PRS. Proposal 9: Resource allocation Scheme 1 for dedicated resource pool should be reused for shared resource pool as much as possible. |
| Intel | - For Scheme 1 SL PRS resource allocation in a shared resource pool, existing DCI format 3_0 is extended to support SL PRS resource allocation. - For Scheme 1 SL PRS resource allocation in a dedicated resource pool, a new DCI format is defined for SL PRS resource allocation. |
| ZTE | Proposal 10: For scheme 1 SL-PRS resource allocation, support SL-PRS configuration transfer between LMF and gNB. Proposal 11: Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from gNB through Dynamic grant, or through configured grant type 1/type 2: - DCI format 3_0 with CRC scrambled by a new RNTI (e.g. SL-PRS-RNTI) is used to dynamically or semi-persistently schedule SL-PRS, at lease the following should be indicated: - Resource pool index (either dedicated resource pool or shared resource pool for SL-PRS) - SL-PRS resource configuration index (e.g. SL-PRS resource ID) - Time gap indicating the gap between DCI reception and the first SL-PRS transmission scheduled by DCI - Configuration index indicating which configured grant type 2 is activated by the DCI when UE is configured with multiple CG |
| Sony | Proposal 12: RAN1 to define new DCI format (3_x) for SL-PRS resource allocation purpose. |
| Interdigital | Proposal 8: Re-use DCI format 3_0 for SL PRS resource allocation in Scheme 1. Proposal 9: Study if the same DCI format can be used for Scheme 1 resource allocation for dedicated and shared resource pool. Proposal 10: Study a SL-PRS resource allocation Scheme 1 in which a UE requests and receives resources for SL-PRS transmissions by peer UEs in a SL positioning group. c |
| Apple | Proposal 8: For dynamic resource allocation and type 2 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the SL-PRS grant can be indicated by: - Option 1: Updating DCI 3_0 with a new field such as a “SL-PRS resource index” at least for the shared resource pool. - Option 2: Introducing a new DCI format at least for the dedicated resource pool. Proposal 9: For type 1 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the “sl-ConfiguredGrantConfig” may have a new field such as “sl-PRS_resources”, which indicates the SL-PRS resources in the grant. Proposal 10: For type 2 configured grant resource allocation for SL-PRS, support activating and releasing the configured grant using the DCI format designed for each resource pool type. |
| Qualcomm | Proposal 8: LMF provides the gNB with requisite SL-PRS properties. gNB responds to the LMF with configuration and allocation information. Proposal 9: DCI grants, configured sidelink grant Type 1, and configured sidelink grant Type 2 from Mode 1 are reused for Scheme 1 resource allocation. Proposal 10: A receiver UE obtains SL-PRS configuration from the transmitter UE via SLPP messaging. Proposal 11: A transmitter UE transmits SCI reserving resources for the accompanying SL-PRS similar to sidelink communications. |
| OPPO | Proposal 12: For Scheme 1 SL-PRS resource allocation in dedicated resource pool, support a new DCI format (3_X) with same size as DCI format 3_0, and a RNTI different from SL-RNTI or SL-CS-RNTI is used for the scrambling of DCI format 3_X. Proposal 13: For Scheme 1 SL-PRS resource allocation in shared resource pool, DCI format 3_X as dedicated resource pool is used. Proposal 14: No PUCCH resource is allocated together with the SL grant for SL PRS transmission in shared resource pool. |
| Samsung | Proposal 4: For Scheme 1 SL-PRS resource allocation, introduce at least one new DCI format 3_X for scheduling of SL positioning and study if the same DCI format is applied for shared pool and dedicated pool. - Study if the time domain resource indication are the same for shared and dedicated pool considering TDM-based multiplexing supported only in dedicated pool. - Study if the time domain resource indication and frequency domain resource indication are the same for shared and dedicated pool considering comb-based multiplexing supported in dedicated pool. - Study how to indicate control information for both SL PRS and PSSCH in same slot for shared pool. - Study DCI size alignment for DCI format 3_X. |
| LGE | Proposal 30: For both dedicated and shared resource pool, a new DCI format 3_X is introduced for scheme 1 resource allocation. The new DCI format 3_X field above are supported. Proposal 31: For configured grant type 1, RRC is used for indicating the periodicity, the offset and the activation/release of the configured grant resources, as same as Rel.16. Proposal 32: For configured grant type 2, RRC is used for indicating the periodicity and the offset of the configured grant resources, and a new DCI format 3_X is used for and the activation/release of the configured grant resources, as same as Rel.16. |
| Sharp | Proposal 9: For resource allocation scheme 1, for PSCCH/PSSCH transmissions scheduled by a DCI format 3_0 in a resource pool, - If SL-PRS is not configured in the resource pool, no SL-PRS transmission is allowed to be multiplexed in the PSCCH/PSSCH transmissions (i.e. legacy UE behavior). - If SL-PRS is configured in the resource pool, it is up to the scheduled UE whether to multiplex a SL-PRS transmission in each PSCCH/PSSCH transmission (i.e similarly to multiplexing of SL-CSI-RS in a PSCCH/PSSCH transmission). Proposal 10: DCI format 3_0 with CRC scrambled by a new RNTI is used to schedule SL-PRS transmission(s) in a dedicated resource pool for positioning. |
| Asustek | Proposal 2: For dedicated resource pool in scheme 1, introduce a new DCI format 3_X as SL PRS grant, which allocates SL PRS resource(s) with scheduling information of RE offset and SL PRS occasion indication. |
| lenovo | Proposal 1: In the case of Scheme 1, RAN1 to support a single DCI format 3 for both shared and dedicated resource pools, preferably a new DCI format 3_X for SL positioning purposes, which includes functionality to activate and release CG Type 2 resources. Proposal 2: In the case of Scheme 1 enhancements, RAN1 to support an in-coverage UE to forward SL-PRS resources to an out-of-coverage UE. |
| CEWiT | Proposal 14: In scheme 1 type of resource allocation, gNB may provide configured resource configuration information to the positioning server/LMF. Proposal 15: In dynamic grant type resource allocation in scheme 1, - For shared resource pool, DCI 3_0 can be reused. - For dedicated resource pool DCI 3_0 should be considered as baseline. - Based on resource pool configuration parameter DCI 3_0 fields can be reinterpreted further. |

[MEDIUM] Feature Lead Proposal 3.4-v0

In dynamic grant type resource allocation in scheme 1, downselect among the following alternatives:

- Alt. 1:

  - For dedicated resource pool, a new DCI format (3_X) should be defined.

  - For shared resource pool, DCI format 3_0 is being reused

- Alt. 2:

  - For both dedicated and shared resource pool, DCI format 3_0 is being reused

- Alt. 3:

  - For both dedicated and shared resource pool, the same new DCI format (3_X) should be defined.

- Companies are encouraged to comment whether a new RNTI, different from SL-RNTI or SL-CS-RNTI should be used for the scrambling of the DCI

Companies views

| CATT | Alt 3. In our vew, since some SL-PRS-specific new fileds have to be added into the DCI format to support the scheduling of SL PRS resource for both dedicated RP and shared RP, e.g., SL PRS resource ID, RE offset, such DCI format seems to be different with legacy DCI format 3_0, therefore, for both dedicated RP and shared RP, we prefer to define new DCI format 3_X. |
| --- | --- |
| ZTE | We prefer Alt.2 from UE’s monitoring complexity perspective. DCI format 3_0 with CRC scrambled by different RTNI can be used for either SL-data scheduling in SL communication resource pool, or SL-PRS scheduling in SL-PRS resource pool. For example, DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI is intended for SL communication and DCI format 3_0 with CRC scrambled by “SL-PRS-RNTI” or “SL-PRS-CS-RNTI” is intended for SL-PRS scheduling. |
| Samsung | Support Alt 3. |
| Qualcomm | Our first preference is Alt 2. The resource pool index field in DCI 3-0 can point to either type of resource pool. This minimizes impact on specifications and PDCCH monitoring complexity. For example, if a new DCI format is introduced, RAN1 would need to ensure that the blind decoding and DCI size budget is not exceeded, potentially impacting the size alignment procedure as well. |
| Sharp | We have a similar view as ZTE and QC, and we support Alt 2. On scheduling of SL-PRS (to be multiplexed in a scheduled PSCCH/PSSCH) in a shared resource pool, we don’t see any difference from the legacy scheduling of PSCCH/PSSCH (without multiplexing of SL-PRS). It could be up to UE whether to actually multiplex a SL-PRS in the scheduled PSCCH/PSSCH resource. If a new DCI format is defined, further work on DCI size alignment would be also introduced in RAN1. |
| Continental | We support new DCI formats in both dedicated and shared RPs, but it is not clear why they have to be the same as indicated in the proposal wording. |
| OPPO | In dedicated resource pool, DCI needs to indicate SL PRS resource ID, this field does not exist in DCI format 3-0, therefore a new DCI format 3-X is needed. For shared resource pool, DCI format 3-0 can be resued to schedule resources for both PSSCH and SL PRS. Size of DCI form 3-X should be aligned to DCI format 3-0 to avoid additional blind detection, hence a new RNTI should be used to scramble DCI 3-X. |
| Lenovo | Either Alt. 2 or Alt. 3 with a preference that a single DCI is used |
| LGE | We support Alt 3 as DCI format 3_0 doesn’t include all the necessary information for SL positioning. We think a new RNTI may be needed to differentiate the new DCI format. |
| Nokia, NSB | Alt 2 to minimize specification effort. For dedicated resource pool, the resource pool index field in DCI format 3_0 identifies the dedicated pool. For shared resource pool, one easy option is that the gNB just provides the resource allocation as in legacy sidelink, up to UE how to use the resources for PSSCH and/or SL-PRS. To discuss whether DCI should have control over SL-PRS resource aspects such as comb offset. |

[MEDIUM] Feature Lead Proposal 3.4-v0

In dynamic grant type resource allocation in scheme 1, downselect among the following alternatives:

- Alt. 1:

  - For dedicated resource pool, a new DCI format (3_X) should be defined.

  - For shared resource pool, DCI format 3_0 is being reused

    - Supported: OPPO

- Alt. 2:

  - For both dedicated and shared resource pool, DCI format 3_0 is being reused

    - Supported: ZTE, Qualcomm, Sharp, Lenovo, Nokia, NSB

- Alt. 3:

  - For both dedicated and shared resource pool, the same new DCI format (3_X) should be defined.

    - Supported: CATT, Samsung, Continental (no need to be the same), Lenovo, LGE

- Companies are encouraged to comment whether a new RNTI, different from SL-RNTI or SL-CS-RNTI should be used for the scrambling of the DCI

  - Support new RNTIs: ZTE, OPPO, LGE

## Scheme 2

#### 3.5.1 Resource allocation/selection – General

The following agreement was reached with regards to this topic:

Agreement For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed. Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool. - Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation.

| Futurewei | Proposal 1: For the coexistence between random and sensing-based resource selection in the same resource pool, the random-based selection resources are separate from or a subset of the sensing-based resources. |
| --- | --- |
| CATT, GOHIGH | Proposal 10: No enhancements are needed for coexistence of random selection and sensing-based resource selection in a dedicated resource pool. |
| CMCC | Proposal 6: Centralized scheduling mechanism, e.g., mode 2(d) like method discussed in Rel-16 NR sidelink, can be considered as a candidate resource allocation mechanism for SL-PRS in Rel-18. |
| Apple | Proposal 11: For Scheme 2 SL-PRS resource allocation, there is no need for any enhancements for coexistence of random selection and sensing-based resource selection in a resource pool similar to the Rel-16/Rel-17 design. |

#### 3.5.2 Sensing-based Resource Allocation

Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective Related Agreement from a 9.5.1.1: Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For the scheme 2 sensing-based resource allocation: - Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Note: This means that Rel-17 partial sensing is not considered a starting point for the design Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Modification 6: For the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - Option 3: Provided by higher layers with values TBD - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%): - Options TBD - Modification 8: For the pre-emption of the reserved resources: - Options TBD - Note 1: Other potential modifications and/or other options within each modification are not precluded - Note 2: Multiple options for each potential modification may be supported

| Nokia | Proposal 21: For Scheme 2 resource allocation in shared pools, UE transmitting SL PRS may provide feedback to other transmitting UEs to avoid their resource preemption and reuse the same time/frequency resources via RE-level multiplexing. Proposal 22: For Scheme 2 resource allocation in at least dedicated resource pools, UE performing sensing for SL PRS may use comb size and comb offset as parameters during sensing to determine if reuse of the same time resources via RE-level multiplexing is possible. This sensing operation may be (pre)-configurable. Proposal 26: In the case of a SL PRS allocation for multiple TX / RX UEs and/or under strict timing constraints, discuss how the priority of resource allocation can be raised as a function of the number of involved UEs and/or timing constraints. |
| --- | --- |

##### Modification 1 (RS for S-RSRP)

| vivo | - For a dedicated resource pool, the SL-PRS can be used for RSRP measurement for sensing in scheme 2. |
| --- | --- |
| Spreadtrum | Proposal 16: For the RS used to derive L1 SL-RSRP for resource exclusion, both SL-PRS and PSCCH DMRS are supported. |
| Huawei, HiSilicon | Proposal 25: For dedicated resource pool, - Modification 1: both Option 1 (i.e. SL-PRS) and option 2 (i.e. PSCCH DMRS) can be used according to (pre)-configuration. |
| CATT, GOHIGH | - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: SL-PRS/PSCCH DMRS based on configuration |
| Intel | - PSCCH DMRS RSRP is used for resource exclusion (Option 2). |
| Xiaomi | Proposal 9: S-RSRP measurement used to exclude SL-PRS candidate resource in scheme 2 resource selection in a dedicated resource pool is obtained - Option 1: from measuring DMRS of PSCCH conveying the SCI indicating the reserved SL PRS resource; - Option 2: from measuring SL PRS associated with the SCI indicating the reserved SL PRS resource. |
| ZTE | - Modification 1: both SL-PRS and PSCCH DMRS can be used to derive L1 SL-RSRP for resource exclusion |
| CMCC | - Modification 1: Option 1 or Option 2; |
| Apple | - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 2: PSCCH DMRS |
| Qualcomm | Proposal 12: PSCCH DMRS is used for RSRP measurements for sensing when (re)selecting SL-PRS transmission resources in a dedicated resource pool. |
| OPPO | Proposal 15: Depending on the structure of PSCCH in dedicated resource pool, PSCCH DMRS and/or SL PRS is used for SL RSRP measurement, and PSSCH is not included in dedicated resource pool. |
| Samsung | - SL PRS is used to derive L1 SL-RSRP for resource exclusion |
| LGE | Proposal 21: For scheme 2 in a dedicated resource pool, for SL PRS resource (re)selection, either PSCCH DMRS or SL PRS is (pre)configured to be used for L1 SL RSRP measurement. |
| lenovo | - PSCCH DMRS (Option 1) and SL-PRS (Option 2) for resource exclusion |
| Ericsson | - PSCCH DMRS should be used as RS to derive L1 SL-RSRP for resource exclusion. |
| CEWiT | Proposal 16: In scheme 2 sensing-based resource selection for dedicated resource pool, at least one of SL-PRS, PSCCH DMRS and PSSCH DMRS should be used based on availability for sensing RSRP calculation for resource exclusion. - Relative power variations should be considered for calculation of RSRP. |

Summary:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

  - Option 1: SL-PRS

    - Vivo, Spreadtrum, Huawei, HiSilicon, CATT, GOHIGH, Xiaomi, ZTE, CMCC, Samsung, LGE, lenovo, CEWiT

  - Option 2: PSCCH DMRS

    - Spreadtrum, Huawei, HiSilicon, CATT, GOHIGH, Intel, Xiaomi, ZTE, Apple, Qualcomm, OPPO, CMCC, LGE, lenovo, Ericsson, CEWiT

  - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

    - CEWiT

[MEDIUM] Feature Lead Proposal 3.5.2.1-v0

For Scheme 2, in a dedicated resource pool, for the RS used to derive L1 SL-RSRP for resource exclusion, support both SL-PRS and PSCCH DMRS, according to resource pool (pre)-configuration.

Companies views

| vivo |  | Support |  |
| --- | --- | --- | --- |
| CMCC |  | We think that whether SL-PRS can be used for resource exclusion depends on the discussion under AI 9.5.1.1, whether the sequence ID of SL-PRS is high-layer configured per UE or using the CRC bit of its associated PSCCH, we suggest to agree on PSCCH DMRS first. |  |
| ZTE |  | Support |  |
| Futurewei |  | Support |  |
| CEWiT |  | Support |  |
| CATT |  | Support |  |
| Spreadtrum |  | Support |  |
| InterDigital |  | We support the proposal. |  |
| Huawei, HiSilicon |  | Support. This proposal is impacted by the conclusion of the SL-PRS sequence ID. |  |
| Samsung |  | First preference is SL PRS only but we can accept this if this is the majority view. |  |
| Qualcomm |  | Not support. RAN1 introduced sensing using PSCCH DMRS to enable the introduction of new transmission types with minimum impact on sensing procedure. Evaluations in R16 showed that PSCCH DMRS provide sufficient accuracy for sensing. As others noted, using SL-PRS is not a general solution. |  |
| Sharp |  | We support the proposal. |  |
| Continental |  | Support. |  |
| OPPO |  | OK. |  |

##### [MEDIUM] Feature Lead Proposal 3.5.2.1-v0

For Scheme 2, in a dedicated resource pool, for the RS used to derive L1 SL-RSRP for resource exclusion, support both SL-PRS and PSCCH DMRS, according to resource pool (pre)-configuration.

- Support:

  - vivo, ZTE, Futurewei, CEWiT, CATT, Spreadtrum, Interdigital, Samsung (1st preference SL-PRS only), Sharp, Continental, OPPO

- Do not support:

  - Qualcomm (Only PSCCH DMRS)

- Wait for progress in 9.5.1.1:

  - CMCC, Huawei, HiSilicon

##### 3.5.2.2 Modification 2 (resource selection window)

| vivo |  | - In the dedicated resource pool, the resource selection window is provided by higher layers. |
| --- | --- | --- |
| spreadtrum |  | Proposal 17: For the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget. |
| Huawei, HiSilicon |  | Reusing legacy NR sidelink communication for others. |
| CATT, GOHIGH | - Modification 2: For the resource selection window: For the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget |  |
| Intel | - A SL-PRS delay budget is introduced to define the RSW (Option 1) |  |
| ZTE | - Modification 2: For resource selection window, it depends on whether there is Packet Delay Budget (PDB) defined for SL-PRS, if not, the selection window is provided by higher layers. |  |
| CMCC | - Modification 2: Option 1; |  |
| Apple | - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget |  |
| Qualcomm | Proposal 13: Higher layers directly indicate the SL-PRS resource selection window to lower layers in both a dedicated resource pool and a shared resource pool. |  |
| Samsung | - For the resource selection window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget. |  |
| LGE | Proposal 19: For scheme 2 in a dedicated resource pool, for scheme 2 SL PRS resource (re)selection, the resource selection window is determined within a delay budget for SL PRS transmission, similar to Rel.6 SL. - FFS how the delay budget is determined |  |
| Lenovo | - Option 1 – a new delay budget for positioning is provided |  |
| Ericsson | - Regarding the selection window and parameters T1 and T2, we support to have the start and the end of the selection window provided by higher layers. |  |
| CEWiT | Proposal 17: In scheme 2 for the derivation of the selection window, using the legacy approach as a starting point (option 1). - Location delay budge (LDB) should be specified by higher layer. |  |

[MEDIUM] Feature Lead Proposal 3.5.2.2-v0

For Scheme 2, in a dedicated resource pool, with regards to the resource selection window, use the legacy approach as a starting point, by substituting the Packet Delay Budget (PDB) with a new delay budget.

- FFS: Details

Companies views

| vivo |  | Based on our understanding, PDB is the maximum boundary for resource selection window. In NR sidelink, the resource selection window is identified as [n+T1, n+T2]. Values of T1 and T2 are up to UE implementation. However, the values of T2 should be subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). And T2min is provided by higher layer. In our opionin, the above mechanism can be reused. |  |
| --- | --- | --- | --- |
| CMCC |  | Location service is not a data packet, it does not have a packet delay budge, we’are wondering what is the delay budget of a location service? |  |
| ZTE |  | We are not sure whether we have a packet delay budget defined for SL positioning. |  |
| Futurewei |  | Same as ZTE, what would be this new delay budget? |  |
| CEWiT |  | Support. The new delay budget (Location delay budget) can be specified by higher layer based on the QoS specified by the application/entity requesting the positioning service. |  |
| CATT |  | OK with the proposal. |  |
| InterDigital |  | We support the proposal. |  |
| Huawei, HiSilicon |  | Support. |  |
| Samsung |  | support |  |
| Qualcomm |  | Not support As noted by others, defining a new delay budget involves work in other WGs. The alternative of higher layers directly indicating the selection window avoids that issue and enables coordination between higher layers among UEs. |  |
| Sharp |  | We support the proposal. |  |
| Continental |  | We are generally OK with the proposal, but prefer to use a different term like positioning delay budget than Packet Delay Budget since there is no packet associated with positioning. |  |
| Nokia, NSB |  | Support. |  |

- Modification 2: For the resource selection window:

  - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

  - Option 2: the selection window is provided by higher layers

##### [MEDIUM] Feature Lead Proposal 3.5.2.2-v0

For Scheme 2, in a dedicated resource pool, with regards to the resource selection window, use the legacy approach as a starting point, by substituting the Packet Delay Budget (PDB) with a new delay budget.

- FFS: Details

- Support the proposal:

  - CEWIT, CATT, Interdigital, Huawei, HiSilicon, Samsung, Sharp, Continental (rewording needed), Nokia, NSB

- Doesn’t support with the proposal

  - Vivo, CMCC, ZTE (1st preference), Futurewei, Qualcomm,

##### 3.5.2.3 Modification 3 (number of priorities)

| vivo |  | - In the dedicated resource pool, multiple L1 SL-PRS priorities are allowed in a resource pool to let UE with higher SL-PRS priority can reserve resource with higher probability. |
| --- | --- | --- |
| spreadtrum |  | Proposal 18: Multiple L1 SL-PRS priority are allowed in a resource pool should be support. |
| Huawei, HiSilicon | Proposal 25: For dedicated resource pool, - Modification 3: Option 2 (i.e. Multiple L1 SL-PRS priorities are allowed in a resource pool) is supported. |  |
| CATT, GOHIGH | - Modification 3: For the SL-PRS priority: Multiple L1 SL-PRS priority are allowed in a resource pool |  |
| Intel | - A single L1 SL-PRS priority is allowed (Option 1). |  |
| ZTE | - Modification 3: For the SL-PRS priority, ultiple L1 SL-PRS priority are allowed in a resource pool. |  |
| CMCC | - Modification 3: Option 1; |  |
| Interdigital | Proposal 12: Multiple L1 SL-PRS priorities are allowed in a resource pool.c |  |
| Apple | • Modification 3: For the SL-PRS priority: o Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool |  |
| Qualcomm | Proposal 14: Multiple L1 SL-PRS priority are allowed in a dedicated resource pool. |  |
| OPPO | Proposal 16: Whether single or multiple priority are allowed in dedicated resource pool is up to higher layer. |  |
| Samsung | - For the SL-PRS priority, multiple L1 SL-PRS priority are allowed in a resource pool. |  |
| LGE | Proposal 24: SL PRS priority is determined based on the priority of the associated SL positioning service. |  |
| lenovo | - Option 2 – multiple L1 SL-PRS priorities in a dedicated resource pool |  |
| Ericsson | - Regarding the SL-PRS priority, SL-PRS transmissions can have different priority in a resource pool. |  |
| CEWiT | Proposal 18: In scheme 2, with regards to the SL-PRS priority (option 1) a single L1 SL-PRS priority should be allowed in for each resource pool. |  |

Summary:

- Modification 3: For the SL-PRS priority:

  - Option 1: A single L1 SL-PRS priority is allowed in a resource pool

    - Intel, CMCC, CEWiT,

  - Option 2: Multiple L1 SL-PRS priority are allowed  in a resource pool

    - Vivo, Spreadtrum, Huawei, HiSilicon, CATT, GOHIGH, ZTE, Interdigital, Apple, Qualcomm, OPPO, Samsung, LGE, Lenovo, Ericsson,

[MEDIUM] Feature Lead Proposal 3.5.2.3-v0

For Scheme 2, in a dedicated resource pool, Multiple L1 SL-PRS priority are allowed  in a resource pool

Companies views

| vivo |  | Support |  |
| --- | --- | --- | --- |
| CMCC |  | Support. |  |
| ZTE |  | Support. |  |
| Futurewei |  | OK |  |
| CATT |  | OK |  |
| InterDigital |  | Support |  |
| Huawei, HiSilicon |  | Support. |  |
| Samsung |  | support |  |
| Qualcomm |  | Support |  |
| Sharp |  | We support the proposal. |  |
| OPPO |  | OK |  |
| Nokia, NSB |  | Support. |  |

[MEDIUM] Feature Lead Proposal 3.5.2.3-v0

For Scheme 2, in a dedicated resource pool, Multiple L1 SL-PRS priority are allowed  in a resource pool

- Support: vivo, CMCC, ZTE, Futurewei, CATT, Interdigital, Huawei, HiSilicon, Samsung, Qualcomm, Sharp, OPPO, Nokia, NSB

##### 3.5.2.4 Modification 4 (candidate resource definition)

| vivo | - In the dedicated resource pool, the SL PRS resource level resource exclusion should be supported in Scheme 2 resource allocation. |
| --- | --- |
| Huawei, HiSilicon | Proposal 25: For dedicated resource pool, - Modification 4: The candidate resource should base on the SL-PRS resource, which will be determined by SL PRS resource ID and slot index etc. |
| CATT, GOHIGH | - Modification 4: For the definition of a candidate resource within the resource selection window: Based on the SL-PRS resource |
| Intel | - For a dedicated resource pool, resource exclusion is based on SL-PRS resources and not SL communication sub-channels . - Candidate SL PRS resources are all potentially TDM’ed or comb-multiplexed. The minimum RE distance for comb multiplexing is (pre)-configured. |
| Xiaomi | Proposal 8: A candidate SL PRS resource is determined by the time/frequency position/size and comb offset in a dedicated resource pool. |
| ZTE | - Modification 4: The definition of a candidate resource within the resource selection window refers to the definition of a SL-PRS resource (e.g. comb offset, symbol offset…). |
| Interdigital | Proposal 13: Study both FDMed and TDMed candidate resources for SL-PRS resource (re)selection in a dedicated resource pool. |
| OPPO | Proposal 18: A candidate resource within the resource selection window is defined as SL PRS resource in a dedicated resource pool. Proposal 19: When TDM-based multiplexing of SL PRS from different UEs in a slot is supported, candidate resources within the resource selection window for a UE correspond to same set of OFDM symbols in a slot. |
| Samsung | - For the definition of a candidate resource within the resource selection window, comb pattern and TDM based multiplexing are considered. |
| Ericsson | - A resource candidate can be defined by a slot in time and the number of PRBs, and the comb-offset. |
| CEWiT | Proposal 19: In scheme 2, the definition of a candidate resource within the resource selection window, the resource elements according to the {OFDM symbols, comb-offsets} or slot occupied by a SL-PRS resource is used. - Slot is used if PSSCH is expected to be transmitted along with the SL-PRS |

Summary

- Modification 4: A candidate resource within the resource selection window is defined as SL PRS resource.

  - Vivo, Huawei, HiSilicon, CATT, GOHIGH, Intel, ZTE, Interdigital, OPPO,

[MEDIUM] Feature Lead Proposal 3.5.2.4-v0

- For Scheme 2, in a dedicated resource pool, a candidate resource within the resource selection window is defined as SL PRS resource.

Companies views

| vivo |  | Support. |  |
| --- | --- | --- | --- |
| CMCC |  | Support. |  |
| ZTE |  | OK |  |
| Futurewei |  | OK |  |
| CATT |  | OK |  |
| InterDigital |  | We support the proposal. |  |
| Huawei, HiSilicon |  | Support. |  |
| Samsung |  | support |  |
| Qualcomm |  | Support |  |
| Sharp |  | We support the proposal. |  |
| Continental |  | Support. |  |
| OPPO |  | OK |  |
| Nokia, NSB |  | Support. |  |

[MEDIUM] Feature Lead Proposal 3.5.2.4-v0

- For Scheme 2, in a dedicated resource pool, a candidate resource within the resource selection window is defined as SL PRS resource.

  - Support: vivo, CMCC, ZTE, Futurewei, CATT, Interdigital, Huawei, HiSilicon, Samsung, Qualcomm, Sharp, Continental, OPPO, Nokia, NSB

##### 3.5.2.5 Modification 5 (reservation internal)

| vivo | - In the dedicated resource pool - For periodic resource reservation, the candidate values of reservation period can be configured by higher layer parameter, and further detailed value is indicated in SCI. - For aperiodic resource reservation, the reservation interval can be indicated by TRIV in the SCI. |
| --- | --- |
| spreadtrum | Proposal 19: For the reservation interval of SL-PRS, it provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. |
| Huawei, HiSilicon | Reusing legacy NR sidelink communication for others. |
| CATT, GOHIGH | - Modification 5: For the reservation interval of SL-PRS: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. |
| Intel | - The reservation interval is (pre)-configured pre resource pool and provided to the resource re-selection procedure by higher layers. |
| ZTE | - Modification 5: The determination of reservation interval for SL-PRS can refer to the periodicities defined for DL-PRS. |
| Apple | - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. |
| Samsung | - For the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. |
| LGE | Proposal 25: For periodic or semi-persistent SL PRS transmission, the resource reservation interval in SCI carries the periodicity of SL PRS transmission at least for scheme 2. Otherwise, the resource reservation interval field is set to zero. |
| CEWiT | Proposal 20: For scheme 2 sensing window length (T0), window should be is equal to or larger than the largest reservation interval. - Window should be configured by higher layer. |

[MEDIUM] Feature Lead Proposal 3.5.2.5-v0

- For Scheme 2, in a dedicated resource pool, with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

  - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point.

Companies views

| vivo | If the reservation interval is used for periodic resource reservation for SL-PRS, we think the candidate values of reservation period can be configured by higher layer parameters and further indicated in SCI. If the reservation interval is used for aperiodic resource reservation for SL-PRS, we think the reservation interval can be indicated by TRIV in the SCI directly. |
| --- | --- |
| CMCC | Support. |
| ZTE | We think the “reservation interval” in this proposal represents periodicity of periodic SL-PRS. |
| Futurewei | Support with the understanding that represents periodicity. |
| CATT | OK with the proposal |
| InterDigital | We support the proposal. |
| Samsung | support |
| Qualcomm | Support |
| OPPO | OK |
| Nokia, NSB | Support. |

[MEDIUM] Feature Lead Proposal 3.5.2.5-v0

- For Scheme 2, in a dedicated resource pool, with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

  - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point.

##### 3.5.2.6 Modification 6 (sensing window)

| vivo | - In the dedicated resource pool, - The sensing window length should be provided by higher layers (option 3), and the values can be associated with the largest reservation period of SL-PRS. |
| --- | --- |
| spreadtrum | Proposal 20: For the sensing window length (T_0), the legacy (pre-)configuration with values (100 msec, 1100 msec) can be reused. |
| Huawei, HiSilicon | Reusing legacy NR sidelink communication for others. |
| CATT, GOHIGH | - Modification 6: For the sensing window length (T0): Equal to or larger than the largest reservation interval |
| ZTE | - Modification 6: Sensing window size should be equal to or larger than the largest reservation interval. |
| CMCC | - Modification 6: Option 2; The candidate reservation interval values should be determined first. |
| Apple | • Modification 6: For the sensing window length ( ): o Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) |
| Qualcomm | Proposal 15: For the sensing window length (T0) use the legacy (pre-)configuration with values (100 msec, 1100 msec). |
| Samsung | - For the sensing window length (T0), Use the legacy (pre-)configuration with values (100 msec, 1100 msec). |
| LGE | Proposal 18: For scheme 2 in a dedicated resource pool, for scheme 2 SL PRS resource (re)selection, the sensing window is determined with the (pre)configured values (100ms, 1100ms), similar to Rel.6 SL. |
| Ericsson | - Regarding the sensing window and the parameter T_0, we support to have the legacy (pre-)configuration with values (100ms, 1100ms). |

Summary

- Modification 6: For the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

    - Spreadtrum, Huawei, HiSilicon, Apple, Qualcomm, LGE, Ericsson

  - Option 2: Equal to or larger than the largest reservation interval

    - Vivo, CATT, GOHIGH, ZTE

  - Option 3: Provided by higher layers with values TBD

##### [LOW] Feature Lead Proposal 3.5.2.6-v0

- For Scheme 2, in a dedicated resource pool, with regards to the sensing window length, downselect between option 1 and option 2:

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

##### Companies views

| vivo | Sensing window length should be pre-configured by higher layer, and the values can be associated with the largest reservation period of SL-PRS. If the reserved value of SL-PRS is larger than 1000ms, e.g., 2000ms, and the sensing window is configured as 1100ms, UE 2 in slot n-1900 reserve a resource in slot n+100, and UE 1 performs resource selection at slot n, the UE 1 cannot according to sensing result of the slot n-1900 which is not in the sensing window [n-1100, n-1], and cannot exclude the resource n+100 in the selection window. Then, a collsion between UE 1 and UE 2 may happen. As illustrated in the following. |
| --- | --- |
| ZTE | We have the same understanding as vivo. Sensing window length is provided by RRC but it is s associated with the reservation interval of SL-PRS. In order to make sure at least one occasion appears in a sensing window, sensing window size for SL-PRS should be equal to or larger than the reservation interval. As shown below, if sensing window size is smaller than SL-PRS reservation interval Prsvp, a UE intended to transmit SL-PRS in selection window cannot detect the SL-PRS outside the sensing window, thus it is possible that UE can choose a candidate SL-PRS resource which has conflict with another UE. |
| InterDigital | We support the proposal and we prefer Option 1. |

##### 3.5.2.7 Modification 7 (initial RSRP threshold, target resource ratio)

| vivo | - The legacy value in the NR sidelink can be considered for the initial RSRP threshold & stepsize, target resource ratio for SL-PRS as the starting point. |
| --- | --- |
| Huawei, HiSilicon | - Reusing legacy NR sidelink communication for others. |
| Xiaomi | Proposal 10: For candidate SL-PRS resource which occupies overlapping PRB with reserved resource but with different comb offset, resource exclusion is performed with an additional S-RSRP threshold. |
| ZTE | - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%), RSRP threshold and target resource ratio should be provided in each dedicated SL-PRS resource pool |
| Qualcomm | Proposal 16: In a dedicated resource pool, the initial S-RSRP threshold & stepsize, target resource ratio X(%), pre-emption, and re-evaluation are reused from sidelink communications. |
| LGE | Proposal 22: For scheme 2 in a dedicated resource pool, for SL PRS resource (re)selection, the initial RSRP threshold, the increment step size, and the target resource ratio are (pre)configured per resource pool. |
| Ericsson | - Changing the RSRP thresholds, step size, and target resource ratio X% requires careful study and analysis through simulations. Therefore, we recommend avoiding any modifications to the legacy values without proper simulations. A S-RSRP threshold & step size, target resource ratio X(%) for SL PRS resource (re)selection are selected as those for SL data resource (re)selection. |

##### [LOW] Feature Lead Proposal 3.5.2.7-v0

- For Scheme 2, in a dedicated resource pool, for the initial S-RSRP threshold & stepsize, target resource ratio X(%), reuse the legacy values from NR sidelink.

##### Companies views

| vivo | Support. |
| --- | --- |
| ZTE | Support. |
| InterDigital | We support the proposal. |
| Nokia, NSB | Support. |

##### 3.5.2.8 Modification 8 (premption)

| vivo | - Whether to support re-evaluation and pre-emption for SL-PRS should be discussed first. |
| --- | --- |
| Huawei, HiSilicon | - Reusing legacy NR sidelink communication for others. |
| ZTE | - Modification 8: For the pre-emption of the reserved resources, whether pre-emption is disabled or enabled and a priority level can be indicated in a SL-PRS resource pool |
| Intel | - Pre-emption is not defined for SL-PRS transmissions in a dedicated resource pool. - Pre-emption of reserve resources: In our understanding as all SL-PRS transmissions have the same priority pre-emption should not be defined for SL-PRS transmissions in a dedicated resource pool. |
| Samsung | - FFS: pre-emption and re-evaluation |
| LGE | Proposal 26: Rel.16 pre-emption procedure is reused based on SL PRS priority and RSRP value of other UE’s reserved resource. |

[MEDIUM] Feature Lead Proposal 3.5.2.8-v0

- For Scheme 2, in a dedicated resource pool, with regards to the resource (re)-selection procedure

  - Alt. 1: do not support re-evaluation, nor pre-emption for SL-PRS.

  - Alt. 2: support re-evaluation for SL-PRS using the Rel-16 re-evaluation as a starting point.

  - Alt. 3: support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

Companies views

| vivo |  | Support alt. 2. |  |
| --- | --- | --- | --- |
| CMCC |  | We support Alt. 3. |  |
| ZTE |  | Alt. 1. Considering we only have one meeting left except this one. We prefer to focus on more urgent issue for SL-PRS and de-prioritize the discussion for re-evaluation and pre-emption. |  |
| Futurewei |  | Alt 3 if multiple priorities are supported. |  |
| CATT |  | OK with the proposal |  |
| InterDigital |  | We support the proposal and prefer Alt. 3 to enhance the SL-PRS transmission reliablity in Scheme 2. |  |
| Huawei, HiSilicon |  | Ok with Alt3 |  |
| Samsung |  | Support Alt 1 |  |
| Qualcomm |  | Alt 3 Reevaluation was not listed as a potential modification to the resource (re)selection procedure in the previous agreement. In our understanding, it is already included in the SL-PRS procedure, unless there is a future agreement to the contrary. In Rel-16, re-evaluation was shown to significantly improve both system performance and the UE’s performance. |  |
| Sharp |  | Alt. 3. |  |
| Continental |  | We support Alt 3 |  |
| OPPO |  | Alt 3. Both re-evaluation and pre-emption should be supported to mitigate resource collision in a resource pool allowing aperiodic transmission. |  |
| Nokia, NSB |  | Support. Prefer Alt. 3 |  |

[MEDIUM] Feature Lead Proposal 3.5.2.8-v1

- For Scheme 2, in a dedicated resource pool, with regards to the resource (re)-selection procedure

  - Alt. 1: do not support re-evaluation, nor pre-emption for SL-PRS.

    - ZTE, Samsung

  - Alt. 2: support re-evaluation for SL-PRS using the Rel-16 re-evaluation as a starting point.

    - vivo

  - Alt. 3: support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

    - CMCC, Futurewei, Interdigital, Huawei, HiSilicon, Qualcomm, Sharp, Continental, OPPO, Nokia, NSB

##### 3.5.2.9 Additional Modifications

| ZTE | - Resource exclusion mechanism: whether the UE performs resource exclusion or not is not only related to the SL-PRS RSRP measurement and Tx priority (both UEs), but also related to whether the two UEs have positioning signaling interaction - Resource(s) exclusion is based on resources overlapping with reserved resource(s) of other UE whose RSRP measurement is larger than a RSRP threshold. - FDMed PRS multiplexing between UEs having positioning serving interaction can NOT be supported. |
| --- | --- |
| Interdigital | Proposal 11: SL-PRS repetition number is indicated by higher layer for SL-PRS resource selection. |
| Fraunhofer | Proposal 7: Support the exchange of assistance data (AD) for the selection of preferred resources or the exclusion of resources. |
| LGE | Proposal 20: For scheme 2 in a dedicated resource pool, for SL PRS resource (re)selection, the (pre)configured min. number of the candidate SL PRS resources are selected by UE implementation with the resource selection window, similar to Rel.6 SL. Proposal 23: For scheme 2 in a dedicated resource pool, for SL PRS resource (re)selection, if the multiplexing of different SL PRS comb patterns is allowed, the candidate resources that partially overlap with other UE’s reserved resources are excluded if - RSRP related to other UE’s reserved resource is above a (pre)configured threshold, and - the RE overlap portion of either candidate resource or SL PRS resource reserved by other UE is above a (pre)configured threshold. |
| Asustek | Proposal 3: In scheme 2, sensing-based SL PRS resource allocation is introduced mostly based on sidelink mode 2 design, and further considers interference issue due to resource differentiation in RE level. |
| Ericsson | For shared RP: - Include SL PRS in the list sl-RS-ForSensing for candidate reference signals for RSRP measurement in the sensing procedure - FFS: include a DMRS+PRS RSRP measurement in the list of possible RS for sensing. |

#### 3.5.3 Congestion control

Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

| Nokia | Proposal 31: UEs involved in SL positioning may exchange information on the channel congestion they observe, so as to adapt their SL PRS configuration, e.g., SL PRS bandwidth depending on their relative congestion status. |
| --- | --- |
| vivo | - The number of SL PRS resources can be used in the calculation CR/CBR for SL-PRS in the dedicated resource pool. - The number of subchannels can be reused in the calculation CR/CBR for SL-PRS in the shared resource pool. - One CR/CBR measurement parameter for both PSSCH and SL-PRS for positioning in the shared resource can be defined. (i.e., Separate CR/CBR measurements for SL-PRS and PSSCH in the shared resource pool are not needed.) - The resource of SL-PRS transmitted by anchor UE can be calculated in the CR/CBR of the target UE at least in the unicast scenario. - The CR and CBR measurement window defined in NR sidelink can be reused for SL-PRS CR and CBR measurement separately (i.e., [n-a, n-1] for CBR measurement and [n-a, n+b] for CR measurement). - The number of CBR ranges defined as 8 in NR sidelink can be reused in SL positioning. |
| Huawei, HiSilicon | Proposal 29: For the CBR and CR definition of SL-PRS of dedicated resource pool - the subchannel in the definition of CR is changed to SL-PRS resource; - the RSSI measurement on all subchannel in the definition of CBR is changed to different RE offsets. Proposal 30: The following parameter of a SL-PRS configuration will be impacted by the congestion control mechanism of SL-PRS within the dedicated resource pool: - Maximum transmission power. |
| Continental | Proposal 2.1: In a dedicated RP, the UE could follow a similar congestion control mechanism as used for SL communication, but with corresponding measurements SL-P RSSI, SL-P CR and SL-P CBR performed on resources used for SL positioning. Proposal 2.2: Use shared measurements and a shared priority value for congestion control for both SL positioning and SL communication in a shared RP. Proposal 2.3: The shared measurements in the shared RP may be computed by the UE by appropriately weighting and combining the measurements on SL positioning and SL communication resources using a weighting parameter provided by the network. Proposal 2.4: At least the following parameters of a SL PRS configuration should be impacted by the congestion control mechanism in both dedicated and shared RPs: - Periodicity of SL PRS - Number of subchannels used for SL positioning related transmissions - Maximum number of SL PRS or measurement report (re-)transmissions - Maximum SL PRS transmission power Proposal 2.5: The UE adjusts the transmission parameters for SL positioning and/or SL communication in a shared RP so that the SL shared CR limit is satisfied for each shared priority value. |
| CATT,GOHIGH | Proposal 29: SL RSSI for SL positioning in a dedicated resource pool is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH, SL PRS and PSSCH (if PSSCH is included in the dedicated resource pool), starting from the 2nd OFDM symbol. Proposal 30: SL CBR for SL positioning in a dedicated resource pool measured in slot n is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2^µ slots, according to the new higher layer parameter sl-TimeWindowSizeCBR-positioning. Proposal 31: SL CR for SL positioning evaluated at slot n is defined as the total number of sub-channels used for its transmissions related to SL positioning in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b]. Proposal 32: The maximum transmission power of SL-PRS, the number of occupied subchannels of SL-PRS, and CR limit of SL-PRS are impacted by SL CBR for SL positioning. Proposal 33: The congestion control processing time for SL positioning should be based on µ of Table 8.1.6-1 and Table 8.1.6-2 in TS 38.214 for UE processing capability 1 and 2 respectively, where µ corresponds to the subcarrier spacing of the sidelink channel with which the SL-PRS is to be transmitted. Proposal 34: The congestion control parameters for SL positioning should be determined based on the existing congestion control mechanisms in Rel-16 V2X. - CBR measurement time window for SL positioning is equal to 100ms or 100·2^µ slots; - CR measurement time window for SL positioning is equal to 1000ms or 1000·2^µ slots; - Up to 16 CBR ranges for SL positioning should be defined. Proposal 35: For the shared resource pool, the legacy congestion control mechanisms should be reused in order to ensure backward compatibility with legacy Rel-16/17 UEs. |
| ZTE | Proposal 13: For congestion control mechanism in shared resource pool: - Either a new SL-PRS RSSI is introduced or existing SL RSSI is reused with slight modification - The definition of CR and CBR should reuse the legacy definition in Rel-16/17 SL communication. - CR can be evaluated separately for SL-PRS and PSSCH Proposal 14: For congestion control mechanisms in dedicated resource pool: - SL-PRS CR and SL-PRS CBR should be based on either - Alt.1: PSCCH which is associated with SL-PRS transmission - Alt.2: SL-PRS - Alt.3: both SL-PRS and PSCCH which is associated with SL-PRS transmission - The definition of SL-PRS CBR and CR is associated with SL-PRS resource allocation granularity and the SL-PRS resource configuration in a SL-PRS resource pool. - For comb-based multiplexing, SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and comb size. - For TDM-based multiplexing, the definition and configuration of SL-PRS CR and SL-PRS CBR is at least associated with the number of occupied SL-PRS/PSCCH resource(s) and the number of symbols for each SL-PRS resource. - CR and CBR measurement time window: the window size is configured per dedicated SL-PRS resource pool. - SL-PRS congestion processing time: based on both SCS and UE capability. - There are mapping between congestion measurements, SL-PRS priority and SL-PRS transmission parameters. |
| Interdigital | Proposal 25: Re-use the legacy PSSCH/PSCCH CBR measurement for transmissions including SL-PRS in a shared resource pool. Proposal 26: Consider different RSSI threshold in the CBR measurement for SL-PRS transmissions multiplexed with SCI-2 only and SL-PRS transmissions multiplexed with SL data. Proposal 27: Introduce a CBR measurement based on RSSI measurement based on a granularity of SL-PRS resource in a dedicated resource pool. Proposal 28: CBR-triggered adjustment on SL-PRS transmission configuration includes at least transmit power, BW and comb pattern. Proposal 29: Support a CR measurement based on sub-channels used per SL-PRS resource in a dedicated resource pool. Proposal 30: Re-use the legacy PSSCH/PSCCH CR measurement for transmissions including SL-PRS in a shared resource pool. Proposal 31: Support separate CR measurement for transmissions including SL-PRS and SCI-2 only in a shared resource pool. |
| Apple | Proposal 18: For congestion control the following need to be defined: - The CR and CBR should be estimated separately for the dedicated resource pool and the shared resource pool - The CBR/CR for the shared resource pool should be the same for SL positioning and SL communications. - CBR and CR definition for SL-PRS - For the shared resource pool, the CBR and CR definitions for a UE are identical with that for the legacy SL communications - For the shared resource pools, there is a need to normalize the time and frequency resources used in the estimates. - Define a SL-PRS allocation unit (made up of the frequency domain allocation unit (e.g. sub-channels) and time domain allocation unti (e.g. sub-slot)). - The Sidelink-PRS Channel Occupancy Ratio (SL-PRS CR) evaluated at slot n is defined as the total number SL-PRS allocation units divided by the comb size used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured SL-PRS allocation units in the transmission pool over [n-a, n+b] - SL-PRS Channel Busy Ratio (SL CBR) measured in slot n is defined as the portion of PRS-allocation units in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1] - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - # of SL-PRS resources - # physical resources for SL-PRS resource e.g. increase comb-size, reduce time resource - # of retransmissions - Transmission power - CR and CBR measurement time window - Can be configured separately from that of the time window for the shared channel - Congestion control processing time - Can be configured separately from that of the time window for the shared channel |
| Qualcomm |  |
| OPPO | Proposal 21: The following as congestion control mechanism for SL communication are reused for congestion control in SL positioning for both shared resource pool and dedicated resource pool: - Congestion control processing time is same as SL communication; - Up to 16 CBR ranges as SL communication are defined; - CBR measurement time window is [100ms or 100 slots]; - CR measurement time window is [1000ms or 1000 slots]; Proposal 22: The definition of CR and CBR in shared resource pool are same as those for SL communication. Proposal 23: In dedicated resource pool CR is defined based on the number of SL PRS resources occupied by a UE, and CBR is defined as the portion of OFDM symbols with SL RSSI exceeding the (pre-)configured threshold. Proposal 24: At least following parameters for SL PRS can be adjusted according to measured CBR level: - CR limit; - Max transmission power of SL PRS; - Max transmission number or minimum periodicity of SL PRS; |
| Samsung | Proposal 7: In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. - The parameters impacted by congestion control include SL PRS CombSize, SL PRS symbol length, SL PRS bandwidth, and SL PRS max power. - The priority value for SL PRS is provided by high layer of UE - If the priority of SL PRS and PSSCH is different, max function can be used. - CBR measurement for SL PRS can be reported to gNB. - Congestion control for SL PRS can restrict CR limit of SL PRS by CBR and priority. - FFS: The definition of CBR/RSSI/CR for SL positioning. |
| LGE | Proposal 45: SL PRS resource is the granularity of RSSI measurement for CBR estimation in SL positioning. Proposal 46: For a dedicated resource pool, SL PRS CBR is estimated as follows. - SL PRS Channel Busy Ratio (CBR) measured in slot n is defined as the portion of SL PRS resources in the resource pool used for SL PRS transmission, whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100•2µ slots (µ=SCS), according to higher layer parameter sl-TimeWindowSizeCBRforSLPRS. Proposal 47: For a dedicated resource pool, SL PRS CR is estimated as follows. - SL PRS Channel Occupancy Ratio (SL PRS CR) evaluated at slot n is defined as the total number of SL PRS resources used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured SL PRS resources in the transmission pool over [n-a, n+b], which is used for SL PRS transmission. Proposal 48: For a shared resource pool, CBR and CR definition in Rel.16 NR SL are reused. Proposal 49: CBR estimation for SL positioning can be based on the followings. - RSSI measurement on SL PRS - RSSI measurement on PSCCH/PSSCH |
| Ericsson | - For the dedicated pool, redefine CBR and CR based on the granularity of the resource reservation. - To adjust the load of the transmission on a pool the UE can switch between the configured dedicated pools. |

By going over all the above proposals, we can start the discussion on congestion control with the following 4 proposals in sections 3.5.3.1-3.5.3.4:

##### 3.5.3.1 Congestion control for Shared Resource Pool

[MEDIUM] Feature Lead Proposal 3.5.3.1-v0

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP,

  - CBR and CR mechanisms from Rel.16 NR SL are reused, with the following potential changes:

    - Modification 1: with regards to the evaluation of the CR and CBR

      - Alt. 1: CR/CBR can be evaluated separately for SL-PRS and PSSCH.

      - Alt. 2: One CR/CBR shall be evaluated for both SL-PRS and PSSCH.

    - Modification 2: with regards to the RSSI evaluation for CBR

      - Alt. 1: PSCCH, PSSCH is being used  (same as legacy)

      - Alt. 2: PSCCH, PSSCH and SL PRS can be used

    - Note: other potential modifications are not precluded

Companies views

| vivo |  | Alt 2+Alt 1 |  |
| --- | --- | --- | --- |
| ZTE |  | For modification 1, in the legacy CR definition, there is note saying “SL CR can be computed per priority level”, we think priority value of SL-PRS can be integer or decimal, for example SL-PRS priority is 2.5 which implies lower priority than data with priority 2, and higher priority than data with priority 3. Therefore we prefer Alt.1. For modification 2, We support Alt.2. SL-PRS should also be considered. It is not enough to only measure the RSSI in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH. We prefer the following modification: Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH or additionally for SL-PRS, starting from the 2nd OFDM symbol. |  |
| Continental |  | For Modification 1: We support Alt. 2. A shared CR, CBR and priority value for SL positioning and communication in a shared RP can be easily computed by linearly weighting the corresponding measurements as outlined in our proposals. Hence we do not support Alt. 1. For Modification 2: we support Alt. 2. |  |
| Futurewei |  | Alt 2 + Alt 1 |  |
| CATT |  | For Modification 1: we prefer Alt.2. For Modification 2: we prefer Alt. 2. |  |
| Huawei, HiSilicon |  | Regarding Alt2 for Mod1, is it saying the legacy mechanism is used and SL-PRS transmission can not affect that evalut result by legacy? |  |
| Samsung |  | No PSSCH in dedicated resource pool. So, we thnk Alt 2 in modification 1 is not work. We support Alt 1 and in the dedicated pool, CR/CBR can be evaluated for SL-PRS only. For modification 2, we think that only SL PRS should used for RSSI evaluation. No PSSCH in dedicated resource pool. |  |
| Qualcomm |  | CBR is a measure of congestion in the system, the source of this congestion does not impact the outcome. It only measures the proportion of resources with high RSSI, regardless of their type or origin. Similarly, CR is a measure of the UE’s contribution to congestion regardless of the transmission content. Modificaiton 1: Alt 2 We think there is woding issue in Modification 2. In SL communications, RSSI is not measured only PSCCH and PSSCH. It is measured on a subchannel in a slot that configured for PSCCH/PSSCH, i.e. it is a received power measurement on a bandwidth that could be used for a sidelink transmission: Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol. The distinction is important, for example, in the SL communication definition, the UE will still have an RSSI measurement even if it was unable to decode SCI-1, or if the interference came from other technologies. Since, RAN1 agreed to include both PSCCH and PSSCH with the SL-PRS transmission, the definition does not need to be updated for the shared resource pool. For example, PSFCH is not mentioned in the SL communication definition. Modification 2: with regards to the RSSI evaluation for CBR - Alt. 1: PSCCH, PSSCH is being used (same as legacy) Keep existing SL RSSI definition in 38.215 - Alt. 2: PSCCH, PSSCH and SL PRS can be used Explicitly include SL-PRS in the SL RSSI definition in 38.215 With those changes we prefer Alt 1. |  |
| OPPO |  | In shared RP, the resource granularity is sub-channel, same as SL communication, thus we support Alt 2 for Modification 1. For Modificaiton 2, it depends on whether configuration would be changed or not. In principle, the SL RSSI should be measured on all SL OFDM symbols in a slot. |  |
| Nokia, NSB |  | Support. Prefer Alt. 2 + Alt. 2 In the shared pool, congestion control behaviour of a Rel-18 UE which supports SL-PRS should be similar to the behaviour of a legacy UE (which is not aware of SL-PRS and measures RSSI over all REs including those used for SL-PRS), to ensure fairness between these UEs. We are not sure that Mod 1-Alt. 1 and Mod 2-Alt.1 are feasible: Current SL RSSI measurement does not depend on ability to decode SCI. But if the UE is supposed to treat SL-PRS differently from PSCCH/PSSCH then it would have to be able to decode all SCIs from all UEs contributing to RSSI in order to determine which REs are used for SL-PRS; and, if there is overlap in an RE between SL-PRS from some UEs and PSSCH from other UEs then to determine which fraction of the RSSI measured is due to SL-PRS and which fraction is due to PSSCH – that is not feasible. |  |

[MEDIUM] Feature Lead Proposal 3.5.3.1-v0

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP,

  - CBR and CR mechanisms from Rel.16 NR SL are reused, with the following potential changes:

    - Modification 1: with regards to the evaluation of the CR and CBR

      - Alt. 1: CR/CBR can be evaluated separately for SL-PRS and PSSCH.

        - ZTE

      - Alt. 2: One CR/CBR shall be evaluated for both SL-PRS and PSSCH.

        - Vivo, Continental, Futurewei, CATT, Qualcomm, OPPO, Nokia, NSB

    - Modification 2: with regards to the RSSI evaluation for CBR

      - Alt. 1: PSCCH, PSSCH is being used  (same as legacy)

        - Vivo, Futurewei, Qualcomm(with the change shown below), OPPO(?)

      - Alt. 2: PSCCH, PSSCH and SL PRS can be used

        - ZTE, Continental, CATT

    - Note: other potential modifications are not precluded

[MEDIUM] Feature Lead Proposal 3.5.3.1-v1

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP,

  - CBR and CR mechanisms from Rel.16 NR SL are reused, with the following potential changes:

    - Modification 1: with regards to the evaluation of the CR and CBR

      - One CR/CBR shall be evaluated for both SL-PRS and PSSCH.

- Modification 2: with regards to the RSSI evaluation for CBR

- Alt. 1: Keep existing SL RSSI definition in 38.215:

  - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol.

- Alt. 2: Explicitly include SL-PRS in the SL RSSI definition in 38.215 as follows:

  - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH or additionally for SL-PRS, starting from the 2nd OFDM symbol.

##### 3.5.3.2 Congestion control for dedicated resource pool

[MEDIUM] Feature Lead Proposal 3.5.3.2-v0

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, using the congestion control mechanisms from Rel.16 NR SL as a starting point, consider the following modifications:

- Modification 1: For the definition of SL PRS CR and CBR:

  - Alt. 1: Reuse the NR SL legacy definition (i.e., based on the number of subchannels in a slot)

  - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration

    - FFS: Details, including:

      - Whether the definition should be different depending on whether comb-based or TDM-based multiplexing is possible in a resource pool

      - whether there is a need to define a “SL-PRS allocation unit”

      - whether to define a separate CBR for different comb-offsets in a slot

- Modification 2: For the evaluation of RSSI used in the CBR definition:

  - Alt. 1: PSCCH is being used

  - Alt. 2: SL PRS is being used

  - Alt. 3: Both PSCCH and SL PRS is being used

- Modification 3: For the CR and CBR measurement time window size,

  - it can be separately configured for a dedicated resource pool and could take the values:

    - Alt. 1: 100ms or 100·2^µ slots

    - Alt. 2: other (Please provide a different option)

- Modification 4: For the congestion control processing time,

  - It should be based on both SCS and a new UE capability-(ies)

    - FFS: Details

- Note: other potential modifications are not precluded

Companies views

| ZTE |  | General ok with FL’s proposal. Just provide our preference for each modification. Modification 1: For the definition of SL PRS CR and CBR: - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration Modification 2: For the evaluation of RSSI used in the CBR definition: There can be mapping relationships between SCI and its associated SL-PRS resource(s) (e.g. one SCI resource/config is associated with one SL-PRS resource). that is also why we think SL-PRS CR and SL-PRS CBR can be based on either PSCCH/SCI or SL-PRS, or both PSCCH/SCI and SL-PRS. Modification 3: For the CR and CBR measurement time window size, - it can be separately configured for a dedicated resource pool and could take the values: - Alt. 1: 100ms or 100·2^µ slots Modification 4: For the congestion control processing time, - It should be based on both SCS and a new UE capability-(ies) |  |
| --- | --- | --- | --- |
| Continental |  | For Modification 1: we support Alt. 1 For Modification 2: we support Alt. 3 For Modification 3: we support Alt. 1 |  |
| Futurewei |  | Alt 1 Alt 3 Alt 1 OK |  |
| CATT |  | For Modification 1: we prefer Alt. 1 For Modification 2: we prefer Alt. 3 For Modification 3: we prefert Alt. 1 |  |
| Huawei, HiSilicon |  | For Modification 1: Alt. 2 For Modification 2: Alt. 2 For Modification 3: Alt. 1 |  |
| Samsung |  | We think some duplicated issue with 3.5.3.1-v0 We are OK for FL’s direction. @FL, you consider the down-selection in the next meeting. Is it correct understanding? |  |
| Qualcomm |  | Modification 1: Alt 1. Randomizing resources used is the function of the resource selection procedure. CBR only measures system congestion and could be used with CR to reduce the UE’s contribution to the overall congestion. For Modification 2, we have the same comment on wording as in P 3.5.3.1: The RSSI measurement is not performed on a specific PHY channel, but on all symbols in a slot that can be used for transmission of those changes. We propose the following: RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS. Modification 3: Alt 1. |  |
| OPPO |  | Modification 1: Alt 2. Subchannel may not be defined in dedicate RP, even configured, the resource granularity would not be subchannel any more if TDMed resources are configured. Modificaiton 2: Alt 3. Modification 3: Alt. 1. Moficatiion 4: OK. |  |
| Nokia, NSB |  | Support. Mod 1: Alt 2 Mod 2: Alt 3 Mod 3: Alt 1 |  |

[MEDIUM] Feature Lead Proposal 3.5.3.2-v1

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, using the congestion control mechanisms from Rel.16 NR SL as a starting point, consider the following modifications:

- Modification 1: For the definition of SL PRS CR and CBR:

  - Alt. 1: Reuse the NR SL legacy definition (i.e., based on the number of subchannels in a slot)

    - Suport: Continental, Futurewei, CATT, Qualcomm

  - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration

    - FFS: Details, including:

      - Whether the definition should be different depending on whether comb-based or TDM-based multiplexing is possible in a resource pool

      - whether there is a need to define a “SL-PRS allocation unit”

      - whether to define a separate CBR for different comb-offsets in a slot

    - Support: ZTE, OPPO, Nokia, NSB

- Modification 2: For the evaluation of RSSI used in the CBR definition:

  - RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS

    - Support: ZTE, Continental, FUturewei, CATT, OPPO, Nokia, NSB

- Modification 3: For the CR and CBR measurement time window size,

  - it can be separately configured for a dedicated resource pool and could take the values:

    - Alt. 1: 100ms or 100·2^µ slots

          - ZTE, Continental, Futurewei, CATT, OPPO, Nokia, NSB

    - Alt. 2: other (Please provide a different option)

- Modification 4: For the congestion control processing time,

  - It should be based on both SCS and a new UE capability-(ies)

    - FFS: Details

- Note: other potential modifications are not precluded

##### 3.5.3.3 Parameters which can be impacted

[MEDIUM] Feature Lead Proposal 3.5.3.3-v0

In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority (companies are encouraged to comment on their preferred option(s)):

- Option 1: SL PRS transmission power

- Option 2: Periodicity of SL PRS

- Option 3: Number of occupied subchannels of SL PRS

- Option 4: Number of SL PRS resources in a slot

- Option 5: comb-size of a SL PRS resource in a slot

- Option 7: Number of OFDM symbols of a SL PRS resource in a slot

- Option 8: Number of SL PRS (re-)transmissions

- Option 9: Number of measurement report (re-)transmissions

Note: Companies are encouraged to comment if their preferred options apply to only the dedicated resource pool or both the shared and dedicated resource pool.

Companies views

| ZTE |  | At least Option 1 should be supported. For the others, we think it is related to how we define CBR and CR for SL positioning. |  |
| --- | --- | --- | --- |
| Continental |  | At least Options 1, 2, 3, 7, 8 and 9 should be supported. |  |
| CATT |  | We prefer Opttion 1, 2, 3, 4, 5, 7. |  |
| Huawei, HiSilicon |  | At least Option 1 |  |
| Samsung |  | We prefer Options at least 1, 3, 5, 7. |  |
| Qualcomm |  | For shared resource pools: Option 1 + Up to UE implementation how to meet the target CBR/CR including dropping transmissions (as in SL communications). For dedicated resource pools: Option 1 + Up to UE implementation how to meet the target CBR/CR including dropping transmissions (as in SL communications). Given the agreement on using only RP bandwidth for SL-PRS in the dedicated resource pool, Option 3 cannot be applied in a dedicated resource pool. |  |
| Nokia, NSB |  | At least Option 1, Option 8 Regarding Option 9 – since this is a higher layer message it will be subject to the legacy SL communications congestion control, no need to include it here. |  |

##### 3.5.3.4 General proposal

[MEDIUM] Feature Lead Proposal 3.5.3.4-v0

In Scheme 2, with regards to the congestion control for SL PRS:

- Maximum number of CBR ranges for SL positioning to be defined:

  - Alt. 1: 8

  - Alt. 2: 16

- CBR measurement for SL PRS can be reported to gNB

##### Companies views

| ZTE |  | Generally ok. We support Alt.1 to reuse the same maximum number of CBR range as that for sidelink communication. maxCBR-Config-r16 INTEGER ::= 8 -- Maximum number of CBR range configurations for sidelink communication |  |
| --- | --- | --- | --- |
| Continental |  | We are OK with the proposal. However, we propose that the CBR ranges may be different for dedicated and shared RPs. |  |
| Huawei, HiSilicon |  | Alt1 |  |
| Samsung |  | We are OK with the proposal. |  |
| Qualcomm |  | Alt 1 |  |
| OPPO |  | Alt 2 maxCBR-Level-r16; -- Maximum number of CBR levels. INTEGER; ::= 16. |
| Nokia, NSB |  | Alt 2 Note that in our understanding this relates to the legacy maxCBR-Level-r16, which is 16; perhaps the proposal needs to be clarified. |  |

#### 3.5.4 Inter-UE coordination for SL-PRS

| Nokia | Proposal 23: For code-domain multiplexing (CDM) of SL PRS transmissions, study IUC like mechanism to determine if SL PRS sequence multiplexing is acceptable at the receiver UE. Proposal 24: For frequency-domain multiplexing of SL PRS transmissions, study IUC like mechanism to determine if comb-based multiplexing of SL PRS is acceptable at the receiver UE. Proposal 27: Support IUC for SL PRS conflict avoidance/resolution. Consider comb offset coordination, SL PRS sequence selection, and CDM to share the same time-frequency resource and SL PRS muting patterns in IUC. |
| --- | --- |
| Futurewei | Proposal 3: Use the IUC Scheme 1 to provide non-preferred resource sets for SL-PRS transmission. Proposal 10: A SL UE may recommend preferred or non-preferred SL-PRS resources for other SL UEs participants in the positioning process. Proposal 11: Use the IUC Scheme 2 to provide the preferred resources for multiple SL-PRS same slot multiplexing. Proposal 12: For the SL-PRS shared resource pools reuse the existing IUC signaling to provide preferred resources for SL-PRS. Proposal 13: For the dedicated SL-PRS resource pools support PSFCH to provide the preferred resources for SL-PRS. |
| Toyota | Proposal 6: Support inter-UE coordination signaling also for the dedicated resource pool. |
| vivo | - The preferred resource sets and non-preferred resource sets can be considered as the IUC information in scheme 2 SL-PRS resource allocation. |
| spreadtrum | Proposal 21: Inter-UE coordination (IUC) in Mode 2 should be supported for SL-PRS resource allocation. |
| Huawei, HiSilicon | Proposal 26: Support that a UE can recommend a SL-PRS resource transmission from another UE. - Inter-UE coordination Scheme-1 can be used as baseline. Proposal 27: To enable the inter-UE coordination Scheme-1 mechanism for SL-PRS transmission, the SCI can include: - Providing/Requesting indicator in SCI - Allocated SL-PRS resource information when the Providing/Requesting indicator indicates ‘Providing’ - The recommended SL-PRS resource transmission from the responding UE when the Providing/Requesting indicator indicates ‘Requesting’. - The related inter-UE coordination information can be indicated in the first stage SCI. |
| Intel | - SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges. - Definition of IUC for dedicated resource pools is deprioritized for Rel-18 SL positioning discussions. |
| Xiaomi | Proposal 11: IUC message from a UE to the other UE to assist SL PRS resource allocation shall be supported. |
| ZTE | Proposal 15: Support SL-PRS IUC to maximize resource utilization and minimize resource conflicts. - Reuse the Rel-17 Scheme 1 |
| Panasonic | Proposal 8: Support inter-UE coordination signaling for SL-PRS for both dedicated and shared resource pool with both Rel-17 Scheme 1 and 2 IUC are used as a starting point. |
| NEC | Proposal 11: Both Scheme 1 IUC procedure and Scheme 2 IUC procedure can be considered as the starting point for SL-PRS transmission on at least dedicated resource pool for positioning. Proposal 12: The IUC procedure could be applied based on necessary enhancement on the positioning related indication. Proposal 13: The SL PRS resource conflict indication should be considered, and corresponding conflict avoidance/solution mechanism should be studied. |
| Apple | Proposal 16: For Rel-17 type sensing, the IUC schemes supported will be determined by the channels can be included in or linked with the dedicated resource pool. In Rel-18, a dedicated RP may be associated with a shared RP that carries this information. |
| Qualcomm | Proposal 17: Deprioritize inter-UE coordination in the dedicated resource pool until essential aspects of sidelink positioning are complete. |
| OPPO | Proposal 25: IUC scheme 1 is supported for SL PRS resource selection in shared resource pool. Proposal 26: FFS whether IUC scheme 1 is supported for SL PRS resource selection in dedicated resource pool. Proposal 27: IUC scheme 2 should be applied to SL PRS resource selection in shared resource pool and dedicated resource pool for SL positioning. Proposal 28: Conflict indication for the already used SL PRS resource(s) should be supported for SL positioning. |
| Samsung | Proposal 9: For Scheme 2, with regards to Inter-UE coordination for SL-PRS, both Rel-17 Scheme 1 and 2 IUC are used as a starting point. - Inter-UE coordination provides additional information of comb/time offset for comb-based/TDM-based multiplexing of SL-PRS from different UEs in a slot. |
| LGE | Proposal 40: The resource selection based on the inter-UE coordination message defined in Rel.17 SL is supported for SL PRS resource selection. Proposal 41: In IUC-based SL PRS resource selection for SL TDOA positioning, the handling of the multiple IUC messages are needed. |
| lenovo | Proposal 5: RAN1 to further consider IUC scheme 1 and 2 for dedicated SL-PRS resource pool and study details. |

[HIGH] Feature Lead Proposal 3.5.4-v0

- For the shared resource pool, reuse the existing IUC signaling of both Scheme 1 and Scheme 2.

  - SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges.

- For the dedicated resource pool, support IUC signaling and

  - Alt. 1: use Rel-17  IUC Scheme 1 as a starting point

  - Alt. 2: use both Rel-17 Scheme 1 and 2 IUC as a starting point.

Companies views

| vivo |  | For the second bullet, we only can accept Alt 1. Otherwise, we cannot support IUC mechanism in dedicated resource pool. |  |
| --- | --- | --- | --- |
| CMCC |  | We are fine with the proposal. Regarding the 2nd bullet for dedicated pool, we prefer Alt. 1, because we think that only PSCCH should be included in addition to SL-PRS resource in the dedicate resource pool, and IUC scheme 2 which requires PSFCH slots is naturally precluded. |  |
| ZTE |  | Generally ok. For dedicated resource pool, we are ok with Alt.1 and use Rel-17 IUC scheme 1 as a starting point. |  |
| OPPO |  | Support the proposal in general. But in current specification for determining preferred/non-prefered resource set and resource conflict a condition “UE is a destination UE of a TB/PSSCH” is required, this is not applicable for SL PRS only transmission. We propose to add “FFS how to adapt the conditions for determining preferred/non-preferred resource set and resource conflict” under the first bullet. For dedicated resource pool, support Scheme 2 only is also acceptable for us if companies have concern on the work load. |  |
| LGE |  | We support the 1st bullet for making a progress. We do not support the 2nd bullet as we don’t support PSSCH transmission in a dedicated resource pool. It is related to Proposal 3.2.2.1-v0. |  |
| Futurewei |  | Support. Alt 2 |  |
| Samsung |  | OK in general. For the second bullet (for the dedicated resource pool), we add one more bullet as FFS: additional information of comb/time offset for comb-based/TDM-based multiplexing of SL-PRS from different UEs in a slot. |  |
| CEWiT |  | Support. Prefer Alt 1 |  |
| CATT |  | Support. For the last bullet, we support Alt.2 |  |
| Panasonic |  | Ok, prefer alt 1. |  |
| Spreadtrum |  | Support. We prefer Alt 2. |  |
| InterDigital |  | We support the proposal. We prefer Alt. 2. |  |
| Huawei, HiSilicon |  | For the dedicated RP, Alt. 1 may be sufficient. |  |
| SONY |  | Okay. For the second bullet point, we prefer ALT-2 |  |
| Nokia, NSB |  | OK. For dedicated pool, we prefer Alt. 2. |  |

[HIGH] Feature Lead Proposal 3.5.4-v0

- For the shared resource pool, reuse the existing IUC signaling of both Scheme 1 and Scheme 2.

  - SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges.

- For the dedicated resource pool, support IUC signaling and

  - Alt. 1: use Rel-17  IUC Scheme 1 as a starting point

    - Vivo, CMCC, ZTE, CEWiT, Panasonic, Huawei, HiSilicon

  - Alt. 2: use both Rel-17 Scheme 1 and 2 IUC as a starting point.

    - Futurewei, CATT, Spreadtrum, Interdigital, Sony, Nokia

- Not support IUC in dedicated RP

  - LGE

#### 3.5.5 UE-A reserving SL-PRS resources for UE-B

| Nokia | Proposal 25: Support the resource allocation mechanisms that allow one UE to reserve one or more SL PRS transmission opportunities for other UE(s) in a SL positioning session. Support a feedback mechanism to enable such a scheme. |
| --- | --- |
| Toyota | Proposal 5: Support that a UE can reserve an SL-PRS resource for the transmission of another UE. |
| Spreadtrum | Proposal 22: UE A can select PRS resources in dedicated resource pool for UE B should be supported. |
| CATT, GOHIGH | Proposal 26: In order to solve the issue of SL-PRS resource confliction, the inter-UE coordination mechanism for SL-PRS should be introduced in the dedicated resource pool. Proposal 27: Inter-UE coordination for SL-PRS in the dedicated resource pool should reuse the existing inter-UE coordination mechanism for PSSCH in Rel-17 NR sidelink enhancements as much as possible to reduce the workload of standardization and the complexity of UE implementation. Proposal 28: Existing inter-UE coordination Scheme 1 and Scheme 2 for PSSCH in Rel-17 NR sidelink enhancements should be the starting point for inter-UE coordination for SL-PRS in the dedicated resource pool. |
| Panasonic | Proposal 4: For a SL UE requests other SL UE(s) to transmit or receive SL-PRS, the requesting UE should also indicate the resources to be used for SL-PRS. Proposal 9: Support that a UE can reserve a SL-PRS resource for the transmission of another UE. |
| Interdigital | Proposal 15: Support a UE to transmit SL-PRS in resources selected and indicated by a peer UE in a position group using SL-PRS resource allocation Scheme 2. Proposal 19: A transmitting UE selects SL-PRS resources in a received resource set based on IUC Scheme 1. Proposal 20: Support conflict indication for SL-PRS resource reservation based on IUC Scheme 2. |
| Fraunhofer | Proposal 5: For scheme 2, support that UE-A can reserve a SL-PRS resource for the transmission UE-B. Note: UE-B can only use the reserved resources under conditions that do not conflict with the sensing mechanism. |
| Apple | Proposal 17: UE-A reserving SL-PRS resources for UE-B: To support SL-RTT, it may be necessary for UE-A and UE-B to transmit their SL-PRS in a specific order. This may be by - UE-A explicitly reserving a resource for UE-B. In this case, the resource reserved for UE-B may be indicated with a flag to prevent an RSRP check - UE-A indicates a time window within which UE-B should reserve a resource |
| OPPO | Proposal 20: Do not support that a UE reserves a SL-PRS resource for the transmission of another UE. |
| Sharp | Proposal 11: For resource allocation scheme 2, support that a UE can reserve a SL-PRS resource for the transmission of another UE. |
| Lenovo | Proposal 4: RAN1 to support a UE that can reserve a SL-PRS for the transmission of another UE under certain restrictions/conditions, FFS conditions/restrictions, e.g., UEs with required capability, for measurements with strict timing constraints, limited to a dedicated resource pool, dependent on the number of UEs involved, type of coverage scenario partial coverage and out-of-coverage scenarios. |
| ZTE | Proposal 16: For SL-PRS resource allocation scheme 2, DO NOT support that a UE reserves a SL-PRS resource for the transmission of another UE. |

[HIGH] Feature Lead Proposal 3.5.5-v1

For scheme 2,

- Alt. 1: support that a UE can reserve a SL-PRS resource for the transmission of another UE.

- Alt. 2: Do not support that a UE can reserve a SL-PRS resource for the transmission of another UE.

Companies views

| vivo |  | Alt .2 |  |
| --- | --- | --- | --- |
| CMCC |  | Alt. 2 |  |
| ZTE |  | Alt.2. We have serious concern for Alt.1 and we do already agreed that both sensing and random resource selection is supported for scheme 2. |  |
| OPPO |  | Support Alt 2. Please note that one release (Rel-17) is used to support inter-UE coordination, i.e., “UE-A recommends a resource set to UE-B”, starting with a study phase to justify the feasibility and benefit. But we have not seen any evaluation on the feasibility, benefit and system impact of Alt. 1 so far, however, the scope and specification impact foreseen are enomous. |  |
| LGE |  | We need clarification on Alt. 1. In one way, Alt. 1 may mean that UE1 directly indicates UE2’s SL PRS resource, as discussed similar to mode2-d in Rel.16 SL communication agenda. In the other way, Alt. 1 may mean that UE2’s SL PRS resource can be derived from UE1’s SL PRS resource in SL RTT. The latter one is similar to PSFCH resource determination, where PSFCH resource is determined based on PSSCH resource. We do not support the former one, but support the latter one. Let’s clarificy Alt. 1 as follows. We support Alt 1b. - Alt. 1: support that a UE(i.e. UE1) can reserve a SL-PRS resource for the transmission of another UE(i.e. UE2). - Alt 1a. UE2’s SL PRS resource is indicated by UE’1 higher or lower layer signaling - Alt 1b. UE2’s SL-PRS resource is implicitly derived by UE1’s SL PRS resource |  |
| Futurewei |  | We are OK Alt1 and could compromise to having “ recommend” instead of “reserve” |  |
| Samsung |  | Support Alt 2 |  |
| CATT |  | We prefer Alt 1 with the following update, so it is just like the R17 IUC scheme 1. Alt. 1: support that a UE can recommend reserve a SL-PRS resource for the transmission of another UE. |  |
| Panasonic |  | We prefer alt 1. |  |
| Spreadtrum |  | Support. We support Alt 1. |  |
| Huawei, HiSilicon |  | Alt1 is ok since this help the RTT based and grouping based SL positioning resource selection. However, based on the last meeting discussion and for progress, we can be ok with recommended SL-PRS resource for another UE to transmit. |  |
| SONY |  | ALT-2 |  |
| Nokia, NSB |  | Support Alt. 1. |  |
| NEC |  | We prefer Alt 1 and also could compromise with “recommend” instead of “reserve”. |  |

#### 3.5.6 Random Resource selection

Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed

## Feedback mechanisms

| Nokia | Proposal 32: Support ACK/NACK feedback for unicast and groupcast SL PRS transmissions to allow or prevent retransmission of a SL PRS transmission. cProposal 33: Support employing different SL PRSs (e.g., two different cyclic shifts of a SL PRS sequence) from a UE to convey ACK/NACK feedback to trigger retransmission of SL PRS from the corresponding UE or not. Proposal 34: SL PRS transmissions can be re-configured based on higher layer feedback from the intended receiver that accounts for changing link conditions and positioning QoS requirements. |
| --- | --- |
| Panasonic | Proposal 10: SL-PRS support feedback for unicast and groupcast SL PRS transmissions and feedback-based retransmissions. |
| NEC | Proposal 10: The feedback of SL-PRS reception from receiving UE, i.e. PSFCH-like channel, should be supported in either shared resource pool or dedicated resource pool. |
| LGE | Proposal 37: SL PRS repetition or retransmission is supported as follows. - Blind repetition - Feedback-based retransmission |
| Ericsson | - ACK/NACK feedback for unicast and groupcast SL PRS transmissions and lower-layer Feedback-based Retransmissions is not supported. |

[HIGH] Feature Lead Proposal 3.6-v1

With regards to feedback mechanisms,

- Alt.1: Support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in this release

- Alt.2: Do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in this release

Companies views

| vivo | At least, for dedicated resource pool, lower layer feedback-based retransmission is not supported. |
| --- | --- |
| CMCC | Alt. 2 |
| ZTE | Alt. 2. We do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmission in this release. We already have measurement report and associated measurement quality. |
| OPPO | Support Alt.1 with “ACK/NACK feedback for SL-PRS or” removed, as ACK/NACK is determined based on CRC, it it not possible for SL PRS. |
| LGE | We support Alt 1. Feedback-based SL PRS retransmission improves the resource utilization and congestion control by avoiding unnecessary transmission. The condition of feedback can be discussed further. |
| Futurewei | Support Alt 1 |
| Samsung | Support Alt 2. |
| CATT | Alt 2. |
| Panasonic | Alt 1. |
| InterDigital | We support Alt. 1. It is essential to provide the feedback to improve the SL-PRS transmission reliability. |
| Huawei, HiSilicon | Alt2. |
| SONY | ALT-1 |
| Nokia, NSB | Alt 1 |

[HIGH] Feature Lead Proposal 3.6-v1

With regards to feedback mechanisms,

- Alt.1: Support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in this release

  - Interdigital, Panasonic, Futurewei, OPPO, LGE, Sony, Nokia

- Alt.2: Do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in this release

  - CMCC, ZTE,, Samsung, CATT, Huawei, HSilicon, vivo (at least for dedicated RP)

# SL-PRS cast types

| Futurewei | Proposal 14: For multi-RTT support SL-PRS multicast transmission, where multicast groups may be defined by some specific criteria (e.g., location-zone, RSRP, LOS/NLOS, requirement on its sync source/location information). |
| --- | --- |
| CATT, GHOGH | Proposal 36: “Cast type indicator”, “Source ID”, “Destination ID” fields (similar to SL communication) are included in SCI to support unicast, groupcast and broadcast SL-PRS transmissions. |
| Apple | Proposal 19: The SL PRS may be unicast, broadcast or multi-cast. For broadcast and multicast SL PRS/SRS in the shared resource pool, RAN1 should study the following two options: • Option 1: Grouping is the same as PSSCH i.e. the PSSCH and the PRS are bundled together • Option 2: Grouping is independent of the PSSCH. The SL-communications target should at least be a member of the SL-positioning target. Proposal 20: Broadcast and multicast transmission in the dedicated resource pool can use the design for SL communications as a starting point in the new SL-PRS SCI. |
| OPPO | Proposal 30: “Cast type indicator” and “Destination ID” fields as SL communication should be included in SCI to support unicast, groupcast and broadcast, where the destination ID is provided by higher layer.c |
| Sharp | Proposal 12: SCI is used for the signaling of the parameters of broadcast SL PRS. |

# Configuration/activation/deactivation/triggering of SL-PRS

## Triggering of SL-PRS

Agreement With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement. Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

| Toyota | Proposal 3: Down-select between SCI, MAC-CE, and SL-PRS as lower-layer signaling. Proposal 4: Confirm the working assumption that the UE-A can request UE-B to transmit SL-PRS via lower-layer signaling sent by UE-A. |
| --- | --- |
| Huawei,HiSilicon | Proposal 28: Confirm the following WA which are made in RAN1#112b-e meeting: - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Lower layer signaling corresponds to SCI |
| CATT, GOHIGH | Proposal 25: For SL-PRS resource allocation scheme 2, with regards to the triggering of SL-PRS, support the working assumption: - Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A |
| Intel | - SL MAC-CE can be used to trigger the transmission of SL-PRS of another UE. - SL-PRS reception can trigger the transmission of SL-PRS for the case of RTT exchanges. |
| Panasonic | Proposal 3: The triggering of SL-PRS transmission or reception could be lower layer signalling or higher layer signalling depending on the use cases. |
| NEC | Proposal 14: Confirm the work assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signalling sent by UE-A. |
| Sony | Proposal 9: Confirm the working assumption on support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS Proposal 10: The lower layer signalling to request SL-PRS transmission from UE-A to UE-B is using SCI. |
| CMCC | Proposal 9: With regards to the triggering of SL-PRS, the working assumption from RAN1#112bis meeting should be confirmed, i.e., support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Lower layer signaling should correspond to SCI. |
| Interdigital | Proposal 22: Confirm WA and support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. Proposal 23: Support SL-PRS transmission triggered by another UE via lower signal signaling corresponding to at least SCI and/or PSFCH. |
| Fraunhofer | Proposal 6: For scheme 2, Support UE-A triggering UE-B to transmit a SL-PRS signal based on the reception of the SL-PRS signal of UE-A |
| Apple | Proposal 12: In Scheme 2, with regards to the triggering of SL-PRS, confirm the working assumption that Rel-18 supports for UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - It is up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A. The signaling can be in the SCI |
| Qualcomm | Proposal 18: Only SL-PRS reception is supported as a lower-layer request for SL-PRS transmission. |
| OPPO | Proposal 29: In Scheme 1 or Scheme 2, support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A, the lower layer signaling is SCI. |
| Samsung | Proposal 12: In Scheme 2, with regards to the triggering of SL PRS, support UE-A to request UE-B to transmit SL PRS via SCI signaling sent by UE-A. - FFS: how to apply this in SL positioning methods of SL RTT, SL AOA, and SL-TDOA. |
| LGE | Proposal 35: Confirm WA for SL PRS resource selection triggering with understanding that MAC layer can be UE-B’s higher layer to transmit SL PRS in response to UE-A’s lower layer request. Proposal 36: In SL RTT, UE-A’s SL PRS can be a lower layer request to trigger UE-B’s SL PRS transmission. |
| lenovo | Proposal 9: RAN1 to confirm the working assumption of supporting UE-A to request UE-B to transmit SL-PRS via lower layer signalling sent by UE-A. Proposal 10: RAN1 to consider the Scheme 1 triggering of SL-PRS by a UE’s own higher-layers. Also support Scheme 1 triggering of SL-PRS by another. FFS whether lower-layer or higher-layer signalling is used. |
| ITL | Proposal 2: In Scheme 2, with regards to the triggering of SL-PRS, following working assumption should be supported. - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. ▪ UE-A: target UE, UE-B: only master UE among multiple anchor UEs |

Summary:

In Scheme 2, with regards to the triggering of SL-PRS,

- Confirm the WA

    - Toyota, Huawei, HiSilicon, CATT, GOHIGH, NEC, Sony, CMCC, Interdigital, Fraunhofer, Apple, LGE, lenovo, ITL

- With regards to the lower-layer signaling support

  - Alt 1: SCI

    - Huawei, HiSilicon, Sony, CMCC, Interdigital, Apple, Samsung

  - Alt 2: SL-PRS

    - Intel (for RTT), Qualcomm, LGE (for RTT)

  - Alt 3: MAC-CE

    - Intel

  - Alt. 4: PSFCH

    - Interdigital

[HIGH] Feature Lead Proposal 5.1-v1

- In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA.

  - With regards to the lower-layer signaling decide during this meeting whether to support one or both of the following 2 options:

    - Opt. 1: SCI

    - Opt. 2: SL-PRS

Companies views

| vivo | Opt.1 is preferred, SL-PRS based triggering is unclear to us, since some method doesn’t need other UE transmit SL-PRS so how to distinguish which SL-PRS is used to trigger the SL PRS transmission and which is not would be not clear. |
| --- | --- |
| CMCC | Support and we prefer Opt. 1. |
| ZTE | One clarification question, how to use SL-PRS as lower-layer signaling? |
| Continental | Support and prefer Opt. 1. |
| OPPO | Support the proposal. In Scheme 2 SL PRS should always be transmitted with SCI, therefore which lower-layer signaling(s) is used for triggering only has impact to intra-UE signaling, i.e., from PHY to SLPP, we propose to support both Opt.1 and Opt. 2 for simplicity. |
| LGE | We support Opt. 2 with relation to Proposal 3.5.5-v1. We think that MAC layer can determine SL PRS transmission similar to PSFCH-based retransmission. |
| Futurewei | Prefer Opt 1 |
| Samsung | Support (prefer Opt1) |
| CEWIT | Support. We prefer opt 1. |
| CATT | Support. We prefer Opt. 1. |
| Panasonic | Both (also ok with opt 1 only) |
| Spreadtrum | Support Opt. 1. |
| InterDigital | We support Opt. 1. |
| Huawei, HiSilicon | We support Opt.1: SCI and confirm the WA. |
| Sharp | We are fine with the proposal and we support Opt. 1. |
| SONY | Support, particularly Opt.1 |

## Activation/Deactivation of SL-PRS

| Qualcomm | Proposal 19: Support SL-PRS transmission activation/deactivation at the physical layer by the UE’s own higher layers only. |
| --- | --- |

## Configuration of SL-PRS

| Nokia | Proposal 28: A UE (e.g., target UE) may assist configuring parameters of SL PRS (e.g., bandwidth, periodicity, etc.) for other UEs (e.g., anchor UEs) such as by providing (non-)preferred set of SL PRS parameters. Proposal 29: Periodic or aperiodic (pre-)configured SL PRS can be activated or deactivated based on indications coming from higher layers, e.g., based on parameters such as distance, time thresholds and/or conditions such as number of SL PRS requests, etc. that are (pre-)configurable. Proposal 30: SL-PRS transmission can be re-configured in response to changing target-anchor link conditions according to channel information report from receiver UE. |
| --- | --- |
| vivo | - The higher layer signaling can be used to pre-configure SL PRS resource(s) per resource pool. - For the support of the multiple SL-PRS transmitted in a slot, the following parameters should be preconfigured to determine the resource of SL-PRS. - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation, the parameters can be determined based on the following options: - Option 1. Based on the pre-defined mapping rule of PSCCH and SL-PRS; - Option 2. Indicate SL PRS in the PSCCH. |
| Qualcomm | Proposal 20: Higher layer signaling (SLPP) is used for configuration of SL-PRS. |

# SL-PRS Time domain behavior

The following agreement has been made:

Agreement With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - FFS: any additional details, including whether or not higher layers can start/stop transmission. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: any additional details - Aperiodic SL-PRS - SL-PRS is transmitted at least once after [triggering/request] - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions. - FFS: any additional details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering

The following related proposals were found in the contributions:

| CATT, GOHIGH | Proposal 6: Resource allocation Scheme 1 should support both SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation. Proposal 21: For SL-PRS transmissions without periodic reservation, the maximum number of reservations and transmission after triggering should be 2 or 3, which is similar with Rel-16 V2X design. Proposal 22: SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation should be applicable to both resource allocation scheme 1 and scheme 2. Proposal 23: The similarities or commonalities between periodic SL-PRS and semi-persistent SL-PRS should be further studied for resource allocation scheme 2. Proposal 24: The SL-PRS transmissions without periodic reservation should be triggered explicitly by the related signalling procedure. |
| --- | --- |
| ZTE | Proposal 18: For SL-PRS transmissions without periodic reservation, the maximum number of reservations signaled in an SCI is (pre-)configurable at least with a value of 2 or 3, which is similar with Rel-16 sidelink. - SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation are applicable to both resource allocation scheme 1 and scheme 2. |
| CMCC | Proposal 12: For SL-PRS, in Scheme 2, semi-persistent transmission with a transmission periodicity after activation and until deactivation is not supported. Proposal 13: For SL-PRS, in Scheme 1, semi-persistent transmission with a transmission periodicity after activation and until deactivation can be supported. - A DCI similar to format 3_0 is used for the activation/deactivation. Proposal 14: R16 DL PRS like always-on periodic transmission of SL-PRS is applicable for Scheme 1 only. |
| Samsung | Proposal 10: In both shared and dedicated resource pool, the maximum number of reserved resources for SL-PRS transmission signaled in an SCI is 2. - FFS: (pre-)configurable with other maximum values of 1 and 3 |
| lenovo | Proposal 7: RAN1 to at least support the following time domain behaviour for SL-PRS transmissions: - Support of semi-persistent SL-PRS by re-using the existing SL framework as best as possible, e.g., using the configured grant scheduling mechanism with lower and/or higher layer activation/deactivation commands. - Support of aperiodic SL-PRS by re-using the existing SL framework as best as possible, e.g., the existing dynamic grant scheduling framework based on a UE request of SL-PRS resources to the gNB. Proposal 8: At least semi-persistent SL-PRS and aperiodic SL-PRS is applicable to both Scheme 1 and Scheme 2 resource allocation procedures. |
| Ericsson | - For the UE receiving SL PRS, periodicity (if any) is included in assistance data (LPP/SLPP) and reflected in the SCI reservation field. - For the SL-PRS transmissions without periodic reservation, use the maximum number of transmissions from legacy sidelink (i.e. up to 3 occasions in time resource assignment). - For periodic reception of SL PRS, reservation periodicity values are the same as for legacy sidelink. |

[LOW] Feature Lead Proposal 6-v4

For the a dedicated resource pool, the maximum number of reserved resources signaled in an SCI  is (pre-)configurable at least with a value of 2, which is similar with Rel-16 sidelink.

- FFS: value of 1, 3

- FFS: shared resource pool

# Other Proposals

| Futurewei | Proposal 15: For RTT-based positioning a maximum time gap (allowable) between the first SL-PRS transmission and the last SL-PRS transmission from the responding SL UE should be supported. |
| --- | --- |
| Toyota | Proposal 1: Resolve remaining open issues such that 10 msec – 1 sec positioning service latency requirements can be met, and resource-efficient solutions can be supported. |
| CMCC | Proposal 8: Some enhancements on resource allocation mechanism for RTT type solution should be introduced in Rel-18. - The motivation is to guarantee the interval b/w the SL-PRS resources selected by the two UEs small enough and mitigate the impact on clock drift. |
| Interdigital | Proposal 14: Study Scheme 2 sensing enhancements to improve SL-PRS transmission reliability, e.g., based on BW aggregation. Proposal 21: Study SL muting indication and signalling. |
| LGE | Proposal 39: When UE-A transmits SL PRS to UE-B, UE-A indicates to UE-B the required maximum responding time, within which e.g. the responding SL PRS in SL RTT or the measurement report in SL TDOA should be transmitted to UE-A. Proposal 43: When TX and RX SL PRS resources collide each other, a prioritization rule is necessary to drop either operation. Proposal 44: When SL PRS resource and UL transmission resource collide each other, a prioritization rule is necessary to drop either operation. Proposal 50: SL synchronization procedure is performed by the UE that performs SL positioning. Proposal 51: The synchronization errors between anchor UEs for SL TDOA shall be less than a (pre)configured threshold. Proposal 52: For RSTD-based SL TDOA, If anchor UE is directly synchronized to gNB/eNB, anchor UE transmits SL PRS with advancing the transmission time by TA/2, where TA is the timing advance used for UL transmission. Proposal 54: Displacement of UE by UE movement between transmission(s) and receptions(s) is included in the measurement report to the location calculation entity (e.g. LMF or server UE). |
| lenovo | Proposal 6: RAN1 to support enhanced resource allocation/coordination schemes for a SL Positioning Group where multiple target UEs are involved that are based on the existing Scheme 1 and 2 procedures, e.g., resource configuration forwarding/sharing by a member UE with other member UE(s) (enhancement to Scheme 1), inter-UE coordination by member UE with other member UE(s), sensing information coordination and server UE reserving SL-PRS resources for other UEs within a SL positioning group (enhancement to Scheme 2). |
| ITL | Proposal 3: The periodicity of DL PRS allocation based on unit of slots should be changed. Following could be starting points for each of options for resource allocation mechanism for SL-PRS. - For Option 1(sensing based resource allocation) : the periodicity of SL-PRS allocation is based on P_reserve - For Option 2(random resource selection) : the periodicity of SL-PRS allocation is based on sl-TimeResource (length = L_bitmap) |
| Ericsson | - The listening UEs receives the SL PRS configuration via higher layers, i.e., from the LMF when the UE is in LMF coverage. - This includes UEs connected to a LMF via a relaying UE, using UE assisted positioning. - For the measuring UEs, SLPP / LPP assistance data includes periodicity or start/stop information for the SL PRS resource. - When out of coverage, the listening UE receives the SL PRS configuration via higher layers, i.e., from the transmitting UE’s higher layer, via SLPP. - To reduce the probability of collisions, coordinated transmission of reservation messages in SCI should be studied. |

# Proposal for Online Discussion

##### OFFLINE CONSENSUS

For Scheme 2, in a dedicated resource pool,

- Multiple L1 SL-PRS priority are allowed  in a resource pool

- A SL PRS resource within the resource selection window is used as a candidate resource

- with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

  - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point.

- with regards to the resource (re)-selection procedure

- support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

##### OFFLINE CONSENSUS

In dynamic grant type resource allocation in scheme 1,

- For shared resource pool, DCI format 3_0 is being used as a starting point

  - Alt. 1: Indication SL-PRS specific information is explicitly included

    - FFS: Which SL-PRS specific information

  - Alt. 2: Indication SL-PRS specific information is not explicitly included

- FFS: Dedicated resource pool

##### [MEDIUM] Feature Lead Proposal 3.2.2.3-v2

In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects:

- the first PSCCH symbol is mapped to the 2nd symbol available for SL tranmsissions in a slot

  - Note: 1st symbol available for SL tranmsissions in a slot is for PSCCH AGC similar to legacy

- PSCCH DM-RS in the slot is being reused from legacy

- The number of PSCCH symbol(s) is (pre-)configured to:

  - Alt. 1: 2 or 3 symbols (same as legacy)

    - LGE, CMCC, Huawei, HiSilicon, Qualcomm, Samsung, vivo, Sony, Lenovo, Fraunhofer, Intel, Apple

  - Alt. 3: 1, or 2 or 3 symbols

    - Ericsson, CEWiT, OPPO, CATT, ZTE

- The number of PRBs is (pre-)configured using the legacy values

  - FFS: reconsider if 1-symbol PSCCH is supported

##### [MEDIUM] Feature Lead Proposal 3.5.3.1-v2

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP, CBR and CR mechanisms from Rel.16 NR SL are reused, with the following changes:

    - Modification 1: with regards to the evaluation of the CR and CBR

      - One CR/CBR shall be evaluated for both SL-PRS and PSSCH

    - Modification 2: with regards to the RSSI evaluation for CBR

      - Alt. 1: Keep existing SL RSSI definition in 38.215

      - Alt. 2: Explicitly include SL-PRS in the SL RSSI definition in 38.215 as follows:

  - Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH or additionally for SL-PRS, starting from the 2nd OFDM symbol.

##### [MEDIUM] Feature Lead Proposal 3.5.3.2-v2

For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, consider the following modifications:

- Modification 1: For the definition of SL PRS CR and CBR:

  - Alt. 1: Reuse the NR SL legacy definition (i.e., based on the number of subchannels in a slot)

  - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration

    - FFS: Details, including:

      - Whether the definition should be different depending on whether comb-based or TDM-based multiplexing is possible in a resource pool

      - whether there is a need to define a “SL-PRS allocation unit”

      - whether to define a separate CBR for different comb-offsets in a slot

- Modification 2: For the evaluation of RSSI used in the CBR definition:

  - RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS

- Modification 3: For the CR and CBR measurement time window size,

  - it can be separately configured for a dedicated resource pool and could take the values: 100ms or 100·2^µ slots

- Modification 4: For the congestion control processing time,

  - It should be based on both SCS and a new UE capability-(ies)

- Note: other potential modifications are not precluded

##### [MEDIUM] Feature Lead Proposal 3.5.3.3-v2

In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. Consider further the following parameter(s):

- Option 1: SL PRS transmission power

- Option 2: Periodicity of SL PRS

- Option 3: Number of occupied subchannels of SL-PRS (for shared resource pool)

- Option 4: Number of SL PRS resources in a slot

- Option 5: comb-size of a SL PRS resource in a slot

- Option 7: Number of OFDM symbols of a SL PRS resource in a slot

- Option 8: Number of SL PRS (re-)transmissions

- FFS: Other options are not precluded

##### [MEDIUM] Feature Lead Proposal 3.5.3.4-v1

In Scheme 2, with regards to the congestion control for SL PRS:

- CBR measurement for SL PRS can be reported to gNB

##### [LOW] Feature Lead Proposal 6-v4

For the a dedicated resource pool, the maximum number of reserved resources signaled in an SCI  is (pre-)configurable at least with a value of 2, which is similar with Rel-16 sidelink.

- FFS: value of 1, 3

# Proposal for Offline Discussion

# 10 References

- R1-2304347	On resource allocation for SL positioning reference signal	Nokia, Nokia Shanghai Bell

- R1-2304367	Discussion on resource allocation for SL PRS	FUTUREWEI

- R1-2304412	Discussion on resource allocation for SL positioning reference signal	TOYOTA Info Technology Centerc

- R1-2304486	Discussion on resource allocation for SL positioning reference signal	vivo

- R1-2304564	Discussion on resource allocation for SL positioning reference signal	Spreadtrum Communications

- R1-2304623	Discussion on SL-PRS resource allocation	Huawei, HiSilicon

- R1-2304676	Considerations on resource allocation for SL positioning	Continental Automotive

- R1-2304737	On resource allocation for SL positioning reference signal	CATT, GOHIGH

- R1-2304830	On resource allocation for SL positioning	Intel Corporation

- R1-2304908	Discussion on resource allocation for SL positioning reference signal	xiaomi

- R1-2304929	Discussion on resource allocation for SL positioning reference signal	ZTE

- R1-2304968	Discussion on Resource Allocation for SL-PRS 	Panasonic

- R1-2305004	Discussion on resource allocation for SL positioning reference signal	NEC

- R1-2305043	On Resource Allocation for SL Positioning Reference Signal	Sony

- R1-2305100	Discussion on resource allocation for SL positioning reference signal	CMCC

- R1-2305127	Resource allocation for SL positioning reference signal	InterDigital, Inc.

- R1-2305213	Considerations on SL-PRS resource allocation	Fraunhofer IIS, Fraunhofer HHI

- R1-2305249	On Resource allocation for SL positioning reference signal	Apple

- R1-2305343	Resource Allocation for SL-PRS	Qualcomm Incorporated

- R1-2305429	Discussion on resource allocation for SL positioning reference signal	OPPO

- R1-2305520	On Resource Allocation for SL Positioning Reference Signal	Samsung

- R1-2305636	Discussion on resource allocation for SL positioning reference signal	LG Electronics

- R1-2305686	Resource allocation for SL positioning reference signal	Sharp

- R1-2305702	Discussion on resource allocation for SL PRS	ASUSTeK

- R1-2305741	Discussion on SL Positioning Resource Allocation	Lenovo

- R1-2305785	Considerations on resource allocation for SL positioning	ITL

- R1-2305831	Resource allocation for SL positioning reference signal	Ericsson

- R1-2305839	Resource allocation for SL-PRS	MediaTek (Chengdu) Inc.

- R1-2305903	Further discussion on SL positioning resource allocation for SL positioning reference signal	CEWiT

# 11Previous Agreements for SL Positioning Solutions

## RAN1 #109-e

Agreement

Study power control mechanisms for SL-PRS transmission, including whether it is necessary.

Agreement

With regards to the Positioning methods supported using SL measurements study further the following methods:

  - RTT-type solutions using SL

    - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT

  - SL-AoA

    - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study

  - SL-TDOA

  - SL-AoD

    - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu.

    - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study

- Consider in the study at least the following aspects:

  - Definition(s) of the corresponding SL measurements for each method

  - Which method is applicable to absolute or relative positioning or ranging, including whether such categorization is needed to be discussed.

  - For angle-based methods, antenna configuration consideration(s) using practical UE capabilities

  - Per-panel location, if UE uses multiple panels.

  - UE’s mobility, especially for V2X scenarios

  - Impact of synchronization error(s) between UEs

  - Existing SL measurements (e.g. RSSI, RSRP), and UE ID information etc, may be used.

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method.

- Note: When the study of carrier phase positioning and the evaluations of sidelink positioning have progressed, it can be reviewed whether carrier phase for sidelink can be considered in further work. Checkpoint at RAN1#110-e-Bis to see if sufficient information is available for this review.

- Note: Companies are encouraged to describe the role of SL nodes and their interaction/coordination participating in each method.

Agreement

With regards to the numerologies of the SL-PRS, limit the study to those supported for NR Sidelink.

- Note 1: NR Sidelink supports {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2

- Note 2: This doesn’t imply that SL-PRS FR2-specific optimization(s) are expected to be studied

Agreement

Study new reference signal for SL positioning/ranging using the existing PRS/SRS design and SL design framework as a starting point.

- The study could at least include: Sequence design, frequency domain pattern, time domain pattern (e.g. number of symbols, repetitions, etc), time domain behavior, configuration/triggering/activation/de-activation of the SL-PRS, AGC time, Tx-Rx Turanround time, supportable bandwidth(s), multiplexing options with other SL channels, randomization/orthogonalization options.

- Note: The study of existing SL reference signal for SL positioning/ranging is not precluded. Companies are encouraged to perform performance evaluation/comparison to investigate whether such reference signals can meet the positioning accuracy requirements.

Agreement

With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options:

- Option 1: High-layer-only signaling involvement in the SL-PRS configuration

  - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS.

  - Based on the study, this option may correspond to

    - A SL-PRS configuration that is a single-shot or multiple shots

    - A high-layer configuration that may be received from an LMF, a gNB, or a UE

- Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

  - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

- Option 3: Only lower-layer signaling involvement in the SL-PRS configuration

  - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI

- Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

Agreement

With regards to the Sidelink Positioning measurement report,

- Study the contents of the measurement report  (e.g. time stamp(s), quality metric(s), ID(s), angular/timing/power measurements, etc)

- Study the time domain behavior of the measurement report (e.g. one-shot, triggered, aperiodic, semi-persistent, periodic)

- FFS whether the Sidelink Positioning measurement can be a high-layer report and/or a lower layer report.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

- Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

- Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

- Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes.

- SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources (other parameters are not precluded) including its bandwidth and periodicity.

- Continue discussion on additional terminology clarification(s) such as: Initiator UE, Responder UE, Sidelink Positioning group, reference UE, etc, including whether such terminology is needed within RAN1 discussion.

Agreement

For the purpose of RAN1 discussion during this study item, at least the following terminology is used:

- Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

  - FFS: clarification of the knowledge of the location of the anchor UE

Agreement

With regards to the frequency domain pattern, study further a Comb-N SL-PRS design. Study at least the following aspects:

- N>=1 (where N=1 corresponds to full RE mapping pattern)

- Fully staggered SL-PRS pattern (e.g., M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used), Partially staggered SL-PRS pattern (e.g., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used), Unstaggered SL-PRS patterns (e.g., M symbol(s) of SL-PRS with comb- N, at each symbol a same RE offset is used, N > 1)

- The number of symbols of SL-PRS within a slot

  - Any relation to the comb-N option

  - RE offset pattern repetitions within a slot

- FFS: Other frequency domain pattern(s)

Agreement

For a potential new SL PRS, study further the following

- Number of symbol(s) for AGC and/or Rx-Tx turnaround time

- Conditions under which AGC training and/or Rx-Tx turnaround time are needed

Agreement

With regards to the SL Positioning resource allocation, study further the following 2 options for SL Positioning resource (pre-)configuration:

- Option 1: Dedicated resource pool for SL-PRS

  - Include in the study at least the following aspects:

    - which slots can be used, SL frame structure, SL positioning slot structure, multiplexing of SL-PRS with control information (if included in the same slot)

    - positioning measurement report

    - whether a dedicated frequency allocation (e.g., layer/BWP) is needed for SL PRS

    - resource allocation procedure(s) of SL-PRS

    - This option may or may not include control information (i.e., configuration/activation/deactivation/triggering of SL-PRS) for the purpose of SL positioning operation

- Option 2: Shared resource pool with sidelink communication.

  - Include in the study at least the following aspects:

    - co-existence between SL communication and SL positioning, backward compatibility

    - Multiplexing considerations of SL-PRS with other PHY channels (PSCCH, PSSCH, PSFCH) and any modifications in the SL-slot structure

Agreement

With regards to the SL-PRS resource allocation, study the following two schemes:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

  - Applicable regardless of the network coverage

- FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g. IUC-like solutions).

- Note: Other Schemes are not precluded to be studied

- FFS how to handle resource allocation of SL-Positioning measurement report

## RAN1 #110-e

Agreement

With regards to the Positioning methods supported using at least SL measurements, potential candidate positioning methods include at least the following:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- Note: other methods can still be studied

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified.

Agreement

A new reference signal should be introduced for supporting SL positioning/ranging.

Agreement

Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging:

- Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

  - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS.

- Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

  - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS

Agreement

With regards to the SL Positioning resource allocation, one of the following alternatives should be introduced for supporting SL positioning/ranging:

- Alt. 1: only dedicated resource pool(s) can be (pre-)configured for SL-PRS

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS

- Note: whether other signals/channels can be present in the dedicated resource pool can be further discussed

Agreement

For the content of the sidelink positioning measurement report, potential elements may include at least the following:

- One or more sidelink positioning measurement(s)

- Timestamp(s) associated with a sidelink positioning measurement

- Quality metric(s) associated with a sidelink positioning measurement

- Identification Information for a sidelink positioning measurement

- FFS any detail for the above

Agreement

For the sequence of the new reference signal for SL positioning/ranging, down select between Alt 1 and Alt 2:

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

- Alt. 2: ZC-based (SRS sequence as a starting point)

Agreement

With regards to the frequency domain pattern, a Comb-N SL-PRS occupying M symbol(s) design should be introduced for the support of NR SL positioning

- Note: there could be multiple values for M, N

Agreement

Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects:

- Resource selection mechanism for SL-PRS

- Inter-UE coordination

- Aspects for congestion control mechanisms for SL-PRS

Agreement

- With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further.

- With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

Agreement

With regards to the frequency domain pattern for multi-symbol SL-PRS, prioritize partially and fully staggered SL-PRS.

- Note: this does not preclude comb N=1

- FFS: single symbol SL-PRS, if supported

## RAN1 #110-bis-e

Agreement

- With regards to the RTT-type solutions using SL, down-select between the following 2 alternatives:

  - Alt. 1: it corresponds to a single-sided RTT method

  - Alt. 2: it may correspond to either a single-sided or double-sided RTT method

    - With regards to the double-sided RTT,

      - companies are encouraged to analyze and evaluate the effect in performance for the single-sided SL RTT due to clock drift

      - Study the order of the SL-PRS transmissions for double-sided RTT

      - Study the impact of UE mobility

    - FFS study whether there is or what is the spec impact of double-sided RTT method

- Note: the above may correspond to RTT with one or multiple devices

Agreement

For the study of SL-AoA positioning method,

- Both SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement should be included

  - FFS: Definition of the measurements

- Study further whether other measurements can be applicable

- Study further the frame of reference (LCS or GCS)

Agreement

With regards to SL-TDOA positioning method for SL-only positioning,

- It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE

- Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed.

Agreement

- From the potential candidate Positioning methods using at least SL measurements, at least the following should be introduced:

  - RTT-type solution(s) using SL

  - SL-AoA

  - SL-TDOA

- FFS: SL-AoD

Agreement

Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options:

- Opt. 1: through higher layers from the LMF

- Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB

  - Up to further discussion which one or more of these shall be applicable

Agreement

For the sequence of the new reference signal for SL positioning/ranging, use

- Alt. 1: pseudorandom-based. Use existing sequence of DL-PRS as a starting point.

Agreement

From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one)

- Broadcast (as a working assumption).

- FFS: Applicability of the above cast types

Agreement

With regards to the frequency and time domain pattern of a SL-PRS resource within a slot has the following characteristics:

- With regards to the value N (comb size) and the number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround:

  - At least the following values are considered as potential candidate values: N = {1,2,4,6,8,12}

  - FFS: the values considered as potential candidate values for M

  - FFS1: Whether to consider N>12 as a potential candidate value(s)

- The symbols of a SL-PRS resource within a slot are consecutive symbols

  - FFS: consecutive and/or non-consecutive symbols for shared resource pool (if supported)

- FFS: RE-Offset sequence within a SL-PRS resource, including whether to have in the end of the SL-PRS pattern a symbol with the same RE-offset as the first symbol, for phase-tracking purpose

Agreement

For a dedicated resource pool for SL positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

    - FFS: Details

    - FFS: definition of PSSCH associated with SL-PRS transmission(s)

- Note: Companies are encouraged to provide their analysis and views on the above

Agreement

With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

Agreement

With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning:

- Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s)

  - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration.

  - FFS: Whether SCI is single stage SCI or two stage SCI

- FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

Agreement

With regards to the Positioning methods supported using SL-PRS measurements

- at least the following measurements are considered:

  - SL Rx-Tx measurement

  - SL RSTD measurement

  - SL RSRP measurement

  - SL RSRPP measurement

  - SL RTOA measurement

  - SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- Companies are encouraged to study other measurements (e.g., time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor) and provide their analysis into why they are needed in light of the above measurements.

- Companies are encouraged to study potential enhancements, such as SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement

- FFS any additional measurements

Agreement

At least for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives:

  - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool

  - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool

- Note: Companies are encouraged to provide their analysis and views on the above alternatives

- FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

Agreement

Study further the granularity of time-domain resource allocation for SL-PRS transmission.

Agreement

With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective:

- Periodic SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity

  - FFS: any additional details, including whether or not higher layers can start/stop transmission.

- Semi-persistent SL-PRS

  - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

  - FFS: any additional details

- Aperiodic SL-PRS

  - SL-PRS is transmitted at least once after [triggering/request]

    - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions.

  - FFS: any additional details

- FFS: Applicability of the above time behaviors for scheme 1 & scheme 2

- FFS: Rx UE behavior is separately discussed.

- FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

## RAN1 #111

Agreement

SL-AoD is included as a potential candidate positioning method, and

- SL-AoD should be deprioritized over the remaining methods that have been recommended to be introduced.

Agreement

With regards to the SL Positioning resource allocation, support

- Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

- Note: this does not imply that the design is the same for both types of resources pools

- Note: shared resources pool(s) should be supported with backward compatibility

Agreement

From RAN1 perspective, at least the following 2 operation scenarios are recommended for normative work:

- Operation Scenario 1: PC5-only-based positioning.

- Operation Scenario 2: Combination of Uu- and PC5-based positioning.

Agreement

For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options:

- Option 1: A sensing based resource allocation should be introduced

- Option 2: A random resource selection should be introduced

- In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

Agreement

With regards to the RTT-type solutions using SL, both single-sided and double-sided RTT methods should be introduced

- Strive to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT.

- Note: a UE should be able to support single-sided RTT without having to support double-sided RTT

Agreement

Capture the following TP into the TR 38.859 as a conclusion:

For the solutions for sidelink positioning,

- The following 2 operation scenarios are recommended for normative work

  - Operation Scenario 1: PC5-only-based positioning.

  - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work.

  - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

- A new sidelink reference signal (SL-PRS) is recommended for normative work.

  - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point.

  - SCI can be used for reserving/indicating one or more SL-PRS resources

- Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation.

- With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work.

  - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

Agreement

A dedicated SL-PRS resource pool is (pre-)configured in the only SL BWP of a carrier.

Agreement

With regards to the power control for SL-PRS at least Open Loop PC should be introduced.

Agreement

For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced.

- A UE is not required to support both DL-TDOA-like operation and UL-TDOA-like operation

Agreement

With regards to the Positioning methods supported using SL-PRS measurements

- at least the following measurements should be introduced:

  - SL-PRS based Rx-Tx measurement

  - SL-PRS based RSTD measurement

  - SL-PRS based RSRP measurement

  - SL-PRS based RSRPP measurement

  - SL-PRS based RTOA measurement

  - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

Agreement

Update the agreed TP into the conclusion section of the TR 38.859 as follows:

For the solutions for sidelink positioning,

- The following 2 operation scenarios are recommended for normative work

  - Operation Scenario 1: PC5-only-based positioning.

  - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

- RTT-type solution(s) using SL, SL-AoA and SL-TDOA are recommended for normative work.

  - both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

  - For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation is recommended for normative work.

  - For the support of the above methods the following measurements are recommended for normative work:

    - SL-PRS based Rx-Tx measurement

    - SL-PRS based RSTD measurement

    - SL-PRS based RSRP measurement

    - SL-PRS based RSRPP measurement

    - SL-PRS based RTOA measurement

    - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

- A new sidelink reference signal (SL-PRS) is recommended for normative work.

  - Such a reference signal should use a Comb frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS should be used as a starting point.

  - SCI can be used for reserving/indicating one or more SL-PRS resources

  - With regards to the power control for SL-PRS at least Open Loop PC is recommended for normative work.

- Both a resource allocation Scheme 1 and Scheme 2 is recommended for normative work, where Scheme 1 corresponds to a network-centric operation SL-PRS resource allocation and Scheme 2 corresponds to UE autonomous SL-PRS resource allocation.

  - For resource allocation mechanism for SL-PRS in Scheme 2, a sensing based resource allocation, or a random resource selection, or both, should be introduced, where the legacy designs for UE autonomous resource allocation are used as a starting point.

- With regards to the SL-PRS transmission, both dedicated resource pool and shared resource pool with Rel-16/Rel-17/Rel-18 SL communication are recommended for normative work.

  - For SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

## RAN1 #112

- Agreement

- A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools).

- SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

- Agreement

For a dedicated resource pool for positioning:

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214).

  - FFS: additional slots that can be used for SL PRS is not precluded

- Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

- Agreement

For a dedicated resource pool for Positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further.

- Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Agreement

Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

Agreement

Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication).

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

  - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool

- FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

Agreement

With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications.

  - Study if any changes are needed

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options:

- Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes higher layer triggering from another UE

- Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Agreement

For SL-PRS transmission, at least support the following

- SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication

  - FFS: whether/what changes are needed

- SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation.

  - FFS: Maximum number of reservations and transmissions after triggering

Agreement

For the scheme 2 sensing-based resource allocation,

- Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool.

  - Study if/what changes are needed

- Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc)

- From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

## RAN1#112-bis

Agreement

For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a

- Dynamic grant

  - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point

- Configured grant type 1

  - the SL-PRS transmission(s) follows the higher layer configuration

- Configured grant type 2

  - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats)

- The above mechanisms use NR Rel-16 mode-1 signaling as a starting point

- FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool.

- FFS: Further details

Agreement

For a dedicated resource pool for positioning:

- No additional slots are needed to be supported

Agreement

For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier.

- A UE can be (pre-)configured with one or more dedicated SL resource pools.

- A UE can be (pre-)configured with one or more shared SL resource pools.

Agreement

Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool.

- Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation.

Agreement

For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point.

- Study at least the following aspects on potential changes over the existing congestion control mechanisms:

  - CBR and CR definition for SL-PRS

  - Which parameters  of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters

  - CR and CBR measurement time window

  - Congestion control processing time

  - Number of CBR ranges

  - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Agreement

With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits

- Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication

  - FFS: Details including a new or existing 2nd stage SCI

Agreement

In a shared resource pool:

- SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot

  - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting:

    - Alt. A.1: Only TDMing is supported

    - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported

      - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases

      - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS

    - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification

  - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting:

    - Alt. B.1: Only TDMing is supported

    - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported

  - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting):

    - Alt. C.1: 2nd SCI only

    - Alt. C.2: 2nd SCI and SL-SCH

    - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH”

  - FFS: Handling of PT-RS and SL-PRS

Agreement

For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot.

- FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

Agreement

For the scheme 2 sensing-based resource allocation:

- Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Note: This means that Rel-17 partial sensing is not considered a starting point for the design

Agreement

For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications:

- Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion:

  - Option 1: SL-PRS

  - Option 2: PSCCH DMRS

  - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool)

- Modification 2: For the resource selection window:

  - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget

  - Option 2: the selection window is provided by higher layers

- Modification 3: For the SL-PRS priority:

  - Option 1: A single L1 SL-PRS priority is allowed in a resource pool

  - Option 2: Multiple L1 SL-PRS priority are allowed  in a resource pool

- Modification 4: For the definition of a candidate resource within the resource selection window:

  - Options TBD

- Modification 5: For the reservation interval of SL-PRS:

  - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured.

- Modification 6: For the sensing window length (T0):

  - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec)

  - Option 2: Equal to or larger than the largest reservation interval

  - Option 3: Provided by higher layers with values TBD

- Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%):

  - Options TBD

- Modification 8: For the pre-emption of the reserved resources:

  - Options TBD

- Note 1: Other potential modifications and/or other options within each modification are not precluded

- Note 2: Multiple options for each potential modification may be supported

Agreement

In Scheme 2, with regards to the triggering of SL-PRS,

- Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers

- Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A

  - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS