3GPP TSG RAN WG1 #113		                                                       R1-2306223

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16

Source: 	Moderators (AT&T, NTT DOCOMO, INC.)

Title: 	Updated RAN1 UE features list for Rel-18 NR after RAN1#113

Document for:	Endorsement

- Introduction

This contribution includes the updated RAN1 UE features list for Rel-18 NR after RAN1 #113 meeting.

- NR_MIMO_evo_DL_UL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: separate row for TRP-specific BFR with unified TCI framework FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated joint TCI states per CC |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams FFS: maximum number of activated DL TCI states per CC FFS: maximum number of activated UL TCI states per CC FFS: separate row for TRP-specific BFR with unified TCI framework |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | FFS: separate rows intra-cell and inter-cell multi-TRP FFS: separate row for additional support of DCI format 1_1 and 1_2 configured with [TCI selection field] FFS: what to do/signal about two default beams |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | FFS: separate rows for resource set |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Support of two TCI states for CJT Tx scheme for PDSCH | FFS: how to signal QCL subset |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA | FFS: merge with 40-2-2 FFS: inter-/intra-cell |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-5 | TCI states of one CORESET Pool associated to both TAGs | This is a WA FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-8 | The maximum number of TAGs all CCs |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel. 16 type-II CJT must support this FG |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1a | Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-2 | Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1 | Prerequisite 40-3-1-1a |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | FFS: split for mode 1/2 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-4 | Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook | FFS: split for mode 1/2 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only A-CSI only FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement Note: A UE that supports CSI enhancement for Rel 17 type-II CJT must support this FG |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-6 | Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 | Prerequisite 40-3-1-5a |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | FFS: split for mode ½ |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | FFS: split for mode 1/2 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-9 | Support for 2NN1N2 >32 [conditioned upon support for 40-3-1-1 and/or 40-3-1-2] | FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously FFS: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CJT CSI measurement |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-10 | Additional support for dynamic selection of N<N_TRP [conditioned upon support for 40-3-1-1 and/or 40-3-1-2] |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-11 | Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot | analogous to 23-9-5 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-12 | Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-13 | Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-14 | Support of Support of L=6 for Rel-16-based CJT type-II codebook | Only for N_TRP=1 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-15 | dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook | Update 40-3-1-1 with “support of N=N_TRP only” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-16 | dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook | Update 40-3-1-5 with “support of N=N_TRP only” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-17 | Number of N_L>1 combinations across for Rel-16-based CJT type-II codebook | Update 40-3-1-1 with “support of N_L=1 only” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-18 | Number of N_L>1 combinations across for Rel-17-based CJT type-II codebook | Update 40-3-1-5 with “support of N_L=1 only” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-19 | Support of SP CSI for Rel-16-based CJT type-II codebook | Update 40-3-1-1 with “A-CSI only” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-20 | Support of SP CSI for Rel-17-based CJT type-II codebook | Update 40-3-1-5 with “A-CSI only” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-21 | Active CSI-RS resources and ports in the presence of mixed codebooks | FFS: which codebooks |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-22 | Max supported offset between CSI-RS (from different TRPs) |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-23 | Unequal number of spatial basis selection for multi-TRP CJT |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1 | Support of Rel-16-based doppler measurement | - Support X=1 CQI based on one slot - Support R=1 - Support parameter combinations with L=2,4 - Support for rank = 1,2 FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-2 | Support R=2 for Rel-16-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=1 based on averaging first and last slot of WCSI, for Rel-16-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-4 | Support of Rel-17-based doppler measurement | - Support of M = 1 - Support for rank = 1,2 FFS: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-5 | Support of M=2 and R=1 for Rel-17-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-6 | Support R=2 for Rel-17-based doppler codebook |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7 | support of l = (n – nCSI,ref ) for CSI reference slot |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Number of delay values | Number Y>1 of delay values |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1a | delay values larger than D_basic | Note: candidate component values cover both range 1 values and range 2 values |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Phase report | Support of phase report |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | FFS: Per-CC, across all CCs, and, simultaneous across all CCs (analogous to 2-33 or 16-1g) |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-6 | Maximum number of TRS resource sets in a report configuration |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol [3) DL PTRS RE mapping for Rel.18 enhanced DMRS ports for PDSCH with rank 1-4] [4) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.] |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1a | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1b | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1c | Support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1d | Support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1e | Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1f | Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-1g | Support enhanced DMRS {type 1, both type 1 and type 2} for PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-4 | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-5 | Support Rel-18 DL DMRS with single DCI based M-TRP | FFS: separate row for Additional row(s) of Rel.18 DMRS ports for single-DCI based M-TRP |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-5b | Support Rel-18 DL DMRS with M-DCI based M-TRP |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols [3) Support 1 symbol FL DMRS and 2 additional DMRS symbols] [4) UL PTRS RE mapping for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4] |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6b | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6c | Support DMRS type (uplink) for Rel.18 enhanced DMRS ports for PUSCH | Support DMRS {type 1, both type 1 and type 2} |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6d | Supported 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6e | Support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-6f | Support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-10 | DMRS port configuration for PUSCH with rank 5-8 | Candidate values: {Rel. 15 DMRS, Rel. 18 DMRs, borh} |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-4-12 | UL PTRS for PUSCH with rank 5-8 |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS with 8 Tx ports—antenna switching | FFS: separate row for comb8 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-4a | SRS with 8 Tx ports—codebook | FFS: separate row for comb8 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-6 | SRS with TDMed 8 Tx ports—antenna switching | FFS: separate row for comb8 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-5-7 | SRS with TDMed 8 Tx ports—codebook | FFS: merge with antenna switching |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based STx2P SDM scheme for PUSCH—codebook | FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | FFS: separate row for “2 PTRS ports for SDM scheme” FFS: separate row “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Associated CSI-RS resources for noncodebook single-DCI based STx2P SDM scheme for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-2 | New DMRS port entry for single-DCI based SDM scheme |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based STx2P SFN scheme for PUSCH—codebook | FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | FFS: separate row for “Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP” |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Associated CSI-RS resources for noncodebook single-DCI based STx2P SFN scheme for PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-3a | codebook multi-DCI based STx2P PUSCH+PUSCH | FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b | noncodebook multi-DCI based STx2P PUSCH+PUSCH | FFS: separate rows for - DG-PUSCH+CG-PUSCH - CG-PUSCH+CG-PUSCH - DG-PUSCH+DG-PUSCH FFS: how to signal - Partial-overlapping PUSCHs in time and non-overlapping in frequency - Overlapping PUSCHs in time and partially overlapping in frequency - fully overlapping in both frequency and time - fully overlapping in frequency, partially overlapping in time - partially or non-overlapping in frequency, partially or fully overlapping in time |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI based STx2P SFN scheme for PUCCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | FFS: Full-coherent codebook parameters as separate row or component |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-2a | Association between CSI-RS and SRS for non-codebook case |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-3 | Maximum number of layers [for rank 5-8] | FFS: split for codebook and noncodebook is in this row or separate FFS: CBG based PUSCH with rank >4 whether component or new row |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-4 | UL full power transmission mode 0 |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-4a | UL full power transmission mode 1 | This is a WA |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-4b | UL full power transmission mode 2 | This is a WA |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-5 | PUSCH [for codebook coherency subset] |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | (N1,N2) for coherent codebook |  |  |  |  |  |  |  |  |  |  |  |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-1 | Common SL PRS Processing Capability [across all resource pools] | FFS: separate rows for [for SL-TDOA DL type/ SL-TDOA UL type /SL-AOA\RTT-type solutions using SL single sided/ RTT-type solutions using SL single sided double-sided] incl SL-PRS resources |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-4a | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-4c | Transmitting SL-PRS scheme 2 in a dedicated resource pool |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-5 | SL-PRS congestion control [in a dedicated/shared resource pool] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-7 | SL PRS measurement for [SL-TDOA DL type/ SL-TDOA UL type /SL-AOA\RTT-type solutions using SL single sided/ RTT-type solutions using SL single sided double-sided] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-8 | Support of random selection [in a dedicated resource pool] | FFS: merge with 41-1-4 |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-10 | Support of [full\partial] sensing [in a dedicated resource pool] | FFS: merge with 41-1-4 |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-11 | TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-12 | Comb-based multiplexing for SL-PRS [transmission/reception] from different UEs in the same slot |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-13 | Reporting the additional paths for SL positioning |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-14 | LoS/NLoS indicator for SL positioning |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-17 | Open loop SL power control and SL RSRP report [in dedicated resource pool] | FFS: merge with 41-1-4 for SL PC and 41-1-3 for RSRP report |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-19 | UE can provide ARP location for sidelink as assistance data |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-1-19a | Report of ARP-ID with SL positioning measurements |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-1 | DL RSCP reporting based on DL PRS in RRC_CONNECTED | FFS: separate rows for DL PRS processing capabilities\measurement for CPP measurements in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-1a | DL RSCPD reporting based on DL PRS in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-2 | DL RSCP reporting based on DL PRS in RRC_INACTIVE | FFS: separate rows for DL PRS processing capabilities\measurement for CPP measurements in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-2a | DL RSCPD reporting based on DL PRS in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-3 | Measurement on indicated DL PRS resource sets [within the indicated time window(s) for UE based and UE assisted] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-2-4 | UE-based Carrier Phase Positioning | Components are 1. Support of carrier phase measurement 2. Support of Assistance data for UE-based Carrier Phase Positioning |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-3-1 | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state for initial UL BWP |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-3-2 | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state configured outside initial UL BWP |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-3-3 | [Processing capability for/Support of] PRS in RRC_IDLE |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-1 | DL PRS processing capabilities for aggregated PRS processing [of 2 PFLs] in intra-band contiguous [within a MG] [for RRC_CONNECTED sate, RRC_INACTIVE state, RRC_IDLE] | FFS: separate row for number of PFLs FFS: total aggregated BW and BW combinations |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-3 | Support of PRS bandwidth aggregation in RRC_CONNECTED — DL-TDOA |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-3a | Support of PRS bandwidth aggregation in RRC_CONNECTED —Multi-RTT |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-4 | Support of PRS bandwidth aggregation in RRC_ INACTIVE — DL-TDOA |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-4a | Support of PRS bandwidth aggregation in RRC_ INACTIVE —Multi-RTT |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-5 | Support of PRS bandwidth aggregation in RRC_IDLE — [DL-TDOA and Multi-RTT] |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-6 | Support of positioning SRS bandwidth aggregation in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-7 | Support of positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-4-8 | Support of positioning SRS bandwidth aggregation in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-5-1 | Support of PRS with Rx frequency hopping [and measurement report] [within a MG] [in RRC_CONNECTED/RRC_INACTIVE/RRC_IDLE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability |  |  |  |  |  |  |  |  |  |  |
| 41. NR_pos_enh2 | 41-5-2 | Support of positioning SRS with Tx frequency hopping [in RRC_CONNECTED/RRC_INACTIVE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability |  |  |  |  |  |  |  |  |  |  |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. Netw_Energy_NR | 42-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 42. Netw_Energy_NR | 42-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 42. Netw_Energy_NR | 42-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e and RAN1 #113]

- NR_netcon_repeater

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd 7.Support periodic beam indication for access link 8. Priority flag for periodic indication |  | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 | Optional without capability signaling |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Component 2 candidate values: - 15 kHz: {[0,] 1 [, 2,3,4]} - 30 kHz: {[0,]1 [, 2,3,4]} - 60 kHz: {[0,]1, 2 [ ,3,4]} - 120 kHz: {[0,]1, 2 [, 3,4]} | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of [temporary] MAC CE override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. NR_NTN_enh | 44-1 | PUCCH repetition for Msg4 HARQ-ACK | 1. Support repetition transmission of PUCCH for Msg4 HARQ-ACK on common PUCCH resource 2. Support receiving repetition factor in system information [3. Support receiving repetition factor in DCI scheduling Msg4 PDSCH] [4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition] [5. Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration] |  | Yes | No | UE does not support PUCCH repetition for Msg4 HARQ-ACK | Per Band | N/A | N/A | N/A | A UE that includes [RAN2 parameter name] in [RRC Setup Request] must support FG 44-1 [Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional without capability signaling |
| 44. NR_NTN_enh | 44-2 | NTN DMRS bundling enhancement for PUSCH | 1. Support of DM-RS bundling for PUSCH over consecutive slots 2. Support of pre-compensation to keep phase rotation due to timing drift within the phase difference limit [3. Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit.] | 30-4a/b | Yes | No | UE does not support DM-RS bundling enhancement for PUSCH in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional with capability signaling |
| 44. NR_NTN_enh | 44-3 | UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN | 1. Support UE Rx-Tx Measurement and Report for Multi-RTT positioning with single satellite in NTN [2. Support of assistance information report [to mitigate the timing error] in Multi-RTT positioning in NTN] | 13-4, 13-8 | Yes | No | UE does not support Multi-RTT positioning with single satellite in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104] | Optional with capability signaling |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. NR_Mob_enh2 | 45-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 45. NR_Mob_enh2 | 45-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 45. NR_Mob_enh2 | 45-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e and RAN1 #113]

- NR_UAV

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_UAV | 46-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 46. NR_UAV | 46-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 46. NR_UAV | 46-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e and RAN1 #113]

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-s1 | Transmission using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR PSCCH/PSSCH/ [PSFCH] TXs in 15kHz and/or 30kHz SCSs. Candidate value sets: {[15KHz, 30kHz, both]} FFS Rx capability including supported SCS | [15-3, 15-6, 15-11] | [Yes] | No |  | [Per band] | N/A | N/A |  | [This is the basic FG for LTE/NR sidelink co-channel coexistence.] [UE supports EUTRA mode 4 SL in the same band] | [Optional with capability signalling] [For UE supports NR sidelink co-channel coexist with EUTRA sidelink, UE must support this FG.] |

- NR_redcap_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_redcap_enh | 48-1 | RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1 | The following components are the same as for supportOfRedCap-r17 (28-1): 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP The following components are new compared to supportOfRedCap-r17 (28-1): [11. DL/UL peak data rate target of 10 Mbps] 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS 13. Relaxed RAR-PDSCH processing timeline FFS whether to add additional components |  | Y |  | Network assumes the UE is not a RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1. | [Per UE] | [No] | [No] (FR1 only) |  | A UE supporting this FG is not required to support FG 6-1. A UE supporting this FG is not allowed to support FG 28-1. The specifications for a UE supporting FG 28-1 (‘RedCap UE’) also apply for a UE supporting this FG (FG 48-1) unless stated otherwise. It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A. | Optional with capability signaling UEs supporting Rel-18 eRedCap UE complexity reduction feature(s) indicate support of this FG instead of FG 28-1 (supportOfRedCap-r17). |
| 48. NR_redcap_enh | 48-2 | RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1 | The capabilities of FG 48-2 are the same as for FG 48-1 except that the following restriction does not apply: 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS [13. Relaxed RAR-PDSCH processing timeline] | 48-1 | Y |  | Network assumes the UE is not a RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1. | [Per UE] | [No] | [No] (FR1 only) |  |  | Optional with capability signaling |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] FFS whether to report max number of sets of cells supported by UE across PUCCH groups 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field 10) The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD/TDD scheduling cell - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) HARQ feedback based on Type 1 HARQ codebook 7) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}. FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) HARQ feedback based on Type 1 HARQ codebook, FFS Type 2 HARQ codebook 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type:value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether this component is reported per reported value in component 3 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether this component is reported per reported value in component 3 [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 9) The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD scheduling cell - Two unicast DCI per slot of scheduling cell [for the set of cells] for TDD scheduling cell - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b 5) Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}, FFS whether to separately report for primary and secondary PUCCH cell groups, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support monitoring both legacy DCI format(s) (0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI | Details FFS |  |  |  |  |  |  |  |  |  |  |
| 49. NR_MC_enh | 49-3y | Advanced UE capability for larger number of unicast UL DCI | Details FFS |  |  |  |  |  |  |  |  |  |  |
| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | At least one of {49-1, 49-1b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | At least one of {49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5 | Type 2 HARQ CB support for DCI format 1_3 | HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 10-16 (Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 25-6 (Enhanced Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per band pair per band combination, details up to RAN2 | N/A | N/A | N/A | This FG is based on the following agreements. RAN1 will not discuss the detail of this FG and the detail is up to RAN2 Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching. | Optional with capability signaling |
| 49. NR_MC_enh | 49-Y | Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots | If two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching, and X us is reported with a candidate value set of {0us, 500us} - The reported value X is applied to both one TAG case and two-TAG case (if UE supports two-TAG case) Note: If the UE reports 0us, the minimum separation time is not applied | 49-X | Yes |  | [two uplink switching cannot be triggered in two consecutive reference slots for UL transmissions on more than 2 bands] | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |

- NR_XR_Enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 50. NR_XR_Enh | 50-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 50. NR_XR_Enh | 50-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e and RAN1 #113]

- NR_FR1_lessthan_5MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-x | Support for 3 MHz channel bandwidth | 1) Reception of 12 RB PBCH based on [RB-level puncturing] [at least] for band n100 FFS 15 PRBs 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS |  | Yes | N/A | UE is not able to support 3 MHz channel bandwidth | [Per Band] | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only | Optional with capability signalling |

- NR_DSS_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_DSS_enh | 52-1 | Reception of NR PDCCH candidates overlapping with LTE CRS REs | Reception of NR PDCCH candidates that overlap with LTE CRS REs within a NR carrier using 15 kHz SCS 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM pattern by configuration of one CRS rate matching pattern via lte-CRS-ToMatchAround 2) Reception of a NR PDCCH candidate in REs that overlap with LTE CRS: candidate value set {a) when at least one symbol of the NR PDCCH candidate is not overlapped with LTE CRS, b) when some or all of symbols of NR PDCCH candidate overlap with LTE CRS} 3) Reception of NR PDCCH candidates that overlap with LTE CRS REs on the X-th symbols of an NR slot. Candidate values for X: {only 2nd symbol, 1st and 2nd symbols} 4) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS] that are monitored within the first 3 OFDM symbols of a slot | 5-28 | Yes | N/A | UE is not required to support reception of NR PDCCH candidates overlapping with LTE CRS REs when it is provided with LTE CRS RM pattern by higher layers | Per Band | N/A | N/A | N/A | [Note: From UE perspective, PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15] For component 2, RAN1 considers support value b) in component 2 only if RAN4 performance requirements for value b) are not defined Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| 52. NR_DSS_enh | 52-1a | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns | 1) Reception of NR PDCCH candidates in REs that overlap with LTE CRS when UE is provided with LTE CRS RM patterns by configuration of one or multiple non-overlapping CRS rate matching patterns via lte-CRS-PatternList1-r16 [or lte-CRS-PatternList3-r18] | 52-1, 14-1 | Yes | N/A |  | Per Band | N/A | N/A | N/A | Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. | Optional with capability signaling |
| 52. NR_DSS_enh | 52-1b | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 1) NR PDCCH that overlaps with LTE CRS REs is in [Type-1 CSS with dedicated RRC configuration, Type-3 CSS, and/or USS] that are monitored within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot | 52-1, 22-12 | Yes | N/A |  | [Per Band] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 52. NR_DSS_enh | 52-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier (regardless of support or configuration of multi-TRP) | 1) Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier (regardless of support or configuration of multi-TRP) for the case when crs-RateMatchPerCoresetPoolIndex is not configured 2) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS: {2,3,4,5,6} 3) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS: {1,2,3} | 5-28 | Yes | N/A | UE does not support two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier | Per Band | N/A | FR1 only | N/A | If a UE supports FG52-2 and FG14-1, FG14-1 is reported for list1/2 and FG52-2 is reported for list3/4 | Optional with capability singaling |
| 52. NR_DSS_enh | 52-2a | Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier | Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier for the case when crs-RateMatchPerCoresetPoolIndex is configured | 52-2, 16-2a | Yes | N/A |  | Per Band | N/A | FR1 only | N/A |  | Optional with capability singaling |

- NR_BWP_wor

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell 3. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | Note: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig [Note: If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations.] This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP with interruptions | 1. UE is allowed to performs RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell 3. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. | [1-7, 2-24, 2-31,] 53-3 | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP and meet interruptions requirements | [Per band] | No | No | n/a | This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured. The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig [If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations.] This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell 3. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per band | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-x |  |  |  |  |  |  |  |  |  |  |  |  |
| 54. NR_cov_enh2 | 54-y |  |  |  |  |  |  |  |  |  |  |  |  |
| 54. NR_cov_enh2 | 54-z |  |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#112bis-e and RAN1 #113]

- TEI18

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-1 | additionalSR-Periodicities-r18 | Indicates whether the UE supports the following SR periodicities in the periodicityAndOffset parameter as specified in TS 38.331: -5sl for 30 kHz SCS -5sl and 10sl for 120 kHz SCS Candidate values {30 kHz SCS, 120 kHz SCS, both} |  | Yes | N/A | If the network implements the TS 38.331 CR on new SR periodicities and the UE does not according to the capability indication, the network will not assign the new SR periodicities. Legacy behaviour applies. | Per UE | No | No | N/A |  | Optional with capability signaling |
| 55. TEI18 | 55-2a | 1-symbol PRS for MG-based measurement in RRC_CONNECTED state | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot inside a MG in RRC_CONNECTED state FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 13-1 | No | n/a | 1-symbol PRS is not supported for MG-based measurement in RRC_CONNECTED state | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 55. TEI18 | 55-2b | 1-symbol PRS for outside MG in RRC_CONNECTED state | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot outside a MG in RRC_CONNECTED state FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-3 | No | n/a | 1-symbol PRS is not supported for outside MG in RRC_CONNECTED state | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 55. TEI18 | 55-2c | 1-symbol PRS in RRC_INACTIVE state | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot in RRC_INACTIVE state FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-6 | No | n/a | 1-symbol PRS is not supported in RRC_INACTIVE state | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 55. TEI18 | 55-2d | 1-symbol PRS for PDC | 1. Support of 1-symbol PRS with comb sizes from {2, 4, 6, 12} 2. Max number of single-symbol DL PRS resources it can process in a slot for PDC FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 25-19a | Yes | n/a | 1-symbol PRS is not supported for PDC | per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 55. TEI18 | 55-3 | Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1 | 1. Multi-PUSCH scheduling by single DCI format 0_1 for the operation with non-contiguous allocation | 10-17 | Yes | N/A | For operation on FR1, scheduling multiple PUSCHs by a DCI format 0_1 in non-contiguous slots is not supported. | Per Band | N/A | N/A | N/A |  | Optional with capability signalling |