3GPP TSG RAN WG1 Meeting #113		R1-2306238

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.5

Source:	Rapporteur (Intel Corporation)

Title:	RAN1 agreements for Rel-18 WI on Expanded and Improved NR Positioning

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made for the Rel-18 WI on ‘Expanded and Improved NR Positioning’ (WI code NR_pos_enh2, WID in [1]) up until RAN1#113.

# Table of contents

Introduction	1

Table of contents	1

1	General aspects and LSs	2

RAN1 #112:	2

RAN1 #112bis-e:	2

RAN1 #113:	3

2	SL positioning reference signal	4

RAN1 #112:	4

RAN1 #112bis-e:	5

RAN1 #113:	7

3	Measurements and reporting for SL positioning	8

RAN1 #112:	8

RAN1 #112bis-e:	10

RAN1 #113:	11

4	Resource allocation for SL positioning reference signal	13

RAN1 #112:	13

RAN1 #112bis-e:	14

RAN1 #113:	16

5	NR DL and UL carrier phase positioning	18

RAN1 #112:	18

RAN1 #112bis-e:	19

RAN1 #113:	21

6	LPHAP (Low Power High Accuracy Positioning)	23

RAN1 #112:	23

RAN1 #112bis-e:	25

RAN1 #113:	25

7	Bandwidth aggregation for positioning measurements	26

RAN1 #112:	26

RAN1 #112bis-e:	28

RAN1 #113:	30

8	Positioning for RedCap UEs	32

RAN1 #112:	32

RAN1 #112bis-e:	33

RAN1 #113:	34

References	35

- General aspects and LSs

## RAN1 #112:

For response to SA2 LS in R1-2301915 Conclusion Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report. Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact. Agreement The draft LS reply in R1-2302145 is endorsed. Final LS in R1-2302146.

## RAN1 #112bis-e:

For response to RAN2 LS on error source distributions in R1-2302282 Agreement Parameters for the overbound Gaussian distribution can be mean and standard deviation. Agreement From RAN1 perspective, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality, rtd-Quality-r16) and uncertainty information (e.g., LocationUncertainty-r16) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859. Agreement From RAN1’s perspective, zero is a valid possible option for the mean value for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859. Agreement From RAN1’s perspective, the RAN2 agreement “the error sources are overbounded by a Gaussian distribution” can be confirmed for the error sources listed in Table 6.1.1-2 in TR 38.859 if the error source follows a Gaussian distribution. Agreement The draft LS to RAN2 on error source distributions is endorsed in R1-2304146. Final LS R1-2304147. For response to RAN2 LS on RAN dependency for ranging & sidelink positioning in R1-2302281 Agreement The draft LS response in Appendix of R1-2304092 is endorsed with the following revision: NR_pos_enh2-Core. Final LS in R1-2304152.

## RAN1 #113:

Higher layer parameters for Rel-18 Positioning The latest list of higher layer parameters for Rel-18 positioning at the end of RAN1 #113: R1-2306237 FLS on list of RRC parameters on Rel-18 WI on expanded and improved NR positioning Rapporteur (Intel Corporation) For response to RAN2 LS on the RAT-dependent positioning integrity in R1-2304332 For Q0 (RAN2 working assumption) Agreement From RAN1’s perspective, no concerns are identified for the RAN2 working assumption “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN” For Q1 (Beam related information as error sources) Agreement Regarding whether beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources or not, RAN1 made the following conclusion in RAN1#111, “RAN1 could not reach consensus on whether beam information (NR-TRP -BeamAntennaInfo) and boresight direction of DL PRS (NR-DL -PRS -BeamInfo) are error sources or not for DL-AoD for UE -based positioning integrity mode.”, where the definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857. For Q2 (Application of DNU flags for TRP/UE positioning measurements) Agreement As agreed in RAN1#110b-e, from RAN1 perspective, study of the application of DNU flag for determination of positioning integrity is within the scope of RAN2 discussion. Specification impact(s) of DNU flag(s) can be discussed in RAN2. Agreement The draft LS in R1-2306156 is endorsed. Final LS in R1-2306157. For response to SA2 Reply LS on Sidelink positioning procedure in R1-2304306 Agreement RAN1 provide the following reply to SA2 and cc to RAN2: “From RAN1 perspective, it is feasible to specify relative velocity in Rel-18. RAN1 assumes that there is no RAN1 specification impact.” Agreement The draft LS in R1-2306140 is endorsed. Final LS in R1-2306208. For response to RAN4 Reply LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs in R1-2304316 Agreement Response to RAN4 on the need of additional switching time: - It is RAN1’s view that for UL SRS for positioning Tx frequency hopping, switching time before the first hop and after the last hop need to be defined for the SRS for positioning with Tx frequency hopping. - RAN4 is kindly requested to evaluate the applicable switching time (if any) required ahead of the first hop and after the last hop, considering potential differences (in e.g. SCS, bandwidth, CP) between initial/active UL BWP and UL SRS for positioning Tx frequency hopping. Agreement The draft LS in R1-2306118 is endorsed. Final LS in R1-2306119.

- SL positioning reference signal

## RAN1 #112:

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095 Agreement A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. Agreement Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities. Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M, N) value is possible. - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M, N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

## RAN1 #112bis-e:

Agreement For SL PRS sequence generation, no additional parameters other than the following input parameters are used: slot number, symbol number, and the parameter nID,seqSL-PRSnID,seqSL-PRS. Agreement TDM-based multiplexing in a slot of SL PRS from different UEs is NOT supported for a shared resource pool. Agreement SL PRS resource sets are not defined in Rel-18. Agreement (M, N) patterns with M > N with full staggering are supported. - In the last (M-N) symbols, the SL PRS symbols are repeated with same order of comb offsets as in the first N symbols. Agreement An AGC symbol preceding a SL PRS resource is not considered as part of the SL PRS resource itself. Agreement For the SL PRS open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss. - The same principle as for PSSCH power control is applied for deciding which (i.e., SL, DL, or SL and DL) pathloss to use. - FFS: SL pathloss reference for open-loop power control for SL PRS. Agreement At least for dedicated SL PRS resource pools, in addition to already-agreed (M, N) = (2, 2), (4, 4), fully staggered pattern with (M, N) = (6, 6) is supported. - FFS: Other values of (M, N). - FFS: Applicability to shared resource pools. Agreement - A SL PRS resource refers to a time-frequency resource within a slot of a dedicated SL PRS resource pool that is used for SL PRS transmission. - FFS: for a shared resource pool - Characteristics associated with a SL PRS resource include at least: - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation, - Note: Additional parameters can be included as/when identified. - FFS: other time domain aspects, if any - A SL PRS resource is identified by a SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. NOTE 1: The above does not imply need for signalling/(pre-)configuration of all these parameters Agreement For SL PRS sequence generation, one of the following options is down-selected to define the parameter nID,seqSL-PRS nID,seqSL-PRS: - Option 1:nID,seqSL-PRS nID,seqSL-PRS is a higher layer parameter. - FFS: How the higher layer parameter is obtained, e.g., (pre-)configuration or via LPP/SLPP, etc. - Option 2:nID,seqSL-PRS nID,seqSL-PRS is based on 12 LSB bits CRC of PSCCH associated with the SL PRS. - Option 3: based on a combination of higher layer parameter from a configured ID list and 12 LSB bits of CRC of PSCCH associated with the SL PRS. - FFS: How the higher layer parameter/ID list is determined/obtained, e.g., (pre-)configuration or via LPP/SLPP, etc. Agreement For a dedicated SL PRS resource pool, options for SL pathloss reference for OLPC for SL PRS are (to be down-selected from): - Option 1: SL PRS as pathloss reference - Option 2: PSCCH DMRS as pathloss reference - Option 3: Both Options 1 and 2 - FFS: Selection between Option 1 and Option 2, including (pre-)configuration. Agreement For shared resource pools, a UE does not map SL-PRS and PSSCH DMRS in the same OFDM symbol(s). Agreement For SL pathloss-based OLPC for SL PRS in unicast, filtered RSRP is reported by a receiving UE. Conclusion For a partially staggered SL PRS pattern (M, N), repetition of a partially staggered SL PRS pattern (M, N) in a slot is not supported.

## RAN1 #113:

Agreement For a shared resource pool - A SL PRS resource refers to a time-frequency resource within a slot that is used for SL PRS transmission. - Characteristics associated with a SL PRS resource in a slot of a shared resource pool include at least: - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation - SL PRS freq domain allocation is not used to identify a unique SL PRS resource ID - A SL PRS resource is identified by a combination of SL PRS resource ID and a SL PRS frequency domain allocation. This combination is unique within a slot of a shared resource pool. NOTE 1: The above does not imply need for signalling/(pre-)configuration of all these parameters Conclusion For a dedicated or shared resource pool, at least the following characteristics are NOT included as part of characteristics of a SL PRS resource: - Periodicity, number of instances/repetitions of SL PRS Working assumption - For SL PRS sequence generation, the parameter is defined as below: - is provided by higher layers to a Tx UE - Details on higher layers, including consideration of Tx UE’s own higher layer, are up to RAN2 - The higher layer parameter is provided to an Rx UE via LPP/SLPP. - FFS: If (pre-)configured for a resource pool and use of SL PRS for sensing is supported, is based on 12 LSB bits CRC of PSCCH associated with the SL PRS - Otherwise (i.e., if not provided by higher layers), is based on 12 LSB bits CRC of PSCCH associated with the SL PRS Agreement For a dedicated resource pool, at least the case where SL PRS bandwidth is same as resource pool bandwidth is supported. Agreement For a shared resource pool, SL PRS bandwidth is same as the bandwidth indicated for PSSCH. Agreement For SL PRS in a shared resource pool, the symbols of a SL-PRS resource within a slot are consecutive symbols. Agreement In a dedicated resource pool, a SL PRS resource is immediately preceded by an AGC symbol unless RAN1 explicitly agrees that an AGC symbol is not included for specific cases (if any). Agreement - In a dedicated resource pool, a SL PRS resource is immediately followed by a gap symbol at least: - if the gap symbol corresponds to the last SL symbol of a slot. - Note: the gap can be used at least for Tx/Rx switching - FFS: when TDM of multiple SL PRS resources within a slot is enabled in the dedicated resource pool - FFS: Other cases. - FFS: for SL PRS resource in a shared resource pool. Agreement Comb-based multiplexing of SL PRS resources from different UEs in a slot is NOT supported for shared resource pools. Conclusion TDM-ed SL PRS resources within a slot from a single UE in a dedicated/shared resource pool is not supported in Rel-18. Agreement Multiple (M,N) pairs within a slot in a dedicated resource pool is supported only when the different (M, N) pairs are always multiplexed via TDM to different sets of symbols in a slot. Only a single (M,N) value can be mapped within one TDM duration (i.e. one set of symbols). Agreement For a shared resource pool, SL PRS transmit power is same as that for PSSCH.

- Measurements and reporting for SL positioning

## RAN1 #112:

Agreement SL PRS reference signal received power (SL PRS-RSRP) - is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. Agreement SL PRS reference signal received path power (SL PRS-RSRPP), - is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. Agreement - SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN Agreement Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. Agreement Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3 Agreement Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE). Agreement Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL-PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded Agreement LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). - LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. - No specification impact for how to set this indicator. - From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18. Agreement Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e. Note: this does not imply a different measurement report content for reporting to LMF or to UE. Table 6.2 Collection of measurement report content reporting to LMF; reporting to UE. SL-PRS based Rx-Tx measurement. SL-PRS based RSTD measurement. SL-PRS based RSRP measurement. SL-PRS based RSRPP measurement. SL-PRS based RTOA measurement. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement. etc.

## RAN1 #112bis-e:

Conclusion - For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission, as in legacy specifications. - When the UE selects a cell as the synchronization reference source, SFN0/SFN is used for SL-PRS based RTOA. - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA. - FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting Agreement - SFN/DFN Initialisation Time can be provided by UE or by LMF. - FFS: which UEs can provide initialisation time (note: which may be decided by other WGs) - FFS: further details of the definition of DFN Initialisation Time - For the definition of SL-PRS based RTOA, update the definition of reference timing as: - the reference timing is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0 provided by SFN/DFN Initialisation Time - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS: The timing of SFN0/SFN for RTOA reference timing is determined by advancing DL SFN0/SFN with TA/2 when the UE is in-coverage and in connected state Agreement Support both the case with and without translation of the LCS to GCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. Agreement SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a transmitting UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901. The SL AoA is determined at the receiving UE’s antenna(s) for a SL channel corresponding to the transmitting UE. Agreement Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path. - No specification impact for how to set the additional path measurements - From RAN1 perspective, no performance requirements are expected to be defined for the additional-path measurements in Rel-18. Agreement For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE. FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs) FFS on quality information of anchor UE location information. Agreement Support per ARP based measurement in sidelink positioning. The ARP related information can be reported along with the SL measurement. FFS on details of ARP related information, including whether TEG ID can be reused for such purpose. Agreement For definition of SL-PRS based Rx-Tx measurement, further consider Alt1 and Alt3 until RAN1#113: - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu Agreement SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, where: - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. FFS: whether or not impact due to mobility or synchronization timing change should be considered for SL RSTD Agreement Support higher layer signaling for sidelink positioning measurement report and report triggering. - Up to RAN2 to discuss detailed signaling design. FFS on SCI based report triggering.

## RAN1 #113:

Agreement For definition of SL-PRS based Rx-Tx measurement, the actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement if the UE optionally reports the Tx time information, otherwise use the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. - FFS: details of the Tx time information - FFS: whether additionally the network or LMF can request the UE to report the Tx time information - Note: the value of Rx-Tx measurement is within [-0.5 0.5] ms Agreement For provision of assistance information for sidelink positioning, the ARP location information can be provided to LMF or UE. - FFS: which UEs can receive the location information (note: which may be decided by other WGs) - FFS: details on the location information, e.g., relative location information - Note: different ARPs have their own location information Agreement For per ARP measurement - The ARP ID of an ARP used for reception can be reported along with SL positioning measurement in measurement report.The ARP ID is used to uniquely identify an ARP associated with a UE - FFS: UE can indicate whether different ARPs for Rx and Tx are used for UE Rx-Tx time difference, if the UE optionally reports the Tx time information - FFS: ARP ID of an ARP used for transmission, and details if supported Agreement Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement - Exchange of synchronization information of anchor UEs between a UE and LMF or another UE. - FFS detailed synchronization information. E.g: synchronization source, relative time difference (RTD), synchronization quality information - FFS other mechanisms Agreement Location information of the target UE based on sidelink positioning measurements can be reported at least to LMF. - FFS: on whether quality information of location is included, e.g., uncertainty etc - Up to other WGs to determine whether location information of the target UE can be reported to another UE - Up to RAN2 for signaling details - FFS: whether and how to report per ARP location information. Agreement For definition of SL-PRS based RSRP measurement in frequency range 2 - SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - If receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. For definition of SL-PRS based RSRPP measurement in frequency range 2 - SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - If receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches. Agreement Support reporting parameters needed for converting LCS to GCS in a similar way as in TS 38.455. - The translation of the LCS to GCS uses the set of angles α (bearing angle), β (downtilt angle), γ (slant angle), which can be reported together with the AoA (ϕ) and ZoA (θ) in LCS. Agreement For provision of assistance information for SL AoA measurement, expected SL-AoA value and uncertainty range can be provided to measuring UE. - No specification impact on how to set the uncertainty range - From RAN1 perspective, no performance requirements are expected to be defined for the uncertainty range in Rel-18 Agreement A time stamp associated to each SL positioning measurement within the report includes at least the followings: - SFN, slot number, and optionally including nr-PhysCellID, nr-ARFCN, nr-CellGlobalID - FFS if at least one of nr-PhysCellID, nr-ARFCN, nr-CellGlobalID is always included - Or DFN and slot number - FFS: sidelink synchronization identity FFS: SL-PRS resource ID is included within the measurement report FFS: symbol number Agreement When GNSS is used for synchronization reference, DFN Initialisation Time is defined based on Tref and OffsetDFN defined for sidelink communications (Subclause 5.8.12 in 38.331). Agreement For SL-PRS based RSTD measurement report, the reference UE information is included in measurement reporting. - FFS: details of the reference UE information

- Resource allocation for SL positioning reference signal

## RAN1 #112:

Agreement - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool. Agreement For a dedicated resource pool for positioning: - The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214). - FFS: additional slots that can be used for SL PRS is not precluded - Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH). Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further). Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

## RAN1 #112bis-e:

Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details Agreement For a dedicated resource pool for positioning: - No additional slots are needed to be supported Agreement For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier. - A UE can be (pre-)configured with one or more dedicated SL resource pools. - A UE can be (pre-)configured with one or more shared SL resource pools. Agreement Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool. - Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation. Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool Agreement With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication - FFS: Details including a new or existing 2nd stage SCI Agreement In a shared resource pool: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. A.1: Only TDMing is supported - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. B.1: Only TDMing is supported - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting): - Alt. C.1: 2nd SCI only - Alt. C.2: 2nd SCI and SL-SCH - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH” - FFS: Handling of PT-RS and SL-PRS Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH Agreement For the scheme 2 sensing-based resource allocation: - Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Note: This means that Rel-17 partial sensing is not considered a starting point for the design Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Modification 6: For the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - Option 3: Provided by higher layers with values TBD - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%): - Options TBD - Modification 8: For the pre-emption of the reserved resources: - Options TBD - Note 1: Other potential modifications and/or other options within each modification are not precluded - Note 2: Multiple options for each potential modification may be supported Agreement In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

## RAN1 #113:

Agreement For a dedicated resource pool for SL positioning, SL-PRS cannot be transmitted in a slot without associated PSCCH. Agreement PSSCH is not included in dedicated resource pool for SL positioning. Agreement With regards to the SCI signaling in a shared resource pool, - Support a new format for 2nd stage SCI. - FFS how to indicate the new 2nd stage SCI format - FFS: If a 2nd stage SCI indicates both SL-PRS and SL-SCH, the cast type, destination ID, source ID are shared. Agreement In shared resource pools, - With regards to PSCCH and SL-PRS multiplexing, support Alt. B.1. from previous agreement (i.e., Only TDMing is supported) Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - With regards to PSSCH and SL-PRS multiplexing, only TDMing is supported for the already agreed comb sizes 1, 2, 4 Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - The PSSCH is used for 2nd SCI and SL-SCH - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case. Agreement For the shared resource pool, reuse the existing IUC signaling of both Scheme 1 and Scheme 2. - SL-PRS transmissions are treated as any other legacy transmission for SL communication when considering IUC information exchanges. Conclusion For Rel-18 sidelink positioning: - For the dedicated resource pool, IUC signalling is not supported - Do not support that a UE can reserve a SL-PRS resource for the transmission of another UE Conclusion Do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in Release 18. Agreement PSFCH is not included in dedicated resource pool for SL positioning. Agreement In the dedicated resource pool, - with regards to the SL-PRS time-domain resource allocation within the resource pool support a - SL-PRS-resource-based allocation - SCI for SL-PRS should at least indicate the following values: - Source ID - Destination ID - Resource reservation period - SL-PRS Priority - Cast type - With regards to the SL-PRS configuration and/or SL-PRS time assignment information, select one alternative at RAN1#114: - Alt. 3.1: support a one-to-one mapping relationship between a PSCCH resource and an associated SL-PRS resource in the same slot. - Note: In this case, there is no need of an explicit signaling of which SL PRS resource for the same slot - Note: Same number of PSCCH resource(s) and SL-PRS resource(s) - Alt. 3.2: explicit signaling of SL PRS resource in the same slot - Alt. 3.3: support a mapping relationship between a PSCCH resource and one or more associated SL-PRS resource(s) in the same slot and explicit signaling of SL PRS resource - Only a one-to-one mapping is used between a PSCCH resource and an associated SL-PRS resource in the same slot if explicit signalling is not used - Note: with a one-to-one mapping, some SL-PRS resources might not be mapped - FFS: details, including (pre)configuration - FFS: Whether and how to indicate SCI resource(s) or SL-PRS resource (s) for a future slot - FFS: Additional information, e.g. SL-PRS request, Positioning Session ID, number of resource reservation periods Agreement In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA for shared and dedicated resource pools. - With regards to the lower-layer signalling, support SCI associated with SL-PRS transmission - FFS: whether this is enabled by (pre)configuration - FFS: to support also SL-PRS Agreement For Scheme 2, in a dedicated resource pool, - Multiple L1 SL-PRS priority are allowed in a resource pool - A SL PRS resource within the resource selection window is used as a candidate resource - with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point. - with regards to the resource (re)-selection procedure - support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point. Agreement In dynamic grant type resource allocation in scheme 1, - For shared resource pool, DCI format 3_0 is being used as a starting point, down-select between the two alternatives below: - Alt. 1: Indication SL-PRS specific information is explicitly included in DCI - FFS: Which SL-PRS specific information - Alt. 2: Indication SL-PRS specific information is not explicitly included in DCI - FFS: Dedicated resource pool Agreement In Scheme 2, congestion control can restrict the range of parameters for SL PRS configuration per resource pool by CBR and priority. Consider further the following parameter(s): - Option 1: SL PRS transmission power - Option 2: Periodicity of SL PRS - Option 3: Number of occupied subchannels of SL-PRS (for shared resource pool) - Option 4: Number of SL PRS resources in a slot - Option 5: comb-size of a SL PRS resource in a slot - Option 7: Number of OFDM symbols of a SL PRS resource in a slot - Option 8: Number of SL PRS (re-)transmissions - FFS: Other options are not precluded Agreement In a dedicated resource pool, with regards to the PSCCH, reuse the PSCCH channel structure of SL communications, at least with regards to the following aspects: - The first PSCCH symbol is mapped to the 2nd symbol available for SL transmissions in a slot - Note: 1st symbol available for SL transmissions in a slot is for PSCCH AGC similar to legacy - PSCCH DM-RS in the slot is being reused from legacy - The number of PSCCH symbol(s) is (pre-)configured to (down-select at RAN1#114): - Alt. 1: 2 or 3 symbols (same as legacy) - Alt. 3: 1, or 2 or 3 symbols - The number of PRBs is (pre-)configured using the legacy values - FFS: reconsider if 1-symbol PSCCH is supported

- NR DL and UL carrier phase positioning

## RAN1 #112:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

## RAN1 #112bis-e:

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements. Agreement To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

## RAN1 #113:

Agreement Support the following definition of the reference point of the UE/TRP carrier phase measurements: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for both frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for both frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for measurement reporting. Agreement Adopt the following modifications on the agreements made in RAN1#112bis-e: To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource sets occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. Agreement If a UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TRP for RSCPD is the same as the reference TRP reported for RSTD. - The target and the reference TRP are in the same PFL Agreement From RAN1’s perspective, carrier phase positioning for UE in RRC_IDLE state is supported for UE-based and UE-assisted positioning in Rel-18. - Note: No additional specification work is expected specifically related to carrier phase positioning for UE in RRC_IDLE state in RAN1. Conclusion From RAN1’s perspective, carrier phase positioning for UE in RRC_CONNECTED state without measurement gap is not supported in Rel-18. Working assumption To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows

- LPHAP (Low Power High Accuracy Positioning)

## RAN1 #112:

For response to SA2 LS in R1-2301914 Agreement The draft LS in R1-2301991 is endorsed. Final LS in R1-2302074. Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement - For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell Agreement - From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold Agreement - For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH Agreement - For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration. Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration.

## RAN1 #112bis-e:

Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, sequenceID in SRS for positioning configuration is commonly configured across cells within the validity area. Working assumption For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the following options: - Option 1: Pathloss RS is absent in the configuration. - Option 2: Pathloss RS is provided in the configuration. Working assumption For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support that spatial relation information can be absent or present in the configuration. Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state: - When the spatial relation information is absent in the configuration, the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources; - When the spatial relation information is provided in the configuration, it is applicable across the cells within the SRS positioning validity area. Further study the configuration details. - FFS: spatial relation information validity criteria, and whether/how to determine UE fallback behavior if validity criteria for spatial relation of the configured RS is not met. Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS. Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support one or multiple of the following alternatives on how to configure pathloss RS (down-selection to be made in RAN1#113 meeting): - Alt. 2-1: Reuse the configuration of pathloss RS in Rel-17; - Reuse the validity criteria of OPLC for SRS transmissions by RRC_INACTIVE UEs in Rel-17, i.e. when the pathloss RS cannot be accurately measured. When the validity criteria of OPLC fail, pathloss is calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB. - Alt. 2-4: Configure a list of candidate pathloss RSs or candidate cells per SRS for positioning resource set. The UE transmits SRS resources in a SRS resource set using a pathloss RS determined from the candidate pathloss RSs based on its DL measurements on the candidate pathloss RSs; - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail. - Alt. 2-5: Reuse the configuration of pathloss RS in Rel-17. The UE only transmits SRS resources in a SRS resource set associated with pathloss RS that can be accurately measured. - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail. - Alt. 2-6: Reuse the configuration of pathloss RS in Rel-17; - A cell-agnostic DL RS can be the pathloss RS in the validity area and the cell-agnostic DL RS itself. - Note: UE power consumption should be considered

## RAN1 #113:

Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support: - Alt. 2-1 (modified): Reuse the configuration of pathloss RS in Rel-17; - FFS: A CD SSB or non-CD SSB can be configured as pathloss RS - If the UE determines that the pathloss RS cannot be accurately measured, pathloss may be calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB. Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, when the spatial relation information is provided in the configuration, support: - Alt. 1-1: Reuse the configuration of spatial relation information in Rel-17. - When the UE determines that the configured RS for the spatial relation information cannot be accurately measured, the UE suspends the transmission of the SRS for positioning resource. Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: - The DL reference timing follows the DL timing of current camping cell. - By default, UE maintains the TA from the last serving cell. - UE can adjust its UL timing according to the change in DL reference timing. - If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens. - Send LS to RAN4 asking about feasibility and necessary conditions (e.g. whether the above behaviour applies when the DL reference timing difference between the last camping cell and current camping cell exceeds a threshold and how UE adjusts it, or additional RRM procedure to obtain the timing difference). Agreement The draft LS in R1-2306247 is endorsed. Final LS in R1-2306248.

- Bandwidth aggregation for positioning measurements

## RAN1 #112:

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis. Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked - SRS resources are per BWP per carrier configuration - FFS whether the link is per SRS resource set basis or per SRS resource basis. Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details Agreement From RAN1 perspective, support UE performs PRS measurement across multiple aggregated PFLs in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE state. Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs Agreement At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation - Support single MAC CE activating positioning SRS resource sets across the linked carriers - FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused - FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way Agreement Study potential power control enhancement of simultaneous transmission of SRS for SRS bandwidth aggregation especially in the case when the total uplink transmission power across multiple carriers exceeds P_c,max Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e., SRS outside BWP and across carriers

## RAN1 #112bis-e:

Agreement Study whether single TRP Tx TEG ID or UE Rx TEG ID is applied across PRSs in aggregated PFLs for TEG information reporting, i.e. single TEG ID is reported across the aggregated PRS resources for TRP Tx TEG association reporting, or for UE Rx TEG ID reporting in the measurement reporting Agreement For PRS bandwidth aggregation across PFLs, select one of the following options in RAN1#113 - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. - Option 3: Per TRP basis and per PRS resource basis. - For each TRP, support new signaling to indicate which PRS resource(s) across PFLs are linked. - For the non-linked PRS resources, no aggregation is assumed even if the conditions are satisfied. Conclusion The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same. - Note: multiple PRS/SRS resources which can be used to determine the start of subframe can be from multiple intra-band continuous carriers, - Note: no RAN1 spec impact - Send an LS to RAN4 to confirm RAN1’s understanding Agreement Draft LS to RAN4 is endorsed in R1-2304081. Final LS in R1-2304082. Agreement Support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. - FFS the details Agreement For PRS resources aggregated across PFLs for DL-TDOA and multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 DL PRS measurement of single PFL with the necessary update. - FFS: In a measurement report element, single RSRP or single RSRPP is reported - In a measurement report element, PFL aggregation indication is supported to indicate whether/which measurement is aggregated - Support new signaling in location information request message to indicate UE whether to perform joint measurement across aggregated PFLs - Single RSTD reference in assistance data and measurement report is used for PRS bandwidth aggregation measurement - FFS RSTD reference is aggregated or not Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3. Agreement For SRS bandwidth aggregation between SRS in two or three carriers, the aggregated SRS resources are of the same SRS resource-Type. Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA. Agreement Support the same power prioritization between the aggregated carriers in the case when total UE transmit power in a transmission occasion I exceeds Pcmaxi. - The UE allocates power to the multiple SRS resources in the transmission occasion i of the aggregated carriers such that the UE’s transmit power in each transmitted resource element is equal. - FFS further details, e.g. power scaling between aggregated carriers Agreement Introduce new UE capability(-ies) to support PRS bandwidth aggregation measurement - FFS the details include the processing capability (N, T), the maximum number of PRS resources that can be process in a slots over the aggregation - FFS the details on the PFL bandwidth combinations, including maximum number of PFLs, the total aggregated bandwidth, etc. - This is applicable for DL-TDOA and Multi-RTT positioning methods Agreement Study whether single UE Tx TEG ID or TRP Rx TEG ID is applied across SRSs in aggregated carriers for TEG information reporting, i.e. single UE Tx TEG ID is reported across the aggregated SRS resources for UE Tx TEG association reporting, or for TRP Rx TEG ID reporting in measurement reporting Agreement Positioning SRS bandwidth aggregation is supported for UEs in RRC_CONNECTED. Positioning SRS bandwidth aggregation is supported for UEs in RRC_INACTIVE state. - For the details, Rel-17 positioning SRS configuration for UE in RRC_INACTIVE state outside initial UL BWP can be the starting point Agreement From RAN1 perspective, MG-based bandwidth aggregation measurement is supported. Decide whether PPW is supported for PRS bandwidth aggregation measurement in RAN1#113 meeting. - FFS the details for PPW if supported Agreement For the case when PRS in one of aggregated PFL is dropped, e.g. because of collision with SSB, select one of the following solutions for LMF based positioning - Alt. 1: Drop positioning measurement in all aggregated PFLs in the same symbol(s) - Alt. 2: Still perform positioning measurement based on the remaining PRSs in other PFL(s) - FFS the details and the difference between MG and PPW if PPW is supported - Note: Up to RAN4 to discuss impact on requirements, if any, for such cases Agreement For SRS bandwidth aggregation across two or three carriers, select one of the following options in RAN1#113 meeting - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied. - Option 3: Per SRS resource basis. - Support new signaling to indicate which SRS resources across carriers are linked. - For the non-linked SRS resources, no aggregation is assumed even if the conditions are satisfied Agreement For the SRS resources across aggregated carriers for UL-TDOA and Multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 SRS measurement of single carrier with the necessary update - FFS: Single RSRP or RSRPP is reported for the SRS resources across aggregated carriers - SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which measurement is aggregated Agreement For positioning SRS aggregation across CCs, if SRS in one of aggregated carriers is dropped in a symbol, select one of the following two options: - Alt. 1: Stop SRS transmission in all aggregated carriers in the same symbol - Alt. 2: SRS is still transmitted in other carriers in the same symbol - FFS: The UE may not be expected to maintain phase continuity across the remaining carriers - FFS the applicable scenario, e.g. the positioning SRS collides with another higher priority SRS or others Agreement For SRS bandwidth aggregation between SRS in two or three carriers, decide whether one or more of the following are needed for the aggregated SRS resources in RAN1#113 meeting - The same timing advance offset or the same TAG - The same periodicityAndOffset, and slotOffset - The same number of SRS resource sets and/or the same number of SRS resources per set - The configuration of same pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier) - FFS the details, e.g. UE determines the transmit power for SRS transmission in a reference carrier and applies the same Tx PSD for SRS transmission in other carriers, or configure a common parameter set for the aggregated carriers - The same antenna port from RAN1 specification perspective - Note: this is to achieve phase continuity between carriers - UE is expected to be configured with SRS resources that maintain a per-symbol uniformly spaced SRS pattern across aggregated bandwidths - Others if any Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, decide whether one or more of the following are needed for the aggregated PRS resources from a TRP in RAN1#113 meeting: - The same antenna port from RAN1 perspective - Note: this is to achieve phase continuity between PFLs - The same periodicity and slot offset - The same muting pattern - The same number of PRS resource sets and/or resources per set for a TRP - The same NR-DL-PRS-SFN0-Offset value - UE is expected to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths - FFS: a per-symbol uniformly spaced PRS pattern across aggregated bandwidths does not preclude dropping some REs in the guardband between two PFLs - Others if any

## RAN1 #113:

Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, the following are needed for the aggregated PRS resources for a TRP: - The same periodicity and slot offset - The same muting pattern - The same NR-DL-PRS-SFN0-Offset value - UE expects to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths in frequency domain (Note: It does not preclude dropping some REs in the guardband between two PFLs). - FFS same antenna port from RAN1 perspective Agreement For PRS bandwidth aggregation across PFLs, support - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement For PRS bandwidth aggregation across PFLs, in a measurement report element, support - Single RSRP or single RSRPP - FFS: the single RSRP/RSRPP is based on aggregated PRS resources across aggregated PFLs - The aggregated reference RSTD - The used PRS resource set IDs for the aggregated measurement which are shared for RSRP/RSRPP and/or timing measurement results Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed. Agreement The draft LS in R1-2306215 is endorsed. Final LS in R1-2306216. Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: - A request to indicate UE which two or three PFLs to be used for performing joint measurement - A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements - Support at least the values of k={-1,-2} - FFS other values e.g. -3, -4, -5, -6 - Send RAN4 an LS to confirm the feasibility Conclusion For PRS bandwidth aggregation, PPW is not supported in Rel-18. Agreement When the UE receives a request to perform aggregated measurements, - TRP(s) that include PRS aggregation have higher priority than the TRPs that do not include PRS aggregation - If 2 or more TRPs include linked resources, then their priority follows the legacy priority, i.e., sorted in the configuration according to priority - If a PRS resource set is linked for aggregation, then it has higher priority compared to the PRS resource set not linked for aggregation. - If both sets in a PFL are linked for aggregation, then their priority follows the legacy priority, i.e., sorted in the configuration according to priority Agreement For SRS bandwidth aggregation between SRS in two or three carriers, the following is needed for the aggregated SRS resources - The same periodicityAndOffset, and slotOffset - The configuration of pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier) - The same configuration of Po and alpha. - Note: UE may either perform pathloss RS measurement across CCs and form a single path loss value to apply across CCs or perform pathloss RS measurement in a single CC and apply across CCs Agreement For SRS bandwidth aggregation across two or three carriers, support - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement To support intra-band contiguous SRS bandwidth aggregation for UE in RRC_INACTIVE state, frequency information (e.g. point A, offset to carrier) of one or two additional carriers with respective SRS configurations should be provided to the UE, where the newly introduced carrier(s) and the carrier of the initial BWP should be intra-band contiguous carriers. Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility. Agreement Draft LS in R1-2306213 is endorsed with the following change: ACTION: RAN1 respectfully asks RAN2 to check the feasibility of the working assumption and inform RAN1 of RAN2’s conclusion on the feasibility. Final LS agreed in R1-2306214. Agreement For positioning SRS aggregation transmission in RRC_INACTIVE state, reuse Rel-17 prioritization rule of SRS outside initial BWP, i.e. SRS is dropped in the symbol(s) of all aggregated carriers where collision occurs. Agreement For a carrier including positioning SRS for aggregation, - Positioning SRS can be transmitted only when the carrier is activated - This is also applicable for the carrier only including positioning SRS for aggregation Agreement With regard to support of aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, at least the existing Rel-17 DCI framework (i.e. use multiple DCIs schedule SRSs in multiple carriers) can be reused - FFS: whether Rel-18 DCI framework for multi-cell PDSCH/PUSCH scheduling with a single DCI (i.e. single DCI schedules SRSs in multiple carriers) can also be reused with or without specification work in RAN1. Agreement For SRS bandwidth aggregation across carriers, support - Single RSRP or RSRPP is reported - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers - The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results

- Positioning for RedCap UEs

## RAN1 #112:

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported. Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Conclusion The scope for RedCap positioning includes FR1 and FR2. Agreement For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance. - Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same. Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources Agreement The draft LS in R1-2302126 is endorsed with the addition of the two agreements above. Final LS in R1-2302127.

## RAN1 #112bis-e:

Agreement For RedCap UEs, SRS for positioning Tx frequency hopping is configured (select one alternative): - Alt 1: within one SRS for positioning resource - Alt 2: across resources, within one SRS for positioning resource set - Alt 3: across resource sets, with all resources in a set corresponding to the same hop sub-bandwidth Conclusion For the positioning of redcap UEs, for the DL PRS reception and UL SRS transmission, the maximum hopping bandwidth for a single hop is 20MHz for FR1 and 100MHz with FR2. Agreement For RedCap UEs, SRS for positioning Tx frequency hopping is configured within one SRS for positioning resource. Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One [or more] measurements where each measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together Agreement For UL SRS Tx hopping, the frequency hopping pattern is configured with overlapping or non-overlapping hops. - FFS: exact patterns to be supported - FFS: whether the overlapping hops may or may not be adjacent in the time domain - Note: RAN1 assumes that no additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops. Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, study whether to support one or both of the following options, according to UE capabilities: - Option 1: UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels - FFS: details on the collision rules

## RAN1 #113:

Agreement The previous agreement is updated as follows: Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One measurement where a measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together Agreement From RAN1 perspective, for DL PRS Rx hopping, a single instance of a measurement gap is used for receiving all the hops for DL PRS with Rx frequency hopping. - Note: this does not assume that the reported measurement has to be based on a single instance of a measurement gap - Send an LS to RAN4 to confirm RAN1’s understanding, and if needed ensure that the measurement gap has the proper duration. Agreement The draft LS in R1-2306226 is endorsed with the following changes: - In the Rel-18 WI Expanded and Improved NR Positioning, in the agenda item “Positioning for redcap UEs”, RAN1 agreed the use of a single instance of a measurement gap for receiving all the hops for DL PRS with Rx frequency hopping: - RAN1 respectfully asks RAN4 Final LS in R1-2306227. Agreement SRS Tx Frequency hopping is supported for both RRC_CONNECTED and RRC_INACTIVE state. Agreement For the SRS Tx hopping pattern configuration support at least the staircase pattern, including a wrapped staircase pattern. - Support configuring the starting PRB of the first hop - FFS: details of signalling of PRB overlap across consecutive hops and bandwidth of each hop Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to [receive/]transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without UL time window (i.e. option 1) - FFS: details on the collision rules - Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group.

# References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning,” Intel Corporation, March 2023

- TR 38.859 v18.0.0	Study on expanded and improved NR positioning

- R1-2301218	Work Plan for Rel-18 WI on Expanded and Improved NR Positioning	Intel Corporation, CATT, Ericsson

- R1-2302047	Discussion of SA2 LS on PRU Procedures	Moderator (Qualcomm Incorporated)

- R1-2302106	Discussion of SA2 LS on PRU Procedures	Moderator (Qualcomm Incorporated)

- R1-2301875	FL summary #1 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2301876	FL summary #2 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2302095	FL summary #3 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2302096	FL summary #4 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2301950	Summary #1 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2301951	Summary #2 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2301952	Summary #3 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2302004	Moderator Summary #1 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2302045	Moderator Summary #2 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2302094	Moderator Summary #3 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2302162	Moderator Summary #4 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2301828	FL Summary #1 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2301829	FL Summary #2 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2301830	FL Summary #3 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2301967	Summary #1 for low power high accuracy positioning	Moderator (CMCC)

- R1-2301968	Summary #2 for low power high accuracy positioning	Moderator (CMCC)

- R1-2302189	Summary #3 for low power high accuracy positioning	Moderator (CMCC)

- R1-2300809	Summary #1 for BW aggregation positioning	Moderator (ZTE)

- R1-2300810	Summary #2 for BW aggregation positioning	Moderator (ZTE)

- R1-2302091	Summary #3 for BW aggregation positioning	Moderator (ZTE)

- R1-2301984	Feature Lead Summary #1 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2301985	Feature Lead Summary #2 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2301986	Feature Lead Summary #3 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2302210	Feature Lead Summary #4 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2302146	LS Reply on PRU Procedures	RAN1, Qualcomm	LS out	Rel-18	NR_pos_enh2-Core, 5G_eLCS_Ph3	To: SA2	Cc: RAN2, RAN3

- R1-2302074	LS reply on the requirement for LPHAP	RAN1, Huawei	LS out	Rel-18	5G_eLCS_Ph3, NR_pos_enh2-Core	To: SA2

- R1-2302127	LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs	RAN1, Ericsson	LS out	Rel-18	NR_pos_enh2-Core	To: RAN4

- R1-2304100	Moderator summary on LS reply for error source distributions (R1-2302282)	Moderator (InterDigital, Inc.)

- R1-2304092	Moderator summary on discussion of RAN2 LS in R1-2302281 on RAN dependency for Ranging & Sidelink Positioning	Moderator (xiaomi)

- R1-2303963	FL summary #1 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2303964	FL summary #2 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2303965	FL summary #3 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2304242	FL summary #4 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2303956	Summary #1 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2303957	Summary #2 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2303958	Summary #3 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2304240	Summary #4 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2303978	Moderator Summary #1 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2304067	Moderator Summary #2 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2304140	Moderator Summary #3 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2304233	Moderator Summary #4 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2303888	FL Summary #1 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2303889	FL Summary #2 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2303890	FL Summary #3 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2304000	Summary #1 for low power high accuracy positioning	Moderator (CMCC)

- R1-2304001	Summary #2 for low power high accuracy positioning	Moderator (CMCC)

- R1-2304002	Summary #3 for low power high accuracy positioning	Moderator (CMCC)

- R1-2303285	Summary #1 for BW aggregation positioning	Moderator (ZTE)

- R1-2303286	Summary #2 for BW aggregation positioning	Moderator (ZTE)

- R1-2304083	Summary #3 for BW aggregation positioning	Moderator (ZTE)

- R1-2304004	Feature Lead Summary #1 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2304005	Feature Lead Summary #2 for Positioning for RedCap Ues	Moderator (Ericsson)

- R1-2304006	Feature Lead Summary #3 for Positioning for RedCap Ues	Moderator (Ericsson)

- R1-2304007	Feature Lead Summary #4 for Positioning for RedCap Ues	Moderator (Ericsson)

- R1-2302146	LS Reply on PRU Procedures	RAN1, Qualcomm	LS out	Rel-18	NR_pos_enh2-Core, 5G_eLCS_Ph3	To: SA2	Cc: RAN2, RAN3

- R1-2302146	LS Reply on PRU Procedures	RAN1, Qualcomm	LS out	Rel-18	NR_pos_enh2-Core, 5G_eLCS_Ph3	To: SA2	Cc: RAN2, RAN3

- R1-2302146	LS Reply on PRU Procedures	RAN1, Qualcomm	LS out	Rel-18	NR_pos_enh2-Core, 5G_eLCS_Ph3	To: SA2	Cc: RAN2, RAN3

- R1-2304147	Reply LS to RAN2 on error source distributions	RAN1, InterDigital, Inc.	LS out	Rel-18	NR_pos_enh2-Core	To:RAN2

- R1-2304152	Reply LS on RAN dependency for Ranging & Sidelink Positioning	RAN1, xiaomi	LS out	Rel-18	NR_pos_enh2	To:RAN2	Cc:SA2

- R1-2304082	LS on measurement definitions for positioning with bandwidth aggregation	RAN1, ZTE	LS out	Rel-18	NR_pos_enh2-Core	To: RAN4

- R1-2306237	FLS on list of RRC parameters on Rel-18 WI on expanded and improved NR positioning	Rapporteur (Intel Corporation)

- R1-2306109	Moderator summary on LS reply on the RAT-dependent positioning integrity (R1-2304332)	Moderator (InterDigital, Inc.)

- R1-2306157	LS reply on the RAT-dependent positioning integrity	RAN1, InterDigital, Inc.	LS out	Rel-18	NR_pos_enh2-Core	To: RAN2

- R1-2306082	Moderator summary on discussion of SA2 LS in R1-2304306 on SL positioning procedure	Moderator (Xiaomi)

- R1-2306208 Reply LS on Sidelink positioning procedure	RAN1, Xiaomi	LS out	Rel-18	Ranging_SL	To: SA2		Cc: RAN2

- R1-2306120	Summary of discussion on LS for DL PRS and UL SRS frequency hopping Switching time	Moderator (Ericsson)

- Reply LS for DL PRS and UL SRS frequency hopping Switching time	RAN1, Ericsson	LS out	Rel-18	NR_pos_enh2-Core	To: RAN4

- R1-2305995	FL summary #1 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2305996	FL summary #2 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2305997	FL summary #3 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2306236	FL summary #4 on SL positioning reference signal	Moderator (Intel Corporation)

- R1-2306097	Summary #1 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2306098	Summary #2 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2306099	Summary #3 on Measurements and reporting for SL positioning	Moderator (vivo)

- R1-2306067	Moderator Summary #1 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2306124	Moderator Summary #2 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2306171	Moderator Summary #3 on resource allocation for SL PRS	Moderator (Qualcomm)

- R1-2305970	FL Summary #1 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2305971	FL Summary #2 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2305972	FL Summary #3 for NR DL and UL carrier phase positioning	Moderator (CATT)

- R1-2306014	Summary #1 for low power high accuracy positioning	Moderator (CMCC)

- R1-2306015	Summary #2 for low power high accuracy positioning	Moderator (CMCC)

- R1-2306016	Summary #3 for low power high accuracy positioning	Moderator (CMCC)

- R1-2304935	Summary #1 for BW aggregation positioning	Moderator (ZTE)

- R1-2304936	Summary #2 for BW aggregation positioning	Moderator (ZTE)

- R1-2304937	Summary #3 for BW aggregation positioning	Moderator (ZTE)

- R1-2306216	LS to RAN4 on SRS and PRS bandwidth aggregation for positioning	RAN1, ZTE	LS out	Rel-18	NR_pos_enh2	To: RAN4

- R1-2306214	LS to RAN2 on SRS bandwidth aggregation for positioning	RAN1, ZTE	LS out	Rel-18	NR_pos_enh2	To: RAN2

- R1-2306114	Feature Lead summary #1 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2306115	Feature Lead summary #2 for Positioning for RedCap UEs	Moderator (Ericsson)

- R1-2306116	Feature Lead summary #3 for Positioning for RedCap UEs	Moderator (Ericsson)