3GPP TSG RAN WG1 #113	                          			  		R1-2306239

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.12.3

Source:	Moderator (InterDigital, Inc.)

Title:	Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and Decision

# Introduction

RAN approved a WI on further coverage enhancements for NR [1]. The WI includes the following objective:

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

The WI also includes corresponding justification point:

- DFT-S-OFDM waveform is beneficial for UL coverage limited scenario because of its lower PAPR compared with CP-OFDM waveform. Currently, UL waveform is configured via RRC and this limitation imposes a large barrier to switch over to DFT-S-OFDM waveform for cell-edge UEs practically.

This contribution summarizes contributions submitted in RAN1#112 under AI 9.14.3 – Dynamic switching between DFT-S-OFDM and CP-OFDM.

Here is the color code used in this summary:

- FL observations

- FL proposals

- Questions for the inputs from companies

- FL summary based on the companies’ input

- RAN1 agreements

A [LP]/[MP]/[HP] tag indicates envisioned priority of each issue in this meeting.

# Contact information

Please input the contact information for each company below:

| Company | Name | Email |
| --- | --- | --- |
| InterDigital | Paul Marinier | paul.marinier at interdigital.com |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001 at jp.panasonic.com |
| Nokia/NSB | Quang Nhan | nhat-quang.nhan@nokia.com |
| Nokia/NSB | Karim Kasan | karim.kasan@nokia.com |
| LG | Duck Hyun Bae | Duckhyun.bae@lge.com |
| Ericsson | Ling Su | Ling.a.su@ericsson.com |
| CMCC | Yongchang Liu | liuyongchang@chinamobile.com |
| Samsung | Carmela Cozzo | carmela.c@samsung.com |
| Sharp | Toshi Nogami | nogami.toshizoh@sharp.co.jp |
| Sharp | Hiro Takahashi | takahashi.hiroki@sharp.co.jp |
| China Telecom | Hang Yin | yinh6@chinatelecom.cn |
| ZTE | Junfeng Zhang | Zhang.junfeng@zte.com.cn |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| Sony | Sam Atungsiri | Sam.atungsiri@sony.com |
| OPPO | Zhisong Zuo | zuozhisong@oppo.com |

# Collection of agreements in RAN1#113

Agreement

Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

Agreement

For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options:

- Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH.

- Details FFS.

- Note: Any MAC CE related decision is up to RAN2

- Option 4: No enhancement.

# Proposals

## Proposals for 1st Online session

FL proposal 1-2: Dynamic waveform switching is supported for a UE configured with multiple UL carriers.

FL proposal 1-3: Dynamic waveform switching is supported for PUSCH(s) scheduled by DCI format 0_3.

FL proposal 2-1: Value “0” of dynamic waveform switching indicator field maps to transform precoding enabled. Value “1” of dynamic waveform switching indicator field maps to transform precoding disabled.

FL proposal 4-1r1: Presence Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

FL proposal 3-1 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. [14 companies] - Details FFS. - Note: reporting PH information for both waveforms is not precluded. - Note: additional trigger for PH for reference PUSCH is not precluded. - Option 2: New trigger of power headroom report based on waveform switching event. [3 companies] - Details FFS. - Option 3: Both Option 1 and Option 2. [2 companies] - Details FFS. - Option 4: No enhancement. [5 companies]

## Proposals for 2nd Online session

FL proposal 4-1r1: Presence Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

FL proposal 3-1 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. [14 companies] - Details FFS. - Note: reporting PH information for both waveforms is not precluded. - Note: additional trigger for PH for reference PUSCH is not precluded. - Option 2: New trigger of power headroom report based on waveform switching event. [3 companies] - Details FFS. - Option 3: Both Option 1 and Option 2. [2 companies] - Details FFS. - Option 4: No enhancement. [5 companies]

FL proposal 1-3: Dynamic waveform switching is supported for PUSCH(s) scheduled by DCI format 0_3.

FL proposal 2-1: Value “0” of dynamic waveform switching indicator field maps to transform precoding enabled. Value “1” of dynamic waveform switching indicator field maps to transform precoding disabled.

## Proposals for 3rd Online session

FL proposal 3-2r3 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for an assumed reference PUSCH using target waveform different from waveform of actual PUSCH. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported with same parameters as actual PUSCH, power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - RAN1 assumes the following RAN2 impacts and design aspects to be decided by RAN2: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Whether power headroom information for assumed PUSCH and legacy Type 1 PH are reported together or not. - Send LS to RAN2 (cc RAN4) - Option 4: No enhancement.

## Proposals for 4th Online session

FL proposal 3-2r5 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select one from the following options: - Option 1: Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - FFS: If actual PUSCH transmission is not scheduled by a DCI with DWS field, power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported with same for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - Power headroom information for assumed PUSCH is supported at least if multiplePHR=False and twoPHRmode is not enabled. FFS: other cases. - For a serving cell with power headroom information of assumed PUSCH to be reported, support only one of alternatives below: - Alt 1: Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - Alt 2: Only power headroom information for assumed PUSCH is reported. - RAN1 assumes the following RAN2 impacts and design aspects to be decided by RAN2: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Whether power headroom information for assumed PUSCH and legacy Type 1 PH are reported together or not. - Send LS to RAN2 (cc RAN4) if Option 1 is selected. - Option 4: No enhancement.

# Topic #1: Applicability of dynamic waveform switching

A set of issues is related to the type of transmission concerned by dynamic waveform switching.

The following cases are considered separately:

- Msg3 PUSCH scheduled by RAR or by TC-RNTI with DCI format 0_0

- UL CA and PUSCH scheduled by DCI format 0_3

## [LP] Issue #1-1: Applicability to msg3 PUSCH

Summary of company views from contributions submitted to RAN1#113

Applicability to PUSCH scheduled by RAR and/or by TC-RNTI with DCI format 0_0

- Indication from RAR/DCI format 0_0 by TC-RNTI: Sony [14], Nokia [16], Oppo [20]

[Justification]

  - Network does not always configure DFT-S-OFDM for msg3 [16]

  - More important for retransmission than initial transmission [16]

  - No issue related to partitioning in case of msg3 retransmission (?) [16]

  - UEs may have different configurations and geometries [20]

[Solution]

  - Signal by aperiodic CSI report bit of the RAR for UE in idle [14]

  - Signal by MAC CE or PDCCH order for RACH for UE in connected or inactive [14]

  - For msg3 retransmission, solution that does not impact DCI format 0_0 [16]

  - Redefine resource allocation [20]

- Linked to use of msg1 repetition: ZTE [4], Panasonic [10], Sony [14], Nokia [16], Oppo [20]

  - UE uses DFT-S-OFDM for msg3 tx/re-tx if multiple PRACH is applied and PUSCH repetition is scheduled [4][20]

  - Does not require additional PRACH resource for waveform selection [10]

  - Requires that PRACH repetition capable UE always support msg3 repetition (?) [10]

  - Repetitions may be sufficient for PRACH but not for msg3 [16]

  - Network can know if a UE is in coverage shortage or not by receiving msg1 [16]

- Linked to use of msg3 repetition: ZTE [4], Intel [8]

- No indication from DCI format 0_0/RAR: ZTE [4], InterDigital [7], China Telecom [9]

- No (use msg3-transformPrecoder): Spreadtrum [3], CATT [6], Lenovo [12], CMCC [15], Qualcomm [18], LG [19], Samsung [22], Sharp [23], NTT DOCOMO [24], Mediatek [25], ETRI [28]

  - Motivation and benefit not clear [3][6][12][25]

  - gNB does not have information on how much power is available [15][18][22]

  - Additional preamble partitioning required for early indication of capability [6][22][24][28]

  - Significant impact on RAR UL grant, fallbackRAR UL grant, DCI format 0_0 [6][15]

  - Loss of PDCCH coverage if bit is added to DCI format 0_0 [28]

  - Latency not a concern for msg3, repetitions are sufficient [18][28]

  - gNB would anyway not configure CP-OFDM if msg3 repetition are supported [22] or in a cell which is large enough to include UEs that require DFT-S-OFDM [23]

  - Should not link waveform of msg3 with msg1 [23]

Observations on applicability to msg3 PUSCH

11 companies prefer to use legacy solution of setting waveform of msg3 according to msg3-transformPrecoder parameter. 6 companies prefer supporting a different solution, e.g. setting based on use of msg1 repetition or msg3 repetition and/or indicating from RAR or DCI format 0_0 by TC-RNTI.

Majority of companies think that benefit of any solution over legacy is unclear given the possibility of configuring msg3-transformPrecoder to enabled in a (large) cell where some UEs may be power-limited for msg3. There are also concerns for specific solutions, such as need for early indication of capability (e.g. if linked to msg3 repetition) and impact on RAR UL grant, DCI format 0_0.

It does not look like more progress can be achieved at this meeting as the number of companies interested in a solution different from legacy has not increased.

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above.

| Company | Comments |
| --- | --- |
| LG | We do not find the further necessity of dynamic waveform switching for the Msg.3 PUSCH transmission. There is already sufficient mechanism, which gNB recognize and determine a UE waveform semi-statically if the UE located in poor coverage. |
| Vivo | Agree that there’s no need to support DWS for msg3 at this stage, though we focused on more important issues in our contribution and didn’t propose this explicitly. |
| Sony | We think that whenever Msg1 needs PRACH multiple transmissions, then Msg3 will also need some coverage extension such as DWS to DFT-s-OFDM. If linked to Msg1 (PRACH multiple transmissions), there is no need for additional early capability indication of Msg3 DWS. gNB knows when PRACH multiple transmission takes place by choice of preamble or RO. So gNB knows to expect Msg3 DWS. We further think that even under these circumstances, whether the DWS is set to DFT-s-OFDM should still be under gNB control – hence our proposal to explicitly signal the waveform in RAR. |
| Lenovo | Don’t support DWS for msg3 initial transmission or retransmission. |
| China Telecom | We are actually acceptable for the msg3 retransmission with DCI format 0_0 scrambled by TC-RNTI, but we don’t support to use the current method with 1 bit new field in DCI 0_0 for indication. |

## [MP] Issue #1-2: Applicability to UL CA and DCI format 0_3

Summary of company views from contributions submitted to RAN1#113

Support DWS when UE is configured with UL carrier aggregation?

- Yes: ZTE [4], Panasonic [10], Qualcomm [18], Ericsson [21].

Intra-band UL carrier aggregation

- Nokia [16] proposes that RAN further study the DWS for multiple PUSCHs in multiple serving cells considering further analysis on consecutive PUSCHs scenario.

- Qualcomm [18] proposes that for any two carriers that belong to the same band, when DWS is enabled and when uplink scheduling would result in simultaneous transmissions across the two carriers, the transmit waveforms are assumed to be the same between the two carriers.

Applicability to PUSCH scheduled by DCI format 0_3

- Yes: Vivo [2][30], (ZTE [4]), InterDigital [7], Intel [8], (Panasonic [10]), NTT DOCOMO [24]

[Justification]

  - Scheduling using DCI format 0_3 is useful for both cell center and cell edge [7]

  - Not supporting would reduce potential benefit of multi-cell scheduling [7]

  - Straightforward application [8]

  - Benefit of DWS is valid irrespective of DCI formats [24]

[Solution]

  - Use multiple bits at least for inter-band serving cells [2][30]

  - Use 1 bit per co-scheduled cell (Type 2) [8][10][24]

    - Coverage condition could be different in different CC’s [24]

  - Use single bit for all carriers based on “majority” TBs [4]

- Discuss under MC WI: Spreadtrum [3], Panasonic [10]

  - Some issues remain for DCI 0_3, too early [3]

  - Need to discuss if waveform may be same or not for more than one co-scheduled cell [3]

- Low priority: China Telecom [9]

Other issues related to UL carrier aggregation

- Ericsson [21] proposes to study how to prevent the problem where a gNB expects higher UE transmission power with waveform switching in an UL carrier, which is prevented by UE allocating more power for the simultaneous UL transmission in another UL carrier.

Observations on applicability to UL CA and PUSCH scheduled by DCI format 0_3

4 companies propose to agree that DWS is supported for the case of UL carrier aggregation. As a first step, moderator proposes the following agreement to confirm this understanding:

FL proposal 1-2: Dynamic waveform switching is supported for a UE configured with multiple UL carriers.

2 companies [16][18] discuss issues that seem related to the case where both UL transmissions are within the same band. At least the proposal in [18] seems to have impact on RAN1 specifications as scheduling restriction.

8 companies discuss applicability of DWS to PUSCH scheduled by DCI format 0_3. 6 companies seem supportive of applying DWS to PUSCH scheduled by DCI format 0_3, while 2 companies think it is too early or low priority. 2 companies think it should be discussed under multi-carrier work item. (One contribution [30] is submitted under 9.17.)

Moderator suggests discussing this issue under this AI given that discussions on MCE in 9.17 are focusing on RRC parameters at this meeting. The first step is to decide on whether to support DWS for this format considering the motivations for both multi-cell scheduling and dynamic waveform switching. If agreed to be supported, the next point of discussion is on whether indication is per cell or for all cells, and any applicable restriction (e.g. for intra-band cells). Moderator’s understanding is that the agreed multi-cell scheduling framework enables any option.

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above. Please feel free to indicate your view on the following:

- If FL proposal 1-2 is acceptable

- Any view on potential restriction of same waveform for intra-band [18] or issues described in [16]

- If you think DWS should be supported for DCI format 0_3 (and why)?

- If yes, whether separate indication per cell (or group of cells) should be supported?

| Company | Comments |
| --- | --- |
| Nokia, NSB | - We are fine with FL proposal 1-2. - We prefer further discussions since several questions are still unclear to us. For example, would applying restriction of same waveform for intra-band means that UE uses same PA for intra-band? Is this always the case or any other exception? How about inter-band cases? - We are fine to support. - We prefer to discussion details on how to support it in MC discussion. Given that several approaches (types) for indicating of different fields have been identified in the MC WI and it’s straightforward to discuss it there. |
| Panasonic | 1) We are fine with FL proposal 1-2. 2) We share the view on RF aspect raised by Nokia and Qualcomm. We think this aspect should also be considered if we enhance PHR reporting since when waveform is switched and RF is shared among carriers, the available PHR can be different even for the carrier in which dynamic waveform switching is not configured. 3) Yes (as captured by above FL summary). 4) We think separate indication per cell is starting point, but we are open the indication per group of cells based considering aspects related to 2). |
| LG | We are fine with this proposal. But if DWS is introduced finally in DCI format 0_3 that scheduling multiple serving cells, further discussion on such DWS field composition within DCI format 0_3 should be in the multi-carrier agenda. In other words, we think this WI should make decision only for whether to support DWS in multi-carrier scenario. |
| Vivo | - Fine with FL’s proposal - At least for inter-band CA, there’s no need to restrict same waveform when DWS is supported. - 0_3 is also for connected UEs, DWS can be supported for more accurate scheduling and this is similar to extending all other features supported for DCI 0-1/2 to DCI0-3 as discussed in MCE topic. Separate indication per cell is preferred at least for inter-band CA case. |
| QC | Might be good to separate the discussion into two. It should be possible to support inter-band CA without any major issues, but intra-band CA needs more discussion. |
| Sony | Proposal 1-2 acceptable |
| ZTE | - Fine with FL proposal 1-2. - For intra-band or inter-band CA, it is still unclear that whether waveform application would be affected by the power amplifier(s). So we can decide whether to apply DWS to CA case first. - Yes. - Need further discussion. |
| Ericsson | - Support - Rel-17 PUSCH repetitions across mTRP is scheduled a single DCI. It is not clear how different waveforms can be indicated with one DCI, if we don’t expand the DWS field in to two bits for this use case. We are open to discuss the waveform restraint for intra-band UL CA, if UE vendors have some implementation restriction. - Yes. - We share the view of Nokia. It is better to discuss the type of the new DCI field in MCE AI. |
| OPPO | FL proposal 1-2 is acceptable under the condition that the DWS for single cell would be reused simply. It is not likely to be coverage limited when you configure multiple cells and want them transmit together. Optimization should not be excessive. 2)OK 3) Acceptable as indicated above. 4) No |
| ETRI | Fine with the proposal 1-2. We think DWS in DCI format 0_3 is about MCE, and the details would be discussed in the MCE maintenance. |
| Spreadtrum | We are fine with FL proposal 1-2. Whether DWS can be supported for DCI format 0_3 needs more discussion. The waveform may be the same or not for more than one co-scheduled cells, which requires further discussion in the MC WI. |
| Lenovo | Fine with FL proposal 1-2. |
| Moderator | @All, thanks for feedback. It seems that FL proposal 1-2 is acceptable to most (all?) companies. In addition, supporting DWS for DCI format 0_3 also seems acceptable, so suggest we agree on FL proposal 1-3 below. |
| Xiaomi | - fine with FL’s proposal 3) ok to support. |
| Apple | 1. Fine with FL proposal 1-2. 2. Open to discuss this issue. 3. OK to support. 4. This is depending on the outcom of question 2. |
| China Telecom | - fine with the proposal - we are open to discuss it. - We can follow the majority’s view. - If 3) is supported, we think it should be per cell. |

FL proposal 1-3: Dynamic waveform switching is supported for PUSCH(s) scheduled by DCI format 0_3.

# Topic #2: Dynamic switching mechanism

## [MP] Issue #2-1: DWS field interpretation and position within DCI format

Summary of company views from contributions submitted to RAN1#113

DWS field interpretation

- Fixed association between field value and waveform: ZTE [4], Huawei [5], InterDigital [7], China Telecom [9], CMCC [15]

  - 0 for DFT-S-OFDM, 1 for CP-OFDM: ZTE [4], InterDigital [7]

    - Align to order used in transformPrecoder parameter [4][7]

    - If switching from BWP that does not support DWS to a BWP that supports DWS, the waveform corresponding to value “0” is indicated due to zero-padding procedure [7].

  - 0 for CP-OFDM, 1 for DFT-S-OFDM: CMCC [15]

DWS field position

- ZTE [4] proposes that DWS field is appended at the end of existing fields of DCI format 0_1/0_2.

Observations on DWS bit interpretation

5 companies discuss DWS field interpretation. All 5 companies suggest adopting a fixed mapping between field value and waveform, which seems straightforward approach. Moderator suggests adopting the order suggested in [4][7] which aligns to transformPrecoder parameter.

FL proposal 2-1: Value “0” of dynamic waveform switching indicator field maps to transform precoding enabled. Value “1” of dynamic waveform switching indicator field maps to transform precoding disabled.

Regarding DWS field position, 1 company propose that it is appended at the end of existing fields. However, during RAN1#112 and RAN1#112bis-e, several companies expressed preference for locating DWS field near the beginning of DCI format to facilitate parsing.

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above. Please feel free to indicate:

- If FL proposal 2-1 is acceptable.

- Preference for the position of the DWS field.

| Company | Comments |
| --- | --- |
| Nokia, NSB | We are generally fine with either way. Though it’s not very straightforward reading “1” as “disabled” and “0” as enabled. We don’t understand much the argument about alignment to transformPrecoder parameter, because such parameter is configured to be enabled or disabled but not linked to 1 or 0. We slightly prefer putting at the end of the existing fields and consider it as zero padding if DWS is not supported. Though we would like to understand benefit of doing otherwise as well. |
| Panasonic | We are fine with FL proposal 2-1. On the position of the DWS field, since per field alignment is agreed in the last meeting, we think the position is not matter. |
| LG | We think this issue should be discussed, including the issue #2-2. To be specific, in case when the BWP switching is indicated from BWP not supporting DWS to a BWP supporting DWS, we think existing zero-padding procedure from TS38.213 can work. In addition, if the “0” of DWS field maps to same transform precoding as higher layer signaling (e.g., msg3-transformPrecoder), the UE trying to do BWP switching will naturally use semi-statically configured waveform. Therefore we suggest following modified proposal. FL proposal 2-1: Value “0” of dynamic waveform switching indicator field maps to same transform precoding enabled as higher layer signaling (e.g., msg3-transformPrecoder). Value “1” of dynamic waveform switching indicator field maps to different transform precoding disabled as higher layer signaling (e.g., msg3-transformPrecoder). |
| Sharp | We are OK with the FL proposal 2-1 and any position of the DWS field can be acceptable. |
| Sony | Proposal 2-1 is acceptable |
| ZTE | Fine with FL proposal 2-1. We prefer the end of the existing fields, but glad to see if any reasonable reason to support other position. |
| Ericsson | FL proposal 2-1 is fine. |
| OPPO | OK for 2-1 No restriction of DWS bit field location is OK. But for simplicity is also in the font as possible. |
| CMCC | Fine. Although it is a little bit strange for reading ‘0’ and ‘1’ as ‘enable’ and ‘disable’. |
| ETRI | We are acceptable to the proposal 2-1, but the value may follow the example of msg3-transformPrecoder, which seems more natural. |
| Spreadtrum | We are fine with proposal 2-1. We think place the DWS field before any field that has waveform-dependent interpretation may help implementation. |
| Lenovo | Fine with FL proposal 2-1. |
| Moderator | @All, thanks for feedback. FL proposal 2-1 seems acceptable to majority, suggest we agree on this proposal. |
| Xiaomi | Fine, also bit weird for the interpretation of the codepoints too. |
| Apple | Ok with proposal 2-1. |
| China Telecom | Support. |

## [MP] Issue #2-2: Bandwidth part switching

Summary of company views from contributions submitted to RAN1#113

Support dynamic waveform switching in case of BWP switching: Intel [8]

- Enable dynamic adjustment of both bandwidth and waveform [8]

Applicable waveform upon BWP switching from BWP not supporting DWS to a BWP supporting DWS:

- RRC-configured waveform: vivo [2], ZTE [4], Nokia [16], ETRI [28]

  - First transmission in the target BWP [2]

  - Safer to UE [4]

  - Should not apply the indication of source BWP in target BWP (?) [4]

- Repurpose bits of another field to indicate waveform: ETRI [28]

  - Can indicate any waveform for first transmission of BWP switch.

- No change necessary: (InterDigital [7]), Qualcomm [18]

  - Existing zero-padding procedure from TS38.213 works [18]

Observations on BWP switching

6 companies discuss the issue of dynamic waveform switching when DWS is not configured in a BWP and the UE receives a DCI that switches to a BWP where DWS is configured. Moderator’s understanding is that according to current specifications, the UE prepends a zero to the (zero-size) DWS bit and applies the waveform associated to the value “0”. 4 companies propose that instead of following the legacy zero-padding procedure, the UE switches to a semi-statically configured waveform or to a waveform indicated by a re-purposed field of the DCI.

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above. Please indicate if you think if the existing zero-padding procedure in case of bandwidth part switching (38.213, section 12) needs to be modified for the DWS field.

| Company | Comments |
| --- | --- |
| LG | We support the DWS when bandwidth part switching case, and existing zero-padding procedure from TS38.213 can work as commented in issue #2-1. |
| Vivo | Zero padding procedure can be there as it is, no need to update. But when BWP is switched, UE should neglect the DWS indication, and apply the RRC waveform given the reasons as FL summarized. |
| ZTE | Support the DWS during the BWP switching. If Zero padding is applied, we should identify the “0” interpretation mentioned in proposal 2-1 first. If Zero padding is not considered in such case. It is nature to use the RRC configured BWP of target BWP when source BWP has no capability of DWS but target BWP has. For other cases such as source BWP has capability of DWS but target BWP has not, simple dropping the DCI indication of BWP is sufficent. |
| Ericsson | The zero-padding procedure and per-field alignment can work in this case. In particular, a UE can refer to the RRC configuration of dynamic waveform switching for the target BWP; if configured, the waveform for the UL transmission in the new BWP is in accordance with the value of DWS field in DCI. |
| OPPO | We also believe the Zero padding schemes are naturally applied. |
| ETRI | We need to firstly discuss whether to support DWS when this field is absent in the source BWP. If DWS is not supported, then the target waveform is determined by the target BWP that is tied to legacy RRC configuration, which is the legacy behavior. If DWS is supported, then we can further discuss how to enable this feature. |
| Lenovo | We share same view as FL that a UE’s behaviour can follow current specification. |
| Moderator | @All, thanks for feedback. @Vivo, ZTE, Ericsson: in my understanding, ignoring the DWS indication to apply RRC configured value instead is not the behaviour defined in section 12 of 38.213. This would be a special solution for DWS. |

### Offline Tuesday

Questions:

- No change / addition to BWP switching procedure of section 12?

  - When switching from a BWP not supporting DWS to a BWP supporting DWS, first transmission uses waveform indicated by value “0” because of UE prepends the 0-bit DWS field size of source BWP with “zero” to match the 1-bit DWS field size of target BWP.

- Modify BWP switching procedure for the case of switching from BWP not supporting DWS to BWP supporting DWS? (use waveform configured by RRC).

  - Do we need to optimize for this case?

Offline discussion summary:

- ETRI thinks that legacy procedure from section 12 of 38.213 may not apply because the field is not present in DCI of source BWP. Moderator, LG think that the procedure applies if considering that the field is present with 0-bit size.

- Intel thinks that for the case under discussion, zero-prepending applies. Intel thinks that there are other cases to consider, such as source BWP supporting DWS but target BWP not supporting DWS. In this case, BWP switching procedure means that UE ignores the DWS field and applies RRC-configured value of target BWP.

## [MP] Issue #2-3: Handling of FDRA type/DMRS type

Summary of company views from contributions submitted to RAN1#113

Related agreements from RAN1#112bis-e

Agreement For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, downselect between following options: - Option 1 (configuration restriction with error case handling): - UE does not expect resourceAllocation set to resourceAllocationType0. - If DFT-S-OFDM is indicated and resourceAllocation set to dynamicSwitch, UE does not expect MSB of FDRA field set to 0. - Option 2 (UE only uses resourceAllocation if CP-OFDM is indicated): - If DFT-S-OFDM is indicated, UE applies type 1 resource allocation. - If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE. - Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE. Agreement For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, downselect between following options: - Option 1 (configuration restriction with error case handling): - UE does not expect dmrs-Type to be set to type2. - Option 2 (UE only uses dmrs-Type if CP-OFDM is indicated): - If DFT-S-OFDM is indicated, UE applies DMRS type 1. - If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

FDRA type

- Option 1 (error case): vivo [2], Spreadtrum [3], ZTE [4], CATT [6], Intel [8], Xiaomi [11], CMCC [15], Apple [17], LG [19], Oppo [20], Ericsson [21], Mediatek [25], Mavenir [26], Transsion [27], ETRI [28]

  - Still possible to configure dynamicSwitch, maintains flexibility of CP-OFDM: [3]

  - Easy and up to proper configuration, gNB implementation issue [3][4][6][11][15][20][25]

  - Less specification impact than Option 2 [4][19][21][26]

  - Contiguous resource allocation is typically utilized [8]

  - Option 2 requires alignment of FDRA field [26]

  - UE operating DWS is not in cell center, likely low-mobility, don’t need full flexibility [28]

  - Specification impact of Option 1? Yes [2]. No [4].

- Option 2 (apply only for CP-OFDM): Huawei [5], InterDigital [7], China Telecom [9], Panasonic [10], Lenovo [12], NEC [13], Nokia [16], (Samsung [22]), Sharp [23], NTT DOCOMO [24], Google [29]

  - Option 1 reduces flexibility [13][16], dynamicSwitch is optional UE capability [5][24]

  - Option 1 adds 1 bit of DCI overhead [7][10][23]

  - Not much difference in specification effort between options [23][29]

  - Restrictions on operation of DFTS-S-OFDM or CP-OFDM reduce motivation for DWS [24]

  - UE configured with DWS may be in CP-OFDM region for a long time [29]

DMRS type

- Option 1 (error case): vivo [2], Spreadtrum [3], ZTE [4], Intel [8], Xiaomi [11], CMCC [15], LG [19], Oppo [20], Ericsson [21], Mediatek [25], Mavenir [26], Transsion [27], ETRI [28]

  - Same handling as FDRA type [3][4][19][20][21][26]

  - Scheduling restriction is not severe [8]

  - Avoid that DWS field controls the DMRS type [15]

  - UE operating DWS is not in cell center, likely low-mobility, don’t need full flexibility [28]

  - Specification impact of Option 1? Yes [2]. No [4].

- Option 2 (apply only for CP-OFDM): Huawei [5], CATT [6], InterDigital [7], China Telecom [9], Panasonic [10], Lenovo [12], NEC [13], Nokia [16], Apple [17], (Samsung [22]), Sharp [23], NTT DOCOMO [24], Google [29]

  - Option 1 reduces maximum number of supported antenna ports, reducing user capacity and throughput [5]. Not compatible with R18 MIMO enhancements [12].

  - Need to allow configuring DMRS type 2 [23] which improves throughput for CP-OFDM. Aligned with intention of DWS [17].

  - Better flexibility [6][7][9][10][13][16]

  - Low specification impact [7][29]

  - Restrictions on operation of DFTS-S-OFDM or CP-OFDM reduce motivation for DWS [24]

  - UE configured with DWS may be in CP-OFDM region for a long time [29]

Other waveform-specific configuration aspects

Vivo [2] proposes that configuration of pi/2 BPSK modulation when DWS is configured is handled as an error case.

Spreadtrum [3] proposes that parameters required by both waveforms are configured when dynamic waveform switching is configured.

Spreadtrum [3] proposes to apply MCS table configured for indicated waveform.

Observations on handling of FDRA type, DMRS type and other waveform-specific configuration aspects

For FDRA type, 15 companies prefer Option 1 (error case) and 11 companies prefer Option 2 (apply only for CP-OFDM).

For DMRS type, 13 companies prefer Option 1 (error case) and 13 companies prefer Option 2 (apply only for CP-OFDM).

2 companies [2][3] discuss other parameters (e.g. tp-pi2BPSK, mcs-Table, mcs-TableTransformPrecoder) that may also require handling. In moderator’s understanding, there is no issue with these parameters since existing specification only considers e.g. mcs-TableTransformPrecoder in case transform precoding is enabled, and tp-pi2BPSK affects parameter q which is present only in MCS tables that are specifically for DFT-S-OFDM.

The views are very split for both cases. Option 1 has the least specification impact (possibly no impact), while Option 2 has a small specification impact. Option 1 introduces some restrictions for the scheduling of CP-OFDM transmissions when DWS is configured:

- For FDRA type, it is not possible to use resource allocation type 0 with CP-OFDM unless dynamicSwitch for frequency allocation is configured.

- For DMRS type, it is not possible to use DMRS type 2 with CP-OFDM.

It could be considered that a restriction related to FDRA is less severe since it would still be possible to use resource allocation type 0 with CP-OFDM by configuring dynamicSwitch at the cost of 1 bit of overhead if UE supports dynamic switching between FDRA types. On the other hand, in the case of DMRS type there does not seem to be any workaround. A UE configured with DWS would not be able to schedule any transmission that requires DMRS type 2. Because of this, two companies [6][17] prefer Option 1 for FDRA type but Option 2 for DMRS type.

To progress towards a decision, moderator suggests analyzing further the potential impact of restrictions that would arise with Option 1. Companies are invited to provide their views on the following questions:

- Is restriction that FDRA type cannot be configured to type 0 for CP-OFDM acceptable (when UE does not support dynamicSwitch)?

  - Why? (e.g. whether Type 0 RA is often/rarely used, whether it is expected to be useful for a UE configured with DWS, whether most UEs support dynamicSwitch, etc.)

- Is restriction that DMRS type cannot be configured to type 2 for CP-OFDM acceptable?

  - Why? (e.g. whether DMRS type 2 is often/rarely used, whether it is expected to be useful for a UE configured with DWS, etc.)

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above. Please feel free to provide views on the questions 1 and 2 above.

| Company | Comments |
| --- | --- |
| Panasonic | 1. On FDRA, supporting dynamic switching between CP-OFDM with resource allocation type 0 and DFT-s-OFDM with resource allocation type 1 means UE should support dynamic switching of resource allocation type regardless of whether UE supports dynamicSwith. Then, we think it is assumed that most UEs support dynamicSwich if dynamic waveform switching is supported. Therefore, we think the restriction that FDRA type cannot be configured to type 0 for CP-OFDM is acceptable. 2. On DMRS type, we prefer not to have the restriction that DMRS type cannot be configured to Type 2 for CP-OFDM. Option 2 allows the alignment with normal CP-OFDM operation, which has the benefit of DMRS orthogonality. |
| LG | We support Option 1 that mainly reason of its simplicity. These FDRA/DMRS configuration restrictions are applied only for DWS, so temporarily avoiding those Type 0 RA or Type 2 DMRS would not be critical within the short period. Moreover, the specification effort on the Option 2 is unnecessary since the dynamic waveform switching might not frequently be used. |
| ZTE | Yes, both the restrictions can be accepted. We support the Option 1 as it is easy to define it as error case. |
| Ericsson | - Type 0 RA is rarely used, therefore a restriction that FDRA type 0 and dynamic waveform switching can’t be configured simultaneously is acceptable. - It is similar to case of question 1. The restriction is acceptable. |
| Spreadtrum | Option 1 is easy and up to a proper configuration to solve the problem. We prefer Option 1. For legacy UE, it applies resource allocation according to resourceAllocation IE. So Option 2 may affect the UE behavior and have impact on the specification. |
| Lenovo | 1. FDRA, if a UE doesn’t support dynamicSwith, the resource allocation type shall be type 1 for both CP-OFDM and DFT-s-OFDM. Although we prefer option 2, but we can accept option 1. 2. DMRS type, we think option 2 shall be supported since in option 1 CP-OFDM can only use DMRS type 1. Besides, eType 1 and eType 2 DMRS types are introduced for CP-OFDM in Rel-18 MIMO, the flexibility will be reduced much for CP-OFDM if option 1 is adopted. |
| Moderator | @All, thanks for the feedback. We can discuss this further. |
| China Telecom | We don’t think the restrictions are acceptable. Though not happens usually, but these base features shouldn’t be ignored for supporting new feature. It is unreasonable if an UE is a Rel-18 UE, then the basic flexibility and function will be worse than the Rel-15 UE. And Option 2 actually doesn’t bring too much specs impact compared with option 1, even the payload of DCI can be smaller. |

## [LP] Issue #2-4: Other issues related to dynamic switching mechanism

Summary of proposals from contributions submitted to RAN1#113

ZTE [4] proposes that during carrier switching, the DCI size should be aligned between the cross-carrier scheduling and self-scheduling when the same DCI format is used.

China Telecom [9] proposes that For UE configured with multi-PUSCH scheduling in time domain in a carrier (i.e. pusch-TimeDomainAllocationListForMultiPUSCH), DCI format 0_1 supports 1-bit field for dynamic waveform switching indication.

- Moderator’s note: this seems already agreed in RAN1#112bis-e.

Panasonic [10] proposes that RRC can configure that PUSCH repetition uses DFT-S-OFDM regardless of configuration of transformPrecoder and msg3-transformPrecoder.

Sony [14] proposes that RAN1 adopts DWS for Type 2 CG-PUSCH signaled by activation (for VoNR).

CMCC [15] proposes to study potential enhancement to enable UE fallback from DWS to legacy DCI.

Oppo [20] proposes that in configured grant, the transmissions of PDCCH scrambled by CS-RNTI with NDI=0 should follow the waveform of the earlier transmission scheduled by PDCCH scrambled by CS-RNTI with NDI=1. The DWS indication field is ignored for PDCCH scrambled by CS-RNTI with NDI=0.

Observations on other issues related to dynamic switching mechanism

The above proposals seem related to aspects that were concluded in earlier meetings and/or seem to have lower priority.

### Pre-meeting comments

Please indicate if you would like to discuss any of the proposals in this section.

| Company | Comments |
| --- | --- |
| OPPO | Thanks for the explanation. As the CS-RNTI with NDI=0 could be CG type2 activation or release, it may not go as the whole CG dynamic switching not agreed. I realize the proposal is not correctly phased. It was intended for the PUSCH after the one by CS-RNTI with NDI=1. In configured grant, the transmissions after PUSCH scheduled by PDCCH scrambled by CS-RNTI with NDI=1 should follow the waveform of the earlier transmission. I saw current agreement may intend to have the waveform back to RRC configuration after the PUSCH by PDCCH scrambled by CS-RNTI with NDI=1. |

# Topic #3: Assistance information for switching waveform

## [HP] Issue #3-1: Assistance information for switching waveform

Summary of company views from contributions submitted to RAN1#113

Related agreement from RAN1#112bis-e

Agreement For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. - Details FFS. - Note: reporting PH information for both waveforms is not precluded. - Note: additional trigger for PH for reference PUSCH is not precluded. - Option 2: New trigger of power headroom report based on waveform switching event. - Details FFS. - Option 3: Both Option 1 and Option 2. - Details FFS. - Option 4: No enhancement.

Summary of benefits/concerns identified for each Option in contributions:

| Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. |  |
| --- | --- |
| Benefits | Concerns |
| - Difference of Pcmax between waveforms function of UE implementation, exact performance difference difficult to get for NW [3][21][24][27] - Providing both PHR at same time minimizes uncertainty of impact of switch [7] - gNB needs to know possible PHR with new waveform [10][13] - Better performance than Option 2 [17] - Without info, if switching from CP-OFDM to DFT-S-OFDM, gNB does not know how much Tx power increase can be and whether to switch from multi-layer PUSCH to single-layer PUSCH [21] - Without info, if switching from DFT-S-OFDM to CP-OFDM, gNB does not know how to schedule FDRA, rank, MCS at the risk that PUSCH transmission fails [21]. | - No simulation results available [4] - gNB can determine if UE is moving to cell edge or cell center based on current PHR [4] - Not necessary to determine waveform though may help for RB/MCS [4] - Multiple entry MAC CE for UL CA needs to be solved [4] - Requires introducing new field in PHR MAC CE [5] - Optimization compared to Option 2, only eliminates performance loss of the first PUSCH transmission [5] - Increases PHR overhead [6][8] - Information may be outdated at time of reporting [8][9] - Benefit is not clear [22] - Large impact to specifications, including RAN2, not required by WID objective, would also be applicable to RRC-based switching [22] - if gNB changes other parameters to maintain UL connection the PHR of target not useful [23]. |

| Option 2: New trigger of power headroom report based on waveform switching event. |  |
| --- | --- |
| Benefits | Concerns |
| - PHR after switching can be used to correct waveform, RB, MCS [4][26] - Switching back should be rare event, can use conservative scheduling [4] - Benefit over Option 4 is on fine adjustment of scheduling parameters [4] - Less specification impact than Option 1, no need to define PHR report or MAC-CE structure [4][26] - Current PHR triggering does not guarantee timely PHR for waveform switching [5] - Limits performance loss (with Option 4) of unsuitable MCS/RB allocations to the first scheduled PUSCH after switching [5] - Switching can happen at any time [9] - gNB can differentiate cell center from cell edge users [26] | - Reduce workload and specification impact [2] - Performance of current PUSCH cannot be guaranteed [3] - Does not provide assistant information for next waveform switching/selection [3] - May result in more frequent switching when PHR of new waveform is very close to old [3] - Cannot be used to determine waveform switching since PHR is reported after [6][12] - Relies on trial and error, PHR difference may be caused by other factors than waveform [7] - Frequent reporting compared to Option 1 [8] - Specification impact of defining new PHR trigger [8] - Some performance degradation after switching [9][12] - Option 2 alone may lead to redundant DWS, not helping gNB to allocate resources [16] - Benefit of PHR report after switching questionable [17] - May not help gNB derive appropriate parameter set immediately after the switching [23]. |

| Option 4: No new assistance information |  |
| --- | --- |
| Benefits | Concerns |
| - No specification impact - Sufficient, gNB can give back-to-back grants with different waveforms to measure difference of SINR [6][8] | - SINR difference may not provide Pcmax difference between waveforms [3] - gNB does not know how to determine RB allocation and MCS [3] - Not effective to transmit PUSCH twice [3] - May result in unsuccessful transmissions, trial and error approach [7] - Back-to-back PUSCHs need similar channel conditions, max power, same configuration, power sharing/P-MPR, etc. [16] |

Preferences for each Option (second preference between ())

Option 1

- vivo [2], Spreadtrum [3], InterDigital [7], China Telecom [9], Panasonic [10], Lenovo [12], NEC [13], Apple [17], LG [19], Ericsson [21], NTT DOCOMO [24], Transsion [27], ETRI [28], Google [29]

Option 2

- ZTE [4], Huawei [5], (China Telecom [9]), Mavenir [26]

Option 3

- Xiaomi [11], Nokia [16]

Option 4

- CATT [6], Intel [8], Oppo [20], Samsung [22], Mediatek [25]

Further discuss:

- Qualcomm [18], Sharp [23], (Intel [8]), CMCC [15], (Mediatek [25])

  - Whether decision to switch waveforms is coupled to rank selection [18]

  - Whether gNB can infer the PH difference between the two waveforms based on PUSCH DMRS [18]

Assumptions of the reference PUSCH (Option 1)

- Same assumption as current PUSCH for RB allocation/MCS: Spreadtrum [3], InterDigital [7], Panasonic [10], Xiaomi [11], Apple [17], Qualcomm [18], Ericsson [21], Transsion [27],

  - Assume a default value if actual PUSCH RB allocation not compatible with target waveform: Spreadtrum [3]

  - Need to adjust in case RB allocation not supported for target [10]

  - Consider MPR, A-MPR and P-MPR when computing PH [18]

- Consider both cases, same or different RB allocation and modulation order: Nokia [16]

  - To provide precise PH related information of target waveform in different RB regions or for different modulations.

Additional triggers (Option 1)

- No additional trigger : vivo [2], Spreadtrum [3], Ericsson [18]

  - Assistance information is not supposed to change very frequently [18]

- By explicit request (MAC CE): InterDigital [7], Google [29]

- PH is lower/higher than threshold: InterDigital [7], Xiaomi [11], NEC [13], Qualcomm [18] (if other waveform has better headroom), Transsion [27]

  - Unclear if 200 ms periodicity suffices for timely information [18]

- PH difference between waveforms is higher than a threshold: InterDigital [7]

- Continuous data error: Xiaomi [11]

Conditions to provide PHR of target waveform (Option 1)

- Change of waveform since last PUSCH transmission: vivo [2]

- No PH for target waveform in case of virtual PH: vivo [2]

- RSRP less than threshold or current path loss is larger than previous moment and exceeds a threshold: Spreadtrum [3]

- PH is lower than a threshold: InterDigital [7]

- PH difference between waveforms is higher than a threshold: InterDigital [7]

- Support shorter values of periodic PHR timer: Apple [17]

- Only for single rank, QPSK: Qualcomm [18]

Contents of assistance information (Option 1)

- Reuse R17 PUSCH repetition solution (two PHs in single MAC CE) [28]

- Assistance information can include Pcmax/PH for target waveform [3][27] or Pcmax/PH difference between waveforms [3]

- May use reserved bits of MAC CE to avoid overhead increase [9]

- Option 1: both PHRs (at least this Option), Option 2: current PH + PH difference of target [16]

- Report both PH’s (easier gNB implementation) [17]

Other proposals related to assistance information

- Panasonic [10] proposes to study reporting of PHR information considering CA/DC scenario

- Xiaomi [11] proposes to support UE reporting of the proposed waveform to the gNB with the one PHR

- Lenovo [12] propose that PHR for a target waveform determined from an actual PUSCH using same waveform or reference PUSCH in case no PUSCH uses same waveform

- Nokia [16] proposes to discuss scenario when report of PHR for current and target waveforms is close to or overlaps with existing PHR of current waveform

- Nokia [16] proposes to study enhancements on assistance signalling enabling DWS feature assigning same or different waveforms to different TPRs in mTRP scenario.

- LG [19] proposes that if dynamic waveform switching is enabled, support independent open-loop power control parameter for each waveform.

- NTT DOCOMO [24] proposes to consider another reporting metric, e.g. actual MPR difference achieved by UE implementation if Option 1 not agreeable.

- Google [29] proposes that if the enhanced PHR (new format or trigger event) for dynamic waveform switching is agreed, study the PH computation and PHR format extension to CA, DC, multi-TRP, and multi-panel transmission.

Observations on assistance information

Based on contributions, the number of companies for each Option as first preference is as follows:

- 14 companies for Option 1

- 3 companies for Option 2

- 2 companies for Option 3

- 5 companies for Option 4

In addition, 5 companies have indicated that they are open for further discussion. One company suggested points to further explore, such as whether decision to switch waveforms is coupled to rank selection or whether it is feasible to use PUSCH DMRS measurement to infer PH difference.

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above. Please also indicate any further technical points to discuss that would help the group make decision.

| Company | Comments |
| --- | --- |
| LG | We support Option 1, but open to all options that provide PH information of the target waveform rather than the existing PHR behavior without any enhancement. The most important thing is that the benefit from the PH information of the target waveform should be justified regardless of which option is adopted. |
| Vivo | According the summary, at least vast majority companies think assistant information report is beneficial, therefore we can remove Option 4. Considering RAN1 have limited time, we can make a decision to down select only one from option 1 and 2. And RAN1 can send an LS to RAN2 and let RAN2 to decide which option is preferred since MAC spec. impact should be considered. |
| ZTE | It seems most specification impact is in RAN2 if Option 1, 2, or 3 is applied. So why not let RAN2 to decide this, unless some RAN1 impacts are identified. |
| Ericsson | We support Option 1. For Option 1, if a UE is currently configured with 2-layer PUSCH transmission with CP-OFDM, we are open to discuss power headroom of DFT-S-OFDM, e.g., whether this is a restriction. |
| ETRI | We prefer option 1 to option 2 since RAN2 seems to be more involved for option 2 discussions. |
| Lenovo | Support option 1. We think only option 1 can provide the accurate PHR and/or Pcmax difference of different waveforms. For option 2, it is unclear how it can facilitate the gNB on determining waveform switching since the waveform switching event already occur. |
| Moderator | @All, thanks for feedback. It seems from contributions that the least popular options are 2 and 3, so maybe a first step would be to downselect these – see FL proposal 3-1 below. |
| Xiaomi | We are option to all PHR enhancements, and share Vivo’s views to first remove option4 to continue discussion. Also fine with the lastest FL proposal for progress. |
| Apple | We support Option 1 and support Proposal 3-1. |
| China Telecom | We prefer Option 1 but are fine with Option 2. |

FL proposal 3-1 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. [14 companies] - Details FFS. - Note: reporting PH information for both waveforms is not precluded. - Note: additional trigger for PH for reference PUSCH is not precluded. - Option 2: New trigger of power headroom report based on waveform switching event. [3 companies] - Details FFS. - Option 3: Both Option 1 and Option 2. [2 companies] - Details FFS. - Option 4: No enhancement. [5 companies]

### Offline Tuesday

Observations:

- Option 2 and 3 have least number of supporters. Many companies do not see significant benefit in obtaining PHR report after switching and prefer a solution allowing the gNB to decide when switching is appropriate.

- Option 1 has the most supporters. However, several companies have strong concerns mainly on following aspects:

  - The potential impact on MAC CE’s for PHR, if both legacy PHR and proposed PHR for target waveform are reported together.

  - The information may be reported too late unless additional trigger is introduced.

Given the above situation, for downselection moderator suggests focusing on Option 1 for any potential enhancement.

FL proposal 3-1 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH. [14 companies] - Details FFS. - Note: reporting PH information for both waveforms is not precluded. - Note: additional trigger for PH for reference PUSCH is not precluded. - Option 2: New trigger of power headroom report based on waveform switching event. [3 companies] - Details FFS. - Option 3: Both Option 1 and Option 2. [2 companies] - Details FFS. - Option 4: No enhancement. [5 companies]

For Option 1, any possibility of addressing concerns?

Minimize impact to MAC CE?

- Send PH for only one waveform in PHR? (need to indicate which waveform is reported)

- Use 1 bit (e.g. R bit) of PHR to indicate PH of target waveform is better than current waveform?

Trigger for timely reporting?

- By explicit request (MAC CE)

- PH is lower/higher than threshold

- PH difference between waveforms is higher than a threshold

- Continuous data error

Offline discussion summary:

- ZTE thinks Option 1 still have many issues, and prefer Option 4 if Option 2 is down-selected.

- Nokia asks what the concern about MAC CE (PHR format) impact is. Moderator thinks it is about reducing RAN2 workload, when considering whether/how to adapt existing formats / options (e.g. twoPHRmode). Vivo thinks introducing additional PHR formats may not be complicated, but trigger conditions would generate a lot of work for RAN2. ZTE, Huawei think that overhead is a concern for UL CA case. Vivo and Nokia think that RAN1 should discuss what is needed first and leave details to RAN2. It may not be needed to support twoPHRmode at the same time.

### Offline discussions Wednesday morning

Offline discussions took place Wednesday morning to determine details of Option 1 following agreement to downselect between Option 1 and Option 4. The outcome of this is reflected in FL proposal 3-2r0:

FL proposal 3-2r0 For reporting of power headroom information for an [assumed] PUSCH using target waveform (Option 1), support the following: - Power headroom information for an assumed PUSCH using target waveform is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - If transform precoding is enabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same parameters as actual PUSCH transmission except that transform precoding is disabled. - If transform precoding is disabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same parameters as actual PUSCH transmission except that transform precoding is enabled. - [FFS: In case assumed PUSCH transmission is not supported with same parameters as actual PUSCH, power headroom information for a target waveform is not computed or reported.] - Power headroom information for an assumed PUSCH includes the following: - [PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - Can be signaled as difference with PCMAX,f,c(i) of actual PUSCH, if reported together with legacy Type 1 power headroom report.] - [Power headroom of assumed PUSCH - Using Type 1 power headroom report formula for an actual PUSCH transmission of section 7.7.1 of 38.213 - Can be signaled as difference with power headroom of actual PUSCH, if reported together with legacy Type 1 power headroom report.] - RAN1 assumes that RAN2 decides and specifies the following aspects: - Trigger/condition for reporting power headroom information for an assumed PUSCH. - [Whether power headroom information for assumed PUSCH is reported together with legacy Type 1 power headroom report.] - Design of MAC CE(s). - Send LS to RAN2, (cc RAN4?)

Notes:

- Text between square brackets needs further discussion.

- Previous agreement used terminology “reference PUSCH” but this creates confusion since in 38.213, “reference PUSCH” refers to the set of parameters for PHR calculation when PUSCH is not transmitted on a serving cell. Such framework cannot be directly reused. To avoid confusion, it is better to use a different terminology. The version above uses “assumed PUSCH” but open to any better suggestion.

- It is clarified that the power headroom information for assumed PUSCH is only supported for the case when there is actual PUSCH transmission on a serving cell.

- The case when the assumed PUSCH is not possible, such as DFT-S-OFDM with rank 2 or FDRA type 0, may need some discussion. The above version captures the simplest handling, i.e. not reported.

- For the definition of “power headroom information”, need to decide if it is PCMAX,f,c(i) only, PH only, or both. If RAN1 agrees that it is reported together with legacy Type 1 power headroom report, it may be directly defined as the difference between the Pcmax (or PH) of assumed and legacy.

### Offline discussions Wednesday afternoon

Further offline discussions took place Wednesday afternoon to determine details of Option 1. Some changes from r0 include:

- Definition of “power headroom information for assumed PUSCH” fixed to PCMAX,f,c(i) of assumed PUSCH. Whether it is delta from actual PUSCH or not can be left to RAN2. This is because the benefit (or not) of compressing information depends on the design of MAC CE (e.g. with octet-alignment there may not be any benefit).

- Applicability to multiplePHR and twoPHRmode is up to RAN2

FL proposal 3-2r1 For reporting of power headroom information for an assumed PUSCH using target waveform (Option 1), support the following: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - If transform precoding is enabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same parameters as actual PUSCH transmission except that transform precoding is disabled. - If transform precoding is disabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same parameters as actual PUSCH transmission except that transform precoding is enabled. - In case assumed PUSCH transmission is not supported with same parameters as actual PUSCH, power headroom information for a target waveform is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Difference between PCMAX,f,c(i) of assumed PUSCH and PCMAX,f,c(i) of actual PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - RAN1 assumes that RAN2 decides and specifies the following aspects: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Applicability when multiplePHR and/or twoPHRmode is configured. - Send LS to RAN2 (cc RAN4)

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | The proposal of two PHs in one report for a serving cell has big RAN2 spec impact especially for MAC-CE. Therefore, RAN1 cannot assume anything RAN2 has decided. Whether two PHs in one report is supported or not should be subject to RAN2 decision. - Single PH (Alt 2) is introduced and let RAN2 decides between Alt1 (two PHs) and Alt2. - Inform RAN4 without any specific question but allow RAN4 feedback if any FL proposal 3-2r1-revised For reporting of power headroom information for an assumed PUSCH using target waveform (Option 1), support the following [subject to RAN2 decision]: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - If transform precoding is enabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same parameters as actual PUSCH transmission except that transform precoding is disabled. - If transform precoding is disabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same parameters as actual PUSCH transmission except that transform precoding is enabled. - In case assumed PUSCH transmission is not supported with same parameters as actual PUSCH, power headroom information for a target waveform is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Difference between PCMAX,f,c(i) of assumed PUSCH and PCMAX,f,c(i) of actual PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - [For each serving cell with PH of assumed PUSCH to be reported, support only one of alternatives below: - Alt 1: Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - Alt 2: Only Power headroom information for assumed PUSCH is reported] - RAN1 assumes that RAN2 decides and specifies the following RAN2 impacts aspects: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Applicability when multiplePHR and/or twoPHRmode is configured. Send LS to RAN2 (for decision) and (cc RAN4 (any feedback is welcome) |

### Offline Thursday

Updated FL proposal modified during offline:

FL proposal 3-2r2 Downselect between Option 1 and Option 4 [can reuse previous agreement framework] For reporting of power headroom information for an assumed PUSCH using target waveform (Option 1), support the following [subject to RAN2 decision]: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - If transform precoding is enabled [waveform different] for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same L1 parameters [used for the calculation of PHR] as actual PUSCH transmission except that transform precoding is disabled. - If transform precoding is disabled for actual PUSCH transmission, UE computes power headroom information of assumed PUSCH using same L1 parameters as actual PUSCH transmission except that transform precoding is enabled. - FFS: (in case different PC loops are supported) - In case assumed PUSCH transmission is not supported with same parameters as actual PUSCH, power headroom information for a target waveform is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - [For a serving cell with PH of assumed PUSCH to be reported, support only one of alternatives below: - Alt 1: Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - Alt 2: Only Power headroom information for assumed PUSCH is reported] - RAN1 assumes the following RAN2 impacts/design: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - [Applicability when multiplePHR and/or twoPHRmode is configured]. - Send LS to RAN2 [for decision] (cc RAN4) [If Option 1 is agreed]

Updated FL proposal addressing comments from offline session that could not be addressed immediately:

- Added details of Option 1 within framework of previous agreements that includes Option 4.

- Reworked bullets starting with “If transform precoding is [disabled/enabled]” into a single bullet and avoids “disabled/enabled” language which could be misinterpreted as a RRC configuration aspect.

- Clarified that the “same parameters” are the ones that are used for the calculation of Pcmax,f,c(i). This also does not preclude separate TPC loops per waveform.

- Removed references to twoPHRmode and multiplePHR

- Alt1/Alt2 is moved under the bullet that lists aspects to be decided by RAN2 (since this impacts a lot MAC CE design).

FL proposal 3-2r3 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options: - Option 1: Reporting of power headroom information for an assumed reference PUSCH using target waveform different from waveform of actual PUSCH. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported with same parameters as actual PUSCH, power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - RAN1 assumes the following RAN2 impacts and design aspects to be decided by RAN2: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Whether power headroom information for assumed PUSCH and legacy Type 1 PH are reported together or not. - Send LS to RAN2 (cc RAN4) - Option 4: No enhancement.

### Updates after Thursday online session

Updated FL proposal addressing comments from online session and previous offline session on Option 1:

- Concern from Intel that power headroom information for assumed PUSCH needs not be supported if DWS is not configured or if the actual PUSCH is by configured grant: Added FFS sub-bullet to first sub-bullet that if actual PUSCH transmission is not scheduled by DCI with DWS field, PH information for assumed PUSCH is not supported.

- Concern from LG that the second sub-bullet of the second bullet could be interpreted as precluding waveform-specific power control (alpha parameters). Modified the wording to clarify that it is only for the parameters that are used for the calculation of PCMAX,f,c(i) (e.g. number of layers, modulation, RB allocation).

- Concerns from Intel related to multiplePHR and twoPHRmode. Added fourth bullet stating that at least the case where multiplePHR is set to False (i.e. PHR for single serving cell) and twoPHRmode is disabled is supported, which should be common understanding. Other cases are FFS.

- Concerns related to reporting PH information of assumed PUSCH together with legacy type 1 PH or not. Instead of leaving the decision to RAN2, at least one company (Apple) prefers that RAN1 decides that it should be reported together while at least one company (Huawei) prefers that RAN1 decides that it should not be reported together. Deleted sub-bullet under bullet related to RAN2 impact/decisions and added FFS bullet identifying both alternatives as suggested by Huawei earlier.

FL proposal 3-2r4 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select one from the following options: - Option 1: Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - FFS: If actual PUSCH transmission is not scheduled by a DCI with DWS field, power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported with same for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - Power headroom information for assumed PUSCH is supported at least if multiplePHR=False and twoPHRmode is not enabled. FFS: other cases. - For a serving cell with power headroom information of assumed PUSCH to be reported, support only one of alternatives below: - Alt 1: Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - Alt 2: Only power headroom information for assumed PUSCH is reported. - RAN1 assumes the following RAN2 impacts and design aspects to be decided by RAN2: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Whether power headroom information for assumed PUSCH and legacy Type 1 PH are reported together or not. - Send LS to RAN2 (cc RAN4) if Option 1 is selected. - Option 4: No enhancement.

After some further offline discussion, FL proposal is updated to the following:

FL proposal 3-2r5 For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select one from the following options: - Option 1: Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH. - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - FFS: If actual PUSCH transmission is not scheduled by a DCI with DWS field, power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported with same for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH includes the following: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - Power headroom information for assumed PUSCH is supported at least if multiplePHR=False and twoPHRmode is not enabled. FFS: other cases. - For a serving cell with power headroom information of assumed PUSCH to be reported, support only one of alternatives below: - Alt 1: Power headroom information for assumed PUSCH and legacy Type 1 PH are reported together. - Alt 2: Only power headroom information for assumed PUSCH is reported. - RAN1 assumes the following RAN2 impacts and design aspects to be decided by RAN2: - Trigger/condition for reporting power headroom information for assumed PUSCH. - Design of MAC CE(s) for the reporting of power headroom information for assumed PUSCH, including whether PCMAX,f,c(i) of assumed PUSCH is reported directly or as difference from PCMAX,f,c(i) of actual PUSCH. - Whether power headroom information for assumed PUSCH and legacy Type 1 PH are reported together or not. - Send LS to RAN2 (cc RAN4) if Option 1 is selected. - Option 4: No enhancement.

# Topic #4: RRC parameters

## [HP] Issue #4-1: Configuration of DWS

Summary of company views from contributions submitted to RAN1#113

Related agreement from RAN1#112bis-e

Agreement For configuration of 1-bit dynamic waveform switching indication in DCI format 0_1/0_2 per a carrier, downselect between following options: - Option 1: Separate configuration of presence of dynamic waveform switching field for DCI format 0_1 and DCI format 0_2. - Option 2: Common configuration of presence of dynamic waveform switching field for DCI format 0_1 and DCI format 0_2.

Common or separate configurability of presence of DWS field for DCI format 0_1 and 0_2?

- Option 1 (Separate): Spreadtrum [3], ZTE [4], Huawei [5], CATT [6], InterDigital [7], Intel [8], China Telecom [9], NEC [13], CMCC [15], Qualcomm [18], LG [19], Samsung [22], NTT DOCOMO [24], Mavenir [26], Google [29]

  - Many features configured separately between DCI formats [3][4][6][7][18]

  - DCI format 0_2 typically targets smaller DCI size, useful to have possibility of disabling DWS since it increases DCI size [3][5][7][8][9][15][19][24][29]

  - Better flexibility [6][13]

  - More straightforward [22][26]

- Option 2 (Common): Vivo [2], Xiaomi [11], Apple [17], Oppo [20], Mediatek [25]

  - Same waveform is used for both in legacy [2][17]

  - Benefit of separate configuration not significant [20]

  - Sufficient [11][25], DWS is useful for UEs between cell center and cell edge [25]

Details on RRC parameter(s)

- New parameter(s): ZTE [4], InterDigital [7]

  - Waveform configuration falls back to legacy if not present [4]

  - Cleaner approach [7]

- Add possible value to existing transformPrecoder parameter: (Samsung [22]?)

- Discuss: LG [19]

Observations on RRC parameter(s)

On the configurability of presence of DWS field, 15 companies prefer Option 1 (separate between DCI formats) and 5 companies prefer Option 2 (common to both DCI formats). Majority of companies observe that DWS can increase DCI size significantly compared to semi-static configuration of DFT-S-OFDM and think it would be useful to have possibility of not configuring it only for DCI format 0_2 since it typically targets small DCI size.

Considering majority view, moderator suggests agreeing on the following. The proposal includes “for a BWP” since it is consistent with previous agreement that DWS is configurable per bandwidth part.

FL proposal 4-1: Presence of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

Regarding details of RRC parameters (e.g. new parameters or extend existing parameter), few companies have specific proposals. This could be discussed in a further step.

### Pre-meeting comments

Please indicate if your company position was incorrectly captured or if you would like to add your company position to the summary above. Please feel free to indicate if FL proposal 4-1 is acceptable.

| Company | Comments |
| --- | --- |
| Nokia, NSB | Although we don’t see the strong need to have separate configurations, we can support majority view for the sake of progress. Just one question for clarification, would “presence of DWS indicator field” a correct wording be case the field should always be present, but the applicability of DWS functionality should be the one to be configured? |
| Panasonic | We support FL proposal 4-1. |
| LG | Agree with the FL proposal 4-1. |
| Sharp | We support the FL proposal 4-1. |
| ZTE | Support. |
| Ericsson | We are fine with FL proposal 4-1. |
| OPPO | We also have similar clarification as Nokia, would the configuration of DWS indication is more proper in the bullet? |
| CMCC | Fine. |
| ETRI | Support the proposal 4-1. |
| Spreadtrum | Support the proposal 4-1. |
| Lenovo | Support FL proposal 4-1. |
| Moderator | @Nokia, NSB: Thanks for flexibility. @Nokia, NSB, OPPO: Fine to change “presence” with “configuration” since the language “presence” for DCI fields is not used in 38.212 (instead, “0 bit”). Please check FL proposal 4-1r1 below. |
| Xiaomi | we can go with the majority. |
| China Telecom | Support. |

FL proposal 4-1r1: Presence Configuration of dynamic waveform switching indicator field, for a BWP, is separately configurable between DCI format 0_1 and DCI format 0_2.

### Offline Tuesday

One company (Mediatek) asked about motivation for separate vs common configurability given that a UE that is under sufficiently good coverage to receive one DCI format (e.g. 0_1) should also be able to receive the other DCI format (e.g. 0_2). Moderator explained that DCI format 0_2 is targeting higher reliability (e.g. for URLLC) thus smaller payload may be beneficial. Mediatek asked how other companies see this with respect with features. One company (Spreadtrum) thinks this will be discussed during UE features discussion.

Offline status: FL proposal 4-1r1 seems agreeable.

# Topic #5: Other issues

## [LP] Issue #5-1: Other issues

Summary of company views from contributions submitted to RAN1#113

Samsung [22] proposes to prioritize PUSCHs with CP-OFDM for PUSCH determination of UCI multiplexing when a PUCCH overlaps with multiple PUSCHs with different waveforms.

Samsung [22] proposes that separate beta offset and alpha values are used for UCI multiplexing in a PUSCH with different waveforms.

Spreadtrum [3] propose that PUSCH transmission in RRC-connected can support dynamic waveform switching.

Spreadtrum [3] proposes to keep current maximum rank 1 limitation for DFT-S-OFDM.

- Moderator’s note: this seems out of scope for this WI.

Nokia [16] proposes that if DWS is supported for m-TRP, RAN1 to further study relative power tolerance restrictions in consecutive PUSCH transmissions with different waveforms in mTRP scenario and to further considering potential capability indication on the duration to achieve a target power step deltaP using non-monotonic sweeping.

- Moderator’s note: this seems more related to RAN4 specifications?

### Pre-meeting comments

Please indicate if you would support any of the above proposals.

| Company | Comments |
| --- | --- |
| OPPO2 | We would like to add another issue for discussion. In current situation, it is unclear if the CS-RNTI with NDI=1 indicate a waveform, should this waveform be activated in the following transmission? We think this should be supported as the waveform can be switched in some period without new DCI. The proposal is: In configured grant, the transmissions of PDCCH scrambled by CS-RNTI with NDI=0 should follow the waveform of the earlier transmission scheduled by PDCCH scrambled by CS-RNTI with NDI=1. Please have this also in the discussion in the other issues. |
| Moderator | @OPPO2: this is captured in section 6.4. However, my understanding is that CS-RNTI with NDI=0 is CG type 2 activation and the waveform follows RRC-configured value within ConfiguredGrantConfig in that case. |
| OPPO | Thanks for reply. Sorry for missing the 6.4. Some following up in that section. |

# References

| 1 | RP-221858 | Revised WID on Further NR coverage enhancements | China Telecom |
| --- | --- | --- | --- |
| 2 | R1-2304505 | Discussions on remaining issues of dynamic waveform switching | vivo |
| 3 | R1-2304582 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | Spreadtrum Communications |
| 4 | R1-2304600 | Discussion on dynamic waveform switching | ZTE |
| 5 | R1-2304651 | Discussion on dynamic waveform switching for coverage enhancement | Huawei, HiSilicon |
| 6 | R1-2304719 | Dynamic switching between DFT-S-OFDM and CP-OFDM | CATT |
| 7 | R1-2304799 | Dynamic switching between DFT-S-OFDM and CP-OFDM | InterDigital, Inc. |
| 8 | R1-2304809 | Dynamic switching between DFT-S-OFDM and CP-OFDM waveform | Intel Corporation |
| 9 | R1-2304867 | Discussion on dynamic waveform switching between DFT-s-OFDM and CP-OFDM | China Telecom |
| 10 | R1-2304870 | Discussion on dynamic waveform switching | Panasonic |
| 11 | R1-2304886 | Discussion on dynamic switching between DFT-s-OFDM and CP-OFDM | xiaomi |
| 12 | R1-2304977 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | Lenovo |
| 13 | R1-2305000 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | NEC |
| 14 | R1-2305054 | Considerations on dynamic waveform switching for various PUSCH types | Sony |
| 15 | R1-2305117 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | CMCC |
| 16 | R1-2305138 | Dynamic switching between DFT-s-OFDM and CP-OFDM | Nokia, Nokia Shanghai Bell |
| 17 | R1-2305273 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | Apple |
| 18 | R1-2305363 | Dynamic switching between DFT-S-OFDM and CP-OFDM | Qualcomm Incorporated |
| 19 | R1-2305394 | Discussion on dynamic waveform switching for NR coverage enhancement | LG Electronics |
| 20 | R1-2305455 | Considerations on dynamic switching between DFT-S-OFDM and CP-OFDM | OPPO |
| 21 | R1-2305485 | Discussion on Dynamic UL Waveform Switching | Ericsson |
| 22 | R1-2305540 | Dynamic switching between DFT-S-OFDM and CP-OFDM | Samsung |
| 23 | R1-2305570 | Dynamic switching between DFT-S-OFDM and CP-OFDM for Rel-18 CovEnh | Sharp |
| 24 | R1-2305618 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | NTT DOCOMO, INC. |
| 25 | R1-2305666 | Dynamic switching between waveforms | MediaTek Inc. |
| 26 | R1-2305682 | Discussion on solutions for NR dynamic switching between DFT-S-OFDM and CP-OFDM | Mavenir |
| 27 | R1-2305714 | Discussion of dynamic switching between DFT-S-OFDM and CP-OFDM | Transsion Holdings |
| 28 | R1-2305803 | Dynamic switching between DFT-S-OFDM and CP-OFDM | ETRI |
| 29 | R1-2305914 | Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM | Google Inc. |
| 30 | R1-2304222 | Summary #5 on dynamic switching between DFT-S-OFDM and CP-OFDM | Moderator (InterDigital) |
| 30 | R1-2304517 | Discussion on Rel-18 Multi-carrier enhancements, eDSS and NCR | vivo |

# Appendix: Previous agreements

RAN1#112b-e

Agreement

For DCI format 0_1/0_2 containing dynamic waveform indication, bit width of each field is set to the maximum between the bit width of the field if transform precoding is disabled and the bit width of the field if transform precoding is enabled, if different.

- If, for the waveform indicated in the DCI, the bit width N of a field would be smaller than the bit width of the field set as per the above, UE decodes the field using N least significant bits. If N=0, the UE ignores the field for the indicated waveform.

Agreement

For potential enhancements to assist the scheduler in determining waveform switching, RAN1 to select 1 from the following options:

- Option 1: Reporting of power headroom information for a reference PUSCH using target waveform different from waveform of actual PUSCH.

  - Details FFS.

  - Note: reporting PH information for both waveforms is not precluded.

  - Note: additional trigger for PH for reference PUSCH is not precluded.

- Option 2: New trigger of power headroom report based on waveform switching event.

  - Details FFS.

- Option 3: Both Option 1 and Option 2.

  - Details FFS.

- Option 4: No enhancement.

Conclusion

For PUSCH transmission scheduled by C-RNTI with DCI format 0_0, UE considers transform precoding enabled or disabled according to msg3-transformPrecoder as in legacy.

Agreement

Dynamic waveform switching is configured separately for each BWP, within PUSCH-Config.

Agreement

For UE configured with multi-PUSCH scheduling in time domain in a carrier (i.e. pusch-TimeDomainAllocationListForMultiPUSCH), DCI format 0_1 supports 1-bit field for dynamic waveform switching indication.

- When configured, 1-bit field indicates waveform for all scheduled PUSCH transmissions.

Agreement

For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, downselect between following options:

- Option 1 (configuration restriction with error case handling):

- UE does not expect resourceAllocation set to resourceAllocationType0.

- If DFT-S-OFDM is indicated and resourceAllocation set to dynamicSwitch, UE does not expect MSB of FDRA field set to 0.

- Option 2 (UE only uses resourceAllocation if CP-OFDM is indicated):

- If DFT-S-OFDM is indicated, UE applies type 1 resource allocation.

- If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE.

- Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE.

Agreement

For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, downselect between following options:

- Option 1 (configuration restriction with error case handling):

- UE does not expect dmrs-Type to be set to type2.

- Option 2 (UE only uses dmrs-Type if CP-OFDM is indicated):

- If DFT-S-OFDM is indicated, UE applies DMRS type 1.

- If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

Agreement

For configuration of 1-bit dynamic waveform switching indication in DCI format 0_1/0_2 per a carrier, downselect between following options:

- Option 1: Separate configuration of presence of dynamic waveform switching field for DCI format 0_1 and DCI format 0_2.

- Option 2: Common configuration of presence of dynamic waveform switching field for DCI format 0_1 and DCI format 0_2.

RAN1#112

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

RAN1#111

Agreement

For DCI based solution,

- For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI

Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion

Note: It does not imply that the waveform switching indication applies to other transmission or not

- Indication from non-UL scheduling DCI is not supported.

Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed

Working Assumption

Support new 1-bit field for dynamic waveform indication from UL scheduling DCI

- Note: no change of the current size alignment procedure between UL DCI and DL DCI

Agreement

Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching:

- Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

  - Target waveform can be same or different from waveform of an actual PUSCH transmission

  - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission

  - FFS determination of target waveform, target RB allocation, target modulation order

  - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms

- PHR triggering enhancements, e.g.

  - Network-triggered PHR

  - PH becomes lower (higher) than a threshold

  - PHR triggered by waveform switching

- Reporting of recommended waveform or request to switch waveform

- Other solutions not precluded

RAN1#110bis-e

Agreement

Dynamic waveform switching enhancement in R18 is only applicable to PUSCH channel.

Working Assumption

Support at least one of the following options for the dynamic waveform indication in R18:

Alt 1: Indication from an UL scheduling DCI

- Alt 1-A: New field in scheduling DCI

- Alt 1-B: Reuse existing field in scheduling DCI

  - Alt 1-B-1: Explicit indication by repurposing field, e.g.

    - Add one column to TDRA table

    - Add one column to MCS table(s)

    - Other solutions not precluded

  - Alt 1-B-2: Implicit determination from condition(s) on scheduling information, e.g.

    - RA type, MSB of RA

    - Number of RBs (below threshold or multiple of 2,3,5)

    - Location of RB allocation within carrier and the associated MPR

    - MCS below threshold

    - Number of PUSCH repetitions (or whether PUSCH repetition is used) and/or TBoMS

    - Number of DMRS CDM group(s) without data

    - Precoding information and number of layers

    - SRI

    - Condition over multiple types of scheduling information

    - Other types of scheduling information not precluded

- Indicated waveform applies at least to the scheduled PUSCH transmission

  - FFS: Whether it also applies to subsequent transmissions, and of which type

- FFS: DCI formats can contain the indication

- FFS: Indication applies only if condition(s) are satisfied (e.g. PDCCH occasion, /RNTI, /Search space of the scheduling DCI, latest PHR reported by the UE, etc.)

Alt 2: Indication from a non-UL scheduling DCI

- FFS: DCI formats that can provide the indication (e.g. Downlink DCI, UE-group common DCI)

- FFS: Types of subsequent transmissions to which indication is applicable

Agreement

To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as:

- Reporting power headroom related information

- Other solutions are not precluded

Agreement

Dynamic waveform switching enhancement in R18 is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1.

- Note: The above does not imply that dynamic switching enhancement in R18 is applicable or not applicable to other cases of PUSCH (e.g. PUSCH transmission with a Type 1 or Type 2 configured grant, PUSCH scheduled by DCI format 0_0).

R1-2210749	Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)