3GPP TSG RAN WG1 Meeting #113  						 			                                                                                                             R1-2306250

Incheon, South Korea, May 22nd   – May 26th, 2021

Agenda Item:	8.15

Source:	       Moderator (MediaTek)

Title:	       RRC parameters for Rel-18 IoT NTN enhancements

Document for:	Discussion and Decision

# Introduction

Rapporteur is to provide initial input on higher layer signalling under agenda item 9.9 for Release 18 IoT_NTN_Enh according to RAN1#113 agenda.

# Disabling of HARQ feedback for IoT NTN

## Related RRC parameters

Based on the agreements to date (up to RAN1#112bis-e), an initial list of RRC parameters for Rel-18 IoT_NTN_Enh related to 9.9.3  Disabling of HARQ feedback is provided below:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_Enh | HARQ feedback disabling for eMTC CE mode A and mode B | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-r18 | new | downlinkHARQ-FeedbackDisabled-r18 | Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID | Bitmap [TBD length] | FFS | Per UE | UE-specific | 36.331 |  |
| IoT_NTN_Enh | HARQ feedback disabling for NB-IoT | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-NB-r18 | new | downlinkHARQ-FeedbackDisabled-NBr18 | Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID | Bitmap [TBD length] | FFS | Per UE | UE-specific | 36.331 |  |
| IoT_NTN_Enh | HARQ feedback disabling for eMTC CE Mode B | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-DCI-r18 |  | downlinkHARQ-FeedbackDisabled-DCI-r18 | Indicate whether DCI indication is used to directly indicate / override RRC configuration for disabling HARQ feedback | BOOLEAN |  | Per UE | UE-specific | 36.331 |  |
| IoT_NTN_Enh | HARQ feedback disabling for NB-IoT | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-DCI-NB-r18 |  | downlinkHARQ-FeedbackDisabled-DCI-NB-r18 | Indicate whether DCI indication is used to directly indicate / override RRC configuration for disabling HARQ feedback | BOOLEAN |  | Per UE | UE-specific | 36.331 |  |
| IoT_NTN_Enh | HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | 36.213 |  |  |  | harq-FeedbackEnablingforSPSactive-r18 |  | harq-FeedbackEnablingforSPSactive-r18 | If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation. Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation | BBOLEAN |  | Per UE | UE-specific | 36.331 |  |

## Company views

| Companies | Comments |
| --- | --- |
| Ericsson | - For the HL parameters related to the DCI-based solutions it has been written “downlinkHARQ-FeedbackDisabled-DCI”, to be equality descriptive for the semi-static approach, perhaps we should update the parameter name as follows “downlinkHARQ-FeedbackDisabled-Bitmap”. - One question, do we need to append “-r18”? If I’m not wrong the suffix “-r18” should be used when the parameter exists from before and a change has been made to it, in such cases there is a need to differentiate the parameter from the existing one by using the “-rXX” suffix. - There is a typo in the last row for NB-IoT: BBOLEAN |

## Updated list of RRC parameters based on company views for first round

On Ericsson comments, moderator view is that proposed revisions are minor editorial revisions that are acceptable.

Based on comments, the revised list of RRC parameters for Rel-18 IoT_NTN_Enh related to 9.9.3  Disabling of HARQ feedback is provided below

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initiql access or IDLE:INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_Enh | HARQ feedback disabling for eMTC CE mode A and mode B | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-Bitmap | new | downlinkHARQ-FeedbackDisabled-Bitmap | Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID | Bitmap [TBD length] | FFS | Per UE | No | 36.331 |  |
| IoT_NTN_Enh | HARQ feedback disabling for NB-IoT | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-Bitmap-NB | new | downlinkHARQ-FeedbackDisabled-Bitmap-NB | Used to disable the DL HARQ feedback, sent in the uplink, per HARQ process ID | Bitmap [TBD length] | FFS | Per UE | No | 36.331 |  |
| IoT_NTN_Enh | HARQ dynamic feedback disabling for eMTC CE Mode B | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-DCI |  | downlinkHARQ-FeedbackDisabled-DCI | Indicate whether DCI indication is used to directly indicate / override RRC configuration for disabling HARQ feedback | BOOLEAN |  | Per UE | No | 36.331 |  |
| IoT_NTN_Enh | HARQ dynamic feedback disabling for NB-IoT | 36.213 |  |  |  | downlinkHARQ-FeedbackDisabled-DCI-NB |  | downlinkHARQ-FeedbackDisabled-DCI-NB | Indicate whether DCI indication is used to directly indicate / override RRC configuration for disabling HARQ feedback | BOOLEAN |  | Per UE | No | 36.331 |  |
| IoT_NTN_Enh | HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | 36.213 |  |  |  | harq-FeedbackEnablingforSPSactive |  | harq-FeedbackEnablingforSPSactive | If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation. Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation | BOOLEAN |  | Per UE | No | 36.331 |  |

# Improved GNSS operations for IoT NTN

## Related RRC parameters

Based on the agreements to date (up to RAN1#112bis-e), an initial list of RRC parameters for Rel-18 IoT_NTN_Enh related to 9.9.4 Improved GNSS operations is provided below:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for eMTC | 36.213 |  |  |  | GNSSGapLength-r18 | new | downlinkHARQ-FeedbackDisabled-r18 | Indicates the length of configured GNSS measurement gap. | The gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. | FFS | Per UE | UE-specific or cell- specific | 36.331 | The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for NB-IoT | 36.213 |  |  |  | GNSSGapLength-NB-r18 | new | downlinkHARQ-FeedbackDisabled-NB-r18 | Indicates the length of configured GNSS measurement gap. | The gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. | FFS | Per UE | UE-specific or cell- specific | 36.331 | The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB |
| IoT_NTN_Enh | GNSS assistance information report for eMTC | 36.213 |  |  |  | GNSSPositionFixtDuration-r18 | new | GNSSPositionFixtDuration-r18 | UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | N-bit field at least including with [1,2] seconds as candidate component values FFS: value of N, other component value(s) of candidate GNSS position fix time duration (e.g. N=3, with value in [1,2,3,7,13,19,25, X] seconds, and X is FFS). | FFS | Per UE | UE-specific | 36.331 |  |
| IoT_NTN_Enh | GNSS assistance information report for NB-IoT | 36.213 |  |  |  | GNSSPositionFixtDuration-NB-r18 | new | GNSSPositionFixtDuration-NB-r18 | UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | N-bit field at least including with [1,2] seconds as candidate component values FFS: value of N, other component value(s) of candidate GNSS position fix time duration (e.g. N=3, with value in [1,2,3,7,13,19,25, X] seconds, and X is FFS). | FFS | Per UE | UE-specific | 36.331 |  |

## Company views

| Companies | Comments |
| --- | --- |
| Ericsson | - One question, do we need to append “-r18”? If I’m not wrong the suffix “-r18” should be used when the parameter exists from before and a change has been made to it, in such cases there is a need to differentiate the parameter from the existing one by using the “-rXX” suffix. - Typo: GNSSPositionFixtDuration --> GNSSPositionFixtDuration - Parameter name is incorrect: - downlinkHARQ-FeedbackDisabled-r18 GNSSGapLength-r18 - downlinkHARQ-FeedbackDisabled-NB-r18 GNSSGapLength-NB-r18 |

## Updated list of RRC parameters based on company views for first round

On Ericsson comments, moderator view is that proposed revisions are minor editorial revisions that are acceptable.

Based on comments, a revised list of RRC parameters for Rel-18 IoT_NTN_Enh related to 9.9.4 Improved GNSS operations is provided below:

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for eMTC | 36.213 |  |  |  | GNSSGapLength | new | GNSSGapLength | Indicates the length of configured GNSS measurement gap. | The gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. | FFS | Per UE | UE-specific or cell- specific | 36.331 | The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for NB-IoT | 36.213 |  |  |  | GNSSGapLength-NB | new | GNSSGapLength-NB | Indicates the length of configured GNSS measurement gap. | The gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. | FFS | Per UE | UE-specific or cell- specific | 36.331 | The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB |
| IoT_NTN_Enh | GNSS assistance information report for eMTC | 36.213 |  |  |  | GNSSPositionFixDuration | new | GNSSPositionFixDuration | UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | N-bit field at least including with [1,2] seconds as candidate component values FFS: value of N, other component value(s) of candidate GNSS position fix time duration (e.g. N=3, with value in [1,2,3,7,13,19,25, X] seconds, and X is FFS). |  | Per UE | UE-specific | 36.331 |  |
| IoT_NTN_Enh | GNSS assistance information report for NB-IoT | 36.213 |  |  |  | GNSSPositionFixtDuration-NB | new | GNSSPositionFixtDuration-NB | UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | N-bit field at least including with [1,2] seconds as candidate component values FFS: value of N, other component value(s) of candidate GNSS position fix time duration (e.g. N=3, with value in [1,2,3,7,13,19,25, X] seconds, and X is FFS). |  | Per UE | UE-specific | 36.331 |  |

| Companies | Comments |
| --- | --- |
| Qualcomm | A parameter to enable autonomous gaps is needed according to today’s agreement: Agreement For NB-IoT and eMTC, at least for the case where the network configuration does not include a periodicity (if supported), for autonomous GNSS re-acquisition, the UE may re-acquire GNSS autonomously during GNSS measurement timer, the start time of the autonomous GNSS measurement timer is based on the original GNSS validity duration. - FFS: additional delay and details of delay (if any), e.g. delay can be zero or can be equal to/larger than the duration X where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition. - Note1: Autonomous GNSS re-acquisition mechanism is enabled or disabled by network. - Note2: The length of GNSS measurement timer can be configured by network and the length of GNSS measurement timer is equal to the latest reported GNSS position fix time duration for measurement when the length of GNSS measurement timer is not configured - Note3: The autonomous GNSS re-acquisition can be periodic in certain conditions without further spec impact |
| Huawei, HiSiicon | In the same agreement, quoted by Qualcomm, in Note 2, a configuration of autonomous GNSS measurement timer will be provided. It should be added as well As for the GNSSGapLength and GNSSGapLength-NB, what I heard it is discussing in RAN2 whether it is carried in triggering MAC CE or RRC. Maybe we can hold it after RAN2 decision. |
| Moderator | Based on RAN1 agreement, we updated Value range for this RRC parameter GNSSPositionFixtDuration-NB and GNSSPositionFixtDuration-NB in table below UE reports one GNSS position fix time duration for GNSS measurement via a 4-bit field with component values [1,2,3,4,5,6,7,13,19,25,31] - FFS: other component values If RAN2 discussing GNSSGapLength and GNSSGapLength-NB is carried in triggering MAC CE or RRC, then this HL parameter can wait until RAN2 concludes discussions. |

## Updated list of RRC parameters based on company views for second round

Based on RAN1 agreement, we updated Value range for this RRC parameter GNSSPositionFixtDuration-NB and GNSSPositionFixtDuration-NB in table below

UE reports one GNSS position fix time duration for GNSS measurement via a 4-bit field with component values [1,2,3,4,5,6,7,13,19,25,31]

FFS: other component values

On Huawei comment, if RAN2 discussing GNSSGapLength and GNSSGapLength-NB is carried in triggering MAC CE or RRC, then this HL parameter can wait until RAN2 concludes discussions.

On Qualcomm and Huawei comments, a Higher Layer parameter to enable autonomous gaps and a configuration of autonomous GNSS measurement timer are added

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initiql access or IDLE:INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for NB-IoT—triggered for eMTC | 36.213 |  |  |  | GNSSPositionFixDuration | new | GNSSPositionFixDuration | UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | [1,2,3,4,5,6,7,13,19,25,31] |  | Per UE | No | 36.331 | 4-bit field UE report |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for NB-IoT—triggered for NB-IoT | 36.213 |  |  |  | GNSSPositionFixtDuration-NB | new | GNSSPositionFixtDuration-NB | UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | [1,2,3,4,5,6,7,13,19,25,31] |  | Per UE | No | 36.331 | 4-bit field UE report |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for eMTC -—autonomous | 36.213 |  |  |  | GNSSAutonomousEnable | new | GNSSAutonomousEnable | Autonomous GNSS re-acquisition mechanism is enabled or disabled by network. | BOOLEAN |  | Per UE | No | 36.331 |  |
| IoT_NTN_Enh | GNSS position fix in RRC Connected state for NB-IoT -—autonomous | 36.213 |  |  |  | GNSSAutonomousEnable-NB | new | GNSSAutonomousEnable-NB | Autonomous GNSS re-acquisition mechanism is enabled or disabled by network. | BOOLEAN |  | Per UE | No | 36.331 |  |

# Reference