3GPP TSG RAN WG1 Meeting#113	R1-2306303

# Incheon, Korea, May 22nd – 26th, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR Sidelink operation on shared spectrum |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-06-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of specification support for NR sidelink operation on shared spectrum |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce to support NR sidelink operation on shared specturm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifcation does not support NR sidelink operation on shared spectrum. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 4.0, 4.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.212, TS 38.214, TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Contention window for a given priority class

Maximum contention window for a given priority class

Minimum contention window for a given priority class

Maximum channel occupancy time for a given priority class

Maximum Uplink channel occupancy time for a given priority class

Energy detection threshold

Maximum energy detection threshold

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AUL-DFI	Autonomous UL Downlink feedback indication

CAPC	Channel access priority class

COT	Channel Occupancy Time

LAA	Licensed Assisted Access

MCOT	Maximum Channel Occupancy Time

SL	Sidelink

# 4	Channel access procedure

## 4.0	General

Unless otherwise noted, the definitions below are applicable for the following terminologies used in this specification:

-	A channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum.

-	A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. The basic unit for sensing is a sensing slot with a duration Tsl=9μs. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration, and determines that the detected power for at least 4μs within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy.

-	A channel occupancy refers to transmission(s) on channel(s) by eNB/gNB/UE(s) after performing the corresponding channel access procedures in this clause.

-	A Channel Occupancy Time refers to the total time for which eNB/gNB/UE and any eNB/gNB/UE(s) sharing the channel occupancy perform transmission(s) on a channel after an eNB/gNB/UE performs the corresponding channel access procedures described in this clause. For determining a Channel Occupancy Time, if a transmission gap is less than or equal to 25μs, the gap duration is counted in the channel occupancy time. A channel occupancy time can be shared for transmission between an eNB/gNB and the corresponding UE(s).

-	A DL transmission burst is defined as a set of transmissions from an eNB/gNB without any gaps greater than 16μs. Transmissions from an eNB/gNB separated by a gap of more than 16μs are considered as separate DL transmission bursts. An eNB/gNB can transmit transmission(s) after a gap within a DL transmission burst without sensing the corresponding channel(s) for availability.

-	A UL transmission burst is defined as a set of transmissions from a UE without any gaps greater than 16μs. Transmissions from a UE separated by a gap of more than 16μs are considered as separate UL transmission bursts. A UE can transmit transmission(s) after a gap within a UL transmission burst without sensing the corresponding channel(s) for availability.

-	A SL transmission burst is defined as a set of SL transmissions from a UE without any gaps greater than 16μs. The SL transmissions from a UE separated by a gap of more than 16μs, are considered as separate SL transmission bursts. A UE can transmit SL transmission(s) after a gap within a SL transmission burst without sensing the corresponding channel(s) for availability.

-	A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. The discovery burst can be any of the following:

-	Transmission(s) initiated by an eNB that includes a primary synchronization signal (PSS), secondary synchronization signal (SSS) and cell-specific reference signal(s)(CRS) and may include non-zero power CSI reference signals (CSI-RS).

-	Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS).

## 4.5	Sidelink Channel access procedures

A UE operating in sidelink resource allocation mode 1 or mode 2 and performing SL transmission(s) on channel(s) shall perform the procedures described in this clause for the UE to access the channel(s) on which the transmission(s) are performed.

In this clause, transmissions from a UE are considered as separate SL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in clause 4.5.5 when applicable.

A UE can access a channel on which SL transmission(s) are performed according to one of Type 1 or Type 2 SL channel access procedures as described in clauses 4.5.1 and 4.5.2, respectively.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s), the applicable channel access priority class is defined in Table 4.5-1.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including PSSCH with user plane data and associated PSCCH, the UE determines the corresponding SL channel access priority class p  in Table 4.5-1 following the procedures described in [Clause x.x.x in [Y]].

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including PSFCH or S-SSB transmission(s), the UE shall use the channel access priority class p=1 in Table 4.5-1.

A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tslmcot,p where the channel access procedure is performed based on the channel access priority class p  associated with the UE transmissions, as given in Table 4.5-1.

If a UE fails to access the channel(s) prior to an intended SL transmission(s), Layer 1 notifies higher layers about the channel access failure and the channel(s) that the UE fails to access.

Table 4.5-1: Channel Access Priority Class (CAPC) for SL

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tslmcot,p | allowed CWp sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For p=3,4, Tslmcot,p=10ms if the higher layer parameter sl-absenceOfAnyOtherTechnology-r18 is provided, otherwise, Tslmcot,p=6ms. NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms. |  |  |  |  |  |

### 4.5.1	Type 1 SL channel access procedure

This clause describes channel access procedures by a UE where the time duration spanned by the sensing slots that are sensed to be idle before a SL transmission(s) is random. The clause is applicable to the SL transmission(s) including [at least any of] PSSCH/PSCCH or PSFCH or S-SSB.

A UE may transmit the transmission using Type 1 channel access procedure after first sensing the channel to be idle during the sensing slot durations of a defer duration Td, and after the counter N is zero in step 4. The counter N is adjusted by sensing the channel for additional sensing slot duration(s) according to the steps described below.

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the UE chooses to decrement the counter, set N=N-1;

3)	sense the channel for an additional sensing slot duration, and if the additional sensing slot duration is idle, go to step 4; else, go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel until either a busy sensing slot is detected within an additional defer duration Td or all the sensing slots of the additional defer duration Td are detected to be idle;

6)	if the channel is sensed to be idle during all the sensing slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

If a UE has not transmitted a SL transmission on a channel on which SL transmission(s) are performed after step 4 in the procedure above, the UE may transmit a transmission on the channel, if the channel is sensed to be idle at least in a sensing slot duration Tsl when the UE is ready to transmit the transmission and if the channel has been sensed to be idle during all the sensing slot durations of a defer duration Td immediately before the transmission. If the channel has not been sensed to be idle in a sensing slot duration Tsl when the UE first senses the channel after it is ready to transmit, or if the channel has not been sensed to be idle during any of the sensing slot durations of a defer duration Td immediately before the intended transmission, the UE proceeds to step 1 after sensing the channel to be idle during the sensing slot durations of a defer duration Td.

The defer duration Td consists of duration Tf=16μs immediately followed by mp consecutive sensing slot durations where each sensing slot duration is Tsl=9μs, and Tf includes an idle sensing slot duration Tsl at start of Tf.

CWmin,p≤CWp≤CWmax,p is the contention window. CWp adjustment is described in clause 4.5.4.

CWmin,p and CWmax,p are chosen before step 1 of the procedure above.

mp, CWmin,p, and CWmax,p are based on a channel access priority class p as shown in Table 4.5-1.

### 4.5.2	Type 2 SL channel access procedure

This clause describes channel access procedures by UE where the time duration spanned by the sensing slots that are sensed to be idle before a SL transmission(s) is deterministic.

Type 2A SL channel access procedure as described in clause 4.5.2.1 are applicable to the following transmission(s) performed by a UE:

- If a UE intends to transmit a SL transmission at least  25μs  after a SL transmission by another UE in a shared channel occupancy as described in clause 4.5.3, the UE uses Type 2A SL channel access procedures for the SL transmission.

- If a UE intends to transmit only S-SSB in transmission(s) where the time duration of S-SSB transmission(s) is at most 1ms with a duty cycle of at most 120, the UE uses Type 2A SL channel access procedures for the SL transmission(s).

Type 2B or Type 2C SL channel access procedure as described in clause 4.5.2.2 and 4.5.2.3, respectively, are applicable to the transmission(s) performed by a UE following transmission(s) by a UE after a gap of 16μs or up to 16μs, respectively, in a shared channel occupancy as described in clause 4.5.3.

#### 4.5.2.1	Type 2A SL channel access procedure

When a UE uses Type 2A SL channel access procedures for a transmission, the UE may transmit the transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_sl=25μs. -The interval Tshort_sl consists of a duration Tf=16μs immediately followed by one sensing slot and Tfincludes a sensing slot at start of Tf. The channel is considered to be idle for Tshort_sl if both sensing slots of Tshort_sl.are sensed to be idle.

#### 4.5.2.2	Type 2B SL channel access procedure

When a UE uses Type 2B SL channel access procedures for a transmission, the UE may transmit the transmission immediately after sensing the channel to be idle within a duration of Tf=16μs. Tf includes a sensing slot that occurs within the last 9us of Tf. The channel is considered to be idle within the duration Tf if the channel is sensed to be idle for total of at least 5us with at least 4us of sensing occurring in the sensing slot.

#### 4.5.2.3	Type 2C SL channel access procedure

When a UE uses Type 2C SL channel access procedures for a transmission, the UE does not sense the channel before the transmission. The duration of the corresponding SL transmission is at most 584us.

### 4.5.3	SL channel access procedures in a shared channel occupancy

When a UE initiates a channel occupancy using the channel access procedures described in clause 4.5.1 on a channel to transmit SL transmission(s), the UE can provide a channel occupancy sharing information that includes at least the source and destination IDs, the corresponding channel access priority class, the remaining channel occupancy duration, and the frequency domain information of the channel occupancy. [The channel occupancy sharing information can also include additional IDs and additional time and frequency domain information of the channel occupancy].

For the case when a UE transmits SL transmission(s) in a shared channel occupancy initiated by a UE, the channel access priority class value corresponding the SL transmission(s) is at most equal to the channel access priority class value provided by the channel access priority class in the channel occupancy sharing information.

When a UE initiates a channel occupancy to transmit SL transmission(s) within a RB set(s) and provides channel occupancy sharing information with a unicast PSCCH/PSSCH transmission within the  RB set(s), another UE may transmit unicast PSCCH/PSSCH transmission(s) sharing the initiated channel occupancy within the  RB set(s), if the destination and source IDs in the corresponding SL control information match the source and destination IDs, respectively, in the unicast PSCCH/PSSCH transmission carrying the channel occupancy information [or match a pair of additional source and destination IDs if provided by the channel occupancy sharing information. Another UE may transmit groupcast or broadcast PSCCH/PSSCH transmissions sharing the initiated channel occupancy within the RB set(s), if the destination ID in the corresponding SL control information matches an additional destination ID if provided by the channel occupancy sharing information].

When a UE initiates a channel occupancy to transmit SL transmission(s) within a RB set(s) and provides channel occupancy sharing information with a groupcast or broadcast PSCCH/PSSCH transmission within the RB set(s), another UE may transmit a groupcast or broadcast PSCCH/PSSCH transmission(s) sharing the initiated channel occupancy within the RB set(s), if the destination ID in the corresponding SL control information matches the destination ID in the groupcast or broadcast PSCCH/PSSCH transmission carrying the channel occupancy sharing information [or matches an additional ID if provided by the channel occupancy sharing information. Another UE may transmit unicast PSCCH/PSSCH transmissions sharing the initiated channel occupancy within the RB set(s), if the destination and source IDs in the corresponding SL control information match a pair of additional source and destination IDs if provided by the channel occupancy sharing information].

When a UE initiates a channel occupancy to transmit SL transmission(s) within a RB set(s) and provides channel occupancy sharing information with a PSSCH/PSCCH transmission within the RB set(s), another UE may transmit a S-SSB transmission(s) sharing the initiated channel occupancy within the RB set(s).

When a UE initiates a channel occupancy to transmit SL transmission(s) within a RB set(s) and provides channel occupancy sharing information with a PSSCH/PSCCH transmission with a PSSCH/PSCCH transmission within the RB set(s), another UE may transmit a PSFCH transmission(s) sharing the initiated channel occupancy within the RB set(s) if at least one of the PSFCH transmission(s) is intended for the UE initiating the channel occupancy.

If a UE shares a channel occupancy initiated by another UE using the channel access procedures described in clause 4.5.1 on a channel to transmit SL transmission(s), the UE may transmit a SL transmission that follows the SL transmission by the UE that has initiated the channel occupancy after a gap as follows:

-	If the gap is at least 25μs, the UE can transmit the SL transmission on the channel after performing Type 2A channel access procedures as described in clause 4.5.2.1.

-	If the gap is 16μs, the UE can transmit the SL transmission on the channel after performing Type 2B channel access procedures as described in clause 4.5.2.2.

-	If the gap is up to 16μs, the UE can transmit the SL transmission on the channel after performing Type 2C channel access as described in clause 4.5.2.3.

When a UE uses channel access procedures to initiate a SL transmission(s) and shares the corresponding channel occupancy with another UE that transmits a SL transmission(s), the UE may transmit a SL transmission(s) within its channel occupancy that follows the SL transmission(s) that share the initiated channel occupancy.

### 4.5.4	Contention window adjustment procedures for SL transmissions

If a UE transmits a SL transmission(s) including PSSCH(s) using Type 1 channel access procedures associated with the channel access priority class p on a channel and the SL transmission(s) is enabled with explicit HARQ-ACK feedback including ‘ACK’/‘NACK’, the UE maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in clause 4.5.1 for the SL transmission(s) applying the following procedures:

1)	For every priority class p∈1,2,3,4, set CWp=CWmin,p.

2)	If a HARQ-ACK feedback corresponding to the PSSCH(s) for unicast SL transmission(s) in the reference duration for the latest channel occupancy initiated by the UE, is available:

-	If the HARQ-ACK feedback includes only ‘ACK’, go to step 1; otherwise go to step 4.

3)	If a HARQ-ACK feedback corresponding to the PSSCH(s) for groupcast SL transmission(s) in the reference duration for the latest channel occupancy initiated by the UE, is available:

-	If HARQ-ACKFeedbackRatioforContentionWindowAdjustment-GC-Option2 is provided by higher layers:

-	The UE calculates the ratio between the number of received ‘ACK’ in the HARQ-ACK feedback and [the number of UE(s) from which the corresponding ‘ACK’/’NACK’ in the HARQ-ACK feedback is expected]. If the calculated ratio is equal to or larger than HARQ-ACKFeedbackRatioforContentionWindowAdjustment-GC-Option2, go to step 1; otherwise go to step 4.

-	Otherwise:

-	If the HARQ-ACK feedback includes at least an ‘ACK’, go to step 1; otherwise go to step 4.

4)	Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

5)	For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2.

The reference duration in the procedure above is defined as follows:

-	The reference duration corresponding to a channel occupancy initiated by the UE including SL transmission(s) of PSSCH(s) is defined in this clause as a duration starting from the beginning of the channel occupancy initiated by the UE including SL transmission (s) of PSSCH(s) until the end of the first slot where at least one PSSCH with HARQ-ACK feedback(s) including ‘ACK’/’NACK’ is transmitted.

If a UE transmits a SL transmission(s) including PSSCH(s) using Type 1 channel access procedures associated with the channel access priority class p on a channel and the SL transmission(s) is not associated with explicit HARQ-ACK feedback(s) by the corresponding UE(s), the UE adjusts CWp before step 1 in the procedures described in clause 4.5.1, using the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p  has not been used for any SL transmissions on the channel, CWp=CWmin,p is used. If the latest CWp≠CWmax,p value is consecutively used for [X] times for generation of Ninit as described in clause 4.5.1 for SL transmission(s) including PSSCH(s) without associated explicit HARQ-ACK feedback(s), the CWp is increased for every priority class p∈1,2,3,4 to the next higher allowed value.

The following applies to the procedures described in this clause for contention window adjustment:

-	If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p.

-	If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit. K is selected by UE from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4.

### 4.5.5	Energy detection threshold adaptation procedure

A UE accessing a channel on which SL transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max.

XThresh_max is determined as follows:

-	If the UE is configured with higher layer parameter sl-maxEnergyDetectionThreshold-r18,

-	XThresh_max is set equal to the value signalled by the higher layer parameter;

-	otherwise

-	the UE shall determine X'Thresh_max according to the procedure described in clause 4.5.5.1;

-	if the UE is configured with higher layer parameter sl-energyDetectionThresholdOffset-r18

-	XThresh_max is set by adjusting X'Thresh_max according to the offset value signalled by the higher layer parameter;

-	otherwise

-	the UE shall set XThresh_max=X'Thresh_max.

[If the higher layer parameter sl-absenceOfAnyOtherTechnology-r18 is not configured to a UE, and the higher layer parameter ue-toUE-COT-SharingED-Threshold is configured to the UE, the UE should use the UE's transmit power in determining the resulting energy detection threshold ue-toUE-COT-SharingED-Threshold. ]

[For the case where a UE performs channel access procedures as described in clause 4.5.1 for SL transmission(s) and indicates channel occupancy sharing information, XThresh_max is set equal to the value provided by the higher layer parameter ue-toUE-COT-SharingED-Threshold, if provided.]

#### 4.5.5.1	Default maximum energy detection threshold computation procedure

If the higher layer parameter sl-absenceOfAnyOtherTechnology-r18 is provided

-	X'Thresh_max=min&Tmax+10dB&Xr where

-	Xr is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB

otherwise

-	X'Thresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX

where

-	TA=10dB;

-	PH=23dBm;

-	PTX is set to the value of PCMAX_H,c as defined in [3];

-	Tmax(dBm)=10⋅log103.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz);

-	BWMHz is the single channel bandwidth in MHz.

The higher layer parameter sl-absenceOfAnyOtherTechnology-r18 is not expected to be provided if the channel(s) where UE performing SL transmission(s) is overlapped with either an LAA Scell(s) on channel(s) or channel(s) where gNB/UE performing DL/UL transmission(s).

### 4.5.6	Channel access procedures for transmission(s) on multiple channels

If a UE

-	is scheduled to transmit on a set of channels C, and if the SL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C, or

-	intends to perform sidelink transmissions on configured resources on the set of channels C, and if the SL transmissions are configured to start transmissions at the same time on all channels in the set of channels C, or

-	intends to perform sidelink transmissions on selected resources on the set of channel C, and if SL transmissions are to start at the same time on all channels in the set of channels C

the followings are applicable:

-	A UE can access multiple channels on which only PSFCH transmissions are performed, according to one of the Type A or Type B procedures described in clause 4.5.6.1 and 4.5.6.2, respectively[, or according to the procedures in clause 4.5.6.3].

-	A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in clause 4.5.6.3.

#### 4.5.6.1	Type A multi-channel access procedures for PSFCH transmissions

A UE can access multiple channels on which only PSFCH transmissions are performed, according to the procedures described in this clause.

A UE shall perform channel access on each channel ci∈C, according to the procedures described in clause 4.5.1, where C is a set of channels on which the UE intends to transmit, and i=0,1,…q-1, and q is the number of channels on which the UE intends to transmit.

The counter N described in clause 4.5.1 is determined for each channel ci and is denoted as Nci. Nci is maintained according to clause 4.5.6.1.1 or 4.5.6.1.2.

[For determining CWp for channel ci, any PSSCH that fully or partially overlaps with channel ci, is used in the procedures described in clause 4.5.4.]

##### 4.5.6.1.1	Type A1 multi-channel access procedures

Counter N as described in clause 4.5.1 is independently determined for each channel ci and is denoted as Nci.

If the absence of any other technology sharing the channel cannot be guaranteed on a long term basis (e.g. by level of regulation), when the UE ceases transmission on any one channel cj∈C, for each channel ci≠cj, the UE can resume decrementing Nci when idle sensing slots are detected either after waiting for a duration of 4⋅Tsl, or after reinitializing Nci, for performing channel access procedures, respectively.

##### 4.5.6.1.2	Type A2 multi-channel access procedures

Counter N is determined as described in clause 4.5.1 for channel cj∈C, and is denoted as Ncj, where cj is the channel that has the largest CWp value. For each channel ci, Nci=Ncj.

When the UE ceases the PSFCH transmissions, on any one channel for which Nci is determined, the UE shall reinitialize Nci for all channels, respectively.

#### 4.5.6.2	Type B multi-channel access procedures for PSFCH transmissions

A UE can access multiple channels on which only PSFCH transmissions are performed, according to the procedures described in this clause.

A channel cj∈C is selected by the UE as follows:

-	the UE selects cj by uniformly randomly choosing cj from C before each transmission on multiple channels ci∈C, or

-	the UE selects cj no more frequently than once every 1 second,

where C is a set of channels on which the UE intends to transmit, i=0,1,…q-1, and q is the number of channels on which the UE intends to transmit PSFCH transmissions.

To transmit on channel cj

-	the UE shall perform channel access on channel cj according to the procedures described in clause 4.5.1 with the modifications described in clause 4.5.6.2.1 or 4.5.6.2.2, for accessing the channel to perform PSFCH transmissions.

To transmit on channel ci≠cj, ci∈C

-	for each channel ci, the UE shall sense the channel ci for at least a sensing interval Tmc=25μs immediately before transmitting on channel cj, and the UE may transmit on channel ci immediately after sensing the channel ci to be idle for at least the sensing interval Tmc, for accessing the channel to perform PSFCH transmissions. The channel ci is considered to be idle for Tmc if the channel is sensed to be idle during all the time durations in which such idle sensing is performed on the channel cj in given interval Tmc.

[The UE shall not transmit a transmission on a channel ci≠cj, ci∈C, for a period exceeding Tmcot,p as given in Table 4.5-1, where the value of Tmcot,p is determined using the channel access parameters used for channel cj, for accessing the channel to perform PSFCH transmissions.]

For the procedures in this clause, the channels of the set of channels C selected by the UE for PSFCH transmissions, is a subset of the RB sets in the (pre-)configured sidelink resource pool.

[For determining CWp for channel ci, any PSSCH that fully or partially overlaps with channel ci, is used in the procedures described in clause 4.5.4.]

##### 4.5.6.2.1	Type B1 multi-channel access procedure

A single CWp value is maintained for the set of channels C.

[For determining CWp for channel access on channel cj, step 2 of the procedure described in clause 4.5.4 is modified as follows

-	if at least Z=80% of HARQ-ACK values corresponding to PSSCH transmission(s) in the reference duration of all channels ci∈C are determined as NACK, increase CWp for each priority class p∈1,2,3,4 to the next higher allowed value; otherwise, go to step 1.]

##### 4.5.6.2.2	Type B2 multi-channel access procedure

A CWp value is maintained independently for each channel ci∈C using the procedure described in clause 4.5.4.

For determining Ninit for channel cj, CWp value of channel cj1∈C is used, where cj1 is the channel with largest CWp among all channels in set C.

#### 4.5.6.3	[Multi-channel access procedures for SL transmissions]

A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in this clause.

If a UE intends to transmit SL transmissions on a set of channels C, the following is applicable:

-	if Type 1 channel access procedure is used for SL transmissions on the set of channels C,

-	the UE may transmit on channel ci∈C using Type 2A channel access procedure as described in clause 4.5.2.1,

-	if the channel frequencies of the set of channels C is a subset of the sets of channel frequencies defined in clause X.X in [2X], and

-	if Type 2A channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and

-	if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.5.1,

-	where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C.

-	the UE may transmit on channel ci∈C using Type 1 channel access procedure as described in clause 4.5.1

-	the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured with SL resources.

-	[the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an SL bandwidth part as described in clause X of [8], and the UE fails to access any of the channels of the SL bandwidth part.]