3GPP TSG-RAN WG1 Meeting #113	R1-2306321

# Incheon, Korea, May 22-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR positioning enhancement in Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-06-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR positioning enhancement in Rel-18 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-18 NR positioning enhancement: - Section 3.3: Add the abbreviation of SL-PRS - Section 8.3.1.2: Define the new SCI format for scheduling SL-PRS in the dedicated resource pool - Section 8.4.1.4: Define the new 2nd-stage SCI for scheduling SL-PRS in the shared resource pool |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR positioning enhancement in Rel-18 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 8.3.1.2 (new), 8.4.1.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CG	Configured grant

CG-DFI	CG downlink feedback information

CG-UCI	CG uplink control information

CORESET	Control resource set

COT	Channel occupancy time

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MBS	Multicast broadcast services

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSSCH	Physical sidelink shared channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SL-BCH	Sidelink broadcast channel

SL PRS               Sidelink positioning reference signal

SL-SCH	Sidelink shared channel

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

< Unchanged parts are omitted >

8.3	Sidelink control information on PSCCH

SCI carried on PSCCH is a 1st-stage SCI, which transports sidelink scheduling information.

8.3.1	1st-stage SCI formats

The fields defined in each of the 1st-stage SCI formats below are mapped to the information bits a0 to aA-1 as follows:

Each field is mapped in the order in which it appears in the description, with the first field mapped to the lowest order information bit a0 and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to a0.

< Unchanged parts are omitted >

8.3.1.2	SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS.

The following information is transmitted by means of the SCI format 1-B:

-	Priority – x bits as specified in clause x.x of [12, TS 23.287] and clause x.x of [8, TS 38.321].

-	Source ID – x bits as defined in clause x.x of [6, TS 38.214].

-	Destination ID – x bits as defined in clause x.x of [6, TS 38.214].

-	Cast type indicator – 2 bits as defined in clause x.x of [6, TS 38.214].

-	Resource reservation period – log2Nrsv_period bits as defined in clause xx of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-PRSResourceReservePeriodList, if higher layer parameter sl-PRSResourceReservePeriodList is configured; 0 bit otherwise.

< Unchanged parts are omitted >

8.4	Sidelink control information on PSSCH

SCI carried on PSSCH is a 2nd-stage SCI, which transports sidelink scheduling information, and/or inter-UE coordination related information.

8.4.1	2nd-stage SCI formats

The fields defined in each of the 2nd-stage SCI formats below are mapped to the information bits  a0 to aA-1 as follows:

Each field is mapped in the order in which it appears in the description, with the first field mapped to the lowest order information bit a0 and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to a0.

< Unchanged parts are omitted >

#### 8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS.

The following information is transmitted by means of the SCI format 2-D:

-	HARQ process number – 4 bits.

-	New data indicator – 1 bit.

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

< Unchanged parts are omitted >