3GPP TSG RAN WG1 #113			R1-230nnnn

Incheon, Korea, May 22nd – May 26th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.7

Agenda Item:		9.16.7

Document for: 	Endorsement

- 9.16.7	UE features for NR sidelink evolution

Initial input on UE features for NR sidelink evolution to be provided by rapporteur.

Proposal 2-1-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-1 | SL channel access for dynamic channel access mode | UE supports 1) SL Type 1 channel access and contention window size adjustment 2) SL Type 2A channel access 3) SL Type 2B channel access 4) SL Type 2C channel access 5) 20MHz LBT bandwidth [6) CP extension up to 1 symbol in 15kHz SCS] 7) CP extension up to 2 symbols in 30kHz and 60kHz SCS [8) CP extension up to 2 symbols if the UE supports 60kHz SCS] FFS whether to merge with the FG with COT sharing | [At least one of 15-25 except Component 3 and 4 in 15-2, 15-3 except Component 3] | [No] | No | UE does not support channel access for NR sidelink operation in unlicensed spectrum. | [Per band] | n/a | n/a |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must support this FG.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-2-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-2 | SL multi-channel access for dynamic channel access mode | 1) UE supports multi-channel access procedures for PSSCH transmission in multiple RB sets 2) UE supports Type A and Type B multi-channel access procedures for concurrent PSFCH transmissions in multiple RB sets FFS whether to separate this FG for PSFCH and S-SSB FFS whether to introduce FG(s) for transmission on multiple RB sets FFS whether to merge with 47-1 | [47-1, 15-11, and at least one of 15-25 except Component 3 and 4 in 15-2, 15-3 except Component 3] | No | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in unlicensed spectrum. | [Per band] | n/a | n/a |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-3-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-3 | Transmitting /Receiving UE to UE COT sharing information | [1) UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing] 2) UE supports indicating in SCI COT sharing information 3) UE supports monitoring SCI to read COT sharing information FFS whether to separate this FG for transmission and reception FFS whether to merge with FG 47-1 | [47-1, 15-1, and at least one of 15-25 except Component 3 and 4 in 15-2, 15-3 except Component 3] | No | [No] | [UE does not support UE-to-UE COT sharing for NR sidelink operation in unlicensed spectrum.] | Per band | n/a | n/a |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-4-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-5 | MCSt [Resource allcation for] Multi-consecutive slots transmission | UE supports 1) MCSt in SL Mode 1 resource allocation 2) MCSt in SL Mode 2 resource allocation | [47-1, At least one of 15-25 except Component 3 and 4 in 15-2, 15-3 except Component 3] | Yes | No | UE does not support resource allcation for multi-consecutive slots transmission | [Per UE band] | n/a | n/a |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-5:

- Introduce following FG

| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | 1) UE supports transmitting/receiving interlace RB-based PSCCH/PSSCH/PSFCH FFS whether to merge with FG 47-m2 | [47-1, at least one of 15-1 except Component 5, 15-25 except Component 3 and 4 in 15-2 and 15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation.] | Optional with capability signalling [For UE supports NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m2 | Interlace RB-based PSCCH/PSSCH/PSFCH reception | 1) UE supports receiving interlace RB-based PSCCH/PSSCH/PSFCH FFS whether to merge with FG 47-m1 | [47-1, 15-1 except Component 5] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH reception | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation.] | Optional with capability signalling [For UE supports NR sidelink in unlicensed spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported.] |

roposal 2-6-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1) UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot FFS whether to merge with FG 47-m4 | [47-1, at least one of 15-25 except Component 3 and 4 in 15-2 and 15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1) UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot FFS whether to merge with FG 47-m3 | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | [Per band] | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] | Optional without capability signalling [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |

Proposal 2-7-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | 1) UE supports N PSFCH occasion(s) per PSCCH/PSSCH FFS whether to separate for transmission and reception capabilities | 15-11 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] Candidate values for N are {TBD} | Optional with capability signalling [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-8:

- Introduce following FGs

| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1) UE supports transmitting legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain | [15-4, 47-1] | No | No | UE does not support transmission of legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] Candidate values for N are {TBD} | Optional without capability signalling [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m7 | Receiving SSB repetitions within one RB set | 1) UE supports receiving S-SSB which is transmitted in a manner that repeat legacy S-PSS/S-SSS/PSBCH N times in frequency domain | 15-4 | No | No | UE does not support reception of S-SSB which is transmitted in a manner that repeat legacy S-PSS/S-SSS/PSBCH N times in frequency domain | Per band | N/A | N/A |  |  | [Optional with capability signalling] |

Proposal 2-9:

- Introduce following FGs

| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1) UE supports transmitting S-SSB on additional S-SSB occasion(s) | [15-4, 47-1] | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m9 | Receiving of S-SSB on additional S-SSB occasion(s) | 1) UE supports receiving S-SSB transmitted on additional S-SSB occasion(s) | [15-4] | No | No | UE does not support receiving S-SSB transmitted on additional S-SSB occasion(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |

Proposal 2-10:

- Introduce following FG

| 47. NR_SL_enh2 | 49-m10 | Transmission and reception with contiguous PSCCH/PSSCH/PSFCH waveform | 1. Transmit using contiguous frequency domain resource allocation for PSCCH/PSSCH/PSFCH 2. Receive using contiguous frequency domain resource allocation for PSCCH/PSSCH/PSFCH if the UE supports SL reception. |  | Yes | No |  | Per band | NA | NA | NA |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3-1-a:

- Introduce following FG

| 47. NR_SL_enh2 | 47-s1 | Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing | 1) UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] using LTE sidelink resource reservation information that excludes NR SL candidate resources overlapping with EUTRA SL reserved resources by other EUTRA SL UE 2) UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] that excludes NR SL candidate resources overlapping with EUTRA SL resources selected to be used for EUTRA SL module’s own transmission 3) UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs [in 15kHz and 30kHz SCSs] that excludes NR SL candidate resources overlapping with EUTRA SL resources associated with non-monitored subframe in EUTRA SL module 4) UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs in 30kHz SCS that selects at least the first of NR SL slots overlapping with an EUTRA SL subframe and can select the subsequent overlapping NR SL slot 5) UE supports NR SL Mode 2 resource (re)selection for NR PSCCH/PSSCH TXs that excludes NR SL candidate resources where the corresponding NR PSFCH transmission occasions overlap with EUTRA SL reserved resources by other EUTRA SL UE in time domain 26) UE supports NR PSCCH/PSSCH TXs in 15kHz and/or 30kHz SCSs. Candidate value sets: {[15KHz, 30kHz, both]} | [15-3, 15-6, 15-11] UE supports EUTRA SL in the same band | [Yes] | No |  | [Per band] | N/A | N/A |  | This is the basic FG for LTE/NR sidelink co-channel coexistence. [UE supports EUTRA SL in the same band] | [Optional with capability signalling] For UE supports NR sidelink co-channel coexist with EUTRA sidelink, UE must support this FG. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2304511	Discussion on UE features for Rel-18 sidelink	vivo

R1-2304668	UE features for NR sidelink evolution	Huawei, HiSilicon

R1-2305279	On UE Features for Rel-18 NR Sidelink Evolution	Apple

R1-2305371	UE Features for Sidelink Evolution	Qualcomm Incorporated

R1-2305470	Initial UE features list for Rel-18 NR sidelink evolution WI	OPPO, Huawei, HiSilicon, LG Electronics

R1-2305622	Discussion on UE features for NR SL evolution	NTT DOCOMO, INC.

R1-2305697	Discussion on UE features for NR sidelink evolution	ZTE, Sanechips

R1-2305722	Initial views on UE features for Sidelink evolution	Nokia, Nokia Shanghai Bell