3GPP TSG RAN WG1 #113			R1-230nnnn

Incheon, Korea, May 22nd – May 26th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.8

Agenda Item:		9.16.8

Document for: 	Endorsement

- 9.16.8	UE features for eRedCap

Initial input on UE features for eRedCap to be provided by rapporteur.

Proposal 2-1a:

  - Introduce following FGs

| 48. NR_redcap_enh | 48-1 | RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1 | The following components are the same as for supportOfRedCap-r17 (28-1): 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP The following components are new compared to supportOfRedCap-r17 (28-1): 11. DL/UL peak data rate target of 10 Mbps [FFS whether fixed or minimum]. The constraint of vLayers·Qm·f >= 4 is relaxed to vLayers·Qm·f [>= X] 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast (including Msg3/MsgA PUSCH and Msg4 PDSCH) per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS 13. Relaxed random access processing timeline [details FFS] |  | Y |  | Network assumes the UE is not a RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1. | [Per UE] | [No] | [No] (FR1 only) |  | A UE supporting this FG is not required to support FG 6-1. A UE supporting this FG is not allowed to support FG 28-1. The specifications for a UE supporting FG 28-1 (‘RedCap UE’) also apply for a UE supporting this FG (FG 48-1) unless stated otherwise. It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A. | Optional with capability signaling UEs supporting Rel-18 eRedCap UE complexity reduction feature(s) indicate support of this FG instead of FG 28-1 (supportOfRedCap-r17). |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_redcap_enh | 48-2 | RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1 | The capabilities of FG 48-2 are the same as for FG 48-1 except that the following restriction does not apply: 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast (including Msg3/MsgA PUSCH and Msg4 PDSCH) per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Component 11 is different from FG 48-1 as: DL/UL peak data rate target of 10 Mbps [FFS whether fixed or minimum]. The constraint of vLayers·Qm·f >= 4 is relaxed to vLayers·Qm·f [>= Y] | 48-1 | Y |  | Network assumes the UE is not a RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1. | [Per UE] | [No] | [No] (FR1 only) |  |  | Optional with capability signaling |

Alternative Proposal 2-2-b:

- In the LS to RAN2 for Rel-18 UE features for NR, ask RAN2 to update the prerequisite FGs for FGs 28-1a and 28-3 as: 28-1 or 48-1

R1-2304337	Initial input on UE features for Rel-18 eRedCap	Rapporteur (Ericsson)

R1-2304361	Discussion on UE features for R18 RedCap	FUTUREWEI

R1-2304512	Discussion on UE features for R18 eRedCap	vivo

R1-2304795	On support of legacy features for Rel-18 eRedCap UEs	Ericsson

R1-2305372	UE featuers for eRedCap	Qualcomm Incorporated

R1-2305457	Rel-18 RedCap UE features	OPPO

R1-2305544	UE features for eRedcap	Samsung

R1-2305623	Discussion on UE features for eRedCap	NTT DOCOMO, INC.

R1-2305662	UE features for eRedCap	MediaTek Inc.

R1-2305723	Initial views on UE features for eRedCap	Nokia, Nokia Shanghai Bell

R1-2305855	On eRedcap features	Nordic Semiconductor ASA

R1-2305928	UE features for eRedCap	Huawei, HiSilicon