3GPP TSG RAN WG1 #114		R1-2306760

Toulouse, France, August 21st – August 25th, 2023

Source:	vivo

Title:	Discussion on positioning for RedCap UEs

Agenda Item:	9.5.5

Document for:	Discussion and Decision

Introduction

At the RAN#98e meeting, the WID on expanded and improved NR positioning RP-223549 [1] was approved including the following objective related to RedCap positioning:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

In this contribution, we will present our views on RedCap positioning.

Potential design of DL and UL frequency hopping

In this section, we present our views on potential design of DL and UL frequency hopping.

DL PRS Rx frequency hopping

PPW-based Rx frequency hopping

In Rel-17, PPW-based PRS measurement is limited to the case where the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the PPW. PPW is pre-configured per BWP, and can be activated by the gNB when needed. Therefore, for PPW-based Rx frequency hopping, a potential solution is that PPW is still configured per BWP, and UE performs Rx frequency hopping through BWP switching. Each Rx hop is associated with a BWP and corresponding PPW. The following figure is shown as an example.

Figure 1 PPW-based Rx frequency hopping

In our understanding, compared with MG-based Rx hopping, PPW-based Rx frequency hopping is not preferred with the following reasons.

- PPW-based method requires BWP switching, however, the current BWP switching mechanism cannot be used directly. For example, for DCI-based BWP switching, UE can switch to a new BWP based on the field ‘BWP indicator’ in DCI 1-1, and at the same time perform data scheduling at the new BWP. For RRC-based BWP switching, UE can switch to a new BWP after RRC reconfiguration or scell activation. For timer-based BWP switching, when the UE has no data scheduling for a period of time, it can automatically switch to the default BWP. The current BWP switching mechanism is mainly used for data transmission, and cannot be used for Rx hopping for pure positioning purposes. In addition, even if the signaling-based BWP switching can be applied between adjacent hops, the BWP switching latency and complexity are also unacceptable. Perhaps the most suitable ‘BWP switching mechanism’ for Rx hopping is the ‘automatic continuous’ BWP switching, that is, after one reception is completed, it is automatically switched to another BWP to complete another reception until a round of Rx hopping is completed. However, such ‘automatic continuous’ BWP switching will bring huge specification impact.

- In the previous meetings, to minimize specification impact, some companies proposed that PPW-based method can be limited to Type-1A PPW only and simplify BWP switching operation to RF retuning only where the switching time can be included in the PPW as MG. In our view, such enhancements will cause the PPW to lose its association with the BWP, making the PPW as a special type of MG, except that ‘such MG’ can have flexible priorities. We expect PPW to have more differentiation from MG rather than duplication. In addition, such enhancements also do not reduce specification impact. For example, the collision issues with UL transmission for half-duplex Redcap UE, the complex processing capability (N,T) and (N2,T2) for Rx hopping within PPW should also be considered.

- For PRS Rx frequency hopping, it is sufficient to only apply MG-based method, no need to extend to PPW-based method. In Rel-17, PPW was introduced for low-latency purpose. If the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP, PPW can be applied without additional procedures of MG request and configuration, so that physical layer latency is reduced. But for RedCap positioning Rx frequency hopping, there seems no such low latency requirement and scenario. The introduction of PPW-based method is unnecessary.

- For PRS Rx frequency hopping, PPW-based method is not supported.

Report of measurement

Agreement The previous agreement is updated as follows: Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: - A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning - One measurement where a measurement is associated with one received hop - FFS: indication of how many received hops / which received hops where used in the measurement report. - Note: no new measurement definition is introduced in RAN1 - FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

Based on the agreement, there are 2 mechanisms for report of measurement.

- Mechanism 1: A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning

- Mechanism 2: One measurements where a measurement is associated with one received hop

In our view, Mechanism 1 is the basic measurement and report mechanism as long as the requirement can be met; and Mechanism 2 can be a fallback measurement and report mechanism when UE/gNB is not able to accurately provide the required measurement, e.g., PRS symbols for certain hops are punctured due to imperfect RF retuning, or some repetitions of PRS are muted. Therefore, we don’t think both mechanisms can be used together. In the measurement report, an indication may be needed to distinguish measurements based on multiple hops or one hop.

- An indication in measurement report may be needed to distinguish measurements based on multiple hops or one hop.

SRS for positioning frequency hopping

SRS for positioning frequency hopping design

Regarding SRS for positioning frequency hopping, the following agreement was achieved.

Agreement SRS Tx Frequency hopping is supported for both RRC_CONNECTED and RRC_INACTIVE state. Agreement For the SRS Tx hopping pattern configuration support at least the staircase pattern, including a wrapped staircase pattern. - Support configuring the starting PRB of the first hop - FFS: details of signalling of PRB overlap across consecutive hops and bandwidth of each hop Agreement Response to RAN4 on the need of additional switching time: - It is RAN1’s view that for UL SRS for positioning Tx frequency hopping, switching time before the first hop and after the last hop need to be defined for the SRS for positioning with Tx frequency hopping. - RAN4 is kindly requested to evaluate the applicable switching time (if any) required ahead of the first hop and after the last hop, considering potential differences (in e.g. SCS, bandwidth, CP) between initial/active UL BWP and UL SRS for positioning Tx frequency hopping.

It was agreed that support configuring the starting PRB of the first hop. Based on the above agreements, it seems that the first hop here seems to be the first hop in time domain. However, in the agreed CR, corresponding descriptions are captured as ‘the starting PRB of the first frequency hop’. So, further clarification regarding ‘the starting PRB of the first hop’ in pervious agreement is needed. In our view, to align with the agreed CR, the agreement can be updated as ‘Support configuring the starting PRB of the first frequency hop’.

The reduced capability UE may be configured via [higher layer parameter], subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration. The reduced capability UE transmit frequency hopping is configured to perform within one SRS resource for positioning, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping may be configured with overlapping or non-overlapping frequency hops in the frequency domain. When the reduced capability UE is configured to perform transmit frequency hopping it expects to be configured via [higher layer parameter] with the starting PRB of the first frequency hop.

- Clarify that ‘the first hop’ for starting PRB configuration in previous agreement is ‘the first frequence hop’.

- Update the previous agreement as the following.

Agreement For the SRS Tx hopping pattern configuration support at least the staircase pattern, including a wrapped staircase pattern. - Support configuring the starting PRB of the first frequency hop - FFS: details of signalling of PRB overlap across consecutive hops and bandwidth of each hop

Frequency domain design

Figure 2 Regular staircase pattern and wrapped staircase pattern of 4 hops

It was agreed in RAN1#113, SRS Tx hopping pattern support the staircase pattern as the above figure and configuring the starting PRB of the first frequency hop. In our view, the starting PRB of the first frequency hop can reuse the parameter freqDomainShift nshift as Rel-15/16/17 SRS, which previously represented the frequency domain starting position of the SRS.

In addition, to further determine the frequency domain position of remaining hops, the configuration should also include the following parameters:

- Bandwidth of each hop

- Overlapping bandwidth

- Number of hops

- Subband index of the first hop, it should be noted that the first hop here is first time hop

Due to wrapped staircase pattern, without the subband index of the first hop, it may be hard to derive the frequency location of following hops. For the subband index of each hop, subband index#0 indicates the subband with the lowest position in the frequency domain, and increases sequentially.

The following is an example of frequency domain mapping of multiple hops, trying to reuse the mapping framework in TS38.211.

The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC noffsetFH=(n1+nSRS)mod NhopMhopSRS-MoverlapSRSNscRB Where k0(pi) in the frequency domain position of the first frequency hop, nshift is the starting PRB of the first frequency hop. noffsetFH is the frequency offset of each hop, where - n1 is the subband index of the first hop(first time hop) - Nhop is the number of hops - MhopSRS is bandwith of each hop - MoverlapSRS is the overlapping bandwidth between hops - nSRS is the quantity associated with time domain characteristic for each hop, which counts the number of SRS transmissions for each SRS symbol of each hop. It is an existing quantity for SRS resource mapping.

- The starting PRB of the first frequency hop can reuse the parameter freqDomainShift nshift in TS38.211 as Rel-15/16/17 SRS, which previously denotes the frequency domain starting position of the SRS resource.

- Support the following parameters for SRS for positioning frequency hopping:

- Bandwidth of each hop

- Overlapping bandwidth

- Number of hops

- Subband index of the first hop (first time hop)

- The following can be supported example of frequency domain mapping of multiple hops.

The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC noffsetFH=(n1+nSRS)mod NhopMhopSRS-MoverlapSRSNscRB Where k0(pi) in the frequency domain position of the first frequency hop, nshift is the starting PRB of the first frequency hop. noffsetFH is the frequency offset of each hop, where - n1 is the subband index of the first hop (first time hop) - Nhop is the number of hops - MhopSRS is bandwith of each hop - MoverlapSRS is the overlapping bandwidth between hops - nSRS is the quantity associated with time domain characteristic for each hop, which counts the number of SRS transmissions for each SRS symbol of each hop. It is an existing quantity for SRS resource mapping.

Time domain design

As discussed above, existing nSRS is the quantity associated with time domain characteristic for each hop, which counts the number of SRS transmissions for each SRS symbol of each hop. Through nSRS, we can establish the relationship between each symbol of each hop and the hop subband, and further determine the starting PRB of each symbol of each hop in the frequency domain. And for the time domain design, the purpose is to determine the value of nSRS corresponding to each symbol of each hop.

- For SRS for positioning frequency hopping, support reusing the quantity nSRS, which is associated with time domain characteristic for each hop and counts the number of SRS transmissions for each SRS symbol of each hop.

From the time domain perspective, the design of frequency hopping may include the design intra-slot frequency hopping and inter-slot frequency hopping. In our view, the design principle should try to reuse the parameters/quantities of the existing SRS mapping to avoid unnecessary normative effort.

For intra-slot frequency hopping design, it may include the following components.

- Intra-slot time gap between adjacent hops within a SRS resource should be introduced considering switching time between adjacent hops. So, with the introduction of intra-slot time gap, the following components should be considered.

- Indication of symbol-level ‘time gap’, e.g., introduce a parameter NsymbSRS_gap symbol-level ‘time gap’.

- Update the following concept of NsymbSRS, since OFDM symbols for an SRS resource are not fully consecutive due to ‘time gap’ between hops.

- NsymbSRS∈1,2,4,8,10,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping

- Reuse quantity l'∈0,1,…,NsymbSRS-1, which is the OFDM symbol number within a slot for the SRS resource

- Update the quantity ‘l'+l0’ considering ‘time gap’ which denotes the actual symbol position within a slot for a SRS resource.

e.g., the existing quantity ‘l'+l0’ as the following used for SRS resource mapping should be updated

aKTCk'+k0(pi),l'+l0(pi)=1NapβSRSr(pi)(k',l')k'=0,1,…,Msc,bSRS-1l'=0,1,...,NsymbSRS-10otherwise

e.g., existing l'+l0 may be updated to  →  l'/R*NsymbSRS_gap+l'+l0,

where R is an existing parameter which can be reused to denote that one hop occupies R adjacent OFDM symbols as discussed in the following, NsymbSRS_gap is the number of symbols of time gap between adjacent hops.

- Reuse the repetition factor R to denote that one hop occupies R adjacent OFDM symbols

- The quantity R can be used to calculate the quantity nSRS, which counts the number of SRS transmissions. Additionally, it can help to determine the position of time gap within a slot.

- For intra-slot frequency hopping, nSRS= l'/R.

- Reuse the Table of the quantity koffsetl' (that is, the table of relative RE offset), if there is no change towards the quantity l'∈0,1,…,NsymbSRS-1.

Table 6.4.1.4.3-2: The offset koffsetl' for SRS as a function of KTC and l'.

| KTC | koffset0, …, koffsetNsymbSRS-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NsymbSRS=1 | NsymbSRS=2 | NsymbSRS=4 | NsymbSRS=8 | NsymbSRS=12 |
| 2 | 0 | 0,1 | 0,1,0,1 | - | - |
| 4 | - | 0, 2 | 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3, 0, 2, 1, 3 |
| 8 | - | - | 0, 4, 2, 6 | 0, 4, 2, 6, 1, 5, 3, 7 | 0, 4, 2, 6, 1, 5, 3, 7, 0, 4, 2, 6 |

The following figure is an example of intra-slot frequency hopping, where NsymbSRS=4, l'∈0,1,…,3, R=2 and koffsetl'={0,1,0,1}, time gap=2 symbols.

Figure 3 4-symbol SRS resource within a slot for intra-slot frequency hopping

For some cases with larger switching time (e.g., 210us), larger comb size and number of symbols for the SRS resource, or multiple SRS resources within a slot, frequency hopping cannot be completed within a slot and inter-slot frequency hopping should be supported.

For inter-slot frequency hopping design, it may include the following components

- Inter-slot repetition within a SRS resource should be introduced.

- The inter-slot repetition mechanism of DL PRS can be reused. E.g., introduce inter-slot repetition factor and corresponding time gap for a SRS resource like DL PRS.

- Update the following function of the quantity nSRS to further consider inter-slot repetition within a SRS resource, since it only considers inter-slot hopping across different periodical SRS instances.

Nslotframe,μnf+ns,fμ-ToffsetTSRS⋅NsymbSRSR+l'R  may be updated to →

Nslotframe,μnf+ns,fμ-Toffset mod TSRSTgap,slotSRSNsymbSRSR+l'R

Where

(1) Nslotframe,μnf+ns,fμ-ToffsetSRS-Toffset,resSRS mod TperSRSTgap,slotSRS denotes the inter-slot repetition index, where Nslotframe,μis the number of slots within a radio frame, nf is the radio frame index, ns,fμ is the slot index of the SRS resource within a radio frame, is Toffsetthe period offset of SRS resource, TSRS is the SRS resource period, Tgap,slotSRS is the intra-slot time gap of the SRS resource.

(2) NsymbSRSR denotes number of hops within a slot, where NsymbSRS is the number of symbols of the SRS resource, R is the number of symbols of each hop.

(3) l'R denotes the hop index within a slot, where l’ is the OFDM symbol number within a slot for the SRS resource.

- For intra-slot time domain design of SRS for positioning frequency hopping, support the following:

- Reuse following parameters and mechanism for SRS frequency hopping

- The quantity of OFDM symbol number within a slot for an SRS resource: l'∈0,1,…,NsymbSRS-1

- Table of the koffsetl'  , that is, the Table of Relative RE Offset

- Repetition factor R, which denotes the number of OFDM symbols of each hop

- The function of the quantity nSRS for intra-slot hopping

nSRS= l'/R

- Introduce symbol-level ‘time gap’ within an SRS resource

- Update the existing concept of NsymbSRS for ‘concecutive OFDM symbols’, considering symbol-level ‘time gap’

- Update symbol position within a slot for an SRS resource: l'+l0, considering symbol-level ‘time gap’, e.g,

- Update  l'+l0 to l'/R*NsymbSRS_gap+l'+l0, where NsymbSRS_gapis the symbol-level time gap

- For inter-slot time domain design of SRS for positioning frequency hopping, support the following:

- Introduce inter-slot repetition like DL PRS within an SRS resource, just like DL PRS

- Update the function of the quantity nSRS considering inter-slot repetition within an SRS resource, e.g.,

nSRS=Nslotframe,μnf+ns,fμ-Toffset mod TSRSTgap,slotSRSNsymbSRSR+l'R Where (1) Nslotframe,μnf+ns,fμ-ToffsetSRS-Toffset,resSRS mod TperSRSTgap,slotSRS denotes the inter-slot repetition index, where Nslotframe,μis the number of slots within a radio frame, nf is the radio frame index, ns,fμ is the slot index of the SRS resource within a radio frame including repeated SRS slot, is Toffsetthe period offset of SRS resource, TSRS is the SRS resource period, Tgap,slotSRS is the inter-slot time gap of the SRS resource. (2) NsymbSRSR denotes number of hops within a slot, where NsymbSRS is the number of symbols of the SRS resource, R is the number of symbols of each hop. (3) l'R denotes the hop index within a slot, where l’ is the OFDM symbol number within a slot for the SRS resource.

Overlapping bandwidth sizes between hops

In this subsection, different bandwidths of overlapping bandwidth between hops are evaluated with the assumption of  the same RS overhead for 5 hops.

Figure 4 Frequency hopping performance with different overlapping bandwidth

Table 1 Evaluation results for frequency hopping with different overlapping bandwidths

| Method | Pattern | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Hopping with phase error compensation | 1PRB | 0.07 | 0.1 | 0.12 | 0.19 |
|  | 4PRB | 0.04 | 0.06 | 0.07 | 0.14 |
|  | 8PRB | 0.05 | 0.06 | 0.07 | 0.1 |
|  | 12PRB | 0.06 | 0.07 | 0.09 | 0.13 |
|  | 16PRB | 0.07 | 0.09 | 0.11 | 0.17 |

Based on the evaluation results, we observe that

- In InF-SH scenario, with a small overlapping bandwidth of 1 PRB or 4 PRBs, the performance of frequency hopping is sufficient to meet the requirement of RedCap positioning.

- Under the assumption of same RS overhead, as the overlapping bandwidth increases, the accuracy will increase within a certain range (e.g., 8 PRBs) because of the increased performance of phase error compensation; but beyond this range (e.g., 8 PRBs), the accuracy will decrease due to the decrease in the bandwidth span of frequency hopping.

- For the sizes of overlapping bandwidth for different hops, the balance between phase error compensation performance and bandwidth span of frequency hopping should be considered.

- A size smaller than 8 PRBs can be supported

Collision rules of SRS for positioning frequency hopping

Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to [receive/]transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without UL time window (i.e. option 1) - FFS: details on the collision rules - Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group.

For Option 1, SRS along with the switching time is treated as higher priority than other signals/channels. In our view, this case may be applied to the scenarios with very low latency requirement where the UE needs to transmit multiple SRS hop within a short time and treats multiple hops as a whole transmission rather than independent transmissions. For example, the UE needs to perform continuous frequency hopping in a short period of time (e.g, FH within a slot as the following figure), the time gap between hops is only used for hop switching without any other signals/channel plugged in. When collision with other UL signals/channels happens in a subset of symbols of frequency hopping, UE is only expected to transmit FH SRS for positioning. For the UL time window, the period can be the same as SRS resource(s) and the start position can be a symbol with a distance T before the first Symbol, and the end position can be a symbol with a distance T later than the last symbol of the last hop, wherein, T is the switch time between active BWP and FH transmission. Regarding [receive] other DL signals/channels, our view is to delete the [receive]. For TDD and FDD case, DL reception will not be affected even within the UL time window; while for HD-UE case, when collision happens, we don’t think all the DL signals/channels should be dropped, since there is still some DL signals/channels with higher priority, such as SSB.

Figure 5 SRS frequency hopping within a slot with high priority

- Support the following modification for UL time window of FH SRS for positioning.

- Option 1: UL time window where the UE is not expected to [receive/]transmit other signals/channels and is only expected to transmit FH SRS for positioning.

Option 2 provides more flexibility for the network to handle SRS FH with other signal/channel priorities, which can be consistent with the rules of Rel-16/17. In Rel-16/17, regardless of whether it is in the RRC_CONNECTED state or the RRC_INACTIVE state, whether along with switching time or not, SRS has always been a low priority and dropped.

For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs.

If the transmission of SRS for positioning outside the initial BWP in RRC_INACTIVE mode along with the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the SRS for positioning transmission is dropped in the symbol(s) where the collision occurs.

For Option 2, the collision rules of R16/17 should be reused as much as possible, and corresponding enhancements considering the new issues for frequency hopping should be made based on this.

For SRS for positioning frequency hopping, the dropping rules with other UL and DL signals/channels may have some differences compared to legacy dropping rules of SRS for positioning. For legacy dropping rules of SRS for positioning, the SRS is generally dropped in the symbols where the collision occurs, e.g., for operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs. But for SRS for positioning frequency hopping, it may not be suitable for UE to only drop the affected symbols when collision happens in some cases.

- Alt 1: UE drops all the SRS hops

One case is that the UE treats multiple hops as a whole transmission rather than independent transmissions, such as multiple hops within a UL time window, so the dropping rule is also applied to multiple hops. For example, the UE needs to perform continuous frequency hopping in a short period of time (e.g, in a slot), the gap between hops is only used for hop switching. When collision with other DL or UL signals/channels happens in a subset of symbols of frequency hopping, it is natural to drop all the SRS hops.

Or, due to insufficient UE capability or low complexity requirement, the UE cannot accept switching from frequency hopping transmission to active BWP for other signals/channels reception/transmission, and then switching back to frequency hopping transmission again, so that when collision happens, it is better to drop all the SRS hops.

Or, the collision symbols may largely affect the performance of frequency hopping, which makes it unnecessary for other hops to be transmitted, so all the SRS hops can be dropped. For example, one hop is an intermediate hop, which is very important for compensating phase errors. If the symbols in this hop are affected, it will cause a huge performance loss for frequency hopping.

Regarding details of dropping all SRS hops, it may include the following cases:

- Case 1: UE drops all the symbols of all SRS hops, when it determines the presence of other signals/channels before the starting symbol of the first hop (including potential switching time).

- Case 2: UE drops remaining symbols of all SRS hops after a time, when it determines the presence of other signals/channels after the starting symbol of the first hop (including potential switching time).

- Alt 2: UE drops affected hop(s)

Alt 2 may be applied to the case where multiple hops are relative independent, but within a hop, UE treats multiple SRS symbols as a whole transmission, so the dropping rule is applied to each hop. When collision happens, the UE only drop affected hops, instead of all hops. For example, DL or UL signals/channels in active BWP can be interspersed between adjacent hops. In addition, dropping the affected hops to receive/transmit other signals/channels does not increase the complexity of the UE and within the UE capability. So, there is no need to drop all the SRS hops.

Or, the collision symbols have minor impact on the performance of frequency hopping. For example, one hop is the last hop within multiple hops, even without this hop, other hops can be used to estimate the phase error and get sufficient performance. So, only the affected hops can be dropped.

Regarding details of dropping affected hop(s), it may include the following cases:

- Case 1: UE drops all the SRS symbols of the hop, when it determines the presence of other signals/channels before the starting symbol of the hop (including potential switching time).

- Case 2: UE drops remaining symbols of the hops after a time, when it determines the presence of other signals/channels after the starting symbol of the hop (including potential switching time).

- Alt 3: UE drops affected symbol(s)

Alt 3 is the most flexible, which is the same as current dropping rule for SRS transmission. However, it may bring higher UE complexity and require higher UE capability. For example, symbol-level dropping may cause UE to frequently switch from active BWP to SRS hops, which is not complexity and power saving friendly.

In addition, the other signals/channels which may collide with SRS for positioning frequency hopping can include:

- Other UL signals/channels

- DL signals/channels in TDD

- DL signals/channels for HD-FD UEs in FDD.

For Option 2, if UE capability support all 3 alternatives, the gNB can further indicate which alternative is applied when collision happens.

- For Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels, SRS with frequency hopping is dropped.

- The dropping rules should include the following based on different scenarios and UE capabilities

- Alt 1: UE drops all the SRS hops

- Alt 2: UE drops affected hop(s)

- Alt 3: UE drops affected symbol(s)

- If UE capability support all 3 alternatives, the gNB can further indicate which alternative is applied when collision happens

- The other DL/UL reception/transmission should include other UL signals/channels, DL signals/channels in TDD, DL signals/channels for half-duplex UE (HD-UE) in FDD

Conclusion

In this contribution, we discuss RedCap positioning with the following observation and proposals.

- In InF-SH scenario, with a small overlapping bandwidth of 1 PRB or 4 PRBs, the performance of frequency hopping is sufficient to meet the requirement of RedCap positioning.

- Under the assumption of same RS overhead, as the overlapping bandwidth increases, the accuracy will increase within a certain range (e.g., 8 PRBs) because of the increased performance of phase error compensation; but beyond this range (e.g., 8 PRBs), the accuracy will decrease due to the decrease in the bandwidth span of frequency hopping.

- For PRS Rx frequency hopping, PPW-based method is not supported.

- An indication in measurement report may be needed to distinguish measurements based on multiple hops or one hop.

- Clarify that ‘the first hop’ for starting PRB configuration in previous agreement is ‘the first frequence hop’.

- Update the previous agreement as the following.

- The starting PRB of the first frequency hop can reuse the parameter freqDomainShift nshift in TS38.211 as Rel-15/16/17 SRS, which previously denotes the frequency domain starting position of the SRS resource.

- Support the following parameters for SRS for positioning frequency hopping:

- Bandwidth of each hop

- Overlapping bandwidth

- Number of hops

- Subband index of the first hop (first time hop)

- The following can be supported example of frequency domain mapping of multiple hops.

The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC noffsetFH=(n1+nSRS)mod NhopMhopSRS-MoverlapSRSNscRB Where k0(pi) in the frequency domain position of the first frequency hop, nshift is the starting PRB of the first frequency hop. noffsetFH is the frequency offset of each hop, where - n1 is the subband index of the first hop (first time hop) - Nhop is the number of hops - MhopSRS is bandwith of each hop - MoverlapSRS is the overlapping bandwidth between hops - nSRS is the quantity associated with time domain characteristic for each hop, which counts the number of SRS transmissions for each SRS symbol of each hop. It is an existing quantity for SRS resource mapping.

- For SRS for positioning frequency hopping, support reusing the quantity nSRS, which is associated with time domain characteristic for each hop and counts the number of SRS transmissions for each SRS symbol of each hop.

- For intra-slot time domain design of SRS for positioning frequency hopping, support the following:

- Reuse following parameters and mechanism for SRS frequency hopping

- The quantity of OFDM symbol number within a slot for an SRS resource: l'∈0,1,…,NsymbSRS-1

- Table of the koffsetl'  , that is, the Table of Relative RE Offset

- Repetition factor R, which denotes the number of OFDM symbols of each hop

- The function of the quantity nSRS for intra-slot hopping

nSRS= l'/R

- Introduce symbol-level ‘time gap’ within an SRS resource

- Update the existing concept of NsymbSRS for ‘concecutive OFDM symbols’, considering symbol-level ‘time gap’

- Update symbol position within a slot for an SRS resource: l'+l0, considering symbol-level ‘time gap’, e.g,

- Update  l'+l0 to l'/R*NsymbSRS_gap+l'+l0, where NsymbSRS_gapis the symbol-level time gap

- For inter-slot time domain design of SRS for positioning frequency hopping, support the following:

- Introduce inter-slot repetition like DL PRS within an SRS resource, just like DL PRS

- Update the function of the quantity nSRS considering inter-slot repetition within an SRS resource, e.g.,

nSRS=Nslotframe,μnf+ns,fμ-Toffset mod TSRSTgap,slotSRSNsymbSRSR+l'R Where (1) Nslotframe,μnf+ns,fμ-ToffsetSRS-Toffset,resSRS mod TperSRSTgap,slotSRS denotes the inter-slot repetition index, where Nslotframe,μis the number of slots within a radio frame, nf is the radio frame index, ns,fμ is the slot index of the SRS resource within a radio frame including repeated SRS slot, is Toffsetthe period offset of SRS resource, TSRS is the SRS resource period, Tgap,slotSRS is the inter-slot time gap of the SRS resource. (2) NsymbSRSR denotes number of hops within a slot, where NsymbSRS is the number of symbols of the SRS resource, R is the number of symbols of each hop. (3) l'R denotes the hop index within a slot, where l’ is the OFDM symbol number within a slot for the SRS resource.

- For the sizes of overlapping bandwidth for different hops, the balance between phase error compensation performance and bandwidth span of frequency hopping should be considered.

- A size smaller than 8 PRBs can be supported

- Support the following modification for UL time window of FH SRS for positioning.

- Option 1: UL time window where the UE is not expected to [receive/]transmit other signals/channels and is only expected to transmit FH SRS for positioning.

- For Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels, SRS with frequency hopping is dropped.

- The dropping rules should include the following based on different scenarios and UE capabilities

- Alt 1: UE drops all the SRS hops

- Alt 2: UE drops affected hop(s)

- Alt 3: UE drops affected symbol(s)

- If UE capability support all 3 alternatives, the gNB can further indicate which alternative is applied when collision happens

- The other DL/UL reception/transmission should include other UL signals/channels, DL signals/channels in TDD, DL signals/channels for half-duplex UE (HD-UE) in FDD

- References

- RP-223549, “New WID on Expanded and Improved NR Positioning”

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 112bis-e, e-Meeting, April 17th – April 26th, 2023

- R1-2302496, Discussion on positioning for RedCap UEs, vivo, RAN1#112bis-e, April 17th – April 26th, 2023

- R4-2306659, LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs, RAN4#106bis-e, April 17th – April 26th, 2023