3GPP TSG RAN WG1 #114		                                                       R1-2307083

Toulouse, France, August 21st – 25th, 2023

Source:	CATT

Title:	SBFD evaluation results

Agenda Item:	9.3.1

Document for:	Decision

- Introduction

In this contribution, updated SLS results of SBFD deployment case1 in FR1 are provided.

- SLS results

  - Indoor office

    - SBFD Alt 2

      - Large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

Table 1: Evaluation results of SBFD Alt 2 for indoor in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 336.84 | 279.85 | -16.92 | 225.83 | 179.82 | -20.37 | 90.71 | 71.03 | -21.69 |
|  | 5% | 245 .65 | 192.71 | -21.55 | 162.63 | 121.44 | -25.33 | 74.80 | 52.97 | -29.18 |
|  | 50% | 327.14 | 291.86 | -10.78 | 224.90 | 185.76 | -17.40 | 89.86 | 71.63 | -20.28 |
|  | 95% | 428.15 | 341.75 | -20.18 | 272.74 | 220.00 | -19.34 | 106.77 | 83.12 | -22.15 |
| DL Tail-UPT CDF (Mbps) | Mean | 218.61 | 196.07 | -10.31 | 144.31 | 129.03 | -10.59 | 65.17 | 53.90 | -17.30 |
|  | 5% | 136.24 | 134.16 | -1.53 | 99.20 | 83.71 | -15.61 | 50.10 | 42.62 | -14.93 |
|  | 50% | 221.85 | 202.31 | -8.81 | 148.74 | 134.50 | -9.57 | 65.29 | 54.80 | -16.07 |
|  | 95% | 277.20 | 237.18 | -14.44 | 180.03 | 154.15 | -14.38 | 76.08 | 61.81 | -18.76 |
| DL Median-UPT CDF (Mbps) | Mean | 347.60 | 274.83 | -20.93 | 238.67 | 185.09 | -22.45 | 88.31 | 68.68 | -22.23 |
|  | 5% | 249.78 | 186.22 | -25.45 | 164.46 | 121.13 | -26.35 | 71.29 | 52.06 | -26.98 |
|  | 50% | 332.67 | 280.50 | -15.68 | 239.85 | 191.76 | -20.05 | 89.74 | 68.55 | -23.61 |
|  | 95% | 442.41 | 336.60 | -23.92 | 304.28 | 230.41 | -24.28 | 103.90 | 78.64 | -24.31 |
| UL Average-UPT CDF (Mbps) | Mean | 131.50 | 193.87 | 47.43 | 79.45 | 109.25 | 37.51 | 23.47 | 36.62 | 56.06 |
|  | 5% | 74.10 | 135.73 | 83.17 | 53.01 | 82.72 | 56.05 | 16.80 | 25.53 | 51.95 |
|  | 50% | 130.48 | 191.90 | 47.07 | 82.37 | 107.48 | 30.48 | 24.11 | 36.19 | 50.08 |
|  | 95% | 203.08 | 265.54 | 30.76 | 106.58 | 133.57 | 25.32 | 32.56 | 47.97 | 47.31 |
| UL Tail-UPT CDF (Mbps) | Mean | 84.96 | 143.68 | 69.11 | 47.05 | 70.51 | 49.86 | 18.23 | 26.34 | 44.47 |
|  | 5% | 50.68 | 96.25 | 89.92 | 29.44 | 45.11 | 53.23 | 11.54 | 18.58 | 60.97 |
|  | 50% | 82.39 | 140.28 | 70.26 | 46.97 | 72.17 | 53.65 | 18.72 | 26.69 | 42.58 |
|  | 95% | 103.99 | 188.06 | 80.84 | 61.45 | 84.10 | 36.86 | 21.16 | 31.59 | 49.32 |
| UL Median-UPT CDF (Mbps) | Mean | 122.09 | 188.71 | 54.57 | 77.77 | 114.63 | 47.40 | 22.82 | 34.14 | 49.63 |
|  | 5% | 75.30 | 121.23 | 61.00 | 46.26 | 79.68 | 72.24 | 16.80 | 26.38 | 57.01 |
|  | 50% | 125.88 | 184.53 | 46.59 | 83.94 | 112.18 | 33.64 | 23.55 | 33.93 | 44.05 |
|  | 95% | 159.89 | 250.99 | 56.98 | 107.24 | 149.75 | 39.64 | 30.47 | 41.43 | 35.99 |
| DL Packet-Latency CDF (ms) | Mean | 11.80 | 14.15 | 19.92 | 17.48 | 21.01 | 20.19 | 46.59 | 56.09 | 20.40 |
|  | 5% | 6.97 | 10.55 | 51.36 | 12.04 | 15.72 | 30.56 | 34.43 | 41.79 | 21.39 |
|  | 50% | 11.40 | 13.58 | 19.12 | 16.47 | 19.90 | 20.83 | 45.43 | 54.54 | 20.06 |
|  | 95% | 17.32 | 19.94 | 15.13 | 26.78 | 30.26 | 12.99 | 59.60 | 75.95 | 27.42 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 7.21 | 4.91 | -31.90 | 12.47 | 8.72 | -30.07 | 39.48 | 26.89 | -31.89 |
|  | 5% | 4.79 | 2.89 | -39.67 | 8.09 | 5.47 | -32.39 | 24.30 | 18.52 | -23.79 |
|  | 50% | 6.21 | 4.63 | -25.44 | 11.33 | 8.44 | -25.51 | 38.85 | 26.22 | -32.50 |
|  | 95% | 12.00 | 7.76 | -35.33 | 21.45 | 13.10 | -38.93 | 57.84 | 36.46 | -36.97 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.02 | 5.16 | -0.86 | 19.89 | 16.52 | -3.37 | 42.22 | 32.98 | -9.24 |
|  | Type-2 | 7.52 | 8.06 | 0.54 | 24.86 | 25.82 | 0.96 | 52.78 | 51.54 | -1.24 |
| UL RU (%) | Type-1 | 1.53 | 2.45 | 0.92 | 5.74 | 8.05 | 2.31 | 10.25 | 18.48 | 8.23 |
|  | Type-2 | 7.66 | 6.81 | -0.85 | 28.69 | 22.36 | -6.33 | 51.27 | 51.34 | 0.07 |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.20 |  | 1.10 | 1.00 |  | 5.00 | 5.10 |  |
|  | UL | 0.30 | 0.30 |  | 1.40 | 1.3 |  | 5.60 | 5.60 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 2 compared with legacy TDD based on the simulation results from Table 1:

•	SBFD with Alt 2 degrades mean,5%,50% and 95% of DL average-UPT CDF in the range of {-29.18%~-10.78%} and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {25.32%~83.17%} for different traffic loads.

•	SBFD with Alt 2 increases mean,5%,50% and 95% of DL packet-latency CDF in the range of {12.99%~51.36%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-39.67%~-23.79%} for different traffic loads.

Hence, SBFD improves UL average-UL UPT at all load conditions for indoor office than that of the legacy TDD at the cost of degradation of DL average-UPT, UL packet-latency of SBFD is decreased at all three load conditions for indoor office scenario at the cost of increased DL packet-latency. The gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD.

Observation 1: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 improves UL average-UPT at all three load conditions at the cost of degraded DL average-UPT.

Observation 2: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 decrease the UL packet-latency at the cost of increased DL packet-latency.

Observation 3: For indoor scenario (FR1) with SBFD Alt 2 and large packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD.

      - Small packet (4Kbytes for DL and 1Kbyte for UL)

Table 2: Evaluation results of SBFD Alt 2 for indoor in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.70 | 37.50 | -0.53 | 37.62 | 37.49 | -0.35 | 33.18 | 31.85 | -4.00 |
|  | 5% | 35.30 | 35.67 | 1.05 | 35.26 | 35.67 | 1.16 | 19.01 | 18.51 | -2.62 |
|  | 50% | 37.76 | 37.51 | -0.66 | 37.72 | 37.51 | -0.56 | 35.15 | 34.06 | -3.10 |
|  | 95% | 40.05 | 39.49 | -1.40 | 39.84 | 39.49 | -0.88 | 39.72 | 39.62 | -0.25 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.49 | 23.35 | -0.60 | 23.49 | 23.36 | -0.55 | 20.29 | 18.85 | -7.13 |
|  | 5% | 22.12 | 22.00 | -0.54 | 21.93 | 21.93 | 0.00 | 7.22 | 9.42 | 30.49 |
|  | 50% | 23.43 | 23.25 | -0.77 | 23.38 | 23.28 | -0.43 | 22.17 | 21.54 | -2.84 |
|  | 95% | 25.40 | 24.66 | -2.91 | 25.19 | 24.81 | -1.51 | 25.80 | 26.78 | 3.82 |
| DL Median-UPT CDF (Mbps) | Mean | 39.60 | 39.44 | -0.40 | 39.53 | 39.42 | -0.28 | 35.77 | 34.70 | -3.00 |
|  | 5% | 36.88 | 36.75 | -0.35 | 36.52 | 36.82 | 0.82 | 23.46 | 20.20 | -13.90 |
|  | 50% | 39.32 | 39.39 | 0.18 | 39.32 | 39.38 | 0.15 | 37.32 | 37.38 | 0.16 |
|  | 95% | 42.88 | 42.32 | -1.31 | 43.03 | 42.26 | -1.79 | 42.03 | 42.16 | 0.30 |
| UL Average-UPT CDF (Mbps) | Mean | 4.55 | 10.64 | 133.85 | 4.55 | 10.64 | 133.85 | 4.28 | 8.46 | 97.38 |
|  | 5% | 3.97 | 10.35 | 160.71 | 3.99 | 10.35 | 159.40 | 2.95 | 7.38 | 150.39 |
|  | 50% | 4.49 | 10.63 | 136.75 | 4.54 | 10.63 | 134.14 | 4.24 | 8.34 | 96.52 |
|  | 95% | 5.31 | 11.00 | 107.16 | 5.29 | 11.00 | 107.94 | 5.88 | 9.88 | 68.03 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.83 | 8.18 | 189.05 | 2.84 | 8.18 | 188.03 | 2.59 | 4.62 | 78.12 |
|  | 5% | 2.68 | 8.11 | 202.61 | 2.68 | 8.00 | 198.51 | 1.48 | 3.79 | 155.83 |
|  | 50% | 2.79 | 8.21 | 194.27 | 2.78 | 8.23 | 196.04 | 2.73 | 4.45 | 62.94 |
|  | 95% | 3.13 | 8.36 | 167.09 | 3.16 | 8.44 | 167.09 | 3.32 | 5.67 | 70.61 |
| UL Median-UPT CDF (Mbps) | Mean | 4.57 | 10.64 | 132.82 | 4.57 | 10.63 | 132.60 | 4.45 | 9.46 | 112.36 |
|  | 5% | 3.62 | 10.19 | 181.49 | 3.60 | 10.17 | 182.50 | 2.92 | 8.53 | 192.05 |
|  | 50% | 4.47 | 10.63 | 137.81 S | 4.49 | 10.60 | 136.08 | 4.14 | 9.39 | 126.68 |
|  | 95% | 5.87 | 11.23 | 91.31 | 5.81 | 11.20 | 92.77 | 6.89 | 10.67 | 54.92 |
| DL Packet-Latency CDF (ms) | Mean | 0.87 | 0.86 | -1.15 | 0.87 | 0.86 | -1.15 | 1.01 | 1.05 | 4.16 |
|  | 5% | 0.52 | 0.52 | 0.00 | 0.52 | 0.52 | 0.00 | 0.53 | 0.54 | 0.90 |
|  | 50% | 0.81 | 0.81 | 0.00 | 0.80 | 0.81 | 1.25 | 0.86 | 0.88 | 2.19 |
|  | 95% | 1.35 | 1.36 | 0.74 | 1.35 | 1.35 | 0.00 | 1.69 | 1.95 | 15.31 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.78 | 0.76 | -57.30 | 1.76 | 0.76 | -56.82 | 1.97 | 0.95 | -51.58 |
|  | 5% | 0.60 | 0.52 | -13.33 | 0.59 | 0.52 | -11.86 | 0.63 | 0.53 | -16.21 |
|  | 50% | 1.77 | 0.75 | -57.63 | 1.74 | 0.75 | -56.90 | 1.85 | 0.84 | -54.41 |
|  | 95% | 2.86 | 0.97 | -66.08 | 2.84 | 0.97 | -65.85 | 3.07 | 1.75 | -42.96 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.12 | 5.21 | -0.91 | 19.87 | 16.37 | -3.5 | 51.29 | 40.72 | -10.57 |
|  | Type-2 | 7.65 | 8.15 | 0.5 | 24.84 | 25.58 | 0.74 | 64.11 | 63.63 | -0.48 |
| UL RU (%) | Type-1 | 1.51 | 2.81 | 1.3 | 5.68 | 7.96 | 2.28 | 13.04 | 24.08 | 11.04 |
|  | Type-2 | 7.57 | 7 .81 | 0.24 | 28.4 | 22.12 | -6.28 | 65.19 | 66.88 | 1.69 |
| Unfinished/dropped Packet Rate (%) | DL | 0.2 | 0.3 |  | 1.2 | 1.1 |  | 5.50 | 5.70 |  |
|  | UL | 0.3 | 2.81 |  | 1.3 | 1.4 |  | 6.10 | 6.10 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 2 compared with legacy TDD based on the simulation results from Table 2:

•	SBFD with Alt 2 degrades mean,5%,50% and 95% of DL average-UPT CDF in the range of {-4.00%~-0.25%} except a improvement of   1.05% and 1.16% at 5%-tile of UL average-UPT CDF for low and median traffic load respectively, and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {68.03%~160.71%} for different traffic loads.

•	SBFD with Alt 2 increases or decreases mean,5%,50% and 95% of DL packet-latency CDF in the range of {-1.15%~15.31%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-66.08%~-11.86%} for different traffic loads.

Hence, SBFD improves UL average-UL UPT at all load conditions for indoor office than that of the legacy TDD and slightly improve or degrade DL average-UPT, UL packet-latency of SBFD is decreased at all load conditions for indoor office scenario and DL packet-latency is slightly decreased or increased. The gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD, the loss of DL average-UPT at least comes from less DL resources for SBFD compared to legacy TDD and the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

Observation 4: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 improve UL average-UPT at all load conditions at the cost of degraded DL average-UPT.

Observation 5: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 reduce the UL packet-latency at the cost of increased DL packet-latency.

Observation 6: For indoor scenario (FR1) with SBFD Alt 2 and small packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD, the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD and the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

    - SBFD Alt 4

      - Large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

Table 3: Evaluation results of SBFD Alt 4 for indoor in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 336.84 | 392.24 | 16.45 | 225.83 | 242.57 | 7.41 | 90.71 | 93.95 | 3.58 |
|  | 5% | 245.65 | 264.30 | 7.59 | 162.63 | 192.06 | 18.10 | 74.80 | 76.11 | 1.75 |
|  | 50% | 327.14 | 407.96 | 24.71 | 224.90 | 243.91 | 8.45 | 89.86 | 93.20 | 3.72 |
|  | 95% | 428.15 | 487.53 | 13.87 | 272.74 | 299.90 | 9.96 | 106.77 | 110.96 | 3.92 |
| DL Tail-UPT CDF (Mbps) | Mean | 218.61 | 248.67 | 13.75 | 144.31 | 158.71 | 9.98 | 65.17 | 69.89 | 7.24 |
|  | 5% | 136.24 | 165.47 | 21.45 | 99.20 | 106.23 | 7.09 | 50.10 | 51.17 | 2.13 |
|  | 50% | 221.85 | 259.21 | 16.84 | 148.74 | 166.54 | 11.97 | 65.29 | 70.43 | 7.87 |
|  | 95% | 277.20 | 306.23 | 10.47 | 180.03 | 199.21 | 10.65 | 76.08 | 82.16 | 7.99 |
| DL Median-UPT CDF (Mbps) | Mean | 347.60 | 392.37 | 12.88 | 238.67 | 265.63 | 11.30 | 88.31 | 92.02 | 4.20 |
|  | 5% | 249.78 | 275.09 | 10.13 | 164.46 | 196.69 | 19.60 | 71.29 | 75.06 | 5.29 |
|  | 50% | 332.67 | 389.67 | 17.13 | 239.85 | 266.55 | 11.13 | 89.74 | 90.71 | 1.08 |
|  | 95% | 442.41 | 493.50 | 11.55 | 304.28 | 334.80 | 10.03 | 103.90 | 109.93 | 5.81 |
| UL Average-UPT CDF (Mbps) | Mean | 130.32 | 161.12 | 23.63 | 79.45 | 93.66 | 17.89 | 23.47 | 26.50 | 12.93 |
|  | 5% | 73.17 | 112.24 | 53.40 | 53.01 | 65.12 | 22.84 | 16.80 | 23.33 | 38.85 |
|  | 50% | 130.48 | 157.09 | 20.39 | 82.37 | 95.77 | 16.27 | 24.11 | 26.49 | 9.86 |
|  | 95% | 203.08 | 227.10 | 11.83 | 106.58 | 110.21 | 3.41 | 32.56 | 29.81 | -8.46 |
| UL Tail-UPT CDF (Mbps) | Mean | 84.96 | 106.29 | 25.11 | 47.05 | 57.32 | 21.83 | 18.23 | 21.10 | 15.73 |
|  | 5% | 50.68 | 80.22 | 58.29 | 29.44 | 45.23 | 53.63 | 11.54 | 17.54 | 51.96 |
|  | 50% | 82.39 | 104.91 | 27.33 | 46.97 | 57.62 | 22.67 | 18.72 | 21.45 | 14.58 |
|  | 95% | 103.99 | 125.90 | 21.07 | 61.45 | 67.14 | 9.26 | 21.16 | 23.04 | 8.90 |
| UL Median-UPT CDF (Mbps) | Mean | 122.09 | 169.24 | 38.62 | 77.77 | 94.94 | 22.08 | 22.82 | 25.23 | 10.58 |
|  | 5% | 75.30 | 125.19 | 66.25 | 46.26 | 72.49 | 56.70 | 16.80 | 22.32 | 32.84 |
|  | 50% | 125.88 | 171.71 | 36.41 | 83.94 | 95.62 | 13.91 | 23.55 | 25.37 | 7.71 |
|  | 95% | 159.89 | 190.42 | 19.09 | 107.24 | 115.48 | 7.68 | 30.47 | 28.16 | -7.57 |
| DL Packet-Latency CDF (ms) | Mean | 11.80 | 10.50 | -11.02 | 17.48 | 16.41 | -6.12 | 46.59 | 43.71 | -6.18 |
|  | 5% | 6.97 | 6.56 | -5.88 | 12.04 | 11.96 | -0.66 | 34.43 | 33.96 | -1.35 |
|  | 50% | 11.40 | 9.66 | -15.26 | 16.47 | 15.39 | -6.56 | 45.43 | 42.74 | -5.91 |
|  | 95% | 17.32 | 16.97 | -2.02 | 26.78 | 25.38 | -5.23 | 59.60 | 56.45 | -5.29 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 7.21 | 5.79 | -19.69 | 12.47 | 11.02 | -11.63 | 39.48 | 36.37 | -7.88 |
|  | 5% | 4.79 | 3.42 | -28.60 | 8.09 | 6.55 | -19.04 | 24.30 | 26.57 | 9.34 |
|  | 50% | 6.21 | 5.58 | -10.14 | 11.33 | 10.84 | -4.32 | 8.85 | 36.52 | -5.99 |
|  | 95% | 12.00 | 8.86 | -26.17 | 21.45 | 15.60 | -27.29 | 57.84 | 44.38 | -23.27 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.02 | 5.53 | -0.49 | 19.89 | 19.16 | -0.73 | 42.22 | 41.26 | -0.96 |
|  | Type-2 | 7.52 | 6.91 | -0.61 | 24.86 | 23.95 | -0.91 | 2.78 | 51.57 | -1.21 |
| UL RU (%) | Type-1 | 1.53 | 1.65 | 0.12 | 5.74 | 5.53 | -0.21 | 10.25 | 10.17 | -0.08 |
|  | Type-2 | 7.66 | 8.22 | 0.56 | 28.69 | 27.65 | -1.04 | 51.27 | 50.84 | -0.43 |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.20 |  | 1.10 | 1.20 |  | 5.00 | 4.90 |  |
|  | UL | 0.3 | 0.3 |  | 1.4 | 1.3 |  | 5.60 | 5.80 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 4 compared with legacy TDD based on the simulation results from Table 3:

•	SBFD with Alt 4 improves mean,5%,50% and 95% of DL average-UPT CDF in the range of {1.75%~24.71%} and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {3.41%~53.4%} for different traffic loads, except a degradation of -8.46% at 95%-tile of UL average-UPT CDF for high traffic load.

•	SBFD with Alt 4 decreases mean,5%,50% and 95% of DL packet-latency CDF in the range of {-15.26%~-0.66%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-28.6%~-4.32%} for different traffic loads, except a improvement of 9.34% at 5%-tile of UL packet-latency CDF for high traffic load.

Hence, compared to legacy TDD, SBFD with Alt 4 can improve the UL average-UPT at low/medium load conditions and DL average-UPT at all load conditions, decreases DL packet-latency at all load conditions and reduce UL packet-latency at low and median loads. The gain of UL average-UPT at least comes from more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the gain of DL average-UPT for SBFD at least comes from more DL transmission opportunities and reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

Observation 7: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 improves UL average-UPT at low/medium load conditions and improves DL average-UPT at all load conditions.

Observation 8: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 decrease DL packet-latency at all load conditions and decrease UL packet-latency at low and median traffic loads.

Observation 9: For indoor scenario (FR1) with SBFD Alt 4 and large packet size, the gain of UL average-UPT at least comes from more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the gain of DL average-UPT for SBFD at least comes from more DL transmission opportunities and reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

      - Small packet (4Kbytes for DL and 1Kbyte for UL)

Table 4: Evaluation results of SBFD Alt 4 for indoor in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.70 | 42.76 | 13.42 | 37.62 | 42.64 | 13.34 | 33.18 | 36.50 | 10.01 |
|  | 5% | 35.30 | 41.58 | 17.79 | 35.26 | 41.59 | 17.95 | 19.01 | 20.83 | 9.61 |
|  | 50% | 37.76 | 42.81 | 13.37 | 37.72 | 42.69 | 13.18 | 35.15 | 39.88 | 13.46 |
|  | 95% | 40.05 | 43.92 | 9.66 | 39.84 | 43.64 | 9.54 | 39.72 | 43.25 | 8.90 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.49 | 32.68 | 39.12 | 23.49 | 32.67 | 39.08 | 20.29 | 24.97 | 23.05 |
|  | 5% | 22.12 | 32.42 | 46.56 | 21.93 | 32.36 | 47.56 | 7.22 | 11.10 | 53.77 |
|  | 50% | 23.43 | 32.85 | 40.20 | 23.38 | 32.88 | 40.63 | 22.17 | 25.80 | 16.42 |
|  | 95% | 25.40 | 33.55 | 32.09 | 25.19 | 33.51 | 33.03 | 25.80 | 33.21 | 28.74 |
| DL Median-UPT CDF (Mbps) | Mean | 39.60 | 42.82 | 8.13 | 39.53 | 42.91 | 8.55 | 35.77 | 38.12 | 6.58 |
|  | 5% | 36.88 | 40.66 | 10.25 | 36.52 | 41.06 | 12.43 | 23.46 | 22.15 | -5.61 |
|  | 50% | 39.32 | 42.79 | 8.83 | 39.32 | 43.00 | 9.36 | 37.32 | 40.45 | 8.39 |
|  | 95% | 42.88 | 44.74 | 4.34 | 43.03 | 44.71 | 3.90 | 42.03 | 45.26 | 7.68 |
| UL Average-UPT CDF (Mbps) | Mean | 4.55 | 10.71 | 135.38 | 4.55 | 10.69 | 134.95 | 4.28 | 7.79 | 81.77 |
|  | 5% | 3.97 | 10.02 | 152.39 | 3.99 | 10.06 | 152.13 | 2.95 | 6.25 | 112.14 |
|  | 50% | 4.49 | 10.65 | 137.19 | 4.54 | 10.65 | 134.58 | 4.24 | 7.78 | 83.29 |
|  | 95% | 5.31 | 11.47 | 116.01 | 5.29 | 11.35 | 114.56 | 5.88 | 9.37 | 59.41 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.83 | 8.23 | 190.81 | 2.84 | 8.22 | 189.44 | 2.59 | 4.27 | 64.52 |
|  | 5% | 2.68 | 7.98 | 197.76 | 2.68 | 8.02 | 199.25 | 1.48 | 3.14 | 111.79 |
|  | 50% | 2.79 | 8.24 | 195.34 | 2.78 | 8.22 | 195.68 | 2.73 | 4.20 | 53.90 |
|  | 95% | 3.13 | 8.65 | 176.36 | 3.16 | 8.65 | 173.73 | 3.32 | 5.63 | 69.28 |
| UL Median-UPT CDF (Mbps) | Mean | 4.57 | 10.72 | 134.57 | 4.57 | 10.72 | 134.57 | 4.45 | 8.91 | 100.04 |
|  | 5% | 3.62 | 10.03 | 177.07 | 3.60 | 9.79 | 171.94 | 2.92 | 7.29 | 149.46 |
|  | 50% | 4.47 | 10.71 | 139.60 | 4.49 | 10.63 | 136.75 | 4.14 | 8.80 | 112.36 |
|  | 95% | 5.87 | 11.43 | 94.72 | 5.81 | 11.77 | 102.58 | 6.89 | 10.38 | 50.65 |
| DL Packet-Latency CDF (ms) | Mean | 0.87 | 0.76 | -12.64 | 0.87 | 0.76 | -12.64 | 1.01 | 0.89 | -11.63 |
|  | 5% | 0.52 | 0.52 | 0.00 | 0.52 | 0.52 | 0.00 | 0.53 | 0.52 | -1.15 |
|  | 50% | 0.81 | 0.74 | -8.64 | 0.80 | 0.75 | -6.25 | 0.86 | 0.80 | -6.98 |
|  | 95% | 1.35 | 0.97 | -28.15 | 1.35 | 0.97 | -28.15 | 1.69 | 1.49 | -12.00 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.78 | 0.76 | -57.30 | 1.76 | 0.76 | -56.82 | 1.97 | 1.04 | -47.27 |
|  | 5% | 0.60 | 0.52 | -13.33 | 0.59 | 0.52 | -11.86 | 0.63 | 0.53 | -15.90 |
|  | 50% | 1.77 | 0.74 | -58.19 | 1.74 | 0.75 | -56.90 | 1.85 | 0.88 | -52.22 |
|  | 95% | 2.86 | 0.97 | -66.08 | 2.84 | 0.97 | -65.85 | 3.07 | 1.94 | -36.96 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.12 | 5.92 | -0.2 | 19.87 | 19.02 | -0.85 | 51.29 | 53.46 | 2.17 |
|  | Type-2 | 7.65 | 7.40 | -0.25 | 24.84 | 23.77 | -1.07 | 64.11 | 66.83 | 2.71 |
| UL RU (%) | Type-1 | 1.51 | 1.57 | 0.06 | 5.68 | 5.75 | 0.07 | 13.04 | 12.99 | -0.05 |
|  | Type-2 | 7.57 | 7.87 | 0.3 | 28.40 | 28.74 | 0.34 | 65.19 | 64.94 | -0.25 |
| Unfinished/dropped Packet Rate (%) | DL | 0.20 | 0.30 |  | 1.20 | 1.30 |  | 5.50 | 5.57 |  |
|  | UL | 0.30 | 0.30 |  | 1.30 | 1.20 |  | 6.10 | 6.30 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 4 compared with legacy TDD based on the simulation results from Table 4:

•	SBFD with Alt 4 improves mean,5%,50% and 95% of  DL average-UPT CDF in the range of {8.9%~17.95%} and improves mean,5%,50% and 95% of  UL average-UPT CDF in the range of {59.41%~152.39%} for different traffic loads.

•	SBFD with Alt 4 decreases mean,5%,50% and 95% of DL packet-latency CDF in the range of {-28.15%~-1.15%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-66.08%~-11.86%} for different traffic loads.

Hence, SBFD improves UL average-UL UPT at all load conditions for indoor office than that of the legacy TDD and slightly improve or degrade DL average-UPT, both UL packet-latency of SBFD and DL packet-latency are decreased at all load conditions for indoor office scenario. The gain of UL average-UPT at least comes more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

Observation 10: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 improves both DL average-UPT and UL average-UPT at all load conditions.

Observation 11: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 decrease the UL packet-latency and DL packet-latency at all load conditions.

Observation 12: For indoor scenario (FR1) with SBFD Alt 4 and small packet size, the gain of UL average-UPT at least comes more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

  - Urban macro

    - SBFD Alt 2

      - Large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

Table 5: Evaluation results of SBFD Alt 2 for Urban macro in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 240.51 | 182.17 | -24.26 | 141.11 | 101.38 | -28.15 | 60.16 | 40.17 | -33.22 |
|  | 5% | 173.52 | 119.39 | -31.20 | 104.76 | 68.34 | -34.76 | 45.51 | 27.95 | -38.58 |
|  | 50% | 245.29 | 186.72 | -23.88 | 143.67 | 107.11 | -25.44 | 61.16 | 43.62 | -28.69 |
|  | 95% | 297.41 | 241.89 | -18.67 | 169.65 | 134.77 | -20.56 | 71.21 | 53.36 | -25.06 |
| DL Tail-UPT CDF (Mbps) | Mean | 173.17 | 133.98 | -22.63 | 107.47 | 78.27 | -27.17 | 46.91 | 30.94 | -34.05 |
|  | 5% | 124.12 | 102.42 | -17.48 | 78.97 | 61.50 | -22.13 | 34.88 | 25.48 | -26.96 |
|  | 50% | 175.21 | 136.76 | -21.95 | 110.94 | 82.49 | -25.64 | 47.92 | 33.21 | -30.69 |
|  | 95% | 209.18 | 151.71 | -27.48 | 136.17 | 97.12 | -28.68 | 54.40 | 37.09 | -31.82 |
| DL Median-UPT CDF (Mbps) | Mean | 239.17 | 175.20 | -26.75 | 140.21 | 98.19 | -29.97 | 60.16 | 40.55 | -32.60 |
|  | 5% | 167.92 | 115.23 | -31.37 | 102.88 | 67.40 | -34.48 | 45.47 | 26.87 | -40.91 |
|  | 50% | 244.58 | 180.87 | -26.05 | 142.37 | 102.68 | -27.88 | 61.89 | 41.54 | -32.89 |
|  | 95% | 291.84 | 235.37 | -19.35 | 168.71 | 131.30 | -22.17 | 71.39 | 51.78 | -27.47 |
| UL Average-UPT CDF (Mbps) | Mean | 92.91 | 125.36 | 36.00 | 52.72 | 69.92 | 32.62 | 20.44 | 25.85 | 26.45 |
|  | 5% | 43.22 | 82.71 | 91.38 | 29.29 | 52.34 | 78.72 | 13.10 | 20.29 | 54.91 |
|  | 50% | 94.69 | 126.19 | 33.27 | 54.82 | 70.93 | 29.38 | 21.07 | 26.40 | 25.33 |
|  | 95% | 128.50 | 158.95 | 23.69 | 73.60 | 87.90 | 19.43 | 28.96 | 31.51 | 8.80 |
| UL Tail-UPT CDF (Mbps) | Mean | 51.14 | 78.06 | 52.64 | 34.25 | 50.92 | 48.65 | 13.31 | 17.68 | 32.77 |
|  | 5% | 28.08 | 52.06 | 85.41 | 22.31 | 35.31 | 58.27 | 9.23 | 12.56 | 36.20 |
|  | 50% | 52.70 | 80.12 | 52.02 | 35.46 | 52.00 | 46.65 | 13.76 | 17.93 | 30.35 |
|  | 95% | 64.34 | 93.14 | 44.76 | 46.57 | 64.58 | 38.69 | 18.22 | 22.59 | 23.98 |
| UL Median-UPT CDF (Mbps) | Mean | 86.24 | 122.34 | 41.86 | 51.37 | 69.74 | 35.75 | 19.52 | 25.34 | 29.81 |
|  | 5% | 42.81 | 79.19 | 84.97 | 27.38 | 47.58 | 73.78 | 12.09 | 19.63 | 62.33 |
|  | 50% | 88.43 | 121.76 | 37.69 | 53.66 | 70.59 | 31.56 | 20.45 | 26.29 | 28.53 |
|  | 95% | 123.43 | 149.75 | 21.32 | 67.07 | 80.38 | 19.86 | 25.69 | 29.05 | 13.10 |
| DL Packet-Latency CDF (ms) | Mean | 16.23 | 20.82 | 28.29 | 28.02 | 37.27 | 33.00 | 65.03 | 88.22 | 35.66 |
|  | 5% | 11.51 | 13.51 | 17.44 | 22.06 | 28.28 | 28.20 | 50.12 | 68.38 | 36.44 |
|  | 50% | 15.69 | 20.95 | 33.46 | 27.09 | 36.72 | 35.55 | 63.10 | 87.14 | 38.10 |
|  | 95% | 23.36 | 28.32 | 21.21 | 38.16 | 46.98 | 23.11 | 86.60 | 108.03 | 24.74 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 11.29 | 8.54 | -24.35 | 17.96 | 14.42 | -19.74 | 48.48 | 39.94 | -17.62 |
|  | 5% | 5.79 | 4.84 | -16.54 | 10.73 | 9.32 | -13.12 | 33.28 | 29.45 | -11.52 |
|  | 50% | 10.66 | 8.32 | -21.96 | 18.05 | 14.22 | -21.18 | 47.77 | 39.01 | -18.35 |
|  | 95% | 18.35 | 11.36 | -38.10 | 26.71 | 19.32 | -27.66 | 70.56 | 52.48 | -25.62 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 5.92 | 5.98 | 0.05 | 21.29 | 19.55 | -1.74 | 41.57 | 37.72 | -3.85 |
|  | Type-2 | 7.40 | 9.34 | 1.93 | 26.61 | 30.54 | 3.93 | 51.96 | 58.94 | 6.98 |
| UL RU (%) | Type-1 | 1.61 | 2.90 | 1.28 | 5.96 | 11.44 | 5.49 | 10.39 | 21.05 | 10.66 |
|  | Type-2 | 8.05 | 8.04 | -0.01 | 29.78 | 31.78 | 2.00 | 51.96 | 58.48 | 6.52 |
| Unfinished/dropped Packet Rate (%) | DL | 0.10 | 0.30 |  | 1.00 | 1.00 |  | 4.20 | 4.50 |  |
|  | UL | 0.30 | 0.40 |  | 1.30 | 1.20 |  | 4.70 | 4.80 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 2 compared with legacy TDD based on the simulation results from Table 5:

•	SBFD with Alt 2 degrades mean,5%,50% and 95% of DL average-UPT CDF in the range of {-38.58%~-18.67%} and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {8.8%~91.38%} for different traffic loads.

•	SBFD with Alt 2 increases mean,5%,50% and 95% of DL packet-latency CDF in the range of {17.44%~38.1%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-38.1%~-11.52%} for different traffic loads.

Hence, SBFD improves UL average-UL UPT at all load conditions for urban macro than that of the legacy TDD at the cost of degradation of DL average-UPT, UL packet-latency of SBFD is decreased at all three load conditions for urban macro at the cost of increased DL packet-latency. The gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD.

Observation 13: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 improves UL average-UPT at all three load conditions at the cost of decreased DL average-UPT.

Observation 14: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 decrease the UL packet-latency at the cost of increased DL packet-latency.

Observation 15: For Urban Macro (FR1) with SBFD Alt 2 and large packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD.

      - Small packet (4Kbytes for DL and 1Kbyte for UL)

Table 6: Evaluation results of SBFD Alt 2 for Urban macro in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 2: {DDDSU} vs. {XXXXU} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.72 | 37.43 | -0.75 | 37.70 | 35.58 | -5.69 | 26.73 | 17.78 | -33.48 |
|  | 5% | 35.30 | 35.07 | -0.66 | 35.39 | 30.40 | -14.32 | 11.65 | 6.60 | -43.33 |
|  | 50% | 37.76 | 37.43 | -0.89 | 37.80 | 37.25 | -1.60 | 29.05 | 20.82 | -28.31 |
|  | 95% | 40.05 | 39.40 | -1.62 | 39.98 | 39.54 | -1.18 | 37.50 | 30.29 | -19.23 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.47 | 23.33 | -0.67 | 23.50 | 22.07 | -6.06 | 14.84 | 9.90 | -33.26 |
|  | 5% | 22.12 | 21.79 | -1.56 | 22.11 | 16.99 | -23.16 | 5.17 | 4.59 | -11.14 |
|  | 50% | 23.51 | 23.30 | -0.83 | 23.45 | 23.01 | -1.87 | 15.27 | 10.44 | -31.66 |
|  | 95% | 25.32 | 24.70 | -2.48 | 25.39 | 25.11 | -1.09 | 24.14 | 14.81 | -38.64 |
| DL Median-UPT CDF (Mbps) | Mean | 39.60 | 39.46 | -0.37 | 39.55 | 37.62 | -4.88 | 31.37 | 20.12 | -35.86 |
|  | 5% | 36.95 | 36.72 | -0.53 | 36.89 | 34.56 | -6.33 | 10.55 | 6.26 | -40.67 |
|  | 50% | 39.32 | 39.40 | 0.28 | 39.41 | 38.57 | -2.11 | 34.81 | 23.07 | -33.73 |
|  | 95% | 42.84 | 42.29 | -1.29 | 42.81 | 42.68 | -0.30 | 40.75 | 35.02 | -14.06 |
| UL Average-UPT CDF (Mbps) | Mean | 4.66 | 10.69 | 129.47 | 4.58 | 10.66 | 132.53 | 3.87 | 6.68 | 72.64 |
|  | 5% | 3.71 | 10.15 | 173.71 | 3.73 | 10.21 | 173.91 | 1.84 | 5.76 | 212.73 |
|  | 50% | 4.60 | 10.68 | 131.91 | 4.54 | 10.64 | 134.27 | 3.99 | 6.53 | 63.61 |
|  | 95% | 5.79 | 11.17 | 92.82 | 5.57 | 11.07 | 98.61 | 5.15 | 7.83 | 51.96 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.97 | 8.28 | 178.85 | 2.91 | 8.20 | 181.57 | 2.19 | 3.43 | 56.91 |
|  | 5% | 2.68 | 8.05 | 200.10 | 2.67 | 8.04 | 200.77 | 0.92 | 2.74 | 196.92 |
|  | 50% | 2.83 | 8.24 | 190.63 | 2.83 | 8.19 | 189.94 | 2.10 | 3.35 | 59.85 |
|  | 95% | 3.57 | 8.68 | 143.59 | 3.34 | 8.48 | 153.68 | 3.12 | 4.24 | 35.94 |
| UL Median-UPT CDF (Mbps) | Mean | 4.77 | 10.75 | 125.35 | 4.66 | 10.67 | 128.70 | 4.24 | 7.84 | 84.94 |
|  | 5% | 3.43 | 9.83 | 186.71 | 3.26 | 9.92 | 203.95 | 1.90 | 6.17 | 224.15 |
|  | 50% | 4.54 | 10.76 | 136.88 | 4.43 | 10.66 | 140.82 | 4.17 | 7.96 | 90.79 |
|  | 95% | 6.70 | 11.55 | 72.55 | 6.22 | 11.44 | 83.97 | 6.43 | 9.56 | 48.69 |
| DL Packet-Latency CDF (ms) | Mean | 0.88 | 0.87 | -1.14 | 0.89 | 0.91 | 1.33 | 1.34 | 1.75 | 30.19 |
|  | 5% | 0.54 | 0.52 | -2.11 | 0.52 | 0.52 | -0.40 | 0.55 | 0.59 | 7.36 |
|  | 50% | 0.81 | 0.80 | -0.83 | 0.81 | 0.82 | 1.39 | 0.96 | 1.40 | 46.00 |
|  | 95% | 1.35 | 1.37 | 1.39 | 1.36 | 1.41 | 3.16 | 4.19 | 3.91 | -6.79 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.75 | 0.75 | -56.85 | 1.76 | 0.75 | -57.31 | 2.17 | 1.20 | -44.61 |
|  | 5% | 0.60 | 0.52 | -13.23 | 0.60 | 0.52 | -13.46 | 0.62 | 0.54 | -13.68 |
|  | 50% | 1.76 | 0.74 | -57.84 | 1.77 | 0.75 | -57.96 | 1.90 | 0.99 | -47.87 |
|  | 95% | 2.85 | 0.97 | -65.95 | 2.86 | 0.97 | -66.03 | 4.78 | 2.32 | -51.49 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.23 | 5.14 | -1.09 | 19.52 | 16.81 | -2.71 | 50.10 | 41.19 | -8.91 |
|  | Type-2 | 7.79 | 8.03 | 0.25 | 24.40 | 26.27 | 1.87 | 62.62 | 64.36 | 1.74 |
| UL RU (%) | Type-1 | 1.45 | 2.90 | 1.45 | 5.77 | 10.37 | 4.60 | 12.71 | 24.00 | 11.29 |
|  | Type-2 | 7.23 | 8.04 | 0.82 | 28.83 | 28.80 | -0.03 | 63.57 | 66.66 | 3.10 |
| Unfinished/dropped Packet Rate (%) | DL | 0.30 | 0.20 |  | 1.00 | 0.90 |  | 5.90 | 5.20 |  |
|  | UL | 0.20 | 0.40 |  | 1.20 | 1.30 |  | 6.10 | 5.90 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 2 compared with legacy TDD based on the simulation results from Table 6:

•	SBFD with Alt 2 degrades mean,5%,50% and 95% of DL average-UPT CDF in the range of {-43.33%~-0.66%}, and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {51.96%~212.73%} for different traffic loads.

•	SBFD with Alt 2 increases or decreases mean,5%,50% and 95% of DL packet-latency CDF in the range of {-6.79%~46%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-66.03%~-13.23%} for different traffic loads.

Hence, SBFD improves UL average-UL UPT at all load conditions for urban macro than that of the legacy TDD at the cost of degradation of DL average-UPT, UL packet-latency of SBFD is decreased at all load conditions for urban macro and DL packet-latency is decreased or increased. The gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD, the loss of DL average-UPT at least comes from less DL resources for SBFD compared to legacy TDD.

Observation 16: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 improve UL average-UPT at all load conditions at the cost of degraded DL average-UPT.

Observation 17: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 reduce the UL packet-latency at the cost of increased DL packet-latency.

Observation 18: For Urban Macro (FR1) with SBFD Alt 2 and small packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD, the loss of DL average-UPT at least comes from less DL resources for SBFD compared to legacy TDD.

    - SBFD Alt 4

      - Large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

Table 7: Evaluation results of SBFD Alt 4 for Urban macro in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 240.51 | 233.47 | -2.93 | 141.11 | 132.54 | -6.07 | 60.16 | 55.86 | -7.14 |
|  | 5% | 173.52 | 155.08 | -10.63 | 104.76 | 89.52 | -14.54 | 45.51 | 36.80 | -19.14 |
|  | 50% | 245.29 | 240.68 | -1.88 | 143.67 | 137.33 | -4.41 | 61.16 | 56.32 | -7.91 |
|  | 95% | 297.41 | 294.03 | -1.14 | 169.65 | 165.94 | -2.18 | 71.21 | 68.48 | -3.83 |
| DL Tail-UPT CDF (Mbps) | Mean | 173.17 | 156.16 | -9.82 | 107.47 | 92.08 | -14.32 | 46.91 | 37.11 | -20.90 |
|  | 5% | 124.12 | 115.52 | -6.93 | 78.97 | 69.09 | -12.52 | 34.88 | 30.55 | -12.41 |
|  | 50% | 175.21 | 159.43 | -9.01 | 110.94 | 94.52 | -14.80 | 47.92 | 38.70 | -19.23 |
|  | 95% | 209.18 | 186.49 | -10.85 | 136.17 | 115.62 | -15.10 | 54.40 | 42.82 | -21.28 |
| DL Median-UPT CDF (Mbps) | Mean | 239.17 | 230.39 | -3.67 | 140.21 | 131.58 | -6.16 | 60.16 | 55.74 | -7.36 |
|  | 5% | 167.92 | 143.13 | -14.76 | 102.88 | 87.04 | -15.39 | 45.47 | 34.81 | -23.44 |
|  | 50% | 244.58 | 237.45 | -2.91 | 142.37 | 136.67 | -4.01 | 61.89 | 56.30 | -9.03 |
|  | 95% | 291.84 | 287.31 | -1.55 | 168.71 | 164.88 | -2.27 | 71.39 | 68.46 | -4.10 |
| UL Average-UPT CDF (Mbps) | Mean | 92.91 | 110.86 | 19.32 | 52.72 | 56.95 | 8.02 | 20.44 | 21.20 | 3.72 |
|  | 5% | 43.22 | 82.10 | 89.96 | 29.29 | 45.18 | 54.26 | 13.10 | 18.66 | 42.46 |
|  | 50% | 94.69 | 111.86 | 18.14 | 54.82 | 58.86 | 5.55 | 21.07 | 22.04 | 4.64 |
|  | 95% | 128.50 | 130.23 | 1.34 | 73.60 | 69.25 | -5.91 | 28.96 | 25.72 | -11.20 |
| UL Tail-UPT CDF (Mbps) | Mean | 51.14 | 68.14 | 33.24 | 34.25 | 45.93 | 34.09 | 13.31 | 18.86 | 41.68 |
|  | 5% | 28.08 | 52.36 | 86.45 | 22.31 | 35.40 | 58.68 | 9.23 | 14.15 | 53.41 |
|  | 50% | 52.70 | 69.62 | 32.10 | 35.46 | 46.27 | 30.50 | 13.76 | 19.18 | 39.43 |
|  | 95% | 64.34 | 74.55 | 15.87 | 46.57 | 52.09 | 11.87 | 18.22 | 20.15 | 10.58 |
| UL Median-UPT CDF (Mbps) | Mean | 86.24 | 103.75 | 20.30 | 51.37 | 55.56 | 8.15 | 19.52 | 20.28 | 3.88 |
|  | 5% | 42.81 | 79.46 | 85.59 | 27.38 | 43.15 | 57.60 | 12.09 | 17.11 | 41.51 |
|  | 50% | 88.43 | 105.15 | 18.91 | 53.66 | 57.41 | 7.00 | 20.45 | 21.05 | 2.91 |
|  | 95% | 123.43 | 124.23 | 0.65 | 67.07 | 66.28 | -1.17 | 25.69 | 24.41 | -4.98 |
| DL Packet-Latency CDF (ms) | Mean | 16.23 | 16.77 | 3.35 | 28.02 | 29.37 | 4.82 | 65.03 | 69.64 | 7.09 |
|  | 5% | 11.51 | 12.97 | 12.70 | 22.06 | 24.92 | 12.98 | 50.12 | 56.72 | 13.16 |
|  | 50% | 15.69 | 16.30 | 3.83 | 27.09 | 29.04 | 7.19 | 63.10 | 69.56 | 10.24 |
|  | 95% | 23.36 | 23.60 | 1.03 | 38.16 | 40.48 | 6.07 | 86.60 | 93.76 | 8.27 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 11.29 | 9.52 | -15.72 | 17.96 | 16.16 | -10.04 | 48.48 | 45.78 | -5.57 |
|  | 5% | 5.79 | 5.71 | -1.43 | 10.73 | 11.03 | 2.76 | 33.28 | 34.73 | 4.34 |
|  | 50% | 10.66 | 9.19 | -13.83 | 18.05 | 16.94 | -6.12 | 47.77 | 46.41 | -2.85 |
|  | 95% | 18.35 | 14.80 | -19.33 | 26.71 | 20.57 | -22.97 | 70.56 | 52.56 | -25.51 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 5.92 | 6.88 | 0.95 | 21.29 | 21.37 | 0.09 | 41.57 | 44.47 | 2.90 |
|  | Type-2 | 7.40 | 8.60 | 1.19 | 26.61 | 26.72 | 0.11 | 51.96 | 55.59 | 3.63 |
| UL RU (%) | Type-1 | 1.61 | 1.79 | 0.17 | 5.96 | 5.82 | -0.14 | 10.39 | 10.43 | 0.03 |
|  | Type-2 | 8.05 | 8.93 | 0.87 | 29.78 | 29.10 | -0.68 | 51.96 | 52.13 | 0.17 |
| Unfinished/dropped Packet Rate (%) | DL | 0.10 | 0.20 |  | 1.00 | 1.10 |  | 4.20 | 4.30 |  |
|  | UL | 0.30 | 0.40 |  | 1.30 | 1.40 |  | 4.70 | 4.80 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 4 compared with legacy TDD based on the simulation results from Table 7:

•	SBFD with Alt 4 degrades mean,5%,50% and 95% of DL average-UPT CDF in the range of {-19.14%~-1.14%} and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {1.34%~89.96%} for different traffic loads, except a degradation of -5.91% and -11.2% at 95%-tile of UL average-UPT CDF for median and high traffic load respectively.

•	SBFD with Alt 4 increases mean,5%,50% and 95% of DL packet-latency CDF in the range of {1.03%~13.16%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-25.51%~-1.43%} for different traffic loads, except a improvement of 2.76% and 4.34% at 5%-tile of UL packet-latency CDF for median and high traffic load respectively.

Hence, compared to legacy TDD, SBFD with Alt 4 degrades DL average-UPT at all load conditions and improve or degrade UL average-UPT, increases DL packet-latency at all load conditions and increase or decrease UL packet-latency. The gain of UL average-UPT at least comes from more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the loss of DL average-UPT for SBFD at least comes from inter-subband interference.

Observation 19: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 improves UL average-UPT at low/medium load conditions and improves DL average-UPT at all load conditions.

Observation 20: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 decrease DL packet-latency at all load conditions and decrease UL packet-latency at low and median traffic loads.

Observation 21: For Urban Macro (FR1) with SBFD Alt 4 and large packet size, the gain of UL average-UPT at least comes from more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the loss of DL average-UPT for SBFD at least comes from inter-subband interference.

      - Small packet (4Kbytes for DL and 1Kbyte for UL)

Table 8: Evaluation results of SBFD Alt 4 for Urban macro in FR1 in SBFD Deployment Case 1

| Metrics |  | SBFD Alt 4: {DDDSU} vs. {XXXXX} |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL and UL arrival rate for baseline static TDD (Type-2 RU: <10%, 20%-40% and ≥50%) |  |  |  |  |  |  |  |  |
|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) | TDD | SBFD | Gain (%) |
| DL Average-UPT CDF (Mbps) | Mean | 37.72 | 40.72 | 7.96 | 37.70 | 37.12 | -1.54 | 26.73 | 22.65 | -15.27 |
|  | 5% | 35.30 | 35.12 | -0.51 | 35.39 | 32.34 | -8.62 | 11.65 | 7.94 | -31.88 |
|  | 50% | 37.76 | 42.04 | 11.33 | 37.80 | 38.21 | 1.09 | 29.05 | 25.26 | -13.03 |
|  | 95% | 40.05 | 43.11 | 7.65 | 39.98 | 40.44 | 1.16 | 37.50 | 34.09 | -9.10 |
| DL Tail-UPT CDF (Mbps) | Mean | 23.47 | 30.22 | 28.75 | 23.50 | 23.72 | 0.94 | 14.84 | 12.54 | -15.53 |
|  | 5% | 22.12 | 22.35 | 1.02 | 22.11 | 19.93 | -9.87 | 5.17 | 5.07 | -1.87 |
|  | 50% | 23.51 | 32.62 | 38.76 | 23.45 | 24.96 | 6.42 | 15.27 | 13.26 | -13.17 |
|  | 95% | 25.32 | 33.13 | 30.86 | 25.39 | 27.06 | 6.59 | 24.14 | 19.65 | -18.59 |
| DL Median-UPT CDF (Mbps) | Mean | 39.60 | 41.23 | 4.12 | 39.55 | 38.83 | -1.82 | 31.37 | 24.12 | -23.10 |
|  | 5% | 36.95 | 36.81 | -0.36 | 36.89 | 34.99 | -5.16 | 10.55 | 7.80 | -26.09 |
|  | 50% | 39.32 | 42.08 | 7.03 | 39.41 | 39.81 | 1.02 | 34.81 | 27.35 | -21.43 |
|  | 95% | 42.84 | 44.40 | 3.63 | 42.81 | 43.57 | 1.79 | 40.75 | 38.06 | -6.60 |
| UL Average-UPT CDF (Mbps) | Mean | 4.66 | 10.67 | 129.07 | 4.58 | 10.59 | 131.07 | 3.87 | 5.83 | 50.67 |
|  | 5% | 3.71 | 10.11 | 172.72 | 3.73 | 10.03 | 169.07 | 1.84 | 4.78 | 159.37 |
|  | 50% | 4.60 | 10.66 | 131.47 | 4.54 | 10.59 | 133.23 | 3.99 | 5.69 | 42.58 |
|  | 95% | 5.79 | 11.13 | 92.12 | 5.57 | 11.14 | 99.83 | 5.15 | 7.18 | 39.24 |
| UL Tail-UPT CDF (Mbps) | Mean | 2.97 | 8.24 | 177.41 | 2.91 | 8.17 | 180.31 | 2.19 | 3.14 | 43.28 |
|  | 5% | 2.68 | 8.05 | 200.15 | 2.67 | 8.02 | 199.97 | 0.92 | 2.42 | 162.95 |
|  | 50% | 2.83 | 8.21 | 189.52 | 2.83 | 8.21 | 190.36 | 2.10 | 3.20 | 52.66 |
|  | 95% | 3.57 | 8.52 | 139.02 | 3.34 | 8.56 | 156.10 | 3.12 | 3.79 | 21.41 |
| UL Median-UPT CDF (Mbps) | Mean | 4.77 | 10.70 | 124.45 | 4.66 | 10.66 | 128.54 | 4.24 | 6.57 | 54.98 |
|  | 5% | 3.43 | 9.86 | 187.47 | 3.26 | 9.78 | 199.53 | 1.90 | 4.71 | 147.68 |
|  | 50% | 4.54 | 10.71 | 135.86 | 4.43 | 10.62 | 139.97 | 4.17 | 6.23 | 49.28 |
|  | 95% | 6.70 | 11.57 | 72.73 | 6.22 | 11.58 | 86.25 | 6.43 | 8.97 | 39.57 |
| DL Packet-Latency CDF (ms) | Mean | 0.88 | 0.81 | -7.94 | 0.89 | 0.90 | 0.83 | 1.34 | 1.45 | 7.74 |
|  | 5% | 0.54 | 0.52 | -2.41 | 0.52 | 0.51 | -1.72 | 0.55 | 0.57 | 5.38 |
|  | 50% | 0.81 | 0.76 | -6.29 | 0.81 | 0.81 | 0.62 | 0.96 | 1.22 | 27.04 |
|  | 95% | 1.35 | 1.34 | -0.75 | 1.36 | 1.39 | 1.86 | 4.19 | 3.59 | -14.40 |
| DL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean | 1.75 | 0.75 | -56.83 | 1.76 | 0.76 | -56.67 | 2.17 | 1.39 | -35.86 |
|  | 5% | 0.60 | 0.52 | -13.16 | 0.60 | 0.52 | -13.44 | 0.62 | 0.55 | -11.10 |
|  | 50% | 1.76 | 0.74 | -57.75 | 1.77 | 0.75 | -57.84 | 1.90 | 1.22 | -35.67 |
|  | 95% | 2.85 | 0.97 | -66.01 | 2.86 | 0.97 | -65.93 | 4.78 | 2.54 | -46.81 |
| UL UE- Average-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.23 | 7.25 | 1.02 | 19.52 | 21.47 | 1.95 | 50.10 | 51.52 | 1.42 |
|  | Type-2 | 7.79 | 9.06 | 1.28 | 24.40 | 26.84 | 2.43 | 62.62 | 64.40 | 1.77 |
| UL RU (%) | Type-1 | 1.45 | 1.84 | 0.39 | 5.77 | 6.20 | 0.43 | 12.71 | 12.63 | -0.08 |
|  | Type-2 | 7.23 | 9.18 | 1.96 | 28.83 | 31.02 | 2.19 | 63.57 | 63.14 | -0.42 |
| Unfinished/dropped Packet Rate (%) | DL | 0.30 | 0.20 |  | 1.00 | 0.90 |  | 5.90 | 5.60 |  |
|  | UL | 0.20 | 0.30 |  | 1.20 | 1.20 |  | 6.10 | 6.10 |  |
| Note | Definition of gain for UPT: X%=SBFD UPT/TDD UPT-1. For Latency, the increase can be calculated as: Increase (%) = SBFD latency / TDD latency – 1. |  |  |  |  |  |  |  |  |  |

The followings are observed for SBFD with Alt 4 compared with legacy TDD based on the simulation results from Table 8:

•	SBFD with Alt 4 improves or degrades mean,5%,50% and 95% of DL average-UPT CDF in the range of {-31.88%~11.33%} and improves mean,5%,50% and 95% of UL average-UPT CDF in the range of {39.24%~172.72%} for different traffic loads.

•	SBFD with Alt 4 increases or decreases mean,5%,50% and 95% of DL packet-latency CDF in the range of {-14.4%~27.04%} and decreases mean,5%,50% and 95% of UL packet-latency CDF in the range of {-66.01%~-11.1%} for different traffic loads.

Hence, SBFD improves UL average-UL UPT at all load conditions for urban macro than that of the legacy TDD and improve or degrade DL average-UPT, UL packet-latency of SBFD is decreased at all load conditions for urban macro and DL packet-latency is decreased or increased. The gain of UL average-UPT at least comes more UL transmission opportunities for SBFD compared to legacy TDD, the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from inter-subband interference.

Observation 22: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 improves both DL average-UPT and UL average-UPT at all load conditions.

Observation 23: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 decrease the UL packet-latency, and increases or decreases DL packet-latency.

Observation 24: For Urban Macro (FR1) with SBFD Alt 4 and small packet size, the gain of UL average-UPT at least comes more UL transmission opportunities for SBFD compared to legacy TDD, the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from inter-subband interference.

- Conclusions

In this contribution, updated SLS results of SBFD-case1 are provided. Our observations are:

Observation 1: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 improves UL average-UPT at all three load conditions at the cost of degraded DL average-UPT.

Observation 2: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 decrease the UL packet-latency at the cost of increased DL packet-latency.

Observation 3: For indoor scenario (FR1) with SBFD Alt 2 and large packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD.

Observation 4: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 improve UL average-UPT at all load conditions at the cost of degraded DL average-UPT.

Observation 5: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 reduce the UL packet-latency at the cost of increased DL packet-latency.

Observation 6: For indoor scenario (FR1) with SBFD Alt 2 and small packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD, the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD and the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

Observation 7: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 improves UL average-UPT at low/medium load conditions and improves DL average-UPT at all load conditions.

Observation 8: For indoor scenario (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 decrease DL packet-latency at all load conditions and decrease UL packet-latency at low and median traffic loads.

Observation 9: For indoor scenario (FR1) with SBFD Alt 4 and large packet size, the gain of UL average-UPT at least comes from more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the gain of DL average-UPT for SBFD at least comes from more DL transmission opportunities and reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

Observation 10: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 improves both DL average-UPT and UL average-UPT at all load conditions.

Observation 11: For indoor scenario (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 decrease the UL packet-latency and DL packet-latency at all load conditions.

Observation 12: For indoor scenario (FR1) with SBFD Alt 4 and small packet size, the gain of UL average-UPT at least comes more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD.

Observation 13: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 improves UL average-UPT at all three load conditions at the cost of decreased DL average-UPT.

Observation 14: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 2 decrease the UL packet-latency at the cost of increased DL packet-latency.

Observation 15: For Urban Macro (FR1) with SBFD Alt 2 and large packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from less DL resources for SBFD compared to legacy TDD.

Observation 16: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 improve UL average-UPT at all load conditions at the cost of degraded DL average-UPT.

Observation 17: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 2 reduce the UL packet-latency at the cost of increased DL packet-latency.

Observation 18: For Urban Macro (FR1) with SBFD Alt 2 and small packet size, the gain of UL average-UPT at least comes from more UL resources and more UL transmission opportunities for SBFD compared to legacy TDD, the loss of DL average-UPT at least comes from less DL resources for SBFD compared to legacy TDD.

Observation 19: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 improves UL average-UPT at low/medium load conditions and improves DL average-UPT at all load conditions.

Observation 20: For Urban Macro (FR1) with large packet size, compared to legacy TDD, SBFD with Alt 4 decrease DL packet-latency at all load conditions and decrease UL packet-latency at low and median traffic loads.

Observation 21: For Urban Macro (FR1) with SBFD Alt 4 and large packet size, the gain of UL average-UPT at least comes from more UL transmission opportunities for SBFD compared to legacy TDD and the loss of UL average-UPT for SBFD at least comes from inter-subband interference, the loss of DL average-UPT for SBFD at least comes from inter-subband interference.

Observation 22: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 improves both DL average-UPT and UL average-UPT at all load conditions.

Observation 23: For Urban Macro (FR1) with small packet size, compared to legacy TDD, SBFD with Alt 4 decrease the UL packet-latency, and increases or decreases DL packet-latency.

Observation 24: For Urban Macro (FR1) with SBFD Alt 4 and small packet size, the gain of UL average-UPT at least comes more UL transmission opportunities for SBFD compared to legacy TDD, the gain of DL average-UPT for SBFD at least comes from reduced HARQ-ACK feedback latency for SBFD compared to legacy TDD and the loss of DL average-UPT for SBFD at least comes from inter-subband interference.

- Annex

Table Annex-1. System-level simulation assumptions for NR Full Duplex

| Parameters | Indoor office | Urban macro |  |
| --- | --- | --- | --- |
| Layout | Single layer 12BSs per 120m x 50m | Single layer Macro layer: Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |
| Carrier frequency | 4G |  |  |
| Deployment case | Case 1 |  |  |
| BS antenna configuration | SBFD antenna configuration option-2 : = (4,4,2,1,1; 4,4) = (0.5, 0.5)λ, +45°/-45° polarization | SBFD antenna configuration option-2 : = (8,8,2,1,1;2,8) = (0.5, 0.8)λ, +45°/-45° polarization |  |
| BS Tx power | 24dBm (38.901) | 49 dBm (38.901) |  |
| UE antenna height | 1.0m | Same as 3D-UMa in TR36.873 |  |
| Traffic model | FTP traffic model 3 Each UE is assigned both UL traffic and DL traffic Asymmetric packet size: 0.5Mbyte for DL and 0.125 Mbytes for UL; 4Kbytes for DL and 1Kbyte for UL UL Traffic load: low UL RU (<10%), medium UL RU (20%-30%), and high UL RU ([≥50%]). DL Traffic load: low DL RU (<10%), medium DL RU (20%-30%), and high DL RU (≥50%). Note: Type-2 RU definition (calculated per link direction) is used |  |  |
| UE-to-UE Channel model | Large-scale channel parameters: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |  | Large-scale channel parameters: UMi-Street canyon in TR 38.901(hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802, ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| co-site inter-sector co-channel inter-subband CLI modelling | NA |  | 75dB for spatial isolation |
| Transmission mode | SU-MIMO |  |  |
| Channel estimation | Realistic |  |  |
| Overhead | No extra overhead |  |  |