3GPP TSG-RAN WG1 Meeting #114	R1-2307455

Toulouse, France, August 21st – August 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the interpretation of the number of scheduled PDSCH during DCI-based BWP switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If pdsch-TimeDomainResourceAllocationListForMultiPDSCH is configured for DCI format 1_1, when interpretating DCI format 1_1 fields, field size of NDI/RV fields depend on the number of scheduled PDSCH indicated by the TDRA field. If BWP switching is indicated by DCI format 1_1, it is not clear whether “the number of scheduled PDSCH” are interpretated based on the TDRA table in the active BWP or in the indicated BWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that “the number of scheduled PDSCH” is interpretated based on the TDRA table in the indicated BWP, for interpretating NDI/RV fields in DCI format 1_1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If pdsch-TimeDomainResourceAllocationListForMultiPDSCH is configured for DCI format 1_1, how to interpretate “the number of scheduled PDSCH” is not clear when interpretating NDI/RV fields in DCI format 1_1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR. |  |  |  |  |  |  |  |  |

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

=============================== Unchanged Text Omitted ===================================

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit if pdsch-TimeDomainResourceAllocationListForMultiPDSCH is not configured or if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainResourceAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, the number of scheduled PUSCH(s) is interpretated based on the indicated bandwidth part.

-	Redundancy version – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if pdsch-TimeDomainResourceAllocationListForMultiPDSCH is not configured or if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainResourceAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

-	If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, the number of scheduled PUSCH(s) is interpretated based on the indicated bandwidth part.

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit if pdsch-TimeDomainResourceAllocationListForMultiPDSCH is not configured or if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainResourceAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, the number of scheduled PUSCH(s) is interpretated based on the indicated bandwidth part.

-	Redundancy version – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if pdsch-TimeDomainResourceAllocationListForMultiPDSCH is not configured or if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainResourceAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

-	If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, the number of scheduled PUSCH(s) is interpretated based on the indicated bandwidth part.

=============================== Unchanged Text Omitted ===================================