3GPP TSG-RAN WG1 Meeting #114                                                                                                             	                                         R1- 2307919

Toulouse, August 21-25, 2023

Agenda item:	9.2.3.2

Source: 	Qualcomm Incorporated

Title: 	Other aspects on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1] with the following goals briefly summarized as below.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

Use cases to focus on:

- Initial set of use cases includes:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

- The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

- PHY layer aspects, e.g., (RAN1)

- Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

- Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

Quite a lot of progress has been made in RAN1 meetings so far, towards achieving the SI objectives. In this contribution, we discuss various aspects of the above-mentioned goals for the beam management use case.

# Common aspects related to spatial and temporal beam prediction

Because some of the specification aspects related to the two agreed use case cases (BM-Case1 and BM-Case2) are identical, we first consider the spec impacts that are common to both use cases in this section, and then consider aspects specific to each use case in later sections. We consider different phases of ML workflow in the following sections and discuss the corresponding specification impact related to each phase.

## Spec impact for model development and data collection

Model development and training strategies is a multi-faceted problem that require extensive testing and tuning. As elaborated in [2], on-device models today and in the near future need offline engineering for model development. This includes model development, training, quantization, compiling the model to hardware primitives with power, area, and latency consideration, target-chip-specific run-time binary image generation, and going through full UE testing. This is like today’s non-ML implementations that go through similar offline development and extensive UE testing, and ML algorithms will not be exceptions. Various options can be discussed in RAN1 in terms of different levels of network control, but offline development and training should be the focus to guarantee a concrete outcome that can lead to specification work in the potential Rel-19 WI. In RAN1 #110 meeting, it is agreed that both alternatives of AI/ML model training at NW side and AI/ML model training at UE side should be supported, but the issue of online versus offline training is considered as a separate discussion. The following proposal addresses the training procedure for beam prediction (one-sided AI/ML) use cases.

In RAN1 #110b-e an FL proposal was discussed in which it was proposed to start with the assumption of offline training for the agenda item 9.2.3.2 and defer discussions of online training to wait for the conclusion/agreement of Agenda item 9.2.1 whether online training is supported or not, but eventually there was no consensus. Our view is that we can separate the discussion to UE-side and NW-side AI/ML models. At least for UE-side AI/ML models, we can focus and agree on offline training scenario, and the discussion on NW-side AI/ML models can wait for the conclusion/agreement of agenda item 9.2.1. There was a similar separation already in RAN1 #110b-e for data collection, model monitoring, etc., and we believe a similar separation here would facilitate the progress of this aspect. With this being said, we have the following proposal:

#### Proposal 1

For the sub use case BM-Case1 and BM-Case2 and for UE-side AI/ML models, Agenda item 9.2.3.2 should focus on offline training scenario, in which the development and training of the AI/ML model happens offline without the need to involve 3gpp signaling

## Association/mapping of beams within Set A and beams within Set B

Let us consider the following agreement from RAN1 #112:

Agreement (RAN1 #112) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

In this section, we discuss aspects related to the association/mapping of beams within Set A and beams within Set B. For this purpose, we consider different phases of ML workflow, and discuss how the association/mapping of beams within Set A and beams within Set B should be characterized across training and inference phases. Let us consider a given model-ID and assume this model-ID has been pre-defined in 3gpp (per agreement in 9.2.1, this can be a logical model-ID). Given the same model-ID, for UE-side AI/ML models, various levels of consistency should be ensured for Set A and Set B beams across training and inference. Here are these various levels explained in more detail:

- Set size consistency for Set B, Set A: the same number of RS resources configured as Set B, Set A beams across training and inference. For instance, if the number of resources configured for Set B and Set A during training (e.g., for wide-to-narrow beam prediction) are 8 and 32, the same number of resources must be configured for Set B and Set A during inference. This is applicable to both sub-use cases of spatial beam prediction, namely wide-to-narrow beam prediction and Set B subset of Set A, as well as BM-Case2.

- Order/indexing consistency: UE shall use L1-RSRP with respect to a resource index (RI) associated with Set B, e.g., SetB-RI =𝑚 to determine input value with respect to the 𝑚th input feature across training & inference; UE shall use L1-RSRP with respect to a resource index (RI) associated with Set A, e.g., SetA-RI=n as label with respect to the 𝑛th output feature during training, and use value from the 𝑛th output feature to derive prediction results with respect to SetA-RI =𝑛 during inference. In the context of wide-to-narrow beam prediction, SetB-RI and SetA-RI would correspond to SSBRI and CRI, respectively. This is applicable to both sub-use cases of spatial beam prediction, namely wide-to-narrow beam prediction and Set B subset of Set A, as well as BM-Case2.

- QCL consistency: if QCL source with respect to SetA-RI=𝑗 corresponds to SetB-RI=𝑘 during training, then QCL source with respect to SetA-RI =𝑗 should also correspond to SetB-RI =𝑘 during inference. This consistency level is particularly applicable to wide-to-narrow beam prediction.

- Beam shape consistency: Relative pointing direction and beamwidth difference between physical beams with respect to two resources should be under predefined tolerances, across two Set B resource sets for training & inference, as well as across two Set A resource sets for training & inference. Figures 2 and 3 illustrate the scenario in which the beam shapes for Set B and Set A are NOT consistent across training (Case 1) and inference (Case 2), in which case the trained model for Case 1 may not be applicable to the inference setting (Case 2).

AI/ML Model

(ID#X)

Output features

mth input feature

Ground truth label

L1-RSRP wrt SetB-RI=m

nth output feature

L1-RSRP wrt SetA-RI=n

Input features

Training

Set A consistency

Set B consistency

Set B consistency

AI/ML Model

(ID#X)

Output features

mth input feature

L1-RSRP wrt SetA-RI=n

L1-RSRP wrt SetB-RI=m

nth output feature

Inference

Input features

Figure 1 Consistency of Set A and Set B beams across training and inference

Let us consider the sub-use case of wide-to-narrow beam prediction. As we see in Figure 2, even though we have set size consistency, order/indexing consistency, and even QCL consistency across training (Case 1) and inference (Case 2) for Set B and Set A, as long as we do not have beam shape consistency, the learned mapping from Set B to Set A may be quite different due to different codebooks (and different resulting mapping/association/relation) for Set A and Set B in Case 1 and Case 2. For this illustrative example, both the beam boresight directions as well as 3dB beamwidth of the beams in Set A (Case 2) are different compared to beams in Set A’ (Case 1). The same statement holds true for beams in Set B (Case 2) compared to beams in Set B’ (Case 1).

Figure 2 Illustration of two cases for Set A and Set B beams based on different possible gNB codebooks (Set B is not a subset of Set A)

Now, let us consider another sub-use case in which Set B is a subset of Set A. This scenario is depicted in Figure 3, in which two different cases represent two different codebook designs. As we see in Figure 3, even though we have set size consistency and order/indexing consistency for Set B and Set A beams across training (Case 1) and inference (Case 2), due to different codebook structures the learned mapping from Set B to Set A may be quite different for Case 1 (training), compared to Case 2 (inference). This is not just an intuitive assertion, and this fact has been observed by quite a few companies through evaluations in 9.2.3.1. For the example cases in Figure 3, both the beam boresight directions as well as 3dB beamwidth of the beams in Set A (Case 2) are different compared to beams in Set A’ (Case 1).

Figure 3 Illustration of two cases for Set A and Set B beams based on different possible gNB codebooks (Set B subset of Set A)

Based on the discussion above, the following proposal states some examples we can have for association/mapping of beams in Set A and beams in Set B:

#### Proposal 2

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding aspects related to association/mapping of beams within Set A and beams within Set B, study mechanisms to ensure consistency of Set B beams and Set A beams across training and inference, including one or more of the following

- Set size consistency for Set B, Set A: consistency in number of beams and/or associated resources for Set B and Set A, across training and inference

- Order/indexing consistency: consistency in ordering of resources (e.g., resource index consistency) for Set B beams and Set A beams, across training and inference

- QCL consistency: consistency in QCL relationship of Set A beams with respect to Set B beams, across training and inference

- Beam shape consistency: relative pointing direction and beamwidth difference between physical beams with respect to Set A and Set B resources across training and inference should be under predefined tolerances

We further share a more detailed example on how Proposal 2 could work in practice as below:

Model training: suppose that we are training a UE side AI/ML model whose inputs comprise of L1-RSRPs of 8 SSBs (as Set-B beams for training purposes) and outputs comprise of predicted L1-RSRPs of 32 CSI-RSs (as Set-A beams for training purposes). Particularly, this can be an AI/ML model for a wide-to-narrow beam prediction problem. In order to collect data for training such an AI/ML model, NW should configure the UE with a 1st SSB resource set, wherein the SSBRIs (from 1 to 8) are defined based on the entry-IDs associated with the respective SSBs associated with the SSB resource set, as well as a 1st CSI-RS resource set, wherein the CRIs (from 1 to 32) are defined based on the entry-IDs associated with the respective CSI-RSs associated with the CSI-RS resource set. The CSI-RSs in the 1st CSI-RS resource set would share identical periodicity as the SSBs in the 1st SSB resource set. UE may collect such L1-RSRPs regarding the SSBs and CSI-RSs across various time occasions and upload the data to a proprietary server for offline model training. During model training, it is expected that the L1-RSRP regarding SSBRI=m (1≤m≤8) from the 1st SSB resource set should be used to determine input value regarding the mth input feature of the AI/ML model, and the L1-RSRP regarding CRI=n (1≤n≤32) from the 1st CSI-RS resource set should be used to determine ground truth label associated with the nth output feature of the AI/ML model. Such offline trained AI/ML model can then be downloaded back to the UE for future model inference.

- Certain mechanisms may be used to identify a model-ID associated with the above training procedure (e.g., predefined through 3GPP and/or identified before UE collects the data through signalling between NW and UE).

Model inference: before actual model inference, suppose certain mechanisms can be used to identify the model-ID associated with the AI/ML model trained above (e.g., through signalling between gNB and UE involved in the model inference procedure). Let us also assume that NW also asks the UE to report its predicted L1-RSRPs regarding the strongest 4 beams out of the 32 predicted ones (e.g., through a CSI report setting). Thus, Based on Proposal 2, we should have:

- Set size consistency for Set B, Set A: NW should configure a 2nd SSB resource set (as Set-B beams for inference purposes), wherein the SSBRIs (from 1 to 8) are defined based on the entry-IDs associated with the respective SSBs in the SSB resource set. Additionally, NW may further configure a CSI-RS resource set with a 2nd CSI-RS resource set (as Set-A beams for inference purposes), wherein the CRIs (from 1 to 32) are defined based on the entry-IDs associated with the respective CSI-RSs in the CSI-RS resource set. Note that periodicity of the CSI-RSs in the 2nd CSI-RS resource set could be much longer then the SSBs in the 2nd SSB resource set (e.g., only for performance monitoring purposes). These SSB and CSI-RS resource sets can be associated with the CSI report setting where the NW requests the UE to report its prediction results (e.g., following conventional L1-RSRP and CRI reporting frameworks, wherein the CRIs should be associated with the 2nd CSI-RS resource set configured above).

- Order/indexing consistency: UE should use the L1-RSRP regarding SSBRI=m (1≤m≤8) from the 2nd SSB resource set to determine input value regarding the mth input feature and use the nth output feature to derive prediction results regarding CRI=n (1≤n≤32) from the 2nd CSI-RS resource set.

- QCL consistency: suppose that the CSI-RSs in the 1st CSI-RS resource set are supposed to be at least QCLed with the SSBs in the 1st SSB resource set, and the CSI-RSs in the 2nd CSI-RS resource set are supposed to be at least QCLed with the SSBs in the 2nd SSB resource set. If CSI-RS associated with CRI=𝑗 in the 1st CSI-RS resource set is QCLed with the SSB associated with SSBRI=𝑘 in the 1st SSB resource set, then the CSI-RS associated with CRI=𝑗 should also be QCLed with the SSB associated with SSBRI=𝑘 in the 2nd SSB resource set.

- Beam shape consistency: this includes at least one of pointing direction consistency and beam width consistency as follows.

  - Pointing direction consistency: if pointing direction of the actual beam associated with SSBRI/CRI=k is offset referring to the actual beam associated with SSBRI/CRI=j in a certain LCS dimension within the 1st SSB/CSI-RS resource set for X-deg, the pointing direction of the actual beam associated with SSBRI/CRI=k should also be offset referring to the actual beam associated with SSBRI/CRI=j in the same LCS dimension within the 2nd SSB/CSI-RS resource set for X±Y-deg, where the value of Y (Y>0 & Y≪|X|) can be separately identified/defined.

  - Beam width consistency: the 3dB beamwidth difference btw the actual beam associated with SSBRI/CRI=k within the 1st SSB/CSI-RS resource set and the actual beam associated with SSBRI/CRI=k within the 2nd SSB/CSI-RS resource set, should be no more than ±Z, where the value of Z>0 can be separately identified/defined.

The above more detailed example is illustrated in Figure 4 below.

Figure 4 An example illustrating how Proposal 2 could work in practice

## Type of Beam Prediction

Let us consider the agreements related to types of beam prediction so far:

Agreement (RAN1 #110) For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact

Conclusion (RAN1 #112) For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized.

Beam pair prediction comes with numerous practical challenges, some of them stemming from the fact that DL Rx beam changes at a much faster pace compared to DL Tx beam and this makes the prediction task much more challenging. Let us consider UE-side and NW-side beam pair prediction. For UE-side beam pair prediction, the most relevant part in terms of specification impact is the predicted DL TX beam and the distinction in terms of specification impact with DL Tx beam prediction is minimal, if any. On the other hand, NW-side beam pair prediction has been discussed in the past few meetings, but there have been serious feasibility concerns that have not been addressed, even at a conceptual level.

In the evaluations across companies, many of these practical challenges are not modelled, and even so, the performance of beam pair prediction (e.g., in terms of agreed KPIs including top-K beam prediction accuracy) is generally worse (in some cases much worse) than DL Tx beam prediction. With this being said, based on all the observations we have had for the evaluations related to beam pair prediction [3], we believe DL Tx beam prediction should be prioritized in the context of Rel-18 SI.

#### Proposal 3

For BM-Case1 and BM-Case2 prioritize the study of DL Tx beam prediction.

# Specific aspects related to temporal prediction

We consider BM-Case2 with a UE-side AI/ML model. A sub use case of temporal beam prediction is beam blockage prediction. Let us assume that a UE can predict blockage based on the history of beam measurements (potentially along with other auxiliary inputs to the AI/ML model such as position information, information from sensors, e.g., camera, etc.). One of the ways that this UE capability could be useful is that UE can proactively indicate to gNB that a blockage is imminent, and the gNB can take this information into account and proactively switch the downlink beam to a secondary beam. The existing methods for beam failure detection and recovery are reactive in nature, in which the blockage event is detected first, and then the beam failure recovery procedure is initiated.

#### Proposal 4

For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signalling to report the following information of AI/ML model inference to NW:

Predicted beam blockage/failure

# Discussion on UE features and functionality prospects for BM use case

Following along the discussion in [2] regarding “overall framework of features, functionalities and models”, we discuss some of the aspects specific to the beam management use case in this section. As compared to the legacy features, one fundamental distinction of AI-ML-enabled features is the need to support dynamic capabilities in addition to static capabilities. This occurs mainly because there may be supported functionalities and/or trained models for some cells, scenarios, configurations but not for others. The purpose of this section is to discuss some prospects related to these aspects for the beam management use case.

A given feature (or a configuration of a feature or a functionality) can be supported by a given UE for a specific cell, e.g., cell-A. On the other hand, the same feature (or a configuration of a feature or a functionality) for the same UE may not be supported for cell-B. That’s why, UE needs to dynamically indicate the supported feature in the case of handover when UE moves to cell-B. An example for “a configuration” could be Set A/Set B configurations for beam prediction.

Suppose that three Features (or Feature Groups), denoted as F1, F2, and F3, have been defined for the AI/ML beam management sub-use-case. A UE may support multiple features/functionalities of this beam management use case denoted by F1, F2, F3 in a cell-specific manner.  For example, suppose that the UE supports F1 and F2 for cell-A, F3 for cell-B, and does not support AI/ML-based beam prediction for cell-C. This implies that in the case of handover from cell A to cell B, the UE needs to communicate the supported features/functionalities with NW dynamically to indicate that F1 and F2 are no more supported, whereas F3 is. Likewise, each time the UE enters cell-C it should notify NW that there is no support for F1 and F2 so that NW may deactivate this feature/functionality.

Another aspect related to the beam management use case is the prospect of assistance information. For instance, let us consider the following agreement:

Agreement (RAN1 #112) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

The “association/mapping of beams within Set A and beams within Set B” can be considered as an example of assistance information from NW to UE to help UE improve prediction accuracy.

As another example, suppose that there is one Feature, denoted as F1, that has been defined for the sub-use-case. Suppose that cell-A uses a certain beam codebook that is provided as an assistance information (e.g., as a component of the Feature F1) in the form of Set A and Set B relationship. Suppose that cell-B uses a different beam codebook that results in a different relationship between Set A and Set B. Cell-B also provides it via an assistance information (e.g., as a component of the Feature F1). Suppose that UE supports the Set A/B relationship of cell-A but not of cell-B. This implies that in the case of handover from cell A to cell B, the UE needs to communicate that UE no longer supports the Feature F1 so that NW may deactivate F1. Likewise, in case of handover from cell B to cell A, the UE needs to communicate that UE now supports the Feature F1 so that NW may activate F1.

To summarize, AI/ML enabled features can be reported by both static and dynamic UE capabilities. Although it is clear from legacy framework that UEs report the supported features with UE capability reporting for static capabilities, it should be studied how to handle dynamic capability of AI/ML enabled features. To this aim, the key consideration is that NW and UE should be on the same page regarding whether a given AI/ML-enabled Feature is supported on the given cell/configuration/scenario.

# Model performance monitoring

We consider UE-side AI/ML models and discuss the corresponding signalling aspects.

UE-side AI/ML Models

The on-device models trained for the purpose of spatial and temporal beam prediction may be subject to performance degradation in certain scenarios, deployments, or use cases, when they get deployed in the field. One main reason for performance degradation could be the fact that UE-side AI/ML models get deployed in an environment that they had not encountered during training. There needs to be a mechanism defined in Spec through which the performance of on-device AI/ML models for beam prediction can be monitored. Along these lines, the following agreement was made in RAN1 110b-e which includes different variations of UE-side model monitoring:

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side Model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/deactivation/switching/fallback operation

- Atl2. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/fallback operation

In light of the following agreement:

Agreement (RAN1 #112) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded

As an example of “configuration/signalling from gNB to UE for performance monitoring”, we consider dedicated reference signals from gNB to UE to help UE in comparing predicted elements from Set A with actual measurements from Set A. The predicted elements could refer to predicted top-K beams from Set A and/or predicted L1-RSRP of the top-K beams from Set A.

#### Proposal 5

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following signalling aspects related to configuration/signalling from gNB to UE for performance monitoring:

Dedicated RS from gNB to UE for performance monitoring

# Conclusions

In this paper, we discussed signalling aspects related to beam prediction use case and made the following proposals:

Proposal 1: For the sub use case BM-Case1 and BM-Case2 and for UE-side AI/ML models, Agenda item 9.2.3.2 should focus on offline training scenario, in which the development and training of the AI/ML model happens offline without the need to involve 3gpp signaling

Proposal 2: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding aspects related to association/mapping of beams within Set A and beams within Set B, study mechanisms to ensure consistency of Set B beams and Set A beams across training and inference, including one or more of the following

- Set size consistency for Set B, Set A: consistency in number of beams and/or associated resources for Set B and Set A, across training and inference

- Order/indexing consistency: consistency in ordering of resources (e.g., resource index consistency) for Set B beams and Set A beams, across training and inference

- QCL consistency: consistency in QCL relationship of Set A beams with respect to Set B beams, across training and inference

- Beam shape consistency: relative pointing direction and beamwidth difference between physical beams with respect to Set A and Set B resources across training and inference should be under predefined tolerances

Proposal 3: For BM-Case1 and BM-Case2 prioritize the study of DL Tx beam prediction.

Proposal 4: For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signalling to report the following information of AI/ML model inference to NW:

- Predicted beam blockage/failure

Proposal 5: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following signalling aspects related to configuration/signalling from gNB to UE for performance monitoring:

- Dedicated RS from gNB to UE for performance monitoring

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R1-2305327, “General aspects of AI-ML framework”, Qualcomm Incorporated.

- R1-2307918, “Evaluation on AI/ML for beam management”, Qualcomm Incorporated.

- R2-2203315, “Introduction of R17 Positioning Enhancements in LPP”, 3GPP TSG-RAN WG2 #117e, Feb. 2022.