3GPP TSG RAN WG1 #114			R1-2307943

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.9.2

Source: 	Qualcomm Incorporated

Title: 	Network verified UE location for NR NTN

Document for:	Discussion/Decision

# Introduction

In RAN1#112bis-e [1], it was agreed to study the UE and gNB RX-TX time differences as in the flowing:

Agreement

For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements:

- Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215.

  - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted

- Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215.

  - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS

- Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4

FFS: One or multiple SRS can be used in determining the arrival time

FFS: Additional enhancement including additional information to be reported, if justified.

In RAN1#113, the following agreements were made regarding Network verified UE location [2]:

Agreement

For network verified UE location in NTN, satellite ephemeris information should be available at the LMF.

Agreement

For network verified UE location in NTN common TA information should be reported at least from gNB to LMF.

Working assumption

In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF.

In RAN#100, the following proposal was agreed [3]:

- TSG-RAN tasks RAN1 to focus its network verified UE location work on Alt1 for the combination of “UE and gNB receive-transmit time difference measurements”:

  - Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215.

    - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted

- If the work at RAN1 is not completed at RAN#101, the nwk verified UE location will be dropped from RAN1

In this contribution, we discuss the accuracy of different options of UE and gNB RX-TX time difference measurements and proposed solutions for single-sat RTT based verification of UE location. We also discuss the advantages of using multiple satellites for network verification of UE location in NTN.

# Single-Sat Multi-RTT

RTT can be measured by UE reporting the RX-TX time difference between a DL reference signal and an UL reference signal as shown in Figure 1. In Figure 1(a), UE sends an SRS after receiving the DL RS from the network and reports the time difference between the DL RS and the SRS. In such case, an RTT measurement, RTT_m is calculated as  T2-T1.    In Figure 1 (b), the SRS is sent before a DL RS and the time difference between the SRS and the DL RS is reported. In such case, RTT_m is calculated as T1-T2.

Figure 1. RTT measurement based on RX-TX time differences at UE and gNB.

The above mechanism is used in TN and can also be used in NTN where satellites are moving. Implementation of the above mechanism, however, could be different due to the following differences between TN and NTN with LEO satellites:

- Subframe duration at UE is constant in TN but time varying in NTN with LEO satellites. In NTN, the duration of a received DL subframe for 15 KHz SCS can be up to 50 ns longer or shorter than 1 ms and the duration of a UL subframe can also be up to 50 ns longer or shorter than 1 ms. When the DL subframe duration is longer than 1 ms the UL subframe duration will be shorter than 1 ms. Note that the maximal and the minimal subframe duration above assumes 0 degree elevation angle for the satellite seen by both UE and gateway.

- In TN, UE TA tends to be constant without network TA commands and UE autonomous TA adjustment. The latter in TN is not expected to happen frequently. In NTN with LEO satellites, UE TA can vary up to 100 ns from one subframe to the next.

For NTN, the RTT_m is obtained as the sum of two propagation delays at different times in Figure 1, i.e., t0 and t1. For verification of UE location, the so-obtained RTT_m can be an approximate of the RTT at time (t0+t1)/2, i.e.,

RTTt0+t12≈RTTm=Propagation Delay t0+Propagation Delay t1,  when t1-t0   is small

The error, |RTT(|RTTt0+t12-RTTm|, by considering both satellite movement and earth rotation is shown to be less than 3 ns when the t1-t0 is less than 200 ms [4].

Unlike in TN where the gNBs are assumed stationary, the high-speed movement of the satellite is exploited to find or verify the UE location. As such, an RTT measurement in NTN needs to be accurately time stamped. Depending on the elevation angle, the difference between the two RTTs at 1ms apart can be up to 100 ns. In the example shown in Figure 1 for NTN, t0, t1, T1, and T2 need to be provided to the LMF to find a RTT and its associated time.

For gNB RX-TX time difference, the time duration of a subframe at the reference point is known to the network. As such Option 2 with the time stamp of SRS is equivalent to Option 4 as shown in Figure 2. In Figure 2, the duration between subframe i and subframe j is known to the Network and LMF as long as indices i and j are known.

Figure 2. Option 2 and Option 4 for the definition of the gNB RX-TX time difference.

Due to satellite movements, arrival time of UL signals with respect to the corresponding DL slot times may change frequently. As a result, a measurement of gNB RX-TX time difference can only be made with respect to one SRS. Hence we have the following proposal.

Proposal 1: For RTT determination in NTN, supports the legacy gNB RX-TX time difference with the following change

  - The start of the uplink subframe #i is determined by the received SRS resource that starts within the subframe.

  - Note: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS.

Alt 2 and Alt 3 for the UE and gNB RX-TX time difference combination provide LMF the needed information for RTT calculation without any error if the measurements are perfect.

Below, we discuss Option 1 and Option 3 for the definition of UE RX-TX timing difference that are used in Alt 1 UE and gNB RX-TX time difference combination.

Option 1: The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX

Where:

  - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP.

For a Transmission Point

  - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time.

  - TUE-TX is the UE transmit timing of the uplink subframe corresponding to subframe #i received from the TP

  - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP.

FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP)

Option-1 UE RX-TX time difference is effectively the TA of subframe #i. If a UE RX-TX time difference report is not tied to an SRS transmission, the error can be far larger than the required accuracy for single-sat multi-RTT based network verification of UE location. For instance, the maximal UL transmit timing error for 15 kHz SCS can be  up to 29*64*Tc, or about 944 ns. Such a large error does not allow location verification with 10 km accuracy in NTN.

If Option-1 UE RX-TX time difference is to be measured with respect to an actual SRS transmission, then there will be an error in the UE received timing of downlink subframe #i, which include the measurement error of PRS, ephemeris propagation error, and GNSS error. This is because the arrival time of subframe #i is predicted based on PRS arrival times before the SRS transmission, the ephemeris propagation, and UE GNSS location. The direct dependency on UE GNSS accuracy makes this design less desirable.

Figure 3. RTT measurement error using a TA report of an SRS and the associated gNB RX-TX time difference. UE report based on calculated arrive time of slot #i which depends on previous PRS measurement, ephemeris propagation, and UE GNSS.

Observation 1: For Option 1 of the definition of UE RX-TX time difference

- If the measurement is not tied to an SRS transmission, the measurement error can be up to 944 ns for 15 KHz SCS according to the existing UL transmit timing requirements.

- If the measurement is tied to an SRS transmission, there exists an error that depends on UE GNSS accuracy.

Option 3: The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options:

- Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i

- Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset

- Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity.

The legacy R17 definition of RX-Tx time difference is illustrated as in Figure 3 [4].  The difference of  #i and #j in NTN can be up to 20 for LEO satellites. In addition, noting that the DL and UL subframe durations at UE are unknown to the LMF, the error between the derived TA based on Option 3 UE RX-TX time difference and the true TA of subframe #i can be large.

To illustrate the issue of Option 3, let’s take an example as shown in Figure 4 and assume the following:

- Perfect downlink synchronization and UL transmit timing.

- The true TA of subframe #i, i.e., the time duration between DL subframe #i  and UL subframe#i is 19.9 ms.

- The elevation angle during the period of concern is constant and the DL subframe duration is 999.95 ms and hence UL subframe duration is 1000.05 ms.

Since the time duration of 19 UL subframes is 20*1000.05=20001 ms, j=i+20 in the figure and the legacy UE RX-TX time difference is

TUE_RX-TX=20001 – 19900= 101 ms

Similarly, it can be calculated that h=i-20 and the time difference between downlink subframe #h and UL subframe #i is 99  ms  in the figure. For the above example, the offset will be 20 for all the three sub options. However, the relationship between the TA of subframe #i, 19.9 ms, and the reports, TUE_RX-TX and the offset, is unclear unless the slot durations are known.

As can be seen from the above, option 3 introduces additional errors even when the measurements were made without any error.

Observation 2:  Option 3 of UE RX-TX time difference is not suitable for RTT measurement unless the slot durations at UE are known to the LMF.

Figure 4. Option 3 of UE RX-TX time difference.

From the above, Alt-1 without additional enhancements is not suitable for RTT measurement in NTN. On the other hand, in Alt-2 and Alt-3 UE and gNB RX-TX time difference measurements and reports need to be coordinated to allow the calculation of RTT. This coordination makes implementation more complicated and is undesirable. As a comparison, UE and gNB RX-TX time difference measurements are decoupled in TN, i.e., UE and gNB have the freedom to choose the PRS and SRS for RX-TX timing difference measurements.

In addition, if LMF is made known about the slot durations at UE, legacy UE RX-TX time difference can be used. To inform the LMF about slot duration at UE, DL Doppler observed by the UE can be reported.

Proposal 2: For RTT determination in NTN, legacy UE RX-TX time difference is reported together with the following:

- Time stamps of the DL slot number and UL slot number of the UE RX-TX time difference

- The DL Doppler of the DL slot associated with the UE RX-TX time difference.

To illustrate the feasibility of the above proposals, consider the example shown in the following figure:

Figure 5. Measurements and reports of UE and gNB RX-TX time difference.

In the above figure, UE reports UE RX-TX time difference and the associated time stamps, downlink slot i, uplink slot j, and DL Doppler D. After slot j, gNB reports gNB RX-TX time difference and the associated slot number j+m. Note that, the UE and gNB RX-TX time difference reports are decoupled, i.e., UE report is associated with DL slot i and UL slot j, whereas gNB report is associated with slot j+m. With the above reports, LMF calculates the RTT with the following steps:

- DL slot duration at UE for a given DL doppler (D in ppm) is calculated as:

    - Tsu(i)=Tsa(i)*(1e6-D)/1e6

where Tsa(i) is the duration of DL slot i at satellite, which can be calculated using common TA parameter.

- TA of slot j is calculated as:

  - TA (j)=(j-i)*Tsu(i)+TUE_RX-TX

- TA of slot (j+m) is calculated as

  - TA(j+m)=TA(j)+2*m*(Tsu(i)-TS), where Ts is the nominal slot duration ,i.e., 1ms for 15 kHz SCS.

- RTT(t) =TA(j+m)+TgNB_RX-TX(j+m) with t being the time of the middle of DL slot j+m and UL slot j+m at ULSRP

In the above, we have assumed that the Doppler during the period between slot i and j+m is constant. In practice, the Doppler is time variant and so do slot durations at UE. The maximal Doppler variation rate occurs at 900 elevation angle. For 600 km orbit, the maximal Doppler variation rate is 0.34 ppm/s or 0.068 ppm per 200 ms.

In addition, the above calculation of RTT is affected by the accuracy of the Doppler report. Since UE is required to achieve 0.1 ppm UL frequency accuracy, we can assume the accuracy of the Doppler report being 0.1 ppm without additional reference signals for Doppler measurement. It can be found that the error due to a Doppler error, De, is

((j-i)*De/1e6+2*m*De/1e6)*Tsa≈((j-i)*De/1e6+2*m*De/1e6)*Ts<2*(j+m-i)*De/1e6*Ts

Assuming the gap between the DL PRS and UL SRS at gNB, j+m-i, is less than 200, i.e., a gap of 200 ms, the above error is less than 40 ns, which is far smaller than 101 ms in the example of Option 3 UE RX-TX time difference without Doppler report shown in Fig.4, and sufficient for network verified UE location.

Observation 3: For the accuracy of the Doppler for RTT calculation in NTN, if the time gap between the associated UE RX-TX time difference and gNB RX-TX time difference is less than 200 ms, existing requirement for  UL transmit frequency error (0.1 ppm) is sufficient.

In the above discussion, we have assumed that the measurements are made based on the beginning of a subframe although both SRS and PRS may not start at the beginning of a subframe. The beginning of a subframe that contains an SRS or PRS may not be known due to the code Doppler. For instance, the beginning of a subframe that contains a PRS at the end may differ by up to 50 ns depending on if the UE considers the Doppler in the calculation. For better accuracy, the derivation of the time of the beginning of a subframe based on received PRS and SRS needs to be defined and known to all parties. One way is to derive the beginning of a subframe based on the received signal and assume 0 Doppler as shown in Figure 6.

Figure 6. The actual arrival time of a subframe vs the calculated arrival time assuming 0 Doppler.

Proposal 3. For UE and gNB RX-TX measurements in NTN, the time of the beginning of a subframe is determined by assuming zero Doppler for symbols before the DL-RS or SRS for positioning in the subframe.

# Multi-Sat Cases

With a LEO constellation with global coverage, a UE will certainly be able to receive from multiple satellites due to the imbalance between UL and DL coverage [4]. Hence, it is desirable to consider UE location verification with multiple satellites to take the advantage of better geometry in triangulation.

Proposal 4: Support network verification of UE location using multiple satellites with the following assumptions:

- UE is connected to one satellite

- UE can detect DL RS from multiple satellites

- The involved satellites are synchronized at uplink synchronization reference points

# Conclusions

In this contribution, we have discussed enhancements to support single-satellite multi-RTT based network verification of UE location and the advantages of using multiple satellites. Observations and proposals are summarized below.

Observation 1: For Option 1 of the definition of UE RX-TX time difference

- If the measurement is not tied to an SRS transmission, the measurement error can be up to 944 ns for 15 KHz SCS according to the existing UL transmit timing requirements.

- If the measurement is tied to an SRS transmission, there exists an error that depends on UE GNSS accuracy.

Observation 2:  Option 3 of UE RX-TX time difference is not suitable for RTT measurement unless the slot durations at UE are known to the LMF.

Observation 3: For the accuracy of the Doppler for RTT calculation in NTN, if the time gap between the associated UE RX-TX time difference and gNB RX-TX time difference is less than 200 ms, existing requirement for  UL transmit frequency error (0.1 ppm) is sufficient.

Proposal 1: For RTT determination in NTN, supports the legacy gNB RX-TX time difference with the following change

  - The start of the uplink subframe #i is determined by the received SRS resource that starts within the subframe.

  - Note: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS.

Proposal 2: For RTT determination in NTN, legacy UE RX-TX time difference is reported together with the following:

- Time stamps of the DL slot number and UL slot number of the UE RX-TX time difference

- The DL Doppler of the DL slot associated with the UE RX-TX time difference.

Proposal 3. For UE and gNB RX-TX measurements in NTN, the time of the beginning of a subframe is determined by assuming zero Doppler for symbols before the DL-RS or SRS for positioning in the subframe.

Proposal 4: Support network verification of UE location using multiple satellites with the following assumptions:

- UE is connected to one satellite

- UE can detect DL RS from multiple satellites

- The involved satellites are synchronized at uplink synchronization reference points

# References

- Chair’s note, RAN1#112bis-e

- Chair’s note, RAN1#113.

- RP 231482, “Way forward for the Rel-18 WID NR_NTN_enh,”

- R1-2212137, “Network verified UE location for NR NTN”, Qualcomm, 3GPP RAN1#111.